# A.6	NR standalone tests with all NR cells in FR1

## A.6.1	SA: RRC_IDLE state mobility

### A.6.1.1	Cell re-selection to NR

#### A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case

##### A.6.1.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements specified in clause 4.2.2.3.

##### A.6.1.1.1.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.1.2-1, A.6.1.1.1.2-2 and A.6.1.1.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.6.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.1.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -130 |  |  | -130 |  |  |
|  |  | 3 | -127 |  |  | -127 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 16 | 13 | 16 | -infinity | 16 | 13 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 2 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 3 | -79 | -82 | -79 | -infinity | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -53.94 | -52.21 | -52.21 | Same as parameters specified in Cell 1 columns- |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -52.21 | -52.21 |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85 | -46.12 | -46.12 |  |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  |  | 60 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.1.1.1.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 34 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to an already detected cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2.2.3-1 in clause 4.2.2.3

Tevaluate, NR_ intra	See Table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

This gives a total of 33.28 s, allow 34 s for the cell re-selection delay to a newly detectable cell and 7.68 s for the cell re-selection delay to an already detected cell in the test case, which we allow 8 s.

#### A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case

##### A.6.1.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.4.

##### A.6.1.1.2.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.6.1.1.2.2-1, A.6.1.1.2.2-2 and A.6.1.1.2.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

Table A.6.1.1.2.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.1.2.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  | 1, 2, 3 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
|  | Neighbour cell |  | 1, 2, 3 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.2.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -84 | -102 | -infinity | -86 |
|  |  | 2 | -84 | -84 | -84 | -102 | -infinity | -86 |
|  |  | 3 | -81 | -81 | -81 | -99 | -infinity | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -55.88 | -68.60 | -70.05 | -57.78 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -55.88 | -68.60 | -70.05 | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -49.79 | -62.50 | -63.96 | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.1.1.2.3	Test Requirements

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, NR_ inter	See Table 4.2.2.4-1 in clause 4.2.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

#### A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

##### A.6.1.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for UE fulfilling low mobility criterion specified in clause 4.2.2.9.2

##### A.6.1.1.3.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.3.2-1, A.6.1.1.3.2-2 and A.6.1.1.3.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.6.1.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.3.2-2: General test parameters for FR1 intra frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 | The UE reselects to cell 1 during T2 period |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 25 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 25 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.3.2-3: Cell specific test parameters for FR1 intra frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |
|  |  | 3 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |
| SSearchDeltaP | dB | 1, 2, 3 | 3 |  | 3 |  |
| TSearchDeltaP | s | 1, 2, 3 | 5 |  | 5 |  |
| Cell_selection_and_ reselection_quality_ measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |
|  | dB | 1, 2, 3 | -3.11 | 2.79 | 2.79 | -3.11 |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note2 | dBm/15 kHz | 1, 2, 3 | -98 |  |  |  |
|  | dB | 1, 2, 3 | 13 | 16 | 16 | 13 |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -82 | -82 | -85 |
|  |  | 2 | -85 | -82 | -82 | -85 |
|  |  | 3 | -82 | -79 | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -52.21 | -52.21 | specified in Cell 1 columns- |  |
|  | dBm/9.36 MHz | 2 | -52.21 | -52.21 |  |  |
|  | dBm/38.16 MHz | 3 | -46.12 | -46.12 |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  | 60 |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.1.1.3.3	Test Requirements

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected cell shall be less than 17 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell shall be less than 17 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detected cell can be expressed as: Tevaluate,NR_Intra + TSI-NR,

Where:

Tevaluate,NR_Intra	See Table 4.2.2.9.2-1 in clause 4.2.2.9.2 for reselection to Cell 2 during T1 with UE fulfilling low mobility criterion. 15.36s.

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s, allow 17 s for the cell re-selection delay to an already detected cell for UE fulfilling low mobility criterion in the test case.

#### A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion

##### A.6.1.1.4.1	Test Purpose and Environment

This test is to verify the relaxed cell re-selection requirement for UEs configured with not-at-cell edge criterion specified in clause 4.2.2.9.3.

##### A.6.1.1.4.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.4.2-1, A.6.1.1.4.2-2 and A.6.1.1.4.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas.

Table A.6.1.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.4.2-2: General test parameters for FR1 intra frequency NR cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active Cell |  | 1, 2, 3 | Cell1 | The UE camps on Cell 1 in the initial phase |
|  | Neighbour Cells |  | 1, 2, 3 | Cell2 |  |
| T1 end condition | Active Cell |  | 1, 2, 3 | Cell2 | The UE shall fulfil the not-at-cell edge criterion and reselect to cell 2 during T1 period during T1. |
|  | Neighbour Cells |  | 1, 2, 3 | Cell1 |  |
| T2 end condition | Active Cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to Cell 1 during T2 |
|  | Neighbour Cells |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between Cells |  |  | 1 | 3 ms | Asynchronous Cells |
|  |  |  | 2 | 3 s | Synchronous Cells |
|  |  |  | 3 | 3 s | Synchronous Cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all Cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 20 | T1 needs to be defined so that Cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 20 | T2 needs to be defined so that Cell re-selection reaction time is taken into account. |

Table A.6.1.1.4.2-3: Cell specific test parameters for FR1 intra frequency NR cell re-selection test case in AWGN for UE fulfilling not-at-cell edge criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |
|  |  | 3 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |
|  | dB | 1 | -3.11 | 2.79 | 2.79 | -3.11 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 13 | 16 | 16 | 13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -82 | -82 | -85 |
|  |  | 2 | -85 | -82 | -82 | -85 |
|  |  | 3 | -82 | -79 | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -52.21 | -52.21 | -52.21 | -52.21 |
|  | dBm/9.36 MHz | 2 | -52.21 | -52.21 | -52.21 | -52.21 |
|  | dBm/38.16 MHz | 3 | -46.12 | -46.12 | -46.12 | -46.12 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  | 60 |  |
| SsearchThresholdP | dB | 1, 2, 3 | 50 | Not sent | Not sent | 50 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both Cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other Cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.1.1.4.3	Test Requirements

The cell re-selection delay to an already detected cell for UE configured with cellEdgeEvaluation criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected cell for UE configured with cellEdgeEvaluation criterion shall be less than 17s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detected cell for UE configured with relaxed measurement criterion can be expressed as: Tevaluate,NR_Intra + TSI-NR,

Where:

Tevaluate,NR_Intra	See Table 4.2.2.9.3-1 for UE fulfilling not-at-cell edge criterion in clause 4.2.2.9.3.

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a Cell; 1280ms is assumed in this test case.

This gives a total of 16.64s, allow 17s for the cell re-selection delay to an already detected cell for UE fulfilling  not-at-cell edge criterion in the test case.

#### A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

##### A.6.1.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.10.2, for UE fulfilling low mobility relaxed measurement criterion.

##### A.6.1.1.5.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.6.1.1.5.2-1, A.6.1.1.5.2-2 and A.6.1.1.5.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1]. So, Cell 2 and Cell 1 configure the UE as follows:

lowMobilityEvalutation [2] criterion is configured according to the parameters listed in Table A.6.1.1.5.2-3;

cellEdgeEvaluation [2] criterion is not configured;

combineRelaxedMeasCondition [2] is not configured;

Table A.6.1.1.5.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.1.5.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase, it fulfills Low Mobility relaxation measurements criterion, and during T1 period the UE reselects to cell 1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 25 s | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 25 s | T2 is defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.5.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |  |
|  |  | 3 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | -4 | 12 |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | -4 |  | 12 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -102 |  | -86 |
|  |  | 2 | -84 | -84 | -102 |  | -86 |
|  |  | 3 | -81 | -81 | -99 |  | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -62.50 |  | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 |  | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  | Not sent |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  | 50 |  |  |
| SSearchDeltaP | dB | 1, 2, 3 | 3 |  | 3 |  |  |
| TSearchDeltaP | s | 1, 2, 3 | 5 |  | 5 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.1.1.5.3	Test Requirements

The cell reselection delay to an already detected lower priority cell for UE fulfilling low mobility relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to a lower priority cell for UE fulfilling low mobility relaxed measurements shall be less than 17 s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling low mobility relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to an already detected higher priority cell for UE fulfilling low mobility relaxed measurements shall be less than 17 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a known lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Tevaluate, NR_ inter	See Table 4.2.2.10.2-1 in clause 4.2.2.10.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s, allow 17s  for the cell re-selection delay to an already detected lower priority cell and 16.64s for the cell re-selection delay to an already detected higher priority cell, which we allow 17s for UE fulfilling low mobility relaxed measurements in the test case.

#### A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion

##### A.6.1.1.6.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.10.3, for UE fulfilling not-at-cell edge relaxed measurement criterion.

##### A.6.1.1.6.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.6.1.1.6.2-1, A.6.1.1.6.2-2 and A.6.1.1.6.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE not-at-cell edge as defined in clause 5.2.4.9.2 in [1]. So, Cell 2 and Cell 1configures the UE as follows:

cellEdgeEvaluation [2] criterion is configured according to the parameters listed in Table A.6.1.1.5.2-3;

lowMobilityEvalutation [2] criterion is not configured;

combineRelaxedMeasCondition [2] is not configured;

Table A.6.1.1.6.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.1.6.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase, it fulfills Not-at-cell edge relaxation measurements criterion, and during T1 period the UE reselects to cell 1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T2 |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB Configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 20 s | T1 is defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 20 s | T2 is defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.6.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN for UE fulfilling not-at-cell edge criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |  |
|  |  | 3 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | -4 | 12 |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | -4 |  | 12 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -102 |  | -86 |
|  |  | 2 | -84 | -84 | -102 |  | -86 |
|  |  | 3 | -81 | -81 | -99 |  | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -68.60 |  | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -62.50 |  | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 |  | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  | Not sent |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  | 50 |  |  |
| SSearchThresholdP | dB | 1, 2, 3 | 50 |  | 50 |  |  |
| SSearchThresholdQ | s | 1, 2, 3 | Not Configured |  |  |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.1.1.6.3	Test Requirements

The cell reselection delay to an already detected lower priority cell for UE fulfilling not-at-cell edge relaxed measurements is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected lower priority cell for UE fulfilling not-at-cell edge relaxed measurements shall be less than 17 s.

The cell reselection delay to an already detected higher priority cell for UE fulfilling not-at-cell-edge relaxed measurements is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to an already detected higher priority cell for UE fulfilling not-at-cell-edge  relaxed measurements shall be less than 17 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Tevaluate, NR_ inter	See Table 4.2.2.10.3-1 in clause 4.2.2.10

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s , allow 17s for the cell re-selection delay to an already detected lower priority cell and 16.64s for the cell re-selection delay to an already higher priority cell, which we allow 17s  for UE fulfilling not-at-cell edge relaxed measurements in the test case.

#### A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16

##### A.6.1.1.7.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for UE configured with highSpeedMeasFlag-r16 specified in clause 4.2.2.3.

##### A.6.1.1.7.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.1.x-1, A.6.1.1.1.x-2 and A.6.1.1.1.x-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. highSpeedMeasFlag-r16 is broadcasted to UE.

Table A.6.1.1.7.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.7.2-2: General test parameters for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial | Active cell |  | 1, 2, 3 | Cell1 |  |
| condition | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.32 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.7.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  |  | SS-RSRP |  |  |
|  | dB | 1 | 16 | -3.11 | 2.79 |  | -infinity | 2.79 | -3.11 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
|  | dB | 1 | 16 | 13 | 16 |  | -infinity | 16 | 13 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82 | -85 | -82 |  | -infinity | -82 | -85 |
|  |  | 2 | -82 | -85 | -82 |  | -infinity | -82 | -85 |
|  |  | 3 | -79 | -82 | -79 |  | -infinity | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -53.94 | -52.21 | -52.21 |  | Same as parameters specified in Cell 1 columns |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -52.21 | -52.21 |  |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85 | -46.12 | -46.12 |  |  |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 |  | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  |  |  | 60 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  | AWGN 1944Hz Note4 |  |  |  |
| Propagation Condition |  | 3 | AWGN |  |  | AWGN 3334Hz Note5 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |  |  |  |

##### A.6.1.1.7.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 4 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell shall be less than 3 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2.2.3-2 in clause 4.2.2.3

Tevaluate, NR_ intra	See Table 4.2.2.3-2 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

This gives a total of 3.84s, allow 4s for the cell re-selection delay to a newly detectable cell and 2.24 s for the cell re-selection delay to an already detected cell in the test case, which we allow 3 s.

### A.6.1.2	Inter-RAT E-UTRAN cell re-selection

#### A.6.1.2.1	Cell reselection to higher priority E-UTRAN

##### A.6.1.2.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements specified in clause 4.2.2.5 when the E-UTRAN cell is of higher priority.

##### A.6.1.2.1.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.1.2-1, A.6.1.2.1.2-2, A.6.1.2.1.2-3 and A.6.1.2.1.2-4. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. NR cell 1 is already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of higher priority than cell 1.

Table A.6.1.2.1.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.1.2-2: General test parameters for NR to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase and during T2 period the UE reselects to cell 2. |
| T2 end | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | 2 during T2. |
| T3 end | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell |
| condition | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | 1 during T3 for iteration of the tests. |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 | 4 |  |
| E-UTRAN PRACH |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in |
| configuration index |  |  | 4, 5, 6 | 4 | TS 36.211 [23] |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | >7 | During T1, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.1.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH parameters |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| Dedicated CORESET parameters |  | 1, 4 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| SSB parameters |  | 1, 4 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.2 FR1 |  |  |
| NR SMTC parameters |  | 1, 4 | SMTC.2 |  |  |
|  |  | 2, 5 | SMTC.1 |  |  |
|  |  | 3, 6 | SMTC.1 |  |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
|  | dBm/SCS | 1, 4 | -98 |  |  |
|  |  | 2, 5 | -98 |  |  |
|  |  | 3, 6 | -95 |  |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| SS-RSRP | dBm/SCS | 1, 4 | -84 | -84 | -84 |
|  |  | 2, 5 | -84 | -84 | -84 |
|  |  | 3, 6 | -81 | -81 | -81 |
|  | dB | 1, 4 | 14 | 14 | 14 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4 | 14 | 14 | 14 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -55.88 | -55.88 | -55.88 |
|  | dBm/9.36 MHz | 2, 5 | -55.88 | -55.88 | -55.88 |
|  | dBm/38.16 MHz | 3, 6 | -49.79 | -49.79 | -49.79 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Threshx, highP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |  |

Table A.6.1.2.1.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |
| RSRP | dBm/15 KHz | -infinity |  | -86 | -102 |
|  | dB | -infinity |  | 12 | -4 |
|  | dB | -infinity |  | 12 | -4 |
| TreselectionEUTRAN | S | 0 |  |  |  |
| SnonintrasearchP | dB | Not sent |  |  |  |
| Threshx, highP | dB | 48 |  |  |  |
| Threshserving, lowP | dB | 44 |  |  |  |
| Threshx, lowP (Note 2) | dB | 50 |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, Low which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |  |  |

##### A.6.1.2.1.3	Test Requirements

The cell reselection delay to a higher priority E-UTRAN cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, E-UTRAN	See Table 4.2.2.5-1 in clause 4.2.2.5

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority E-UTRAN cell.

#### A.6.1.2.2	Cell reselection to lower priority E-UTRAN

##### A.6.1.2.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements specified in clause 4.2.2.5 when the E-UTRAN cell is of lower priority.

##### A.6.1.2.2.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.2.2-1, A.6.1.2.2.2-2, A.6.1.2.2.2-3 and A.6.1.2.2.2-4. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

Table A.6.1.2.2.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.2.2-2: General test parameters for NR to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 534 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 |  |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.2.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 5 | SMTC.1 |  |
|  |  | 3, 6 | SMTC.1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 4 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -51.69 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, lowP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table A.6.1.2.2.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP (Note 2) | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.6.1.2.2.3	Test Requirements

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN	See Table 4.2.2.5-1 in clause 4.2.2.5

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 7.68 s, allow 8 s for the cell re-selection delay to a lower priority E-UTRAN cell.

#### A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion

##### A.6.1.2.3.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection when UE fulfills the low mobility criterion specified in clause 4.2.2.11.2 and the E-UTRAN cell is of lower priority.

##### A.6.1.2.3.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.3.2-1, A.6.1.2.3.2-2, A.6.1.2.3.2-3 and A.6.1.2.3.2-4. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1]. So, Cell 1 configures the UE as follows:

-	lowMobilityEvalutation [2] criterion is configured according to the parameters listed in Table A.6.1.2.3.2-3;

-	cellEdgeEvaluation [2] criterion is not configured;

-	combineRelaxedMeasCondition [2] is not configured

Table A.6.1.2.3.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.3.2-2: General test parameters for NR to E-UTRAN cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase, it fulfills Low Mobility relaxation measurements criterion, and during T1 period the UE reselects to cell 2 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 with higher priority during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.64 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 24 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 24 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.3.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC pattern 2 |  |
|  |  | 2, 5 | SMTC pattern 1 |  |
|  |  | 3, 6 | SMTC pattern 1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 4 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -51.69 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, lowP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| SSearchDeltaP | dB | 1, 2, 3, 4, 5, 6 | 3 |  |
| TSearchDeltaP | s | 1, 2, 3, 4, 5, 6 | 5 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, low which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table A.6.1.2.3.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | Not sent |  |
| Threshx, high (Note 2) | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.6.1.2.3.3	Test Requirements

time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCConnectionRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 17  s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN	See Table 4.2.2.11.2-1 in clause 4.2.2.11.2

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 15.36 (Tevaluate, E-UTRAN) + 1.28 (TSI-E-UTRA) = 16.64 s, allow 17 s for the cell re-selection delay to a lower priority E-UTRAN cell for UE fulfilling low mobility criterion.

#### A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion

##### A.6.1.2.4.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements when UE fulfills  not-at-cell edge criterion specified in clause 4.2.2.11.3 when the E-UTRAN cell is of lower priority.

##### A.6.1.2.4.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.4.2-1, A.6.1.2.4.2-2, A.6.1.2.4.2-3 and A.6.1.2.4.2-4. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

As specified in the Test Purpose, the UE is configured with the relaxed measurement criterion for UE with not-at-cell edge defined in clause 5.2.4.9.2 in [1]. So, Cell 1 configures the UE as follows:

-	lowMobilityEvalutation [2] criterion is not configured;

-	cellEdgeEvaluation [2] criterion is configured according to the parameters listed in Table A.6.1.2.4.2-3;

-	combineRelaxedMeasCondition [2] is not configured

Table A.6.1.2.4.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.4.2-2: General test parameters for NR to E-UTRAN cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase and fulfill the not at the cell edge criteria. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.64 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 4, 5, 6 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 24 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 24 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.4.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC |  | 1, 4 | CR.1.1 FDD |  |
| configuration |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC |  | 1, 4 | CCR.1.1 FDD |  |
| configuration |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC pattern 2 |  |
|  |  | 2, 5 | SMTC pattern 1 |  |
|  |  | 3, 6 | SMTC pattern 1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 4 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| SSearchThresholdP | dB | 1, 2, 3, 4, 5, 6 | 32 | 32 |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -51.69 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 60 |  |
| Threshx, highP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table A.6.1.2.4.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | Not sent |  |
| Threshx, high (Note 2) | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.6.1.2.4.3	Test Requirements

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCConnectionRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 17s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN	See Table 4.2.2.11.3-1 in clause 4.2.2.11.3

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s, allow 17 s for the cell re-selection delay to a lower priority E-UTRAN cell for UE fulfilling not-at-cell edge criterion.

#### A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16

##### A.6.1.2.5.1	Test Purpose and Environment

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements for UE configured with highSpeedMeasFlag-r16 specified in clause 4.2.2.5 when the E-UTRAN cell is of lower priority.

##### A.6.1.2.5.2	Test Parameters

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.6.1.2.5.2-1, A.6.1.2.5.2-2, A.6.1.2.5.2-3 and A.6.1.2.5.2-4. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1. The E-UTRAN cell 2 is indicated by NR cell 1 as an HST cell.

Table A.6.1.2.5.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.2.5.2-2: General test parameters for NR to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase. |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.32 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 77 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.2.5.2-3: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 4 | CR.1.1 FDD |  |
|  |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 4 | CCR.1.1 FDD |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC pattern 2 |  |
|  |  | 2, 5 | SMTC pattern 1 |  |
|  |  | 3, 6 | SMTC pattern 1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 4 | -102 | -86 |
|  |  | 2, 5 | -102 | -86 |
|  |  | 3, 6 | -99 | -83 |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -4 | 12 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -51.69 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Threshx, highP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN 1944HzNote3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, highP which is included in NR system information, and is a threshold for the E-UTRA target cell. Note 3: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. |  |  |  |  |

Table A.6.1.2.5.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP (Note 2) | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP | dB | 50 |  |
| Propagation Condition |  | AWGN 1944Hz |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, highP which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.6.1.2.5.3	Test Requirements

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 3 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN_HST + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN_HST	See Table 4.2.2.5-2 in clause 4.2.2.5

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 2.24 s, allow 3 s for the cell re-selection delay to a lower priority E-UTRAN cell.

## A.6.2	SA: RRC_INACTIVE state mobility

## A.6.3	RRC_CONNECTED state mobility

### A.6.3.1	Handover

#### A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell

##### A.6.3.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements specified in clause 6.1.1.2.

##### A.6.3.1.1.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.1.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.1.2-2, and A.6.3.1.1.2-3.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.6.3.1.1.2-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.1.2-2: General test parameters Intra-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T3 |  | T1 |  | T2 |  | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |  | 1 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | dB | 8 | -3.3 |  | -3.3 |  | -Infinity |  | 2.36 |  | 2.36 |  |
|  |  |  | dB | 8 | 8 |  | 8 |  | -Infinity |  | 11 |  | 11 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -90 |  | -Infinity |  | -87 |  | -87 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -87 |  | -Infinity |  | -84 |  | -84 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.06 |  | -57.06 |  | -61.41 |  | -57.06 |  | -57.06 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -50.96 |  | -50.96 |  | -55.31 |  | -50.96 |  | -50.96 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 62 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

#### A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell

##### A.6.3.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements specified in clause 6.1.1.2.

##### A.6.3.1.2.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.2.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.2.2-2, and A.6.3.1.2.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.6.3.1.2.2-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.2.2-2: General test parameters Intra-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.6.3.1.2.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf. 2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 8 | -0.64 |  | -Infinity |  | -0.64 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 8 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -Infinity |  | -90 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -Infinity |  | -87 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -58.71 |  | -61.41 |  | -58.71 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -52.60 |  | -55.31 |  | -52.60 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 92 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 82 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 92 ms.

#### A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell

##### A.6.3.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 inter frequency handover requirements specified in clause 6.1.1.2.

##### A.6.3.1.3.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.3.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.3.2-2, and A.6.3.1.3.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.6.3.1.3.2-1: Inter-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.3.2-2: General test parameters Inter-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.6.3.1.3.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 |  | -Infinity |  | -93 |  |
|  | Config 3 |  | dBm/SCS | -91 | -91 |  | -Infinity |  | -90 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -63.85 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -57.75 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.3.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 122 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 132 ms.

#### A.6.3.1.4	 SA NR - E-UTRAN handover

##### A.6.3.1.4.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1. This test shall verify the NR to E-UTRAN handover requirements as specified in clause 6.1.2.1.

The test comprises of one NR carrier and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.6.3.1.4-1. General test parameters are provided in Table A.6.3.1.4-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.6.3.1.4-3 and A.6.3.1.4-4 respectively.

Table A.6.3.1.4-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.4-2: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | As specified in Table A.6.3.1.4-3 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2EUTRAN |  | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.4-3: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |
| Duplex mode |  |  | 1, 4 | FDD |  |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1, 4 | TRS.1.1 FDD |  |  |
|  |  |  | 2, 5 | TRS.1.1 TDD |  |  |
|  |  |  | 3, 6 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2, 4, 5 | -96 |  |  |
|  |  |  | 3, 6 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -100 | -104 | -100 |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -100 | -104 | -100 |
|  |  |  | 3, 6 | -97 | -101 | -97 |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | 12 | 0 | -4 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | 12 | 0 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -88 | -104 | -104 |
|  |  |  | 3, 6 | -85 | -101 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.78 | -73.04 | -70.59 |
|  |  | dBm/38.16 MHz | 3, 6 | -53.68 | -66.9448 | -64.49 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.3.1.4-4: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |  |
|  |  | 4, 5, 6 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8 | 78 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 78 | 78 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -90 | -90 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -90 | -90 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -67.21 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

##### A.6.3.1.4.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 85 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 6.1.2.1.

Tinterrupt = 35 ms in the test; Tinterrupt is defined in clause 6.1.2.1.

This gives a total of 85 ms.

#### A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell

##### A.6.3.1.5.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1. This test shall verify the NR to E-UTRAN handover requirements for the case when the target E-UTRAN cell is unknown as specified in clause 6.1.2.1.

The test comprises of one NR carrier and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable. No Gap pattern shall be configured.

A RRC message implying handover shall be sent to the UE during period T1. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.6.3.1.5-1. General test parameters are provided in Table A.6.3.1.5-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.6.3.1.5-3 and A.6.3.1.5-4 respectively.

Table A.6.3.1.5-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.5-2: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

Table A.6.3.1.5-3: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  | 1, 4 | FDD |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |
| TRS configuration |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  | 3, 6 | TRS.1.2 TDD |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  |  | 3, 6 | -95 |  |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | 0 | 0 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -98 | -98 |
|  |  |  | 3, 6 | -95 | -95 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -67.04 | -67.04 |
|  |  | dBm/38.16 MHz | 3, 6 | -60.94 | -60.94 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.6.3.1.5-4: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |
|  |  | 4, 5, 6 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -91 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -91 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -70.22 | -62.43 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.6.3.1.5.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 165 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 6.1.2.1.

Tinterrupt = 115 ms in the test; Tinterrupt is defined in clause 6.1.2.1.

This gives a total of 165 ms.

#### A.6.3.1.6	 SA NR - UTRAN FDD handover

##### A.6.3.1.6.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT UTRAN FDD handover when operating in standalone (SA) operation with PCell in FR1. This test shall verify the NR to UTRAN FDD handover requirements as specified in clause 6.1.2.2.1.

The test comprises of one NR carrier and one UTRA FDD carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN FDD neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.6.3.1.6-1. General test parameters are provided in Table A.6.3.1.6-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.6.3.1.6-3 and A.6.3.1.6-4 respectively.

Table A.6.3.1.6-1: Supported test configurations for SA inter-RAT UTRAN FDD handover tests

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, UTRAN FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.6-2: General test parameters for SA inter-RAT UTRAN FDD handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  |  | 2 | 1 UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| Inter-RAT (UTRAN FDD) measurement quantity |  |  | CPICH Ec/N0 |  |
| b2-Threshold1 |  | dBm | As specified in Table A.6.3.1.6-3 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2UTRA-FDD |  | dB | -18 | Absolute UTRAN CPICH Ec/Io threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.6.3.1.6-3: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3 | 1 |  |  |
| Duplex mode |  |  | 1 | FDD |  |  |
|  |  |  | 2, 3 | TDD |  |  |
| TDD Configuration |  |  | 2 | TDDConf.1.1 |  |  |
|  |  |  | 3 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1 | SR.1.1 FDD |  |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1 | CR.1.1 FDD |  |  |
|  |  |  | 2 | CR.1.1 TDD |  |  |
|  |  |  | 3 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1 | TRS.1.1 FDD |  |  |
|  |  |  | 2 | TRS.1.1 TDD |  |  |
|  |  |  | 3 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2 | SSB.1 FR1 |  |  |
|  |  |  | 3 | SSB.2 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2 | -96 |  |  |
|  |  |  | 3 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3 | -100 |  |  |
| NocNote2 |  | dBm/SCS | 1, 2, | -100 |  |  |
|  |  |  | 3 | -97 |  |  |
| Ês/Noc |  | dB | 1, 2, 3 | 12 | -4 | -4 |
| Ês/IotNote3 |  | dB | 1, 2, 3 | 12 | -4 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -88 | -104 | -104 |
|  |  |  | 3 | -85 | -101 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2 | -59.78 | -70.59 | -70.59 |
|  |  | dBm/38.16 MHz | 3 | -53.68 | -64.49 | -64.49 |
| Propagation condition |  |  | 1, 2, 3 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.3.1.6-4: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 2)

| Parameter | Unit | Cell 2 (UTRA) |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| UTRA RF Channel Number |  | 2 |  |  |
| CPICH_Ec/Ior | dB | -10 |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |
| DCH_Ec/Ior | dB | N/A | N/A | Note 1 |
| OCNS_Ec/Ior | dB | -0.941 | 0.941 | Note 2 |
|  | dB | -infinity | -1.8 | -1.8 |
|  | dBm/3,84 MHz | -70 | -70 | -70 |
| CPICH_Ec/Io | dB | -infinity | -14 | -14 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: The DPCH level is controlled by the power control loop Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior . |  |  |  |  |

##### A.6.3.1.6.2	Test Requirements

The UE shall start to transmit the UL DPCCH to Cell 2 less than 190 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms, which is specified in clause 5.3.1.1.1.

Tinterrupt = 140 ms in the test; Tinterrupt is defined in clause 5.3.1.1.2. 	This gives a total of 190 ms.

#### A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1

##### A.6.3.1.7.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency DAPS handover requirements in synchronous scenario specified in clause 6.1.3.2.

##### A.6.3.1.7.2	Test Parameters

Supported test configurations are shown in Table A.6.3.1.7.2-1. Both handover delay and interruption length are tested by using the parameters in Table A.6.3.1.7.2-2, and A.6.3.1.7.2-3.The test consists of five successive time periods, with time durations of T1, T2, T3, T4, and T5 respectively.

Before the start of T1, the UE is connected to the cell1 and not aware of the cell2. During T1, the UE does not have any timing information of the cell2.

Starting T2, the cell2 becomes detectable. During T2, the UE performs cell detection and measurements on the cell2 and shall send event report to the network. After receiving the event report A3, the network sends a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing DAPS handover command is sent to the UE. During T3, UE shall be able to perform random access, DL reception or UL transmission in the cell2 while the DL scheduling and UL feedback in the cell1 shall be avoided. After successful RACH procedure of the cell2, UE is scheduled with PDSCH from cell1 and cell2 in alternative TTIs where both cell1 and cell2 belong to the same TAG. In the end the network sends a RRC message implying cell1 release to the UE. During T3, the handover delay Dhandover1 for target cell addition need to be verified.

The start of T4 is the instant when the last TTI containing cell1 release command is sent to the UE. During T4, the UE shall accomplish the release actions within Dhandover2.

Starting T5, the UE stops sending the periodical CSI report to the cell1.

Table A.6.3.1.7.2-1: Intra-frequency DAPS handover in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.7.2-2: General test parameters synchronous Intra-frequency DAPS handover in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | Dhandover2 | DHandover2 is defined in clause 6.1.3.2.1 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.7.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency DAPS handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 |  | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  | 1 |  |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | dB | 8 | -1.5 | -1.5 | -1.5 | -1.5 | -Infinity | 0.36 | 0.36 | 0.36 | 0.36 |  |
|  |  |  | dB | 8 | 8 | 8 | 8 | 8 | -Infinity | 9 | 9 | 9 | 9 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 | -90 | -90 | -90 | -Infinity | -89 | -89 | -89 | -89 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 | -87 | -87 | -87 | -Infinity | -86 | -86 | -86 | -86 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -58.21 | -58.21 | -58.21 | -58.21 | -61.41 | -58.21 | -58.21 | -58.21 | -58.21 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -52.11 | -52.11 | -52.11 | -52.11 | -55.31 | -52.11 | -52.11 | -52.11 | -52.11 |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.7.3 Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The target cell add delay Dhandover1 can be expressed as: TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.2.2.

If the target cell is known, then Tsearch = 0 ms

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

After successful RACH to cell 2 and until the start of time period T4, UE shall be able to receive PDSCH alternatively from cell 1 and cell 2. UE is not expected to transmit UL to both cell 1 and cell 2 in the same TTI.

The UE shall release cell 1 less than Dhandover2 = (TRRC_procedure + Tinterrupt2) from the beginning of time period T4.

NOTE:	Dhandover2 is defined in clause 6.1.3.2.1.

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.2.2.

UE shall not report CSI to cell 1 during T5.

#### A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1

##### A.6.3.1.8.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency DAPS handover requirements in asynchronous scenario specified in clause 6.1.3.2.

##### A.6.3.1.8.2	Test Parameters

Supported test configurations are shown in Table A.6.3.1.8.2-1. Both handover delay and interruption length are tested by using the parameters in Table A.6.3.1.8.2-2, and A.6.3.1.8.2-3.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4, and T5 respectively.

Before the start of T1, the UE is connected to the cell1 and not aware of the cell2. During T1, the UE does not have any timing information of the cell2.

Starting T2, the cell2 becomes detectable. During T2, the UE performs cell detection and measurements on the cell2 and shall send event report to the network. After receiving the event report A3, the network sends a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing DAPS handover command is sent to the UE. During T3, UE shall be able to perform random access, DL reception or UL transmission in the cell2 while the DL scheduling and UL feedback in the cell1 shall be avoided. After successful RACH procedure of the cell2, UE is scheduled with PDSCH from cell1 and cell2 in alternative TTIs where both cell1 and cell2 belong to the same TAG. In the end the network sends a RRC message implying cell1 release to the UE. During T3, the handover delay Dhandover1 for target cell addition needs to be verified.

The start of T4 is the instant when the last TTI containing cell1 release command is sent to the UE by cell2. During T4, the UE shall accomplish the release actions within Dhandover2.

Starting T5, the UE stops sending the periodical CSI report to the cell1.Table A.6.3.1.8.2-1: Intra-frequency DAPS handover in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.8.2-2: General test parameters Intra-frequency asynchronous DAPS handover in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 7 s | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | Dhandover2 | DHandover2 is defined in clause 6.1.3.2.1 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.8.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency DAPS handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 |  | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  | 1 |  |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | dB | 8 | -1.5 | -1.5 | -1.5 | -1.5 | -Infinity | 0.36 | 0.36 | 0.36 | 0.36 |  |
|  |  |  | dB | 8 | 8 | 8 | 8 | 8 | -Infinity | 9 | 9 | 9 | 9 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 | -90 | -90 | -90 | -Infinity | -89 | -89 | -89 | -89 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 | -87 | -87 | -87 | -Infinity | -86 | -86 | -86 | -86 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -58.21 | -58.21 | -58.21 | -58.21 | -61.41 | -58.21 | -58.21 | -58.21 | -58.21 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -52.11 | -52.11 | -52.11 | -52.11 | -55.31 | -52.11 | -52.11 | -52.11 | -52.11 |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.8.3 Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The target cell add delay Dhandover1 can be expressed as: TRRC_procedure + Tsearch + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tsearch, TIU, Tprocessing, T∆ and Tmargin are defined in clause 6.1.1.2.2.

If the target cell is known, then Tsearch = 0 ms

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

After successful RACH to cell 2 and until the start of time period T4, UE shall be able to receive PDSCH alternatively from cell 1 and cell 2. UE is not expected to transmit UL to both cell 1 and cell 2 in the same TTI.

The UE shall release cell 1 less than Dhandover2 = (TRRC_procedure + Tinterrupt2) from the beginning of time period T4.

NOTE:	Dhandover2 is defined in clause 6.1.3.2.1.

TRRC_procedure  = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt2 is defined in clause 6.1.3.2.2.

UE shall not report CSI to cell 1 during T5.

#### A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1

##### A.6.3.1.9.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra-band inter-frequency synchronous DAPS handover requirements specified in clause 6.1.3.2.

##### A.6.3.1.9.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.9.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.9.2-2, and A.6.3.1.9.2-3.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. The test scenario comprises of two carriers and one cell on each carrier Gap pattern ID gp0 as specified in Table 9.1.2-1 is configured before T2 in the test case.

Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A3. After receiving the Event A3, the test system shall send a RRC message implying DAPS handover to the UE.

T3 is defined as the end of the last TTI containing the RRC message implying DAPS handover. During T3 UE shall be able to perform random access to cell 2. Cell 1 is continuously scheduled in DL during T3. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perfrom DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. At the end of T3 cell 2 shall send an RRC message implying cell 1 release command.

T4 is defined as the end of the last TTI containing the RRC message implying DAPS handover. Cell 2 is continuously scheduled in DL during T4. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stops to send CSI report to the source cell. And the test system shall observe the periodic reporting of CSI for cell 1 during T5.

Table A.6.3.1.9.2-1: Intra-band inter-frequency synchronous DAPS handover in SA for FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.9.2-2: General test parameters for intra-band inter-frequency synchronous DAPS  handover test in SA for FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 0 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | 10 + Tinterrupt2 | Tinterrupt2 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-5 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.9.2-3: Cell specific test parameters for intra-band inter-frequency synchronous DAPS  handover test in SA for FR1

| Parameter |  |  | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  | 2 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |  |  |  |
|  |  |  | dB | 8 | 8 | 8 | 8 | 8 | -Infinity | 8 | 8 | 8 | 8 |
|  |  |  | dB | 8 | 8 | 8 | 8 | 8 | -Infinity | 8 | 8 | 8 | 8 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 | -90 | -90 | -90 | -Infinity | -90 | -90 | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 | -87 | -87 | -87 | -Infinity | -87 | -87 | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -61.41 | -61.41 | -61.41 | -61.41 | -70.05 | -61.41 | -61.41 | -61.41 | -61.41 |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -55.31 | -55.31 | -55.31 | -55.31 | -63.94 | -55.31 | -55.31 | -55.31 | -55.31 |
| Propagation condition |  |  | - | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.9.3 Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

During T3 UE is allowed to cause Tinterrupt1 interruption to cell 1. Tinterrupt1 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-2. When UE is transmitting PRACH preamble to cell 2, interruption to cell 1 is allowed.

During T4 UE is allowed to cause Tinterrupt2 interruption to cell 1. Tinterrupt2 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-5.

UE shall finish cell 1 release in T4 and shall not send any CSI reports to cell 1 during T5.

The rate of correct handovers observed during repeated tests shall be at least 90%.

#### A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1

##### A.6.3.1.10.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra-band inter-frequency asynchronous DAPS handover requirements specified in clause 6.1.3.2.

##### A.6.3.1.10.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.10.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.10.2-2, and A.6.3.1.10.2-3.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. The test scenario comprises of two carriers and one cell on each carrier Gap pattern ID gp0 as specified in Table 9.1.2-1 is configured before T2 in the test case.

Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A3. After receiving the Event A3, the test system shall send a RRC message implying DAPS handover to the UE.

T3 is defined as the end of the last TTI containing the RRC message implying DAPS handover.During T3 UE shall be able to perform random access to cell 2. Cell 1 is continuously scheduled in DL during T3. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perfrom DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. At the end of T3 cell 2 shall send an RRC message implying cell 1 release command.

T4 is defined as the end of the last TTI containing the RRC message implying DAPS handover. Cell 2 is continuously scheduled in DL during T4. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stops to send CSI report to the source cell. Andthe test system shall observe the periodic reporting of CSI for cell 1 during T5.

Table A.6.3.1.10.2-1: Intra-band inter-frequency asynchronous DAPS handover in SA for FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.10.2-2: General test parameters for intra-band inter-frequency asynchronous DAPS  handover test in SA for FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 10 s | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |
| T4 |  | ms | 10 + Tinterrupt2 | Tinterrupt2 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-5 |
| T5 |  | ms | 100 |  |

Table A.6.3.1.10.2-3: Cell specific test parameters for intra-band inter-frequency asynchronous DAPS  handover test in SA for FR1

| Parameter |  |  | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  | 2 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |  |  |  |
|  |  |  | dB | 8 | 8 | 8 | 8 | 8 | -Infinity | 8 | 8 | 8 | 8 |
|  |  |  | dB | 8 | 8 | 8 | 8 | 8 | -Infinity | 8 | 8 | 8 | 8 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 | -90 | -90 | -90 | -Infinity | -90 | -90 | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 | -87 | -87 | -87 | -Infinity | -87 | -87 | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -61.41 | -61.41 | -61.41 | -61.41 | -70.05 | -61.41 | -61.41 | -61.41 | -61.41 |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -55.31 | -55.31 | -55.31 | -55.31 | -63.94 | -55.31 | -55.31 | -55.31 | -55.31 |
| Propagation condition |  |  | - | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.1.10.3 Test Requirements

The UE shall start to transmit the PRACH to cell 2 less than 72 ms from the beginning of time period T3.

During T3 UE is allowed to cause Tinterrupt1 interruption to cell 1. Tinterrupt1 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-2. When UE is transmitting PRACH preamble to cell 2, interruption to cell 1 is allowed.

During T4 UE is allowed to cause Tinterrupt2 interruption to cell 1. Tinterrupt2 is defined in clause 6.1.3.2.2 Table 6.1.3.2.2-5.

UE shall finish cell 1 release in T4 and shall not send any CSI reports to cell 1 during T5.

The rate of correct handovers observed during repeated tests shall be at least 90%.

#### A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1

##### A.6.3.1.11.1	Test Purpose and Environment

This test is to verify the requirement for the FR1-to-FR1 inter-band inter-frequency synchronous DAPS handover requirements specified in clause 6.1.3.2.

##### A.6.3.1.11.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.11.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.11.2-2, A.6.3.1.11.2-3 and A.6.3.1.11.2-4.

The test scenario comprises of two bands each with one cell. The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

Before the start of T1, the UE is connected to Cell 1 (source PCell) on radio channel 1 but is not aware of Cell 2 (neighbour cell) on radio channel 2. During T1, the UE shall not have any timing information of Cell 2.

Before the start of T2, the UE in the measurement control information that event-triggered reporting with Event A3 is configured for neighbour cell (Cell 2), and the UE is configured with the measurement gaps (gap pattern ID # 0). Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A3. After receiving the Event A3, the test system shall send a RRC m`essage implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing the RRC message implying DAPS handover to Cell 2 (target PCell) is sent to the UE. During T3, the UE shall be able to perform random access to Cell 2. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perform DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. After the RACH procedure is completed, the test system shall send a RRC message to the UE to release Cell 1 (source cell) on radio channel 1.

The start of T4 is the instant when the last TTI containing the RRC message implying source cell release is sent to the UE. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stops to send CSI report to the source cell.

Table A.6.3.1.11.2-1: Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 5 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 7 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 9 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.11.2-2: General test parameters for inter-band inter-frequency synchronous DAPS handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | PCell on RF channel number 1 |
|  | Neighbouring cell |  | Cell 2 | Neighbour cell on RF channel number 2 |
| Final condition | Active cell |  | Cell 2 | PCell on RF channel number 2 |
|  | Neighbouring cell |  | Cell 1 | Neighbour cell on RF channel number 1 |
| A3-Offset |  | dB | -6 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  | s | 33 | Synchronous cells |
| DRX |  |  | OFF |  |
| Measurement gap pattern Id |  |  | #0 | Gaps are configured before T2. |
| T1 |  | s | 5 |  |
| T2 |  | s | <5 |  |
| T3 |  | s | <0.5 |  |
| T4 |  | ms | 10+Tinterrupt2 | Tinterrupt2 as defined in Table 6.1.3.2.2-6 for synchronous DAPS HO |
| T5 |  | ms | 100 |  |

Table A.6.3.1.11.2-3: Cell specific test parameters for inter-band inter-frequency synchronous DAPS handover from FR1 to FR1 (Cell 1)

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |
| Duplex mode |  | Config 1,4,7 |  | FDD |  |  |  |  |
|  |  | Config 2,3,5,6,8,9 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1,4,7 |  | Not Applicable |  |  |  |  |
|  |  | Config 2,5,8 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3,6,9 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1,4,7 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2,5,8 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3,6,9 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1,4,7 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2,5,8 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3,6,9 |  | 40: NRB,c = 106 |  |  |  |  |
| TRS configuration |  | Config 1,4,7 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2,5,8 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3,6,9 |  | TRS.1.2 TDD |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4,7 |  | SR.1.1 FDD |  |  |  |  |
|  |  | Config 2,5,8 |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3,6,9 |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  | Config 1,4,7 |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 2,5,8 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3,6,9 |  | CR2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |
| SSB Configuration |  | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3,6,9 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3,6,9 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3,6,9 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 2 |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Note2 | Config 1,2,4,5,7,8 |  | dBm/SCS | -98 | -98 | -98 | -98 | -98 |
|  | Config 3,6,9 |  |  | -95 | -95 | -95 | -95 | -95 |
|  |  |  | dB | 4 | 4 | 4 | 4 | 4 |
|  |  |  | dB | 4 | 4 | 4 | 4 | 4 |
| SSB_RP | Config 1,2,4,5,7,8 |  | dBm/SCS | -94 | -94 | -94 | -94 | -94 |
|  | Config 3,6,9 |  | dBm/SCS | -91 | -91 | -91 | -91 | -91 |
| IoNote3 | Config 1,2,4,5,7,8 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -64.59 | -64.59 | -64.59 |
|  | Config 3,6,9 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -58.49 | -58.49 | -58.49 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.6.3.1.11.2-4: Cell specific test parameters for inter-band inter-frequency synchronous DAPS handover from FR1 to FR1 (Cell 2)

| Parameter |  |  | Unit | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 2 |  |  |  |  |
| Duplex mode |  | Config 1,2,3 |  | FDD |  |  |  |  |
|  |  | Config 4,5,6,7,8,9 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1,2,3 |  | Not Applicable |  |  |  |  |
|  |  | Config 4,5,6 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 7,8,9 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1,2,3 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 4,5,6 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 7,8,9 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1,2,3 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 4,5,6 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 7,8,9 |  | 40: NRB,c = 106 |  |  |  |  |
| TRS configuration |  | Config 1,2,3 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 4,5,6 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 7,8,9 |  | TRS.1.2 TDD |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,2,3 |  | SR.1.1 FDD |  |  |  |  |
|  |  | Config 4,5,6 |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 7,8,9 |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  | Config 1,2,3 |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 4,5,6 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 7,8,9 |  | CR2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |
| SSB Configuration |  | Config 1,2,3,4,5,6 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 7,8,9 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,3,4,5,6 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 7,8,9 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,3,4,5,6 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 7,8,9 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 2 |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Note2 | Config 1,2,3,4,5,6 |  | dBm/SCS | -98 | -98 | -98 | -98 | -98 |
|  | Config 7,8,9 |  |  | -95 | -95 | -95 | -95 | -95 |
|  |  |  | dB | -Infinity | 4 | 4 | 4 | 4 |
|  |  |  | dB | -Infinity | 4 | 4 | 4 | 4 |
| SSB_RP | Config 1,2,3,4,5,6 |  | dBm/SCS | -Infinity | -94 | -94 | -94 | -94 |
|  | Config 7,8,9 |  | dBm/SCS | -Infinity | -91 | -91 | -91 | -91 |
| IoNote3 | Config 1,2,3,4,5,6 |  | dBm/ 9.36MHz | -70.05 | -64.59 | -64.59 | -64.59 | -64.59 |
|  | Config 7,8,9 |  | dBm/ 38.16MHz | -63.94 | -58.49 | -58.49 | -58.49 | -58.49 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.3.1.11.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 72 ms from the beginning of time period T3. During Dhandover1, the interruption on Cell 1 shall not exceed Tinterrupt1 as defined in Table 6.1.3.2.2-3 for synchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover1 can be expressed as: TRRC_procedure + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

The UE shall complete to release Cell 1 less than (10 ms + Tinterrupt2) from the beginning of time period T4. During Dhandover2, the interruption on Cell 2 shall not exceed Tinterrupt2 as defined in Table 6.1.3.2.2-6 for synchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

#### A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1

##### A.6.3.1.12.1	Test Purpose and Environment

This test is to verify the requirement for the FR1-to-FR1 inter-band inter-frequency asynchronous DAPS handover requirements specified in clause 6.1.3.2.

##### A.6.3.1.12.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.12.2-1. Both handover delay and interruption length are tested by using the parameters in table A.6.3.1.12.2-2, A.6.3.1.12.2-3 and A.6.3.1.12.2-4.

The test scenario comprises of two bands each with one cell. The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

Before the start of T1, the UE is connected to Cell 1 (source PCell) on radio channel 1 but is not aware of Cell 2 (neighbour cell) on radio channel 2. During T1, the UE shall not have any timing information of Cell 2.

Before the start of T2, the UE in the measurement control information that event-triggered reporting with Event A3 is configured for neighbour cell (Cell 2), and the UE is configured with the measurement gaps (gap pattern ID # 0). Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A3. After receiving the Event A3, the test system shall send a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the last TTI containing the RRC message implying DAPS handover to Cell 2 (target PCell) is sent to the UE. During T3, the UE shall be able to perform random access to Cell 2. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perform DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. After the RACH procedure is completed, the test system shall send a RRC message to the UE to release Cell 1 (source cell) on radio channel 1.

The start of T4 is the instant when the last TTI containing the RRC message implying source cell release is sent to the UE. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stops to send CSI report to the source cell.

Table A.6.3.1.12.2-1: Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 5 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 7 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 9 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.1.12.2-2: General test parameters for inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  |  | Cell 1 |  |
|  | Neighbouring cell |  |  | Cell 2 |  |
| Final condition | Active cell |  |  | Cell 2 |  |
| A3-Offset |  |  | dB | -4 |  |
| Hysteresis |  |  | dB | 0 |  |
| Time To Trigger |  |  | s | 0 |  |
| Filter coefficient |  |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  | Config 1,2,4,5 | ms | 0.5 | Asynchronous cells |
|  |  | Config3,6,7,8,9 | ms | 0.25 |  |
| DRX |  |  |  | OFF |  |
| Measurement gap pattern Id |  |  |  | #0 | Gaps are configured before T2. |
| T1 |  |  | s | 5 |  |
| T2 |  |  | s | <5 |  |
| T3 |  |  | s | <0.5 |  |
| T4 |  |  | ms | 10+Tinterrupt2 | Tinterrupt2 as defined in Table 6.1.3.2.2-6 for asynchronous DAPS HO. |
| T5 |  |  | ms | 100 |  |

Table A.6.3.1.12.2-3: Cell specific test parameters for inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1 (Cell 1)

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |
| Duplex mode |  | Config 1,4,7 |  | FDD |  |  |  |  |
|  |  | Config 2,3,5,6,8,9 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1,4,7 |  | Not Applicable |  |  |  |  |
|  |  | Config 2,5,8 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3,6,9 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1,4,7 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2,5,8 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3,6,9 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1,4,7 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2,5,8 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3,6,9 |  | 40: NRB,c = 106 |  |  |  |  |
| TRS configuration |  | Config 1,4,7 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2,5,8 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3,6,9 |  | TRS.1.2 TDD |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4,7 |  | SR.1.1 FDD |  |  |  |  |
|  |  | Config 2,5,8 |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3,6,9 |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  | Config 1,4,7 |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 2,5,8 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3,6,9 |  | CR2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |
| SSB Configuration |  | Config 1,2,4,5,7,8 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3,6,9 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3,6,9 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,4,5,7,8 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3,6,9 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 2 |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Note2 | Config 1,2,4,5,7,8 |  | dBm/SCS | -98 | -98 | -98 | -98 | -98 |
|  | Config 3,6,9 |  |  | -95 | -95 | -95 | -95 | -95 |
|  |  |  | dB | 4 | 4 | 4 | 4 | 4 |
|  |  |  | dB | 4 | 4 | 4 | 4 | 4 |
| SSB_RP | Config 1,2,4,5,7,8 |  | dBm/SCS | -94 | -94 | -94 | -94 | -94 |
|  | Config 3,6,9 |  | dBm/SCS | -91 | -91 | -91 | -91 | -91 |
| IoNote3 | Config 1,2,4,5,7,8 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -64.59 | -64.59 | -64.59 |
|  | Config 3,6,9 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -58.49 | -58.49 | -58.49 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.6.3.1.12.2-4: Cell specific test parameters for inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1 (Cell 2)

| Parameter |  |  | Unit | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 2 |  |  |  |  |
| Duplex mode |  | Config 1,2,3 |  | FDD |  |  |  |  |
|  |  | Config 4,5,6,7,8,9 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1,2,3 |  | Not Applicable |  |  |  |  |
|  |  | Config 4,5,6 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 7,8,9 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1,2,3 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 4,5,6 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 7,8,9 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1,2,3 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 4,5,6 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 7,8,9 |  | 40: NRB,c = 106 |  |  |  |  |
| TRS configuration |  | Config 1,2,3 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 4,5,6 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 7,8,9 |  | TRS.1.2 TDD |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,2,3 |  | SR.1.1 FDD |  |  |  |  |
|  |  | Config 4,5,6 |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 7,8,9 |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  | Config 1,2,3 |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 4,5,6 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 7,8,9 |  | CR2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |
| SSB Configuration |  | Config 1,2,3,4,5,6 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 7,8,9 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,3,4,5,6 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 7,8,9 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,3,4,5,6 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 7,8,9 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 2 |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Note2 | Config 1,2,3,4,5,6 |  | dBm/SCS | -98 | -98 | -98 | -98 | -98 |
|  | Config 7,8,9 |  |  | -95 | -95 | -95 | -95 | -95 |
|  |  |  | dB | -Infinity | 4 | 4 | 4 | 4 |
|  |  |  | dB | -Infinity | 4 | 4 | 4 | 4 |
| SSB_RP | Config 1,2,3,4,5,6 |  | dBm/SCS | -Infinity | -94 | -94 | -94 | -94 |
|  | Config 7,8,9 |  | dBm/SCS | -Infinity | -91 | -91 | -91 | -91 |
| IoNote3 | Config 1,2,3,4,5,6 |  | dBm/ 9.36MHz | -70.05 | -64.59 | -64.59 | -64.59 | -64.59 |
|  | Config 7,8,9 |  | dBm/ 38.16MHz | -63.94 | -58.49 | -58.49 | -58.49 | -58.49 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.3.1.12.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 72 ms from the beginning of time period T3. During Dhandover1, the interruption on Cell 1 shall not exceed Tinterrupt1 as defined in Table 6.1.3.2.2-3 for asynchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover1 can be expressed as: TRRC_procedure + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 20 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 72 ms.

The UE shall complete to release Cell 1 less than (10 ms + Tinterrupt2) from the beginning of time period T4. During Dhandover2, the interruption on Cell 2 shall not exceed Tinterrupt2 as defined in Table 6.1.3.2.2-6 for asynchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

### A.6.3.2	RRC Connection Mobility Control

#### A.6.3.2.1	SA: RRC Re-establishment

##### A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1

A.6.3.2.1.1.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 with known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.6.3.2.1.1.1-1, table A.6.3.2.1.1.1-2 and table A.6.3.2.1.1.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table A.6.3.2.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.3.2.1.1.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 0 | Radio link failure timer; |
| T311 |  | ms | 1, 2, 3 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | FR1 PRACH configuration 1 | Table A.3.8.2.1-1 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | ms | 1, 2, 3 | 240 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1, 2, 3 | 2 |  |

Table A.6.3.2.1.1.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  | T3 | T1 | T2 |  |  | T3 |
| TDD configuration |  | 1 | N/A |  |  |  |  | N/A |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  |  | SR.1.1 FDD |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  |  | SSB |  |  |  |  |
|  | dB | 1 | 1.54 |  | -infinity |  | -infinity | -3.79 | 4 |  |  | 4 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 7 |  | -infinity |  | -infinity | 4 | 4 |  |  | 4 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -91 |  | -infinity |  | -infinity | -94 | -94 |  |  | -94 |
|  |  | 2 | -91 |  | -infinity |  | -infinity | -94 | -94 |  |  | -94 |
|  |  | 3 | -88 |  | -infinity |  | -infinity | -91 | -91 |  |  | -91 |
| Io | dBm/9.36 MHz | 1 | -60.74 |  | -64.59 |  | -64.59 | -60.74 | -64.59 |  |  | -64.59 |
|  | dBm/9.36 MHz | 2 | -60.74 |  | -64.59 |  | -64.59 | -60.74 | -64.59 |  |  | -64.59 |
|  | dBm/38.16 MHz | 3 | -54.65 |  | -58.50 |  | -58.50 | -54.65 | -58.50 |  |  | -58.50 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

A.6.3.2.1.1.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to a known NR intra frequency cell shall be less than 1.6 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 200 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 1545 ms, allow 1.6 s in the test case.

##### A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1

A.6.3.2.1.2.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR1 without known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.6.3.2.1.2.1-1, table A.6.3.2.1.2.1-2 and table A.6.3.2.1.2.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

Table A.6.3.2.1.2.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.3.2.1.2.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 0 | Radio link failure timer; |
| T311 |  | ms | 1, 2, 3 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | FR1 PRACH configuration 1 | Table A.3.8.2.1-1 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | ms | 1, 2, 3 | 240 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1, 2, 3 | 5 |  |

Table A.6.3.2.1.2.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 | T3 | T1 | T2 |  |  | T3 |
| RF Channel Number |  | 1, 2, 3 | 1 |  |  |  | 2 |  |  |  |  |
| TDD configuration |  | 1 | N/A |  |  |  | N/A |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |  | N/A | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |  | N/A | N/A |  | N/A | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  | SSB |  |  |  |  |
|  | dB | 1 | 4 |  | -infinity | -infinity | -infinity | -infinity |  |  | 7 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 4 |  | -infinity | -infinity | -infinity | -infinity |  |  | 7 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -94 |  | -infinity | -infinity | -infinity | -infinity |  |  | -91 |
|  |  | 2 | -94 |  | -infinity | -infinity | -infinity | -infinity |  |  | -91 |
|  |  | 3 | -91 |  | -infinity | -infinity | -infinity | -infinity |  |  | -88 |
| Io | dBm/9.36 MHz | 1 | -64.59 |  | -70. 05 | -70. 05 | -70. 05 | -70. 05 |  |  | -62.26 |
|  | dBm/9.36 MHz | 2 | -64.59 |  | -70. 05 | -70. 05 | -70. 05 | -70.05 |  |  | -62.26 |
|  | dBm/38.16 MHz | 3 | -58.50 |  | -63.94 | -63.94 | -63.94 | -63.94 |  |  | -56.15 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

A.6.3.2.1.2.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than 3 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2

Tidentify_intra_NR = 800 ms

Tidentify_inter_NR = 800 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 2945 ms, allow 3 s in the test case.

##### A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing

A.6.3.2.1.3.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 without serving cell timing is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.6.3.2.1.3.1-1, table A.6.3.2.1.3.1-2 and table A.6.3.2.1.3.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table A.6.3.2.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.3.2.1.3.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 6000 | Radio link failure timer configured by RLF-TimersAndConstants |
| T311 |  | ms | 1, 2, 3 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | FR1 PRACH configuration 1 | Table A.3.8.2.1-1 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | s | 1, 2, 3 | 6.24 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1, 2, 3 | 3 |  |

Table A.6.3.2.1.3.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -94 | -infinity | -infinity | -infinity | -infinity | -94 |
|  |  | 2 | -94 | -infinity | -infinity | -infinity | -infinity | -94 |
|  |  | 3 | -91 | -infinity | -infinity | -infinity | -infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.59 | -infinity | -infinity | -infinity | -infinity | -64.59 |
|  | dBm/9.36 MHz | 2 | -64.59 | -infinity | -infinity | -infinity | -infinity | -64.59 |
|  | dBm/38.16 MHz | 3 | -58.50 | -infinity | -infinity | -infinity | -infinity | -58.50 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.6.3.2.1.3.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell without serving cell timing shall be less than 2.2 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 800 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 [2] for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 2145 ms, allow 2.2 s in the test case.

#### A.6.3.2.2	Random Access

##### A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone

A.6.3.2.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.2 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1. Supported test parameters are shown in Table A.6.3.2.2.1.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.6.3.2.2.1.1-2.

Table A.6.3.2.2.1.1-1: Supported test configurations for contention based random access test in FR1 for NR standalone

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.6.3.2.2.1.1-2: General test parameters for contention based random access test in FR1 for NR Standalone

| Parameter |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  |  | SSB pattern 1 in FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  |  | SSB pattern 2 in FR1 |  |
| Number of SSBs per SS-burst |  |  |  |  | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  |  | 0,1 | Different from the definition in A.3.10 |
| Duplex Mode for Cell 1 |  | Config 1 |  |  | FDD |  |
|  |  | Config 2 |  |  | TDD |  |
| TDD Configuration |  | Config 2 |  |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  |  | OP.1 | As defined in A.3.2.1. |
| PDSCH parameters Note 4 |  | Config 1 |  |  | SR.1.1 FDD | As defined in A.3.1.1. |
|  |  | Config 2 |  |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channe |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  |  | Config 2 |  | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1 |  | dBm/15kHz | -98 |  |
|  |  | Config 2 |  |  | -101 |  |
|  |  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1 |  | dBm/15kHz | -98 |  |
|  |  | Config 2 |  |  | -101 |  |
|  |  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 |  | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  |  | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  |  | FR1 PRACH configuration 1 | As defined in A.3.8. |
| Propagation Condition |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: Void Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

A.6.3.2.2.1.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.6.3.2.2.1.2.1	Random Access Preamble Transmission

To test the UE behavior specified in Clause 6.2.2.2.1.1 the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.1.2.2	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2.2.1.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.1.2.3	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2.2.1.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.1.2.4	Receiving an UL grant for msg3 retransmission

To test the UE behavior specified in clause 6.2.2.2.1.4 the System Simulator shall provide an UL grant for msg3 retransmission following a successful Random Access Response.

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

A.6.3.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

A.6.3.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

The UE shall send ACK if the Contention Resolution is successful.

A.6.3.2.2.1.2.7	Contention Resolution Timer expiry

To test the UE behavior specified in Clause 6.2.2.2.1.6 the System Simulator shall not send a response to a msg3.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

##### A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone

A.6.3.2.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.2 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1. Supported test parameters are shown in Table A.6.3.2.2.2.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.6.3.2.2.2.1-2 for SSB-based non-contention based random access test (Test 1) and CSI-RS-based non-contention based random access test (Test 2). Test 2 is only applicable to UE which supports csi-RSRP-AndRSRQ-MeasWithSSB or csi-RSRP-AndRSRQ-MeasWithoutSSB.

Table A.6.3.2.2.2.1-1: Supported test configurations for non-contention based random access test in FR1 for NR standalone

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.6.3.2.2.2.1-2: General test parameters for non-contention based random access test in FR1 for NR Standalone

| Parameter |  |  |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  |  | SSB pattern 1 in FR1 | SSB pattern 1 in FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  |  | SSB pattern 2 in FR1 | SSB pattern 2 in FR1 |  |
| Number of SSBs per SS-burst |  |  |  |  | 2 | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  |  | 0,1 | 0,1 | Different from the definition in A.3.10 |
| CSI-RS Configuration |  | Config 1 |  |  | N/A | CSI-RS.1.1 FDD | As defined in A.3.1.4 |
|  |  | Config 2 |  |  |  | CSI-RS.2.1 TDD |  |
| Duplex Mode for Cell 1 |  | Config 1 |  |  | FDD | FDD |  |
|  |  | Config 2 |  |  | TDD | TDD |  |
| TDD Configuration |  | Config 2 |  |  | TDDConf.2.1 | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  |  | OP.1 | OP.1 | As defined in A.3.2.1. |
| PDSCH parameters Note 4 |  | Config 1 |  |  | SR.1.1 FDD | SR.1.1 FDD | As defined in A.3.1.1. |
|  |  | Config 2 |  |  | SR.2.1 TDD | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  |  | Config 2 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  |  | Config 2 |  | CCR.2.1 TDD | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  | dB |  |  |  |
| SSB with index 0 |  |  |  | dB | 3 | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1 |  | dBm/15kHz | -98 | -98 |  |
|  |  | Config 2 |  |  | -101 | -101 |  |
|  |  |  |  | dB | 3 | 3 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -95 | -95 |  |
| SSB with index 1 |  |  |  | dB | -17 | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1 |  | dBm/15kHz | -98 | -98 |  |
|  |  | Config 2 |  |  | -101 | -101 |  |
|  |  |  |  | dB | -17 | -17 |  |
|  | SS-RSRP Note 3 |  |  | dBm/ SCS | -115 | -115 |  |
| Io Note 2 |  | Config 1 |  | dBm | -65.3/9.36MHz | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  |  | -62.2/38.16MHz | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  |  | dBm/ SCS | -5 | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  |  | dBm | 23 | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  |  | FR1 PRACH configuration 2 | FR1 PRACH configuration 3 | As defined in A.3.8.2. |
| Propagation Condition |  |  |  | - | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: Void Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |  |

A.6.3.2.2.2.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.6.3.2.2.2.2.1	SSB-based Random Access Preamble Transmission

In Test-1, to test the UE behavior specified in Clause 6.2.2.2.2.1 for SSB-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the Random Access Preamble which has the Preamble Index associated with the SSB with index 0.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.2.2.2	CSI-RS-based Random Access Preamble Transmission

In Test-2, to test the UE behavior specified in Clause 6.2.2.2.2.1 for CSI-RS-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs configured, the System Simulator shall receive the Random Access Preamble which has the Preamble Index associated with the CSI-RS configured.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-OccasionList.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.2.2.3	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2.2.2.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.2.2.4	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2.2.2.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone

A.6.3.2.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the 2-step RA type random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.3 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1. Supported test parameters are shown in Table A.6.3.2.2.3.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.6.3.2.2.3.1-2.

Table A.6.3.2.2.3.1-1: Supported test configurations for 2-step RA type contention based random access with successRAR test in FR1 for NR standalone

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.6.3.2.2.3.1-2: General test parameters for 2-step RA type contention based random access with successRAR test in FR1 for NR standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB pattern 1 in FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB pattern 2 in FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.10 |
| Duplex Mode for Cell 2 |  | Config 1 |  | FDD |  |
|  |  | Config 2 |  | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  | Config 1 |  | SR.1.1 FDD | As defined in A.3.1.1. |
|  |  | Config 2 |  | SR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  | FR1 MsgA configuration 1 | As defined in A.3.20.2.1. |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

A.6.3.2.2.3.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.6.3.2.2.3.2.1	MsgA Transmission

To test the UE behavior specified in Clause 6.2.2.3.1.1 the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured msgA-RSRP-ThresholdSSB.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first MsgA preamble transmission shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.3.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2.3.1.2 the System Simulator shall transmit a MsgB containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for MsgB(s) and shall transmit an ACK if the MsgB with a successRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble and if the Contention Resolution is successful.

The UE may stop monitoring for MsgB(s) and shall transmit the msg3 if the MsgB with a fallbackRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB(s) contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.3.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2.3.1.3 the System Simulator shall transmit a MsgB containing a successRAR message and a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if no MsgB  is received within the MsgB Response window.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone

A.6.3.2.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.3 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1. Supported test parameters are shown in Table A.6.3.2.2.4.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.6.3.2.2.4.1-2.

Table A.6.3.2.2.4.1-1: Supported test configurations for non-contention based random access test in FR1 for NR standalone

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.6.3.2.2.4.1-2: General test parameters for non-contention based random access test in FR1 for NR Standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB pattern 2 in FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.10 |
| Duplex Mode for Cell 1 |  | Config 1 |  | TDD |  |
| TDD Configuration |  | Config 1 |  | TDDConf.2.1 |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.3.2.1. |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.2.1 TDD | As defined in A.3.1.1. |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| msgA-RSRP-ThresholdSSB |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  | FR1 MsgA configuration 2 | As defined in A.3.20.2.2. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

A.6.3.2.2.4.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.6.3.2.2.4.2.1	MsgA Transmission

To test the UE behavior specified in Clause 6.2.2.3.2.1, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0.

In addition, the System Simulator shall receive the MsgA PRACH on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belongs to the PRACH occasions permitted by the restrictions given first by the msgA-SSB-SharedRO-MaskIndex if configured, or next by the ra-ssb-OccasionMaskIndex if configured.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.4.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2.3.2.2 the System Simulator shall transmit a MsgB containing a fallbackRAR containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for MsgB(s) and shall transmit the msg3 containing the payload of MsgA PUSCH if the MsgB with a fallbackRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble. The UE shall monitor contention resolution as described in clause 8.2A in TS 38.213 [3].

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB’s contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA and msg3 transmissions shall be within the accuracy specified in Clause 7.1.2.

A.6.3.2.2.4.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2.3.2.3 the System Simulator shall transmit a MsgB containing a successRAR message and a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA transmission power when the backoff time expires if no MsgB  is received within the MsgB Response window.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be -22 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

#### A.6.3.2.3	SA: RRC Connection Release with Redirection

##### A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1

A.6.3.2.3.1.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR to NR requirements specified in clause 6.2.3.2.1.

A.6.3.2.3.1.2	Test Parameters

Supported test configurations are shown in table A.6.3.2.3.1.2-1. The time delay is tested by using the parameters in table A.6.3.2.3.1.2-2, and A.6.3.2.3.1.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.6.3.2.3.1.2-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.2.3.1.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

Table A.6.3.2.3.1.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -64.59 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -58.49 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

A.6.3.2.3.1.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 2240 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

where:

TRRC_procedure_delay = 110 msin the test.

Tidentify-NR = 680 ms in the test.

TSI-NR = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH = 170 ms in the test.

This gives a total of 2240 ms.

##### A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN

A.6.3.2.3.2.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR to E-UTRAN requirements specified in clause 6.2.3.2.2.

A.6.3.2.3.2.2	Test Parameters

Supported test configurations are shown in table A.6.3.2.3.2.2-1. The time delay is tested by using the parameters in table A.6.3.2.3.2.2-2, A.6.3.2.3.2.2-3 and A.6.3.2.3.2.2-4.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.6.3.2.3.2.2-1: Redirection from NR to E-UTRAN test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.2.3.2.2-2: General test parameters for Redirection from NR to E-UTRAN test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

Table A.6.3.2.3.2.2-3: Cell specific test parameters for Redirection from NR to E-UTRAN (cell 1)

| Parameter |  |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |
| RF Channel Number |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |  |  |
|  |  | Config 2 |  | SSB.1 FR1 |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |
| BWP BW |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |  |  |
|  |  | Config 2,5 |  | SR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | SR2.1 TDD |  |  |
| CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |  |
|  |  | Config 2,5 |  | CR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | CR2.1 TDD |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| SMTC configuration |  | Config 1,2,4,5 |  | SMTC.1 FR1 |  |  |
|  |  | Config 3,6 |  | SMTC.2 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 kHz |  |  |
|  |  | Config 3,6 |  | 30 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 kHz |  |  |
|  |  | Config 3,6 |  | 30 kHz |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -98 |  |  |
|  | Config 3,6 |  |  | -95 |  |  |
|  |  |  | dB | 4 | 4 |  |
|  |  |  | dB | 4 | 4 |  |
| IoNote3 | Config 1,2,4,5 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  |
|  | Config 3,6 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  |
| Propagation condition |  |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.3.2.3.2.2-4: Cell specific test parameters for Redirection from NR to E-UTRAN (cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |
|  |  | 4, 5, 6 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 4 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 4 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -94 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -94 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -70.22 | -64.76 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

A.6.3.2.3.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 2205 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to E-UTRAN observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_E-UTRA = TRRC_procedure_delay + Tidentify-E-UTRA + TSI-E-UTRA + TRACH,

where:

TRRC_procedure_delay = 110 ms  in the test.

Tidentify-E-UTRA = 800 ms in the test.

TSI-E-UTRA = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 36.331 for the target E-UTRA cell.

TRACH = 15 ms in the test.

This gives a total of 2205 ms.

### A.6.3.3	Conditional handover

#### A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1

##### A.6.3.3.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency conditional handover requirements specified in clause 6.1.4.2.

##### A.6.3.3.1.2	Test Parameters

Supported test configurations are shown in table A.6.3.3.1.2-1. Both conditional handover delay and interruption length are tested by using the parameters in table A.6.3.3.1.2-2, and A.6.3.3.1.2-3.

The test consists of two successive time periods, with time durations of T1 and  T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2.

Table A.6.3.3.1.2-1: Intra-frequency conditional handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.1.2-2: General test parameters Intra-frequency conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency conditional handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 8 | -3.3 |  | -Infinity |  | 2.36 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 11 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -Infinity |  | -87 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -Infinity |  | -84 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.06 |  | -61.41 |  | -57.06 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -50.96 |  | -55.31 |  | -50.96 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.3.1.3 Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution = 800 +62 +10=872ms from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms

#### A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1

##### A.6.3.3.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR conditional FR1-NR FR1 inter frequency conditional handover requirements specified in clause 6.1.4.2.

##### A.6.3.3.2.2	Test Parameters

Supported test configurations are shown in table A.6.3.3.2.2-1. Both conditional handover delay and interruption length are tested by using the parameters in table A.6.3.3.2.2-2, and A.6.3.3.2.2-3.

The test scenario comprises of two carriers and one cell on each carrier Gap pattern ID gp0 is configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. NR shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2.  At the start of T2, cell 2 becomes detectable and meets the handover condition.

Table A.6.3.3.2.2-1: Inter-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.2.2-2: General test parameters Inter-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset in handover condition |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.2.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| Gap pattern ID |  |  |  | gp0 |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 |  | -Infinity |  | -93 |  |
|  | Config 3 |  | dBm/SCS | -91 | -91 |  | -Infinity |  | -90 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -63.85 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -57.75 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.3.2.3 Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution =920 +62 +10=992 ms from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms excluding any transmissions which do not occur due to measurement gaps.

Inter-frequency CHO FR1-FR1 920 (Tmeasure)+62 (Tinterrupt)+10 (TCHO_execution) = 992 62 m

## A.6.4	Timing

### A.6.4.1	UE transmit timing

#### A.6.4.1.1	NR UE Transmit Timing Test for FR1

##### A.6.4.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table A.6.4.1.1.1-1.

Table A.6.4.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

For this test a single NR cell is used. Table A.6.4.1.1.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.6.4.1.1.1-3.

Table A.6.4.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2,3 | 1 |  | 1 |  |
| TDD configuration |  | 1 | Not Applicable |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |  |  |
|  |  | 2 | 10: NRB,c = 52 |  |  |  |
|  |  | 3 | 40: NRB,c = 106 |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| DRx Cycle | ms | 1,2,3 | N/A |  |  | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |  |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |  |
| SMTC Configuration |  | 1,2 | SMTC.1 |  |  |  |
|  |  | 3 | SMTC.2 |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3 | -98 |  | -98 |  |
| Note2 | dBm/SCS | 1,2 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
|  |  | 1,2,3 | 3 |  | 3 |  |
|  |  | 1,2,3 | 3 |  | 3 |  |
| SS-RSRPNote3 | dBm/SCS | 1,2 | -95 |  | -95 |  |
|  |  | 3 | -92 |  | -92 |  |
| IoNote3 | dBm/9.36MHz | 1,2 | -65.2 |  | -65.2 |  |
|  | dBm/38.1MHz | 3 | -59.2 |  | -59.2 |  |
| Propagation condition |  | 1,2,3 | AWGN |  |  |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 | SRSConf.3Note6 |  |  |
|  |  | 3 | SRSConf.1Note6 | SRSConf.2Note6 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.6.4.1.1.1-3 |  |  |  |  |  |  |

Table A.6.4.1.1.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | SRSConf.3 | Comments |
| --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 | 0 |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2 25 for test configuration 3 | 25 | 14 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 0 | sl320, 0 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | 0 | Any 10 bit number |

Table A.6.4.1.1.1-4: Void

##### A.6.4.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1) Setup NR PCell according to parameters given in Table A.6.4.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.6.4.1.1.2-1

Table A.6.4.1.1.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first detected path (in time) of DL SSB.  Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

### A.6.4.2	UE timer accuracy

### A.6.4.3	Timing advance

#### A.6.4.3.1	SA FR1 timing advance adjustment accuracy

##### A.6.4.3.1.1	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment delay and accuracy requirement defined in clause 7.3.

##### A.6.4.3.1.2	Test Parameters

Supported test configurations are shown in table A.6.4.3.1.2-1. Both timing advance adjustment delay and accuracy are tested by using the parameters in table A.6.4.3.1.2-2, A.6.4.3.1.2-3 and A.6.4.3.1.2-4.

In all test cases, single cell is used. Each test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.6.4.3.1.2-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.4 in TS 38.321 [7]. The Timing Advance Command value shall be set to 31, which according to Clause 4.2 in TS 38.213 [3] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.6.4.3.1.2-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at slot n+k for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321 [7], shall be configured so that it does not expire in the duration of the test.

Table A.6.4.3.1.2-1: Timing advance supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.4.3.1.2-2: General test parameters for timing advance

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.3.9.2.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.3.9.2.2-1 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.3.9.3.1-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.3.9.3.2-1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For 15 kHz SCS NTA_new = NTA_old + 8192*Tc For 30 kHz SCS NTA_new = NTA_old + 4096*Tc (based on equation in clause 4.2 of TS 38.213 [3]) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.6.4.3.1.2-3: Cell specific test parameters for timing advance

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DRx Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  |
| CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2,5 |  | TRS.1.1 TDD |  |
|  |  | Config 3,6 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |
|  |  | Config 3 |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |
|  |  | Config 3 |  | 30 kHz |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | 3 |  |
|  |  |  | dB | 3 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -67.57 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -62.58 |  |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.6.4.3.1.2-4: Sounding Reference Symbol Configuration for timing advance

| Field |  | Value | Comment |
| --- | --- | --- | --- |
| c-SRS | Config 1,2 | 12 | Frequency hopping is disabled |
|  | Config 3 | 24 |  |
| b-SRS |  | 0 |  |
| b-hop |  | 0 |  |
| freqDomainPosition |  | 0 | Frequency domain position of SRS |
| freqDomainShift |  | 0 |  |
| groupOrSequenceHopping |  | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset |  | sl5=2 for SCS 15kHz sl5=4 for SCS 30kHz | Once every 5 slots |
| pathlossReferenceRS |  | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage |  | Codebook | Codebook based UL transmission |
| startPosition |  | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols |  | n1 |  |
| repetitionFactor |  | n1 |  |
| combOffset-n2 |  | 0 | transmissionComb setting |
| cyclicShift-n2 |  | 0 |  |
| nrofSRS-Ports |  | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |  |

##### A.6.4.3.1.3	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. k+1 slots after the reception of the timing advance command, where k=5.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

## A.6.5	Signalling characteristics

### A.6.5.1	Radio link Monitoring

In the following clause, any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

-	UE output power higher than Transmit OFF power -50 dBm (as defined in TS 38.101-1 [18]) means uplink signal

-	UE output power equal to or less than Transmit OFF power -50 dBm (as defined in TS 38.101-1 [18]) means no uplink signal.

#### A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

##### A.6.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.6.5.1.1.1-1. The test parameters are given in Tables A.6.5.1.1.1-2, A.6.5.1.1.1-3, and A.6.5.1.1.1-4 below. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 1.

Table A.6.5.1.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.1.1-2: General test parameters for FR1 out-of-sync testing in non-DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2 |  | Table A.3.8.2.1-1 |
|  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.48 |
| T3 |  |  | s | 0.48 |
| D1 |  |  | s | 0.44 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.6.5.1.1.1-3: Cell specific test parameters for FR1 (Cell 1) for out-of-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 |
|  | Config 2 |  | 1 | -7 | -15 |
|  | Config 3 |  | 1 | -7 | -15 |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.6.5.1.1.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. |  |  |  |  |  |

Table A.6.5.1.1.1-4: Measurement gap configuration for out-of-sync tests in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note: Ensure that RLM RS is partially overlapped with measurement gap |  |

Figure A.6.5.1.1.1-1: SNR variation for out-of-sync testing

##### A.6.5.1.1.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

##### A.6.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.6.5.1.2.1-1. The test parameters are given in Tables A.6.5.1.2.1-2, and A.6.5.1.2.1-3 below. There is one cell (Cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms.

Table A.6.5.1.2.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.2.1-2: General test parameters for FR1 in-sync testing in non-DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2, 3 |  | TDD |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD |
|  |  |  | Config 2 |  | CCR.1.1 TDD |
|  |  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  |  | Config 1 |  | SSB.1 FR1 |
|  |  |  | Config 2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |
|  |  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 kHz |
|  |  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  |  | Config 1, 2 |  | Table A.3.8.2.1-1 |
|  |  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX |  |  |  |  | OFF |
| Gap pattern ID |  |  |  |  | N.A. |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 1000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  |  | TRS.1.1 FDD |
|  |  | Config 2, 5 |  |  | TRS.1.1 TDD |
|  |  | Config 3, 6 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | 0.2 |
| T2 |  |  |  | s | 0.2 |
| T3 |  |  |  | s | 0.24 |
| T4 |  |  |  | s | 0.2 |
| T5 |  |  |  | s | 0.88 |
| D1 |  |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.1.2.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 3 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.6.5.1.2.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.1.2.1-4: Void

Figure A.6.5.1.2.1-1: SNR variation for in-sync testing

##### A.6.5.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode

##### A.6.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.6.5.1.3.1-1. The test parameters are given in Tables A.6.5.1.3.1-2, and A.6.5.1.3.1-3. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.1.3.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.3.1-2: General test parameters for FR1 out-of-sync testing in DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.3 FDD |
|  |  | Config 2 |  | CCR.1.3 TDD |
|  |  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  | 15 kHz |
|  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  | Config 1, 2 |  | Table A.3.8.2.1-1 |
|  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.68 |
| T3 |  |  | s | 0.68 |
| D1 |  |  | s | 0.64 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.6.5.1.3.1-3: Cell specific test parameters for FR1 (Cell 1) for out-of-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 |
|  | Config 2 |  | 1 | -7 | -15 |
|  | Config 3 |  | 1 | -7 | -15 |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.6.5.1.3.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. |  |  |  |  |  |

Table A.6.5.1.3.1-4: Void

Table A.6.5.1.3.1-5: Void

Table A.6.5.1.3.1-6: Void

Figure A.6.5.1.3.1-1: SNR variation for out-of-sync testing

##### A.6.5.1.3.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode

##### A.6.5.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.6.5.1.4.1-1. The test parameters are given in Tables A.6.5.1.4.1-2, and A.6.5.1.4.1-3. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.1.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.4.1-2: General test parameters for FR1 in-sync testing in DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2, 3 |  | TDD |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD |
|  |  |  | Config 2 |  | CCR.1.1 TDD |
|  |  |  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration |  |  | Config 1 |  | SSB.1 FR1 |
|  |  |  | Config 2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |
|  |  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 kHz |
|  |  |  | Config 3 |  | 30 kHz |
| PRACH Configuration |  |  | Config 1, 2 |  | Table A.3.8.2.1-1 |
|  |  |  | Config 3 |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX Configuration |  |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  |  | N.A. |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 2000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |
|  |  | Config 2 |  |  | TRS.1.1 TDD |
|  |  | Config 3 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | 0.2 |
| T2 |  |  |  | s | 0.2 |
| T3 |  |  |  | s | 0.64 |
| T4 |  |  |  | s | 0.2 |
| T5 |  |  |  | s | 0.88 |
| D1 |  |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.1.4.1-3: Cell specific test parameters for FR1 (Cell 1) for in-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 3 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.6.5.1.4.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.1.4.1-4: Void

Table A.6.5.1.4.1-5: Void

Figure A.6.5.1.4.1-1: SNR variation for in-sync testing.

##### A.6.5.1.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode

##### A.6.5.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.5.1-1, A.6.5.1.5.1-2, A.6.5.1.5.1-3, and A.6.5.1.5.1-3A below. There is one cell, cell 1 which is the PCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.5.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting of 5ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test. In the test, SSB0 is configured as the BFD-RS.

Table A.6.5.1.5.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.5.1-2: General test parameters for FR1 PCell for CSI-RS out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.3 FDD |
|  | Config 2 |  | CCR.1.3 TDD |
|  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State.2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.48 |
| T3 |  | s | 0.48 |
| D1 |  | s | 0.44 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.5.1-3: Cell specific test parameters for FR1 for CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 |
|  | Config 2 |  | 1 | -7 | -15 |
|  | Config 3 |  | 1 | -7 | -15 |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.6.5.1.5.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is [A.3.6]. |  |  |  |  |  |

Table A.6.5.1.5.1-3A: Measurement gap configuration for FR1 CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: Void |  |

Table A.6.5.1.5.1-4: Void

Figure A.6.5.1.5.1-1: SNR variation for CSI-RS out-of-sync testing

##### A.6.5.1.5.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

The UE shall stop transmitting uplink signal in Cell 1 no later than time point C (D1 ms after the start of the time duration T3) on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode

##### A.6.5.1.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.6.1-1, A.6.5.1.6.1-2, and A.6.5.1.6.1-3 below. There is one cells, cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.6.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is not enabled. In the test, SSB0 is configured as the BFD-RS.

Table A.6.5.1.6.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.6.1-2: General test parameters for FR1 PCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State.2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 1000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 0.44 |
| T4 |  | s | 0.2 |
| T5 |  | s | 0.88 |
| T6 |  | S | 0.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.6.1-3: Cell specific test parameters for FR1 for CSI-RS in-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 3 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.6.5.1.6.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is specified in clause A.3.6.1.1. |  |  |  |  |  |  |  |

Table A.6.5.1.6.1-4: Void

Figure A.6.5.1.6.1-1: SNR variation for CSI-RS in-sync testing

##### A.6.5.1.6.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode

##### A.6.5.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.7.1-1, A.6.5.1.7.1-2, and A.6.5.1.7.1-3 below. There is one cell, cell 1 is the PCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.7.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. In the test, SSB0 is configured as the BFD-RS.

Table A.6.5.1.7.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.7.1-2: General test parameters for FR1 PCell for CSI-RS out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.3 FDD |
|  | Config 2 |  | CCR.1.3 TDD |
|  | Config 3 |  | CCR.2.2 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State.2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 1.28 |
| T3 |  | s | 1.28 |
| D1 |  | s | 1.24 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.7.1-3: Cell specific test parameters for FR1 for CSI-RS out-of-sync radio link monitoring in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 |
|  | Config 2 |  | 1 | -7 | -15 |
|  | Config 3 |  | 1 | -7 | -15 |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.6.5.1.7.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is specified in clause A.3.6.1.1. |  |  |  |  |  |

Table A.6.5.1.7.1-4: Void

Table A.6.5.1.7.1-5: Void

Table A.6.5.1.7.1-6: Void

Figure A.6.5.1.7.1-1: SNR variation for CSI-RS out-of-sync testing

##### A.6.5.1.7.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on PCell.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 (PCell) at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

The UE shall stop transmitting uplink signal in Cell 1 (PCell) no later than time point C (D1 ms after the start of the time duration T3) on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode

##### A.6.5.1.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.8.1-1, A.6.5.1.81-2, A.6.5.1.8.1-3 and A.6.5.1.8.1-3A below. There is one cells, cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.8.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test. In the test, SSB0 is configured as the BFD-RS.

Table A.6.5.1.8.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.1.8.1-2: General test parameters for FR1 PCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State.2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 2000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 1.24 |
| T4 |  | s | 0.2 |
| T5 |  | s | 1.88 |
| T6 |  | s | 1.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.1.8.1-3: Cell specific test parameters for FR1 for CSI-RS in-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 2 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 3 |  | 1 | -7 | -15 | -4.5 | 1 |
|  | Config 1 | dBm/15kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.6.5.1.8.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is specified in clause A.3.6.1.1. |  |  |  |  |  |  |  |

Table A.6.5.1.8.1-3A: Measurement gap configuration for FR1 CSI-RS in-sync radio link monitoring in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: Void |  |

Table A.6.5.1.8.1-4: Void

Table A.6.5.1.8.1-5: Void

Table A.6.5.1.8.1-6: Void

Figure A.6.5.1.8.1-1: SNR variation for CSI-RS in-sync testing

##### A.6.5.1.8.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.2	Interruption

#### A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1

##### A.6.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE missed ACK/NACK rate does not exceed the limits at NR PSCell interruptions during the measurement on the deactivated NR SCC. This test will verify the missed ACK/NACK rate for PCell in standalone NR specified in clause 8.2.2.2. Supported test configurations are shown in table A.6.5.2.1.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.6.5.2.1.1-2 and A 6.5.2.1.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell, Cell2 is an NR deactivated SCell. Cell1 shall be configured as PCell and Cell2 shall be configured as SCell.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2 and the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector. During T1, PCell is continuously scheduled in DL.

Table A.6.5.2.1.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD – FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD – FDD duplex mode |
| 4 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD – TDD duplex mode |
| 5 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD – TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.2.1.1-2: General test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell2 | Deactivated SCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to Cell1 and Cell2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

Table A.6.5.2.1.1-3: NR cell specific test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | FDD |
|  | Config 2,5 |  | TDD | TDD |
|  | Confiq 3 |  | TDD | FDD |
|  | Confiq 4 |  | FDD | TDD |
| TDD configuration | Config 1 |  | Not Applicable | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.1 | Not Applicable |
|  | Confiq 4 |  | Not Applicable | TDDConf.1.1 |
|  | Confiq 5 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel | Config 1,2,3,4 |  | Note 9 | Note 9 |
|  | Config 5 |  | Note 9 | Note 9 |
| BWoccupied | Config 1,2,3,4 | RB | 52 Note 7 | 52 Note 7 |
|  | Config 5 |  | 106 Note 8 | 106 Note 8 |
| Initial DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.0.1 | DLBWP.0.1 |
|  | Config 5 |  | DLBWP.0.1 | DLBWP.0.1 |
| Dedicated DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.1.1 | DLBWP.1.1 |
|  | Config 5 |  | DLBWP.1.1 | DLBWP.1.1 |
| Initial UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.0.1 |  |
|  | Config 5 |  | ULBWP.0.1 |  |
| Dedicated UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.1.1 |  |
|  | Config 5 |  | ULBWP.1.1 |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  | Config 2 |  | SR.1.2 TDD | SR.1.2 TDD |
|  | Config 3 |  | SR.1.2 TDD | SR.1.1 FDD |
|  | Confiq 4 |  | SR.1.1 FDD | SR.1.2 TDD |
|  | Confiq 5 |  | SR.2.1 TDD | SR.2.1 TDD |
| CSI-RS for tracking | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.1 TDD | TRS.1.1 FDD |
|  | Confiq 4 |  | TRS.1.1 FDD | TRS.1.1 TDD |
|  | Confiq 5 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  | Config 3 |  | CR.1.1 TDD | CR.1.1 FDD |
|  | Confiq 4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  | Confiq 5 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  | Config 3 |  | CCR.1.1 TDD | CCR.1.1 FDD |
|  | Config 4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  | Config 5 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns | Config 1,2,3,4 |  | OP.1 Note 7 | OP.1 Note 7 |
|  | Config 5 |  | OP.1 Note 8 | OP.1 Note 8 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.4 |
| SSB Configuration | Config 1,2,3,4 |  | SSB.1 FR1 | SSB.5 FR1 |
|  | Config 5 |  | SSB.2 FR1 | SSB.6 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 | -87 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1,2,3,4 | dBm/SCS | -104 | -104 |
|  | Config 5 |  | -101 | -101 |
| IoNote3 | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 | -58.96 |
|  | Config 5 | dBm/ 38.16MHz | -52.86 | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 3 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2 defined in clause 12 of TS 38.213 [3]. Note 7: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 8: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 9: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |

##### A.6.5.2.1.2	Test Requirements

The UE shall be continuously scheduled on PCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on PCell.

If the NR PCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on NR PCell immediately before and immediately after an SMTC. Each interruption on NR PCell shall not exceed the value defined in Table A.6.5.2.1.2-1.

If the NR PCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. the interruption on NR PCell shall not exceed the value defined in Table A.6.5.2.1.2-2.

Table A.6.5.2.1.2-1: Interruption duration if the PCell is not in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |

Table A.6.5.2.1.2-2: Interruption duration if the PCell is in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 2 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.2.2	SA interruptions at NR SRS carrier based switching

##### A.6.5.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit aperiodic SRS, the UE can perform carrier based switching to one carrier not configured for PUCCH/PUSCH transmission from a carrier with PUCCH/PUSCH transmission. The test will partly verify the interruption requirements on PCell in clause 8.2.2.2.9.

##### A.6.5.2.2.2	Test Parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the FR1 PCell and Cell 2 is activated SCell on the TDD SCC which operats in downlink without PUCCH/PUSCH. The UE is configured with the SRS switching between PCell and SCell.The test parameters for PCell and SCell are given in Table A.6.5.2.2.2-2 and A.6.5.2.2.2-3 below. The test consists of two successive time periods, with duration of T1 and T2, respectively. Immediately at the beginning of T2, the UE is triggered for SRS switching.

The test equipment verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in PCell.

Table A.6.5.2.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.5.2.2.2-2: General test parameters for SA interruptions at NR SRS carrier based switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1 |
| Configured SCell |  | Cell 2 | Activated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 5 |  |
| T2 | ms | 40 | UE shall perform SRS switching during T2 |

Table A.6.5.2.2.2-3: Cell specific test parameters for SA interruptions at NR SRS carrier based switching

| Parameter |  |  |  | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Duplex mode |  | Config 1 |  |  | FDD | TDD | FDD | TDD |
|  |  | Config 2,3 |  |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  |  | N/A | TDDConf.1.1 | N/A | TDDConf.1.1 |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1,2 |  | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |
| Downlink initial BWP Configuration |  |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP Configuration |  |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |
| TCI state |  |  |  |  | TCI.State.0 |  |  |  |
| TRS Configuration |  |  |  |  | TRS.1.1 TDD |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD | SR.1.1 TDD | SR.1.1 FDD | SR.1.1 TDD |
|  |  | Config 2 |  |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD | SR2.1 TDD | SR2.1 TDD | SR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD | CCR.1.1 TDD | CCR.1.1 FDD | CCR.1.1 TDD |
|  |  | Config 2 |  |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD | CR.1.1 TDD | CR.1.1 FDD | CR.1.1 TDD |
|  |  | Config 2 |  |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD | CR2.1 TDD | CR2.1 TDD | CR2.1 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| SRS Configuration | Config 1,2 |  |  |  | SRS.1 TDD |  |  |  |
|  | Config 3 |  |  |  | SRS.2 TDD |  |  |  |
| SSB Configuration | Config 1,2 |  |  |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  |  |  | SSB.2 FR1 |  |  |  |
| SMTC configuration |  |  |  |  | SMTC.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | Config 1,2,4,5 | dBm/15kHz | -104 |  |  |  |
|  |  |  | Config 3,6 |  | -101 |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |
| SS-RSRPNote3 |  |  | Config 1,2,4,5 | dBm/SCS | -87 |  |  |  |
|  |  |  | Config 3,6 |  | -84 |  |  |  |
| SCH_RP Note 3 |  |  |  | dBm/15 kHz | -87 |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |

Table A.6.5.2.2.2-4: Void

##### A.6.5.2.2.3	Test Requirements

The UE shall be scheduled on PCell continuously throughout the test. During the time duration T2, the interruption on PCell shall not be more than the values specified for SA in clause 8.2.2.2.9.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.3	SCell Activation and Deactivation Delay

#### A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle

##### A.6.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.1.1-1 below. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.1.1-1: known FR1 SCell activation in non-DRX for 160ms SCell measurement cycle supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.1.1-2: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 15 | The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.1.1-3: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | Not applicable |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1,2 |  | MHz | Note 7 |  |  |  |  |  |
|  |  | Config 3 |  |  | Note 7 |  |  |  |  |  |
| BWoccupied |  | Config 1,2 |  | RB | 52 Note 5 |  |  |  |  |  |
|  |  | Config 3 |  |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  |  |  | DLBWP.0.2 |  |  |  |  |  |
| TCI state |  |  |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration |  | Config 1 |  |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |  |  | - |  |  |
|  |  | Config 2 |  |  | SR.1.1 TDD |  |  | - |  |  |
|  |  | Config 3 |  |  | SR.2.1 TDD |  |  | - |  |  |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD |  |  | - |  |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | - |  |  |
|  |  | Config 3 |  |  | CCR.2.1 TDD |  |  | - |  |  |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD |  |  | - |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |  | - |  |  |
|  |  | Config 3 |  |  | CR.2.1 TDD |  |  | - |  |  |
| OCNG Patterns | Config 1,2 |  |  |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  |  |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  |  |  | periodic |  |  | N/A |  |  |
| reportQuantity |  |  |  |  | cri-RI-PMI-CQI |  |  | N/A |  |  |
| CSI reporting periodicity |  | Config 1,2 |  | slot | 5 |  |  | N/A |  |  |
|  |  | Config 3 |  |  | 10 |  |  | N/A |  |  |
| CSI reporting offset |  | Config 1,2 |  | slot | 2 |  |  | N/A |  |  |
|  |  | Config 3 |  |  | 4 |  |  | N/A |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | Config 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  |  |  | Config 3 |  | -101 |  |  |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |  |  |
|  |  |  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 |  |  | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |
|  |  |  | Config 3 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  |  |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Io Note3 |  |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |
|  |  |  | Config 3 | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |  |

##### A.6.5.3.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after slot (n+1+THARQ+3msNR slot length). UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in clause 8.3.

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot m+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During T3 the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

#### A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle

##### A.6.5.3.2.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.6.5.3.1.1. The supported test configurations are the same as defined in clause A.6.5.3.1.1. The test parameters are the same except those described in the following clause. The listed parameter values in Tables A.6.5.3.2.1-1 will replace the values of corresponding parameters in Tables A.4.5.3.1.1-1.

Table A.6.5.3.2.1-1: General test parameters for known FR1 SCell activation case, 320ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |

##### A.6.5.3.2.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + Trs + 5ms.

#### A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

##### A.6.5.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

The supported test configurations are shown in table A.6.5.3.1.1-1 below. The test parameters are given in Tables A.6.5.3.1.1-2 and cell-specific parameters in A.6.5.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two NR carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.6.5.3.3.1-1: General test parameters for unknown FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |

##### A.6.5.3.3.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms as defined in clause 8.3.

#### A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1

##### A.6.5.3.4.1		Test Purpose and Environment

The purpose of this test is to verify fulfillment of direct SCell activation delay and interruption requirements at SCell addition as defined in clause 8.3.4 and 8.2.2, respectively. The supported test configurations are shown in Table A.6.5.3.4.1-1.

The test scenario comprises one PCell (Cell 1) and one SCell (Cell 2) as outlined in Table A.6.5.3.4.1-2. Cell-specific parameters are provided in Table A.6.5.3.4.1-3.

The test consists of two successive time periods with duration T1 and T2, respectively. There are two carriers, each with one cell. Cell 1 (PCell) is on RF channel 1 (PCC), and Cell 2 (SCell) is on RF channel 2 (SCC). Cell 1 and Cell 2 both operate according to one of the configurations in Table A.6.5.3.4.1-1.

Before the test starts the UE is connected to Cell 1 on RF channel 1. The UE is only monitoring RF channel 1 and is not aware of Cell 2 on RF channel 2.

The UE is continuously scheduled in PCell throughout the test.

At the beginning of T1 the UE is configured to measure RF channel 2 in measurement gaps. During T1, the UE detects and measures Cell 2 on RF channel 2, and sends a measurement report containing Cell 2 to the test equipment. After having received a measurement report containing Cell 2, the test equipment deconfigures the measurement gaps and thereafter sends a RRC connection reconfiguration message to the UE by which it configures the SCell (Cell 2) in activated state (sCellState is set to activated). The time between reception of the last measurement report carrying SCell and transmission of the RRC connection reconfiguration message directly activating SCell is kept short enough to allow the SCell to remain known to the UE.

Time period T2 starts when the UE receives the RRC connection reconfiguration message at the UE antenna connector. The corresponding slot at which the message is received at the UE antenna connector is denoted n. The UE shall complete activation of the SCell no later than in slot n + NdirectNR slot length, as specified in clause 8.3.4. From slot n+ NdirectNR slot length and onwards the UE shall report valid CSI both for PCell and SCell.

The test equipment verifies the activation time by counting the slots between the RRC connection reconfiguration message is sent and until CSI report with non-zero CQI for both PCell and SCell is received.

The test equipment verifies that interruptions on other serving cells are within the requirements by counting ACK/NACKs transmitted in PCell.

Table A.6.5.3.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Note: The UE is only required to be tested in one of the supported test configurations

Table A.6.5.3.4.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Inter-frequency neighbor cell (SCell to-be) |  | Cell 2 | Inter-frequency neighbor cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern |  | gp0 | Measurement gap is used during parts of time period T1 for detection of Cell 2. |
| CSI reporting periodicity | ms | 2 | CSI reporting periodicity for periodic reporting of CQI for PCell and, when added, SCell. |
| SCell measurement cycle (measCycleSCell) | ms | 160 | Measurement cycle for SCell does not come into effect in direct activation at SCell addition. |
| Timing offset between Cell 1 and Cell 2 | s | MRTD | The value of maximum timing offset depends upon the carrier aggregation scenario. |
| T1 | s | 7 | During this time period the PCell shall be known and Cell 2 shall be detected as an inter-frequency neighbor cell. |
| T2 | s | 1 | During this time period Cell 2 shall be configured and directly activated as SCell. |

Table A.6.5.3.4.1-3: NR Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration |  | Initial DL |  | DLBWP.0.1 |  |  | --- | DLBWP.0.1 |
|  |  | Initial UL |  | ULBWP.0.1 |  |  |  | --- |
|  |  | Dedicated DL |  | DLBWP.1.1 |  |  |  | DLBWP.1.1 |
|  |  | Dedicated UL |  | ULBWP.1.1 |  |  |  | --- |
| TCI state |  |  |  | TCI.State.0 |  |  | --- | TCI.State.0 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |  |  | --- | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  | CSI-RS.2.1 TDD |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  |  | --- | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  | --- | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD |  |  | --- | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |  |  | --- |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  |
| OCNG Pattern |  |  |  | OP.1 |  |  | OP.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  | SMTC.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 |  | dBm/15kHz | -104 |  |  | -104 |  |
|  | Config 3 |  |  | -101 |  |  | -101 |  |
| Ês/Iot |  |  | dB | 17 |  |  | 17 |  |
| Ês/Noc |  |  | dB | 17 |  |  | 17 |  |
| SS-RSRP Note3 | Config 1,2 |  | dBm/SCS | -87 |  |  | -87 |  |
|  | Config 3 |  |  | -84 |  |  | -84 |  |
| Io Note3 | Config 1,2 |  | dBm/9.36 MHz | -59.0 |  |  | -59.0 |  |
|  | Config 3 |  | dBm/38.16 MHz | -52.9 |  |  | -52.9 |  |
| Propagation condition |  |  |  | AWGN |  |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |  | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.5.3.4.2		Test Requirements

The UE shall complete the direct activation of the SCell no later than at slot n + NdirectNR slot length.

The UE shall report non-zero CQI for SCell from slot n + NdirectNR slot length and onwards throughout time period T2.

The interruption on PCell during direct activation of the SCell shall occur within the interruption window specified in clause 8.3.4 and shall not exceed the length specified in clause 8.2.2.2.11.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1

##### A.6.5.3.5.1	Test Purpose and Environment

This test is to verify the requirement for the FDD-FDD and TDD-TDD intra frequency handover with direct SCell activation requirements specified in subclause 8.3.5.

Supported test configurations are shown in table A.6.5.3.5.1-1. Both handover with direct SCell activation requirements are tested by using the parameters in table A.6.5.3.5.1-2, and A.6.5.3.5.1-3.

The test scenario comprises of three NR FDD or NR TDD FR1 carriers and the 3 cells as given in tables A.6.5.3.5.1-1 and A.6.5.3.5.1-2. The test consists of three successive time periods, with time durations of T1, T2, and T3 respectively.

At the start of time duration T1, the UE is in connected mode with PCell and SCell1 (cell 2) is in activated state and UE is reporting CQI for both PCell and SCell1.

Time period T2 starts when UE receives a handover command to Cell 3 that also activates SCell1 (Cell2). This is done using an RRCReconfiguration message with parameter sCellState set to activated for the SCell1 (Cell 2). The message is sent from the test equipment to the UE and is received in a subframe # denoted n at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (n + Ndirect).

Time period T3 starts at (n + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell1.

Table A.6.5.3.5.1-1: Intra-frequency handover with direct SCell activation from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode SCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode SCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | PCell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode SCell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.3.5.1-2: General test parameters Intra-frequency handover with direct SCell activation from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | PCell |  | Cell 1 |  |
|  | SCell |  | Cell 2 |  |
|  | Target cell |  | Cell 3 |  |
| Final condition | PCell |  | Cell 3 |  |
|  | SCell |  | Cell 2 |  |
|  | neighbour cell |  | Cell 1 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 | UE is in connected mode with PCell and SCell1 (cell 2) is in activated state. UE receives a handover command |
| T2 |  | s | Ndirect | UE shall accomplish the activation of the SCell |
| T3 |  | s | 1 |  |
| THARQ |  | slot | k | k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting |  | ms | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k |  | ms |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.6.5.3.5.1-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover with direct SCell activation test case

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  | 1 |  |  | 2 |  |  | 1 |  |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |
| BWchannel | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP BW | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| DRx Cycle |  | ms | Not Applicable |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |
|  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |
|  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  | SMTC pattern 1 |  |  |  |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |
|  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |
| BWP configuraiton | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  |  |  |  |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |  |  |  |  |
|  |  | dB | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  | dB | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| SSB_RP | Config 1,2 | dBm/SCS | -90 | -90 | -90 | -90 | -90 | -90 | -90 | -90 | -90 |
|  | Config 3 | dBm/SCS | -87 | -87 | -87 | -87 | -87 | -87 | -87 | -87 | -87 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -61.41 | -57.06 | -57.06 | -61.41 | -57.06 | -61.41 | -57.06 | -57.06 | -61.41 |
|  | Config 3 | dBm/ 38.16MHz | -55.31 | -50.96 | -50.96 | -55.31 | -50.96 | -55.31 | -50.96 | -50.96 | -55.31 |
| Propagation condition |  | - | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.5.3.5.2	Test Requirements

The UE shall be capable to transmit valid CSI report for the directly activated SCell1 no later than in subframe n+Ndirect.

The rate of correct observed SCell1 direct activation delay during repeated tests shall be at least 90%.

NOTE:	The SCell activation delay, Ndirect, can be expressed as: Ndirect = TRRC_process + Tinterrupt + T2 + T3 + Tactivation_time + TCSI_Reporting - 3ms, where:

TRRC_Process: RRC procedure delay defined in clause 12 of TS 38.331 [2],

Tinterrupt: Interruption time during handover as specified in clause 6.1.1,

T2: Delay from slot n+TRRC_Process+TinterruptNR slot length  until UE has obtained a valid TA command for the target PCell,

T3: Delay for applying the received TA for uplink transmission in the target PCell, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213,

Tactivation_time and TCSI_Reporting are specified in clause 8.3.2, where the following definitions of TFirstSSB and TFirstSSB_MAX as defined in section 8.3.5 shall apply:

- TFirstSSB: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + (𝑇𝑅𝑅𝐶_𝑃𝑟𝑜𝑐𝑒𝑠𝑠+𝑇𝑖𝑛𝑡𝑒𝑟𝑟𝑢𝑝𝑡+𝑇2+𝑇3)/(N𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ)

- TFirstSSB_MAX: the time to the end of the first complete SSB burst indicated by the SMTC after slot n + (𝑇𝑅𝑅𝐶𝑃𝑟𝑜𝑐𝑒𝑠𝑠+𝑇𝑖𝑛𝑡𝑒𝑟𝑟𝑢𝑝𝑡+𝑇2+𝑇3)/(N𝑅 𝑠𝑙𝑜𝑡 𝑙𝑒𝑛𝑔𝑡ℎ)

This gives a total of Ndirect = 10 + 52 + TIU + T2 + T3 + Tactivation_time + TCSI_Reporting - 3 ms = 62 + 10 + 13 + 6 + 20 + 2 - 3 = 94 ms for test configurations 1 and 2.

This gives a total of Ndirect = 10 + 52 + TIU + T2 + T3 + Tactivation_time + TCSI_Reporting - 3 ms = 62 + 10 + 13 + 6 + 20 + 2 - 3 = 94 ms for test configuration 3.

During T3 the UE shall send valid CSI reports for PCell and SCell1 with non-zero CQI index and continue to send CSI reports for PCell and SCell1 (Cell 2) with non-zero CQI index until the end of T3.

All of the above test requirements shall be fulfilled in order for the observed SCell1 direct activation delay to be counted as correct.

### A.6.5.4	UE UL carrier RRC reconfiguration Delay

#### A.6.5.4.1	UE UL carrier RRC reconfiguration Delay

Table A.6.5.4.1-1 - Table A.6.5.4.1-4 : Void

##### A.6.5.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that when the UE receives a RRC message implying NR UL or Supplementary UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within the time limits specified in clause 8.4.2 and 8.4.3 for configuring and deconfiguring, respectively.

There are two cells: FR1 PCell (cell 1) and FR1 SCell (cell 2). Both NR uplink and supplementary uplink are broadcast by ServingCellConfigCommonSIB. The test parameters for PCell and SCell are given in Table A. 6.5.4.1.1-1, Table A.6.5.4.1.1-2, Table A.6.5.4.1.1-3 and Table A.6.5.4.1.1-4 below.  In test 1, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, NR uplink of cell 2 is configured to UE. At the start of T2, a supplementary uplink of cell 2 is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the supplementary uplink is released through RRCReconfiguration.

In test 2, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, supplementray uplink on cell 2 is configured to UE. At the start of T2, a NR uplink is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the NR uplink is released through RRCReconfiguration.

Table A.6.5.4.1.1-1: Supported test configurations

| Configuration | PCell (Cell 1) | SCell (Cell 2) |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 2 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 3 | 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| 4 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 5 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 6 | 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| 7 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 8 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode; SUL: 15 kHz SCS, ≥10 MHz bandwidth, SUL duplex mode |
| 9 | 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, ≥40 MHz bandwidth, SUL duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: he UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |  |

Table A.6.5.4.1.1-2: General test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on Pcell

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 1, 2 | Two radio channels are used for these two tests. |
| Active cell |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Cell 1: FR1 PCell Cell 2: FR1 SCell | PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Normal |  |
| DRX |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Measurement gap pattern Id |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Filter coefficient |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 0 | L3 filtering is not used |
| T1 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T2 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T3 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |

Table A.6.5.4.1.1-3: NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on PCell (Cell 1)

| Parameter | Unit | Test Configuration | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 |  |  | 1 |  |  |
| TDD configuration |  | Conf 1, 2, 3 | N/A |  |  | N/A |  |  |
|  |  | Conf 4, 5, 6 | TDD Conf.1.1 |  |  | TDD Conf.1.1 |  |  |
|  |  | Conf 7, 8, 9 | TDD Conf.2.1 |  |  | TDD Conf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 2, 3 | Note 6 |  |  | Note 6 |  |  |
|  |  | Conf 4, 5, 6 | Note 6 |  |  | Note 6 |  |  |
|  |  | Conf 7, 8, 9 | Note 6 |  |  | Note 6 |  |  |
|  |  | Conf 1, 2, 3 | 52 Note 4 |  |  | 52 Note 4 |  |  |
| BWoccupied | RB | Conf 4, 5, 6 | 52 Note 4 |  |  | 52 Note 4 |  |  |
|  |  | Conf 7, 8, 9 | 106 Note 5 |  |  | 106 Note 5 |  |  |
| PDSCH reference measurement channel as defined in A.3.1.1 |  | Conf 1, 2, 3 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | SR 2.1 TDD |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET reference measurement channel as defined in A.3.1.2 |  | Conf 1, 2, 3 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| RMC CORESET reference measurement channel as defined in A.3.1.3 |  | Conf 1, 2, 3 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 3, 4, 5, 6 | OP.1 Note 4 |  |  | OP.1 Note 4 |  |  |
|  |  | Config 7, 8, 9 | OP.1 Note 5 |  |  | OP.1 Note 5 |  |  |
| SSB configuration |  | Conf 1, 2, 3, 4, 5, 6 | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 7, 8, 9 | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 2 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 3 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 4 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 5 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 6 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 7 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 8 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 9 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1,2,3,4,5,6 | -102 |  |  | -102 |  |  |
|  |  | Conf 7,8,9 | -99 |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1,2,3,4,5,6 | -86 | -86 | -86 | -86 | -86 | -86 |
|  |  | Conf 7,8,9 | -83 | -83 | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1,2,3,4,5,6 | -57.9 | -57.9 | -57.9 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 7,8,9 | -51.8 | -51.8 | -51.8 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |

Table A.6.5.4.1.1-4: NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on SCell (Cell 2)

| Parameter | Unit | Test Configuration | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 2 |  |  |  | 2 |  |  |
| TDD configuration |  | Conf 1, 4, 7 | N/A |  |  |  | N/A |  |  |
|  |  | Conf 2, 5, 8 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | Conf 3, 6, 9 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 4, 7 | Note 6 |  |  |  | Note 6 |  |  |
|  |  | Conf 2, 5, 8 | Note 6 |  |  |  | Note 6 |  |  |
|  |  | Conf 3, 6, 9 | Note 6 |  |  |  | Note 6 |  |  |
|  |  | Conf 1, 4, 7 | 52 Note 4 |  |  |  | 52 Note 4 |  |  |
| BWoccupied | RB | Conf 2, 5, 8 | 52 Note 4 |  |  |  | 52 Note 4 |  |  |
|  |  | Conf 3, 6, 9 | 106 Note 5 |  |  |  | 106 Note 5 |  |  |
| PUSCH parameters for NR UL carrier |  | Conf 1, 4, 7 | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |  | N/A | G-FR1-A3-10 in [13] | N/A |
|  |  | Conf 2, 5, 8 | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |  | N/A | G-FR1-A3-10 in [13] | N/A |
|  |  | Conf 3, 6, 9 | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] |  | N/A | G-FR1-A3-14 in [13] | N/A |
| PUCCH parameters For NR UL carrier |  | Conf 1, 4, 7 | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |  | N/A | N/A | N/A |
|  |  | Conf 2, 5, 8 | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |  | N/A | N/A | N/A |
|  |  | Conf 3, 6, 9 | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] |  | N/A | N/A | N/A |
| PUSCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | G-FR1-A3-10 in [13] | N/A |  | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |
|  |  | Conf 2, 5, 8 | N/A | G-FR1-A3-10 in [13] | N/A |  | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] | G-FR1-A3-10 in [13] |
|  |  | Conf 3, 6, 9 | N/A | G-FR1-A3-14 in [13] | N/A |  | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] | G-FR1-A3-14 in [13] |
| PUCCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |
|  |  | Conf 2, 5, 8 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] | Table 8.3.3.1.2-1 in [13] |
|  |  | Conf 3, 6, 9 | N/A | N/A | N/A |  | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] | Table 8.3.3.1.2-2 in [13] |
| PDSCH reference measurement channel as defined in A.3.1.1 |  | Conf 1, 4, 7 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | SR 2.1 TDD |  |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET reference measurement channel as defined in A.3.1.2 |  | Conf 1, 4, 7 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| RMC CORESET reference measurement channel as defined in A.3.1.3 |  | Conf 1, 4, 7 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 4, 5, 7, 8 | OP.1 Note 4 |  |  |  | OP.1 Note 4 |  |  |
|  |  | Conf 3, 6, 9 | OP.1 Note 5 |  |  |  | OP.1 Note 5 |  |  |
| SSB configuration |  | Conf 1, 2, 4, 5, 7,8 | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 3, 6, 9 | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  |  | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 2 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 3 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 4 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 5 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 6 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 7 | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 8 | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 9 | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -102 |  |  |  | -102 |  |  |
|  |  | Conf 3, 6, 9 | -99 |  |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -86 | -86 |  | -86 | -86 | -86 | -86 |
|  |  | Conf 3, 6, 9 | -83 | -83 |  | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1, 2, 4, 5, 7,8 | -57.9 | -57.9 |  | -57.9 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 3, 6, 9 | -51.8 | -51.8 |  | -51.8 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |

##### A.6.5.4.1.2	Test Requirements

In test 1 the UE shall be ready to start transmission on the supplementary uplink carrier on SCell within 20ms from the start of T2.

In test 1 the UE shall stop the transmission on the supplementary uplink carrier on SCell within 20ms from the start of T3.

In test 2 the UE shall be ready to start transmission on the NR uplink carrier on SCell within 20ms from the start of T2.

In test 2 the UE shall stop the transmission on the NR uplink carrier on SCell within 20ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed UE UL carrier configuration delay and UE UL carrier release delay to be counted as correct. The rate of correct observed UE UL carrier configuration delay and UE UL carrier release delay during repeated tests shall be at least 90%.

#### A.6.5.4.2	Void

### A.6.5.5	Beam Failure Detection and Link recovery procedures

#### A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode

##### A.6.5.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.1.1-1, A.6.5.5.1.1-2, A.6.5.5.1.1-3 and A.6.5.5.1.1-4 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.1.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.6.5.5.1.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 1.

Table A.6.5.5.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.1.1-2: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |
|  |  |  |  | Config 2, 3 |  | TDD |  |
| BWchannel |  |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration |  |  |  | Config 1 |  | Not Applicable |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  |  | Config 3 |  | CR.2.1 TDD |  |
| SSB Configuration |  |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  |  | Config 3 |  | SSB.4 FR1 |  |
| SMTC Configuration |  |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  |  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  |  | Config 1, 2 |  | Table A.3.8.2.2-1 |  |
|  |  |  |  | Config 3 |  | Table A.3.8.2.2-1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  |  | Aggregation level |  |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  |  | gp0 |  |
| gapOffset |  |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1, 2 |  |  |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 0.2 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 0.37 |  |
| T3 |  |  |  |  | s | 0.24 |  |
| T4 |  |  |  |  | s | 0 |  |
| T5 |  |  |  |  | s | 0.17 |  |
| D1 |  |  |  |  | s | 0.13 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table A.6.5.5.1.1-3: Cell specific test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 2 |  | -108 | -108 | -88 | -88 | -88 |
|  | Config 3 |  | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.5.1.1-4: Void

Figure A.6.5.5.1.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

A.6.5.5.1.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 120+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode

##### A.6.5.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.2.1-1, A.6.5.5.2.1-2, A.6.5.5.2.1-3, A.6.5.5.2.1-4 and A.6.5.5.2.1-5 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.2.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.6.5.5.2.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.5.2.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.2.1-2: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |
|  |  |  | Config 2, 3 |  | TDD |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |
| SSB Configuration |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  | Config 3 |  | SSB.4 FR1 |  |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  | Table A.3.8.2.2-1 |  |
|  |  |  | Config 3 |  | Table A.3.8.2.2-1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  | 2 |  |
|  |  | Aggregation level |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  |  | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | Absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1, 2 |  |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 5.17 |  |
| T3 |  |  |  | s | 3.24 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 1.97 |  |
| D1 |  |  |  | s | 1.93 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

Table A.6.5.5.2.1-3: Cell specific test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 2 |  | -108 | -108 | -88 | -88 | -88 |
|  | Config 3 |  | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.5.2.1-4: Void

Table A.6.5.5.2.1-5: Void

Figure A.6.5.5.2.1-1: SNR and L1-RSRP variation for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.6.5.5.2.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 1920+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode

##### A.6.5.5.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.3.1-1, A.6.5.5.3.1-2, and below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.3.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0 in the active cell to emulate CSI-RS based beam failure. Figure A.6.5.5.3.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 3 ms. In the test, DRX configuration is not enabled.

Table A.6.5.5.3.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.3.1-2: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode | Config 1 |  |  |  | FDD |  |
|  | Config 2, 3 |  |  |  | TDD |  |
| TDD Configuration | Config 1 |  |  |  | Not Applicable |  |
|  | Config 2 |  |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  |  | TDDConf.2.1 |  |
| CORESET Reference Channel | Config 1 |  |  |  | CR.1.1 FDD | A.3.1.2 |
|  | Config 2 |  |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  |  | SSB.3 FR1 | A.3.10 |
|  | Config 2 |  |  |  | SSB.3 FR1 |  |
|  | Config 3 |  |  |  | SSB.4 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  |  | SMTC.1 | A.3.11 |
|  | Config 3 |  |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  |  | 15 KHz |  |
|  | Config 3 |  |  |  | 30 KHz |  |
| PRACH Configuration | Config 1, 2 |  |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
|  | Config 3 |  |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  |  | 0 |  |
| OCNG parameters |  |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  | Aggregation level |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  |  | 1 | N |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 and q1 |  |  | Config 1 |  | CSI-RS.1.2 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  | CSI-RS.1.1 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS |  |  | Config 1 |  | CSI-RS.1.2 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 0.2 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 0.18 |  |
| T3 |  |  |  | s | 0.14 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 0.08 |  |
| D1 |  |  |  | s | 0.04 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

Table A.6.5.5.3.1-3: Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | 10 | 10 | 10 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |
|  | Config 2 |  | -108 | -108 | -88 | -88 | -88 |
|  | Config 3 |  | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.5.3.1-4: Void

Table A.6.5.5.3.1-5: Void

Figure A.6.5.5.3.1-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

##### A.6.5.5.3.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 30+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.6.5.5.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.4.1-1, A.6.5.5.4.1-2, A.6.5.5.4.1-3, and A.6.5.5.4.1-4 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.4.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0 in the active cell to emulate CSI-RS based beam failure. Figure A.6.5.5.4.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.5.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.4.1-2: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode | Config 1 |  |  | FDD |  |
|  | Config 2, 3 |  |  | TDD |  |
| TDD Configuration | Config 1 |  |  | Not Applicable |  |
|  | Config 2 |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  | TDDConf..21 |  |
| CORESET Reference Channel | Config 1 |  |  | CR.1.1 FDD | A.3.1.2 |
|  | Config 2 |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  | SSB.3 FR1 | A.3.10 |
|  | Config 2 |  |  | SSB.3 FR1 |  |
|  | Config 3 |  |  | SSB.4 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  | SMTC.1 | A.3.11 |
|  | Config 3 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  | 15 KHz |  |
|  | Config 3 |  |  | 30 KHz |  |
| PRACH Configuration | Config 1, 2 |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
|  | Config 3 |  |  | FR1 PRACH configuration 4 | A.3.8.2 |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1, 2 | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  | -95 |  |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 and q1 | Config 1 |  |  | CSI-RS.1.2 FDD | A.3.14 .1 |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.1.1 FDD | A.3.14.1 |
|  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| TRS configuration | Config 1 |  |  | TRS.1.1 FDD |  |
|  | Config 2 |  |  | TRS.1.1 TDD |  |
|  | Config 3 |  |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS | Config 1 |  |  | CSI-RS.1.2 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 8.37 |  |
| T3 |  |  | s | 6.44 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.5.4.1-3: Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | 10 | 10 | 10 |
| CSI-RS_RP of set q1 | Config 1 | dB/SCS kHz | -110 | -110 | -88 | -88 | -88 |
|  | Config 2 |  | -110 | -110 | -88 | -88 | -88 |
|  | Config 3 |  | -107 | -107 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 KHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Table A.6.5.5.4.1-4: Void

Table A.6.5.5.4.1-5: Void

Table A.6.5.5.4.1-6: Void

Figure A.6.5.5.4.1-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in DRX mode

##### A.6.5.5.4.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 1920+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode

##### A.6.5.5.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP without schedulingRequestID-BFR-SCell-r16 configuration, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.5.1-1, A.6.5.5.5.1-2, and below. There are two cells, cell 1 is the PCell and cell 2 is the SCell, in the test. UE is not provided by schedulingRequestID-BFR-SCell-r16, i.e., no configuration for PUCCH transmission resources, and UE shall perform the random access procedure to recover the beam failure. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.5.1-1 shows the SNR of the CSI-RS in set q0 in the active SCell to emulate beam failure. Figure A.6.5.5.5.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.  Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is not enabled.

Table A.6.5.5.5.1-1: Supported test configurations for FR1 PCell and SCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.5.1-2: General test parameters for FR1 SCell for beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Active SCell |  |  |  |  | Cell 2 |  |
| RF Channel Number |  |  |  |  | 2 |  |
| Duplex mode | Config 1 |  |  |  | FDD |  |
|  | Config 2, 3 |  |  |  | TDD |  |
| TDD Configuration | Config 1 |  |  |  | Not Applicable |  |
|  | Config 2 |  |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  |  | TDDConf.2.1 |  |
| CORESET Reference Channel | Config 1 |  |  |  | CR.1.1 FDD | A.3.1.2 |
|  | Config 2 |  |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  |  | SSB.1 FR1 | A.3.10 |
|  | Config 2 |  |  |  | SSB.1 FR1 |  |
|  | Config 3 |  |  |  | SSB.2 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  |  | SMTC.1 | A.3.11 |
|  | Config 3 |  |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  |  | 15 KHz |  |
|  | Config 3 |  |  |  | 30 KHz |  |
| PRACH Configuration | Config 1, 2, 4, 5 |  |  |  | Table A.3.8.2.1-1 |  |
|  | Config 3, 6 |  |  |  | Table A.3.8.2.1-1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 in activated SCell |  |  |  |  | 0 |  |
| OCNG parameters |  |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  | Aggregation level |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  | N.A. |  |
| schedulingRequestID-BFR-SCell-r16 |  |  |  |  | absent | When the field is absent, the random access procedure will be triggered for SCell BFR |
| SSB Index assigned as CBD RS (q1) in activated SCell |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  |  | -95 |  |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 |  |  | Config 1 |  | CSI-RS.1.2 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  | CSI-RS.1.1 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.1 TDD |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS |  |  | Config 1 |  | CSI-RS.1.2 FDD | A.3.14 |
|  |  |  | Config 2 |  | CSI-RS.1.2 TDD |  |
|  |  |  | Config 3 |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  | 2 |  |
| T1 |  |  |  | s | 0.2 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 0.18 |  |
| T3 |  |  |  | s | 0.14 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 0.17 |  |
| D1 |  |  |  | s | 0.13 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

Table A.6.5.5.5.1-3: Cell specific test parameters for FR1 SCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Cell1 | Test 1 Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 to T5 | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 |  | 5 | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | -10 | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 |  | -10 | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 |  | -10 | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/ | -108 | -108 | -108 | -88 | -88 | -88 |
|  | Config 2 | SCS kHz | -108 | -108 | -108 | -88 | -88 | -88 |
|  | Config 3 |  | -105 | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 | dBm/ | -98 | -98 |  |  |  |  |
|  | Config 2 | 15kHz | -98 | -98 |  |  |  |  |
|  | Config 3 |  | -98 | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |  |

Figure A.6.5.5.5.1-1: SNR and L1-RSRP variation for beam failure detection and link recovery testing in for SCell non-DRX mode

##### A.6.5.5.5.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 120+10 ms after the start of T5, the UE shall transmit preamble for UL-SCH resource application, followed by MAC-CE on the assigned uplink resources containing  a beam associated with the candidate beam set q1. The UE shall not transmit preamble earlier than time point B.

During T5, the System Simulator shall transmit a Random Access Response to UE after the System Simulator receives the preamble from UE. The UE shall transmit the msg.3 containing candidate beam set q1 for SCell BFR if UE receives the Random Access Response.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode

##### A.6.5.5.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based  beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP without schedulingRequestID-BFR-SCell-r16 configuration, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.6.5.5.6.1-1, A.6.5.5.6.1-2, A.6.5.5.6.1-3, and A.6.5.5.6.1-4 below. There are two cells, cell 1 is the PCell and cell 2 is the SCell, in the test. UE is not provided by schedulingRequestID-BFR-SCell-r16, i.e., no configuration for PUCCH transmission resources, and UE shall perform the random access procedure to recover the beam failure. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.5.6.1-1 shows the SNR of the CSI-RS in set q0 in the active SCell to emulate beam failure. Figure A.6.5.5.6.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled in SCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.5.6.1-1: Supported test configurations for FR1 PCell and SCell

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.5.6.1-2: General test parameters for FR1 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Active SCell |  |  |  | Cell 2 |  |
| RF Channel Number |  |  |  | 2 |  |
| Duplex mode | Config 1 |  |  | FDD |  |
|  | Config 2, 3 |  |  | TDD |  |
| TDD Configuration | Config 1 |  |  | Not Applicable |  |
|  | Config 2 |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  | TDDConf..21 |  |
| CORESET Reference Channel | Config 1 |  |  | CR.1.1 FDD | A.3.1.2 |
|  | Config 2 |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  | SSB.1 FR1 | A.3.10 |
|  | Config 2 |  |  | SSB.1 FR1 |  |
|  | Config 3 |  |  | SSB.2 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  | SMTC.1 | A.3.11 |
|  | Config 3 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  | 15 KHz |  |
|  | Config 3 |  |  | 30 KHz |  |
| PRACH Configuration | Config 1, 2, 4, 5 |  |  | Table A.3.8.2.1-1 |  |
|  | Config 3, 6 |  |  | Table A.3.8.2.1-1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 in activated SCell |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 | A.3.2.1 |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  |  | N.A. |  |
| schedulingRequestID-BFR-SCell-r16 |  |  |  | absent | When the field is absent, the random access procedure will be triggered for SCell BFR |
| SSB Index assigned as CBD RS (q1) in activated SCell |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | Config 1, 2 | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  | -95 |  |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 | Config 1 |  |  | CSI-RS.1.2 FDD | A.3.14 .1 |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.1.1 FDD | A.3.14.1 |
|  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| TRS configuration | Config 1 |  |  | TRS.1.1 FDD |  |
|  | Config 2 |  |  | TRS.1.1 TDD |  |
|  | Config 3 |  |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS | Config 1 |  |  | CSI-RS.1.2 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 8.37 |  |
| T3 |  |  | s | 6.44 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.6.5.5.6.1-3: Cell specific test parameters for FR1 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Cell 1 | Test 1 Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 to T5 | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
|  | Config 3 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | -10 | -10 | -10 | 10 | 10 | 10 |
|  | Config 2 | dB | -10 | -10 | -10 | 10 | 10 | 10 |
|  | Config 3 | dB | -10 | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/ | -110 | -110 | -110 | -88 | -88 | -88 |
|  | Config 2 | SCS kHz | -110 | -110 | -110 | -88 | -88 | -88 |
|  | Config 3 |  | -107 | -107 | -107 | -85 | -85 | -85 |
|  | Config 1 | dBm/15 | -98 | -98 |  |  |  |  |
|  | Config 2 | kHz | -98 | -98 |  |  |  |  |
|  | Config 3 |  | -98 | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |  |

Figure A.6.5.5.6.1-1: SNR and L1-RSRP variation for beam failure detection and link recovery testing for SCell in DRX mode

##### A.6.5.5.6.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 120+10 ms after the start of T5, the UE shall transmit preamble for UL-SCH resource application, followed by MAC-CE on the assigned uplink resources containing  a beam associated with the candidate beam set q1. The UE shall not transmit preamble earlier than time point B.

During T5, the System Simulator shall transmit a Random Access Response to UE after the System Simulator receives the preamble from UE. The UE shall transmit the msg.3 containing candidate beam set q1 for SCell BFR if UE receives the Random Access Response.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.6	Active BWP switch

#### A.6.5.6.1	DCI-based and Timer-based Active BWP Switch

##### A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA

A.6.5.6.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations are shown in Table A.6.5.6.1.1.1-1 below. The test scenario comprises of one PCell (Cell 1) and one SCell (Cell 2) as given in Table A.6.5.6.1.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.1.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

UE is configured with 2 different UE-specific downlink bandwidth parts for SCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for PCell, BWP-0 in Cell 1 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in SCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in PCell.

-	UE is configured with a bwp-InactivityTimer timer value for SCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for SCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in SCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of SCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell on PCell no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on SCell’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to BWP switch on SCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on SCell (Cell 2).

During T3,

The time period T3 starts from the slot #j, where j is the first  slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of SCell’s slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell on PCell at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on SCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to BWP switch of SCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in SCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of SCell, respectively.

Table A.6.5.6.1.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD - FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD - TDD duplex mode |
| 3 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD - FDD duplex mode |
| 4 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD - TDD duplex mode |
| 5 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD - TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.6.5.6.1.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.6.5.6.1.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  | Unit | Cell 1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | FDD |
|  | Config 2,5 |  | TDD | TDD |
|  | Config 3 |  | TDD | FDD |
|  | Config 4 |  | FDD | TDD |
| TDD configuration | Config 1 |  | Not Applicable | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.1 | Not Applicable |
|  | Config 4 |  | Not Applicable | TDDConf.1.1 |
|  | Config 5 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel | Config 1,2,3,4 |  | Note 7 | Note 7 |
|  | Config 5 |  | Note 7 | Note 7 |
| BWoccupied | Config 1,2,3,4 | RB | 52 Note 5 | 52 Note 5 |
|  | Config 5 |  | 106 Note 6 | 106 Note 6 |
| Active BWP ID |  |  | 1, 2 | 0 |
| Initial DL BWP Configuration |  |  | DLBWP.0.2Note4 |  |
| Initial UL BWP Configuration |  |  | ULBWP.0.2Note4 |  |
| Active DL BWP-0 Configuration |  |  | DLBWP.0.2Note4 | N.A. |
| Active DL BWP-1 Configuration |  |  | N.A. | DLBWP.1.1Note4 |
| Active DL BWP-2 Configuration |  |  | N.A. | DLBWP.1.3Note4 |
| Active UL BWP-0 Configuration |  |  | ULBWP.0.2Note4 | N.A. |
| Active UL BWP-1 Configuration |  |  | N.A. | ULBWP.1.1Note4 |
| Active UL BWP-2 Configuration |  |  | N.A. | ULBWP.1.3Note4 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  | Config 3 |  | SR.1.1 TDD | SR.1.1 FDD |
|  | Config 4 |  | SR.1.1 FDD | SR.1.1 TDD |
|  | Config 5 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  | Config 3 |  | CR.1.1 TDD | CR.1.1 FDD |
|  | Config 4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  | Config 5 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.2 FDD | CCR.1.2 FDD |
|  | Config 2 |  | CCR.1.2 TDD | CCR.1.2 TDD |
|  | Config 3 |  | CCR.1.2 TDD | CCR.1.2 FDD |
|  | Config 4 |  | CCR.1.2 FDD | CCR.1.2 TDD |
|  | Config 5 |  | CCR.2.4 TDD | CCR.2.4 TDD |
| OCNG Patterns | Config 1,2,3,4 |  | OP.1 Note 5 |  |
|  | Config 5 |  | OP.1 Note 6 |  |
| SSB Configuration | Config 1,2,3,4 |  | SSB.1 FR1 |  |
|  | Config 5 |  | SSB.2 FR1 |  |
| SMTC Configuration |  |  | SMTC.1 |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 | Config 1,2,3,4 | dBm/SCS | -104 | -104 |
|  | Config 5 |  | -101 | -101 |
| NocNote 2 |  | dBm/15KHz | -104 | -104 |
| SS-RSRP Note 3 | Config 1,2,3,4 | dBm/SCS | -87 | -87 |
|  | Config 5 |  | -84 | -84 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| IoNote3 | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 | -58.96 |
|  | Config 5 | dBm/ 38.16MHz | -52.86 | -52.86 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 RBs) from FC,low, and Io is independent of the BWchannel configured. Note 7: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |

A.6.5.6.1.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of PCell interruption during SCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA

A.6.5.6.1.2.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6.

The supported test configurations are shown in Table A.6.5.6.1.2.1-1. The test scenario comprises of one cell (Cell 1) as given in Table A.6.5.6.1.2.1-2. Cell-specific parameters of the cell are specified in Table A.6.5.6.1.2.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE is configured with 2 different UE-specific downlink bandwidth parts, BWP-1 and BWP-2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

-	UE is configured with a bwp-InactivityTimer timer value for Cell1.

The cell has constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for DL BWP switch, sent from the test equipment to the UE, is received at the UE side in Cell1’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH on the first DL slot that occurs after the beginning of Cell1’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the Cell1 no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on Cell1’s BWP-2 starting from the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

During T2, the test equipment won’t transmit DCI format for PDSCH reception on Cell1.

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH on the first DL slot that occurs after the beginning of Cell1’s slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the Cell1 at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on Cell1’s BWP-1 starting from the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The test equipment verifies the DL BWP switch time by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

Table A.6.5.6.1.2.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: A UE which fulfils the requirements in test case A.6.5.6.1.1 can skip the test cases in A.6.5.6.1.2. |  |

Table A.6.5.6.1.2.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | 200 |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.6.5.6.1.2.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2,3 |  | TDD |
| TDD configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  |  | Config 1 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | Config 1,2,3 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  |  | Config 1,2,3 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  |  | Config 1,2,3 |  | DLBWP.1.3 Note 4 |
| Initial UL BWP Configuration |  |  | Config 1,2,3 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  |  | Config 1,2,3 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  |  | Config 1,2,3 |  | ULBWP.1.3 Note 4 |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |
|  |  |  | Config 2 |  | SR.1.1 TDD |
|  |  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | Config 1 |  | CCR.1.2 FDD |
|  |  |  | Config 2 |  | CCR.1.2 TDD |
|  |  |  | Config 3 |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  |  | Config 1,2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 1x2 Low |
| TRS Configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2 |  |  | dBm/SCS | -104 |
|  | Config 3 |  |  |  | -101 |
| NocNote 2 |  |  |  | dBm/15kHz | -104 |
| SS-RSRP Note 3 | Config 1,2 |  |  | dBm/SCS | -87 |
|  | Config 3 |  |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.6.5.6.1.2.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed Cell1 active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

#### A.6.5.6.2	RRC-based Active BWP Switch

##### A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA

A.6.5.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in clause 8.6.

The supported test configurations are shown in Table A.6.5.6.2.1.1-1. The test scenario comprises of one Cell (Cell 1) as given in Table A.6.5.6.2.1.1-2. Cell-specific parameters of Cell are specified in Table A.6.5.6.2.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in Cell 1.

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated bandwidth part configuration, sent from the test equipment to the UE, is completely received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its bandwidth part with the updated bandwidth part BWP-1 of final condition.

The UE shall be able to receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length as defined in clause 8.6.3 and starts to report valid ACK/NACK for the PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1 on BWP-1 of final condition. The UE shall be continuously scheduled on PCell’s BWP-1 of final condition starting from the first DL slot right after slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length.

TRRCprocessingDelay and TBWPswitchDelayRRC are defined in clause 8.6.3.

The test equipment verifies the DL BWP switch time in Cell by counting the time from the time when the RRC Reconfiguration message including updated BWP configuration is sent till the time when a vaild ACK/NACK is received is received.

Table A.6.5.6.2.1.1-1: DL BWP switch supported test configurations in SA scenario

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.6.2.1.1-2: General test parameters for DL BWP switch in SA scenario

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

Table A.6.5.6.2.1.1-3: NR Cell specific test parameters for DL BWP switch in SA scenario

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  |  | FDD |
|  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  | Config 1 |  |  | Not Applicable |
|  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  | Config 1 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  | 1 |
| Initial DL BWP Configuration |  | Config 1,2, 3 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  | Config 1,2, 3 |  |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration | Config 1, 2, 3 |  |  | DLBWP.1.3 |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3 |  |  | ULBWP.1.3 |
| Final Condition | Active DL BWP-1 Configuration | Config 1, 2, 3 |  |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration | Config 1, 2, 3 |  |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |
|  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD |
|  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.2 FDD |
|  |  | Config 2 |  |  | CCR.1.2 TDD |
|  |  | Config 3 |  |  | CCR.2.4 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2 |  |  | SSB.1 FR1 |
|  |  | Config 3 |  |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  | Config 1 |  | TRS.1.1 FDD |
|  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  | Config 1,2 |  | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.6.5.6.2.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for the Cell from the first DL slot that occurs right after the begining of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

##### A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA

###### A.6.5.6.3.1.1	Test Purpose and Environment

The purpose of this test is to verify requirements on the DL BWP switch delay on multiple CCs and interruption requirement for NR victim cell, both defined in clause 8.6.

The supported test configurations are shown in Table A.6.5.6.3.1.1-1 below. The test scenario comprises of one NR PCell (Cell 1) and two NR SCells (Cell 2 and Cell 3) as given in Table A.6.5.6.3.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.3.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) and SCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 3) to ensure that the UE will have ACK/NACK sending.Before the test starts,

-	UE is connected to PCell (Cell 1) on radio channel 1 (PCC), and SCell (Cell 2) on radio channel 2 (SCC) and SCell (Cell 3) on radio channel 3(SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PCell and SCell (Cell 2), BWP-1 and BWP-2, in Cell 1 and Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with a single UE-specific downlink bandwidth part, BWP-0, for SCell (Cell 3). BWP-0 includes the bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell and SCell (Cell 2).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell (Cell 3).

-	UE is configured with a bwp-InactivityTimer timer value for PCell and SCell (Cell 2).

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for both PCell and SCell (Cell 2) DL BWP switch, sent from the test equipment to the UE, is received at the UE side in both PCell’s and SCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2 at both PCell and SCell (Cell 2).

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s and SCell (Cell 2)’s DL slot (i+ TMultipleBWPswitchDelay) as defined in clause 8.6.2A.1 and starts to report valid ACK/NACK for the both PCell and SCell (Cell 2) no later than the first UL slot that occurs after the beginning of slot (i+ TMultipleBWPswitchDelay +k1). The UE shall be continuously scheduled on both PCell’s and SCell (Cell 2)’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+ TMultipleBWPswitchDelay).

The starting time of SCell (Cell 3) interruption due to BWP switch on PCell and SCell (Cell 2) shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PCell (Cell 1) and SCell (Cell 2).

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1 on both PCell and SCell (Cell 2).

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s an SCell (Cell 2)’s slot (j+ TMultipleBWPswitchDelay) as defined in clause 8.6.2A.1 and starts to report valid ACK/NACK for the PCell and SCell (Cell 2) no later than the first UL slot that occurs after the beginning of slot (j+ TMultipleBWPswitchDelay +k1). The UE shall be continuously scheduled on PCell’s and SCell (Cell 2)’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+ TMultipleBWPswitchDelay).

The starting time of SCell (Cell 3) interruption due to BWP switch of PCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in PCell and SCells by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to SCell (Cell 3) is carried out in the correct time span by monitoring ACK/NACK sent in SCell (Cell 3) during BWP switch of PCell, respectively.

Table A.6.5.6.3.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD -FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – FDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 5 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD - TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.6.3.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2; Cell 3 | SCell on RF channel number 2 and number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCells (Cell 2 and Cell 3) |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for Cell 1 on SCC. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for Cell 2 on SCC. |
| Cell2 and Cell 3 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |

Table A.6.5.6.3.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  | Unit | Cell 1 and Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 | FR1 |
| Duplex mode |  | Config 1 |  | FDD | FDD |
|  |  | Config 2,5 |  | TDD | TDD |
|  |  | Config 3 |  | TDD | FDD |
|  |  | Config 4 |  | FDD | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.1.1 | Not Applicable |
|  |  | Config 4 |  | Not Applicable | TDDConf.1.1 |
|  |  | Config 5 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | Config 1,2,3,4 |  | 10 MHz: NRB,c = 52 | 10 MHz: NRB,c = 52 |
|  |  | Config 5 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 1, 2 | 0 |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |  |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 |  |
| Active DL BWP-0 Configuration |  |  |  | N.A. | DLBWP.0.2Note4 |
| Active DL BWP-1 Configuration |  |  |  | DLBWP.1.1Note4 | N.A. |
| Active DL BWP-2 Configuration |  |  |  | DLBWP.1.3Note4 | N.A. |
| Active UL BWP-0 Configuration |  |  |  | N.A. | ULBWP.0.2Note4 |
| Active UL BWP-1 Configuration |  |  |  | ULBWP.1.1Note4 | N.A. |
| Active UL BWP-2 Configuration |  |  |  | ULBWP.1.3Note4 | N.A. |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  | SR.1.1 TDD | SR.1.1 FDD |
|  |  | Config 4 |  | SR.1.1 FDD | SR.1.1 TDD |
|  |  | Config 5 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | Config 3 |  | CR.1.1 TDD | CR.1.1 FDD |
|  |  | Config 4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  |  | Config 5 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.1.1 TDD | CCR.1.1 FDD |
|  |  | Config 4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  |  | Config 5 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |  |
| SSB Configuration |  | Config 1,2,3,4 |  | SSB.1 FR1 |  |
|  |  | Config 5 |  | SSB.2 FR1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2,3,4 |  | dBm/SCS | -104 | -104 |
|  | Config 5 |  |  | -101 | -101 |
| NocNote 2 |  |  | dBm/15KHz | -104 | -104 |
| SS-RSRP Note 3 | Config 1,2,3,4 |  | dBm/SCS | -87 | -87 |
|  | Config 5 |  |  | -84 | -84 |
| Ês/Iot |  |  | dB | 17 | 17 |
| Ês/Noc |  |  | dB | 17 | 17 |
| IoNote3 |  | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 | -58.96 |
|  |  | Config 5 | dBm/ 38.16MHz | -52.86 | -52.86 |
| Propagation Condition |  |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

###### A.6.5.6.3.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for both PCell and SCell (Cell 2) from the first UL slot that occurs after the beginning of DL slot (i+ TMultipleBWPswitchDelay +k1).

During T3, the UE shall start to send the ACK/NACK for both PCell and SCell (Cell 2) from the first UL slot that occurs after the beginning of DL slot (j+ TMultipleBWPswitchDelay +k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TMultipleBWPswitchDelay defined in 8.6.2A.1.

All of the above test requirements shall be fulfilled in order for the observed Cell1 and Cell2 active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of SCell (Cell 3) interruption during PCell and SCell (Cell 2) active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell (Cell 3) shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed PCell and SCell (Cell 2) active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+ TMultipleBWPswitchDelay +k1), (j+ TMultipleBWPswitchDelay +k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

#### A.6.5.6.4	SCell dormancy switch

##### A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time

A.6.5.6.4.1.1	Test Purpose and Environment

The purpose of this test is to verify the SCell dormancy switch delay requirements defined in clause 8.6 when the UE is triggered to switch between dormancy to non-dormancy and non-dormancy to dormancy outside the DRX active time. Further the test purpose is to verify the interruption rate on other serving cells when the UE performing CSI and RRM measurements on dormant SCell(s) as defined in clause 8.2.2.2.12 and also to verify the interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

In the test scenario UE is connected to one PCell (Cell 1) in FR1 and one SCell in FR1. In the test the SCell is switched from non-dormancy to dormancy, and vice versa, at a point in time before start of onDuration. The UE is configured to monitor PDCCH for DCI format 2_6 at ps-Offset before the start of onDuration. Two tests are specified, where a UE that only supports triggering within the first three OFDM symbols of a slot shall undergo Test1 only, and a UE that supports triggering also in remaining OFDM symbols of a slot shall undergo both Test1 and Test2. In the tested scenario, ps-Offset is selected to correspond to the dormancy switching time specified in clause 8.6.

The supported test configurations are shown in Table A.6.5.6.4.1.1-1. The general test configuration is given in Table A.6.5.6.4.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.4.1.1-3.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for PCell, BWP-0 in Cell 1 before starting the test.

-	UE is configured with 2 different UE-specific downlink bandwidth parts for SCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in PCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in SCell.

-	UE is indicated in dormantBWP -Id that the active DL BWP is BWP-2 in the SCell.

-	UE is configured with DRX.

-	UE is configured to monitor DCI format 2_6, and to be active during onDuration even when no DCI format 2_6 is detected (ps-WakeUp).

All cells have constant signal levels throughout the test.

The test consists of 4 successive time periods, with durations of T1, T2, T3 and T4, respectively.

During T1,

Time period T1 starts when a DCI format 2_6 command intended for dormant BWP switch in a SCell from non-dormancy to dormancy, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i (at ps-Offset before onDuration). Upon reception of the PDCCH indicating entering dormant BWP in PCell (i.e. through cross-carrier scheduling), UE shall switch the DL BWP-1 to DL BWP-2 in SCell, i.e., switching from non-dormant BWP to dormant BWP and the UE shall complete the switching before the start of onDuration.

The UE shall be able to receive PDCCH on PCell no later than the first DL slot that occurs after the beginning of PCell’s DL slot (i+ TdormantBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK on the PCell no later than the first UL slot that occurs after the beginning of slot (i+N) as defined in clause 10.3 in TS38.213. The UE shall be continuously scheduled on PCell’s BWP-0 no later than the first DL slot that occurs after the beginning of slot (i+ TdormantBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to dormancy switching on SCell shall occur within the dormant BWP switch delay, i.e. before start of onDuration.

The UE shall not transmit signals on SCell after the beginning of PCell’s DL slot (i+ TdormantBWPswitchDelay) as defined in clause 8.6. The UE shall not be scheduled on SCells BWP-1 no later than the first DL slot that occurs after the beginning of slot (i+ TdormantBWPswitchDelay).

Time period T2 starts when T1 is completed. During T2, the test equipment continues to schedule the UE continuously in PCell. The UE shall carry out CSI and RRM measurements on the dormant SCells. The UE shall report ACK/NACK in PCell in response to scheduled PDSCH, with the maximum loss of transmitted ACK/NACKs fulfilling the requirement in clause 8.2.2.2.12. The test equipment verifies that the loss of ACK/NACKs is no larger than 1.5%.

Time period T3 starts when T2 is completed. During T3, the test equipment does not schedule the UE, by which the inactivity timer expires and the UE stops monitoring PDCCH except for signalling using DCI format 2_6 at wake-up signalling occasions.

During T4,

Time period T4 starts when a DCI format 2_6 command for leaving dormant BWP in SCell, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted j (at ps-Offset before onDuration). Upon reception of the PDCCH indicating leaving dormant BWP in PCell (i.e. through cross-carrier scheduling), UE shall switch the DL BWP-2 to DL BWP-1 in SCell, i.e., switching from dormant BWP to non-dormant BWP.

The UE shall be able to receive PDSCH on PCell and SCell no later than the first DL slot that occurs after the beginning of PCell’s DL slot (j+ TdormantBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK on the PCell (for both PCell and SCell) no later than the first UL slot that occurs after the beginning of slot (j+N) as defined in clause 10.3 in TS 38.213. The UE shall be continuously scheduled on PCell’s BWP-0 no later than the first DL slot that occurs after the beginning of slot (j+ TdormantBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to dormancy switching on SCell shall occur within the dormant BWP switch delay.

The UE shall be ready to transmit signals on SCell no later than the first DL slot that occurs after the beginning of PCell’s DL slot (j+ TdormantBWPswitchDelay) as defined in clause 8.6. The UE shall be ready to continuously scheduled on SCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+ TdormantBWPswitchDelay).

The test equipment verifies the DL dormant BWP switch time in SCell by counting the slots from the time when the dormant BWP switch command is received till an ACK/NACK on PCell is received.

The test equipment verifies that potential interruption to PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during dormant BWP switch of SCell (i.e. before start of onDuration), respectively.

Table A.6.5.6.4.1.1-1: SCell dormancy switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD -FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – FDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 5 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD - TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.6.4.1.1-2: General test parameters for SCell dormancy switch in SA

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | 1, 2 |  | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 |  | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 |  | SCell on RF channel number 2. |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | For both PCell and SCell |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 |  | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 |  | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 |  | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| OFDM symbol range in slot for transmission of DCI with dormancy indication |  | 0 – 2 | 3 – 11 |  |
| T1 | s | 0.2 |  |  |
| T2 | s | 0.2 |  |  |

Table A.6.5.6.4.1.1-3: NR Cell specific test parameters for SCell dormancy switch in SA

| Parameter |  |  | Unit | Cell 1 | Cell2 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 | FR1 |
| Duplex mode |  | Config 1 |  | FDD | FDD |
|  |  | Config 2,5 |  | TDD | TDD |
|  |  | Config 3 |  | TDD | FDD |
|  |  | Config 4 |  | FDD | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.1.1 | Not Applicable |
|  |  | Config 4 |  | Not Applicable | TDDConf.1.1 |
|  |  | Config 5 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | Config 1,2,3,4 |  | 10 MHz: NRB,c = 52 | 10 MHz: NRB,c = 52 |
|  |  | Config 5 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 0 | 1 |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |  |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 |  |
| Active DL BWP-0 Configuration |  |  |  | DLBWP.0.2Note4 | N.A. |
| Active DL BWP-1 Configuration |  |  |  | N.A. | DLBWP.1.1Note4 |
| Active DL BWP-2 Configuration |  |  |  | N.A. | DLBWP.1.3Note4 |
| Active UL BWP-0 Configuration |  |  |  | ULBWP.0.2Note4 | N.A. |
| Active UL BWP-1 Configuration |  |  |  | N.A. | ULBWP.1.1Note4 |
| Active UL BWP-2 Configuration |  |  |  | N.A. | ULBWP.1.3Note4 |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  | SR.1.1 TDD | SR.1.1 FDD |
|  |  | Config 4 |  | SR.1.1 FDD | SR.1.1 TDD |
|  |  | Config 5 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | Config 3 |  | CR.1.1 TDD | CR.1.1 FDD |
|  |  | Config 4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  |  | Config 5 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters, Test 1 |  | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.1.1 TDD | CCR.1.1 FDD |
|  |  | Config 4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  |  | Config 5 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| Dedicated CORESET parameters, Test 2 |  | Config 1 |  | CCR.1.5 FDD | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.5 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.1.5 TDD | CCR.1.1 FDD |
|  |  | Config 4 |  | CCR.1.5 FDD | CCR.1.1 TDD |
|  |  | Config 5 |  | CCR.2.3 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |  |
| SSB Configuration |  | Config 1,2,3,4 |  | SSB.1 FR1 |  |
|  |  | Config 5 |  | SSB.2 FR1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2,3,4 |  | dBm/SCS | -104 | -104 |
|  | Config 5 |  |  | -101 | -101 |
| NocNote 2 |  |  | dBm/15KHz | -104 | -104 |
| SS-RSRP Note 3 | Config 1,2,3,4 |  | dBm/SCS | -87 | -87 |
|  | Config 5 |  |  | -84 | -84 |
| Ês/Iot |  |  | dB | 17 | 17 |
| Ês/Noc |  |  | dB | 17 | 17 |
| IoNote3 |  | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 | -58.96 |
|  |  | Config 5 | dBm/ 38.16MHz | -52.86 | -52.86 |
| Propagation Condition |  |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.6.5.6.4.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+N) (i.e. from the start of onDuration).

During time period T2, the UE shall transmit ACK/NACKs in response to scheduling in PCell and the rate of missed ACK/NACKs shall be no more than 1.5%.

During T4, the UE shall start to send the ACK/NACK for PCell and SCell from the first UL slot that occurs after the beginning of DL slot (j+N) (i.e. from the start of onDuration).

Where, N is the timing that UE provide HARQ-ACK information in response to a detection of a DCI format 2_6 indicating SCell dormancy as specified in [3].

All of the above test requirements shall be fulfilled in order for the observed SCell dormant BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T4, the start time of PCell interruption during SCell dormant BWP switch shall not happen outside the dormant BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for dormant BWP switch in clause 8.6.

NOTE:	During T1, T4 if there are no uplink resources for reporting the ACK/NACK in the first DL slot that occurs after the beginning of DL slot (i+ N), (j+ N), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time

A.6.5.6.4.2.1		Test Purpose and Environment

The purpose of this test is to verify fulfillment of SCell dormancy switching delay requirements in clause 8.6.2A, requirements on interruptions due to SCell dormancy switching in clause 8.2.2.2.12.1, and requirements on interruptions due to CSI and RRM measurements on dormant SCells in clauses 8.2.2.2.12.2 and 8.2.2.2.12.3, respectively. In the tested scenario, the UE is connected to PCell and two SCells in FR1, and the SCells are switched from non-dormancy to dormancy, and back, during active time. Depending on UE capability on whether DCI for dormancy switching can be received also later than within the initial three OFDM symbols of a slot, the UE may have to undergo one or two sets of tests. A UE that only supports triggering during within the first three OFDM symbols of a slot shall only undergo Test1 and Test2, whereas a UE that supports triggering also in remaining OFDM symbols of a slot shall undergo Test1 through Test4.

The supported test configurations are provided in Table A.6.5.6.4.2.1-1 below. General test parameters are provided in Table A.6.5.6.4.2.1-2, and cell-specific parameters are provided in Table A.6.5.6.4.2.1-3 below.

The tests consist of three consecutive time periods T1, T2, and T3, respectively.

Three carriers are used in the test, each within FR1 and each with one cell. Cell 1 (PCell) is on RF channel 1 (PCC), Cell 2 (SCell1) is on RF channel 2 (SCC1), and Cell 3 (SCell2) is on RF channel 3 (SCC2). All three cells have constant signal levels throughout the test. The UE is continuously scheduled in PCell throughout the test.

Before the test starts,

UE is connected to Cell 1 (PCell), Cell 2 (SCell1) and Cell 3 (SCell2).

UE is configured with a single UE-specific downlink bandwidth part, BWP-0, for Cell 1. BWP-0 includes the bandwidth of the initial DL BWP and SSB.

UE is configured with one non-dormant and one dormant UE-specific downlink bandwidth part, BWP-0 and BWP-1, respectively, for Cell 2 and Cell 3. BWP-0 includes the bandwidth of the initial DL BWP and SSB.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP in Cell 1 is BWP-0.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP in Cell 2 is BWP-0.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP in Cell 3 is BWP-0.

UE is continuously scheduled in PCell, SCell1 and SCell2.

T1 starts at the point in time at which the UE receives a DCI with dormancy indication on PDCCH in PCell at the antenna connector, in a slot # denoted m, pertaining to dormancy indication for switching SCell1 and SCell2 from non-dormancy to dormancy. The UE shall complete switching of the SCells to dormancy by the end of slot m + ceil(TMultipleBWPswitchDelay/NR slot length) + 1 in Test1 and Test2, and slot m + ceil(TMultipleBWPswitchDelay/NR slot length) + 2 in Test3 and Test4, as specified in clause 8.6.2A. Any PCell interruptions due to the switching between non-dormant and dormant BWPs shall fulfill requirements in clause 8.2.2.2.12.1. The test equipment verifies that interruptions due to switching from non-dormancy to dormancy are within the requirements by analysing HARQ feedback transmitted in PCell for PCell.

During T2, the UE is carrying out CSI and RRM measurements on dormant SCell1 and SCell2. Any PCell interruptions due to CSI and RRM measurements shall fulfill requirements in clauses 8.2.2.2.12.2 and 8.2.2.2.12.3, respectively. The test equipment verifies that the interruptions are within the allowed percentages by counting ACK/NACKs in PCell. At the end of T2, the test equipment transmits a DCI with dormancy indication on PDCCH in PCell carrying a dormany indication for switching SCell1 and SCell2 from dormancy to non-dormancy.

T3 starts at the point in time at which the UE receives a DCI with dormancy indication on PDCCH in PCell at the antenna connector, in a slot # denoted n, pertaining to dormancy indication for switching SCell1 and SCell2 from dormancy to non-dormancy. The UE shall complete switching of the SCells to non-dormancy by the end of slot n + ceil(TMultipleBWPswitchDelay/NR slot length) + 1 in Test1 and Test2, and slot n + ceil(TMultipleBWPswitchDelay/NR slot length) + 2 in Test3 and test4, as specified in clause 8.6.2A. Any PCell interruptions due to the switching between dormant and non-dormant BWPs shall fulfill requirements in clause 8.2.2.2.12.1. The test equipment verifies that interruptions due to switching from dormancy to non-dormancy are within the requirements by analysing HARQ feedback transmitted in PCell for PCell. The test equipment verifies the switching delay by analysing HARQ feedback transmitted in PCell for SCells.

Table A.6.5.6.4.2.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Note: The UE is only required to be tested in one of the supported test configurations

Table A.6.5.6.4.2.1-2: General test parameters

| Parameter | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 | Test3 | Test4 |  |
| NR RF Channel Number |  | 1, 2, 3 |  |  |  | Three NR radio channels are used for this test |
| Active PCell |  | Cell 1 |  |  |  | Primary cell on NR RF channel number 1 in FR1 |
| SCell1 |  | Cell 2 |  |  |  | SCell1 on NR RF channel number 2 in FR1 |
| SCell2 |  | Cell 3 |  |  |  | SCell2 on NR RF channel number 3 in FR1 |
| CP length |  | Normal |  |  |  |  |
| DRX |  | OFF |  |  |  | Continuous monitoring of primary cell |
| CSI reporting periodicity, Non-dormant BWP | ms | 2 |  |  |  | CSI reporting periodicity for periodic reporting of CQI for PCell and non-dormant SCells |
| CSI reporting periodicity, Dormant BWP | ms | 40 |  |  |  | CSI reporting periodicity for periodic reporting of CQI for dormant SCells |
| Timing offset between Cell 1 and Cell 2 | ns | 0 |  |  |  |  |
| Timing offset between Cell 1 and Cell 3 | ns | 0 |  |  |  |  |
| Triggering DCI format |  | 1_1 | 0_1 | 1_1 | 0_1 | Triggering DCI format for triggering during active time |
| OFDM symbol range in slot for transmission of DCI with dormancy indication |  | 0 – 2 |  | 3 – 11 |  | Test1 and Test3 are based on that triggering DCI is received within the first three OFDM symbols of a slot.Test2 and Test4 are based on that the triggering DCI is received later than within the first three OFDM symbols of a slot. |
| T1 | s | 0.2 |  |  |  |  |
| T2 | s | 5 |  |  |  |  |
| T3 | s | 0.2 |  |  |  |  |

Table A.6.5.6.4.2.1-3: NR Cell specific test parameters

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range |  |  |  | FR1 |  |  |
| NR RF channel |  |  |  | 1 | 2 | 3 |
| Duplex mode |  | Config 1 |  | FDD |  |  |
|  |  | Config 2,3 |  | TDD |  |  |
| TDD configuration |  | Config 2 |  | TDDConf.1.1 |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 | --- | --- |
| Downlink active non-dormant BWP-0 configuration |  |  |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| Downlink active dormant BWP-1 configuration |  |  |  | --- | DLBWP.1.1 | DLBWP.1.1 |
| Uplink active BWP-0 configuration |  |  |  | ULBWP.1.1 | --- | --- |
| TCI state |  |  |  | TCI.State.0 | TCI.State.0 | TCI.State.0 |
| CSI-RS configuration for CSI reporting, Non-dormant BWP |  | Config 1 |  | CSI-RS.1.1 FDD | CSI-RS.1.1 FDD | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD | CSI-RS.1.1 TDD | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD | CSI-RS.2.1 TDD | CSI-RS.2.1 TDD |
| CSI-RS configuration for CSI reporting, Dormant BWP |  | Config 1 |  | --- | CSI-RS.1.6 FDD | CSI-RS.1.6 FDD |
|  |  | Config 2 |  |  | CSI-RS.1.5 TDD | CSI-RS.1.5 TDD |
|  |  | Config 3 |  |  | CSI-RS.2.6 TDD | CSI-RS.2.6 TDD |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| Dedicated CORESET parameters, Test 1,2 |  | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| Dedicated CORESET parameters, Test 3,4 |  | Config 1 |  | CCR.1.5 FDD | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.5 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.2.3 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD | --- | --- |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |
| OCNG Pattern |  |  |  | OP.1 | OP.1 | OP.1 |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| SMTC configuration |  |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 |  | dBm/15kHz | -104 | -104 | -104 |
|  | Config 3 |  |  | -101 | -101 | -101 |
| Ês/Iot |  |  | dB | 17 | 17 | 17 |
| Ês/Noc |  |  | dB | 17 | 17 | 17 |
| SS-RSRP Note3 | Config 1,2 |  | dBm/SCS | -87 | -87 | -87 |
|  | Config 3 |  |  | -84 | -84 | -84 |
| Io Note3 | Config 1,2 |  | dBm/9.36 MHz | -59.0 | -59.0 | -59.0 |
|  | Config 3 |  | dBm/38.16 MHz | -52.9 | -52.9 | -52.9 |
| Propagation condition |  |  |  | AWGN | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| Note 1: OCNG shall be used such that the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.5.6.4.2.2	Test Requirements

During T1, any interruption on PCell due to dormancy switching of SCells shall be within the requirement specified in clause 8.2.2.2.12.1.

During T2, interruptions on PCell due to CSI and RRM measurements on dormant SCells shall be within the interruption rate requirements specified in clauses 8.2.2.2.12.2 and 8.2.2.2.12.3, respectively.

During T3, any interruption on PCell due to dormancy switching of SCells shall be within the requirement specified in clause 8.2.2.2.12.1. Monitoring of PDCCH for SCell in SCell shall be resumed within the dormancy switching time specified in clause 8.6.2A.

For an event to be considered to be correct, all requirements above have to be fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs

##### A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA

A.6.5.6.5.1.1	Test Purpose and Environment

The purpose of this test is to verify requirements on the RRC-based DL BWP switch delay on multiple CCs defined in clause 8.6.

The supported test configurations are shown in Table A.6.5.6.5.1.1-1 below. The test scenario comprises of one NR PCell (Cell 1) and one NR SCell (Cell 2) as given in Table A.6.5.6.5.1.1-2. NR Cell-specific parameters are specified in Table A.6.5.6.5.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) and SCell (Cell 2) to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 2 (SCell) on radio channel 2 (SCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for PCell and SCell (Cell 2).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PCell and SCell (Cell 2).

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated bandwidth part configuration for both PCell and SCell (Cell 2), sent from the test equipment to the UE, is completely received at the UE side in PCell’s and SCell’s slot # denoted i. The UE shall reconfigure its bandwidth part with the updated bandwidth part BWP-1 of final condition for both PCell and SCell (Cell 2).

The UE shall be able to receive PDSCH on PCell and SCell (Cell 2) from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length as defined in clause 8.6.3A.1 and starts to report valid ACK/NACK for the PCell and SCell (Cell 2) from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length+k1 on BWP-1 of final condition. The UE shall be continuously scheduled on PCell’s and SCell (Cell 2)’s BWP-1 of final condition starting from the first DL slot right after slot i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length.

TRRCprocessingDelay, TBWPswitchDelayRRC and DRRC are defined in clause 8.6.3A.1, N=2 in this test case.

The test equipment verifies the DL BWP switch time in PCell and SCell by counting the slots from the time when the RRC Reconfiguration message including updated BWP configuration is sent till the time when a vaild ACK/NACK is received.

Table A.6.5.6.5.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD – FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 3 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – FDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 5 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.6.5.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell (Cell 2) |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cell on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cell on SCC. |
| Cell 2 timing offset to Cell 1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | [0.2] |  |

Table A.6.5.6.5.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |  |
| Duplex mode |  | Config 1 |  | FDD | FDD |
|  |  | Config 2,5 |  | TDD | TDD |
|  |  | Config 3 |  | TDD | FDD |
|  |  | Config 4 |  | FDD | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.1.1 | Not Applicable |
|  |  | Config 4 |  | Not Applicable | TDDConf.1.1 |
|  |  | Config 5 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | Config 1,2,3,4 |  | 10 MHz: NRB,c = 52 | 10 MHz: NRB,c = 52 |
|  |  | Config 5 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 1 |  |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |  |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 |  |
| Initial Condition | Active DL BWP-1 Configuration |  |  | DLBWP.1.3 Note4 |  |
|  | Active UL BWP-1 Configuration |  |  | ULBWP.1.3 Note4 |  |
| Final Condition | Active DL BWP-1 Configuration |  |  | DLBWP.1.1 Note4 |  |
|  | Active UL BWP-1 Configuration |  |  | ULBWP.1.1 Note4 |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | Config 3 |  | SR.1.1 TDD | SR.1.1 FDD |
|  |  | Config 4 |  | SR.1.1 FDD | SR.1.1 TDD |
|  |  | Config 5 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | Config 3 |  | CR.1.1 TDD | CR.1.1 FDD |
|  |  | Config 4 |  | CR.1.1 FDD | CR.1.1 TDD |
|  |  | Config 5 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.1.1 TDD | CCR.1.1 FDD |
|  |  | Config 4 |  | CCR.1.1 FDD | CCR.1.1 TDD |
|  |  | Config 5 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |  |
| SSB Configuration |  | Config 1,2,3,4 |  | SSB.1 FR1 |  |
|  |  | Config 5 |  | SSB.2 FR1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
|  |  | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.1 TDD | TRS.1.1 FDD |
|  |  | Config 4 |  | TRS.1.1 FDD | TRS.1.1 TDD |
|  |  | Config 5 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |
| Propagation Condition |  |  |  | AWGN |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2,3,4 |  | dBm/SCS | -104 |  |
|  | Config 5 |  |  | -101 |  |
| SS-RSRP Note 3 | Config 1,2,3,4 |  | dBm/SCS | -87 |  |
|  | Config 5 |  |  | -84 |  |
| Ês/Iot |  |  | dB | 17 |  |
| Ês/Noc |  |  | dB | 17 |  |
| IoNote3 |  | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 |  |
|  |  | Config 5 | dBm/ 38.16MHz | -52.86 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.6.5.6.5.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for both PCell and SCell (Cell 2) from the first DL slot that occurs right after the begining of slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length) and starts to report valid ACK/NACK for both PCell and SCell (Cell 2) from the first UL slot that occurs after the beginning of DL slot (i+ TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch on PCell and SCell (Cell 2) within the time duration TRRCprocessingDelay+TBWPswitchDelayRRC+DRRC defined in 8.6.3A.1.

All of the above test requirements shall be fulfilled in order for the observed PCell and SCell (Cell 2) active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+ TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length +k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

### A.6.5.7	DL interruptions at switching between two uplink carriers

#### A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA

##### A.6.5.7.1.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.2.2.10. The test case is applicable for an uplink band pair of an inter-band FDD-TDD CA configuration when the capability uplinkTxSwitchingPeriod is present.

There are two cells: FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2). The test parameters for the two cells are given in Table A.6.5.7.1.1-1, Table A.6.5.7.1.1-2 and Table A.6.5.7.1.1-3 below.

For NR FDD carrier (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the slot overlapping with the special slot of the NR TDD carrier (Cell 2):

- symbol#12 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

- otherwise,

  - symbol #8 if UE capability uplinkTxSwitchingPeriod is 210us or

  - symbol #9 if UE capability uplinkTxSwitchingPeriod is 140us or

  - symbol #10 if UE capability uplinkTxSwitchingPeriod is 35us.

For NR TDD carrier (Cell 2), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol in the special slot:

- symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

- otherwise,

  - symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

  - symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

  - symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7.1.1-2: General test parameters for DL interruptions at switching between two uplink carriers in FDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.1.5 FDD Cell 2: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7.1.1-3: Cell specific test parameters for DL interruptions at switching between two uplink carriers in FDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | TDD |
| TDD configuration | Config 1 |  | N/A | TDDConf.2.1 except that: S=’11DL:1GP:2UL’; nrofDownlinkSymbols:11 nrofUplinkSymbols: 2 |
| BWchannel | Config 1 |  | 10 MHz: NRB,c = 52 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0resourceMappingnrofSymbols: n2 | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.1 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -87 | -84 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - |
|  |  | dBm/ 38.16MHz | - | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 0 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |

##### A.6.5.7.1.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.1.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA

##### A.6.5.7.2.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching between two uplink carriers defined in clause 8.2.2.2.10. The test case is applicable for an uplink band pair of an inter-band TDD-TDD CA configuration when the capability uplinkTxSwitchingPeriod is present.

There are two cells: FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2). The test parameters for the two cells are given in Table A.6.5.7.2.1-1, Table A.6.5.7.2.1-2 and Table A.6.5.7.2.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the special slot:

- symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

- otherwise,

  - symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

  - symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

  - symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

For NR TDD SCell (Cell 2), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol on the 2nd special slot of every 8 slots:

- symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption-r16;

- otherwise,

  - symbol #4 if UE capability uplinkTxSwitchingPeriod is 210us or

  - symbol #5 if UE capability uplinkTxSwitchingPeriod is 140us or

  - symbol #8 if UE capability uplinkTxSwitchingPeriod is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, uplinkTxSwitching is indicated to UE.

Table A.6.5.7.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.6.5.7.2.1-2: General test parameters for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2 | Two radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7.2.1-3: Cell specific test parameters for DL interruptions at switching between two uplink carriers in TDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 |
| Duplex mode | Config 1 |  | TDD | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that S=’11DL:1GP:2UL’; nrofDownlinkSymbols:11 nrofUplinkSymbols: 2 | TDDConf.2.2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRS configuration in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -84 | -84 |
| Ês/Iot |  | dB | 17 | 17 |
| Ês/Noc |  | dB | 17 | 17 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - |
|  |  | dBm/ 38.16MHz | - | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - | 0 |
| Propagation Condition |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |

##### A.6.5.7.2.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.1.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.8	UE specific CBW change

#### A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX

##### A.6.5.8.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE specific CBW change delay requirement defined in clause 8.13.

The supported test configurations are shown in Table A.6.5.8.1.1-1. The test scenario comprises of one Cell (Cell 1), which is PCell as given in Table A.6.5.8.1.1-2. Cell-specific parameters are specified in Table A.6.5.8.1.1-3.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE sends ACK/NACK during the test.

Before the test starts:

- UE is connected to Cell 1 (PCell) on radio channel 1.

- UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1 (PCell).

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PCell.

- UE has been configured with UE specific CBW (CBW-1).

- UE is indicated in SCS-SpecificCarrier [2] that the UE specific CBW is CBW-1 as the initial condition in Cell 1 (PCell).

Cell1 (PCell) has constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration containing SCS-SpecificCarrier with updated UE specific CBW, sent from the test equipment to the UE, is completely received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its UE specific CBW with the updated CBW-2 for the final condition.

The UE shall be able to receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length as defined in clause 8.13 and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1 on the PCell’s BWP-1 on CBW-2 for the final condition. The UE shall be continuously scheduled on the PCell’s BWP-1 on CBW-2  for the final condition starting from the first DL slot right after slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length.

TRRCprocessingDelay and TCBWchangeDelayRRC are defined in clause 8.13.

The test equipment verifies the UE specific CBW switching delay in PCell by estimating the time from the moment the RRC Reconfiguration message including updated UE specific CBW configuration is sent until the moment a vaild ACK/NACK is received.

Table A.6.5.8.1.1-1: Supported test configurations for UE specific CBW change in SA scenario

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.8.1.1-2: General test parameters for UE specific CBW change in SA scenario

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

Table A.6.5.8.1.1-3: NR Cell specific test parameters for UE specific CBW change in SA scenario

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  |  | FDD |
|  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  | Config 1 |  |  | Not Applicable |
|  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  | Config 1 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3 |  |  | 40 MHz: NRB,c = 106 |
| Active DL BWP ID |  | Config 1,2, 3 |  |  | 1 |
| Initial DL BWP Configuration (BWP-1) |  | Config 1,2, 3 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  | Config 1,2, 3 |  |  | ULBWP.0.2 |
| Initial Condition | Active DLCBW-1 Configureation | Config 1, 2, 3 |  |  | DLCBW.1.1 |
|  | Active UL CBW-1 Configuration | Config 1, 2, 3 |  |  | ULCBW.1.1 |
| Final Condition | Active DLCBW-1 Configureation | Config 1, 2, 3 |  |  | DLCBW.1.2 |
|  | Active UL CBW-1 Configuration | Config 1, 2, 3 |  |  | ULCBW.1.2 |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD |
|  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  | Config 3 |  |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  |  | CR.1.1 FDD |
|  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  | Config 3 |  |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  |  | CCR.1.1 FDD |
|  |  | Config 2 |  |  | CCR.1.1 TDD |
|  |  | Config 3 |  |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2 |  |  | SSB.1 FR1 |
|  |  | Config 3 |  |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  | Config 1 |  | TRS.1.1 FDD |
|  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  | Config 1,2 |  | dBm/ 9.36MHz | -58.96 |
|  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.1 is linked with ULBWP.0.1; DLBWP.1.1 is linked with ULBWP.1.1; as defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

##### A.6.5.8.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for the PCell from the first DL slot that occurs right after the begining of slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

All of the above test requirements shall be fulfilled in order for the observed UE specific CBW change delay on the PCell to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

## A.6.6	Measurement procedure

### A.6.6.1	Intra-frequency Measurements

#### A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX

##### A.6.6.1.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.1.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table A.6.6.1.1.1-1 and A.6.6.1.1.1-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.1.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.1.1.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.1.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.2	SA event triggered reporting tests without gap under DRX

##### A.6.6.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.2.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.2.2-1, A.6.6.1.2.2-2 and A.6.6.1.2.2-3 below. In the measurement controlinformation, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.2.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

Table A.6.6.1.2.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.2.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX

##### A.6.6.1.3.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

##### A.6.6.1.3.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.3.1-1 and A.6.6.1.3.1-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.6.6.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.3.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.1.3.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.2 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.3.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX

##### A.6.6.1.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

##### A.6.6.1.4.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.4.2-1, A.6.6.1.4.2-2 and A.6.6.1.4.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.4.2-2: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1.. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

Table A.6.6.1.4.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1 with DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.2 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: Table A.6.6.1.4.2-1The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Table A.6.6.1.4.2-1Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Table A.6.6.1.4.2-1SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.1.4.2-4: Void

Table A.6.6.1.4.2-5: Void

##### A.6.6.1.4.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading

##### A.6.6.1.5.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.5.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in Table A.6.6.1.5.2-1 and A.6.6.1.5.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.1.5.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.5.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

Table A.6.6.1.5.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0,1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.5.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading

##### A.6.6.1.6.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 9.2.6.2 and 9.2.6.3.

##### A.6.6.1.6.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for FDD PCell and neighbour cell are given in Table A.6.6.1.6.2-1 and A.6.6.1.6.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

Table A.6.6.1.6.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |

Table A.6.6.1.6.2-2: General test parameters for SA intra-frequency event triggered reporting with gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1 | 40 |  |
| Measurement gap length | ms | 1 | 6 |  |
| Measurement gap offset | ms | 1 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

Table A.6.6.1.6.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting with gap for FDD PCell in FR1 with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| IInitial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.6.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16

##### A.6.6.1.7.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event for UE configured with highSpeedMeasFlag-r16. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2.

##### A.6.6.1.7.2	Test parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table A.6.6.1.7.2-1, A.6.6.1.7.2-2 and A.6.6.1.7.2-3 below. In the measurement controlinformation, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.1.7.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.1.7.2-2: General test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasFlag-r16 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.7 | 640ms DRX cycle |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 6 |  |

Table A.6.6.1.7.2-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 with DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity |  | -1.46 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity |  | 4 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity |  | -94 |
|  |  | 2 | -94 | -94 | -Infinity |  | -94 |
|  |  | 3 | -91 | -91 | -Infinity |  | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 |  | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 |  | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -58.50 |  | -56.16 |
| Propagation Condition |  | 1, 2 | AWGN |  |  | AWGN 1944Hz Note 4 |  |
|  |  | 3 | AWGN |  |  | AWGN 3334Hz Note 5 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |

##### A.6.6.1.7.3	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 5120 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.2	Inter-frequency Measurements

#### A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used

##### A.6.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.2.1.1-1, A.6.6.2.1.1-2 and A.6.6.2.1.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.1.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.6.6.2.1.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.6.6.2.1.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1 | 1 |  |

Table A.6.6.2.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

##### A.6.6.2.1.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 760 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used

##### A.6.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.2.2.1-1, A.6.6.2.2.1-2 and A.6.6.2.2.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.2.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.6.6.2.2.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

UE needs to be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.2.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 11 | 1.1 | 11 |  |

Table A.6.6.2.2.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | NA |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | NA |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  | NA |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  |  | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 |  |  | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  |  | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  |  | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  |  | -70.05 | -62.2 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  |  | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

Table A.6.6.2.2.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.6.6.2.2.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.6.6.2.2.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.3	Void

#### A.6.6.2.4	Void

#### A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used

##### A.6.6.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.6.6.2.5.1-1, A.6.6.2.5.1-2 and A.6.6.2.5.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.5.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.6.6.2.5.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.6.6.2.5.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.5.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 1 |  |

Table A.6.6.2.5.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -58.4 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.5.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1040 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 880 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used

##### A.6.6.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.6.6.2.6.1-1, A.6.6.2.6.1-2 and A.6.6.2.6.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.6.6.2.6.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.6.6.2.6.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.6.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.6.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 1.3 | 13.5 | 1.3 | 13.5 |  |

Table A.6.6.2.6.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.2.6.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 12160 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 12160 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.7	Void

#### A.6.6.2.8	Void

#### A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern

##### A.6.6.2.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event when mandatory gap pattern with 3ms MGL is configured.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.2.9.1-1, A.6.6.2.9.1-2 and A.6.6.2.9.1-3.

In test 1 measurement gap pattern configuration # 3 as defined in Table A.6.6.2.9.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #2 as defined in Table A.6.6.2.9.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #2, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.6.6.2.9.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.9.1-2: General test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 3 | 2 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1 | 1 |  |

Table A.6.6.2.9.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.1 |  |  | SMTC.4 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

##### A.6.6.2.9.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used

##### A.6.6.2.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE which supports interFrequencyMeas-Nogap-r16 makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search without measurement gap requirements in clause 9.3.9.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on RF channel 2. The SSB of cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from cell 1 and cell 2 should be different in frequency location within the cell bandwidth. The test parameters are given in Tables A.6.6.2.10.1-1, A.6.6.2.10.1-2 and A.6.6.2.10.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.2.10.1-1: SA event triggered reporting tests when DRX is used for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.10.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 when DRX is used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.2.10.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 when DRX is used

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | NA |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | NA |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  |  | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 |  |  | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  |  | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  |  | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  |  | -70.05 | -62.2 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  |  | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

Table A.6.6.2.10.1-4: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting when DRX is used

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.6.6.2.10.2	Test Requirements

In test config 1, UE is required to report SSB time index. UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test config 2 and 3, UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 900 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used

##### A.6.6.2.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.9.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from cell 1 and cell 2 should be different in frequency location within the cell bandwidth. The test parameters are given in Tables A.6.6.2.11.1-1, A.6.6.2.11.1-2 and A.6.6.2.11.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.6.6.2.11.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.2.11.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without gap

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table A.6.6.2.11.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without gap

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

##### A.6.6.2.11.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

The UE is not required to read the neighbour cell SSB index in this test.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.3	Inter-RAT Measurements

#### A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1

##### A.6.6.3.1.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2 and 9.4.3.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.3.1.1-1. General test parameters are provided in Table A.6.6.3.1.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.3.1.1-3 and A.6.6.3.1.1-4, respectively.

Table A.6.6.3.1.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.1.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.1.1-2 and A.6.6.3.1.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.3.1.1-3 |  |  |  |

Table A.6.6.3.1.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | --96 |  |
|  |  |  |  | 3, 6 | --93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 116 | 70 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 116 | 70 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | --88 | --104 |
|  |  |  |  | 3, 6 | --85 | --101 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | --88 | --104 |
|  |  |  |  | 3, 6 | --85 | --101 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | --59.94 | --73.04 |
|  |  |  | dBm/38.16 MHz | 3, 6 | --53.84 | --66.93 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.1.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.3.1.2	Test Requirements

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1

##### A.6.6.3.2.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2 and 9.4.3. There are two test cases. In test 1 the UE shall be configured with DRX cycle of 40 ms. In test 2 the UE shall be configured with DRX cycle of 640 ms.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indctated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.6.6.3.2.1-1. General test parameters are provided in Table A.6.6.3.2.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.3.2.1-3 and A.6.6.3.2.1-4, respectively.

Table A.6.6.3.2.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.2.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| NR RF Channel Number |  | 1 |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.2.1-2 and A.6.6.3.2.1-3. |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 |  | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | DRX.1 | DRX.7 | DRX cycle configurations DRX.1 and DRX.7 are defined in Table A.3.3.1-1 and Table A.3.3.7-1 respectively. |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 15 |  |
| Note 1: Values are defined in Table A.6.6.3.2.1-3 |  |  |  |  |

Table A.6.6.3.2.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 16 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16 | 16 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -88 |
|  |  |  |  | 3, 6 | -85 | -85 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -88 |
|  |  |  |  | 3, 6 | -85 | -85 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94 | -59.94 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84 | -53.84 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.2.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | -Infinity |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | -Infinity |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -Infinity |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -Infinity |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NRB,c /50) | -76.22+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.6.6.3.2.2	Test Requirements

In test 1, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

In test 2, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 12.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16

##### A.6.6.3.3.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements for UE configured with highSpeedMeasFlag-r16 in standalone (SA) operation with PCell in FR1 when DRX is used. This test shall partly verify the cell search and measurement requirements in Clauses 9.4.2 and 9.4.3.

In the test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

The UE shall be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore the UE shall be allocated with PUSCH resource at every DRX cycle.

Supported test configurations are shown in table A.6.6.3.3.1-1. General test parameters are provided in Table A.6.6.3.3.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.3.3.1-3 and A.6.6.3.3.1-4, respectively.

Table A.6.6.3.3.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.3.3.1-2: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.3.1-2 and A.6.6.3.3.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX.6 | DRX cycle configurations DRX.6 is defined in Table A.3.3.1-6. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.3.3.1-3 |  |  |  |

Table A.6.6.3.3.1-3: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.3.3.1-4: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NRB,c /50) | -59.16+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN1944 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.6.6.3.3.2	Test Requirements

In the test, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 4.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.4	L1-RSRP measurement for beam reporting

#### A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used

##### A.6.6.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.6.6.4.1.1-1.

Table A.6.6.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.1.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.1.2-1 and Table A.6.6.4.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.1.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used

##### A.6.6.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.6.6.4.2.1-1.

Table A.6.6.4.2.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.2.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.2.2-1 and Table A.6.6.4.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.2.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used

##### A.6.6.4.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in Table A.6.6.4.3.1-1.

Table A.6.6.4.3.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.3.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.3.2-1 and Table A.6.6.4.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 80ms from the beginning of the test, the DCI trigger comes in slot n (0 for Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in Table A.6.6.4.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.4.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.3.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.4.3.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.20.1.1 and relative accuracy requirement in clause 10.1.20.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used

##### A.6.6.4.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in Table A.6.6.4.4.1-1.

Table A.6.6.4.4.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.4.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.4.2-1 and Table A.6.6.4.4.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 80ms from the beginning of the test, the DCI trigger comes in slot n (0 for Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in Table A.6.6.4.4.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.4.4.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.4.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.4.4.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.20.1.1 and relative accuracy requirement in clause 10.1.20.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16

##### A.6.6.4.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement when UE is configured with highSpeedMeasFlag-r16. This test will partly verify the L1-RSRP measurement requirements for UE configured with highSpeedMeasFlag-r16 in clause 9.5.4.1, with the testing configurations for NR cells in Table A.6.6.4.5.1-1.

Table A.6.6.4.5.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.4.5.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.4.5.2-1 and Table A.6.6.4.5.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.5.2-1: General test parameters for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.8 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN 1944 Hz |
|  | 3 |  | AWGN 3334 Hz |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.5.2-2: SSB specific test parameters for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.4.5.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than [1920ms] plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.5

#### A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1

##### A.6.6.5.1.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE makes correct event-triggered reporting of inter-RAT UTRAN FDD measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in Clause 9.4.6.

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN FDD neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

Supported test configurations are shown in table A.6.6.5.1.1-1. General test parameters are provided in Table A.6.6.5.1.1-2 below. Test parameters for Cell 1 and Cell 2, valid for both time duration T1 and T2, are provided in Tables A.6.6.5.1.1-3 and A.6.6.5.1.1-4, respectively.

Table A.6.6.5.1.1-1: Supported test configurations in SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, UTRA FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, UTRA FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, UTRA FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.5.1.1-2: General test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  | 2 | 1 UTRA carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.5.1.1-3 and A.6.6.5.1.1-4. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| Inter-RAT UTRA measurement quantity |  | CPICH Ec/Io | Measurement quantity for Cell 2 |
| b1-ThresholdUTRA-FDD | dB | -16.5 | CPICH Ec/Io threshold for SS-RSRP measurement on cell1 for event B1 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.5.1.1-3 |  |  |  |

Table A.6.6.5.1.1-3: PCell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1 | SR.1.1 FDD |  |
|  |  |  |  | 2 | SR.1.1 TDD |  |
|  |  |  |  | 3 | SR.2.1 TDD |  |
| CORESET reference channel |  |  |  | 1 | CR.1.1 FDD |  |
|  |  |  |  | 2 | CR.1.1 TDD |  |
|  |  |  |  | 3 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2 | SSB.1 FR1 |  |
|  |  |  |  | 3 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1 | TRS.1.1 FDD |  |
|  |  |  |  | 2 | TRS.1.1 TDD |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2 | -106 |  |
|  |  |  |  | 3 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3 | ETDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.5.1.1-4: UTRAN neighbour cell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |  |

##### A.6.6.5.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 2.4s from the start of period T2, i.e. when Cell 2 becomes detectable. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.6	CLI measurements

#### A.6.6.6.1	SRS-RSRP measurement with DRX

##### A.6.6.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SRS-RSRP measurement. This test will verify the SRS-RSRP measurement requirements in clause 9.7.2.5 with the testing configurations for NR cells in Table A.6.6.6.1.1-1.

Table A.6.6.6.1.1-1: Applicable NR configurations for FR1 SRS-RSRP test

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.6.1.2	Test Parameters

One cell is deployed in the test, which is FR1 PCell (Cell 1). The test parameters for PCell is given in Table A.6.6.6.1.2-1 and A.6.6.6.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system transmits SRS resource for measurement in the DL slot according to the SRS configuration in Table A.6.6.6.1.2-4 and the test parameters for the (virtual) neighbour cell UE in Table A. 6.6.6.1.2-3. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

Table A.6.6.6.1.2-1: General test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.6.6.6.1.2-3 |
|  |  | 2 | SRSConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -97 |  |
|  |  | 2 | -95 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between DL from serving cell and SRS from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.6.6.6.1.2-2: NR Cell specific test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |
|  |  | 2 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |
|  |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |
|  |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table A.6.6.6.1.2-3: NR Cell specific test parameters for SRS-RSRP event triggered reporting for neighbour cell UE

| Parameter | Unit | Test configuration | Neighbour cell UE |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
|  | dB | 1 | -infinity | 4 |
|  |  | 2 |  |  |
|  | dB | 1 | -infinity | 4 |
|  |  | 2 |  |  |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | -infinity | -94 |
|  |  | 2 | -infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -70.05 | -64.59 |
|  | dBm/38.16 MHz | 2 | -63.96 | -58.50 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.6.6.6.1.2-4: SRS configuration for measurement reporting

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 12 | 12 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset | sl640, 4 | sl640, 9 |  |
|  | sequenceId | 0 | 0 | Any 10 bit number |

##### A.6.6.6.1.3	Test Requirements

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 1920 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.6.2	CLI-RSSI measurement with DRX

##### A.6.6.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of CLI-RSSI measurement. This test will verify the CLI-RSSI measurement requirements in clause 9.7.3.5 with the testing configurations for NR cells in Table A.6.6.6.2.1-1.

Table A.6.6.6.2.1-1: Applicable NR configurations for FR1 CLI-RSSI test

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.6.2.2	Test Parameters

One cell is deployed in the test, which are FR1 PCell (Cell 1). The test parameters for PCell is given in Table A.6.6.6.2.2-1 and A.6.6.6.2.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for CLI-RSSI measurement resource and on 1 data symbol before. The CLI-RSSI measurement resource configuration is in Table A.6.6.6.2.2-3.

Table A.6.6.6.2.2-1: General test parameters for CLI-RSSI event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | NR Cell 1 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| CLI-RSSI configuration |  | 1 | CLI-RSSIConf.1 | Table A.6.6.6.2.2-3 |
|  |  | 2 | CLI-RSSIConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -93 |  |
|  |  | 2 | -93 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between DL from serving cell and OCNG from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 2 |  |

Table A.6.6.6.2.2-2: NR Cell specific test parameters for CLI-RSSI event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |
|  |  | 2 | SR.2.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
|  |  | 2 |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |
|  |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |
|  |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns Note 3 |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| on CLI-RSSI measurement resource Note 2 | dBm/15 kHz | 1 | -116 | -108 |
|  |  | 2 |  |  |
| on CLI-RSSI measurement resource Note 2 | dBm/SCS | 1 | -116 | -108 |
|  |  | 2 | -113 | -105 |
| Io on CLI-RSSI measurement resource | dBm/9.36 MHz | 1 | -88.05 | -80.05 |
|  | dBm/38.16 MHz | 2 | -81.96 | -74.00 |
| Io on CLI-RSSI measurement resource | dBm/1.08 MHz | 1 | -97.43 | -89.43 |
|  | dBm/1.08 MHz | 2 | -97.44 | -89.44 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |  |

Table A.6.6.6.2.2-3: CLI-RSSI measurement resource configuration for measurement reporting

|  | Field | CLI-RSSIConf.1 | CLI-RSSIConf.2 |
| --- | --- | --- | --- |
| RSSI-Resource | rssi-ResourceId | 0 | 0 |
|  | rssi-SCS | 15 | 30 |
|  | startPRB | 0 | 0 |
|  | nrofPRBs | 52 | 106 |
|  | startPosition | 3 | 3 |
|  | nrofSymbols | 11 | 11 |
|  | rssi-PeriodicityAndOffset | sl640, 4 | sl640, 9 |

##### A.6.6.6.2.3	Test Requirements

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 640 ms from the beginning of time period T2. The nominal RSSI used to evaluate the requirement shall be based on Io.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.7	NR measurements with autonomous gaps

#### A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1

##### A.6.6.7.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of intra-frequency CGI identification of an NR neighbour cell in FR1 with autonomous gaps. This test shall partly verify the measurement requirements in Clause 9.11.

##### A.6.6.7.1.2	Test Parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the FR1 PCell and Cell 2 is an FR1 neighbour cell on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table A.6.6.7.1.1-2 and A.6.6.7.1.3-2 below. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable.  A measurement object is configured for the frequency of the PCell and it is indicated to the UE that event-triggered reporting with Event A3 is used. The UE is expected to detect and send a measurement report with Event A3.

A new RRC message triggering CGI identification shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying CGI identification is sent to the UE.

The test equipment verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in PCell during T3 untill a measurement report with CGI is sent.

Table A.6.6.71.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.7.1.2-2: General test parameters for SA intra-frequency CGI identification of NR neighbor cell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

Table A.6.6.7.1.2-3: NR Cell specific test parameters for SA intra-frequency CGI identification of NR neighbor cell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.7.1.3	Test Requirements

The UE shall send a measurement report containing the CGI of cell 2 within 252 ms from the start of time period T3.

Test requirement 	= RRC Procedure delay + Tidentify_CGI + reporting delay

= 10 + 240 + 2ms from the start of T3

= 252 ms

The UE shall be scheduled continuously throughout the test. From the start of T3 until 252 ms, the interruption on PCell shall not be more than the values specified for SA in clause 8.2.2.2.14.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

##### A.6.6.7.2.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in NR SA in clause 9.4.7.

The test scenario comprises of one NR carrier and an E-UTRA carrier and two cells as given in tables A.6.6.7.2.1-1, A.6.6.7.2.1-2, A.6.6.7.2.1-3 and A.6.6.7.2.1-4. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event B2. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.6.6.7.2.1-1: Supported test configurations of inter-RAT E-UTRAN cell using autonomous gap in SA

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.7.2.1-2: General test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| LTE Channel Bandwidth | MHz | 10 |  |
| LTE PDSCH/PCFICH/PDCCH/PHICH parameters |  |  | As specified in clause A.3.7.2.1 |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| useAutonomousGaps |  | TRUE | As specified in clause 5.5.3.1 in TS 38.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.6.6.7.2.1-3: PCell specific test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR

| Parameter |  |  | Unit | Configuration | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |  | T3 |  |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |  |  |  |
|  |  |  |  | 4, 5, 6 | TDD |  |  |  |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |  |  |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |  |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |  |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |  |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |  |  |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |  |  |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |  |  |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |  |  |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |  |  |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |  |  |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |  |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |  |  |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |  |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |  |  |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |  |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |  |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |  |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |  |  |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |  |  |  |
|  |  |  |  | 3, 6 | -95 |  |  |  |  |
| b2-Threshold2EUTRA |  |  | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -109 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |  |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |  |  |  |
|  |  |  |  | 3, 6 | -103 |  |  |  |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  | -108 |
|  |  |  |  | 3, 6 | -85 |  | -105 |  | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  | -108 |
|  |  |  |  | 3, 6 | -85 |  | -105 |  | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 |  | -75.92 |  | -75.92 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 |  | -69.82 |  | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |  |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

Table A.6.6.7.2.1-4: Cell specific test parameters for inter-RAT E-UTRAN cell for identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 10 MHz: NRB,c = 50 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.3 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 10 MHz: OP.10 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.6.6.7.2.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 200 milliseconds from the start of T3.

Test requirement = RRC Procedure delay with additional margin + Tidentify_CGI,E-UTRAN + reporting delay

= 15 + 30 + 150 + 2ms from the start of T3

= 197 ms, allow 200 ms.

-	The UE shall be scheduled continuously throughout the test, and from the start of T3 until 200 ms at least the number of ACK/NACK specified in NOTE 2 shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The overall ACK/NACK number is caused by two parts. Firstly, at least X ACK/NACK shall be sent during identifying the cell global identifier of cell 2, where X is defined in Table 8.2.2.2.15-1. Secondly, given that continuous DL data allocation, additional 20/40 ACK/NACK shall be sent from the start of T3 until 200 ms excludes 150 ms for identifying the cell global identifier of cell 2.

### A.6.6.8	L1-SINR measurement for beam reporting

#### A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used

##### A.6.6.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements in clause 9.8.4.1, with the testing configurations for NR cells in Table A.6.6.8.1.1-1.

Table A.6.6.8.1.1-1: Applicable NR configurations for FR1 CSI-RS based L1-SINR test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.8.1.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.8.1.2-1 and Table A.6.6.8.1.2-2 below.

In the CSI-RS measurement configuration, UE is indicated to perform L1-SINR measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. After 80ms from the beginning of the test, the DCI trigger comes in slot n (1 Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in Table A.6.6.8.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.6.6.8.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.2 FDD |
|  | 2 |  | CSI-RS 1.2 TDD |
|  | 3 |  | CSI-RS 2.2 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.8.1.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.8.1.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.27.1.1 and relative accuracy requirement in clause 10.1.27.1.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

##### A.6.6.8.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements in clause 9.8.4.2, with the testing configurations for NR cells in Table A.6.6.8.2.1-1.

Table A.6.6.8.2.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.8.2.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.8.2.2-1 and Table A.6.6.8.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the SSBs and the associated CSI-RS resources, and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD measurements based on the SSBs, and UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-RS resources as IMR.

Table A.6.6.8.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | c |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.1A FDD |
|  | 2 |  | CSI-RS 1.1A TDD |
|  | 3 |  | CSI-RS 2.1A TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity-r16 | 1~3 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| L1-SINR reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.8.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.8.2.2-3: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 |  | CSI-RS#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/CSI-RS SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/CSI-RS SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.8.2.3	Test Requirements

The UE shall send L1-SINR report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-SINR report including results of both SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1 while meeting the accuracy requirement in clause 10.1.27.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used

##### A.6.6.8.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements with CSI-RS based CMR and dedicated IMR cofigured in clause 9.8.4.3, with the testing configurations for NR cells in Table A.6.6.8.3.1-1.

Table A.6.6.8.3.1-1: Applicable NR configurations for FR1 L1-SINR test with CMR and dedicated IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.6.6.8.3.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.6.8.3.2-1 and Table A.6.6.8.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the configured CSI-RS as CMR and an associated CSI-IM as IMR, and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. UE is also configured to measure L1-SINR based on SSB. After 80ms from the beginning of the test, the DCI trigger comes in slot n (1 Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in Table A.6.6.8.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs, and UE is configured to perform L1-SINR measurement based on the CSI-RS as CMR and the CSI-IM as IMR.

Table A.6.6.8.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| CSI-IM configuration | 1 |  | CSI-IM.1.2 FDD |
|  | 2 |  | CSI-IM.1.2 TDD |
|  | 3 |  | CSI-IM.2.2 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

A.6.6.8.3.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.6.8.3.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 as CMR + CSI-IM#0 as IMR and CSI-RS#1 as CMR + CSI-IM#1 as IMR while meeting the accuracy requirement in clause 10.1.27.3.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.9	Idle Mode CA/DC Measurements

#### A.6.6.9.1	SA Idle mode CA/DC measurement for FR1

##### A.6.6.9.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE performs the required measurements on the serving cell and the configured inter-frequency carrier for idle mode measurement reporting after the UE has entered Idle mode. This test will partly verify the Idle mode CA/DC measurements requirements in clause 4.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.9.1.1-1, A.6.6.9.1.1-2, A.6.6.9.1.1-3 and A.6.6.9.1.1-4.

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. Druing T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with early measurement reporting with channel 2. Beam level reporting for early measurements is not configured. The connection is released at the end of T1. T2 starts when the connection is released. During the time periods T2 UE is in Idle mode. At T3 the UE is paged for connection setup and requested by the network to send idle mode measurements.

Table A.6.6.9.1.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.1.1-2: General test parameters for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | [0.32] | The value shall be used for all cells in the test. |
| T331 | s |  | 300 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 10 |  |
| T2 | s | Config 1,2,3 | [11.52] |  |
| T3 | s | Config 1,2,3 | 10 |  |

Table A.6.6.9.1.1-3: Cell specific test parameters for connected mode for SA Idle mode CA/DC measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 |  | T3 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 | -91 | -91 | -infinity |  | -98 |  | -98 |
|  |  |  | Config 3 | -88 | -88 | -88 | -infinity |  | -95 |  | -95 |
|  |  | dB | Config 1,2,3,4,5,6 | 7 | 7 | 7 | - infinity |  | 0 |  | 0 |
|  |  | dB | Config 1,2,3 | 7 | 7 | 7 | infinity |  | 0 |  | 0 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -62.26 | -62.26 | -62.26 | -70.5 |  | -67.04 |  | -67.04 |
|  |  | dBm/38.16MHz | Config 3 | -56.15 | -56.15 | -56.15 | -63.94 |  | -60.93 |  | -60.93 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.6.9.1.1-4: Cell specific test parameters for idle mode for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1,2,3 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | [14] | [14] | [14] | -infinity | [12] | [12] |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | [-98] |  |  |  |  |  |
|  |  | 2 | [-98] |  |  |  |  |  |
|  |  | 3 | [-95] |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | [-98] |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | [7] | [7] | [7] | -infinity | [0] | [0] |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | [-91] | [-91] | [-91] | -infinity | [-98] | [-98] |
|  |  | 2 | [-91] | [-91] | [-91] | -infinity | [-98] | [-98] |
|  |  | 3 | [-88] | [-88] | [-88] | -infinity | [-95] | [-95] |
| Io | dBm/9.36 MHz | 1 | [-62.26] | [-62.26] | [-62.26] | [-70.5] | [-67.04] | [-67.04] |
|  | dBm/9.36 MHz | 2 | [-62.26] | [-62.26] | [-62.26] | [-70.5] | [-67.04] | [-67.04] |
|  | dBm/38.16 MHz | 3 | [-56.15] | [-56.15] | [-56.15] | [-63.94] | [-60.93] | [-60.93] |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  |  | Not sent |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

##### A.6.6.9.1.2	Test Requirements

The UE behaviour during time durations T2 and T3 shall be as follows:

During the time period T2 the UE is in Idle mode and the signal level of cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to Section 4.4.

At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for Cell 2 meets the requirements in Section 10.X and test ends.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.10	CSI-RS based intra-frequency Measurements

#### A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX

##### A.6.6.10.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA CSI-RS based L3 intra-frequency requirements in clauses 9.10.2.

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell and neighbour cell are given in Table A.6.6.10.1.1-1 and A.6.6.10.1.1-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.6.6.10.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.6.10.1.1-2: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified and measured. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS configuration for RRM |  | 1 | CSI-RS.RRM.FR1.1 FDD |  |
|  |  | 2 | CSI-RS.RRM.FR1.1 TDD |  |
|  |  | 3 | CSI-RS.RRM.FR1.2 TDD |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells | s | 1 | 4.7 | Asynchronous cells. The timing of Cell 2 is CP later than the timing of Cell 1. |
|  |  | 2 | 4.7 | Synchronous cells |
|  |  | 3 | 2.35 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 1 |  |

Table A.6.6.10.1.1-3: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| CSI-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | --64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | --58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.10.1.2	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.11	CSI-RS based inter-frequency Measurements

#### A.6.6.11.1	 SA event triggered reporting tests with gap under DRX

##### A.6.6.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA CSI-RS based L3 inter-frequency measurement requirements in clause 9.10.3.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.6.6.11.1.1-1, A.6.6.11.1.1-2 and A.6.6.11.1.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.6.6.11.1.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.6.6.11.1.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.6.6.11.1.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.11.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.5 | DRX.5 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells | s | Config 1 | 4.7 |  | Asynchronous cells. The timing of Cell 2 is CP later than the timing of Cell 1. |
|  |  | Config 2 | 4.7 |  | Synchronous cells. |
|  |  | Config 3 | 2.35 |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 10 | 10 |  |

Table A.6.6.11.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| CSI-RS configuration for RRM |  |  | Config 1 | CSI-RS.RRM.FR1.1 FDD |  | CSI-RS.RRM.FR1.1 FDD |  |
|  |  |  | Config 2 | CSI-RS.RRM.FR1.1 TDD |  | CSI-RS.RRM.FR1.1 TDD |  |
|  |  |  | Config 3 | CSI-RS.RRM.FR1.2 TDD |  | CSI-RS.RRM.FR1.2 TDD |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| CSI-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

##### A.6.6.11.1.2	Test Requirements

In test 1 with per-UE gap and test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 9280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.6.6.12	RSTD measurements

#### A. 6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA

##### A. 6.6.12.1.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations are specified in Table A.6.6.12.1.1-1.

Table A.6.6.12.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All 3 cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The general test parameters are listed in Table A.6.6.12.1.1-2, and cell specific test parameters are listed in Table A.6.6.12.1.1-3.

Table A.6.6.12.1.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355[34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and secondplaces in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | [1.28] | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.1.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| SSB |  | dB | 10 | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 2 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 3 | dBm/ 38.16MHz | -63.20 | -63.96 | -63.96 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.1.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -69.59 | -69.93 | -69.93 |
|  | Config 2 | dBm/ 96.48MHz | -69.59 | -69.93 | -69.93 |
|  | Config 3 | dBm/ 38.16MHz | -63.72 | -63.89 | -63.89 |
| PRS |  | dB | -6 | 13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

##### A.6.6.12.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 9.9.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.1.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD1970049

#### A. 6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA

##### A. 6.6.12.2.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 9.9.2 in an environment with AWGN propagation conditions in FR1 in standalone scenario when dual positioning frequency layers are configured.

The supported test configurations are specified in Table A.6.6.12.2.1-1.

Table A.6.6.12.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 3 is on a different RF channel with Cell 1 and Cell 2.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #0 before T2.

The general test parameters are listed in Table A.6.6.12.2.1-2, and cell specific test parameters are listed in Table A.6.6.12.2.1-3.

Table A.6.6.12.2.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355[34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC configuration | Config 1 |  | SMTC.2 |  |
|  | Config 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.1.1 FDD | As specified in clause A.3.1.2.1 |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Initial BWP configuration | Config 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration | Config 1,2,3 |  | DLBWP.1.1 |  |
| Active UL BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |
| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.xx |
|  | Config 2 |  | PRS.1.2 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#0 | GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 500 | The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | [1.28] | The length of the time interval that follows immediately after time interval T1 |

Table A.6.6.12.2.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 2 |
| Positiong frequency layer |  |  | 1 | 1 | 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 | dBm/SCS | -98 |  |  |
|  | Config 3 | dBm/SCS | -95 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 2 | dBm/ 9.36MHz | -68.63 | -70.05 | -70.05 |
|  | Config 3 | dBm/ 38.16MHz | -63.20 | -63.96 | -63.96 |
| SSB RP Note4 | Config 1 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 2 | dBm/SCS | -88 | -Infinity | -Infinity |
|  | Config 3 | dBm/SCS | -88 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.6.6.12.2.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2 and T3

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| NR RF Channel Number |  |  | 1 | 1 | 2 |
| Positiong frequency layer |  |  | 1 | 1 | 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 | FR1 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -98 | -98 | -98 |
|  | Config 2 | dBm/SCS | -98 | -98 | -98 |
|  | Config 3 | dBm/SCS | -95 | -95 | -95 |
| PRS | Config 1 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 2 | dB | -5.45 | -11.67 | -11.67 |
|  | Config 3 | dB | -5.45 | -11.67 | -11.67 |
| Io Note 4 | Config 1 | dBm/ 9.36MHz | -69.59 | -69.93 | -69.93 |
|  | Config 2 | dBm/ 96.48MHz | -69.59 | -69.93 | -69.93 |
|  | Config 3 | dBm/ 38.16MHz | -63.72 | -63.89 | -63.89 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

##### A.6.6.12.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 9.9.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.1.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD_1970049.

### A.6.6.13 PRS-RSRP measurements

#### A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer

##### A.6.6.13.1.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements specified in clause 9.9.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.6.13.1.1-1.

Table A.6.6.13.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cell. Both cells are on the same NR RF channel in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.6.13.1.1-2, and cell specific test parameters are listed in Table A.6.6.13.1.1-3.

Table A.6.6.13.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | [5] |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.13.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -108 |
|  |  | 2 | -Infinity | -101 | -Infinity | -108 |
|  |  | 3 | -Infinity | -98 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -88 | -88 |
|  |  | 2 | -88 | -88 | -88 | -88 |
|  |  | 3 | -85 | -85 | -85 | -85 |
| Io | dBm/9.36 MHz | 1 | N/A | -62.25 | N/A | -62.25 |
|  | dBm/9.36 MHz | 2 |  | -62.25 |  | -62.25 |
|  | dBm/38.16 MHz | 3 |  | -56.16 |  | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.13.1.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.3.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer

##### A.6.6.13.2.1	Test purpose and Environment

The purpose of the test is to verify that the PRS-RSRP measurement meets the delay requirements specified in clause 9.9.3.5 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.6.13.2.1-1.

Table A.6.6.13.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR1. Cell 2 is a neighbour cell on a different NR RF channel #2 in FR1. The test consists of two consecutive time intervals, with duration of T1 and T2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The general test parameters are listed in Table A.6.6.13.2.1-2, and cell specific test parameters are listed in Table A.6.6.13.2.1-3.

Table A.6.6.13.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 | Cell 1 and Cell 2 are on differnet positioning frequency layers |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | [10] |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.13.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 2 | PRS.1.4 FR1 |  | PRS.1.4 FR1 |  |
|  |  | 3 | PRS.2.4 FR1 |  | PRS.2.4 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -108 |
|  |  | 2 | -Infinity | -101 | -Infinity | -108 |
|  |  | 3 | -Infinity | -98 | -Infinity | -105 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -88 | -88 | -88 | -88 |
|  |  | 2 | -88 | -88 | -88 | -88 |
|  |  | 3 | -85 | -85 | -85 | -85 |
| Io | dBm/9.36 MHz | 1 | N/A | -62.25 | N/A | -62.25 |
|  | dBm/9.36 MHz | 2 |  | -62.25 |  | -62.25 |
|  | dBm/38.16 MHz | 3 |  | -56.16 |  | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.13.2.2	Test Requirements

The UE shall perform and report the PRS-RSRP measurements for Cell 1 and Cell 2, within the time limit specified in clause 9.9.3.5, starting from the beginning of time interval T2.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.6.14	UE Rx-Tx time difference measurements

#### A.6.6.14.1 UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA

##### A.6.6.14.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.6.14.1.1-1.

Table A.6.6.14.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.14.1.1-2 and Table A.6.6.14.1.1-3 respectively.

Table A.6.6.14.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/9.36 MHz | 1 | N/A | -67.67 | N/A | -67.67 |
|  | dBm/9.36 MHz | 2 |  | -67.67 |  | -67.67 |
|  | dBm/38.16 MHz | 3 |  | -61.57 |  | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.14.1.1-4: Void

##### A.6.6.14.1.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.14.2 UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA

##### A.6.6.14.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR1 in standalone scenario when dual positioning frequency layers are configured.

The supported test configurations in listed in Table A.6.6.14.2.1-1.

Table A.6.6.14.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Cell 1 and Cell2 are on different RF channels in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #0 or ID #24 before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.6.6.14.2.1-2 and Table A.6.6.14.2.1-3 respectively.

Table A.6.6.14.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1, 2, 3 | 1 | For Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 2 | For Cell 2 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Measurement gap |  | 1, 2, 3 | GP#24 or GP#0 Note 1 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| DRX |  | 1, 2, 3 | OFF |  |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

Table A.6.6.14.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -3 | -Infinity | -13 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 | -Infinity | -111 |
|  |  | 2 | -Infinity | -101 | -Infinity | -111 |
|  |  | 3 | -Infinity | -98 | -Infinity | -108 |
| Io | dBm/9.36 MHz | 1 | N/A | -68.28 | N/A | -69.63 |
|  | dBm/9.36 MHz | 2 |  | -68.28 |  | -69.63 |
|  | dBm/38.16 MHz | 3 |  | -62.19 |  | -63.54 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.6.6.14.2.1-4: Void

##### A.6.6.14.2.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

### A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting

#### A.6.6.15.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly retains the detected cell status for the idle mode CA measurement when UE transitions from RRC Connected mode to Idle mode, when the UE has entered Idle mode. Additionally, test that the UE performs the required measurements on the serving cell and the configured inter-RAT carrier for idle mode measurement reporting. This test will partly verify the Idle mode CA measurements in clause 4.4. In the test, connected mode DRX configuration is not configured in either PCell or PSCell.

Additionally, the purpose of this test is to verify that the SS-RSRP, SS-RSRQ, RSRP and RSRQ measurement accuracy is within the specified limits. This test will verify the accuracy requirements in Sections 10.1.2B and 10.1.7B for intra-frequency measurements and section 10.2.4 and 10.2.5 for the inter-RAT measurements for the supported test configurations in tables A.6.6.15.1-4 and A.6.6.15.1-5.

The supported test configurations are given in Table A.6.6.15.1-1. The test parameters are given in Tables A.6.6.15.1-2, A.6.6.15.1-3, A.6.6.15.1-4 and A.6.6.15.1-5 below. In the test there are two cells, cell 1, which is the PCell in connected, and serving cell in idle mode, on radio channel 1 in FR1, and cell 2, which is the PSCell in connected, and measured LTE inter-RAT cell in idle mode, on radio channel 2 in LTE.

For the purpose of testing absolute accuracy in idle mode in this set of test cases the cells in idle mode are on different carrier frequencies (NR FR1 and LTE).  The absolute accuracy of RSRP and RSRQ inter-RAT measurements are tested by using the parameters in Table A.6.6.15.1-4 and Table A.6.6.15.1-5. In all test cases, Cell 1 is the serving and Cell 2 the target cell.

The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. During T1 cell 2, the PSCell, shall be configured.

Time duration T2 starts when UE has transmitted random access preamble on the PSCell. After T2, the UE is configured with idle mode CA measurements with the PSCell carrier as the target carrier. The connection is released [500]ms after T2 when the UE has sent random access preamble on the PSCell.

T3 starts when the connection is released. During the time periods T3 and T4 the UE is in Idle mode with the serving cell on the FR1 carrier. The UE is configured to perform inter-RAT idle mode CA/DC measurements on Cell 2 carrier. After the connection release and during T3, [1000] ms after T3 is started, the signal level of the inter-RAT carrier configured for idle mode CA/DC measurements is changed at which time T4 starts. T5 starts [65]s after T4, when the UE is paged for connection setup and UE is requested by the network to report idle mode CA/DC measurements.

Table A.6.6.15.1-1: Supported test configurations for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Config | Description |
| --- | --- |
| 1 | FR1 FDD SSB SCS 15kHz BW 10MHz – LTE FDD 10MHz |
| 2 | FR1 FDD SSB SCS 15kHz BW 10MHz – LTE TDD 10MHz |
| 3 | FR1 TDD SSB SCS 30kHz BW 40MHz – LTE FDD 10MHz |
| 4 | FR1 TDD SSB SCS 30kHz BW 40MHz – LTE TDD 10MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.15.1-2: General test parameters for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| PSCell |  | Cell 2 | PSCell on RF channel number 2 in LTE |
| DRX |  | OFF | For both PCell and PSCell once configured |
| PRACH configuration in Cell 2 |  | [PRACH_2CE] | PRACH configuration as specified in Clause A.3.16 in TS 36.133 |
| CSI reporting periodicity and offset configuration for Cell 2 | ms | 2 |  |
| T1 | s | [0.5] | During this time the PCell is known and PSCell is configured. |
| T2 | s | [0.5] | PSCell access. |
| T3 + T4 | s | [66] | During this time the UE is configured to perform inter-frequency measurements in idle mode on the PSCell carrier. |
| T5 | s | [0.5] | UE is paged and connection is setup. Network requests measurement report from the UE. |

Table A.6.6.15.1-3: Cell specific test parameters for NR cell for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | T1 | T2 | T3 |  |  |  | T4 |  |  |  | T5 |
| AoA setup |  | 1,2,3,4 | N/A |  |  |  |  |  |  |  |  |  |  |
| Assumption for UE beams Note 5 R: Rough |  | 1,2,3,4 | N/A | N/A | R |  |  |  | R |  |  |  | N/A |
| Frequency Range |  | 1,2,3,4 | FR1 |  |  |  |  |  |  |  |  |  |  |
| Duplex mode |  | 1, 2 | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | 3, 4 | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD Confguration 1: TDDConf.1.1 2: TDDConf.2.1 |  | 1,2 | - |  |  |  |  |  |  |  |  |  |  |
|  |  | 3,4 | 1 | 1 | 2 |  |  |  | 2 |  |  |  | 1 |
| BWchannel 1: 10: NRB,c = 52 2: 40: NRB,c = 106 | MHz | 1, 2 | 1 | 1 | - |  |  |  | - |  |  |  | 1 |
|  |  | 3, 4 | 2 | 2 | - |  |  |  | - |  |  |  | 2 |
| Initial Downlink BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Initial Uplink BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Dedicated Downlink BWP configuration 1: DLBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - |  |  | - |  |  |  |  | 1 |
| Dedicated Uplink BWP configuration 1: ULBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - |  |  | - |  |  |  |  | 1 |
| PDSCH Reference Measurement Channel 1: SR.1.1 FDD 2: SR.2.1 TDD | FDD | 1,2 | 1 | 1 | 1 |  |  | 1 |  |  |  |  | 1 |
|  | TDD | 3,4 | 2 | 2 | 2 |  |  | 2 |  |  |  |  | 2 |
| TRS configuration |  | 1,2,3,4 | - |  |  |  |  |  |  |  |  |  |  |
| TCI state |  | 1,2,3,4 | - |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET parameters | FDD | 1,2 | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  | TDD | 3,4 | CR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET parameters | FDD | 1,2 | CCR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  | TDD | 3,4 | CCR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG PatternsNote1 |  | 1,2,3,4 | OP.1 defined in A.3.2.1 |  |  |  |  |  |  |  |  |  |  |
| SSB configuration 1: SSB.1 FR1 2: SSB.2 FR1 |  | 1,2 | 1 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3,4 | 2 |  |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | 1,2,3,4 | SMTC.2 |  |  |  |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3,4 | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PBCH to PBCH DMRS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDCCH DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDSCH to PDSCH |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of OCNG DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| Noc Note2 | dBm/ 15kHz | 1,2 | [-98] | [-98] | [-98] |  |  | [-98] |  |  |  |  | [-98] |
|  |  | 3,4 |  |  |  |  |  |  |  |  |  |  |  |
| Noc Note2 | dBm/SCS | 1,2 | [-98] | [-98] | [-98] |  | [ -98] |  |  |  |  | [-98] |  |
|  |  | 3,4 | [-95] | [-95] | [-95] |  | [-95] |  |  |  |  | [-95] |  |
| Ês/Iot | dB | 1,2,3,4 | [5] | [5] | [5] |  | [5] |  |  |  |  | [5] |  |
| Ês/Noc | dB | 1,2,3,4 | [5] | [5] | [5] |  | [5] |  |  |  |  | [5] |  |
| SS-RSRPNote3,4 | dBm/SCS | 1,2 | [-93] | [-93] | [-93] |  | [-93] |  |  |  |  | [-93] |  |
|  |  | 3,4 | [-90] | [-90] | [-90] |  | [-90] |  |  |  |  | [-90] |  |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2 | [-63.85] | [-63.85] | -[-63.85] | -[-63.85] |  |  |  |  | [-63.85] |  |  |
|  | dBm/ 38.16 MHz | 3,4 | [-57.76] | [-57.76] | -[-57.76] | -[-57.76] |  |  |  |  | [-57.76] |  |  |
| Qrxlevmin | dBm/SCS | 1 | - | - | [-140] |  |  |  |  | - |  |  |  |
|  |  | 2 | - | - | [-137] |  |  |  |  | - |  |  |  |
|  |  | 3 | - | - | [TBD] | [TBD] |  |  |  | - |  |  |  |
|  |  | 4 | - | - | [TBD] | [TBD] |  |  |  | - |  |  |  |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 |  |  |  | - |  |  |  |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 |  |  |  | - |  |  |  |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 |  |  |  | - |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | SS-RSRP |  |  |  |  |  |  |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  |  |  |  | - |  |  |  |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | [TBD] |  |  |  |  | - |  |  |  |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | [TBD] |  |  |  |  | - |  |  |  |
| Threshx, high | dB | 1,2,3,4 | - |  | [48] |  |  |  |  | - |  |  |  |
| Threshserving, low | dB | 1,2,3,4 | - |  | [44] |  |  |  |  | - |  |  |  |
| Threshx, low | dB | 1,2,3,4 | - |  | [50] |  |  |  |  | - |  |  |  |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  |  |  |  | - |  |  |  |

Table A.6.6.15.1-4: Cell specific test parameters for LTE cell for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Frequency Range |  | 1,2,3,4 | LTE |  |  |  |  |
| Duplex mode |  | 1, 3 | FDD |  |  |  |  |
|  |  | 2, 4 | TDD |  |  |  |  |
| BWchannel | MHz | 1,2,3,4 | 10 | 10 | - | - | 10 |
| Measurement bandwidth |  | 1,2,3,4 | - | - | 22-27 | 22-27 | - |
| PDSCH Reference Measurement Channel 1: R.1 FDD 2: R.1 TDD | FDD | 1,3 | 1 | 1 | - | - | 1 |
|  | TDD | 2,4 | 2 | 2 | - | - | 2 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 and A.3.1.2.2 in 36.133 1: R.6 FDD 2: R.6 TDD |  | 1,3 | 1 | 1 | - | - | 1 |
|  |  | 2,4 | 2 | 2 | - | - | 2 |
| OCNG Patterns defined in A.3.2.1.1 (OP.2 FDD) and A.3.2.1.2 (OP.2 TDD) in 36.133 1: OP.2 FDD 2: OP.2 TDD |  | 1,3 | 1 |  |  |  |  |
|  |  | 2,4 | 2 |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |
| PBCH_RA | dB | 1,2,3,4 | N/A | N/A | 0 | 0 | N/A |
| PBCH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PSS_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| SSS_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PCFICH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PHICH_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PHICH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PDCCH_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PDCCH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PDSCH_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PDSCH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| OCNG_RANote 1 |  |  | N/A | N/A | 0 | 0 | N/A |
| OCNG_RBNote 1 |  |  | N/A | N/A | 0 | 0 | N/A |
| Noc Note2 | dBm/ 15kHz | 1,2 | [-98] | [-98] | [-98] | [-98] | [-98] |
|  |  | 3,4 |  |  |  |  |  |
| Ês/Iot | dB | 1,2,3,4 | [5] | [5] | [-3] | [8] | [5] |
| Ês/Noc | dB | 1,2,3,4 | [5] | [5] | [-3] | [8] | [5] |
| SS-RSRPNote3,4 | dBm/SCS | 1,2,3,4 | [-93] | [-93] | [-101] | [-90] | [-93] |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2, 3, 4 | FFS | FFS | [FFS] | [FFS] | FFS |
| Qrxlevmin | dBm/SCS | 1 | - | - | [-140] |  | - |
|  |  | 2 | - | - | [-137] |  | - |
|  |  | 3 | - | - | [TBD] | [TBD] | - |
|  |  | 4 | - | - | [TBD] | [TBD] | - |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | RSRP and RSRQ |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  | - |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | [TBD] |  | - |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | [TBD] |  | - |
| Threshx, high | dB | 1,2,3,4 | - |  | [48] |  | - |
| Threshserving, low | dB | 1,2,3,4 | - |  | [44] |  | - |
| Threshx, low | dB | 1,2,3,4 | - |  | [50] |  | - |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  | - |

Table A.6.6.15.1-5: General idle mode test parameters for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Serving cell |  | 1, 2, 3, 4 | Cell1 | The UE camps on cell 1 which is the former PCell. |
| Neighbour cell |  | 1, 2, 3, 4 | Cell2 | The UE shall perform inter-frequency measurements on cell 2 which is the former PSCell. |
| RF Channel Number |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  | 1, 2, 3, 4 | 3 s | Synchronous cells |
| Access Barring Information | - | 1, 2, 3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  | 1, 2 | SSB.1 FR1 | Serving cell |
|  |  | 3, 4 | SSB.2 FR1 | Serving cell |
| SMTC configuration Serving cell |  | 1, 2, 3, 4 | SMTC.2 |  |
| DRX cycle length | s | 1, 2, 3, 4 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  | 1, 2, 3, 4 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  | 1, 2, 3, 4 | Not configured |  |
| T3 | s | 1, 2, 3, 4 | [0.5] | T3 needs to be defined so that cell measurement time is taken into account. |
| T4 | s | 1, 2, 3, 4 | [65] | T4 needs to be defined so that cell measurement time is taken into account. |

#### A.6.6.15.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During time durations T1 the UE shall start transmitting preamble on PSCell. During T2 the UE perform intra-frequency measurements on the PCell and the PSCell.

During the time-period T3 the connection is released, and UE enters idle mode. During the time period T3 and T4 the UE is camped in Idle mode and at T4 the signal level of cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to Section 4.4.

At the start of T5 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell including idle mode CA/DC measurement from cell 2.

After receiving the requested early measurement report, the test equipment verifies that the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in Sections 10.1.2B and 10.1.7B and Sections 10.2.4 and 10.2.5, respectively and test ends.

The rate of correct events observed during repeated tests shall be at least 90%.

## A.6.7	Measurement Performance requirements

Unless explicitly stated otherwise:

-	Reported measurements shall be within defined range of accuracy limits defined in Clause 10 for at least 90 % of the reported cases. If multiple measurement performance requirements are verified in the same test, the reported measurements for each requirement shall be within defined range of accuracy limits of the corresponding requirement defined in Clause 10 for at least 90% of the reported cases.

-	Measurements are performed in RRC_CONNECTED state.

-	The reference channels assume transmission of PDSCH with a maximum number of 5 HARQ transmissions unless otherwise specified.

### A.6.7.1	SS-RSRP

#### A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.2.1.1 and 10.1.2.1.2 for intra-frequency measurements.

##### A.6.7.1.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in A.6.7.1.1.2-2. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell.

Table A.6.7.1.1.2-1: SS-RSRP  Intra frequency SS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.1.1.2-2: SS-RSRP Intra frequency test parameters

| Parameter |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |  |  | Cell 1 |  |  | Cell 2 |  |  |
| Cell ID |  |  |  |  |  |  | 489 | 0 |  | 489 |  | 0 |  |  |  | 489 |  | 0 |  |  |  |
| SSB ARFCN |  |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |  |  | freq1 |  |  |  |  |  |
| Duplex mode |  |  |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  |  | Config 1 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  | Config 1 |  | TRS.1.1 FDD |  | NA |  | TRS.1.1 FDD |  |  | NA | TRS.1.1 FDD |  |  |  |  |  | NA |
|  |  |  |  |  | Config 2 |  | TRS.1.1 TDD |  | NA |  | TRS.1.1 TDD |  |  | NA | TRS.1.1 TDD |  |  |  |  |  | NA |
|  |  |  |  |  | Config 3 |  | TRS.1.2 TDD |  | NA |  | TRS.1.2 TDD |  |  | NA | TRS.1.2 TDD |  |  |  |  |  | NA |
| DRX Cycle |  |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  |  | SR.1.1 FDD |  | - | SR.1.1 FDD |  | - |  |  |  | SR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  |  | SR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |  |  | SR2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  |  | CR.1.1 FDD |  | - | CR.1.1 FDD |  | - |  |  |  | CR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |  |  |  | CR2.1 TDD |  |  |  |  |  |
| Control channel RMC |  |  |  | Config 1 |  |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD |  | - |  |  |  | CCR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |  |  | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |  |  |  | CCR2.1 TDD |  |  |  |  |  |
| SSB configuration |  | Config 1 |  |  |  |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 2 |  |  |  |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  |  |  |  | SSB.2 FR1 |  | SSB.2 FR1 | SSB.2 FR1 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 |  |  |  | ms | - |  | 3 | - |  | 3 |  |  |  | - |  |  |  | 3 |  |
|  |  | Config 2,3 |  |  |  | s | - |  | 3 | - |  | 3 |  |  |  | - |  |  |  | 3 |  |
| SMTC configuration |  | Config 1 |  |  |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 30kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  | dB | 0 |  | 0 | 0 |  |  | 0 |  |  |  | 0 |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15KhZ | -106 |  |  | -88 |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  |  | -94 |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
| Note2 | Config 1,2 |  |  |  |  | dBm/SCS | -106 |  |  | -88 |  |  |  |  |  |  | Same as Noc/15kHz |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  |  | -91 |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -109 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -108 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  |  |  |
|  |  |  |  |  |  | dB | 2.46 |  | -5.97 | 2.46 |  |  | -5.97 |  |  |  | -0.01 |  |  |  | -4.76 |
|  |  |  |  |  |  | dB | 6 |  | 1 | 6 |  |  | 1 |  |  |  | 3 |  |  |  | 0 |
| SS-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | dBm/SCS | -100 |  | -105 | -82 |  |  | -87 |  |  |  | -111.00 |  |  |  | -114.00 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  | -110.50 |  |  |  | -113.50 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  | -110.00 |  |  |  | -113.00 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  | -109.50 |  |  |  | -112.50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  | -109.00 |  |  |  | -112.00 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  | -108.50 |  |  |  | -111.50 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  | -108.00 |  |  |  | -111.00 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  | -107.50 |  |  |  | -110.50 |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  | Not applicableNote 5 | -85 |  |  | -90 |  |  |  | -108.00 |  |  |  | -111.00 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -107.50 |  |  |  | -110.50 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -107.00 |  |  |  | -110.00 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -106.50 |  |  |  | -109.50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -106.00 |  |  |  | -109.00 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -105.50 |  |  |  | -108.50 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -105.00 |  |  |  | -108.00 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -104.50 |  |  |  | -107.50 |
| IoNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/ 9.36MHz | -70.09 |  |  | -52.09 |  |  |  |  |  |  | -80.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -79.53 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -79.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -78.53 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -78.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -77.53 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -77.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -76.53 |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/ 38.16MHz | Not applicableNote 5- |  |  | -51.99 |  |  |  |  |  |  | -73.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -73.44 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -72.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -72.44 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -71.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -71.44 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -70.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -70.44 |  |  |  |  |
| Propagation condition |  |  |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.1.1.3	Test Requirements

The SS-RSRP measurement accuracy for cell 1 and cell 2 shall fulfil absolute requirement in clause 10.1.2.1.1 and relative requirement in clause 10.1.2.1.2.

#### A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.4.1.1 and 10.1.4.1.2 for inter-frequency measurements with the testing configurations for NR cells in Table A.6.7.1.2.1-1.

Table A.6.7.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.1.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.6.7.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.6.7.1.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.6.7.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  |  | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 |  |  | freq2 | freq1 |  | freq2 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD |  |  | - | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 FDD |  |  |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD |  |  | - | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD |  |  | - | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 FDD |  |  | - | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD |  |  | - | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD |  |  | - | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD |  |  | - | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1 | ms | - | 3 |  |  | - | 3 |  |
|  |  | 2,3 | s | - | 3 |  |  | - | 3 |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  |  | SMTC.1 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  |  | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4,5 | dBm/SSB SCS | -94.65 |  |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  |  |  | ( for Channel 2 +8dB) |  | -112.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -111.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -111.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -110.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -109.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -109.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -108.50 |
|  |  | 1~3 | dB | 10 |  |  | 10 | 13 |  | -3 |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4,5 | dBm/SCS | -84.65 |  |  |  | (RSRP for Cell 2 +25dB) |  | -118.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -117.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -117.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -116.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -116.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -115.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -115.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -81.65 |  |  |  | (RSRP for Cell 2 +25dB) |  | -115.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -112.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.50 |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4,5 | dBm/ 9.36MHz | -56.28 |  |  |  | Io for Channel 2 +19.75dB)T |  | -85.28 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -84.78 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -83.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -82.78 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -82.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -81.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 38.16MHz | -50.19 |  |  |  | Io for Channel 2 +19.75dB)T |  | -79.19 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -78.69 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -77.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -76.69 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -76.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -75.69 |
|  |  | 1~3 | dB | 10 |  |  | 10 | 13 |  | -3 |
| Propagation condition |  | 1~3 | - | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.4.1.1 and relative requirement in clause 10.1.4.1.2.

#### A.6.7.1.3	Void

### A.6.7.2	SS-RSRQ

#### A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.1.1.

##### A.6.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is tested by using the parameters in Table A.6.7.2.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  |  |  | Test 2 |  |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  |  | Cell 2 |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  |  | freq1 |  |  |  |  | freq1 |  |  |  | freq1 |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |
| Gap Pattern ID |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |  | - |  |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD |  | - |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  |  | Config 3 |  | SR2.1 TDD |  |  |  |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  | - |  |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| Control Channel RMC |  |  | Config 1 |  | CCR.1.1 FDD |  | - |  |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD |  | - |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| TRS Configuration |  |  | Config 1 |  | TRS.1.1 FDD |  | - |  |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  | - |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  |  | OP. 1 |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  | Config 1 | ms | - | 3 |  |  | - |  | 3 |  | - |  | 3 |  |
|  |  |  | Config 2,3 | s | - | 3 |  |  | - |  | 3 |  | - |  | 3 |  |
| SMTC configuration |  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RS for tracking |  |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -85 |  |  |  |  | -101 |  |  |  | -114 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -113 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -112 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -111 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  |  |  |  | - |  |  |  | -114 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -113 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -112 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -111 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  |  |  |  | -101 |  |  |  | -114 -113.5 -113 -112.5 -112 -111.5 -111 -110.5 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  |  |  |  | - |  |  |  | -111 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -110 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -109 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -108 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  |
|  |  |  |  | dB | -1.76 |  |  |  |  | -4.7 |  |  |  | -5..46 |  | -5.46 |
|  |  |  |  | dB | 3 |  | 3 |  |  | -2.9 |  | -2.9 |  | -4 |  | -4 |
| SS-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -82 |  | -82 |  |  | -103.9 |  | -103.9 |  | -118 |  | -118 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -117.5 |  | -117.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -117 |  | -117 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -116.5 |  | -116.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -116 |  | -116 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -115.5 |  | -115.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -115 |  | -115 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -114.5 |  | -114.5 |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 |  | -85 |  |  | - |  | - |  | -115 |  | -115 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -114.5 |  | -114.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -114 |  | -114 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -113.5 |  | -113.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -113 |  | -113 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -112.5 |  | -112.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -112 |  | -112 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -111.5 |  | -111.5 |
| SS-RSRQ Note3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 |  | -14.77 |  |  | -16.76 |  | -16.76 |  | -17.34 |  | -17.34 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 |  | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -50 |  |  |  |  | -70 |  |  |  | -83.5 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -83 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -82.5 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -82 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -81.5 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -81 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -80.5 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -80 |  |  |
|  |  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -50 |  |  |  |  | - |  |  |  | -77.4 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -76.9 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -76.4 |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -75.9 |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -75.4 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -74.9 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -74.4 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -73.9 |  |  |
| Propagation condition |  |  |  | - | AWGN |  | AWGN |  |  | AWGN |  | AWGN |  | AWGN |  | AWGN |
| Antenna configuration |  |  |  |  | 1x2 |  | 1x2 |  |  | 1x2 |  | 1x2 |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.7.1.1.

#### A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.1.1 and 10.1.9.1.2.

##### A.6.7.2.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.6.7.2.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test parameters in Table A.6.7.2.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.2.2.2-1: SS-RSRQ Inter frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.2.2.2-2: SS-RSRQ Inter frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |  |  | Cell 1 |  |  | Cell 2 |  |
| SSB ARFCN |  |  |  |  | freq1 | freq2 |  |  | freq1 |  |  |  | freq2 |  | freq1 |  | freq2 |  |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gap pattern ID |  | Config 1,2,3 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  |  | SR.1.1 FDD | - |  |  | SR.1.1 FDD |  |  | - |  |  | SR.1.1 FDD |  |  | - |  |
|  |  | Config 2,5 |  |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3,6 |  |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |  |  | SR2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD | - |  |  | R.1.1 FDD |  |  | - |  |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |  |  | CR2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD | - |  |  | CCR.1.1 FDD |  |  | - |  |  | CCR.1.1 FDD |  |  | - |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  |  |  | CCR2.1 TDD |  |  |  |  |  | CCR2.1 TDD |  |  |  |  |
| TRS Configuration |  | Config 1 |  |  | TRS.1.1 FDD | - |  |  | TRS.1.1 FDD |  |  | - |  |  | TRS.1.1 FDD |  |  | - |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |  |  |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 |  | ms | - |  |  | 3 |  | - | 3 |  |  | - |  | 3 |  |  |  |
|  |  | Config 2,3 |  | s | - |  |  | 3 |  | - | 3 |  |  | - |  | 3 |  |  |  |
| SMTC configuration |  | Config 1 |  |  | SMTC pattern 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | SMTC pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  |  | SSB pattern 1 in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SSB pattern 2 in FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  |  |  | -106 |  |  |  |  |  | -116 |  |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
| Note2 | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -86.27 |  |  |  | -113 |  |  |  |  |  | -116 |  |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  |  |  | -106 |  |  |  |  |  | -116 |  |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -83.27 |  |  |  | -110 |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  |  |
|  |  |  |  | dB | -1.75 |  |  |  | -1.75 |  |  |  |  |  | 3 |  |  | -1.75 |  |
|  |  |  |  | dB | -1.75 |  |  |  | -1.75 |  |  |  |  |  | 3 |  |  | -1.75 |  |
| SS-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -81.93 | -81.93 |  |  | -107.75 |  |  | -107.75 |  |  | -113 |  |  |  | -117.75 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  | -117.25 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  | -116.75 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  | -116.25 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  | -115.75 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  | -115.2 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  | -114.75 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  | -114.25 |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85.02 | -85.02 |  |  | -111.75 |  |  | -111.75 |  |  | -110 |  |  |  | -114.75 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  | -114.25 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -109 |  |  |  | -113.75 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  |  | -113.25 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -108 |  |  |  | -112.75 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  |  | -112.2 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -107 |  |  |  | -111.75 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -106.5 |  |  |  | -111.25 |
| SS-RSRQNote3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 |  |  | -40.59 |  |  | -40.59 |  |  | 12.56T |  |  |  | 14.76T |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -50 |  |  |  | -75.83 |  |  |  |  |  | -83.28 |  |  |  | -85.83 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -82.78 |  |  |  | -85.33 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -82.28 |  |  |  | -84.83 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -81.78 |  |  |  | -84.33 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -81.28 |  |  |  | -83.83 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -80.78 |  |  |  | -83.33 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -80.28 |  |  |  | -82.83 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -79.78 |  |  |  | -82.33 |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -50 |  |  |  | -76.73 |  |  |  |  |  | -77.19 |  |  |  | -79.73 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -76.69 |  |  |  | -79.23 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -76.19 |  |  |  | -78.73 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -75.69 |  |  |  | -78.23 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -75.19 |  |  |  | -77.73 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -74.69 |  |  |  | -77.23 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -74.19 |  |  |  | -76.73 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -73.69 |  |  |  | -76.53 |
| Propagation condition |  |  |  | - | AWGN | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |
| Antenna configuration |  |  |  |  | 1x2 | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.9.1.1 and 10.1.9.1.2.

### A.6.7.3	SS-SINR

#### A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.12.1.1.

##### A.6.7.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.3.1.2-1. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in Table A.6.7.3.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| TRS configuration |  |  |  | Config 1 |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |  |
|  |  |  |  | Config 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |
|  |  |  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | SR.2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |
|  |  |  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| SMTC configuration |  |  | Config 1 Config 2,3 |  |  | SMTC.2 |  |  |  |  |  |  |
|  |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  | Config 1 Config 2,3 |  | ms | - |  | 3 |  | - | 3 |  |
|  |  |  |  |  | ms | - |  | 3 |  | - | 3 |  |
| SSB configuration |  |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -93 |  |  | -116 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -115.5 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -115 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -114.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -114 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -113.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -113 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -112.5 |  |  |  |
| Note2 | Config 1,2 |  |  |  | dBm/SCS | -93 |  |  | Same as Noc for 15 kHz |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -90 |  |  | -113 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -112.5 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -112 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -111.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -111 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -110.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -110 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -109.5 |  |  |  |
|  |  |  |  |  | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  |  |  | dB | 4.54 | 2.66 |  | -4 |  |  | -4 |
| SS-RSRPNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -88.46 | -90.34 |  | -120 |  |  | -120 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -119.5 |  |  | -119.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -119 |  |  | -119 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -118.5 |  |  | -118.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -118 |  |  | -118 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -117.5 |  |  | -117.5 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -117 |  |  | -117 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -116.5 |  |  | -116.5 |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85.46 | -87.34 |  | -117 |  |  | -117 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -116.5 |  |  | -116.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -116 |  |  | -116 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -115.5 |  |  | -115.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -115 |  |  | -115 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -114.5 |  |  | -114.5 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -114 |  |  | -114 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -113.5 |  |  | -113.5 |
| SS-SINR Note3 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |
| IoNote3 |  | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.5 |  |  | -85.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -85.01 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -84.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -84.01 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -83.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -83.01 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -82.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -82.01 |  |  |  |
|  |  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.41 |  |  | -79.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -78.91 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -78.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -77.91 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -77.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -76.91 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -76.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -75.91 |  |  |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  | - | 1x2 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.3.1.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.12.1.1.

#### A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.14.1.1 and 10.1.14.1.2.

##### A.6.7.3.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.6.7.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table A.6.7.3.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.3.2.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.3.2.2-2: SS-SINR Inter frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  | Test 2 |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  | Cell 2 | Cell 1 | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  |  | freq1 |  | freq2 | freq1 | freq2 | freq1 | freq2 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| Gap pattern ID |  |  |  |  | 0 | - |  | 0 | - | 0 |  | - |
| TRS configuration |  | Config 1 |  |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - | SR.1.1 FDD |  | - |
|  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - | CR.1.1 FDD |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 |  | ms | - | 3 |  | - | 3 | - |  | 3 |
|  |  | Config 2,3 |  | s | - | 3 |  | - | 3 | - |  | 3 |
| SMTC configuration |  | Config 1 |  |  | SMTC pattern 2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | SMTC pattern 1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | kHz | 15 |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  | 0 | 0 | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -88 |  |  | -108.5 |  | -119.5 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -119 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -118.5 |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -118 |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -117.5 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -117 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -116.5 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -116 |  |  |
| Note2 | Config 1,2 |  |  | dBm/SCS | -88 |  |  | -108.5 |  | Same as Noc for 15kHz |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85 |  |  | -105.5 |  | -116.5 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -116 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -115.5 |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -115 |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114.5 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -114 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -114.5 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -113 |  |  |
|  |  |  |  | dB | -1.75 |  | -1.75 | 20 | 20 | -4.0 |  | -4.0 |
|  |  |  |  | dB | -1.75 |  |  | 20 |  | -4.0 |  |  |
| SS-RSRP Note3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -89.75 |  |  | -88.5 |  | -123.5 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -123 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -122.5 |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -122 |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -121.5 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -121 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -120.5 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -120 |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -86.75 |  |  | -85.5 |  | -120.5 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -120 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -119.5 |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -119 |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -118.5 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -118 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -117.5 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -117 |  |  |
| SS-SINRNote3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -1.75 |  |  | 20 |  | -4.0 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.83 |  |  | -60.5 |  | -90.09 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -89.59 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -89.09 |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -88.59 |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -88.09 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -87.59 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -87.09 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -86.59 |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.73 |  |  | -54.41 |  | -84 |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -83.5 |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -83 |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -82.5 |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -82 |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -81.5 |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -81 |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -80.5 |  |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.3.2.3	Test Requirements

The SS-SINR measurement accuracy shall fulfil the requirements in clause 10.1.14.1.1 and 10.1.14.1.2.

### A.6.7.4	L1-RSRP measurement for beam reporting

#### A.6.7.4.1	SSB based L1-RSRP measurement

##### A.6.7.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.5.2 and clause 10.1.19.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.6.7.4.1.1-1.

Table A.6.7.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.4.1.2	Test parameters

In this set of test cases there one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.6.7.4.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.6.7.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity |  | 1~3 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | -3 |
| SSB RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.4.1.3	Test Requirements

The L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in clauses 10.1.19.1.

#### A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off

##### A.6.7.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.5.3 and clause 10.1.19.2 for L1-RSRP measurements based on CSI-RS with the testing configurations for NR cells in Table A.6.7.4.2.1-1.

Table A.6.7.4.2.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.4.2.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.4.2.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.6.7.4.2.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.6.7.4.2.2-1: FR1 CSI-RS based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity |  | 1~3 |  | cri-RSRP | cri-RSRP |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | -3 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.4.2.3	Test Requirements

The L1-RSRP measurement accuracy for CSI-RS#0 and CSI-RS#1 of Cell 1 shall fulfil the requirements in clause 10.1.19.2.

### A.6.7.5	E-UTRAN RSRP

#### A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell

##### A.6.7.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.2.2 for SA inter-RAT E-UTRAN RSRP measurements.

##### A.6.7.5.1.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an E-UTRAN inter-RAT neighbour cell. Supported test configurations are shown in table A.6.7.5.1.2-1. The measurement accuracy of SA inter-RAT E-UTRAN RSRP are tested by using the parameters in A.6.7.5.1.2-2 and A.6.7.5.1.2-3.

Table A.6.7.5.1.2-1: Inter-RAT E-UTRAN RSRP supported test configurations with FR1 serving cell

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.5.1.2-2: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.6.7.5.1.2-3: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| E-UTRA RF channel number |  |  | 1 |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |
|  | Config 4, 5, 6 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -91.65 | -117 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -116.5 |
|  | Bands FDD_C, TDD_C |  |  | -116 |
|  | Bands FDD_D |  |  | -115.5 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -115 |
|  | Bands FDD_G Note 8 |  |  | -114 |
|  | Bands FDD_H |  |  | -113.5 |
| Ês/Noc |  | dB | 10 | -4 |
| Ês/IotNote5 |  | dB | 10 | -4 |
| RSRPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -81.65 | -121 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -120.5 |
|  | Bands FDD_C, TDD_C |  |  | -120 |
|  | Bands FDD_D |  |  | -119.5 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -119 |
|  | Bands FDD_G Note 8 |  |  | -118 |
|  | Bands FDD_H |  |  | -117.5 |
| SCH_RPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -81.65 | -121 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -120.5 |
|  | Bands FDD_C, TDD_C |  |  | -120 |
|  | Bands FDD_D |  |  | -119.5 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -119 |
|  | Bands FDD_G Note 8 |  |  | -118 |
|  | Bands FDD_H |  |  | -117.5 |
| IoNote5 | Bands FDD_A Note 9, TDD_A | dBm/Ch BW | -53.45 + 10log(NRB,c/50) | -87.76 + 10log(NRB,c/50) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  | -87.26 + 10log(NRB,c/50) |
|  | Bands FDD_C, TDD_C |  |  | -86.76 + 10log(NRB,c/50) |
|  | Bands FDD_D |  |  | -86.26 + 10log(NRB,c/50) |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  | -85.76 + 10log(NRB,c/50) |
|  | Bands FDD_G Note 8 |  |  | -84.76 + 10log(NRB,c/50) |
|  | Bands FDD_H |  |  | -84.26 + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [15]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

##### A.6.7.5.1.3	Test Requirements

The SA inter-RAT E-UTRAN RSRP measurement accuracy for cell 2 shall fulfil absolute requirement in clause 10.2.2.

### A.6.7.6	E-UTRAN RSRQ

#### A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell

##### A.6.7.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.2.3 for SA inter-RAT E-UTRAN RSRQ measurements.

##### A.6.7.6.1.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an E-UTRAN inter-RAT neighbour cell. Supported test configurations are shown in table A.6.7.6.1.2-1. The measurement accuracy of SA inter-RAT E-UTRAN RSRQ are tested by using the parameters in A.6.7.6.1.2-2 and A.6.7.6.1.2-3.

Table A.6.7.6.1.2-1: Inter-RAT E-UTRAN RSRQ supported test configurations with FR1 serving cell

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.6.1.2-2: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | dB |
| Ês/IotNote3 |  | dB | dB |
| SS-RSRQNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRQ, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.6.7.6.1.2-3: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -83 | -104.70 | -119.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -119 |
|  | Bands FDD_C, TDD_C |  |  |  | -118.5 |
|  | Bands FDD_D |  |  |  | -118 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -117.5 |
|  | Bands FDD_G Note 8 |  |  |  | -116.5 |
|  | Bands FDD_H |  |  |  | -116 |
| Ês/Noc |  | dB | -1.75 | -4.0 | -4.0 |
| Ês/IotNote5 |  | dB | -1.75 | -4.0 | -4.0 |
| RSRPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -84.75 | -108.70 | -123.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -123 |
|  | Bands FDD_C, TDD_C |  |  |  | -122.5 |
|  | Bands FDD_D |  |  |  | -122 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -121.5 |
|  | Bands FDD_G Note 8 |  |  |  | -120.5 |
|  | Bands FDD_H |  |  |  | -120 |
| RSRQNote5 | Bands FDD_A Note 9, TDD_A | dB | -14.76 | -16.25 | -16.25 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |
|  | Bands FDD_C, TDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  |  |
|  | Bands FDD_G Note 8 |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote5 | Bands FDD_A Note 9, TDD_A | dBm/Ch BW | -53 + 10log(NRB,c /50) | -75.46 + 10log(NRB,c /50) | -90.26 + 10log(NRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -89.76 + 10log(NRB,c /50) |
|  | Bands FDD_C, TDD_C |  |  |  | -89.26 + 10log(NRB,c /50) |
|  | Bands FDD_D |  |  |  | -88.76 + 10log(NRB,c /50) |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -88.26 + 10log(NRB,c /50) |
|  | Bands FDD_G Note 8 |  |  |  | -87.26 + 10log(NRB,c /50) |
|  | Bands FDD_H |  |  |  | -86.76 + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [15]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

##### A.6.7.6.1.3	Test Requirements

The SA inter-RAT E-UTRAN RSRQ measurement accuracy for cell 2 shall fulfil absolute requirement in clause 10.2.3.

### A.6.7.7	E-UTRAN RS-SINR

#### A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell

##### A.6.7.7.1.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN RS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.2.4 for SA inter-RAT E-UTRAN RS-SINR measurements.

##### A.6.7.7.1.2	Test parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an E-UTRAN inter-RAT neighbour cell. Supported test configurations are shown in table A.6.7.7.1.2-1. The measurement accuracy of SA inter-RAT E-UTRAN RS-SINR are tested by using the parameters in A.6.7.7.1.2-2 and A.6.7.7.1.2-3.

Table A.6.7.7.1.2-1: Inter-RAT E-UTRAN RS-SINR supported test configurations with FR1 serving cell

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, E-UTRAN TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, E-UTRAN TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.7.1.2-2: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RS-SINRNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RS-SINR, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.6.7.7.1.2-3: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -88 | -108.50 | -119.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -119 |
|  | Bands FDD_C, TDD_C |  |  |  | -118.5 |
|  | Bands FDD_D |  |  |  | -118 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -117.5 |
|  | Bands FDD_G Note 8 |  |  |  | -116.5 |
|  | Bands FDD_H |  |  |  | -116 |
| CRS Ês/Noc1 |  | dB | -1.75 | 20.0 | -4.0 |
| CRS Ês/IotNote5 |  | dB | -1.75 | 20.0 | -4.0 |
| RSRPNote5 | Bands FDD_A Note 9, TDD_A | dBm/15kHz | -89.75 | -88.50 | -123.5 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -123 |
|  | Bands FDD_C, TDD_C |  |  |  | -122.5 |
|  | Bands FDD_D |  |  |  | -122 |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -121.5 |
|  | Bands FDD_G Note 8 |  |  |  | -120.5 |
|  | Bands FDD_H |  |  |  | -120 |
| RS-SINRNote5 | Bands FDD_A Note 9, TDD_A | dB | -1.75 | 20 | -4.0 |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  |  |
|  | Bands FDD_C, TDD_C |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  |  |
|  | Bands FDD_G Note 8 |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |
| IoNote5 | Bands FDD_A Note 9, TDD_A | dBm/Ch BW | -53.79 + 10log(NRB,c /50) | -60.56 + 10log(NRB,c /50) | -93.48 + 10log(NRB,c /50) |
|  | Bands FDD_B1, FDD_B2 Note 10 |  |  |  | -92.98 + 10log(NRB,c /50) |
|  | Bands FDD_C, TDD_C |  |  |  | -92.48 + 10log(NRB,c /50) |
|  | Bands FDD_D |  |  |  | -91.98 + 10log(NRB,c /50) |
|  | Bands FDD_E, FDD_F Note 7, TDD_E |  |  |  | -91.48 + 10log(NRB,c /50) |
|  | Bands FDD_G Note 8 |  |  |  | -90.48 + 10log(NRB,c /50) |
|  | Bands FDD_H |  |  |  | -89.98 + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4a: Void. Note 5: CRS Ês/Iot, RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [15]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

##### A.6.7.7.1.3	Test Requirements

The SA inter-RAT E-UTRAN RS-SINR measurement accuracy for cell 2 shall fulfil absolute requirement in clause 10.2.4.

### A.6.7.8	CLI measurements

#### A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell

##### A.6.7.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.22.1.1 with the testing configurations for NR cells in Table A.6.7.8.1.1-1.

Table A.6.7.8.1.1-1: Applicable NR configurations for FR1 SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 15kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.8.1.2	Test parameters

In this set of test cases there is one cell in the test, FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.8.1.2-1 below. The test parameter for the (virtual) neighbor cell UE transmitting SRS are given in Table A.6.7.8.1.2-2.

Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS configuration in Table A.6.7.8.1.2-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

Table A.6.7.8.1.2-1: FR1 test parameters for SRS-RSRP accuracy for PCell

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~2 |  | freq1 | freq1 | freq1 |
| Duplex mode |  | 1~2 |  | TDD | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 2 |  | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 40: NRB,c = 106 | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 2 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 2 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~2 |  | OP.1 | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 2 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~2 |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1~2 | s | 17.67 | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1~2 | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 4 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 3 | 2 |  | Not applicableNote 4 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 4: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table A.6.7.8.1.2-2: FR1 test parameters for SRS-RSRP accuracy for neighbour cell UE

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| on SRS |  | 1~2 | dB | 1 | 1 | 1 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -105 | -87 | -113 |
|  | NR_TDD_FR1_C |  |  |  |  | -112 |
|  | NR_TDD_FR1_D |  |  |  |  | -111.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -111 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -87 | -110 |
|  | NR_TDD_FR1_C |  |  |  |  | -109 |
|  | NR_TDD_FR1_D |  |  |  |  | -108.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -108 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/9.36 MHz | -74.51 | -56.51 | -82.51 |
|  | NR_TDD_FR1_C |  |  |  |  | -81.51 |
|  | NR_TDD_FR1_D |  |  |  |  | -81.01 |
|  | NR_TDD_FR1_E |  |  |  |  | -79.51 |
|  | NR_TDD_FR1_A NOTE 5 | 2 | dBm/38.16 MHz | Not applicableNote 6 | -53.42 | -76.42 |
|  | NR_TDD_FR1_C |  |  |  |  | -75.42 |
|  | NR_TDD_FR1_D |  |  |  |  | -74.92 |
|  | NR_TDD_FR1_E |  |  |  |  | -74.42 |
| on SRS |  | 1~2 | dB | 1 | 1 | 1 |
| Propagation condition |  | 1~2 |  | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1~2 |  | 1x2 | 1x2 | 1x2 |
| SRS configuration |  | 1 |  | SRSConf.1 | SRSConf.1 | SRSConf.1 |
|  |  | 2 |  | SRSConf.2 | SRSConf.2 | SRSConf.2 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table A.6.7.8.1.2-3: SRS configuration parameters for FR1 SRS-RSRP accuracy

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |
|  | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
| SRS-Resource | SRS-ResourceId | 0 | 0 |
|  | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n1 | n1 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 12 | 12 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20, 9 | sl40, 19 |
|  | sequenceId | 0 | 0 |

#### A.6.7.8.1.3	Test Requirements

The SRS-RSRP measurement accuracy shall fulfil the requirements in clauses 10.1.22.1.1.

#### A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell

##### A.6.7.8.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CLI-RSSI measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.22.2.1 with the testing configurations for NR cells in Table A.6.7.8.2.1-1.

Table A.6.7.8.2.1-1: Applicable NR configurations for FR1 CLI-RSSI accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.8.2.2	Test parameters

In this set of test cases there is one cell in the test, the FR1 PSCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.8.2.2-1 below.

Before the test UE is configured to perform CLI-RSSI measurement. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for CLI-RSSI resource and on 1 data symbol before. The CLI-RSSI measurement resource configuration is in Table A.6.7.8.2.2-2.

Table A.6.7.8.2.2-1: FR1 test parameters for CLI-RSSI accuracy

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD configuration | 1 |  | TDDConf.1.1 |
|  | 2 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 TDD |
|  | 2 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 TDD |
|  | 2 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 TDD |
|  | 2 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR1 |
|  | 2 |  | SSB.2 FR1 |
| OCNG Patterns Note6 | 1~2 |  | OP.1 |
| TRS configuration | 1 |  | TRS.1.1 TDD |
|  | 2 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| Time offset between DL from serving cell and OCNG from test system | 1~2 | s | 17.67 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| on CLI-RSSI measurement resource Note2 | 1 | dBm/15kHz | -106 |
|  | 2 |  | -106 |
| on CLI-RSSI measurement resource Note2 | 1 | dBm/ BWP SCS | -106 |
|  | 2 |  | -103 |
| on CLI-RSSI measurement resource | 1~2 | dB | -Infinity |
| RSRP on CLI-RSSI measurement resource Note3 | 1~2 | dBm/ BWP SCS | -Infinity |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/9.36 MHz | -78.05 |
|  | 2 | dBm/38.16 MHz | -71.96 |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/1.08 MHz | -87.43 |
|  | 2 |  | -87.44 |
| on CLI-RSSI measurement resource | 1~2 | dB | -Infinity |
| Propagation condition | 1~2 |  | AWGN |
| Antenna configuration | 1~2 |  | 1x2 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |

Table A.6.7.8.2.2-2: CLI-RSSI measurement resource configuration for FR1 CLI-RSSI accuracy

|  | Field | Config | SRSConf.1 |
| --- | --- | --- | --- |
| CLI-RSSI measurement resource | rssi-ResourceId | 1~2 | 0 |
|  | rssi-SCS | 1 | 15kHz |
|  |  | 2 | 30kHz |
|  | startPRB | 1~2 | 0 |
|  | nrofPRBs | 1 | 52 |
|  |  | 2 | 106 |
|  | startPosition | 1~2 | 3 |
|  | nrofSymbols | 1~2 | 11 |
|  | rssi-PeriodicityAndOffset | 1 | sl20, 9 |
|  |  | 2 | sl40, 19 |

##### A.6.7.8.2.3	Test Requirements

The CLI-RSSI measurement accuracy shall fulfil the requirements in clauses 10.1.22.2.1.

### A.6.7.9	L1-SINR measurement for beam reporting

#### A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off

##### A.6.7.9.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.8.4.1 and clause 10.1.27.1 for L1-SINR measurements based on CSI-RS with the testing configurations for NR cells in Table A.6.7.9.1.1-1.

Table A.6.7.9.1.1-1: Applicable NR configurations for FR1 L1-SINR test with CSI-RS based CMR and no dedicated IMR configured

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.9.1.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.9.1.2-1 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in Table A.6.7.9.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.6.7.9.1.2-1: FR1 CSI-RS based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | -3 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.9.1.3	Test Requirements

The L1-SINR measurement accuracy for CSI-RS#0 and CSI-RS#1 of Cell 1 shall fulfil the requirements in clause 10.1.27.1.

#### A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR

##### A.6.7.9.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.8.4.2 and clause 10.1.27.2 for L1-SINR measurements with SSB based CMR and dedicated CSI-RS based IMR, with the testing configurations for NR cells in Table A.6.7.9.2.1-1.

Table A.6.7.9.2.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.9.2.2	Test parameters

In this set of test cases there one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.9.2.2-1 below. The absolute accuracy of L1-SINR measurements are tested by using the parameters in Table A.6.7.9.2.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources and one CSI-RS resource set with two CSI-RS resource. UE is configured to perform RLM and BFD measurement based on the SSB resources 0 and 1. UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-RS resources as IMR.

Table A.6.7.9.2.2-1: FR1 SSB based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| CSI-RS configuration |  | 1 |  | CSI-RS 1.1A FDD | CSI-RS 1.1A FDD |
|  |  | 2 |  | CSI-RS 1.1A TDD | CSI-RS 1.1A TDD |
|  |  | 3 |  | CSI-RS 2.1A TDD | CSI-RS 2.1A TDD |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | ssb-Index-SINR-r16 | ssb-Index-SINR-r16 |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | 0 |
|  |  | 1~6 | dB | 10 | 0 |
| SSB RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -84.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -113 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -86.04 |
|  | NR_FDD_FR1_B |  |  |  | -85.54 |
|  | NR_TDD_FR1_C |  |  |  | -85.04 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -84.54 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -84.04 |
|  | NR_FDD_FR1_F |  |  |  | -83.54 |
|  | NR_FDD_FR1_G |  |  |  | -83.04 |
|  | NR_FDD_FR1_H |  |  |  | -82.54 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -79.94 |
|  | NR_FDD_FR1_B |  |  |  | -79.44 |
|  | NR_TDD_FR1_C |  |  |  | -78.94 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -78.44 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -77.94 |
|  | NR_FDD_FR1_F |  |  |  | -77.44 |
|  | NR_FDD_FR1_G |  |  |  | -76.94 |
|  | NR_FDD_FR1_H |  |  |  | -76.44 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.9.2.3	Test Requirements

The L1-SINR measurement accuracy for SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1 of Cell 1 shall fulfil the requirements in clauses 10.1.27.2.

#### A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR

##### A.6.7.9.3.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will partly verify the requirements in Clauses 9.8.4.3 and clause 10.1.27.3 for L1-SINR measurements based on CSI-RS as CMR and CSI-IM as IMR with the testing configurations for NR cells in Table A.6.7.9.3.1-1.

Table A.6.7.9.3.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.9.3.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.9.3.2-1 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in Table A.6.7.9.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources and one CSI-IM resource set with two CSI-IM resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. UE is configured to perform L1-SINR measurement based on the configured CSI-RS as CMR and CSI-IM as IMR.

Table A.6.7.9.3.2-1: FR2 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS configuration as CMR |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| CSI-IM configuration as IMR |  | 1 |  | CSI-IM 1.3 FDD | CSI-IM 1.3 FDD |
|  |  | 2 |  | CSI-IM 1.3 TDD | CSI-IM 1.3 TDD |
|  |  | 3 |  | CSI-IM 2.3 TDD | CSI-IM 2.3 TDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -94.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 | -114 |
|  | NR_FDD_FR1_B |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  | -110.5 |
|  |  | 1~3 | dB | 10 | -3 |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/CSI-RS SCS | -84.65 | -120 |
|  | NR_FDD_FR1_B |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  | -116.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 | -117 |
|  | NR_FDD_FR1_B |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  | -113.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 | -87.28 |
|  | NR_FDD_FR1_B |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  | -83.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 | -81.19 |
|  | NR_FDD_FR1_B |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  | -77.69 |
|  |  | 1~3 | dB | 10 | -3 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.6.7.9.3.3	Test Requirements

The L1-SINR measurement accuracy for CSI-RS#0+CSI-IM#0 and CSI-RS#1+CSI-IM# of Cell 1 shall fulfil the requirements in clause 10.1.27.3.A.7	NR standalone tests with one or more NR cells in FR2.

### A.6.7.10	CSI-RSRP

#### A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.10.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.2.3.1 and 10.1.2.3.2 for CSI-RS intra-frequency measurements.

##### A.6.7.10.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.10.1.2-1. Both absolute and relative accuracy of CSI-RSRP intra-frequency measurements are tested by using the parameters in A.6.7.10.1.2-2. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell.

Table A.6.7.10.1.2-1: CSI-RSRP intra frequency supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.10.1.2-2: CSI-RSRP intra frequency test parameters

| Parameter |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |  |  | Cell 1 |  |  | Cell 2 |  |  |
| Cell ID |  |  |  |  |  |  | 489 | 0 |  | 489 |  | 0 |  |  |  | 489 |  | 0 |  |  |  |
| SSB ARFCN |  |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |  |  | freq1 |  |  |  |  |  |
| Duplex mode |  |  |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  |  | Config 1 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  | Config 1 |  | TRS.1.1 FDD |  | NA |  | TRS.1.1 FDD |  |  | NA | TRS.1.1 FDD |  |  |  |  |  | NA |
|  |  |  |  |  | Config 2 |  | TRS.1.1 TDD |  | NA |  | TRS.1.1 TDD |  |  | NA | TRS.1.1 TDD |  |  |  |  |  | NA |
|  |  |  |  |  | Config 3 |  | TRS.1.2 TDD |  | NA |  | TRS.1.2 TDD |  |  | NA | TRS.1.2 TDD |  |  |  |  |  | NA |
| DRX Cycle |  |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  |  | SR.1.1 FDD |  | - | SR.1.1 FDD |  | - |  |  |  | SR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  |  | SR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  |  |  | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  |  | CR.1.1 FDD |  | - | CR.1.1 FDD |  | - |  |  |  | CR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  |  | CR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |  |  | CR.2.1 TDD |  |  |  |  |  |
| Control channel RMC |  |  |  | Config 1 |  |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD |  | - |  |  |  | CCR.1.1 FDD |  |  |  |  | - |
|  |  |  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |  |  | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |  |  |  | CCR2.1 TDD |  |  |  |  |  |
| SSB configuration |  | Config 1 |  |  |  |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 2 |  |  |  |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |
|  |  | Config 3 |  |  |  |  | SSB.2 FR1 |  | SSB.2 FR1 | SSB.2 FR1 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1,2 |  |  |  | s | - |  | 4.7 | - |  | 4.7 |  |  |  | - |  |  |  | 4.7 |  |
|  |  | Config 3 |  |  |  | s | - |  | 2.35 | - |  | 2.35 |  |  |  | - |  |  |  | 2.35 |  |
| SMTC configuration |  | Config 1 |  |  |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RS configuration for RRM |  | Config 1 |  |  |  |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  |  |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  |  | 30kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  | dB | 0 |  | 0 | 0 |  |  | 0 |  |  |  | 0 |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15KhZ | -106 |  |  | -88 |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  |  | -94 |  |  |  |  |  |  | -114 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
| Note2 | Config 1,2 |  |  |  |  | dBm/SCS | -106 |  |  | -88 |  |  |  |  |  |  | Same as Noc/15kHz |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  |  | -91 |  |  |  |  |  |  | -111 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -109 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -108 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  |  |  |
|  |  |  |  |  |  | dB | 2.46 |  | -5.97 | 2.46 |  |  | -5.97 |  |  |  | -0.01 |  |  |  | -4.76 |
|  |  |  |  |  |  | dB | 6 |  | 1 | 6 |  |  | 1 |  |  |  | 3 |  |  |  | 0 |
| CSI-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | dBm/SCS | -100 |  | -105 | -82 |  |  | -87 |  |  |  | -111.00 |  |  |  | -114.00 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  | -110.50 |  |  |  | -113.50 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  | -110.00 |  |  |  | -113.00 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  | -109.50 |  |  |  | -112.50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  | -109.00 |  |  |  | -112.00 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  | -108.50 |  |  |  | -111.50 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  | -108.00 |  |  |  | -111.00 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  | -107.50 |  |  |  | -110.50 |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  |  | Not applicableNote 5 |  | Not applicableNote 5 | -85 |  |  | -90 |  |  |  | -108.00 |  |  |  | -111.00 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -107.50 |  |  |  | -110.50 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -107.00 |  |  |  | -110.00 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -106.50 |  |  |  | -109.50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -106.00 |  |  |  | -109.00 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -105.50 |  |  |  | -108.50 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -105.00 |  |  |  | -108.00 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -104.50 |  |  |  | -107.50 |
| IoNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/ 9.36MHz | -70.09 |  |  | -52.09 |  |  |  |  |  |  | -80.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -79.53 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -79.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -78.53 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -78.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -77.53 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -77.03 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -76.53 |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/ 38.16MHz | Not applicableNote 5- |  |  | -51.99 |  |  |  |  |  |  | -73.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  | -73.44 |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  | -72.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  | -72.44 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  | -71.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  | -71.44 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  | -70.94 |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  | -70.44 |  |  |  |  |
| Propagation condition |  |  |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB and CSI-RS SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.10.1.3	Test Requirements

The CSI-RSRP measurement accuracy for cell 1 and cell 2 shall fulfil absolute requirement in clause 10.1.2.3.1 and relative requirement in clause 10.1.2.3.2.

#### A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.10.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.4.3.1 and 10.1.4.3.2 for CSI-RS inter-frequency measurements with the testing configurations for NR cells in Table A.6.7.10.2.1-1.

Table A.6.7.10.2.1-1: Applicable NR configurations for FR1 inter-frequency CSI-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.6.7.10.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.6.7.10.2.2-1 below. Both absolute and relative accuracy of CSI-RSRP inter-frequency measurements are tested by using the parameters in Table A.6.7.10.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.6.7.10.2.2-1: CSI-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 |  | freq1 |  | freq2 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 FDD | - |  | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1,2 | s | - |  | 4.7 | - | 4.7 |  |
|  |  | 3 | s | - |  | 2.35 | - | 2.35 |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| CSI-RS configuration for RRM |  | 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  | CSI-RS.RRM.FR1.1 FDD |  |  |
|  |  | 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  | CSI-RS.RRM.FR1.1 TDD |  |  |
|  |  | 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  | CSI-RS.RRM.FR1.2 TDD |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SSB SCS | -94.65 |  |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  |  | ( for Channel 2 +8dB) |  | -112.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -112.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -111.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -110.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.50 |
|  |  | 1~3 | dB | 10 | 10 |  | 13 |  | -3 |
| CSI-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SCS | -84.65 |  |  | (RSRP for Cell 2 +25dB) |  | -118.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -117.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -117.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -115.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -81.65 |  |  | (RSRP for Cell 2 +25dB) |  | -115.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.50 |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/ 9.36MHz | -56.28 |  |  | Io for Channel 2 +19.75dB |  | -85.28 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -84.78 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -83.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -82.78 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -82.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -81.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 38.16MHz | -50.19 |  |  | Io for Channel 2 +19.75dB |  | -79.19 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -78.69 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -77.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -76.69 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -76.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -75.69 |
|  |  | 1~3 | dB | 10 | 10 |  | 13 |  | -3 |
| Propagation condition |  | 1~3 | - | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

##### A.6.7.10.2.3	Test Requirements

The CSI-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirement in clause 10.1.4.3.1 and relative requirement in clause 10.1.4.3.2.

### A.6.7.11	CSI-RSRQ

#### A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.2.

##### A.6.7.11.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.11.1.2-1. The absolute accuracy of CSI-RSRQ intra-frequency measurement is tested by using the parameters in Table A.6.7.11.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.11.1.2-1: Intra frequency CSI-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.11.1.2-2: CSI-RSRQ Intra frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  | Test 2 |  |  |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 |  |  | Cell 2 |  | Cell 1 |  | Cell 2 |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |
| Gap Pattern ID |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |
| BWP configuration |  |  |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD |  |  | - |  | SR.1.1 FDD |  | - |
|  |  |  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |  | - |  | CR.1.1 FDD |  |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |
| Control Channel RMC |  |  |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  |  | - |  | CCR.1.1 FDD |  | - |
|  |  |  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |
| TRS Configuration |  |  |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |  | - |  | TRS.1.1 FDD |  | - |
|  |  |  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |
|  |  |  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  |  |  | OP. 1 |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 |  |  | s | - | 4.7 |  | - | 4.7 |  | - |  | 4.7 |  |
|  |  | Config 2,3 |  |  | s | - | 2.35 |  | - | 2.35 |  | - |  | 2.35 |  |
| CSI-RS configuration for RRM |  | Config 1 |  |  |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1 |  |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |
| SSB configuration |  |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 | 0 |  |  | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -85 |  | -101 |  |  |  |  | -114 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -113.5 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -113 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -112.5 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -112 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -111.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -111 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -110.5 |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  | - |  |  |  |  | -114 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -113.5 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -113 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -112.5 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -112 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -111.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -111 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -110.5 |  |  |
| Note2 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  | -101 |  |  |  |  | -114 -113.5 -113 -112.5 -112 -111.5 -111 -110.5 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  | - |  |  |  |  | -111 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -110.5 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -110 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -109.5 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -109 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -108.5 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -108 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -107.5 |  |  |
|  |  |  |  |  | dB | -1.76 |  | -4.7 |  |  |  |  | -5..46 |  | -5.46 |
|  |  |  |  |  | dB | 3 | 3 | -2.9 |  |  | -2.9 |  | -4 |  | -4 |
| SS-RSRP/ CSI-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | dBm/SCS | -82 | -82 | -103.9 |  |  | -103.9 |  | -118 |  | -118 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -117.5 |  | -117.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -117 |  | -117 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -116.5 |  | -116.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -116 |  | -116 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -115.5 |  | -115.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -115 |  | -115 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -114.5 |  | -114.5 |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 | -85 | - |  |  | - |  | -115 |  | -115 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -114.5 |  | -114.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -114 |  | -114 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -113.5 |  | -113.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -113 |  | -113 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -112.5 |  | -112.5 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -112 |  | -112 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -111.5 |  | -111.5 |
| SS-RSRQ/CSI-RSRQ Note3 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -16.76 |  |  | -16.76 |  | -17.34 |  | -17.34 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -50 |  | -70 |  |  |  |  | -83.5 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -83 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -82.5 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -82 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -81.5 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -81 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -80.5 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -80 |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -50 |  | - |  |  |  |  | -77.4 |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  | -76.9 |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  | -76.4 |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  | -75.9 |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  | -75.4 |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  | -74.9 |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  | -74.4 |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  | -73.9 |  |  |
| Propagation condition |  |  |  |  | - | AWGN | AWGN | AWGN |  |  | AWGN |  | AWGN |  | AWGN |
| Antenna configuration |  |  |  |  |  | 1x2 | 1x2 | 1x2 |  |  | 1x2 |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.11.1.3	Test Requirements

The CSI-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.7.2.

#### A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.11.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.2.1 and 10.1.9.2.2.

##### A.6.7.11.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.6.7.11.2.2-1. Both absolute accuracy and relative accuracy requirements of CSI-RSRQ inter-frequency measurement are tested by using test parameters in Table A.6.7.11.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.11.2.2-1: CSI-RSRQ Inter frequency CSI-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.11.2.2-2: CSI-RSRQ Inter frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  |  | Test 2 |  |  |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |
| Gap pattern ID |  | Config 1,2,3 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  |  | SR.1.1 FDD | - |  |  | SR.1.1 FDD |  |  | - |  | SR.1.1 FDD |  | - |  |
|  |  | Config 2,5 |  |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3,6 |  |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD | - |  |  | R.1.1 FDD |  |  | - |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |  | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD | - |  |  | CCR.1.1 FDD |  |  | - |  | CCR.1.1 FDD |  | - |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  |  |  | CCR2.1 TDD |  |  |  |  | CCR2.1 TDD |  |  |  |
| TRS Configuration |  | Config 1 |  |  | TRS.1.1 FDD | - |  |  | TRS.1.1 FDD |  |  | - |  | TRS.1.1 FDD |  | - |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 |  | s | - |  |  | 4.7 |  | - | 4.7 |  | - |  | 4.7 |  |  |
|  |  | Config 3 |  | s | - |  |  | 2.35 |  | - | 2.35 |  | - |  | 2.35 |  |  |
| CSI-RS configuration for RRM |  | Config 1 |  |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  |  | 0 |  |  | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  |  |  | -106 |  |  |  |  | -116 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |
| Note2 | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -86.27 |  |  |  | -113 |  |  |  |  | -116 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |
| Note2 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  |  |  | -106 |  |  |  |  | -116 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -113.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -83.27 |  |  |  | -110 |  |  |  |  | -113 |  |  |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -112 |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -111 |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -110 |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  |  |
|  |  |  |  | dB | -1.75 |  |  |  | -1.75 |  |  |  |  | 3 |  | -1.75 |  |
|  |  |  |  | dB | -1.75 |  |  |  | -1.75 |  |  |  |  | 3 |  | -1.75 |  |
| SS-RSRP/CSI-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -81.93 | -81.93 |  |  | -107.75 |  |  | -107.75 |  | -113 |  |  | -117.75 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -112.5 |  |  | -117.25 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -112 |  |  | -116.75 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -111.5 |  |  | -116.25 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -111 |  |  | -115.75 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -110.5 |  |  | -115.2 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -110 |  |  | -114.75 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  | -114.25 |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85.02 | -85.02 |  |  | -111.75 |  |  | -111.75 |  | -110 |  |  | -114.75 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -109.5 |  |  | -114.25 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -109 |  |  | -113.75 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -108.5 |  |  | -113.25 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -108 |  |  | -112.75 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -107.5 |  |  | -112.2 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -107 |  |  | -111.75 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -106.5 |  |  | -111.25 |
| SS-RSRQ/CSI-RSRQNote3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 |  |  | -40.59 |  |  | -40.59 |  | 12.56T |  |  | 14.76T |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -50 |  |  |  | -75.83 |  |  |  |  | -83.28 |  |  | -85.83 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -82.78 |  |  | -85.33 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -82.28 |  |  | -84.83 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -81.78 |  |  | -84.33 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -81.28 |  |  | -83.83 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -80.78 |  |  | -83.33 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -80.28 |  |  | -82.83 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -79.78 |  |  | -82.33 |
|  | Config 3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -50 |  |  |  | -76.73 |  |  |  |  | -77.19 |  |  | -79.73 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  | -76.69 |  |  | -79.23 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  | -76.19 |  |  | -78.73 |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  | -75.69 |  |  | -78.23 |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  | -75.19 |  |  | -77.73 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  | -74.69 |  |  | -77.23 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  | -74.19 |  |  | -76.73 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  | -73.69 |  |  | -76.53 |
| Propagation condition |  |  |  | - | AWGN | AWGN |  |  | AWGN |  |  | AWGN |  | AWGN |  | AWGN |  |
| Antenna configuration |  |  |  |  | 1x2 | 1x2 |  |  | 1x2 |  |  | 1x2 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.11.2.3	Test Requirements

The CSI-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.9.2.1 and 10.1.9.2.2.

### A.6.7.12	CSI-SINR

#### A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.12.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.12.2.1.

##### A.6.7.12.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.6.7.12.1.2-1. The absolute accuracy of CSI-SINR intra-frequency measurement is tested by using the parameters in Table A.6.7.12.1.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

Table A.6.7.12.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.12.1.2-2: CSI-SINR Intra frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| TRS configuration |  |  |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |
|  |  |  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | SR.2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |
|  |  |  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  |  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |  |  |  |  |  |
| CSI-RSSI-Measurement |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  | Config 1,2 |  | s | - |  | 2.35 |  | - | 2.35 |  |
|  |  |  | Config 3 |  | s | - |  | 1.17 |  | - | 1.17 |  |
| SSB configuration |  |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| SMTC configuration |  |  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |
|  |  |  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |
| CSI-RS for mobility |  |  |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |
|  |  |  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |
|  |  |  |  | Config 3 |  | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -93 |  |  | -116 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -115.5 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -115 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -114.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -114 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -113.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -113 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -112.5 |  |  |  |
| Note2 | Config 1,2 |  |  |  | dBm/SCS | -93 |  |  | Same as Noc for 15 kHz |  |  |  |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -90 |  |  | -113 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -112.5 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -112 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -111.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -111 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -110.5 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -110 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -109.5 |  |  |  |
|  |  |  |  |  | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  |  |  | dB | 4.54 | 2.66 |  | -4 |  |  | -4 |
| CSI-RSRPNote3 | Config 1,2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -88.46 | -90.34 |  | -120 |  |  | -120 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -119.5 |  |  | -119.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -119 |  |  | -119 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -118.5 |  |  | -118.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -118 |  |  | -118 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -117.5 |  |  | -117.5 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -117 |  |  | -117 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -116.5 |  |  | -116.5 |
|  | Config 3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85.46 | -87.34 |  | -117 |  |  | -117 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -116.5 |  |  | -116.5 |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -116 |  |  | -116 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -115.5 |  |  | -115.5 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -115 |  |  | -115 |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -114.5 |  |  | -114.5 |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -114 |  |  | -114 |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -113.5 |  |  | -113.5 |
| CSI-SINR Note3 |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |
| IoNote3 |  | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.5 |  |  | -85.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -85.01 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -84.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -84.01 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -83.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -83.01 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -82.51 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -82.01 |  |  |  |
|  |  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.41 |  |  | -79.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_B |  |  |  |  | -78.91 |  |  |  |
|  |  |  |  | NR_TDD_FR1_C |  |  |  |  | -78.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  | -77.91 |  |  |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  | -77.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_F |  |  |  |  | -76.91 |  |  |  |
|  |  |  |  | NR_FDD_FR1_G |  |  |  |  | -76.41 |  |  |  |
|  |  |  |  | NR_FDD_FR1_H |  |  |  |  | -75.91 |  |  |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  | - | 1x2 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-SINR, CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.12.1.3	Test Requirements

The CSI-SINR measurement accuracy shall fulfil the requirements in clause 10.1.12.2.1.

#### A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

##### A.6.7.12.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.14.2.1 and 10.1.14.2.2.

##### A.6.7.12.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.6.7.12.2.2-1. Both absolute accuracy and relative accuracy requirements of CSI-SINR inter-frequency measurement are tested by using test parameters in Table A.6.7.12.2.2-2. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A.6.7.12.2.2-1: CSI-SINR Inter frequency CSI-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.7.12.2.2-2: CSI-SINR Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 |  | freq1 |  |  | freq2 |  | freq1 |  | freq2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |  | SR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - |  | R.1.1 FDD |  |  | - |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |  | CCR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 | s | - |  | 2.35 |  | - | 2.35 |  | - |  | 2.35 |  |  |
|  |  | Config 3 | s | - |  | 1.17 |  | - | 1.17 |  | - |  | 1.17 |  |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RS for mobility |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -88 |  |  | -108.5 |  |  |  |  | -119.5 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -119 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -118.5 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -118 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -117.5 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -117 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -116.5 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -116 |  |  |  |
| Note2 | Config 1,2 N |  | dBm/15kHz | -88 |  |  | -108.5 |  |  |  |  | Same as Noc for 15kHz T |  |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85 |  |  | -105.5 |  |  |  |  | -116.5 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -116 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -115.5 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -115 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -114 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -114.5 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -113 |  |  |  |
|  |  |  | dB | -1.75 | -1.75 |  | 15 |  |  | 15 |  | -4.0 |  |  | -4.0 |
|  |  |  | dB | -1.75 |  |  | 15 |  |  |  |  | -4.0 |  |  |  |
| CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -89.75 |  |  | -93.5 |  |  |  |  | -123.5 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -123 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -122.5 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -122 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -121.5 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -121 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -120.5 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -120 |  |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -86.75 |  |  | -90.5 |  |  |  |  | -120.5 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -120 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -119.5 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -119 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -118.5 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -118 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -117.5 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -117 |  |  |  |
| CSI-SINRNote3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -1.75 |  |  | 15 |  |  |  |  | -4.0 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.83 |  |  | -65.4 |  |  |  |  | -90.09 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -89.59 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -89.09 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -88.59 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -88.09 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -87.59 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -87.09 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -86.59 |  |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.73 |  |  | -59.3 |  |  |  |  | -84 |  |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  | -83.5 |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  | -83 |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  | -82.5 |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  | -82 |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  | -81.5 |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  | -81 |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  | -80.5 |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-SINR, CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.6.7.12.2.3	Test Requirements

The CSI-SINR measurement accuracy shall fulfil the requirements in clause 10.1.14.2.1 and 10.1.14.2.2.

### A.6.7.13	RSTD measurements

#### A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer

##### A.6.7.13.1.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.7.13.1.1-1.

Table A.6.7.13.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. Both cells are on the same NR RF channel in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

Table A.6.7.13.1.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  |  | Cell 2 | Cell 1 |  | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 |  |  | Freq1 | freq1 |  | Freq1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  | 2 |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  | 3 |  | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode | 1 |  | FDD |  |  |  | FDD |  |  |
|  | 2 |  | TDD |  |  |  | TDD |  |  |
|  | 3 |  | TDD |  |  |  | TDD |  |  |
| TDD configuration | 1 |  | N/A |  |  |  | N/A |  |  |
|  | 2 |  | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  | 3 |  | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |  |  | - | SR.1.1 FDD |  | - |
|  | 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  | 3 |  | SR.2.1 FDD |  |  |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |  |  | - | CR.1.1 FDD |  | - |
|  | 2 |  | CR.1.1 TDD |  |  | - | CR.1.1 TDD |  | - |
|  | 3 |  | CR.2.1 FDD |  |  | - | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |  |  | - | CCR.1.1 FDD |  | - |
|  | 2 |  | CCR.1.1 TDD |  |  | - | CCR.1.1 TDD |  | - |
|  | 3 |  | CCR.2.1 TDD |  |  | - | CCR.2.1 TDD |  | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  | 2 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  | 3 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  |  |
|  | 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  | 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  |  | - | 3 |  |
|  | 2,3 |  | - | 3 |  |  | - | 3 |  |
| SMTC configuration | 1 |  | SMTC.2 |  |  |  | SMTC.2 |  |  |
|  | 2,3 |  | SMTC.1 |  |  |  | SMTC.1 |  |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  |  |  | PRS.1.2 FR1 |  |  |
|  | 2 |  | PRS.1.1 FR1 |  |  |  | PRS.1.2 FR1 |  |  |
|  | 3 |  | PRS.2.1 FR1 |  |  |  | PRS.2.2 FR1 |  |  |
| PRS muting info | 1~3 |  | ‘10’ | ‘01’ |  |  | ‘10’ | ‘01’ |  |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  |  | N/A | 3 |  |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  |  | N/A | 5 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  |  | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  |  | -98 |  |  |
|  | 3 |  | -95 |  |  |  | -95 |  |  |
|  | 1~3 | dB | -6 |  |  | -13 | -6 |  | -13 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -104 |  |  | -111 | -104 |  | -111 |
|  | 3 |  | -101 |  |  | -108 | -101 |  | -108 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -69.07 |  |  | -69.83 | -69.07 |  | -69.83 |
|  | 3 | dBm/ 38.16MHz | -62.98 |  |  | -63.74 | -62.98 |  | -63.74 |
|  | 1~3 | dB | -6 |  |  | -13 | -6 |  | -13 |
| Propagation condition | 1~3 | - | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

##### A.6.7.13.1.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer

##### A.6.7.13.2.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.6.7.13.2.1-1.

Table A.6.7.13.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR1. Cell 2 is a neighbour cell on a different NR RF channel #2 in FR1. GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

Table A.6.7.13.2.1-2: RSTD accuracy test parameters

| Parameter | Config | Unit | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  |  | Cell 2 | Cell 1 |  | Cell 2 |
| PRS ARFCN | 1~3 |  | freq1 |  |  | freq2 | freq1 |  | freq2 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  | 2 |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  | 3 |  | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |
| Duplex mode | 1 |  | FDD |  |  |  | FDD |  |  |
|  | 2 |  | TDD |  |  |  | TDD |  |  |
|  | 3 |  | TDD |  |  |  | TDD |  |  |
| TDD configuration | 1 |  | N/A |  |  |  | N/A |  |  |
|  | 2 |  | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  | 3 |  | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |  |  | - | SR.1.1 FDD |  | - |
|  | 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  | 3 |  | SR.2.1 FDD |  |  |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |  |  | - | CR.1.1 FDD |  | - |
|  | 2 |  | CR.1.1 TDD |  |  | - | CR.1.1 TDD |  | - |
|  | 3 |  | CR.2.1 FDD |  |  | - | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |  |  | - | CCR.1.1 FDD |  | - |
|  | 2 |  | CCR.1.1 TDD |  |  | - | CCR.1.1 TDD |  | - |
|  | 3 |  | CCR.2.1 TDD |  |  | - | CCR.2.1 TDD |  | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  | 2 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  | 3 |  | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns | 1~3 |  | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration | 1 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  |  |
|  | 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  | 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 | 1 | s | - | 3 |  |  | - | 3 |  |
|  | 2,3 |  | - | 3 |  |  | - | 3 |  |
| SMTC configuration | 1 |  | SMTC.2 |  |  |  | SMTC.2 |  |  |
|  | 2,3 |  | SMTC.1 |  |  |  | SMTC.1 |  |  |
| PRS configuration | 1 |  | PRS.1.1 FR1 |  |  |  | PRS.1.2 FR1 |  |  |
|  | 2 |  | PRS.1.1 FR1 |  |  |  | PRS.1.2 FR1 |  |  |
|  | 3 |  | PRS.2.1 FR1 |  |  |  | PRS.2.2 FR1 |  |  |
| Expected RSTD | 1, 2, 3 | s | N/A | 3 |  |  | N/A | 3 |  |
| Expected RSTD uncertainty | 1, 2, 3 | s | N/A | 5 |  |  | N/A | 5 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  |  | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | 1,2 | dBm/ SCS | -98 |  |  |  | -98 |  |  |
|  | 3 |  | -95 |  |  |  | -95 |  |  |
|  | 1~3 | dB | -6 |  |  | -13 | -6 |  | -13 |
| PRS-RSRPNote3 | 1,2 | dBm/SCS | -104 |  |  | -111 | -104 |  | -111 |
|  | 3 |  | -101 |  |  | -108 | -101 |  | -108 |
| IoNote3 | 1,2 | dBm/ 9.36MHz | -69.07 |  |  | -69.83 | -69.07 |  | -69.83 |
|  | 3 | dBm/ 38.16MHz | -62.98 |  |  | -63.74 | -62.98 |  | -63.74 |
|  | 1~3 | dB | -6 |  |  | -13 | -6 |  | -13 |
| Propagation condition | 1~3 | - | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration | 1~3 |  | 1x2 |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

##### A.6.7.13.2.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

### A.6.7.14	PRS-RSRP measurements

#### A.6.7.14.1	SA: measurement accuracy with PRS in FR1

##### A.6.7.14.1.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.6.7.14.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in table A.6.7.14.1.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in A.6.7.14.1.2-2. In all test cases, Cell 1 is the PCell.

Table A.6.7.14.1.2-1: PRS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.6.7.14.1.2-2: PRS-RSRP test parameters

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  | Cell 2 | Cell 1 | Cell 2 |  |
| Cell ID |  |  |  | 489 |  | 0 | 489 | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  | freq1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | NA | TRS.1.1 FDD | NA |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  | NA | TRS.1.1 TDD | NA |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | NA | TRS.1.2 TDD | NA |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| Measurement gap |  |  |  | GP#24 or GP#0 Note 7 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - | CR.1.1 FDD | - |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| PRS configuration |  | Config 1 |  | PRS.1.2 FR1 |  | PRS.1.2 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |  |
|  |  | Config 2 |  | PRS.1.2 FR1 |  | PRS.1.2 FR1 | PRS.1.2 FR1 | PRS.1.2 FR1 |  |
|  |  | Config 3 |  | PRS.2.2 FR1 |  | PRS.2.2 FR1 | PRS.2.2 FR1 | PRS.2.2 FR1 |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 2 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |  |
| Time offset with Cell 1 |  | Config 1 | ms | - |  | 3 | - | 3 |  |
|  |  | Config 2,3 | s | - |  | 3 | - | 3 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/15KhZ | -106 |  |  | -88 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  | -94 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -106 |  |  | -88 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicableNote 5 |  |  | -91 |  |  |
|  |  |  | dB | 2.46 | -5.97 |  | 2.46 |  | -5.97 |
|  |  |  | dB | 6 | 1 |  | 6 |  | 1 |
| PRS-RSRP Note3 | Config 1, 2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/SCS | -100 |  | -105 | -82 |  | -87 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H |  | Not applicable Note 5 |  | Not applicable Note 5 | -85 |  | -90 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/9.36MHz | -70.09 |  |  | -52.09 |  |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | dBm/38.16MHz | Not applicable Note 5 |  |  | -51.99 |  |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |  |  |  |

##### A.6.7.14.1.3	Test Requirements

The PRS-RSRP measurement accuracy shall fulfil absolute requirement in clause 10.1.24.2.1 and relative requirement in clause 10.1.24.2.2.

### A.6.7.15	UE Rx-Tx time difference measurements

#### A.6.7.15.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA

##### A.6.7.15.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.7.15.1.1-1.

Table A.6.7.15.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.7.15.1.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in Table A.6.7.15.1.2-1.

Table A.6.7.15.1.2-2: SRS configuration for UE Rx-Tx time difference measurement accuracy test

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | 1 |  | 1 |  |
| Measurement gap |  | 1,2,3 | GP#24 or GP#0 Note 4 |  |  |  |
| DRX |  | 1,2,3 | OFF |  |  |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A |  | 3 |  |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  | PRS.2.2 FR1 |  |
| SRS configuration |  | 1 | POS-SRS.1 |  | N/A |  |
|  |  | 2 | POS-SRS.1 |  | N/A |  |
|  |  | 3 | POS-SRS.2 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 | -Infinity | -108 |
|  |  | 2 | -Infinity | -100 | -Infinity | -108 |
|  |  | 3 | -Infinity | -97 | -Infinity | -105 |
| Io | dBm/9.36 MHz | 1 | N/A | -67.67 | N/A | -67.67 |
|  | dBm/9.36 MHz | 2 |  | -67.67 |  | -67.67 |
|  | dBm/38.16 MHz | 3 |  | -61.57 |  | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |

Table A.6.7.15.1.2-2: Void

##### A.6.7.15.1.3	Test requirements

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

## A.7.1	SA: RRC_IDLE state mobility

### A.7.1.1	Cell re-selection to NR

#### A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case

##### A.7.1.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements specified in clause 4.2.2.3.

##### A.7.1.1.1.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.7.1.1.1.2-1, A.7.1.1.1.2-2 and A.7.1.1.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.7.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.7.1.1.1.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| Final condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cell |  | 1, 2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 135 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2 | 35 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.7.1.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
|  |  | 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
|  |  | 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
|  |  | 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  |  | OP.4 |  |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  | 1, 2 | 66 |  |  | 66 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -138 |  |  | -138 |  |  |
|  |  | 2 | -135 |  |  | -135 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
|  | dB | 1 | 8 | -3 | 1.5 | -infinity | 1.5 | -3 |
|  |  | 2 |  |  |  |  |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 8 | -3 | 1.5 | -infinity | 1.5 | -3 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -96 | -91.5 | -infinity | -91.5 | -96 |
|  |  | 2 | -82 | -93 | -88.5 | -infinity | -88.5 | -93 |
| Io on SSB symbols | dBm/95.04 MHz | 1 | -59.37 | -63.40 | -62.47 | -64.01 | -62.47 | -63.40 |
| of each cell |  | 2 | -57.18 | -62.86 | -61.67 | -64.01 | -61.67 | -62.86 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

##### A.7.1.1.1.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than 130 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon cell 1.

The cell re-selection delay to an already detected cell shall be less than 27 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Tevaluate, NR_ intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 4.2.2.3-1 in clause 4.2.2.3

Tevaluate, NR_ intra	See Table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 129.28 s, allow 130 s for the cell re-selection delay to a newly detectable cell and 26.88 s for the cell re-selection delay to an already detected cell in the test case, which we allow 27 s.

#### A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case

##### A.7.1.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.4.

##### A.7.1.1.2.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.7.1.1.2.2-1, A.7.1.1.2.2-2 and A.7.1.1.2.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.7.1.1.2.2-1: Supported test configurations

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.7.1.1.2.2-2: General test parameters for FR2 inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 35 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | 95 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.7.1.1.2.2-3: Cell specific test parameters for FR2 inter frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
|  |  | 2 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  |  | Rough |  |  |
|  | dB | 1 | 10.5 | 10.5 | 8 | -10.5 | -infinity | 8.5 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  | -93 |  |  |
|  |  | 2 | -90 |  |  | -90 |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  | -102 |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 10.5 | 10.5 | 8 | -10.5 | -infinity | 8.5 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -83.5- | -83.5 | -85 | -103.5 | -infinity | -84.5 |
|  |  | 2 | -80.5 | -80.5 | -82 | -100.5 | -infinity | -80.5 |
| Io | dBm/95.04 MHz | 1, 2 | -54.05 | -54.05 | -55.37 | -63.64 | -54.01 | -54.94 |
| Treselection | s | 1, 2 | -54.05 | -54.05 | -55.37 | -63.64 | -54.01 | -54.94 |
| SnonintrasearchP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

##### A.7.1.1.2.3	Test Requirements

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps  on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon cell 2.

The cell re-selection delay to a higher priority cell shall be less than 87 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon cell 1.

The cell re-selection delay to a lower priority cell shall be less than 27 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, NR_ inter	See Table 4.2.2.4-1 in clause 4.2.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 86.88 s, allow 87 s for the cell re-selection delay to a higher priority cell and 26.88 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 27 s.

#### A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

##### A.7.1.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for UE configured with relaxed measurement criterion specified in clause 4.2.2.9.2.

##### A.7.1.1.3.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.7.1.1.3.2-1, A.7.1.1.3.2-2 and A.7.1.1.3.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. During T1 and T2, only criteria lowMobilityEvalutation is configured and fulfilled, where (SrxlevRef – Srxlev) < SSearchDeltaP. UE has not registered with network for the tracking area containing cell2.

Table A.7.1.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.7.1.1.3.2-2: General test parameters for FR2 intra-frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Active cell |  | 1, 2 | Cell1 | The UE reselects to cell 1 during T2 period |
|  | Neighbour cells |  | 1,2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 100 |  |
| T2 |  | s | 1, 2 | 100 |  |

Table A.7.1.1.3.2-3: Cell specific test parameters for FR2 intra-frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| PDSCH RMC |  | 1 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| configuration |  | 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  | OP.4 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |  |
|  |  | 2 | -137 |  | -137 |  |  |
| SSearchDeltaP | dB | 1, 2 | 6 |  | 6 |  |  |
| TSearchDeltaP | s | 1,2 | 5 |  | 5 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  | Setup 1 defined in A.3.15.1 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |  |
|  | dB | 1 | -3 | 1.5 | 1.5 | -3 |  |
|  |  | 2 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  | dB | 1 | -3 | 1.5 | 1.5 | -3 |  |
|  |  | 2 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -96 | -91.5 | -91.5 | -96 |  |
|  |  | 2 | -93 | -88.5 | -88.5 | -93 |  |
| Io on SSB symbols of each cell | dBm/95.04 MHz | 1 | -63.40 | -62.47 | -62.47 | -63.40 |  |
|  |  | 2 | -62.86 | -61.67 | -61.67 | -62.86 |  |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 |  |
| SintrasearchP | dB | 1, 2 | 50 |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.1.1.3.3	Test Requirements

The cell reselection delay to an already detected cell for UE fulfilling low mobility relaxed criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected cell shall be less than 79 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detectable cell can be expressed as: Tevaluate, NR_Intra + TSI-NR,

Where:

Tevaluate, NR_Intra	See Table 4.2.2.9.1-1 in clause 4.2.2.9,

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 78.08 s, allow 79s for the cell re-selection delay to an already detected cell for UE fulfilling low mobility criterion in the test case.

#### A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion

##### A.7.1.1.4.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for UE configured with relaxed measurement criterion specified in clause 4.2.2.9.3.

##### A.7.1.1.4.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.7.1.1.4.2-1, A.7.1.1.4.2-2 and A.7.1.1.4.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. During T1 and T2, only criteria cellEdgeEvaluation is configured and fulfilled, where Srxlev > SSearchThresholdP. UE has not registered with network for the tracking area containing cell2.

Table A.7.1.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.7.1.1.4.2-2: General test parameters for FR2 intra-frequency NR cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1,2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 100 |  |
| T2 |  | s | 1, 2 | 100 |  |

Table A.7.1.1.4.2-3: Cell specific test parameters for FR2 intra-frequency NR cell re-selection test case in AWGN for UE fulfilling not-at-cell edge criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |  |
| PDSCH RMC |  | 1 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| configuration |  | 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| RMC configuration |  | 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  | OP.4 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |  |
|  |  | 2 | -137 |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  | Setup 1 defined in A.3.15.1 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |  |
|  | dB | 1 | -3 | 1.5 | 1.5 | -3 |  |
|  |  | 2 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  | dB | 1 | -3 | 1.5 | 1.5 | -3 |  |
|  |  | 2 |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -96 | -91.5 | -91.5 | -96 |  |
|  |  | 2 | -93 | -88.5 | -88.5 | -93 |  |
| Io on SSB symbols of each cell | dBm/95.04 MHz | 1 | -63.40 | -62.47 | -62.47 | -63.40 |  |
|  |  | 2 | -62.86 | -61.67 | -61.67 | -62.86 |  |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 |  |
| SSearchThresholdP |  | 1, 2 | 35 | 35 | 35 | 35 |  |
| SintrasearchP | dB | 1, 2 | 50 |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.1.1.4.3	Test Requirements

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected cell shall be less than 79 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detected cell can be expressed as: Tevaluate, NR_Intra + TSI-NR,

Where:

Tevaluate, NR_Intra	See Table 4.2.2.9.3-1 in clause 4.2.2.9,

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 78.08 s, allow 79s for the cell re-selection delay to an already detected cell for UE fulfilling  not-at-cell edge criterion in the test case.

#### A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

##### A.7.1.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements for UE fulfilling low mobility criterion specified in clause 4.2.2.10.2.

##### A.7.1.1.5.2	Test Parameters

The test scenario comprises of 2 cells (Cell 1 and Cell 2) on 2 different NR carriers respectively as given in tables A.7.1.1.5.2-1, A.7.1.1.5.2-2 and A.7.1.1.5.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2. Cell 2 is of higher priority than Cell 1. The UE is configured with lowMobilityEvalutation criterion [2].

Table A.7.1.1.5.2-1: Supported test configurations

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.7.1.1.5.2-2: General test parameters for FR2 inter frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell2 and fulfils low mobility (lowMobilityEvalutation [2]) criterion. |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 final condition | Active cell |  | 1, 2 | Cell1 | The UE reselects to low priority cell1 during T1 |
|  | Neighbour cell |  | 1, 2 | Cell2 |  |
| T2 final condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to high priority cell2 during T2 |
|  | Neighbour cell |  |  | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 85 | T1 needs to be long enough to allow cell re-selection to already known cell1 |
| T2 |  | s | 1, 2 | [85] | T2 needs to be long enough to allow cell re-selection to already known cell2 |

Table A.7.1.1.5.2-3: Cell specific test parameters for FR2 inter frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |
|  |  | 2 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |
| Cell_selection_and_reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  | Setup 1 defined in A.3.15.1 |  |
| Beam assumptionNote 4 |  | 1, 2 | Rough |  | Rough |  |
|  | dB | 1, 2 | 8 | [8] | -3 | [8] |
| Note2 | dBm/SCS | 1 | -93 |  | -93 |  |
|  |  | 2 | -90 |  | -90 |  |
| Note2 | dBm/15 kHz | 1, 2 | -102 |  | -102 |  |
|  | dB | 1, 2 | 8 | 8 | -3 | 8 |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -85 | -96 | -85 |
|  |  | 2 | -82 | -82 | -93 | -82 |
| Io | dBm/95.04 MHz | 1 | -55.37 | -55.37 | -62.25 | -55.37 |
|  |  | 2 | -52.37 | -52.37 | -59.25 | -52.37 |
| TreselectionNR | s | 1, 2 | 0 |  | 0 |  |
| SnonintrasearchP | dB | 1, 2 | 50 |  | Not sent |  |
| SSearchDeltaP | dB | 1, 2 | 6 |  | 6 |  |
| TSearchDeltaP | s | 1, 2 | 5 |  | 5 |  |
| Threshx, high | dB | 1, 2 | 48 |  | 48 |  |
| Threshserving, low | dB | 1, 2 | 44 |  | 44 |  |
| Threshx, low | dB | 1, 2 | 50 |  | 50 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

##### A.7.1.1.5.3	Test Requirements

The cell reselection delay to an already detected low priority cell (Cell 1) for UE fulfilling low mobility criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected high priority cell, Cell 2, shall be less than 79 s.

The cell reselection delay to an already detected high priority cell (Cell 2) for UE fulfilling low mobility criterion is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected high priority cell, Cell 2, shall be less than [79] s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE 1:	The cell re-selection delay to an already detected low priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR

NOTE 2:	The cell re-selection delay to an already detected higher priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR

Where:

Tevaluate, NR_ inter	See Table 4.2.2.10.2-1 in clause 4.2.2.10.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 78.08 s, allow 79 s for the cell re-selection delay to an already detected low priority cell for UE fulfilling low mobility criterion in the test case.

This gives a total of 78.08 s, allow 79 s for the cell re-selection delay to an already detected high priority cell for UE fulfilling low mobility criterion in the test case.

#### A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion

##### A.7.1.1.6.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements for UE fulfilling not-at-cell edge criterion specified in clause 4.2.2.10.3.

##### A.7.1.1.6.2	Test Parameters

The test scenario comprises of 2 cells (Cell 1 and Cell 2) on 2 different NR carriers respectively as given in tables A.7.1.1.6.2-1, A.7.1.1.6.2-2 and A.7.1.1.6.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2. Cell 2 is of higher priority than Cell 1. The UE is configured with cellEdgeEvaluation criterion [2].

Table A.7.1.1.6.2-1: Supported test configurations

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.7.1.1.6.2-2: General test parameters for FR2 inter frequency NR cell re-selection test case for UE fulfilling not-at-cell edge criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell2 and fulfils not-at-cell edge (cellEdgeEvaluation [2]) criterion. |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 final condition | Active cell |  | 1, 2 | Cell1 | The UE reselects to low priority cell1 during T1 |
|  | Neighbour cell |  | 1, 2 | Cell2 |  |
| T2 final condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to high priority cell2 during T2 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 85 | T1 needs to be long enough to allow cell re-selection to already known cell. |
| T2 |  | s | 1, 2 | [85] | T2 needs to be long enough to allow cell re-selection to already known cell. |

Table A.7.1.1.6.2-3: Cell specific test parameters for FR2 inter frequency NR cell re-selection test case in AWGN for UE fulfilling not-at-cell edge criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  | -140 |  |
|  |  | 2 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  | Setup 1 defined in A.3.15.1 |  |
| Beam assumptionNote 4 |  | 1, 2 | Rough |  | Rough |  |
|  | dB | 1, 2 | 8 | 8 | -3 | 8 |
| Note2 | dBm/SCS | 1 | -93 |  | -93 |  |
|  |  | 2 | -90 |  | -90 |  |
| Note2 | dBm/15 kHz | 1, 2 | -102 |  | -102 |  |
|  | dB | 1, 2 | 8 | 8 | -3 | 8 |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -85 | -96 | -85 |
|  |  | 2 | -82 | -82 | -93 | -82 |
| Io | dBm/95.04 MHz | 1 | -55.37 | -55.37 | -62.25 | -55.37 |
|  |  | 2 | -52.37 | -52.37 | -59.25 | -52.37 |
| SSearchThresholdP |  | 1, 2 | 35 | 35 | 35 | 35 |
| TreselectionNR | s | 1, 2 | 0 |  | 0 |  |
| SnonintrasearchP | dB | 1, 2 | 50 |  | Not sent |  |
| Threshx, high | dB | 1, 2 | 48 |  | 48 |  |
| Threshserving, low | dB | 1, 2 | 44 |  | 44 |  |
| Threshx, low | dB | 1, 2 | 50 |  | 50 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

##### A.7.1.1.6.3	Test Requirements

The cell reselection delay to an already detected low priority cell (Cell 1) for UE fulfilling not-at-cell edge criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected low priority cell, Cell 1, shall be less than 79 s.

The cell reselection delay to an already detected high priority cell (Cell 2) for UE fulfilling not-at-cell edge criterion is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected high priority cell, Cell 2, shall be less than 79 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE 1:	The cell re-selection delay to an already detected low priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR

NOTE 2:	The cell re-selection delay to an already detected higher priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR

Where:

Tevaluate, NR_ inter	See Table 4.2.2.10.3-1 in clause 4.2.2.10.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

This gives a total of 78.8 s, allow 79 s for the cell re-selection delay to an already detected low priority cell for UE fulfilling  not-at-cell edge criterion in the test case.

This gives a total of 78.08 s, allow 79 s for the cell re-selection delay to an already detected high priority cell for UE fulfilling  not-at-cell edge criterion in the test case.

## A.7.2	SA: RRC_INACTIVE state mobility

## A.7.3	RRC_CONNECTED state mobility

### A.7.3.1	Handover

#### A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell

##### A.7.3.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR2 inter frequency handover requirements specified in clause 6.1.1.5.

##### A.7.3.1.1.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.2.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.1.1.2-2, and A.7.3.1.1.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.7.3.1.1.2-1: Inter-frequency handover from FR1 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.3.1.1.2-2: General test parameters Inter-frequency handover from FR1 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Offset |  | dBm | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.7.3.1.1.2-3: Cell specific test parameters for NR FR1-FR2 Inter frequency handover test case

| Parameter |  |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |  | T1 |  | T2 |
| Assumption for UE beamsNote 6 |  |  |  |  | N/A |  |  | Rough |  |  |
| AoA setup |  |  |  |  | NA |  |  | Setup 1 as defined in A.3.15 |  |  |
| NR RF Channel Number |  |  |  |  | 1 |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  | TDD |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  | TDDConf.3.1 |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  | TDDConf.3.1 |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  | TDDConf.3.1 |  |  |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |  |  |
| BWP BW |  |  | Config 1 | MHz | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 1 |  | 52 |  | 66 |  |  |  |
| Data RBs allocated |  |  | Config 2 |  | 52 |  | 66 |  |  |  |
|  |  |  | Config 3 |  | 106 |  | 66 |  |  |  |
| DRx Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |  | SR3.1 TDD |  |  |  |
|  |  |  | Config 2 |  | SR.1.1 TDD |  | SR3.1 TDD |  |  |  |
|  |  |  | Config 3 |  | SR2.1 TDD |  | SR3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  | CR3.1 TDD |  |  |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  | CR3.1 TDD |  |  |  |
|  |  |  | Config 3 |  | CR2.1 TDD |  | CR3.1 TDD |  |  |  |
| Control Channel RMC |  |  | Config 1 |  | CCR.1.1 FDD |  | CCR.3.1 TDD |  |  |  |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.3.1 TDD |  |  |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  | SSB.3 FR2 |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  | SSB.3 FR2 |  |  |  |
| SMTC configuration |  |  | Config 1,2 |  | SMTC.1 |  | SMTC.1 |  |  |  |
|  |  |  | Config 3 |  | SMTC.2 |  | SMTC.1 |  |  |  |
| SMTC configuration |  |  | Config 1,2 |  | SMTC.1 |  | SMTC.1 |  |  |  |
|  |  |  | Config 3 |  | SMTC.2 |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 kHz |  | 120 kHz |  |  |  |
|  |  |  | Config 3 |  | 30 kHz |  | 120 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 kHz |  | 120 kHz |  |  |  |
|  |  |  | Config 3 |  | 30 kHz |  | 120 kHz |  |  |  |
| PRACH configuration |  |  |  |  | FR1 PRACH configuration 1 |  | FR2 PRACH configuration 1 |  |  |  |
| TRS configuration |  | Config 1 |  |  | TRS.1.1 FDD |  | TRS.2.1 TDD |  |  |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  | TRS.2.1 TDD |  |  |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  |  | N/A |  | TCI.State.2 |  |  |  |
| BWP configuraiton |  |  | Initial DL BWP |  | DLBWP.0.1 |  | DLBWP.0.1 |  |  |  |
|  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  | DLBWP.1.1 |  |  |  |
|  |  |  | Initial UL BWP |  | ULBWP.0.1 |  | ULBWP.0.1 |  |  |  |
|  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | dBm/15kHz | Link only, see clause A.3.7A |  | -104.7 |  |  |  |
| Note2 | Config 1,2 |  |  | dBm/SCS |  |  | -95.7 |  |  |  |
|  | Config 3 |  |  |  |  |  | -95.7 |  |  |  |
|  |  |  |  | dB |  |  | -Infinity |  | 10 |  |
|  |  |  |  | dB |  |  | -Infinity |  | 10 |  |
| IoNote3 | Config 1,2 |  |  | dBm/ BW |  |  | -66.7 |  | -56.3 |  |
|  | Config 3 |  |  | dBm/ BW |  |  | -66.7 |  | -56.3 |  |
| Propagation condition |  |  |  | - |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

##### A.7.3.1.1.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 572 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = [10] ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 562 ms in the test. Tinterrupt is defined in clause 6.1.1.5.2.

This gives a total of 572 ms.

#### A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell

##### A.7.3.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 intra frequency handover requirements specified in clause 6.1.1.4.

##### A.7.3.1.2.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.2.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.1.2.2-2, and A.7.3.1.2.2-3.

The test scenario comprises of carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.7.3.1.2.2-1: Intra-frequency handover from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.1.2.2-2: General test parameters Intra-frequency handover from FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Offset |  | dBm | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.7.3.1.2.2-3: Cell specific test parameters for NR FR2-FR2 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  |  |  | Rough |  |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |  |  |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| Data RBs allocated |  |  |  | 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |  |  |  |  |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |  |  |  |  |  |
|  |  |  | dB | 6 | -1.8 |  | -Infinity |  | 0 |  |
|  |  |  | dB | 6 | 6 |  | -Infinity |  | 7 |  |
| IoNote3 |  |  | dBm/ BW | -59.7 | -56.7 |  | -59.7 |  | -56.7 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

##### A.7.3.1.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 232  ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 222 ms in the test. Tinterrupt is defined in clause 6.1.1.4.2.

This gives a total of 232 ms.

#### A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell

##### A.7.3.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 inter frequency handover requirements specified in clause 6.1.1.4.

##### A.7.3.1.3.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.3.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.1.3.2-2, and A.7.3.1.3.2-3.

The test scenario comprises of carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.7.3.1.3.2-1: Inter-frequency handover from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.1.3.2-2: General test parameters Inter-frequency handover from FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Offset |  | dB | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.7.3.1.3.2-3: Cell specific test parameters for NR FR2-FR2 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  |  |  | Rough |  |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |  |  |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| Data RBs allocated |  |  |  | 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |  | -104.7 |  |  |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |  | -95.7 |  |  |  |
|  |  |  | dB | 5 | 5 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 5 | 5 |  | -Infinity |  | 5 |  |
| IoNote3 |  |  | dBm/ BW | -60.5 | -60.5 |  | -66.7 |  | -60.5 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

##### A.7.3.1.3.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 552 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 542 ms in the test. Tinterrupt is defined in clause 6.1.1.4.2.

This gives a total of 552 ms.

#### A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2

##### A.7.3.1.4.1	Test Purpose and Environment

This test is to verify the requirement for the FR1-to-FR2 Inter-band inter-frequency synchronous DAPS handover requirements specified in clause 6.1.3.4.

##### A.7.3.1.4.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.4.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.1.4.2-2, A.7.3.1.4.2-3 and A.7.3.1.4.2-4.

The test scenario comprises of two bands each with one cell. The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

Before the start of T1, the UE is connected to Cell 1 (source PCell) on radio channel 1 but is not aware of Cell 2 (neighbour cell) on radio channel 2. During T1, the UE shall not have any timing information of Cell 2.

Before the start of T2, the UE in the measurement control information that event-triggered reporting with Event A4 is configured for neighbour cell (Cell 2), and the UE is configured with the measurement gaps (gap pattern ID # 0). Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A4. After receiving the Event A4, the test system shall send a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the test system receives the ACK of the PDSCH corresponding to the last TTI containing the RRC message implying DAPS handover to Cell 2 (target PCell) sent to the UE. During T3, the UE shall be able to perform random access to Cell 2. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perform DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. After the RACH procedure is completed, the test system shall send a RRC message to the UE to release Cell 1 (source cell) on radio channel 1.

The start of T4 is the instant when the test system receives the ACK of the PDSCH corresponding to the last TTI containing the RRC message implying source cell release sent to the UE. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stops to send CSI report to the source cell.

Table A.7.3.1.4.2-1: Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.3.1.4.2-2: General test parameters for Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Threshold |  | dBm | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  | s | 33 | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | <5 |  |
| T3 |  | s | <0.5 |  |
| T4 |  | ms | 10+Tinterrupt2 | Tinterrupt2 as defined in Table 6.1.3.4.2-2 for synchronous DAPS HO |
| T5 |  | ms | 100 |  |

Table A.7.3.1.4.2-3: Cell specific test parameters for Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2 (Cell 1 in FR1)

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| SMTC Configuration |  | Config 1,2 |  | SMTC.1 |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 2 |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | NA Link only, see clause A.3.7A |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS |  |  |  |  |  |
|  | Config 3 |  |  |  |  |  |  |  |
|  |  |  | dB |  |  |  |  |  |
|  |  |  | dB |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz |  |  |  |  |  |
|  | Config 3 |  | dBm/ 38.16MHz |  |  |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.7.3.1.4.2-4: Cell specific test parameters for Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2 (Cell 2 in FR2)

| Parameter |  | Unit | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Assumption for UE beamsNote 6 |  |  | Rough |  |  |  |  |
| AoA setup |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |
| NR RF Channel Number |  |  | 2 |  |  |  |  |
| Duplex mode | Config 1,2,3 |  | TDD |  |  |  |  |
| TDD configuration | Config 1,2,3 |  | TDDConf.3.1 |  |  |  |  |
| BWchannel | Config 1,2,3 | MHz | 100: NRB,c = 66 |  |  |  |  |
| BWP BW | Config 1,2,3 | MHz | 100: NRB,c = 66 |  |  |  |  |
| TRS configuration | Config 1,2,3 |  | TRS.2.1 TDD |  |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel | Config 1,2,3 |  | SR3.1 TDD |  |  |  |  |
| CORESET Reference Channel | Config 1,2,3 |  | CR3.1 TDD |  |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration | Config 1,2,3 |  | SSB.1 FR2 |  |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,3 | kHz | 120 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,3 | kHz | 120 kHz |  |  |  |  |
| PRACH configuration |  |  | FR2 PRACH configuration 2 |  |  |  |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |  |  |  |
| BWP | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 | -104.7 | -104.7 | -104.7 | -104.7 |
| Note2 |  | dBm/SCS | -95.7 | -95.7 | -95.7 | -95.7 | -95.7 |
|  |  | dB | -Infinity | 10 | 10 | 10 | 10 |
|  |  | dB | -Infinity | 10 | 10 | 10 | 10 |
| IoNote3 |  | dBm/ 9.36MHz | -66.7 | -55.4 | -55.4 | -55.4 | -55.4 |
| Propagation condition |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

##### A.7.3.1.4.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 92 ms from the beginning of time period T3. During Dhandover1, the interruption on Cell 1 shall not exceed Tinterrupt1 as defined in Table 6.1.3.4.2-1 for synchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover1 can be expressed as: TRRC_procedure + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 40 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 92 ms.

The UE shall complete to release Cell 1 less than (10 ms + Tinterrupt2) from the beginning of time period T4. During Dhandover2, the interruption on Cell 2 shall not exceed Tinterrupt2 as defined in Table 6.1.3.4.2-2 for synchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

#### A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2

##### A.7.3.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the FR1-to-FR2 Inter-band inter-frequency asynchronous DAPS handover requirements specified in clause 6.1.3.4.

##### A.7.3.1.5.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.5.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.1.5.2-2, A.7.3.1.5.2-3 and A.7.3.1.5.2-4.

The test scenario comprises of two bands each with one cell. The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5 respectively.

Before the start of T1, the UE is connected to Cell 1 (source PCell) on radio channel 1 but is not aware of Cell 2 (neighbour cell) on radio channel 2. During T1, the UE shall not have any timing information of Cell 2.

Before the start of T2, the UE in the measurement control information that event-triggered reporting with Event A4 is configured for neighbour cell (Cell 2), and the UE is configured with the measurement gaps (gap pattern ID # 0). Starting T2, Cell 2 becomes known to the UE. During T2, the UE shall report Event A4. After receiving the Event A4, the test system shall send a RRC message implying DAPS handover to the UE.

The start of T3 is the instant when the test system receives the ACK of the PDSCH corresponding to the last TTI containing the RRC message implying DAPS handover to Cell 2 (target PCell) sent to the UE. During T3, the UE shall be able to perform random access to Cell 2. DL schedule and UL feedback to cell 1 shall be avoided when UE is required to perform DL reception or UL transmission in PRACH procedure in cell 2, except preamble transmission. After the RACH procedure is completed, the test system shall send a RRC message to the UE to release Cell 1 (source cell) on radio channel 1.

The start of T4 is the instant when the test system receives the ACK of the PDSCH corresponding to the last TTI containing the RRC message implying source cell release sent to the UE. During T4, the UE shall perform source cell release.

Starting T5, the UE shall stops to send CSI report to the source cell.

Table A.7.3.1.5.2-1: Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.3.1.5.2-2: General test parameters for Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A4-Threshold |  | dBm | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  | s | 62.5 | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | <5 |  |
| T3 |  | s | <0.5 |  |
| T4 |  | ms | 10+Tinterrupt2 | Tinterrupt2 as defined in Table 6.1.3.4.2-2 for asynchronous DAPS HO. |
| T5 |  | ms | 100 |  |

Table A.7.3.1.5.2-3: Cell specific test parameters for Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2 (Cell 1 in FR1)

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| SMTC Configuration |  | Config 1,2 |  | SMTC.1 |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 2 |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | NA Link only, see clause A.3.7A |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS |  |  |  |  |  |
|  | Config 3 |  |  |  |  |  |  |  |
|  |  |  | dB |  |  |  |  |  |
|  |  |  | dB |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz |  |  |  |  |  |
|  | Config 3 |  | dBm/ 38.16MHz |  |  |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.7.3.1.5.2-4: Cell specific test parameters for Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2 (Cell 2 in FR2)

| Parameter |  | Unit | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Assumption for UE beamsNote 6 |  |  | Rough |  |  |  |  |
| AoA setup |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |
| NR RF Channel Number |  |  | 2 |  |  |  |  |
| Duplex mode | Config 1,2,3 |  | TDD |  |  |  |  |
| TDD configuration | Config 1,2,3 |  | TDDConf.3.1 |  |  |  |  |
| BWchannel | Config 1,2,3 | MHz | 100: NRB,c = 66 |  |  |  |  |
| BWP BW | Config 1,2,3 | MHz | 100: NRB,c = 66 |  |  |  |  |
| TRS configuration | Config 1,2,3 |  | TRS.2.1 TDD |  |  |  |  |
| DRX Cycle |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel | Config 1,2,3 |  | SR3.1 TDD |  |  |  |  |
| CORESET Reference Channel | Config 1,2,3 |  | CR3.1 TDD |  |  |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration | Config 1,2,3 |  | SSB.1 FR2 |  |  |  |  |
| SMTC Configuration |  |  | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,3 | kHz | 120 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2,3 | kHz | 120 kHz |  |  |  |  |
| PRACH configuration |  |  | FR2 PRACH configuration 2 |  |  |  |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |  |  |  |
| BWP | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.3 |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.3 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -104.7 | -104.7 | -104.7 | -104.7 | -104.7 |
| Note2 |  | dBm/SCS | -95.7 | -95.7 | -95.7 | -95.7 | -95.7 |
|  |  | dB | -Infinity | 10 | 10 | 10 | 10 |
|  |  | dB | -Infinity | 10 | 10 | 10 | 10 |
| IoNote3 |  | dBm/ 9.36MHz | -66.7 | -55.4 | -55.4 | -55.4 | -55.4 |
| Propagation condition |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

##### A.7.3.1.5.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 92 ms from the beginning of time period T3. During Dhandover1, the interruption on Cell 1 shall not exceed Tinterrupt1 as defined in Table 6.1.3.4.2-1 for asynchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover1 can be expressed as: TRRC_procedure + TIU + Tprocessing + T∆ + Tmargin, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

TIU = 20 ms in the test. TIU is defined in clause 6.1.1.2.2.

T∆ = 20 ms in the test. T∆ is defined in clause 6.1.1.2.2.

Tprocessing = 40 ms in the test. Tprocessing is defined in clause 6.1.1.2.2.

Tmargin = 2 ms in the test. Tmargin is defined in clause 6.1.1.2.2.

This gives a total of 792 ms.

The UE shall complete to release Cell 1 less than (10 ms + Tinterrupt2) from the beginning of time period T4. During Dhandover2, the interruption on Cell 2 shall not exceed Tinterrupt2 as defined in Table 6.1.3.4.2-2 for asynchronous DAPS HO.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay Dhandover2 can be expressed as: TRRC_procedure + Tinterrupt2, where:

TRRC_procedure = 10 ms and is specified in clause 12 in TS 38.331 [2].

### A.7.3.2	RRC Connection Mobility Control

#### A.7.3.2.1	SA: RRC Re-establishment

##### A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2

A.7.3.2.1.1.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.7.3.2.1.1.1-1, table A.7.3.2.1.1.1-2 and table A.7.3.2.1.1.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure.

Table A.7.3.2.1.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.1.1.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | FR2 PRACH configuration 1 | Table A.3.8.3.1-1 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1 | 5 |  |

Table A.7.3.2.1.1.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
|  | dB | 1 | -0.12 | -infinity | -infinity | -3.46 | 2 | 2 |
| Note2 | dBm/15 kHz | 1 | -104.7 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -95.7 |  |  |  |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | 2 | 2 | 2 |
| SS-RSRP Note3 | dBm/SCS | 1 | -91.7 | -infinity | -infinity | -93.7 | -93.7 | -93.7 |
| Io | dBm/95.04 MHz | 1 | -59.64 | -62.59 | -62.59 | -59.94 | -62.59 | -62.59 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

A.7.3.2.1.1.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell shall be less than 5 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 3520 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 4865 ms, allow 5 s in the test case.

##### A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2

A.7.3.2.1.2.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.7.3.2.1.2.1-1, table A.7.3.2.1.2.1-2 and table A.7.3.2.1.2.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

Table A.7.3.2.1.2.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.1.2.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | FR2 PRACH configuration 1 | Table A.3.8.3.1-1 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133) |
| T3 |  | s | 1 | 6 |  |

Table A.7.3.2.1.2.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| AoA setup |  | 1 | Setup 3 as specified in clause A.3.15 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  | 1 | 24 |  |  | 24 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| Note2 | dBm/15 kHz | 1 | -92.1 |  |  | -92.1 |  |  |
| Note2 | dBm/SCS | 1 | -83.1 |  |  | -83.1 |  |  |
|  | dB | 1 | 0 | -infinity | -infinity | -infinity | -infinity | 0 |
| BB Note 5 | dB | 1 | -1.01 | -infinity | -infinity | -infinity | -infinity | -1.01 |
| SSB_RP Note3 | dBm/SCS | 1 | -83.1 | -infinity | -infinity | -infinity | -infinity | -83.1 |
| Io | dBm/95.04 MHz | 1 | -55.46 | -infinity | -infinity | -infinity | -infinity | -55.46 |
| Propagation Condition |  | 1 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that a constant total transmitted power is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

A.7.3.2.1.2.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than 6 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2

Tidentify_intra_NR = 1600 ms

Tidentify_inter_NR = 2080 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 5025 ms, allow 6 s in the test case.

##### A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing

###### A.7.3.2.1.3.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR2 without serving cell timing is within the specified limits. These tests will verify the requirements in clause 6.2.1.

The test parameters are given in table A.7.3.2.1.3.1-1, table A.7.3.2.1.3.1-2 and table A.7.3.2.1.3.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table A.7.3.2.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.1.3.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 6000 | Radio link failure timer configured by RLF-TimersAndConstants |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | FR2 PRACH configuration 1 | Table A.3.8.3.1-1 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 10.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133) |
| T3 |  | s | 1 | 5 |  |

Table A.7.3.2.1.3.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| Note2 | dBm/15kHz | 1 | -104.7 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -95.7 |  |  |  |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| SS-RSRP Note3 | dBm/SCS | 1 | -90.7 | -infinity | -infinity | -infinity | -infinity | -90.7 |
| Io | dBm/95.04 MHz | 1 | -60.52 | -66.71 | -60.52 | -60.52 | -66.71 | -60.52 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

###### A.7.3.2.1.3.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell without serving cell timing shall be less than 5 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 3520 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 [2] for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 4865 ms, allow 5 s in the test case.

#### A.7.3.2.2	Random Access

##### A.7.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone

A.7.3.2.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.2 and Clause 7.1.2 in an AWGN model.

For this test one cell is used, with the configuration of Cell 1 configured as PCell or SCell in FR2. Supported test parameters are shown in Table A.7.3.2.2.1.1-1. UE capable of SA with PCell or SCell in FR2 needs to be tested by using the parameters in Table A.7.3.2.2.1.1-2 and Table A.7.3.2.2.1.1-3.

Table A.7.3.2.2.1.1-1: Supported test configurations for contention based random access test in FR2 for NR Standalone

| Config | Description |
| --- | --- |
| 1 | NR PSCell/SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.2.1.1-2: General test parameters for contention based random access test in FR2 for NR Standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 | As defined in A.3.10 |
| CSI-RS for tracking |  | Config 1 |  | TRS.2.1 TDD |  |
| Duplex Mode for Cell 1 |  | Config 1 |  | TDD |  |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 | As defined in A.3.1.4 |
| BWchannel |  | Config 1 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated |  | Config 1 |  | 24 |  |
| OCNG Pattern Note 1 |  |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH Reference Channel Note 2 | Config 1 |  |  | SR.3.1 TDD | As defined in A.3.1.1. |
| RMSI CORESET Reference Channel | Config 1 |  |  | CR.3.1 TDD | As defined in A.3.1.2 |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [2]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [19] |
| PRACH Configuration |  |  |  | FR2 PRACH configuration 1 | As defined in A.3.8.3, with exceptions as defined below |
| rsrp-ThresholdSSB |  |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  |  | dBm | -100 | As defined in TS 38.331 [2] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |  |

Table A.7.3.2.2.1.1-3: OTA-related test parameters for contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | As defined in A.3.15.1 |
| Assumption for UE beamsNote 3 |  |  | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 |  |
|  | Es/IotBB | dB | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 |  |
|  | Es/IotBB | dB | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN |  |
| Note 1: No articial noise is applied in this test. Note 2: Void. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

A.7.3.2.2.1.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.7.3.2.2.1.2.1	Random Access Preamble Transmission

To test the UE behavior specified in Clause 6.2.2.2.1.1 the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.1.2.2	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2.2.1.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.1.2.3	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2.2.1.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 2 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.1.2.4	Receiving an UL grant for msg3 retransmission

To test the UE behavior specified in clause 6.2.2.2.1.4 the System Simulator shall provide an UL grant for msg3 retransmission following a successful Random Access Response.

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

A.7.3.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

A.7.3.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

The UE shall send ACK if the Contention Resolution is successful.

A.7.3.2.2.1.2.7	Contention Resolution Timer expiry

To test the UE behavior specified in Clause 6.2.2.2.1.6 the System Simulator shall not send a response to a msg3.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

##### A.7.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone

A.7.3.2.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.2 and Clause 7.1.2 in an AWGN model.

For this test one cell is used, with the configuration of Cell 1 configured as PCell or SCell in FR2. Supported test parameters are shown in Table A.7.3.2.2.2.1-1. UE capable of SA with PCell or SCell in FR2 needs to be tested by using the parameters in Table A.7.3.2.2.2.1-2 and Table A.7.3.2.2.2.1-3 for SSB-based non-contention based random access test (Test 1) and CSI-RS-based non-contention based random access test (Test 2). Test 2 is only applicable to UE which supports csi-RSRP-AndRSRQ-MeasWithSSB or csi-RSRP-AndRSRQ-MeasWithoutSSB.

Table A.7.3.2.2.2.1-1: Supported test configurations for non-contention based random access test in FR2 for NR Standalone

| Config | Description |
| --- | --- |
| 1 | NR PSCell/SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.2.2.1-2: General test parameters for non-contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | SSB.1 FR2 | As defined in A.3.10 |
| CSI-RS for tracking | Config 1 |  | TRS.2.1 TDD | TRS.2.1 TDD |  |
| CSI-RS Configuration | Config 1 |  | N/A | CSI-RS.3.1 TDD | As defined in A.3.1.4 |
| Duplex Mode for Cell 2 | Config 1 |  | TDD | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | TDDConf.3.1 |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |  |
| Data RBs allocated | Config 1 |  | 24 | 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | OP.3 | As defined in A.3.2.1. |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR3.1 TDD | SR3.1 TDD | As defined in A.3.1.1. |
| NR RF Channel Number |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | +20 +ΔUL | As defined in TS 38.331 [2]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [19] |
| PRACH Configuration |  |  | FR2 PRACH configuration 2 | FR2 PRACH configuration 3 | As defined in A.3.8.3, with exceptions as defined below. |
| rsrp-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | -100 | As defined in TS 38.331 [2] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |  |

Table A.7.3.2.2.2.1-3: OTA-related test parameters for non-contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | Setup 1 | As defined in A.3.15.1 |
| Assumption for UE beamsNote 3 |  |  | Rough | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 | -80.6 |  |
|  | Es/IotBB | dB | 21.09 | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 | -95.0 |  |
|  | Es/IotBB | dB | 6.69 | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN | AWGN |  |
| Note 1: No articial noise is applied in this test. Note 2: void. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

A.7.3.2.2.2.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.7.3.2.2.2.2.1	SSB-based Random Access Preamble Transmission

In Test-1, to test the UE behavior specified in Clause 6.2.2.2.2.1 for SSB-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the Random Access Preamble which has the Preamble Index associated with the SSB with index 0.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.2.2.2	CSI-RS-based Random Access Preamble Transmission

In Test-1, to test the UE behavior specified in Clause 6.2.2.2.2.1 for CSI-RS-based Random Access Preamble tranmsision, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs configured, the System Simulator shall receive the Random Access Preamble which has the Preamble Index associated with the CSI-RS configured.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and the selected PRACH occasion shall belongs to the PRACH occassions permitted by the restrictions given by the ra-OccasionList.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.2.2.3	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2.2.2.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for Random Access Response(s) if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.2.2.4	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2.2.2.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 2 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2.2. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone

A.7.3.2.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the 2-step RA type random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.3 and Clause 7.1.2 in an AWGN model.

For this test one cell is used, with the configuration of Cell 1 configured as PCell or SCell in FR2. Supported test parameters are shown in Table A.7.3.2.2.3.1-1. UE capable of SA with PCell or SCell in FR2 needs to be tested by using the parameters in Table A.7.3.2.2.3.1-2 and Table A.7.3.2.2.3.1-3.

Table A.7.3.2.2.3.1-1: Supported test configurations for 2-step RA type contention based random access test in FR2 for NR Standalone

| Config | Description |
| --- | --- |
| 1 | NR PSCell/SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.2.3.1-2: General test parameters for 2-step RA type contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | As defined in A.3.10 |
| Duplex Mode for Cell 1 | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | As defined in A.3.1.4 |
| BWchannel | Config 1 | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR.3.1 TDD | As defined in A.3.1.1. |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | As defined in A.3.1.2 |
| NR RF Channel Number |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [2]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [19] |
| MsgA Configuration |  |  | FR2 MsgA configuration 1 | As defined in A.3.20.3, with exceptions as defined below |
| msgA-RSRP-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | As defined in TS 38.331 [2] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PMsgA0 -1), where PMsgA0 is the measured first MsgA PRACH power with -80.6dBm/SCS applied, msgA-PreambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send MsgA. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table A.7.3.2.2.3.1-3: OTA-related test parameters for 2-step RA type contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 2b | As defined in A.3.15.1 |
| Assumption for UE beamsNote 2 |  |  | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 |  |
|  | Es/IotBB | dB | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 |  |
|  | Es/IotBB | dB | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN |  |
| Note 1: No articial noise is applied in this test. Note 2: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

A.7.3.2.2.3.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.7.3.2.2.3.2.1	MsgA Transmission

To test the UE behavior specified in Clause 6.2.2.3.1.1 the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured msgA-RSRP-ThresholdSSB.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first MsgA preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.3.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2.3.1.2 the System Simulator shall transmit a MsgB containing a fallbackRAR message and a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for MsgB(s) and shall transmit the msg3 if the MsgB with a fallbackRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit MsgA with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB’s contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first MsgA PRACH shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.3.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2.3.1.3 the System Simulator shall transmit a MsgB containing a fallbackRAR message and Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 2 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if no MsgB is received within the MsgB Response window.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first MsgA PRACH shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.7.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone

A.7.3.2.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in Clause 6.2.2.3 and Clause 7.1.2 in an AWGN model.

For this test one cell is used, with the configuration of Cell 1 configured as PCell or SCell in FR2. Supported test parameters are shown in Table A.7.3.2.2.4.1-1. UE capable of SA with PCell or SCell in FR2 needs to be tested by using the parameters in Table A.7.3.2.2.4.1-2 and Table A.7.3.2.2.4.1-3.

Table A.7.3.2.2.4.1-1: Supported test configurations for non-contention based random access test for 2-step RA type in FR2 for NR Standalone

| Config | Description |
| --- | --- |
| 1 | NR PSCell/SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.2.4.1-2: General test parameters for non-contention based random access test for 2-step RA type in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | As defined in A.3.10 |
| Duplex Mode for Cell 2 | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | As defined in A.3.2.1. |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR3.1 TDD | As defined in A.3.1.1. |
| NR RF Channel Number |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [2]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power (PCMAX,f,c) |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [19] |
| MsgA Configuration |  |  | FR2 MsgA configuration 2 | As defined in A.3.20.3, with exceptions as defined below. |
| msgA-RSRP-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| msgA-PreambleReceivedTargetPower |  | dBm | -100 | As defined in TS 38.331 [2] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PMsgA0 -1), where PMsgA0 is the measured first MsgA PRACH power with -80.6dBm/SCS applied, msgA-PreambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send MsgA. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table A.7.3.2.2.4.1-3: OTA-related test parameters for non-contention based random access test for 2-step RA type in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | As defined in A.3.15.1 |
| Assumption for UE beams Note 2 |  |  | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 |  |
|  | Es/IotBB | dB | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 |  |
|  | Es/IotBB | dB | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

A.7.3.2.2.4.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.7.3.2.2.4.2.1	MsgA Transmission

In Test-1, to test the UE behavior specified in Clause 6.2.2.3.2.1 for MsgA transmission, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the MsgA which has the Preamble Index associated with the SSB with index 0.

In addition, the System Simulator shall receive the MsgA on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belongs to the PRACH occasions permitted by the restrictions given first by the msgA-SSB-SharedRO-MaskIndex if configured, or next by the ra-ssb-OccasionMaskIndex if configured.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.4.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2.3.2.2 the System Simulator shall transmit a MsgB containing a successRAR MAC subPDU corresponding to the transmitted Random Access Preamble after 3 MsgA transmissions have been received by the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble.

The UE may stop monitoring for MsgB if the MsgB contains a successRAR MAC subPDU corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA transmission power if all received Random Access Response Reception has not been considered as successful.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.7.3.2.2.4.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2.3.2.3 the System Simulator shall transmit a MsgB corresponding to the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 2 preambles.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA transmission power when the backoff time expires if no MsgB is received within the MsgB Response window configured in RACH-ConfigGenericTwoStepRA.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2.3. The power of the first preamble shall be 0.6 dBm to be received at TE with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-2 [19], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-2 [19].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

#### A.7.3.2.3	SA: RRC Connection Release with Redirection

##### A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2

A.7.3.2.3.1.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR to NR requirements specified in clause 6.2.3.2.1.

A.7.3.2.3.1.2	Test Parameters

Supported test configurations are shown in table A.7.3.2.3.1.2-1. The time delay is tested by using the parameters in table A.7.3.2.3.1.2-2, and A.7.3.2.3.1.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.7.3.2.3.1.2-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.3.1.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 3.2 |  |

Table A.7.3.2.3.1.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  |  |  | Rough |  |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |  |  |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| Data RBs allocated |  |  |  | 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 FR2 |  |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |  | -104.7 |  |  |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |  | -95.7 |  |  |  |
|  |  |  | dB | 5 | 5 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 5 | 5 |  | -Infinity |  | 5 |  |
| IoNote3 |  |  | dBm/ BW | -60.5 | -60.5 |  | -66.7 |  | -60.5 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

A.7.3.2.3.1.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 3160 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

where:

TRRC_procedure_delay = 110 ms in the test.

Tidentify-NR = 1760 ms in the test.

TSI-NR = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH = 10 ms in the test.

This gives a total of 3160 ms.

### A.7.3.3	Conditional Handover

#### A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2

##### A.7.3.3.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 intra frequency conditional handover requirements specified in clause 6.1.4.4.

##### A.7.3.3.1.2	Test Parameters

Supported test configurations are shown in table A.7.3.3.2.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.3.2.2-2, and A.7.3.3.2.2-3.

The test scenario comprises of two cells. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. NR shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2. Starting T2, cell 2 becomes detectable.

Table A.7.3.3.1.2-1: Intra-frequency conditional handover from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.3.1.2-2: General test parameters for conditional Intra-frequency handover from FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset for condition |  | dBm | -1 | Trigger HO to cell which may be measured as -1dB relative to cell 1. Actual SS-RSRP is 5dB stronger. |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.7.3.3.1.2-3: Cell specific test parameters for NR FR2-FR2 conditional Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |  |  |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  |  |  |  |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB.1 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| TCI configuration |  |  |  | CSI-RS.Config.0 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |  | -104.7 |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -98.7 |  |  | -98.7 |  |  |  |
|  |  |  | dB | 6 | -5.33 |  | -Infinity |  | 4.02 |  |
|  |  |  | dB | 6 | 6 |  | -Infinity |  | 11 |  |
| IoNote3 | Config 1 |  | dBm/ BW | -62.7 | -57.2 |  | -62.7 |  | -57.2 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

##### A.7.3.3.1.2.3	Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution =1600+62+10=1672 ms (power class 1) or 1080+62+10 =1152 (PC2/3/4) 62 ms=1152 ms (power classes 2,3 and 4) from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms excluding any transmissions which do not occur due to scheduling restrictions.

#### A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell

##### A.7.3.3.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 inter frequency conditional handover requirements specified in clause 6.1.4.4.

##### A.7.3.3.2.2	Test Parameters

Supported test configurations are shown in table A.7.3.3.3.2-1. Both conditional handover delay and interruption length are tested by using the parameters in table A.7.3.3.3.2-2, and A.7.3.3.3.2-3.

The test scenario comprises of two carriers and one cell on each carrier. Gap pattern ID gp0 is configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. NR shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2.  At the start of T2, cell 2 becomes detectable and meets the handover condition.

Table A.7.3.3.2.2-1: Inter-frequency conditional handover from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.3.2.2-2: General test parameters Inter-frequency conditional handover from FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset for handovercondition |  | dB | FFS |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 7 |  |

Table A.7.3.3.2.2-3: Cell specific test parameters for NR FR2-FR2 Inter frequency conditional handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |  |  |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  |  |  |  |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| Gap pattern ID |  |  |  | gp0 |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB.1 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| TCI configuration |  |  |  | CSI-RS.Config.0 |  |  |  |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |  | -104.7 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -95.7 |  |  | -95.7 |  |  |  |
|  | Config 3 |  |  | -95.7 |  |  | -95.7 |  |  |  |
|  |  |  | dB | 5 | 5 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 5 | 5 |  | -Infinity |  | 5 |  |
| IoNote3 | Config 1,2 |  | dBm/ BW | -60.5 | -60.5 |  | -66.7 |  | -60.5 |  |
|  | Config 3 |  | dBm/ BW | -60.5 | -60.5 |  | -66.7 |  | -60.5 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

##### A.7.3.3.2.3 Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution = 6720+62+10ms=6792 ms (power class 1) or 4160+62+10ms =4232ms (power classes 2,3 and 4) from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms excluding any transmissions which do not occur due to scheduling restrictions. excluding any transmissions which do not occur due to scheduling restrictions.

## A.7.4	Timing

### A.7.4.1	UE transmit timing

#### A.7.4.1.1	NR UE Transmit Timing Test for FR2

##### A.7.4.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table 7.4.1.1.1-1.

Table A.7.4.1.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |

For this test a single NR cell is used. Tables A.7.4.1.1.1-2 and A.7.4.1.1.1-2A define the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.4.1.1.1-3.

Table A.7.4.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  |  | Test2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1 | Freq1 |  |  | Freq1 |  |
| TDD configuration |  | 1 |  | TDDConf.3.1 |  |  |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |  |  |  |
| Data RBs allocated |  | 1 | 66 |  |  |  |  |
| Initial BWP Configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  | 1 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  |  |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  |  |  |
| DRx Cycle | ms | 1 | N/A |  |  |  | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1 | SR.3.3 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  | 1 | CR.3.2 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.3.7 TDD |  |  |  |  |
| OCNG Patterns |  | 1 | OP.1 |  |  |  |  |
| SSB Configuration |  | 1 | SSB.4 FR2 |  |  |  |  |
| SMTC Configuration |  | 1 | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 | 120 |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Propagation condition |  | 1 | AWGN |  |  |  |  |
| SRS Config |  | 1 | SRSConf.1Note6 |  | SRSConf.2Note6 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.7.4.1.1.1-3 |  |  |  |  |  |  |  |

Table A.7.4.1.1.1-2A: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |
| Assumption for UE beamsNote 6 |  | Fine |  |
| Note1 | dBm/15kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -100 |  |
|  | dB | 4 |  |
| SSB_RPNote2 | dBm/SCS Note4 | -96 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table A.7.4.1.1.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 17 | 17 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl2560, 4 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

Table A.7.4.1.1.1-4: Void

##### A.7.4.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test:

1)	Setup NR PCell according to parameters given in Table A.7.4.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 13792

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.7.4.1.1.2-1

Table A.7.4.1.1.2-1 Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 240 | +8*64Tc | +4*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first detected path (in time) of DL SSB.  Skip this step for test 2 with DRX confiured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.4.2	UE timer accuracy

### A.7.4.3	Timing advance

#### A.7.4.3.1	SA FR2 timing advance adjustment accuracy

##### A.7.4.3.1.1	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment delay and accuracy requirement defined in clause 7.3.

##### A.7.4.3.1.2	Test Parameters

Supported test configurations are shown in table A.7.4.3.1.2-1. Both timing advance adjustment delay and accuracy are tested by using the parameters in table A.7.4.3.1.2-2, A.7.4.3.1.2-3 and A.7.4.3.1.2-4.

In all test cases, single cell is used. Each test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.4.3.1.2-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in clause 6.1.3.4 in TS 38.321 [7]. The Timing Advance Command value shall be set to 31, which according to clause 4.2 in TS 38.213 [3] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.4.3.1.2-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at slot n+k for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321 [7], shall be configured so that it does not expire in the duration of the test.

Table A.7.4.3.1.2-1: Timing advance supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.4.3.1.2-2: General test parameters for timing advance

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.3.9.2.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.3.9.2.2-1 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.3.9.3.1-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.3.9.3.2-1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For 120 kHz SCS NTA_new = NTA_old + 1024*Tc (based on equation in clause 4.2 of TS 38.213 [3]) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.4.3.1.2-3: Cell specific test parameters for timing advance

| Parameter | Unit | Test1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Duplex mode |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |
| BWP BW | MHz | 100: NRB,c = 66 |  |
| DRx Cycle | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  |
| CORESET Reference Channel |  | CR.3.1 TDD |  |
| OCNG Patterns |  | OCNG pattern 1 |  |
| TRS configuration |  | TRS.2.1 TDD |  |
| PDSCH/PDCCH TCI state |  | TCI.State.2 |  |
| SMTC configuration |  | SMTC.1 FR2 |  |
| SSB Configuration |  | SSB.3 FR2 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 kHz |  |
| PUCCH/PUSCH subcarrier spacing | kHz | 120 kHz |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Propagation condition | - | AWGN |  |
| Note 1: OCNG shall be used such that the resources in the cell in this test are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone |  |  |  |

Table A.7.4.3.1.2-3A: OTA related test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |
| Assumption for UE beamsNote 6 |  | Fine |  |
| Note1 | dBm/15kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -103 |  |
|  | dB | 4 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -99 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table A.7.4.3.1.2-4: Sounding Reference Symbol Configuration for timing advance

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 16 | Frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=0 | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | Codebook | Codebook based UL transmission |
| startPosition | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

##### A.7.4.3.1.3 Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. k+1 slots after the reception of the timing advance command, where k = 11.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

## A.7.5	Signaling characteristics

### A.7.5.1	Radio link Monitoring

In the following clause, any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

Editor note: The metric for the detection of the UE UL transmitted signal by the TE is FFS.

#### A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

##### A.7.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.7.5.1.1.1-1. The test parameters are given in Tables A.7.5.1.1.1-2, A.7.5.1.1.1-3, and A.7.5.1.1.1-4 below. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.5.1.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states, and Figure A.7.5.1.1.1-2 shows the Time multiplexed downlink transmissions from each Angle of Arrival. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In addition to RLM-RS radio link monitoring using SSB index 0 and SSB index 1, the UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 1.

Table A.7.5.1.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table A.7.5.1.1.1-2: General test parameters for FR2 out-of-sync testing in non-DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 |  | 100: NRB,c = 66 |
| Data RBs allocated |  | Config 1 |  | 24 |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.3.4 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration |  | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz |
| PRACH Configuration |  | Config 1 |  | Table A.3.8.3.4 |
| SSB index assigned as RLM RS |  | Config 1 |  | 0,1 |
| OCNG parameters |  |  |  | OP.5 |
| CP length |  |  |  | Normal |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI reporting |  | Config 1 |  | CSI-RS.3.1 TDD |
| TCI states for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| CSI-RS for tracking |  | Config 1 |  | TRS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 9.68 |
| T3 |  |  | s | 9.68 |
| D1 |  |  | s | 9.64 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.7.5.1.1.1-3: OTA related cell specific test parameters for FR2 (Cell 1) for out-of-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  |  | Setup 3 defined in A.3.15 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| Assumption for UE beams Note 5 |  |  | Rough |  |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  | Not sent |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| ssb-Index 0 SNR | Config 1 | dB | 2Note 6 | -6Note 6 | -15 |  |  |  |
| ssb-Index 1 SNR | Config 1 |  | Not sent |  |  | 2Note 6 | -15 | -15 |
|  | Config 1 | dBm/ 15kHz | -92.1 |  |  | -92.1 |  |  |
| Time multiplexing of the downlink transmissions from each AoA |  |  | Defined in Figure A.7.5.1.1.1-2 |  |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  | TDL-A 30ns 75Hz |  |  |
| Note 1: OCNG shall be used such that and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 6: This value allows up to 1dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |  |

Table A.7.5.1.1.1-4: Measurement gap configuration for out-of-sync tests in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |

Figure A.7.5.1.1.1-1: SNR variation for out-of-sync testing

Figure A.7.5.1.1.1-2: Time multiplexed downlink transmissions

##### A.7.5.1.1.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

##### A.7.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.7.5.1.2.1-1.The test parameters are given in Tables A.7.5.1.2.1-2, and A.7.5.1.2.1-3 below. There is one cell (Cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states, and Figure A.7.5.1.2.1-2 shows the Time multiplexed downlink transmissions from each Angle of Arrival. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms.

Table A.7.5.1.2.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table A.7.5.1.2.1-2: General test parameters for FR2 in-sync testing in non-DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 |  | 100: NRB,c = 66 |
| Data RBs allocated |  | Config 1 |  | 24 |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.3.1 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration |  | Config 1 |  | SMTC.3 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz |
| PRACH Configuration |  | Config 1 |  | Table A.3.8.3.4 |
| SSB index assigned as RLM RS |  | Config 1 |  | 0,1 |
| OCNG parameters |  |  |  | OP.5 |
| CP length |  |  |  | Normal |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 4000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI reporting |  | Config 1 |  | CSI-RS.3.1 TDD |
| TCI states for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| CSI-RS for tracking |  | Config 1 |  | TRS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 1.88 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 3.84 |
| D1 |  |  | s | 3.8 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.7.5.1.2.1-3: OTA related cell specific test parameters for FR2 (Cell 1) for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 3 defined in A.3.15 |  |  |  |  |  |  |  |  |  |
|  |  |  | AoA1 |  |  |  |  | AoA2 |  |  |  |  |
| Assumption for UE beams Note 5 |  |  | Rough |  |  |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  | Not sent |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| ssb-Index 0 SNR | Config 1 | dB | 2Note 6 | -6Note 6 | -15 | -4.5 | 2Note 6 |  |  |  |  |  |
| ssb-Index 1 SNR | Config 1 |  | Not sent |  |  |  |  | 2Note 6 | -15 | -15 | -15 | -15 |
|  | Config 1 | dBm/ 15kHz | -92.1 |  |  |  |  | -92.1 |  |  |  |  |
| Time multiplexing of the downlink transmissions from each AoA |  |  | Defined in Figure A.7.5.1.2.1-2 |  |  |  |  |  |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 6: This value allows up to 1dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.1.2.1-4: Void

Figure A.7.5.1.2.1-1: SNR variation for in-sync testing

Figure A.7.5.1.2.1-2: Time multiplexed downlink transmissions

##### A.7.5.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode

##### A.7.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.7.5.1.3.1-1. The test parameters are given in Tables A.7.5.1.3.1-2, and A.7.5.1.3.1-3. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.5.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.5.1.3.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table A.7.5.1.3.1-2: General test parameters for FR2 out-of-sync testing in DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 |  | 100: NRB,c = 66 |
| Data RBs allocated |  | Config 1 |  | 66 |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.3.4 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration |  | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz |
| PRACH Configuration |  | Config 1 |  | Table A.3.8.3.4 |
| SSB index assigned as RLM RS |  | Config 1 |  | 0,1 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI reporting |  | Config 1 |  | CSI-RS.3.1 TDD |
| TCI states for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| CSI-RS for tracking |  | Config 1 |  | TRS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 14.48 |
| T3 |  |  | s | 14.48 |
| D1 |  |  | s | 14.44 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.7.5.1.3.1-3: OTA related cell specific test parameters for FR2 (Cell 1) for out-of-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |
| Assumption for UE beams Note 5 |  |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| ssb-Index 0 SNR | Config 1 | dB | 2Note 6 | -6Note 6 | -15 |
| ssb-Index 1 SNR | Config 1 |  | 2Note 6 | -15 | -15 |
|  | Config 1 | dBm/15KHz | -104.7dBm |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 6: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |

Table A.7.5.1.3.1-4: Void

Table A.7.5.1.3.1-5: Void

Figure A.7.5.1.3.1-1: SNR variation for out-of-sync testing

##### A.7.5.1.3.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode

##### A.7.5.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR2 radio link monitoring requirements in clause 8.1.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.7.5.1.4.1-1. The test parameters are given in Tables A.7.5.1.4.1-2, and A.7.5.1.4.1-3. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.1.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.5.1.4.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 120 KHz, data SCS 120KHz, BW 100 MHz |

Table A.7.5.1.4.1-2: General test parameters for FR2 in-sync testing in DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 |  | 100: NRB,c = 66 |
| Data RBs allocated |  | Config 1 |  | 66 |
| DL initial BWP configuration |  | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | TDDConf.3.1 |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.3.1 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration |  | Config 1 |  | SMTC.3 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 120 KHz |
| PRACH Configuration |  | Config 1 |  | Table A.3.8.3.4 |
| SSB index assigned as RLM RS |  | Config 1 |  | 0,1 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| In sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 0 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.11 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 4000 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS for CSI reporting |  | Config 1 |  | CSI-RS.3.1 TDD |
| TCI states for PDCCH/PDSCH |  |  |  | TCI.State.2 |
| CSI-RS for tracking |  | Config 1 |  | TRS.2.1 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.2 |
| T3 |  |  | s | 2.8 |
| T4 |  |  | s | 0.2 |
| T5 |  |  | s | 3.88 |
| D1 |  |  | s | 3.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.7.5.1.4.1-3: OTA related cell specific test parameters for FR2 (Cell 1) for in-sync radio link monitoring test in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 5 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| ssb-Index 0 SNR | Config 1 | dB | 2Note 6 | -6Note 6 | -15 | -4.5 | 2Note 6 |
| ssb-Index 1 SNR | Config 1 |  | 2Note 6 | -15 | -15 | -15 | -15 |
|  | Config 1 | dBm/15KHz | -104.7dBm |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG.3 Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 6: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |

Table A.7.5.1.4.1-4: Void

Table A.7.5.1.4.1-5: Void

Figure A.7.5.1.4.1-1: SNR variation for in-sync testing

##### A.7.5.1.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode

##### A.7.5.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR2 PCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.7.5.1.5.1-1, A.7.5.1.5.1-2, A.7.5.1.5.1-3 and A.7.5.1.5.1-4 below. There is one cell, cell 1 which is the PCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.5.1.5.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 10 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test. In the test, SSB0 and SSB1 are configured as BFD-RS.

Table A.7.5.1.5.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table A.7.5.1.5.1-2: General test parameters for FR2 PCell for CSI-RS out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.3.4 TDD CCR.3.6 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.2 |
| CP length |  |  | Normal |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | *gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.35 |
| T3 |  | s | 0.35 |
| D1 |  | s | 0.31 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.7.5.1.5.1-3: Cell specific test parameters for FR2 for CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  |  | Setup 3 defined in A.3.15 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  | Not sent |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR on RLM-RS1 | Config 1 | dB | 2Note 11 | -6Note 11 | -15 |  |  |  |
| SNR on RLM-RS2 | Config 1 |  | Not sent |  |  | 2Note 11 | -14 | -15 |
|  | Config 1 | dBm/ 15kHz | -92.1 |  |  | -92.1 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.1.5.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |  |

Table A.7.5.1.5.1-4: Measurement gap configuration for FR2 CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: RLM RS is partially overlapped with measurement gap |  |

Figure A.7.5.1.5.1-1: SNR variation for CSI-RS out-of-sync testing

##### A.7.5.1.5.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

The UE shall stop transmitting uplink signal in Cell 1 no later than time point C (D1 second after the start of the time duration T3) on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode

##### A.7.5.1.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR2 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.7.5.1.6.1-1, A.7.5.1.6.1-2 and A.7.5.1.6.1-3 below. There is one cells, cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.1.6.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 10 ms. In the test, DRX configuration is not enabled. In the test, SSB0 and SSB1 are configured as BFD-RS.

Table A.7.5.1.6.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table A.7.5.1.6.1-2: General test parameters for FR2 PCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD CCR.3.3 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.2 |
| CP length |  |  | Normal |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 1000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 0.24 |
| T4 |  | s | 0.2 |
| T5 |  | s | 0.88 |
| D1 |  | s | 0.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.7.5.1.6.1-3: Cell specific test parameters for FR2 for CSI-RS in-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 3 defined in A.3.15 |  |  |  |  |  |  |  |  |  |
|  |  |  | AoA1 |  |  |  |  | AoA2 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  | Not sent |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |  |  |  |  |
| SNR on RLM-RS1 | Config 1 | dB | 2Note 11 | -6Note 11 | -15 | -4.5 | 2Note 11 |  |  |  |  |  |
| SNR on RLM-RS2 | Config 1 |  | Not sent |  |  |  |  | 2Note 11 | -14 | -15 | -15 | -14 |
|  | Config 1 | dBm/ 15KHz | -92.1 |  |  |  |  | -92.1 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.5.1.6.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.7.5.1.6.1-1: SNR variation for CSI-RS in-sync testing

##### A.7.5.1.6.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode

##### A.7.5.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR2 PCell CSI-RS Out-of-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.7.5.1.7.1-1, A.7.5.1.7.1-2, and A.7.5.1.7.1-3 below. There is one cell, cell 1 is the PCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.5.1.7.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 10 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. In the test, SSB0 and SSB1 are configured as BFD-RS.

Table A.7.5.1.7.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table A.7.5.1.7.1-2: General test parameters for FR2 PCell for CSI-RS out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.3.4 TDD CCR.3.6 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 1.28 |
| T3 |  | s | 1.28 |
| D1 |  | s | 1.24 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.7.5.1.7.1-3: Cell specific test parameters for FR2 for CSI-RS out-of-sync radio link monitoring in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| AoA setup |  | dB | Setup 1 defined in A.3.15 |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS1 | Config 1 | dB | 2Note 11 | -6Note 11 | -15 |
| SNR on RLM-RS2 | Config 1 | dB | 2Note 11 | -14 | -15 |
|  | Config 1 | dBm/15KHz | -104.7 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.1.7.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |

Figure A.7.5.1.7.1-1: SNR variation for CSI-RS out-of-sync testing

##### A.7.5.1.7.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on PCell.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 (PCell) at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

The UE shall stop transmitting uplink signal in Cell 1 (PCell) no later than time point C (D1 secondafter the start of the time duration T3) on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode

##### A.7.5.1.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR2 PCell CSI-RS In-sync radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.7.5.1.8.1-1, A.7.5.1.8.1-2, A.7.5.1.8.1-3 and A.7.5.1.8.1-4 below. There is one cells, cell 1which is the PCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.1.8.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 10 ms. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test. In the test, SSB0 and SSB1 are configured as BFD-RS.

Table A.7.5.1.8.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table A.7.5.1.8.1-2: General test parameters for FR2 PCell for CSI-RS in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | TDD |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD CCR.3.3 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |
| SMTC Configuration | Config 1 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | *[gp0] |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 2000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 1.64 |
| T4 |  | s | 0.2 |
| T5 |  | s | 1.88 |
| D1 |  | s | 1.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.7.5.1.8.1-3: Cell specific test parameters for FR2 for CSI-RS in-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | dB | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS1 | Config 1 | dB | 2Note 11 | -6Note 11 | -15 | -4.5 | 2Note 11 |
| SNR on RLM-RS2 | Config 1 | dB | 2Note 11 | -14 | -15 | -15 | -14 |
|  | Config 1 | dBm/15KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.5.1.8.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |

Table A.7.5.1.8.1-4: Measurement gap configuration for FR2 CSI-RS in-sync radio link monitoring in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: RLM RS is partially overlapped with measurement gap |  |

Figure A.7.5.1.8.1-1: SNR variation for CSI-RS in-sync testing

##### A.7.5.1.8.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2

##### A.7.5.1.9.1	Test Purpose and Environment

The purpose is to verify that the NR UE correctly follows the RLM scheduling restrictions requirements defined in clause 8.1.7. This test verifies that the UE correctly receive the PDCCH scheduled on the symbols right before the RLM SSB symbols without overlap so that it sends ACK/NACK correctly. The test case is only applicable to UE which supports pdcch-MonitoringAnyOccasions or pdcch-MonitoringAnyOccasionsWithSpanGap.

The test parameters are given in table A.7.5.1.9.1-1, table A.7.5.1.9.1-2 and table A.7.5.1.9.1-3 below. The UE is required during time period T1 to transmit ACK/NACK correctly upon scheduling of PDSCH.

Table A.7.5.1.9.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 120 kHz RMC SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.1.9.1-2: General test parameters for NR RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | 1 | 1 |  |
| SSB configuration |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  | 1 | SMTC pattern 1 |  |
| DRX cycle length | s | 1 | OFF |  |
| T1 | s | 1 | 5 | During T1 the UE is required to correctly transmit ACK/NACK |

Table A.7.5.1.9.1-3: Cell specific test parameters for NR RLM scheduling restriction test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
| AoA setup |  | 1 | Setup 3 defined in A.3.15.3 |  |
|  |  |  | AoA1 | AoA2 |
| Assumption for UE beams Note 1 |  |  | Rough | Rough |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1 | 24 |  |
| PDSCH Reference measurement channel |  | 1 | SR.3.2 TDD | Not sent |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD | Not sent |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.2 TDD | Not sent |
| TRS configuration |  | 1 | TRS.2.1 TDD | TRS.2.2 TDD |
| PDCCH/PDSCH TCI state |  | 1 | TCI.State.2 | N/A |
| OCNG Pattern |  | 1 | OP.5 defined in A.3.2.1 | Not sent |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |
| RLM-RS |  | 1 | SSB with index 0 | SSB with index 1 |
|  | dBm/15kHz | 1 | -92.1 | -92.1 |
| Note2 | dBm/SCS | 1 | -83.1 | -83.1 |
|  | dB | 1 | 2 | 2 |
| BB Note 4 | dB | 1 | 1 | 1 |
| SSB_RP Note3 | dBm/SCS | 1 | -81.1 | -81.1 |
| Io | dBm/95.04 MHz | 1 | -54.35 | -54.35 |
| Time multiplexing of the downlink transmissions from each AoA |  | 1 | Defined in Figure A.7.5.1.9.1-1 |  |
| Propagation Condition |  | 1 | AWGN | AWGN |
| Note 1: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |

Figure A.7.5.1.9.1-1: Time multiplexed downlink transmissions

##### A.7.5.1.9.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.1.7.3.

### A.7.5.2	Interruption

#### A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2

##### A.7.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE missed ACK/NACK rate does not exceed the limits at NR PSCell interruptions during the measurement on the deactivated NR SCC. This test will verify the missed ACK/NACK rate for PCell in standalone NR specified in clause 8.2.2.2. Supported test configurations are shown in table A.7.5.2.1.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.7.5.2.1.1-2 and A.7.5.2.1.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell, Cell2 is an NR deactivated SCell. Cell1 shall be configured as PCell and Cell2 shall be configured as SCell.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. The point in time at which the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector, defines the start of time period T1. During T1, PCell is continuously scheduled in DL.

Table A.7.5.2.1.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD – TDD duplex mode |

Table A.7.5.2.1.1-2: General test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell2 | Deactivated SCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to Cell1 and Cell2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

Table A.7.5.2.1.1-3: NR cell specific test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter |  | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR2 |  |
| Duplex mode |  |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  |
| BWchannel |  |  | 100 MHz: NRB,c = 66 |  |
| Data RBs allocated |  |  | 66 |  |
| Initial DL BWP Configuration |  |  | DLBWP.0.2Note4 |  |
| Initial UL BWP Configuration |  |  | ULBWP.0.2 Note6 |  |
| Downlink dedicated BWP Configuration |  |  | DLBWP.1.1 |  |
| Uplink dedicated BWP configuration |  |  | ULBWP.1.1 |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  |
| RMSI CORESET parameters |  |  | CR.3.1 TDD |  |
| Dedicated CORESET parameters |  |  | CCR.3.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SMTC Configuration |  |  | SMTC.1 |  |
| SSB Configuration |  |  | SSB.1 FR2 |  |
| TCI State |  |  | TCI.State.0 |  |
| TRS Configuration |  |  | TRS.2.1 TDD |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Time offset to Cell1 Note 3 |  | s | - | 3 |
| Propagation Condition |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2 defined in clause 12 of of TS 38.213 [3]. |  |  |  |  |

Table A.7.5.2.1.1-4: OTA related test parameters for interruptions during measurements on deactivated NR SCC in standalone NR

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration |  |  | Setup1 according to table A.3.15.1 | Setup 1according to table A.3.15.1 |
| Assumption for UE beams Note 6 |  |  | Rough | Rough |
| Note1 | NR_TDD_FR2_A | dBm/15kHz | -104.7 | -104.7 |
|  | NR_TDD_FR2_B |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |
| Note1 | NR_TDD_FR2_A | dBm/SCS | -95.7 | -95.7 |
|  | NR_TDD_FR2_B |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |
| SSB_RPNote2 | NR_TDD_FR2_A | dBm/120KHz Note3 | -88.7 | -88.7 |
|  | NR_TDD_FR2_B |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |
|  |  | dB | 7 | 7 |
|  |  | dB | 7 | 7 |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -58.92 | -58.92 |
|  | NR_TDD_FR2_B |  |  |  |
|  | NR_TDD_FR2_F |  |  |  |
|  | NR_TDD_FR2_G |  |  |  |
|  | NR_TDD_FR2_T |  |  |  |
|  | NR_TDD_FR2_Y |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |

##### A.7.5.2.1.2	Test Requirements

The UE shall be continuously scheduled on PCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on PCell.

If the NR PCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on NR PCell immediately before and immediately after an SMTC. Each interruption on NR PCell shall not exceed the value defined in Table A.7.5.2.1.2-1.

If the NR PCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell no earlier than 4 slots before an SMTC and no later than 4 slots after the SMTC. the interruption on NR PCell shall not exceed the value defined in Table A.7.5.2.1.2-2.

Table A.7.5.2.1.2-1: Interruption duration if the PCell is not in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 3 | 0.125 | 4 |

Table A.7.5.2.1.2-2: Interruption duration if the PCell is in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 3 | 0.125 | 8 + SMTC duration |

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.2.2	SA interruptions at NR SRS carrier-based switching

##### A.7.5.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit aperiodic SRS, the UE can perform SRS carrier-based switching to a carrier not configured for PUCCH/PUSCH transmission from a carrier with PUCCH/PUSCH transmission. The test will partly verify the interruption requirements on PCell in clause 8.2.2.2.9.

##### A.7.5.2.2.2	Test Parameters

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the FR2 PCell. Cell 2 is an activated FR2 SCell on the TDD SCC which operats in downlink without PUCCH/PUSCH. The UE is configured with the SRS switching between PCell and SCell.The test parameters for PCell and SCell are given in Tables A.7.5.2.2.2-2, A.7.5.2.2.2-3, and A.7.5.2.2.2-4 below. The test consists of two successive time periods, with duration of T1 and T2, respectively. Immediately at the beginning of T2, the UE is triggered for SRS switching. The UE shall be scheduled on PCell continuously throughout the test.

The test equipment verifies that potential interruption is carried out correctly by monitoring ACK/NACK sent in PCell.

Table A.7.5.2.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.7.5.2.2.2-2: General test parameters for SA interruptions at NR SRS carrier-based switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1 |
| Configured SCell |  | Cell 2 | Activated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of PCell |
| T1 | s | 5 |  |
| T2 | ms | 100 | UE shall perform SRS switching during T2 |

Table A.7.5.2.2.2-3: Cell-specific test parameters for SA interruptions at NR SRS carrier-based switching

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR2 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD configuration | Config 1 |  | TDDConf.3.1 |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |  |
| Downlink initial BWP Configuration | Config 1 |  | DLBWP.0.1 |  |
| Downlink dedicated BWP Configuration | Config 1 |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration | Config 1 |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |  |
| SRS configuration | Config 1 |  | SRS.3 TDD |  |
| TRS configuration | Config 1 |  | TRS.2.1 TDD |  |
| TCI state | Config 1 |  | TCI.State.0 |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.3.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |  |
| RMC CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SSB Configuration |  |  | SSB.1 FR2 |  |
| SMTC Configuration | Config 1 |  | SMTC.1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Ês/Noc |  | dB | 17 |  |
| Propagation Condition |  |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.5.2.2.2-4: OTA related test parameters

| Parameter | Unit | Test 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |
| Assumption for UE beams Note 6 |  | Fine |  |
| Note 1 | dBm/15kHzNote 4 | -112 |  |
| Note 1 | dBm/SCSNote 3 | -103 |  |
|  | dB | 4 |  |
| SS-RSRP Note 2 | dBm/SCS Note 4 | -99 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note 4 | -68.5 |  |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. NOTE 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. NOTE 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |

##### A.7.5.2.2.3	Test Requirements

During T2, interruption on PCell due to SRS carrier-based switching between Cell 1 and Cell 2 shall not exceed the required values specified in clause 8.2.2.2.9.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.5.3	SCell Activation and Deactivation Delay

#### A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX

##### A.7.5.3.1.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.6.5.3.1.1 except the PCell and SCell are in FR2 intra-band.

The supported test configurations are shown in table A.7.5.3.1.1-1 below. The general test parameters are the same as defined in Table A.6.5.3.1.1-2 except those described in Tables A.7.5.3.1.1-2, and cell specific test parameters are described in Tables A.7.5.3.1.1-3. OTA related test parameters are shown in table A.7.5.3.1.1-4 below.

Table A.7.5.3.1.1-1: Supported test configurations for FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |

Table A.7.5.3.1.1-2: General test parameters for FR2 SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channels are used for this test, cell 1 and cell2 use RF channel 1 and 2, respectively. |

Table A.7.5.3.1.1-3: Cell specific test parameters for FR2 SCell activation case

| ParameterNote 5 | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 | T3 |
| SSB ARFCN |  | freq1 |  |  | freq2 |  |  |  |
| Duplex mode |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | TDDConf.3.1 |  |  |  |  |  |  |
| Downlink initial BWP Configuration |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP Configuration |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |  |  |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| Data RBs allocated |  | 66 |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  |  | - |  |  |  |
| RMSI CORESET Parameters |  | CR.3.1 TDD |  |  | - |  |  |  |
| Dedicated CORESET Parameters |  | CCR.3.1 TDD |  |  | - |  |  |  |
| OCNG Patterns |  | OP.1 |  |  |  |  |  |  |
| SSB Configuration |  | SSB.1 FR2 |  |  |  |  |  |  |
| SMTC Configuration |  | SMTC.1 |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | CSI-RS.3.1 TDD |  |  |  |  |  |  |
| reportConfigType |  | periodic |  |  |  | N/A |  |  |
| reportQuantity |  | cri-RI-CQI |  |  |  | N/A |  |  |
| CSI reporting periodicity | slot | 40 |  |  |  | N/A |  |  |
| CSI reporting offset | slot | 4 |  |  |  | N/A |  |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Propagation conditions |  | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: Void |  |  |  |  |  |  |  |  |

Table A.7.5.3.1.1-4: OTA related test parameters for FR2 SCell activation case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  | Setup 1 according to table A.3.15.1 |  |  | Setup 1 according to table A.3.15.1 |  |  |
| Assumption for UE beams Note 7 |  | Rough |  |  | Rough |  |  |
| Note1 | dBm/15kHzNote4 | -104.7 |  |  | -104.7 |  |  |
| Note1 | dBm/SCSNote3 | -95.7 |  |  | -95.7 |  |  |
|  | dB | 7 |  |  | 7 |  |  |
| SSB_RPNote2 | dBm/SCS Note4 | -88.7 |  |  | -88.7 |  |  |
|  | dB | 7 |  |  | 7 |  |  |
| IoNote2 | dBm/95.04 MHz Note4 | -58.92 |  |  | -58.92 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

##### A.7.5.3.1.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time will be replaced with the value TFirstSSB + 5ms as defined in clause 8.3.

#### A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2

##### A.7.5.3.2.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.7.5.3.1.1 except the PCell is in FR1 and SCell is in FR2.

The supported test configurations are defined in Table A.7.5.3.2.1-1. The general test parameters are the same as defined in Table A.6.5.3.1.1-2 except that the length of T2 is 2s. And cell specific test parameters are described in Tables A.7.5.3.2.1-2. OTA related test parameters are defined in Table A.7.5.3.2.1-3.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on NR. During T1 the SCell is powered off and UE is not aware of SCell.

A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a slot # denoted m. The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2.

During T2, the test equipment monitors the L1-RSRP measurement reporting for the SCell. The time when test equipment receives a valid L1-RSRP report is denoted as slot m+TL1-RSRP. In the next DL slot after slot m+TL1-RSRP, the test equipment sends a MAC message for the activation of the TCI state of the RMC CORESET of the SCell. In the same slot, the test equipment also sends an RRC message to configure the CSI-RS resources for SCell.

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PCell during activation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell1 deactivation command is sent until CSI reporting for SCell1 is discontinued.

Table A.7.5.3.2.1-1: Supported test configurations for FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Table A.7.5.3.2.1-2: Cell specific test parameters for FR2 SCell activation case

| ParameterNote 5 |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| SSB ARFCN |  |  | Freq1 |  |  |  | Freq2 |  |  |
| Duplex mode | Config 1 |  | FDD |  |  |  | TDD |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  | TDDConf.3.1 |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP Configuration | Config 1,2,3 |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP Configuration | Config 1,2,3 |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration | Config 1,2,3 |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration | Config 1,2,3 |  | ULBWP.1.1 |  |  |  |  |  |  |
| TRS configuration | Config 1,2,3 |  | N/A |  |  |  | TRS.2.1 TDD |  |  |
| TCI state | Config 1,2,3 |  | TCI.State.0 |  |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| Data RBs allocated | Config 1,2 |  | 52 |  |  |  | 66 |  |  |
|  | Config 3 |  | 106 |  |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  | - |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Parameters | Config 1 |  | CR.1.1 FDD |  |  |  | - |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |  |
| Dedicated CORESET Parameters | Config 1 |  | CCR.1.1 FDD |  |  |  | - |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  | SSB.3 FR2 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting | Config 1~3 |  | N/A |  |  |  | N/A | CSI-RS.3.1 TDD Note 6 | CSI-RS.3.1 TDD |
| reportConfigType for CSI reporting |  |  | periodic |  |  |  | N/A |  |  |
| reportConfigType for L1-RSRP |  |  | periodic |  |  |  | N/A |  |  |
| reportQuantity for CSI reporting |  |  | cri-RI-CQI |  |  |  | N/A |  |  |
| reportQuantity for L1-RSRP |  |  | ssb-Index-RSRP |  |  |  | N/A |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |  | N/A |  |  |
|  | Config 3 |  | 10 |  |  |  |  |  |  |
| L1-RSRP reporting periodicity Note 7 | Config 1,2 | slot | 5 |  |  |  | N/A |  |  |
|  | Config 3 |  | 10 |  |  |  |  |  |  |
| CSI reporting offset | Config 1,2 | slot | 2 |  |  |  | N/A |  |  |
|  | Config 3 |  | 4 |  |  |  |  |  |  |
| L1-RSRP reporting offset | Config 1,2 | slot | 2 |  |  |  | N/A |  |  |
|  | Config 3 |  | 4 |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  | N/A Link only, see clause A.3.7A |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: All parameters apply for configuration 1 and 2 Note 6: CSI-RS for CSI measurement is (re)configured in the next DL slot after slot m+TL1-RSRP during T2. Note 7: L1-RSRP measurement and reporting are configured to the the UE prior to the start of time period T1. |  |  |  |  |  |  |  |  |  |

Table A.7.5.3.2.1-3: OTA related test parameters for FR1 PCell activation case with FR2 SCell

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 |  | T3 |  |
| Angle of arrival configuration |  |  | N/A |  |  | According to clause A.3.15.1 |  |  |  |  |
| Assumption for UE beams Note 7 |  |  | N/A |  |  | Rough |  |  |  |  |
| Note 1 | Config 1,2,3 | dBm/15kHz | Link only, see clause A.3.7A |  |  | -104.7 |  |  |  |  |
| Note 1 | Config 1,2,3 | dBm/SCS |  |  |  | -95.7 |  |  |  |  |
|  | Config 1,2,3 | dB |  |  |  | -∞ |  | 7 |  | 7 |
|  | Config 1,2,3 | dB |  |  |  | -∞ |  | 7 |  | 7 |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS |  |  |  | -∞ |  | -88.7 |  | -88.7 |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz |  |  |  | -66.68 |  | -58.92 |  | -58.92 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |

##### A.7.5.3.2.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after slot (m+k). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption.  Whether CSI report in a slot was interrupted is checked by monitoring ACK/NACK sent in PCell in the slot.

During T2 the UE shall start sending valid L1-RSRP report for the SCell in the configured slots for CSI reporting after slot (m+TL1-RSRP), where TL1-RSRP is no larger than

3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report

as defined in clause 8.3.2. For this test case, TFirstSSB_MAX=TSMTC_MAX=Trs=20ms; TL1-RSRP, measure=160ms and TL1-RSRP, report=5ms, which allows TL1-RSRP 680 ms.

During T2 the UE shall start sending CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, where

- THARQ is defined in Table A.5.5.3.1.1-2

- Tactivation_time = 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, which allows 710 ms

- TCSI_Reporting = 10ms

- NR slot length is 0.125ms for this test case.

During T3 the UE shall stop sending CSI reports for both SCells no later than slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T2 interruption of PCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length  to m+1+THARQ+3ms+TXNR slot length, as defined in clause 8.3, where TX =20ms.

During T3 the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption of PCell due to activation of SCell shall not be more than the values specified for SA in Clause 8.2.2.2.7.

#### A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX

##### A.7.5.3.3.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in clause A.7.5.3.1.1 except the PCell and SCell are in FR2 inter-band.

The supported test configurations are shown in table A.7.5.3.3.1-1 below. The general test parameters are described in Tables A.7.5.3.3.1-2, and cell specific test parameters are described in Tables A.7.5.3.3.1-3. OTA related test parameters are shown in table A.7.5.3.3.1-4 below.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on NR. During T1 the SCell is powered off and UE is not aware of SCell. A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a slot # denoted m.

The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 is increased to same level as for cell 2. During T2, the test equipment monitors the L1-RSRP measurement reporting for the SCell. The time when test equipment receives a valid L1-RSRP report is denoted as slot m+TL1-RSRP. In the next DL slot after slot m+TL1-RSRP, the test equipment sends a MAC message for the activation of the TCI state of the RMC CORESET of the SCell. In the same slot, the test equipment also sends an RRC message to configure the CSI-RS resources for SCell.

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a slot # denoted n, is received at the UE antenna connector.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PSCell during activation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell1 deactivation command is sent until CSI reporting for SCell1 is discontinued.

Table A.7.5.3.3.1-1: Supported test configurations for FR2 SCell activation in FR2 inter-band

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |

Table A.7.5.3.3.1-2: General test parameters for FR2 SCell activation in FR2 inter-band

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channels are used for this test. RF channel number 1 is in band 1 and RF channel number 2 is in band 2, where bands 1 and 2 are inter-band CA operating bands in FR2 as specified in Table 5.2A.2-1 in TS38.101-2. |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | ≤8 | A random value from 0s to 8s |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 2 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.7.5.3.3.1-3: Cell specific test parameters for FR2 SCell activation in FR2 inter-band

| ParameterNote 5 | Unit | T1 |  | T2 |  | T3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Downlink initial BWP Configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Downlink dedicated BWP Configuration |  | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| TRS configuration |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| TCI state |  | TCI.State.0 |  | TCI.State.0 |  | TCI.State.0 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Parameters |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Dedicated CORESET Parameters |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| CSI-RS configuration |  | NA | NA | NA | CSI-RS.3.1 TDD Note 2 | NA | CSI-RS.3.1 TDD |
| CSI reporting periodicity Note 3 |  | NA | 5 | NA | 5 | NA | 5 |
| OCNG Patterns |  | OP.1 |  |  |  |  |  |
| SSB Configuration |  | SSB.1 FR2 |  |  |  |  |  |
| SMTC Configuration |  | SMTC.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Propagation conditions |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: CSI-RS for CSI measurement is (re)configured in the next DL slot after slot m+TL1-RSRP during T2. Note 3: L1-RSRP measurement and reporting are configured to the the UE prior to the start of time period T1. |  |  |  |  |  |  |  |

Table A.7.5.3.3.1-4: OTA related test parameters for FR2 SCell activation in FR2 inter-band

| ParameterNote 6 | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| AoA setup |  | Setup 3 as specified in clause A.3.15 |  |  |  |  |  |
|  |  | AoA1 |  |  | AoA2 |  |  |
| Assumption for UE beams Note 7 |  | Rough |  |  | Rough |  |  |
| Note1 | dBm/15kHzNote4 | -92.1 |  |  | -92.1 |  |  |
| Note1 | dBm/SCSNote3 | -83.1 |  |  | -83.1 |  |  |
|  | dB | 0 |  |  | 0 |  |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -83.1 |  |  | -83.1 |  |  |
|  | dB | 0 |  |  | 0 |  |  |
| IoNote2 | dBm/95.04 MHz Note4 | -51.1 |  |  | -51.1 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: All parameters apply for configuration 1 Note 7: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

##### A.7.5.3.3.2	Test Requirements

During T2 the UE shall start sending CSI report for the SCell in the configured slots for CSI reporting after slot (m+k). UE shall send the first CSI report for SCell in slot (m+k), or in the next available uplink resource for CSI reporting if slot (m+k) was subject to interruption. Whether CSI report in a slot was interrupted is checked by monitoring ACK/NACK sent in PCell in the slot.

During T2, the UE shall start sending valid L1-RSRP report for the SCell in the configured slots for CSI reporting after slot (m+TL1-RSRP), where TL1-RSRP is no larger than 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report as defined in clause 8.3.2. For this test case, TFirstSSB_MAX=TSMTC_MAX=Trs=20ms; TL1-RSRP, measure=480ms and TL1-RSRP, report=5ms, which allows TL1-RSRP =1000ms.

During T2, the UE shall start sending CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot m+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, where

-	THARQ is defined in Table A.7.5.3.3.1-2

-	Tactivation_time = 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}, which allows 1030ms

-	TCSI_Reporting = 10ms

-	NR slot length is 0.125ms for this test case.

During T2, the interruption of PCell during SCell activation shall not happen outside the slot m+1+THARQNR slot length  to m+1+THARQ+3ms+TXNR slot length, where TX =20ms.

During T3, the UE shall stop sending CSI reports for SCell no later than slot n+THARQ+3msNR slot length, as defined in clause 8.3.

During T3, the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3msNR slot length as defined in clause 8.3.

#### A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2

##### A.7.5.3.4.1	Test Purpose and Environment

The purpose of this test is to verify that the delay and interruption for direct SCell activation delay at SCell addition are within the requirements stated in clause 8.3.4.

The supported test configurations are shown in Table A.7.5.3.4.1-1 below. The general test parameters are given in Table A.7.5.3.4.1-2 and cell-specific test parameters in Table A.7.5.3.4.1-3. OTA related test parameters are shown in Table A.7.5.3.4.1-4.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two FR2 carriers and two NR cells. Before the test starts the UE is connected to Cell 1 (PCell) on carrier #1, but is not aware of Cell 2 on NR carrier #2. Cell 1 and Cell 2 have constant signal levels throughout the test. The UE is monitoring the PCell. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 is monitored by the UE. During T1, Cell 2 should be detected and measured by the UE such that it meets the condition for known cell defined in clause 8.3.4 for direct SCell activation.

Time period T2 starts when the RRCReconfiguration message for the configuration and activation of Cell 2 (the SCell), which is sent from the test equipment, is received at the UE antenna connector in a slot # denoted m. The test equipment shall set the parameter sCellState to activated for the SCell, which causes Cell 2 to become configured and activated.

Time period T3 starts at (m + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during the activation of SCell. The test equipment verifies the activation time by counting the slots from the time when the SCell activation message is sent until a CQI report with other than CQI index 0 is received.

Table A.7.5.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.3.4.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channels (1,2) in FR2 are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured and activated SCell |  | Cell 2 | Configured and activated SCell on NR RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| T1 | s | 7 | During this time the measurement for Cell 2 is configured, and Cell 2 is detected. |
| T2 | s | Ndirect | During this time the UE shall configure and activate Cell 2 as SCell. |
| T3 | ms | 100 | During this time the UE shall report valid CQI for both PCell and SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| k | slot |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.7.5.3.4.1-3: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq1 |  |  | freq2 |  |  |
| Duplex mode | Config 1 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | TDDConf.3.1 |  |  |  |  |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |  |  |  |  |  |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |  |  |  |  |  |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |  |  |  |  |  |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |  |  |  |  |  |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |  |  |  |  |  |
| Timing offset to Cell 1 |  | ms | Not Applicable |  |  | 0 |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMC CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration | Config 1 |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| CSI-RS configuration | Config 1 |  | CSI-RS.3.1 TDD |  |  | CSI-RS.3.1 TDD |  |  |
| CSI reporting periodicity | Config 1 | ms | 5 |  |  | 5 |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration | Config 1 |  | SSB.1 FR2 |  |  | SSB.1 FR2 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |  |

Table A.7.5.3.4.1-4: OTA related test parameters

| ParameterNote 6 | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 |  |  |  |  |  |
| Assumption for UE beamsNote 7 |  | Rough |  |  | Rough |  |  |
| Note1 | dBm/15kHzNote4 | -112 |  |  | -112 |  |  |
| Note1 | dBm/SCSNote3 | -102.97 |  |  | -102.97 |  |  |
|  | dB | 14 |  |  | 14 |  |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -88.97 |  |  | -88.97 |  |  |
|  | dB | 14 |  |  | 14 |  |  |
| IoNote2 | dBm/95.04 MHz Note4 | -59.81 |  |  | -59.81 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: All parameters apply for configuration 1 Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.5.3.4.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in the first available uplink resource after slot (m+k). UE is allowed to postpone CSI report to next available UL resource if an available uplink resource is subject to interruption.  Whether CSI report in a slot was interrupted is checked by monitoring ACK/NACK sent in PCell in the slot.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot m+NdirectNR slot length , where

Ndirect = TRRC_Process + T1 + Tactivation_time + TCSI_Reporting - 3ms,

-	TRRC_Process = 16ms, which is the RRC procedure delay defined for SCell addition in clause 12 of TS 38.331 [2],

-	T1 is the delay from slot m + TRRC_Process until the transmission of RRCReconfigurationComplete message,

-	Tactivation_time = TFirstSSB+ 5ms = 25ms,

-	TCSI_Reporting = 10ms

This gives a total of Ndirect = 16 + T1 + 25 + 10 - 3 = (48 + T1) ms, and NR slot length is 0.125ms.

During T3 the UE shall send CSI reports for SCell with non-zero CQI index and continue to send CSI reports for SCell  with non-zero CQI index until the end of T3.

During T2 interruption of PSCell during SCell activation shall not happen outside the window from slot m+1 to slot  m+1+TRRC_Process+T1+TXNR slot length as defined in clause 8.3.4, where TX =20ms.

The interruption of PCell due to activation of SCell shall not be more than the values specified for NR SA in clause 8.2.2.2.11.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot m+NdirectNR slot length as defined in clause 8.3.4 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

#### A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2

##### A.7.5.3.5.1	Test Purpose and Environment

This test is to verify the requirements specified in sub clause 8.3.5 for the FR2 handover with direct SCell activation.

The test scenario comprises of three FR2 cells, one source PCell (Cell 1), one target PCell (Cell 2) and one SCell (Cell 3). The test consists of three successive time periods, with time durations of T1, T2, and T3 respectively.

At the start of time duration T1, the UE is in connected mode with PCell (Cell 1). Both Cell 2 and Cell 3 are known to UE and UE is reporting CQI for all Cell 1.

Time period T2 starts when UE receives a handover command that initiate handover of UE to Cell2 and also activates Cell 3. This is done using an RRCConnectionReconfiguration message with parameter sCellState set to activated for the Cell 3. The message is sent from the test equipment to the UE and is received in a slot number n at the UE antenna connector. The UE shall accomplish the handover, addition and activation of the SCell no later than slot (n +NdirectNR slot length).

Time period T3 starts at (n +NdirectNR slot length), at which point UE shall be reporting a valid CSI for both Cell 2 and Cell 3 as given in tables A.7.5.3.5.1-1 and A.7.5.3.5.1-2.

Table A.7.5.3.5.1-1: Supported test configurations for FR2 handover with direct SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | SCell: NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.3.5.1-2: General test parameters for FR2 handover with direct SCell activation case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2, 3 | Three NR radio channels are used for this test, Cell 1, Cell2 and Cell 3 use RF channel 1, 2 and 3 respectively. |
| A4-Offset |  | dBm | -120 |  |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| Initial conditions | Source cell |  | Cell 1 | Source Cell |
|  | Target cell |  | Cell 2 | Neighbour cell |
|  | SCell |  | Cell 3 | SCell is not added and activated |
| Final condition | Source cell |  | Cell 2 | Cell 2 is Source cell after handover |
|  | Neighbour cell |  | Cell 1 | Neighbour cell |
|  | SCell |  | Cell 3 | SCell is added and activated |

Table A.7.5.3.5.1-3: Cell specific test parameters for FR2 SCell activation case

| ParameterNote 5 | Unit | T1 |  |  | T2 |  |  | T3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |
| SSB ARFCN |  | freq1 | freq2 | freq 3 | freq1 | freq2 | freq 3 | freq1 | freq2 | freq3 |
| Duplex mode |  | TDD |  |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| Downlink initial BWP Configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP Configuration |  | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| TCI state |  | TCI.State.0 |  |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  | - | SR.3.1 TDD |  | - | SR.3.1 TDD |  |  |
| RMSI CORESET Parameters |  | CR.3.1 TDD |  | - | CR.3.1 TDD |  | - | CR.3.1 TDD |  |  |
| Dedicated CORESET Parameters |  | CCR.3.1 TDD |  | - | CCR.3.1 TDD |  | - | CCR.3.1 TDD |  |  |
| OCNG Patterns |  | OP.1 |  |  |  |  |  |  |  |  |
| SSB Configuration |  | SSB.1 FR2 |  |  |  |  |  |  |  |  |
| SMTC Configuration |  | SMTC.1 |  |  |  |  |  |  |  |  |
| PRACH configuration |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 0 |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Void |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.3.5.1-4: OTA related test parameters for FR2 SCell activation case

| ParameterNote 6 | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  | Setup 1 according to table A.3.15.1 |  |  | Setup 1 according to table A.3.15.1 |  |  | Setup 1 according to table A.3.15.1 |  |  |
| Assumption for UE beams Note 7 |  | Rough |  |  | Rough |  |  | Rough |  |  |
| Note1 | dBm/15kHzNote4 | -112 |  |  | -112 |  |  | -112 |  |  |
| Note1 | dBm/SCSNote3 | -102.97 |  |  | -102.97 |  |  | -102.97 |  |  |
|  | dB | 14 |  |  | 14 |  |  | 14 |  |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -88.97 |  |  | -88.97 |  |  | -88.97 |  |  |
|  | dB | 14 |  |  | 14 |  |  | 14 |  |  |
| IoNote2 | dBm/95.04 MHz Note4 | -88.80 |  |  | -88.80 |  |  | -88.80 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |

##### A.7.5.3.5.2	Test Requirements

The UE shall be capable to transmit valid CSI report for PCell (Cell 2) and to the directly activated SCell1 no later than in slot n+ Ndirect.

The SCell activation delay, Ndirect, can be expressed as: Ndirect = TRRC_process + Tinterrupt + T2 + T3 + Tactivation_time + TCSI_Reporting - 3ms; Where:

TRRC_Process: RRC procedure delay defined in clause 12 of TS 38.331 and it is equal to 16ms,

Tinterrupt: Interruption time during handover as specified in clause 6.1.1. The value to be verified in the test is 52 ms (Tinterrupt = 0 ms for Tsearch + 10ms for TIU + 20 ms for Tprocessing + 20ms for T∆ + 2 ms for Tmargin ms) by assuming known SCell and SMTC.1 configuration.

T2: Delay from slot n+TRRC_Process+TinterruptNR slot length  until UE has obtained a valid TA command for the target PCell,

T3: Delay for applying the received TA for uplink transmission in the target PCell, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213,

Tactivation_time and TCSI_Reporting are specified in clause 8.3.2, where the following definitions of TFirstSSB and TFirstSSB_MAX as defined in section 8.3.5 shall apply:

During time period T2 of the test, the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TSMTC_SCell + 5ms, as defined in clause 8.3.

During time period T3 of the test, the UE shall stop sending CSI reports for SCell at latest in a slot m+THARQ+3msNR slot length, as defined in clause 8.3.

During time period T2 of the test, interruption of PCell / PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3.

During time period T3 of the test, the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in clause 8.3.

The interruption on any activated serving cell shall not be more than the values specified for SA in clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During time period T2 of the test, if there are no uplink resources for reporting the valid CSI in a slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.7.5.4	Void

### A.7.5.5	Beam Failure Detection and Link recovery procedures

#### A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

##### A.7.5.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR2 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.7.5.5.1.1-1, A.7.5.5.1.1-2, A.7.5.5.1.1-3 and A.7.5.5.1.1-4 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.1.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.7.5.5.1.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 1.

Table A.7.5.5.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 2 | TDD duplex mode, 240 kHz SSB SCS, 100 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.7.5.5.1.1-2: General test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  | Config 1, 2 |  |  |  | TDD |  |
| BWchannel |  | Config 1, 2 |  |  |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | Config 1, 2 |  |  |  | 66 |  |
| DL initial BWP configuration |  | Config 1, 2 |  |  |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1, 2 |  |  |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1, 2 |  |  |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1, 2 |  |  |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1, 2 |  |  |  | TDDConf.3.1 |  |
| CORESET Reference Channel |  | Config 1, 2 |  |  |  | CR. 3.1 TDD |  |
| SSB Configuration |  | Config 1 |  |  |  | SSB.1 FR2 |  |
|  |  | Config 2 |  |  |  | SSB.2 FR2 |  |
| SMTC Configuration |  | Config 1, 2 |  |  |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  |  |  | 120 KHz |  |
| PRACH Configuration |  | Config 1, 2 |  |  |  | FR2 PRACH configuration 2 | A.3.8.3 |
| SSB index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  |  | 2 |  |
|  | Aggregation level |  |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  |  | 6 |  |
| DRX |  |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  |  | gp0 |  |
| gapOffset |  |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp- |  |  | Config 1 |  | dBm/SSB | -94.5 | Threshold used |
| ThresholdSSB |  |  | Config 2 |  | SCS | -91.5 | for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  |  | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI reporting |  |  |  | Config 1, 2 |  | CSI-RS.3.1 TDD |  |
| TCI states |  |  |  |  |  | TCI.State.0 |  |
| CSI-RS for tracking |  |  |  | Config 1, 2 |  | TRS.2.1 TDD |  |
| SSB index assigned as RLM RS |  |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 2.61 |  |
| T3 |  |  |  |  | s | 1.64 |  |
| T4 |  |  |  |  | S | 0 |  |
| T5 |  |  |  |  | s | 1.01 |  |
| D1 |  |  |  |  | s | 0.97 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Editor’s note: An additional RS for RLM, different from BFD-RS at constant high SNR shall be configured as part of the test configuration.

Table A.7.5.5.1.1-3: Cell specific test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5Note 11 | -3Note 11 | -12 | -12 | -12 |
|  | Config 2 |  | 5Note 11 | -3Note 11 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 2 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| SSB_RP of set q1 | Config 1 | dBm/SSB | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
|  | Config 2 | SCS | -101.5 | -101.5 | -81.5 | -81.5 | -81.5 |
|  | Config 1 | dBm/120 KHz | -104.7 |  |  |  |  |
|  | Config 2 |  | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE hich supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |

Table A.7.5.5.1.1-4: Void

Figure A.7.5.5.1.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.7.5.5.1.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 960+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode

##### A.7.5.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR2 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.7.5.5.2.1-1, A.7.5.5.2.1-2, A.7.5.5.2.1-3, A.7.5.5.2.1-4 and A.7.5.5.2.1-5 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.2.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.7.5.5.2.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.5.5.2.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 2 | TDD duplex mode, 240 kHz SSB SCS, 100 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Table A.7.5.5.2.1-2: General test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  |  | Config 1, 2 |  |  | TDD |  |
| BWchannel |  |  | Config 1, 2 |  |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1, 2 |  |  | 66 |  |
| DL initial BWP configuration |  |  | Config 1, 2 |  |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2 |  |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | Config 1, 2 |  |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2 |  |  | ULBWP.1.1 |  |
| TDD Configuration |  |  | Config 1, 2 |  |  | TDDConf.3.1 |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  |  | CR.3.1 TDD |  |
|  |  |  | Config 2 |  |  | CR.3.2 TDD |  |
| SSB Configuration |  |  | Config 1 |  |  | SSB.1 FR2 |  |
|  |  |  | Config 2 |  |  | SSB.2 FR2 |  |
| SMTC Configuration |  |  | Config 1, 2 |  |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  |  | 120 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  |  | FR2 PRACH configuration 2 | A.3.8.3 |
| SSB index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  |  | 2 |  |
|  | Aggregation level |  |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  |  | 6 |  |
| DRX |  |  |  |  |  | DRX.3 | A.3.3.3 |
| Gap pattern ID |  |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp- |  | Config 1 |  |  | dBm/SSB | -94.5 | Threshold used |
| ThresholdSSB |  | Config 2 |  |  | SCS | -91.5 | for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI reporting |  |  |  | Config 1, 2 |  | CSI-RS.3.1 TDD | A.3.14.2 |
| TCI states |  |  |  |  |  | TCI.State.0 |  |
| CSI-RS for tracking |  |  |  | Config 1, 2 |  | TRS.2.1 TDD |  |
| SSB index assigned as RLM RS |  |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 3.37 |  |
| T3 |  |  |  |  | s | 2.8 |  |
| T4 |  |  |  |  | s | 0 |  |
| T5 |  |  |  |  | s | 0.61 |  |
| D1 |  |  |  |  | s | 0.57 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table A.7.5.5.2.1-3: Cell specific test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5Note 11 | -3Note 11 | -12 | -12 | -12 |
|  | Config 2 |  | 5Note 11 | -3Note 11 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
|  | Config 2 |  | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| SSB_RP of set q1 | Config 1 | dBm/SSB | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
|  | Config 2 | SCS | -101.5 | -101.5 | -81.5 | -81.5 | -81.5 |
|  | Config 1 | dBm/120 KHz | -104.7 |  |  |  |  |
|  | Config 2 |  | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause [A.3.6]. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |

Table A.7.5.5.2.1-4: Void

Table A.7.5.5.2.1-5: Void

Figure A.7.5.5.2.1-1: SNR and L1-RSRP variation for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.7.5.5.2.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 560+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode

##### A.7.5.5.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.7.5.5.3.1-1, A.7.5.5.3.1-2, and A.7.5.5.3.1-3 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.3.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0 in the active cell to emulate CSI-RS based beam failure. Figure A.7.5.5.3.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled.

Table A.7.5.5.3.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table A.7.5.5.3.1-2: General test parameters for FR2 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated | Config 1 |  | 66 |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | A.3.1.2 |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | A.3.10 |
| SMTC Configuration | Config 1 |  | SMTC.3 | A.3.11 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120KHz |  |
| PRACH Configuration | Config 1 |  | FR2 PRACH configuration 4 | A.3.8.3 |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  | 0 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |
| OCNG parameters |  |  | OP.1 | A.3.2.1 |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | OFF |  |
| Gap pattern ID |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | dBm/SCS kHz | -94.5 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 and q1 | Config 1 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD | A.3.14.2 |
| csi-RS-Index assigned as RLM RS |  |  | 0, 1 | A.3.14.2 |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 1.17 |  |
| T3 |  | s | 0.9 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.7.5.5.3.1-3: Cell specific test parameters for FR2 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumptpion for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5Note 11 | -3Note 11 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
|  | Config 1 | dBm/15 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.3.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |

Table A.7.5.5.3.1-4: Void

Table A.7.5.5.3.1-5: Void

Figure A.7.5.5.3.1-1: SNR and L1-RSRP variation for CSI-RS based beam failure detection and link recovery testing in non-DRX mode

##### A.7.5.5.3.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.7.5.5.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in clause 8.5.

The test parameters are given in Tables A.7.5.5.4.1-1, A.7.5.5.4.1-2, A.7.5.5.4.1-3, and A.7.5.5.4.1-4 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.4.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0 in the active cell to emulate CSI-RS based beam failure. Figure A.7.5.5.4.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.5.5.4.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table A.7.5.5.4.1-2: General test parameters for FR2 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated | Config 1 |  | 66 |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | A.3.1.2 |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | A.3.10 |
| SMTC Configuration | Config 1 |  | SMTC.3 | A.3.11 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |  |
| PRACH Configuration | Config 1 |  | FR2 PRACH configuration 4 | A.3.8.3 |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  | 0 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| PDSCH/PDCCH TCI state |  |  | TCI.State.2 |  |
| OCNG parameters |  |  | OP.1 | A.3.2.1 |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | DRX.3 | A.3.3.3 |
| Gap pattern ID |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | dBm/SCS kHz | -94.5 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 and q1 | Config 1 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD | A.3.14.2 |
| csi-RS-Index assigned as RLM RS | Config 1 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 5.43 |  |
| T3 |  | s | 5.16 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.7.5.5.4.1-3: Cell specific test parameters for FR2 PCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5Note 11 | -3Note 11 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
|  | Config 1 | dBm/120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.4.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |

Table A.7.5.5.4.1-4: Void

Table A.7.5.5.4.1-5: Void

Table A.7.5.5.4.1-6: Void

Figure A.7.5.5.4.1-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in DRX mode

##### A.7.5.5.4.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

##### A.7.5.5.5.1	Test Purpose and Environment

The purpose is to test scheduling availability restrictions when the UE is performing beam failure detection or when the UE is performing L1-RSRP measurement for candidate beam detection, when no DRX is used. This test will verify the scheduling availability restriction requirements in clause 8.5.7 and 8.5.8.

The test parameters are given in Tables A.7.5.5.5.1-1, A.7.5.5.5.1-2 and A.7.5.5.5.1-3 below. There is one cell, cell 1 which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.5.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.7.5.5.5.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. This test will focus on the scheduling availability during beam failure detection) and candidate beam detection. In the test, DRX configuration is not enabled. Test is to test the scheduling availability restriction of UE performing beam failure detection and candidate beam detection when SSB RS configured for Beam failure detection and candidate beam detection. During the test the UE is scheduled to transmit continuously in UL.

Table A.7.5.5.5.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.5.5.1-2: General test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated | Config 1 |  | 66 |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |  |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |  |
| CORESET Reference Channel | Config 1 |  | CR. 3.1 TDD |  |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |  |
| SMTC Configuration | Config 1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |  |
| PRACH Configuration | Config 1 |  | FR2 PRACH configuration 2 | A.3.8.3 |
| SSB index assigned as BFD RS (q0) |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  | 1 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | TCI.State.0 |  |
| OCNG parameters |  |  | OP.1 |  |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | OFF | DRX is not in use |
| Gap pattern ID |  |  | N.A. | No measurement gap pattern is configured |
| ssb-Index |  |  | 2 | Number of SSB indexes used for beam failure detection |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the 10% |
| rsrp-ThresholdSSB |  | dBm/SCS kHz | -94.5 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see TS 38.321 [7], clause 5.17 |
| beamFailureDetectionTimer |  |  | pbfd4 | see TS 38.321 [7], clause 5.17 |
| CSI Configuration for reporting | Config 1 |  | CSI-RS.3.1 TDD | A.3.14.2 |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 2.6 |  |
| T3 |  | s | 1.64 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 1.01 |  |
| D1 |  | s | 0.97 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.7.5.5.5.1-3: Cell specific test parameters for FR2 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| AoA Setup |  |  | Setup1 defined in A.3.15.1 |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5Note 11 | -3Note 11 | -12 | -12 | -12 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
| SNR_SSB of set q1 | Config 1 | dB | -12 | -12 | 5 | 5 | 5 |
|  | Config 1 | dBm/15KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.5.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband. |  |  |  |  |  |  |  |

Figure A.7.5.5.5.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.7.5.5.5.2	Test Requirements

The UE behaviour during time duration T3 follows the requirements defined in clause 8.5.7.3:

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on BFD-RS symbols to be measured for beam failure detection.

The UE behaviour during time durations T4 and T5 follows the requirements defined in clause 8.5.8.3:

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on reference symbols to be measured for candidate beam detection.

#### A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode

##### A.7.5.5.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for an active SCell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the SCell with schedulingRequestID-BFR-SCell-r16 configuration, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 SCell requirements in clause 8.5.

The test parameters are given in Tables A.7.5.5.6.1-1, A.7.5.5.6.1-2 and A.7.5.5.6.1-3. There are two cells, cell 1 is the active PCell and cell 2 is the active SCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.6.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0 in the active SCell to emulate CSI-RS based beam failure. Figure A.7.5.5.6.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is not enabled.

Table A.7.5.5.6.1-1: Supported test configurations for FR2 PCell and SCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table A.7.5.5.6.1-2: General test parameters for FR2 SCell for beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Active SCell |  |  | Cell 2 |  |
| RF Channel Number |  |  | 2 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | A.3.1.2 |
| SSB Configuration | Config 1 |  | SSB.3 FR2 | A.3.10 |
| SMTC Configuration | Config 1 |  | SMTC.3 | A.3.11 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120KHz |  |
| PRACH Configuration | Config 1 |  | Table A.3.8.3.1-1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 in activated SCell |  |  | 0 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |
| OCNG parameters |  |  | OP.1 | A.3.2.1 |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | OFF |  |
| Gap pattern ID |  |  | N.A. |  |
| schedulingRequestID-BFR-SCell-r16 |  |  | Configured |  |
| Periodicity of PUCCH for SR configuration for BFR on SCell |  | slots | 40 | 5ms |
| Offset of PUCCH for SR configuration for BFR on SCell |  | slots | 5 |  |
| PUCCH parameters for SR configuration for BFR on SCell |  |  | Table 8.3.3.1.2-1 in [13] |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 in activated SCell |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | dBm/SCS kHz | -94.5 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | NA | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 and q1 | Config 1 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD | A.3.14.2 |
| csi-RS-Index assigned as RLM RS |  |  | 0, 1 | A.3.14.2 |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 1.17 |  |
| T3 |  | s | 0.9 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.7.5.5.6.1-3: Cell specific test parameters for FR2 SCell for beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Cell 1 | Test 1 Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 to T5 | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumptpion for UE beams Note 10 |  |  | Rough | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | 0.2 | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | -104.5 | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
| Noc | Config 1 | dBm/ 15kHz | -104.7 | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.6.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

Figure A.7.5.5.6.1-1: SNR and L1-RSRP variation for beam failure detection and link recovery testing for SCell in non-DRX mode

##### A.7.5.5.6.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 in A.7.5.5.6.1 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initial link recovery. During T4 and T5 the UE measures and evaluates beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit PUCCH with LRR, followed by BFR MAC CE containing a beam associated with the candidate beam set q1. The UE shall not transmit PUCCH with an LRR with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode

##### A.7.5.5.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for an active SCell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the SCell with schedulingRequestID-BFR-SCell-r16 configuration, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 SCell requirements in clause 8.5.

The test parameters are given in Tables A.7.5.5.7.1-1, A.7.5.5.7.1-2 and A.7.5.5.7.1-3. There are two cell, cell 1 is the active PCell and cell 2 is the active SCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.5.7.1-1 shows the variation of the downlink SNR of the CSI-RS in set q0 in the active SCell to emulate CSI-RS based beam failure. Figure A.7.5.5.7.1-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.5.5.7.1-1: Supported test configurations for FR2 PCell and SCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |

Table A.7.5.5.7.1-2: General test parameters for FR2 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Active SCell |  |  | Cell 2 |  |
| RF Channel Number |  |  | 2 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | A.3.1.2 |
| SSB Configuration | Config 1 |  | SSB.3 FR2 | A.3.10 |
| SMTC Configuration | Config 1 |  | SMTC.3 | A.3.11 |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |  |
| PRACH Configuration | Config 1 |  | Table A.3.8.3.1-1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 in activated SCell |  |  | 0 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |
| OCNG parameters |  |  | OP.1 | A.3.2.1 |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | DRX.3 | A.3.3.3 |
| Gap pattern ID |  |  | N.A. |  |
| schedulingRequestID-BFR-SCell-r16 |  |  | Configured |  |
| Periodicity of PUCCH for SR configuration for BFR on SCell |  | slots | 40 | 5ms |
| Offset of PUCCH for SR configuration for BFR on SCell |  | slots | 5 |  |
| PUCCH parameters for SR configuration for BFR on SCell |  |  | Table 8.3.3.1.2-1 in [13] |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 in activated SCell |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB |  | dBm/SCS kHz | -94.5 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for q0 and q1 | Config 1 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD | A.3.14.2 |
| csi-RS-Index assigned as RLM RS | Config 1 |  | CSI-RS.3.2 TDD | A.3.14.2 |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 5.43 |  |
| T3 |  | s | 5.16 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.7.5.5.7.1-3: Cell specific test parameters for FR2 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Cell1 | Test 1 Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 to T5 | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | 0.2 | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q1 | Config 1 | dBm/ SCS kHz | -104.5 | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
| Noc | Config 1 | dBm/15 kHz | -104.7 | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.7.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

Figure A.7.5.5.7.1-1: SNR and L1-RSRP variation for beam failure detection and link recovery testing for SCell in DRX mode

##### A.7.5.5.7.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 in A.7.5.5.7.1 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initial link recovery. During T4 and T5 the UE measures and evaluates beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit PUCCH with LRR, followed by BFR MAC CE containing a beam associated with the candidate beam set q1. The UE shall not transmit PUCCH with an LRR with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.5.6	Active BWP switch

#### A.7.5.6.1	DCI-based and Timer-based Active BWP Switch

##### A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA

A.7.5.6.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations are shown in Table A.7.5.6.1.1.1-1 below. The test scenario comprises of one PCell (Cell 1) and one SCell (Cell 2) as given in Table A.7.5.6.1.1.1-2. NR Cell-specific parameters are specified in Table A.7.5.6.1.1.1-3 below. OTA related test parameters are shown in table A.7.5.6.1.1.1-4 below.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

UE is configured with 2 different UE-specific downlink bandwidth parts for SCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for PSCell, BWP-0 in Cell 1 before starting the test.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in SCell.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in PCell.

UE is configured with a bwp-InactivityTimer timer value for SCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for SCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in SCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of SCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell on PCell no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on SCell’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to BWP switch on SCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on SCell (Cell 2).

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the half subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of SCell’s DL slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell on PCell at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on SCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to BWP switch of SCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in SCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of SCell, respectively.

Table A.7.5.6.1.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD -TDD duplex mode |
| Note 1: Void |  |

Table A.7.5.6.1.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.7.5.6.1.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter | Unit | Cell 1 | Cell2 |
| --- | --- | --- | --- |
| Frequency Range |  | FR2 | FR2 |
| Duplex mode |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  |
| BWchannel |  | 100 MHz: NRB,c = 66 |  |
| Active BWP ID |  | 0 | 1, 2 |
| Downlink initial BWP Configuration |  | DLBWP.0.2 |  |
| Uplink initial BWP Configuration |  | ULBWP.0.2 |  |
| Downlink active BWP-0 Configuration |  | DLBWP.0.2 | - |
| Downlink active BWP-1 Configuration |  | - | DLBWP.1.1 |
| Downlink active BWP-2 Configuration |  | - | DLBWP.1.3 |
| Uplink active BWP-0 Configuration |  | ULBWP.0.2 | - |
| Uplink active BWP-1 Configuration |  | - | ULBWP.1.1 |
| Uplink active BWP-2 Configuration |  | - | ULBWP.1.3 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  |
| TRS configuration |  | TRS.2.1 TDD |  |
| TCI state |  | TCI.State.0 |  |
| RMSI CORESET parameters |  | CR.3.1 TDD |  |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |  |
| OCNG Patterns |  | OP.1 |  |
| SSB Configuration |  | SSB.1 FR2 |  |
| SMTC Configuration |  | SMTC.1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Propagation Condition |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.5.6.1.1.1-4: OTA related test parameters for BWP switching test case

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined in clause A.3.15.1 | Setup 1 defined in clause A.3.15.1 |
| Assumtion for UE beams Note 6 |  | Fine | Fine |
| Note1 | dBm/15kHz | -112 | -112 |
| Note1 | dBm/SCS | -103 | -103 |
| SS-RSRPNote2 | dBm/SCS Note3 | -85 | -85 |
|  | dB | 18 | 18 |
| IoNote4 | dBm/95.04 MHz Note4 | -56 | -56 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |

A.7.5.6.1.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for SCell on PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of PCell interruption during SCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed SCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK.

##### A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA A.7.5.6.1.2.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations are shown in Table A.7.5.6.1.2.1-1 below. The test scenario comprises of one NR PCell (Cell 1) and one NR SCell (Cell 2). The general parameters are given in Table A.7.5.6.1.2.1-2. NR Cell-specific parameters are specified in Table A.7.5.6.1.2.1-3 below. OTA related test parameters are shown in table A.7.5.6.1.2.1-4 below.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

UE is configured with 2 different UE-specific downlink bandwidth parts for SCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for PCell, BWP-0 in Cell 1 before starting the test.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in SCell.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in PCell.

UE is configured with a bwp-InactivityTimer timer value for SCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for SCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in SCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of SCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell on PCell no later than the first UL slot that occurs after the begining of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on SCell’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to BWP switch on SCell shall occur within the BWP switch delay if the UE doesn’t support per-FR gap, otherwise no interruption due to BWP switch on PCell is allowed.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on SCell (Cell 2).

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the half subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of SCell’s DL slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell on PCell at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on SCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The starting time of PCell (Cell 1) interruption due to BWP switch of SCell shall occur within the BWP switch delay if the UE doesn’t support per-FR gap, otherwise no interruption due to BWP switch on PCell is allowed.

The test equipment verifies the DL BWP switch time in SCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of SCell, respectively.

Table A.7.5.6.1.2.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode SCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | PCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode SCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | PCell: NR 30 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode SCell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.6.1.2.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 2 | Two NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.7.5.6.1.2.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  | Unit | Cell 1 | Cell2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR2 |
| Duplex mode | Config 1 |  | FDD | TDD |
|  | Config 2,3 |  | TDD |  |
| TDD configuration | Config 1 |  | Not Applicable | TDDConf.3.1 |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2 | MHz | 10 MHz: NRB,c = 52 | 100 MHz: NRB,c = 66 |
|  | Config 3 |  | 40 MHz: NRB,c = 106 |  |
| Active BWP ID |  |  | 0 | 1, 2 |
| Downlink initial BWP Configuration |  |  | DLBWP.0.2 |  |
| Uplink initial BWP Configuration |  |  | ULBWP.0.2 |  |
| Downlink active BWP-0 Configuration |  |  | DLBWP.0.2 | - |
| Downlink active BWP-1 Configuration |  |  | - | DLBWP.1.1 |
| Downlink active BWP-2 Configuration |  |  | - | DLBWP.1.3 |
| Uplink active BWP-0 Configuration |  |  | ULBWP.0.2 | - |
| Uplink active BWP-1 Configuration |  |  | - | ULBWP.1.1 |
| Uplink active BWP-2 Configuration |  |  | - | ULBWP.1.3 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.3.1 TDD |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.3.1 TDD |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.3.1 TDD |
|  | Config 2 |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 | SSB.1 FR2 |
|  | Config 3 |  | SSB.2 FR1 |  |
| TRS configuration | Config 1,2,3 |  | - | TRS.2.1 TDD |
| TCI state | Config 1,2,3 |  | TCI.State.0 | TCI.State.0 |
| SMTC Configuration |  |  | SMTC.1 |  |
| Correlation Matrix and Antenna Configuration |  |  | NA Link only, see clause A.3.7A | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Propagation Condition |  |  | NA Link only, see clause A.3.7A | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.7.5.6.1.2.1-4: OTA related test parameters for BWP switching test case

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | -NA Link only, see clause A.3.7A | Setup 1 defined in clause A.3.15.1 |
| Assumption for UE beams Note 6 |  |  | Fine |
| Note1 | dBm/15kHz |  | -112 |
| Note1 | dBm/SCS |  | -103 |
| SS-RSRPNote2 | dBm/SCS Note3 |  | -85 |
|  | dB |  | 18 |
| IoNote4 | dBm/95.04 MHz Note4 |  | -56 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |

A.7.5.6.1.2.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

If the UE doesn’t support per-FR gap,

-	During T1 and T3, the start time of SCell interruption during PCell active BWP switch shall not happen outside the BWP switch delay.

-	The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

Otherwise no interruption due to BWP switch on SCell is allowed.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA

###### A.7.5.6.1.3.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6. Supported test configurations are shown in Table A.7.5.6.1.3.1-1.

The test scenario comprises of one cell (Cell 1) as given in Table A.7.5.6.1.3.1-2. Cell-specific parameters of NR PCell is specified in Table A.7.5.6.1.3.1-3 below. The OTA related test parameters for FR2 is shown in Table A.7.5.6.1.3.1-4.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE is configured with 2 different UE-specific downlink bandwidth parts, BWP-1 and BWP-2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

-	UE is configured with a bwp-InactivityTimer timer value for Cell1.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for DL BWP switch, sent from the test equipment to the UE, is received at the UE side in Cell 1’s slot # denoted i. The UE should switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH on the first DL slot that occurs after the beginning of Cell 1’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the Cell 1 no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on Cell 1’s BWP-2 starting from the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

During T2, the test equipment won’t transmit DCI format for PDSCH reception on Cell 1.

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the half subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH on the first DL slot that occurs after the beginning of Cell 1’s DL slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the Cell 1 at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on Cell 1’s BWP-1 starting from the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The test equipment verifies the DL BWP switch time by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK is received.

Table A.7.5.6.1.3.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. Note 2: A UE which fulfils the requirements in test case A.7.5.6.1.1 or A.7.5.6.1.2 can skip the test cases in A.7.5.6.1.3. |  |

Table A.7.5.6.1.3.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| bwp-InactivityTimer | ms | [200] |  |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |

Table A.7.5.6.1.3.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Active BWP ID |  | 1, 2 |
| Initial DL BWP Configuration |  | DLBWP.0.2 Note 2 |
| Active DL BWP-1 Configuration |  | DLBWP.1.1 Note 2 |
| Active DL BWP-2 Configuration |  | DLBWP.1.3 Note 2 |
| Initial UL BWP Configuration |  | ULBWP.0.2 Note 2 |
| Active UL BWP-1 Configuration |  | ULBWP.1.1 Note 2 |
| Active UL BWP-2 Configuration |  | ULBWP.1.3 Note 2 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.1 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State |  | TCI.State.0 |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |

Table A.7.5.6.1.3.1-4: OTA related test parameters for DL BWP switch in SA

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined in clause A.3.15.1 |
| Assumption for UE beams Note 6 |  | Fine |
| NocNote 1 | dBm/15 kHz | -112 |
| NocNote 1 | dBm/SCS | -103 |
| SS-RSRP Note 2 | dBm/120 kHz Note3 | -85 |
| Ês/Iot | dB | 18 |
| Ês/Noc Note 5 | dB | 18 |
| IoNote2 | dBm/95.04 MHz Note4 | -56 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |

###### A.7.5.6.1.3.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

#### A.7.5.6.2	RRC-based Active BWP Switch

##### A.7.5.6.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA

A.7.5.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in clause 8.6.3. Supported test configurations are shown in Table A.7.5.6.2.1.1-1.

The test scenario comprises of one PCell (Cell 1) as given in Table A.7.5.6.2.1.1-2. Cell-specific parameters of PCell are specified in Table A.7.5.6.2.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1 (PCell).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PCell.

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated bandwidth part configuration, sent from the test equipment to the UE, is received at the UE side in PSCell’s slot # denoted i. The UE shall reconfigure its bandwidth part with the updated bandwidth part BWP-1 of final condition.

The UE shall be able to completely receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length as defined in clause 8.6.3 and starts to report valid ACK/NACK for the PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1. The UE shall be continuously scheduled on PSCell’s BWP-1 starting from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length.

TRRCprocessingDelay and TBWPswitchDelayRRC are defined in clause 8.6.3.

The test equipment verifies the DL BWP switch time in PSCell by counting the time from the time when the RRC Reconfiguration message including updated BWP configurationis sent till the time when RRC Reconfiguration Complete message is received.

Table A.7.5.6.2.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.6.2.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| T1 | s | [0.2] |  |

Table A.7.5.6.2.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR2 |
| Duplex mode |  |  | TDD |
| TDD configuration |  |  | TDDConf.3.1 |
| BWchannel |  |  | 100 MHz: NRB,c = 66 |
| Active BWP ID |  |  | 1 |
| Initial Condition | Active DL BWP-1 Configuration |  | DLBWP.0.2 |
|  | Active UL BWP-1 Configuration |  | ULBWP.1.3 |
| Final Condition | Active DL BWP-1 Configuration |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration |  | ULBWP.1.1 |
| 'PDSCH Reference measurement channel |  |  | SR.3.1 TDD |
| RMSI CORESET parameters |  |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  |  | CCR.3.1 TDD |
| OCNG Patterns |  |  | OP.1 |
| SSB Configuration |  |  | SSB.1 FR2 |
| SMTC Configuration |  |  | SMTC.1 |
| TCI State |  |  | TCI.State.0 |
| TRS Configuration |  |  | TRS.2.1 TDD |
| Antenna Configuration |  |  | 1x2 |
| Propagation Condition |  |  | AWGN |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |

Table A.7.5.6.2.1.1-4: OTA related test parameters for BWP switching test case

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  |  | Setup 1 according to table A.3.15 |
| Assumption for UE beams Note 5 |  |  | Fine |
| Note1 | NR_TDD_FR2_A | dBm/15kHz | -112 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| Note1 | NR_TDD_FR2_A | dBm/SCS | -103 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note3 | -85 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
|  |  | dB | 18 |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -56 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |

A.7.5.6.2.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for PCell from the first DL slot that occurs after the beginning of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for the PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs

##### A.7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA

A.7.5.6.3.1.1	Test Purpose and Environment

The purpose of this test is to verify fulfillment of DL BWP switch delay requirement defined in clauses 8.6.2A.1 and 8.6.2B.1, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations are shown in Table A.7.5.6.3.1.1-1 below. The test scenario comprises one PCell (Cell 1) and two SCells (Cell 2 and Cell 3) as given in Table A.7.5.6.3.1.1-2. NR cell-specific parameters are provided in Table A.7.5.6.3.1.1-3, and OTA related test parameters in Table A.7.5.6.3.1.1-4 below.

The test consists of three consecutive time periods with durations T1, T2 and T3, respectively.

PDCCHs indicating new transmissions shall be transmitted in PCell, SCell1 and SCell2 throughout time periods T1 and T3 to ensure that UE sends ACK/NACKs for PDSCH reception in PCell, SCell1 and SCell2. During T2, there shall be scheduling on PDSCH in PCell only.

Before the test starts,

UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 2 (SCell1) on radio channel 2 (SCC1), and Cell 3 (SCell2) on radio channel 3 (SCC2).

UE is configured with a single UE-specific downlink bandwidth part, BWP-0, for Cell 1 (PCell). BWP-0 includes the bandwidth of the initial DL BWP and SSB.

UE is configured with two different UE-specific downlink bandwidth parts, BWP-1 and BWP-2, for Cell 2 (SCell1). BWP-1 and BWP-2 include the bandwidth of the initial DL BWP and SSB.

UE is configured with two different UE-specific downlink bandwidth parts, BWP-3 and BWP-4, for Cell 3 (SCell2). BWP-3 and BWP-4 include the bandwidth of the initial DL BWP and SSB.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in PCell.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in SCell1.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-3 in SCell2.

UE is configured with a bwp-InactivityTimer timer value for SCell1 and SCell2, respectively.

All cells have constant signal levels throughout the test.

Time period T1 starts when the UE simultaneously receives DCI format 1_1 commands for DL BWP switch in SCell1 and SCell2, respectively, in a slot # denoted m. The UE shall switch its SCell1 bandwidth part from BWP-1 to BWP-2, and its SCell2 bandwidth part from BWP-3 to BWP-4. The UE shall be able to receive PDSCH in SCell1 and SCell2 starting from the first DL slot that occurs after slot (m+TMultipleBWPswitchDelay) as defined in clause 8.6.2A.1, and to transmit ACK/NACKs in SCell1 and SCell2 from the first UL slot that occurs after (m+TBWPswitchDelay+k1) where k1 is specified in [7]. The UE shall be continuously scheduled in SCell1 BWP-2 and SCell2 BWP-4 no later than in the first DL slot that occurs after slot (m+TMultipleBWPswitchDelay). The starting time of any interruption on PCell due to DL BWP switching of SCell1 and SCell2 shall occur within the BWP switching delay. The length of any interruption on PCell due to DL BWP switching of SCell1 and SCell2 shall fulfill requirements in clause 8.2.2.2.5.

Time period T2 starts when the test equipment ceases to schedule the UE on PDSCH in SCell1 and SCell2, thereby causing the bwp-InactivityTimer timers for SCell1 and SCell2 to be running until expiry.

Time period T3 starts at the beginning of the first DL half-subframe immediately after the earliest of the bwp-InactivityTimer timers expires, in a slot # denoted n. The UE shall switch its SCell1 bandwidth part from BWP-2 to BWP-1, and its SCell2 bandwidth part from BWP-4 to BWP-3. The UE shall be able to receive PDSCH in SCell1 and SCell2 starting from the first DL slot that occurs after slot (n+TMultipleBWPswitchDelay) as defined in clause 8.6.2B.1, and to transmit ACK/NACKs in SCell1 and SCell2 from the first UL slot that occurs after slot (n+TMultipleBWPswitchDelay+k1). The UE shall be continuously scheduled in SCell1 BWP-1 and SCell2 BWP-3 no later than in the first DL slot that occurs after slot (n+TMultipleBWPswitchDelay). The starting time of any interruption on PCell due to DL BWP switching of SCell1 and SCell2 shall occur within the BWP switching delay. The length of any interruption on PCell due to DL BWP switching of SCell1 and SCell2 shall fulfill requirements in clause 8.2.2.2.5.

The test equipment verifies the DL BWP switch time by counting the slots from the time when the BWP switch commands are received or bwp-InactivityTimer timers expire until ACK/NACKs are sent in SCell1 and SCell2, respectively.

The test equipment verifies that potential interruptions of PCell due to DL BWP switching on SCell1 and SCell2 are carried out within the correct time span, and are within the correct length, by monitoring ACK/NACKs sent in PCell for PCell.

Table A.7.5.6.3.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD -TDD duplex mode |

Table A.7.5.6.3.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2, 3 | Three NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell1 |  | Cell 2 | SCell1 on RF channel number 2. |
| Active SCell2 |  | Cell 3 | SCell2 on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| sCellDeactivationTimer | ms | --- | Same value applies for SCell1 and SCell2. The value infinity is applied. |
| bwp-InactivityTimer | ms | 200 | Same value applies for SCell1 and SCell2. |
| Cell2 timing offset to Cell1 | s | 0 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| Cell3 timing offset to Cell1 | s | 0 |  |
| T1 | s | 0.2 | During T1, DCI-based simultaneous BWP switching of SCell1 and SCell2 is carried out. |
| T2 | s | 0.2 | During T2 bwp-InactivityTimer timers shall run to expiry. |
| T3 | s | 0.2 | During T3, timer-based simultaneous BWP switching of SCell1 and SCell2 is carried out. |

Table A.7.5.6.3.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| Frequency Range |  | FR2 |  |  |
| NR RF channel |  | 1 | 2 | 3 |
| Duplex mode |  | TDD |  |  |
| TDD configuration |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 100: NRB,c = 66 |  |  |
| Active Downlink BWP ID |  | 0 | 1, 2 | 3, 4 |
| Downlink initial BWP Configuration |  | DLBWP.0.2 | DLBWP.0.2 | DLBWP.0.2 |
| Uplink initial BWP Configuration |  | ULBWP.0.2 | ULBWP.0.2 | ULBWP.0.2 |
| Downlink active BWP-0 Configuration |  | DLBWP.1.1 | --- | --- |
| Downlink active BWP-1 Configuration |  | --- | DLBWP.1.1 | --- |
| Downlink active BWP-2 Configuration |  | --- | DLBWP.1.3 | --- |
| Downlink active BWP-3 Configuration |  | --- | --- | DLBWP.1.1 |
| Downlink active BWP-4 Configuration |  | --- | --- | DLBWP.1.3 |
| Uplink active BWP-0 Configuration |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |
| TCI state |  | TCI.State.0 |  |  |
| RMSI CORESET parameters |  | CR.3.1 TDD |  |  |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |  |  |
| OCNG Patterns |  | OP.1 |  |  |
| SSB Configuration |  | SSB.3 FR2 |  |  |
| SMTC Configuration |  | SMTC.1 |  |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |
| Propagation Condition |  | AWGN | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.5.6.3.1.1-4: OTA related test parameters for BWP switching test case

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined in clause A.3.15.1 |  |  |
| Assumtion for UE beams Note4 |  | Fine | Fine | Fine |
| Noc Note1 | dBm/15kHz | [-111.7] | [-111.7] | [-111.7] |
| Noc Note1 | dBm/SCS | [-102.7] | [-102.7] | [-102.7] |
| Ês/Noc | dB | [7] | [7] | [7] |
| SSB-RPNote2 | dBm/SCS | [-95.7] | [-95.7] | [-95.7] |
| Ês/Iot | dB | [7] | [7] | [7] |
| IoNote3 | dBm/95.04 MHz | [-65.9] | [-65.9] | [-65.9] |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SSB-RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |

A.7.5.6.3.1.2	Test Requirements

During T1, the UE shall start to send ACK/NACKs in SCell1 and SCell2 from the first UL slot that occurs after the beginning of DL slot (m+TMultipleBWPswitchDelay+k1).

During T3, the UE shall start to send ACK/NACKs in SCell1 and SCell2 from the first UL slot that occurs after the beginning of DL slot (n+TMultipleBWPswitchDelay+k1).

.

During T1 and T3, the start of any interruption on PCell due to active BWP switching on SCell1 and SCell2 shall not happen outside the BWP switching delay TMultipleBWPswitchDelay, and the length of any interruption shall not exceed the length specified in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed active BWP switch delays in SCell1 and SCell2 to be considered correct.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.6.4	SCell dormancy switch

##### A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time

A.7.5.6.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the Dormant SCell BWP switch delay requirements are within the requirements stated in section 8.6 for UE configured with a single downlink SCell, when the dormancy indication is received in any of the first 3 OFDM symbols or is received after the first 3 OFDM symbols.

The Supported test configurations are given in Table A.7.5.6.4.1.1-1. The test parameters are given in Tables A.7.5.6.4.1.1-2 and cell-specific parameters in A.7.5.6.4.1.1-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A6 is used The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. There are two carriers both in FR2, with one cell on the PCC and 2 cells on SCC. Cell 1, Cell 2 and Cell 3 operate in either FDD or TDD duplex mode according to test configuration. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) with configured and activated SCell (SCell1) on radio channel 2 (SCC1). The UE is not aware of Cell 3 on radio channel 2 (SCC1). The UE is reporting CSI and shall not report CQI index 0 (out-of-range) in the available uplink resources to report CQI for the SCell. The UE shall be continuously scheduled in the PCell throughout the whole test.

The UE receives a DCI-based BWP switch command by which the SCell1 (Cell 2) is requested to switch the active BWP to the dormant BWP.

The point in time at which the DCI message is received at the UE antenna connector, in a subframe # denoted n, defines the start of time period T1. The UE shall accomplish the BWP switch to the dormant BWP latest in subframe (n + TBWPswitchDelay + X). The UE shall continue to shall report valid CQI if the UE has available uplink resources to report CQI for the dormant SCell. The UE shall continue to shall report L1-RSRP if the UE has available uplink resources to report L1-RSRP for the Dormant SCell. Any PCell interruption due to BWP switch on the SCell shall occur in the subframes n to (n+ TBWPswitchDelay + X).

Time T2 start at T1 + (TBWPswitchDelay + X). During T2 the UE shall continue to measure and report CQI and L1-RSRP in the available uplink resources to report CQI and L1-RSRP for the SCell.

Time T3 starts at T2 + 500ms. During T3 the UE shall continue to measure and report CQI and L1-RSRP in the available uplink resources to report CQI and L1-RSRP for the SCell.

Starting at T4 = T3 + 500ms, Cell 3 becomes detectable. During T3 the UE shall continue to measure and report CQI and L1-RSRP in the available uplink resources to report CQI and L1-RSRP for the SCell. The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T4. The UE is not required to read the neighbour cell SSB index in this test.

At time T5 starting at T4 + 1500ms a a DCI-based BWP switch command by which the SCell1 (Cell 2) is requested to switch the active BWP to the non-dormant BWP.

The point in time at which the DCI message is received at the UE antenna connector, in a subframe # denoted n, defines the start of time period T6. The UE shall accomplish the BWP switch to the non-dormant BWP latest in subframe (n + TBWPswitchDelay + X). The UE shall continue to shall report valid CQI if the UE has available uplink resources to report CQI for the non-dormant SCell. The UE shall continue to shall report L1-RSRP if the UE has available uplink resources to report L1-RSRP for the non-dormant SCell. Any PCell interruption due to BWP switch on the SCell shall occur in the subframes n to (n+ TBWPswitchDelay + X).

During T2, T3 and T4 the total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements and RRM measurements, clause 8.2.2.2.12.3, on dormant SCells, shall not exceed [0.5]%.

During T2, T3 and T4 the total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from L1-RSRP measurements and RRM measurements, clause 8.2.2.2.12.x, on dormant SCells, shall not exceed [0.5]%.

During T2, T3 and T4 the total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from RRM measurements and RRM measurements, clause 8.2.2.2.12.3, on dormant SCells, shall not exceed [0.5]%

During T1, T2, T3, T4, T5 and T6, the UE shall be continuously scheduled in the SCell1.

Table A.7.5.6.4.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.6.4.1.1-2: General test parameters for dormancy SCell in NR SA with PCell and SCell in FR2

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| PCell |  | 1 | Cell 1 |  |  |  |  |
| SCell |  | 1 | Cell 2 |  |  |  |  |
| Neighbour cell |  | 1 | Cell 3 |  |  |  | Cell to be identified. |
| RF Channel Number |  | 1 | 1 |  |  |  | cell 1 |
| RF Channel Number |  | 1 | 2 |  |  |  | Cell 2 and Cell 3 |
| Measurement gap type |  | 1 |  |  |  |  | No measurement gaps configured |
| SSB configuration |  | 1 | SSB.1 FR2 |  |  |  | for all cells |
| SMTC configuration |  | 1 | SMTC.1 |  |  |  | all cells |
| CSI-RS parameters |  | 1 | CSI-RS.3.2 FDD |  |  |  |  |
| CSI reporting periodicity, Non-dormant BWP | ms |  | 2 |  |  |  |  |
| CSI reporting periodicity, Dormant BWP | ms |  | 40 |  |  |  |  |
| Timing offset between the cells | ms |  | 0 |  |  |  |  |
| Triggering DCI format |  |  | 1_1 | 0_1 | 1_1 | 0_1 | Triggering DCI format |
| OFDM symbol range in slot for transmission of DCI with dormancy indication |  |  | 0 – 2 |  | 3 – 11 |  | Test1 and Test3 are based on that triggering DCI is received within the first three OFDM symbols of a slot. Test2 and Test4 are based on that the triggering DCI is received after the first three OFDM symbols of a slot |
| A3-Offset | dB | 1 | -4.5 |  |  |  |  |
| CP length |  | 1 | Normal |  |  |  |  |
| Hysteresis | dB | 1 | 0 |  |  |  |  |
| Time To Trigger | s | 1 | 0 |  |  |  |  |
| Filter coefficient |  | 1 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | 1 | OFF |  |  |  |  |
| T1 | s | 1 | 5 |  |  |  |  |
| T2 | s | 1 | 5 |  |  |  |  |

Table A.7.5.6.4.1.1-3: NR Cell specific test parameters for dormancy SCell in NR SA with PCell and SCell in FR2

| Parameter | Unit | Test configuration | Cell 1, Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration, Test 1,2 |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| Dedicated CORESET RMC configuration, Test 3,4 |  |  | CCR.3.2 TDD |  | CCR.3.1 TDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |
| Downlink initial BWP configuration |  | 1 | DLBWP.0.1 |  | N/A |  |
| Uplink initial BWP configuration |  | 1 | ULBWP.0.1 | N/A | N/A |  |
| Downlink active non-dormant BWP configuration |  | 1 | N/A | DLBWP.1.2 | N/A |  |
| Downlink active dormant BWP configuration |  | 1 | DLBWP.1.2 |  | N/A |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 | N/A | N/A |  |
| RLM-RS |  | 1 | CSI-RS |  | N/A |  |
| EPRE ratio of PSS to SSS | dB |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 4 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 4 |  |  |  |  |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 | -62.25 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: OCNG shall be used such that the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols |  |  |  |  |  |  |

A.7.5.6.4.1.2	Test Requirements

During T1 the UE shall switch to the dormant BWP.

During T2, T3, T4 and T5 the UE shall not send ACK/NACK for the PDSCH data scheduled on the SCell.

During T2, T3, T4 and T5 the UE shall continue to send CSI reports for SCell1 with non-zero CQI index.

During T2, T3, T4 and T5 the UE shall continue to send L1-RSRP reports for SCell.

During T4 the UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T4.

During T2, T3, T4 and T5, the missing ACK/NACK sent in PCell shall be less than 1.5% of the total number of the expected ACK/NACK.

During T6, the UE shall send ACK/NACK for the PDSCH data scheduled after subframe (n+ TBWPswitchDelay + X) for the SCell1.

All of the above test requirements shall be fulfilled in order for the observed SCell1 BWP switch delays, Pcell interruption rate, correct CSI and L1-RSRP reporting and event triggeres reporting. The rate of correct observed SCell1 hibernation delay, activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

##### A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time

A.7.5.6.4.2.1		Test Purpose and Environment

The purpose of this test is to verify fulfillment of SCell dormancy switching delay requirements in clause 8.6.2A when the UE is triggered to switch between non-dormancy and dormancy outside DRX active time. In the tested scenario, the UE is connected to PCell in FR1and two SCells in FR2, and the SCells are switched from non-dormancy to dormancy, and vice versa, at a point in time before start of onDuration. The UE is configured to monitor PDCCH for DCI format 2_6 at ps-Offset before the start of onDuration. Two tests are specified, where a UE that only supports triggering within the first three OFDM symbols of a slot shall undergo Test1 only, and a UE that supports triggering also in remaining OFDM symbols of a slot shall undergo both Test1 and Test2. In the tested scenario, ps-Offset is selected to correspond to the dormancy switching time specified in clause 8.6.2A.

The supported test configurations are provided in Table A.7.5.6.4.2.1-1 below. General test parameters are provided in Table A.7.5.6.4.2.1-2, and cell-specific parameters are provided in Table A.7.5.6.4.2.1-3 below. OTA-related test parameters are provided in Table A.7.5.6.4.2.1-4.

The tests consist of four consecutive time periods, T1, T2, T3 and T4, respectively.

Three carriers are used in the test. Cell 1 (PCell) is on RF channel 1 (PCC) in FR1, and Cell 2 (SCell1) and Cell 3 (SCell2) are on RF channels 2 (SCC1) and 3 (SCC2) in FR2, respectively. All three cells have constant signal levels throughout the test.

Before the test starts,

UE is connected to Cell 1 (PCell), Cell 2 (SCell1) and Cell 3 (SCell2).

UE is configured with a single UE-specific downlink bandwidth part, BWP-0, for Cell 1. BWP-0 includes the bandwidth of the initial DL BWP and SSB.

UE is configured with one non-dormant and one dormant UE-specific downlink bandwidth part, BWP-0 and BWP-1, respectively, for Cell 2 and Cell 3. BWP-0 includes the bandwidth of the initial DL BWP and SSB.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP in Cell 1 is BWP-0.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP in Cell 2 is BWP-0.

UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP in Cell 3 is BWP-0.

UE is configured with DRX.

UE is configured to monitor DCI format 2_6, and to be active during onDuration even when no DCI format 2_6 is detected (ps-WakeUp).

Time period T1 starts when the UE at ps-Offset before onDuration detects a DCI format 2_6 carrying dormancy indication that indicates that SCell1 and SCell2 are to be switched from non-dormancy to dormancy. The UE shall switch active bandwidth parts for SCell1 and SCell2, respectively, from non-dormant BWP-0 to dormant BWP-1. The UE shall complete the switching before the start of onDuration. The test equipment schedules the UE continuously with new data indications in PCell starting from beginning of onDuration. The test equipment verifies that the UE is transmitting HARQ feedback for PCell from the beginning of onDuration and thus verifies that the UE has completed interruptions due to dormancy switching before the start of onDuration.

Time period T2 starts when T1 is completed. The test equipment continues to schedule the UE continuously in PCell. The UE shall carry out CSI and RRM measurements on the dormant SCells. The UE shall report ACK/NACK in PCell in response to scheduled PDSCH, with the maximum loss of transmitted ACK/NACKs fulfilling the requirement in clause 8.2.2.2.12. The test equipment verifies that the loss of ACK/NACKs is no larger than 1.5%.

Time period T3 starts when T2 is completed. During T3, the test equipment does not schedule the UE, by which the inactivity timer expires and the UE stops monitoring PDCCH except for signalling using DCI format 2_6 at wake-up signalling occasions.

Time period T4 starts when the UE at ps-Offset before onDuration detects a DCI format 2_6 carrying dormancy indication that indicates that SCell1 and SCell2 are to be switched from dormancy to non-dormancy. The UE shall switch active bandwidth parts for SCell1 and SCell2, respectively, from dormant BWP-1 to non-dormant BWP-0. The UE shall complete the switching before the start of onDuration. The test equipment schedules the UE with new data indication in PCell, SCell1 and SCell2 during onDuration. The UE shall receive in PCell, SCell1 and SCell2 and send HARQ feedback for PCell, SCell1 and SCell2 via PCell. The test equipment verifies that the UE is transmitting HARQ feedback for PCell, SCell1 and SCell2 from the beginning of onDuration, and thus verifies that the UE has completed interruptions due to dormancy switching before the start of onDuration.

Table A.7.5.6.4.2.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | PCell: 15kHz SSB SCS, 10MHz bandwidth, FDD duplex mode SCells: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | PCell: 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode SCells: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode SCells: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to undergo test for one of the supported test configurations. |  |

Table A.7.5.6.4.2.1-2: General test parameters

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test1 | Test2 |  |
| NR RF Channel Number |  | 1, 2, 3 |  | Three NR radio channels are used for this test |
| Active PCell |  | Cell 1 |  | PCell on RF channel number 1 in FR1 |
| Active SCell1 |  | Cell 2 |  | SCell1 on RF channel number 2 in FR2 |
| Active SCell2 |  | Cell 3 |  | SCell2 on RF channel number 3 in FR2 |
| CSI reporting periodicity, Non-dormant BWP | ms | 2 |  | CSI reporting periodicity for periodic reporting of CQI for PCell and non-dormant SCells |
| CSI reporting periodicity, Dormant BWP | ms | 40 |  | CSI reporting periodicity for periodic reporting of CQI for dormant SCells |
| CP length |  | Normal |  |  |
| DRX |  | DRX.8 |  | For both PCell and SCells. See clause A.3.3.8. |
| ps-Offset |  | Depending on UE capability |  | Monitoring of DCI 2_6 ahead of start of drx-onDurationTimer. Value of ps-Offset shall correspond to SCell dormancy switching time for switching of two SCells, as specified in clause 8.6.2A. Actual value depends on reported UE capabilities. |
| ps-WakeUp |  | true |  | Wake up for onDuration in case DCI format 2_6 is not detected. |
| Cell 2 timing offset to Cell 1 | µs | <24 |  | Timing offset shall be less than MRTD for FR1-FR2 CA, and leave margin for timing difference between Cell2 and Cell3. |
| Cell 3 timing offset to Cell 2 | ns | <260 |  | Timing offset shall be less than MRTD for FR2 intra-band non-contiguous CA. |
| OFDM symbol range in slot for transmission of DCI with dormancy indication |  | 0 – 2 | 3 – 11 | Test1 is based on that triggering DCI is received within the first three OFDM symbols of a slot.Test2 is based on that the triggering DCI is received later than within the first three OFDM symbols of a slot. |
| T1 | s | 0.2 |  | During this time the SCells are switched from non-dormancy to dormancy. |
| T2 | s | 10 |  | During this time the SCells are dormant. |
| T3 | s | 0.2 |  | During this time the UE is not scheduled in PCell. |
| T4 | s | 0.2 |  | During this time the SCells are switched from dormancy to non-dormancy. |

Table A.7.5.6.4.2.1-3: Cell specific test parameters

| Parameter |  | Unit | Cell 1 | Cell2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR2 | FR2 |
| NR RF channel |  |  | 1 | 2 | 3 |
| Duplex mode | Config 1 |  | FDD | TDD | TDD |
|  | Config 2,3 |  | TDD |  |  |
| TDD configuration | Config 1 |  | --- | TDDConf.3.1 | TDDConf.3.1 |
|  | Config 2 |  | TDDConf.1.1 |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |
| BWchannel | Config 1,2 | MHz | 10: NRB,c = 52 | 100: NRB,c = 66 | 100: NRB,c = 66 |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |
| Downlink initial BWP Configuration |  |  | DLBWP.0.2 | DLBWP.0.2 | DLBWP.0.2 |
| Uplink initial BWP Configuration |  |  | ULBWP.0.2 | --- | --- |
| Downlink active non-dormant BWP-0 Configuration |  |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| Downlink active dormant BWP-1 Configuration |  |  | --- | DLBWP.1.1 | DLBWP.1.1 |
| Uplink active BWP-0 Configuration |  |  | ULBWP.0.2 | --- | --- |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.3.1 TDD | SR.3.1 TDD |
|  | Config 2 |  | SR.1.1 TDD |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |
| CSI-RS configuration for CSI reporting, Non-dormant BWP | Config 1 |  | CSI-RS.1.1 FDD | CSI-RS.3.1 TDD | CSI-RS.3.1 TDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |
| CSI-RS configuration for CSI reporting, Dormant BWP |  |  | --- | CSI-RS.3.5 TDD | CSI-RS.3.5 TDD |
| TRS configuration | Config 1 |  | TRS.1.1 FDD | TRS.2.1 TDD | TRS.2.1 TDD |
|  | Config 2 |  | TRS.1.1 TDD |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |
| TCI state |  |  | TCI.State.0 | TCI.State.0 | TCI.State.0 |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | --- | --- |
|  | Config 2 |  | CR.1.1 TDD |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |
| Dedicated CORESET parameters, Test 1,2 | Config 1 |  | CCR.1.1 FDD | CCR.3.1 TDD | CCR.3.1 TDD |
|  | Config 2 |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |
| Dedicated CORESET parameters, Test 3,4 | Config 1 |  | CCR.1.5 FDD | CCR.3.1 TDD | CCR.3.1 TDD |
|  | Config 2 |  | CCR.1.5 TDD |  |  |
|  | Config 3 |  | CCR.2.3 TDD |  |  |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 | SSB.1 FR2 | SSB.1 FR2 |
|  | Config 3 |  | SSB.2 FR1 |  |  |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  |  |  |
| Propagation Condition |  |  | N/A Link only, see clause A.3.7A | AWGN | AWGN |
| Note 1: OCNG shall be used such that the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |

Table A.7.5.6.4.2.1: OTA related test parameters

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration |  | N/A Link only, see clause A.3.7A | Setup 1 defined in clause A.3.15.1 |  |
| Assumtion for UE beams Note6 |  |  | Fine | Fine |
| Noc Note1 | dBm/15kHz |  | -112 | -112 |
| Noc Note1 | dBm/SCS |  | -103 | -103 |
| SS-RSRPNote2 | dBm/SCS Note3 |  | -85 | -85 |
| Ês/Iot | dB |  | 18 | 18 |
| IoNote4 | dBm/95.04 MHz Note4 |  | -56 | -56 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |

###### A.7.5.6.4.2.2		Test Requirements

Starting from onDuration in time period T1, the UE shall transmit ACK/NACK in response to scheduling in PCell. There shall be no loss of ACK/NACK.

During time period T2, the UE shall transmit ACK/NACKs in response to scheduling in PCell and the rate of missed ACK/NACKs shall be no more than 1.5%.

Starting from onDuration in time period T4, the UE shall transmit ACK/NACK in response to scheduling in PCell, SCell1 and SCell2. There shall be no loss of ACK/NACK.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs

##### A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA

A.7.5.6.5.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement for simultaneous RRC-based BWP switch on multiple CCs defined in clause 8.6.3A.

The supported test configurations are shown in Table A.7.5.6.5.1.1-1. The test scenario comprises one PCell (Cell 1) and one SCell (Cell 2) as given in Table A.7.5.6.5.1.1-2. NR cell-specific parameters are provided in Table A.7.5.6.5.1.1-3, and OTA related test parameters in Table A.7.5.6.5.1.1-4.

PDCCHs indicating new transmissions shall be transmitted in PCell and SCell throughout to ensure that UE sends ACK/NACKs for PDSCH reception in PCell, SCell.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), to Cell 2 (SCell) on radio channel 2 (SCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell1 (PCell), Cell 2 (SCell).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition on Cell1 (PCell), Cell 2 (SCell).

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated bandwidth part configuration in Cell1 and Cell2, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its bandwidth part with the updated bandwidth part BWP-1 of final condition in Cell1 and Cell2.

The UE shall be able to receive PDSCH on Cell 1 and Cell 2 at the beginning of the DL slot right after PCell’s DL slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length) as defined in clause 8.6.3A and be ready for the reception of uplink grant for the PCell no later than at the beginning of the DL slot right after slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length). The UE shall be continuously scheduled on Cell 1’s BWP-1and Cell 2’s BWP-1 starting from the beginning of the DL slot right after slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length).

TRRCprocessingDelay , TBWPswitchDelayRRC and DRRC are defined in clause 8.6.3A.

The test equipment verifies the DL BWP switch time in Cell 1 and Cell 2 by counting the time from the time when the RRC Reconfiguration message including updated BWP configuration is sent till the time when RRC Reconfiguration Complete message is received.

Table A.7.5.6.5.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD -TDD duplex mode |

Table A.7.5.6.5.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2, 3 | Three NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell2 timing offset to Cell1 | s | 0 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | [0.2] | During T1, RRC-based simultaneous BWP switching of PCell and SCell is carried out. |

Table A.7.5.6.5.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR2 |  |  |  |  |  |
| NR RF channel |  |  |  | 1 |  |  | 2 |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |
| Active Downlink BWP ID |  |  |  | 0 |  |  | 1 |  |  |
| Downlink initial BWP Configuration |  |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Uplink initial BWP Configuration |  |  |  | ULBWP.0.2 |  |  | ULBWP.0.2 |  |  |
| Initial Condition | Active DL BWP-1 Configuration |  |  |  | DLBWP.1.3 |  | DLBWP.1.3 |  |  |
|  | Active UL BWP-1 Configuration |  |  |  | ULBWP.1.3 |  | ULBWP.1.3 |  |  |
| Final Condition | Active DL BWP-1 Configuration |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  |  |
|  | Active UL BWP-1 Configuration |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  |  |
| PDSCH Reference measurement channel |  |  |  | SR.3.1 TDD |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  |  |  | TCI.State.0 |  |  |  |  |  |
| RMSI CORESET parameters |  |  |  | CR.3.1 TDD |  |  |  |  |  |
| Dedicated CORESET parameters |  |  |  | CCR.3.1 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |  |  |  |
| Propagation Condition |  |  |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |  |  |  |  |

Table A.7.5.6.3.1.1-4: OTA related test parameters for BWP switching test case

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined in clause A.3.15.1 |  |
| Assumtion for UE beams Note4 |  | Fine | Fine |
| Noc Note1 | dBm/15kHz | [-111.7] | [-111.7] |
| Noc Note1 | dBm/SCS | [-102.7] | [-102.7] |
| Ês/Noc | dB | [7] | [7] |
| SSB-RPNote2 | dBm/SCS | [-85] | [-85] |
| Ês/Iot | dB | [18] | [18] |
| IoNote3 | dBm/95.04 MHz | [-56] | [-56] |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SSB-RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the center of the quiet zone Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |

A.7.5.6.5.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for PCell and SCell in the beginning of the DL slot right after slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length).

All of the above test requirements shall be fulfilled in order for the observed PCell and SCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.5.7	PSCell addition and release delay

#### A.7.5.7.1	Addition and Release Delay of known NR PSCell

##### A.7.5.7.1.1	Test Purpose and Environment

The purpose of this test is to verify the PSCell addition and release delay requirements defined in clauses 8.9.2 and 8.9.3, respectively, for the case where the PSCell is known to the UE at the time of addition.

The supported test configurations are given in Table A.7.5.7.1.1-1. The test scenario comprises two NR cells, Cell 1 and Cell 2, on radio channel 1 in FR1 and radio channel 2 in FR2, respectively. Test parameters are given in Tables A.7.5.7.1.1-2, A.7.5.7.1.1-3 and A.7.5.7.1.1-4 below. The test consists of five time periods with durations T1, T2, T3, T4 and T5, respectively.

At the start of T1, the UE shall be connected to Cell 1 (PCell) on radio channel 1 (PCC) and shall only monitor PCC and hence be unaware of Cell 2 (PSCell-to-be) on radio channel 2. Before the start of T2, the test system shall send measurement control information including measurement gap configuration and event-triggered reporting configuration for measurements on radio channel 2.

During T2, the UE shall identify Cell 2 and send an event-triggered report. When the tests system receives the report, it shall send updated measurement control information where the measurement gap pattern is released. Before the start of T3, the test system shall send a RRC message instructing the UE to add PSCell (Cell 2), and further instructing the UE to report CSI periodically in the PSCell once it has been added. Reception by the UE of this RRC message defines the start of T3.

During T3, the UE shall carry out random access towards the PSCell. Reception by the test system of the PRACH preamble defines the start of T4.

During T4, the UE shall send periodic CSI reports in PSCell. After having received at least one such report, the test system shall send a RRC message instructing the UE to release the PSCell. Reception by the UE of the RRC message defines the start of T5.

During T5, the UE shall release the PSCell.

Table A.7.5.7.1.1-1: Supported test configurations for FR2 PSCell

| Config | Description |
| --- | --- |
| 1 | FR1 FDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 2 | FR1 TDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 3 | FR1 TDD SSB SCS 30kHz BW 40MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.7.1.1-2: General test parameters for PSCell addition and release delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| Neighbour cell |  |  | Cell 2 | Neighbour cell (PSCell-to-be) on RF channel number 2 in FR2 |
| A4 | Hysteresis | dB | 0 | Hysteresis for event A4 |
|  | Threshold RSRP | dBm | -118 | Threshold for event A4 |
|  | Time to Trigger | S | 0 | Time to trigger for event A4 |
| DRX |  |  | OFF | For both PCell and PSCell once activated |
| Measurement gap pattern ID |  |  | 0 | Gaps are configured before T2 and released before T3. |
| PRACH configuration in Cell 2 |  |  | FR2 PRACH configuration 2 | PRACH configuration as specified in Clause A.3.8.3.2. |
| CSI reporting periodicity and offset configuration for Cell 2 |  | ms | 2 |  |
| T1 |  | s | 5 | During this time the PCell is known and Cell 2 is unknown. |
| T2 |  | s | 1 | During this time the UE shall identify neighbour cell 2 and report event B1. |
| T3 |  | s | 1 | During this time the UE adds the PSCell. |
| T4 |  | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T5 |  | s | 1 | During this time the UE releases the PSCell. |

Table A.7.5.7.1.1-3: NR Cell specific test parameters for PSCell addition and release delay

| Parameter | Unit | Config | Cell 1 | Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| Frequency Range |  | 1,2,3 | FR1 | FR2 |  |  |  |  |
| Duplex mode |  | 1 | FDD | TDD |  |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  | 1 | – | TDDConf.3.1 |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 10: NRB,c = 52 | 100: NRB,c = 66 |  |  |  |  |
|  |  | 3 | 40: NRB,c = 106 |  |  |  |  |  |
| Data RBs allocated |  | 1,2 | 52 | 48 |  |  |  |  |
|  |  | 3 | 106 |  |  |  |  |  |
| Initial Downlink BWP configuration |  | 1,2,3 | DLBWP.0.1 | DLBWP.0.1 |  |  |  |  |
| Initial Uplink BWP configuration |  | 1,2,3 | ULBWP.0.1 | ULBWP.0.1 |  |  |  |  |
| Dedicated Downlink BWP configuration |  | 1,2,3 | DLBWP.1.1 | DLBWP.1.1 |  |  |  |  |
| Dedicated Uplink BWP configuration |  | 1,2,3 | ULBWP.1.1 | ULBWP.1.1 |  |  |  |  |
| PDSCH Reference Measurement Channel |  | 1 | SR.1.1 FDD | SR.3.3 TDD |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| TRS configuration |  | 1,2,3 | – | TRS.2.1 TDD |  |  |  |  |
| TCI state |  | 1,2,3 | – | TCI.State.0 |  |  |  |  |
| RMSI CORESET parameters |  | 1 | CR.1.1 FDD | CR.3.2 TDD |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET parameters |  | 1 | CCR.1.1 FDD | CCR.3.7 TDD |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |  |
| OCNG PatternsNote1 |  | 1,2,3 | OP.1 | OP.3 |  |  |  |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 | SSB.2 FR2 |  |  |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |  |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 | SMTC.1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 15 | 120 |  |  |  |  |
|  |  | 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| Propagation Condition |  | 1,2,3 | N/A | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: Void |  |  |  |  |  |  |  |  |

Table A.7.5.7.1.1-4: OTA related test parameters for PSCell addition and release delay

| Parameter | Unit | Config | Cell 1 | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| Angle of arrival configuration |  | 1,2,3 | Link only, see clause A.3.7A | Setup 2a according to clause A.3.15.2.1 |  |  |  |  |
| Assumption for UE beams Note 3 |  |  |  | Rough |  |  |  |  |
| Ês | dBm/SCS | 1,2,3 |  | -∞ | -81 |  |  |  |
| SSB_RP Note1, Note2 | dBm/SCS | 1,2,3 |  | -∞ | -81 |  |  |  |
| BB Note1, Note 4 | dB | 1,2,3 |  | -∞ | 4.88 |  |  |  |
| Io Note 1, Note2 | dBm/95.04 MHz | 1,2,3 |  | N/A | -56.41 |  |  |  |
| Note 1: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone. Note 3: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

###### A.7.5.7.1.2	Test Requirements

The UE shall transmit the PRACH preamble to PSCell at latest 112 ms into T3.

The UE shall transmit at least one periodic CSI report for PSCell during T4.

The UE shall stop transmitting CSI reports for PSCell at latest 20 ms into T5.

All of the above test requirements shall be fulfilled in order for the observed PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.5.7.2	Addition and Release Delay of unknown NR PSCell

##### A.7.5.7.2.1	Test Purpose and Environment

The purpose of this test is to verify the PSCell addition and release delay requirements defined in clauses 8.9.2 and 8.9.3, respectively, for the case where the PSCell is unknown to the UE at the time of addition.

The supported test configurations are given in Table A.7.5.7.2.1-1. The test scenario comprises two NR cells, Cell 1 and Cell 2, on radio channel 1 in FR1 and radio channel 2 in FR2, respectively. Test parameters are given in Tables A.7.5.7.2.1-2, A.7.5.7.2.1-3 and A.7.5.7.2.1-4 below. The test consists of four time periods with durations T1, T2, T3 and T4, respectively.

At the start of T1, the UE shall be connected to Cell 1 (PCell) on radio channel 1 (PCC) and shall only monitor PCC and hence be unaware of Cell 2 (PSCell-to-be) on radio channel 2. At the end of T1, the test system shall send a RRC message instructing the UE to add PSCell (Cell 2), and further instructing the UE to report CSI periodically in the PSCell once it has been added. Reception by the UE of this RRC message defines the start of T2.

During T2, the UE shall identify PSCell and carry out random access towards the PSCell. Reception by the test system of the PRACH preamble defines the start of T3.

During T3, the UE shall send periodic CSI reports in PSCell. After having received at least one such report, the test system shall send a RRC message instructing the UE to release the PSCell. Reception by the UE of the RRC message defines the start of T4.

During T4, the UE shall release the PSCell.

Table A.7.5.7.2.1-1: Supported test configurations for FR2 PSCell

| Config | Description |
| --- | --- |
| 1 | FR1 FDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 2 | FR1 TDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 3 | FR1 TDD SSB SCS 30kHz BW 40MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.7.2.1-2: General test parameters for PSCell addition and release delay

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| Neighbour cell |  | Cell 2 | Neighbour cell (PSCell-to-be) on RF channel number 2 in FR2 |
| DRX |  | OFF | For both PCell and PSCell once activated |
| PRACH configuration in Cell 2 |  | FR2 PRACH configuration 2 | PRACH configuration as specified in Clause A.3.8.3.2. |
| CSI reporting periodicity and offset configuration for Cell 2 | ms | [2] |  |
| T1 | s | 5 | During this time the PCell is known and Cell 2 is unknown. |
| T2 | s | 1 | During this time the UE adds the PSCell. |
| T3 | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T4 | s | 1 | During this time the UE releases the PSCell. |

Table A.7.5.7.2.1-3: NR Cell specific test parameters for PSCell addition and release delay

| Parameter | Unit | Config | Cell 1 | Cell2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 |
| Frequency Range |  | 1,2,3 | FR1 | FR2 |  |  |  |
| Duplex mode |  | 1 | FDD | TDD |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |
| TDD configuration |  | 1 | – | TDDConf.3.1 |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel | MHz | 1,2 | 10: NRB,c = 52 | 100: NRB,c = 66 |  |  |  |
|  |  | 3 | 40: NRB,c = 106 |  |  |  |  |
| Data RBs allocated |  | 1,2 | 52 | 48 |  |  |  |
|  |  | 3 | 106 |  |  |  |  |
| Initial Downlink BWP configuration |  | 1,2,3 | DLBWP.0.1 | DLBWP.0.1 |  |  |  |
| Initial Uplink BWP configuration |  | 1,2,3 | ULBWP.0.1 | ULBWP.0.1 |  |  |  |
| Dedicated Downlink BWP configuration |  | 1,2,3 | DLBWP.1.1 | DLBWP.1.1 |  |  |  |
| Dedicated Uplink BWP configuration |  | 1,2,3 | ULBWP.1.1 | ULBWP.1.1 |  |  |  |
| PDSCH Reference Measurement Channel |  | 1 | SR.1.1 FDD | SR.3.3 TDD |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |
| TRS configuration |  | 1,2,3 | – | TRS.2.1 TDD |  |  |  |
| TCI state |  | 1,2,3 | – | TCI.State.0 |  |  |  |
| RMSI CORESET parameters |  | 1 | CR.1.1 FDD | CR.3.2 TDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET parameters |  | 1 | CCR.1.1 FDD | CCR.3.7 TDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |
| OCNG PatternsNote1 |  | 1,2,3 | OP.1 | OP.3 |  |  |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 | SSB.2 FR2 |  |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 15 | 120 |  |  |  |
|  |  | 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |
| Propagation Condition |  | 1,2,3 | AWGN | AWGN |  |  |  |
| Note 1: OCNG shall be used such that and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: Void |  |  |  |  |  |  |  |

Table A.7.5.7.2.1-4: OTA related test parameters for PSCell addition and release delay

| Parameter | Unit | Config | Cell 1 | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  | 1,2,3 | Link only, see clause A.3.7A | Setup 2a according to clause A.3.15.2.1 |  |  |  |
| Assumption for UE beams Note 3 |  |  |  | Rough |  |  |  |
| Ês | dBm/SCS | 1,2,3 |  | -∞ | -81 |  |  |
| SSB_RP Note1, Note 2 | dBm/SCS | 1,2,3 |  | -∞ | -81 |  |  |
| BB Note1, Note 4 | dB | 1,2,3 |  | -∞ | 4.88 |  |  |
| Io Note 1, Note 2 | dBm/95.04 MHz | 1,2,3 |  | N/A | -56.41 |  |  |
| Note 1: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone. Note 3: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

##### A.7.5.7.2.2	Test Requirements

The UE shall transmit the PRACH preamble to PSCell at latest 572 ms into T2.

The UE shall transmit at least one periodic CSI report for PSCell during T3.

The UE shall stop transmitting CSI reports for PSCell at latest 20 ms into T4.

All of the above test requirements shall be fulfilled in order for the observed PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.5.8	Active TCI state switch delay

#### A.7.5.8.1	MAC-CE based active TCI state switch

##### A.7.5.8.1.1	NR PCell FR2 active TCI state switch for a known TCI state

A.7.5.8.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the active TCI state switch delay requirement defined in clause 8.10.3. Supported test configuration is shown in Table A.7.5.8.1.1.1-1.

The test scenario comprises of one NR PCell (Cell 1) as given in Table A.7.5.8.1.1.1-2. Cell-specific parameters of NR PCell are specified in Table A.7.5.8.1.1.1-3 below. The OTA related test parameters for FR2 are shown in Table A.7.5.8.1.1.1-4.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is configured with 2 different TCI states for PCell, PDCCH TCI state 0 (QCL’d to SSB0) and TCIstate 1 (QCL’d to SSB1), in Cell 1 before starting the test.

-	UE is indicated in TCI state 0 as the active PDCCH TCI state

The test consists of two time periods, T1 and T2. During T1 only SSB to which PDCCH-TCI-state0 is QCL’d is transmitted. At the beginning of T2, the SSB corresponding to TCI state 1 starts transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms of UE providing L1-RSRP report with results for both SSB0 and SSB1, UE receives a MAC-CE command indicating a switch to TCI state 1. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

The test equipment verifies that UE can be scheduled on PCell on TCI state 0 till n+ THARQ +3 ms. The test equipment also verifies the TCI state switch time in PCell by scheduling the UE on TCI state 1 after n+ THARQ +3 ms + (Tfirst-SSB + TSSB-proc).

Table A.7.5.8.1.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.8.1.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |

Table A.7.5.8.1.1.1-3: NR Cell specific test parameters for TCI state switch

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Data RBs allocated |  | 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.5 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State 0 |  | TCI.State.0 |
| TCI State 1 |  | TCI.State.1 |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.8.1.1.1-4: OTA related test parameters for TCI state switch

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |  |
|  |  | T1 | T2 | T1 |  | T2 |
| Angle of arrival configuration |  | Setup 3 according to clause A.3.15.3 |  |  |  |  |
|  |  | AoA1 |  |  | AoA2 |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |
| Ês | dBm/SCS | -80.6 | -80.6 | -Infinity |  | -80.6 |
| SSB_RP Note 2 | dBm/ SCS | -80.6 | -80.6 | -Infinity |  | -80.6 |
| BB Note 7 | dB | 8.3 | 8.3 | -Infinity |  | 8.3 |
| IoNote2 | dBm/95.04 MHz Note4 | -56.0 | -56.0 | -Infinity |  | -56.0 |
| Note 1: Void Note 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

A.7.5.8.1.1.2	Test Requirements

During T2, UE shall send L1-RSRP report with results for both SSB0 and SSB1.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive on TCI state 0 till   n+ THARQ +3 ms

-	be able to start receiving on TCI state 1 after n+ THARQ +5 ms + Tfirst-SSB

#### A.7.5.8.2	RRC based active TCI state switch

##### A.7.5.8.2.1	NR PCell FR2 active TCI state switch for a known TCI state

A.7.5.8.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the active TCI state switch delay requirement defined in clause 8.10.3. Supported test configuration is shown in Table A.7.5.8.2.1.1-1.

The test scenario comprises of one NR PCell as given in Table A.7.5.8.2.1.1-2. Cell-specific parameters of NR PCell is specified in Table A.7.5.8.2.1.1-3 below. The OTA related test parameters for FR2 is shown in Table A.7.5.8.2.1.1-4.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is configured with 1 TCI state for PCell, PDCCH-TCI-state0 (QCL’d to SSB0)

-	UE is indicated in TCI state0 as the active TCI state

The test consists of two time periods, T1 and T2. During T1 only SSB to which TCI-state0 is QCL’d is transmitted. At the beginning of T2, the SSB corresponding to TCI-state1 starts transmitting. The UE is configured to provide periodic L1-RSRP reports.  In slot n which is within 1280 ms of UE providing L1-RSRP report with results for both SSB0 and SSB1, UE receives a RRC command indicating a switch to TCI-state1.

The test equipment verifies the TCI state switch time in PCell by scheduling the UE on TCI state 1 after n+ TRRC_processing  + Tfirst-SSB + 2ms.

Table A.7.5.8.2.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.8.2.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |
| T2 | s | 2 |  |

Table A.7.5.8.2.1.1-3: NR Cell specific test parameters for TCI state switch

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Data RBs allocated |  | 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.5 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State 0 |  | TCI. State.0 |
| TCI State 1 |  | TCI.State.1 |
| reportConfigType |  | ssb-Index-RSRP |
| reportConfigType |  | periodic |
| Number of reported RS |  | 2 |
| L1-RSRP reporting period | slot | 640 |
| timeRestrictionForChannelMeasurements |  | configured |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.8.2.1.1-4: OTA related test parameters for TCI state switch

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |  |
|  |  | T1 | T2 | T1 |  | T2 |
| Angle of arrival configuration |  | Setup 3 according to clause A.3.15.3 |  |  |  |  |
|  |  | AoA1 |  |  | AoA2 |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |
| Ês | dBm/SCS | -80.6 | -80.6 | -Infinity |  | -80.6 |
| SSB_RP Note 2 | dBm/ SCS | -80.6 | -80.6 | -Infinity |  | -80.6 |
| BB Note 7 | dB | 8.3 | 8.3 | -Infinity |  | 8.3 |
| IoNote2 | dBm/95.04 MHz Note4 | -56.0 | -56.0 | -Infinity |  | -56.0 |
| Note 1: Void Note 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

A.7.5.8.2.1.2	Test Requirements

During T2, UE shall send L1-RSRP report with both SSB0 and SSB1.

After receiving RRC command in slot n, UE shall be able to start receiving on TCI state 1 after n+ TRRC_processing  + Tfirst-SSB + 2ms.

### A.7.5.9	Uplink spatial relation switch delay

#### A.7.5.9.1	MAC-CE based Spatial Relation switch

##### A.7.5.9.1.1		NR PCell FR2 spatial relation associated with known DL-RS

A.7.5.9.1.1.1	Test Purpose and Environment

The purpose of this test is to verify fulfillment of the uplink spatial relation switch delay requirement defined in clause 8.12.3 by a UE capable of beam correspondence without the need for UL beam sweeping. The supported test configurations are shown in Table A.7.5.9.1.1.1-1.

The test scenario comprises one PCell (Cell 1) as outlined in Table A.7.5.9.1.1.1-2. Cell-specific parameters are provided in Table A.7.5.9.1.1.1-3. OTA-related test parameters are provided in Table A.7.5.9.1.1.1-4.

Throughout the test, PDCCH indicating new transmissions shall ge sent continuously on PCell to ensure that the UE will send ACK/NACKs on PUCCH.

Before the test starts,

UE is connected to Cell 1 on radio channel 1.

UE is configured with a single TCI state, TCI State-0, which is QCLed with SSB0.

UE is configured with two spatial relation information configurations Spatial Relation Info-0 and Spatial Relation Info-1 for PUCCH, each associated with SSB0 and SSB1, respectively.

UE is indicated via MAC-CE activation of PUCCH-SpatialRelationInfoId corresponding to Spatial Relation Info-0

UE is configured with a CSI measurement configuration indicating L1-RSRP measurements on SSB0 and SSB1 with periodic reporting. The L1-RSRP measurement period is influenced by the following: the higher layer parameter timeRestrictionForChannelMeasurement is configured, measured SSBs are fully overlapping with SMTC window, and there are no conflicts with measurement gaps.

The test consists of two time periods, T1 and T2. During T1 only the SSB associated with PDCCH TCI state-0 and PUCCH Spatial Relation Info-0 is transmitted. At the beginning of T2, transmission of the SSB associated with PUCCH Spatial Relation Info-1 starts. The UE conducts periodic L1-RSRP measurements and SSB-Index-RSRP reporting for SSB0 and SSB1. In slot n, which is within 1280ms after UE receiving both SSB0 and SSB1, and after reporting valid results for both the SSB0 and the SSB1, the UE receives a MAC-CE indicating a switch of spatial relation to PUCCH Spatial Relation Info 1

The test equipment verifies that the UE transmits according to PUCCH Spatial Relation Info 0 up until slot n + THARQ/NR slot length + 3Nslotsubframe,µ, and according to PUCCH Spatial Relation Info 1 from slot n + THARQ/NR slot length + 3Nslotsubframe,µ + 1 and onwards.

Table A.7.5.9.1.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.9.1.1.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 160 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0, SSB1 | L1-RSRP measurements of SSB0 and SSB1. |
| Number of reported RS |  | 2 | L1-RSRP reporting of measurements on SSB0 and SSB1. |
| T1 | s | [0.2] |  |
| T2 | s | [2] |  |

Table A.7.5.9.1.1.1-3: NR Cell specific test parameters

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.1 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State-0 Configuration |  | TCI.State.0 |
| reportConfigType |  | ssb-Index-RSRP |
| reportConfigType |  | periodic |
| timeRestrictionForChannelMeasurements |  | configured |
| TRS Configuration |  | TRS.2.1 TDD |
| Spatial Relation Info-0 Configuration |  | PUCCH.SRI.0 |
| Spatial Relation Info-1 Configuration |  | PUCCH.SRI.1 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.9.1.1.1-4: OTA related test parameters

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |  |
|  |  | T1 | T2 | T1 | T2 |  |
| Angle of arrival configuration |  | Setup 3 according to clause A.3.15.3 |  |  |  |  |
|  |  | AoA1 |  | AoA2 |  |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |
| NocNote 1 | dBm/15 kHz | -92.1 |  |  |  |  |
| NocNote 1 | dBm/SCS | -83.1 |  |  |  |  |
| Ês/Noc | dB | 1 |  | -infinity |  | 1 |
| SS-RSRP Note 2 | dBm/120 kHz Note3 | -82.1 |  | -infinity |  | -82.1 |
| IoNote2 | dBm/95.04 MHz Note4 | -50.6 |  | -54.1 |  | -50.6 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |

###### A.7.5.9.1.1.2	Test Requirements

During T2, the UE shall send L1-RSRP report with results for SSB0 and SSB1.

After receiving MAC-CE command in slot n, the UE shall:

Continue transmitting using PUCCH spatial relation associated with SSB0 up to and including slot n + THARQ/NR slot length + 3Nslotsubframe,µ

Start transmitting using PUCCH spatial relation associated with SSB1 from slot n + THARQ/NR slot length + 3Nslotsubframe,µ + 1 and onwards.

The rate of correct events observed during repeated tests shall be at least [90]%.

#### A.7.5.9.2	RRC based spatial relation switch

##### A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS

###### A.7.5.9.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the RRC based spatial relation switch delay requirement defined in clause 8.12.5 by a UE capable of beam correspondence without the need for UL beam sweeping. In the test the higher layer parameter timeRestrictionForChannelMeasurements is configured. Supported test configuration is shown in Table A.7.5.9.2.1.1-1.

The test scenario comprises of one PCell (Cell 1) as given in Table A.7.5.9.2.1.1-2. Cell-specific parameters of PCell is specified in Table A.7.5.9.2.1.1-3 below. The OTA related test parameters for FR2 is shown in Table A.7.5.9.2.1.1-4.

Periodic SRS is transmitted on PCell (Cell 1), and the SRS configuration is SRSConf.1 given in Table A.5.4.1.1.1-3.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is configured with 1 SRS-SpatialRelation0 associated with SSB0.

-	UE is indicated SRS-SpatialRelation0 as the active SRS spatial relation.

The test consists of two time periods, T1 and T2. During T1 only SSB0 to which SRS-SpatialRelation0 associated is transmitted. UE shall transmit periodic SRS with SRS-SpatialRelation0 on the UL of the PCell.

T2 start when the tester initiates transmission of SSB1 corresponding to SRS-SpatialRelation1. The UE is configured to transmit periodic L1-RSRP reports.

In slot n, which is within [1280]ms of UE providing the L1-RSRP report with results for both SSB0 and SSB1, the UE receives an RRC command indicating a switch to transmit periodic SRS with target SRS-SpatialRelation1. The UE shall be able to transmit periodic SRS with target spatial relation (SRS-SpatialRelation1) on PCell in slot n + TRRC_processing/NR slot length +1.

Table A.7.5.9.2.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.9.2.1.1-2: General test parameters for spatial relation switch associated with a known DL-RS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| timeRestrictionForChannelMeasurements |  | configured | Time domain measurement restriction for the channel (signal) measurements (see TS 38.214 [19], clause 5.2.1.1) |
| T1 | s | 0.5 |  |
| T2 | s | 1.5 |  |

Table A.7.5.9.2.1.1-3: NR Cell specific test parameters for spatial relation switch associated with a known DL-RS

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.1 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| SRS-SpatialRelation0 |  | SRS.SRI0 |
| SRS-SpatialRelation1 |  | SRS.SRI1 |
| reportConfigType |  | ssb-Index-RSRP |
| reportConfigType |  | periodic |
| Number of reported RS |  | 2 |
| L1-RSRP reporting period | slot | 160 |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.9.2.1.1-4: OTA related test parameters for spatial relation switch associated with a known DL-RS

| Parameter | Unit | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |
|  |  | T1 | T2 | T1 | T2 |
| Angle of arrival |  | Setup 3 according to clause A.3.15.3 |  |  |  |
| configuration |  | AoA1 |  | AoA2 |  |
| Assumption for UE beamsNote 6 |  | Rough |  | Rough |  |
| NocNote 1 | dBm/15 kHz | -92.1 |  |  |  |
| NocNote 1 | dBm/SCS | -83.1 |  |  |  |
| Ês/Noc | dB | 1 | 1 | -Infinity | 1 |
| SS-RSRP Note 2 | dBm/120 kHz Note3 | -82.1 | -82.1 | -Infinity | -82.1 |
| IoNote2,Note6 | dBm/95.04 MHz Note4 | -50.6 | -50.6 | -54.1 | -50.6 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

###### A.7.5.9.2.1.2	Test Requirements

During T1 UE shall send L1-RSRP report with SSB0 to which SRS-SpatialRelation0 is associated. During T2, UE shall send L1-RSRP report with SSB1 to which SRS-SpatialRelation1 is associated.

After receiving RRC command in slot n, UE shall be able to transmit target periodic SRS with SRS-SpatialRelation1 on the PCell in the slot n +  TRRC_processing/NR slot length + 1.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.5.10	UE specific CBW change

#### A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA

##### A.7.5.10.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE specific CBW change delay requirement defined in clause 8.13. Supported test configurations are shown in Table A.7.5.10.1.1-1.

The test scenario comprises of one PCell (Cell 1) as given in Table A.7.5.10.1.1-2. Cell-specific parameters of PCell are specified in Table A.7.5.10.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK transmission.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1 (PCell).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PCell.

-	UE has been configured with UE-specific CBW (CBW-1)

-	UE is indicated in SCS-SpecificCarrier [2] that the UE-specific CBW is CBW-1 as the initial condition in Cell 1 (PCell).

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated CBW configuration, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its CBW with the updated CBW of final condition.

The UE shall be able to completely receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length as defined in clause 8.13.2 and starts to report valid ACK/NACK for the PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1. The UE shall be continuously scheduled on PCell’s new CBW starting from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length.

TRRCprocessingDelay and TCBWchangeDelayRRC are defined in clause 8.13.

The test equipment verifies the UE specific CBW switch time in PCell by counting the time from the time when the RRC Reconfiguration message including updated CBW configurations sent till the time when RRC Reconfiguration Complete message is received.

Table A.7.5.10.1.1-1: UE specific CBW change supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.10.1.1-2: General test parameters for UE specific CBW change in NR SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| T1 | s | [0.2] |  |

Table A.7.5.10.1.1-3: NR Cell specific test parameters for UE specific CBW change in NR SA

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR2 |
| Duplex mode |  |  | TDD |
| TDD configuration |  |  | TDDConf.3.1 |
| BWchannel |  |  | 100 MHz: NRB,c = 66 |
| Active DL BWP ID |  |  | 1 |
| Initial DL BWP Configuration (BWP-1) |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  |  | ULBWP.0.2 |
| Initial Condition | Active DL CBW-1 Configuration |  | DLCBW.1.1 |
|  | Active UL CBW-1 Configuration |  | ULCBW.1.1 |
| Final Condition | Active DL CBW-1 Configuration |  | DLCBW.1.2 |
|  | Active UL CBW-1 Configuration |  | ULCBW.1.2 |
| 'PDSCH Reference measurement channel |  |  | SR.3.1 TDD |
| RMSI CORESET parameters |  |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  |  | CCR.3.1 TDD |
| OCNG Patterns |  |  | OP.1 |
| SSB Configuration |  |  | SSB.1 FR2 |
| SMTC Configuration |  |  | SMTC.1 |
| TCI State |  |  | TCI.State.0 |
| TRS Configuration |  |  | TRS.2.1 TDD |
| Antenna Configuration |  |  | 1x2 |
| Propagation Condition |  |  | AWGN |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.7.5.10.1.1-4: OTA related test parameters for UE specific CBW change test case

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  |  | Setup 1 according to table A.3.15 |
| Assumption for UE beams Note 5 |  |  | Fine |
| Note1 | NR_TDD_FR2_A | dBm/15kHz | -112 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| Note1 | NR_TDD_FR2_A | dBm/SCS | -103 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note3 | -85 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
|  |  | dB | 18 |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -56 |
|  | NR_TDD_FR2_B |  |  |
|  | NR_TDD_FR2_F |  |  |
|  | NR_TDD_FR2_G |  |  |
|  | NR_TDD_FR2_T |  |  |
|  | NR_TDD_FR2_Y |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |

##### A.7.5.10.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for PCell from the first DL slot that occurs after the beginning of slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length and starts to report valid ACK/NACK for the PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TCBWchangeDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

All of the above test requirements shall be fulfilled in order for the observed PCell UE specific CBW change delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

## A.7.6	Measurement procedure

### A.7.6.1	Intra-frequency Measurements

#### A.7.6.1.1	SA event triggered reporting test without gap under non-DRX

##### A.7.6.1.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.7.6.1.1.1-1.

Table A.7.6.1.1.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.1.1.1-2, A.7.6.1.1.1-3 and A.7.6.1.1.1-4 below.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.7.6.1.1.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| offsetMO | dB | 1, 2 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | 1, 2 | -11 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.7.6.1.1.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1 | 24 |  | 24 |  |
|  |  | 2 | 48 |  | 48 |  |
| Intial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.2 TDD |  | N/A |  |
|  |  | 2 | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |
|  |  | 2 | CR.3.2 TDD |  | CR.3.2 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
|  |  | 2 | CCR.3.7 TDD |  | CCR.3.7 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 120 |  | 120 |  |
| OCNG Patterns |  | 1, 2 | OP.5 |  | N/A |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.7 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.8 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |

Table A.7.6.1.1.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |  |
| AoA setup |  | 1, 2 | Setup 3 defined in A.3.15.3 |  |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |  |
| Es | dBm/SCS | 1 | -89 | -89 | -Infinity |  | -89 |
|  |  | 2 | -86 | -86 | -Infinity |  | -86 |
| BB Note 5 | dB | 1, 2 | -0.12 | -0.12 | -Infinity | -0.12 |  |
| SSB_RP | dBm/SCS | 1 | -89 | -89 | -Infinity | -89 |  |
|  |  | 2 | -86 | -86 | -Infinity | -86 |  |
|  | dBm/95.04MHz | 1 | -64.41 | -64.41 | See Cell 1 columns |  |  |
|  |  | 2 | -61.41 | -61.41 |  |  |  |
| Time multiplexing of the downlink transmissions from each AoA |  | 1, 2 | Defined in Figure A.7.6.1.1.1-1 |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Void Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

Figure A.7.6.1.1.1-1: Time multiplexed downlink transmissions (Config 1 example)

##### A.7.6.1.1.2	Test Requirements

In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	2.4s for a UE supporting power class 1,

-	1.44s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.1.2	SA event triggered reporting test without gap under DRX

##### A.7.6.1.2.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.7.6.1.2.1-1.

Table A.7.6.1.2.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.1.2.1-2 ~ 6.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.1.2.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2 | -6 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX.7 | DRX related parameters are defined in Table A.7.6.1.2.1-5 |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 10 | 52 |  |

Table A.7.6.1.2.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1, 2 | 66 |  | 66 |  |
| Intial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.2 TDD |  | N/A |  |
|  |  | 2 | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |
|  |  | 2 | CR.3.2 TDD |  | CR.3.2 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
|  |  | 2 | CCR.3.7 TDD |  | CCR.3.7 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 120 |  | 120 |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |

Table A.7.6.1.2.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |
| BB Note 5 | dB | 1, 2 | 3.77 | -1.52 | -Infinity | -1.52 |
| Note 2 | dBm/15 KHz | 1, 2 | -98 |  |  |  |
| Note 2 | dBm/SCS | 1 | -89 |  |  |  |
|  |  | 2 | -86 |  |  |  |
| SSB_RP | dBm/SCS | 1 | -85 | -85 | -Infinity | -85 |
|  |  | 2 | -82 | -82 | -Infinity | -82 |
|  | dB | 1, 2 | 4 | 4 | -Infinity | 4 |
|  | dBm/95.04MHz | 1, 2 | -54.53 | -52.18 | See Cell 1 columns |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

Table A.7.6.1.2.1-5: Void

Table A.7.6.1.2.1-6: Void

##### A.7.6.1.2.2	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	7.2s for a UE supporting power class 1,

-	4.32s for a UE supporting power class 2, 3 and 4

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	51.2s for a UE supporting power class 1,

-	30.72s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX

##### A.7.6.1.3.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.7.6.1.3.1-1.

Table A.7.6.1.3.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.1.3.1-2 ~ 4 below.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.7.6.1.3.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps without DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| Gap type |  | 1, 2 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2 | 40 |  |
| Measurement gap length | ms | 1, 2 | 6 |  |
| Measurement gap offset | ms | 1, 2 | 39 |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.3.2 TDD |  |
| offsetMO | dB | 1, 2 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | 1, 2 | -11 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.7.6.1.3.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1 | 24 |  | 24 |  |
|  |  | 2 | 48 |  | 48 |  |
| Intial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | CSI-RS |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.2 TDD |  | N/A |  |
|  |  | 2 | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |
|  |  | 2 | CR.3.2 TDD |  | CR.3.2 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
|  |  | 2 | CCR.3.7 TDD |  | CCR.3.7 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 120 |  | 120 |  |
| OCNG Patterns |  | 1, 2 | OP.5 |  | N/A |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.7 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.8 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |

Table A.7.6.1.3.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 3 defined in A.3.15.3 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 4 |  | 1,2 | Rough |  | Rough |  |
| Es | dBm/SCS | 1 | -89 | -89 | -Infinity | -89 |
|  |  | 2 | -86 | -86 | -Infinity | -86 |
| BB Note 5 | dB | 1, 2 | -0.12 | -0.12 | -Infinity | -0.12 |
| SSB_RP | dBm/SCS | 1 | -89 | -89 | -Infinity | -89 |
|  |  | 2 | -86 | -86 | -Infinity | -86 |
|  | dBm/95.04MHz | 1 | -64.41 | -64.41 | See Cell 2 columns |  |
|  |  | 2 | -61.41 | -61.41 |  |  |
| Time multiplexing of the downlink transmissions from each AoA |  | 1,2 | Defined in Figure A.7.6.1.3.1-1 |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Void Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

Figure A.7.6.1.3.1-1: Time multiplexed downlink transmissions (Config 1 example)

##### A.7.6.1.3.2	Test Requirements

In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	3.2s for a UE supporting power class 1,

-	1.92s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX

##### A.7.6.1.4.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.7.6.1.4.1-1.

Table A.7.6.1.4.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.1.4.1-2, A.7.6.1.4.1-3 and A.7.6.1.4.1-4 below.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.1.4.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps with DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  | One TDD carrier frequency is used for the NR cells. |
| Gap type |  | 1, 2 | Per-UE gaps |  |  |
| Measurement gap repitition periodicity | ms | 1, 2 | 40 |  |  |
| Measurement gap length | ms | 1, 2 | 6 |  |  |
| Measurement gap offset | ms | 1, 2 | 39 |  |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.3.2 TDD |  |  |
| A3-Offset | dB | 1, 2 | -6 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX.7 | DRX related parameters are defined in Table A.7.6.1.2.1-5 |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 10 | 52 |  |

Table A.7.6.1.4.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps with DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1, 2 | 66 |  | 66 |  |
| Intial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SCSI-RS |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.12TDD |  | N/A |  |
|  |  | 2 | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |
|  |  | 2 | CR.3.2 TDD |  | CR.3.2 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
|  |  | 2 | CCR.3.7 TDD |  | CCR.3.7 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  | 1, 2 | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 120 |  | 120 |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN |  |

Table A.7.6.1.4.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps with DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 |  | 1,2 | Rough |  |  |  |
| BB Note 5 | dB | 1, 2 | 3.77 | -1.52 | -Infinity | -1.52 |
| Note 2 | dBm/15 KHz | 1, 2 | -98 |  |  |  |
| Note 2 | dBm/SCS | 1 | -89 |  |  |  |
|  |  | 2 | -86 |  |  |  |
| SSB_RP | dBm/SCS | 1 | -85 | -85 | -Infinity | -85 |
|  |  | 2 | -82 | -82 | -Infinity | -82 |
|  | dB | 1, 2 | 4 | 4 | -Infinity | 4 |
|  | dBm/95.04MHz | 1,2 | -54.53 | -52.18 | See Cell 1 columns |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

Table A.7.6.1.4.1-5: Void

Table A.7.6.1.4.1-6:Void

##### A.7.6.1.4.2	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	7.2s for a UE supporting power class 1,

-	4.32s for a UE supporting power class 2, 3 and 4

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	51.2s for a UE supporting power class 1,

-	30.72s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.7.6.2	Inter-frequency Measurements

#### A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)

##### A.7.6.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.1.1-1, A.7.6.2.1.1-2, and A.7.6.2.1.1-3.

Measurement gap pattern configuration # 13 as defined in Table A.7.6.2.1.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.1.1-1.

Table A.7.6.2.1.1-1 SA event triggered reporting tests without SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.7.6.2.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| offsetMO | dB | Config 1 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | Config 1 | -11 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 5.2 for PC1; 3.5 for other PC |  |

Table A.7.6.2.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.3.15 |  |  |  |
|  |  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  | 1,2 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 |  | -87 |  |
| SSB-RP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 | -Infinity | -87 |
| BB Note 8 |  | dB | Config 1 | 1.89 | 1.89 | -Infinity | 1.89 |
| Io Note3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

##### A.7.6.2.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

The UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)

##### A.7.6.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.2.1-1, A.7.6.2.2.1-2, and A.7.6.2.2.1-3.

In test 1&2 measurement gap pattern configuration # 13 as defined in Table A.7.6.2.2.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.2.1-1.

UE needs to be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.2.1-1: SA event triggered reporting tests without SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.7.6.2.2.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC |  |

Table A.7.6.2.2.1-3: Cell specific test parameters for CA inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.2.2.2	Test Requirements

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

7680 for UE supporting power class 1, or

4800 for UE supporting other power class.

In test 2  the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

81920 for UE supporting power class 1, or

51200 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)

##### A.7.6.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.3.1-1, A.7.6.2.3.1-2, and A.7.6.2.3.1-3.

Measurement gap pattern configuration # 13 as defined in Table A.7.6.2.3.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.3.1-1.

Table A.7.6.2.3.1-1: SA event triggered reporting tests with SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.7.6.2.3.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | Config 1 | -11 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 for PC1; 4.5 for other PC |  |

Table A.7.6.2.3.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.3.15 |  |  |  |
|  |  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 |  | -87 |  |
| SSB-RP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 | -Infinity | -87 |
| BB Note 8 |  | dB | Config 1 | 1.89 | 1.89 | N/A | 1.89 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

##### A.7.6.2.3.2	Test Requirements

In test 1 with per-UE gap and in test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class.

The UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)

##### A.7.6.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.4.1-1, A.7.6.2.4.1-2, and A.7.6.2.4.1-3.

In test 1&2 measurement gap pattern configuration # 13 as defined in Table A.7.6.2.4.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.4.1-1.

UE needs to be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.4.1-1: SA event triggered reporting tests with SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.7.6.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC |  |

Table A.7.6.2.4.1-3: Cell specific test parameters for CA inter-frequency event triggered reporting with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.2.4.2	Test Requirements

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

10080 for UE supporting power class 1, or

6240 for UE supporting other power class.

In test 2 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

107520 for UE supporting power class 1, or

66560 for UE supporting other power class.

In test 1 and 2 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)

##### A.7.6.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2. The test parameters and configurations are given in Tables A.7.6.2.5.1-1, A.7.6.2.5.1-2, and A.7.6.2.5.1-3.

In test 1 per-UE measurement gap pattern configuration # 0 as defined in Table A.7.6.2.5.1-2 is provided for a UE that does not support per-FR gap and in test 2 no gap pattern is configured as defined in Table A.7.6.2.5.1-2. If the UE supports per-FR gap, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.5.1-1.

Table A.7.6.2.5.1-1 SA event triggered reporting tests without SSB index reading for FR1-FR2

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.7.6.2.5.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | One NR FR1 and one NR FR2 carrier frequency is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | Gap not configured | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 | N/A |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| CSI-RS for tracking parameters on NR RF Channel 1 |  | Config 1 | TRS.1.1 FDD |  |  |
|  |  | Config 2 | TRS.1.1 TDD |  |  |
|  |  | Config 3 | TRS.1.2 TDD |  |  |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -105 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 5.2 for PC1; 3.5 for other PC | 3 for PC1; 2 for other PC |  |

Table A.7.6.2.5.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | N/A |  | Setup 1 as specified in clause A.3.15 |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 52 |  | 66 |  |
|  |  |  | Config 2 | 52 |  | 66 |  |
|  |  |  | Config 3 | 106 |  | 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1,2,3 |  |  | -Infinity | -87 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2 |  |  | -Infinity | -87 |
|  |  | Note5 | Config 3 |  |  | -Infinity | -87 |
| BB Note 8 |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 14.69 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1,2,3 | A.3.7A |  | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

##### A.7.6.2.5.2	Test Requirements

In test 1 with per-UE gap and in test 2 with per-FR gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

In test 2, without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

2560 for UE supporting power class 1, or

1600 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)

##### A.7.6.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.6.1-1, A.7.6.2.6.1-2, and A.7.6.2.6.1-3.

In test 1&2 per-UE measurement gap pattern configuration # 0 as defined in Table A.7.6.2.6.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 no gap pattern is configured as defined in Table A.7.6.2.6.1-2. If a UE supports per-FR gap  it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.6.1-1.

UE needs to be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.6.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR2

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.7.6.2.6.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | One NR FR1 and one NR FR2 carrier frequency is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | Gap not configured |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 |  | N/A |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  |  |  | As specified in clause A.3.10.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  |  |  | As specified in clause A.3.10.2 |
| CSI-RS for tracking parameters on NR RF Channel 1 |  | Config 1 | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 | TRS.1.2 TDD |  |  |  |  |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -105 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC |  |

Table A.7.6.2.6.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | NA |  | Setup 1 as specified in clause A.3.15 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 52 |  | 66 |  |
|  |  |  | Config 2 | 52 |  | 66 |  |
|  |  |  | Config 3 | 106 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
|  |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | NA Link only, see clause A.3.7A |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1,2 |  |  | -95.7 |  |
|  |  |  | Config 3 |  |  | -95.7 |  |
| SSB_RP Note 3 |  | dBm/SCS Note5 | Config 1,2 |  |  | -Infinity | -86.7 |
|  |  |  | Config 3 |  |  | -Infinity | -86.7 |
|  |  | dB | Config 1,2,3 |  |  | -Infinity | 9 |
|  |  | dB | Config 1,2,3 |  |  | -Infinity | 9 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 |  |  | - | - |
|  |  | dBm/38.16MHz | Config 3 |  |  | - | - |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 |  |  | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone |  |  |  |  |  |  |  |

##### A.7.6.2.6.2	Test Requirements

In test 1 with per-UE gap and in test 3 without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

7680 for UE supporting power class 1, or

4800 for UE supporting other power class.

In test 2 with per-UE gap and in test 4 without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

81920 for UE supporting power class 1, or

51200 for UE supporting other power class.

In test 1, 2, 3 and 4 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)

##### A.7.6.2.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

n this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2. The test parameters and configurations are given in Tables A.7.6.2.7.1-1, A.7.6.2.7.1-2, and A.7.6.2.7.1-3.

In test 1 per-UE measurement gap pattern configuration # 0 as defined in Table A.7.6.2.7.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement no gap pattern is configured as defined in Table A.7.6.2.7.1-2. If the UE supports per-FR gap, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.7.1-1.

Table A.7.6.2.7.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR2

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.7.6.2.7.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | One NR FR1 and one NR FR2 carrier frequency is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | Gap not configured | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 | N/A |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| CSI-RS for tracking parameters on NR RF Channel 1 |  | Config 1 | TRS.1.1 FDD |  |  |
|  |  | Config 2 | TRS.1.1 TDD |  |  |
|  |  | Config 3 | TRS.1.2 TDD |  |  |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| a4-Threshold | dBm | Config 1,2,3,4,5,6 | -105 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 7 for PC1; 4.5 for other PC | 3.5 for PC1; 2.5 for other PC |  |

Table A.7.6.2.7.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | NA |  | Setup 1 as specified in clause A.3.15 |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 52 |  | 66 |  |
|  |  |  | Config 2 | 52 |  | 66 |  |
|  |  |  | Config 3 | 106 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1,2, 3 |  |  | -Infinity | -87 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2 |  |  | -Infinity | -87 |
|  |  | Note5 | Config 3 |  |  | -Infinity | -87 |
| BB Note 8 |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 14.69 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1,2,3 | A.3.7A |  | Infinity | -58.01 |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

##### A.7.6.2.7.2	Test Requirements

In test 1 with per-UE gap and in test 2 with per-FR gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class.

In test 2 without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

3360 for UE supporting power class 1, or

2080 for UE supporting other power class.

In test 1 and 2 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)

##### A.7.6.2.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2. The test parameters and configurations are given in Tables A.7.6.2.8.1-1, A.7.6.2.8.1-2, and A.7.6.2.8.1-3.

In test 1&2 per-UE measurement gap pattern configuration # 0 as defined in Table A.7.6.2.8.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement no gap pattern is configured as defined in Table A.7.6.2.8.1-2.If a UE supports per-FR gap , it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.8.1-1.

UE needs to be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.8.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR2

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.7.6.2.8.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | One NR FR1 and one NR FR2 carrier frequency is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | Gap not configured |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 |  | N/A |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  |  |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  |  |  | As specified in clause A.3.10.1 |
| CSI-RS for tracking parameters on NR RF Channel 1 |  | Config 1 | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 | TRS.1.2 TDD |  |  |  |  |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  |  |  | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -105 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 11 for PC1; 6.5 for other PCTBD | 108 for PC1; 67 for other PCTBD | 11 for PC1; 6.5 for other PCTBD | 108 for PC1; 67 for other PCTBD |  |

Table A.7.6.2.8.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | NA |  | Setup 1 as specified in clause A.3.15 |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 52 |  | 66 |  |
|  |  |  | Config 2 | 52 |  | 66 |  |
|  |  |  | Config 3 | 106 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  |  |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1,2 |  |  | -95.7 |  |
|  |  |  | Config 3 |  |  | -95.7 |  |
| SSB_RP Note 3 |  | dBm/SCS Note5 | Config 1,2 |  |  | -Infinity | -86.7 |
|  |  |  | Config 3 |  |  | -Infinity | -86.7 |
|  |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 9 |
|  |  | dB | Config 1,2,3 | A.3.7A |  | -Infinity | 9 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 |  |  | - | - |
|  |  | dBm/38.16MHz | Config 3 |  |  | - | - |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 |  |  | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.2.8.2	Test Requirements

In test 1 with per-UE gap and in test 3 without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

10080 for UE supporting power class 1, or

6240 for UE supporting other power class.

In test 2 with per-UE gap and in test 4 without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

107520 for UE supporting power class 1, or

66560 for UE supporting other power class.

In test 1, 2, 3 and 4 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)

##### A.7.6.2.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.2.9.1-1, A.7.6.2.9.1-2, and A.7.6.2.9.1-3.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.2.9.1-1.

Table A.7.6.2.9.1-1 SA event triggered reporting tests without SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: Void. |  |

Table A.7.6.2.9.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection (GP17)

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 17 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -30 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 6 (PC1) 4 (other PC) |  |

Table A.7.6.2.9.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection (GP17)

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.3.15 |  |  |  |
|  |  |  |  | AoA1 |  | AoA2 |  |
| Beam assumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| TCI configuration |  |  | Config 1 | CSI-RS.Config.0 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | N/A |  | N/A |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | N/A |  | N/A |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 | -Infinity | -87 |
|  |  | dB | Config 1 | N/A | N/A | -Infinity | N/A |
|  |  | dB | Config 1 | N/A | N/A | -Infinity | N/A |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.2.9.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 5120ms (PC1) or 3200ms (other than PC1) from the beginning of time period T2.

The UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.10	SA event triggered reporting test without gap under non-DRX

##### A.7.6.2.10.1	Test Purpose and Environment

The purpose of this test is to verify that if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the network, the UE makes correct reporting of an event. This test will partly verify the inter-frequency without gap cell search requirements in clause 9.3.9. Supported test configurations are shown in table A.7.6.2.10.1-1.

Table A.7.6.2.10.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

There are two cells in the test, NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2. The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from cell 1 and cell 2 should be different in frequency location within the cell bandwidth.  The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.2.10.1-2, A.7.6.2.10.1-3 and A.7.6.2.10.1-4 below.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.7.6.2.10.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | PCell (Cell 1) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | 1 | Cell 2 | NR cell 2 is on NR RF channel number 2. |
| RF Channel Number |  | 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| SMTC configuration |  | 1 | SMTC.1 |  |
| A3-Offset | dB | 1 | -6 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX |  | 1 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1 | 3 s | Synchronous cells |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

Table A.7.6.2.10.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Intial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1 | TCI.State.2 |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |

Table A.7.6.2.10.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |  |  |
| Beam assumptionNote 4 |  | 1 | Rough |  | Rough |  |
|  | dB | 1 | 4 | 4 | -Infinity | 8 |
| Note 2 | dBm/15 KHz | 1 | -102 |  |  |  |
| Note 2 | dBm/SCS | 1 | -93 |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -89 | -89 | -Infinity | -85 |
|  | dB | 1 | 4 | 4 | -Infinity | 8 |
|  | dBm/95.04MHz | 1 | -58.56 |  | -55.38 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

##### A.7.6.2.10.2	Test Requirements

In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	2.4s for a UE supporting power class 1,

-	1.44s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.2.11	SA event triggered reporting test without gap under DRX

##### A.7.6.2.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD inter-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2. Supported test configurations are shown in table A.7.6.2.11.1-1.

Table A.7.6.2.11.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) on NR RF channel 1 and a FR2 neighbour cell (Cell 2) on NR RF channel 2. The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from cell 1 and cell 2 should be different in frequency location within the cell bandwidth. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.2.11.1-2 ~ 6.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided  with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.11.1-2: General test parameters for inter-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | 1, 2 | 2 TDD carrier frequency are used for the NR cells. |
| Active cell |  | 1, 2 | PCell (Cell 1) | Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. Cell 2 is on NR RF channel number 2. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| A3-Offset | dB | 1, 2 | -6 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 52 |  |

Table A.7.6.2.11.1-3: NR Cell specific test parameters for inter-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1, 2 | 1 |  | 2 |  |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Intial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | N/A |  |
| RLM-RS |  | 1, 2 | SSB |  | N/A |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | N/A |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |

Table A.7.6.2.11.1-4: NR OTA Cell specific test parameters for inter-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |
|  | dB | 1, 2 | 4 | -1.46 | -Infinity | -1.46 |
| Note 2 | dBm/15 KHz | 1, 2 | -98 |  |  |  |
| Note 2 | dBm/SCS | 1 | -89 |  |  |  |
|  |  | 2 | -86 |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -85 | -85 | -Infinity | -85 |
|  |  | 2 | -82 | -82 | -Infinity | -82 |
|  | dB | 1, 2 | 4 | 4 | -Infinity | 4 |
|  | dBm/95.04MHz | 1 | -54.53 | -52.18 | -54.53 | -52.18 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |

##### A.7.6.2.11.2	Test Requirements

In test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	51.2s for a UE supporting power class 1,

-	30.72s for a UE supporting power class 2, 3 and 4est

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.7.6.3	L1-RSRP measurement for beam reporting

#### A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used

#### A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used

##### A.7.6.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.7.6.3.1.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15

Table A.7.6.3.1.1-1: Applicable NR configurations for FR2 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.7.6.3.1.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.6.3.1.2-1 and Table A.7.6.3.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.7.6.3.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| Data RBs allocated | 1~2 |  | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD |
|  | 2 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
|  | 2 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
|  | 2 |  | CCR.3.7 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | Off |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 |
| L1-RSRP reporting period | 1~2 | slot | 320 |
| T1 | 1~2 | s | 5 |
| T2 | 1~2 | s | 2 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.3.1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 | 1-2 |  | Rough |  |  |  |
| Note2 | 1~2 | dBm/15kHz | -105 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96 |  |  |  |
|  | 2 |  | -93 |  |  |  |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | -87 |
|  | 2 |  | -93 | -93 | -Infinity | -84 |
| Io Note3 | 1 | dBm/95.04MHz | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 2 |  | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

##### A.7.6.3.1.3	Test Requirements

The UE shall send L1-RSRP report every 320 slots. No later than X ms plus 320 slots from the beginning of time period T2, UE shall send L1-RSRP report including the results for both SSB#0 and SSB#1 while meeting the accuracy requirements defined in clause 10.1.20.1, where X is

-	1680 for UE supporting power class 1

-	1200 for UE supporting power class 2,3 or 4.

The reported L1-RSRP value shall include the Rx antenna gain in the range of -10 to +20 dB.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used

##### A.7.6.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1, with the testing configurations for NR cells in Table A.7.6.3.2.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15

Table A.7.6.3.2.1-1: Applicable NR configurations for FR2 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.7.6.3.2.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.6.3.2.2-1 and Table A.7.6.3.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.7.6.3.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| Data RBs allocated | 1~2 |  | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD |
|  | 2 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
|  | 2 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
|  | 2 |  | CCR.3.7 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | DRX.3 |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 |
| L1-RSRP reporting period | 1~2 | slot | 320 |
| T1 | 1~2 | s | 5 |
| T2 | 1~2 | s | 3 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.3.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 | 1-2 |  | Rough |  |  |  |
| Note2 | 1~2 | dBm/15kHz | -105 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96 |  |  |  |
|  | 2 |  | -93 |  |  |  |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | -87 |
|  | 2 |  | -93 | -93 | -Infinity | -84 |
| Io Note3 | 1 | dBm/95.04MHz | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 2 |  | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

##### A.7.6.3.2.3	Test Requirements

The UE shall send L1-RSRP report every 320 slots. No later than X ms plus 320 slots from the beginning of time period T2, UE shall send L1-RSRP report including the results for both SSB#0 and SSB#1 while meeting the accuracy requirements defined in clause 10.1.20.1, where X is

-	2880 for UE supporting power class 1

-	1920 for UE supporting power class 2,3 or 4.

The reported L1-RSRP value shall include the Rx antenna gain in the range of -10 to +20 dB.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used

##### A.7.6.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in Table A.7.6.3.3.1-1.

Table A.7.6.3.3.1-1: Applicable NR configurations for FR2 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.7.6.3.3.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.6.3.3.2-1 and Table A.7.6.3.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 480ms from the beginning of the test, the DCI trigger comes in slot 1 of a frame and UE provides the report back based on the reporting configuration as defined in Table A.7.6.3.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.7.6.3.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 8 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.3.3.2-1: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.3.15.1 |  |
| Beam AssumptionNote 4 | 1 |  | Rough | Rough |
| Note1 | 1 | dBm/15kHz | -105 |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |
|  | 1 | dB | 0 | 9 |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1 | dB | 0 | 9 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

##### A.7.6.3.3.3	Test Requirements

After 480ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the accuracy requirements defined in clause 10.1.20.1. The reported L1-RSRP value shall include the Rx antenna gain in the range of [-10 ~ +20] dB.

For absolute accuracy of CSI-RS0 and absolute accuracy of CSI-RS1, the UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table A.7.6.3.3.3-1.

For relative accuracy of CSI-RS0 compared with CSI-RS1, the UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in Table 10.1.20.2.2-1.

Table A.7.6.3.3.3-1: L1-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| CSI-RS0 | CSI-RS _RP0 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP0 +δ + Gmax |
| CSI-RS1 | CSI-RS _RP1 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP1 +δ + Gmax |
| Note 1: CSI-RS_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the CSI-RS n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.20.2.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

#### A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used

##### A.7.6.3.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.2, with the testing configurations for NR cells in Table A.7.6.3.4.1-1.

Table A.7.6.3.4.1-1: Applicable NR configurations for FR2 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.7.6.3.4.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.6.3.4.2-1 and Table A.7.6.3.4.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 1440ms from the beginning of the test, the DCI trigger comes in slot 1 of a frame and UE provides the report back based on the reporting configuration as defined in Table A.7.6.3.4.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.7.6.3.4.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | DRX.3 |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 8 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.3.4.2-1: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.3.15.1 |  |
| Beam AssumptionNote 4 | 1 |  | Rough | Rough |
| Note1 | 1 | dBm/15kHz | -105 |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |
|  | 1 | dB | 0 | 9 |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1 | dB | 0 | 9 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

##### A.7.6.3.3.3	Test Requirements

After 1440ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the accuracy requirements defined in clause 10.1.20.1. The reported L1-RSRP value shall include the Rx antenna gain in the range of [-10 ~ +20] dB.

For absolute accuracy of CSI-RS0 and absolute accuracy of CSI-RS1, the UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table A.7.6.3.4.3-1.

For relative accuracy of CSI-RS0 compared with CSI-RS1, the UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in Table 10.1.20.2.2-1.

Table A.7.6.3.4.3-1: L1-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| CSI-RS0 | CSI-RS _RP0 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP0 +δ + Gmax |
| CSI-RS1 | CSI-RS _RP1 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP1 +δ + Gmax |
| Note 1: CSI-RS_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the CSI-RS n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.20.2.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

### A.7.6.4	CLI measurements

#### A.7.6.4.1	SRS-RSRP measurement with non-DRX

##### A.7.6.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SRS-RSRP measurement. This test will verify the SRS-RSRP measurement requirements in clause 9.7.2.5 with the testing configurations for NR cells in Table A.7.6.4.1.1-1.

Table A.7.6.4.1.1-1: Applicable NR configurations for FR2 SRS-RSRP test

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.7.6.4.1.2	Test Parameters

One cell is deployed in the test, which is FR2 PCell (Cell 1). The test parameters for PCell is given in Table A.7.6.4.1.2-1 ~ A.7.6.4.1.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system transmits SRS resource for measurement in the DL slot according to the SRS configuration in Table A.7.6.4.1.2-4 and the test parameters for the (virtual) neighbour cell UE in Table A. 7.6.4.1.2-3. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 2 data symbols before SRS to be transmitted.

Table A.7.6.4.1.2-1: General test parameters for SRS-RSRP event triggered reporting for PCell in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| RF Channel Number |  | 1 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.7.6.4.1.2-4 |
| CP length |  | 1 | Normal |  |
| i1-Threshold | dBm | 1 | -103 |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 | OFF | Non-DRX |
| Time offset between DL from serving cell and SRS from test system | s | 1 | 10.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 1 |  |

Table A.7.6.4.1.2-2: NR Cell specific test parameters for SA SRS-RSRP event triggered reporting for PCell in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns |  | 1 | OP.1 |  |
| TRS configuration |  |  | TRS.2.1. TDD |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| Propagation Condition |  | 1 | AWGN |  |

Table A.7.6.4.1.2-3: NR OTA Cell specific test parameters for SA SRS-RSRP event triggered reporting for PCell and neighbour cell UE in FR2

| Parameter | Unit | Test configuration | Cell 1 |  | Neighbour cell UE |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |  |  |
| Beam assumption Note 4 |  | 1 | Fine |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | -98 |  |
| Note 2 | dBm/SCS | 1 | -89 |  | -89 |  |
|  | dB | 1 | - | - | -infinity | 4 |
|  | dB | 1 | - | - | -infinity | 4 |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | - | - | -infinity | -94 |
| Io | dBm/95.04 MHz | 1 | -70.01 | -68.82 | -70.01 | -68.82 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |

Table A.7.6.4.1.2-4: SRS configuration for measurement reporting

|  | Field | SRSConf.1 | Comments |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |  |
|  | srs-ResourceIdList | 0 |  |
|  | resourceType | Periodic |  |
|  | Usage | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 |  |
|  | nrofSRS-Ports | Port1 |  |
|  | transmissionComb | n2 |  |
|  | combOffset-n2 | 0 |  |
|  | cyclicShift-n2 | 0 |  |
|  | resourceMapping startPosition | 0 |  |
|  | resourceMapping nrofSymbols | n1 |  |
|  | resourceMapping repetitionFactor | n1 |  |
|  | freqDomainPosition | 0 |  |
|  | freqDomainShift | 0 |  |
|  | freqHopping c-SRS | 12 |  |
|  | freqHopping b-SRS | 0 |  |
|  | freqHopping b-hop | 0 |  |
|  | groupOrSequenceHopping | Neither |  |
|  | resourceType | Periodic |  |
|  | periodicityAndOffset | sl40, 25 |  |
|  | sequenceId | 0 | Any 10 bit number |

##### A.7.6.4.1.3	Test Requirements

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 60 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.4.2	CLI-RSSI measurement with non-DRX

##### A.7.6.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of CLI-RSSI measurement. This test will verify the CLI-RSSI measurement requirements in clause 9.7.3.5 with the testing configurations for NR cells in Table A.7.6.4.2.1-1.

Table A.7.6.4.2.1-1: Applicable NR configurations for FR2 CLI-RSSI test

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.7.6.4.2.2	Test Parameters

One cell is deployed in the test, which is FR2 PCell (Cell 1). The test parameters for PCell is given in Table A.7.6.4.2.2-1 ~ A.7.6.4.2.2-3 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for CLI-RSSI measurement resource and on 2 data symbols before. The CLI-RSSI measurement resource configuration is in Table A.7.6.4.2.2-4.

Table A.7.6.4.2.2-1: General test parameters for CLI-RSSI event triggered reporting for PCell in FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | NR Cell 1 |  |
| RF Channel Number |  | 1 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| CLI-RSSI configuration |  | 1 | CLI-RSSIConf.1 | Table A.7.6.4.2.2-4 |
| CP length |  | 1 | Normal |  |
| i1-Threshold | dBm | 1 | -94.5 |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX |  | 1 | OFF | Non-DRX |
| Time offset between DL from serving cell and OCNG from test system | s | 1 | 10.67 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 1 |  |

Table A.7.6.4.2.2-2: NR Cell specific test parameters for CLI-RSSI event triggered reporting for PCell in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |
| PUSCH parameters |  | 1 | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |
| OCNG Patterns Note 1 |  | 1 | OP.1 |  |
| TRS configuration |  |  | TRS.2.1. TDD |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  |
| Propagation Condition |  | 1 | AWGN |  |
| Note 1: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |  |

Table A.7.6.4.2.2-3: NR OTA Cell specific test parameters for CLI-RSSI event triggered reporting for PCell in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |
| Beam assumption Note 3 |  | 1 | Fine | Fine |
| on CLI-RSSI measurement resource Note 2 | dBm/15 kHz | 1 | -119 | -108 |
| on CLI-RSSI measurement resource Note 2 | dBm/SCS | 1 | -110 | -99 |
| Io on CLI-RSSI measurement resource | dBm/95.04 MHz | 1 | -81.01 | -70.01 |
| Io on CLI-RSSI measurement resource | dBm/1.08 MHz | 1 | -100.46 | -89.46 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |

Table A.7.6.4.2.2-4: CLI-RSSI measurement resource configuration for measurement reporting

|  | Field | CLI-RSSIConf.1 |
| --- | --- | --- |
| RSSI-Resource | rssi-ResourceId | 0 |
|  | rssi-SCS | 120 |
|  | startPRB | 0 |
|  | nrofPRBs | 66 |
|  | startPosition | 3 |
|  | nrofSymbols | 11 |
|  | rssi-PeriodicityAndOffset | sl40, 25 |

##### A.7.6.4.2.3	Test Requirements

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 5ms from the beginning of time period T2. The nominal RSSI used to evaluate the requirement shall be based on Io.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.7.6.5	NR Measurements with autonomous gaps

#### A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)

##### A.7.6.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an CGI. This test will partly verify the SA inter-frequency NR cell search requirements in clause 8.2.1.2.16 and 9.11

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.5.1.1-1, A.7.6.5.1.1-2, and A.7.6.5.1.1-3.

Measurement gap patterns are configured. During T1 the UE shall report event A3 for cell 2. Within 3 seconds of the event report, the test equipment shall add a measurement reporting configuration using ReportConfigNR which containsa ReportCGI IE with cellForWhichToReportCGI set to the physical Cell ID of cell 2 and including the optional IE useAutonomousGaps-r16

In the measurement control information, it is indicated to the UE to decode the CGI of the neighbour cell using autonomous gaps. The test consists of two time phases, T1 and T2. Time period T2 begins 10ms after the test equipment has transmitted the RRC reconfiguration message containing the ReportCGI IE.

Supported test configurations are shown in table A.7.6.5.1.1-1.

Table A.7.6.5.1.1-1 SA interfrequency CGI reporting test in autonomous gaps

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.5.1.1-2: General test parameters for SA interfrequency CGI reporting in autonomous gaps

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| SI-RNTI scheduling rate | ms |  | 40 ms | S-RNTI scheduled on four occasions per 160ms transmission period |
| A3-Offset | dB | Config 1 | -30 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | <10 | UE expected to report event A3 for cell 2 within 5,2s (PC1)or 3.5s (other PC) of the start of T1. Test equipment shall configure CGI reporting within 3s after receiving the event A3 report. T2 begins 10ms after test equipment has transmitted the RRC reconfiguration to configure CGI reporting. |
| T2 | s | Config 1 | 1 |  |

Table A.7.6.5.1.1-3: Cell specific test parameters SA interfrequency CGI reporting in autonomous gaps

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.3.15 |  |  |  |
|  |  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  | 1,2 | Rough |  |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | Not sent |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| TCI configuration |  |  | Config 1 | CSI-RS.Config.0 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -99.03 |  | -99.03 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -90 |  | -90 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 |  | -93 |  |
|  |  | dB | Config 1 | 3 |  | -3 |  |
|  |  | dB | Config 1 | 3 |  | -3 |  |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -56.25 |  | -59.25 |  |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.5.1.2	Test Requirements

The UE shall report the CGI of cell 2 within 25*Tsmtc + 6*Tsi-rnti+20ms +2ms= 762ms from the start of T2, allow 765ms.  The rate of correct events observed during repeated tests shall be at least 90%.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 775 ms the number of interrupted slots shall not exceed the allowed number [as defined in clause 8.2.1.2.16 plus TBD].

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.7.6.6	L1-SINR measurement for beam reporting

#### A.7.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used

##### A.7.6.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements in clause 9.8.4.1, with the testing configurations for NR cells in Table A.7.6.6.1.1-1.

Table A.7.6.6.1.1-1: Applicable NR configurations for FR2 CSI-RS based L1-SINR test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.7.6.6.1.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.6.6.1.2-1 and Table A.7.6.6.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. After 160ms from the beginning of the test, the DCI trigger comes in slot 8 of a frame and UE provides the report back based on the reporting configuration as defined in Table A.7.6.6.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.7.6.6.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-SINR |
| reportQuantity-r16 | 1 |  | cri-SINR-r16 |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 26 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.6.1.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.3.15.1 |  |
| Beam assumptionNote 3 | 1 |  | Rough |  |
| Note1 | 1 | dBm/15kHz | -105 |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |
|  | 1 | dB | 0 | 9 |
| CSI-RS RSRP Note3 | 1 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1 | dB | 0 | 9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

##### A.7.6.6.1.3	Test Requirements

After 160ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the accuracy requirements defined in clause 10.1.28.1.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used

##### A.7.6.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements in clause 9.8.4.2, with the testing configurations for NR cells in Table A.7.6.6.2.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15

Table A.7.6.6.2.1-1: Applicable NR configurations for FR2 L1-SINR measurement test with SSB based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.7.6.6.2.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.6.6.2.2-1 and Table A.7.6.6.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the SSBs and the associated CSI-IM resources, and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD measurements based on the SSBs, and UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-IM resources as IMR.

Table A.7.6.6.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~2 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~2 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~2 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| CSI-IM configuration | 1~2 |  | CSI-IM.3.1 TDD |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | DRX.3 |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity-r16 | 1~2 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~2 |  | 2 |
| L1-SINR reporting period | 1~2 | slot | 640 |
| T1 | 1~2 | s | 5 |
| T2 | 1~2 | s | 2 |
| Propagation condition | 1~2 |  | AWGN |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.6.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam assumptionNote 4 | 1~2 |  | Rough |  |  |  |
| Note2 | 1~2 | dBm/15kHz | -105 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96 |  |  |  |
|  | 2 |  | -93 |  |  |  |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| SSB RSRP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | -87 |
|  | 2 |  | -93 | -93 | -Infinity | -84 |
| Io Note3 | 1 | dBm/95.04MHz | -67.5 | -67.5 | -71.1 | -60.7 |
|  | 2 |  | -67.5 | -67.5 | -71.1 | -60.7 |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

##### A.7.6.6.2.3	Test Requirements

The UE shall send L1-SINR report every 640 slots. No later than X ms plus 640 slots from the beginning of time period T2, UE shall send L1-SINR report including the results for both SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 while meeting the accuracy requirements defined in clause 10.1.28.2, where X is

-	2880 for UE supporting power class 1

-	1920 for UE supporting power class 2,3 or 4.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used

##### A.7.6.6.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-SINR measurement. This test will partly verify the L1-SINR measurement requirements with CSI-RS based CMR and dedicated IMR cofigured in clause 9.8.4.3, with the testing configurations for NR cells in Table A.7.6.6.3.1-1.

Table A.7.6.6.3.1-1: Applicable NR configurations for FR2 L1-SINR test with CMR and dedicated IMR

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.7.6.6.3.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.6.6.3.2-1 and Table A.7.6.6.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-SINR measurement on the configured CSI-RS as CMR and an associated CSI-RS as IMR, and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources and the associated IMR. UE is also configured to measure L1-SINR based on SSB. After 1440ms from the beginning of the test, the DCI trigger comes in slot 8 of a frame and UE provides the report back based on the reporting configuration as defined in Table A.7.6.6.3.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs, and UE is configured to perform L1-SINR measurement based on the CSI-RS as CMR and the CSI-RS as IMR.

Table A.7.6.6.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS as CMR configuration | 1 |  | CSI-RS.3.3 TDD |
| CSI-RS as IMR configuration | 1 |  | CSI-RS.3.2A TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | DRX.3 |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity-r16 | 1 |  | cri-SINR-r16 |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 26 |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.6.6.3.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 3 | 1~2 |  | Rough |  |
| Note1 | 1~2 | dBm/15kHz | -105 |  |
| Note1 | 1~2 | dBm/SSB SCS | -95.97 |  |
|  | 1~2 | dB | 0 | 9 |
| CSI-RS RSRP Note2 | 1~2 | dBm/SSB SCS | -95.97 | -86.97 |
| Io Note2 | 1~2 | dBm/95.04MHz | -63.97 | -57.47 |
|  | 1~2 | dB | 0 | 9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

##### A.7.6.6.3.3	Test Requirements

After 1440ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 as CMR + CSI-RS#0 as IMR and CSI-RS#1 as CMR + CSI-RS#1 as IMR while meeting the accuracy requirement in clause 10.1.28.3. The reported L1-SINR value shall consider the Rx antenna gain in the range of [-10 ~ +20] dB when calculated.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.7.6.7	CSI-RS based intra-frequency Measurements

#### A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement

##### A.7.6.7.1.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency measurement requirements in clause 9.10.2 and 9.10.3. Supported test configurations are shown in table A.7.6.7.1.1-1.

Table A.7.6.7.1.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and CSI-RS SCS, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.6.7.1.1-2 ~ 6.

In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.7.1.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active cell |  | 1 | PCell (Cell 1) |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| CSI-RS resource configuration |  | 1 | CSI-RS.RRM.FR2.1 TDD |  |
| A3-Offset | dB | 1 | -6 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX |  | 1 | DRX.1 | DRX related parameters are defined in Table A.3.3 |
| Time offset between Cell 1 and Cell 2 | s | 1 | 0.58 |  |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 10 |  |

Table A.7.6.7.1.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Intial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1 | TCI.State.2 |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| SMTC |  | 1 | SMTC.1 |  |  |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
| CSI-RS |  | 1 | CSI-RS.RRM.FR2.1 TDD |  |  |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |

Table A.7.6.7.1.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap with DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |  |  |
| Beam assumptionNote 4 |  | 1 | Fine |  | Fine |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
| Note 2 | dBm/SCS | 1 | -86 |  |  |  |
| CSI-RSRP | dBm/SCS | 1 | -82 | -82 | -Infinity | -82 |
| SS-RSRP | dBm/SCS | 1 | -82 | -82 | -Infinity | -82 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  | dBm/95.04MHz | 1 | -54.53 | -52.18 | -54.53 | -52.18 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |

##### A.7.6.7.1.2	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

-	9.6s for a UE supporting power class 1,

-	5.76s for a UE supporting power class 2, 3 and 4

The UE is required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.7.6.8	CSI-RS based inter-frequency Measurements

#### A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)

##### A.7.6.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event for CSI-RS based L3 measurement. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.10.3.5.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in Tables A.7.6.8.1.1-1, A.7.6.8.1.1-2, and A.7.6.8.1.1-3.

In test 1&2 measurement gap pattern configuration # 13 as defined in Table A.7.6.8.1.1-2 is provided for UE that does not support per-FR gap and for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Supported test configurations are shown in table A.7.6.8.1.1-1.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.8.1.1-1: SA event triggered reporting tests for CSI-RS based L3 measurement for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB and CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.8.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 CSI-RS based L3 measurement

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| SMTC configuration |  | Config 1 | SMTC.1 | As specified in clause A.3.11 |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells | s | Config 1 | 0.58 | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 for PC1; 4.5 for other PC |  |

Table A.7.6.8.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 CSI-RS based L3 measurement

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| TCI configuration |  |  | Config 1 | CSI-RS.Config.0 |  | N/A |  |
| CSI-RS configuration for RRM |  |  |  | - |  | CSI-RS.RRM.FR2.1 TDD |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| CSI-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

##### A.7.6.2.2.2	Test Requirements

In the test the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.7.6.9	RSTD measurements

#### A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA

##### A.7.6.9.1.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 9.9.2 in an environment with AWGN propagation conditions in FR2 in standalone scenario when single positioning frequency layer is configured.

Supported test configurations are shown in table A.7.7.1.1-1. The test parameters are as given in Table 7.6.7.1.1-2, Table A.7.6.9.1.1-3 and Table A.7.6.9.1.1-4.

Table A.7.6.9.1.1-1: Supported test configurations for NR RSTD

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel distributed in single positioning frequency layers.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #13 before T2.

Table A.7.6.9.1.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 38.215 [4] and TS 37.355[34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.2 FR2 |  |
| SMTC configuration | Config 1 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.3.1 TDD | As specified in clause A.3.1.2.1 |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
| PRS Configuration | Config 1 |  | PRS.1.1. FR2 | As specified in clause A.3.31 |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#[13] | GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 500 | The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [24] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | [1.28] | The length of the time interval that follows immediately after time interval T1 |
| AoA setup |  |  | Setup 1 | As defined in A.3.15.1 |
| Beam assumption |  |  | Rough | Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |

Table A.7.6.9.1.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.5 FDD | N/A | N/A |
| Note 3 | Config 1 | dBm/SCS | -89 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/ 95.04MHz | -58.86 | -60.01 | -60.01 |
| SSB RP Note4 | Config 1 | dBm/SCS | -89 | -Infinity | -Infinity |
|  |  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.7.6.9.1.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| RF Channel Number |  |  | 1 | 1 | 1 |
| Positiong frequency layer |  |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR2 PRACH configuration 1 | FR2 PRACH configuration 1 | FR2 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -89 | -89 | -89 |
| PRS | Config 1 | dB | -5.44 | -11.67 | -11.67 |
| Io | Config 1 | dBm/ 9.36MHz | -59.65 | -59.92 | -59.92 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

Table A.7.6.9.1.-5: Void

##### A.7.6.9.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 9.9.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.1.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD_1970049.

#### A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA

##### A.7.6.9.2.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 9.9.2 in an environment with AWGN propagation conditions in FR2 in standalone scenario when dual positioning frequency layer is configured.

Supported test configurations are shown in table A.7.6.9.2.1-1. The test parameters are as given in Table 7.6.7.2.1-2, Table A.7.6.9.2.1-3 and , Table A.7.6.9.2.1-4.

Table A.7.6.9.2.1-1: Supported test configurations for NR RSTD

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the 2 RF channels distributed in dual positioning frequency layers.

The test consists of two consecutive time intervals, with duration of T1 and T2. During time duration T1, the UE shall not have any timing information of Cell 2 and Cell 3. All three cells transmit PRS during T2.Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The NR-DL-TDOA-ProvideAssistanceData and nr-DL-TDOA-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the DL-TDOA assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID # 24 or #13 before T2.

Table A.7.6.9.2.1-2: General test parameters for RSTD measurement reporting delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Reference cell |  |  | Cell 1 | Reference cell is the cell in the DL-TDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 37.355[34]. The reference cell is the PCell in this test case. |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at the first and second places in the neighbour cell list in the DL-TDOA assistance data. |
| SSB configuration | Config 1 |  | SSB.2 FR2 |  |
| SMTC configuration | Config 1 |  | SMTC.1 |  |
| PDSCH RMC configuration | Config 1 |  | SR.1.1 FDD |  |
| RMSI CORESET RMC configuration | Config 1 |  | CR.3.1 TDD | As specified in clause A.3.1.2.1 |
| Dedicated CORESET RMC configuration | Config 1 |  | CR.1.1 FDD |  |
| PRS Configuration | Config 1 |  | PRS.1.1. FR2 | As specified in clause A.3.31 |
| Physical cell ID PCI |  |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| Measurement gap |  |  | GP#24 or GP#13 | GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured |
| Radio frame receive time offset between the cells at the UE antenna connector |  | s | Cell 2 to Cell 1: 0 Cell 3 to Cell 1: 3 | PRS are transmitted from synchronous cells |
| Expected RSTD |  | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells |  | s | 5 | The corresponding parameter in the DL-TDOA assistance data specified in TS 37.355[34] is the expectedRSTD-Uncertainty index |
| Number of cells provided in DL-TDOA assistance data |  |  | 16 | Including the reference cell |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [24] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |
| T1 |  | s | 3 | The length of the time interval from the beginning of each test |
| T2 |  | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |
| AoA setup |  |  | Setup 1 | As defined in A.3.15.1 |
| Beam assumption |  |  | Rough | Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |

Table A.7.6.9.2.1-3: Cell-specific test parameters for RSTD measurement reporting delay during T1

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 | 2 |
| Positiong frequency layer |  |  | 1 | 1 | 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | N/A | N/A |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note 3 | Config 1 | dBm/SCS | -89 |  |  |
| PRS |  | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | Config 1 | dBm/95.04MHz | -58.86 | -60.01 | -60.01 |
| SSB RP Note4 | Config 1 | dBm/SCS | -89 | -Infinity | -Infinity |
|  |  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SSB RP and Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.7.6.9.2.1-4: Cell-specific test parameters for RSTD measurement reporting delay during T2 and T3

| Parameter |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T2 | T2 | T2 |
| RF Channel Number |  |  | 1 | 1 | 2 |
| Positiong frequency layer |  |  | 1 | 1 | 2 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  |  | OP.1 | OP.1 | OP.1 |
| PRACH configuration |  |  | FR2 PRACH configuration 1 | FR2 PRACH configuration 1 | FR2 PRACH configuration 1 |
| Note 3 | Config 1 | dBm/SCS | -89 | -89 | -89 |
| PRS | Config 1 | dB | -5.44 | -11.67 | -11.67 |
| Io | Config 1 | dBm/ 9.36MHz | -59.65 | -59.92 | -59.92 |
| PRS |  | dB | -6 | -13 | -13 |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

Table A.7.6.9.2.1-5: Void

##### A.7.6.9.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 9.9.2.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the DL-TDOA assistance data, Cell 1, within the time duration specified in section 9.9.1.5 starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 10.1.23.3, i.e., between RSTD_0000000 and RSTD_1970049.

### A.7.6.10 PRS-RSRP measurements

#### A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer

##### A.7.6.10.1.1	Test Purpose and Environment

The purpose of the test is to verify the PRS RSRP measurement requirements specified in Clause 9.9.3.5 for single positioning frequency layer under AWGN propagation conditions in standalone scenario. Supported test configurations are shown in table A.7.6.10.1.1-1

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The test parameters are as given in table A.7.6.10.1.1-2, and table A.7.6.10.1.1-3.

Table A. 7.6.6.1.1-1: supported test configurations for PRS RSRP measurement for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.10.1.1-2: General test parameters for PRS RSRP measurement reporting delay

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | Config 1 | NR cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| Gap Pattern Id |  | Config 1 | GP#13 or GP#24Note1 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC parameters |  | Config 1 | SMTC.1 | As specified in clause A.3.11 |
| SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| Expected RSTD | s | Config 1 | 3 |  |
| Expected RSTD uncertainty | s | Config 1 | 5 |  |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#[13] is configured. |  |  |  |  |

Table A.7.6.10.1.1-3: Cell-specific test parameters for PRS RSRP measurement reporting delay

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.3.1 TDD |  | - |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | - |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| PRS configuration |  |  | Config 1 | PRS.1.1 FR2 |  | PRS.1.2 FR2 |  |
| PRS muting configuration |  |  | Config 1 | ‘10’ |  | ‘01’ |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -102 |  | -102 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -93 |  | -93 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -Infinity | -96 | -Infinity | -103 |
| PRS |  | dB | Config 1 | -Infinity | -3 | -Infinity | -10 |
| PRS |  | dB | Config 1 | -Infinity | -3 | -Infinity | -10 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.56 |  | -55.38 |  |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.10.1.2	Test Requirements

The PRS RSRP measurement time fulfils the requirements specified in Clause 9.9.3.5.The UE shall perform and report the PRS RSRP measurements for Cell 2 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

#### A.7.6.10.2 PRS-RSRP reporting delay test case for dual positioning frequency layer

##### A.7.6.10.2.1	Test Purpose and Environment

The purpose of the test is to verify the PRS RSRP measurement requirements specified in Clause 9.9.3.5 for dual positioning frequency layers under AWGN propagation conditions in standalone scenario. Supported test configurations are shown in table A.7.6.10.2.1-1

There are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the different frequency from the PCell.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. Both cells transmit PRS during T2.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

The test parameters are as given in table A.7.6.10.2.1-2, and table A.7.6.10.2.1-3.

Table A.7.6.10.2.1-1: supported test configurations for PRS RSRP measurement for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.10.2.1-2: General test parameters for PRS RSRP measurement reporting delay

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | Config 1 | NR cell 2 | Cell 2 is a neighbour cell in the positioning assistance data. |
| Gap Pattern Id |  | Config 1 | GP#[13] or GP#24Note1 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC parameters |  | Config 1 | SMTC.1 | As specified in clause A.3.11 |
| SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| Expected RSTD | s | Config 1 | 3 |  |
| Expected RSTD uncertainty | s | Config 1 | 5 |  |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#[13] is configured. |  |  |  |  |

Table A.7.6.10.2.1-3: Cell-specific test parameters for PRS RSRP measurement reporting delay

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.3.1 TDD |  | - |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | - |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| PRS configuration |  |  | Config 1 | PRS.1.1 FR2 |  | PRS.1.2 FR2 |  |
| PRS muting configuration |  |  | Config 1 | ‘10’ |  | ‘01’ |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -92.7 | -92.7 | -Infinity | -85.7 |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -Infinity | -92.7 | -Infinity | -85.7 |
| PRS |  | dB | Config 1 | -Infinity | -3 | -Infinity | -10 |
| PRS |  | dB | Config 1 | -Infinity | -3 | -Infinity | -10 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.10.2.2	Test Requirements

The PRS RSRP measurement time fulfils the requirements specified in Clause 9.9.3.5.The UE shall perform and report the PRS RSRP measurements for Cell 2 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

### A.7.6.11	UE Rx-Tx time difference measurements

#### A.7.6.11.1 UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA

##### A.7.6.11.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR2 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.7.6.11.1.1-1.

Table A.7.6.11.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR2.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #13 or ID #24 before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.7.6.11.1.1-2 and Table A.7.6.11.1.1-3 respectively.

Table A.7.6.11.1.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1 | 1 | For both Cell 1 and Cell 2 |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |
| SSB configuration |  | 1 | SSB.2 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| Measurement gap |  | 1 | GP#24 or GP#13 Note 1 |  |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | OFF |  |
| Time offset between serving and neighbour cells | s | 1 | 3 | Synchronous cells |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 20 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. |  |  |  |  |

Table A.7.6.11.1.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  | 1 | Rough |  | Rough |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.1 FR2 |  | PRS.1.1 FR2 |  |
| PRS muting info |  | 1 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.3 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -89 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -91 | -Infinity | -99 |
| Io | dBm/95.04 MHz | 1 | N/A | -57.63 | N/A | -57.63 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

Table A.7.6.11.1.1-4: Void

##### A.7.6.11.1.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.7.6.11.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA

##### A.7.6.11.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in AWGN propagation condition in FR2 in standalone scenario when dual positioning frequency layers are cnfigured.

The supported test configurations in listed in Table A.7.6.11.2.1-1.

Table A.7.6.11.2.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on different RF channels in FR2.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 and Cell 2 mute PRS transmission during T1 and transmit PRS during T2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE during T1. The last TTI containing the two messages shall be provided to the UE T ms before the start of T2, where T = 50 ms is the maximum processing time of the multi-RTT assistance data and location information request.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance containing the PRS resources.

The UE is configured with measurement gap pattern ID #13 or ID #24 before T2.

The UE is configured to transmit SRS during T2.

The general test parameters and cell specific test parameters are as given in Table A.7.6.11.2.1-2 and Table A.7.6.11.2.1-3 respectively.

Table A.7.6.11.2.1-2: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 | Cell 1 is the PCell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| Neighbour cell |  | 1 | Cell 2 | Cell 2 is a neighbour cell in NR-Multi-RTT-ProvideAssistanceData [34]. |
| RF Channel Number |  | 1 | 1 | For Cell 1 |
| RF Channel Number |  | 1 | 2 | For Cell 2 |
| BWchannel | MHz | 1 | 100: NRB,c = 66 |  |
| SSB configuration |  | 1 | SSB.2 FR2 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
| Measurement gap |  | 1 | GP#24 or GP#13 Note 1 |  |
| CP length |  | 1 | Normal |  |
| DRX |  | 1 | OFF |  |
| Time offset between serving and neighbour cells | s | 1 | 3 | Synchronous cells |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 20 |  |
| Note 1: GP#24 is configured if UE supports MG#24, otherwise GP#13 is configured. |  |  |  |  |

Table A.7.6.11.2.1-3: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  | 1 | Rough |  | Rough |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.1 FR2 |  | PRS.1.1 FR2 |  |
| PRS muting info |  | 1 | ‘10’ |  | ‘01’ |  |
| SRS configuration |  | 1 | POS-SRS.3 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -89 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -91 | -Infinity | -99 |
| Io | dBm/95.04 MHz | 1 | N/A | -57.89 | N/A | -59.60 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

Table A.7.6.11.1.1-4: Void

##### A.7.6.11.2.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 and Cell 2 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

## A.7.7	Measurement Performance requirements

Unless explicitly stated otherwise:

-	Reported measurements shall be within defined range of accuracy limits defined in Clause 10 for at least 90 % of the reported cases. If multiple measurement performance requirements are verified in the same test, the reported measurements for each requirement shall be within defined range of accuracy limits of the corresponding requirement defined in Clause 10 for at least 90% of the reported cases.

-	Measurements are performed in RRC_CONNECTED state.

-	The reference channels assume transmission of PDSCH with a maximum number of 5 HARQ transmissions unless otherwise specified.

### A.7.7.1	SS-RSRP

#### A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.3.1.1 and 10.1.3.1.2 for intra-frequency measurements.

##### A.7.7.1.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in Table A.7.7.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in Table A.7.7.1.1.2-2 and A.7.7.1.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The TCI status for Cell 1 is defined in Table A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1. The test consists of two time phases T1 and T2.

Table A.7.7.1.1.2-1: SS-RSRP Intra frequency SS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.1.1.2-2: SS-RSRP  Intra frequency general test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  | freq1 |  | freq1 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 24 |  | 24 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.2 TDD | - | SR.3.2 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Dedicated CORESET Reference channel |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.3 | OP.3 | OP.3 | OP.3 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset with Cell 1 | s | - | 3 | - | 3 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: Void |  |  |  |  |  |

Table A.7.7.1.1.2-3: SS-RSRP Intra frequency OTA related test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 7 |  | Rough |  | Rough |  |
| Note1 | dBm/15kHzNote4 | -91.6 |  | N/A |  |
| Note1 | dBm/SCSNote4 | -82.6 |  | N/A |  |
|  | dB | 6.0 | 1.0 | N/A | N/A |
| Es | dBm/SCSNote4 |  |  | (Table B.2.2-2 Rx Beam Peak +2.1dB) | (Table B.2.2-2 Rx Beam Peak +2.1dB) |
| SSB_RPNote2 | dBm/SCS | -76.6 | -81.6 | (Table B.2.2-2 Rx Beam Peak +2.1dB) | (Table B.2.2-2 Rx Beam Peak +2.1dB) |
| BB Note6 | dB | 2.44 | -5.98 | -5.98 | -5.98 |
| IoNote2 | dBm/95.04 MHz Note4 | -50.05 |  | (Table B.2.2-2 Rx Beam Peak +29.70dB) |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.1.1.3	Test Requirements

The SS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.3.1.1 and relative accuracy requirements in clause 10.1.3.1.2. The following requirements are to be verified:

During T1:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table A.7.7.1.1.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 10.1.3.1.2-1.

During T2:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in table A.7.7.1.1.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 10.1.3.1.2-1.

During T1 and T2:

Relative accuracy of Cell 1 during T2 compared with Cell 1 during T1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 10.1.3.1.2-1

Relative accuracy of Cell 2 during T2 compared with Cell 2 during T1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in Table 10.1.3.1.2-1.

Table A.7.7.1.1.3-1: SS-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| Cell 1 | SSB_RP1 -δ +Gmin ≤ Reported RSRP(dBm) ≤ SSB_RP1 +δ +Gmax |
| Cell 2 | SSB_RP2 -δ +Gmin ≤ Reported RSRP(dBm) ≤ SSB_RP2 +δ +Gmax |
| Note 1: SSB_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.3.1.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

#### A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.5.1.1 and 10.1.5.1.2 for intrer-frequency measurements with the testing configurations for NR cells in Table A.7.7.1.2.1-1.

Table A.7.7.1.2.1-1: Applicable NR configurations for FR2 inter-frequency SS-RSRP accuracy test

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.7.7.1.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.7.1.2.2-1 and Table A.7.7.1.2.2-2 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.7.7.1.2.2-1 and Table A.7.7.1.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.7.7.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN | 1~2 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1~2 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated | 1 |  | 24 |  | 24 |  |
|  | 2 |  | 48 |  | 48 |  |
| Gap pattern ID |  |  | 0 |  | 0 |  |
| Duplex mode | 1~2 |  | TDD |  | TDD |  |
| TDD configuration | 1~2 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD | - | SR.3.2 TDD | - |
|  | 2 |  | SR.3.3 TDD |  | SR.3.3 TDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
|  | 2 |  | CR.3.2 TDD |  | CR.3.2 TDD |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
|  | 2 |  | CCR.3.7 TDD |  | CCR.3.7 TDD |  |
| SSB configuration | 1 |  | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  | 2 |  | SSB.4 FR2 |  | SSB.4 FR2 |  |
| PDSCH/PDCCH subcarrier spacing | 1~2 | kHz | 120 |  | 120 |  |
| OCNG Patterns | 1~2 |  | OP.3 |  | OP.3 |  |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |  | DLBWP.1.3 ULBWP.1.3 |  |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |  | TCI.State.2 |  |
| SMTC configuration | 1~2 |  | SMTC.1 |  | SMTC.1 |  |
| Time offset between Cell 2 and Cell 1 | 1~2 | s | 3 |  | 3 |  |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition | 1~2 | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration | 1~2 | - | 1x2 | 1x2 | 1x2 | AWGN |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void. |  |  |  |  |  |  |

Table A.7.7.1.2.2-2: SS-RSRP inter frequency OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration | 1~2 |  | Setup 4b according to clause A.3.15.4.2 |  | Setup 4b according to clause A.3.15.4.2 |  |
|  |  |  | AoA1 Spherical coverage | AoA2 Rx Beam Peak | AoA1 Spherical coverage | AoA2 Rx Beam Peak |
| Assumption for UE beamsNote 7 | 1~2 |  | Rough |  | Rough |  |
| Note1 | 1 | dBm/15kHzNote4 | -90.6 | -90.6 | (Table B.2.3-2 Rx Beam PeakNote 8 +1.97dB) | (Table B.2.3-2 Rx Beam PeakNote 8 -3.03dB) |
|  | 2 |  | -93.7 | -93.7 |  |  |
| Note1 | 1 | dBm/SCSNote4 | -81.6 | -81.6 | (Table B.2.3-2 Rx Beam PeakNote 8 +11.0dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +6.0dB) |
|  | 2 |  | -81.7 | -81.7 | (Table B.2.3-2 Rx Beam PeakNote 8 +14.0dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +9.0dB) |
|  | 1~2 | dB | 6.0 | 6.0 | 17.0 | -1.0 |
| SSB_RPNote2 | 1 | dBm/SCS | -75.6 | -75.6 | (Table B.2.3-2 Rx Beam PeakNote 8 +28.0dB) | (Table B.2. 3-2 Rx Beam PeakNote 8 +5.0dB) |
|  | 2 |  | -75.7 | -75.7 | (Table B.2.3-2 Rx Beam PeakNote 8 +31.0dB) | (Table B.2. 3-2 Rx Beam PeakNote 8 +8.0dB) |
| (SSB_RPCell 1 – SSB_RPCell 2) | 1~2 | dB | 0 |  | 23.00 |  |
| BBNote6 | 1 | dB | 5.26 | 5.96 | 9.53 | -3.46 |
|  | 2 |  | 4.61 | 5.91 |  |  |
| IoNote2 | 1 | dBm/95.04 MHz Note4 | -50.00 | -50.00 | (Table B.2.3-2 Rx Beam PeakNote 8 +52.68dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +33.13dB) |
|  | 2 |  | -50.09 | -50.09 | (Table B.2.3-2 Rx Beam PeakNote 8 +55.69dB) | (Table B.2.3-2 Rx Beam PeakNote 8 +36.14dB) |
| (Iofreq 1 – Io freq 2) | 1~2 | dB | 0 |  | 19.55 |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot, Io, (SSB_RPCell 2 – SSB_RPCell 1) and (Iofreq 2 – Io freq 1) levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP or ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: The value in Table B.2.3-2 is the Minimum SSB_RP for SCSSSB = 120 kHz, selected according to the operating band of Cell 2 and UE power class, without ∆MBP,n adjustment. |  |  |  |  |  |  |

##### A.7.7.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirements in clause 10.1.5.1.1 and the relative requirements in clause 10.1.5.1.2.

Test 1:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in Table A.7.7.1.2.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in A.7.7.1.2.3-2.

Test 2:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported SS-RSRP is in the range shown in Table A.7.7.1.2.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported SS-RSRP meets the requirements in A.7.7.1.2.3-2.

Table A.7.7.1.2.3-1: SS-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3,4 |
| --- | --- |
| Cell 1 | SSB_RP1 -δ +Gmin +X ≤ Reported RSRP(dBm) ≤ SSB_RP1 +δ +Gmax |
| Cell 2 | SSB_RP2 -δ +Gmin ≤ Reported RSRP(dBm) ≤ SSB_RP2 +δ +Gmax |
| Note 1: SSB_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.5.1.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class Note 4: X is the Spherical coverage gain difference in dB, derived as (UE Refsens - UE Spherical coverage) from TS 38.101-2 [19] clauses 7.3.2 and 7.3.4, selected according to the UE power class and operating band. X is always a negative value. |  |

Table A.7.7.1.2.3-2: SS-RSRP relative accuracy test requirement

|  | Test requirement Notes1,2,3,4 |
| --- | --- |
| Cell 2 – Cell 1 | SSB_RP2 - SSB_RP1 -δ ≤ Reported RSRP(dB) ≤ SSB_RP2 - SSB_RP1 +δ –(X) |
| Note 1: SSB_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration Note 2: δ is the RSRP relative accuracy requirement from Table 10.1.5.1.2-1 Note 3: Void Note 4: X is the Spherical coverage gain difference in dB, derived as (UE Refsens - UE Spherical coverage) from TS 38.101-2 [19] clauses 7.3.2 and 7.3.4, selected according to the UE power class and operating band. X is always a negative value. |  |

#### A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell

##### A.7.7.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.5.1.1 for inter-frequency measurements with the testing configurations in Table A.7.7.1.3.1-1.

Table A.7.7.1.3.1-1: Applicable NR configurations for FR2 inter-frequency SS-RSRP accuracy test

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |  |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |  |

##### A.7.7.1.3.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) in FR1 and Cell 2 in FR2 . The test parameters for the Cell 1 and Cell 2 are given in Table A.7.7.1.3.2-1 and Table A.7.7.1.3.2-2 below. Absolute accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.7.7.1.3.2-1 and Table A.7.7.1.3.2-2. The inter-frequency measurements are supported by a measurement gap.

Table A.7.7.1.3.2-1: SS-RSRP inter-frequency test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 | 100: NRB,c = 66 | 10: NRB,c = 52 | 100: NRB,c = 66 |
|  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| Data RBs allocated | 1,2 |  | 52 | 24 | 52 | 66 |
|  | 3 |  | 106 |  | 106 |  |
| Duplex mode | 1 |  | FDD | TDD | FDD | TDD |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD configuration | 1 |  | N/A | TDDConf.3.1 | N/A | TDDConf.3.1 |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 | SSB.3 FR2 | SSB.1 FR1 | SSB.3 FR2 |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 | OP.3 | OP.1 | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.3 ULBWP.1.3 |  | DLBWP.1.3 ULBWP.1.3 |  |
| TRS Configuration | 1~3 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDCCH/PDSCH TCI Configuration | 1~3 |  | TCI.State.2 |  | TCI.State.2 |  |
| SMTC configuration | 1~3 |  | SMTC.1 |  | SMTC.1 |  |
| Time offset between Cell 2 and Cell 1 | 1~3 | s | 3 |  | 3 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition | 1~3 | - | NA Link only, see clause A.3.7A | AWGN | NA Link only, see clause A.3.7A | AWGN |
| Antenna configuration | 1~3 | - |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void |  |  |  |  |  |  |

Table A.7.7.1.3.2-2: SS-RSRP inter-frequency OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration according to clause A.3.15 |  |  | NA | Setup 2b | NA | Setup 2b |
| Assumption for UE beamsNote 4 |  |  | N/A | Rough | N/A | Rough |
|  | 1~3 | dBm/15kHz |  | -90 |  | NA |
|  | 1~3 | dBm/SSB SCS |  | -80.97 |  | NA |
|  | 1~3 | dB |  | 5 |  | NA |
| Es | 1~3 | dBm/SCS |  |  |  | (Table B.2.3-2 Spherical coverage +1dB) |
| SSB_RPNote1 | 1~3 | dBm/SCS | Link only, see clause A.3.7A | -76.0 | Link only, see clause A.3.7A | Table B.2.3-2 Spherical coverage +1dB) |
| BBNote6 | 1~3 | dB |  | 4.35 |  | -3.81 |
| IoNote1 | 1~3 | dBm/ 95.04MHz |  | 50.18 |  | SSB_RP+28.98 |
| Note 1: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Void Note 3: No additional noise is added by the test system in Test 2. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

##### A.7.7.1.3.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 2 shall fulfil the Absolute requirement in clause 10.1.5.1.1.

### A.7.7.2	SS-RSRQ

#### A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.8.1.1.

##### A.7.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configurations are shown in Table A.7.7.2.1.2-1. . The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.7.7.2.1.2-2 and Table A.7.7.2.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.7.7.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  | Cell 2 | Cell 1 | Cell 2 |  |
| SSB ARFCN |  |  | Freq1 |  |  | Freq1 |  |  |
| Duplex mode |  |  | TDD |  |  | TDD |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  |  | 66 |  |  | 66 |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| TCI state |  |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD |  | - | CR.3.1 TDD |  |  |
| Control channel RMC |  |  | CCR.3.1 TDD |  | - | CCR.3.1 TDD | - |  |
| OCNG Patterns |  |  | OP.1 |  | OP.1 | OP.1 | OP.1 |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  |  | SSB.1 FR2 |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  | 120 | 120 | 120 |  |
| SS-RSSI-Measurement |  |  | Not Applicable |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  | 0 | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Propagation condition |  |  | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  |  | 1x2 | 1x2 |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void Note 5: Void |  |  |  |  |  |  |  |  |

Table A.7.7.2.1.2-3: SS-RSRQ Intra frequency OTA related test parameters

|  | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |  | Setup 1according to clause A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 9 |  | Rough |  |  |  |  |  |  |
| Note1 | dBm/15kHzNote4 | -95 |  |  |  | -95 |  |  |
| Note1 | dBm/SCSNote3 | -86 |  |  |  | -86 |  |  |
|  | dB | 3 |  | 3 |  | -3 |  | -3 |
| SSB_RPNote2 | dBm/SCS Note4 | -83 |  | -83 |  | -89 |  | -89 |
| SS-RSRQ Note2 | dB | -14.77 |  | -14.77 |  | -16.81 |  | -16.81 |
|  | dB | -1.76 |  | -1.76 |  | -4.76 |  | -4.76 |
| IoNote2 | dBm/95.04 MHz Note4 | -50 |  |  |  | -54 |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRQ, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Void Note 7: Void Note 8: Void Note 9: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

##### A.7.7.2.1.3	Test Requirements

The SS-RSRQ absolute measurement accuracy in test 1shall be within the range Nominal SS-RSRQ+2.5dB to Nominal SS-RSRQ-2.5dB and the SS-RSRQ measurement accuracy in test 2 shall be within the range Nominal RSRQ+3.5dB to Nominal RSRQ-3.5dB  according to the requirements in clause 10.1.8.1.1. Nominal RSRQ is the value shown in table A.7.7.2.1.2-3.

#### A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell

##### A.7.7.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.9.1.1 and 10.1.9.1.2 for inter-frequency measurement.

##### A.7.7.2.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.7.7.2.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-frequency measurement are tested by using test parameters in Table A.7.7.2.2.2-2 and Table A.7.7.2.2.2-3.. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A. 7.7.2.2.2-1: SS-RSRQ Inter frequency SS-RSRQ supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.2.2.2-2: SS-RSRQ Inter frequency general test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | Freq1 | freq2 | freq1 | Freq2 |
| SSB Configuration |  |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |
| CSI-RS for tracking |  |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| Duplex mode |  |  | TDD |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | 66 |  | 66 |  |
|  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
| BWP configuration | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation conditions |  |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |  |  |  |

Table A.7.7.2.2.2-3: SS-RSRQ Inter frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| AoA setup |  | Setup 1 in clause A.3.15. |  | Setup 1 in clause A.3.15. |  |
| Assumption for UE beamsNote 8 |  | Rough |  | Rough |  |
| Note1 | dBm/15kHzNote4 | -94.03 | -94.03 | -94.03 | -94.03 |
| Note1 | dBm/SCSNote3 | -85.0 | -85.0 | -85.0 | -85.0 |
|  | dB | -1.75 | -1.75 | -3 | -1.75 |
| SSB_RPNote2 | dBm/SCS Note4 | -86.75 | -86.75 | -88 | -88 |
| SS-RSRQNote2 | dB | -14.75 | -14.75 | -15.56 | -15.56 |
|  | dB | -1.75 | -1.75 | -3 | -3 |
| IoNote2 | dBm/95.04 MHz Note4 | -53.8 | -53.8 | -54.25 | -54.25 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRQ, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Void Note 7: Void Note 8: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.2.2.3	Test Requirements

The SS-RSRQ absolute measurement accuracy in test 1 shall be within the range Nominal SS-RSRQ+2.5dB to Nominal SS-RSRQ -2.5dB and the SS-RSRQ measurement accuracy in test 2 shall be within the range Nominal SS-RSRQ +3.5dB to Nominal SS-RSRQ -3.5dB  according to the requirements in clause 10.1.10.1.1.

The SS-RSRQ relative measurement accuracy shall fulfil the requirements in clause 10.1.10.1.2.

### A.7.7.3	SS-SINR

#### A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.13.1.1.

##### A.7.7.3.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configurations are shown in Table A.7.7.3.1.2-1. . The absolute accuracy of SS-SINR intra-frequency measurement is test by using the parameters in Table A.7.7.3.1.2-2 and Table A.7.7.3.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The TCI status for Cell 1 is defined in Table  A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1.

Table A.7.7.3.1.2-1: SS-SINR Intra frequency SS-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.3.1.2-2: SS-SINR Intra frequency test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | Freq2 |  | Freq2 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 66 |  | 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD |  |
| Dedicated RMSI CORESET Reference Channel |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 |  |  |  |
| SSB configuration |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| SS-RSSI-Measurement |  | Not Applicable |  |  |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void. Note 4: Void |  |  |  |  |  |

Table A.7.7.3.1.2-3: SS-SINR Intra frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  | Setup 1 according to clause A.3.15.1 |  |
| Assumption for UE beamsNote 9 |  | Rough |  | Rough |  |
| Note1 | dBm/15kHz Note4 | -105 |  | -105 |  |
| Note1 | dBm/SCS Note3 | -96 |  | -96 |  |
|  | dB | 4.54 | 2.66 | -3 | -3 |
| SSB_RPNote2 | dBm/SCS Note4 | -91.46 | -93.34 | -99 | -99 |
| SS-SINR Note2 | dB | 0 | -3.2 | -4.76 | -4.76 |
|  | dB | 0 | -3.2 | -4.76 | -4.76 |
| IoNote2 | dBm/95.04 MHz Note4 | -59.2 |  | -64 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Void Note 7: Void Note 8: Void Note 9: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.3.1.3	Test Requirements

The SS-SINR absolute measurement accuracy in test 1 shall be within the range Nominal SS-SINR+3B to Nominal SS-SINR -3dB and the SS-SINR measurement accuracy in test 2 shall be within the range Nominal SS-SINR +3.5dB to Nominal SS-SINR -3.5dB  according to the requirements in clause 10.1.10.13.1.

#### A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell

##### A.7.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.15.1.1 and 10.1.15.1.2 for inter-frequency measurement.

##### A.7.7.3.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.7.7.3.2.2-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table A.7.7.3.2.2-2 and Table A.7.7.3.2.2-3. In all test cases, Cell 1 is the PCell and Cell 2 is target cell. The TCI status for Cell 1 is defined in Table  A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1.

Table A.7.7.3.2.2-1: SS-SINR Inter frequency SS-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.3.2.2-2: SS-SINR Inter frequency general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 66 |  | 66 |  | 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |  |  |  |  |

Table A.7.7.3.2.2-3: SS-SINR Inter frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration | degrees | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 10 |  | Rough |  | Rough |  | Rough |  |
| Note1 | dBm/15kHz Note4 | -105 | -105 | -105 | -105 | -105 | -105 |
| Note1 | dBm/SCS Note3 | -96 | -96 | -96 | -96 | -96 | -96 |
|  | dB | -0.5 | -0.5 | 11.0 | 11.0 | -3.0 | -3.0 |
| SSB_RPNote2 | dBm/SCS Note4 | -96.5 | -96.5 | -85 | -85 | -99 | -99 |
| SS-SINRNote2 | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
|  | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
| IoNote2 | dBm/95.04 MHz Note4 | -69.3 | -69.3 | -55.4 | -55.4 | -65.24 | -65.24 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SSB_RP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.7.3.2.3	Test Requirements

The SS-SINR absolute measurement accuracy in test 1 shall be within the range Nominal SS-SINR +3dB to Nominal SS-SINR -3dB and the SS-SINR measurement accuracy in test 2 shall be within the range Nominal SS-SINR +3.5dB to Nominal SS-SINR -3.5dB  according to the requirements in clause 10.1.15.1.1.

The SS-SINR relative measurement accuracy shall fulfil the requirements in clause 10.1.15.1.2.

### A.7.7.4	L1-RSRP measurement for beam reporting

#### A.7.7.4.1	SSB based L1-RSRP measurement

##### A.7.7.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 9.5.2 and clause 10.1.20.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.7.7.4.1.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Table A.7.7.4.1.1-1: Applicable NR configurations for FR2 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.7.7.4.1.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.7.4.1.2-1 and Table A.7.7.4.1.2-2 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.7.7.4.1.2-1 and Table A.7.7.4.1.2-2.

Here is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.7.7.4.1.2-1: FR2 SSB based L1-RSRP general test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 | freq1 |
| Duplex mode | 1~2 |  | TDD | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Data RBs allocated | 1~2 |  | 66 | 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD | SR.3.2 TDD |
|  | 2 |  | SR.3.3 TDD | SR.3.3 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
|  | 2 |  | CR.3.2 TDD | CR.3.2 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
|  | 2 |  | CCR.3.7 TDD | CCR.3.7 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 | SSB.2 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 | TCI.State.2 |
| SMTC configuration | 1~2 |  | SMTC.1 | SMTC.1 |
| reportConfigType | 1~2 |  | periodic | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 | 2 |
| L1-RSRP reporting period | 1~2 |  | slot320 | slot320 |
| Propagation condition | 1~2 |  | AWGN | AWGN |
| Antenna configuration | 1~2 |  | 1x2 | 1x2 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table A.7.7.4.1.2-2: FR2 SSB based L1-RSRP OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SSB0 | SSB1 | SSB0 | SSB1 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 4 |  |  | Rough |  | Rough |  |
|  | 1~2 | dBm/15kHz | -100 |  | n.a. |  |
|  | 1 | dBm/SSB SCS | -91 |  | n.a. |  |
|  | 2 |  | -88 |  | n.a. |  |
|  | 1~2 | dB | 10 | -2 | n.a. |  |
| SSB_RPNote1 | 1 | dBm/SCS | -81 | -93 | As in Table B.2.4-2 |  |
|  | 2 |  | -78 | -90 | As in Table B.2.4-2 |  |
| IoNote1 | 1~2 | dBm/ 95.04MHz | -51.57 |  | SSB_RP+28.98 |  |
|  | 1~2 | dB | 10 | -2 | n.a. |  |
| Note 1: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Void. Note 3: No additional noise is added by the test system in Test 2. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

##### A.7.7.4.1.3	Test Requirements

After 320ms from the beginning of the test, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in clauses 10.1.20.1. The following requirements are to be verified:

For Test 1:

Absolute accuracy of SSB0 and absolute accuracy of SSB1. The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table A.7.7.4.1.3-1.

Relative accuracy of SSB0 compared with SSB1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in Table 10.1.20.1.2-1.

For Test 2:

Absolute accuracy of SSB0 and absolute accuracy of SSB1. The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table A.7.7.4.1.3-1.

Relative accuracy of SSB0 compared with SSB1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in Table 10.1.20.1.2-1.

Table A.7.7.4.1.3-1: L1-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| SSB0 | SSB_RP0 -δ + Gmin ≤ Reported RSRP(dBm) ≤ SSB_RP0 +δ + Gmax |
| SSB1 | SSB_RP1 -δ + Gmin ≤ Reported RSRP(dBm) ≤ SSB_RP1 +δ + Gmax |
| Note 1: SSB_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the SSB n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.20.1.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

#### A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off

##### A.7.7.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 9.5.3 and clause 10.1.20.2 for L1-RSRP measurements based on CSI-RS with the testing configurations for NR cells in Table A.7.7.4.2.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Table A.7.7.4.2.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.7.7.4.2.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.7.4.2.2-1 and Table A.7.7.4.2.2-2 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.7.7.4.2.2-1 and Table A.7.7.4.2.2-2.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.7.7.4.2.2-1: FR2 CSI-RS based L1-RSRP general test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 | freq1 |
| Duplex mode | 1 |  | TDD | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 | SSB.1 FR2 |
| OCNG Patterns | 1 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 | TCI.State.2 |
| SMTC configuration | 1 |  | SMTC.1 | SMTC.1 |
| CSI-RS | 1 |  | CSI-RS.3.2 TDD | CSI-RS.3.2 TDD |
| reportConfigType | 1 |  | periodic | periodic |
| reportQuantity | 1 |  | cri-RSRP | cri-RSRP |
| Number of reported RS | 1 |  | 2 | 2 |
| L1-RSRP reporting period | 1 |  | slot80 | slot80 |
| Propagation condition | 1 |  | AWGN | AWGN |
| Antenna configuration | 1 |  | 1x2 | 1x2 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table A.7.7.4.2.2-2: FR2 CSI-RS based L1-RSRP OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CSI-RS0 | CSI-RS1 | CSI-RS0 | CSI-RS1 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 4 |  |  | Rough |  | Rough |  |
|  | 1~2 | dBm/15kHz | -100 |  | n.a. |  |
|  | 1~2 | dBm/SSB SCS | -91 |  | n.a. n.a. |  |
|  | 1~2 | dB | 10 | -2 | n.a. |  |
| CSI-RS-RSRPNote1 | 1~2 | dBm/SCS | -81 | -93 | As in Table B.2.4-2 |  |
| IoNote1 | 1~2 | dBm/ 95.04MHz | -59.86 |  | SS-RSRP+28.98 |  |
|  | 1~2 | dB | -51.57 | -2 | n.a. |  |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: No additional noise is added by the test system in Test 2. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

##### A.7.7.4.2.3	Test Requirements

After 640ms from the beginning of the test, the L1-RSRP measurement accuracy for CSI-RS#0 and CSI-RS#1 of Cell 1 shall fulfil the requirements in clause 10.1.20.2. The following requirements are to be verified:

For Test 1:

Absolute accuracy of CSI-RS0 and absolute accuracy of CSI-RS1. The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table A.7.7.4.2.3-1.

Relative accuracy of CSI-RS0 compared with CSI-RS1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in Table 10.1.20.2.2-1.

For Test 2:

Absolute accuracy of CSI-RS0 and absolute accuracy of CSI-RS1. The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table A.7.7.4.2.3-1.

Relative accuracy of CSI-RS0 compared with CSI-RS1. The UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in Table 10.1.20.2.2-1.

Table A.7.7.4.2.3-1: L1-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| CSI-RS0 | CSI-RS _RP0 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP0 +δ + Gmax |
| CSI-RS1 | CSI-RS _RP1 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP1 +δ + Gmax |
| Note 1: CSI-RS_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the CSI-RS n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.20.2.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

### A.7.7.5	CLI measurements

#### A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell

##### A.7.7.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.22.1.1 with the testing configurations for NR cells in Table A.7.7.5.1.1-1.

Table A.7.7.5.1.1-1: Applicable NR configurations for FR2 SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 120 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.7.7.5.1.2	Test parameters

In this set of test cases there is one cell in the test, FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.7.5.1.2-1 and A.7.7.5.1.2-2 below. The test parameter for the (virtual) neighbor cell UE transmitting SRS are given in Table A.7.7.5.1.2-2.

Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS configuration in Table A.7.7.5.1.2-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 2 data symbols before SRS to be transmitted.

Table A.7.7.5.1.2-1: FR2 test parameters for SRS-RSRP accuracy

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 | freq1 |
| Duplex mode | 1 |  | TDD | TDD |
| TDD configuration | 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR2 | SSB.3 FR2 |
| OCNG Patterns | 1 |  | OP.1 | OP.1 |
| TRS configuration | 1 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system | 1 | s | 10.76 | 10.67 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1 |  | AWGN | AWGN |
| Antenna configuration | 1 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.7.5.1.2-2: SRS-RSRP accuracy OTA related test parameters for PCell and Neighbour cell UE in FR2

| Parameter | Unit | T1 | T2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined A.3.15.1 | Setup 1 defined A.3.15.1 |
| Beam assumption Note 5 |  | Fine | Fine |
| Note1 | dBm/15kHzNote3 | -100 | N/A |
| Note1 | dBm/SCSNote3 | -91 | N/A |
|  | dB | 2 | N/A |
| Es | dBm/SCSNote3 |  | (Table B.2.7-2 Rx Beam Peak) |
| SRS_RPNote2 | dBm/SCS | -89 | (Table B.2.7-2 Rx Beam Peak) |
| BB Note4 | dB | >1 | 1 |
| IoNote2 | dBm/95.04 MHz Note3 | -57.89 | (Table B.2.7-2 Rx Beam Peak +50.79dB) |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |

Table A.7.7.5.1.2-3: SRS configuration parameters for FR2 SRS-RSRP accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |
|  | srs-ResourceIdList | 0 |
|  | resourceType | Periodic |
|  | Usage | Codebook |
| SRS-Resource | SRS-ResourceId | 0 |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n2 |
|  | combOffset-n2 | 0 |
|  | cyclicShift-n2 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n1 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | 12 |
|  | freqHopping b-SRS | 0 |
|  | freqHopping b-hop | 0 |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | sl160,25 |
|  | sequenceId | 0 |

##### A.7.7.5.1.3	Test Requirements

The SRS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.22.1.1. The following requirements are to be verified:

During T1:

The UE is deemed to meet the requirement if the reported SRS-RSRP is in the range shown in table A.7.7.5.1.3-1.

During T2:

The UE is deemed to meet the requirement if the reported SRS-RSRP is in the range shown in table A.7.7.5.1.3-1.

Table A.7.7.5.1.3-1: SRS-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| SRS | SRS_RP -δ +Gmin ≤ Reported SRS-RSRP(dBm) ≤SRS_RP +δ +Gmax |
| Note 1: SRS_RP is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.22.1.1-2, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

#### A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell

##### A.7.7.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CLI-RSSI measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.22.2.1 with the testing configurations for NR cells in Table A.7.7.5.2.1-1.

Table A.7.7.5.2.1-1: Applicable NR configurations for FR2 CLI-RSSI accuracy test

| Config | Description |
| --- | --- |
| 1 | 120 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.7.7.5.2.2	Test parameters

In this set of test cases there is one cell in the test, FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.7.5.2.2-1 and A.7.7.5.2.2-2 below.

Before the test UE is configured to perform CLI-RSSI measurement. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for CLI-RSSI resource and on 2 data symbol before. The CLI-RSSI measurement resource configuration is in Table A.7.7.5.2.2-3.

Table A.7.7.5.2.2-1: FR2 test parameters for CLI-RSSI accuracy

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 | freq1 |
| Duplex mode | 1 |  | TDD | TDD |
| TDD configuration | 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR2 | SSB.3 FR2 |
| OCNG Patterns Note2 | 1 |  | OP.1 | OP.1 |
| TRS configuration | 1 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and OCNG from test system | 1 | s | 10.67 | 10.67 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1 |  | AWGN | AWGN |
| Antenna configuration | 1 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |  |

Table A.7.7.5.2.2-2: CLI-RSSI accuracy OTA related test parameters

| Parameter | Unit | T1 | T2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined A.3.15.1 |  |
| Beam assumption Note 5 |  | Fine |  |
| on CLI-RSSI measurement resource Note1 | dBm/15kHzNote3 | -100 |  |
| on CLI-RSSI measurement resource Note1 | dBm/SCSNote3 | -91 |  |
| on CLI-RSSI measurement resource | dB | -Infinity |  |
| RSRP on CLI-RSSI measurement resource Note2 | dBm/SCS | -Infinity |  |
| BB on CLI-RSSI measurement resource Note4 | dB | -Infinity |  |
| Io on CLI-RSSI measurement resource Note2 | dBm/95.04 MHz Note3 | -62.01 |  |
| Io on CLI-RSSI measurement resource Note2 | dBm/1.08 MHz | -81.46 |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |

Table A.7.7.5.2.2-3: CLI-RSSI measurement resource configuration for FR2 CLI-RSSI accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| CLI-RSSI measurement resource | rssi-ResourceId | 0 |
|  | rssi-SCS | 120kHz |
|  | startPRB | 0 |
|  | nrofPRBs | 66 |
|  | startPosition | 3 |
|  | nrofSymbols | 11 |
|  | rssi-PeriodicityAndOffset | sl160, 25 |

##### A.7.7.5.2.3	Test Requirements

The CLI-RSSI measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.22.2.1. The following requirements are to be verified:

During T1:

The UE is deemed to meet the requirement if the reported CLI-RSSI is in the range shown in table A.7.7.5.2.3-1.

During T2:

The UE is deemed to meet the requirement if the reported CLI-RSSI is in the range shown in table A.7.7.5.2.3-1.

Table A.7.7.5.2.3-1: CLI-RSSI absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
|  | Io -δ +Gmin ≤ Reported CLI-RSSI(dBm) ≤Io +δ +Gmax |
| Note 1: Io is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for 1.08MHz Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.22.1.1-2, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

### A.7.7.6	L1-SINR measurement for beam reporting

#### A.7.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off

##### A.7.7.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 9.8.4.1 and clause 10.1.28.1 for L1-SINR measurements based on CSI-RS with the testing configurations for NR cells in Table A.7.7.6.1.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Table A.7.7.6.1.1-1: Applicable NR configurations for FR2 L1-SINR test with CSI-RS based CMR and no dedicated IMR configured

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.7.7.6.1.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.7.6.1.2-1 and Table A.7.7.6.1.2-2 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in Table A.7.7.6.1.2-1 and Table A.7.7.6.1.2-2.

There is no measurement gap configured in the test. Before the test, UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB.

Table A.7.7.6.1.2-1: FR2 CSI-RS based L1-SINR general test parameters

| Parameter | Config | Unit | Test 1 |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| SMTC configuration | 1 |  | SMTC.1 |
| CSI-RS | 1 |  | CSI-RS.3.2 TDD |
| reportConfigType | 1 |  | periodic |
| reportQuantity | 1 |  | cri-SINR-r16 |
| nrofReportedRS | 1 |  | 2 |
| L1-SINR reporting period | 1 |  | slot80 |
| Propagation condition | 1 |  | AWGN |
| Antenna configuration | 1 |  | 1x2 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |

Table A.7.7.6.1.2-2: FR2 CSI-RS based L1-SINR OTA related test parameters

| Parameter | Config |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | CSI-RS0 | CSI-RS1 |
| Angle of arrival configuration |  |  |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 4 |  |  |  | Rough |  |
|  | 1~2 |  | dBm/15kHz | -100 |  |
|  | 1~2 |  | dBm/SSB SCS | -91 |  |
|  | 1~2 |  | dB | 10 | -2 |
| CSI-RS-RSRPNote1 | 1~2 |  | dBm/SCS | -81 | -93 |
| IoNote1 | 1~2 |  | dBm/ 95.04MHz | -59.86 |  |
|  | 1~2 |  | dB | 10 | -2 |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: Void. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.6.1.3	Test Requirements

After 640ms from the beginning of the test, the L1-SINR measurement accuracy for CSI-RS#0 and CSI-RS#1 of Cell 1 shall fulfil the requirements in clauses 10.1.28.1. The following requirements are to be verified:

For Test 1:

Absolute accuracy of CSI-RS0 and absolute accuracy of CSI-RS1. The UE is deemed to meet the requirement if the reported L1-SINR is in the range shown in Table A.7.7.6.1.3-1.

Relative accuracy of CSI-RS0 compared with CSI-RS1. The UE is deemed to meet the requirement if the difference in reported L1-SINR meets the requirements in Table 10.1.28.1.2-1.

Table A.7.7.6.1.3-1: L1-SINR absolute accuracy test requirement

|  | Test requirement Notes1,2 |
| --- | --- |
| CSI-RS0 | L1-SINR0-δ≤ Reported SINR(dB) ≤L1-SINR0+δ |
| CSI-RS1 | L1-SINR1-δ ≤ Reported SINR(dB) ≤L1-SINR1+δ |
| Note 1: L1-SINRn is the equivalent SINR received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the CSI-RS n under consideration Note 2: δ is the SINR absolute accuracy requirement from Table 10.1.28.2.1-1, selected according to the Io used in the test |  |

#### A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR

##### A.7.7.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 9.8.4.2 and clause 10.1.28.2 for L1-SINR measurements with SSB based CMR and CSI-IM based IMR, with the testing configurations for NR cells in Table A.7.7.6.2.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Table A.7.7.6.2.1-1: Applicable NR configurations for FR2 L1-SINR measurement test with SSB based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE FDD, NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.7.7.6.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.7.6.2.2-1 and Table A.7.7.6.2.2-2 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in Table A.7.7.6.2.2-1 and Table A.7.7.6.2.2-2.

Here is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources and one CSI-IM resource set with two CSI-IM resource. UE is configured to perform RLM and BFD measurement based on the SSB resources 0 and 1. UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-IM resources as IMR.

Table A.7.7.6.2.2-1: FR2 L1-SINR general test parameters with SSB based CMR and CSI-IM based IMR

| Parameter | Config | Unit | Test 1 |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~2 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~2 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~2 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| CSI-IM configuration | 1~2 |  | CSI-IM 3.1 TDD |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity-r16 | 1~2 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~2 |  | 2 |
| L1-SINR reporting period | 1~2 |  | slot640 |
| Propagation condition | 1~2 |  | AWGN |
| Antenna configuration | 1~2 |  | 1x2 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |

Table A.7.7.6.2.2-2: FR2 L1-SINR SSB specific test parameters

| Parameter | Config | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | SSB0 | SSB1 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 4 |  |  | Rough |  |
|  | 1~2 | dBm/15kHz | -100 |  |
|  | 1 | dBm/SSB SCS | -91 |  |
|  | 2 |  | -88 |  |
|  | 1~2 | dB | 10 | -2 |
| SS-RSRPNote1 | 1 | dBm/SCS | -81 | -93 |
|  | 2 |  | -78 | -90 |
| IoNote1 | 1~2 | dBm/95.04 MHz | -51.57 |  |
|  | 1~2 | dB | 10 | -2 |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

##### A.7.7.6.2.3	Test Requirements

After 640ms from the beginning of the test, the L1-SINR measurement accuracy for SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 of Cell 1 shall fulfil the requirements in clauses 10.1.28.2. The following requirements are to be verified:

For Test 1:

Absolute accuracy of SSB#0+CSI-IM#0 and absolute accuracy of SSB#1+CSI-IM#1. The UE is deemed to meet the requirement if the reported L1-SINR is in the range shown in Table A.7.7.6.2.3-1.

Relative accuracy of SSB#0+CSI-IM#0 compared with SSB#1+CSI-IM#1. The UE is deemed to meet the requirement if the difference in reported L1-SINR meets the requirements in Table 10.1.28.2.2-2.

Table A.7.7.6.2.3-1: L1-SINR absolute accuracy test requirement

|  | Test requirement Notes1,2 |
| --- | --- |
| SSB#0+CSI-IM#0 | L1_SINR0 -δ ≤ Reported SINR(dB) ≤ L1_SINR0 +δ |
| SSB#1+CSI-IM#1 | L1_SINR1 -δ ≤ Reported SINR(dB) ≤ L1_SINR1 +δ |
| Note 1: L1_SINRn is the equivalent SINR received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the SSB#n+CSI-IM#n under consideration Note 2: δ is the SINR absolute accuracy requirement from Table 10.1.28.2.1-2, selected according to the Io used in the test |  |

#### A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR

##### A.7.7.6.3.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will partly verify the requirements in Clauses 9.8.4.3 and clause 10.1.28.3 for L1-SINR measurements based on CSI-RS as both CMR and IMR with the testing configurations for NR cell in Table A.7.7.6.3.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Table A.7.7.6.3.1-1: Applicable NR configurations for FR2 L1-SINR measurement test with CSI-RS based both CMR based IMR

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.7.7.6.3.2	Test parameters

In this set of test cases there are one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.7.6.3.2-1 and Table A.7.7.6.3.2-2 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in Table A.7.7.6.3.2-1 and Table A.7.7.6.3.2-2.

There is no measurement gap configured in the test. Before the test, UE is configured two CSI-RS resource sets with two CSI-RS resources for each set. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. UE is configured to perform L1-SINR measurement based on the configured CSI-RS as both CMR and IMR.

Table A.7.7.6.3.2-1: FR2 L1-SINR measurement test with CSI-RS based both CMR and IMR

| Parameter | Config | Unit | Test 1 |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| SMTC configuration | 1 |  | SMTC.1 |
| CSI-RS configuration as CMR | 1 |  | CSI-RS.3.2 TDD |
| CSI-RS configuration as IMR | 1 |  | CSI-RS.3.3A TDD |
| reportConfigType | 1 |  | periodic |
| reportQuantity-r16 | 1 |  | cri-SINR-r16 |
| nrofReportedRS | 1 |  | 2 |
| L1-RSRP reporting period | 1 |  | slot80 |
| Propagation condition | 1 |  | AWGN |
| Antenna configuration | 1 |  | 1x2 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |

Table A.7.7.6.3.2-2: FR2 CSI-RS based L1-SINR measurement OTA related test parameters

| Parameter | Config | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | CSI-RS0 | CSI-RS1 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 4 |  |  | Rough |  |
|  | 1~2 | dBm/15kHz | -100 |  |
|  | 1~2 | dBm/SSB SCS | -91 |  |
|  | 1~2 | dB | 10 | 0 |
| CSI-RS-RSRPNote1 | 1~2 | dBm/SCS | -81 | -91 |
| IoNote1 | 1~2 | dBm/ 95.04MHz | -59.86 |  |
|  | 1~2 | dB | 10 | 0 |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: No additional noise is added by the test system in Test 2. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

##### A.7.7.6.3.3	Test Requirements

After 640ms from the beginning of the test, the L1-SINR measurement accuracy for CSI-RS#0+CSI-RS#2 and CSI-RS#1+CSI-RS#3 of Cell 1 shall fulfil the requirements in clause 10.1.28.3. The following requirements are to be verified:

Absolute accuracy of CSI-RS#0 and absolute accuracy of CSI-RS#1. The UE is deemed to meet the requirement if the reported L1-SINR is in the range shown in Table A.7.7.6.3.3-1.

Relative accuracy of CSI-RS#0 compared with CSI-RS#1. The UE is deemed to meet the requirement if the difference in reported L1-SINR meets the requirements in Table 10.1.28.3.2-1.

Table A.7.7.6.3.3-1: L1-SINR absolute accuracy test requirement

|  | Test requirement Notes1,2 |
| --- | --- |
| CSI-RS#0 | L1-SINR0 -δ≤ Reported SINR(dBm) ≤L1-SINR 0 +δ |
| CSI-RS#1 | L1-SINR 1 -δ≤ Reported SINR(dBm) ≤L1-SINR 1 +δ |
| Note 1: L1-SINRn is the equivalent SINR received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the CSI-RS#n under consideration Note 2: δ is the SINR absolute accuracy requirement from Table 10.1.28.3.1-1. |  |

### A.7.7.7	CSI-RSRP

#### A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.7.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.3.2.1 and 10.1.3.2.2 for intra-frequency measurements.

##### A.7.7.7.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in Table A.7.7.7.1.2-1. Both absolute and relative accuracy of CSI-RSRP intra-frequency measurements are tested by using the parameters in Table A.7.7.7.1.2-2 and A.7.7.7.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The TCI status for Cell 1 is defined in Table A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1. The test consists of two time phases T1 and T2.

Table A.7.7.7.1.2-1: CSI-RSRP Intra frequency CSI-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.7.1.2-2: CSI-RSRP Intra frequency general test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 24 |  | 100: NRB,c = 24 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.3 | OP.3 | OP.3 | OP.3 |
| SMTC configuration |  | SMTC.1 |  | SMTC.1 |  |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| CSI-RS configuration for RRM |  | CSI-RS.RRM.FR2.1 TDD |  |  |  |
| Time offset with Cell 1 | s | - | 0.58 | - | 0.58 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |

Table A.7.7.7.1.2-3: CSI-RSRP Intra frequency OTA related test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 7 |  | Rough |  | Assumption for UE beamsNote 7 |  |
| Note1 | dBm/15kHzNote4 | -91.6 |  | N/A |  |
| Note1 | dBm/SCSNote4 | -82.6 |  | N/A |  |
|  | dB | 6.0 | 1.0 | N/A | N/A |
| Es | dBm/SCSNote4 |  |  | (Table B.2.2.2-2 Rx Beam Peak +2.1dB) | (Table B.2.2.2-2 Rx Beam Peak +2.1dB) |
| CSI-RS_RPNote2 | dBm/SCS | -76.6 | -81.6 | (Table B.2.2.2-2 Rx Beam Peak +2.1dB) | (Table B.2.2.2-2 Rx Beam Peak +2.1dB) |
| BB Note6 | dB | 2.44 | -5.98 | -5.98 | -5.98 |
| IoNote2 | dBm/95.04 MHz Note4 | -50.05 |  | (Table B.2.2.2-2 Rx Beam Peak +29.70dB) |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.7.1.3	Test Requirements

The CSI-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.3.2.1 and relative accuracy requirements in clause 10.1.3.2.2. The following requirements are to be verified:

During T1:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported CSI-RSRP is in the range shown in table A.7.7.7.1.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported CSI-RSRP meets the requirements in Table 10.1.3.2.2-1.

During T2:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported CSI-RSRP is in the range shown in table A.7.7.7.1.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported CSI-RSRP meets the requirements in Table 10.1.3.2.2-1.

During T1 and T2:

Relative accuracy of Cell 1 during T2 compared with Cell 1 during T1. The UE is deemed to meet the requirement if the difference in reported CSI-RSRP meets the requirements in Table 10.1.3.2.2-1

Relative accuracy of Cell 2 during T2 compared with Cell 2 during T1. The UE is deemed to meet the requirement if the difference in reported CSI-RSRP meets the requirements in Table 10.1.3.2.2-1.

Table A.7.7.7.1.3-1: CSI-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| Cell 1 | CSI-RS_RP1 -δ +Gmin ≤ Reported RSRP(dBm) ≤ CSI-RS_RP1 +δ +Gmax |
| Cell 2 | CSI-RS_RP2 -δ +Gmin ≤ Reported RSRP(dBm) ≤ CSI-RS_RP2 +δ +Gmax |
| Note 1: CSI-RS_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.3.2.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

#### A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.7.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.5.2.1 and 10.1.5.2.2 for inter-frequency measurements with the testing configurations for NR cells in Table A.7.7.7.2.1-1.

Table A.7.7.7.2.1-1: Applicable NR configurations for FR2 inter-frequency CSI-RSRP accuracy test

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.7.7.7.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.7.7.2.2-1 and Table A.7.7.7.2.2-2 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.7.7.7.2.2-1 and Table A.7.7.7.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.7.7.7.2.2-1: CSI-RSRP inter-frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| BWchannel |  |  | 100: NRB,c = 24 |  | 100: NRB,c = 24 |  |
| Gap pattern ID |  |  | 0 |  | 0 |  |
| Duplex mode |  |  | TDD | TDD | TDD | TDD |
| TDD configuration |  |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Dedicated CORESET Reference Channel |  |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| SSB configuration |  |  | SSB.3 FR2 |  | SSB.3 FR2 |  |
| SMTC configuration |  |  | SMTC.1 |  | SMTC.1 |  |
| OCNG Patterns |  |  | OP.3 |  | OP.3 |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  |  | DLBWP.1.3 ULBWP.1.3 |  | DLBWP.1.3 ULBWP.1.3 |  |
| TRS Configuration |  |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDCCH/PDSCH TCI Configuration |  |  | TCI.State.2 |  | TCI.State.2 |  |
| CSI-RS configuration for RRM |  |  | CSI-RS.RRM.FR2.1 TDD |  | CSI-RS.RRM.FR2.1 TDD |  |
| Time offset between Cell 2 and Cell 3 |  | s | 0.58 |  | 0.58 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | - | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |

Table A.7.7.7.2.2-2: SS-RSRP inter frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 4b according to clause A.3.15.4.2 |  | Setup 4b according to clause A.3.15.4.2 |  |
|  |  | AoA1 Spherical coverage | AoA2 Rx Beam Peak | AoA1 Spherical coverage | AoA2 Rx Beam Peak |
| Note1 | dBm/15kHzNote4 | -90.6 | -90.6 | (Table B.2.3.2-2 Rx Beam Peak +1.97dB) | (Table B.2.3.2-2 Rx Beam Peak -3.03dB) |
| Assumption for UE beamsNote 7 |  | Rough |  | Rough |  |
| Note1 | dBm/SCSNote4 | -81.6 | -81.6 | (Table B.2.3.2-2 Rx Beam Peak +11.0dB) | (Table B.2.3.2-2 Rx Beam Peak +6.0dB) |
|  | dB | 6.0 | 6.0 | 17.0 | -1.0 |
| CSI-RS_RPNote2 | dBm/SCS | -75.60 | -75.60 | (Table B.2.3.2-2 Rx Beam Peak +28.0dB) | (Table B.2.3.2-2 Rx Beam Peak +5.0dB) |
| (CSI-RS_RPCell 1 – CSI-RS_RPCell 2) | dB | 0 |  | 23.00 |  |
| BB Note6 | dB | 5.29 | 5.96 | 8.86 | -3.92 |
| IoNote2 | dBm/95.04 MHz Note4 | -50.03 | -50.03 | (Table B.2.3.2-2 Rx Beam Peak +52.68dB) | (Table B.2.3.2-2 Rx Beam Peak +33.13dB) |
| (Iofreq 1 – Io freq 2) | dB | 0 |  | 19.55 |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS_RP, Es/Iot, Io, (CSI-RS_RPCell 2 – CSI-RS_RPCell 1) and (Iofreq 2 – Io freq 1) levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP or ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.7.2.3	Test Requirements

The CSI-RSRP measurement accuracy for Cell 1 and Cell 2 shall fulfil the absolute requirements in clause 10.1.5.2.1 and the relative requirements in clause 10.1.5.2.2.

Test 1:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported CSI-RSRP is in the range shown in Table A.7.7.7.2.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported CSI-RSRP meets the requirements in A.7.7.7.2.3-2.

Test 2:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported CSI-RSRP is in the range shown in Table A.7.7.7.2.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported CSI-RSRP meets the requirements in A.7.7.7.2.3-2.

Table A.7.7.7.2.3-1: CSI-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3,4 |
| --- | --- |
| Cell 1 | CSI-RS_RP1 -δ +Gmin +X ≤ Reported RSRP(dBm) ≤ CSI-RS_RP1 +δ +Gmax |
| Cell 2 | CSI-RS_RP2 -δ +Gmin ≤ Reported RSRP(dBm) ≤ CSI-RS_RP2 +δ +Gmax |
| Note 1: CSI-RS_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.5.2.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class Note 4: X is the Spherical coverage gain difference in dB, derived as (UE Refsens - UE Spherical coverage) from TS 38.101-2 [19] clauses 7.3.2 and 7.3.4, selected according to the UE power class and operating band. X is always a negative value. |  |

Table A.7.7.7.2.3-2: CSI-RSRP relative accuracy test requirement

|  | Test requirement Notes1,2,3,4 |
| --- | --- |
| Cell 2 – Cell 1 | CSI-RS_RP2 – CSI-RS_RP1 -δ ≤ Reported RSRP(dB) ≤ CSI-RS_RP2 – CSI-RS_RP1 +δ –(X) |
| Note 1: CSI-RS_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration Note 2: δ is the RSRP relative accuracy requirement from Table 10.1.5.2.2-1 Note 3: Void Note 4: X is the Spherical coverage gain difference in dB, derived as (UE Refsens - UE Spherical coverage) from TS 38.101-2 [19] clauses 7.3.2 and 7.3.4, selected according to the UE power class and operating band. X is always a negative value. |  |

### A.7.7.8	CSI-RSRQ

#### A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.8.2.1.

##### A.7.7.8.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configurations are shown in Table A.7.7.8.1.2-1. The absolute accuracy of CSI-RSRQ intra-frequency measurement is tested by using the parameters in Table A.7.7.8.1.2-2 and Table A.7.7.8.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.7.7.8.1.2-1: CSI-RSRQ Intra frequency CSI-RSRQ supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.8.1.2-2: CSI-RSRQ Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | Freq1 |  | Freq1 |  |
| Duplex mode |  |  | TDD |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| TCI state |  |  | TCI.State.0 |  | TCI.State.0 |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD | - | CR.3.1 TDD |  |
| Control channel RMC |  |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 |
| Time offset with Cell 1 |  | s | - | 0.58 | - | 0.58 |
| SMTC configuration |  |  | SMTC.1 |  |  |  |
| SSB configuration |  |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 | 120 | 120 | 120 |
| CSI-RS configuration for RRM |  |  | CSI-RS.RRM.FR2.1 TDD |  |  |  |
| EPRE ratio of CSI-RS to SSS |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
|  |  | dB | 3 | 3 | -3 | -3 |
| Propagation condition |  |  | AWGN |  | AWGN |  |
| Antenna configuration |  |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |

Table A.7.7.8.1.2-3: CSI-RSRQ Intra frequency OTA related test parameters

|  | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |  | Setup 1according to clause A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 9 |  | Rough |  |  |  |  |  |  |
| Note1 | dBm/15kHzNote4 | -95 |  |  |  | -95 |  |  |
| Note1 | dBm/SCSNote3 | -86 |  |  |  | -86 |  |  |
| CSI-RSRPNote2 | dBm/SCS Note4 | -83 |  | -83 |  | -89 |  | -89 |
| CSI-RSRQ Note2 | dB | -14.77 |  | -14.77 |  | -16.81 |  | -16.81 |
|  | dB | -1.76 |  | -1.76 |  | -4.76 |  | -4.76 |
| IoNote2 | dBm/95.04 MHz Note4 | -50 |  |  |  | -54 |  | -54 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RSRQ, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: CSI-RSRQ and CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in Clause 3.5.2. Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

##### A.7.7.8.1.3	Test Requirements

The CSI-RSRQ absolute measurement accuracy in test 1 shall be within the range Nominal CSI-RSRQ+2.5 dB to Nominal CSI-RSRQ-3.5 dB and the CSI-RSRQ measurement accuracy in test 2 shall be within the range Nominal CSI-RSRQ+3.5 dB to Nominal CSI-RSRQ-4.5 dB according to the requirements in clause 10.1.8.2.1 with an additional -1dB margin reflecting the possible impact of UE self noise in the test. Nominal RSRQ is the value shown in table A.7.7.8.1.2-3.

#### A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell

##### A.7.7.8.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.10.2.1 and 10.1.10.2.2 for inter-frequency measurement.

##### A.7.7.8.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.7.7.8.2.2-1. Both absolute accuracy and relative accuracy requirements of CSI-RSRQ inter-frequency measurement are tested by using test parameters in Table A.7.7.8.2.2-2 and Table A.7.7.8.2.2-3. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

Table A. 7.7.2.2.2-1: CSI-RSRQ Inter frequency supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.8.2.2-2: CSI-RSRQ Inter frequency general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq2 | freq1 | Freq2 |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 |
| Time offset with Cell 1 | s | - | 0.58 | - | 0.58 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| CSI-RS configuration for RRM |  | CSI-RS.RRM.FR2.1 TDD |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of CSI-RS to SSS |  |  |  |  |  |
|  | dB | -1.75 | -1.75 | -3 | -1.75 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |

Table A.7.7.8.2.2-3: CSI-RSRQ Inter frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| AoA setup |  | Setup 1 in clause A.3.15. |  | Setup 1 in clause A.3.15. |  |
| Assumption for UE beamsNote 8 |  | Rough |  | Rough |  |
| Note1 | dBm/15kHzNote4 | -94.03 |  | -94.03 |  |
| Note1 | dBm/SCSNote3 | -85.0 |  | -85.0 |  |
| CSI-RSRPNote2 | dBm/SCS Note4 | -86.75 | -86.75 | -88 | -88 |
| CSI-RSRQNote2 | dB | -14.75 | -14.75 | -15.56 | -15.56 |
|  | dB | -1.75 | -1.75 | -3 | -3 |
| IoNote2 | dBm/95.04 MHz Note4 | -53.8 | -53.8 | -54.25 | -54.25 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RSRQ, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: CSI-RSRQ and CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.8.2.3	Test Requirements

The CSI-RSRQ absolute measurement accuracy in test 1 shall be within the range Nominal CSI-RSRQ+2.5dB to Nominal CSI-RSRQ -3.5dB and the CSI-RSRQ measurement accuracy in test 2 shall be within the range Nominal CSI-RSRQ +3.5dB to Nominal CSI-RSRQ -4.5dB according to the requirements in clause 10.1.10.2.1 with an additional -1dB margin reflecting the possible impact of UE self noise in the test.

The CSI-RSRQ relative measurement accuracy shall fulfil the requirements in clause 10.1.10.2.2.

### A.7.7.9	CSI-SINR

#### A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell

##### A.7.7.9.1.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.13.2.1.

##### A.7.7.9.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configurations are shown in Table A.7.7.9.1.2-1. . The absolute accuracy of CSI-SINR intra-frequency measurement is test by using the parameters in Table A.7.7.9.1.2-2 and Table A.7.7.9.1.2-3. In all test cases, Cell 1 is the PCell and Cell 2 the target cell. The TCI status for Cell 1 is defined in Table A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1.

Table A.7.7.9.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.9.1.2-2: CSI-SINR Intra frequency test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | Freq2 |  | Freq2 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD |  |
| Dedicated RMSI CORESET Reference Channel |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| Time offset with Cell 1 | s | - | 0.29 | - | 0.29 |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 |  |  |  |
| SSB configuration |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |
| CSI-RS configuration for RRM |  | CSI-RS.RRM.FR2.1 TDD |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| SS-RSSI-Measurement |  | Not Applicable |  |  |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of CSI-RS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to SSSNote 1 |  |  |  |  |  |
|  | dB | 4.54 | 2.66 | -3 | -3 |
| Propagation conditions |  | AWGN |  |  |  |
| Antenna configuration |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-SINR and CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |

Table A.7.7.9.1.2-3: CSI-SINR Intra frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  | Setup 1 according to clause A.3.15.1 |  |
| Assumption for UE beamsNote 9 |  | Rough |  | Rough |  |
| Note1 | dBm/15kHz Note4 | -105 |  | -105 |  |
| Note1 | dBm/SCS Note3 | -96 |  | -96 |  |
| CSI-RSRPNote2 | dBm/SCS Note4 | -91.46 | -93.34 | -99 | -99 |
| CSI-SINR Note2 | dB | 0 | -3.2 | -4.76 | -4.76 |
|  | dB | 0 | -3.2 | -4.76 | -4.76 |
| IoNote2 | dBm/95.04 MHz Note4 | -59.2 |  | -64 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: CSI-SINR and CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in clause 3.5.2. Note 7: Void Note 8: Void Note 9: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.9.1.3	Test Requirements

The CSI-SINR absolute measurement accuracy in test 1 shall be within the range Nominal CSI-SINR+XdB to Nominal CSI-SINR –X-1dB and the CSI-SINR measurement accuracy in test 2 shall be within the range Nominal CSI-SINR +YdB to Nominal CSI-SINR –Y-1dB according to the requirements in clause 10.1.13.2.1 with an additional -1dB margin reflecting the possible impact of UE self noise in the test. The relative CSI-SINR measurement accuracy shall fulfil the requirements in clause 10.1.13.2.1.

Editor’s note: The values of X and Y are pending on the accuracy requirement discussion

#### A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell

##### A.7.7.9.2.1	Test Purpose and Environment

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.15.2.1 and 10.1.15.2.2 for inter-frequency measurement.

##### A.7.7.9.2.2	Test Parameters

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table A.7.7.9.2.2-1. Both absolute accuracy and relative accuracy requirements of CSI-SINR inter-frequency measurement are tested by using test parameters in Table A.7.7.9.2.2-2 and Table A.7.7.9.2.2-3. In all test cases, Cell 1 is the PCell and Cell 2 is target cell. The TCI status for Cell 1 is defined in Table A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1.

Table A.7.7.9.2.2-1: CSI-SINR Inter frequency CSI-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB and CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.9.2.2-2: CSI-SINR Inter frequency general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |  |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |  |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  | TCI.State.0 |  |  |  |  |  |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Time offset with Cell 1 | s | - | 0.29 | - | 0.29 | - | 0.29 |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| CSI-RS configuration for RRM |  | CSI-RS.RRM.FR2.1 TDD |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to SSSNote 1 |  |  |  |  |  |  |  |
|  | dB | -0.5 | -0.5 | 11.0 | 11.0 | -3.0 | -3.0 |
| Propagation conditions |  | AWGN |  |  |  |  |  |
| Antenna configuration |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-SINR, CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-SINR and CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table A.7.7.9.2.2-3: CSI-SINR Inter frequency OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration | degrees | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 10 |  | Rough |  | Rough |  | Rough |  |
| Note1 | dBm/15kHz Note4 | -105 |  | -105 |  | -105 |  |
| Note1 | dBm/SCS Note3 | -96 |  | -96 |  | -96 |  |
| CSI-RSRPNote2 | dBm/SCS Note4 | -96.5 | -96.5 | -85 | -85 | -99 | -99 |
| CSI-SINRNote2 | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
|  | dB | -0.5 | -0.5 | 11 | 11 | -3.0 | -3.0 |
| IoNote2 | dBm/95.04 MHz Note4 | -69.3 |  | -55.4 |  | -65.24 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: CSI-SINR and CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in clause 3.5.2. Note 7: Void Note 8: Void Note 9: Void Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.7.9.2.3	Test Requirements

The CSI-SINR absolute measurement accuracy in test 1 shall be within the range Nominal CSI-SINR +XdB to Nominal CSI-SINR –X-1dB and the CSI-SINR measurement accuracy in test 2 shall be within the range Nominal CSI-SINR +YdB to Nominal CSI-SINR –Y-1dB according to the requirements in clause 10.1.15.2.1 with an additional -1dB margin reflecting the possible impact of UE self noise in the test.

The CSI-SINR relative measurement accuracy shall fulfil the requirements in clause 10.1.15.2.2.

### A.7.7.10	RSTD measurements

#### A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer

##### A.7.7.10.1.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions.

The supported test configurations are specified in Table A.7.7.10.1.1-1.

Table A.7.7.10.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell. Cell 2 is a neighbour cells. Both cells are on the same NR RF channel in FR2. GP#13 is configured for the test. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

Table A.7.7.10.1.1-2: RSTD accuracy test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| PRS ARFCN |  | freq1 |  | freq1 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.3 | OP.3 | OP.3 | OP.3 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 |
| PRS configuration |  | PRS.1.2 FR2 | PRS.1.2 FR2 | PRS.1.2 FR2 | PRS.1.2 FR2 |
| PRS muting info |  | ‘10’ | ‘01’ | ‘10’ | ‘01’ |
| Expected RSTD | s | N/A | 3 | N/A | 3 |
| Expected RSTD uncertainty | s | N/A | 5 | N/A | 5 |
| Time offset with Cell 1 | s | - | 3 | - | 3 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |

Table A.7.7.10.1.1-3: RSTD accuracy OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 5 |  | Rough |  | Rough |  |
| Note1 | dBm/SCSNote3 | -98 |  | -98 |  |
|  | dB | -6 | -13 | -6 | -13 |
| PRS-RSRPNote2 | dBm/SCS | -104 | -111 | -104 | -111 |
| BB Note4 | dB | -6 | -13 | -6 | -13 |
| IoNote2 | dBm/95.04 MHz Note3 | -68.04 | -68.80 | -68.04 | -68.80 |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.10.1.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

#### A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer

##### A.7.7.10.2.1	Test purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the accuracy requirements specified in clause 10.1.23.2 in an environment with AWGN propagation conditions. The NR-TDOA-ProvideAssistanceData and NR-TDOA-RequestLocationInformation message as defined in TS 37.355 shall be provided to the UE before the start of the test. The test duration should be larger than the UE measurement period as defined in clause 9.9.2.

The supported test configurations are specified in Table A.7.7.10.2.1-1.

Table A.7.7.10.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

In the test there are two synchronous cells: Cell 1 and Cell 2. Cell 1 is the reference as well as the PCell on NR RF channel #1 in FR2. Cell 2 is a neighbour cell on a different NR RF channel #2 in FR2. GP#13 is configured for the test.

Table A.7.7.10.2.1-2: RSTD accuracy test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| PRS ARFCN |  | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - | Not applicable | - |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.3 | OP.3 | OP.3 | OP.3 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 |
| PRS configuration |  | PRS.1.1 FR2 | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.2 FR2 |
| Expected RSTD | s | N/A | 3 | N/A | 3 |
| Expected RSTD uncertainty | s | N/A | 5 | N/A | 5 |
| Time offset with Cell 1 | s | - | 3 | - | 3 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |

Table A.7.7.10.2.1-3: RSTD accuracy OTA related test parameters

| Parameter | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 5 |  | Rough |  | Rough |  |
| Note1 | dBm/SCSNote3 | -98 |  | -98 |  |
|  | dB | -6 | -13 | -6 | -13 |
| PRS-RSRPNote2 | dBm/SCS | -104 | -111 | -104 | -111 |
| BB Note4 | dB | -6 | -13 | -6 | -13 |
| IoNote2 | dBm/95.04 MHz Note3 | -68.04 | -68.80 | -68.04 | -68.80 |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.10.2.2	Test Requirements

The RSTD measurement accuracy for Cell 2 shall fulfil the absolute requirement in clause 10.1.23.2.

### A.7.7.11	PRS-RSRP measurements

#### A.7.7.11.1	SA measurement accuracy with PRS in FR2

##### A.7.7.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that the PRS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.24.2.1 and 10.1.24.2.2.

##### A.7.7.11.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Supported test configurations are shown in Table A.7.7.11.1.2-1. Both absolute and relative accuracy of PRS-RSRP measurements are tested by using the parameters in Table A.7.7.11.1.2-2 and A.7.7.11.1.2-3. In all test cases, Cell 1 is the PCell. The TCI status for Cell 1 is defined in Table A.3.16.2-1 and TRS configuration for Cell 1 is defined in Table A.3.17.2.1-1. The test consists of two time phases T1 and T2.

Table A.7.7.11.1.2-1: PRS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.7.11.1.2-2: PRS-RSRP general test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  | freq1 |  | freq1 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 100: NRB,c = 24 |  | 100: NRB,c = 24 |  |
| Downlink initial BWP configuration |  | DLBWP.0.1 | - | DLBWP.0.1 | - |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 | - | DLBWP.1.1 | - |
| Uplink initial BWP configuration |  | ULBWP.0.1 | - | ULBWP.0.1 | - |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 | - | ULBWP.1.1 | - |
| DRX cycle configuration |  | Not applicable | - | Not applicable | - |
| Measurement gap |  | GP#0 or GP#24 Note2 |  |  |  |
| TRS configuration |  | TRS.2.1 TDD | - | TRS.2.1 TDD | - |
| TCI state |  | TCI.State.0 | - | TCI.State.0 | - |
| PDSCH Reference measurement channel |  | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  | OP.3 | OP.3 | OP.3 | OP.3 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset with Cell 1 | s | - | 3 | - | 3 |
| PRS configuration |  | PRS.1.1 FR2 | PRS.1.2 FR2 | PRS.1.1 FR2 | PRS.1.2 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Propagation conditions |  | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |

Table A.7.7.11.1.2-3: PRS-RSRP OTA related test parameters

| Parameter | Unit | T1 |  | T2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15.1 |  |  |  |
| Assumption for UE beamsNote 7 |  | Rough |  | Rough |  |
| Note1 | dBm/15kHzNote4 | -91.6 |  | N/A |  |
| Note1 | dBm/SCSNote4 | -82.6 |  | N/A |  |
|  | dB | 6.0 | 1.0 | N/A | N/A |
| Es | dBm/SCSNote4 | - | - | (Table B.2.2-2 Rx Beam Peak +2.1dB) | (Table B.2.2-2 Rx Beam Peak +2.1dB) |
| PRS_RPNote2 | dBm/SCS | -76.6 | -81.6 | (Table B.2.2-2 Rx Beam Peak +2.1dB) | (Table B.2.2-2 Rx Beam Peak +2.1dB) |
| BB Note6 | dB | 2.44 | -5.98 | -5.98 | -5.98 |
| IoNote2 | dBm/95.04 MHz Note4 | -50.05 |  | (Table B.2.2-2 Rx Beam Peak +29.70dB) |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: PRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.7.7.11.1.3	Test Requirements

The PRS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clause 10.1.24.2.1 and relative requirement in clause 10.1.24.2.2. The following requirements are to be verified:

During T1:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported PRS-RSRP is in the range shown in table A.7.7.11.1.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported PRS-RSRP meets the requirements in Table 10.1.24.2.2-2.

During T2:

Absolute accuracy of Cell 1 and absolute accuracy of Cell 2. The UE is deemed to meet the requirement if the reported PRS-RSRP is in the range shown in table A.7.7.11.1.3-1.

Relative accuracy of Cell 2 compared with Cell 1. The UE is deemed to meet the requirement if the difference in reported PRS-RSRP meets the requirements in Table 10.1.24.2.2-2.

During T1 and T2:

Relative accuracy of Cell 1 during T2 compared with Cell 1 during T1. The UE is deemed to meet the requirement if the difference in reported PRS-RSRP meets the requirements in Table 10.1.24.2.2-2.

Relative accuracy of Cell 2 during T2 compared with Cell 2 during T1. The UE is deemed to meet the requirement if the difference in reported PRS-RSRP meets the requirements in Table 10.1.24.2.2-2.

Table A.7.7.11.1.3-1: PRS-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| Cell 1 | PRS_RP1 -δ +Gmin ≤ Reported RSRP(dBm) ≤ PRS_RP1 +δ +Gmax |
| Cell 2 | PRS_RP2 -δ +Gmin ≤ Reported RSRP(dBm) ≤ PRS_RP2 +δ +Gmax |
| Note 1: PRS_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the cell n under consideration. Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.24.2.1-2, selected according to the Io used in the test. Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

### A.7.7.12	UE Rx-Tx time difference measurements

#### A.7.7.12.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA

##### A.7.7.12.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR2 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.7.7.12.1.1-1.

Table A.7.7.12.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB and PRS SCS, 100 MHz bandwidth, TDD duplex mode |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR2.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test.

The UE is configured with measurement gap pattern ID #0 or ID #24 before the test.

The UE is configured to transmit SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.7.7.12.1.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in Table A.7.7.12.1.2-1.

Table A.7.7.12.1.2-1: SRS configuration for UE Rx-Tx time difference measurement accuracy test

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| AoA setup |  | 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  | 1 | Rough |  | Rough |  |
| Measurement gap |  | 1 | GP#24 or GP#0 Note 8 |  |  |  |
| DRX |  | 1 | OFF |  |  |  |
| Time offset with Cell 1 | s | 1 | N/A |  | 3 |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.2.1 TDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | N/A |  |
| PRS configuration |  | 1 | PRS.1.1 FR2 |  | PRS.1.1 FR2 |  |
| SRS configuration |  | 1 | POS-SRS.3 |  | N/A |  |
| Note 2 | dBm/SCS | 1 | -89 |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
| PRS | dB | 1 | -Infinity | -2.41 | -Infinity | -12.12 |
| PRS | dB | 1 | -Infinity | -2 | -Infinity | -10 |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -91 | -Infinity | -99 |
| Io | dBm/95.04 MHz | 1 | N/A | -57.63 | N/A | -57.63 |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |  |  |

Table A.7.7.12.1.2-2: Void

##### A.7.7.12.1.3	Test requirements

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

# A.8	E-UTRA standalone tests for NR RRM

Editor notes: All NR RRM tests under E-UTRA standalone operations are included in this Annex. All EN-DC related NR RRM tests are in A.4 and A.5.

## A.8.1	Void

## A.8.2	RRC_IDLE state mobility

### A.8.2.1	Inter-RAT NR Cell re-selection

#### A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1

##### A.8.2.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the E-UTRAN to NR inter-RAT cell reselection requirements specified in clause 4.2.2.5.6 in TS 36.133 [15].

The test scenario comprises of 1 E-UTRA cell and 1 NR cell as given in tables A.8.2.1.1.1-1, A.8.2.1.1.1-2, A.8.2.1.1.1-3 and A.8.2.1.1.1-4. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. E-UTRA cell 1 is already identified by the UE prior to the start of the test. Cell 2 is of higher priority than cell 1.

Table A.8.2.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.2.1.1.1-2: General test parameters for E-UTRA cell re-selection FR1 NR cell test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE camps on cell 2 in the initial phase |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T1 end condition | Active cell |  |  | Cell1 | During T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  |  | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3, 4, 5, 6 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2, 5 | 3 s | Synchronous cells |
|  |  |  | 3, 6 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.8.2.1.1.1-3: Cell specific test parameters for NR cell 2

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.2 FR1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Qhysts | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4, 5, 6 | SS-RSRP |  |  |
|  | dB | 1, 4 | -4 | -infinity | 12 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |
|  |  | 2, 5 | -98 |  |  |
|  |  | 3, 6 | -95 |  |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |  |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4 | -4 | -infinity | 12 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -102 | -infinity | -86 |
|  |  | 2, 5 | -102 | -infinity | -86 |
|  |  | 3, 6 | -99 | -infinity | -83 |
| Io | dBm/9.36 MHz | 1, 4 | -68.60 | -70.05 | -57.78 |
|  | dBm/9.36 MHz | 2, 5 | -68.60 | -70.05 | -57.78 |
|  | dBm/38.16 MHz | 3, 6 | -62.50 | -63.95 | -51.69 |
| Treselection | s | 1, 2, 3, 4, 5, 6 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.2.1.1.1-4: Cell specific test parameters for E-UTRA cell 1

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -84 |
|  | dB | 14 | 14 | 14 |
|  | dB | 14 | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | 50 |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.8.2.1.1.2	Test Requirements

The cell reselection delay to a higher priority NR cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2 in TS 36.133 [15]

Tevaluate, NR	See Table 4.2.2.5.6-1 in clause 4.2.2.5.6 in TS 36.133 [15]

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority NR cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

#### A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16

##### A.8.2.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the E-UTRAN to NR inter-RAT cell reselection requirements specified in clause 4.2.2.5.6 in 36.133 [15].

The test scenario comprises of 1 E-UTRA cell and 1 NR cell as given in tables A.8.2.1.2.1-1, A.8.2.1.2.1-2, A.8.2.1.2.1-3 and A.8.2.1.2.1-4. In SIB of the E-UTRA cell, highSpeedInterRAT-NR-r16 is configured and the carrier of NR cell is configured with highSpeedCarrierNR-r16. The test consists of two time periods, with time duration of T1 and T2 respectively. Both E-UTRA cell 1 and NR cell 2 are already identified by the UE prior to the start of the test. NR cell 2 is of lower priority than E-UTRA cell 1.

Table A.8.2.1.2.1-1: Supported test configurations for UE configured with highSpeedInterRAT-NR-r16

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.2.1.2.1-2: General test parameters in E-UTRA cell re-selection FR1 NR cell test case for UE configured with highSpeedInterRAT-NR-r16

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1 |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3, 4, 5, 6 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2, 5 | 3 s | Synchronous cells |
|  |  |  | 3, 6 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 0.32 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 87 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.8.2.1.2.1-3: Cell specific test parameters for NR cell 2 in E-UTRA cell re-selection FR1 NR cell test case for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.1.1 FDD |  |
|  |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.1.1 FDD |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
| Pcompensation | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Qhysts | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Qoffsets, n | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4, 5, 6 | SS-RSRP |  |
|  | dB | 1, 4 | 14 | 14 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |
|  |  | 2, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | 14 | 14 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -84 | -84 |
|  |  | 2, 5 | -84 | -84 |
|  |  | 3, 6 | -81 | -81 |
| Io | dBm/9.36 MHz | 1, 4 | -55.88 | -55.88 |
|  | dBm/9.36 MHz | 2, 5 | -55.88 | -55.88 |
|  | dBm/38.16 MHz | 3, 6 | -47.79 | -47.79 |
| Treselection | s | 1, 2, 3, 4, 5, 6 | 0 |  |
| Snonintrasearch | dB | 1, 2, 3, 4, 5, 6 | Not sent |  |
| Threshx, high | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, low | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, low | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN 3334 Note 4 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |

Table A.8.2.1.2.1-4: Cell specific test parameters for E-UTRA cell 1 in E-UTRA cell re-selection FR1 NR cell test case for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
| Note 2 | dBm/15 kHz | -98 |  |
| RSRP Note 3 | dBm/15 KHz | -102 | -86 |
|  | dB | -4 | 12 |
|  | dB | -4 | 12 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | 50 |  |
| Threshx, high | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN 1944 Hz Note4 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. |  |  |  |

##### A.8.2.1.2.2	Test Requirements

The cell reselection delay to a lower priority NR cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 3 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR + TSI-NR,

Where:

Tevaluate, NR	See Table 4.2.2.5.6-2 in clause 4.2.2.5.6 in [15]

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 2.24 s, allow 3 s for the cell re-selection delay to a lower priority NR cell.

### A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting

#### A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1

##### A.8.2.2.1.1	Test Purpose and Environment

This test is to verify the requirement for the E-UTRAN to NR inter-RAT Idle mode DC measurement requirements specified in clause 4.9.2.4 in TS 36.133 [15]. This test is also to verify the accuracy requirement for the E-UTRAN to NR inter-RAT Idle mode DC measurement requirements specified in clause 9.11.1A and 9.11.2A in TS 36.133 [15]. Supported test configurations are shown in Table A.8.2.2.1.1-1.

Table A.8.2.2.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

The test scenario comprises of 1 E-UTRA cell (Cell 1) and 1 NR cell (Cell 2). The the test parameters and applicability for the E-UTRAN cell are defined in Table A.8.2.2.1.1-4. The general test parameters and the cell specific test parameters for the NR cell are speficied in Table A.8.2.2.1.1-2 and Table A.8.2.2.1.1-3, respectively.

The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Prior to the start of the time duration T1, the UE shall be connected to Cell 1. During T1, Cell 2 shall be powered off. At the end of T1, the RRC connection to Cell 1 is released and UE is configured Idle mode DC measurement on the carrier frequency of Cell 2. Time duration T2 starts when the RRC connection is released, and during the T2 UE is in Idle mode. Cell 2 shall be powered on from the beginning of T2. At beginning of T3 the UE is paged for connection setup and requested by the network to send idle mode measurements.

Table A.8.2.2.1.1-2: General test parameters

| Parameter | Unit | Test | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  | configuration |  |  |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRAN Cell 1 |  |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR Cell 2 |  |
| RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1: Cell 1 2: Cell 2 |  |
| DRX cycle length | s | 1, 2, 3, 4, 5, 6 | 1.28 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2, 3, 4, 5, 6 | 3 s |  |
| T1 | s | 1, 2, 3, 4, 5, 6 | 0.5 |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 71 |  |
| T3 | s | 1, 2, 3, 4, 5, 6 | 2 |  |
| T331 | s | 1, 2, 3, 4, 5, 6 | 300 |  |

Table A.8.2.2.1.1-3: Cell specific test parameters for NR cell 2

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.2 FR1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |
|  | dB | 1, 4 | -infinity | 4 | 4 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -98 |  |  |
|  |  | 2, 5 | -98 |  |  |
|  |  | 3, 6 | -95 |  |  |
| Note2 | dBm/15 kHz | 1, 4 | -98 |  |  |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4 | -infinity | -4 | -4 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -infinity | -102 | -102 |
|  |  | 2, 5 | -infinity | -102 | -102 |
|  |  | 3, 6 | -infinity | -99 | -99 |
| SS-RSRQ Note3 | dB | 1, 4 | -infinity | -16.25 | -16.25 |
|  |  | 2, 5 | -infinity | -16.25 | -16.25 |
|  |  | 3, 6 | -infinity | -16.25 | -16.25 |
| Io | dBm/9.36 MHz | 1, 4 | -70.05 | -68.60 | -68.60 |
|  | dBm/9.36 MHz | 2, 5 | -70.05 | -68.60 | -68.60 |
|  | dBm/38.16 MHz | 3, 6 | -63.96 | -62.50 | -62.50 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.2.2.1.1-4: Cell specific test parameters for E-UTRA cell 1

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -84 |
| RSRQ Note 3 | dB | -10.96 | -10.96 | -10.96 |
|  | dB | 14 | 14 | 14 |
|  | dB | 14 | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | 50 |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| beamMeasConfigIdle |  | True |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.8.2.2.1.2	Test Requirements

At the beginning of the time-period T2 the connection is released, and UE enters idle mode. During the time period T2 the UE is in Idle mode and Cell 2 is active. The UE shall not perform reselection. The UE shall perform Idle Mode DC measurement according to clause 4.9.2.4 in TS 36.133 [15]. UE shall be able to detect, acqure the SSB index and measure the SS-RSRP and SS-RSRQ from Cell 2 for Idle mode DC measurement during T2.

NOTE:	The Idle mode DC measurement period for the test setup can be expressed as: Thigher_priority_search + TSSB_index,NR + Tevaluate, NR.

Where:

Thigher_priority_search	See clause 4.2.2 in TS 36.133 [15]

TSSB_index,NR	See Table 4.9.2.4-1 in clause 4.9.2.4 in TS 36.133 [15]

Tevaluate, NR	See Table 4.2.2.5.6-1 in clause 4.2.2.5.6 in TS 36.133 [15]

This gives a total of 70.24 s, allow 71 s for the T2.

At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in Section 9.1.2B in TS 36.133 [15] and Section 9.1.3B, respectively and test ends.

The rate of correct events observed during repeated tests shall be at least 90%.

## A.8.3	RRC_CONNECTED state mobility

### A.8.3.1	Handover

#### A.8.3.1.1	E-UTRAN - NR handover in FR1

##### A.8.3.1.1.1	Test Purpose and Environment

This test shall verify the E-UTRAN to NR FR1 handover requirements as specified in clause 6.1.2.1 specified in clause 5.3.4 in TS 36.133 [15].

The test comprises of one E-UTRA carrier and one NR carrier. There are two cells and one cell on each carrier. Cell 1 is the E-UTRAN and Cell 2 is an inter-RAT NR neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 of TS 36.133 [15] is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.8.3.1.1-1. General test parameters are provided in Table A.8.3.1.1-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.8.3.1.1-3 and A.8.3.1.1-4 respectively.

Table A.8.3.1.1-1: Supported test configurations for E-UTRAN inter-RAT NR handover

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.3.1.1-2: General test parameters for E-UTRAN inter-RAT NR handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | E-UTRAN cell |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | -83 | Absolute E-UTRAN RSRP threshold for event B2 |
| b2-Threshold2NR |  | dBm | As specified in Table A.8.3.1.1-4 | Absolute NR SS-RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts [15] |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.8.3.1.1-3: Cell specific test parameters for E-UTRAN inter-RAT NR handover (Cell 1)

| Parameter | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |  |
|  |  | 4, 5, 6 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 7 | 7 | 7 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | 7 | 7 | 7 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -91 | -91 | -91 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -91 | -91 | -91 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -62.43 | -62.43 | -62.43 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

Table A.8.3.1.1-4: Cell specific test parameters E-UTRAN inter-RAT NR handover (Cell 2)

| Parameter |  | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |
| Duplex mode |  |  | 1, 4 | FDD |  |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |  |
| b2-Threshold2NR |  | dBm | 1, 2, 4, 5 | -106 |  |  |
|  |  |  | 3, 6 | -103 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -98 |  |  |
|  |  |  | 3, 6 | -95 |  |  |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | -inifinity | 0 | 0 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | -inifinity | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -inifinity | -98 | -98 |
|  |  |  | 3, 6 | -inifinity | -95 | -95 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -70.05 | -67.04 | -67.04 |
|  |  | dBm/38.16 MHz | 3, 6 | -63.96 | -60.94 | -60.94 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.8.3.1.1.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 112 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in TS36.133.

Tinterrupt = 62 ms in the test; Tinterrupt is defined in TS36.133 clause 5.3.4.3.

This gives a total of 112 ms.

## A.8.4	Measurement procedure

### A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay

#### A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX

##### A.8.4.1.1.1	Test Purpose and Environment

The purpose of this test is to partly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell in FR1 is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [15] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and no DRX is configured.

The tests consist of a single time period of duration T1. Two carriers are used in the tests: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier with the NR neighbour cell (Cell 2).

Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 1 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report.

The supported test configurations are listed in Table A.8.4.1.1.1-1 below. Test parameters and cell-specific parameters for the NR cell are provided in Tables A.8.4.1.1.1-2 and A.8.4.1.1.1-3 below, respectively. Cell-specific parameters for the E-UTRA cell are provided in Table A.3.7.2.1-1 in clause A.3.7.2.1.

Table A.8.4.1.1.1-1: Applicable E-UTRA and NR configurations for inter-RAT SFTD measurement delay test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.4.1.1.1-2: Applicable E-UTRA and NR configurations for inter-RAT SFTD measurement delay test

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One NR FR1 carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4,5,6 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| SSB configuration |  | Config 1,4 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2,5 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3,6 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Frame time offset between serving and neighbour cells | ms | Config 1,2,3,4 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | s | Config 5,6 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| T1 | s | Config 1,2,3,4,5,6 | 1 |  |  |

Table A.8.4.1.1.1-3: Cell specific test parameters for Cell 2 in inter-RAT SFTD measurement delay test

| Parameter | Unit | Test configuration | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |
| Duplex mode |  | Config 1,4 | FDD |
|  |  | Config 2,3,5,6 | TDD |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |
|  |  | Config 2,5 | 10: NRB,c = 52 |
|  |  | Config 3,6 | 40: NRB,c = 106 |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |
|  |  | Config 3,6 | TDDConf.2.1 |
| OCNG Pattern defined in A.3.2.1.1 |  | Config 1,2,3,4,5,6 | OP.1 |
| SMTC configuration |  | Config 1,2,3,4,5,6 | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |
|  |  | Config 3,6 | 30 |
| EPRE ratio of PSS to SSS | dB | Config 1,2,3,4,5,6 | 0 |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 | dB |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 | dB |  |  |
| Noc Note2 | dBm/15kHz |  | -98 |
| Noc Note2 | dBm/SCS | Config 1,2,4,5 | -98 |
|  |  | Config 3,6 | -95 |
| SS-RSRP Note 3, 4 | dBm/SCS | Config 1,2,4,5 | -94 |
|  |  | Config 3,6 | -91 |
| Ês/Iot | dB | Config 1,2,3,4,5,6 | 4 |
| Ês/Noc | dB | Config 1,2,3,4,5,6 | 4 |
| Io Note 3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 |
|  | dBm/38.16MHz | Config 3,6 | -58.50 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

##### A.8.4.1.1.2	Test Requirements

Following the start of T1, the UE shall detect Cell 2 and determine the relative time difference between Cell 1 and Cell 2. At latest at TRRC_procedure_delay + Tmeasure_SFTD1 after the beginning of time duration T1, the UE shall send a measurement report on SFTD between Cell 1 and Cell 2.

The observed rate of successful SFTD reports in repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2×TTIDCCH longer than the measurement reporting delays above due to TTI insertion uncertainty of the measurement report in DCCH.

#### A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX

##### A.8.4.1.2.1	Test Purpose and Environment

The purpose of this test is to partly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell in FR1 is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [15] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and DRX is configured.

The tests consist of a single time period of duration T1. Two carriers are used in the tests: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier with the NR neighbour cell (Cell 2).

Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 1 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report.

The supported test configurations are listed in Table A.8.4.1.2.1-1 below. Test parameters are provided in Tables A.8.4.1.2.1-2 below. Cell-specific parameters for the E-UTRA and NR cells are provided in Table A.3.7.2.1-1 in clause A.3.7.2.1, and Table A.8.4.1.1.1-3 in clause A.8.4.1.1.1, respectively.

Table A.8.4.1.2.1-1: Applicable E-UTRA and NR configurations for inter-RAT SFTD measurement delay test in DRX

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.4.1.2.1-2: Applicable E-UTRA and NR configurations for inter-RAT SFTD measurement delay test in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One NR FR1 carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4,5,6 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| SSB configuration |  | Config 1,4 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2,5 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3,6 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2,3,4,5,6 | DRX.4 |  | DRX configuration as specified in clause A.3.3.4 |
| Frame time offset between serving and neighbour cells | ms | Config 1,2,4,5 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | s | Config 3,6 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| T1 | s | Config 1,2,3,4,5,6 | 1 |  |  |

##### A.8.4.1.2.2	Test Requirements

Following the start of T1, the UE shall detect Cell 2 and determine the relative time difference between Cell 1 and Cell 2. At latest at the earliest DRX activity time following upon  TRRC_procedure_delay + Tmeasure_SFTD1 from the beginning of time duration T1, the UE shall send a measurement report on SFTD between Cell 1 and Cell 2.

The observed rate of successful SFTD reports in repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2×TTIDCCH longer than the measurement reporting delays above due to TTI insertion uncertainty of the measurement report in DCCH.

### A.8.4.2	E-UTRA – NR Inter-RAT Measurements

#### A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used

##### A.8.4.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21 of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.1.1-1, A.8.4.2.1.1-2, A.8.4.2.1.1-3 and A.8.4.2.1.1-4.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.8.4.2.1.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.8.4.2.1.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.1.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.1.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 | 19 | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 1 | 1 |  |
| Note 1: The value of b2-Threshold1 is defined in Table A.8.4.2.1.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.8.4.2.1.1-4 |  |  |  |  |  |

Table A.8.4.2.1.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.8.4.2.1.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

##### A.8.4.2.1.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used

##### A.8.4.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21 of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.2.1-1, A.8.4.2.2.1-2, A.8.4.2.2.1-3 and A.8.4.2.2.1-4.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.8.4.2.2.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.8.4.2.2.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.2.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.2.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  | Test 3 | Test 4 |  |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  |  | 4 |  |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  |  | 19 |  |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  |  |  |  |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  |  |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 |  | DRX.9 | DRX.12 |  | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  |  |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  |  |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 |  | 11 | 2 |  | 11 |  |
| Note 1: The value of b2-Threshold1 is defined in Table A.8.4.2.2.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.8.4.2.2.1-4 |  |  |  |  |  |  |  |  |  |

Table A.8.4.2.2.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.8.4.2.2.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

##### A.8.4.2.2.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used

##### A.8.4.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1.  The test parameters are given in Tables A.8.4.2.3.1-1, A.8.4.2.3.1-2, A.8.4.2.3.1-3 and A.8.4.2.3.1-4.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.8.4.2.3.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.8.4.2.3.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.3.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.3.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 | 19 | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 | 1 |  |
| Note 1: The value of b2-Threshold1 is defined in Table A.8.4.2.3.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.8.4.2.3.1-4 |  |  |  |  |  |

Table A.8.4.2.3.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.8.4.2.3.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

##### A.8.4.2.3.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1040 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used

##### A.8.4.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.4.1-1, A.8.4.2.4.1-2, A.8.4.2.4.1-3 and A.8.4.2.4.1-4.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.8.4.2.4.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.8.4.2.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.4.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.4.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  | Test 3 | Test 4 |  |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  |  | 4 |  |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  |  | 19 |  |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  |  |  |  |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  |  |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 |  | DRX.9 | DRX.12 |  | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  |  |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  |  |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 |  | 13 | 2 |  | 13 |  |
| Note 1: The value of b2-Threshold1 is defined in Table A.8.4.2.4.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.8.4.2.4.1-4 |  |  |  |  |  |  |  |  |  |

Table A.8.4.2.4.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.8.4.2.4.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | TDL-C 300ns 100Hz |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

##### A.8.4.2.4.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 12160 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 12160 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used

##### A.8.4.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.5.1-1, A.8.4.2.5.1-2 and A.8.4.2.5.1-3.

The cell specific test parameters for E-UTRA cell1 as PCell are defined in clause A.3.7.2.2.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.8.4.2.5.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.8.4.2.5.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [16] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have timing information of NR cell 2.

Table A.8.4.2.5.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR2 in non-DRX

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.5.1-2: General test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | One E-UTRAcarrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.3.7.2.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [16]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 10 |  |  |
| T2 | s | 1, 2 | 6 | 3 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table A.8.4.2.5.1-3 |  |  |  |  |  |

Table A.8.4.2.5.1-3: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in non-DRX

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| AoA setup defined in A.3.15.2.1 |  |  | 1, 2 | Setup 2a |  |
| Assumption for UE beamsNote 5 |  |  | 1, 2 | Rough |  |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.3.2.1.1 |  |  | 1, 2 | OP.3 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -112 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Ês |  | dBm/SCS | 1, 2 | -Infinity | -80.6 |
| SSB_RP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -80.6 |
| BB Note 6 |  | dB | 1, 2 | -Infinity | 8.3 |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -Infinity | -56.0 |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 6: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |

##### A.8.4.2.5.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is not required to report SSB time index.

Table A.8.4.2.5.2-1: Test requirements for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in non-DRX

| Test case | Measurement reporting delay (ms) |  |
| --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms |
| UE power class 3 | 3200 | 1600 |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used

##### A.8.4.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 1.  The test parameters are given in Tables A.8.4.2.6.1-1, A.8.4.2.6.1-2 and A.8.4.2.6.1-3.

The cell specific test parameters for E-UTRA cell1 as PCell are defined in clause A.3.7.2.2.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.8.4.2.6.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.8.4.2.6.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [16] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have timing information of NR cell 2.

Table A.8.4.2.6.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR2 in DRX

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.6.1-2: General test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  | Test 3 | Test 4 |  |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 |  |  |  |  |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.3.7.2.2. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  |  | 4 |  |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  |  | 19 |  |  | As specified in TS 36.331 [16]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  |  |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 |  | DRX.9 | DRX.12 |  | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  |  |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  |  |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 6 |  | 83 | 6 |  | 83 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table A.8.4.2.6.1-3 |  |  |  |  |  |  |  |  |  |

Table A.8.4.2.6.1-3: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| AoA setup defined in A.3.15.1 |  |  | 1, 2 | Setup 1 |  |
| Assumption for UE beamsNote 5 |  |  | 1, 2 | Rough |  |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.3.2.1.1 |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -106 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | dBm/15kHz | 1, 2 | -104.7 |  |
| Note2 |  | dBm/SCS | 1, 2 | -95.7 |  |
| SSN_RP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -87.7 |
|  |  | dB | 1, 2 | -Infinity | 8 |
|  |  | dB | 1, 2 | -Infinity | 8 |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -66.7 | -58.0 |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.8.4.2.6.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D3 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D4 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is not required to report SSB time index.

Table A.8.4.2.6.2-1: Test requirements for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Test case | Measurement reporting delay (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms | Test 3: D3 ms | Test 4: D4 ms |
| UE power class 3 | 4800 | 51200 | 4800 | 51200 |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used

##### A.8.4.2.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.7.1-1, A.8.4.2.7.1-2 and A.8.4.2.7.1-3.

The cell specific test parameters for E-UTRA cell1 as PCell are defined in clause A.3.7.2.2.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.8.4.2.7.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.8.4.2.7.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.7.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR2 in non-DRX

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.7.1-2: General test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  | 1, 2 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Numbers |  | 1, 2 | 1 |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.3.7.2.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [16]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 5 | 3 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table A.8.4.2.5.1-3 |  |  |  |  |  |

Table A.8.4.2.7.1-3: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in non-DRX

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| AoA setup defined in A.3.15.1 |  |  | 1, 2 | Setup 1 |  |
| Assumption for UE beamsNote 5 |  |  | 1, 2 | Rough |  |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.3.2.1.1 |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -106 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | dBm/15kHz | 1, 2 | -104.7 |  |
| Note2 |  | dBm/SCS | 1, 2 | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -87.7 |
|  |  | dB | 1, 2 | -Infinity | 8 |
|  |  | dB | 1, 2 | -Infinity | 8 |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -66.7 | -58.0 |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.8.4.2.7.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

Table A.8.4.2.7.2-1: Test requirements for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in non-DRX

| Test case | Measurement reporting delay (ms) |  |
| --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms |
| UE power class 3 | 4160 | 2080 |

#### A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used

##### A.8.4.2.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21 of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.8.1-1, A.8.4.2.8.1-2 and A.8.4.2.8.1-3.

The cell specific test parameters for E-UTRA cell1 as PCell are defined in clause A.3.7.2.2.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.8.4.2.8.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.8.4.2.8.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.8.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR2 in DRX

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.8.1-2: General test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 |  |  |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.3.7.2.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 |  | 19 |  | As specified in TS 36.331 [16]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |  |  |
| CP length |  | 1, 2 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | DRX.9 | DRX.12 | DRX.9 | DRX.12 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |  |  |
| T2 | s | 1, 2 | 7 | 70 | 7 | 70 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table A.8.4.2.8.1-3 |  |  |  |  |  |  |  |

Table A.8.4.2.8.1-3: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| AoA setup defined in A.3.15.1 |  |  | 1, 2 | Setup 1 |  |
| Assumption for UE beamsNote 5 |  |  | 1, 2 | Rough |  |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.3.2.1.1 |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -106 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | dBm/15kHz | 1, 2 | -104.7 |  |
| Note2 |  | dBm/SCS | 1, 2 | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -87.7 |
|  |  | dB | 1, 2 | -Infinity | 8 |
|  |  | dB | 1, 2 | -Infinity | 8 |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -66.7 | -58.08 |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

##### A.8.4.2.8.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D3 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D4 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is required to report SSB time index.

Table A.8.4.2.8.2-1: Test requirements for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in DRX

| Test case | Measurement reporting delay (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms | Test 3: D3 ms | Test 4: D4 ms |
| UE power class 3 | 6240 | 66560 | 6240 | 66560 |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16

##### A.8.4.2.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements when UE is configured with highSpeedInterRAT-NR-r16.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Tables A.8.4.2.9.1-1, A.8.4.2.9.1-2, A.8.4.2.9.1-3 and A.8.4.2.9.1-4.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.8.4.2.9.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR1 for UE configured with highSpeedInterRAT-NR-r16

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.8.4.2.9.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.6 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 5 |  |
| Note 1: The value of b2-Threshold1 is defined in Table A.8.4.2.9.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.8.4.2.9.1-4 |  |  |  |  |

Table A.8.4.2.9.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting with NR neigbour cell in FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.8.4.2.9.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection for UE configured with highSpeedInterRAT-NR-r16

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -91 |
|  |  | 3, 6 | -Infinity | -88 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -62.26 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2, 4, 5 | AWGN1944 |  |
|  |  | 3,6 | AWGN3334 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

##### A.8.4.2.9.2	Test Requirements

The UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 4.8s from the beginning of time period T2. The UE is required to read the neighbour cell SSB index and report the acquired SSB index in this test.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.8.5	Measurement performance

### A.8.5.1	SFTD accuracy

#### A.8.5.1.1	SFTD accuracy

##### A.8.5.1.1.1	Test Purpose

The purpose of this set of tests is to verify that the SFTD measurement accuracy is within the specified limits. This test will verify the requirements as specified in clause 9.1.27 in TS 36.133 [15] for inter-RAT FR1 SFTD measurements.

##### A.8.5.1.1.2	Test Environment

Supported test configurations are shown in Table A.8.5.1.1.2-1. In this set of test cases there are two cells on different carriers. Cell 1 is E-UTRAN PCell and Cell 2 is inter-RAT NR FR1 target cell. The test parameters of cell 1 are given in clause A.8.5.1.1.2-2. The test parameters of cell 2 are given in Table A.8.5.1.1.2-3. The SFTD between PCell and target cell shall be set by the test equipment to one of the time differences in Table A.8.5.1.1.2-4.

Table A.8.5.1.1.2-1: Supported test configurations for SFTD accuracy

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.5.1.1.2-2: Test parameters for SFTD accuracy (Cell 1)

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | -3 |
| Ês/Iot | dB | -3 |
| RSRP Note5 | dBm/15 kHz | -107 |
| SCH_RP Note5 | dBm/15 kHz | -107 |
| Io Note5 | dBm/Ch BW | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table A.8.5.1.1.2-3: Test parameters for SFTD accuracy (Cell 2)

| Parameter |  | Config | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~6 |  | freq1 |
| Duplex mode |  | 1,4 |  | FDD |
|  |  | 2,5 |  | TDD |
|  |  | 3,6 |  | TDD |
| TDD Configuration |  | 1,4 |  | N/A |
|  |  | 2,5 |  | TDDConf.1.1 |
|  |  | 3,6 |  | TDDConf.2.1 |
| BWchannel |  | 1,4 | MHz | 10: NRB,c = 52 |
|  |  | 2,5 |  | 10: NRB,c = 52 |
|  |  | 3,6 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1,4 |  | SR.1.1 FDD |
|  |  | 2,5 |  | SR.1.1 TDD |
|  |  | 3,6 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1,4 |  | CR.1.1 FDD |
|  |  | 2,5 |  | CR.1.1 TDD |
|  |  | 3,6 |  | CR.2.1 TDD |
| RMC CORESET Reference Channel |  | 1,4 |  | CCR.1.1 FDD |
|  |  | 2,5 |  | CCR.1.1 TDD |
|  |  | 3,6 |  | CCR.2.1 TDD |
| SSB configuration |  | 1,4 |  | SSB.1 FR1 |
|  |  | 2,5 |  | SSB.1 FR1 |
|  |  | 3,6 |  | SSB.2 FR1 |
| SMTC configuration |  | 1~6 |  | SMTC.1 |
| DL BWP configuration |  | 1~6 |  | DLBWP.1.1 |
| UL BWP configuration |  | 1~6 |  | ULBWP.1.1 |
| OCNG Patterns |  | 1~6 |  | OP.1 |
| EPRE ratio of PSS to SSS |  | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~6 | dBm/15kHz | -104 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2,4,5 | dBm/SSB SCS | -104 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3,6 |  | -101 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  |  | 1~6 | dB | -3 |
|  |  | 1~6 | dB | -3 |
| SS-RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2,4,5 | dBm/SCS | -107 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3,6 |  | -104 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2,4,5 | dBm/9.36 MHz | -74.28 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3,6 | dBm/38.16 MHz | -68.18 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_F |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
| Propagation condition |  | 1~6 |  | AWGN |
| Antenna configuration |  | 1~6 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |

Table A.8.5.1.1.2-4: Timing offsets for SFTD accuracy test

| Configuration | SFN offset between PCell and PSCell | Frame boundary offset between PCell and PSCell (Ts) |
| --- | --- | --- |
| 1 | 100 | -122000 |
| 2 | 300 | -60540 |
| 3 | 500 | 1000 |
| 4 | 700 | 62540 |
| 5 | 900 | 124000 |

##### A.8.5.1.1.3	Test Requirements

The SFTD reported by the UE consists of 2 elements, SFN offset and frame boundary offset between PCell and inter-RAT NR target cell. The reported SFTD accuracy shall fulfil the requirement in clause 9.1.27 in TS 36.133 [15].

### A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements

#### A.8.5.2.1	SS-RSRP

##### A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell

A.8.5.2.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.1 in TS 36.133 [15] for inter-RAT FR1 SS-RSRP measurements.

A.8.5.2.1.1.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.1.1.2-1. In this test case there are two cells on different carriers. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1. Cell 2 is the inter-RAT NR FR1 target cell. The absolute accuracy requirements of SS-RSRP inter-RAT measurement is tested by using test parameters in Table A.8.5.2.1.1.2-2.

Table A.8.5.2.1.1.2-1: SS-RSRP Inter-RAT SS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.5.2.1.1.2-2: SS-RSRP inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| SMTC configruation |  |  |  | SMTC.1 |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | Config 1,2,3,4,5,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -94.65 |  | -117 |  |
|  |  | NR_FDD_FR1_B |  |  |  | -116.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  | -116 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -115.5 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  | -115 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -114.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -113.5 |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -94.65 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -91.65 |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -110.5 |  |
|  |  |  | dB | 10 |  | -4 |  |
|  |  |  | dB | 10 |  | -4 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -84.65 |  | -121 |  |
|  |  | NR_FDD_FR1_B |  |  |  | -120.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  | -120 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -119.5 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  | -119 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -118.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  | -118 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -117.5 |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -81.65 |  | -118 |  |
|  |  | NR_FDD_FR1_B |  |  |  | -117.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  | -117 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -116.5 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  | -116 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -115.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  | -115 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -114.5 |  |
| IoNote3 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -56.28 |  | -87.76 |  |
|  |  | NR_FDD_FR1_B |  |  |  | -87.26 |  |
|  |  | NR_TDD_FR1_C |  |  |  | -86.76 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -86.26 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  | -85.76 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -85.26 |  |
|  |  | NR_FDD_FR1_G |  |  |  | -84.76 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -84.26 |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -50.19 |  | -84.76 |  |
|  |  | NR_FDD_FR1_B |  |  |  | -84.26 |  |
|  |  | NR_TDD_FR1_C |  |  |  | -83.76 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  | -83.26 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  | -82.76 |  |
|  |  | NR_FDD_FR1_F |  |  |  | -82.26 |  |
|  |  | NR_FDD_FR1_G |  |  |  | -81.76 |  |
|  |  | NR_FDD_FR1_H |  |  |  | -81.26 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

A.8.5.2.1.1.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.1 in TS 36.133 [15].

##### A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell

###### A.8.5.2.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.1 in TS 36.133 [15] for inter-RAT FR2 SS-RSRP measurements.

###### A.8.5.2.1.2.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.1.2.2-1. In this test case there are two cells on different carriers. Absolute accuracy requirements of SS-RSRP inter-RAT measurement are tested by using test setup in Table A.8.5.2.1.2.2-2 and Table A.8.5.2.1.2.2-3. In all test cases, Cell 2 is target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.8.5.2.1.2.2-1: SS-RSRP Inter-RAT SS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.8.5.2.1.2.2-2: SS-RSRP Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| PDSCH Reference measurement channel |  | - | - |
| RMSI CORESET Reference Channel |  | - | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |

Table A.8.5.2.1.2.2-3: SS-RSRP Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 | Setup 1 according to A.3.15.1 |
| Assumption for UE beamsNote 10 |  | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105 | N/A |
| Note1 | dBm/SCS Note4 | -96 | N/A |
| Es | dBm/SCS Note4 |  | (Table B.2.3-2 Rx Beam Peak +1dB) (Note 7) |
|  | dB | 11 | N/A |
| SSB_RPNote2 | dBm/SCS Note4 | -85 | (Table B.2.3-2 Rx Beam Peak +1dB) (Note 7) |
| BB Note 2, Note 9 | dB | 9.97 | -3.81 |
| IoNote2 | dBm/95.04 MHz Note4 | -55.65 | (Table B.2.3-2 Rx Beam Peak +30dB) (Note 8) |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone. Note 5: Void Note 6: Void Note 7: SSB_RP is applied at 1dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 8: Io is applied at 10log10(792)dB+1dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 9: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |

###### A.8.5.2.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.1 in TS 36.133 [15].

#### A.8.5.2.2	SS-RSRQ

##### A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell

A.8.5.2.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.2 in TS 36.133 [15] for inter-RAT FR1 SS-RSRQ measurements.

A.8.5.2.2.1.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.2.1.2-1. In this test case there are two cells on different carriers. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1. Cell 2 is the inter-RAT NR FR1 target cell. The absolute accuracy requirements of SS-RSRP inter-RAT measurement is tested by using test parameters in Table A.8.5.2.2.1.2-2.

Table A.8.5.2.2.1.2-1: SS-RSRQ Inter-RAT SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.5.2.2.1.2-2: SS-RSRQ inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configruation |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  | -106 |  | -116 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -115.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -115 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -114.5 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -113.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -112.5 |  |
|  | Config 3,6 |  |  | -86.27 |  | -113 |  | Same as Noc for Config 1,2,4,5 |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -80.18 |  | -106 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -83.27 |  | -110 |  | -113 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -112.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -110.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -109.5 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -81.93 |  | -107.75 |  | -117.75 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -117.25 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -116.75 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -116.25 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -115.75 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -115.25 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -114.75 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -114.25 |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85.02 |  | -111.75 |  | -114.75 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -114.25 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -113.75 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -113.25 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -112.75 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -112.25 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -111.75 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -111.25 |  |
| SS-RSRQ Note3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -14.77 |  | -40.59 |  | -14.76 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -50 |  | -75.83 |  | -85.83 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -85.33 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -84.83 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -84.33 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -83.83 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -83.33 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -82.83 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -82.33 |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -50 |  | -76.73 |  | -79.73 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -79.23 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -78.73 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -78.23 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -77.73 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -77.23 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -76.73 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -76.53 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

A.8.5.2.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.2 in TS 36.133 [15].

##### A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell

###### A.8.5.2.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.2 in TS 36.133 [15] for inter-RAT FR2 SS-RSRQ measurements.

###### A.8.5.2.2.2.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.2.2.2-1. In this test case there are two cells on different carriers. Absolute accuracy requirements of SS-RSRQ inter-RAT measurement are tested by using test setup in Table A.8.5.2.2.2.2-2 and Table A.8.5.2.2.2.2-3. In all test cases, Cell 2 is target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.8.5.2.2.2.2-1: SS-RSRQ Inter-RAT SS-RSRQ supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.8.5.2.2.2.2-2: SS-RSRQ Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| PDSCH Reference measurement channel |  | - | - |
| RMSI CORESET Reference Channel |  | - | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |

Table A.8.5.2.2.2.2-3: SS-RSRQ Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 | Setup 1 according to A.3.15.1 |
| Assumption for UE beamsNote 10 |  | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -104.7 | (Table B.2.3-2 Rx Beam Peak -5dB) (Note 7) |
| Note1 | dBm/SCS Note4 | -95.7 | (Table B.2.3-2 Rx Beam Peak +4dB) (Note 7) |
|  | dB | -0.5 | -1.75 |
| SSB_RPNote2 | dBm/SCS Note4 | -96.2 | (Table B.2.3-2 Rx Beam Peak +2.25dB) (Note 8) |
| SS-RSRQNote2 | dB | -3.27 | -14.82 |
| Note2 | dB | -0.5 | -1.75 |
| IoNote2 | dBm/95.04 MHz Note4 | -63.95 | (Table B.2.3-2 Rx Beam Peak +35.22dB) (Note 9) |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, SS-RSRQ, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone. Note 5: Void Note 6: Void Note 7: Noc for SCS 15kHz is applied at -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 for beam peak. Noc for SCS 120kHz is applied at 4dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 8: SSB_RP is applied at 2.25dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 9: Io is applied at 10log10(792)+6.22dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |

###### A.8.5.2.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.2 in TS 36.133 [15].

In this test case there are two cells on different carriers and measurement gaps are provided

#### A.8.5.2.3	SS-SINR

##### A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell

A.8.5.2.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS- SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.3 in TS 36.133 [15] for inter-RAT FR1 SS-SINR measurements.

A.8.5.2.3.1.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.3.1.2-1. In this test case there are two cells on different carriers. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1. Cell 2 is the inter-RAT NR FR1 target cell. The absolute accuracy requirements of SS-RSRP inter-RAT measurement is tested by using test parameters in Table A.8.5.2.3.1.2-2.

Table A.8.5.2.3.1.2-1: SS- SINR Inter-RAT SS- SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.5.2.3.1.2-2: SS-SINR inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configruation |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -880 |  | -108.5 |  | -119.5 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -119 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -118.5 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -118 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -117.5 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -117 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -116.5 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -116 |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | [-80] |  | -88 |  | -108.5 |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85 |  | -105.5 |  | -116.5 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -116 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -115.5 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -115 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -114.5 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -114 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -114.5 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -113] |  |
|  |  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
|  |  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -89.75 |  | -88.5 |  | -123.5 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -123 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -122.5 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -122 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -121.5 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -121 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -120.5 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -120 |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -86.75 |  | -85.5 |  | -120.5 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -120 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -119.5 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -119 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -118.5 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -118 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -117.5 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -117 |  |
| SS-SINR Note3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -1.75 |  | 20 |  | -4.0 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2,4,5 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.83 |  | -60.5 |  | -90.09 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -89.59 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -89.09 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -88.59 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -88.09 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -87.59 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -87.09 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -86.59 |  |
|  | Config 3,6 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.73 |  | -54.41 |  | -84 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -83.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -83 |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -82.5 |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -82 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -81.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -81 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -80.5 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

A.8.5.2.3.1.3	Test Requirements

The SS-SINR measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.3 in TS 36.133 [15].

##### A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell

###### A.8.5.2.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS- SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.3 in TS 36.133 [15] for inter-RAT FR2 SS-SINR measurements.

###### A.8.5.2.3.2.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.3.2.2-1. In this test case there are two cells on different carriers. Absolute accuracy requirements of SS-SINR inter-RAT measurement are tested by using test setup in Table A.8.5.2.3.2.2-2 and A.8.5.2.3.2.2-3. In all test cases, Cell 2 is target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.8.5.2.3.2.2-1: SS-SINR Inter-RAT SS-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.8.5.2.3.2.2-2: SS-SINR Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 | freq1 |
| Duplex mode |  | TDD | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |
| PDSCH Reference measurement channel |  | - | - | - |
| RMSI CORESET Reference Channel |  | - | - | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |  |

Table A.8.5.2.3.2.2-3: SS-SINR Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 | Setup 1 according to A.3.15.1 | Setup 1 according to A.3.15.1 |
| Assumption for UE beamsNote 10 |  | Rough | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -104.7 | -104.7 | (Table B.2.3-2 Rx Beam Peak -5dB) (Note 7) |
| Note1 | dBm/SCS Note4 | -95.7 | -95.7 | (Table B.2.3-2 Rx Beam Peak +4dB) (Note 7) |
|  | dB | -0.5 | 11 | -1.0 |
| SSB_RPNote2 | dBm/SCS Note4 | -96.2 | -84.7 | (Table B.2.3-2 Rx Beam Peak +3dB) (Note 8) |
| SS-SINRNote2 | dB | -0.5 | 11 | -1.0 |
| Note2 | dB | -0.5 | 11 | -1.0 |
| IoNote2 | dBm/95.04 MHz Note4 | -63.95 | -55.38 | (Table B.2.3-2 Rx Beam Peak +35.54dB) (Note 9) |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, SS-SINR, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. Note 5: Void Note 6: Void Note 7: Noc for SCS 15kHz is applied at -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 for beam peak. Noc for SCS 120kHz is applied at 4dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 8: SSB_RP is applied at 3dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 9: Io is applied at level 10log10(792)+6.54dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |

###### A.8.5.2.3.2.3	Test Requirements

The SS-SINR measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.3 in TS 36.133 [15].

# A.9	V2X Tests

## A.9.1	V2X Tests in FR1

### A.9.1.1	Test for V2X UE Transmit Timing

#### A.9.1.1.1	Test for GNSS as Synchronization Reference Source

##### A.9.1.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the UE timing requirements as specified in clause 12.2.2, when the GNSS is used as timing reference. For this test, the UE is triggered by the test loop function to transmit for V2X sidelink communication.

Table A.9.1.1.1.1-1 defines test parameters for UE transmit timing accuracy tests for V2X. There is one GNSS based synchronization source during the test. The test system can emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B.4.1.

Table A.9.1.1.1.1-1: V2X Sidelink Test Parameters for UE Transmit Timing Tests for GNSS as Timing Reference

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1 | TDD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Active cell |  | None |  |
| Active SyncRef UE |  | None |  |
| V2X sidelink communication preconfiguration |  | As specified in section A.3.21.2 | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  | CC.1A HD | As specified in Table A.3.21.3-1 |
| PSSCH Reference Measurement Channel |  | CD.1A HD | As specified in Table A.3.21.3-2 |
| Propagation condition |  | AWGN |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

##### A.9.1.1.1.2	Test requirements

For parameters specified in Tables A.9.1.1.1-1, the timing accuracy for V2X sidelink transmission shall be within the limits defined in clause 12.2.2. The timing accuracy is verified by using PSSCH transmissions.

#### A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source

##### A.9.1.1.2.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for V2X sidelink transmissions specified in clause 12.2.5, when SyncRef UE is used as timing reference. For this test, the UE is triggered by the test loop function to transmit for V2X sidelink communication.

Table A.9.1.1.2.1-1 defines test parameters for UE transmit timing accuracy tests for V2X sidelink Communication. There is one active SyncRef UE in this test without either serving cell and or GNSS signals. Before the test starts, the UE has been synchronized to the SyncRef UE. The transmit timing accuracy is verified by using the transmission timing of PSSCH transmissions.

Table A.9.1.1.2.1-1: General Test Parameters for V2X UE Transmit Timing Test for SyncRef UE as Timing Reference

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | TDD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 3 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Active cell |  |  | None |  |
| Active SyncRef UE |  |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on uplink of RF channel number 1 |
| V2X sidelink communication preconfiguration |  |  | As specified in section A.3.21.2 | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  |  | CC.1A HD | As specified in Table A.3.21.3-1 |
| PSSCH Reference Measurement Channel |  |  | CD.1A HD | As specified in Table A.3.21.3-2 |
| Note1,2 |  | dBm/30kHz | -95 |  |
| SyncRef UE 1 | sl-SSB-TimeAllocation |  | sl-SSB-TimeAllocation1 |  |
|  | slssid |  | 30 |  |
|  | inCoverage |  | TRUE | In MIB-SL |
|  | networkControlledSyncTx |  | ON |  |
|  | V2X sidelink communication configuration |  | As specified in section A.3.21.2 | IE values unless specified otherwise in this test. |
|  |  |  | 3 |  |
|  | PSBCH-RSRP Note1, Note 2 | dBm/30kHz | -92 |  |
| Propagation condition |  |  | AWGN |  |
| Note 1: PSBCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: PSSSS Es/Noc and PSPSS Es/Noc are set the same as PSBCH Es/Noc. Note 3: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

##### A.9.1.1.2.2	Test requirements

For parameters specified in Tables A.9.1.1.2.1-1, the timing accuracy for V2X sidelink transmission shall be within the limits defined in clause 12.2.5. The timing accuracy is verified by using PSSCH transmissions.

#### A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source

##### A.9.1.1.3.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for V2X sidelink transmissions specified in clause 12.2.3, when the downlink timing of the serving cell (RRC_IDLE) or PCell (RRC_CONNECTED) on a non-V2X sidelink carrier is used as timing reference. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X sidelink communication.

This test is applicable for V2X sidelink communication capable UEs that support NR Uu and sidelink operation.

Table A.9.1.1.3.1-1, A.9.1.1.3.1-2 and A.9.1.1.3.1-3 define test parameters for UE transmit timing accuracy tests for V2X sidelink Communication. There is one active cell (PCell) in this test. The transmit timing accuracy is verified by using the transmission timing of PSSCH transmissions.

Table A.9.1.1.3.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.9.1.1.3.1-2: V2X Sidelink Test Parameters for V2X UE Transmit Timing Accuracy Test for gNB as Timing Reference

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1 | TDD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| Active cell |  | Cell 1 |  |
| Active SyncRef UE |  | None |  |
| V2X sidelink communication configuration |  | As specified in section A.3.21.2 | IE values unless specified otherwise in this test. |
| PSCCH Reference Measurement Channel |  | CC.1A HD | As specified in Table A.3.21.3-1 |
| PSSCH Reference Measurement Channel |  | CD.1A HD | As specified in Table A.3.21.3-2 |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

Table A.9.1.1.3.1-3: Cell Test Parameters for V2X UE Transmit Timing Accuracy Test for gNB as Timing Reference

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| RF Channel Number |  |  | 2 |
| Duplex Mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| Channel Bandwidth (BWchannel) | Config 1,2 | MHz | 10: NRB,c = 52 |
|  | Config 3 |  | 40: NRB,c = 106 |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |
| DRX Cycle |  |  | N/A |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.2.1 TDD |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | SMTC.2 |
| OCNG Patterns |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -98 |
| Note2 | Config 1,2 | dBm/SCS | -98 |
|  | Config 3 |  | -95 |
|  |  | dB | 3 |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -95 |
|  | Config 3 |  | -92 |
| Io Note 3 | Config 1,2 | dBm/9.36 MHz | -65.2 |
|  | Config 3 | dBm/38.1 MHz | -59.2 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

##### A.9.1.1.3.2	Test requirements

For parameters specified in Tables A.9.1.1.3.1-1 A.9.1.1.3.1-2 and A.9.1.1.3.1-3, the timing accuracy for V2X sidelink transmission shall be within the limits defined in clause 12.2.3. The timing accuracy is verified by using PSSCH transmissions.

### A.9.1.2	Test for Initiation/Cease of SLSS Transmission with V2X Sidelink Communication

#### A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX

##### A.9.1.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the V2X UE meets the requirements related to the maximum evaluation time allowed to initiate and cease SLSS transmissions defined in clause 12.3.1.1, when the reference timing used for sidelink transmissions is a NR serving cell in FR1 on a non-V2X sidelink carrier. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X sidelink communication.

This test is applicable for V2X sidelink communication capable UEs that support NR Uu and sidelink operation.

Supported test configurations for FR1 NR cell are shown in Table A.9.1.2.1.1-1.

Table A.9.1.2.1.1-1: Supported Test Configurations for FR1 NR cell as synchronization reference source

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to pass in one of the supported test configurations in FR1. Note 2: For NR SL, SL BW is one between 20MHz and 40MHz, and SL SCS is 30kHz. |  |

The test parameters are given in Table A.9.1.2.1.1-2 and Table A.9.1.2.1.1-3 below. There is one active cell in this test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the SS-RSRP of the PCell is above syncTxThreshIC and the UE is not expected to be transmitting SLSS.

During T2, the SS-RSRP of the PCell is lowered below syncTxThreshIC and the UE is expected to initiate SLSS transmissions.

During T3, the SS-RSRP of the PCell is increased back to be above syncTxThreshIC and the UE is expected to cease SLSS transmissions.

Table A.9.1.2.1.1-2: Test Parameters for Initiation/Cease of SLSS Transmission Test for FR1 NR cell as synchronization reference source

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 | Serving cell on RF channel number 1 |
| Active SyncRef UE |  | None |  |
| Active V2X UE |  | V2X UE | Transmitting SLSS+MIB-SL on RF channel number 2(TDD carrier in Band n47 or n38) |
| V2X sidelink communication configuration |  | As specified in Table A.3.21.2-2 | IE values unless specified otherwise in this test |
| networkControlledSyncTx |  | Not configured |  |
| syncTxThreshIC | dBm/SCS | -110 | In SIB12 |
| DRX |  | OFF |  |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

Table A.9.1.2.1.1-3: FR1 NR Cell Specific Test Parameters for Initiation/Cease of SLSS Transmission Test for FR1 NR cell as synchronization reference source

| Parameter |  | Unit | Cell1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| NR RF Channel Number |  |  | 1 |  |  |
| Duplex Mode | Config 1 |  | FDD |  |  |
|  | Config 2,3 |  | TDD |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |
| Channel Bandwidth (BWchannel) | Config 1,2 | MHz | 10:NRB,c = 52 |  |  |
|  | Config 3 |  | 40:NRB,c = 106 |  |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| DRx Cycle |  | ms | N/A |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |
| SMTC Configuration | Config 1 |  | SMTC.2 |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -110 |  |  |
|  | Config 1,2 | dBm /SCS | -110 |  |  |
|  | Config 3 |  | -107 |  |  |
|  |  | dB | 4.5 | -4.5 | 4.5 |
|  |  | dB | 4.5 | -4.5 | 4.5 |
| SS-RSRPNote3 | Config 1,2 | dBm /SCS | -105.5 | -114.5 | -105.5 |
|  | Config 3 |  | -102.5 | -111.5 | -102.5 |
| IoNote3 | Config 1,2 | dBm /9.36MHz | -76.2 | -80.7 | -76.2 |
|  | Config 3 | dBm/ 38.16MHz | -70.1 | -74.6 | -70.1 |
| Propagation condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |

##### A.9.1.2.1.2	Test Requirements

The SLSS transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the SLSS transmission.

The SLSS transmission initiation delay shall be less than 0.56 s.

The SLSS transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the SLSS transmission.

The SLSS transmission cease delay shall be less than 0.56 s.

The rate of correct initiation/cease delay of SLSS transmissions observed during repeated tests shall be at least 90%.

NOTE:	The initiation/cease delay of SLSS transmissions can be expressed as: Tevaluate,SLSS + SLSS period,

Where:

Tevaluate,SLSS = 0.4 sec (as specified in clause 12.3.1.1);

SLSS period = 160ms.

#### A.9.1.2.2	Test for SyncRef UE as synchronization reference source

##### A.9.1.2.2.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the evaluation time allowed to initiate and cease SLSS transmissions defined in clause 12.3.1.4, when the reference timing used for sidelink transmissions is a SyncRef UE. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X sidelink communication.

The test parameters are given in Table A.9.1.2.2.1-1 and Table A.9.1.2.2.1-2 below. There are neither active cells nor GNSS signals in this test. There is one active SyncRef UE (SyncRef UE 1) in this test. The test system shall emulate SyncRef UE 1 to transmit SLSS and MIB-SL every synchronization period.

Prior to start of test, test system is required to ensure that the V2X UE is synchronized to the SyncRef UE 1 and is transmitting SLSS + MIB-SL as derived from the SLSS + MIB-SL of SyncRef UE 1 as per clause 5.8.5.3 of TS 38.331[2]. For the test configuration, the SLSSID used by the V2X UE shall be 30 with inCoverage IE in MIB-SL set as FALSE. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the PSBCH-RSRP of SyncRef UE 1 is above syncTxThreshOOC and the UE is not expected to be transmitting SLSS.

During T2, the PSBCH-RSRP of SyncRef UE 1 is lowered below syncTxThreshOOC and the UE is expected to initiate SLSS transmissions.

During T3, the PSBCH-RSRP of SyncRef UE 1 is increased back to be above syncTxThreshOOC and the UE is expected to cease SLSS transmissions.

Table A.9.1.2.2.1-1: Test Parameters for Initiation/Cease of SLSS Transmission Test for SyncRef UE as synchronization reference source

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | None |  |
| Active SyncRef UE |  | SyncRef UE 1 | Transmitting SLSS + MIB-SL on RF channel number 1(TDD carrier in Band n47 or n38) |
| Active V2X UE |  | V2X UE | Transmitting SLSS + MIB-SL on RF channel number 1(TDD carrier in Band n47 or n38) |
| V2X sidelink communication preconfiguration |  | As specified in Table A.3.21.2-2 | IE values unless specified otherwise in this test |
| networkControlledSyncTx |  | Not configured |  |
| syncTxThreshOoC | dBm/30kHz | -97 |  |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

Table A.9.1.2.2.1-2: SyncRef UE Specific Test Parameters for Initiation/Cease of SLSS Transmission Test for SyncRef UE as synchronization reference source

| Parameter | Unit | Cell1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| NR RF Channel Number |  | 1 |  |  |
| V2X SL communication resource pool configuration |  | As specified in Table A.3.21.2-2 |  |  |
| Channel Bandwidth (BWchannel) Note3 | MHz | 20(NRB,c = 50) or 40(NRB,c = 100) |  |  |
| SLSSID |  | 30 |  |  |
| inCoverage |  | FALSE |  |  |
| networkControlledSyncTx |  | ON |  |  |
| Note1 | dBm/30 kHz | -98 |  |  |
|  | dB | 5.5 | -3.5 | 5.5 |
| PSBCH | dB | 5.5 | -3.5 | 5.5 |
| PSBCH-RSRPNote2 | dBm/30 kHz | -92.5 | -101.5 | -92.5 |
| IoNote2 | dBm /3.96MHz | -70.2 | -75.2 | -70.2 |
| Propagation condition |  | AWGN |  |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: PSBCH-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Io level is based on the allocated RBs for PSPSS/PSSSS/PSBCH symbols. Note 3: The UE is only required to be tested in one of the supported test configurations. Note 4: PSSSS Es/Noc and PSPSS Es/Noc are set the same as PSBCH Es/Noc. |  |  |  |  |

##### A.9.1.2.2.2	Test Requirements

The SLSS transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the SLSS transmission.

The SLSS transmission initiation delay shall be less than 0.8 s.

The SLSS transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the SLSS transmission.

The SLSS transmission cease delay shall be less than 0.8 s.

The rate of correct initiation/cease delay of SLSS transmissions observed during repeated tests shall be at least 90%.

NOTE:	The initiation/cease delay of SLSS transmissions can be expressed as: Tevaluate,SLSS + SLSS period,

Where:

-	Tevaluate,SLSS = 0.64 sec (as specified in clause 12.3.1.4);

-	SLSS period = 160ms.

### A.9.1.3	Test for V2X Synchronization Reference Selection/Reselection

#### A.9.1.3.1	Test for GNSS configured as the highest priority

##### A.9.1.3.1.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to SyncRef UE selection / reselection defined in clause 12.4, when GNSS is configured as the highest priority. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.9.1.3.1.1-1and A.9.1.3.1.1-2 below. There are no GNSS signals in this test. There are three active SyncRef UEs (SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3) in this test. The test system shall emulate SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3 to transmit SLSS and MIB-SL every SLSS period.

The test system can verify the selection / reselection of SyncRef UE by monitoring the SLSS ID used by the V2X UE for its SLSS+MIB-SL transmissions. When the V2X UE is not synchronized to any SyncRef UE, then the V2X UE shall use the SLSS ID pre-configured in the V2X UE. When the V2X UE is synchronized to a SyncRef UE, the V2X UE shall derive its SLSS ID from the SLSS ID of the SyncRef UE as per clause 5.8.5.3 of TS 38.331[2].

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. SyncRef UE 1, SyncRef UE 2 and SyncRef UE 3 are all powered off before starting the test. During T1, SyncRef UE 1 is powered ON and the V2X UE will select SyncRef UE 1 as synchronization source. During T2, SyncRef UE 2 is powered ON and the V2X UE will select SyncRef UE 2 as the synchronization source. During T3, a higher priority SyncRef UE 3 is additionally powered ON and the V2X UE will reselect to the higher priority SyncRef UE 2 as the synchronization source.

Table A.9.1.3.1.1-1: Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active synchronization source |  | Sync Ref UE 1 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 30 and in-coverage set as TRUE in MIB-SL. |
| T2 end condition | Active synchronization source |  | Sync Ref UE 2 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 336 and in-coverage set as FALSE in MIB-SL. |
| Final condition | Active synchronization source |  | Sync Ref UE 3 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 0 and in-coverage set as FALSE in MIB-SL. |
| Active SyncRef UEs |  |  | SyncRef UE 1 SyncRef UE 2 SyncRef UE 3 | Transmitting SLSS+MIB-SL on RF channel number 1 (TDD carrier in Band n47 or n38) |
| Timing offset among SyncRef UEs |  | s | CP/2 | Synchronous |
| Frequency offset of SyncRef UE 1 |  | ppm | 0 |  |
| Frequency offset of SyncRef UE 2 |  | ppm | 5 |  |
| Frequency offset of SyncRef UE 3 |  | ppm | 10 |  |
| V2X sidelink Communication configuration |  |  | As specified in Table A. 3.21.2-2 | IE values unless specified otherwise in this test. |
| typeTxSync |  |  | gnss |  |
| slssid |  |  | 30 |  |
| syncTxThreshIC |  |  | +infinity |  |
| T1 |  | s | 24 |  |
| T2 |  | s | 16 |  |
| T3 |  | s | 3.2 |  |

Table A.9.1.3.1.1-2: SyncRef UE Specific Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority

| Parameter | Unit | SyncRef UE 1 |  |  | SyncRef UE 2 |  |  | SyncRef UE 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1(TDD carrier in Band n47 or n38) |  |  |  |  |  |  |  |  |
| SCS | kHz | 30 |  |  |  |  |  |  |  |  |
| Channel Bandwidth (BWchannel) Note 4 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |  |  |  |  |  |  |  |
| V2X Sidelink Communication resource pool configuration |  | As specified in Table A.3.21.2-2 |  |  |  |  |  |  |  |  |
| networkControlledSyncTx |  | N/A |  |  | N/A |  |  | ON |  |  |
| syncTxThreshOoC | dBm/15 kHz | +infinity |  |  | +infinity |  |  | N/A |  |  |
| slssid |  | 30 |  |  | 336 |  |  | 0 |  |  |
| inCoverage (in MIB-SL) |  | TRUE |  |  | FALSE |  |  | FALSE |  |  |
| Note1 | dBm/30 kHz | -95 |  |  |  |  |  |  |  |  |
|  | dB | 3 | 0 | 0 | -infinity | 0 | 0 | -infinity | -infinity | 3 |
|  | dB | 0 | -4.76 | -4.76 | -infinity | 0 | -4.76 | -infinity | -infinity | 0 |
| PSBCH-RSRPNote2, Note 3 | dBm/30 kHz | -92 | -95 | -95 | -infinity | -95 | -95 | -infinity | -infinity | -92 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: PSBCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: PSSSS Es/Iot is set the same as PSPSS/PSBCH Es/Iot. Note 4: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |  |  |  |  |  |  |

##### A.9.1.3.1.2	Test Requirements

1) During T1, SyncRef UE reselection delay is defined as the time from the beginning of T1 to the time UE is synchronized to SyncRef UE 1, and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 1 as the synchronization source. For the test configuration, the SLSS ID will be changed to 30 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T1.

The SyncRef UE reselection delay shall be less than 8.8sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 8sec (as specified in sub-clause 12.4)

-	Tevaluate,SLSS = 0.64 sec (as specified in sub-clause 12.3)

-	SLSS period = 160ms

This gives a total of 8.8 seconds.

2) During T2, SyncRef UE selection delay is defined as the time from the beginning of T2 to the time UE changes its synchronization source from SyncRef UE 1 to SyncRef UE 2 and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 2 as the synchronization source. For the test configuration, the SLSS ID will be changed to 336 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T2.

The SyncRef UE selection delay shall be less than 8.8sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 8sec (as specified in sub-clause 12.4)

-	Tevaluate,SLSS = 0.64 (as specified in sub-clause 12.3)

-	SLSS period = 160ms

This gives a total of 8.8seconds.

3) During T3, SyncRef UE reselection delay is defined as the time from the beginning of T3 to the time UE changes its synchronization source from SyncRef UE 2 to SyncRef UE 3, and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 3 as the synchronization source. For the test configuration, the SLSS ID will still be 0 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T3.

The SyncRef UE reselection delay shall be less than 2.4sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 1.6sec (as specified in sub-clause 12.4)

-	Tevaluate,SLSS = 0.64 (as specified in sub-clause 12.3)

-	SLSS period = 160ms

This gives a total of 2.4seconds.

The test system will verify that the V2X UE does not drop or delay more than 6% of its V2X data and SLSS transmissions during the duration of T2, and does not drop or delay more than 30% of its SLSS transmissions during the duration of T3.

The rate of correct SyncRef UE selection / reselection observed during repeated tests shall be at least 90%.

#### A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority

##### A.9.1.3.2.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to SyncRef UE selection / reselection defined in clause 12.4, when gNB is configured as the highest priority. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

This test is applicable for V2X sidelink communication capable UEs that support gNB as synchronization source and sidelink operation.

Supported test configurations for FR1 NR cell are shown in Table A.9.1.3.2.1-1.

Table A. 9.1.3.2.1-1: Supported Test Configurations for FR1 NR cell

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

The test parameters are given in Table A.9.1.3.2.1-2and A.9.1.3.2.1-3 below. There are no active cells and GNSS is reliable during the whole test. The test system can emulate and send the GNSS signal to the test UE. The test parameters for GNSS signals are defined in B. 4.1. There are two active SyncRef UEs (SyncRef UE 1 and SyncRef UE 2) in this test. The test system shall emulate SyncRef UE 1 and SyncRef UE 2 to transmit SLSS and MIB-SL every SLSS period.

The test system can verify the selection / reselection of SyncRef UE by monitoring the SLSS ID used by the V2X UE for its SLSS+MIB-SL transmissions. When the V2X UE is not synchronized to any SyncRef UE, then the V2X UE shall use the SLSS ID pre-configured in the V2X UE. When the V2X UE is synchronized to a SyncRef UE, the V2X UE shall derive its SLSS ID from the SLSS ID of the SyncRef UE as per clause 5.8.5.3 of TS 38.331[2].

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, both SyncRef UE 1 and SyncRef UE 2 are powered off and the V2X UE will select GNSS as synchronization source. During T2, SyncRef UE 1 is powered ON and the V2X UE will select SyncRef UE 1 as the synchronization source. During T3, a higher priority SyncRef UE 2 is additionally powered ON and the V2X UE will reselect to the higher priority SyncRef UE 2 as the synchronization source.

Table A.9.1.3.2.1-2: Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for FR1 NR Cell configured as the highest priority

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active synchronization source |  | GNSS | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 0 and in-coverage set as TRUE in MIB-SL. |
| T2 end condition | Active synchronization source |  | Sync Ref UE 1 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 336+59 and in-coverage set as FALSE in MIB-SL. |
| Final condition | Active synchronization source |  | Sync Ref UE 2 | UE transmits for V2X Sidelink Communication and SLSS+MIB-SL with SLSS ID = 30 and in-coverage set as FALSE in MIB-SL. |
| Active cell |  |  | None |  |
| Active SyncRef UEs |  |  | SyncRef UE 1 SyncRef UE 2 | Transmitting SLSS+MIB-SL on RF channel number 1 |
| Timing offset between SyncRef UE 1 and SyncRef UE 2 |  | ms | 3 | Asynchronous |
| Frequency offset of SyncRef UE 1 |  | ppm | 0 |  |
| Frequency offset of SyncRef UE 2 |  | ppm | 5 |  |
| V2X sidelink Communication preconfiguration |  |  | As specified in Table A.3.21.2-2 | IE values unless specified otherwise in this test. |
| syncPriority |  |  | gnb |  |
| syncTxThreshOoC |  |  | 13 (+infinity) |  |
| T1 |  | s | 24 |  |
| T2 |  | s | 16 |  |
| T3 |  | s | 16 |  |

Table A.9.1.3.2.1-3: SyncRef UE Specific Test Parameters for V2X Synchronization Reference Selection/Reselection Tests for FR1 NR Cell configured as the highest priority

| Parameter | Unit | SyncRef UE 1 |  |  | SyncRef UE 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1(TDD carrier in Band n47 or n38) |  |  |  |  |  |
| SCS | kHz | 30 |  |  |  |  |  |
| Channel Bandwidth (BWchannel) Note 4 | MHz | 20(NRB,c = 50) or 40(NRB,c = 100) |  |  |  |  |  |
| V2X Sidelink Communication resource pool configuration |  | As specified in Table A.3.21.2-2 |  |  | As specified in Table A.3.21.2-2 |  |  |
| networkControlledSyncTx |  | N/A |  |  | ON |  |  |
| syncTxThreshOoC | dBm/15 kHz | +infinity |  |  | N/A |  |  |
| slssid |  | 336+59 |  |  | 30 |  |  |
| inCoverage (in MIB-SL) |  | FALSE |  |  | TRUE |  |  |
| Note1 | dBm/30 kHz | -95 |  |  |  |  |  |
|  | dB | -infinity | 0 | 0 | -infinity | -infinity | 3 |
|  | dB | -infinity | 0 | -4.76 | -infinity | -infinity | 0 |
| PSBCH-RSRP Note2, Note 3 | dBm/30 kHz | -infinity | -95 | -95 | -infinity | -infinity | -92 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: PSBCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: PSSSS Es/Iot is set the same as PSPSS/PSBCH Es/Iot. Note 4: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |  |  |  |

##### A.9.1.3.2.2	Test Requirements

1) During T2, SyncRef UE selection delay is defined as the time from the beginning of T2 to the time UE is synchronized to SyncRef UE 1 and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 1 as the synchronization source. For the test configuration, the SLSS ID will be changed to 336+59 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T2.

The SyncRef UE selection delay shall be less than 8.8sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 8sec (as specified in sub-clause 12.4)

-	Tevaluate,SLSS = 0.64sec (as specified in sub-clause 12.3)

-	SLSS period = 160ms

This gives a total of 8.8 seconds.

2) During T3, SyncRef UE reselection delay is defined as the time from the beginning of T3 to the time UE changes its synchronization source from SyncRef UE 1 to SyncRef UE 2, and changes its SLSS transmissions timing and SLSS ID to follow SyncRef UE 2 as the synchronization source. For the test configuration, the SLSS ID will be changed to 30 (with in-coverage IE in MIB-SL set to FALSE) after SyncRef UE selection delay from start of T3.

The SyncRef UE reselection delay shall be less than 8.8sec. The SyncRef UE selection/reselection delay can be expressed as:

SyncRef UE selection/reselection delay = Tdetect,SyncRef UE + Tevaluate,SLSS + SLSS period

Where

-	Tdetect,SyncRef UE = 8sec (as specified in sub-clause 12.4)

-	Tevaluate,SLSS = 0.64 sec (as specified in sub-clause 12.3)

-	SLSS period = 160ms

This gives a total of 8.8 seconds.

The test system will verify that the V2X UE does not drop or delay more than 6% of its V2X data and SLSS transmissions during the duration of T2 and T3.

The rate of correct SyncRef UE selection / reselection observed during repeated tests shall be at least 90%.

### A.9.1.4	Test for L1 SL-RSRP Measurement

#### A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection

##### A.9.1.4.1.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to autonomous resource selection / reselection for V2X UE in mode 2 defined in clause 12.5. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.9.1.4.1.1-1and A. 9.1.4.1.1-2 below. There are 50 active V2X sidelink UEs (UE0~UE49) in this test. Both the UE under test and active V2X sidelink UEs select GNSS as synchronization reference source. The test system can emulate and send the GNSS signal to the test UE and active V2X sidelink UEs. The test parameters for GNSS signals are defined in B.4.1. The test system shall emulate the active V2X sidelink UEs to transmit PSCCH/PSSCH every 5ms. At the beginning of whole test, the test equipment shall send one message with a SL-SCH MAC PDU as specified in Clause 6.1.6 in TS 38.321[7], in order to make sure that the UE under test needs continuously transmit PSCCH/PSSCH.

The test consists of two duration T1 and T2. During T1, the signal from Test Equipement are configured such that

-	the measured PSSCH-RSRP for 10 active V2X sidelink UEs(UE20~UE29) is above the measurement threshold, and the resource occupied by the 10 active V2X sidelink UEs is expected to be excluded in the resource selection procedure and,

-	the measured PSSCH-RSRP for other 40 active V2X sidelink UEs(UE0~UE19, UE30~UE49) is low the measurement threshold, and the resource occupied by the 40 active V2X sidelink UEs is expected to be included in the resource selection procedure.

During T2, the signal from Test Equipement are configured such that

-	the measured PSSCH-RSRP for the 10 active V2X sidelink UEs(UE20~UE29) is below the measurement threshold, and the resource occupied by the 10 active V2X sidelink UEs is expected to be included in the resource selection procedure and,

-	the measured PSSCH-RSRP for other 40 active V2X sidelink UEs(UE0~UE19, UE30~UE49) is above the measurement threshold, and the resource occupied by the 40 active V2X sidelink UEs is expected to be excluded in the resource selection procedure.

Table A.9.1.4.1.1-1: Test Parameters for V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | TDD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 2 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink communication pre-configuration |  |  | As specified in Table A.3.21.2-1 and A.3.21.2-3 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  | 1111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 5 | Indicates the number of sub-channels for TX resource pool |
| sl-SubchannelSize-r16 included in SL-ResourcePool |  |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| Number of Active Sidelink UEs |  |  | 50 | Active Sidelink UE i = 0, .., 49 |
| SL-ThresPSSCH-RSRP |  |  | 12 | Corresponding -106 dBm as defined in Section 6.3.5 in TS38.331[2] |
| Active Sidelink UEs (UE i = 0, .., 49) | V2X sidelink Communication preconfiguration |  | As specified in Table A.3.21.2-1 And A.3.21.2-3 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i} Note1 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-StartRB-Subchannel-r16 included in SL-ResourcePool |  | floor(i/10)x10 | Indicates the lowest RB index of the subchannel with the lowest index. UE 0~9 start RB=0; UE 10~19 start RB=10; UE 20~29 start RB=20; UE 30~39 start RB=30; UE 40~49 start RB=40; |
|  | sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| Timing offset among Active Sidelink UEs |  | s | CP/2 | Synchronous |
| Note 1: {1i} is a sequence of nine 0’s with one 1 in (mod(i,10 )+1’th position. Note 2: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

Table A.9.1.4.1.1-2: Active Sidelink UE Specific Test Parameters for V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements

| Parameter | Unit | Active Sidelink UE i (i = 0, .., 49) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel)Note 5 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS | kHz | 30 |  |
| PSCCH RMC (defined in A.3.21.3) | - | CC.1A HD |  |
| PSSCH RMC (defined in A.3.21.3) | - | CD.1A HD |  |
| Note1 | dBm/30 kHz | -111 | -121 |
| Note3 | dB | 10 |  |
| Note2,3 | dB | 10 |  |
| Note4 | dB | 0 | 20 |
| Note2,4 | dB | 0 | 20 |
| PSSCH-RSRP1 Note 2,3 | dBm/30kHz | -101 | -111 |
| PSSCH -RSRP2 Note 2,4 | dBm/30kHz | -111 | -101 |
| SL-RSSI1 Note 2,3 | dBm/3.6 MHz | -79.79 | -89.79 |
| SL-RSSI2 Note 2,4 | dBm/3.6 MHz | -87.20 | -80.17 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, PSSCH-RSRP and SL-RSSI levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: For UE 20 to 29, occupying subchannel #2 Note 4: For UE 0 to 19 and 30 to 49, occupying subchannel #0/1/3/4 Note 5: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

##### A.9.1.4.1.2	Test Requirements

The test time  T1 and T2 should be long enough. The rate of PSSCH transmissions on the resources on subchannel #2 shall be less than 10% during T1. The rate of PSSCH transmissions on the resources on subchannel #2 shall be more than 90% during T2.

#### A.9.1.4.2	Test for V2X UE Resource Pre-emption

##### A.9.1.4.2.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to autonomous resource pre-emption for V2X UE in mode 2 defined in clause 12.5. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A. 9.1.4.2.1-1and A.12. 9.1.4.1-2 below. There is one active V2X sidelink UE in this test. Both the UE under test and the active V2X sidelink UE select GNSS as synchronization reference source. The test system can emulate and send the GNSS signal to the test UE and active V2X sidelink UEs. The test parameters for GNSS signals are defined in B.4.1. At the beginning of whole test, the test equipment shall send one message with a SL-SCH MAC PDU as specified in Clause 6.1.6 in TS 38.321[7], in order to make sure that the UE under test needs continuously transmit PSCCH/PSSCH.

The test consists of two duration T1 and T2. During T1, the signal from Test Equipement are configured such that the active V2X sidelink UE is not transmitting. The UE under test shall transmit SL data and reserve future resources. The resource reservation is decoded by the active V2X sidelink UE. The point in time at which resource reservation from the UE under test is decoded by the active V2X sidelink UE defines the start of time period T2. During T2, the active V2X sidelink UE reserves the same resource as the UE under test with high priority data no later than slot n- Tpre-empt.

Table A. 9.1.4.2.1-1: Test Parameters for V2X UE Resource Pre-emption Tests for PSSCH-RSRP measurements

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | TDD carrier in Band n47 and n38 |
| Channel Bandwidth (BWchannel) Note 1 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| V2X sidelink communication pre-configuration |  |  | As specified in Table A.3.21.2-1 and A.3.21.2-3 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool in SL-ResourcePool in sl-TxPoolSelectedNormal-r16 |  |  | 10000000000000000000 | Indicates the time resource of resource pool within sl-Period. (see TS 38.213 [3]) Note that this is for Tx pool. |
| sl-TimeResource-r16 included in SL-ResourcePool in sl-RxPool-r16 |  |  | 11111111111111111111 | Indicates the time resource of resource pool within sl-Period. (see TS 38.213 [3]) Note that this is for Rx pool. |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 | Indicates the number of subchannels in the corresponding resource pool, which consists of contiguous PRBs only |
| sl-SubchannelSize-r16 included in SL-ResourcePool |  |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| sl-StartRB-Subchannel-r16 included in SL-ResourcePool |  |  | 10 | Indicates the lowest RB index of the subchannel with the lowest index. |
| Number of Active Sidelink UEs |  |  | 1 |  |
| SL-ThresPSSCH-RSRP |  |  | 12 | Corresponding -106 dBm as defined in Section 6.3.8 in TS38.331[2] |
| Active Sidelink UEs | V2X sidelink Communication preconfiguration |  | As specified in Table A.3.21.2-1 and A.3.21.2-3 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-StartRB-Subchannel-r16 included in SL-ResourcePool |  | 10 | Indicates the lowest RB index of the subchannel with the lowest index. |
|  | Sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB |
| Timing offset among Active Sidelink Ues |  | s | CP/2 | Synchronous |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

Table A.9.1.4.2.1-2: Active Sidelink UE Specific Test Parameters for V2X UE Resource Pre-emption Tests for PSSCH-RSRP measurements

| Parameter | Unit | Active Sidelink UE |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel) Note 3 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS | kHz | 30 |  |
| PSCCH RMC (defined in A.3.21.3) | - | CC.1A HD |  |
| PSSCH RMC (defined in A.3.21.3) | - | CD.1A HD |  |
| Note1 | dBm/30 kHz | N/A | -100 |
| PSCCH | dB |  | 5 |
| PSSCH | dB |  | 5 |
| PSCCH Note2 | dB |  | 5 |
| PSSCH Note2 | dB |  | 5 |
| PSSCH-RSRP Note 2 | dBm/30kHz |  | -95 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other Ues and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, PSSCH-RSRP have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: The UE is only required to be tested in one of the supported test configurations. |  |  |  |

##### A.9.1.4.2.2	Test Requirements

The test time T1 and T2 should be long enough. The UE under test is required to trigger resource reselection and not to transmit on the reserved resource at slot n when the high priority reservation is transmitted by the active V2X sidelink UE before n-Tpre-empt, where

Tpre-empt = T3+Tproc,0

T3 = 2ms and Tproc,0= 1 slot for FR1.

The rate of PSSCH transmissions on the resources at slot n shall be less than 10% during repeated tests.

#### A.9.1.4.3	Test for V2X UE Resource Re-evaluation

##### A.9.1.4.3.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to autonomous resource re-evaluation for V2X UE in mode 2 defined in clause 12.5. For this test, the UE is triggered by the test loop function or the upper layers to transmit for V2X Sidelink Communication.

The test parameters are given in Table A.9.1.4.3.1-1, A.9.1.4.3.1-2 and A.9.1.4.3.1-3 below. There are 130 active V2X sidelink Ues in this test. The first 100 active V2X sidelink Ues are scheduled with 50ms periodicity. The last 30 active V2X sidelink Ues are aperiodic service UE with retransmission reservation period equaling 15ms.

Both the UE under test and active V2X sidelink Ues select GNSS as synchronization reference source. The test system can emulate and send the GNSS signal to the test UE and active V2X sidelink Ues. The test parameters for GNSS signals are defined in B.4.1.

The test consists of three duration T0, T1, T2.

During T0, the signal from Test Equipement are configured. The resource occupied by the active V2X sidelink UEs is expected to be excluded in the resource selection procedure such that the measured PSSCH-RSRP is above the measurement threshold. The test equipment shall just configure the resource pool for the test UE without the MAC PDU for transmission channel configuration.

During T1, the signal from Test Equipement are configured. Some of the resource occupied by the active V2X sidelink Ues is expected to be excluded in the resource selection procedure such that the measured PSSCH-RSRP is above the measurement threshold and some of the resource occupied by the active V2X sidelink Ues is expected to be included in the resource selection procedure such that the measured PSSCH-RSRP is below the measurement threshold. The test system shall emulate the active V2X sidelink Ues to transmit PSCCH/PSSCH every 50ms according to the RSRP level specified in the Table A. 9.1.4.3.1-2, but UE #0~29 will be silent during T2.

At the end of T1, where slot index mod 100 = 99, the test equipment shall send one message with a SL-SCH MAC PDU as specified in Clause 6.1.6 in TS 38.321[7], in order to make sure that the UE under test shall be scheduled to periodically transmit PSCCH/PSSCH.

During T2, the additional aperiodic active V2X sidelink UEs from Test Equipement are configured in the beginning 30 slots, and the resource occupied by these active V2X sidelink UEs is expected to be excluded in the resource re-evaluation procedure such that the measured PSSCH-RSRP is above the measurement threshold shown in Table A. 9.1.4.3.1-2. The test system shall emulate the active V2X sidelink UEs to transmit PSCCH/PSSCH with the maximum number of reserved PSCCH/PSSCH resources equalling n2 and time resource assignment interval as 15ms.

During T2, the test UE is expeted to reselect the resources and transmit the PSCCH/PSSCH in the newly re-evaluated resources.

Table A.9.1.4.3.1-1: Test Parameters for V2X UE Resource Selection Tests for Re-evaluation

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | TDD carrier in Band n47 and n38 |
| Channel Bandwidth (BWchannel) Note 2 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink communication pre-configuration |  |  | As specified in Table A.3.21.2-2 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 | Indicates the number of sub-channels for TX resource pool |
| sl-SubchannelSize-r16 included in SL-ResourcePool |  |  | 10 |  |
| sl-StartRB-Subchannel-r16 |  |  | 0 |  |
| Number of Active Sidelink UEs |  |  | 130 | Active Sidelink UE i = 0, .., 129 |
| SL-ThresPSSCH-RSRP-r16 |  |  | 17 | Corresponding -96 dBm as defined in Section 6.3.5 in TS38.331[2] |
| Active Sidelink UEs(UE i=0-99) | V2X sidelink Communication preconfiguration |  | As specified in Table A.3.21.2-2 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i}Note1 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [3]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize-r16 included in SL-ResourcePool |  | 10 | Indicates the size of sub-channels for TX resource pool |
|  | sl-ResourceReservePeriod2-r16 | ms | 50 |  |
| Active Sidelink UEs(UE i= 100-129) | V2X sidelink Communication preconfiguration |  | As specified in Table A.3.21.2-2 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i}Note1 | Indicates the bitmap of the TX resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213 [3]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-SubchannelSize included in SL-ResourcePool |  | 10 | Indicates the size of sub-channels for TX resource pool |
|  | sl-MultiReserveResource-r16 |  | enabled |  |
|  | sl-MaxNumPerReserve-r16 |  | n2 |  |
|  | sl-ResourceReservePeriod2-r16 |  | 0 | Unit:ms |
| Timing offset among Active Sidelink UEs |  | s | CP/2 | Synchronous |
| T0 |  | s | 1 |  |
| T1 |  | ms | 50 |  |
| T2 |  | ms | 50 |  |
| Note 1: {1i} is a sequence of ninety-nine 0’s with one 1 in (mod(i,100)+1)’th position. Note 2: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

Table A.9.1.4.3.1-2: Active Sidelink UE Specific Test Parameters for V2X UE Resource Selection Tests for Re-evaluation (UE #0...99)

| Parameter | Unit | Active Sidelink UE i (i = 0, .., 99) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel) Note 7 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS | kHz | 30 |  |
| PSCCH RMC (defined in A.3.21.3) | - | CC.1A HD |  |
| PSSCH RMC (defined in A.3.21.3) | - | CD.1A HD |  |
| Note1 | dBm/SCS | -103 |  |
| PSSCH1 Note 3 | dB | 22 | 22 |
| PSSCH2 Note 4 | dB | 2 | 2 |
| PSSCH3 Note 5 | dB | 12 | -infinity |
| PSSCH4 Note 6 | dB | 12 | 12 |
| PSSCH1 Note2,3 | dB | 22 | 22 |
| PSSCH2 Note2,4 | dB | 2 | 2 |
| PSSCH3 Note2,5 | dB | 12 | -infinity |
| PSSCH4 Note2,6 | dB | 12 | 12 |
| PSSCH -RSRP1 Note 2, 3 | dBm/SCS | -81 | -81 |
| PSSCH -RSRP2 Note 2, 4 | dBm/SCS | -101 | -101 |
| PSSCH -RSRP3 Note 2, 5 | dBm/SCS | -91 | -infinity |
| PSSCH -RSRP4 Note 2, 6 | dBm/SCS | -91 | -91 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: , PSSCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: UE #50~64 and UE #85~99 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #50-64 and #85-99. Note 4: UE #30~49 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #30-49. Note 5: UE #0~29 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #0-29. Note 6: UE #65~84 will periodically occupy the subchannels on the slot with “#slot index mod 100” = #65-84. Note 7: The UE is only required to be tested in one of the supported configurations. |  |  |  |

Table A.9.1.4.3.1-3: Active Sidelink UE Specific Test Parameters for V2X UE Resource Selection Tests for Re-evaluation (UE #100…129)

| Parameter | Unit | Active Sidelink UE i (i = 100, .., 129) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| NR RF Channel Number | - | 1 |  |
| Channel Bandwidth (BWchannel) Note 4 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS | kHz | 30 |  |
| PSCCH RMC (defined in A.3.21.3) | - | CC.1A HD |  |
| PSSCH RMC (defined in A.3.21.3) | - | CD.1 A HD |  |
| Note1 | dBm/SCS | -103 |  |
| PSSCH | dB | -infinity | 22 |
| PSSCH Note2 | dB | -infinity | 22 |
| PSSCH-RSRP Note 2, Note 3 | dBm/SCS | -infinity | -81 |
| Antenna Configuration | - | 1x2 |  |
| Propagation Condition | - | AWGN |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, PSSCH-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: UE #100~129 will occupy the subchannels on the slots with “#slot index mod 100”= #0-29 during T2. Note 4: The UE is only required to be tested in one of the supported configurations. |  |  |  |

##### A.9.1.4.3.2	Test Requirements

The rate of PSSCH transmissions on the resources of the subchannels which are occupied by UE #65-84 shall be more than 90% during T2.

### A.9.1.5	Test for Congestion Control Measurement

#### A.9.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the V2X UE makes correct reporting of an event. This test will verify the congestion control measurement requirements in section 12.6.

The test parameters are given in Table A.9.1.5.1-1,  Table A.9.1.5.1-2 and A.9.1.5.1-3 below. In the measurement control information it is indicated to the V2X UE that event-triggered reporting with Event C1 is used. There are 4 active V2X sidelink UEs in this test. The test system shall emulate the active sidelink UE to transmit PSCCH/PSSCH every 50ms. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During T1, all of active V2X sidelink UEs are configured to transmit PSCCH/PSSCH with lower transmission power every 50ms. During T2, all of active V2X sidelink UEs are configured to transmit PSCCH/PSSCH with higher transmission power every 50ms.

Supported test configurations for FR1 NR cell are shown in Table A.9.1.5.1.1-1.

Table A.9.1.5.1.1-1: Supported Test Configurations for FR1 NR cell

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.9.1.5.1-2: General test parameters for Congestion Control Measurement Test for V2X UE

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | TDD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel)Note 2 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink communication configuration |  |  | As specified in Table A.3.21.2-1 and A.3.21.2-3 | IE values unless specified otherwise in this test. |
| sl-TimeResource-r16 included in SL-ResourcePool |  |  | 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 11111111111111111111 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
| sl-NumSubchannel-r16 included in SL-ResourcePool |  |  | 1 | ENUMERATED {n1} |
| sl-SubchannelSize included in SL-ResourcePool |  |  | 10 | ENUMERATED {n10} |
| sl-StartRB-Subchannel-r16 |  |  | 0 |  |
| threshS-RSSI-CBR |  |  | 19 | Corresponding -74dBm as defined in Section 6.3.8 in TS38.331[2] |
| Number of Active Sidelink UEs every 50ms |  |  | 4 | Active Sidelink UE i, where i = 0, 1, 2, 3 |
| Active Sidelink UEs (i = 0,1,2,3) | V2X sidelink Communication configuration |  | As specified in Table A.3.21.2-1 and A.3.21.2-3 | IE values unless specified otherwise in this test. |
|  | sl-TimeResource-r16 included in SL-ResourcePool |  | {1i}Note1 | Indicates the bitmap of the TX and Rx resource pool, which is defined by repeating the bitmap within a SFN cycle (see TS 38.213[3]) |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool |  | 1 |  |
|  | sl-SubchannelSize included in SL-ResourcePool |  | 10 |  |
| Timing offset between V2X UE and Active Sidelink UEs |  | s | CP/2 | Synchronous |
| c1-Threshold-r16 |  |  | 2 | Corresponding 0.02 as defined in Section 6.3.2 in TS38.331[2] |
| sl-CBR-RangeConfigList-r16 |  |  | [2 100] | Two ranges are defined by this list: 0 to 0.02 and 0.02 to 1 |
| sl-CR-Limit-r16 |  |  | 10 and 100 | Corresponding to the two CBR ranges: if CBR > 0.02, CR < 0.001, otherwise CR > 0.0001 |
| sl-ThresPSSCH-RSRP-r16 |  |  | 12 | Configure threshold <-98.64dBm/30kHz to ensure not blocking transmission |
| Hysteresis |  |  | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: {1i} is a sequence of ninety nine 0’s with one 1 in i+1’th position. Note 2: The UE is only required to be tested in one of the channel bandwidths. |  |  |  |  |

Table A.9.1.5.1-3: Active sidelink UE specific test parameters for Congestion Control Measurement Test for V2X UE

| Parameter | Unit | Active Sidelink UE i (i = 0, 1, 2, 3) |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  |
| NR RF Channel Number |  | 1 |  |  |
| Channel Bandwidth (BWchannel) Note 7 | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |  |
| SCS | kHz | 30 |  |  |
| PSCCH RMC (defined in A.3.21.3) |  | CC.1A HD |  |  |
| PSSCH RMC (defined in A.3.21.3) |  | CD.1A HD |  |  |
| Note1 | dBm/30 kHz | -103 |  |  |
|  | dB | 4.35 |  | 10.32 |
| PSSCH-RSRP Note 2 | dBm/30 kHz | -98.65 |  | -92.68 |
| SL-RSSI1 Note 2,3 | dBm/3.6 MHz | -76.5 |  | -71.5 |
| SL-RSSI2 Note 2,4 | dBm/3.6 MHz | -82.21 |  | -82.21 |
| Io1 Note 2,5 | dBm/3.6 MHz | -76.5 |  | -71.5 |
| Io2 Note 2,6 | dBm/3.6 MHz | -82.21 |  | -82.21 |
| Propagation Condition | - | AWGN |  |  |
| Note 1: Interference from other UEs and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: PSSCH Es/Noc, PSSCH-RSRP, SL-RSSI1, SL-RSSI2, Io1 and Io2 levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SL-RSSI1 is the SL-RSSI level measured on the slot# 0 - 3 with “SFN mod 5 = 0”. Note 4: SL-RSSI2 is the SL-RSSI level measured on the slot# 4-9 with “SFN mod 5 = 0” and the slot# 0-9 with “SFN mod 5 = 1,…, 4”. Note 5: Io1 is the Io level measured on the slot# 0 - 3 with “SFN mod 5 = 0”. Note 6: Io2 is the Io level measured on the slot# 4-9 with “SFN mod 5 = 0” and the slot# 0-9 with “SFN mod 5 = 1,…, 4”. Note 7: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

#### A.9.1.5.2	Test Requirements

For UEs that support NR Uu and sidelink operation, the UEs shall not send event C1 triggered measurement reports during T1 and shall send event C1 triggered measurement reports during T2.

For UEs that support sidelink operation only, the UE channel occupancy ratio shall be larger than 0.001 during T1, and the UE channel occupancy ratio shall be smaller than 0.001 curing T2.

The rate of correct events observed during repeated tests shall be at least 98%.

### A.9.1.6	Test for Interruption

#### A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication

##### A.9.1.6.1.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to V2X sidelink communication defined in clause 12.7.1 under the following additional conditions:

-	The UE is out of coverage on the V2X sidelink carrier and is associated with a serving cell on a non-V2X sidelink carrier

This test is applicable for V2X sidelink communication capable UEs that support inter-band concurrent V2X sidelink operation.

For this test, the UE is triggered by the test loop function or the upper layers to monitor V2X sidelink communication.

The test parameters are given in Table A.9.1.6.1.1-1, Table A.9.1.6.1.1-2, Table A.9.1.6.1.1-3 and Table A.9.1.6.1.1-4. The test consists of one active cell (PCell) on the serving RF channel 1, and there are no active cells on RF channel 2. On RF channel 2, the test consists of 8 active Sidelink UEs in this test transmitting V2X sidelink communication.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the V2X sidelink communication transmission from other active Sidelink UEs on the V2X sidelink communication resources.

During T2, the test system establishes a RRC connection with the UE. No PDSCH traffic is scheduled for UE, and the UE is expected to transmit SidelinkUEInformationNR indicating sl-RxInterestedFreqList. On reception of SidelinkUEInformationNR, the test system shall send RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformationNR for up to 2 second, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on PCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during V2X sidelink communication (no missed ACK/NACKs are allowed).

Table A.9.1.6.1.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR Uu: FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR Uu: TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR Uu: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.9.1.6.1.1-2: Test Parameters for Interruptions due to V2X Sidelink Communication

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number | - | 1, 2 | RF channel 1 is non-V2X sidelink carrier RF channel 2 is V2X sidelink carrier |
| Active cell | - | Cell 1 | PCell on RF channel number 1 |
| CP length of Cell 1 | - | Normal |  |
| T1 | s | 5.12 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to 2 second if UE does not transmit SidelinkUEInformationNR during this period. |  |
| T3 | s | 10 |  |

Table A.9.1.6.1.1-3: Slidelink Communication Configuration for Interruptions due to V2X Sidelink Communication

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | - | 2 | TDD carrier in Band n47 or n38 |
| Channel Bandwidth (BWchannel) Note 1 |  | MHz | 20 (NRB,c = 50) or 40 (NRB,c = 100) |  |
| SCS |  | kHz | 30 |  |
| V2X sidelink Communication configuration |  | - | As specified in section A.3.21.2 | IE values unless specified otherwise in this test. |
| Number of Active Sidelink UEs |  | - | 8 | Active Sidelink UE i = 0, .., 7 |
| Active Sidelink UEs (UE i = 0, .., 7) | V2X sidelink Communication configuration | - | As specified in section A.3.21.2 | IE values unless specified otherwise in this test. |
|  | PSCCH Reference Measurement Channel | - | CC.1A HD | As specified in Table A.3.21.3-1 |
|  | PSSCH Reference Measurement Channel | - | CD.1A HD | As specified in Table A.3.21.3-2 |
|  | sl-NumSubchannel-r16 included in SL-ResourcePool | - | 1 | Indicates the number of sub-channels for TX resource pool |
|  | sl-StartRB-Subchannel-r16 included in SL-ResourcePool | - | i | Indicates the lowest RB index of the subchannel with the lowest index for active Sidelink UE i = 0, .., 7. |
|  | PSBCH-RSRP | dBm/30kHz | -95 |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

Table A.9.1.6.1.1-4: Cell specific test parameters for interruptions due to V2X slidelink communication

| Parameter |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF Channel Number |  |  | 1 |  |  |
| UE RRC state |  |  | IDLE | CONNECTED |  |
| Duplex Mode | Config 1 |  | FDD |  |  |
|  | Config 2,3 |  | TDD |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |
| Channel Bandwidth (BWchannel) | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP Configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| DRX Cycle |  |  | N/A |  |  |
| PDSCH Reference measurement channel | Config 1 |  | N/A | None | SR.1.1 FDD |
|  | Config 2 |  | N/A | None | SR.1.1 TDD |
|  | Config 3 |  | N/A | None | SR.2.1 TDD |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference Channel | Config 1 |  | CCR.1.1 FDD |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |
| SSB configuration | Config 1,2 |  | SSB.1 FR1 |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |
| SMTC Configuration |  |  | SMTC.2 |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 | Config 1,2,3 | dBm/15 kHz | -98 |  |  |
| Note2 | Config 1,2 | dBm/SCS | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
|  |  | dB | 3 |  |  |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -95 |  |  |
|  | Config 3 |  | -92 |  |  |
| Io Note 3 | Config 1,2 | dBm/9.36 MHz | -65.3 |  |  |
|  | Config 3 | dBm/38.1 MHz | -59.2 |  |  |
| Antenna Configuration |  |  | 1x2 |  |  |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

##### A.9.1.6.1.2	Test Requirements

The UE shall be continuously scheduled on PCell on RF channel 1 during T3. During T3, 100% of all expected ACK/NACKs shall be transmitted by the V2X UE.

# A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1

Editor’s note: Test cases for EN-DC with NR PSCell under CCA and SCell under CCA are also included here.

## A.10.1	RRC_CONNECTED state mobility

### A.10.1.1	RRC connection mobility control

#### A.10.1.1.1	Random Access

##### A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA

###### A.10.1.1.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in clause 6.2.2A.2 and clause 7.1.2 in an AWGN model.

For this test two cells are used, with the configuration of Cell 1 (E-UTRA PCell) specified in clause A.3.7A.2.1 and Cell 2 configured as PSCell in FR1. Cell 1 is on a licensed band and cell 2 is subjected to CCA. Supported test parameters are shown in Table A.10.1.1.1.1.1-1. UE capable of EN-DC with PSCell in FR1 needs to be tested by using the parameters in Table A.10.1.1.1.1.1-2.

Table A.10.1.1.1.1.1-1: Supported test configurations for contention based random access test in FR1 for PSCell with CCA

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.10.1.1.1.1.1-2: General test parameters for contention based random access test in FR1 for PSCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1,2 |  | SSB.1 CCA | As defined in A.3.10A |
|  | Note 5, 6 |  |  | Config 1,2 |  | SSB.2 CCA | As defined in A.3.10A |
| DBT Window Configuration |  |  |  | Config 1,2 |  | DBT.1 | As specified in A.3.28.1 |
| DL CCA model |  |  |  | Config 1,2 |  | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  | Config 1,2 |  | As specified in A.3.26.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1,2 |  | TDD |  |
| TDD Configuration |  |  |  | Config 1,2 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1,2 |  | SR.1.1 CCA | As defined in A.3.1A.1. |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | 3 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | -17 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  |  | Config 1,2 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  |  |  | FR1 PRACH configuration 1 under CCA | As defined in A.3.8A.2. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75 / 0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

###### A.10.1.1.1.1.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.10.1.1.1.1.2.1	Random Access Preamble Transmission

To test the UE behavior specified in Clause 6.2.2A.2.1.1 the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB, if the UL CCA is successful.

The three requirements below are relevant for all cases of PRACH transmissions described within the whole clause A.10.1.1.1.1.2:

The System Simulator shall implement the UL CCA model of A.3.26.2 for the RACH occasions where PRACH transmissions are expected. The System Simulator shall monitor the RACH occasions to detect if the UE is transmitting PRACH preambles. If a PRACH transmission is detected on a RACH occasion that is expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit PRACH for semi-static channel access mode; for dynamic channel access mode it is assumed that RACH occasions are always scheduled within a UE-initiated COT.

In case of UL CCA failure, The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.10.1.1.1.1.2.2	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2A.2.1.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble. In case of CCA DL failure, the test equipment should delay the transmission of Random Access Response.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting msg3 when CCA UL failure. If a msg3 transmission is detected on a grant expected to have UL CCA failure, the test is considered as failed.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.10.1.1.1.1.2.3	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2A.2.1.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles. In case of CCA DL failure, the test equipment should delay the transmission of Random Access Response.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission

To test the UE behavior specified in clause 6.2.2A.2.1.4, the System Simulator shall provide an UL grant for msg3 retransmission following a successful Random Access Response if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting msg3 when CCA UL failure. If a msg3 is detected on a grant expected to have UL CCA failure, the test is considered as failed.

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

A.10.1.1.1.1.2.5		Contention Resolution Timer expiry

To test the UE behavior specified in Clause 6.2.2A.2.1.6 the System Simulator shall not send a response to a msg3.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

##### A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA

###### A.10.1.1.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in clause 6.2.2A.2 and clause 7.1.2 in an AWGN model.

For this test two cells are used, with the configuration of Cell 1 (E-UTRA PCell) specified in clause A.3.7A.2.1 and Cell 2 configured as PSCell in FR1. Cell 1 is on a licensed band and cell 2 is subjected to CCA. Supported test parameters are shown in Table A.10.1.1.1.2.1-1. UE capable of EN-DC with PSCell in FR1 needs to be tested by using the parameters in Table A.10.1.1.1.2.1-2.

Table A.10.1.1.1.2.1-1: Supported test configurations for non-contention based random access test in FR1 for PSCell with CCA

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.10.1.1.1.2.1-2: General test parameters for non-contention based random access test in FR1 for PSCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1,2 |  | SSB.1 CCA | As defined in A.3.10A |
|  | Note 5, 6 |  |  | Config 1,2 |  | SSB.2 CCA | As defined in A.3.10A |
| DBT Window Configuration |  |  |  | Config 1,2 |  | DBT.1 | As specified in A.3.28.1 |
| DL CCA model |  |  |  | Config 1,2 |  | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  | Config 1,2 |  | As specified in A.3.26.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1,2 |  | TDD |  |
| TDD Configuration |  |  |  | Config 1,2 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1,2 |  | SR.1.1 CCA | As defined in A.3.1A.1. |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | 3 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | -17 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  |  | Config 1,2 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  |  |  | FR1 PRACH configuration 2 under CCA | As defined in A.3.8A.2. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75 / 0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

###### A.10.1.1.1.2.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission

In Test-1, to test the UE behavior specified in Clause 6.2.2A.2.2.1 for SSB-based Random Access Preamble transmission, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the Random Access Preamble which has the Preamble Index associated with the SSB with index 0.

The three requirements below are relevant for all cases of PRACH transmissions described within the whole clause A.10.1.1.1.2.2:

The System Simulator shall implement the UL CCA model of A.3.26.2 for the RACH occasions where PRACH transmissions are expected. The System Simulator shall monitor the RACH occasions to detect if the UE is transmitting PRACH preambles. If a PRACH transmission is detected on a RACH occasion that is expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit PRACH for semi-static channel access mode; for dynamic channel access mode it is assumed that RACH occasions are always scheduled within a UE-initiated COT.

In case of UL CCA failure, The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.10.1.1.1.2.2.2	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2A.2.2.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble. In case of CCA DL failure, the test equipment should delay the transmission of Random Access Response.

The UE may stop monitoring for Random Access Response(s) if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting msg3 when CCA UL failure. If a msg3 is detected on a grant expected to have UL CCA failure, the test is considered as failed.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.10.1.1.1.2.2.3	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2A.2.2.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles. In case of CCA DL failure, the test equipment should delay the transmission of Random Access Response.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA

###### A.10.1.1.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in clause 6.2.2A.3 and clause 7.1.2 in an AWGN model.

For this test two cells are used, with the configuration of Cell 1 (E-UTRA PCell) specified in clause A.3.7A.2.1 and Cell 2 configured as PSCell in FR1. Cell 1 is on a licensed band and cell 2 is subjected to CCA. Supported test parameters are shown in Table A.10.1.1.1.3.1-1. UE capable of EN-DC with PSCell in FR1 needs to be tested by using the parameters in Table A.10.1.1.1.3.1-2.

Table A.10.1.1.1.3.1-1: Supported test configurations for 2-step RA type contention based random access test in FR1 for PSCell with CCA

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.10.1.1.1.3.1-2: General test parameters for 2-step RA type contention based random access test in FR1 for PSCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1,2 |  | SSB.1 CCA | As defined in A.3.10A |
|  | Note 5, 6 |  |  | Config 1,2 |  | SSB.2 CCA | As defined in A.3.10A |
| DBT Window Configuration |  |  |  | Config 1,2 |  | DBT.1 | As specified in A.3.28.1 |
| DL CCA model |  |  |  | Config 1,2 |  | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  | Config 1,2 |  | As specified in A.3.26.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1,2 |  | TDD |  |
| TDD Configuration |  |  |  | Config 3,4 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1,2 |  | SR.1.1 CCA | As defined in A.3.1A.1. |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | 3 |  |
|  |  | SS-RSRP Note 2 |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | -17 |  |
|  |  | SS-RSRP Note 2 |  |  | dBm/ SCS | -115 |  |
| Io |  |  |  | Config 1,2 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  |  |  | FR1 MsgA configuration 1 under CCA | As defined in A.3.20A.2. |
| msgA-RSRP-ThresholdSSB |  |  |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75 / 0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

###### A.10.1.1.1.3.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.10.1.1.1.3.2.1	MsgA Transmission

To test the UE behaviour specified in Clause 6.2.2A.3.1.1 the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured msgA-RSRP-ThresholdSSB, if the UL CCA is successful.

below are relevant for all cases of MsgA transmissions described within the clause A.10.1.1.1.3.2:

The System Simulator shall implement the UL CCA model for the MsgA occasions (i.e. both MsgA PRACH and MsgA PUSCH occasions) where MsgA transmissions are expected. The System Simulator shall monitor the MsgA occasions to detect if the UE is transmitting MsgA. If a MsgA transmission is detected on MsgA occasions that are expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit MsgA for semi-static channel access mode; for dynamic channel access mode it is assumed that MsgA occasions are always scheduled within a UE-initiated COT.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated PRACH transmission power in case of UL CCA failure.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated PRACH transmission power in case of UL CCA failure. In addition, the power applied to all MsgA transmission shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.10.1.1.1.3.2.2	MsgB Reception

To test the UE behaviour specified in Clause 6.2.2A.3.1.2 the System Simulator shall transmit a MsgB with fallbackRAR containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble. In case of CCA DL failure, the test equipment should delay the transmission of MsgB.

The UE may stop monitoring for MsgB(s) and shall transmit the msg3 if the MsgB with a fallbackRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting msg3 when CCA UL failure. If a msg3 is detected on a grant expected to have UL CCA failure, the test is considered as failed.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB’s contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble .

In addition, the power applied to all MsgA transmission shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.10.1.1.1.3.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2A.3.1.3 the System Simulator shall transmit a MsgB with fallbackRAR containing a successRAR message and a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles. In case of CCA DL failure, the test equipment should delay the transmission of MsgB.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if no MsgB  is received within the MsgB Response window.

In addition, the power applied to all MsgA transmission shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA

###### A.10.1.1.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in clause 6.2.2A.3 and clause 7.1.2 in an AWGN model.

For this test two cells are used, with the configuration of Cell 1 (E-UTRA PCell) specified in clause A.3.7.2.1 and Cell 2 configured as PSCell in FR1. Cell 1 is on a licensed band and cell 2 is subjected to CCA. Supported test parameters are shown in Table A.10.1.1.1.4.1-1. UE capable of EN-DC with PSCell in FR1 needs to be tested by using the parameters in Table A.10.1.1.1.4.1-2.

Table A.10.1.1.1.4.1-1: Supported test configurations for non-contention based random access test for 2-step RA type in FR1 for PSCell with CCA

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table A.10.1.1.1.4.1-2: General test parameters for non-contention based random access test for 2-step RA type in FR1 for PSCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1,2 |  | SSB.1 CCA | As defined in A.3.10A |
|  | Note 5, 6 |  |  | Config 1,2 |  | SSB.2 CCA | As defined in A.3.10A |
| DBT Window Configuration |  |  |  | Config 1,2 |  | DBT.1 | As specified in A.3.28.1 |
| DL CCA model |  |  |  | Config 1,2 |  | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  | Config 1,2 |  | As specified in A.3.26.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1,2 |  | TDD |  |
| TDD Configuration |  |  |  | Config 1,2 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1,2 |  | SR.1.1 CCA | As defined in A.3.1A.1. |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | 3 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1,2 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | -17 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  |  | Config 1,2 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power (PCMAX,f,c) |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  |  |  | FR1 MsgA configuration 2 under CCA | As defined in A.3.20A.2. |
| msgA-RSRP-ThresholdSSB |  |  |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75 / 0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

###### A.10.1.1.1.4.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.10.1.1.1.4.2.1	MsgA Transmission

In Test-1, to test the UE behavior specified in Clause 6.2.2A.3.2.1 for MsgA transmission, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the MsgA which has the Preamble Index associated with the SSB with index 0.

In addition, the System Simulator shall receive the MsgA on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given first by the msgA-SSB-SharedRO-MaskIndex if configured, or next by the ra-ssb-OccasionMaskIndex if configured.

The three requirements below are relevant for all cases of MsgA transmissions described within the clause A.10.1.1.1.4.2:

The System Simulator shall implement the UL CCA model for the MsgA occasions (i.e. both MsgA PRACH and MsgA PUSCH occasions) where MsgA transmissions are expected. The System Simulator shall monitor the MsgA occasions to detect if the UE is transmitting MsgA. If a MsgA transmission is detected on MsgA occasions that are expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit MsgA for semi-static channel access mode; for dynamic channel access mode it is assumed that MsgA occasions are always scheduled within a UE-initiated COT.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated PRACH transmission power in case of UL CCA failure.

In addition, the power applied to all MsgA transmission shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.10.1.1.1.4.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2A.3.2.2 the System Simulator shall transmit a MsgB containing a successRAR MAC subPDU corresponding to the transmitted Random Access Preamble after 5 MsgA transmissions have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble. In case of CCA DL failure, the test equipment should delay the transmission of MsgB.

The UE may stop monitoring for MsgB if the MsgB contains a successRAR MAC subPDU corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated MsgA transmission power if Random Access Responses Reception has not been considered as successful.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2A.3. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.10.1.1.1.4.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2A.3.2.3 the System Simulator shall transmit a MsgB corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles. In case of CCA DL failure, the test equipment should delay the transmission of MsgB.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated MsgA transmission power when the backoff time expires if no MsgB is received within the MsgB Response window configured in RACH-ConfigGenericTwoStepRA.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2A.3. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

## A.10.2	Timing

### A.10.2.1	UE transmit timing

#### A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA

##### A.10.2.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb when PSCell is subject to DL CCA and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2. Supported test configurations are shown in Table 10.2.1.1.1-1.

Table A.10.2.1.1.1-1: Supported test configurations for UE transmit timing test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

The test consists of E-UTRA PCell and NR PSCell, which is subject to DL CCA. The configuration for E-UTRA is given in A.3.7.2.1. Table A.10.2.1.1.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.10.2.1.1.1-3.

Table A.10.2.1.1.1-2: Cell Specific Test Parameters for UE Transmit Timing test

| Parameter |  | Unit | Config | Test1 | Test2 |
| --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  |  | 1,2 | Freq1 | Freq1 |
| TDD configuration |  |  | 1,2 | TDDConf.1.1 CCA |  |
| BWchannel |  | MHz | 1,2 | 40: NRB,c = 106 |  |
| Initial BWP Configuration |  |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP Configuration |  |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |
| DRX Cycle |  | ms | 1,2 | N/A | DRX.8Note5 |
| DL CCA model |  |  | 1,2 | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | 1,2 | As specified in clause A.3.26.2.2 |  |
| PDSCH Reference |  |  | 1,2 | SR.1.1 CCA |  |
| CORESET Reference |  |  | 1,2 | CR.1.1 CCA |  |
| OCNG Patterns |  |  | 1,2 | OCNG pattern 1 |  |
| SSB configuration | Semi- static channel acces |  | 1,2 | SSB.1 CCA |  |
|  | Dymamic channel acces |  | 1,2 | SSB.2 CCA |  |
| SMTC configuration |  |  | 1,2 | SMTC.1 FR1 |  |
| TRS configuration |  |  | 1,2 | TRS.1.2 TDD |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  |  | 1,2 | 0.9375 | 0.9375 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  |  | 1,2 | 0.75 | 0.75 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  |  | 1,2 | 0.75 | 0.75 |
| UL CCA probability (PCCA_UL) |  |  | 1,2 | 1 | 1 |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 1,2 | 0 | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | dBm/30 kHz | 1,2 | -95 | -95 |
|  |  |  | 1,2 | 3 | 3 |
|  |  |  | 1,2 | 3 | 3 |
| SS-RSRPNote3 |  | dBm/30 kHz | 1,2 | -92 | -92 |
| IoNote3 |  | dBm/38.1MHz | 1,2 | -59.2 | -59.2 |
| Propagation condition |  |  | 1,2 | AWGN |  |
| SRS Config |  |  | 1,2 | SRSConf.1Note6 | SRSConf.2Note6 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRX related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.10.2.1.1.1-3. Note 7: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.26.2. Note 8: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

Table A.10.2.1.1.1-3: SRS Configuration for UE transmit timing

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2 25 for test configuration 3 | 25 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 0 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

##### A.10.2.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1)	Set up E-UTRA PCell according to parameters given in Table A.3.7.2.1-1 and setup NR PSCell according to parameters given in Table A.10.2.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset)×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.10.2.1.1.2-1

Table A.10.2.1.1.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first detected path (in time) of DL SSB. Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.10.2.2	UE timing advance

#### A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA

##### A.10.2.2.1.1	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment delay and accuracy requirement defined in clause 7.3.

##### A.10.2.2.1.2	Test Parameters

Supported test configurations are shown in table A.10.2.2.1.2-1. Both timing advance adjustment delay and accuracy are tested by using the parameters in table A.10.2.2.1.2-2, A.10.2.2.1.2-3 and A.10.2.2.1.2-4. The configuration of Cell 1 (LTE PCell) is specified in clause A.3.7.2.1.

In all test cases, two cells are used. Cell 1 is the PCell in the primary Timing Advance Group (pTAG) and cell 2 is the PSCell which is subject to DL CCA is in the secondary Timing Advance Group (sTAG). Each test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands for sTAG are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.10.2.2.1.2-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured for PSCell in sTAG.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element for sTAG, as specified in clause 6.1.3.4 in TS 38.321 [7]. The Timing Advance Command value shall be set to 31, which according to clause 4.2 in TS 38.213 [3] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance for sTAG used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements for sTAG, with Timing Advance Command value specified in table A.10.2.2.1.2-2. This value shall result in changes of the timing advance for sTAG used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in clause 7.3.2.1, the UE adjusts its uplink timing at slot n+k for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in clause 5.2 in TS 38.321, shall be configured so that it does not expire in the duration of the test.

Table A.10.2.2.1.2-1: Supported test configurations for timing advance test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE supporting EN-DC only on NR band(s) with shared spectrum access is required to be tested |  |

Table A.10.2.2.1.2-2: General test parameters for timing advance test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | Cell 1: 1 Cell 2: 2 | 1 for E-UTRAN PCell 2 for NR PSCell |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.3.9.2.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.3.9.2.2-1 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.3.9.3.1-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.3.9.3.2-1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For 30 kHz SCS NTA_new = NTA_old + 4096*Tc (based on equation in clause 4.2 of TS 38.213 [3]) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.10.2.2.1.2-3: Cell specific test parameters for timing advance test

| Parameter |  |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| TDD configuration |  |  | Config 1,2 |  | TDDConf.1.1 CCA |  |
| BWchannel |  |  | Config 1,2 | MHz | 40: NRB,c = 106 |  |
| BWP BW |  |  | Config 1,2 | MHz | 40: NRB,c = 106 |  |
| DRX Cycle |  |  | Config 1,2 | ms | Not Applicable |  |
| DL CCA model |  |  | Config 1,2 |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | Config 1,2 |  | As specified in clause A.3.26.2.2 |  |
| PDSCH Reference |  |  | Config 1,2 |  | SR.1.1 CCA |  |
| CORESET Reference |  |  | Config 1,2 |  | CR.1.1 CCA |  |
| TRS configuration |  |  | Config 1,2 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  | Config 1,2 |  | OCNG pattern 1 |  |
| SSB Configuration |  | Semi- static channel acces | Config 1,2 |  | SSB.1 CCA |  |
|  |  | Dymamic channel acces | Config 1,2 |  | SSB.2 CCA |  |
| SMTC configuration |  |  | Config 1,2 |  | SMTC.1 FR1 |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  |  | Config 1,2 |  | 1 |  |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  |  | Config 1,2 |  | 1 |  |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  |  | Config 1,2 |  | 1 |  |
| UL CCA probability PCCA |  |  | Config 1,2 |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | Config 1,2 |  |  | dBm/30 kHz | -95 |  |
|  | Config 3,6 |  |  |  | -95 |  |
|  |  |  |  | dB | 3 |  |
|  |  |  |  | dB | 3 |  |
| IoNote3 | Config 1,2 |  |  | dBm/38.16MHz | -62.58 |  |
| Propagation condition |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.26.2. Note 5: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |

Table A.10.2.2.1.2-4: Sounding Reference Symbol Configuration for timing advance test

| Field |  | Value | Comment |
| --- | --- | --- | --- |
| c-SRS | Config 1,2 | 24 | Frequency hopping is disabled |
| b-SRS |  | 0 |  |
| b-hop |  | 0 |  |
| freqDomainPosition |  | 0 | Frequency domain position of SRS |
| freqDomainShift |  | 0 |  |
| groupOrSequenceHopping |  | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset |  | sl5=4 for SCS 30kHz | Once every 5 slots |
| pathlossReferenceRS |  | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage |  | Codebook | Codebook based UL transmission |
| startPosition |  | 0 | resourceMapping setting: SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols |  | n1 |  |
| repetitionFactor |  | n1 |  |
| combOffset-n2 |  | 0 | transmissionComb setting |
| cyclicShift-n2 |  | 0 |  |
| nrofSRS-Ports |  | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |  |

##### A.10.2.2.1.3	Test Requirements

The UE shall apply the signalled Timing Advance value for PSCell in sTAG to the transmission timing at the designated activation time i.e. k+1 slots after the reception of the timing advance command, where k=5.

The Timing Advance adjustment accuracy for PSCell in sTAG shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

## A.10.3	Signalling characteristics

### A.10.3.1	Radio link monitoring

#### A.10.3.1.1	Introduction

In the test cases specified in clause A.10.3.1, any uplink signal transmitted by the UE is used for detecting the in-/out-of-sync state of the UE. In terms of measurement, the uplink signal is verified based on the UE output power:

- UE output power higher than Transmit OFF power -50 dBm (as defined in TS 38.101-3 [20]) means uplink signal

- UE output power equal to or less than Transmit OFF power -50 dBm (as defined in TS 38.101-3 [20]) means no uplink signal.

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

#### A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode

##### A.10.3.1.2.1	Test purpose and environment

The purpose of this test is to verify that the UE properly detects the out-of-sync and in-sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR1 PSCell radio link monitoring requirements in clause 8.1A.

In the test, UE is configured to perform RLM based on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.10.3.1.2.1-1. The test parameters are given in Tables A.10.3.1.2.1-2, A.10.3.1.2.1-3, and A.10.3.1.2.1-4 below. There are two cells in the test: Cell 1 is the E-UTRAN PCell, and Cell 2 is the FR1 PSCell which operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

The test consists of three successive time periods, with time duration of T1, T2 and T3, respectively. Figure A.10.3.1.2.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE transmits according to UL CCA model. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40 ms) in the test.

Table A.10.3.1.2.1-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.3.1.2.1-2: General test parameters for PSCell out-of-sync testing in non-DRX mode.

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1, Test 2 |
| Active E-UTRA PCell |  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  |  | 1 |
| Active PSCell |  |  |  | Cell 2 |
| RF Channel Number |  |  |  | 2 |
| DL CCA model |  |  |  | As specified in clause A.3.20.2.1 |
| UL CCA model |  |  |  | As specified in clause A.3.20.2.2 |
| Duplex mode |  | Config 1,2 |  | TDD |
| BWchannel |  | Config 1,2 | MHz | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1,2 |  | [DLBWP.0.1] |
| DL dedicated BWP configuration |  | Config 1,2 |  | [DLBWP.1.1] |
| UL initial BWP configuration |  | Config 1,2 |  | [ULBWP.0.1] |
| UL dedicated BWP configuration |  | Config 1,2 |  | [ULBWP.1.1] |
| TDD configuration |  | Config 1,2 |  | TBD |
| CORESET Reference Channel |  | Config 1,2 |  | TBD |
| SSB configuration |  | Config 1,2 |  | TBD |
| DBT window configuration |  | Config 1,2 |  | TBD |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | 30 kHz |
| PRACH Configuration |  | Config 1,2 |  | TBD |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | [OP.1] |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | [1-0] |
|  | Number of Control OFDM symbols |  |  | [2] |
|  | Aggregation level |  | CCE | [8] |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | [4] |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | [4] |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | [6] |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | TBD |
| T311 timer |  |  | ms | TBD |
| N310 |  |  |  | TBD |
| N311 |  |  |  | TBD |
| CSI-RS configuration for CSI reporting |  | Config 1,2 |  | [CSI-RS.2.1 TDD] |
| CSI-RS for tracking |  | Config 1,2 |  | [TRS.1.2 TDD] |
| T1 |  |  | s | TBD |
| T2 |  |  | s | TBD |
| T3 |  |  | s | TBD |
| D1 |  |  | s | TBD |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. NOTE 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.10.3.1.2.1-3: Cell-specific test parameters for PSCell out-of-sync testing in non-DRX mode.

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| DL CCA probability PCCA_DL |  | Note 6,8 |  | TBD |  |  | TBD |  |  |
|  |  | Note 7,8 |  |  |  |  |  |  |  |
| UL CCA probability PCCA_UL |  |  |  | TBD |  |  | TBD |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 4 |  |  | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |  |
| SNRNote 3,4 on RLM-RS | Config 1,2 |  | dB | 1 | [-7] | -15 | 1 | TBD | TBD |
| SNR on other channels and signals | Config 1,2 |  | dB | 1 |  |  | 1 |  |  |
|  | Config 1,2 |  | dBm/SCS | -95 |  |  | -95 |  |  |
| Propagation condition |  |  |  | TDL-C 300 ns 100 Hz |  |  | TDL-C 300 ns 100 Hz |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during muted slots or during DBT windows. NOTE 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 3: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT windows. NOTE 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3, respectively, in Figure A.10.3.1.2.1-1. NOTE 5: The SNR values are specified for testing a UE which supports 2 RX on at least one band. For testing of a UE which supports 4 RX on all bands, the SNR during T3 is A.3.6. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |

Table A.10.3.1.2.1-4: Measurement gap configuration for PSCell out-of-sync testing in non-DRX mode.

| Field | Test 1 | Test 2 |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | 0 | 0 |
| NOTE 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. NOTE 2: Ensure that RLM RS is partially overlapped with measurement gap. |  |  |

Figure A.10.3.1.2.1-1: SNR variation for out-of-sync testing.

##### A.10.3.1.2.2	Test requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

- During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

- The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

In Test 1, the UE is verified to meet the out-of-sync requirement for RLM-RS SSB Es/Iot <-7 dB.

In Test 2, the UE is verified to meet the out-of-sync requirement for RLM-RS SSB Es/Iot ≥-7 dB.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode

##### A.10.3.1.3.1	Test purpose and environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR1 PSCell radio link monitoring requirements in clause 8.1A.

In the test, UE is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.10.3.1.3.1-1. The test parameters are given in Tables A.10.3.1.3.1-2, and A.10.3.1.3.1-3 below. There are two cells in the test: Cell 1 is the E-UTRAN PCell, and Cell 2 is the FR1 PSCell which operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.10.3.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE transmits according to UL CCA model.

Table A.10.3.1.3.1-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.3.1.3.1-2: General test parameters for PSCell in-sync testing in non-DRX mode.

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active E-UTRA PCell |  |  |  |  | Cell 1 |
| E-UTRA RF Channel Number |  |  |  |  | 1 |
| Active PSCell |  |  |  |  | Cell 2 |
| RF Channel Number |  |  |  |  | 2 |
| DL CCA model |  |  |  |  | As specified in clause A.3.20.2.1 |
| UL CCA model |  |  |  |  | As specified in clause A.3.20.2.2 |
| Duplex mode |  |  | Config 1,2 |  | TDD |
| BWchannel |  |  | Config 1,2 | MHz | 40: NRB,c = 106 |
| DL initial BWP configuration |  |  | Config 1,2 |  | [DLBWP.0.1] |
| DL dedicated BWP configuration |  |  | Config 1,2 |  | [DLBWP.1.1] |
| UL initial BWP configuration |  |  | Config 1,2 |  | [ULBWP.0.1] |
| UL dedicated BWP configuration |  |  | Config 1,2 |  | [ULBWP.1.1] |
| TDD Configuration |  |  | Config 1,2 |  | TBD |
| CORESET Reference Channel |  |  | Config 1,2 |  | TBD |
| SSB Configuration |  |  | Config 1,2 |  | TBD |
| DBT window configuration |  |  | Config 1,2 |  | TBD |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 |  | 30 kHz |
| PRACH Configuration |  |  | Config 1,2 |  | TBD |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX |  |  |  |  | OFF |
| Gap pattern ID |  |  |  |  | N/A |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | TBD |
| T311 timer |  |  |  | ms | TBD |
| N310 |  |  |  |  | TBD |
| N311 |  |  |  |  | TBD |
| CSI-RS configuration for CSI reporting |  | Config 1,2 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1,2 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | TBD |
| T2 |  |  |  | s | TBD |
| T3 |  |  |  | s | TBD |
| T4 |  |  |  | s | TBD |
| T5 |  |  |  | s | TBD |
| D1 |  |  |  | s | TBD |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.10.3.1.3.1-3: Cell-specific test parameters for PSCell in-sync testing in non-DRX mode.

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA_DL |  |  | TBD |  |  |  |  |
| UL CCA probability PCCA_UL |  |  | TBD |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1,2 | dB | 1 | [-7] | [-15] | [-4.5] | 1 |
| SNR on other channels and signals | Config 1,2 | dB | 1 |  |  |  |  |
|  | Config 1,2 | dBm/SCS | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during muted slots or during DBT windows. NOTE 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 3: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT windows. NOTE 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.10.3.1.2.1-1. Note 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4 RX on all bands, the SNR during T3 and T4 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Figure A.10.3.1.2.1-1: SNR variation for in-sync testing.

##### A.10.3.1.3.2	Test requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode

##### A.10.3.1.4.1	Test purpose and environment

##### A.10.3.1.4.2	Test requirements

#### A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode

##### A.10.3.1.5.1	Test purpose and environment

##### A.10.3.1.5.2	Test requirements

### A.10.3.2	Interruption

#### A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA

##### A.10.3.2.1.1	Test Purpose and Environment

The purpose of this test is to verify E-UTRAN PCell and NR PSCell interruptions during Scell operations on an NR SCC with CCA, This test will verify the interruption requirements for E-UTRAN PCell and NR PSCell in EN-DC specified in TS 38.133 clause 8.2.1 and 8.3A. Supported test configurations are shown in table A.10.3.2.1.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.10.3.2.1.1-2 and A.10.3.2.1.1-3 below. The E-UTRAN cell specific test parameters are provided in Table A.3.7.2.1-1. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is LTE PCell, Cell2 and Cell3 is NR PSCell and NR SCell. Both of cell 2 and cell 3 are subject to CCA. The test consists of five time periods, with duration of T1, T2, T3, T4 and T5. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. Throughout the test, the LTE PCell and NR PSCell are continuously scheduled in DL. The power of signals on cell 1,2 and 3 is not modified during the test.

Prior to T1, a connection is started with cell 2 as the PSCell, and measurements of cell 3 are configured with gap pattern 0, such that cell 3 is reported. This ensures that cell 3 is known at the start of time period T1 and is not itself part of the tested requirement.

The point in time at which the RRC message implying SCell addition is received at the UE antenna connector, defines the start of time period T1. Measurement gap pattern 0 shall be stopped when the SCell is configured.

The point in time at which the RRC message implying SCell addition is received at the UE antenna connector, defines the start of time period T1.

The point in time at which the MAC-CE message implying SCell activation is received at the UE antenna connector, defines the start of time period T2.

The point in time at which the MAC-CE message implying SCell deactivation is received at the UE antenna connector, defines the start of time period T3.

The point in time at which deactivation delay requirement in section 8.3A are satisfied defines the start of time period T4

The point in time at which the RRC message implying SCell release is received at the UE antenna connector, defines the start of time period T5.

Table A.10.3.2.1.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD NR without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD NR without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.3.2.1.1-2: General test parameters for Interruptions during measurements on deactivated NR SCC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | One is E-UTRAN RF channel and the other two are NR RF channels |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on NR RF channel number 3. |
| CP length |  | Normal | Applicable to Cell1, Cell2 and Cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| T1 | s | <10 |  |
| T2 | s | <10 |  |
| T3 | s | <10 |  |
| T4 | s | <10 |  |
| T5 | s | <10 |  |

Table A.10.3.2.1.1-3: NR cell specific test parameters for Interruptions during measurements on deactivated NR SCC

| Parameter |  | Unit | Cell2 |  |  |  |  | Cell3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| TDD configuration | Config 1,2 |  | TDDConf.1.1 CCA |  |  |  |  | TDDConf.1.1 CCA |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 40: NRB,c = 106 |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |
| DL CCA model | Config 1,2 |  | As specified in clause A.3.20.2.1 |  |  |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |
| DL CCA probability for semi-static channel accessNote6,8 | PCCA_DL |  | 0.9375 |  |  |  |  | 0.9375 |  |  |  |  |
| DL CCA probability for dynamic channel accessNote7,8 | PCCA_DL_1 |  | 0.75 |  |  |  |  | 0.75 |  |  |  |  |
|  | PCCA_DL_2 |  | 0.75 |  |  |  |  | 0.75 |  |  |  |  |
| UL CCA model | Config 1,2 |  | As specified in clause A.3.20.2.2 |  |  |  |  | --- |  |  |  |  |
| UL CCA probability for semi-static channel access | PCCA_UL |  | 0.87 |  |  |  |  | --- |  |  |  |  |
| UL CCA probability for dynamic channel access | PCCA_UL |  | 0.75 |  |  |  |  | --- |  |  |  |  |
| Initial BWP Configuration | Config 1,2 |  | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Dedicated DL BWP Configuration | Config 1,2 |  | DLBWP.1.1 |  |  |  |  | DLBWP.1.1 |  |  |  |  |
| Initial UL BWP Configuration | Config 1,2 |  | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Dedicated UL BWP Configuration | Config 1,2 |  | ULBWP.1.1 |  |  |  |  | ULBWP.1.1 |  |  |  |  |
| PDSCH reference meassurement channel | Config 1,2 |  | SR.1.1 CCA |  |  |  |  | - |  |  |  |  |
| RMSI CORESET Parameters | Config 1,2 |  | CR.1.1 CCA |  |  |  |  | CR.1.1 CCA |  |  |  |  |
| PDCCH CORESET Parameters | Config 1,2 |  | CCR.1.1 CCA |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
| TRS configuration | Config 1,2 |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  | OP.1 |  |  |  |  |
| SSB configuration for semi-static channel accessNote6,8 | Config 1,2 |  | SSB.1 CCA |  |  |  |  | SSB.1 CCA |  |  |  |  |
| SSB configuration for dynamic channel accessNote7,8 | Config 1,2 |  | SSB.2 CCA |  |  |  |  | SSB.2 CCA |  |  |  |  |
| SMTC Configuration | Config 1,2 |  | SMTC.1 |  |  |  |  | SMTC.1 |  |  |  |  |
| DBT window configuration | Config 1,2 |  | DBT.1 |  |  |  |  | DBT.1 |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  | TCI.State.0 |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  | 1x2 Low |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 |  |  |  |  | -104 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  | -87 |  |  |  |  |
| Ês/Iot |  | dB | 17 |  |  |  |  | 17 |  |  |  |  |
| Ês/Noc |  | dB | 17 |  |  |  |  | 17 |  |  |  |  |
| IoNote3 | Config 1,2 | dBm/38.16MHz | -52.86 |  |  |  |  | -52.86 |  |  |  |  |
| Time offset to Cell1 Note 4 |  | ms | 3 |  |  |  |  | 3 |  |  |  |  |
| Time offset to Cell2 Note 5 |  | s | - |  |  |  |  | 3 |  |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. OCNG is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Receive time difference of signals received between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell at the UE antenna connector including time alignment error between the two cells Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 8: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.10.3.2.1.2	Test Requirements

The UE shall meet the interruption requirements for SCell addition on both the victim PSCC in clause 8.2.1 and the vicitim LTE PCell in clause 7.32 of [15] during time T1

The UE shall meet the interruption requirements for SCell activation on both the victim PSCell in clause 8.2.1 and the vicitim LTE PCell in clause 7.32 of [15] during time T2. There shall be a single interruption with time window as specified in clause 8.3A.2

The UE shall meet the interruption requirements for SCell deactivation on both the victim PSCell in clause 8.2.1 and the vicitim LTE PCell in clause 7.32 of [15] during time T3. There shall be a single interruption with time window as specified in clause 8.3A,3

The UE shall meet the interruption requirements for deactivated SCell measurements on both the victim PSCell in clause 8.2.1 and the vicitim LTE PCell in clause 7.32 of [15] during time T43. The interruptions shall be within the time window as specified in clause 8.3A,3

The UE shall meet the interruption requirements for SCell release on both the victim PSCell in clause 8.2.1 and the vicitim LTE PCell in clause 7.32 of [15] during time T5.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.10.3.3	SCell activation and deactivation delay

#### A.10.3.3.1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle

##### A.10.3.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation delays for NR SCell, with NR PSCell and NR SCell both under CCA, are within the requirements stated in clause 8.3A, when the SCell is known by the UE at the time of activation and the configured SCell measurement cycle is 160 ms.

The supported test configurations are shown in Table A.10.3.3.1.1-1.

The test parameters are given in Table A.10.3.3.1.1-2 and cell-specific parameters for NR cells are provided in Table A.10.3.3.1.1-3 below. Cell-specific parameters for EUTRA PCell are provided in clause A.3.7.2.1.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell: Cell 1 (PCell) on radio channel 1 (PCC) in E-UTRA, Cell 2 (PSCell) on radio channel 2 (PSCC) in NR, and Cell3 (SCell) on radio channel 3 (SCC) in NR. Before the test starts the UE is connected to Cell 1 and Cell 2, but is not aware of Cell 3, as the UE is only monitoring PCC and PSCC. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 3) becomes configured on radio channel 2. The UE now starts monitoring the SCC. At the end of T1, the test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. The UE shall be able to report a valid CSI in PSCell for the activated SCell at latest in slot m +  (THARQ+ Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, as defined in clause 8.3A.2. The UE shall start reporting CSI in PSCell in first available uplink resource for CSI reporting following slot m+ THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption shall fall within the time window specified in clause 8.3A.2.

The point in time at which the MAC message is received by at the UE antenna connector, in a slot # denoted n, defines the start of time period T3. The UE shall complete the activation at latest in slot n+THARQ+3msNR slot length. Any PSCell interruption shall fall within the time window specified in clause 8.3A.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PSCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received, while taking into account CCA failures on SCC.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.10.3.3.1.1-1: Supported test configurations for SCell Activation and Deactivation of known NR SCell with NR PSCell and SCell under CCA, 160 ms SCell measurement cycle

| Configuration | Description |
| --- | --- |
| 1 | PCC: LTE FDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | PCC: LTE TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.3.3.1.1-2: General test parameters for known SCell activation case with NR PSCell and SCell under CCA, 160 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3 | Three radio channels (1, 2, 3) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on E-UTRAN RF channel number 1. |
| Active PSCell |  | Cell 2 | Primary secondary cell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on NR RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 3 timing offset to Cell 2 | s | 0 |  |
| Time alignment error between Cell 3 and Cell 2 | s | TAE as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time PCell and PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 10+5⋅2µDL | The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] µDL is the subcarrier spacing configuration for DL |

Table A.10.3.3.1.1-3: Cell specific test parameters for known SCell activation case with NR PSCell and SCell under CCA, 160 ms SCell measurement cycle

| Parameter |  | Unit | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1,2 |  | TDD |  |  | TDD |  |  |
| TDD configuration | Config 1,2 |  | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |  |
| BWchannel | Config 1,2 | MHz | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |  | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |  | As specified in clause A.3.26.2.2 |  |  |
| DL CCA probability for semi-static channel accessNote5,7 | PCCA_DL |  | 0.9375 |  |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel accessNote6,7 | PCCA_DL_1 |  | 0.75 |  |  | 0.75 |  |  |
|  | PCCA_DL_2 |  | 0.75 |  |  | 0.75 |  |  |
| UL CCA probability for semi-static channel access | PCCA_UL |  | 0.87 |  |  | 0.87 |  |  |
| UL CCA probability for dynamic channel access | PCCA_UL |  | 0.75 |  |  | 0.75 |  |  |
| LCCA_DL Note 8 |  |  | 2 |  |  | 2 |  |  |
| WCCA_DL Note 8 |  |  | Tactivation_time_withCCA |  |  | Tactivation_time_withCCA |  |  |
| Initial downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Initial uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Dedicated downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Dedicated uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1,2 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | Config 1,2 |  | SR.1.1 CCA |  |  | SR.1.1 CCA |  |  |
| Dedicated CORESET parameters | Config 1,2 |  | CCR.1.3 CCA |  |  | CCR.1.3 CCA |  |  |
| RMSI CORESET parameters | Config 1,2 |  | CR.1.1 CCA |  |  | CR.1.1 CCA |  |  |
| OCNG Patterns Note1 |  |  | OP.1 |  |  | OP.1 |  |  |
| SSB Configuration for semi-static channel accessNote5,7 | Config 1,2 |  | SSB.1 CCA |  |  | SSB.1 CCA |  |  |
| SSB Configuration for dynamic channel accessNote6,7 | Config 1,2 |  | SSB.2 CCA |  |  | SSB.2 CCA |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  | SMTC.1 |  |  |
| DBT window configuration |  |  | DBT.1 |  |  | DBT.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 | dBm/15kHz | -104 |  |  | -104 |  |  |
| Noc Note2 | Config 1,2 | dBm/SCS | -101 |  |  | -101 |  |  |
| Ês/Iot |  | dB | 17 |  |  | 17 |  |  |
| Ês/Noc |  | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -84 |  |  | -84 |  |  |
| IoNote3 | Config 1,2 | dBm/ 38.16MHz | -52.87 |  |  | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that resources in the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. OCNG is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 8: As specified in clause 8.3A for L1,max, L2,1,max, L2,2,max, L3,1,max, and L3,2,max |  |  |  |  |  |  |  |  |

##### A.10.3.3.1.2	Test Requirements

During T2, starting from the slot specified in clause 4.3 of TS 38.213 [3] and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

During T2, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in first available uplink resource for CSI reporting following slot m + (THARQ+ Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB + L1*Trs + 5ms, as specified in clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in clause 8.3A.3.

During T2, interruption on PSCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB.

During T3, interruption on PSCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PSCell shall not be more than specified for EN-DC in clause 8.2.1.2.4.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

#### A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 320 ms SCell measurement cycle

##### A.10.3.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation delays for NR SCell, with NR PSCell and NR SCell both under CCA, are within the requirements stated in clause 8.3A, when the SCell is known by the UE at the time of activation and the configured SCell measurement cycle is 320 ms.

The supported test configurations are same as in Table A.10.3.3.1.1-1 above.

The test parameters are same as in Table A.10.3.3.1.1-2 above, except for parameters listed below in Table A.10.3.3.2.1-1. The cell-specific parameters are same as in Table A.10.3.3.1.1-3 above.

The test execution is the same as described in clause A.10.3.3.1 above.

Table A.10.3.3.2.1-1: General test parameters for known NR SCell activation with NR PSCell and SCell under CCA, 320 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |

##### A.10.3.3.2.2	Test Requirements

During T2, starting from the slot specified in clause 4.3 of TS 38.213 [3] and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

During T2, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in first available uplink resource for CSI reporting following slot m + (THARQ+ Tactivation_time_withCCA + TCSI_reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB_MAX + L2,1*TSMTC_MAX + (1 +L2,2)*Trs + 5ms, as specified in clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in clause 8.3A.3.

During T2, interruption on PSCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB_MAX + L2,1* TSMTC_MAX.

During T3, interruption on PSCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PSCell shall not be more than specified for EN-DC in clause 8.2.1.2.4.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

#### A.10.3.3.3 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA

##### A.10.3.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation delays for NR SCell, with NR PSCell and NR SCell both under CCA, are within the requirements stated in clause 8.3A, when the SCell is unknown to the UE at the time of activation.

The supported test configurations are same as in Table A.10.3.3.1.1-1 above.

The test parameters are same as in Table A.10.3.3.1.1-2 above, except for parameters listed below in Table A.10.3.3.3.1-1. The cell-specific parameters are same as in Table A.10.3.3.1.1-3 above.

The test execution is the same as described in clause A.10.3.3.1 above.

Table A.10.3.3.3.1-1: General test parameters for unknown NR SCell activation with NR PSCell and SCell under CCA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | s | 0.1 | During this time period PCell and PSCell shall be known and the SCell configured, but not detected. |

##### A.10.3.3.3.2	Test Requirements

During T2, starting from the slot specified in clause 4.3 of TS 38.213 [3] and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

During T2, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in first available uplink resource for CSI reporting following slot m + (THARQ+ Tactivation_time_withCCA + TCSI_reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB_MAX + (1 + L3,1)*TSMTC_MAX + (2 + L3,2)*Trs + 5ms, as specified in clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in clause 8.3A.3.

During T2, interruption on PSCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB_MAX + L3,1* TSMTC_MAX.

During T3, interruption on PSCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PSCell shall not be more than specified for EN-DC in clause 8.2.1.2.4.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

### A.10.3.4	Beam failure detection and link recovery procedures

#### A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode

##### A.10.3.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5A.

The test parameters are given in Tables A.10.3.4.1.1-1, A.10.3.4.1.1-2, A.10.3.4.1.1-3 and A.10.3.4.1.1-4 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell which operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.10.3.4.1.1-1 shows the variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure A.10.3.4.1.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. The UE transmits the reporting according to UL CCA model. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40 ms) in test 1.

Table A.10.3.4.1.1-1: Supported test configurations for FR1 PSCell with CCA

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.10.3.4.1.1-2: General test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 | Test 2 |  |
| Active E-UTRA PCell |  |  |  |  |  | Cell 1 | Cell 1 |  |
| E-UTRA RF Channel Number |  |  |  |  |  | 1 | 1 |  |
| Active PSCell |  |  |  |  |  | Cell 2 | Cell 2 |  |
| RF Channel Number |  |  |  |  |  | 2 | 2 |  |
| DL CCA model |  |  |  |  |  | As specifieed in A.3.20.2.1 | As specifieed in A.3.20.2.1 |  |
| UL CCA model |  |  |  |  |  | As specified in A.3.20.2.2 | As specified in A.3.20.2.2 |  |
| Duplex mode |  |  |  | Config 1, 2 |  | TDD | TDD |  |
| BWchannel |  |  |  | Config 1, 2 | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  |  | Config 1, 2 |  | DLBWP.0.1 | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  |  | Config 1, 2 |  | DLBWP.1.1 | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  |  | Config 1, 2 |  | ULBWP.0.1 | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  |  | Config 1, 2 |  | ULBWP.1.1 | ULBWP.1.1 |  |
| TDD configuration |  |  |  | Config 1, 2 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |  |
| CORESET Reference Channel |  |  |  | Config 1, 2 |  | CR.1.1 CCA | CR.1.1 CCA |  |
| SSB Configuration |  |  |  | Config 1, 2 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |  |
| DBT Window Configuration |  |  |  | Config 1, 2 |  | DBT.1 | DBT.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1, 2 |  | 30 KHz | 30 KHz |  |
| PRACH Configuration |  |  |  | Config 1, 2 |  | Table A.3.8.2.2-1 | Table A.3.8.2.2-1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  | 0 | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  | 1 | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 | OP.1 |  |
| CP length |  |  |  |  |  | Normal | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low | 2x2 Low |  |
| Beam failure |  | DCI format |  |  |  | 1-0 | 1-0 |  |
| detection transmission parameters |  | Number of Control OFDM symbols |  |  |  | 2 | 2 |  |
|  |  | Aggregation level |  |  | CCE | 8 | [2] |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size | REG bundle size |  |
|  |  | REG bundle size |  |  |  | 6 | 6 |  |
| DRX |  |  |  |  |  | OFF | OFF |  |
| Gap pattern ID |  |  |  |  |  | gp0 | gp0 |  |
| gapOffset |  |  |  |  |  | 0 | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1, 2 |  |  |  | dBm/SCS kHz | -95 | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  | db0 | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | n1 | see TS 38.321 [7], clause 5.17 |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | pbfd4 | see TS 38.321 [7], clause 5.17 |
| CSI-RS configuration for CSI reporting |  |  | Config 1, 2 |  |  | CSI-RS.2.1 TDD | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  |  | Config 1, 2 |  |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  |  | 0,1 | 0,1 |  |
| T310 timer |  |  |  |  | ms | [1000] | [1000] |  |
| N310 |  |  |  |  |  | [2] | [2] |  |
| T1 |  |  |  |  | s | [0.2] | [0.2] | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | [0.93] | [0.85] |  |
| T3 |  |  |  |  | s | [0.52] | [0.44] |  |
| T4 |  |  |  |  | s | [0] | [0] |  |
| T5 |  |  |  |  | s | [0.45] | [0.45] |  |
| D1 |  |  |  |  | s | [0.41] | [0.41] |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |  |  |

Table A.10.3.4.1.1-3: Cell specific test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA,DL | Note 10, 12 |  |  | [1.0] | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
|  | Note 11, 12 |  |  | [1.0]/[1.0] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA,UL |  |  |  | [1.0] | [1.0] | [1.0] | [1.0] | [1.0] |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 |  | Config 1, 2 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 |  | Config 1, 2 | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 |  | Config 1, 2 | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  |  | Config 1, 2 | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause [A.3.6A]. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |

Table A.10.3.4.1.1-4: Cell specific test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  | Unit | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA.DL |  | Note 10, 12 |  | [1.0] | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
|  |  | Note 11, 12 |  | [1.0]/[1.0] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA.UL |  |  |  | [1.0] | [1.0] | [1.0] | [1.0] | [1.0] |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1, 2 |  | dB | 5 | [2] | [-4] | [-4] | [-4] |
| SNR_SSB of set q1 | Config 1, 2 |  | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1, 2 |  | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  | Config 1, 2 |  | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause [A.3.6A]. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |

Figure A.10.3.4.1.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.10.3.4.1.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [410] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

In Test 1, the UE is verified to meet the beam failure detection for BFD-RS SSB Es/Iot < -7 dB.

In Test 2, the UE is verified to meet the beam failure detection for BFD-RS SSB Es/Iot ≥ -7 dB.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode

##### A.10.3.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving PSCell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5A.

The test parameters are given in Tables A.10.3.4.2.1-1, A.10.3.4.2.1-2, A.4.5.5.2.1-3 and A.10.3.4.2.1-4 below. There are two cells, cell 1 is the E-UTRAN PCell, and cell 2 is the PSCell which operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.10.3.4.2.1-1 shows the variation of the downlink SNR of the PCell and the SNR of the SSB in set q0 in the active PSCell to emulate SSB based beam failure. Figure A.10.3.4.2.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. The UE transmits the reporting according to UL CCA model. In the test, DRX configuration is enabled in PSCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.10.3.4.2.1-1: Supported test configurations for FR1 PSCell with CCA

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.10.3.4.2.1-2: General test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 | Test 2 |  |
| Active E-UTRA PCell |  |  |  |  |  | Cell 1 | Cell 1 |  |
| E-UTRA RF Channel Number |  |  |  |  |  | 1 | 1 |  |
| Active PSCell |  |  |  |  |  | Cell 2 | Cell 2 |  |
| RF Channel Number |  |  |  |  |  | 2 | 2 |  |
| DL CCA model |  |  |  |  |  | As specified in A.3.26.2.1 | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  |  |  | As specified in A.3.26.2.2 | As specified in A.3.26.2.2 |  |
| Duplex mode |  |  |  | Config 1, 2 |  | TDD | TDD |  |
| BWchannel |  |  |  | Config 1, 2 | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  |  | Config 1, 2 |  | DLBWP.0.1 | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  |  | Config 1, 2 |  | DLBWP.1.1 | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  |  | Config 1, 2 |  | ULBWP.0.1 | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  |  | Config 1, 2 |  | ULBWP.1.1 | ULBWP.1.1 |  |
| TDD configuration |  |  |  | Config 1, 2 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |  |
| CORESET Reference Channel |  |  |  | Config 1, 2 |  | CR.1.1 CCA | CR.1.1 CCA |  |
| SSB Configuration |  |  |  | Config 1, 2 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |  |
| DBT Window Configuration |  |  |  | Config 1, 2 |  | DBT.1 | DBT.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1, 2 |  | 30 KHz | 30 KHz |  |
| PRACH Configuration |  |  |  | Config 1, 2 |  | Table A.3.8.2.2-1 | Table A.3.8.2.2-1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  | 0 | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  | 1 | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 | OP.1 |  |
| CP length |  |  |  |  |  | Normal | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low | 2x2 Low |  |
| Beam failure |  | DCI format |  |  |  | 1-0 | 1-0 |  |
| detection transmission parameters |  | Number of Control OFDM symbols |  |  |  | 2 | 2 |  |
|  |  | Aggregation level |  |  | CCE | 8 | [2] |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size | REG bundle size |  |
|  |  | REG bundle size |  |  |  | 6 | 6 |  |
| DRX |  |  |  |  |  | DRX.7 | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  |  |  |  | N.A. | N.A. |  |
| gapOffset |  |  |  |  |  | 0 | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1, 2 |  |  |  | dBm/SCS kHz | -95 | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  | db0 | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | n1 | see TS 38.321 [7], clause 5.17 |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | pbfd4 | see TS 38.321 [7], clause 5.17 |
| CSI-RS configuration for CSI reporting |  |  | Config 1, 2 |  |  | CSI-RS.2.1 TDD | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  |  | Config 1, 2 |  |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  |  | 0,1 | 0,1 |  |
| T310 timer |  |  |  |  | ms | [1000] | [1000] |  |
| N310 |  |  |  |  |  | [2] | [2] |  |
| T1 |  |  |  |  | s | [1] | [1] | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | [9.01] | [8.37] |  |
| T3 |  |  |  |  | s | [5.16] | [4.52] |  |
| T4 |  |  |  |  | s | [0] | [0] |  |
| T5 |  |  |  |  | s | [3.89] | [3.39] |  |
| D1 |  |  |  |  | s | [3.85] | [3.85] |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |  |  |

Table A.10.3.4.2.1-3: Cell specific test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA_DL | Semi-static channel access PCCA_DLNote 10, 12 |  |  | 1.0 | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
|  | Dynamic channel access, PCCA_DL_1/PCCA_DL_2 Note 11, 12 |  |  | 1.0/1.0 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 |
| UL CCA probability PCCA,UL |  |  |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| LCCA_DL |  |  |  | N/A | 3 |  |  |  |
| WCCA_DL |  |  | ms | N/A | TEvaluate_CBD_SSB_CCA Note 13 |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 |  | Config 1, 2 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 |  | Config 1, 2 | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 |  | Config 1, 2 | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  |  | Config 1, 2 | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6A. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 13: As defined in Table 8.5A.5.2-1, where LCBD,max=3 for TDRX > 320. |  |  |  |  |  |  |  |  |

Table A.10.3.4.2.1-4: Cell specific test parameters for FR1 PSCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  | Unit | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA _DL |  | Semi-static channel access PCCA_DLNote 10, 12 |  | 1.0 | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
|  |  | Dynamic channel access, PCCA_DL_1/PCCA_DL_2 Note 11, 12 |  | 1.0/1.0 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 |
| UL CCA probability PCCA,UL |  |  |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| LCCA_DL |  |  |  | N/A | 3 |  |  |  |
| WCCA_DL |  |  | ms | N/A | TEvaluate_CBD_SSB_CCA Note 13 |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1, 2 |  | dB | 5 | [2] | [-4] | [-4] | [-4] |
| SNR_SSB of set q1 | Config 1, 2 |  | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1, 2 |  | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  | Config 1, 2 |  | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6A. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 13: As defined in Table 8.5A.5.2-1, where LCBD,max=3 for TDRX > 320. |  |  |  |  |  |  |  |  |

Figure A.10.3.4.2.1-1: SNR and L1-RSRP variation for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.10.3.4.2.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [3850] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

In Test 1, the UE is verified to meet the beam failure detection for BFD-RS SSB Es/Iot < -7 dB.

In Test 2, the UE is verified to meet the beam failure detection for BFD-RS SSB Es/Iot ≥ -7 dB.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

### A.10.3.5	Active BWP switching

#### A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC

##### A.10.3.5.1.1	Test Purpose and Environment

The purpose of this test is to verify the UL BWP switch delay requirement defined in clause 8.6.4.

The supported test configurations are shown in Table A.10.3.5.1.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1), and one NR PSCell (Cell 2) as given in A.10.3.5.1.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell is specified in Table A.10.3.5.1.1-2. SRS configuration used in the test is specified in Table A.10.3.5.1.1-4.

The UE shall be configured with PRACH configuration on UL BWP on which the UE shall switch after the consistent UL LBT failure detection.

Before the test starts,

- UE is connected to Cell 1 on radio channel 1 and Cell 2 on radio channel 2.

- UE is configured with 2 different UE-specific downlink and uplink bandwidth parts on Cell 2: DL BWP-1, DL BWP-2, UL BWP-1 and UL BWP-2 before starting the test. DL BWP-1 and DL BWP-2 always include bandwidth of the initial DL BWP and SSB. UL BWP-1 and UL BWP-2 always include bandwidth of the SRS.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is DL BWP-1.

- UE is indicated in firstActiveUplinkBWP-Id that the active UL BWP is UL BWP-1.

- UE is configured with LBT-FailureRecoveryConfig parameters for Cell 2.

The cell has constant signal levels throughout the test. The test consists of 2 successive time periods, with durations of T1 and T2, respectively.

During T1,

- Time period T1 starts when the UE has received the SRS configuration for periodic SRS transmission on active UL BWP-1.

- The UE shall perform UL CCA before SRS transmission.

- The parameter UL CCA probability PCCA is set to 0 during T1. This requires the test system to set energy level above the detection level during portion of the UL slot where the UE performs UL CCA. This in turn forces the UE to fail the UL CCA. The UE consistently fails UL CCA during T1 and is therefore unable to transmit SRS.

During T2,

- T2 starts when the UE detects consistent UL LBT failures i.e. when total number of UL LBT failures in Cell 2 on active UL BWP-1 exceeds lbt-FailureInstanceMaxCount during lbt-FailureDetectionTimer.

- The UE upon detected consistent UL LBT failure starts the LBT recovery mechanism, which requires the UE to switch to active UL BWP-2 in Cell 2 and to send PRACH in the active UL BWP-2.

- Staring from T2, the UE shall be able to send PRACH in the active UL BWP-2 within the delay specified in clause 8.6.4.

Table A.10.3.5.1.1-1: Supported test configurations for UL BWP switch test in EN-DC

| Config | Description |
| --- | --- |
| 1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.10.3.5.1.1-2: General test parameters for UL BWP switch in EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| lbt-FailureDetectionTimer [2] | ms | 80 | Parameter configured by IE: LBT-FailureRecoveryConfig [1] |
| lbt-FailureInstanceMaxCount [2] |  | 4 | Parameter configured by IE: LBT-FailureRecoveryConfig [1] |
| T1 | s | 0.1 | During T1 consistent LBT failure is detected on active UL BWP-1 |
| T2 | s | 0.1 | During T2 UE sends PRACH on active UL BWP-2 |

Table A.10.3.5.1.1-3: NR Cell specific test parameters for UL BWP switch test in EN-DC

| Parameter |  |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| TDD configuration |  | Config 1, 2 |  | TDDConf.1.1 CCA |  |
| BWchannel |  | Config 1, 2 |  | 40 MHz: NRB,c = 106 |  |
| DL CCA model |  | Config 1, 2 |  | As specified in clause A.3.20.2.1 |  |
| UL CCA model |  | Config 1, 2 |  | As specified in clause A.3.20.2.2 |  |
| Active BWP ID |  | Config 1, 2 |  | 1, 2 |  |
| Initial DL BWP Configuration |  | Config 1, 2 |  | DLBWP.0.2 Note 4 |  |
| Active DL BWP-1 Configuration |  | Config 1, 2 |  | DLBWP.1.1 Note 4 |  |
| Active DL BWP-2 Configuration |  | Config 1, 2 |  | DLBWP.1.3 Note 4 |  |
| Initial UL BWP Configuration |  | Config 1, 2 |  | ULBWP.0.2 Note 4 |  |
| Active UL BWP-1 Configuration |  | Config 1, 2 |  | ULBWP.1.1 Note 4 |  |
| Active UL BWP-2 Configuration |  | Config 1, 2 |  | ULBWP.1.3 Note 4 |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  | SR.1.1 CCA |  |
| RMSI CORESET parameters |  | Config 1, 2 |  | CR.1.1 CCA |  |
| Dedicated CORESET parameters |  | Config 1, 2 |  | CCR.1.1 CCA |  |
| OCNG Patterns |  | Config 1, 2 |  | OP.1 |  |
| SSB Configuration | Semi- static channel acces | Config 1, 2 |  | SSB.1 CCA |  |
|  | Dymamic channel acces | Config 1, 2 |  | SSB.2 CCA |  |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 FR1 |  |
| Correlation Matrix and Antenna Configuration |  | Config 1, 2 |  | 1x2 Low |  |
| TRS Configuration |  | Config 1, 2 |  | TRS.1.2 TDD |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1, 2 |  | 1 | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  | Config 1, 2 |  | 1 | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  | Config 1, 2 |  | 1 | 1 |
| UL CCA probability (PCCA_UL) |  | Config 1, 2 |  | 0 | 1 |
| PRACH configuration |  | Config 1, 2 |  | N/A | Configuration #1 in Table A.3.8.2.1-1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1, 2 | dBm/SCS | -101 |  |
| SS-RSRP Note 3 |  | Config 1, 2 | dBm/SCS | -84 |  |
| Ês/Iot |  | Config 1, 2 | dB | 17 |  |
| Ês/Noc |  | Config 1, 2 | dB | 17 |  |
| IoNote3 |  | Config 1, 2 | dBm/ 38.16MHz | -52.86 |  |
| Propagation Condition |  |  |  | AWGN |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.20.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

Table A.10.3.5.1.1-4: Sounding Reference Symbol Configuration for UL BWP Switch Test in EN-DC

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 24 | Frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=4 for SCS 30kHz | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | Codebook | Codebook based UL transmission |
| startPosition | 0 | resourceMapping setting: SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

##### A.10.3.5.1.2	Test Requirements

The UE capable of bwp-SwitchingDelay type1 [2] shall start to transmit the PRACH on active UL BWP-2 of Cell 2 (PSCell) less than 21.5 ms from the beginning of time period T1.

The UE capable of bwp-SwitchingDelay type2 [2] shall start to transmit the PRACH on active UL BWP-2 of Cell 2 (PSCell) less than 23 ms from the beginning of time period T1.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE: The above delay is calculated as follows:’

The active UL BWP switch delay from UL BWP-1 to UL BWP-2 can be expressed as:

TBWPswitchDelay*Tslot +1*Tslot + (1+ L3)*TSSB,RO + 10 ms

Where:

TBWPswitchDelay = 1 ms (2 slots) and 2.5 ms (5 slots) for bwp-SwitchingDelay [2] type1 and type2 UE capabilities according to clause 8.6.4.

Tslot = It is the slot length. It is 0.5 ms for 30 kHz.

L3 = It is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure. L3= 0 during T2 since PCCA = 1.

TSSB,RO = 10 ms according to FR1 PRACH configuration 1.

This gives a total of 21.5 ms and 23 ms for type1 and type2 UE respectively.

#### A.10.3.5.2	DCI-based and Timer-based Active BWP Switch

##### A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC

A.10.3.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS38.133 clause 8.6, and interruption requirement for E-UTRA victim cell defined in TS36.133 clause 7.32.2.7. Supported test configurations are shown in Table A.10.3.5.2.1.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1), and one NR PSCell (Cell 2) as given in Table A.10.3.5.2.1.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell is specified in Table A.10.3.5.2.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

PDCCHs indicating new transmissions shall be sent continuously on PSCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PSCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PSCell.

-	UE is configured with a bwp-InactivityTimer timer value for PSCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PSCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PSCell no later than at the beginning of the DL slot right after DL slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PSCell’s BWP-2 starting from the beginning of the DL slot right after DL slot (i+TBWPswitchDelay).

The starting time of PCell(Cell 1) interruption due to BWP switch on PSCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PSCell(Cell 2).

During T3,

The time period T3 starts from the slot #j, where j is the beginning slot of the DL subframe immediately after the bwp-InactivityTimer timer expires. The UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PSCell at latest at the beginning of the DL slot right after DL slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PSCell’s BWP-1 starting from the beginning of the DL slot right after DL slot (j+TBWPswitchDelay).

The starting time of PCell(Cell 1) interruption due to BWP switch of PSCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in PSCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK is received.

The test equipment verifies that potential interruption to E-UTRA PCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell during BWP switch of PSCell, respectively.

Table A.10.3.5.2.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: A UE which fulfils the requirements in test case A.10.3.5.2.2 can skip the test cases in A.10.3.5.2.1. Note 3: The UE supporting EN-DC with only NR band(s) with shared spectrum access is required to be test. |  |

Table A.10.3.5.2.1.1-2: General test parameters for DL BWP switch in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| DL CCA model |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  | As specified in clause A.3.26.2.2 |  |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.10.3.5.2.1.1-3.: NR Cell specific test parameters for DL BWP switch in synchronous EN-DC

| Parameter |  |  | Unit | Cell 2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1,2 |  | TDD |
| TDD configuration |  | Config 1,2 |  | TDDConf.1.1 CCA |
| BWchannel |  | Config 1,2 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  | Config 1,2 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  | Config 1,2 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  | Config 1,2 |  | DLBWP.1.3 Note 4 |
| Initial UL BWP Configuration |  | Config 1,2 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  | Config 1,2 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  | Config 1,2 |  | ULBWP.1.3 Note 4 |
| PDSCH Reference measurement channel |  | Config 1,2 |  | SR.1.1 CCA |
| RMSI CORESET parameters |  | Config 1,2 |  | CR.1.1 CCA |
| Dedicated CORESET parameters |  | Config 1,2 |  | CCR.1.1 CCA |
| OCNG Patterns |  | Config 1,2 |  | OP.1 |
| SSB Configuration | Semi- static channel acces | Config 1,2 |  | SSB.1 CCA |
|  | Dymamic channel acces | Config 1,2 |  | SSB.2 CCA |
| SMTC Configuration |  | Config 1,2 |  | SMTC.1 |
| TRS Configuration |  | Config 1,2 |  | TRS.1.2 TDD |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1,2 |  | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  | Config 1,2 |  | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  | Config 1,2 |  | 1 |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1,2 |  | 1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | Config 1,2 | dBm/SCS | -101 |
| SS-RSRP Note 3 |  | Config 1,2 | dBm/SCS | -84 |
| Ês/Iot |  | Config 1,2 | dB | 17 |
| Ês/Noc |  | Config 1,2 | dB | 17 |
| IoNote3 |  | Config 1,2 | dBm/38.16MHz | -59 |
| Propagation Condition |  |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.26.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |

A.10.3.5.2.1.2	Test Requirements

During T1, the UE shall start to send the ACK for PSCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK for PSCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1, the start time of PCell interruption during PSCell active BWP switch shall not happen outside the BWP switch delay.

During T3, the start time of PCell interruption of during PSCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in TS36.133 Clause 7.32.2.7.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK in the DL slot right after DL slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK.

##### A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC

A.10.3.5.2.2.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirements for NR victim cell defined in clause 8.2.1.2.7 and interruption requirement for E-UTRA victim cell defined in clause 7.32.2.7 of TS 36.133 [15]. Supported test configurations are shown in Table A.10.3.5.2.2.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1), one NR PSCell (Cell 2) and one NR SCell (Cell 3) as given in Table A.10.3.5.2.2.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell and SCell are specified in Table A.10.3.5.2.2.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) and SCell (Cell 3) to ensure that the UE will have ACK/NACK sending.

PDCCHs indicating new transmissions shall be sent continuously on PSCell (Cell 2) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 2 and the time duration of T2.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 2 (PSCell) on radio channel 2 (PSCC) and Cell 3 (SCell) on radio channel 3 (SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PSCell, BWP-1 and BWP-2, in Cell 2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for SCell, BWP-0 in Cell 3 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PSCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell.

-	UE is configured with a bwp-InactivityTimer timer value for PSCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PSCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PSCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PSCell no later than at the beginning of the DL slot right after slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PSCell’s BWP-2 starting from the beginning of the DL slot right after slot (i+TBWPswitchDelay).

PCell(Cell 1) interruption due to BWP switch on PSCell shall occur within the BWP switch delay.

SCell(Cell 3) interruption due to BWP switch on PSCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PSCell(Cell 2).

During T3,

The time period T3 starts from the slot #j, where j is the beginning slot of the DL subframe immediately after the slot wherein bwp-InactivityTimer timer expires. The UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PSCell at latest at the beginning of the DL slot right after slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PSCell’s BWP-1 starting from the beginning of the DL slot right after slot (j+TBWPswitchDelay).

PCell(Cell 1) interruption due to BWP switch of PSCell shall occur within the BWP switch delay.

SCell(Cell 3) interruption due to BWP switch of PSCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in PSCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK is received.

The test equipment verifies that potential interruption to E-UTRA PCell and NR SCell is carried out in the correct time span by monitoring ACK/NACK sent in PCell and SCell during BWP switch of PSCell, respectively.

Table A.10.3.5.2.2.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: A UE which fulfils the requirements in test case A.10.3.5.2.2 can skip the test cases in A.10.3.5.2.1. Note 3: NR configuration is the same for PSCell and SCells. Note 4: The UE supporting EN-DC with only NR band(s) with shared spectrum access is required to be tested. |  |

Table A.10.3.5.2.2.1-2: General test parameters for DL BWP switch in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2, 3 | Two NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| Active SCell |  | Cell 3 | SCell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DL CCA model |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  | As specified in clause A.3.26.2.2 |  |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| Cell3 timing offset to cell2 | s | 3 | Synchronous cells |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.10.3.5.2.2.1-3: NR Cell specific test parameters for DL BWP switch in synchronous EN-DC

| Parameter |  |  | Unit | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |  |
| Duplex mode |  | Config 1,2 |  | TDD |  |
| TDD configuration |  | Config 1,2 |  | TDDConf.1.1 CCA |  |
| BWchannel |  | Config 1,2 |  | 40 MHz: NRB,c = 106 |  |
| Active BWP ID |  |  |  | 1, 2 | 0 |
| Initial BWP Configuration |  | Config 1,2 |  | DLBWP.0.2 | DLBWP.0.2 |
| Active BWP-0 Configuration |  | Config 1,2 |  | NA | DLBWP.0.2 |
| Active BWP-1 Configuration |  | Config 1,2 |  | DLBWP.1.3 | NA |
| Active BWP-2 Configuration |  | Config 1,2 |  | DLBWP.1.1 | NA |
| PDSCH Reference measurement channel |  | Config 1,2 |  | SR.1.1 CCA |  |
| RMSI CORESET parameters |  | Config 1,2 |  | CR.1.1 CCA |  |
| Dedicated CORESET parameters |  | Config 1,2 |  | CCR.1.1 CCA |  |
| OCNG Patterns |  | Config 1,2 |  | OP.1 |  |
| SSB Configuration | Semi- static channel acces | Config 1,2 |  | SSB.1 CCA |  |
|  | Dymamic channel acces | Config 1,2 |  | SSB.2 CCA |  |
| SMTC Configuration |  | Config 1,2 |  | SMTC.1 |  |
| TRS Configuration |  | Config 1,2 |  | TRS.1.2 TDD |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1,2 |  | 1 | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  | Config 1,2 |  | 1 | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  | Config 1,2 |  | 1 | 1 |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1,2 |  | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 |  |
| Propagation Condition |  |  |  | AWGN |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 | dBm/SCS kHz | -101 | -101 |
| SS-RSRP Note 3 |  | Config 1,2 | dBm/SCS kHz | -84 | -84 |
| Ês/Iot |  | Config 1,2 | dB | 17 | 17 |
| Ês/Noc |  | Config 1,2 | dB | 17 | 17 |
| IoNote3 |  | Config 1,2 | dBm/38.16MHz | -59 | -59 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.26.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

A.10.3.5.2.2.2	Test Requirements

During T1, the UE shall start to send the ACK for PSCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK for PSCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1, the start of the interruption of PCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

During T3, the start of the interruption of PCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of PCell shall not be longer than the interruption duration specified for active BWP switch in clause 7.32.2.7 of TS 36.133 [15].

During T1, the start of the interruption of SCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

During T3, the start of the interruption of SCell during PSCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.6.2.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK in the DL slot right after slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK.

Editor’s note: FFS value of k1 for type 1 and type 2 UE.

#### A.10.3.5.3	RRC-based Active BWP Switch

##### A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC

A.10.3.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in clause 8.6.3. Supported test configurations are shown in Table A.10.3.5.3.1.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1) and one NR PSCell (Cell 2) as given in Table A.10.3.5.3.1.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell are specified in Table A.10.3.5.3.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and to Cell 2 (PSCell) on radio channel 2 (PSCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1 (PSCell).

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in PSCell.

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated bandwidth part configuration, sent from the test equipment to the UE, is completely received at the UE side in PSCell’s slot # denoted i. The UE shall reconfigure its bandwidth part with the updated bandwidth part BWP-1 of final condition.

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC) as defined in clause 8.6.3 and be ready for the reception of uplink grant for the PSCell no later than at the beginning of the DL slot right after slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC). The UE shall be continuously scheduled on PSCell’s BWP-1 starting from the beginning of the DL slot right after slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC).

TRRCprocessingDelay and TBWPswitchDelayRRC are defined in clause 8.6.3.

The test equipment verifies the DL BWP switch time in PSCell by counting the time from the time when the RRC Reconfiguration message including updated BWP configurationis sent till the time when RRC Reconfiguration Complete message is received.

Table A.10.3.5.3.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD, With CCA: NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE supporting EN-DC with only NR band(s) with shared spectrum access is required to be tested. |  |

Table A.10.3.5.3.1.1-2: General test parameters for DL BWP switch in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| DL CCA model |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  | As specified in clause A.3.26.2.2 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| T1 | s | 0.2 |  |

Table A.10.3.5.3.1.1-3: NR Cell specific test parameters for DL BWP switch in synchronous EN-DC

| Parameter |  |  |  | Unit | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1,2 |  | TDD |
| TDD configuration |  |  | Config 1,2 |  | TDDConf.1.1 CCA |
| BWchannel |  |  | Config 1,2 |  | 40 MHz: NRB,c = 106 |
| Active DL BWP ID |  |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | Config 1,2 |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  |  | Config 1,2 |  | ULBWP.0.2 |
| Initial Condition |  | Active DL BWP-1 Configuration | Config 1,2 |  | DLBWP.1.3 |
| Final Condition |  | Active DL BWP-1 Configuration | Config 1,2 |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  |  | Config 1,2 |  | ULBWP.0.2 |
| Active UL BWP-1 Configuration |  |  | Config 1,2 |  | ULBWP.1.3 |
| Active UL BWP-2 Configuration |  |  | Config 1,2 |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  |  | Config 1,2 |  | SR.1.1 CCA |
| RMSI CORESET parameters |  |  | Config 1,2 |  | CR.1.1 CCA |
| Dedicated CORESET parameters |  |  | Config 1,2 |  | CCR.1.1 CCA |
| OCNG Patterns |  |  | Config 1,2 |  | OP.1 |
| SSB Configuration | Semi- static channel acces |  | Config 1,2 |  | SSB.1 CCA |
|  | Dymamic channel acces |  | Config 1,2 |  | SSB.2 CCA |
| SMTC Configuration |  |  | Config 1,2 |  | SMTC.1 |
| TRS Configuration |  |  | Config 1,2 |  | TRS.1.2 TDD |
| DL CCA probability for semi-static channel access (PCCA_DL) |  |  | Config 1,2 |  | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  |  | Config 1,2 |  | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  |  | Config 1,2 |  | 1 |
| DL CCA probability for semi-static channel access (PCCA_DL) |  |  | Config 1,2 |  | 1 |
| Antenna Configuration |  |  |  |  | 1x2 |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  |  | Config 1,2 | dBm/SCS kHz | -101 |
| SS-RSRP Note 3 |  |  | Config 1,2 | dBm/SCS kHz | -84] |
| Ês/Iot |  |  | Config 1,2 | dB | 17 |
| Ês/Noc |  |  | Config 1,2 | dB | 17 |
| IoNote3 |  |  | Config 1,2 | dBm/38.16MHz | -59 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.26.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

A.10.3.5.3.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for PSCell in the beginning of the DL slot right after  slot (i+ TRRCprocessingDelay+TBWPswitchDelayRRC ).

All of the above test requirements shall be fulfilled in order for the observed PSCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.10.3.6	PSCell addition and release delay

#### A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA

##### A.10.3.6.1.1	Test purpose and environment

The purpose of this test is to verify that the NR PSCell addition and release delays on the carrier under CCA under EN-DC are within the requirements stated in clause 7.31A.2 [15] for the case when the PSCell is known by the UE at the time of addition.

Supported test configurations are shown in A.10.3.6.1.1-1. The test parameters for the E-UTRA cell are given in Table A.3.7.2.1-1. The E-UTRA cell once set up is not changed across time.

The test parameters for NR cell are given in Tables A.10.3.6.1.1-2 and cell-specific parameters in A.10.3.6.1.1-3 below. The test consists of five successive time periods with duration of T1, T2, T3, T4 and T5 respectively. There are two carriers each with one cell. Before the test starts the UE is connected to Cell 1 (E-UTRA PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (NR PSCell) on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

Before the start of T2, the UE in the measurement control information that event-triggered reporting with Event B1 is configured for neighbour cell (Cell2). Before the start of T2 the UE is configured with the measurement gaps (gap pattern Id # 0). The Cell2 becomes known to the UE during T2. Therefore, during T2 the UE shall report Event B1. After receiving the Event B1, the test system shall send a RRC message to the UE to release the measurement gaps.

The test system shall send a RRC message to the UE to add PSCell (Cell 2) on radio channel 2. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added. The RRC message to add PSCell shall be sent to the UE during period T2, after the measurement gaps are released by the test system. The point in time at which the RRC message to add PSCell (Cell2) is received at the UE antenna connector defines the start of period T3.

The test system shall observe the periodic reporting of CSI for PSCell during T4. The point in time at which the UE has sent PRACH to the PSCell (Cell 2) defines the start of period T4.

The test system shall send a RRC message to the UE to release PSCell (Cell 2) on radio channel 2. The RRC message to release PSCell (Cell2) shall be sent to the UE during period T4, after the UE has sent at least one CQI report with non-zero CQI index for PSCell (Cell 2). The point in time at which the RRC message to release PSCell (Cell2) is received at the UE antenna connector defines the start of period T5.

Table A.10.3.6.1.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD, NR SCS 30 kHz, BW 40 MHz, TDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.10.3.6.1.1-2: General Test Parameters for PSCell Addition and Release

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| RF Channel Number |  |  |  | 1, 2 | Two radio channels are used for this test. One for E-UTRA cell and second for NR Cell on the carrier under CCA |
| Initial | Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  |  | Cell2 | Neighbour cell on RF channel number 2. |
| Final | Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Condition | Neighbour Cell |  |  | Cell2 | PSCell released on RF channel number 2. |
| B1 | Hysteresis |  | dB | 0 | Hysteresis for evaluation of event B1. |
|  | Threshold RSRP |  | dBm | -93 | Actual RSRP threshold for event B1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time to Trigger |  | S | 0 |  |
| DRX |  |  |  | OFF | Continuous monitoring of primary cell |
| DL CCA model |  | Dynamic channel accessNote 1, 3 |  |  | As specified in clause A.3.20.2.1 |
|  |  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model |  | Dynamic channel access Note 1, 3 |  |  | As specified in clause A.3.20.2.2 |
|  |  | Semi-static channel access Note 2,3 |  |  |  |
| Measurement gap pattern Id |  |  |  | 0 | Gaps are configured before T2 and released before T3. |
| PRACH configuration on cell2 |  |  |  | FR1 PRACH configuration 2 | Captured in A.3.8.2.1 |
| CQI/PMI periodicity and offset configuration index on cell2 |  |  |  | 2ms | CQI reporting for PSCell every uplink subframe |
| Cell-individual offset for cells on RF channel number 1 |  |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T304 |  |  | ms | 500 |  |
| LCCA_DL |  |  |  | 5 |  |
| T1 |  |  | s | 1 | During this time the PCell shall be known and cell2 shall be unknown. |
| T2 |  |  | s | ≥ Tidentify_irat_cca_without_index | Tidentify_irat_cca_without_index is defined in clause 8.1.2.4.21A and 8.1.2.4.22A in TS 36.133 During this time the UE shall identify neighbour cell (cell2) and report event B1. |
| T3 |  |  | s | ≥ Tconfig_PSCell_withCCA | During this time the UE adds the PSCell. Tconfig_PSCell_withCCA is defined in clause 7.31A.2 |
| T4 |  |  | s | 0.5 | During this time the UE sends CSI reports for PSCell. |
| T5 |  |  | s | 0.5 | During this time the UE releases the PSCell. |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.10.3.6.1.1-3: Cell Specific Parameters for PSCell Addition and Release

| Parameter | Unit | Config | Test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| PCCA_DL for dynamic channel access Note 5,7 | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |
| PCCA_DL for semi-static channel access Note 6,7 | - | PCCA_DL=0.9375 | PCCA_DL=0.9375 |  |  |  |  |
| PCCA_UL for dynamic channel access Note 5,7 | - | 1 | 1 |  |  |  |  |
| PCCA_UL for semi-static channel access Note 6,7 | - | 1 | 1 |  |  |  |  |
| E-UTRA RF Channel Number |  | 1,2 | 1 |  |  |  |  |
| NR RF Channel Number |  | 1,2 | 2 |  |  |  |  |
| TDD configuration |  | 1,2 | TDDConf.1.1 CCA |  |  |  |  |
| BWchannel |  | 1,2 | 40: NRB,c = 106 |  |  |  |  |
| Initial BWP Configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |
| PDSCH Reference |  | 1,2 | SR1.1 CCA |  |  |  |  |
| RMSI CORESET Reference |  | 1,2 | CR1.1 CCA |  |  |  |  |
| Dedicated CORESET Reference |  | 1,2 | CCR1.1 CCA |  |  |  |  |
| OCNG Patterns |  | 1,2 | OP.1 |  |  |  |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |  |  |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |  |  |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |  |  |  |
| SMTC configuration |  | 1,2 | SMTC.1 |  |  |  |  |
| TRS Configuration |  | 1,2 | TRS.1.2 TDD |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2 | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2 | N/A | -85 |  |  |  |
| Note2 | dBm/SCS | 1,2 | N/A | -82 |  |  |  |
|  |  | 1,2 | -infinity | 0 |  |  |  |
|  |  | 1,2 | -infinity | 0 |  |  |  |
| SS-RSRPNote3 | dBm/SCS | 1,2 | -infinity | -82 |  |  |  |
| IoNote3 | dBm/38.1MHz | 1,2 | N/A | -51 |  |  |  |
| Propagation condition |  | 1,2 | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |

##### A.10.3.6.1.2	Test Requirements

The UE shall transmit the PRACH to PSCell at latest Tconfig_PSCell_withCCA Note1 into T3.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T4.

The UE shall periodically send CSI reports for PSCell after the UE has sent first CQI report with non-zero CQI index during T4

The UE shall stop sending CSI reports for PSCell in at latest 20 ms into T5.

All the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

Note1:	The PSCell addition delay can be expressed as follows as specified in clause 7.31A.2 [15]:

Tconfig_PSCell_withCCA = TRRC_delay + Tprocessing + Tsearch_withCCA + T∆_withCCA + TPSCell_ DU_withCCA + 2 ms

Where:

TRRC_delay = 20 ms

Tprocessing = 20 ms

Tsearch_withCCA = 0

T∆_withCCA = (1+ L2)*20 ms

TPSCell_ DU_withCCA = 20 ms.

L2 is the number of SMTC occasions not available at the UE for fine time tracking and acquiring full timing information, where L2  LCCA_DL.

### A.10.3.7	Active TCI state switching delay

## A.10.4	Measurement procedure

### A.10.4.1	Intra-frequency measurements

#### A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX

##### A.10.4.1.1.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.10.4.1.1.2	Test parameters

Three cells are deployed in the test, which are E-UTRAN PCell (Cell 1) and two cells on the same carrier frequency with CCA transmitting SSBs in DBT windows according to DL CCA model: PSCell (Cell 2) and a neighbour cell (Cell 3). The test parameters for the three cells are given in Table A.10.4.1.1.2-1 and A.10.4.1.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 3.

FFS: The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

- In the first test (Test 1), the UE is configured with SS-RSRP as Event A3 measurement quantity.

- In the second test (Test 2), the UE is configured with SS-RSRQ as Event A3 measurement quantity.

- In the third test (Test 3), the UE is configured with SS-SINR as Event A3 measurement quantity.

Table A.10.4.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD; NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.10.4.1.1.2-2: General test parameters for intra-frequency event triggered reporting without gaps

Editor’s note: Table TBD

Table A.10.4.1.1.2-3: Cell-specific test parameters for intra-frequency event-triggered reporting without gaps

Editor’s note: Table TBD

##### A.10.4.1.1.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX

##### A.10.4.1.2.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.10.4.1.2.2	Test parameters

##### A.10.4.1.2.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX

##### A.10.4.1.3.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.10.4.1.3.2	Test parameters

##### A.10.4.1.3.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX

##### A.10.4.1.4.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.10.4.1.4.2	Test parameters

##### A.10.4.1.4.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX

##### A.10.4.1.5.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.10.4.1.5.2	Test parameters

##### A.10.4.1.5.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX

##### A.10.4.1.6.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.10.4.1.6.2	Test parameters

##### A.10.4.1.6.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX

##### A.10.4.1.7.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.10.4.1.7.2	Test parameters

##### A.10.4.1.7.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX

##### A.10.4.1.8.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.10.4.1.8.2	Test parameters

##### A.10.4.1.8.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.1.9	RSSI measurement reporting on PSCC

##### A.10.4.1.9.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the intra-frequency RSSI measurement reporting requirements in Section 9.2A.7.1.

##### A.10.4.1.9.2	Test parameters

In the test, the UE is configured to perform intra-frequency RSSI measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.10.4.1.9.2-1. There are two cells in the test: Cell 1 is E-UTRAN PCell on a licensed band, and Cell 2 is PSCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.4.1.9.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.4.1.9.2-2: General test parameters.

Editor’s note: Table TBD

#### A.10.4.1.10	Channel occupancy measurement reporting on PSCC

##### A.10.4.1.10.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the intra-frequency channel occupancy measurement reporting requirements in Section 9.2A.7.2.

##### A.10.4.1.10.2	Test parameters

In the test, the UE is configured to perform intra-frequency channel occupancy measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.10.4.1.10.2-1. There are two cells in the test: Cell 1 is E-UTRAN PCell on a licensed band, and Cell 2 is PSCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.4.1.10.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.4.1.10.2-2: General test parameters.

Editor’s note: Table is TBD

#### A.10.4.1.11	RSSI measurement reporting on SCC

##### A.10.4.1.11.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the intra-frequency RSSI measurement reporting requirements in Section 9.2A.7.1.

##### A.10.4.1.11.2	Test parameters

In the test, the UE is configured to perform intra-frequency RSSI measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.10.4.1.11.2-1. There are three cells in the test: Cell 1 is E-UTRAN PCell on a licensed band, Cell 2 is PSCell operating on a carrier frequency under CCA, Cell 3 is SCell on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1, Cell 2, and Cell 3. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.4.1.11.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.4.1.11.2-2: General test parameters.

Editor’s note: Table TBD

#### A.10.4.1.12	Channel occupancy measurement reporting on SCC

##### A.10.4.1.12.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the intra-frequency channel occupancy measurement reporting requirements in Section 9.2A.7.2.

##### A.10.4.1.12.2	Test parameters

In the test, the UE is configured to perform intra-frequency channel occupancy measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.10.4.1.12.2-1. There are three cells in the test: Cell 1 is E-UTRAN PCell on a licensed band, Cell 2 is PSCell operating on a carrier frequency under CCA, and Cell 3 is SCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1, Cell 2, Cell 3. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.4.1.12.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.4.1.12.2-2: General test parameters.

Editor’s note: Table is TBD

### A.10.4.2	Inter-frequency measurements

#### A.10.4.2.1	RSSI measurement reporting

##### A.10.4.2.1.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the inter-frequency RSSI measurement reporting requirements in Section 9.3A.8.

##### A.10.4.2.1.2	Test parameters

In the test, the UE is configured to perform inter-frequency RSSI measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.10.4.2.1.2-1. There are two cells in the test: Cell 1 is E-UTRAN PCell on a licensed band, and Cell 2 is PSCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2. The RSSI measurement is performed on an inter-frequency under CCA. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.4.2.1.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.4.2.1.2-2: General test parameters.

Editor’s note: Table TBD

#### A.10.4.2.2	Channel occupancy measurement reporting

##### A.10.4.2.2.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the inter-frequency channel occupancy measurement reporting requirements in Section 9.3A.9.

##### A.10.4.2.2.2	Test parameters

In the test, the UE is configured to perform inter-frequency channel occupancy measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.10.4.2.2.2-1. There are two cells in the test: Cell 1 is E-UTRAN PCell on a licensed band, and Cell 2 is PSCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2. The channel occupancy measurement is performed on an inter-frequency under CCA. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.4.2.2.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.4.2.2.2-2: General test parameters.

Editor’s note: Table is TBD

#### A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used

##### A.10.4.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test parameters and configurations are given in Tables A.10.4.2.3.1-1, A.10.4.2.3.1-2, and A.10.4.2.3.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.10.4.2.3.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.10.4.2.3.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.10.4.2.3.1-1.

Table A.10.4.2.3.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | E-UTRAN cell: LTE FDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | E-UTRAN cell: LTE TDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.4.2.3.1-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 with CCA (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 with CCA is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1,2 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1,2 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |
| CP length |  | Config 1,2 | Normal |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |
| T2 | s | Config 1,2 | 1.7 | 1.7 |  |

Table A.10.4.2.3.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,2 | TDD |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  |  | Config 1,2 | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| Initial DL BWP |  |  | Config 1,2 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  |  | Config 1,2 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  |  | Config 1,2 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  |  | Config 1,2 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,2 | SR.1.1 CCA |  | - |  |
| CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 CCA |  | - |  |
| SSB parameters | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  | SSB.1 CCA |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | SSB.2 CCA |  | SSB.2 CCA |  |
| DBT window configuration |  |  | Config 1,2 | As defined in A.3.28.1 |  | As defined in A.3.28.1 |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH |  | kHz | Config 1,2 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_UL=1 |  | PCCA_UL=1 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  |  | Config 1,2 | 12 |  | 12 |  |
| WCCA_DL |  | ms | Config 1,2 | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -104 |  | -104 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -101 |  | -101 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/38.16MHz | Config 1,2 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |

##### A.10.4.2.3.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used

##### A.10.4.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2.    The test parameters and configurations are given in Tables A.10.4.2.4.1-1, A.10.4.2.4.1-2, and A.10.4.2.4.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.10.4.2.4.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.10.4.2.4.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.10.4.2.4.1-1.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.10.4.2.4.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | E-UTRAN cell: LTE FDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | E-UTRAN cell: LTE TDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.4.2.4.1-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  |  |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 with CCA (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 with CCA is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1,2 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |  |  |
| CP length |  | Config 1,2 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |  |  |
| T2 | s | Config 1,2 | 2.5 | 17 | 2.5 | 17 |  |

Table A.10.4.2.4.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1,2 | TDD |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1,2 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 CCA |  |  |  |
| Initial DL BWP |  |  | Config 1,2 | DLBWP.0.1 |  |  |  | NA |  |  |  |
| Initial UL BWP |  |  | Config 1,2 | ULBWP.0.1 |  |  |  | NA |  |  |  |
| Dedicated DL BWP |  |  | Config 1,2 | DLBWP.1.1 |  |  |  | NA |  |  |  |
| Dedicated UL BWP |  |  | Config 1,2 | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference |  |  | Config 1,2 | SR.1.1 CCA |  |  |  | - |  |  |  |
| CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 CCA |  |  |  | - |  |  |  |
| SSB parameters | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  |  |  | SSB.1 CCA |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | SSB.2 CCA |  |  |  | SSB.2 CCA |  |  |  |
| DBT window configuration |  |  | Config 1,2 | As defined in A.3.28.1 |  |  |  | As defined in A.3.28.1 |  |  |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH |  | kHz | Config 1,2 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_DL=0.9375 |  |  |  | PCCA_DL=0.9375 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_UL=1 |  |  |  | PCCA_UL=1 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_UL=1 |  |  |  | PCCA_UL=1 |  |  |  |
| LCCA_DL |  |  | Config 1,2 | 5 |  |  |  | 5 |  |  |  |
| WCCA_DL |  | ms | Config 1,2 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -104 |  |  |  | -104 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -101 |  |  |  | -101 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 |  | -91 |  | -Infinity |  | -88 |  |
|  |  | dB | Config 1,2 | 4 |  | 4 |  | -Infinity |  | 7 |  |
|  |  | dB | Config 1,2 | 4 |  | 4 |  | -Infinity |  | 7 |  |
| IoNote3 |  | dBm/38.16MHz | Config 1,2 | -58.49 |  | -58.49 |  | -63.94 |  | -56.15 |  |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table A.10.4.2.4.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.10.4.2.4.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.10.4.2.4.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used

##### A.10.4.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2.    The test parameters and configurations are given in Tables A.10.4.2.5.1-1, A.10.4.2.5.1-2, and A.10.4.2.5.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.10.4.2.5.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.10.4.2.5.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.10.4.2.5.1-1.

Table A.10.4.2.5.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | E-UTRAN cell: LTE FDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | E-UTRAN cell: LTE TDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.4.2.5.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 with CCA (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 with CCA is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1,2 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1,2 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2 | -6 |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |
| CP length |  | Config 1,2 | Normal |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |
| T2 | s | Config 1,2 | 2 | 2 |  |

Table A.10.4.2.5.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter |  | Unit | Test | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,2 | TDD |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  |  | Config 1,2 | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| Initial DL BWP |  |  | Config 1,2 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  |  | Config 1,2 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  |  | Config 1,2 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  |  | Config 1,2 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1,2 | SR.1.1 CCA |  | - |  |
| CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 CCA |  | - |  |
| SSB parameters | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  | SSB.1 CCA |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | SSB.2 CCA |  | SSB.2 CCA |  |
| DBT window configuration |  |  | Config 1,2 | As defined in A.3.28.1 |  | As defined in A.3.28.1 |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH |  | kHz | Config 1,2 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_UL=1 |  | PCCA_UL=1 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  |  | Config 1,2 | 5 |  | 5 |  |
| WCCA_DL |  | ms | Config 1,2 | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | -104 |  | -104 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -101 |  | -101 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/38.16MHz | Config 1,2 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |

##### A.10.4.2.5.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used

##### A.10.4.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2.    The test parameters and configurations are given in Tables A.10.4.2.6.1-1, A.10.4.2.6.1-2, and A.10.4.2.6.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.10.4.2.6.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.10.4.2.6.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.10.4.2.6.1-1.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.10.4.2.6.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | E-UTRAN cell: LTE FDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | E-UTRAN cell: LTE TDD NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.4.2.6.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  |  |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cell |  | Config 1,2 | LTE Cell 1 (PCell) and NR cell 2 with CCA (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 with CCA is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1,2 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2 | 0 |  |  |  |  |
| CP length |  | Config 1,2 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2 | 5 |  |  |  |  |
| T2 | s | Config 1,2 | 3 | 20 | 3 | 20 |  |

Table A.10.4.2.6.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter |  | Unit | Test | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1,2 | TDD |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1,2 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 CCA |  |  |  |
| Initial DL BWP |  |  | Config 1,2 | DLBWP.0.1 |  |  |  | NA |  |  |  |
| Initial UL BWP |  |  | Config 1,2 | ULBWP.0.1 |  |  |  | NA |  |  |  |
| Dedicated DL BWP |  |  | Config 1,2 | DLBWP.1.1 |  |  |  | NA |  |  |  |
| Dedicated UL BWP |  |  | Config 1,2 | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1,2 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference |  |  | Config 1,2 | SR.1.1 CCA |  |  |  | - |  |  |  |
| CORESET Reference Channel |  |  | Config 1,2 | CR.1.1 CCA |  |  |  | - |  |  |  |
| SSB parameters | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  |  |  | SSB.1 CCA |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | SSB.2 CCA |  |  |  | SSB.2 CCA |  |  |  |
| DBT window configuration |  |  | Config 1,2 | As defined in A.3.28.1 |  |  |  | As defined in A.3.28.1 |  |  |  |
| SMTC configuration |  |  | Config 1,2 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH |  | kHz | Config 1,2 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_DL=0.9375 |  |  |  | PCCA_DL=0.9375 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2 | PCCA_UL=1 |  |  |  | PCCA_UL=1 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2 | PCCA_UL=1 |  |  |  | PCCA_UL=1 |  |  |  |
| LCCA_DL |  |  | Config 1,2 | 2 |  |  |  | 2 |  |  |  |
| WCCA_DL |  | ms | Config 1,2 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2 | 104 |  |  |  | 104 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -101 |  |  |  | -101 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 |  | -91 |  | -Infinity |  | -88 |  |
|  |  | dB | Config 1,2 | 4 |  | 4 |  | -Infinity |  | 7 |  |
|  |  | dB | Config 1,2 | 4 |  | 4 |  | -Infinity |  | 7 |  |
| IoNote3 |  | dBm/38.16MHz | Config 1,2 | -58.49 |  | -58.49 |  | -63.94 |  | -56.15 |  |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table A.10.4.2.6.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.10.4.2.6.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.10.4.2.6.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used

##### A.10.4.2.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.  The test parameters and configurations are given in Tables A.10.4.2.7.1-1, A.10.4.2.7.1-2, and A.10.4.2.7.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.10.4.2.7.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.10.4.2.7.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.10.4.2.7.1-1.

Table A.10.4.2.7.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | E-UTRAN cell: LTE FDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 5 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 6 | E-UTRAN cell: LTE TDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.4.2.7.1-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  | Two FR1 NR carrier frequencies are used. NR RF channel 1 is with CCA. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 3 ms |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 1.7 | 1.7 |  |

Table A.10.4.2.7.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4,5,6 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,4 | TDD |  | FDD |  |
|  |  |  | Config 2,3,5,6 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| TDD configuration |  |  | Config 1,4 | TDDConf.1.1 CCA |  | NA |  |
|  |  |  | Config 2,5 | TDDConf.1.1 CCA |  | TDDConf.1.1 |  |
|  |  |  | Config 3,6 | TDDConf.1.1 CCA |  | TDDConf.2.1 |  |
| Initial DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2,3,4,5,6 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3,4,5,6 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 CCA |  | SR.1.1 FDD |  |
|  |  |  | Config 2,5 | SR.1.1 CCA |  | SR.1.1 TDD |  |
|  |  |  | Config 3,6 | SR.1.1 CCA |  | SR.2.1 TDD |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 CCA |  | CR.1.1 FDD |  |
|  |  |  | Config 2,5 | CR.1.1 CCA |  | CR.1.1 TDD |  |
|  |  |  | Config 3,6 | CR.1.1 CCA |  | CR.2.1 TDD |  |
| SSB | Semi-static channel access Note 5,7 |  | Config 1,4 | SSB.1 CCA |  | SSB.1 FR1 |  |
| parameters |  |  | Config 2,5 | SSB.1 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 3,6 | SSB.1 CCA |  | SSB.2 FR1 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,4 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 2,5 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 3,6 | SSB.2 CCA |  | SSB.2 FR1 |  |
| DBT window configuration |  |  | Config 1,2,3,4,5,6 | As defined in A.3.28.1 |  | Not applicable |  |
| SMTC configuration |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
| defined in A.3.11 |  |  | Config 2,3,5,6 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH |  | kHz | Config 1,2,4,5 | 30 |  | 15 |  |
| subcarrier spacing |  |  | Config 3,6 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_DL=0.9375 |  | Not applicable |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | Not applicable |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  | Not applicable |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  | Not applicable |  |
| LCCA_DL |  |  | Config 1,2,3,4,5,6 | 12 |  | 12 |  |
| WCCA_DL |  | ms | Config 1,2,3,4,5,6 | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3,4,5,6 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4,5,6 | -104 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4,5 | -101 |  | -98 |  |
|  |  |  | Config 3,6 | -101 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4,5 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3,6 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | NR Config 1,2,4,5 | -58.49 | -58.49 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | NR Config 3,6 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3,4,5,6 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |

##### A.10.4.2.7.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used

##### A.10.4.2.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.  The test parameters and configurations are given in Tables A.10.4.2.8.1-1, A.10.4.2.8.1-2, and A.10.4.2.8.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.10.4.2.8.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.10.4.2.8.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.10.4.2.8.1-1.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.10.4.2.8.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | E-UTRAN cell: LTE FDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 5 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mod |
| 6 | E-UTRAN cell: LTE TDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.4.2.8.1-2: General test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  |  |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. NR RF channel 1 is with CCA. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 3 ms |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 2.5 | 17 | 2.5 | 17 |  |

Table A.10.4.2.8.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3,4,5,6 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1,4 | TDD |  |  |  | FDD |  |  |  |
|  |  |  | Config 2,3,5,6 | TDD |  |  |  | TDD |  |  |  |
| BWchannel |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  |  | Config 1,4 | TDDConf.1.1 CCA |  |  |  | NA |  |  |  |
|  |  |  | Config 2,5 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3,6 | TDDConf.1.1 CCA |  |  |  | TDDConf.2.1 |  |  |  |
| Initial DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  |  |  | NA |  |  |  |
| Initial UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  |  |  | NA |  |  |  |
| Dedicated DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  |  |  | NA |  |  |  |
| Dedicated UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1,2,3,4,5,6 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3,4,5,6 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 CCA |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2,5 | SR.1.1 CCA |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3,6 | SR.1.1 CCA |  |  |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 CCA |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2,5 | CR.1.1 CCA |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3,6 | CR.1.1 CCA |  |  |  | CR.2.1 TDD |  |  |  |
| SSB | Semi-static channel access Note 5,7 Semi-static channel access Note 5,7 |  | Config 1,4 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
| parameters |  |  | Config 2,5 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3,6 | SSB.1 CCA |  |  |  | SSB.2 FR1 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,4 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 2,5 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3,6 | SSB.2 CCA |  |  |  | SSB.2 FR1 |  |  |  |
| DBT window configuration |  |  | Config 1,2,3,4,5,6 | As defined in A.3.28.1 |  |  |  | Not applicable |  |  |  |
| SMTC configuration |  |  | Config 1,4 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
| defined in A.3.11 |  |  | Config 2,3,5,6 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH |  | kHz | Config 1,2,4,5 | 30 |  |  |  | 15 |  |  |  |
| subcarrier spacing |  |  | Config 3,6 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_DL=0.9375 |  |  |  | Not applicable |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | Not applicable |  |  |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  |  |  | Not applicable |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  |  |  | Not applicable |  |  |  |
| LCCA_DL |  |  | Config 1,2,3,4,5,6 | 5 |  |  |  | 5 |  |  |  |
| WCCA_DL |  | ms | Config 1,2,3,4,5,6 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3,4,5,6 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4,5,6 | -104 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2,4,5 | -101 |  |  |  | -98 |  |  |  |
|  |  |  | Config 3,6 | -101 |  |  |  | -95 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4,5 | -94 |  | -94 |  | -Infinity |  | -91 |  |
|  |  |  | Config 3,6 | -91 |  | -91 |  | -Infinity |  | -88 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 |  | 4 |  | -Infinity |  | 7 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 |  | 4 |  | -Infinity |  | 7 |  |
| IoNote3 |  | dBm/9.36MHz | NR Config 1,2,4,5 | -58.49 |  | -58.49 |  | -70.05 |  | -62.26 |  |
|  |  | dBm/38.16MHz | NR Config 3,6 | -58.49 |  | -58.49 |  | -63.94 |  | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3,4,5,6 | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table A.10.4.2.8.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.10.4.2.8.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.10.4.2.8.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used

##### A.10.4.2.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.  The test parameters and configurations are given in Tables A.10.4.2.9.1-1, A.10.4.2.9.1-2, and A.10.4.2.9.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.10.4.2.9.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.10.4.2.9.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.10.4.2.9.1-1.

Table A.10.4.2.9.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | E-UTRAN cell: LTE FDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 5 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mod |
| 6 | E-UTRAN cell: LTE TDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.4.2.9.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  | Two FR1 NR carrier frequencies are used. NR RF channel 1 is with CCA. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 3 ms |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 1,2,3,4,5,6 | 3 s |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 2 | 2 |  |

Table A.10.4.2.9.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter |  | Unit | Test | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4,5,6 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1,4 | TDD |  | FDD |  |
|  |  |  | Config 2,3,5,6 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| TDD configuration |  |  | Config 1,4 | TDDConf.1.1 CCA |  | NA |  |
|  |  |  | Config 2,5 | TDDConf.1.1 CCA |  | TDDConf.1.1 |  |
|  |  |  | Config 3,6 | TDDConf.1.1 CCA |  | TDDConf.2.1 |  |
| Initial DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  | NA |  |
| Initial UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  | NA |  |
| Dedicated DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  | NA |  |
| Dedicated UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1,2,3,4,5,6 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3,4,5,6 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 CCA |  | SR.1.1 FDD |  |
|  |  |  | Config 2,5 | SR.1.1 CCA |  | SR.1.1 TDD |  |
|  |  |  | Config 3,6 | SR.1.1 CCA |  | SR.2.1 TDD |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 CCA |  | CR.1.1 FDD |  |
|  |  |  | Config 2,5 | CR.1.1 CCA |  | CR.1.1 TDD |  |
|  |  |  | Config 3,6 | CR.1.1 CCA |  | CR.2.1 TDD |  |
| SSB | Semi-static channel access Note 5,7 Semi-static channel access Note 5,7 |  | Config 1,4 | SSB.1 CCA |  | SSB.1 FR1 |  |
| parameters |  |  | Config 2,5 | SSB.1 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 3,6 | SSB.1 CCA |  | SSB.2 FR1 |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,4 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 2,5 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  | Config 3,6 | SSB.2 CCA |  | SSB.2 FR1 |  |
| DBT window configuration |  |  | Config 1,2,3,4,5,6 | As defined in A.3.28.1 |  | Not applicable |  |
| SMTC configuration |  |  | Config 1,4 | SMTC.2 |  | SMTC.5 |  |
| defined in A.3.11 |  |  | Config 2,3,5,6 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH |  | kHz | Config 1,2,4,5 | 30 |  | 15 |  |
| subcarrier spacing |  |  | Config 3,6 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_DL=0.9375 |  | Not applicable |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | Not applicable |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  | Not applicable |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  | Not applicable |  |
| LCCA_DL |  |  | Config 1,2,3,4,5,6 | 5 |  | 5 |  |
| WCCA_DL |  | ms | Config 1,2,3,4,5,6 | TPSS/SSS_sync_inter_cca800 |  | TPSS/SSS_sync_inter_cca800 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3,4,5,6 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4,5,6 | -104 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2,4,5 | -101 |  | -98 |  |
|  |  |  | Config 3,6 | -101 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4,5 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3,6 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | NR Config 1,2,4,5 | -58.49 | -58.49 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | NR Config 3,6 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3,4,5,6 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |

##### A.10.4.2.9.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used

##### A.10.4.2.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the EN-DC inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 on NR RF channel 2.  The test parameters and configurations are given in Tables A.10.4.2.10.1-1, A.10.4.2.10.1-2, and A.10.4.2.10.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.10.4.2.10.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.10.4.2.10.1-2 is provided for UE that support per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

The configuration of LTE cell 1 is defined in table A.3.7.2.1-1. Supported test configurations are shown in table A.10.4.2.10.1-1.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.10.4.2.10.1-1: EN-DC event triggered reporting tests without SSB index reading for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | E-UTRAN cell: LTE FDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | E-UTRAN cell: LTE FDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 5 | E-UTRAN cell: LTE TDD NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mod |
| 6 | E-UTRAN cell: LTE TDD NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.10.4.2.10.1-2: General test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter | Unit | Test | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | configuration | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  |  |  | One E-UTRAN carrier frequency is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. NR RF channel 1 is with CCA. |
| Active cell |  | Config 1,2,3,4,5,6 | LTE Cell 1 (PCell) and NR cell 2 (PScell) |  |  |  | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 2 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2,3,4,5,6 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2,3,4,5,6 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4,5,6 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3,4,5,6 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3,4,5,6 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3,4,5,6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3,4,5,6 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between PCell and PSCell |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 3 ms |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | Config 1,2,3,4,5,6 | 3 s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3,4,5,6 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3,4,5,6 | 3 | 20 | 3 | 20 |  |

Table A.10.4.2.10.1-3: Cell specific test parameters for EN-DC inter-frequency event triggered reporting with SSB time index detection

| Parameter |  | Unit | Test | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | configuration | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3,4,5,6 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1,4 | TDD |  |  |  | FDD |  |  |  |
|  |  |  | Config 2,3,5,6 | TDD |  |  |  | TDD |  |  |  |
| BWchannel |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2,4,5 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3,6 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| TDD configuration |  |  | Config 1,4 | TDDConf.1.1 CCA |  |  |  | NA |  |  |  |
|  |  |  | Config 2,5 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3,6 | TDDConf.1.1 CCA |  |  |  | TDDConf.2.1 |  |  |  |
| Initial DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.0.1 |  |  |  | NA |  |  |  |
| Initial UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.0.1 |  |  |  | NA |  |  |  |
| Dedicated DL BWP |  |  | Config 1,2,3,4,5,6 | DLBWP.1.1 |  |  |  | NA |  |  |  |
| Dedicated UL BWP |  |  | Config 1,2,3,4,5,6 | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1,2,3,4,5,6 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3,4,5,6 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 CCA |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2,5 | SR.1.1 CCA |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3,6 | SR.1.1 CCA |  |  |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 CCA |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2,5 | CR.1.1 CCA |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3,6 | CR.1.1 CCA |  |  |  | CR.2.1 TDD |  |  |  |
| SSB | Semi-static channel access Note 5,7 Semi-static channel access Note 5,7 |  | Config 1,4 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
| parameters |  |  | Config 2,5 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3,6 | SSB.1 CCA |  |  |  | SSB.2 FR1 |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,4 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 2,5 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3,6 | SSB.2 CCA |  |  |  | SSB.2 FR1 |  |  |  |
| DBT window configuration |  |  | Config 1,2,3,4,5,6 | As defined in A.3.28.1 |  |  |  | Not applicable |  |  |  |
| SMTC configuration |  |  | Config 1,4 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
| defined in A.3.11 |  |  | Config 2,3,5,6 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH |  | kHz | Config 1,2,4,5 | 30 |  |  |  | 15 |  |  |  |
| subcarrier spacing |  |  | Config 3,6 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_DL=0.9375 |  |  |  | Not applicable |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | Not applicable |  |  |  |
| UL CCA probability PCCA_UL | Semi-static channel access Note 5,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  |  |  | Not applicable |  |  |  |
|  | Dynamic channel access Note 6,7 |  | Config 1,2,3,4,5,6 | PCCA_UL=1 |  |  |  | Not applicable |  |  |  |
| LCCA_DL |  |  | Config 1,2,3,4,5,6 | 2 |  |  |  | 2 |  |  |  |
| WCCA_DL |  | ms | Config 1,2,3,4,5,6 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3,4,5,6 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3,4,5,6 | -104 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2,4,5 | -101 |  |  |  | -98 |  |  |  |
|  |  |  | Config 3,6 | -101 |  |  |  | -95 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4,5 | -94 |  | -94 |  | -Infinity |  | -91 |  |
|  |  |  | Config 3,6 | -91 |  | -91 |  | -Infinity |  | -88 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 |  | 4 |  | -Infinity |  | 7 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 |  | 4 |  | -Infinity |  | 7 |  |
| IoNote3 |  | dBm/9.36MHz | NR Config 1,2,4,5 | -58.49 |  | -58.49 |  | -70.05 |  | -62.26 |  |
|  |  | dBm/38.16MHz | NR Config 3,6 | -58.49 |  | -58.49 |  | -63.94 |  | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3,4,5,6 | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table A.10.4.2.10.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.10.4.2.10.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.10.4.2.10.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.10.4.3	L1-RSRP measurements for beam reporting

#### A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used

##### A.10.4.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.10.4.3.1.1-1.

Table A.10.4.3.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.10.4.3.1.2	Test parameters

There are two cells in the test, E-UTRAN Pcell (Cell 1) and FR1 PSCell (Cell 2) which operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model. The test parameters and applicability for Cell 1 are defined in A.3.7A.2. The test parameters for the Cell 2 are given in Table A.10.4.3.1.2-1 and Table A.10.4.3.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.10.4.3.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1,2 |  | freq1 |
| DL CCA model | 1,2 |  | As specifieed in A.3.20.2.1 |
| UL CCA model | 1,2 |  | As specified in A.3.20.2.2 |
| Duplex mode | 1,2 |  | TDD |
| TDD Configuration | 1,2 |  | TDDConf.1.1 CCA |
| BWchannel | 1,2 | MHz | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1,2 |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel | 1,2 |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel | 1,2 |  | CCR.1.1 CCA |
| SSB configuration | 1,2 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |
| OCNG Patterns | 1,2 |  | OP.1 |
| Initial BWP Configuration | 1,2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1,2 |  | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration | 1,2 |  | DBT.1 |
| TRS Configuration | 1,2 |  | TRS.1.2 TDD |
| DRX configuration | 1,2 |  | Off |
| reportConfigType | 1,2 |  | periodic |
| reportQuantity | 1,2 |  | ssb-Index-RSRP |
| Number of reported RS | 1,2 |  | 2 |
| L1-RSRP reporting period | 1,2 | slot | 80 |
| T1 | 1,2 | s | 5 |
| T2 | 1,2 | s | 1 |
| EPRE ratio of PSS to SSS |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | 1,2 | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1,2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |

Table A.10.4.3.1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1,2 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1,2 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1,2 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1,2 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1,2 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1,2 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS Res when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.10.4.3.1.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

The UE shall send L1-RSRP report of both SSB0 and SSB1 in Cell 2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used

##### A.10.4.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.10.4.3.1.1-1.

Table A.10.4.3.2.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.10.4.3.2.2	Test parameters

There are two cells in the test, E-UTRAN Pcell (Cell 1) and FR1 PSCell (Cell 2) which operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model. The test parameters and applicability for Cell 1 are defined in A.3.7A.2. The test parameters for the Cell 2 are given in Table A.10.4.3.2.2-1 and Table A.10.4.3.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.10.4.3.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1,2 |  | freq1 |
| DL CCA model | 1,2 |  | As specifieed in A.3.20.2.1 |
| UL CCA model | 1,2 |  | As specified in A.3.20.2.2 |
| Duplex mode | 1,2 |  | TDD |
| TDD Configuration | 1,2 |  | TDDConf.1.1 CCA |
| BWchannel | 1,2 | MHz | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1,2 |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel | 1,2 |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel | 1,2 |  | CCR.1.1 CCA |
| SSB configuration | 1,2 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |
| OCNG Patterns | 1,2 |  | OP.1 |
| Initial BWP Configuration | 1,2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1,2 |  | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration | 1,2 |  | DBT.1 |
| TRS Configuration | 1,2 |  | TRS.1.2 TDD |
| DRX configuration | 1,2 |  | DRX.3 |
| reportConfigType | 1,2 |  | periodic |
| reportQuantity | 1,2 |  | ssb-Index-RSRP |
| Number of reported RS | 1,2 |  | 2 |
| L1-RSRP reporting period | 1,2 | slot | 80 |
| T1 | 1,2 | s | 5 |
| T2 | 1,2 | s | 1 |
| EPRE ratio of PSS to SSS |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | 1,2 | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1,2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |

Table A.10.4.3.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1,2 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1,2 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1,2 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1,2 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1,2 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1,2 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.10.4.3.2.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

The UE shall send L1-RSRP report of both SSB0 and SSB1 in Cell 2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used

##### A.10.4.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.10.4.3.1.1-1.

Table A.10.4.3.3.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.10.4.3.3.2	Test parameters

There are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2), and FR1 SCell (Cell 3). Cell 2 and Cell 3 operate on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model. The test parameters and applicability for Cell 1 are defined in A.3.7A.2. The test parameters for the Cell 2 and Cell 3 are given in Table A.10.4.3.3.2-1 and Table A.10.4.3.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.10.4.3.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Active PScell | 1,2 |  | Cell 2 |
| Active Scell | 1,2 |  | Cell 3 |
| RF Channel Number | 1,2 |  | 1: Cell 2 2: Cell 3 |
| DL CCA model | 1,2 |  | As specifieed in A.3.20.2.1 |
| UL CCA model | 1,2 |  | As specified in A.3.20.2.2 |
| Duplex mode | 1,2 |  | TDD |
| TDD Configuration | 1,2 |  | TDDConf.1.1 CCA |
| BWchannel | 1,2 | MHz | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1,2 |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel | 1,2 |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel | 1,2 |  | CCR.1.1 CCA |
| SSB configuration | 1,2 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |
| OCNG Patterns | 1,2 |  | OP.1 |
| Initial BWP Configuration | 1,2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1,2 |  | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration | 1,2 |  | DBT.1 |
| TRS Configuration | 1,2 |  | TRS.1.2 TDD |
| DRX configuration | 1,2 |  | Off |
| reportConfigType | 1,2 |  | periodic |
| reportQuantity | 1,2 |  | ssb-Index-RSRP |
| Number of reported RS | 1,2 |  | 2 |
| L1-RSRP reporting period | 1,2 | slot | 80 |
| T1 | 1,2 | s | 5 |
| T2 | 1,2 | s | 1 |
| EPRE ratio of PSS to SSS |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | 1,2 | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1,2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |

Table A.10.4.3.3.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1, 2 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1, 2 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1, 2 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1, 2 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1, 2 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1, 2 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1, 2 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1, 2 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1, 2 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.10.4.3.3.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

The UE shall send L1-RSRP report of both SSB0 and SSB1 in Cell 3.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used

##### A.10.4.3.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.10.4.3.4.1-1.

Table A.10.4.3.4.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.10.4.3.4.2	Test parameters

There are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2), and FR1 SCell (Cell 3). Cell 2 and Cell 3 operate on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model. The test parameters and applicability for Cell 1 are defined in A.3.7A.2. The test parameters for the Cell 2 and Cell 3 are given in Table A.10.4.3.4.2-1 and Table A.10.4.3.4.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.10.4.3.4.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Active PScell | 1,2 |  | Cell 2 |
| Active Scell | 1,2 |  | Cell 3 |
| RF Channel Number | 1,2 |  | 1: Cell 2 2: Cell 3 |
| DL CCA model | 1,2 |  | As specifieed in A.3.20.2.1 |
| UL CCA model | 1,2 |  | As specified in A.3.20.2.2 |
| Duplex mode | 1,2 |  | TDD |
| TDD Configuration | 1,2 |  | TDDConf.1.1 CCA |
| BWchannel | 1,2 | MHz | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1,2 |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel | 1,2 |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel | 1,2 |  | CCR.1.1 CCA |
| SSB configuration | 1,2 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |
| OCNG Patterns | 1,2 |  | OP.1 |
| Initial BWP Configuration | 1,2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1,2 |  | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration | 1,2 |  | DBT.1 |
| TRS Configuration | 1,2 |  | TRS.1.2 TDD |
| DRX configuration | 1,2 |  | DRX.3 |
| reportConfigType | 1,2 |  | periodic |
| reportQuantity | 1,2 |  | ssb-Index-RSRP |
| Number of reported RS | 1,2 |  | 2 |
| L1-RSRP reporting period | 1,2 | slot | 80 |
| T1 | 1,2 | s | 5 |
| T2 | 1,2 | s | 1 |
| EPRE ratio of PSS to SSS |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | 1,2 | dB | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1,2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |

Table A.10.4.3.4.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1, 2 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1, 2 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1, 2 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1, 2 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1, 2 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1, 2 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1, 2 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1, 2 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1, 2 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.10.4.3.4.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

The UE shall send L1-RSRP report of both SSB0 and SSB1 in Cell 3.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA

#### A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used

##### A.10.4.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21A of TS 36.133 [15] for E-UTRAN FDD-NR measurements under CCA and clause 8.1.2.4.22A of TS 36.133 [15] for E-UTRAN TDD-NR measurements under CCA.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test parameters are given in Tables A.10.4.4.1.1-1, A.10.4.4.1.1-2, A.10.4.4.1.1-3 and A.10.4.4.1.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.10.4.4.1.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.10.4.4.1.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The UE is tested when MeasTriggerQuantity is configured as RSRP, RSRQ and SINR for each test. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.10.4.4.1.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.10.4.4.1.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | 1 |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  |  | 1, 2 | 1,2 |  | Two FR1 NR carrier frequency under CCA is used. |
| DL CCA model | Dynamic channel accessNote 3, 5 |  |  | As specified in clause A.3.20.2.1 |  |  |
|  | Semi-static channel access Note 4, 5 |  |  |  |  |  |
| UL CCA model | Dynamic channel accessNote 3, 5 |  |  | As specified in clause A.3.20.2.2 |  |  |
|  | Semi-static channel access Note 4, 5 |  |  |  |  |  |
| Active cell |  |  | 1, 2 | E-UTRA cell 1 (PCell) and NR cell 2 with CCA (PSCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | 1, 2 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [16]. |
| b2-Threshold1 |  | dBm | 1, 2 | Note 1 |  | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR |  | dBm | 1, 2 | Note 2 |  | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 3 for event B2 [16] |
| Hysteresis |  | dB | 1, 2 | 0 |  |  |
| CP length |  |  | 1, 2 | Normal |  |  |
| TimeToTrigger |  | s | 1, 2 | 0 |  |  |
| Filter coefficient |  |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  |  | 1, 2 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  |  | 1, 2 | 3s |  | Synchronous cells. |
| T1 |  | s | 1, 2 | 5 |  |  |
| T2 |  | s | 1, 2 | ≥Tidentify_irat_cca_without_index | ≥Tidentify_irat_cca_without_index | Tidentify_irat_cca_without_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| NOTE 1: The value of b2-Threshold1 is defined in Table A.10.4.4.1.1-3 NOTE 2: The value of b2-Threshold2NR is defined in Table A.10.4.4.1.1-4 NOTE 3: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 4: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 5: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |

Table A.10.4.4.1.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 for RSRP |  |
|  |  | 1, 2 | [TBDfor RSRQ] |  |
|  | dB | 1, 2 | [TBD for SINR] |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.10.4.4.1.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T2 | T2 |
| NR RF Channel Number |  | 1, 2 | 2 |  |  | 3 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| PCCA_DL for dynamic channel access Note 6,8 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 7,8 |  | 1, 2 | PCCA_DL=0.9375 |  |  | PCCA_DL=0.9375 |  |
| PCCA_UL for dynamic channel access Note 6,8 |  | 1, 2 | 1 |  |  | 1 |  |
| PCCA_UL for semi-static channel access Note 7,8 |  | 1, 2 | 1 |  |  | 1 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |  | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |  | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |  | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |  | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |  | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |  | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2 | NA |  |  | -98 for SS-RSRP |  |
|  |  | 1, 2 | NA |  |  | [TBD for SS-RSRQ] |  |
|  | dB | 1, 2 | NA |  |  | [TBD For SS-SINR] |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |  | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |  | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -91 |  | -91 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | 4 |  | 4 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | 4 |  | 4 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -58.49 |  | -58.49 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | ETU70 |  |  | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |  | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |

##### A.10.4.4.1.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is not required to report SSB time index. Tidentify_irat_cca_without_index is defined in defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used

##### A.10.4.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21 of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test parameters are given in Tables A.10.4.4.2.1-1, A.10.4.4.2.1-2, A.10.4.4.2.1-3 and A.10.4.4.2.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.10.4.4.2.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.10.4.4.2.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The UE is tested when MeasTriggerQuantity is configured as RSRP, RSRQ and SINR for each test. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.10.4.4.2.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.10.4.4.2.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | 1 |  |  |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  |  | 1, 2 | 1,2 |  |  |  | Two FR1 NR carrier frequency under CCA is used. |
| Active cell |  |  | 1, 2 | E-UTRA cell 1 (PCell) and NR cell 2 with CCA (PSCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| DL CCA model | Dynamic channel accessNote 3, 5 |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |
|  | Semi-static channel access Note 4, 5 |  |  |  |  |  |  |  |
| UL CCA model | Dynamic channel accessNote 3, 5 |  |  | As specified in clause A.3.20.2.2 |  |  |  |  |
|  | Semi-static channel access Note 4, 5 |  |  |  |  |  |  |  |
| Neighbour cell |  |  | 1, 2 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  |  | 1, 2 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  |  | 1, 2 | 39 |  | 19 |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 |  | dBm | 1, 2 | Note 1 |  |  |  | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR |  | dBm | 1, 2 | Note 2 |  |  |  | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 3 for event B2 [16] |
| Hysteresis |  | dB | 1, 2 | 0 |  |  |  |  |
| CP length |  |  | 1, 2 | Normal |  |  |  |  |
| TimeToTrigger |  | s | 1, 2 | 0 |  |  |  |  |
| Filter coefficient |  |  | 1, 2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | 1, 2 | DRX.9 | DRX.10 | DRX.9 | DRX.10 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  |  | 1, 2 | 3s |  |  |  | Synchronous cells. |
| T1 |  | s | 1, 2 | 5 |  |  |  |  |
| T2 |  | s | 1, 2 | ≥Tidentify_irat_cca_without_index |  |  |  | Tidentify_irat_cca_without_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| NOTE 1: The value of b2-Threshold1 is defined in Table A.10.4.4.1.1-3 NOTE 2: The value of b2-Threshold2NR is defined in Table A.10.4.4.1.1-4 NOTE 3: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 4: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 5: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |

Table A.10.4.4.2.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 for RSRP |  |
|  |  | 1, 2 | [TBD for RSRQ] |  |
|  | dB | 1, 2 | [TBD for SINR] |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.10.4.4.2.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T2 | T2 |
| NR RF Channel Number |  | 1, 2 | 2 |  |  | 3 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| PCCA_DL for dynamic channel access Note 6,8 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 7,8 |  | 1, 2 | PCCA_DL=0.9375 |  |  | PCCA_DL=0.9375 |  |
| PCCA_UL for dynamic channel access Note 6,8 |  | 1, 2 | 1 |  |  | 1 |  |
| PCCA_UL for semi-static channel access Note 7,8 |  | 1, 2 | 1 |  |  | 1 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |  | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |  | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |  | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |  | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |  | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |  | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2 | NA |  |  | -98 for SS-RSRP |  |
|  |  | 1, 2 | NA |  |  | [TBD for SS-RSRQ] |  |
|  | dB | 1, 2 | NA |  |  | [TBD For SS-SINR] |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |  | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |  | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -91 |  | -91 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | 4 |  | 4 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | 4 |  | 4 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -58.49 |  | -58.49 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | ETU70 |  |  | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |  | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |

##### A.10.4.4.2.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used

##### A.10.4.4.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test parameters are given in Tables A.10.4.4.3.1-1, A.10.4.4.3.1-2, A.10.4.4.3.1-3 and A.10.4.4.3.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.10.4.4.3.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.10.4.4.3.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The UE is tested when MeasTriggerQuantity is configured as RSRP, RSRQ and SINR for each test. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.10.4.4.3.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.10.4.4.3.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | 1 |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  |  | 1, 2 | 1,2 |  | Two FR1 NR carrier frequency under CCA is used. |
| DL CCA model | Dynamic channel accessNote 3, 5 |  |  | As specified in clause A.3.20.2.1 |  |  |
|  | Semi-static channel access Note 4, 5 |  |  |  |  |  |
| UL CCA model | Dynamic channel accessNote 3, 5 |  |  | As specified in clause A.3.20.2.2 |  |  |
|  | Semi-static channel access Note 4, 5 |  |  |  |  |  |
| Active cell |  |  | 1, 2 | E-UTRA cell 1 (PCell) and NR cell 2 with CCA (PSCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | 1, 2 | NR cell 3 |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [16]. |
| b2-Threshold1 |  | dBm | 1, 2 | Note 1 |  | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR |  | dBm | 1, 2 | Note 2 |  | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 3 for event B2 [16] |
| Hysteresis |  | dB | 1, 2 | 0 |  |  |
| CP length |  |  | 1, 2 | Normal |  |  |
| TimeToTrigger |  | s | 1, 2 | 0 |  |  |
| Filter coefficient |  |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  |  | 1, 2 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  |  | 1, 2 | 3s |  | Synchronous cells. |
| T1 |  | s | 1, 2 | 5 |  |  |
| T2 |  | s | 1, 2 | ≥ Tidentify_irat_cca_with_index | ≥ Tidentify_irat_cca_with_index | Tidentify_irat_cca_with_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| NOTE 1: The value of b2-Threshold1 is defined in Table A.10.4.4.3.1-3 NOTE 2: The value of b2-Threshold2NR is defined in Table A.10.4.4.3.1-4 NOTE 3: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 4: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 5: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |

Table A.10.4.4.3.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 with SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 for RSRP |  |
|  |  | 1, 2 | [TBD for RSRQ] |  |
|  | dB | 1, 2 | [TBD for SINR] |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.10.4.4.3.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T2 | T2 |
| NR RF Channel Number |  | 1, 2 | 2 |  |  | 3 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| PCCA_DL for dynamic channel access Note 6,8 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 7,8 |  | 1, 2 | PCCA_DL=0.9375 |  |  | PCCA_DL=0.9375 |  |
| PCCA_UL for dynamic channel access Note 6,8 |  | 1, 2 | 1 |  |  | 1 |  |
| PCCA_UL for semi-static channel access Note 7,8 |  | 1, 2 | 1 |  |  | 1 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |  | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |  | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |  | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |  | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |  | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |  | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2 | NA |  |  | -98 for SS-RSRP |  |
|  |  | 1, 2 | NA |  |  | [TBD for SS-RSRQ] |  |
|  | dB | 1, 2 | NA |  |  | [TBD For SS-SINR] |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |  | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |  | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -91 |  | -91 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | 4 |  | 4 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | 4 |  | 4 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -58.49 |  | -58.49 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | ETU70 |  |  | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |  | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |

##### A.10.4.4.3.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used

##### A.10.4.4.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are three cells: LTE cell 1 as PCell on E-UTRA RF channel 1, NR cell 2 as PSCell in FR1 with CCA on NR RF channel 1 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test parameters are given in Tables A.10.4.4.4.1-1, A.10.4.4.4.1-2, A.10.4.4.4.1-3 and A.10.4.4.4.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.10.4.4.4.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.10.4.4.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.10.4.4.4.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.10.4.4.4.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 | Test 2 | Test 3 | Test |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | 1 |  |  |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  |  | 1, 2 | 1,2 |  |  |  | Two FR1 NR carrier frequency under CCA is used. |
| DL CCA model | Dynamic channel accessNote 3, 5 |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |
|  | Semi-static channel access Note 4, 5 |  |  |  |  |  |  |  |
| UL CCA model | Dynamic channel accessNote 3, 5 |  |  | As specified in clause A.3.20.2.2 |  |  |  |  |
|  | Semi-static channel access Note 4, 5 |  |  |  |  |  |  |  |
| Active cell |  |  | 1, 2 | E-UTRA cell 1 (PCell) and NR cell 2 with CCA (PSCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | 1, 2 | NR cell 3 |  |  |  | NR cell 3 is on NR RF channel number 2. |
| Gap Pattern Id |  |  | 1, 2 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  |  | 1, 2 | 39 |  | 19 |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 |  | dBm | 1, 2 | Note 1 |  |  |  | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR |  | dBm | 1, 2 | Note 2 |  |  |  | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 3 for event B2 [16] |
| Hysteresis |  | dB | 1, 2 | 0 |  |  |  |  |
| CP length |  |  | 1, 2 | Normal |  |  |  |  |
| TimeToTrigger |  | s | 1, 2 | 0 |  |  |  |  |
| Filter coefficient |  |  | 1, 2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | 1, 2 | DRX.9 | DRX.10 | DRX.9 | DRX.10 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  |  | 1, 2 | 3s |  |  |  | Synchronous cells. |
| T1 |  | s | 1, 2 | 5 |  |  |  |  |
| T2 |  | s | 1, 2 | ≥Tidentify_irat_cca_with_index |  |  |  | Tidentify_irat_cca_with_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| NOTE 1: The value of b2-Threshold1 is defined in Table A.10.4.4.4.1-3 NOTE 2: The value of b2-Threshold2NR is defined in Table A.10.4.4.4.1-4 NOTE 3: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 4: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 5: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |

Table A.10.4.4.4.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 with SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 for RSRP |  |
|  |  | 1, 2 | [TBD for RSRQ] |  |
|  | dB | 1, 2 | [TBD for SINR] |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.10.4.4.4.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T2 | T2 |
| NR RF Channel Number |  | 1, 2 | 2 |  |  | 3 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |
| PCCA_DL for dynamic channel access Note 6,8 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 7,8 |  | 1, 2 | PCCA_DL=0.9375 |  |  | PCCA_DL=0.9375 |  |
| PCCA_UL for dynamic channel access Note 6,8 |  | 1, 2 | 1 |  |  | 1 |  |
| PCCA_UL for semi-static channel access Note 7,8 |  | 1, 2 | 1 |  |  | 1 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |  | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |  | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |  | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |  | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |  | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |  | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2 | NA |  |  | -98 for SS-RSRP |  |
|  |  | 1, 2 | NA |  |  | [TBD for SS-RSRQ] |  |
|  | dB | 1, 2 | NA |  |  | [TBD For SS-SINR] |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |  | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |  | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -91 |  | -91 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | 4 |  | 4 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | 4 |  | 4 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -58.49 |  | -58.49 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | ETU70 |  |  | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |  | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |  |  |  |

##### A.10.4.4.4.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.10.5	Measurement performance

### A.10.5.1	SS-RSRP

#### A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell

##### A.10.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.36.1.1 and 10.1.36.1.2 when the serving cell is subject to CCA.

##### A.10.5.1.1.2	Test parameters

In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell under CCA (Cell 2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model. Supported test configurations are shown in Table A.10.5.1.1.1-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in Table A.10.5.1.1.1-2. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.3.7A.2.1. In all test cases, Cell 2 is the PSCell, and Cell 3 is the target cell.

Table A.10.5.1.1.2-1: SS-RSRP Intra frequency SS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Table A.10.5.1.1.2-2: SS-RSRP Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Physical cell ID |  |  |  | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  |  |  |
| Duplex mode |  | Config 1, 2 |  | TDD |  |  |  |
| TDD configuration |  | Config 1, 2 |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  | Config 1, 2 | MHz | 40: NRB,c = 106 |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |
| TRS configuration |  | Config 1, 2 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1, 2 |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Control Channel RMC |  | Config 1, 2 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| SSB configuration | Semi-static channel access | Config 1, 2 |  | SSB.1 CCA (As defined in A.3.10A ) | SSB.1 CCA (As defined in A.3.10A ) | SSB.1 CCA (As defined in A.3.10A ) | SSB.1 CCA (As defined in A.3.10A ) |
|  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) | SSB.2 CCA (As defined in A.3.10A ) | SSB.2 CCA (As defined in A.3.10A ) | SSB.2 CCA (As defined in A.3.10A ) |
| Time offset with Cell 2 |  | Config 1, 2 | s | - | 3 | - | 3 |
| SMTC configuration |  | Config 1, 2 |  | TBD |  |  |  |
| DBT Window Configuration |  | Config 1, 2 |  | As defined in A.3.28.1 |  |  |  |
| DL CCA model |  | Config 1, 2 |  | As specified in clause A.3.20.2.1 |  |  |  |
| UL CCA model |  | Config 1, 2 |  | As specified in clause A.3.20.2.2 |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 | kHz | 30kHz |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | Config 1, 2 | NR_CCA_FR1_I | dBm/15KhZ | -94 |  | -110 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -109.5 |  |
| Note2 | Config 1, 2 | NR_CCA_FR1_I | dBm/SCS | -91 |  | -107.0 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -106.5 |  |
|  |  |  | dB | 2.46 | -5.97 | -0.01 | -4.76 |
|  |  |  | dB | 6 | 1 | 3 | 0 |
| SS-RSRPNote3 | Config 1, 2 | NR_CCA_FR1_I | dBm/SCS | -85 | -90 | -104.00 | -107.00 |
|  |  | NR_CCA_FR1_J |  |  |  | -103.50 | -106.50 |
| IoNote3 | Config 1, 2 | NR_CCA_FR1_I | dBm/ 38.16MHz | -51.99 |  | -69.94 |  |
|  |  | NR_CCA_FR1_J |  |  |  | -69.44 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

##### A.10.5.1.1.3	Test Requirements

The SS-RSRP measurement accuracy for cell 2 and cell 3 shall fulfil absolute requirement in clause 10.1.2.1.1 and relative requirement in clause 10.1.36.1.1 and 10.1.36.1.2.

#### A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell

##### A.10.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.37.1.1 and 10.1.37.1.2 for inter-frequency measurements with the testing configurations in Table A.10.5.1.2.1-1.

Table A.10.5.1.2.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations on each supported band |  |

##### A.10.5.1.2.2	Test parameters

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on a different frequency than the PSCell. The test parameters and applicability for Cell 1 are defined in A.3.7A.2.1. The test parameters for the Cell 2 and Cell 3 are given in Table A.10.5.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.10.5.1.2.2-1. The inter-frequency measurements are supported by a measurement gap.

Table A.10.5.1.2.2-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  | 1, 2 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel |  | 1, 2 | MHz | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| Gap pattern ID |  |  |  | 0 |  | 0 |  |
| Duplex mode |  | 1, 2 |  | TDD |  | TDD |  |
| TDD configuration |  | 1, 2 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| PDSCH Reference measurement channel |  | 1, 2 |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | 1, 2 |  | CR.1.1 CCA | - | CR.1.1 CCA | - |
| Dedicated CORESET Reference Channel |  | 1, 2 |  | CCR.1.1 CCA | - | CCR.1.1 CCA | - |
| SSB configuration | Semi-static channel access | 1, 2 |  | SSB.1 CCA (As defined in A.3.10A ) |  | SSB.1 CCA (As defined in A.3.10A ) |  |
|  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) |  | SSB.2 CCA (As defined in A.3.10A ) |  |
| OCNG Patterns |  | 1, 2 |  | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1, 2 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration |  | 1, 2 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1, 2 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| Time offset with Cell 2 |  | 1, 2 | s | - | 3 | - | 3 |
| SMTC configuration |  | 1, 2 |  | TBD |  | TBD |  |
| DBT Window Configuration |  | 1, 2 |  | As defined in A.3.28.1 |  | As defined in A.3.28.1 |  |
| DL CCA model |  |  |  | As specified in clause A.3.20.2.1 |  |  |  |
| UL CCA model |  |  |  | As specified in clause A.3.20.2.2 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | 1, 2 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | NR_CCA_FR1_I | 1, 2 | dBm/15kHz | -94.65 |  | ( for Cell 3 +8dB) | -111 |
|  | NR_CCA_FR1_J |  |  |  |  |  | -110.5 |
| Note2 | NR_CCA_FR1_I | 1, 2 | dBm/SSB SCS | -91.65 |  | ( for C 3 +8dB) | -109.00 |
|  | NR_CCA_FR1_J |  |  |  |  |  | -108.50 |
|  |  | 1, 2 | dB | 10 | 10 | 13 | -3 |
| SS-RSRPNote3 | NR_CCA_FR1_I | 1, 2 | dBm/SCS | -81.65 |  | (RSRP for Cell 3 +25dB) | -111.00 |
|  | NR_CCA_FR1_J |  |  |  |  |  | -110.50 |
| IoNote3 | R_CCA_FR1_I | 1, 2 | dBm/ 38.16MHz | -50.19 |  | (Io for Channel 3 +19.75dB) | -75.19 |
|  | NR_CCA_FR1_J |  |  |  |  |  | -74.69 |
|  |  | 1, 2 | dB | 10 | 10 | 13 | -3 |
| Propagation condition |  | 1, 2 | - | AWGN |  | AWGN |  |
| Antenna configuration |  |  |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

##### A.10.5.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 2 and Cell 3 shall fulfil the Absolute requirement in clause 10.1.4.1.1 and Relative requirement in clause 10.1.37.1.1 and 10.1.37.1.2.

### A.10.5.2	SS-RSRQ

#### A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell

##### A.10.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.29.1.1.

##### A.10.5.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table A.10.5.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.10.5.2.1.2-2. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.3.7A.2.1. In all test cases, Cell 2 is the PSCell and Cell 3 is the target cell.

Table A.10.5.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.10.5.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  |  | freq1 |  |  |  |
| Duplex mode |  | Config 1, 2 |  |  | TDD |  |  |  |
| TDD configuration |  | Config 1, 2 |  |  | TDDConf.1.1 CCA |  |  |  |
| BWchannel |  | Config 1, 2 |  | MHz | 40: NRB,c = 106 |  |  |  |
| BWP configuration |  | Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1, 2 |  |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Control Channel RMC |  | Config 1, 2 |  |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| TRS configuration |  | Config 1, 2 |  |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  |  | OP. 1 |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |
| Time offset with Cell 2 |  | Config 1, 2 |  | s | - | 3 | - | 3 |
| SMTC configuration |  | Config 1, 2 |  |  | TBD |  |  |  |
| SSB configuration |  | Semi-static channel access | Config 1, 2 |  | SSB.1 CCA (As defined in A.3.10A ) |  |  |  |
|  |  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |
| PDSCH/PDCCH |  | Config 1, 2 |  | kHz | 30kHz |  |  |  |
| subcarrier spacing |  |  |  |  |  |  |  |  |
| DBT Window Configuration |  | Config 1, 2 |  |  | As defined in A.3.28.1 |  |  |  |
| DL CCA model |  | Config 1, 2 |  |  | As specified in clause A.3.20.2.1 |  |  |  |
| UL CCA model |  | Config 1, 2 |  |  | As specified in clause A.3.20.2.2 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config1, 2 | NR_CCA_FR1_I |  | Bm/15kHz | -91 |  | -110 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  | -109.5 |  |
| Note2 | Config 1, 2 | NR_CCA_FR1_I |  | dBm/SC S | -88 |  | -107 |  |
|  |  | NR_CCA_FR1_J |  |  |  |  | -106.5 |  |
|  |  |  |  | dB | -1.76 |  | -5.46 | -5.46 |
|  |  |  |  | dB | 3 | 3 | -4 | -4 |
| SS-RSRPNote3 | Config 1, 2 | NR_CCA_FR1_I |  | dBm/SCS | -85 | -85 | -111 | -111 |
|  |  | NR_CCA_FR1_J |  |  |  |  | -110.5 | -110.5 |
| SS-RSRQ Note3 |  | NR_CCA_FR1_I |  | dB | -14.77 | -14.77 | -17.34 | -17.34 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |
| IoNote3 | Config 1, 2 | NR_CCA_FR1_I |  | dBm/ 38.16MHz | -50 |  |  | -73.4 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -72.9 |
| Propagation condition |  |  |  | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |

##### A.10.5.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.29.1.1.

#### A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell

##### A.10.5.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limitsThis test will verify the requirements in clause 10.1.30.1.1 and 10.1.30.1.2 for inter-frequency measurements with the testing configurations in Table A.10.5.2.2.2-1.

##### A.10.5.2.2.2	Test Parameters

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on a different frequency than the PSCell. The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 and Cell 3 are given in Table A.4.7.1.2.2-1 below. Both absolute and relative accuracy of RSRP inter-frequency measurements are tested by using the parameters in Table A.10.5.2.2.2-2. The inter-frequency measurements are supported by a measurement gap.

Table A.10.5.2.2.2-1: SS-RSRQ Inter frequency SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table A.10.5.2.2.2-2: SS-RSRQ Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | Config 1, 2 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1, 2 |  | TDDConf.1.1 CCA |  |  |  |  |  |
| BWchannel |  | Config 1, 2 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1, 2 | MHz | 40: NRB,c = 106 |  |  |  |  |  |
| Gap pattern ID |  |  |  | 0 |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  | SR.1.1 CCA |  | SR.1.1 CCA |  | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  | Config 1, 2 |  | CR.1.1 CCA |  | CR.1.1 CCA |  | CR.1.1 CCA |  |
| Dedicated CORESET Reference Channel |  | Config 1, 2 |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  | CCR.1.1 CCA |  |
| TRS configuration |  | Config 1, 2 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| Time offset with Cell 2 |  | Config 1, 2 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1, 2 |  | TBD |  |  |  |  |  |
| SSB configuration | Semi-static channel access | Config 1, 2 |  | SSB.1 CCA (As defined in A.3.10A) |  |  |  |  |  |
|  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A) |  |  |  |  |  |
| DBT Window Configuration |  | Config 1, 2 |  | As defined in A.3.28.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 | kHz | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1, 2 | NR_CCA_FR1_I | dBm/15kHz | -86.27 | -86.27 | -113 | -113 | -112 | -112 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -111.5 | -111.5 |
| Note2 | Config 1, 2 | NR_CCA_FR1_I | dBm/SCS | -83.27 | -83.27 | -110 | -110 | -109 | -109 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -108.5 | -108.5 |
|  |  |  | dB | -1.75 | -1.75 | -1.75 | -1.75 | 3 | -1.75 |
|  |  |  | dB | -1.75 | -1.75 | -1.75 | -1.75 | 3 | -1.75 |
| SS-RSRPNote3 | Config 1, 2 | NR_CCA_FR1_I | dBm/SCS | -85.02 | -85.02 | -111.75 | -111.75 | -106 | -110.75 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -105.5 | -110.25 |
| SS-RSRQ Note3 |  | NR_CCA_FR1_I | dB | -14.77 | -14.77 | -40.59 | -40.59 | -12.56 | -14.76 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  |  |  |
| IoNote3 | Config 1, 2 | NR_CCA_FR1_I | dBm/ 38.16MHz | -50 | -50 | -76.73 | -76.73 | -73.19 | -75.23 |
|  |  | NR_CCA_FR1_J |  |  |  |  |  | -72.69 | -74.73 |
| Propagation condition |  |  |  | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Section 3.5.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |

##### A.10.5.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in clause 10.1.30.1.1 and 10.1.30.1.2.

### A.10.5.3	SS-SINR

### A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell

#### A.10.5.4.1	SSB based L1-RSRP measurement

##### A.10.5.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.10.5.4.1.1-1.

Table A.10.5.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

##### A.10.5.4.1.2	Test parameters

In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell under CCA (Cell 2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 2. The test parameters and applicability for Cell 1 are defined in A.3.7A.2. The test parameters for the Cell 2 are given in Table A.10.5.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.10.5.4.1.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.10.5.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1,2 |  | freq1 | freq1 |
| DL CCA model |  | 1,2 |  | As specifieed in A.3.20.2.1 | As specifieed in A.3.20.2.1 |
| UL CCA model |  | 1,2 |  | As specified in A.3.20.2.2 | As specified in A.3.20.2.2 |
| Duplex mode |  | 1,2 |  | TDD | TDD |
| TDD Configuration |  | 1,2 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |
| BWchannel |  | 1,2 | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 |
| Duplex mode |  | 1,2 |  | TDD | TDD |
| TDD configuration |  | 1,2 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |
| PDSCH Reference measurement channel |  | 1,2 |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  | 1,2 |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  | 1,2 |  | CCR.1.1 CCA | CCR.1.1 CCA |
| SSB configuration for Semi-static channel access |  | 1,2 |  | SSB.3 CCA | SSB.3 CCA |
| SSB configuration for Dynamic channel access |  | 1,2 |  | SSB.4 CCA | SSB.4 CCA |
| OCNG Patterns |  | 1,2 |  | OP.1 | OP.1 |
| TRS configuration |  | 1,2 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1,2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1,2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration |  | 1,2 |  | DBT.1 | DBT.1 |
| reportConfigType |  | 1,2 |  | periodic | periodic |
| reportQuantity |  | 1,2 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS |  | 1,2 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1,2 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1,2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  | 1,2 |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_TDD_FR1_I | 1,2 | dBm/15kHz | -94.65 | [-113] |
| Note2 | NR_TDD_FR1_I | 1,2 | dBm/SCS | -91.65 | [-110] |
|  |  | 1,2 | dB | 10 | -3 |
| SS-RSRPNote3 | NR_TDD_FR1_I | 1,2 | dBm/SCS | -81.65 | [-113] |
| IoNote3 | NR_TDD_FR1_I | 1,2 | dBm/ 38.16MHz | -50.19 | [-77.19] |
|  |  | 1,2 | dB | 10 | -3 |
| Propagation condition |  | 1,2 |  | AWGN | AWGN |
| Antenna configuration |  | 1,2 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |

##### A.10.5.4.1.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in clauses 10.1.33.1.

### A.10.5.5	RSSI

### A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA

#### A.10.5.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the RSSI measurement accuracy is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in Section 10.1.34.1.

#### A.10.5.5.1.2	Test parameters

In all test cases, Cell 1 is E-UTRAN PCell on a licensed band, and Cell 2 is PSCell operating on a carrier frequency under CCA. RSSI is measured on channel number 1. Supported test configurations are shown in table A.10.5.5.1.2-1. The accuracy of RSSI intra-frequency measurements is tested by using the parameters in A.10.5.5.1.2-2 and A.10.5.5.1.2-3. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.5.5.1.2-1: RSSI supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.5.5.1.2-2: RSSI test parameters

| Parameter |  | Configurations | Unit | Test 1 |
| --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |
| RF Channel Number |  |  |  | 1 |
| BWchannel |  |  | MHz | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2 |  | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2 |  | SSB.2 CCA |
| PCCA_DL |  |  |  | TBD |
| PCCA_UL |  |  |  | TBD |
| DL CCA model |  |  |  | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |
| Channel access bandwidth |  |  | MHz | 20 |
| DRX Cycle configuration |  |  | ms | Not Applicable |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -87 |
| Propagation condition |  |  | - | AWGN |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.10.5.5.1.2-3: RSSI RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.10.5.5.1.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 10.1.34.1. The nominal RSSI used to evaluate the requirement shall be based on Io in slots corresponding to RSSI measurement time configuration (RMTC).

### A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA

#### A.10.5.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the RSSI measurement accuracy is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in Section 10.1.34.1.

#### A.10.5.5.2.2	Test parameters

In all test cases, Cell 1 is E-UTRAN PCell on a licensed band, Cell 2 is PSCell operating on a carrier frequency under CCA, Cell 3 is SCell on a carrier frequency under CCA. RSSI is measured on channel number 2. Supported test configurations are shown in table A.10.5.5.2.2-1. The accuracy of RSSI intra-frequency measurements is tested by using the parameters in A.10.5.5.2.2-2 and A.10.5.5.2.2-3. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.5.5.2.2-1: RSSI supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.5.5.2.2-2: RSSI test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.10.5.5.2.2-3: RSSI RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.10.5.5.2.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 10.1.34.1. The nominal RSSI used to evaluate the requirement shall be based on Io in slots corresponding to RSSI measurement time configuration (RMTC).

### A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA

#### A.10.5.5.3.1	Test Purpose and Environment

The purpose of this test is to verify that the RSSI measurement accuracy is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in Section 10.1.34.2.

#### A.10.5.5.3.2	Test parameters

In all test cases, Cell 1 is E-UTRAN PCell on a licensed band, Cell 2 is PSCell operating on a carrier frequency under CCA, and Cell 3 is the neighbour with CCA. RSSI is measured on channel number 2. Supported test configurations are shown in table A.10.5.5.3.2-1. The accuracy of RSSI inter-frequency measurements is tested by using the parameters in A.10.5.5.3.2-2 and A.10.5.5.3.2-3. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.5.5.3.2-1: RSSI supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.5.5.3.2-2: RSSI test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 TDD | NA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 TDD | NA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 TDD | NA |
| OCNG Patterns |  |  |  | OP.1 | NA |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | NA |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.10.5.5.3.2-3: RSSI RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.10.5.5.3.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 10.1.34.2. The nominal RSSI used to evaluate the requirement shall be based on Io in slots corresponding to RSSI measurement time configuration (RMTC).

### A.10.5.6	Channel occupancy

### A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA

#### A.10.5.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy measurement accuracy is within the specified limits. This test will partially verify the channel occupancy measurement accuracy requirements in Section 10.1.35.1.

#### A.10.5.6.1.2	Test parameters

In all test cases, Cell 1 is E-UTRAN PCell on a licensed band, and Cell 2 is PSCell operating on a carrier frequency under CCA. Channel occupancy is measured on channel number 1. Supported test configurations are shown in table A.10.5.6.1.2-1. The accuracy of channel occupancy intra-frequency measurements is tested by using the parameters in A.10.5.6.1.2-2 and A.10.5.6.1.2-3. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.5.6.1.2-1: CO supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.5.6.1.2-2: CO test parameters

| Parameter |  | Configurations | Unit | Test 1 |
| --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |
| RF Channel Number |  |  |  | 1 |
| BWchannel |  |  | MHz | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2 |  | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2 |  | SSB.2 CCA |
| PCCA_DL |  |  |  | TBD |
| PCCA_UL |  |  |  | TBD |
| DL CCA model |  |  |  | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |
| Channel access bandwidth |  |  | MHz | 20 |
| DRX Cycle configuration |  |  | ms | Not Applicable |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -87 |
| Propagation condition |  |  | - | AWGN |
| channelOccupancyThreshold |  |  | dBm | -83 |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.10.5.6.1.2-3: CO RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.10.5.6.1.3	Test Requirements

The nominal reported channelOccupancy shall be TBD. At least 90% of channel occupancy reports made by the UE shall indicate this value.

### A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA

#### A.10.5.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy measurement accuracy is within the specified limits. This test will partially verify the channel occupancy measurement accuracy requirements in Section 10.1.35.1.

#### A.10.5.6.2.2	Test parameters

In all test cases, Cell 1 is E-UTRAN PCell on a licensed band, Cell 2 is PSCell operating on a carrier frequency under CCA, Cell 3 is SCell on a carrier frequency under CCA. Channel occupancy is measured on channel number 2. Supported test configurations are shown in table A.10.5.6.2.2-1. The accuracy of channel occupancy intra-frequency measurements is tested by using the parameters in A.10.5.6.2.2-2 and A.10.5.6.2.2-3. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.5.6.2.2-1: CO supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.5.6.2.2-2: CO test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| channelOccupancyThreshold |  |  | dBm | -83 |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.10.5.6.2.2-3: CO RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.10.5.6.2.3	Test Requirements

The nominal reported channelOccupancy shall be TBD. At least 90% of channel occupancy reports made by the UE shall indicate this value.

### A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA

#### A.10.5.6.3.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy measurement accuracy is within the specified limits. This test will partially verify the channel occupancy measurement accuracy requirements in Section 10.1.35.2.

#### A.10.5.6.3.2	Test parameters

In all test cases, Cell 1 is E-UTRAN PCell on a licensed band, Cell 2 is PSCell operating on a carrier frequency under CCA, and Cell 3 is the neighbour with CCA. Channel occupancy is measured on channel number 2. Supported test configurations are shown in table A.10.5.6.3.2-1. The accuracy of channel occupancy inter-frequency measurements is tested by using the parameters in A.10.5.6.3.2-2 and A.10.5.6.3.2-3. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.10.5.6.3.2-1: CO supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.10.5.5.3.2-2: CO test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 TDD | NA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 TDD | NA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 TDD | NA |
| OCNG Patterns |  |  |  | OP.1 | NA |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | NA |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| channelOccupancyThreshold |  |  | dBm | -83 |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.10.5.6.3.2-3: CO RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.10.5.6.3.3	Test Requirements

The nominal reported channelOccupancy shall be TBD. At least 90% of channel occupancy reports made by the UE shall indicate this value.

# A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1

Editor’s note: Test cases for NR SA with NR PCell under CCA and SCell under CCA are also included here.

## A.11.1	RRC_IDLE state mobility

### A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA

#### A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell

##### A.11.1.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements subject to CCA specified in clause 4.2A.2.3. Supported test configurations are shown in table A. 11.1.1.1.2-1.

##### A.11.1.1.1.2	Test Parameters

The test scenario comprises of 1 NR carrier that is subject to CCA and 2 cells as given in tables A.11.1.1.1.2-1, A.11.1.1.1.2-2 and A.11.1.1.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.11.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.1.1.1.2-2: General test parameters for intra frequency NR cell re-selection test case when subject to CCA

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial | Active cell |  | 1 | Cell1 |  |
| condition | Neighbour cells |  | 1 | Cell2 |  |
| T2 end condition | Active cell |  | 1 | Cell2 |  |
|  | Neighbour cells |  | 1 | Cell1 |  |
| Final condition | Active cell |  | 1 | Cell1 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration | Semi-static channel access |  | 1 | SSB.1 CCA | (As defined in A.3.10A ) |
|  | Dynamic channel access |  |  | SSB.2 CCA |  |
| DBT Window Configuration |  |  | 1 | DBT.1 | As specified in clause A.3.28.1. |
| SMTC confituration |  |  |  | SMTC.1 |  |
| DL CCA model |  |  | 1 | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | 1 | As specified in clause A.3.26.2.2 |  |
| DRX cycle length |  | s | 1 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1 | Not configured |  |
| T1 |  | s | 1 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.11.1.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case in AWGN when subject to CCA

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1 | 0.9 |  |  | 0.9 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1 | 0.75 |  |  | 0.75 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1 | 0.5 |  |  | 0.5 |  |  |
| UL CCA probability PCCA_UL |  | 1 | 1 |  |  | 1 |  |  |
| Md,max |  | 1 | 16 |  |  | 16 |  |  |
| Mm,max |  | 1 | 4 |  |  | 4 |  |  |
| Me,max |  | 1 | 8 |  |  | 8 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 CCA |  |  | SR.1.1 CCA |  |  |
| RMSI CORESET |  | 1 | CR.1.1 CCA |  |  | CR.1.1 CCA |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 CCA |  |  | CCR.1.1 CCA |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -127 |  |  | -127 |  |  |
| Pcompensation | dB | 1 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
| Note2 | dBm/SCS | 1 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  | dB | 1 | 16 | 13 | 16 | -infinity | 16 | 13 |
| SS-RSRP Note3 | dBm/SCS | 1 | -79 | -82 | -79 | -infinity | -79 | -82 |
| Io | dBm/38.16 MHz | 1 | -47.85 | -46.12 | -46.12 | Same as parameters specified in Cell 1 columns- |  |  |
| Treselection | s | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |

##### A.11.1.1.1.3	Test Requirements

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than (25 + Md)*1.28 + TSI_CCA s. Md is the number of DRX cycles with at least one SMTC where there are no SSBs available during the Tdetect,NR_Intra_CCA. If Md > Md,max the UE is required to restart the detection of Cell 2.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell shall be less than (5+Me)*1.28 + TSI_CCA s. Me is the number of DRX cycles with at least one SMTC where there are no SSBs available during the Tevaluate,NR_Intra_CCA. If Me > Me,max the UE is required to restart the evaluation of Cell 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Tdetect, NR_Intra_CCA + TSI_CCA, and to an already detected cell can be expressed as: Tevaluate, NR_ intra_CCA + TSI_CCA,

Where:

Tdetect, NR_Intra_CCA	See Table 4.2A.2.3-1 in clause 4.2A.2.3

Tevaluate, NR_ intra_CCA	See Table 4.2A.2.3-1 in clause 4.2A.2.3

TSI_CCA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell.

This gives a total of (25 + Md)*1.28 + TSI_CCA s for the cell re-selection delay to a newly detectable cell and (5+Me)*1.28 + TSI_CCA s for the cell re-selection delay to an already detected cell in the test case.

#### A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell

##### A.11.1.1.2.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements subject to CCA specified in clause 4.2A.2.4. Supported test configurations are shown in table A.11.1.1.2.2-1.

##### A.11.1.1.2.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers that are subject to CCA respectively as given in tables A.11.1.1.2.2-1, A.11.1.1.2.2-2 and A.11.1.1.2.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

Table A.11.1.1.2.2-1: Supported test configurations

| Configuration | Description of cell 1 with CCA | Description of cell 2 with CCA |
| --- | --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.1.1.2.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case when subject to CCA

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
| T1 end condition | Active cell |  | 1 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1 | Cell2 |  |
| T3 end condition | Active cell |  | 1 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
| RF Channel Number |  |  | 1 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration | Semi-static channel access |  | 1 | SSB.1 CCA (As defined in A.3.10A ) |  |
|  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) |  |
| DBT Window Configuration |  |  | 1 | DBT.1 | As specified in clause A.3.28.1. |
| SMTC configuration |  |  | 1 | SMTC.1 |  |
| DL CCA model |  |  | 1 | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | 1 | As specified in clause A.3.26.2.2 |  |
| DRX cycle length |  | s | 1 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1 | Not configured |  |
| T1 |  | s | 1 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.11.1.1.2.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1 | 0.9 |  |  | 0.9 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1 | 0.75 |  |  | 0.75 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 2 | 0.5 |  |  | 0.5 |  |  |
| UL CCA probability PCCA_UL |  | 1 | 1 |  |  | 1 |  |  |
| Md,max |  | 1 | 16 |  |  | 16 |  |  |
| Mm,max |  | 1 | 4 |  |  | 4 |  |  |
| Me,max |  | 1 | 8 |  |  | 8 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 CCA |  |  | SR.1.1 CCA |  |  |
| RMSI CORESET |  | 1 | CR.1.1 CCA |  |  | CR.1.1 CCA |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 CCA |  |  | CCR.1.1 CCA |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
| Note2 | dBm/SCS | 1 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
| SS-RSRP Note3 | dBm/SCS | 1 | -81 | -81 | -81 | -99 | -infinity | -83 |
| Io | dBm/38.16 MHz | 1 | -49.79 | -49.79 | -49.79 | -62.50 | -infinity | -51.69 |
| Treselection | s | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1 | 50 |  |  | 50 |  |  |
| Threshx, high | dB | 1 | 48 |  |  | 48 |  |  |
| Threshserving, low | dB | 1 | 44 |  |  | 44 |  |  |
| Threshx, low | dB | 1 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |

##### A.11.1.1.2.3	Test Requirements

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 60 + 1.28 x (5 + Me) + TSI_CCA s. Me is the number of DRX cycles with at least one SMTC where there are no SSBs available during the Tevaluate,NR_Intra_CCA. If Me > Me,max the UE is required to restart the evaluation of cell 2.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to a lower priority cell shall be less than 1.28 x (5 + Me) + TSI_CCA s. Me is the number of DRX cycles with at least one SMTC where there are no SSBs available during the Tevaluate,NR_Intra_CCA. If Me > Me,max the UE is required to restart the evaluation of cell 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter_CCA + TSI_CCA, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter_CCA + TSI_CCA,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, NR_ inter_CCA	See Table 4.2A.2.4-1 in clause 4.2A.2.4

TSI_CCA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell.

This gives a total of 60 + 1.28 x (5 + Me) + TSI_CCA s for the cell re-selection delay to a higher priority cell and 1.28 x (5 + Me) + TSI_CCA s for the cell re-selection delay to a lower priority cell in the test case.

### A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA

#### A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA

##### A.11.1.2.1.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause 4.2.2.4 when the serving cell is subject to CCA. Supported test configurations are shown in table A.11.1.2.1.2-1.

##### A.11.1.2.1.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers where the first carrier is subject to CCA as given in tables A.11.1.2.1.2-1, A.11.1.2.1.2-2 and A.11.1.2.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

Table A.11.1.2.1.2-1: Supported test configurations

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.11.1.2.1.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case when serving cell is subject to CCA

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 which is subject to CCA in the initial phase and during T1 period the UE reselects to cell 1 which is an inter-frequency NR cell |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | Cell 1: SSB.1 FR1 Cell 2: SSB.1 CCA for semi-static channel access; Cell 2: SSB.2 CCA for dynamic channel access |  |
|  |  |  | 2 | Cell 1: SSB.1 FR1 Cell 2: SSB.1 CCA for semi-static channel access; Cell 2: SSB.2 CCA for dynamic channel access |  |
|  |  |  | 3 | Cell 1: SSB.2 FR1 Cell 2: SSB.1 CCA for semi-static channel access; Cell 2: SSB.2 CCA for dynamic channel access |  |
| SMTC configuration |  |  | 1 | Cell 1: SMTC pattern 2 Cell 2: N/A |  |
|  |  |  | 2 | Cell 1: SMTC pattern 1 Cell 2: N/A |  |
|  |  |  | 3 | Cell 1: SMTC pattern 1 Cell 2: N/A |  |
| DBT Window Configuration |  |  | 1, 2, 3 | Cell 1: N/A Cell 2: DBT.1 | As specified in clause A.3.28.1. |
| DL CCA model |  |  | 1, 2, 3 | Cell 1: N/A Cell 2: As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | 1, 2, 3 | Cell 1: N/A Cell 2: As specified in clause A.3.26.2.2 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.11.1.2.1.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN when serving cell is subject to CCA

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  |  | TDDConf.1.1.CCA |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1.CCA |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.1.1.CCA |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1, 2, 3 | N/A |  |  |  | 0.9 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1, 2, 3 | N/A |  |  |  | 0.75 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1, 2, 3 | N/A |  |  |  | 0.5 |  |  |
| UL CCA probability PCCA_UL |  | 1, 2, 3 | N/A |  |  |  | 1 |  |  |
| Md,max |  | 1, 2, 3 | N/A |  |  |  | 16 |  |  |
| Mm,max |  | 1, 2, 3 | N/A |  |  |  | 4 |  |  |
| Me,max |  | 1, 2, 3 | N/A |  |  |  | 8 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  |  | SR.1.1 CCA |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  |  | SR.1.1 CCA |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  | SR.1.1 CCA |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  |  | CR.1.1 CCA |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  |  | CR.1.1 CCA |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | CR.1.1 CCA |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  |  | CCR.1.1 CCA |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  |  | CCR.1.1 CCA |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | CCR.1.1 CCA |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  |  | -137 |  |  |
|  |  | 3 | -137 |  |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 |  | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  | -95 |  |  |  |
|  |  | 2 | -98 |  |  | -95 |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | 14 |  | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -84 |  | -102 | -infinity | -83 |
|  |  | 2 | -84 | -84 | -84 |  | -102 | -infinity | -83 |
|  |  | 3 | -81 | -81 | -81 |  | -99 | -infinity | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -55.88 |  | -68.60 | -- | -- |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -55.88 |  | -68.60 | -- | -- |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -49.79 |  | -62.50 | -63.96 | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 |  | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | 50 |  |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |

##### A.11.1.2.1.3	Test Requirements

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 60 + 1.28 x (5 + Me) + TSI_CCA s. Me is the number of DRX cycles with at least one SMTC where there are no SSBs available during the Tevaluate,NR_Intra_CCA. If Me > Me,max the UE is required to restart the evaluation of cell 2.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter_CCA + TSI_CCA, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR.

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, NR_ inter_CCA	See Table 4.2A.2.4-1 in clause 4.2A.2.4

TSI_CCA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell.

Tevaluate, NR_ inter	See Table 4.2.2.4-1 in clause 4.2.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test cases.

This gives a total of 60 + 1.28 x (5 + Me) + TSI_CCA s for the cell re-selection delay to a higher priority cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

### A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA

#### A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA

##### A.11.1.3.1.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements specified in clause  4.2A.2.4 when the target cell is subject to CCA. Supported test configurations are shown in table A. 11.1.3.1.2-1.

##### A.11.1.3.1.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers where the second carrier is subject to CCA as given in tables A.11.1.3.1.2-1, A.11.1.3.1.2-2 and A.11.1.3.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

Table A.11.1.3.1.2-1: Supported test configurations

| Configuration | Description of a cell without CCA | Description of a cell with CCA |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.11.1.3.1.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case when target cell is subject to CCA

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 which is an inter-frequency NR cell in the initial phase and during T1 period the UE reselects to cell 1 which is cell subject to CCA |
|  | Neighbour cell |  | 1, 2, 3 | Cell 1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
|  | Neighbour cell |  | 1, 2, 3 | Cell 1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | Cell 1: SSB.1 CCA for semi-static channel access; Cell 1: SSB.2 CCA for dynamic channel access; Cell 2: SSB.1 FR1 |  |
|  |  |  | 2 | Cell 1: SSB.1 CCA for semi-static channel access; Cell 1: SSB.2 CCA for dynamic channel access; Cell 2: SSB.1 FR1 |  |
|  |  |  | 3 | Cell 1: SSB.1 CCA for semi-static channel access; Cell 1: SSB.2 CCA for dynamic channel access; Cell 2: SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | Cell 1: SMTC.1 Cell 2: SMTC.2 |  |
|  |  |  | 2 | Cell 1: SMTC.1 Cell 2: SMTC.1 |  |
|  |  |  | 3 | Cell 1: SMTC.1 Cell 2: SMTC.1 |  |
| DBT Window Configuration |  |  | 1, 2, 3 | Cell 1: DBT.1 Cell 2: N/A | As specified in clause A.3.28.1. |
| DL CCA model |  |  | 1, 2, 3 | Cell 1: As specified in clause A.3.26.2.1 Cell 2: N/A |  |
| UL CCA model |  |  | 1, 2, 3 | Cell 1: As specified in clause A.3.26.2.2 Cell 2: N/A |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.11.1.3.1.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN when target cell is subject to CCA

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| TDD configuration |  | 1 | TDDConf.1.1.CCA |  |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1.CCA |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.1.1.CCA |  |  |  | TDDConf.2.1 |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1, 2, 3 | 0.9 |  |  |  | N/A |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1, 2, 3 | 0.75 |  |  |  | N/A |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1, 2, 3 | 0.5 |  |  |  | N/A |  |  |
| UL CCA probability PCCA_UL |  | 1, 2, 3 | 1 |  |  |  | N/A |  |  |
| Md,max |  | 1, 2, 3 | 16 |  |  |  | N/A |  |  |
| Mm,max |  | 1, 2, 3 | 4 |  |  |  | N/A |  |  |
| Me,max |  | 1, 2, 3 | 8 |  |  |  | N/A |  |  |
| PDSCH RMC |  | 1 | SR.1.1 CCA |  |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 CCA |  |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.1.1 CCA |  |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 CCA |  |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 CCA |  |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.1.1 CCA |  |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 CCA |  |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 CCA |  |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.1.1 CCA |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -137 |  |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 |  | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -95 |  |  | -98 |  |  |  |
|  |  | 2 | -95 |  |  | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | 14 |  | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -81 | -81 | -81 |  | -102 | -infinity | -86 |
|  |  | 2 | -81 | -81 | -81 |  | -102 | -infinity | -86 |
|  |  | 3 | -81 | -81 | -81 |  | -99 | -infinity | -83 |
| Io | dBm/9.36 MHz | 1 | -- | -- | -- |  | -68.60 | -70.05 | -57.78 |
|  | dBm/9.36 MHz | 2 | -- | -- | -- |  | -68.60 | -70.05 | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -49.79 |  | -62.50 | -63.96 | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 |  | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | 50 |  |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |

##### A.11.1.3.1.3	Test Requirements

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

1.28 x (5 + Me) + TSI_CCA s. Me is the number of DRX cycles with at least one SMTC where there are no SSBs available during the Tevaluate,NR_Intra_CCA. If Me > Me,max the UE is required to restart the evaluation of cell 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR.

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, NR_ inter_CCA	See Table 4.2A.2.4-1 in clause 4.2A.2.4

TSI_CCA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell.

Tevaluate, NR_ inter	See Table 4.2.2.4-1 in clause 4.2.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.s

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority cell and 1.28 x (5 + Me) + TSI_CCA s for the cell re-selection delay to a lower priority cell in the test case.

### A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA

#### A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA

##### A.11.1.4.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR cell subject to CCA to E-UTRAN inter-RAT cell reselection requirements specified in clause 4.2A.2.5 when the E-UTRAN cell is of higher priority.

##### A.11.1.4.1.2	Test Parameters

The test scenario comprises of one NR cell which is subject to CCA and one E-UTRAN cell as given in tables A.11.1.4.1.2-1, A.11.1.4.1.2-2, A.11.1.4.1.2-3 and A.11.1.4.1.2-4. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. NR cell 1 is already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of higher priority than cell 1.

Table A.11.1.4.1.2-1: Supported test configurations

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.11.1.4.1.2-2: General test parameters for NR cell subject to CCA to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 | The UE camps on cell 1 in the initial phase and during T2 period the UE reselects to cell 2. |
| T2 end | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell |
| condition | Neighbour cells |  | 1, 2 | Cell1 | 2 during T2. |
| T3 end | Active cell |  | 1, 2 | Cell1 | The UE shall perform reselection to cell |
| condition | Neighbour cells |  | 1, 2 | Cell2 | 1 during T3 for iteration of the tests. |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DBT Window Configuration |  |  | 1, 2 | DBT.1 | As specified in clause A.3.28.1. |
| SSB configuration |  |  |  | Cell 1: SSB.1 CCA for semi-static channel access; Cell 1: SSB.2 CCA for dynamic channel access; |  |
| DL CCA model |  |  | 1, 2 | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | 1, 2 | As specified in clause A.3.26.2.2 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
|  |  |  | 2 | 4 |  |
| E-UTRAN PRACH |  |  | 1 | 53 | As specified in table 5.7.1-2 in |
| configuration index |  |  | 2 | 4 | TS 36.211 [23] |
| T1 |  | s | 1, 2 | >7 | During T1, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1, 2 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.11.1.4.1.2-3: Cell specific test parameters for NR cell 1 subject to CCA

| Parameter | Unit | Test configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.1.1.CCA |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1, 2 | 0.9 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1, 2 | 0.75 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1, 2 | 0.5 |  |  |
| UL CCA probability PCCA_UL |  | 1, 2 | 1 |  |  |
| Md,max |  | 1, 2 | 16 |  |  |
| Mm,max |  | 1, 2 | 4 |  |  |
| Me,max |  | 1, 2 | 8 |  |  |
| PDSCH parameters |  | 1, 2 | SR.1.1 CCA |  |  |
| RMSI CORESET parameters |  | 1, 2 | CR.1.1 CCA |  |  |
| Dedicated CORESET parameters |  | 1, 2 | CCR.1.1 CCA |  |  |
| SSB parameters |  | 1, 2 | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access |  |  |
| NR SMTC parameters |  | 1, 2 | SMTC.1 |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -137 |  |  |
|  | dBm/SCS | 1, 2 | -95 |  |  |
|  | dBm/15 kHz | 1, 2 | -98 |  |  |
| SS-RSRP |  | 1, 2 | -81 | -81 | -81 |
|  | dB | 1, 2 | 14 | 14 | 14 |
|  | dB | 1, 2 | 14 | 14 | 14 |
| Io | dBm/38.16 MHz | 1, 2 | -49.79 | -49.79 | -49.79 |
| Treselection | S | 1, 2 | 0 |  |  |
| Snonintrasearch | dB | 1, 2 | 50 |  |  |
| Threshx, high (Note 2) | dB | 1, 2 | 48 |  |  |
| Threshserving, low | dB | 1, 2 | 44 |  |  |
| Threshx, low | dB | 1, 2 | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

Table A.11.1.4.1.2-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |
| RSRP | dBm/15 KHz | -infinity |  | -86 | -102 |
|  | dB | -infinity |  | 12 | -4 |
|  | dB | -infinity |  | 12 | -4 |
| TreselectionEUTRAN | S | 0 |  |  |  |
| Snonintrasearch | dB | 50 |  |  |  |
| Threshx, high (Note 2) | dB | 48 |  |  |  |
| Threshserving, low | dB | 44 |  |  |  |
| Threshx, low | dB | 50 |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |  |  |

##### A.11.1.4.1.3	Test Requirements

The cell reselection delay to a higher priority E-UTRAN cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Thigher_priority_search	See clause 4.2.2.7

Tevaluate, E-UTRAN	See Table 4.2.2.5-1 in clause 4.2.2.5

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority E-UTRAN cell.

#### A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA

##### A.11.1.4.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR cell subject to CCA to E-UTRAN inter-RAT cell reselection requirements specified in clause 4.2A.2.5 when the E-UTRAN cell is of lower priority.

The test scenario comprises of one NR cell and one E-UTRAN cell as given in tables A.11.1.4.2.1-1, A.11.1.4.2.1-2, A.11.1.4.2.1-3 and A.11.1.4.2.1-4. The test consists of three successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

Table A.11.1.4.2.1-1: Supported test configurations

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.11.1.4.2.1-2: General test parameters for NR cell subject to CCA to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 | The UE camps on cell 1 in the initial phase. |
| T1 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DBT Window Configuration |  |  | 1, 2 | DBT.1 | As specified in clause A.3.28.1. |
| DL CCA model |  |  | 1, 2 | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | 1, 2 | As specified in clause A.3.26.2.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH |  |  | 1 | 53 | As specified in table 5.7.1-2 in TS 36.211 [23] |
| configuration index |  |  | 2 | 4 |  |
| T1 |  | s | 1, 2 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.11.1.4.2.1-3: Cell specific test parameters for NR cell 1 subject to CCA

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.1.1.CCA |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1, 2 | 0.9 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1, 2 | 0.75 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1, 2 | 0.5 |  |
| UL CCA probability PCCA_UL |  | 1, 2 | 1 |  |
| Md,max |  | 1, 2 | 16 |  |
| Mm,max |  | 1, 2 | 4 |  |
| Me,max |  | 1, 2 | 8 |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 CCA |  |
| RMSI CORESET RMC Configuration |  | 1, 2 | CR.1.1 CCA |  |
| Dedicated CORESET RMC |  | 1, 2 | CCR.1.1 CCA |  |
| Configuration |  |  |  |  |
| SSB configuration |  | 1, 2 | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0 |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0 |  |
| RLM-RS |  | 1, 2 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -137 |  |
|  | dBm/SCS | 1, 2 | -95 |  |
|  | dBm/15 kHz | 1, 2 | -98 |  |
| SS-RSRP | dBm/SCS | 1, 2 | -99 | -83 |
|  | dB | 1, 2 | -4 | 12 |
|  | dB | 1, 2 | -4 | 12 |
| Io | dBm/38.16 MHz | 1, 2 | -62.50 | -51.69 |
| Treselection | S | 1, 2 | 0 |  |
| Snonintrasearch | dB | 1, 2 | 50 |  |
| Threshx, high (Note 2) | dB | 1, 2 | 48 |  |
| Threshserving, low | dB | 1, 2 | 44 |  |
| Threshx, low | dB | 1, 2 | 50 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |

Table A.11.1.4.2.1-4: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -84 | -84 |
|  | dB | 14 | 14 |
|  | dB | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| Snonintrasearch | dB | 50 |  |
| Threshx, high (Note 2) | dB | 48 |  |
| Threshserving, low | dB | 44 |  |
| Threshx, low | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, high which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

##### A.11.1.4.2.2	Test Requirements

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN	See Table 4.2.2.5-1 in clause 4.2.2.5

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 7.68 s, allow 8 s for the cell re-selection delay to a lower priority E-UTRAN cell.

## A.11.2	RRC_CONNECTED state mobility

### A.11.2.1	Handover

#### A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell

##### A.11.2.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR intra frequency handover requirements from FR1 carrier under CCA to FR1 carrier under CCA specified in clause 6.1B.1.2.

##### A.11.2.1.1.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.1.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.1.2-2, and A.11.2.1.1.2-3.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.11.2.1.1.2-1: Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.1.1.2-2: General test parameters Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | On the carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.20.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.20.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T304 |  | ms | 500 |  |
| LCCA_DL |  |  | 5 |  |
| WCCA_DL |  | ms | T304 |  |
| LCCA_UL |  |  | 5 |  |
| WCCA_UL |  | ms | T304 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | ≥ Tinterrupt | Tinterrupt is defined in clause 6.1B.1.2 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.1.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T3 |  | T1 |  | T2 |  | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |  | 1 |  |  |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  | - | PCCA_DL=0.9375 |  |  |  |  |  | PCCA_DL=0.9375 |  |  |  |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  | - | 0.75 |  |  |  |  |  | 0.75 |  |  |  |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  | - | 0.87 |  |  |  |  |  | 0.87 |  |  |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 CCA |  |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 CCA |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| DBT window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |  |  |  |  |  |
| SSB configuration for semi-static channel accessNote 4, 6 |  | Config 1 |  | SSB.1 CCA |  |  |  |  |  |  |  |  |  |  |
| SSB configuration for dynamic channel accessNote 5, 6 |  | Config 1 |  | SSB.2 CCA |  |  |  |  |  |  |  |  |  |  |
| ssb-PositionQCL |  | Config 1 |  | [1] |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -95 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | dB | 8 | -3.3 |  | -3.3 |  | -Infinity |  | 2.36 |  | 2.36 |  |
|  |  |  | dB | 8 | 8 |  | 8 |  | -Infinity |  | 11 |  | 11 |  |
| SSB_RP | Config 1 |  | dBm/SCS | -87 | -87 |  | -87 |  | -Infinity |  | -84 |  | -84 |  |
| IoNote3 | Config 1 |  | dBm/ 38.16MHz | -55.31 | -50.96 |  | -50.96 |  | -55.31 |  | -50.96 |  | -50.96 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.11.2.1.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tinterrupt from the beginning of time period T3, where Tinterrupt is defined in clause 6.1B.1.2

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin

Tsearch = 0.

Tprocessing = 20 ms.

Tmargin = 2 ms.

T∆ = (1+ L2) *20 ms.

TIU = (1+ L3)*10 + 10 ms

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2], L2 is the number of SMTC occasions not available at the UE during the time tracking period where L2  LCCA_DL, and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure, where L3  LCCA_UL. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. The interruption time considering the potential extensions caused by L1, L2 , L3  and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer. The UE behaviour at the T304 timer expiry is detailed in TS 38.331 [2]. Test equipment should make sure that LCCA_DL and LCCA_UL are not exceeded during a test by monitoring the number of CCA failures and preventing additional CCA failures from happening after LCCA_DL or LCCA_UL is reached.

#### A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell

##### A.11.2.1.2.1	Test Purpose and Environment

This test is to verify the requirement intra frequency handover requirements from FR1 carrier under CCA to FR1 carrier under CCA specified in clause 6.1B.1.2.

##### A.11.2.1.2.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.2.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.2.2-2, and A.11.2.1.2.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.11.2.1.2.2-1: Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.1.2.2-2: General test parameters Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | On the carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.20.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.20.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T304 |  | ms | 500 |  |
| LCCA_DL |  |  | 5 |  |
| WCCA_DL |  | ms | T304 |  |
| LCCA_UL |  |  | 5 |  |
| WCCA_UL |  | ms | T304 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | ≥ Tinterrupt | Tinterrupt is defined in clause 6.1B.1.2 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.1.2.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  | - | PCCA_DL=0.9375 |  |  |  | PCCA_DL=0.9375 |  |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  | - | 0.75 |  |  |  | 0.75 |  |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  | - | 0.87 |  |  |  | 0.87 |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 CCA |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 CCA |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| DBT window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |  |
| SSB configuration for semi-static channel access Note 4, 6 |  | Config 1 |  | SSB.1 CCA |  |  |  |  |  |  |
| SSB configuration for dynamic channel access Note 5, 6 |  | Config 1 |  | SSB.2 CCA |  |  |  |  |  |  |
| ssb-PositionQCL |  | Config 1 |  | [1] |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -95 |  |  |  |  |  |  |
|  |  |  | dB | 8 | -0.64 |  | -Infinity |  | -0.64 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 8 |  |
| SSB_RP | Config 1 |  | dBm/SCS | -87 | -87 |  | -Infinity |  | -87 |  |
| IoNote3 | Config 1 |  | dBm/ 38.16MHz | -55.31 | -52.60 |  | -55.31 |  | -52.60 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |

##### A.11.2.1.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tinterrupt from the beginning of time period T3, where Tinterrupt is defined in clause 6.1B.1.2

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin

Tsearch = (1+L1)* 20 ms.

Tprocessing = 20 ms.

Tmargin = 2 ms.

T∆ = (1+ L2) *20 ms.

TIU = (1+ L3)*10 + 10 ms

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2], L1 is the number of SMTC occasions not available at the UE during the intra-frequency detection period, L2 is the number of SMTC occasions not available at the UE during the time tracking period, where L1 + L2  LCCA_DL, and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure, where L3  LCCA_UL. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. The interruption time considering the potential extensions caused by L1, ,L2 , L3  and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer.

#### A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell

##### A.11.2.1.3.1	Test Purpose and Environment

This test is to verify the requirement for inter frequency handover requirements from FR1 carrier under CCA to FR1 carrier under CCA specified in clause 6.1B.1.2.

##### A.11.2.1.3.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.3.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.3.2-2, and A.11.2.1.3.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.11.2.1.3.2-1: Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.1.3.2-2: General test parameters Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | On the carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.20.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.20.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T304 |  | ms | 500 |  |
| LCCA_DL |  |  | 5 |  |
| WCCA_DL |  | ms | T304 |  |
| LCCA_UL |  |  | 5 |  |
| WCCA_UL |  | ms | T304 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | Tinterrupt | Tinterrupt is defined in clause 6.1B.1.2 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.1.3.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  | - | PCCA_DL=0.9375 |  |  |  | PCCA_DL=0.9375 |  |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  | - | 0.75 |  |  |  | 0.75 |  |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  | - | 0.87 |  |  |  | 0.87 |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 CCA |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR1.1 CCA |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 CCA |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| DBT window configuration |  | Config 1 |  | DBT.1 |  |  |  |  |  |  |
| SSB configuration for semi-static channel access Note 4, 6 |  | Config 1 |  | SSB.1 CCA |  |  |  |  |  |  |
| SSB configuration for dynamic channel access Note 5, 6 |  | Config 1 |  | SSB.2 CCA |  |  |  |  |  |  |
| ssb-PositionQCL |  | Config 1 |  | [1] |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -95 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 5 |  |
| SSB_RP | Config 1 |  | dBm/SCS | -91 | -91 |  | -Infinity |  | -90 |  |
| IoNote3 | Config 1 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -57.75 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |

##### A.11.2.1.3.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tinterrupt from the beginning of time period T3, where Tinterrupt is defined in clause 6.1B.1.2

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin

Tsearch = (3+L1’)* 20 ms.

Tprocessing = 20 ms.

Tmargin = 2 ms.

T∆ = (1+ L2) *20 ms.

TIU = (1+ L3)*10 + 10 ms

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2], L1’ is the number of SMTC occasions not available at the UE during the inter-frequency detection period, L2 is the number of SMTC occasions not available at the UE during the time tracking period, where L1’ + L2  LCCA_DL, and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure, where L3  LCCA_UL. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. The interruption time considering the potential extensions caused by , L1´,L2 , L3  and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer.

#### A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell

##### A.11.2.1.4.1	Test Purpose and Environment

This test is to verify the requirement for the NR with CCA FR1-NR FR1 handover requirements specified in clause 6.1.1.2.

##### A.11.2.1.4.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.4.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.4.2-2, and A.11.2.1.4.2-3.

The test consists of three successive time periods, with time durations of T1 T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR with CCA shall send a RRC message implying handover to cell 2. The RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. T3 is defined as the end of the last TTI containing the RRC message implying handover.

Table A.11.2.1.4.2-1: Handover from NR with CCA FR1 to NR FR1 test configuration

| Config | Description |
| --- | --- |
| 1 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.2.1.4.2-2: General test parameters handover from NR with CCA FR1 to NR FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | NR cell with CCA |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.11.2.1.4.2-3: Cell specific test parameters for NR with CCA FR1 – NR FR1 handover test case

| Parameter |  |  | Unit | Test | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | configuration | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1,2,3 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  |  | 1 | TDD |  |  | FDD |  |  |
|  |  |  |  | 2 | TDD |  |  | TDD |  |  |
|  |  |  |  | 3 | TDD |  |  | TDD |  |  |
| TDD configuration |  |  |  | 1 | TDDConf.1.1 CCA |  |  | Not Applicable |  |  |
|  |  |  |  | 2 | TDDConf.1.1 CCA |  |  | TDDConf.1.1 |  |  |
|  |  |  |  | 3 | TDDConf.1.1 CCA |  |  | TDDConf.2.1 |  |  |
| BWchannel |  |  | MHz | 1 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 2 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 3 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| BWP BW |  |  | MHz | 1 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 2 | 40: NRB,c = 106 |  |  | 10: NRB,c = 52 |  |  |
|  |  |  |  | 3 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| DRX Cycle |  |  | ms | 1,2,3 | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | 1 | SR.1.1 CCA |  |  | SR.1.1 FDD |  |  |
|  |  |  |  | 2 | SR.1.1 CCA |  |  | SR.1.1 TDD |  |  |
|  |  |  |  | 3 | SR.1.1 CCA |  |  | SR2.1 TDD |  |  |
| CORESET Reference Channel |  |  |  | 1 | CR1.1 CCA |  |  | CR.1.1 FDD |  |  |
|  |  |  |  | 2 | CR1.1 CCA |  |  | CR.1.1 TDD |  |  |
|  |  |  |  | 3 | CR1.1 CCA |  |  | CR2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  |  |  | 1 | CCR.1.1 CCA |  |  | CCR.1.1 FDD |  |  |
|  |  |  |  | 2 | CCR.1.1 CCA |  |  | CCR.1.1 TDD |  |  |
|  |  |  |  | 3 | CCR.1.1 CCA |  |  | CCR.2.1 TDD |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  |  |  | 1,2,3 | 0.9375 |  |  | N/A |  |  |
| DL CCA probability for dynamic channel access (PCCA_DL_1) |  |  |  | 1,2,3 | 0.75 |  |  | N/A |  |  |
| DL CCA probability for dynamic channel access (PCCA_DL_2) |  |  |  | 1,2,3 | 0.75 |  |  | N/A |  |  |
| UL CCA probability for semi-static channel access PCCA_UL |  |  |  | 1,2,3 | 0.75 |  |  | N/A |  |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  |  |  | 1,2,3 | 0.87 |  |  |  |  |  |
| TRS configuration |  |  |  | 1 | TRS.1.2 TDD |  |  | TRS.1.1 FDD |  |  |
|  |  |  |  | 2 | TRS.1.2 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Semi-static channel access |  | 1,2 | SSB.1 CCA (As defined in A.3.10A ) |  |  | SSB.1 FR1 |  |  |
|  |  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |  |  |
|  |  | Semi-static channel access |  | 3 | SSB.1 CCA (As defined in A.3.10A ) |  |  | SSB.2 FR1 |  |  |
|  |  | Dynamic channel access |  |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |  |  |
| DBT window configuration |  |  |  |  | As defined in A.3.28.1 |  |  | Not applicable |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 1,2 | 30 kHz |  |  | 15 kHz |  |  |
|  |  |  |  | 3 | 30 kHz |  |  | 30 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 1,2 | 30 kHz |  |  | 15 kHz |  |  |
|  |  |  |  | 3 | 30 kHz |  |  | 30 kHz |  |  |
| PRACH configuration |  |  |  |  | FR1 PRACH configuration 1 under CCA |  |  | FR1 PRACH configuration 1 |  |  |
| BWP configuration |  | Initial DL BWP |  | 1,2,3 | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | 1,2,3 | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | 1,2,3 | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | 1,2,3 | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1,2,3 | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  | 1,2,3 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  | 1,2,3 |  |  |  |  |  |  |
| Note2 |  |  | dBm/SCS | 1,2 | [-101] |  |  | -98 |  |  |
|  |  |  |  | 3 | [-101] |  |  | -95 |  |  |
|  |  |  | dB | 1,2,3 | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
|  |  |  | dB | 1,2,3 | 8 | 8 | 8 | -Infinity | 11 | 11 |
| SSB_RP | Config 1 |  | dBm/SCS | 1,2,3 | -90 | -90 | -90 | -Infinity | -87 | -87 |
| IoNote3 | Config 1 |  | dBm/ 9.36MHz | 1,2,3 | -61.41 | -57.06 | -57.06 | -61.41 | -57.06 | -57.06 |
| Propagation condition |  |  | - | 1,2,3 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |

##### A.11.2.1.4.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 112 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 102 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 112 ms.

#### A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell

##### A.11.2.1.5.1	Test Purpose and Environment

This test is to verify the requirement for the NR with CCA FR1-NR FR1 handover requirements specified in clause 6.1.1.2.

##### A.11.2.1.5.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.5.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.5.2-2, and A.12.2.1.7.2-3.

The test scenario comprises of two carriers and one cell on each carrier. Cell 1 is the NR with CCA cell and Cell 2 is an NR neighbour cell. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2.

Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.11.2.1.5.2-1: Handover from NR with CCA FR1 to NR FR1 test configuration

| Config | Description |
| --- | --- |
| 1 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.2.1.5.2-2: General test parameters handover from NR with CCA FR1 to NR FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | NR cell with CCA |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.11.2.1.5.2-3: Cell specific test parameters for NR with CCA FR1 – NR FR1 handover test case

| Parameter | Unit | Configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3 | 1 |  | 2 |  |
| Duplex mode |  | 1 | TDD |  | FDD |  |
|  |  | 2 | TDD |  | TDD |  |
|  |  | 3 | TDD |  | TDD |  |
| DL CCA model |  | 1, 2, 3 | As specified in clause A.3.26.2.1 |  | N/A |  |
| UL CCA model |  | 1, 2, 3 | As specified in clause A.3.26.2.2 |  |  |  |
| TDD configuration |  | 1 | TDDConf.1.1 CCA |  | Not Applicable |  |
|  |  | 2 | TDDConf.1.1 CCA |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.1.1 CCA |  | TDDConf.2.1 |  |
| BWchannel | MHz | 1 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  | 2 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW | MHz | 1 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  | 2 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| DRX Cycle | ms |  | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 CCA |  | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 CCA |  | SR.1.1 TDD |  |
|  |  | 3 | SR.1.1 CCA |  | SR2.1 TDD |  |
| CORESET Reference Channel |  | 1 | CR2.1 TDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR2.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR2.1 TDD |  | CR2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 CCA |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 CCA |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.1.1 CCA |  | CCR.2.1 TDD |  |
| TRS configuration |  | 1 | TRS.1.2 TDD |  | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.2 TDD |  | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  | 1, 2, 3 | 0.9375 |  | N/A |  |
| DL CCA probability for dynamic channel access (PCCA_DL_1) |  | 1, 2, 3 | 0.75 |  | N/A |  |
| DL CCA probability for dynamic channel access (PCCA_DL_2) |  | 1, 2, 3 | 0.75 |  | N/A |  |
| UL CCA probability for semi-static channel access PCCA_UL |  | 1, 2, 3 | 0.75 |  | N/A |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  | 1, 2, 3 | 0.87 |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  |  |
| SMTC Configuration |  | 1, 2, 3 | SMTC.1 |  |  |  |
| DBT window configuration |  | 1, 2, 3 | As defined in A.3.28.1 |  | N/A |  |
| SSB configuration | Semi-static channel access | 1,2 | SSB.1 CCA (As defined in A.3.10A ) |  | SSB.1 FR1 |  |
|  | Dynamic channel access |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |
|  | Semi-static channel access | 3 | SSB.1 CCA (As defined in A.3.10A ) |  | SSB.2 FR1 |  |
|  | Dynamic channel access |  | SSB.2 CCA (As defined in A.3.10A ) |  |  |  |
| ssb-PositionQCL |  |  | [1] |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 | 30 kHz |  | 15 kHz |  |
|  |  | 2 | 30 kHz |  | 15 kHz |  |
|  |  | 3 | 30 kHz |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing | kHz | 1 | 30 kHz |  | 15 kHz |  |
|  |  | 2 | 30 kHz |  | 15 kHz |  |
|  |  | 3 | 30 kHz |  | 30 kHz |  |
| PRACH configuration |  | 1,2,3 | FR1 PRACH configuration 1 |  | FR1 PRACH configuration 1 under CCA |  |
| BWP configuration | Initial DL BWP | 1,2,3 | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP | 1,2,3 | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP | 1,2,3 | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP | 1,2,3 | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | 1,2,3 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | 1,2,3 |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | 1,2,3 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | 1,2,3 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | 1,2,3 |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  | 1,2,3 |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  | 1,2,3 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  | 1,2,3 |  |  |  |  |
| Note2 | dBm/SCS | 1,2,3 | [-101] |  | -98 |  |
|  |  |  | [-101] |  | -95 |  |
|  | dB | 1,2,3 | -Infinity | 5 | 4 | 4 |
|  | dB | 1,2,3 | -Infinity | 5 | 4 | 4 |
| SSB_RP | dBm/SCS | 1,2,3 | -Infinity | -93 | -94 | -94 |
|  |  |  | -Infinity | -90 | -91 | -91 |
| IoNote3 | dBm/ 9.36MHz | 1,2,3 | -70.05 | -63.85 | -64.59 | -64.59 |
|  | dBm/ 38.16MHz | 1,2,3 | -63.94 | -57.75 | -58.49 | -58.49 |
| Propagation condition | - | 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |

##### A.11.2.1.5.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 122 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 132 ms.

#### A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell

##### A.11.2.1.6.1	Test Purpose and Environment

This test is to verify the requirement for inter frequency handover requirements from FR1 to FR1 carrier under CCA specified in clause 6.1B.1.2.

##### A.11.2.1.6.2	Test Parameters

Supported test configurations are shown in table A.11.2.1.6.2-1. Both handover delay and interruption length are tested by using the parameters in table A.11.2.1.6.2-2, and A.11.2.1.6.2-3.

The test scenario comprises of two carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.11.2.1.6.2-1: Inter-frequency handover from FR1 to FR1 carrier under CCA test configurations

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.11.2.1.6.2-2: General test parameters Inter-frequency handover from FR1 to FR1 carrier under CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | Tinterrupt | Tinterrupt is defined in clause 6.1B.1.2 |

Table A.11.2.1.6.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  | Unit | Configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1, 2, 3 | 1 |  | 2 |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  |  | 1, 2, 3 | N/A |  | 0.9375 |  |
| DL CCA probability for dynamic channel access (PCCA_DL_1) |  |  | 1, 2, 3 | N/A |  | 0.75 |  |
| DL CCA probability for dynamic channel access (PCCA_DL_2) |  |  | 1, 2, 3 | N/A |  | 0.75 |  |
| UL CCA probability for semi-static channel access PCCA_UL |  |  | 1, 2, 3 | N/A |  | 0.75 |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  |  | 1, 2, 3 | N/A |  | 0.87 |  |
| TDD configuration |  |  | 1 | N/A |  | TDDConf.1.1.CCA |  |
|  |  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1.CCA |  |
|  |  |  | 3 | TDDConf.1.2 |  | TDDConf.1.1.CCA |  |
| BWchannel |  |  | 1 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |
|  |  |  | 2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |
|  |  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  |  | 1 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |
|  |  |  | 2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |
|  |  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| DRX Cycle |  | ms |  | Not Applicable |  |  |  |
| PDSCH Reference |  |  | 1 | SR.1.1 FDD |  | SR.1.1 CCA |  |
|  |  |  | 2 | SR.1.1 TDD |  | SR.1.1 CCA |  |
|  |  |  | 3 | SR.2.1 TDD |  | SR.1.1 CCA |  |
| CORESET Reference Channel |  |  | 1 | CR.1.1 FDD |  | CR.1.1 CCA |  |
|  |  |  | 2 | CR.1.1 TDD |  | CR.1.1 CCA |  |
|  |  |  | 3 | CR.2.1 TDD |  | CR.1.1 CCA |  |
| Dedicated CORESET RMC configuration |  |  | 1 | CCR.1.1 FDD |  | CCR.1.1 CCA |  |
|  |  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 CCA |  |
|  |  |  | 3 | CCR.2.1 TDD |  | CCR.1.1 CCA |  |
| TRS configuration |  |  | 1 | TRS.1.1 FDD |  | TRS.1.2 TDD |  |
|  |  |  | 2 | TRS.1.1 TDD |  | TRS.1.2 TDD |  |
|  |  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  | 1, 2, 3 | OP.1 |  |  |  |
| SMTC Configuration |  |  | 1, 2, 3 | SMTC.1 |  |  |  |
| DBT window configuration |  |  | 1, 2, 3 | N/A |  | As defined in A.3.28.1 |  |
| SSB configuration |  |  | 1, 2 | SSB.1 FR1 |  | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access; |  |
|  |  |  | 3 | SSB.2 FR1 |  | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access; |  |
| ssb-PositionQCL |  |  |  | N/A |  | [1] |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1 | 15 kHz |  | 30 kHz |  |
|  |  |  | 2 | 15 kHz |  | 30 kHz |  |
|  |  |  | 3 | 30 kHz |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 1 | 15 kHz |  | 30 kHz |  |
|  |  |  | 2 | 15 kHz |  | 30 kHz |  |
|  |  |  | 3 | 30 kHz |  | 30 kHz |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  | FR1 PRACH configuration 1 CCA |  |
| BWP configuration | Initial DL BWP |  | 1, 2, 3 | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  | 1, 2, 3 | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  | 1, 2, 3 | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  | 1, 2, 3 | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  |  |
| Note2 | Config 1 | dBm/SCS | 1, 2 | -98 |  |  |  |
|  |  |  | 3 | -95 |  |  |  |
|  |  | dB |  | 4 | 4 | -Infinity | 5 |
|  |  | dB |  | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1 | dBm/SCS | 1, 2 | -94 | -94 | -Infinity | -93 |
|  |  |  | 3 | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1 | dBm/ 9.36MHz | 1, 2 | -64.59 | -64.59 | -70.05 | -63.85 |
|  |  | dBm/ 38.16MHz | 3 | -58.49 | -58.49 | -63.94 | -57.75 |
| Propagation condition |  | - |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

##### A.11.2.1.6.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tinterrupt from the beginning of time period T3, where Tinterrupt is defined in clause 6.1B.1.2

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2], L1’ is the number of SMTC occasions not available at the UE during the inter-frequency detection period, L2 is the number of SMTC occasions not available at the UE during the time tracking period, and L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure. L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [33]. The interruption time considering the potential extensions caused by L1, L1´,L2 , L3  and by the UL CCA failure detection/recovery mechanism is limited by the T304 timer. The UE behaviour at the T304 timer expiry is detailed in TS 38.331 [2].

#### A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell

##### A.11.2.1.7.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1 carrier under CCA. This test shall verify the NR to E-UTRAN handover requirements as specified in clause 6.1.2.1.

The test comprises of one NR carrier under CCA and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.11.2.1.7-1. General test parameters are provided in Table A.11.2.1.7-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.11.2.1.7-3 and A.11.2.1.7-4 respectively.

Table A.11.2.1.7-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE supporting SA operation only on NR band(s) with shared spectrum access is required to be tested. |  |  |

Table A.11.2.1.7-2: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell on a carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | As specified in Table A.11.2.1.7-3 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2EUTRAN |  | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.11.2.1.7-3: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2 | 1 |  |  |
| TDD Configuration |  |  | 1, 2 | TDDConf.1.1.CCA |  |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  |  | 1, 2 | 0.9375 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  |  | 1, 2 | 0.75 |  |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  |  | 1, 2 | 0.75 |  |  |
| UL CCA probability for semi-static channel access PCCA_UL |  |  | 1, 2 | 0.75 |  |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  |  | 1, 2 | 0.87 |  |  |
| BWchannel |  |  | 1, 2 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 2 | SR.1.1 CCA |  |  |
| CORESET reference channel |  |  | 1, 2 | CR.1.1 CCA |  |  |
| Dedicated CORESET RMC configuration |  |  | 1, 2 | CCR.1.1 CCA |  |  |
| TRS configuration |  |  | 1, 2 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |  |
| DBT window configuration |  |  | 1, 2 | As defined in A.3.28.1 |  |  |
| SSB configuration |  |  | 1, 2 | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access; |  |  |
| b2-Threshold1 |  | dBm | 1, 2 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2 | -100 | -104 | -100 |
| NocNote2 |  | dBm/SCS | 1, 2 | -97 | -101 | -97 |
| Ês/Noc |  | dB | 1, 2 | 12 | 0 | -4 |
| Ês/IotNote3 |  | dB | 1, 2 | 12 | 0 | -4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -85 | -101 | -101 |
| IoNote3 |  | dBm/38.16 MHz | 1, 2 | -53.68 | -66.9448 | -64.49 |
| Propagation condition |  |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |

Table A.11.2.1.7-4: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2 | 2 |  |  |
| Duplex mode |  | 1 | FDD |  |  |
|  |  | 2 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |  |
| BWchannel | MHz | 1, 2 | 10 MHz: NRB,c = 50 |  |  |
| PRACH ConfigurationNote2 |  | 1 | 4 |  |  |
|  |  | 2 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 10 MHz: R.3 FDD |  |  |
|  |  | 2 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 10 MHz: R.6 FDD |  |  |
|  |  | 2 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote3 |  | 1 | 10 MHz: OP.10 FDD |  |  |
|  |  | 2 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1, 2 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2 | -98 |  |  |
| Ês/Noc | dB | 1, 2 | -Infinity | 8 | 78 |
| Ês/IotNote6 | dB | 1, 2 | -Infinity | 78 | 78 |
| RSRPNote6 | dBm/15kHz | 1, 2 | -Infinity | -90 | -90 |
| SCH_RPNote6 | dBm/15kHz | 1, 2 | -Infinity | -90 | -90 |
| IoNote6 | dBm/9MHz | 1, 2 | -67.21 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) | -58.57 +10log(NRB,c/100) |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

##### A.11.2.1.7.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 85 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 6.1.2.1.

Tinterrupt = 35 ms in the test; Tinterrupt is defined in clause 6.1.2.1.

This gives a total of 85 ms.

#### A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell

##### A.11.2.1.8.1	Test Purpose and Environment

The purpose of this set of tests is to verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1 carrier under CCA. This test shall verify the NR to E-UTRAN handover requirements for the case when the target E-UTRAN cell is unknown as specified in clause 6.1.2.1.

The test comprises of one NR carrier under CCA and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable. No Gap pattern shall be configured.

A RRC message implying handover shall be sent to the UE during period T1. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.11.2.1.8-1. General test parameters are provided in Table A.11.2.1.8-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.11.2.1.8-3 and A.11.2.1.8-4 respectively.

Table A.11.2.1.8-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description of a cell with CCA | Description of a cell without CCA |
| --- | --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | LTE 10 MHz bandwidth, FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE supporting SA operation only on NR band(s) with shared spectrum access is required to be tested. |  |  |

Table A.11.2.1.8-2: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell on a carrier under CCA |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

Table A.11.2.1.8-3: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF channel number |  |  | 1, 2 | 1 |  |
| DL CCA probability for semi-static channel access (PCCA_DL)DL CCA probability PCCA_DL |  |  | 1, 2 | 0.9375 |  |
| DL CCA probability for dynamic channel access (PCCA_DL_1) |  |  | 1, 2 | 0.75 |  |
| DL CCA probability for dynamic channel access (PCCA_DL_2) |  |  | 1, 2 | 0.75 |  |
| UL CCA probability for semi-static channel access PCCA_UL |  |  | 1, 2 | 0.75 |  |
| UL CCA probability for dynamic static channel access PCCA_UL |  |  | 1, 2 | 0.87 |  |
| TDD Configuration |  |  | 1, 2 | TDDConf.1.1.CCA |  |
| BWchannel |  | MHz | 1, 2 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  | 1, 2 | SR.1.1 CCA |  |
| CORESET reference channel |  |  | 1, 2 | CR.1.1 CCA |  |
| Dedicated CORESET RMC configuration |  |  | 1, 2 | CCR.1.1 CCA |  |
| TRS configuration |  |  | 1, 2 | TRS.1.2 TDD |  |
| OCNG patternNote1 |  |  | 1, 2 | OP.1 |  |
| BWP | Initial DL BWP |  | 1, 2 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DBT window configuration |  |  | 1, 2 | As defined in A.3.28.1 |  |
| SSB configuration |  |  | 1, 2 | SSB.1 CCA for semi-static channel access; SSB.2 CCA for dynamic channel access; |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2 | -98 |  |
| NocNote2 |  | dBm/SCS | 1, 2 | -95 |  |
| Ês/Noc |  | dB | 1, 2 | 0 | 0 |
| Ês/IotNote3 |  | dB | 1, 2 | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -95 | -95 |
| IoNote3 |  | dBm/38.16 MHz | 1, 2 | -60.94 | -60.94 |
| Propagation condition |  |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

Table A.11.2.1.8-4: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 2 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 10 MHz: NRB,c = 50 |  |
| PRACH ConfigurationNote2 |  | 1 | 4 |  |
|  |  | 2 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 10 MHz: R.3 FDD |  |
|  |  | 2 | 10 MHz: R.0 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 10 MHz: R.6 FDD |  |
|  |  | 2 | 10 MHz: R.6 TDD |  |
| OCNG PatternsNote3 |  | 1 | 10 MHz: OP.10 FDD |  |
|  |  | 2 | 10 MHz: OP.1 TDD |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2 | -98 |  |
| Ês/Noc | dB | 1, 2 | -Infinity | 7 |
| Ês/IotNote6 | dB | 1, 2 | -Infinity | 7 |
| RSRPNote6 | dBm/15kHz | 1, 2 | -Infinity | -91 |
| SCH_RPNote6 | dBm/15kHz | 1, 2 | -Infinity | -91 |
| IoNote6 | dBm/9MHz | 1, 2 | -70.22 | -62.43 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

##### A.11.2.1.8.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 165 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 6.1.2.1.

Tinterrupt = 115 ms in the test; Tinterrupt is defined in clause 6.1.2.1.

This gives a total of 165 ms.

### A.11.2.2	RRC connection mobility control

#### A.11.2.2.1	RRC re-establishment

##### A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1

A.11.2.2.1.1.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay with CCA in FR1 with known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1A.

The test parameters are given in table A.11.2.2.1.1.1-1, table A.11.2.2.1.1.1-2 and table A.11.2.2.1.1.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell with CCA, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table A.11.2.2.1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.2.1.1.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case with CCA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial Condition | Active cell | - | Cell1 | Cell 1 is with CCA. |
|  | Neighbour cells | - | Cell2 | Cell 2 is with CCA. |
| Final condition | Active cell | - | Cell2 |  |
| RF Channel Number |  | - | 1 |  |
| DL CCA model | Dynamic channel accessNote 1, 3 | - | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2, 3 | - |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 | - | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 | - |  |  |
| Time offset between cells |  | - | 3 s | Synchronous cells |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| SSB configuration | Dynamic channel access Note 1, 3 | - | SSB.2 CCA | Table A.3.10A.1.2-1 |
|  | Semi-static channel access Note 2, 3 | - | SSB.1 CCA | Table A.3.10A.1.1-1 |
| DBT window configuration |  | - | DBT.1 | Table A.3.28.1-1 |
| SMTC configuration |  | - | SMTC pattern 1 |  |
| DRX cycle length |  | s | OFF |  |
| PRACH configuration |  | - | FR1 PRACH configuration 1 under CCA | Table A.3.8A.2.1-1 |
| T1 |  | s | 5 |  |
| T2 |  | ms | 480 | Time for the UE to detect RLF |
| T3 |  | s | 2 |  |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.2.1.1.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case with CCA

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  | T3 | T1 | T2 |  |  | T3 |
| TDD configuration | - | TDDConf.1.1 CCA |  |  |  |  | TDDConf.1.1 CCA |  |  |  |  |
| DL CCA probability PCCA_DL for dynamic channel access Note 4,6 | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |
| DL CCA probability PCCA_DL for semi-static channel access Note 5,6 | - | PCCA_DL=0.9375 |  |  |  |  | PCCA_DL=0.9375 |  |  |  |  |
| UL CCA probability PCCA_UL | - | 1 |  |  |  |  | 1 |  |  |  |  |
| PDSCH RMC configuration |  | SR.1.1 CCA |  |  |  |  | SR.1.1 CCA |  |  |  |  |
| RMSI CORESET RMC configuration |  | CR.1.1 CCA |  |  |  |  | CR.1.1 CCA |  |  |  |  |
| Dedicated CORESET RMC configuration |  | CCR.1.1 CCA |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |  |
| TRS configuration |  | TRS.1.2 TDD |  |  |  |  | N/A |  |  |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | DLBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | ULBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | ULBWP.1.1 |  |
| RLM-RS |  | SSB |  |  |  |  | SSB |  |  |  |  |
|  | dB | 1.54 |  | -infinity |  | -infinity | -3.79 | 4 |  |  | 4 |
| Note2 | dBm/SCS | -95 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 7 |  | -infinity |  | -infinity | 4 | 4 |  |  | 4 |
| SS-RSRP Note3 | dBm/SCS | -88 |  | -infinity |  | -infinity | -91 | -91 |  |  | -91 |
| Io | dBm/38.16 MHz | -54.65 |  | -58.50 |  | -58.50 | -54.65 | -58.50 |  |  | -58.50 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 5: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |  |

A.11.2.2.1.1.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to a known NR intra frequency cell with CCA shall be less than  1350 + MAX (200, (5+K1) x 20)  ms.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay_CCA=TUE_re-establish_delay_CCA+TUL_grant

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay_CCA=50 ms+Tidentify_intra_NR_CCA+i=1Nfreq-1Tidentify_inter_NR_CCA,i+TSI-NR_CCA+TPRACH_CCA

Where

Nfreq = 1

Tidentify_intra_NR_CCA = MAX (200 ms, (5+K1) x TSMTC), where

K1 is the number of SMTC occasions not available at the UE due to DL CCA failures during RRC re-establishment period on the carrier with CCA.

TSMTC = 20 ms is the SMTC periodicity.

Tidentify_inter_NR_CCA = 0 ms

TSI-NR_CCA = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH_CCA = TSSB,RO + 10 ms, where:

-	TSSB,RO is the SSB to PRACH occasion association period as defined in Table 8.1-1 of TS 38.213 [39], which is TSSB,RO=10 ms for FR1 PRACH configuration 1 under CCA.

This gives a total of 1350 + MAX (200, (5+K1) x 20) ms.

##### A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1

A.11.2.2.1.2.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay with CCA in FR1 without known target cell is within the specified limits. These tests will verify the requirements in clause 6.2.1A.

The test parameters are given in table A.11.2.2.1.2.1-1, table A.11.2.2.1.2.1-2 and table A.11.2.2.1.2.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell with CCA, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

Table A.11.2.2.1.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.2.1.2.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial Condition | Active cell | - | Cell1 | Cell 1 is with CCA. |
|  | Neighbour cells | - | Cell2 | Cell 2 is with CCA. |
| Final condition | Active cell | - | Cell2 |  |
| RF Channel Number |  | - | 1 |  |
| DL CCA model | Dynamic channel access Note 1, 3 | - | As specified in clause A.3.20.2.1 |  |
|  | Semi-static channel access Note 2, 3 | - |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 | - | As specified in clause A.3.20.2.2 |  |
|  | Semi-static channel access Note 2, 3 | - |  |  |
| Time offset between cells |  | - | 3 s | Synchronous cells |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| SSB configuration | Dynamic channel access Note 1, 3 | - | SSB.2 CCA | Table A.3.10A.1.2-1 |
|  | Semi-static channel access Note 2, 3 | - | SSB.1 CCA | Table A.3.10A.1.1-1 |
| DBT window configuration |  | - | [DBT.1] | Table A.3.28.1-1 |
| SMTC configuration |  | - | SMTC pattern 1 |  |
| DRX cycle length |  | s | OFF |  |
| PRACH configuration |  | - | [TBD] |  |
| T1 |  | s | [5] |  |
| T2 |  | ms | [200] | Time for the UE to detect RLF |
| T3 |  | s | [2] |  |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.2.1.2.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  | T3 | T1 | T2 |  |  | T3 |
| TDD configuration |  | TDDConf.1.1 CCA |  |  |  |  | TDDConf.1.1 CCA |  |  |  |  |
| DL CCA probability PCCA_DL for dynamic channel access Note 4,6 | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |
| DL CCA probability PCCA_DL for semi-static channel access Note 5,6 | - | PCCA_DL=0.9375 |  |  |  |  | PCCA_DL=0.9375 |  |  |  |  |
| UL CCA probability PCCA_UL | - | 1 |  |  |  |  | 1 |  |  |  |  |
| PDSCH RMC configuration |  | SR.1.1 CCA |  |  |  |  | SR.1.1 CCA |  |  |  |  |
| RMSI CORESET RMC configuration |  | CR.1.1 CCA |  |  |  |  | CR.1.1 CCA |  |  |  |  |
| Dedicated CORESET RMC configuration |  | CCR.1.1 CCA |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |  |
| TRS configuration |  | TRS.1.2 TDD |  |  |  |  | N/A |  |  |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | DLBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | ULBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | ULBWP.1.1 |  |
| RLM-RS |  | SSB |  |  |  |  | SSB |  |  |  |  |
|  | dB | 1.54 |  | -infinity |  | -infinity | -3.79 | 4 |  |  | 4 |
| Note2 | dBm/SCS | [-101] |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | [-104] |  |  |  |  |  |  |  |  |  |
|  | dB | 7 |  | -infinity |  | -infinity | 4 | 4 |  |  | 4 |
| SS-RSRP Note3 | dBm/SCS | -88 |  | -infinity |  | -infinity | -91 | -91 |  |  | -91 |
| Io | dBm/9.36 MHz | -54.65 |  | -58.50 |  | -58.50 | -54.65 | -58.50 |  |  | -58.50 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 5: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |  |

A.11.2.2.1.2.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than TBD s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establishdelayCCA=TUEre-establishdelayCCA+TULgrant

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay_CCA=50 ms+Tidentify_intra_NR_CCA+i=1Nfreq-1Tidentify_inter_NR_CCA,i+TSI-NR_CCA+TPRACH_CCA

Where

Tidentify_intra_NR_CCA: 0 ms

Tidentify_inter_NR_CCA,i: MAX (200 ms, ([6]+K2,i) x TSMTC, i),

where

K2,i is the number of SMTC not available at the UE during RRC re-establishment period on the “i” th carrier with CCA

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i.

Nfreq = 2

TSI-NR_CCA = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH_CCA = (1+ K3)*TSSB,RO + 10 ms, where:

-	TSSB,RO is the SSB to PRACH occasion association period as defined in Table 8.1-1 of TS 38.213 [39].

[-	K3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure. K3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [57].]

This gives a total of TBD ms.

##### A.11.2.2.1.3	Intra-frequency RRC Re-establishment with CCA in FR1 without serving cell timing

A.11.2.2.1.3.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay with CCA in FR1 without serving cell timing is within the specified limits. These tests will verify the requirements in clause 6.2.1A.

The test parameters are given in table A.11.2.2.1.3.1-1, table A.11.2.2.1.3.1-2 and table A.11.2.2.1.3.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell with CCA, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

Table A.11.2.2.1.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.2.1.3.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial Condition | Active cell | - | Cell1 | Cell 1 is with CCA. |
|  | Neighbour cells | - | Cell2 | Cell 2 is with CCA. |
| Final condition | Active cell | - | Cell2 |  |
| RF Channel Number |  | - | 1 |  |
| DL CCA model | Dynamic channel accessNote 1,3 | - | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2,3 | - |  |  |
| UL CCA model | Dynamic channel access Note 1,3 | - | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 | - |  |  |
| Time offset between cells |  | - | 3 s | Synchronous cells |
| N310 |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| SSB configuration | Dynamic channel access Note 1, 3 | - | SSB.2 CCA | Table A.3.10A.1.2-1 |
|  | Semi-static channel access Note 2, 3 | - | SSB.1 CCA | Table A.3.10A.1.1-1 |
| DBT window configuration |  | - | DBT.1 | Table A.3.28.1-1 |
| SMTC configuration |  | - | SMTC pattern 1 |  |
| DRX cycle length |  | s | OFF |  |
| PRACH configuration |  | - | FR1 PRACH configuration 1 | Table A.3.8A.2.1-1 |
| T1 |  | s | 5 |  |
| T2 |  | ms | 6 | Time for the UE to detect RLF |
| T3 |  | s | 3 |  |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.2.1.3.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  | T3 | T1 | T2 |  |  | T3 |
| TDD configuration |  | TDDConf.1.1 CCA |  |  |  |  | TDDConf.1.1 CCA |  |  |  |  |
| DL CCA probability PCCA_DL for dynamic channel access Note 4,6 | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  |
| DL CCA probability PCCA_DL for semi-static channel access Note 5,6 | - | PCCA_DL=0.9375 |  |  |  |  | PCCA_DL=0.9375 |  |  |  |  |
| UL CCA probability PCCA_UL | - | 1 |  |  |  |  | 1 |  |  |  |  |
| PDSCH RMC configuration |  | SR.1.1 CCA |  |  |  |  | SR.1.1 CCA |  |  |  |  |
| RMSI CORESET RMC configuration |  | CR.1.1 CCA |  |  |  |  | CR.1.1 CCA |  |  |  |  |
| Dedicated CORESET RMC configuration |  | CCR.1.1 CCA |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |  |
| TRS configuration |  | TRS.1.2 TDD |  |  |  |  | N/A |  |  |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | DLBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | ULBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | ULBWP.1.1 |  |
| RLM-RS |  | SSB |  |  |  |  | SSB |  |  |  |  |
|  | dB | 4 |  | -infinity |  | -infinity | -infinity | -infinity |  |  | 4 |
| Note2 | dBm/SCS | -95 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 7 |  | -infinity |  | -infinity | -infinity | -infinity |  |  | 4 |
| SS-RSRP Note3 | dBm/SCS | -91 |  | -infinity |  | -infinity | -infinity | -infinity |  |  | -91 |
| Io | dBm/38.16 MHz | -58.50 |  | -63.94 |  | -63.94 | -63.94 | -63.94 |  |  | -58.50 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 5: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |  |  |

A.11.2.2.1.3.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell without serving cell timing shall be less than 1350 + MAX (800 ms, (10+ K1) x 20) ms.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay_CCA=TUE_re-establish_delay_CCA+TUL_grant

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay_CCA=50 ms+Tidentify_intra_NR_CCA+i=1Nfreq-1Tidentify_inter_NR_CCA,i+TSI-NR_CCA+TPRACH_CCA

Where,

Nfreq = 1

Tidentify_intra_NR = MAX (800 ms, (10+ K1) x TSMTC), where

K1 is the number of SMTC occasions not available at the UE due to DL CCA failures during RRC re-establishment period on the carrier with CCA.

TSMTC is the SMTC periodicity which is 20 ms.

Tidentify_inter_NR_CCA = 0 ms

TSI-NR_CCA = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 [2] for the target intra-frequency NR cell.

TPRACH_CCA = (1+ K3)*TSSB,RO + 10 ms, where:

-	TSSB,RO is the SSB to PRACH occasion association period as defined in Table 8.1-1 of TS 38.213 [39]. It is 10 ms for FR1 PRACH configuration 1 under CCA.

-	K3 = 0.

This gives total TUE_re-establish_delay_CCA=1350 + MAX (800 ms, (10+ K1) x 20) ms.

##### A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA

A.11.2.2.1.4.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay requirement for RRC re-establishment from NR FR1 carrier without CCA to NR FR1 inter-frequency carrier under CCA with unknown target cell. These tests will verify the requirements in clause 6.2.1A.

The test parameters are given in table A.11.2.2.1.4.1-1, table A.11.2.2.1.4.1-2 and table A.11.2.2.1.4.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

Table A.11.2.2.1.4.1-1: Supported test configurations inter-frequency RRC re-establishment from NR FR1 without under CCA to NR FR1 inter-frequency carrier under CCA

| Configuration | Source cell without CCA | Target cell with CCA |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.11.2.2.1.4.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case from NR FR1 carrier without CCA to NR FR1 intrer-frequency carrier under CCA

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  |  | Cell1 |  |
|  | Neighbour cells |  |  | Cell2 |  |
| Final condition | Active cell |  |  | Cell2 |  |
| RF Channel Number |  |  |  | 1, 2 |  |
| Time offset between cells |  |  |  | 3 s | Synchronous cells |
| DL CCA model | Dynamic channel accessNote 1,3 | - |  | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2,3 | - |  |  |  |
| UL CCA model | Dynamic channel access Note 1,3 | - |  | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 | - |  |  |  |
| N310 |  |  | - | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  |  | - | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  |  | ms | 0 | Radio link failure timer; T310 is disabled |
| T311 |  |  | ms | 5000 | RRC re-establishment timer |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  |  | s | OFF |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 | Table A.3.8A.2.1-1 |
| T1 |  |  | s | 5 |  |
| T2 |  |  | ms | 480 | Time for the UE to detect RLF |
| T3 |  |  | s | ≥ TUE_re-establish_delay_CCA | As defined in clause 6.2.1A |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.11.2.2.1.4.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case from NR FR1 carrier without CCA to NR FR1 intrer-frequency carrier under CCA

| Parameter |  | Test config | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T3 | T1 | T2 |  |  | T3 |
| TDD configuration |  | 1 |  | N/A |  |  |  |  | TDDConf.1.1.CCA |  |  |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  |  |  | TDDConf.1.1.CCA |  |  |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  |  |  | TDDConf.1.1.CCA |  |  |  |  |
| PDSCH RMC configuration |  | 1 |  | SR.1.1 FDD |  |  |  |  | SR.1.1 CCA |  |  |  |  |
|  |  | 2 |  | SR.1.1 TDD |  |  |  |  | SR.1.1 CCA |  |  |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  |  |  | SR.1.1 CCA |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 |  | CR.1.1 FDD |  |  |  |  | CR.1.1 CCA |  |  |  |  |
|  |  | 2 |  | CR.1.1 TDD |  |  |  |  | CR.1.1 CCA |  |  |  |  |
|  |  | 3 |  | CR.2.1 TDD |  |  |  |  | CR.1.1 CCA |  |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 |  | CCR.1.1 FDD |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
|  |  | 2 |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
|  |  | 3 |  | CCR.2.1 TDD |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
| OCNG Pattern |  | 1 |  | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
| SMTC configuration |  | 1,2,3 |  | SMTC.1 |  |  |  |  | SMTC.1 |  |  |  |  |
| SSB configuration | Semi- static channel acces | 1,2 |  | SSB.1 FR1 |  |  |  |  | SSB.1 CCA |  |  |  |  |
|  | Semi- static channel acces | 3 |  | SSB.2 FR1 |  |  |  |  | SSB.1 CCA |  |  |  |  |
|  | Dymamic channel acces | 1,2 |  | SSB.1 FR1 |  |  |  |  | SSB.2 CCA |  |  |  |  |
|  | Dymamic channel acces | 3 |  | SSB.2 FR1 |  |  |  |  | SSB.2 CCA |  |  |  |  |
| Initial DL BWP configuration |  | 1,2,3 |  | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  | 1,2,3 |  | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Active DL BWP confgiuration |  | 1,2,3 |  | DLBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1,2,3 |  | ULBWP.1.1 | N/A |  | N/A |  | N/A |  | N/A | ULBWP.1.1 |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | 1,2,3 |  | N/A | N/A |  | N/A |  | 1 |  | 1 | 0.9375 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_1) |  | 1,2,3 |  | N/A | N/A |  | N/A |  | 1 |  | 1 | 0.75 |  |
| DL CCA probability for for dynamic static channel access (PCCA_DL_2) |  | 1,2,3 |  | N/A | N/A |  | N/A |  | 1 |  | 1 | 0.75 |  |
| UL CCA probability (PCCA_UL) |  | 1,2,3 |  | N/A | N/A |  | N/A |  | 1 |  | 1 | 1 |  |
| RLM-RS |  | 1,2,3 |  | SSB |  |  |  |  | SSB |  |  |  |  |
|  |  | 1,2,3 | dB | 4 |  | -infinity |  | -infinity | -infinity | -infinity |  |  | 7 |
| Note2 |  | 1,2,3 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
| Note2 |  | 1,2 | dBm/SCS | -98 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  | -95 |  |  |  |  |  |  |  |  |  |
|  |  | 1,2,3 | dB | 4 |  | -infinity |  | -infinity | -infinity | -infinity |  |  | 7 |
| SS-RSRP Note3 |  | 1, 2 | dBm/SCS | -94 |  | -infinity |  | -infinity | -infinity | -infinity |  |  | -91 |
|  |  | 3 |  | -91 |  | -infinity |  | -infinity | -infinity | -infinity |  |  | -88 |
| Io |  | 1,2 | dBm/9.36 MHz | -64.59 |  | -70. 05 |  | -70. 05 | -70. 05 | -70.05 |  |  | -62.26 |
|  |  | 3 | dBm/38.16 MHz | -58.50 |  | -63.94 |  | -63.94 | -63.94 | -63.94 |  |  | -56.15 |
| Propagation Condition |  | 1,2,3 |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.20.2. Note 5: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.11.2.2.1.4.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less TUE_re-establish_delay_CCA.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay_CCA=TUE_re-establish_delay_CCA+TUL_grant

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay_CCA=50 ms+Tidentify_intra_NR_CCA+i=1Nfreq-1Tidentify_inter_NR_CCA,i+TSI-NR_CCA+ TPRACH_CCA

Nfreq = 2

Tidentify_intra_NR_CCA = MAX (800 ms, (10+ K1) x 20) ms

Tidentify_inter_NR_CCA = MAX (800 ms, ([13]+K2,2) x 20) ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH_CCA = It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TPRACH_CCA = (1+ K3)*TSSB,RO + 10 ms; where K3=0 and TSSB,RO=10 ms for FR1 PRACH configuration 1 under CCA.

K1 is the number of SMTC occasions not available at the UE due during RRC re-establishment period on the carrier with CCA and with RF channel number # 1.

K2,2 is the number of SMTC occasions not available at the UE during RRC re-establishment period on the carrier with CCA and with RF channel number # 2.

This gives total TUE_re-establish_delay_CCA=1350+MAX (800 ms, (10+ K1) x 20) ms+MAX (800 ms, ([13]+K2,2) x 20) ms.

#### A.11.2.2.2	Random Access

##### A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA

###### A.11.2.2.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in Clause 6.2.2A.2 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1, which operates on a carrier frequency with CCA.  Supported test parameters are shown in Table A.11.2.2.2.1.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.11.2.2.2.1.1-2.

Table A.11.2.2.2.1.1-1: Supported test configurations for contention based random access test for FR1 PCell with CCA

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: Void |  |

Table A.11.2.2.2.1.1-2: General test parameters for contention based random access test for FR1 PCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1 |  | SSB.3 CCA | As defined in A.3.10A |
|  | Note 5, 6 |  |  | Config 1 |  | SSB.4 CCA | As defined in A.3.10A |
| DBT Window Configuration |  |  |  | Config 1 |  | DBT.1 | As specified in A.3.28.1 |
| DL CCA model |  |  |  | Config 1 |  | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  | Config 1 |  | As specified in A.3.26.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1 |  | TDD |  |
| TDD Configuration |  |  |  | Config 1 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1 |  | SR.1.1 CCA | As defined in A.3.1A.1. |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | 3 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | -17 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  |  | Config 1 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power (PCMAX,f,c) |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  |  |  | FR1 PRACH configuration 1 under CCA | As defined in A.3.8A.2. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75 / 0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

###### A.11.2.2.2.1.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.11.2.2.2.1.2.1	Random Access Preamble Transmission

To test the UE behavior specified in Clause 6.2.2A.2.1.1 the System Simulator shall receive the Random Access Preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB, if the UL CCA is successful.

The three requirements below are relevant for all cases of PRACH transmissions described within the whole clause A.11.2.2.2.1.2:

The System Simulator shall implement the UL CCA model of A.3.26.2 for the RACH occasions where PRACH transmissions are expected. The System Simulator shall monitor the RACH occasions to detect if the UE is transmitting PRACH preambles. If a PRACH transmission is detected on a RACH occasion that is expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit PRACH for semi-static channel access mode; for dynamic channel access mode it is assumed that RACH occasions are always scheduled within a UE-initiated COT.

In case of UL CCA failure, The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.11.2.2.2.1.2.2	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2A.2.1.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble. In case of CCA DL failure, the test equipment should delay the transmission of Random Access Response.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting msg3 when CCA UL failure. If a msg3 transmission is detected on a grant expected to have UL CCA failure, the test is considered as failed.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.11.2.2.2.1.2.3	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2A.2.1.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles. In case of CCA DL failure, the test equipment should delay the transmission of Random Access Response.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission

To test the UE behavior specified in clause 6.2.2A.2.1.4 the System Simulator shall provide an UL grant for msg3 retransmission following a successful Random Access Response if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting msg3 when CCA UL failure. If a msg3 is detected on a grant expected to have UL CCA failure, the test is considered as failed.

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2A.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI

To test the UE behavior specified in Clause 6.2.2A.2.1.5 the System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

The UE shall send ACK if the Contention Resolution is successful.

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry

To test the UE behavior specified in Clause 6.2.2A.2.1.6 the System Simulator shall not send a response to a msg3.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

##### A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA

###### A.11.2.2.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in Clause 6.2.2A.2 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1, which operates on a carrier frequency with CCA. Supported test parameters are shown in Table A.11.2.2.2.2.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.11.2.2.2.2.1-2.

Table A.11.2.2.2.2.1-1: Supported test configurations for non-contention based random access test for FR1 PCell with CCA

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: Void. |  |

Table A.11.2.2.2.2.1-2: General test parameters for non-contention based random access test for FR1 PCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1 |  | SSB.3 CCA | As defined in A.3.10A |
|  | Note 5, 6 |  |  | Config 1 |  | SSB.4 CCA | As defined in A.3.10A |
| DBT Window Configuration |  |  |  | Config 1 |  | DBT.1 | As specified in A.3.28.1 |
| DL CCA model |  |  |  | Config 1 |  | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  | Config 1 |  | As specified in A.3.26.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1 |  | TDD |  |
| TDD Configuration |  |  |  | Config 1 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1 |  | SR.1.1 CCA | As defined in A.3.1A.1. |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | 3 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | -17 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  |  | Config 1 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| PRACH Configuration |  |  |  |  |  | FR1 PRACH configuration 2 under CCA | As defined in A.3.8A.2. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75 / 0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

###### A.11.2.2.2.2.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission

In Test-1, to test the UE behavior specified in Clause 6.2.2A.2.2.1 for SSB-based Random Access Preamble transmission, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the Random Access Preamble which has the Preamble Index associated with the SSB with index 0.

The three requirements below are relevant for all cases of PRACH transmissions described within the clause A.11.2.2.2.2.2:

The System Simulator shall implement the UL CCA model of A.3.26.2 for the RACH occasions where PRACH transmissions are expected. The System Simulator shall monitor the RACH occasions to detect if the UE is transmitting PRACH preambles. If a PRACH transmission is detected on a RACH occasion that is expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit PRACH for semi-static channel access mode; for dynamic channel access mode it is assumed that RACH occasions are always scheduled within a UE-initiated COT.

In case of UL CCA failure The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS38.321 [7], and transmit with the calculated PRACH transmission power.

In addition, the System Simulator shall receive the Random Access Preamble on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.11.2.2.2.2.2.2	Random Access Response Reception

To test the UE behavior specified in Clause 6.2.2A.2.2.2 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response not corresponding to the transmitted Random Access Preamble. In case of CCA DL failure, the test equipment should delay the transmission of Random Access Response.

The UE may stop monitoring for Random Access Response(s) if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting msg3 when CCA UL failure. If a msg3 is detected on a grant expected to have UL CCA failure, the test is considered as failed.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

A.11.2.2.2.2.2.3	No Random Access Response Reception

To test the UE behavior specified in clause 6.2.2A.2.2.3 the System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles. In case of CCA DL failure, the test equipment should delay the transmission of Random Access Response.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.2. The power of the first preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all PRACH transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA

###### A.11.2.2.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the 2-step RA type random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in Clause 6.2.2A.3 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1, which operates on a carrier frequency with CCA. Supported test parameters are shown in Table A.11.2.2.2.3.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.11.2.2.2.3.1-2.

Table A.11.2.2.2.3.1-1: Supported test configurations for 2-step RA type contention based random access with successRAR test for FR1 PCell with CCA

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: Void. |  |

Table A.11.2.2.2.3.1-2: General test parameters for 2-step RA type contention based random access with successRAR test for FR1 PCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1 |  | SSB.3 CCA | As defined in A.3.10A |
|  | Note 5, 6 |  |  | Config 1 |  | SSB.4 CCA | As defined in A.3.10A |
| DBT Window Configuration |  |  |  | Config 1 |  | DBT.1 | As specified in A.3.28.1 |
| DL CCA model |  |  |  | Config 1 |  | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  | Config 1 |  | As specified in A.3.26.2.2 |  |
| Duplex Mode for Cell 2 |  |  |  | Config 1 |  | TDD |  |
| TDD Configuration |  |  |  | Config 2 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1 |  | SR.1.1 CCA | As defined in A.3.1A.1. |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| SSB with index 0 |  |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | 3 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | -17 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  |  | Config 1 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power (PCMAX,f,c) |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  |  |  | FR1 MsgA configuration 1 under CCA | As defined in A.3.20A.2. |
| msgA-RSRP-ThresholdSSB |  |  |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75 / 0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

###### A.11.2.2.2.3.2	Test Requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

A.11.2.2.2.3.2.1	MsgA Transmission

To test the UE behavior specified in Clause 6.2.2A.3.1.1 the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured msgA-RSRP-ThresholdSSB, if the UL CCA is successful.

The three requirements below are relevant for all cases of MsgA transmissions described within the clause A.11.2.2.2.3.2:

The System Simulator shall implement the UL CCA model for the MsgA occasions (i.e. both MsgA PRACH and MsgA PUSCH occasions) where MsgA transmissions are expected. The System Simulator shall monitor the MsgA occasions to detect if the UE is transmitting MsgA. If a MsgA transmission is detected on MsgA occasions that are expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit MsgA for semi-static channel access mode; for dynamic channel access mode it is assumed that MsgA occasions are always scheduled within a UE-initiated COT.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated PRACH transmission power in case of UL CCA failure.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated PRACH transmission power in case of UL CCA failure.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2A.3. The power of the first MsgA preamble transmission shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.11.2.2.2.3.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2A.3.1.2 the System Simulator shall transmit a MsgB containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble. In case of CCA DL failure, the test equipment should delay the transmission of MsgB.

The UE may stop monitoring for MsgB(s) and shall transmit an ACK if the MsgB with a successRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble and if the Contention Resolution is successful and if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting ACK in the case of CCA UL failure. If ACK transmission is detected on a grant expected to have UL CCA failure, the test is considered as failed.

The UE may stop monitoring for MsgB(s) and shall transmit the msg3 if the MsgB with a fallbackRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting msg3 when CCA UL failure. If a msg3 is detected on a grant expected to have UL CCA failure, the test is considered as failed.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB(s) contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2A.3. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.11.2.2.2.3.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2A.3.1.3 the System Simulator shall transmit a MsgB containing a successRAR message and a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles. In case of CCA DL failure, the test equipment should delay the transmission of MsgB.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if no MsgB is received within the MsgB Response window.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2A.3. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be  0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

##### A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA

###### A.11.2.2.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits when subject to CCA. This test will verify the requirements in Clause 6.2.2A.3 and Clause 7.1.2 in an AWGN model.

For this test one cell is used and configured as PCell in FR1, which operates on a carrier frequency with CCA. Supported test parameters are shown in Table A.11.2.2.2.4.1-1. UE capable of SA with PCell in FR1 needs to be tested by using the parameters in Table A.11.2.2.2.4.1-2.

Table A.11.2.2.2.4.1-1: Supported test configurations for non-contention based random access test for FR1 PCell with CCA

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: Void. |  |

Table A.11.2.2.2.4.1-2: General test parameters for non-contention based random access test for FR1 PCell with CCA

| Parameter |  |  |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration | Note 4, 6 |  |  | Config 1 |  | SSB.3 CCA | As defined in A.3.10A |
|  | Note 5, 6 |  |  | Config 1 |  | SSB.4 CCA | As defined in A.3.10A |
| DBT Window Configuration |  |  |  | Config 1 |  | DBT.1 | As specified in A.3.28.1 |
| DL CCA model |  |  |  | Config 1 |  | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  | Config 1 |  | As specified in A.3.26.2.2 |  |
| Duplex Mode for Cell 1 |  |  |  | Config 1 |  | TDD |  |
| TDD Configuration |  |  |  | Config 1 |  | TDDConf.1.1 CCA |  |
| OCNG Pattern Note 1 |  |  |  |  |  | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH parameters Note 3 |  |  |  | Config 1 |  | SR.1.1 CCA | As defined in A.3.1A.1. |
| NR RF Channel Number |  |  |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  | dB |  |  |
| msgA-RSRP-ThresholdSSB |  |  |  |  | dBm | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| SSB with index 0 |  |  |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | 3 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured msgA-RSRP-ThresholdSSB |
|  |  |  |  | Config 1 | dBm/15kHz | -101 |  |
|  |  |  |  |  | dB | -17 |  |
|  |  | SS-RSRP |  |  | dBm/ SCS | -115 |  |
| Io Note 2 |  |  |  | Config 1 | dBm | -62.2/38.16MHz | For symbols without SSB index 1 |
| ss-PBCH-BlockPower |  |  |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [2]. |
| Configured UE transmitted power () |  |  |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1. |
| MsgA Configuration |  |  |  |  |  | FR1 MsgA configuration 2 under CCA | As defined in A.3.20A.2. |
| DL CCA probability |  |  | Note 4, 6 |  |  | 0.9375 |  |
| PCCA_DL |  |  | Note 5, 6 |  |  | 0.75 / 0.75 |  |
| LCCA_DL Note 7 |  |  |  |  |  | 4 |  |
| WCCA_DL Note 8 |  |  |  |  |  | Inf |  |
| UL CCA probability |  |  | Note 4, 6 |  |  | 0.87 |  |
| PCCA_UL |  |  | Note 5, 6 |  |  | 0.75 |  |
| LCCA_UL Note 7 |  |  |  |  |  | 5 |  |
| WCCA_UL Note 8 |  |  |  |  |  | Inf |  |
| Semi-static channel access config period Note 4, 6 |  |  |  |  | ms | 2 |  |
| Propagation Condition |  |  |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic channel occupancy and semi-static channel occupancy configuration. Note 7: LCCA_DL and LCCA_UL are chosen such that preambleTransMax > 5 + LCCA_DL + LCCA_UL. Note 8: A window WCCA_DL=WCCA_UL=Inf is used to indicate that LCCA_DL and LCCA_UL are considered during the entire duration of a test run. |  |  |  |  |  |  |  |

###### A.11.2.2.2.4.2	Test Requirements

Non-Contention based random access is triggered by explicitly assigning a random access preamble via dedicated signalling in the downlink. In the test, the non-contention based random access procedure is not initialized for Other SI requested from UE or beam failure recovery.

A.11.2.2.2.4.2.1	MsgA Transmission

To test the UE behavior specified in Clause 6.2.2A.3.2.1, with the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs configured, the System Simulator shall receive the MsgA with a preamble which belongs to one of the Random Access Preambles associated with the SSB with index 0.

In addition, the System Simulator shall receive the MsgA PRACH on the PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and the selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given first by the msgA-SSB-SharedRO-MaskIndex if configured, or next by the ra-ssb-OccasionMaskIndex if configured.

The three requirements below are relevant for all cases of MsgA transmissions described within the clause A.11.2.2.2.4.2:

The System Simulator shall implement the UL CCA model for the MsgA occasions (i.e. both MsgA PRACH and MsgA PUSCH occasions) where MsgA transmissions are expected. The System Simulator shall monitor the MsgA occasions to detect if the UE is transmitting MsgA. If a MsgA transmission is detected on MsgA occasions that are expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit MsgA for semi-static channel access mode; for dynamic channel access mode it is assumed that MsgA occasions are always scheduled within a UE-initiated COT.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated PRACH transmission power in case of UL CCA failure.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.3. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

A.11.2.2.2.4.2.2	MsgB Reception

To test the UE behavior specified in Clause 6.2.2A.3.2.2 the System Simulator shall transmit a MsgB containing a fallbackRAR containing a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a MsgB not corresponding to the transmitted Random Access Preamble. In case of CCA DL failure, the test equipment should delay the transmission of MsgB.

The UE may stop monitoring for MsgB(s) and shall transmit the msg3 containing the payload of MsgA PUSCH if the MsgB with a fallbackRAR contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble if UL CCA is successful. The System Simulator shall monitor if the UE is transmitting msg3 when CCA UL failure. If a msg3 is detected on a grant expected to have UL CCA failure, the test is considered as failed. The UE shall monitor contention resolution as described in clause 8.2A in TS 38.213 [3].

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA PRACH and MsgA PUSCH transmission power when the backoff time expires if all received MsgB’s contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The system simulator shall implement the UL CCA model of A.3.26.2 for the MsgA occasions where MsgA System Simulator transmissions are expected. The System Simulator shall monitor the MsgA occasions to detect if the UE is transmitting MsgA. If a MsgA transmission is detected on a MsgA occasion that is expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit MsgA for semi-static channel access mode.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated MsgA transmission power in case UL CCA failure.

In addition, the power applied to all preambles shall be in accordance with what is specified in Clause 6.2.2A.3. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional preambles shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA and msg3 transmissions shall be within the accuracy specified in Clause 7.1.2.

A.11.2.2.2.4.2.3	No MsgB Reception

To test the UE behavior specified in clause 6.2.2A.3.2.3 the System Simulator shall transmit a MsgB containing a successRAR message and a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator. The System Simulator shall not respond to the first 4 preambles. In case of CCA DL failure, the test equipment should delay the transmission of MsgB.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS 38.321 [7], and transmit with the calculated MsgA transmission power when the backoff time expires if no MsgB is received within the MsgB Response window.

The System Simulator shall implement the UL CCA model of A.3.26.2 for the MsgA occasions where MsgA transmissions are expected. The System Simulator shall monitor the MsgA occasions to detect if the UE is transmitting MsgA. If a MsgA transmission is detected on a MsgA occasion that is expected to have UL CCA failure, the test is considered as failed.

In case of CCA DL failure, the test equipment should verify that the UE does not transmit MsgA for semi-static channel access mode.

The UE shall again perform the Random Access Resource selection procedure specified in clause 5.1.2a in TS38.321 [7], and transmit with the calculated MsgA transmission power in case UL CCA failure.

In addition, the power applied to all MsgA transmissions shall be in accordance with what is specified in Clause 6.2.2A.3. The power of the first MsgA preamble shall be -16 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18]. The power of the first MsgA PUSCH transmission shall be 0.6+3μ+2 dBm with an accuracy specified in clause 6.3.4.2 of TS 38.101-1 [18], where μ indicates the MsgA PUSCH numerology. The relative power applied to additional MsgA transmissions shall have an accuracy specified in clause 6.3.4.3 of TS 38.101-1 [18].

The transmit timing of all MsgA transmissions shall be within the accuracy specified in Clause 7.1.2.

#### A.11.2.2.3	RRC connection release with redirection

##### A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA

A.11.2.2.3.1.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA specified in clause 6.2.3.2.3.

A.11.2.2.3.1.2	Test Parameters

Supported test configurations are shown in table A.11.2.2.3.1.2-1. The time delay is tested by using the parameters in table A.11.2.2.3.1.2-2, and A.11.2.2.3.1.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.11.2.2.3.1.2-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.2.2.3.1.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  |  | Cell 1 | On the carrier under CCA |
|  | Neighbouring cell |  |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  |  | Cell 2 | On the carrier under CCA |
| Filter coefficient |  |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  |  | 3 s | Synchronous cells |
| DL CCA model |  | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.26.2.1 |  |
|  |  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model |  | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.26.2.2 |  |
|  |  | Semi-static channel access Note 2,3 |  |  |  |
| T1 |  |  | s | 5 |  |
| T2 |  |  | s | ≥ Tconnection_release_redirect_NR_CCA | Tconnection_release_redirect_NR_CCA is defined in clause 6.2.3.2.3 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.11.2.2.3.1.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  | - | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  | - | PCCA_DL=0.9375 |  |  |  | PCCA_DL=0.9375 |  |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  | - | 1 |  |  |  | 1 |  |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  | - | 1 |  |  |  | 1 |  |  |
| LCCA_DL Note 7 |  |  |  | N/A |  |  |  | 8 |  |  |
| WCCA_DL Note 7 |  |  | ms | N/A |  |  |  | Tidentify-NR_CCA |  |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 CCA |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 CCA |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| DBT configuration |  |  |  | DBT.1 |  |  |  |  |  |  |
| SSB configuration for semi-static channel access Note 4, 6 |  | Config 1 |  | SSB.1 CCA |  |  |  |  |  |  |
| SSB configuration for dynamic channel access Note 5, 6 |  | Config 1 |  | SSB.2 CCA |  |  |  |  |  |  |
| ssb-PositionQCL |  | Config 1 |  | [1] |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1 | kHz | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1 |  | dBm/SCS | -95 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
| IoNote3 | Config 1 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -58.49 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5 For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 7: As defined in clause 6.2.3.2.3 for Trs ≤ 40 ms. |  |  |  |  |  |  |  |  |  |  |

A.11.2.2.3.1.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tconnection_release_redirect_NR_CCA ms from the beginning of time period T2, where Tconnection_release_redirect_NR_CCA is defined in clause 6.2.3.2.3.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR_CCA = TRRC_procedure_delay + Tidentify-NR_CCA + TSI-NR_CCA + TRACH_CCA,

where:

TRRC_procedure_delay = 110 ms in the test.

Tidentify-NR_CCA = MAX (680 ms, (L1+11)  20 ms) in the test.

TSI-NR =  1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH_CCA is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell.

L1 is the number of SMTC occasions not available at the UE due to DL CCA failures. The test equipment ensure that number of L1 in target cell does not exceed L1,max using the configured LCCA_DL as in clause A.3.26.2.1;

##### A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA

A.11.2.2.3.2.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA specified in clause 6.2.3.2.3.

A.11.2.2.3.2.2	Test Parameters

Supported test configurations are shown in table A.11.2.2.3.2.2-1. The time delay is tested by using the parameters in table A.11.2.2.3.2.2-2, and A.11.2.2.3.2.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.11.2.2.3.2.2-1: Redirection from NR to NR test configurations

| Configuration | Source cell without CCA | Target cell with CCA |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.11.2.2.3.2.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial conditions |  | Active cell |  | Cell 1 | On the carrier without CCA |
|  |  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition |  | Active cell |  | Cell 2 | On the carrier under CCA |
| Filter coefficient |  |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  |  | 3 s | Synchronous cells |
| DL CCA model | Dynamic channel accessNote 1, 3 |  |  | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  |  | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |  |
| T1 |  |  | s | 5 |  |
| T2 |  |  | s | ≥ Tconnection_release_redirect_NR_CCA | Tconnection_release_redirect_NR_CCA is defined in clause 6.2.3.2.3 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.11.2.2.3.2.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  |  | N/A |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  |  | N/A |  | PCCA_DL=0.9375 | PCCA_DL=0.9375 |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  |  | N/A |  | 1 | 1 |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  |  | N/A |  | 1 | 1 |  |
| LCCA_DL Note 7 |  |  |  | N/A |  | 8 |  |  |
| WCCA_DL Note 7 |  |  |  | N/A |  | Tidentify-NR_CCA |  |  |
| Duplex mode |  | Config 1 |  | FDD |  | TDD |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  | TDDConf.1.1 CCA |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  | TDDConf.1.1 CCA |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  | TDDConf.1.1 CCA |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | SR.1.1 CCA |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 CCA |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR.1.1 CCA |  |  |
| RMSI CORESET RMC configuration |  | Config 1 |  | CR.1.1 FDD |  | CR.1.1 CCA |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 CCA |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR.1.1 CCA |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 FDD |  | CCR.1.1 CCA |  |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 CCA |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.1.1 CCA |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration | Semi-static channel acces | Config 1,2 |  | SSB.1 FR1 |  | SSB.1 CCA |  |  |
|  | Dymamic channel acces | Config 3 |  | SSB.2 FR1 |  | SSB.2 CCA |  |  |
|  | Semi-static channel acces | Config 1,2 |  | SSB.1 FR1 |  | SSB.1 CCA |  |  |
|  | Dymamic channel acces | Config 3 |  | SSB.2 FR1 |  | SSB.2 CCA |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  | SMTC.2 FR1 |  |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  | 30 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  | 30 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA in Table A.3.8A.2.1-1 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |
| Note2 |  | Config 1,2 | dBm/SCS | -98 |  | -95 |  |  |
|  |  | Config 3 |  | -95 |  |  |  |  |
|  |  |  | dB | 4 | 4 | -infinity |  | 4 |
|  |  |  | dB | 4 | 4 | -infinity |  | 4 |
| IoNote3 |  | Config 1,2 | dBm/9.36MHz | -64.59 | -64.59 | N/A |  | N/A |
|  |  | Config 3 | dBm/38.16MHz | -58.49 | -58.49 | -63.94 |  | -58.49 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 7: As defined in clause 6.2.3.2.3 for Trs ≤ 40 ms. |  |  |  |  |  |  |  |  |

A.11.2.2.3.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tconnection_release_redirect_NR_CCA ms from the beginning of time period T2, where Tconnection_release_redirect_NR_CCA is defined in clause 6.2.3.2.3.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR_CCA = TRRC_procedure_delay + Tidentify-NR_CCA + TSI-NR_CCA + TRACH_CCA,

where:

TRRC_procedure_delay = 110 ms in the test.

Tidentify-NR_CCA = MAX (680 ms, (L1+11)  20 ms) in the test.

TSI-NR_CCA = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH_CCA is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TRACH_CCA = (1+L2)TSSB,RO + 10 ms; where TSSB,RO = 10 ms for FR1 PRACH configuration 1.

L1 is the number of SMTC occasions not available at the UE due to DL CCA failures. The test equipment shall ensure that L1 does not exceed L1,max. In the test L1,max= LCCA_DL which is defined in clause A.3.26.2.1.

L2 is the consecutive number of SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failures. L2 = 0 in the test.

The total delay, Tconnection_release_redirect_NR_CCA, shall be less than 1410 + MAX (680, (L1+11)20) ms.

## A.11.3	Timing

### A.11.3.1	UE transmit timing

#### A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA

##### A.11.3.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb when PCell is subject to DL CCA and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table 11.3.1.1.1-1

Table A.11.3.1.1.1-1: Supported test configuration for UE transmit timing test

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |

For this test a single NR cell is used. Table A.11.3.1.1.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.11.3.1.1.1-3.

Table A.11.3.1.1.1-2: Cell Specific Test Parameters for UE transmit timing test

| Parameter |  | Unit | Configuration | Test1 | Test2 |
| --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  |  | 1 | 1 | 1 |
| TDD configuration |  |  | 1 | TDDConf.1.1 CCA |  |
| BWchannel |  | MHz | 1 | 40: NRB,c = 106 |  |
| Initial BWP Configuration |  |  | 1 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP Configuration |  |  | 1 | DLBWP.1.1 ULBWP.1.1 |  |
| DRX Cycle |  | ms | 1 | N/A | DRX.8Note5 |
| DL CCA model |  |  | 1 | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | 1 | As specified in clause A.3.26.2.2 |  |
| PDSCH Reference measurement channel |  |  | 1 | SR.1.1 CCA |  |
| RMSI CORESET Reference Channel |  |  | 1 | CR.1.1 CCA |  |
| Dedicated CORESET Reference Channel |  |  | 1 | CCR.1.1 CCA |  |
| OCNG Patterns |  |  | 1 | OP.1 |  |
| SSB configuration | Semi- static channel acces |  | 1 | SSB.1 CCA |  |
|  | Dymamic channel acces |  | 1 | SSB.2 CCA |  |
| SMTC Configuration |  |  | 1 | SMTC.1 FR1 |  |
| TRS configuration |  |  | 1 | TRS.1.2 TDD |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  |  | 1 | 0.9375 | 0.9375 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  |  | 1 | 0.75 | 0.75 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  |  | 1 | 0.75 | 0.75 |
| UL CCA probability (PCCA_UL) |  |  | 1 | 1 | 1 |
| EPRE ratio of PSS to SSS |  | dB | 1 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | dBm/30 KHz | 1 | -95 | -95 |
|  |  |  | 1 | 3 | 3 |
|  |  |  | 1 | 3 | 3 |
| SS-RSRPNote3 |  | dBm/30 kHz | 1 | -92 | -92 |
| IoNote3 |  | dBm/38.1MHz | 1 | -59.2 | -59.2 |
| Propagation condition |  |  | 1 | AWGN |  |
| SRS Config |  |  | 1 | SRSConf.1Note6 | SRSConf.2Note6 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRX related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.11.3.1.1.1-3 Note 7: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.26.2. Note 8: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

Table A.11.3.1.1.1-3: SRS Configuration for UE transmit timing test

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2 25 for test configuration 3 | 25 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 0 | Offset to align with DRX periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

##### A.11.3.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1) Setup NR PCell according to parameters given in Table A.11.3.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.11.3.1.1.2-1

Table A.11.3.1.1.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first detected path (in time) of DL SSB.  Skip this step for test 2 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

### A.11.3.2	UE timing advance

#### A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA

##### A.11.3.2.1.1	Test Purpose and Environment

The purpose of the test is to verify UE Timing Advance adjustment delay and accuracy requirement defined in clause 7.3.

##### A.11.3.2.1.2	Test Parameters

Supported test configurations are shown in table A.11.3.2.1.2-1. Both timing advance adjustment delay and accuracy are tested by using the parameters in table A.11.3.2.1.2-2, A.11.3.2.1.2-3 and A.11.3.2.1.2-4.

In all test cases, single cell is used. Each test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.11.3.2.1.2-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.4 in TS 38.321 [7]. The Timing Advance Command value shall be set to 31, which according to Clause 4.2 in TS 38.213 [3] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.11.3.2.1.2-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at slot n+k for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321 [7], shall be configured so that it does not expire in the duration of the test.

Table A.11.3.2.1.2-1: Supported test configuration for timing advance test

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE supporting SA operation only on NR band(s) with shared spectrum access is required to be tested |  |

Table A.11.3.2.1.2-2: General test parameters for timing advance test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.3.9.2.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.3.9.2.2-1 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.3.9.3.1-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.3.9.3.2-1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For 30 kHz SCS NTA_new = NTA_old + 4096*Tc (based on equation in clause 4.2 of TS 38.213 [3]) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.11.3.2.1.2-3: Cell specific test parameters for timing advance test

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 40: NRB,c = 106 |  |
| DRX Cycle |  | Config 1 | ms | Not Applicable |  |
| DL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.2 |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 CCA |  |
| TRS configuration |  | Config 1 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  | Config 1 |  | OCNG pattern 1 |  |
| SMTC configuration |  | Config 1 |  | SMTC.1 FR1 |  |
| SSB configuration | Semi- static channel acces | Config 1 |  | SSB.1 CCA |  |
|  | Dymamic channel acces | Config 1 |  | SSB.2 CCA |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1 |  | 1 |  |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  | Config 1 |  | 1 |  |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  | Config 1 |  | 1 |  |
| UL CCA probability PCCA |  | Config 1 |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  | Config 1 | dBm/30 kHz | -95 |  |
|  |  |  | dB | 3 |  |
|  |  |  | dB | 3 |  |
| IoNote3 |  | Config 1 | dBm/ 38.16MHz | -62.58 |  |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.26.2. Note 5: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

Table A.11.3.2.1.2-4: Sounding Reference Symbol Configuration for Timing Advance Accuracy Test

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 24 | Frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=4 for SCS 30kHz | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | Codebook | Codebook based UL transmission |
| startPosition | 0 | resourceMapping setting: SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

##### A.11.3.2.1.3	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. k+1 slots after the reception of the timing advance command, where k=5.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

## A.11.4	Signalling characteristics

### A.11.4.1	Radio link monitoring

#### A.11.4.1.1	Introduction

In the test cases specified in clause A.11.4.1, any uplink signal transmitted by the UE is used for detecting the in-/out-of-sync state of the UE. In terms of measurement, the uplink signal is verified based on the UE output power:

- UE output power higher than Transmit OFF power -50 dBm (as defined in TS 38.101-1 [18]) means uplink signal

- UE output power equal to or less than Transmit OFF power -50 dBm (as defined in TS 38.101-1 [18]) means no uplink signal.

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

#### A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode

##### A.11.4.1.2.1	Test purpose and environment

The purpose of this test is to verify that the UE properly detects the out-of-sync and in-sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 PCell radio link monitoring requirements in clause 8.1A.

In the test, UE is configured to perform RLM based on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.11.4.1.2.1-1. The test parameters are given in Tables A.11.4.1.2.1-2, A.11.4.1.2.1-3, and A.11.4.1.2.1-4 below. There is one cell (Cell 1), which is the active NR cell in FR1, in the test. Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

The test consists of three successive time periods, with time duration of T1, T2 and T3, respectively. Figure A.11.4.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE transmits according to UL CCA model. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40 ms) in the test.

Table A.11.4.1.2.1-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.4.1.2.1-2: General test parameters for PCell out-of-sync testing in non-DRX mode.

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1, Test 2 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| DL CCA model |  |  |  | As specified in clause A.3.20.2.1 |
| UL CCA model |  |  |  | As specified in clause A.3.20.2.2 |
| Duplex mode |  | Config 1 |  | TDD |
| BWchannel |  | Config 1 | MHz | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1 |  | [DLBWP.0.1] |
| DL dedicated BWP configuration |  | Config 1 |  | [DLBWP.1.1] |
| UL initial BWP configuration |  | Config 1 |  | [ULBWP.0.1] |
| UL dedicated BWP configuration |  | Config 1 |  | [ULBWP.1.1] |
| TDD configuration |  | Config 1 |  | TBD |
| CORESET Reference Channel |  | Config 1 |  | TBD |
| SSB configuration |  | Config 1 |  | TBD |
| DBT window configuration |  | Config 1 |  | TBD |
| PDSCH/PDCCH subcarrier spacing |  | Config 1 |  | 30 kHz |
| PRACH Configuration |  | Config 1 |  | TBD |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | [OP.1] |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | [1-0] |
|  | Number of Control OFDM symbols |  |  | [2] |
|  | Aggregation level |  | CCE | [8] |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | [4] |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | [4] |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | [6] |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | TBD |
| T311 timer |  |  | ms | TBD |
| N310 |  |  |  | TBD |
| N311 |  |  |  | TBD |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | [CSI-RS.2.1 TDD] |
| CSI-RS for tracking |  | Config 1 |  | [TRS.1.2 TDD] |
| T1 |  |  | s | TBD |
| T2 |  |  | s | TBD |
| T3 |  |  | s | TBD |
| D1 |  |  | s | TBD |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table A.11.4.1.2.1-3: Cell-specific test parameters for PCell out-of-sync testing in non-DRX mode.

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| DL CCA probability PCCA_DL |  | Note 6,8 |  | TBD |  |  | TBD |  |  |
|  |  | Note 7,8 |  |  |  |  |  |  |  |
| UL CCA probability PCCA_UL |  |  |  | TBD |  |  | TBD |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 4 |  |  | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |  |
| SNRNote 3,4 on RLM-RS | Config 1 |  | dB | 1 | [-7] | -15 | 1 | TBD | TBD |
| SNR on other channels and signals | Config 1 |  | dB | 1 |  |  | 1 |  |  |
|  | Config 1 |  | dBm/SCS | -95 |  |  | -95 |  |  |
| Propagation condition |  |  |  | TDL-C 300 ns 100 Hz |  |  | TDL-C 300 ns 100 Hz |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during muted slots or during DBT windows. NOTE 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 3: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT windows. NOTE 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3, respectively, in Figure A.10.3.1.2.1-1. NOTE 5: The SNR values are specified for testing a UE which supports 2 RX on at least one band. For testing of a UE which supports 4 RX on all bands, the SNR during T3 is A.3.6. NOTE 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 8: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |  |

Table A.11.4.1.2.1-4: Measurement gap configuration for PCell out-of-sync testing in non-DRX mode.

| Field | Test 1 | Test 2 |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | 0 | 0 |
| NOTE: Ensure that RLM RS is partially overlapped with measurement gap0 |  |  |

Figure A.11.4.1.2.1-1: SNR variation for out-of-sync testing.

##### A.11.4.1.2.2	Test requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

- During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

- The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

In Test 1, the UE is verified to meet the out-of-sync requirement for RLM-RS SSB Es/Iot <-7 dB.

In Test 2, the UE is verified to meet the out-of-sync requirement for RLM-RS SSB Es/Iot ≥-7 dB.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode

##### A.11.4.1.3.1	Test purpose and environment

The purpose of this test is to verify that the UE properly detects the out-of-sync and in-sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 PCell radio link monitoring requirements in clause 8.1A.

In the test, UE is configured to perform RLM based on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0 and SSB#1, and purpose set to ‘rlf’. Supported test configurations are shown in table A.11.4.1.3.1-1. The test parameters are given in Tables A.11.4.1.3.1-2, and A.11.4.1.3.1-3 below. There is one cell (Cell 1), which is the active NR cell in FR1, in the test. Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model.

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5, respectively. Figure A.11.4.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE transmits according to UL CCA model.

Table A.11.4.1.3.1-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |

Table A.11.4.1.3.1-2: General test parameters for PCell in-sync testing in non-DRX mode.

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| DL CCA model |  |  |  |  | As specified in clause A.3.20.2.1 |
| UL CCA model |  |  |  |  | As specified in clause A.3.20.2.2 |
| Duplex mode |  |  | Config 1 |  | TDD |
| BWchannel |  |  | Config 1 | MHz | 40: NRB,c = 106 |
| DL initial BWP configuration |  |  | Config 1 |  | [DLBWP.0.1] |
| DL dedicated BWP configuration |  |  | Config 1 |  | [DLBWP.1.1] |
| UL initial BWP configuration |  |  | Config 1 |  | [ULBWP.0.1] |
| UL dedicated BWP configuration |  |  | Config 1 |  | [ULBWP.1.1] |
| TDD Configuration |  |  | Config 1 |  | TBD |
| CORESET Reference Channel |  |  | Config 1 |  | TBD |
| SSB Configuration |  |  | Config 1 |  | TBD |
| DBT window configuration |  |  | Config 1 |  | TBD |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1 |  | 30 kHz |
| PRACH Configuration |  |  | Config 1 |  | TBD |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | [OP.1] |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  |  | [1-0] |
|  | Number of Control OFDM symbols |  |  |  | [2] |
|  | Aggregation level |  |  | CCE | [4] |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | [0] |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | [0] |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | [6] |
| Out of sync transmission parameters | DCI format |  |  |  | [1-0] |
|  | Number of Control OFDM symbols |  |  |  | [2] |
|  | Aggregation level |  |  | CCE | [8] |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | [4] |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | [4] |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | [6] |
| DRX |  |  |  |  | OFF |
| Gap pattern ID |  |  |  |  | N/A |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | TBD |
| T311 timer |  |  |  | ms | TBD |
| N310 |  |  |  |  | TBD |
| N311 |  |  |  |  | TBD |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | TBD |
| T2 |  |  |  | s | TBD |
| T3 |  |  |  | s | TBD |
| T4 |  |  |  | s | TBD |
| T5 |  |  |  | s | TBD |
| D1 |  |  |  | s | TBD |
| NOTE 1: All configurations are assigned to the UE prior to the start of time period T1. NOTE 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

Table A.11.4.1.3.1-3: Cell-specific test parameters for PCell in-sync testing in non-DRX mode.

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA_DL |  |  | TBD |  |  |  |  |
| UL CCA probability PCCA_UL |  |  | TBD |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1 | [-7] | [-15] | [-4.5] | 1 |
| SNR on other channels and signals | Config 1 | dB | 1 |  |  |  |  |
|  | Config 1 | dBm/SCS | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in slots with RMC burst transmission and is not transmitted during muted slots or during DBT windows. NOTE 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 3: SNR levels correspond to the signal to noise ratio over the transmitted SSS REs during DBT windows. NOTE 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure A.11.4.1.3.1-1. NOTE 5: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4 RX on all bands, the SNR during T3 and T4 is modified as specified in clause A.3.6. |  |  |  |  |  |  |  |

Figure A.11.4.1.3.1-1: SNR variation for in-sync testing.

##### A.11.4.1.3.2	Test requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

- During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode

##### A.11.4.1.4.1	Test purpose and environment

##### A.11.4.1.4.2	Test requirements

#### A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode

##### A.11.4.1.5.1	Test purpose and environment

##### A.11.4.1.5.2	Test requirements

### A.11.4.2	Interruption

#### A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell

##### A.11.4.2.1.1 Test Purpose and Environment

The purpose of this test is to verify NR PCell interruptions during Scell operations on an NR SCC with CCA. This test will verify the interruption requirements for NR PCell in NR SA specified in TS 38.133 clause 8.2.2 and 8.3A. Supported test configurations are shown in table A.11.4.2.1.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.11.4.2.1.1-2 and A.11.4.2.1.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 and Cell2 are PCell and SCell. Both of cell 1 and cell 2 are subject to CCA. The test consists of five time periods, with duration of T1, T2, T3, T4 and T5. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. Throughout the test, the PCell are continuously scheduled in DL. The power of signals on cell 1 and 2 is not modified during the test.

Prior to T1, a connection is started with cell 1 as the PCell, and measurements of cell 2 are configured with gap pattern 0, such that cell 2 is reported. This ensures that cell 2 is known at the start of time period T1 and is not itself part of the tested requirement.

The point in time at which the RRC message implying Scell addition is received at the UE antenna connector, defines the start of time period T1. Measurement gap pattern 0 shall be stopped when the Scell is configured.

The point in time at which the MAC-CE message implying Scell activation is received at the UE antenna connector, defines the start of time period T2.

The point in time at which the MAC-CE message implying Scell deactivation is received at the UE antenna connector, defines the start of time period T3.

The point in time at which deactivation delay requirement in section 8.3A are satisfied defines the start of time period T4

The point in time at which the RRC message implying Scell release is received at the UE antenna connector, defines the start of time period T5.

Table A.11.4.2.1.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.4.2.1.1-2: General test parameters for Interruptions during measurements on deactivated NR SCC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 |  |
| Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Configured SCell |  | Cell2 | Deactivated SCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to Cell1, Cell2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| T1 | s | <10 |  |
| T2 | s | <10 |  |
| T3 | s | <10 |  |
| T4 | s | <10 |  |
| T5 | s | <10 |  |

Table A.11.4.2.1.1-3: NR cell specific test parameters for Interruptions during measurements on deactivated NR SCC

| Parameter |  | Unit | Cell1 |  |  |  |  | Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| TDD configuration | Config 1 |  | TDDConf.1.1 CCA |  |  |  |  | TDDConf.1.1 CCA |  |  |  |  |
| BWchannel | Config 1 | MHz | 40: NRB,c = 106 |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |
| DL CCA model | Config 1 |  | As specified in clause A.3.20.2.1 |  |  |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |
| DL CCA probability for semi-static channel accessNote6,8 | PCCA_DL |  | 0.9375 |  |  |  |  | 0.9375 |  |  |  |  |
| DL CCA probability for dynamic channel accessNote7,8 | PCCA_DL_1 |  | 0.75 |  |  |  |  | 0.75 |  |  |  |  |
|  | PCCA_DL_2 |  | 0.75 |  |  |  |  | 0.75 |  |  |  |  |
| UL CCA model | Config 1 |  | As specified in clause A.3.20.2.2 |  |  |  |  | --- |  |  |  |  |
| UL CCA probability for semi-static channel access | PCCA_UL |  | 0.87 |  |  |  |  | --- |  |  |  |  |
| UL CCA probability for dynamic channel access | PCCA_UL |  | 0.75 |  |  |  |  | --- |  |  |  |  |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Dedicated DL BWP Configuration | Config 1 |  | DLBWP.1.1 |  |  |  |  | DLBWP.1.1 |  |  |  |  |
| Initial UL BWP Configuration | Config 1 |  | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Dedicated UL BWP Configuration | Config 1 |  | ULBWP.1.1 |  |  |  |  | ULBWP.1.1 |  |  |  |  |
| PDSCH reference meassurement channel | Config 1 |  | SR.1.1 CCA |  |  |  |  | --- |  |  |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 CCA |  |  |  |  | CR.1.1 CCA |  |  |  |  |
| PDCCH CORESET parameters | Config 1 |  | CCR.1.1 CCA |  |  |  |  | CCR.1.1 CCA |  |  |  |  |
| TRS configuration | Config 1 |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  | OP.1 |  |  |  |  |
| SSB configuration for semi-static channel accessNote6,8 | Config 1 |  | SSB.1 CCA |  |  |  |  | SSB.1 CCA |  |  |  |  |
| SSB configuration for dynamic channel accessNote7,8 | Config 1 |  | SSB.2 CCA |  |  |  |  | SSB.2 CCA |  |  |  |  |
| SMTC Configuration | Config 1 |  | SMTC.1 |  |  |  |  | SMTC.1 |  |  |  |  |
| DBT window configuration | Config 1 |  | DBT.1 |  |  |  |  | DBT.1 |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  | TCI.State.0 |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  | 1x2 Low |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 |  |  |  |  | -104 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  | -87 |  |  |  |  |
| Ês/Iot |  | dB | 17 |  |  |  |  | 17 |  |  |  |  |
| Ês/Noc |  | dB | 17 |  |  |  |  | 17 |  |  |  |  |
| IoNote3 | Config 1 | dBm/38.16MHz | -52.86 |  |  |  |  | -52.86 |  |  |  |  |
| Time offset to Cell1 Note 4 |  | ms | 3 |  |  |  |  | 3 |  |  |  |  |
| Time offset to Cell2 Note 5 |  | s | - |  |  |  |  | 3 |  |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that resources in the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. OCNG is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Receive time difference of signals received between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell at the UE antenna connector including time alignment error between the two cells Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 8: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.11.4.2.1.2	Test Requirements

The UE shall meet the interruption requirements for Scell addition on the victim Pcell in clause 8.2.1 during time T1

The UE shall meet the interruption requirements for Scell activation on the victim Pcell in clause 8.2.1during time T2. There shall be a single interruption with time window as specified in clause 8.3A.2

The UE shall meet the interruption requirements for Scell deactivation on the victim PCell in clause 8.2.1during time T3. There shall be a single interruption with time window as specified in clause 8.3A.3

The UE shall meet the interruption requirements for deactivated Scell measurements on the victim PCell in clause 8.2.1 during time T4. The interruptions shall be within the time window as specified in clause 8.3A.3

The UE shall meet the interruption requirements for Scell release on the victim PCell in clause 8.2.1 during time T5.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.11.4.3	SCell activation and deactivation delay

#### A.11.4.3.1 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle

##### A.11.4.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation delays for SCell, with PCell and SCell both under CCA, are within the requirements stated in clause 8.3A, when the SCell is known by the UE at the time of activation and the configured SCell measurement cycle is 160 ms.

The supported test configurations are shown in Table A.11.4.3.1.1-1.

The test parameters are given in Table A.11.4.3.1.1-2 and cell-specific parameters in Table A.11.4.3.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell: Cell 1 (PCell) on radio channel 1 (PCC) in NR with CCA, and Cell2 (SCell) on radio channel 2 (SCC) in NR with CCA. Before the test starts the UE is connected to Cell 1, but is not aware of Cell 2, as the UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. At the end of T1, the test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. The UE shall be able to report a valid CSI in PCell for the activated SCell at latest in slot m + (THARQ+Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, as defined in clause 8.3A.2. The UE shall start reporting CSI in PCell in first available uplink resource for CSI reporting following slot m+ THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption shall fall within the time window specified in clause 8.3.2.

The point in time at which the MAC message is received by at the UE antenna connector, in a slot # denoted n, defines the start of time period T3. The UE shall complete the activation at latest in slot n+THARQ+3msNR slot length. Any PCell interruption shall fall within the time window specified in clause 8.3A.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received, while taking into account CCA failures on SCC.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.11.4.3.1.1-1: Supported test configurations for SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle

| Configuration | Description |
| --- | --- |
| 1 | With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.4.3.1.1-2: General test parameters for known SCell activation with PCell and SCell under CCA, 160 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two radio channels (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | TAE as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 10+5⋅2µDL | The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] µDL is the subcarrier spacing configuration for DL |

Table A.11.4.3.1.1-3: Cell specific test parameters for known SCell activation case with PCell and SCell under CCA, 160 ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | TDD |  |  | TDD |  |  |
| TDD configuration | Config 1 |  | TDDConf.1.1 CCA |  |  | TDDConf.1.1 CCA |  |  |
| BWchannel | Config 1 | MHz | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |  | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |  | As specified in clause A.3.26.2.2 |  |  |
| DL CCA probability for semi-static channel accessNote5,7 | PCCA_DL |  | 0.9375 |  |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel accessNote6,7 | PCCA_DL_1 |  | 0.75 |  |  | 0.75 |  |  |
|  | PCCA_DL_2 |  | 0.75 |  |  | 0.75 |  |  |
| UL CCA probability for semi-static channel access | PCCA_UL |  | 0.87 |  |  | 0.87 |  |  |
| UL CCA probability for dynamic channel access | PCCA_UL |  | 0.75 |  |  | 0.75 |  |  |
| LCCA_DL Note 8 |  |  | 2 |  |  | 2 |  |  |
| WCCA_DL Note 8 |  | ms | Tactivation_time_withCCA |  |  | Tactivation_time_withCCA |  |  |
| Initial downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Initial uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Dedicated downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Dedicated uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 CCA |  |  | SR.1.1 CCA |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.3 CCA |  |  | CCR.1.3 CCA |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 CCA |  |  | CR.1.1 CCA |  |  |
| OCNG Patterns Note1 |  |  | OP.1 |  |  | OP.1 |  |  |
| SSB Configuration for semi-static channel accessNote5,7 | Config 1 |  | SSB.1 CCA |  |  | SSB.1 CCA |  |  |
| SSB Configuration for dynamic channel accessNote6,7 | Config 1 |  | SSB.2 CCA |  |  | SSB.2 CCA |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  | SMTC.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1 | dBm/15kHz | -104 |  |  | -104 |  |  |
| Noc Note2 | Config 1 | dBm/SCS | -101 |  |  | -101 |  |  |
| Ês/Iot |  | dB | 17 |  |  | 17 |  |  |
| Ês/Noc |  | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note3 | Config 1 | dBm/SCS | -84 |  |  | -84 |  |  |
| IoNote3 | Config 1 | dBm/ 38.16MHz | -52.87 |  |  | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that resources in the cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. OCNG is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 8: As specified in clause 8.3A for L1,max, L2,1,max, L2,2,max, L3,1,max, and L3,2,max |  |  |  |  |  |  |  |  |

##### A.11.4.3.1.2	Test Requirements

During T2, starting from the slot specified in clause 4.3 of TS 38.213 [3] and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

During T2, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in first available uplink resource for CSI reporting following slot m + (THARQ+Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB + L1*Trs + 5ms, as specified in clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in clause 8.3A.3.

During T2, interruption on PCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB.

During T3, interruption on PCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PCell shall not be more than specified for SA in clause 8.2.2.2.2.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

#### A.11.4.3.2 SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 320 ms SCell measurement cycle

##### A.11.4.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation delays for SCell, with PCell and SCell under CCA, are within the requirements stated in clause 8.3A, when the SCell is known by the UE at the time of activation and the configured SCell measurement cycle is 320 ms.

The supported test configurations are same as in Table A.11.4.3.1.1-1 above.

The test parameters are same as in Table A.11.4.3.1.1-2 above, except for parameters listed below in Table A.11.4.3.2.1-1. The cell-specific parameters are same as in Table A.11.4.3.1.1-3 above.

The test execution is the same as described in clause A.11.4.3.1 above.

Table A.11.4.3.2.1-1: General test parameters for known SCell activation with PCell and SCell under CCA, 320 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |

##### A.11.4.3.2.2	Test Requirements

During T2, starting from the slot specified in clause 4.3 of TS 38.213 [3] and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

During T2, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in first available uplink resource for CSI reporting following slot m + (THARQ+Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB_MAX + L2,1*TSMTC_MAX + (1 +L2,2)*Trs + 5ms, as specified in clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in clause 8.3A.3.

During T2, interruption on PCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB_MAX + L2,1* TSMTC_MAX.

During T3, interruption on PCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PCell shall not be more than specified for SA in clause 8.2.2.2.2.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

#### A.11.4.3.3 SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA

##### A.11.4.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation delays for SCell, with PCell and SCell under CCA, are within the requirements stated in clause 8.3A, when the SCell is unknown to the UE at the time of activation.

The supported test configurations are same as in Table A.11.4.3.1.1-1 above.

The test parameters are same as in Table A.11.4.3.1.1-2 above, except for parameters listed below in Table A.11.4.3.3.1-1. The cell-specific parameters are same as in Table A.11.4.3.1.1-3 above.

The test execution is the same as described in clause A.11.4.3.1 above.

Table A.11.4.3.3.1-1: General test parameters for unknown SCell activation with PCell ans SCell under CCA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | s | 0.1 | During this time period the PCell shall be known and the SCell configured, but not detected. |

##### A.11.4.3.3.2	Test Requirements

During T2, starting from the slot specified in clause 4.3 of TS 38.213 [3] and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

During T2, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in first available uplink resource for CSI reporting following slot m + (THARQ+Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB_MAX + (1 + L3,1)*TSMTC_MAX + (2 + L3,2)*Trs + 5ms, as specified in clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in clause 8.3A.3.

During T2, interruption on PCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB_MAX + L3,1* TSMTC_MAX.

During T3, interruption on PCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PCell shall not be more than specified for SA in clause 8.2.2.2.2.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

### A.11.4.4	Beam failure detection and link recovery procedures

#### A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode

##### A.11.4.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5A.

The test parameters are given in Tables A.11.4.4.1.1-1, A.11.4.4.1.1-2, A.11.4.4.1.1-3 and A.11.4.4.1.1-4 below. There is one cell, cell 1 which is the active cell, in the test. Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.11.4.4.1.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.11.4.4.1.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. The UE transmits the reporting according to UL CCA mode. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40 ms) in test 1.

Table A.11.4.4.1.1-1: Supported test configurations for FR1 PCell with CCA

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.11.4.4.1.1-2: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  |  |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 | Test 2 |  |
| Active PSCell |  |  |  |  |  | Cell 1 | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 | 1 |  |
| DL CCA model |  |  |  |  |  | As specifieed in A.3.20.2.1 | As specifieed in A.3.20.2.1 |  |
| UL CCA model |  |  |  |  |  | As specified in A.3.20.2.2 | As specified in A.3.20.2.2 |  |
| Duplex mode |  |  |  | Config 1 |  | TDD | TDD |  |
| BWchannel |  |  |  | Config 1 | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  |  | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  |  | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  |  | Config 1 |  | ULBWP.0.1 | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  |  | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |  |
| TDD Configuration |  |  |  | Config 1 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |  |
| CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 CCA | CR.1.1 CCA |  |
| SSB Configuration |  |  |  | Config 1 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |  |
| DBT Window Configuration |  |  |  | Config 1 |  | DBT.1 | DBT.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1 |  | 30 KHz | 30 KHz |  |
| PRACH Configuration |  |  |  | Config 1 |  | Table A.3.8.2.2-1 | Table A.3.8.2.2-1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  | 0 | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  | 1 | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 | OP.1 |  |
| CP length |  |  |  |  |  | Normal | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  | 1-0 | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  |  | 2 | 2 |  |
|  |  | Aggregation level |  |  | CCE | 8 | [2] |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size | REG bundle size |  |
|  |  | REG bundle size |  |  |  | 6 | 6 |  |
| DRX |  |  |  |  |  | OFF | OFF |  |
| Gap pattern ID |  |  |  |  |  | gp0 | gp0 |  |
| gapOffset |  |  |  |  |  | 0 | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1 |  |  |  | dBm/SCS kHz | -95 | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  | db0 | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  |  | CSI-RS.2.1 TDD | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  |  | Config 1 |  |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  |  | 0, 1 | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | [1000] | [1000] |  |
| N310 |  |  |  |  |  | [2] | [2] |  |
| T1 |  |  |  |  | s | [0.2] | [0.2] | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | [0.93] | [0.85] |  |
| T3 |  |  |  |  | s | [0.52] | [0.44] |  |
| T4 |  |  |  |  | s | [0] | [0] |  |
| T5 |  |  |  |  | s | [0.45] | [0.45] |  |
| D1 |  |  |  |  | s | [0.41] | [0.41] |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |  |

Table A.11.4.4.1.1-3: Cell specific test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA,DL |  | Note 10, 12 |  | [1.0] | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
|  |  | Note 11, 12 |  | [1.0]/[1.0] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA,UL |  |  |  | [1.0] | [1.0] | [1.0] | [1.0] | [1.0] |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 |  | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 |  | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 |  | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 |  | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause [A.3.6A]. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |

Table A.11.4.4.1.1-4: Cell specific test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  |  | Unit | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA |  | Note 10, 12 |  | [1.0] | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
|  |  | Note 11, 12 |  | [1.0]/[1.0] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA |  |  |  | [1.0] | [1.0] | [1.0] | [1.0] | [1.0] |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 |  | dB | 5 | [-2] | [-4] | [-4] | [-4] |
| SNR_SSB of set q1 | Config 1 |  | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 |  | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 |  | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause [A.3.6A]. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic cannel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |  |  |  |

Figure A.11.4.4.1.1-1: SNR and L1-RSRP variation SSB for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.11.4.4.1.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [410] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

In Test 1, the UE is verified to meet the beam failure detection for BFD-RS SSB Es/Iot < -7 dB.

In Test 2, the UE is verified to meet the beam failure detection for BFD-RS SSB Es/Iot ≥ -7 dB.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode

##### A.11.4.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5A.

The test parameters are given in Tables A.11.4.4.2.1-1, A.11.4.4.2.1-2, A.11.4.4.2.1-3 and A.11.4.4.2.1-4 below. There is one cell, cell 1 which is the active cell, in the test. Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.11.4.4.2.1-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure A.11.4.4.2.1-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. The UE transmits the reporting according to UL CCA mode. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.11.4.4.2.1-1: Supported test configurations for FR1 PCell with CCA

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.11.4.4.2.1-2: General test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  |  |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 | Test 2 |  |
| Active PSCell |  |  |  |  |  | Cell 1 | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 | 1 |  |
| DL CCA model |  |  |  |  |  | As specified in A.3.26.2.1 | As specified in A.3.26.2.1 |  |
| UL CCA model |  |  |  |  |  | As specified in A.3.26.2.2 | As specified in A.3.26.2.2 |  |
| Duplex mode |  |  |  | Config 1 |  | TDD | TDD |  |
| BWchannel |  |  |  | Config 1 | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  |  | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  |  | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  |  | Config 1 |  | ULBWP.0.1 | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  |  | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 |  |
| TDD Configuration |  |  |  | Config 1 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |  |
| CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 CCA | CR.1.1 CCA |  |
| SSB Configuration |  |  |  | Config 1 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |  |
| DBT Window Configuration |  |  |  | Config 1 |  | DBT.1 | DBT.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1 |  | 30 KHz | 30 KHz |  |
| PRACH Configuration |  |  |  | Config 1 |  | Table A.3.8.2.2-1 | Table A.3.8.2.2-1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  | 0 | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  | 1 | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 | OP.1 |  |
| CP length |  |  |  |  |  | Normal | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  | 1-0 | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  |  | 2 | 2 |  |
|  |  | Aggregation level |  |  | CCE | 8 | [2] |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 | 0 |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size | REG bundle size |  |
|  |  | REG bundle size |  |  |  | 6 | 6 |  |
| DRX |  |  |  |  |  | DRX.7 | DRX.7 | A.3.3.7 |
| Gap pattern ID |  |  |  |  |  | N.A. | N.A. |  |
| gapOffset |  |  |  |  |  | 0 | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | absent | When the field is absent, the UE applies the value 0. (Table 8.1.1-1). |
| rsrp-ThresholdSSB | Config 1 |  |  |  | dBm/SCS kHz | -95 | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  | db0 | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | n1 | see clause 5.17 of TS 38.321 [7] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | pbfd4 | see clause 5.17 of TS 38.321 [7] |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  |  | CSI-RS.2.1 TDD | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  |  | Config 1 |  |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  |  | 0, 1 | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | [1000] | [1000] |  |
| N310 |  |  |  |  |  | [2] | [2] |  |
| T1 |  |  |  |  | s | [1] | [1] | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | [9.01] | [8.37] |  |
| T3 |  |  |  |  | s | [5.16] | [4.52] |  |
| T4 |  |  |  |  | s | [0] | [0] |  |
| T5 |  |  |  |  | s | [3.89] | [3.89] |  |
| D1 |  |  |  |  | s | [3.85] | [3.85] |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |  |

Table A.11.4.4.2.1-3: Cell specific test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA_DL |  | Semi-static channel access PCCA_DLNote 10, 12 |  | 1.0 | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
|  |  | Note 11, 12 |  | 1.0/1.0 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 |
| UL CCA probability PCCA |  |  |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| LCCA_DL |  |  |  | N/A | 3 |  |  |  |
| WCCA_DL |  |  | ms | N/A | TEvaluate_CBD_SSB_CCA Note 13 |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 |  | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 |  | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 |  | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 |  | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6A. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 13: As defined in Table 8.5A.5.2-1, where LCBD,max=3 for TDRX > 320. |  |  |  |  |  |  |  |  |

Table A.11.4.4.2.1-4: Cell specific test parameters for FR1 PCell for SSB-based beam failure detection and link recovery testing in DRX mode

| Parameter |  |  | Unit | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| DL CCA probability PCCA_DL |  | Semi-static channel access PCCA_DLNote 10, 12 |  | 1.0 | 0.9375 | 0.9375 | 0.9375 | 0.9375 |
|  |  | Dynamic channel access, PCCA_DL_1/PCCA_DL_2 Note 11, 12 |  | 1.0/1.0 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 | 0.75/0.75 |
| UL CCA probability PCCA |  |  |  | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
| LCCA_DL |  |  |  | N/A | 3 |  |  |  |
| WCCA_DL |  |  | ms | N/A | TEvaluate_CBD_SSB_CCA Note 13 |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 |  | dB | 5 | [2] | [-4] | [-4] | [-4] |
| SNR_SSB of set q1 | Config 1 |  | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 |  | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  | Config 1 |  | dBm/15 KHz | -98 |  |  |  |  |
| Propagation condition |  |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio the transmitted SSS REs during DBT window. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.4.5.5.1.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6A. Note 10: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 11: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. Note 12: For UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 13: As defined in Table 8.5A.5.2-1, where LCBD,max=3 for TDRX > 320. |  |  |  |  |  |  |  |  |

Figure A.11.4.4.2.1-1: SNR and L1-RSRP variation for SSB-based beam failure detection and link recovery testing in non-DRX mode

##### A.11.4.4.2.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = [3850] ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

In Test 1, the UE is verified to meet the beam failure detection for BFD-RS SSB Es/Iot < -7 dB.

In Test 2, the UE is verified to meet the beam failure detection for BFD-RS SSB Es/Iot ≥ -7 dB.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

### A.11.4.5	Active BWP switching

#### A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA

##### A.11.4.5.1.1	Test Purpose and Environment

The purpose of this test is to verify the UL BWP switch delay requirement defined in clause 8.6.4.

The supported test configurations are shown in Table A.11.4.5.1.1-1. The test scenario comprises of one cell (Cell 1), which is Pcell as given in Table A.11.4.5.1.1-2. Cell-specific parameters of the cell are specified in Table A.11.4.5.1.1-3 below. SRS configuration used in the test is specified in Table A.11.4.5.1.1-4.

Before the test starts,

- UE is connected to Cell 1 on radio channel 1.

- UE is configured with 2 different UE-specific downlink and uplink bandwidth parts: DL BWP-1, DL BWP-2, UL BWP-1 and UL BWP-2 before starting the test. DL BWP-1 and DL BWP-2 always include bandwidth of the initial DL BWP and SSB. UL BWP-1 and UL BWP-2 always include bandwidth of the SRS.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is DL BWP-1.

- UE is indicated in firstActiveUplinkBWP-Id that the active UL BWP is UL BWP-1.

- UE is configured with LBT-FailureRecoveryConfig parameters for Cell 1.

The cell has constant signal levels throughout the test. The test consists of 2 successive time periods, with durations of T1 and T2, respectively.

During T1,

- Time period T1 starts when the UE has received the SRS configuration for periodic SRS transmission on active UL BWP-1.

- The UE shall perform UL CCA before SRS transmission.

- The parameter UL CCA probability PCCA is set to 0 during T1. This requires the test system to set energy level above the detection level during portion of the UL slot where the UE performs UL CCA. This in turn forces the UE to fail the UL CCA. The UE consistently fails UL CCA during T1 and is therefore unable to transmit SRS.

During T2,

- T2 starts when the UE detects consistent UL LBT failures i.e. when total number of UL LBT failures in cell1 on active UL BWP-1 exceeds lbt-FailureInstanceMaxCount during lbt-FailureDetectionTimer.

- The UE upon detected consistent UL LBT failure starts the LBT recovery mechanism, which requires the UE to switch to active UL BWP-2 in Cell 1 and to send PRACH in the active UL BWP-2.

- Staring from T2, the UE shall be able to send PRACH in the active UL BWP-2 within the delay specified in clause 8.6.4.

Table A.11.4.5.1.1-1: Supported test configurations for UL BWP switch test in SA

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: void |  |

Table A.11.4.5.1.1-2: General test parameters for UL BWP switch test in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| lbt-FailureDetectionTimer [2] | ms | 80 | Parameter configured by IE: LBT-FailureRecoveryConfig [1] |
| lbt-FailureInstanceMaxCount [2] |  | 4 | Parameter configured by IE: LBT-FailureRecoveryConfig [1] |
| T1 | s | 0.1 | During T1 consistent LBT failure is detected on active UL BWP-1 |
| T2 | s | 0.1 | During T2 UE sends PRACH on active UL BWP-2 |

Table A.11.4.5.1.1-3: NR Cell specific test parameters for UL BWP switch test in SA

| Parameter |  |  | Unit | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |
| BWchannel |  | Config 1 |  | 40 MHz: NRB,c = 106 |  |
| DL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  | Config 1 |  | As specified in clause A.3.26.2.2 |  |
| Active BWP ID |  | Config 1 |  | 1, 2 |  |
| Initial DL BWP Configuration |  | Config 1 |  | DLBWP.0.2 Note 4 |  |
| Active DL BWP-1 Configuration |  | Config 1 |  | DLBWP.1.1 Note 4 |  |
| Active DL BWP-2 Configuration |  | Config 1 |  | DLBWP.1.3 Note 4 |  |
| Initial UL BWP Configuration |  | Config 1 |  | ULBWP.0.2 Note 4 |  |
| Active UL BWP-1 Configuration |  | Config 1 |  | ULBWP.1.1 Note 4 |  |
| Active UL BWP-2 Configuration |  | Config 1 |  | ULBWP.1.3 Note 4 |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 CCA |  |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.3 CCA |  |
| OCNG Patterns |  | Config 1 |  | OP.1 |  |
| SSB Configuration | Semi- static channel acces | Config 1 |  | SSB.1 CCA |  |
|  | Dymamic channel acces | Config 1 |  | SSB.2 CCA |  |
| SMTC Configuration |  | Config 1 |  | SMTC.1 FR1 |  |
| Correlation Matrix and Antenna Configuration |  | Config 1 |  | 1x2 Low |  |
| TRS Configuration |  | Config 1 |  | TRS.1.2 TDD |  |
| DL CCA probability for semi-static channel access (PCCA_DL) |  | Config 1 |  | 1 | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_1) |  | Config 1 |  | 1 | 1 |
| DL CCA model probability for dynamic static channel access (PCCA_DL_2) |  | Config 1 |  | 1 | 1 |
| UL CCA probability (PCCA_UL) |  | Config 1 |  | 0 | 1 |
| PRACH configuration |  | Config 1 |  | N/A | Configuration #1 in Table A.3.8.2.1-1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1 | dBm/SCS | -101 |  |
| SS-RSRP Note 3 |  | Config 1 | dBm/SCS | -84 |  |
| Ês/Iot |  | Config 1 | dB | 17 |  |
| Ês/Noc |  | Config 1 | dB | 17 |  |
| IoNote3 |  | Config 1 | dBm/ 38.16MHz | -52.86 |  |
| Propagation Condition |  |  |  | AWGN |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. Note 5: Parameters PCCA_DL, PCCA_DL_1, PCCA_DL_2 and PCCA_UL are defined in clause A.3.26.2. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |

Table A.11.4.5.1.1-4: Sounding Reference Symbol Configuration for UL BWP Switch Test

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 24 | Frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=4 for SCS 30kHz | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage | Codebook | Codebook based UL transmission |
| startPosition | 0 | resourceMapping setting: SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

##### A.11.4.5.1.2	Test Requirements

The UE capable of bwp-SwitchingDelay type1 [2] shall start to transmit the PRACH on active UL BWP-2 of Cell 1 (PCell) less than 21.5 ms from the beginning of time period T1.

The UE capable of bwp-SwitchingDelay type2 [2] shall start to transmit the PRACH on active UL BWP-2 of Cell 1 (PCell) less than 23 ms from the beginning of time period T1.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE: The above delay is calculated as follows:’

The active UL BWP switch delay from UL BWP-1 to UL BWP-2 can be expressed as:

TBWPswitchDelay*Tslot +1*Tslot + (1+ L3)*TSSB,RO + 10 ms

Where:

-	TBWPswitchDelay = 1 ms (2 slots) and 2.5 ms (5 slots) for bwp-SwitchingDelay [2] type1 and type2 UE capabilities according to clause 8.6.4.

-	Tslot = It is the slot length. It is 0.5 ms for 30 kHz.

-	L3 = It is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure. L3= 0 during T2 since PCCA = 1.

-	TSSB,RO = 10 ms according to FR1 PRACH configuration 1.

This gives a total of 21.5 ms and 23 ms for type1 and type2 UE respectively.

#### A.11.4.5.2	DCI-based and Timer-based Active BWP Switch

##### A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA

A.11.4.5.2.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6, and interruption requirement on other active serving cell defined in clause 8.2.2.2.5.

The supported test configurations are shown in Table A.11.4.5.2.1.1-1 below. The test scenario comprises of one PCell (Cell 1) and one SCell (Cell 2) as given in Table A.11.4.5.2.1.1-2. NR Cell-specific parameters are specified in Table A.11.4.5.2.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PCell, BWP-1 and BWP-2, in Cell 1 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for SCell, BWP-0 in Cell 2 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell.

-	UE is configured with a bwp-InactivityTimer timer value for PCell.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for PCell DL BWP switch, sent from the test equipment to the UE, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the PCell no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-2 no later than the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

The starting time of SCell (Cell 2) interruption due to BWP switch on PCell shall occur within the BWP switch delay.

During T2, the test equipment won’t transmit DCI format for PDSCH reception on PCell (Cell 1).

During T3,

The time period T3 starts from the slot #j, where j is the first  slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE should switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH no later than the first DL slot that occurs after the beginning of PCell’s slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the SCell at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on PCell’s BWP-1 no later than the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The starting time of SCell (Cell 2) interruption due to BWP switch of PCell shall occur within the BWP switch delay.

The test equipment verifies the DL BWP switch time in PCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The test equipment verifies that potential interruption to SCell is carried out in the correct time span by monitoring ACK/NACK sent in SCell during BWP switch of PCell, respectively.

Table A.11.4.5.2.1.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: Void Note 2: The UE supporting SA operation with only NR band(s) with shared spectrum access is required to be tested. |  |

Table A.11.4.5.2.1.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used in this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| DL CCA model |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  | As specified in clause A.3.26.2.2 |  |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

Table A.11.4.5.2.1.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  | Unit | Cell 1 | Cell2 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |  |
| Duplex mode |  | Config 1 |  | TDD |  |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |  |
| BWchannel |  | Config 1 |  | 40 MHz: NRB,c = 106 |  |
| Active BWP ID |  |  |  | 1, 2 | 0 |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |  |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 |  |
| Active DL BWP-0 Configuration |  |  |  | N.A. | DLBWP.0.2Note4 |
| Active DL BWP-1 Configuration |  |  |  | DLBWP.1.1Note4 | N.A. |
| Active DL BWP-2 Configuration |  |  |  | DLBWP.1.3Note4 | N.A. |
| Active UL BWP-0 Configuration |  |  |  | N.A. | ULBWP.0.2Note4 |
| Active UL BWP-1 Configuration |  |  |  | ULBWP.1.1Note4 | N.A. |
| Active UL BWP-2 Configuration |  |  |  | ULBWP.1.3Note4 | N.A. |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |  |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 CCA |  |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.3 CCA |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SSB Configuration | Semi- static channel acces | Config 1 |  | SSB.1 CCA |  |
|  | Dymamic channel acces | Config 1 |  | SSB.2 CCA |  |
| SMTC Configuration |  | Config 1 |  | SMTC.1 |  |
| DL CCA probability (PCCA_DL) |  | Config 1 |  | 1 | 1 |
| UL CCA probability (PCCA_UL) |  | Config 1 |  | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | Config 1 | dBm/SCS | -101 | -101 |
| SS-RSRP Note 3 |  | Config 1 | dBm/SCS | -84 | -84 |
| Ês/Iot |  | Config 1 | dB | 17 | 17 |
| Ês/Noc |  | Config 1 | dB | 17 | 17 |
| IoNote3 |  | Config 1 | dBm/38.16MHz | -52.86 | -52.86 |
| Propagation Condition |  |  |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.11.4.5.2.1.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of SCell interruption during PCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first DL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA

A.11.4.5.2.2.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement defined in clause 8.6.

The supported test configurations are shown in Table A.11.4.5.2.2.1-1. The test scenario comprises of one cell (Cell 1) as given in Table A.11.4.5.2.2.1-2. Cell-specific parameters of the cell are specified in Table A.11.4.5.2.2.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE is configured with 2 different UE-specific downlink bandwidth parts, BWP-1 and BWP-2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

-	UE is configured with a bwp-InactivityTimer timer value for Cell1.

The cell has constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

During T1,

Time period T1 starts when a DCI format 1_1 command for DL BWP switch, sent from the test equipment to the UE, is received at the UE side in Cell1’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH on the first DL slot that occurs after the beginning of Cell1’s DL slot (i+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the Cell1 no later than the first UL slot that occurs after the beginning of slot (i+TBWPswitchDelay+k1). The UE shall be continuously scheduled on Cell1’s BWP-2 starting from the first DL slot that occurs after the beginning of slot (i+TBWPswitchDelay).

During T2, the test equipment won’t transmit DCI format for PDSCH reception on Cell1.

During T3,

The time period T3 starts from the slot #j, where j is the first slot of the subframe immediately after bwp-InactivityTimer timer expires. The UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1.

The UE shall be able to receive PDSCH on the first DL slot that occurs after the beginning of Cell1’s slot (j+TBWPswitchDelay) as defined in clause 8.6 and starts to report valid ACK/NACK for the Cell1 at latest on the first UL slot that occurs after the beginning of slot (j+TBWPswitchDelay+k1). The UE shall be continuously scheduled on Cell1’s BWP-1 starting from the first DL slot that occurs after the beginning of slot (j+TBWPswitchDelay).

The test equipment verifies the DL BWP switch time by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

Table A.11.4.5.2.2.1-1: DL BWP switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: A UE which fulfils the requirements in test case A.11.4.5.2.1 can skip the test cases in A.11.4.5.2.2. Note 3: The UE supporting SA operation with only NR band(s) with shared spectrum access is required to be tested. |  |

Table A.11.4.5.2.2.1-2: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |  |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |  |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |  |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| DL CCA model |  | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | As specified in clause A.3.26.2.2 |  |  |
| bwp-InactivityTimer | ms | 200 |  |  |
| T1 | s | 0.2 |  |  |
| T2 | s | 0.2 |  |  |
| T3 | s | 0.2 |  |  |

Table A.11.4.5.2.2.1-3: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1 |  | TDD |
| TDD configuration |  | Config 1 |  | TDDConf.1.1 CCA |
| BWchannel |  | Config 1 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  | Config 1 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  | Config 1 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  | Config 1 |  | DLBWP.1.3 Note 4 |
| Initial UL BWP Configuration |  | Config 1 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  | Config 1 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  | Config 1 |  | ULBWP.1.3 Note 4 |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 CCA |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 CCA |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.3 CCA |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration | Semi- static channel acces | Config 1 |  | SSB.1 CCA |
|  | Dymamic channel acces | Config 1 |  | SSB.2 CCA |
| SMTC Configuration |  | Config 1 |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |
| TRS Configuration |  | Config 1 |  | TRS.1.2 TDD |
| DL CCA probability (PCCA_DL) |  | Config 1 |  | 1 |
| UL CCA probability (PCCA_UL) |  | Config 1 |  | 1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| NocNote 2 |  | Config 1 | dBm/SCS | -101 |
| SS-RSRP Note 3 |  | Config 1 | dBm/SCS | -84 |
| Ês/Iot |  | Config 1 | dB | 17 |
| Ês/Noc |  | Config 1 | dB | 17 |
| IoNote3 |  | Config 1 | dBm/38.16 MHz | -52.86 |
| Propagation Condition |  |  |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

A.11.4.5.2.2.2	Test Requirements

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed Cell1 active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+TBWPswitchDelay+k1), (j+TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

#### A.11.4.5.3	RRC-based Active BWP Switch

##### A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA

A.11.4.5.3.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in clause 8.6.

The supported test configurations are shown in Table A.11.4.5.3.1.1-1. The test scenario comprises of one Cell (Cell 1) as given in Table A.11.4.5.3.1.1-2. Cell-specific parameters of Cell are specified in Table A.11.4.5.3.1.1-3 below.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in Cell 1.

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated bandwidth part configuration, sent from the test equipment to the UE, is completely received at the UE side in PCell’s slot # denoted i. The UE shall reconfigure its bandwidth part with the updated bandwidth part BWP-1 of final condition.

The UE shall be able to receive PDSCH on PCell from the first DL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length as defined in clause 8.6.3 and starts to report valid ACK/NACK for the PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1 on BWP-1 of final condition. The UE shall be continuously scheduled on PCell’s BWP-1 of final condition starting from the first DL slot right after slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length.

TRRCprocessingDelay and TBWPswitchDelayRRC are defined in clause 8.6.3.

The test equipment verifies the DL BWP switch time in Cell by counting the time from the time when the RRC Reconfiguration message including updated BWP configuration is sent till the time when a vaild ACK/NACK is received is received.

Table A.11.4.5.3.1.1-1: DL BWP switch supported test configurations in SA scenario

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: Void Note 2: The UE supporting SA operation with only NR band(s) with shared spectrum access is required to be tested. |  |

Table A.11.4.5.3.1.1-2: General test parameters for DL BWP switch in SA scenario

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DL CCA model |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  | As specified in clause A.3.26.2.2 |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

Table A.11.4.5.3.1.1-3: NR Cell specific test parameters for DL BWP switch in SA scenario

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1 |  | TDD |
| TDD configuration |  |  | Config 1 |  | TDDConf.1.1 CCA |
| BWchannel |  |  | Config 1 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  | 1 |
| Initial DL BWP Configuration |  |  | Config 1 |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  |  | Config 1 |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration |  | Config 1 |  | DLBWP.1.3 |
|  | Active UL BWP-1 Configuration |  | Config 1 |  | ULBWP.1.3 |
| Final Condition | Active DL BWP-1 Configuration |  | Config 1 |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration |  | Config 1 |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 CCA |
| RMSI CORESET parameters |  |  | Config 1 |  | CR.1.1 CCA |
| Dedicated CORESET parameters |  |  | Config 1 |  | CCR.1.3 CCA |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  | Semi-static channel acces | Config 1 |  | SSB.1 CCA |
|  |  | Dymamic channel acces | Config 1 |  | SSB.2 CCA |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  | Config 1 |  | TRS.1.2 TDD |
| DL CCA probability (PCCA_DL) |  |  | Config 1 |  | 1 |
| UL CCA probability (PCCA_UL) |  |  | Config 1 |  | 1 |
| Propagation Condition |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  |  | Config 1 | dBm/SCS | -101 |
| SS-RSRP Note 3 |  |  | Config 1 | dBm/SCS | -84 |
| Ês/Iot |  |  | Config 1 | dB | 17 |
| Ês/Noc |  |  | Config 1 | dB | 17 |
| IoNote3 |  |  | Config 1 | dBm/38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |  |

A.11.4.5.3.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for the Cell from the first DL slot that occurs right after the begining of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [7].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.11.4.6	Active TCI state switching delay

## A.11.5	Measurement procedure

### A.11.5.1	Intra-frequency measurements

#### A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX

##### A.11.5.1.1.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.11.5.1.1.2	Test parameters

Two cells are deployed in the test, which are PCell (Cell 1) and a neighbour cell (Cell 2) on the same carrier frequency with CCA transmitting SSBs in DBT windows according to DL CCA model. The test parameters for the two cells are given in Table A.11.5.1.1.2-1 and A.11.5.1.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

FFS: The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

- In the first test (Test 1), the UE is configured with SS-RSRP as Event A3 measurement quantity.

- In the second test (Test 2), the UE is configured with SS-RSRQ as Event A3 measurement quantity.

- In the third test (Test 3), the UE is configured with SS-SINR as Event A3 measurement quantity.

Table A.11.5.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.5.1.1.2-2: General test parameters for intra-frequency event triggered reporting without gaps

Editor’s note: Table TBD

Table A.11.5.1.1.2-3: Cell-specific test parameters for intra-frequency event-triggered reporting without gaps

Editor’s note: Table TBD

##### A.11.5.1.1.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX

##### A.11.5.1.2.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.11.5.1.2.2	Test parameters

##### A.11.5.1.2.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX

##### A.11.5.1.3.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.11.5.1.3.2	Test parameters

##### A.11.5.1.3.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX

##### A.11.5.1.4.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.11.5.1.4.2	Test parameters

##### A.11.5.1.4.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX

##### A.11.5.1.5.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.11.5.1.5.2	Test parameters

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX

##### A.11.5.1.6.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.11.5.1.6.2	Test parameters

##### A.11.5.1.6.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX

##### A.11.5.1.7.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.11.5.1.7.2	Test parameters

##### A.11.5.1.7.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX

##### A.11.5.1.8.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.11.5.1.8.2	Test parameters

##### A.11.5.1.8.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.1.9	RSSI measurement reporting on PCC

##### A.11.5.1.9.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the intra-frequency RSSI measurement reporting requirements in Section 9.2A.7.1.

##### A.11.5.1.9.2	Test parameters

In the test, the UE is configured to perform intra-frequency RSSI measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.11.5.1.9.2-1. There is one cell in the test: Cell 1 which is PCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1.

Table A.11.5.1.9.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.5.1.9.2-2: General test parameters.

Editor’s note: Table TBD

#### A.11.5.1.10	Channel occupancy measurement reporting on PCC

##### A.11.5.1.10.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the intra-frequency channel occupancy measurement reporting requirements in Section 9.2A.7.2.

##### A.11.5.1.10.2	Test parameters

In the test, the UE is configured to perform intra-frequency channel occupancy measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.11.5.1.10.2-1. There is one cell in the test: Cell 1 which is PCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1.

Table A.11.5.1.10.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.5.1.10.2-2: General test parameters.

Editor’s note: Table is TBD

#### A.11.5.1.11	RSSI measurement reporting on SCC

##### A.11.5.1.11.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the intra-frequency RSSI measurement reporting requirements in Section 9.2A.7.1.

##### A.11.5.1.11.2	Test parameters

In the test, the UE is configured to perform intra-frequency RSSI measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.11.5.1.11.2-1. There are two cells in the test: Cell 1 which is PCell operating on a carrier frequency under CCA, and Cell 2 which is SCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2.

Table A.11.5.1.11.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.5.1.11.2-2: General test parameters.

Editor’s note: Tabls TBD

#### A.11.5.1.12	Channel occupancy measurement reporting on SCC

##### A.11.5.1.12.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the intra-frequency channel occupancy measurement reporting requirements in Section 9.2A.7.2.

##### A.11.5.1.12.2	Test parameters

In the test, the UE is configured to perform intra-frequency channel occupancy measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.11.5.1.12.2-1. There are two cells in the test: Cell 1 which is PCell operating on a carrier frequency under CCA, and Cell 2 which is SCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2.

Table A.11.5.1.12.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.5.1.12.2-2: General test parameters.

Editor’s note: Table is TBD

### A.11.5.2	Inter-frequency measurements

#### A.11.5.2.1	RSSI measurement reporting

##### A.11.5.2.1.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the inter-frequency RSSI measurement reporting requirements in Section 9.3A.8.

##### A.11.5.2.1.2	Test parameters

In the test, the UE is configured to perform inter-frequency RSSI measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.11.5.2.1.2-1. There is one cell in the test: Cell 1 which is PCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1. The RSSI measurement is performed on an inter-frequency under CCA.

Table A.11.5.2.1.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.5.2.1.2-2: General test parameters.

Editor’s note: Table TBD

#### A.11.5.2.2	Channel occupancy measurement reporting

##### A.11.5.2.2.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the inter-frequency channel occupancy measurement reporting requirements in Section 9.3A.9.

##### A.11.5.2.2.2	Test parameters

In the test, the UE is configured to perform inter-frequency channel occupancy measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.11.5.2.2.2-1. There is one cell in the test: Cell 1 which is PCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1. The channel occupancy measurement is performed on an inter-frequency under CCA.

Table A.11.5.2.2.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.5.2.2.2-2: General test parameters.

Editor’s note: Table is TBD

#### A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used

##### A.11.5.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements for NR cell with CCA in clause 9.3A.4 and 9.3A.5.

In this test, there are two cells: NR cell 1 with CCA as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test parameters are given in Tables A.11.5.2.3.1-1, A.11.5.2.3.1-2 and A.11.5.2.3.1-3.

In test 1, measurement gap pattern configuration # 0 as defined in Table A.11.5.2.3.1-2 is provided for UE that does not support per-FR gap. In test 2, measurement gap pattern configuration #4 as defined in Table A.11.5.2.3.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.11.5.2.3.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.5.2.3.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cells |  | Config 1 | NR cell 1 with CCA (PCell) |  | NR cell 1 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1 | NR cell 2 with CCA |  | NR cell 2 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 9 | 9 |  |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 1.7 | 1.7 |  |

Table A.11.5.2.3.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1 | 1 |  |  | 2 |  |
| Duplex mode |  |  |  | Config 1 | TDD |  |  |  |  |
| TDD configuration |  |  |  | Config 1 | TDDConf.1.1 CCA |  |  |  |  |
| BWchannel |  |  | MHz | Config 1 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  |  | MHz | Config 1 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | Config 1 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  | Config 1 |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  | Config 1 |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  | Config 1 |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  |  | Config 1 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 | SR.1.1 CCA |  |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1 | CR.1.1 CCA |  |  |  |  |
| SSB parameters |  | Semi-static channel access Note 5,7 |  | Config 1 | SSB.1 CCA |  |  | SSB.1 CCA |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | SSB.2 CCA |  |  | SSB.2 CCA |  |
| DBT window configuration |  |  |  | Config 1 | As defined in A.3.28.1 |  |  | As defined in A.3.28.1 |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1 | SMTC.1 |  |  | SMTC.4 |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1 | PCCA_DL=0.9375 |  |  | PCCA_DL=0.9375 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1 | PCCA_UL=1 |  |  | PCCA_UL=1 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | PCCA_UL=1 |  |  | PCCA_UL=1 |  |
| LCCA_DL |  |  |  | Config 1 | 12 |  |  | 12 |  |
| WCCA_DL |  |  | ms | Config 1 | TPSS/SSS_sync_inter_cca |  |  | TPSS/SSS_sync_inter_cca |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1 | -104 |  | -104 |  |  |
| Note2 |  |  | dBm/SCS | Config 1 | -101 |  | -101 |  |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1 | -91 | -91 |  | -Infinity | -88 |
|  |  |  | dB | Config 1 | 4 | 4 |  | -Infinity | 7 |
|  |  |  | dB | Config 1 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  |  | dBm/9.36MHz | Config 1 | -58.49 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  |  | Config 1 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |

##### A.11.5.2.3.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used

##### A.11.5.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are two cells: NR cell 1 as PCell in FR1 with CCA on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test parameters are given in Tables A.11.5.2.4.1-1, A.11.5.2.4.1-2 and A.11.5.2.4.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.11.5.2.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.11.5.2.4.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.11.5.2.4.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.5.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cells |  | Config 1 | NR cell 1 with CCA (PCell) |  |  |  | NR cell 1 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1 | NR cell 2 with CCA |  |  |  | NR cell 2 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |  |  |
| CP length |  | Config 1 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |  |  |
| T2 | s | Config 1 | 2.5 | 17 | 2.5 | 17 |  |

Table A.11.5.2.4.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T3 | T4 |  | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  |  | Config 1 | 1 |  |  |  |  | 2 |  |  |  |
| Duplex mode |  |  |  | Config 1 | TDD |  |  |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 | TDDConf.1.1 CCA |  |  |  |  |  |  |  |  |
| BWchannel |  |  | MHz | Config 1 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP BW |  |  | MHz | Config 1 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | Config 1 | DLBWP.0.1 |  |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  | Config 1 |  | ULBWP.0.1 |  |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  | Config 1 |  | DLBWP.1.1 |  |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  | Config 1 |  | ULBWP.1.1 |  |  |  |  | NA |  |  |  |
| TRS configuration |  |  |  | Config 1 | TRS.1.2 TDD |  |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1 | OP.1 |  |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 | SR.1.1 CCA |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1 | CR.1.1 CCA |  |  |  |  |  |  |  |  |
| SSB parameters |  | Semi-static channel access Note 5,7 |  | Config 1 | SSB.1 CCA |  |  |  |  | SSB.1 CCA |  |  |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | SSB.2 CCA |  |  |  |  | SSB.2 CCA |  |  |  |
| DBT window configuration |  |  |  | Config 1 | As defined in A.3.28.1 |  |  |  |  | As defined in A.3.28.1 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1 | SMTC.1 |  |  |  |  | SMTC.4 |  |  |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1 | PCCA_DL=0.9375 |  |  |  |  | PCCA_DL=0.9375 |  |  |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1 | PCCA_UL=1 |  |  |  |  | PCCA_UL=1 |  |  |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | PCCA_UL=1 |  |  |  |  | PCCA_UL=1 |  |  |  |
| LCCA_DL |  |  |  | Config 1 | 5 |  |  |  |  | 5 |  |  |  |
| WCCA_DL |  |  | ms | Config 1 | TPSS/SSS_sync_inter_cca |  |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 30 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1 | 0 |  |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1 | -104 |  |  |  | -104 |  |  |  |  |
| Note2 |  |  | dBm/SCS | Config 1 | -101 |  |  |  | -101 |  |  |  |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1 | -91 |  | -91 |  |  | -Infinity |  | -88 |  |
|  |  |  | dB | Config 1 | 4 |  | 4 |  |  | -Infinity |  | 7 |  |
|  |  |  | dB | Config 1 | 4 |  | 4 |  |  | -Infinity |  | 7 |  |
| IoNote3 |  |  | dBm/9.36MHz | Config 1 | -58.49 |  | -58.49 |  |  | -63.94 |  | -56.15 |  |
| Propagation Condition |  |  |  | Config 1 | AWGN |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.11.5.2.4.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.11.5.2.4.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.11.5.2.4.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used

##### A.11.5.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are two cells: NR cell 1 as PCell in FR1 with CCA on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test parameters are given in Tables A.11.5.2.5.1-1, A.11.5.2.5.1-2 and A.11.5.2.5.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.11.5.2.5.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.11.5.2.5.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.11.5.2.5.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.5.2.5.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cells |  | Config 1 | NR cell 1 with CCA (PCell) |  | NR cell 1 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1 | NR cell 2 with CCA |  | NR cell 2 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 9 | 9 |  |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 2 | 2 |  |

Table A.11.5.2.5.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1 | 1 |  |  | 2 |  |
| Duplex mode |  |  |  | Config 1 | TDD |  |  |  |  |
| TDD configuration |  |  |  | Config 1 | TDDConf.1.1 CCA |  |  |  |  |
| BWchannel |  |  | MHz | Config 1 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  |  | MHz | Config 1 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | Config 1 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  | Config 1 |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  | Config 1 |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  | Config 1 |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  |  | Config 1 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 | SR.1.1 CCA |  |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1 | CR.1.1 CCA |  |  |  |  |
| SSB parameters |  | Semi-static channel access Note 5,7 |  | Config 1 | SSB.1 CCA |  |  | SSB.1 CCA |  |
|  |  | Semi-static channel access Note 5,7 |  | Config 1 | SSB.2 CCA |  |  | SSB.2 CCA |  |
| DBT window configuration |  |  |  | Config 1 | As defined in A.3.28.1 |  |  | As defined in A.3.28.1 |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1 | SMTC.1 |  |  | SMTC.4 |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1 | PCCA_DL=0.9375 |  |  | PCCA_DL=0.9375 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1 | PCCA_UL=1 |  |  | PCCA_UL=1 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | PCCA_UL=1 |  |  | PCCA_UL=1 |  |
| LCCA_DL |  |  |  | Config 1 | 5 |  |  | 5 |  |
| WCCA_DL |  |  | ms | Config 1 | TPSS/SSS_sync_inter_cca |  |  | TPSS/SSS_sync_inter_cca |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1 | -104 |  | -104 |  |  |
| Note2 |  |  | dBm/SCS | Config 1 | -101 |  | -101 |  |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1 | -91 | -91 |  | -Infinity | -88 |
|  |  |  | dB | Config 1 | 4 | 4 |  | -Infinity | 7 |
|  |  |  | dB | Config 1 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  |  | dBm/9.36MHz | Config 1 | -58.49 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  |  | Config 1 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |

##### A.11.5.2.5.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used

##### A.11.5.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are two cells: NR cell 1 as PCell in FR1 with CCA on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 with CCA on NR RF channel 2.  The test parameters are given in Tables A.11.5.2.6.1-1, A.11.5.2.6.1-2 and A.11.5.2.6.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.11.5.2.6.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.11.5.2.6.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.11.5.2.6.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.5.2.6.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. Channels 1 and 2 are with CCA. |
| Active cells |  | Config 1 | NR cell 1 with CCA (PCell) |  |  |  | NR cell 1 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1 | NR cell 2 with CCA |  |  |  | NR cell 2 is on NR RF channel number 2 with CCA. |
| DL CCA model |  | Config 1 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |  |  |
| CP length |  | Config 1 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |  |  |
| T2 | s | Config 1 | 3 | 20 | 3 | 20 |  |

Table A.11.5.2.6.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T3 | T4 |  | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  |  | Config 1 | 1 |  |  |  |  | 2 |  |  |  |
| Duplex mode |  |  |  | Config 1 | TDD |  |  |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 | TDDConf.1.1 CCA |  |  |  |  |  |  |  |  |
| BWchannel |  |  | MHz | Config 1 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP BW |  |  | MHz | Config 1 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1 | Config 1 | DLBWP.0.1 |  |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  | Config 1 |  | ULBWP.0.1 |  |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  | Config 1 |  | DLBWP.1.1 |  |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  | Config 1 |  | ULBWP.1.1 |  |  |  |  | NA |  |  |  |
| TRS configuration |  |  |  | Config 1 | TRS.1.2 TDD |  |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1 | OP.1 |  |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 | SR.1.1 CCA |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1 | CR.1.1 CCA |  |  |  |  |  |  |  |  |
| SSB parameters |  | Semi-static channel access Note 5,7 |  | Config 1 | SSB.1 CCA |  |  |  |  | SSB.1 CCA |  |  |  |
|  |  | Semi-static channel access Note 5,7 |  | Config 1 | SSB.2 CCA |  |  |  |  | SSB.2 CCA |  |  |  |
| DBT window configuration |  |  |  | Config 1 | As defined in A.3.28.1 |  |  |  |  | As defined in A.3.28.1 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1 | SMTC.1 |  |  |  |  | SMTC.4 |  |  |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1 | PCCA_DL=0.9375 |  |  |  |  | PCCA_DL=0.9375 |  |  |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1 | PCCA_UL=1 |  |  |  |  | PCCA_UL=1 |  |  |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1 | PCCA_UL=1 |  |  |  |  | PCCA_UL=1 |  |  |  |
| LCCA_DL |  |  |  | Config 1 | 2 |  |  |  |  | 2 |  |  |  |
| WCCA_DL |  |  | ms | Config 1 | TPSS/SSS_sync_inter_cca |  |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 30 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1 | 0 |  |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1 | -104 |  |  |  | -104 |  |  |  |  |
| Note2 |  |  | dBm/SCS | Config 1 | -101 |  |  |  | -101 |  |  |  |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1 | -91 |  | -91 |  |  | -Infinity |  | -88 |  |
|  |  |  | dB | Config 1 | 4 |  | 4 |  |  | -Infinity |  | 7 |  |
|  |  |  | dB | Config 1 | 4 |  | 4 |  |  | -Infinity |  | 7 |  |
| IoNote3 |  |  | dBm/9.36MHz | Config 1 | -58.49 |  | -58.49 |  |  | -63.94 |  | -56.15 |  |
| Propagation Condition |  |  |  | Config 1 | AWGN |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.11.5.2.6.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.11.5.2.6.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.11.5.2.6.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%. In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of

#### A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used

##### A.11.5.2.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements for NR cell with CCA in clause 9.3A.4 and 9.3A.5.

In this test, there are two cells: NR cell 1 with CCA as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.11.5.2.7.1-1, A.11.5.2.7.1-2 and A.11.5.2.7.1-3.

In test 1, measurement gap pattern configuration # 0 as defined in Table A.11.5.2.7.1-2 is provided for UE that does not support per-FR gap. In test 2, measurement gap pattern configuration #4 as defined in Table A.11.5.2.7.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.11.5.2.7.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.5.2.7.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies are used. NR channel 1 is with CCA. |
| Active cell |  | Config 1,2,3 | NR cell 1 (PCell) |  | NR cell 1 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 1,2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.7 | 1.7 |  |

Table A.11.5.2.7.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  |  | Config 1 | TDD |  | FDD |  |
|  |  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 1 | TDDConf.1.1 CCA |  | Not Applicable |  |
|  |  |  |  | Config 2 | TDDConf.1.1 CCA |  | TDDConf.1.1 |  |
|  |  |  |  | Config 3 | TDDConf.1.1 CCA |  | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | Config 1,2 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  |  | MHz | Config 1,2 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP configuration | Initial DL BWP |  |  | Config 1,2,3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  |  | Config 1,2,3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  |  | Config 1,2,3 | SR.1.1 CCA |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1,2,3 | CR.1.1 CCA |  |  |  |
| SSB parameters |  | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  | SSB.1 FR1 |  |
|  |  |  |  | Config 3 | SSB.1 CCA |  | SSB.2 FR1 |  |
|  |  | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  |  | Config 3 | SSB.2 CCA |  | SSB.2 FR1 |  |
| DBT window configuration |  |  |  | Config 1,2,3 | As defined in A.3.28.1 |  | Not applicable |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1,2,3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1,2 | 30 |  | 15 |  |
|  |  |  |  | Config 3 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | PCCA_DL=0.9375 |  | NA |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | NA |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | PCCA_UL=1 |  | NA |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | PCCA_UL=1 |  | NA |  |
| LCCA_DL |  |  |  | Config 1,2,3 | 12 |  | 12 |  |
| WCCA_DL |  |  | ms | Config 1,2,3 | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1,2,3 | -104 |  | -98 |  |
| Note2 |  |  | dBm/SCS | Config 1,2 | -101 |  | -98 |  |
|  |  |  |  | Config 3 | -101 |  | -95 |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1,2 | -91 | -91 | -Infinity | -91 |
|  |  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  |  | dBm/9.36MHz | Config 1,2 | -58.49 | -58.49 | -70.05 | -62.26 |
|  |  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |

##### A.11.5.2.7.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used

##### A.11.5.2.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are two cells: NR cell 1 with CCA as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.11.5.2.8.1-1, A.11.5.2.8.1-2 and A.11.5.2.8.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.11.5.2.8.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.11.5.2.8.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.11.5.2.8.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.5.2.8.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. NR channel 1 is with CCA. |
| Active cell |  | Config 1,2,3 | NR cell 1 (PCell) |  |  |  | NR cell 1 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 1,2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 2.5 | 17 | 2.5 | 17 |  |

Table A.11.5.2.8.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  |  | Config 1 | TDD |  |  |  | FDD |  |  |  |
|  |  |  |  | Config 2,3 | TDD |  |  |  | TDD |  |  |  |
| TDD configuration |  |  |  | Config 1 | TDDConf.1.1 CCA |  |  |  | Not Applicable |  |  |  |
|  |  |  |  | Config 2 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 |  |  |  |
|  |  |  |  | Config 3 | TDDConf.1.1 CCA |  |  |  | TDDConf.2.1 |  |  |  |
| BWchannel |  |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  |  | Config 1,2,3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  |  | Config 1,2,3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1,2,3 | SR.1.1 CCA |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1,2,3 | CR.1.1 CCA |  |  |  |  |  |  |  |
| SSB parameters |  | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  |  | Config 3 | SSB.1 CCA |  |  |  | SSB.2 FR1 |  |  |  |
|  |  | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  |  | Config 3 | SSB.2 CCA |  |  |  | SSB.2 FR1 |  |  |  |
| DBT window configuration |  |  |  | Config 1,2,3 | As defined in A.3.28.1 |  |  |  | Not applicable |  |  |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1,2,3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1,2 | 30 |  |  |  | 15 |  |  |  |
|  |  |  |  | Config 3 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | PCCA_DL=0.9375 |  |  |  | NA |  |  |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | NA |  |  |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | PCCA_UL=1 |  |  |  | NA |  |  |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | PCCA_UL=1 |  |  |  | NA |  |  |  |
| LCCA_DL |  |  |  | Config 1,2,3 | 5 |  |  |  | 5 |  |  |  |
| WCCA_DL |  |  | ms | Config 1,2,3 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1,2,3 | -104 |  |  |  | -98 |  |  |  |
| Note2 |  |  | dBm/SCS | Config 1,2 | -101 |  |  |  | -98 |  |  |  |
|  |  |  |  | Config 3 | -101 |  |  |  | -95 |  |  |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1,2 | -91 |  | -91 |  | -Infinity |  | -91 |  |
|  |  |  |  | Config 3 | -91 |  | -91 |  | -Infinity |  | -88 |  |
|  |  |  | dB | Config 1,2,3 | 4 |  | 4 |  | -Infinity |  | 7 |  |
|  |  |  | dB | Config 1,2,3 | 4 |  | 4 |  | -Infinity |  | 7 |  |
| IoNote3 |  |  | dBm/9.36MHz | Config 1,2 | -58.49 |  | -58.49 |  | -70.05 |  | -62.26 |  |
|  |  |  | dBm/38.16MHz | Config 3 | -58.49 |  | -58.49 |  | -63.94 |  | -56.15 |  |
| Propagation Condition |  |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.11.5.2.8.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.11.5.2.8.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.11.5.2.8.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used

##### A.11.5.2.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are two cells: NR cell 1 with CCA as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.11.5.2.9.1-1, A.11.5.2.9.1-2 and A.11.5.2.9.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.11.5.2.9.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.11.5.2.9.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.11.5.2.9.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.5.2.9.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies are used. NR channel 1 is with CCA. |
| Active cell |  | Config 1,2,3 | NR cell 1 (PCell) |  | NR cell 1 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3 | As specified in clauseA.3.26.2.1 |  |  |
| UL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 1,2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 2 | 2 |  |

Table A.11.5.2.9.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  |  | Config 1 | TDD |  | FDD |  |
|  |  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 1 | TDDConf.1.1 CCA |  | Not Applicable |  |
|  |  |  |  | Config 2 | TDDConf.1.1 CCA |  | TDDConf.1.1 |  |
|  |  |  |  | Config 3 | TDDConf.1.1 CCA |  | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | Config 1,2 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  |  | MHz | Config 1,2 | 40: NRB,c = 106 |  | 10: NRB,c = 52 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP configuration | Initial DL BWP |  |  | Config 1,2,3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  |  | Config 1,2,3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  |  | Config 1,2,3 | SR.1.1 CCA |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1,2,3 | CR.1.1 CCA |  |  |  |
| SSB parameters |  | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  | SSB.1 FR1 |  |
|  |  |  |  | Config 3 | SSB.1 CCA |  | SSB.2 FR1 |  |
|  |  | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.2 CCA |  | SSB.1 FR1 |  |
|  |  |  |  | Config 3 | SSB.2 CCA |  | SSB.2 FR1 |  |
| DBT window configuration |  |  |  | Config 1,2,3 | As defined in A.3.28.1 |  | Not applicable |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1,2,3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1,2 | 30 |  | 15 |  |
|  |  |  |  | Config 3 | 30 |  | 30 |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | PCCA_DL=0.9375 |  | NA |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | NA |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | PCCA_UL=1 |  | NA |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | PCCA_UL=1 |  | NA |  |
| LCCA_DL |  |  |  | Config 1,2,3 | 5 |  | 5 |  |
| WCCA_DL |  |  | ms | Config 1,2,3 | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1,2,3 | -104 |  | -98 |  |
| Note2 |  |  | dBm/SCS | Config 1,2 | -101 |  | -98 |  |
|  |  |  |  | Config 3 | -101 |  | -95 |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1,2 | -91 | -91 | -Infinity | -91 |
|  |  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
|  |  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  |  | dBm/9.36MHz | Config 1,2 | -58.49 | -58.49 | -70.05 | -62.26 |
|  |  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |

##### A.11.5.2.9.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used

##### A.11.5.2.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are two cells: NR cell 1 with CCA as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters are given in Tables A.11.5.2.10.1-1, A.11.5.2.10.1-2 and A.11.5.2.10.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.11.5.2.10.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.11.5.2.10.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.11.5.2.10.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.11.5.2.10.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used. NR channel 1 is with CCA. |
| Active cell |  | Config 1,2,3 | NR cell 1 (PCell) |  |  |  | NR cell 1 is on NR RF channel number 1 with CCA. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| DL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 1,2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 3 | 20 | 3 | 20 |  |

Table A.11.5.2.10.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  |  | Config 1 | TDD |  |  |  | FDD |  |  |  |
|  |  |  |  | Config 2,3 | TDD |  |  |  | TDD |  |  |  |
| TDD configuration |  |  |  | Config 1 | TDDConf.1.1 CCA |  |  |  | Not Applicable |  |  |  |
|  |  |  |  | Config 2 | TDDConf.1.1 CCA |  |  |  | TDDConf.1.1 |  |  |  |
|  |  |  |  | Config 3 | TDDConf.1.1 CCA |  |  |  | TDDConf.2.1 |  |  |  |
| BWchannel |  |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  |  | MHz | Config 1,2 | 40: NRB,c = 106 |  |  |  | 10: NRB,c = 52 |  |  |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  |  | Config 1,2,3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  |  | Config 1,2,3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1,2,3 | SR.1.1 CCA |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1,2,3 | CR.1.1 CCA |  |  |  |  |  |  |  |
| SSB parameters |  | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.1 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  |  | Config 3 | SSB.1 CCA |  |  |  | SSB.2 FR1 |  |  |  |
|  |  | Semi-static channel access Note 5,7 |  | Config 1,2 | SSB.2 CCA |  |  |  | SSB.1 FR1 |  |  |  |
|  |  |  |  | Config 3 | SSB.2 CCA |  |  |  | SSB.2 FR1 |  |  |  |
| DBT window configuration |  |  |  | Config 1,2,3 | As defined in A.3.28.1 |  |  |  | Not applicable |  |  |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1,2,3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1,2 | 30 |  |  |  | 15 |  |  |  |
|  |  |  |  | Config 3 | 30 |  |  |  | 30 |  |  |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | PCCA_DL=0.9375 |  |  |  | NA |  |  |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |  | NA |  |  |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | PCCA_UL=1 |  |  |  | NA |  |  |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | PCCA_UL=1 |  |  |  | NA |  |  |  |
| LCCA_DL |  |  |  | Config 1,2,3 | 2 |  |  |  | 2 |  |  |  |
| WCCA_DL |  |  | ms | Config 1,2,3 | TPSS/SSS_sync_inter_cca |  |  |  | TPSS/SSS_sync_inter_cca |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1,2,3 | -104 |  |  |  | -98 |  |  |  |
| Note2 |  |  | dBm/SCS | Config 1,2 | -101 |  |  |  | -98 |  |  |  |
|  |  |  |  | Config 3 | -101 |  |  |  | -95 |  |  |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1,2 | -91 |  | -91 |  | -Infinity |  | -91 |  |
|  |  |  |  | Config 3 | -91 |  | -91 |  | -Infinity |  | -88 |  |
|  |  |  | dB | Config 1,2,3 | 4 |  | 4 |  | -Infinity |  | 7 |  |
|  |  |  | dB | Config 1,2,3 | 4 |  | 4 |  | -Infinity |  | 7 |  |
| IoNote3 |  |  | dBm/9.36MHz | Config 1,2 | -58.49 |  | -58.49 |  | -70.05 |  | -62.26 |  |
|  |  |  | dBm/38.16MHz | Config 3 | -58.49 |  | -58.49 |  | -63.94 |  | -56.15 |  |
| Propagation Condition |  |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.11.5.2.10.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.11.5.2.10.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.11.5.2.10.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.11.5.3	Inter-RAT E-UTRAN measurements

### A.11.5.4	L1-RSRP measurements for beam reporting

#### A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used

##### A.11.5.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.11.5.4.1.1-1.

Table A.11.5.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.11.5.4.1.2	Test parameters

There is one cell in the test, the FR1 PCell (Cell 1). Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The test parameters for the Cell 1 are given in Table A.11.5.4.1.2-1 and Table A.11.5.4.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.11.5.4.1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| DL CCA model | 1 |  | As specifieed in A.3.20.2.1 |
| UL CCA model | 1 |  | As specified in A.3.20.2.2 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.1.1 CCA |
| BWchannel | 1 | MHz | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 CCA |
| SSB configuration | 1 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration | 1 |  | DBT.1 |
| TRS Configuration | 1 |  | TRS.1.2 TDD |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | periodic |
| reportQuantity | 1 |  | ssb-Index-RSRP |
| Number of reported RS | 1 |  | 2 |
| L1-RSRP reporting period | 1 | slot | 80 |
| T1 | 1 | s | 5 |
| T2 | 1 | s | 1 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |

Table A.11.5.4.1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.11.5.4.1.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used

##### A.11.5.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.11.5.4.2.1-1.

Table A.11.5.4.2.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.11.5.4.2.2	Test parameters

There is one cell in the test, the FR1 PCell (Cell 1). Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The test parameters for the Cell 1 are given in Table A.11.5.4.2.2-1 and Table A.11.5.4.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.11.5.4.2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| DL CCA model | 1 |  | As specifieed in A.3.20.2.1 |
| UL CCA model | 1 |  | As specified in A.3.20.2.2 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.1.1 CCA |
| BWchannel | 1 | MHz | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 CCA |
| SSB configuration | 1 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration | 1 |  | DBT.1 |
| TRS Configuration | 1 |  | TRS.1.2 TDD |
| DRX configuration | 1 |  | DRX.3 |
| reportConfigType | 1 |  | periodic |
| reportQuantity | 1 |  | ssb-Index-RSRP |
| Number of reported RS | 1 |  | 2 |
| L1-RSRP reporting period | 1 | slot | 80 |
| T1 | 1 | s | 5 |
| T2 | 1 | s | 1 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |

Table A.11.5.4.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.11.5.4.2.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used

##### A.11.5.4.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.11.5.4.3.1-1.

Table A.11.5.4.3.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.11.5.4.3.2	Test parameters

There are two cells in the test, the FR1 PCell (Cell 1) and FR1 SCell (Cell 2). Both Cell 1 and Cell 2 operate on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The test parameters for the Cell 1 are given in Table A.11.5.4.3.2-1 and Table A.11.5.4.3.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.11.5.4.3.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Active PCell | 1 |  | Cell 1 |
| Active SCell | 1 |  | Cell 2 |
| RF Channel Number | 1 |  | 1: Cell 1 2: Cell 2 |
| DL CCA model | 1 |  | As specifieed in A.3.20.2.1 |
| UL CCA model | 1 |  | As specified in A.3.20.2.2 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.1.1 CCA |
| BWchannel | 1 | MHz | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 CCA |
| SSB configuration | 1 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration | 1 |  | DBT.1 |
| TRS Configuration | 1 |  | TRS.1.2 TDD |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | periodic |
| reportQuantity | 1 |  | ssb-Index-RSRP |
| Number of reported RS | 1 |  | 2 |
| L1-RSRP reporting period | 1 | slot | 80 |
| T1 | 1 | s | 5 |
| T2 | 1 | s | 1 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |

Table A.11.5.4.3.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.11.5.4.3.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

The UE shall send L1-RSRP report of both SSB0 and SSB1 in Cell 2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used

##### A.11.5.4.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.11.5.4.4.1-1.

Table A.11.5.4.4.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | With CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.11.5.4.4.2	Test parameters

There are two cells in the test, the FR1 PCell (Cell 1) and FR1 SCell (Cell 2). Both Cell 1 and Cell 2 operate on a carrier frequency with CCA and transmits SSBs in DBT windows according to DL CCA model. The test parameters for the Cell 1 are given in Table A.11.5.4.4.2-1 and Table A.11.5.4.4.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.11.5.4.4.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Active PCell | 1 |  | Cell 1 |
| Active SCell | 1 |  | Cell 2 |
| RF Channel Number | 1 |  | 1: Cell 1 2: Cell 2 |
| DL CCA model | 1 |  | As specifieed in A.3.20.2.1 |
| UL CCA model | 1 |  | As specified in A.3.20.2.2 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.1.1 CCA |
| BWchannel | 1 | MHz | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 CCA |
| SSB configuration | 1 |  | SSB.3 CCA for semi-static channel access SSB.4 CCA for dynamic channel access |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration | 1 |  | DBT.1 |
| TRS Configuration | 1 |  | TRS.1.2 TDD |
| DRX configuration | 1 |  | DRX.3 |
| reportConfigType | 1 |  | periodic |
| reportQuantity | 1 |  | ssb-Index-RSRP |
| Number of reported RS | 1 |  | 2 |
| L1-RSRP reporting period | 1 | slot | 80 |
| T1 | 1 | s | 5 |
| T2 | 1 | s | 1 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |

Table A.11.5.4.4.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.11.5.4.4.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

The UE shall send L1-RSRP report of both SSB0 and SSB1 in Cell 2.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.11.6	Measurement performance

### A.11.6.1	SS-RSRP

#### A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA

##### A.11.6.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy on the carrier frequency with CCA is within the specified limits. This test will verify the requirements in clauses 10.1.36.1.1 and 10.1.36.1.2 for intra-frequency measurements under CCA.

##### A.11.6.1.1.2	Test parameters

In this set of test cases all cells are on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model. Supported test configurations are shown in table A.11.6.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in A.11.6.1.1.2-2. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell.

Table A.11.6.1.1.2-1: SS-RSRP  Intra frequency SS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.6.1.1.2-2: SS-RSRP Intra frequency test parameters

| Parameter |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |  |  | Cell 1 |  |  | Cell 2 |  |  |
| Cell ID |  |  |  |  |  |  | 489 | 0 |  | 489 |  | 0 |  |  |  | 489 |  | 0 |  |  |  |
| SSB ARFCN |  |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |  |  | freq1 |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  |  | TDDConf.1.1 CCA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  |  | Config 1 |  | MHz | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  |  | Config 1 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL CCA model |  |  |  |  |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UL CCA model |  |  |  |  |  |  | As specified in clause A.3.20.2.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCA_DL |  |  |  |  |  |  | [TBD] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCA_UL |  |  |  |  |  |  | [TBD] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  | Config 1 |  | TRS.1.2 TDD |  | NA |  | TRS.1.2 TDD |  |  | NA | TRS.1.2 TDD |  |  |  |  |  | NA |
| DRX Cycle |  |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  |  | SR.1.1 CCA |  | - | SR.1.1 CCA |  | - |  |  |  | SR.1.1 CCA |  |  |  |  | - |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  |  | CR.1.1 CCA |  | - | CR.1.1 CCA |  | - |  |  |  | CR.1.1 CCA |  |  |  |  | - |
| Control channel RMC |  |  |  | Config 1 |  |  | CR.1.1 CCA |  | - | CR.1.1 CCA |  | - |  |  |  | CR.1.1 CCA |  |  |  |  | - |
| SSB configuration for semi-static channel access |  |  |  | Config 1 |  |  | SSB.1 CCA |  | SSB.1 CCA | SSB.1 CCA |  | SSB.1 CCA |  |  |  | SSB.1 CCA |  |  |  |  | SSB.1 CCA |
| SSB configuration for dynamic channel access |  |  |  | Config 1 |  |  | SSB.2 CCA |  | SSB.2 CCA | SSB.2 CCA |  | SSB.2 CCA |  |  |  | SSB.2 CCA |  |  |  |  | SSB.2 CCA |
| DBT window configuration |  |  |  | Config 1,2,3 |  |  | DBT.1 |  | DBT.1 | DBT.1 |  | DBT.1 |  |  |  | DBT.1 |  |  |  |  | DBT.1 |
| Time offset with Cell 1 |  | Config 1 |  |  |  | s | - |  | 3 | - |  | 3 |  |  |  | - |  |  |  | 3 |  |
| SMTC configuration |  | Config 1 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1 |  | kHz | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  | dB | 0 |  | 0 | 0 |  |  | 0 |  |  |  | 0 |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1 |  |  |  | NR_TDD_FR1_I |  | Not applicableNote 5 |  |  | -94 |  |  |  |  |  |  | TBD |  |  |  |  |
| Note2 | Config 1 | NR_TDD_FR1_I |  |  |  | dBm/SCS | Not applicableNote 5 |  |  | -91 |  |  |  |  |  |  | TBD |  |  |  |  |
| Note6 |  |  |  |  |  | dB | 2.46 |  | -5.97 | 2.46 |  |  | -5.97 |  |  |  | TBD |  |  |  | TBD |
| Note6 |  |  |  |  |  | dB | 6 |  | 1 | 6 |  |  | 1 |  |  |  | TBD |  |  |  | TBD |
| SS-RSRPNote3,6 | Config 1 |  | NR_TDD_FR1_I |  |  | dBm/SCS | Not applicableNote 5 |  | Not applicableNote 5 | -85 |  |  | -90 |  |  |  | TBD |  |  |  | TBD |
| IoNote3 | Config 1 |  |  | NR_TDD_FR1_I |  | dBm/ 38.16MHz | Not applicableNote 5- |  |  | -51.99 |  |  |  |  |  |  | TBD |  |  |  |  |
| Propagation condition |  |  |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Subtest 1 is not used when testing with 30kHz SSB SCS. NOTE 6: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.11.6.1.1.3	Test Requirements

The SS-RSRP measurement accuracy for cell 1 and cell 2 shall fulfil absolute requirement in clause 10.1.36.1.1 and relative requirement in clause 10.1.36.1.2.

#### A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA

##### A.11.6.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy on the carrier frequency with CCA is within the specified limits. This test will verify the requirements in clauses 10.1.36.1.1 and 10.1.36.1.2 for intra-frequency measurements under CCA.

##### A.11.6.1.2.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1) on the carrier frequency with CCA, and two cells on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model: SCell (Cell 2) and a neighbour cell (Cell 3).  Supported test configurations are shown in table A.11.6.1.2.2-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in A.11.6.1.2.2-2.

Table A.11.6.1.2.2-1: SS-RSRP  Intra frequency SS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.11.6.1.2.2-2: SS-RSRP Intra frequency test parameters

| Parameter |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Cell 2 | Cell 3 |  | Cell 2 |  | Cell 3 |  |  |  | Cell 2 |  |  | Cell 3 |  |  |
| Cell ID |  |  |  |  |  |  | 489 | 0 |  | 489 |  | 0 |  |  |  | 489 |  | 0 |  |  |  |
| SSB ARFCN |  |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |  |  | freq1 |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1 |  |  | TDDConf.1.1 CCA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  |  | Config 1 |  | MHz | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  |  | Config 1 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL CCA model |  |  |  |  |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UL CCA model |  |  |  |  |  |  | As specified in clause A.3.20.2.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCA_DL |  |  |  |  |  |  | [TBD] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCA_UL |  |  |  |  |  |  | [TBD] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  | Config 1 |  | TRS.1.2 TDD |  | NA |  | TRS.1.2 TDD |  |  | NA | TRS.1.2 TDD |  |  |  |  |  | NA |
| DRX Cycle |  |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 |  |  | SR.1.1 CCA |  | - | SR.1.1 CCA |  | - |  |  |  | SR.1.1 CCA |  |  |  |  | - |
| RMSI CORESET Reference Channel |  |  |  | Config 1 |  |  | CR.1.1 CCA |  | - | CR.1.1 CCA |  | - |  |  |  | CR.1.1 CCA |  |  |  |  | - |
| Control channel RMC |  |  |  | Config 1 |  |  | CR.1.1 CCA |  | - | CR.1.1 CCA |  | - |  |  |  | CR.1.1 CCA |  |  |  |  | - |
| SSB configuration for semi-static channel access |  |  |  | Config 1 |  |  | SSB.1 CCA |  | SSB.1 CCA | SSB.1 CCA |  | SSB.1 CCA |  |  |  | SSB.1 CCA |  |  |  |  | SSB.1 CCA |
| SSB configuration for dynamic channel access |  |  |  | Config 1 |  |  | SSB.2 CCA |  | SSB.2 CCA | SSB.2 CCA |  | SSB.2 CCA |  |  |  | SSB.2 CCA |  |  |  |  | SSB.2 CCA |
| DBT window configuration |  |  |  | Config 1,2,3 |  |  | DBT.1 |  | DBT.1 | DBT.1 |  | DBT.1 |  |  |  | DBT.1 |  |  |  |  | DBT.1 |
| Time offset with Cell 1 |  | Config 1 |  |  |  | s | - |  | 3 | - |  | 3 |  |  |  | - |  |  |  | 3 |  |
| SMTC configuration |  | Config 1 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1 |  | kHz | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  | dB | 0 |  | 0 | 0 |  |  | 0 |  |  |  | 0 |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1 |  |  |  | NR_TDD_FR1_I |  | Not applicableNote 5 |  |  | -94 |  |  |  |  |  |  | TBD |  |  |  |  |
| Note2 | Config 1 | NR_TDD_FR1_I |  |  |  | dBm/SCS | Not applicableNote 5 |  |  | -91 |  |  |  |  |  |  | TBD |  |  |  |  |
| Note6 |  |  |  |  |  | dB | 2.46 |  | -5.97 | 2.46 |  |  | -5.97 |  |  |  | TBD |  |  |  | TBD |
| Note6 |  |  |  |  |  | dB | 6 |  | 1 | 6 |  |  | 1 |  |  |  | TBD |  |  |  | TBD |
| SS-RSRPNote3 | Config 1 |  | NR_TDD_FR1_I |  |  | dBm/SCS | Not applicableNote 5 |  | Not applicableNote 5 | -85 |  |  | -90 |  |  |  | TBD |  |  |  | TBD |
| IoNote3 | Config 1 |  |  | NR_TDD_FR1_I |  | dBm/ 38.16MHz | Not applicableNote 5- |  |  | -51.99 |  |  |  |  |  |  | TBD |  |  |  |  |
| Propagation condition |  |  |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Subtest 1 is not used when testing with 30kHz SSB SCS. NOTE 6: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.11.6.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for cell 2 and cell 3 shall fulfil absolute requirement in clause 10.1.36.1.1 and relative requirement in clause 10.1.36.1.2.

### A.11.6.2	SS-RSRQ

### A.11.6.3	SS-SINR

### A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell

#### A.11.6.4.1	SSB based L1-RSRP measurement

##### A.11.6.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.11.6.4.1.1-1.

Table A.11.6.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.11.6.4.1.2	Test parameters

In this set of test cases there one cell in the test, PCell under CCA (Cell 1). Cell 1 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 1. The test parameters for the Cell 1 are given in Table A.11.6.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.11.6.4.1.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.11.6.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1 |  | freq1 | freq1 |
| DL CCA model |  | 1 |  | As specifieed in A.3.20.2.1 | As specifieed in A.3.20.2.1 |
| UL CCA model |  | 1 |  | As specified in A.3.20.2.2 | As specified in A.3.20.2.2 |
| Duplex mode |  | 1 |  | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.1.1 CCA | TDDConf.1.1 CCA |
| BWchannel |  | 1 | MHz | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 CCA | CCR.1.1 CCA |
| SSB configuration Semi-static channel access |  | 1 |  | SSB.3 CCA | SSB.3 CCA |
| SSB configuration for Dynamic channel access |  | 1 |  | SSB.4 CCA | SSB.4 CCA |
| OCNG Patterns |  | 1 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| DBT Window Configuration |  | 1 |  | DBT.1 | DBT.1 |
| reportConfigType |  | 1 |  | periodic | periodic |
| reportQuantity |  | 1 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS |  | 1 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_TDD_FR1_I | 1 | dBm/15kHz | -94.65 | [-113] |
| Note2 | NR_TDD_FR1_I | 1 | dBm/SCS | -91.65 | [-110] |
|  |  | 1 | dB | 10 | -3 |
| SS-RSRPNote3 | NR_TDD_FR1_I | 1 | dBm/SCS | -81.65 | [-113] |
| IoNote3 | NR_TDD_FR1_I | 1 | dBm/ 38.16MHz | -50.19 | [-77.19] |
|  |  | 1 | dB | 10 | -3 |
| Propagation condition |  | 1 |  | AWGN | AWGN |
| Antenna configuration |  | 1 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

##### A.11.6.4.1.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 1 shall fulfil the requirements in clauses 10.1.33.1.

### A.11.6.5	RSSI

### A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA

#### A.11.6.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the RSSI measurement accuracy is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in Section 10.1.34.1.

#### A.11.6.5.1.2	Test parameters

In all test cases, Cell 1 is the PCell with CCA. RSSI is measured on channel number 1. Supported test configurations are shown in table A.11.6.5.1.2-1. The accuracy of RSSI intra-frequency measurements is tested by using the parameters in A.11.6.5.1.2-2 and A.11.6.5.1.2-3.

Table A.11.6.5.1.2-1: Intra frequency RSSI supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.6.5.1.2-2: RSSI Intra frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |
| --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| BWchannel |  |  | MHz | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1 |  | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1 |  | SSB.2 CCA |
| PCCA_DL |  |  |  | TBD |
| PCCA_UL |  |  |  | TBD |
| DL CCA model |  |  |  | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |
| Channel access bandwidth |  |  | MHz | 20 |
| DRX Cycle configuration |  |  | ms | Not Applicable |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -87 |
| Propagation condition |  |  | - | AWGN |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.6.5.1.2-3: RSSI RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.11.6.5.1.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 10.1.34.1. The nominal RSSI used to evaluate the requirement shall be based on Io in slots corresponding to RSSI measurement time configuration (RMTC).

### A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA

#### A.11.6.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the RSSI measurement accuracy is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in Section 10.1.34.1.

#### A.11.6.5.2.2	Test parameters

In all test cases, Cell 1 which is PCell operating on a carrier frequency under CCA, and Cell 2 which is SCell operating on a carrier frequency under CCA. RSSI is measured on channel number 2. Supported test configurations are shown in table A.11.6.5.2.2-1. The accuracy of RSSI intra-frequency measurements is tested by using the parameters in A.11.6.5.2.2-2 and A.11.6.5.2.2-3.

Table A.11.6.5.2.2-1: Intra frequency RSSI supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.6.5.2.2-2: RSSI Intra frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2 For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.11.6.5.2.2-3: RSSI RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.11.6.5.2.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 10.1.34.1. The nominal RSSI used to evaluate the requirement shall be based on Io in slots corresponding to RSSI measurement time configuration (RMTC).

### A.11.6.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA

#### A.11.6.5.3.1	Test Purpose and Environment

The purpose of this test is to verify that the RSSI measurement accuracy is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in Section 10.1.34.2.

#### A.11.6.5.3.2	Test parameters

In all test cases, Cell 1 which is PCell operating on a carrier frequency under CCA, and Cell 2 which is neighbor cell operating on a carrier frequency under CCA. RSSI is measured on channel number 2. Supported test configurations are shown in table A.11.6.5.3.2-1. The accuracy of RSSI intra-frequency measurements is tested by using the parameters in A.11.6.5.3.2-2 and A.11.6.5.3.2-3.

Table A.11.6.5.3.2-1: Inter frequency RSSI supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.6.5.3.2-2: RSSI Inter frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.11.6.5.3.2-3: RSSI RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.11.6.5.3.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 10.1.34.2. The nominal RSSI used to evaluate the requirement shall be based on Io in slots corresponding to RSSI measurement time configuration (RMTC).

### A.11.6.6	Channel occupancy

### A.11.6.6.1 	Intra-frequency channel occupancy measurement accuracy on PCC with CCA

#### A.11.6.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy measurement accuracy is within the specified limits. This test will partially verify the channel occupancy measurement accuracy requirements in Section 10.1.35.1.

#### A.11.6.6.1.2	Test parameters

In all test cases, Cell 1 is the PCell with CCA. channel occupancy is measured on channel number 1. Supported test configurations are shown in table A.11.6.6.1.2-1. The accuracy of channel occupancy intra-frequency measurements is tested by using the parameters in A.11.6.6.1.2-2 and A.11.6.6.1.2-3.

Table A.11.6.6.1.2-1: Intra frequency CO supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.6.6.1.2-2: CO Intra frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |
| --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| BWchannel |  |  | MHz | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1 |  | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1 |  | SSB.2 CCA |
| PCCA_DL |  |  |  | TBD |
| PCCA_UL |  |  |  | TBD |
| DL CCA model |  |  |  | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |
| Channel access bandwidth |  |  | MHz | 20 |
| DRX Cycle configuration |  |  | ms | Not Applicable |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -87 |
| Propagation condition |  |  | - | AWGN |
| channelOccupancyThreshold |  |  | dBm | -83 |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.6.6.1.2-3: CO RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.11.6.6.1.3	Test Requirements

The nominal reported channelOccupancy shall be TBD. At least 90% of channel occupancy reports made by the UE shall indicate this value.

### A.11.6.6.2 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA

#### A.11.6.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy measurement accuracy is within the specified limits. This test will partially verify the channel occupancy measurement accuracy requirements in Section 10.1.35.1.

#### A.11.6.6.2.2	Test parameters

In all test cases, Cell 1 which is PCell operating on a carrier frequency under CCA, and Cell 2 which is SCell operating on a carrier frequency under CCA. Channel occupancy is measured on channel number 2. Supported test configurations are shown in table A.11.6.6.2.2-1. The accuracy of channel occupancy intra-frequency measurements is tested by using the parameters in A.11.6.6.2.2-2 and A.11.6.6.2.2-3.

Table A.11.6.6.2.2-1: Intra frequency CO supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.6.6.2.2-2: CO Intra frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| channelOccupancyThreshold |  |  | dBm | -83 |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.11.6.6.2.2-3: CO RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.11.6.6.2.3	Test Requirements

The nominal reported channelOccupancy shall be TBD. At least 90% of channel occupancy reports made by the UE shall indicate this value.

### A.11.6.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA

#### A.11.6.6.3.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy measurement accuracy is within the specified limits. This test will partially verify the channel occupancy measurement accuracy requirements in Section 10.1.34.2.

#### A.11.6.6.3.2	Test parameters

In all test cases, Cell 1 which is PCell operating on a carrier frequency under CCA, and Cell 2 which is neighbor cell operating on a carrier frequency under CCA. Channel occupancy is measured on channel number 2. Supported test configurations are shown in table A.11.6.6.3.2-1. The accuracy of channel occupancy intra-frequency measurements is tested by using the parameters in A.11.6.6.3.2-2 and A.11.6.6.3.2-3.

Table A.11.6.6.3.2-1: Inter frequency CO supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, bandwidth 40 MHz |

Table A.11.6.6.3.2-2: CO Inter frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| channelOccupancyThreshold |  |  | dBm | -83 |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.11.6.6.3.2-3: CO RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.11.6.6.3.3	Test Requirements

The nominal reported channelOccupancy shall be TBD. At least 90% of channel occupancy reports made by the UE shall indicate this value.

### A.11.6.7	E-UTRAN RSRP

### A.11.6.8	E-UTRAN RSRQ

### A.11.6.9	E-UTRAN SINR

# A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA

## A.12.1	RRC_IDLE state mobility

### A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA

#### A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA

##### A.12.1.1.1.1	Test Purpose and Environment

This test is to verify the requirement for the E-UTRAN to NR inter-RAT cell subject to CCA reselection requirements specified in clause 4.2.2.5.7 in TS 36.133 [15].

The test scenario comprises of 1 E-UTRA cell and 1 NR cell subject to CCA as given in tables A.12.1.1.1.1-1, A.8.2.1.1.1-2, A.8.2.1.1.1-3 and A.8.2.1.1.1-4. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. E-UTRA cell 1 is already identified by the UE prior to the start of the test. Cell 2 is of higher priority than cell 1.

Table A.12.1.1.1.1-1: Supported test configurations

| Configuration | Description of a cell without CCA | Description of a cell with CCA |
| --- | --- | --- |
| 1 | LTE FDD | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.12.1.1.1.1-2: General test parameters for E-UTRA cell re-selection FR1 NR cell subject to CCA test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell 2 in the initial phase |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 end condition | Active cell |  |  | Cell1 | During T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  |  | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| DBT Window Configuration |  |  | 1, 2 | TBD | As specified in clause A.3.28.1. |
| DL CCA model |  |  | 1, 2 | As specified in clause A.3.20.2.1 | DL CCA model |
| UL CCA model |  |  | 1, 2 | As specified in clause A.3.20.2.2 | UL CCA model |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2 | TBD | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | TBD | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | TBD | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.12.1.1.1.1-3: Cell specific test parameters for NR cell 2 subject to CCA

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TBD |  |  |
| DL CCA probability PCCA_DL |  | 1, 2 | TBD |  |  |
| UL CCA probability PCCA_UL |  | 1, 2 | TBD |  |  |
| Md,max |  | 1, 2 | 16 |  |  |
| Mm,max |  | 1, 2 | 4 |  |  |
| Me,max |  | 1, 2 | 8 |  |  |
| PDSCH Reference measurement channel |  | 1, 2 | TBD |  |  |
| RMSI CORESET Reference Channel |  | 1, 2 | TBD |  |  |
| RMC CORESET Reference Channel |  | 1, 2 | TBD |  |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |  |
| SSB configuration |  | 1, 2 | TBD |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  |
|  | dB | 1, 2 | -4 | -infinity | 12 |
| Note2 | dBm/SCS | 1, 2 | -95 |  |  |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  |
|  | dB | 1, 2 | -4 | -infinity | 12 |
| SS-RSRP Note3 | dBm/SCS | 1, 2 | -99 | -infinity | -83 |
| Io | dBm/38.16 MHz | 1, 2 | -62.50 | -63.95 | -51.69 |
| Treselection | s | 1, 2 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2 | 50 |  |  |
| Threshx, highP | dB | 1, 2 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2 | 44 |  |  |
| Threshx, lowP | dB | 1, 2 | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.12.1.1.1.1-4: Cell specific test parameters for E-UTRA cell 1

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -84 |
|  | dB | 14 | 14 | 14 |
|  | dB | 14 | 14 | 14 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | 50 |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.12.1.1.1.2	Test Requirements

The cell reselection delay to a higher priority NR cell subject to CCA is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon cell 2.

The cell re-selection delay to a higher priority cell shall be less than 60 + 1.28 x (5 + Me) + TSI_CCA s. Me is the number of DRX cycles with at least one SMTC where there are no SSBs available during the Tevaluate,NR_Intra_CCA. If Me > Me,max the UE is required to restart the evaluation of cell 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter_CCA + TSI_CCA, and to a lower priority cell can be expressed as: Tevaluate, NR + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2 in TS 36.133 [15]

Tevaluate, NR_ inter_CCA	See Table 4.2.2.5.7-1 in clause 4.2.2.5.7

TSI_CCA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell.

Tevaluate, NR	See Table 4.2.2.5.6-1 in clause 4.2.2.5.6 in TS 36.133 [15]

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority NR cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

## A.12.2	RRC_CONNECTED state mobility

### A.12.2.1	Handover

#### A.12.2.1.1	E-UTRAN - NR with CCA handover

##### A.12.2.1.1.1	Test Purpose and Environment

This test shall verify the E-UTRAN to NR FR1 handover requirements specified in clause 5.3.4A in TS 36.133 [15].

The test comprises of one E-UTRA carrier and one NR carrier with CCA. There are two cells and one cell on each carrier. Cell 1 is the E-UTRAN cell and Cell 2 is an inter-RAT NR neighbour cell with CCA.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 of TS 36.133 [15] is configured before T2 begins to enable inter-RAT frequency monitoring. A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

Supported test configurations are shown in table A.12.2.1.1-1. General test parameters are provided in Table A.12.2.1.1-2. Cell specific test parameters for Cell 1 and Cell 2 are provided in Tables A.12.2.1.1-3 and A.12.2.1.1-4 respectively.

Table A.12.2.1.1-1: Supported test configurations for E-UTRAN inter-RAT NR handover

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR with CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.12.2.1.1-2: General test parameters for E-UTRAN inter-RAT NR handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency with CCA is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | E-UTRAN cell |
|  | Neighbouring cell |  | Cell 2 | NR cell with CCA |
| Final condition | Active cell |  | Cell 2 |  |
| DL CCA model |  |  | As specified in clause A.3.26.2.1 |  |
| UL CCA model |  |  | As specified in clause A.3.26.2.2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | -84 | Absolute E-UTRAN RSRP threshold for event B2 |
| b2-Threshold2NR |  | dBm | As specified in Table A.12.2.1.1-4 | Absolute NR SS-RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts [15] |
| T1 |  | s | [5] |  |
| T2 |  | s | [£5] |  |
| T3 |  | s | [1] |  |

Table A.12.2.1.1-3: Cell specific test parameters for E-UTRAN inter-RAT NR handover with CCA (Cell 1)

| Parameter | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2 | 2 |  |  |
| Duplex mode |  | 1 | FDD |  |  |
|  |  | 2 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 1, 2 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 1, 2 | 1 |  |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1 | 4 |  |  |
|  |  | 2 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1, 2 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2 | -98 |  |  |
| Ês/Noc | dB | 1, 2 | 7 | 7 | 7 |
| Ês/IotNote6 | dB | 1, 2 | 7 | 7 | 7 |
| RSRPNote6 | dBm/15kHz | 1, 2 | -91 | -91 | -91 |
| SCH_RPNote6 | dBm/15kHz | 1, 2 | -91 | -91 | -91 |
| IoNote6 | dBm/9MHz | 1, 2 | -62.43 | -62.43 | -62.43 |
| Propagation Condition |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |  |

Table A.12.2.1.1-4: Cell specific test parameters E-UTRAN inter-RAT NR with CCA handover (Cell 2)

| Parameter |  |  | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  |  | 1, 2 | 1 |  |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 4, 6 |  | 1, 2 | PCCA_DL=0.9375 |  |  |
|  |  | Dynamic channel access Note 5, 6 |  | 1, 2 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 4, 6 |  | 1, 2 | PCCA_UL=0.87 |  |  |
|  |  | Dynamic channel access Note 5, 6 |  | 1, 2 | PCCA_UL=0.75 |  |  |
| LCCA_DL |  |  | - | 1, 2 | 5 |  |  |
| WCCA_DL |  |  | ms | 1, 2 | T304 |  |  |
| LCCA_UL |  |  | - | 1, 2 | 5 |  |  |
| WCCA_UL |  |  | ms | 1, 2 | T304 |  |  |
| T304 |  |  | ms | 1, 2 | 500 |  |  |
| Duplex mode |  |  |  | 1, 2 | TDD |  |  |
| TDD Configuration |  |  |  | 1, 2 | TDDConf.1.1 CCA |  |  |
| BWchannel |  |  | MHz | 1, 2 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  |  | 1, 2 | SR.1.1 CCA |  |  |
| CORESET reference channel |  |  |  | 1, 2 | CR.1.1 CCA |  |  |
| PRACH configuration |  |  |  | 1, 2 | FR1 PRACH configuration 1 under CCA |  |  |
| OCNG patternNote1 |  |  |  | 1, 2 | OP.1 |  |  |
| BWP | Initial DL BWP |  |  | 1, 2 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  |  | 1, 2 | SMTC.1 |  |  |
| SSB configuration |  | Semi-static channel access Note 4, 6 |  | 1, 2 | SSB.1 CCA |  |  |
|  |  | Dynamic channel access Note 5, 6 |  | 1, 2 | SSB.2 CCA |  |  |
| DBT window configuration |  |  |  |  | As defined in A.3.28.1 |  |  |
| b2-Threshold2NR |  |  | dBm | 1 | -105 |  |  |
|  |  |  |  | 2 | -103 |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2 | -98 |  |  |
| NocNote2 |  |  | dBm/SCS | 1, 2 | -95 |  |  |
| Ês/Noc |  |  | dB | 1, 2 | -inifinit | 0 | 0 |
| Ês/IotNote3 |  |  | dB | 1, 2 | -inifinit | 0 | 0 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2 | -inifinit | -95 | -95 |
| IoNote3 |  |  | dBm/38.16 MHz | 1, 2 | -63.96 | -60.94 | -60.94 |
| Propagation condition |  |  |  | 1, 2 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |

##### A.12.2.1.1.2	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than TBD ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in TS36.331.

Tinterrupt = 62 + ( L1´ + L3) * TSMTC; Tinterrupt is defined in TS36.133 clause 5.3.4A.3 where

L1´ is the number of SMTC occasions not available at the UE during the inter-RAT detection period.

L3 is the number of consecutive SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failure.  L3 = 0 for Type 2C UL channel access procedure as defined in TS 37.213 [57].

TSMTC = 20 ms is the SMTC periodicity ms in the test.

This gives a total of 112 + (L1´ + L3)*20 ms.

## A.12.3	Signalling characteristics

### A.12.3.1	Interruptions

## A.12.4	Measurement procedure

### A.12.4.1	E-UTRANNR inter-RAT SFTD measurements

#### A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX

##### A.12.4.1.1.1	Test Purpose and Environment

The purpose of this test is to partly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell under CCA is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [15] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and no DRX is configured.

The tests consist of a single time period of duration T1. Two carriers are used in the tests: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier under CCA with the NR neighbour cell (Cell 2).

Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 2 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report.

The supported test configurations are listed in Table A.12.4.1.1.1-1 below. General test parameters and cell-specific parameters for the NR cell are provided in Tables A.12.4.1.1.1-2 and A.12.4.1.1.1-3 below, respectively. Cell-specific parameters for the E-UTRA cell are provided in clause A.3.7.2.1.

Table A.12.4.1.1.1-1: Applicable test configurations for inter-RAT SFTD measurement delay test with NR under CCA

| Config | Description |
| --- | --- |
| 1 | LTE FDD; NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD; NR: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.12.4.1.1.1-2: General test parameters for inter-RAT SFTD measurement delay test with NR under CCA

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2 | 1 |  | One NR carrier frequencies is used. |
| Active cell |  | Config 1,2 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| CP length |  | Config 1,2 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2 | OFF |  | DRX is not used |
| Frame time offset between serving and neighbour cells | ms | Config 1 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | ms | Config 2 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| SS-RSRP reporting |  | Config 1,2 | No |  | Only SFTD is reported. |
| T1 | s | Config 1,2 | [2] |  | T1 shall exceed Tmeasure_SFTD_LBT_max = 56 × SMTC |

Table A.12.4.1.1.1-3: Cell specific test parameters for Cell 2 in inter-RAT SFTD measurement delay test with NR under CCA

| Parameter |  | Unit | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 |
| Duplex mode |  |  | TDD |
| BWchannel |  | MHz | 40: NRB,c = 106 |
| TDD configuration |  |  | TDDConf.1.1 CCA |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |
| DL CCA probability for semi-static channel accessNote5,7 | PCCA_DL |  | [0.75] |
| DL CCA probability for dynamic channel accessNote6,7 | PCCA_DL_1 |  | [0.75] |
|  | PCCA_DL_2 |  | [0.75] |
| OCNG Pattern defined in A.3.2.1.1Note 1 |  |  | OP.1 |
| SMTC configuration defined in A.3.2.11.1 and A.3.2.11.2 |  |  | SMTC.2 |
| SSB configuration for semi-static channel accessNote5,7 |  |  | SSB.1 CCA |
| SSB configuration for dynamic channel accessNote6,7 |  |  | SSB.2 CCA |
| DBT window configuration |  |  | DBT.1 |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 30 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  | dB |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  | dB |  |
| Noc Note2 |  | dBm/15 kHz | -98 |
| Noc Note2 |  | dBm/SCS | -95 |
| SS-RSRP Note 3, 4 |  | dBm/SCS | -91 |
| Ês/Iot |  | dB | 4 |
| Ês/Noc |  | dB | 4 |
| Io Note 3 |  | dBm/38.16MHz | -62.27 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |

##### A.12.4.1.1.2	Test Requirements

Following the start of T1, the UE shall detect Cell 2 and determine the relative time difference between Cell 1 and Cell 2. At latest at TRRC_procedure_delay + Tmeasure_SFTD_LBT_max after the beginning of time duration T1, the UE shall send a measurement report on SFTD between Cell 1 and Cell 2.

The observed rate of successful SFTD reports in repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2×TTIDCCH longer than the measurement reporting delays above due to TTI insertion uncertainty of the measurement report in DCCH.

### A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA

#### A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used

##### A.12.4.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21A of TS 36.133 [15] for E-UTRAN FDD-NR measurements under CCA and clause 8.1.2.4.22A of TS 36.133 [15] for E-UTRAN TDD-NR measurements under CCA.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1 on a carrier frequency with CCA. The test parameters are given in Tables A.12.4.2.1.1-1, A.12.4.2.1.1-2, A.12.4.2.1.1-3 and A.12.4.2.1.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.12.4.2.1.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.12.4.2.1.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The UE is tested when MeasTriggerQuantity is configured as RSRP, RSRQ and SINR for each test. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.12.4.2.1.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.12.4.2.1.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  | 1, 2 | 1 |  | One FR1 NR carrier frequency under CCA is used. |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |  |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2 | Note 1 |  | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2 | Note 2 |  | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | ≥Tidentify_irat_cca_without_index | ≥Tidentify_irat_cca_without_index | Tidentify_irat_cca_without_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| Note 1: The value of b2-Threshold1 is defined in Table A.12.4.2.1.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.12.4.2.1.1-4 |  |  |  |  |  |

Table A.12.4.2.1.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 for RSRP |  |
|  |  | 1, 2 | [TBDfor RSRQ] |  |
|  | dB | 1, 2 | [TBD for SINR] |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.12.4.2.1.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2 | 2 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |
| PCCA_DL |  | 1, 2 | [TBD] |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2 | -98 for SS-RSRP |  |
|  |  | 1, 2 | [69for SS-RSRQ] |  |
|  | dB | 1, 2 | [54 For SS-SINR] |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |

##### A.12.4.2.1.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is not required to report SSB time index. Tidentify_irat_cca_without_index is defined in defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used

##### A.12.4.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21 of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1 on a carrier frequency with CCA. The test parameters are given in Tables A.12.4.2.2.1-1, A.12.4.2.2.1-2, A.12.4.2.2.1-3 and A.12.4.2.2.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.12.4.2.2.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.12.4.2.2.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The UE is tested when MeasTriggerQuantity is configured as RSRP, RSRQ and SINR for each test. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.12.4.2.2.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.12.4.2.2.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  | 1, 2 | 1 |  |  |  | One FR1 NR carrier frequency under CCA is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |  |  |  |
| Neighbour cell |  | 1, 2 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 |  | 19 |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2 | Note 1 |  |  |  | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2 | Note 2 |  |  |  | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |  |  |
| CP length |  | 1, 2 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.9 | DRX.10 | DRX.9 | DRX.10 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 2 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |  |  |
| T2 | s | 1, 2 | ≥Tidentify_irat_cca_without_index |  |  |  | Tidentify_irat_cca_without_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| Note 1: The value of b2-Threshold1 is defined in Table A.12.4.2.1.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.12.4.2.1.1-4 |  |  |  |  |  |  |  |

Table A.12.4.2.2.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 without SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 for RSRP |  |
|  |  | 1, 2 | [TBD for RSRQ] |  |
|  | dB | 1, 2 | [TBD for SINR] |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.12.4.2.2.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2 | 1 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |
| PCCA_DL |  |  | [TBD] |  |
| CCA model |  | 1, 2 | TBD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2 | -98 for SS-RSRP |  |
|  |  | 1, 2 | [TBD for SS-RSRQ] |  |
|  | dB | 1, 2 | [TBD For SS-SINR] |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |

##### A.12.4.2.2.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_without_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used

##### A.12.4.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1 on a carrier frequency with CCA.  The test parameters are given in Tables A.12.4.2.3.1-1, A.12.4.2.3.1-2, A.12.4.2.3.1-3 and A.12.4.2.3.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.12.4.2.3.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.12.4.2.3.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. The UE is tested when MeasTriggerQuantity is configured as RSRP, RSRQ and SINR for each test. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.12.4.2.3.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.12.4.2.3.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  | 1, 2 | 1 |  | One FR1 NR carrier frequency under CCA is used. |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |  |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2 | Note 1 |  | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2 | Note 2 |  | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | ≥ Tidentify_irat_cca_with_index | ≥ Tidentify_irat_cca_with_index | Tidentify_irat_cca_with_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| Note 1: The value of b2-Threshold1 is defined in Table A.12.4.2.3.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.12.4.2.3.1-4 |  |  |  |  |  |

Table A.12.4.2.3.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 with SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 for RSRP |  |
|  |  | 1, 2 | [TBD for RSRQ] |  |
|  | dB | 1, 2 | [TBD for SINR] |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.12.4.2.3.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2 | 1 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |
| PCCA_DL |  |  | [TBD] |  |
| CCA model |  | 1, 2 | TBD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | SMTC.1 |  |
| DBT window configuration |  | 1, 2 | DBT.1 |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2 | -98 for SS-RSRP |  |
|  |  | 1, 2 | [TBD for SS-RSRQ] |  |
|  | dB | 1, 2 | [TBD For SS-SINR] |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |

##### A.12.4.2.3.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used

##### A.12.4.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the NR inter-RAT cell search requirements in clause 8.1.2.4.21of TS 36.133 [15] for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 [15] for E-UTRAN TDD-NR measurements.

In this test, there are two cells: E-UTRA cell 1 as PCell on E-UTRA RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 1 on a carrier frequency with CCA. The test parameters are given in Tables A.12.4.2.4.1-1, A.12.4.2.4.1-2, A.12.4.2.4.1-3 and A.12.4.2.4.1-4. Cell transmits SSBs in DBT windows according to DL CCA model.

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table A.12.4.2.4.1-2 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table A.12.4.2.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) [16] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

Table A.12.4.2.4.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR1

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| 2 | LTE TDD; NR with CCA: SCS 30 kHz, BW 40 MHz, TDD |
| NOTE: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.12.4.2.4.1-2: General test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  | One E-UTRAcarrier frequency is used. |
| NR RF Chanel Number |  | 1, 2 | 1 |  |  |  | One FR1 NR carrier frequency under CCA is used. |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |  |  |  |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [15]. |
| Measurement gap offset |  | 1, 2 | 39 |  | 19 |  | As specified in TS 36.331 [16]. |
| b2-Threshold1 | dBm | 1, 2 | Note 1 |  |  |  | E-UTRA RSRP/RSRQ/SINR threshold for E-UTRA RSRP measurement on cell 1 for event B2 [16] |
| b2-Threshold2NR | dBm | 1, 2 | Note 2 |  |  |  | SS-RSRP/ SS-RSRQ/ SS-SINR threshold measurement on cell 2 for event B2 [16] |
| Hysteresis | dB | 1, 2 | 0 |  |  |  |  |
| CP length |  | 1, 2 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.9 | DRX.10 | DRX.9 | DRX.10 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 2 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |  |  |
| T2 | s | 1, 2 | ≥Tidentify_irat_cca_with_index |  |  |  | Tidentify_irat_cca_with_index is defined in clause 8.1.2.4.21A.1 and 8.1.2.4.22A.1 in TS 36.133 |
| Note 1: The value of b2-Threshold1 is defined in Table A.12.4.2.4.1-3 Note 2: The value of b2-Threshold2NR is defined in Table A.12.4.2.4.1-4 |  |  |  |  |  |  |  |

Table A.12.4.2.4.1-3: E-UTRAN PCell specific test parameters for NR inter-RAT event triggered reporting in non-DRX with NR neigbour cell in FR1 with SSB time index detection

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2 | 1 |  |
| Duplex mode |  | 1 | FDD |  |
|  |  | 2 | TDD |  |
| TDD special subframe configurationNote1 |  | 2 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 2 | 1 |  |
| BWchannel | MHz | 1, 2 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 2 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 2 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 2 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2 | -77 for RSRP |  |
|  |  | 1, 2 | [TBD for RSRQ] |  |
|  | dB | 1, 2 | [TBD for SINR] |  |
| PBCH_RA | dB | 1, 2 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2 | -104 |  |
| Ês/Noc | dB | 1, 2 | 17 | 17 |
| Ês/IotNote5 | dB | 1, 2 | 17 | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2 | -87 | -87 |
| IoNote5 | dBm/9MHz | 1, 2 | -59.13+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

Table A.12.4.2.4.1-4: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2 | 1 |  |
| TDD configuration |  | 1, 2 | TDDConf.1.1 CCA |  |
| BWchannel | MHz | 1, 2 | 40: NRB,c = 106 |  |
| PCCA_DL |  |  | TBD |  |
| CCA model |  | 1, 2 | TBD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 2 | TBD |  |
| DBT window configuration |  | 1, 2 | TBD |  |
| SSB configuration for semi-static channel access |  | 1, 2 | SSB.1 CCA |  |
| SSB configuration for dynamic channel access |  | 1, 2 | SSB.2 CCA |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2 | -98 for SS-RSRP |  |
|  |  | 1, 2 | [TBD for SS-RSRQ] |  |
|  | dB | 1, 2 | [TBD For SS-SINR] |  |
| EPRE ratio of PSS to SSS |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2 | -98 |  |
| Note2 | dBm/SCS | 1, 2 | -95 |  |
| SS-RSRP Note 3,5 | dBm/SCS | 1, 2 | -Infinity | -88 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| Note 5 | dB | 1, 2 | -Infinity | 7 |
| IoNote3 | dBm/38.16MHz | 1, 2 | -63.95 | -56.16 |
| Propagation Condition |  | 1, 2 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |

##### A.12.4.2.4.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than Tidentify_irat_cca_with_index ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.12.4.2.5	RSSI measurement reporting

##### A.12.4.2.5.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the inter-RAT RSSI measurement reporting requirements in TS 36.133 [15, Section 8.1.2.4.21A.1.5].

##### A.12.4.2.5.2	Test parameters

In the test, the UE is configured to perform inter-RAT RSSI measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.12.4.2.5.2-1. There is one cell in the test: Cell 1 is E-UTRAN PCell on a licensed band. Prior to the start of the time duration T1, the UE is connected to Cell 1. The RSSI measurement is performed on an inter-RAT carrier frequency under CCA. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.12.4.2.5.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.12.4.2.5.2-2: General test parameters.

Editor’s note: Table TBD

#### A.12.4.2.6	Channel occupancy measurement reporting

##### A.12.4.2.6.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the inter-RAT channel occupancy measurement reporting requirements in TS 36.133 [15, Section 8.1.2.4.21A.1.6].

##### A.12.4.2.6.2	Test parameters

In the test, the UE is configured to perform inter-frequency channel occupancy measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.12.4.2.6.2-1. There is one cell in the test: Cell 1 which is E-UTRAN PCell on a licensed band. Prior to the start of the time duration T1, the UE is connected to Cell 1. The channel occupancy measurement is performed on an inter-RAT carrier frequency under CCA. The E-UTRAN PCell setting refers to Table A.3.7.2.1-1.

Table A.12.4.2.6.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Table A.12.4.2.6.2-2: General test parameters.

Editor’s note: Table is TBD

## A.12.5	Measurement performance

### A.12.5.1	E-UTRANNR SFTD

#### A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA

##### A.12.5.1.1.1	Test Purpose

The purpose of this set of tests is to verify that the SFTD measurement accuracy is within the specified limits. This test will verify the requirements as specified in clause 9.1.27 in TS 36.133 [15] for inter-RAT SFTD measurements between E-UTRA PCell and NR target cell under CCA.

##### A.12.5.1.1.2	Test Environment

Supported test configurations are shown in Table A.12.5.1.1.2-1. In this set of test cases there are two cells on different carriers. Cell 1 is E-UTRAN PCell and Cell 2 is inter-RAT NR target cell under CCA. The test parameters of Cell 1 are given in clause A.12.5.1.1.2-2. The test parameters of Cell 2 are given in Table A.12.5.1.1.2-3. The SFTD between PCell and NR target cell shall be set by the test equipment to one of the time differences in Table A.12.5.1.1.2-4.

Table A.12.5.1.1.2-1: Supported test configurations for SFTD accuracy with NR target cell under CCA

| Config | Description |
| --- | --- |
| 1 | LTE FDD NR with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD NR with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.12.5.1.1.2-2: Test parameters for inter-RAT SFTD accuracy with NR target cell under CCA (Cell 1)

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | -3 |
| Ês/Iot | dB | -3 |
| RSRP Note5 | dBm/15 kHz | -107 |
| SCH_RP Note5 | dBm/15 kHz | -107 |
| Io Note5 | dBm/Ch BW | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table A.12.5.1.1.2-3: Test parameters for inter-RAT SFTD accuracy with NR target cell under CCA (Cell 2)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Configuration |  |  | TDDConf.1.1 CCA |
| BWchannel |  | MHz | 40: NRB,c = 106 |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |
| DL CCA probability for semi-static channel accessNote6,8 | PCCA |  | [0.75] |
| DL CCA probability for dynamic channel accessNote7,8 | PCCA_DL_1 |  | [0.75] |
|  | PCCA_DL_2 |  | [0.75] |
| SSB configuration for semi-static channel accessNote6,8 |  |  | SSB.1 CCA |
| SSB configuration for dynamic channel accessNote7,8 |  |  | SSB.2 CCA |
| SMTC configuration |  |  | SMTC.1 |
| DBT window configuration |  |  | DBT.1 |
| OCNG Patterns |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Noc Note2 |  | dBm/15kHz | -104 |
| Noc Note2 |  | dBm/SSB SCS | -101 |
| Ês/Iot |  | dB | -3 |
| Ês/Noc |  | dB | -3 |
| SS-RSRP Note3 |  | dBm/SCS | -104 |
| Io Note3 |  | dBm/38.16 MHz | -68.18 |
| Propagation condition |  |  | AWGN |
| Antenna configuration |  |  | 1x2 |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols in slots with downlink transmission bursts. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 8: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |

Table A.12.5.1.1.2-4: Timing offsets for inter-RAT SFTD accuracy test with NR target cell under CCA

| Configuration | SFN offset between PCell and NR neighbor cell | Frame boundary offset between PCell and NR neighbour cell (Ts) |
| --- | --- | --- |
| 1 | 100 | -122000 |
| 2 | 300 | -60540 |
| 3 | 500 | 1000 |
| 4 | 700 | 62540 |
| 5 | 900 | 124000 |

##### A.12.5.1.1.3	Test Requirements

The SFTD reported by the UE consists of 2 elements, SFN offset and frame boundary offset between PCell and inter-RAT NR target cell. The reported SFTD accuracy shall fulfil the requirement in clause 9.1.27 in TS 36.133 [15].

### A.12.5.2	E-UTRANNR SS-RSRP

### A.12.5.3	E-UTRANNR SS-RSRQ

### A.12.5.4	E-UTRANNR SS-SINR

### A.12.5.5	E-UTRANNR RSSI

### A.12.5.6	E-UTRANNR channel occupancy

# A.13		NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1

## A.13.1	Timing

## A.13.1.1	UE transmit timing

## A.13.1.2	Timing advance

## A.13.2	Signalling characteristics

### A.13.2.1	Interruption

#### A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell

##### A.13.2.1.1.1	Test Purpose and Environment

The purpose of this test is to verify NR PCell interruptions during SCell operations on an NR SCC with CCA, This test will verify the interruption requirements for NR PCell in NR SA specified in TS 38.133 clause 8.2.2 and 8.3A. Supported test configurations are shown in table A.13.2.1.1.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.13.2.1.1.1-2 and A.13.2.1.1.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 and Cell2 are PCell and SCell. Cell 1 is on a licenced band and cell 2 is subject to CCA. The test consists of five time periods, with duration of T1, T2, T3, T4 and T5. Prior to the start of the time duration T1, the UE is connected to Cell1 and Cell2. Throughout the test, the PCell are continuously scheduled in DL. The power of signals on cell 1 and 2 is not modified during the test.

Prior to T1, a connection is started with cell 1 as the PCell, and measurements of cell 2 are configured with gap pattern 0, such that cell 2 is reported. This ensures that cell 2 is known at the start of time period T1 and is not itself part of the tested requirement.

The point in time at which the RRC message implying Scell addition is received at the UE antenna connector, defines the start of time period T1. Measurement gap pattern 0 shall be stopped when the Scell is configured.

The point in time at which the MAC-CE message implying Scell activation is received at the UE antenna connector, defines the start of time period T2.

The point in time at which the MAC-CE message implying Scell deactivation is received at the UE antenna connector, defines the start of time period T3.

The point in time at which deactivation delay requirement in section 8.3A are satisfied defines the start of time period T4

The point in time at which the RRC message implying Scell release is received at the UE antenna connector, defines the start of time period T5.

Table A.13.2.1.1.1-1: Interruptions during measurements on deactivated NR SCC supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.13.2.1.1.1-2: General test parameters for Interruptions during measurements on deactivated NR SCC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 |  |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured dSCell |  | Cell2 | SCell on NR RF channel number 2 |
| CP length |  | Normal | Applicable to Cell1, Cell2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| T1 | s | <10 |  |
| T2 | s | <10 |  |
| T3 | s | <10 |  |
| T4 | s | <10 |  |
| T5 | s | <10 |  |

Table A.13.2.1.1.1-3: NR cell specific test parameters for Interruptions during measurements on deactivated NR SCC

| Parameter |  | Unit | Cell1 |  |  |  |  | Cell2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| TDD configuration | Config 1 |  | --- |  |  |  |  | TDDConf.1.1 CCA |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |
| DL CCA model | Config 1,2,3 |  | --- |  |  |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |
| DL CCA probability for semi-static channel accessNote6,8 | PCCA_DL |  | --- |  |  |  |  | 0.9375 |  |  |  |  |
| DL CCA probability for dynamic channel accessNote7,8 | PCCA_DL_1 |  | --- |  |  |  |  | 0.75 |  |  |  |  |
|  | PCCA_DL_2 |  | --- |  |  |  |  | 0.75 |  |  |  |  |
| Initial BWP Configuration | Config 1,2,3 |  | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Dedicated DL BWP Configuration | Config 1,2,3 |  | DLBWP.1.1 |  |  |  |  | DLBWP.1.1 |  |  |  |  |
| Initial UL BWP Configuration | Config 1,2,3 |  | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| Dedicated UL BWP Configuration | Config 1,2,3 |  | ULBWP.1.1 |  |  |  |  | ULBWP.1.1 |  |  |  |  |
| PDSCH reference meassurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  | --- |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Parameters | Config 1 |  | CR.1.1 FDD |  |  |  |  | --- |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |  |  |  |  |
| PDCCH CORESET Parameters | Config 1 |  | CCR.1.1 FDD |  |  |  |  | --- |  |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |  |  |  |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  | --- |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |
| OCNG Pattern |  |  | OP.1 |  |  |  |  | OP.1 |  |  |  |  |
| SSB configuration for semi-static channel accessNote6,8 | Config 1,2 |  | SSB.1 FR1 |  |  |  |  | SSB.1 CCA |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |
| SSB configuration for dynamic channel accessNote7,8 | Config 1,2 |  | SSB.1 FR1 |  |  |  |  | SSB.2 CCA |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |
| SMTC Configuration | Config 1,2,3 |  | SMTC.1 |  |  |  |  | SMTC.1 |  |  |  |  |
| DBT window configuration | Config 1,2,3 |  | --- |  |  |  |  | DBT.1 |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  | --- |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  | 1x2 Low |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | Config 1,2,3 | dBm/15 kHz | -104 |  |  |  |  | -104 |  |  |  |  |
| NocNote 2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  | -101 |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note 3 | Config 1,2,3 | dBm/15 kHz | -87 |  |  |  |  | -87 |  |  |  |  |
| Ês/Iot |  | dB | 17 |  |  |  |  | 17 |  |  |  |  |
| Ês/Noc |  | dB | 17 |  |  |  |  | 17 |  |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For Cell 2 with CCA model, OCNG is transmitted only in slots with downlink transmission bursts and is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. Note 4: Receive time difference of signals received between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell at the UE antenna connector including time alignment error between the two cells Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 8: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.13.2.1.1.2	Test Requirements

The UE shall meet the interruption requirements for SCell addition on the victim Pcell in clause 8.2.1 during time T1

The UE shall meet the interruption requirements for SCell activation on the victim Pcell in clause 8.2.1during time T2. There shall be a single interruption with time window as specified in clause 8.3A.2

The UE shall meet the interruption requirements for SCell deactivation on the victim PCell in clause 8.2.1during time T3. There shall be a single interruption with time window as specified in clause 8.3A.3

The UE shall meet the interruption requirements for deactivated SCell measurements on the victim PCell in clause 8.2.1 during time T4. The interruptions shall be within the time window as specified in clause 8.3A.3

The UE shall meet the interruption requirements for SCell release on the victim PCell in clause 8.2.1during time T5.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.13.2.2	SCell activation and deactivation delay

#### A.13.2.2.1 SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle

##### A.13.2.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation delays for SCell on NR-U SCC with CCA are within the requirements stated in clause 8.3A, when the SCell is known by the UE at the time of activation and the configured SCell measurement cycle is 160 ms.

The supported test configurations are shown in Table A.13.2.2.1.1-1.

The test parameters are given in Table A.13.2.2.1.1-2 and cell-specific parameters in Table A.13.2.2.1.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell: Cell 1 (PCell) on radio channel 1 (PCC) in NR FR1, and Cell2 (SCell) on radio channel 2 (SCC) in NR with CCA. Before the test starts the UE is connected to Cell 1, but is not aware of Cell 2, as the UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. The UE now starts monitoring the SCC. At the end of T1, the test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a slot # denoted m, defines the start of time period T2. The UE shall be able to report a valid CSI in PCell for the activated SCell at latest in slot m + THARQ+Tactivation_time_withCCA+TCSI_Reporting_withCCANR slot length, as defined in clause 8.3A.2. The UE shall start reporting CSI in PCell in slot m+ THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed.

Any PCell interruption shall fall within the time window specified in clause 8.3A.2. At the end of T2 the test equipment sends a MAC message for deactivation of the SCell.

The point in time at which the MAC message is received by at the UE antenna connector, in a slot # denoted n, defines the start of time period T3. The UE shall complete the activation at latest in slot n+THARQ+3msNR slot length. Any PCell interruption shall fall within the time window specified in clause 8.3A.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received, while taking into account CCA failures on SCC.

The test equipment verifies the deactivation time by counting the slots from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.13.2.2.1.1-1: Supported test configurations for SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode; With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.13.2.2.1.1-2: General test parameters for known SCell activation with SCell under CCA, 160 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two radio channels (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every fourth slot |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | TAE as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 10+5⋅2µDL | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] µDL is the subcarrier spacing configuration for DL |

Table A.13.2.2.1.1-3: Cell specific test parameters for known FR1 SCell activation case with SCell under CCA, 160 ms SCell measurement cycle

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  | TDD |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | --- |  |  | TDDConf.1.1 CCA |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | 10: NRB,c = 52 |  |  | 40: NRB,c = 106 |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DL CCA model |  |  | --- |  |  | As specified in clause A.3.26.2.1 |  |  |
| DL CCA probability for semi-static channel accessNote5,7 | PCCA_DL |  | --- |  |  | 0.9375 |  |  |
| DL CCA probability for dynamic channel accessNote6,7 | PCCA_DL_1 |  | --- |  |  | 0.75 |  |  |
|  | PCCA_DL_2 |  | --- |  |  | 0.75 |  |  |
| PCCA_UL |  |  |  |  |  | 1 |  |  |
| LCCA_DL Note 8 |  |  |  |  |  | 2 |  |  |
| WCCA_DL Note 8 |  | ms |  |  |  | Tactivation_time_withCCA |  |  |
| Initial downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Initial uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Dedicated downlink BWP configuration |  |  | DLBWP.0.2 |  |  | DLBWP.0.2 |  |  |
| Dedicated uplink BWP configuration |  |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| TCI state |  |  | TCI.State.0 |  |  | TCI.State.0 |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  | TRS.1.2 TDD |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  | SR.1.1 CCA |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  | CCR.1.3 CCA |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  | CR.1.1 CCA |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |
| OCNG Patterns Note1 |  |  | OP.1 |  |  | OP.1 |  |  |
| SSB Configuration for semi-static channel accessNote5,7 | Config 1,2 |  | SSB.1 FR1 |  |  | SSB.1 CCA |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| SSB Configuration for dynamic channel accessNote6,7 | Config 1,2 |  | SSB.1 FR1 |  |  | SSB.2 CCA |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  | SMTC.1 |  |  |
| DBT window configuration |  |  | --- |  |  | DBT.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 | dBm/15kHz | -104 |  |  | -104 |  |  |
|  | Config 3 |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 | dBm/SCS | -104 |  |  | -101 |  |  |
|  | Config 3 |  | -101 |  |  |  |  |  |
| Ês/Iot |  | dB | 17 |  |  | 17 |  |  |
| Ês/Noc |  | dB | 17 |  |  | 17 |  |  |
| SS-RSRP Note3 | Config 1,2 | dBm/SCS | -87 |  |  | -84 |  |  |
|  | Config 3 |  | -84 |  |  | -84 |  |  |
| IoNote3 | Config 1,2 |  | -58.96 |  |  | -52.87 |  |  |
|  | Config 3 |  | -52.87 |  |  | -52.87 |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For Cell 2 with CCA model, OCNG is transmitted only in slots with downlink transmission bursts and is not transmitted during muted slots or during DBT windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic cannel access, the UE must be tested under both dynamic and semi-static channel occupancy configurations. Note 8: As specified in clause 8.3A for L1,max, L2,1,max, L2,2,max, L3,1,max, and L3,2,max |  |  |  |  |  |  |  |  |

##### A.13.2.2.1.2	Test Requirements

During T2, the UE shall send the first CSI report for SCell in slot m+1+THARQ+3msNR slot length.

During T2, conditioned on that downlink CCA failures L1 and L2,2 experienced in the SCell fulfill L1 ≤ L1,max and L2,2 ≤ L2,2,max with L1,max = 2 and L2,2,max = 2, respectively, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in slot m + (THARQ+Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB + L1*Trs + 5ms and TCSI_reporting_withCCA = TCSI_reporting + L2,2*TCSI-RS + TCSI_ReportingDelay, as specified in clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in clause 8.3A.3.

During T2, interruption on PCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB.

During T3, interruption on PCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PCell shall not be more than specified for SA in clause 8.2.2.2.2.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

#### A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 320 ms SCell measurement cycle

##### A.13.2.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation delays for SCell on NR-U SCC with CCA are within the requirements stated in clause 8.3A, when the SCell is known by the UE at the time of activation and the configured SCell measurement cycle is 320 ms.

The supported test configurations are same as in Table A.13.2.2.1.1-1 above.

The test parameters are same as in Table A.13.2.2.1.1-2 above, except for parameters listed below in Table A.13.2.2.2.1-1. The cell-specific parameters are same as in Table A.13.2.2.1.1-3 above.

The test execution is the same as described in clause A.13.2.2.1 above, except that downlink CCA failures L2,1 and L2,2 with limits L2,1 ≤ L2,1,max and L2,2 ≤ L2,2,max replace L1 as described in clause 8.3A.2 for activation of known SCell with a measurement cycle larger than 160 ms.

Table A.13.2.2.2.1-1: General test parameters for known SCell activation with SCell under CCA, 320 ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |

##### A.13.2.2.2.2	Test Requirements

During T2, the UE shall send the first CSI report for SCell in slot m+1+THARQ+3msNR slot length.

During T2, conditioned on that downlink CCA failures L2,1 and L2,2 experienced in the SCell fulfill L2,1 ≤ L2,1,max and L2,2 ≤ L2,2,max with L2,1,max = 2 and L2,2,max = 2, respectively, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in slot m + (THARQ+Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB_MAX + L2,1*TSMTC_MAX + (1 +L2,2)*Trs + 5ms and TCSI_reporting_withCCA = TCSI_reporting + TCSI_ReportingDelay, as specified in clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in clause 8.3A.3.

During T2, interruption on PCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB.

During T3, interruption on PCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PCell shall not be more than specified for SA in clause 8.2.2.2.2.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

#### A.13.2.2.3 SCell Activation and Deactivation of unknown SCell under CCA

##### A.13.2.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation delays for SCell on NR-U SCC with CCA are within the requirements stated in clause 8.3A, when the SCell is unknown to the UE at the time of activation.

The supported test configurations are same as in Table A.13.2.2.1.1-1 above.

The test parameters are same as in Table A.13.2.2.1.1-2 above, except for parameters listed below in Table A.13.2.2.3.1-1. The cell-specific parameters are same as in Table A.13.2.2.1.1-3 above.

The test execution is the same as described in clause A.13.2.2.1 above, except that downlink CCA failures L3,1 and L3,2 with limits L3,1 ≤ L3,1,max and L3,2 ≤ L3,2,max replace L1 as described in clause 8.3A.2 for activation of unknown SCell.

Table A.13.2.2.3.1-1: General test parameters for unknown SCell activation with SCell under CCA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | s | 0.1 | During this time period the PCell shall be known and the SCell configured, but not detected. |

##### A.13.2.2.3.2	Test Requirements

During T2, the UE shall send the first CSI report for SCell in slot m+1+THARQ+3msNR slot length.

During T2, conditioned on that downlink CCA failures L3,1 and L3,2 experienced in the SCell fulfill L3,1 ≤ L3,1,max and L3,2 ≤ L3,2,max with L3,1,max = 2 and L3,2,max = 2, respectively, the UE shall send the first valid CSI report (non-zero CQI) for the SCell in slot m + (THARQ+Tactivation_time_withCCA + TCSI_Reporting_withCCA)/NR_slot_length, where Tactivation_time_withCCA = TFirstSSB_MAX + (1 + L3,1)*TSMTC_MAX + (2 + L3,2)*Trs + 5ms and TCSI_reporting_withCCA = TCSI_reporting + TCSI_ReportingDelay, as specified in clause 8.3A.2.

During T3, the UE shall stop sending CSI reports for SCell at latest in slot n+THARQ+3msNR slot length, as defined in clause 8.3A.3.

During T2, interruption on PCell shall not occur outside slot m +1+THARQNR slot length  to slot m +1+THARQ+3+TXNR slot length with TX = TFirstSSB.

During T3, interruption on PCell shall not occur outside slot n +1+THARQ/NR_slot_length to slot n+1+(THARQ +3ms)/NR_slot_length.

The interruption on PCell shall not be more than specified for SA in clause 8.2.2.2.2.

The rate of correctly observed SCell activation delays and SCell deactivation delays shall for repeated tests be at least 90%.

### A.13.2.3	Active TCI state switching delay

## A.13.3	Measurement procedure

### A.13.3.1	Intra-frequency measurements

#### A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX

##### A.13.3.1.1.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.13.3.1.1.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), and two cells on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model: SCell (Cell 2) and a neighbour cell (Cell 3). The test parameters for the three cells are given in Table A.13.3.1.1.2-1 and A.13.3.1.1.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 3.

FFS: The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

- In the first test (Test 1), the UE is configured with SS-RSRP as Event A3 measurement quantity.

- In the second test (Test 2), the UE is configured with SS-RSRQ as Event A3 measurement quantity.

- In the third test (Test 3), the UE is configured with SS-SINR as Event A3 measurement quantity.

Table A.13.3.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.3.1.1.2-2: General test parameters for intra-frequency event triggered reporting without gaps

| Parameter | Unit | Test configuration | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |  |
| Active PCell |  | 1, 2, 3 | Cell 1 |  |  |  |
| Active SCell |  | 1, 2, 3 | Cell 2 |  |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 3 |  |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and Cell 3 |  |  |  |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |  |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |  |  |
| SSB configuration |  | 1 | Cell 1: SSB.1 FR1 Cell 2,3: TBD |  |  |  |
|  |  | 2 | Cell 1: SSB.1 FR1 Cell 2,3: TBD |  |  |  |
|  |  | 3 | Cell 1: SSB.2 FR1 Cell 2,3: TBD |  |  |  |
| SMTC configuration |  | 1 | Cell 1: SMTC.2 Cell 2,3: N/A |  |  |  |
|  |  | 2 | Cell 1: SMTC.1 Cell 2,3: N/A |  |  |  |
|  |  | 3 | Cell 1: SMTC.1 Cell 2,3: N/A |  |  |  |
| DBT window configuration |  | 1, 2, 3 | Cell 1: N/A Cell 2,3: TBD |  |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |  |
| Event A3 measurement quantity |  |  | SS-RSRP | SS-RSRQ | SS-SINR |  |
| CP length |  | 1, 2, 3 | Normal |  |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  |  |  | OFF |
| Time offset between Cell 2 and Cell 3 |  | 1 | 3 ms |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | 2 | 3 ms |  |  | Synchronous cells |
|  |  | 3 | 3 ms |  |  | Synchronous cells |
| T1 | s | 1, 2, 3 | TBD |  |  |  |
| T2 | s | 1, 2, 3 | TBD |  |  |  |

Table A.13.3.1.1.2-3: Cell-specific test parameters for intra-frequency event-triggered reporting without gaps

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| DL CCA probability PCCA_DL |  | 1, 2, 3 | N/A |  | TBD | TBD | TBD | TBD |
| UL CCA probability PCCA_UL |  | 1, 2, 3 | N/A |  | TBD | TBD | TBD | TBD |
| TDD configuration |  | 1 | N/A |  | TBD |  | TBD |  |
|  |  | 2 | TDDConf.1.1 |  | TBD |  | TBD |  |
|  |  | 3 | TDDConf.2.1 |  | TBD |  | TBD |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | CR.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | CR.2.1 TDD |  | TBD |  | TBD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | CCR.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | CCR.2.1 TDD |  | TBD |  | TBD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | TBD |  | TBD |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | TRS.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | TRS.1.2 TDD |  | TBD |  | TBD |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  | TBD |  | TBD |  |
|  |  | 2 | -98 |  | TBD |  | TBD |  |
|  |  | 3 | -95 |  | TBD |  | TBD |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | TBD |  | TBD |  |
|  |  | 2 | -98 |  | TBD |  | TBD |  |
|  |  | 3 | -95 |  | TBD |  | TBD |  |
| Note 5 | dB | 1 | 4 | -1.46 | TBD | TBD | -Infinity | TBD |
|  |  | 2 |  |  | TBD | TBD | -Infinity | TBD |
|  |  | 3 |  |  | TBD | TBD | -Infinity | TBD |
| Note 5 | dB | 1 | 4 | 4 | TBD | TBD | -Infinity | TBD |
|  |  | 2 |  |  | TBD | TBD | -Infinity | TBD |
|  |  | 3 |  |  | TBD | TBD | -Infinity | TBD |
| SS-RSRP Note 3,5 | dBm/SCS kHz | 1 | -94 | -94 | TBD | TBD | -Infinity | TBD |
|  |  | 2 | -94 | -94 | TBD | TBD | -Infinity | TBD |
|  |  | 3 | -91 | -91 | TBD | TBD | -Infinity | TBD |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | TBD | TBD | TBD | TBD |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | TBD | TBD | TBD | TBD |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | TBD | TBD | TBD | TBD |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |  |  |  |  |

##### A.13.3.1.1.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX

##### A.13.3.1.2.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.5.1 and 9.2A.5.2.

##### A.13.3.1.2.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), and two cells on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model: SCell (Cell 2) and a neighbour cell (Cell 3). The test parameters for the three cells are given in Table A.13.3.1.2.2-1 and A.13.3.1.2.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 3.

FFS: The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

- In Test 1 and Test 2, the UE is configured with SS-RSRP as Event A3 measurement quantity.

- In Test 3 and Test 4, the UE is configured with SS-RSRQ as Event A3 measurement quantity.

- In Test 5 and Test 6, the UE is configured with SS-SINR as Event A3 measurement quantity.

Table A.13.3.1.2.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.3.1.2.2-2: General test parameters for intra-frequency event triggered reporting without gaps with DRX

| Parameter | Unit | Test configuration | Value |  |  |  |  |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  | Test 2 |  | Test 3 |  | Test 4 |  | Test 5 | Test 6 |  |
| Active PCell |  | 1, 2, 3 | Cell 1 |  |  |  |  |  |  |  |  |  |  |
| Active SCell |  | 1, 2, 3 | Cell 2 |  |  |  |  |  |  |  |  |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 3 |  |  |  |  |  |  |  |  |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and Cell 3 |  |  |  |  |  |  |  |  |  |  |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |  |  |  |  |  |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | 1 | Cell 1: SSB.1 FR1 Cell 2,3: TBD |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | Cell 1: SSB.1 FR1 Cell 2,3: TBD |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | Cell 1: SSB.2 FR1 Cell 2,3: TBD |  |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | 1 | Cell 1: SMTC.2 Cell 2,3: N/A |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | Cell 1: SMTC.1 Cell 2,3: N/A |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | Cell 1: SMTC.1 Cell 2,3: N/A |  |  |  |  |  |  |  |  |  |  |
| DBT window configuration |  | 1, 2, 3 | Cell 1: N/A Cell 2,3: TBD |  |  |  |  |  |  |  |  |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |  |  |  |  |  |  |  |  |
| Event A3 measurement quantity |  |  | SS-RSRP |  |  | SS-RSRQ |  |  |  | SS-SINR |  |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |  |  |  |  |  |  |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |  |  |  |  |  |  |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |  |  |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  |  |  |  |  |  |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 | DRX.2 |  | DRX.1 |  | DRX.2 |  | DRX.1 |  | DRX.2 |  |
| Time offset between Cell 2 and Cell 3 |  | 1 | 3 ms |  |  |  |  |  |  |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | 2 | 3 ms |  |  |  |  |  |  |  |  |  | Synchronous cells |
|  |  | 3 | 3 ms |  |  |  |  |  |  |  |  |  | Synchronous cells |
| T1 | s | 1, 2, 3 | TBD |  |  |  |  |  |  |  |  |  |  |
| T2 | s | 1, 2, 3 | TBD | TBD |  | TBD |  | TBD |  | TBD |  | TBD |  |

Table A.13.3.1.2.2-3: Cell-specific test parameters for intra-frequency event-triggered reporting without gaps

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| DL CCA probability PCCA_DL |  | 1, 2, 3 | N/A |  | TBD | TBD | TBD | TBD |
| UL CCA probability PCCA_UL |  | 1, 2, 3 | N/A |  | TBD | TBD | TBD | TBD |
| TDD configuration |  | 1 | N/A |  | TBD |  | TBD |  |
|  |  | 2 | TDDConf.1.1 |  | TBD |  | TBD |  |
|  |  | 3 | TDDConf.2.1 |  | TBD |  | TBD |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | CR.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | CR.2.1 TDD |  | TBD |  | TBD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | CCR.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | CCR.2.1 TDD |  | TBD |  | TBD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | TBD |  | TBD |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | TRS.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | TRS.1.2 TDD |  | TBD |  | TBD |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  | TBD |  | TBD |  |
|  |  | 2 | -98 |  | TBD |  | TBD |  |
|  |  | 3 | -95 |  | TBD |  | TBD |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | TBD |  | TBD |  |
|  |  | 2 | -98 |  | TBD |  | TBD |  |
|  |  | 3 | -95 |  | TBD |  | TBD |  |
| Note 5 | dB | 1 | 4 | -1.46 | TBD | TBD | -Infinity | TBD |
|  |  | 2 |  |  | TBD | TBD | -Infinity | TBD |
|  |  | 3 |  |  | TBD | TBD | -Infinity | TBD |
| Note 5 | dB | 1 | 4 | 4 | TBD | TBD | -Infinity | TBD |
|  |  | 2 |  |  | TBD | TBD | -Infinity | TBD |
|  |  | 3 |  |  | TBD | TBD | -Infinity | TBD |
| SS-RSRP Note 3,5 | dBm/SCS kHz | 1 | -94 | -94 | TBD | TBD | -Infinity | TBD |
|  |  | 2 | -94 | -94 | TBD | TBD | -Infinity | TBD |
|  |  | 3 | -91 | -91 | TBD | TBD | -Infinity | TBD |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | TBD | TBD | TBD | TBD |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | TBD | TBD | TBD | TBD |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | TBD | TBD | TBD | TBD |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |  |  |  |  |

##### A.13.3.1.2.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX

##### A.13.3.1.3.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.13.3.1.3.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), and two cells on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model: SCell (Cell 2) and a neighbour cell (Cell 3). The test parameters for the three cells are given in Table A.13.3.1.3.2-1 and A.13.3.1.3.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 3.

FFS: The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

- In the first test (Test 1), the UE is configured with SS-RSRP as Event A3 measurement quantity.

- In the second test (Test 2), the UE is configured with SS-RSRQ as Event A3 measurement quantity.

- In the third test (Test 3), the UE is configured with SS-SINR as Event A3 measurement quantity.

Table A.13.3.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.3.1.3.2-2: General test parameters for intra-frequency event triggered reporting with per-UE gaps

| Parameter | Unit | Test configuration | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |  |
| Active PCell |  | 1, 2, 3 | Cell 1 |  |  |  |
| Active SCell |  | 1, 2, 3 | Cell 2 |  |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 3 |  |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and Cell 3 |  |  |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |  |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |  |  |
| Measurement gap length | ms | 1, 2, 3 | [6] |  |  |  |
| Measurement gap offset | ms | 1, 2, 3 | [39] |  |  |  |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |  |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |  |  |
| SSB configuration |  | 1 | Cell 1: SSB.1 FR1 Cell 2,3: TBD |  |  |  |
|  |  | 2 | Cell 1: SSB.1 FR1 Cell 2,3: TBD |  |  |  |
|  |  | 3 | Cell 1: SSB.2 FR1 Cell 2,3: TBD |  |  |  |
| SMTC configuration |  | 1 | Cell 1: SMTC.2 Cell 2,3: N/A |  |  |  |
|  |  | 2 | Cell 1: SMTC.1 Cell 2,3: N/A |  |  |  |
|  |  | 3 | Cell 1: SMTC.1 Cell 2,3: N/A |  |  |  |
| DBT window configuration |  | 1, 2, 3 | Cell 1: N/A Cell 2,3: TBD |  |  |  |
| CSI-RS parameters in Cell 1 |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |  |
| Event A3 measurement quantity |  |  | SS-RSRP | SS-RSRQ | SS-SINR |  |
| CP length |  | 1, 2, 3 | Normal |  |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  |  |  | OFF |
| Time offset between Cell 2 and Cell 3 |  | 1 | 3 ms |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | 2 | 3 ms |  |  | Synchronous cells |
|  |  | 3 | 3 ms |  |  | Synchronous cells |
| T1 | s | 1, 2, 3 | TBD |  |  |  |
| T2 | s | 1, 2, 3 | TBD |  |  |  |

Table A.13.3.1.3.2-3: Cell-specific test parameters for intra-frequency event-triggered reporting without gap

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| DL CCA probability PCCA_DL |  | 1, 2, 3 | N/A |  | TBD | TBD | TBD | TBD |
| UL CCA probability PCCA_UL |  | 1, 2, 3 | N/A |  | TBD | TBD | TBD | TBD |
| TDD configuration |  | 1 | N/A |  | TBD |  | TBD |  |
|  |  | 2 | TDDConf.1.1 |  | TBD |  | TBD |  |
|  |  | 3 | TDDConf.2.1 |  | TBD |  | TBD |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | CR.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | CR.2.1 TDD |  | TBD |  | TBD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | CCR.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | CCR.2.1 TDD |  | TBD |  | TBD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | TBD |  | TBD |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | TRS.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | TRS.1.2 TDD |  | TBD |  | TBD |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | DLBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  | TBD |  | TBD |  |
|  |  | 2 | -98 |  | TBD |  | TBD |  |
|  |  | 3 | -95 |  | TBD |  | TBD |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | TBD |  | TBD |  |
|  |  | 2 | -98 |  | TBD |  | TBD |  |
|  |  | 3 | -95 |  | TBD |  | TBD |  |
| Note 5 | dB | 1 | 4 | -1.46 | TBD | TBD | -Infinity | TBD |
|  |  | 2 |  |  | TBD | TBD | -Infinity | TBD |
|  |  | 3 |  |  | TBD | TBD | -Infinity | TBD |
| Note 5 | dB | 1 | 4 | 4 | TBD | TBD | -Infinity | TBD |
|  |  | 2 |  |  | TBD | TBD | -Infinity | TBD |
|  |  | 3 |  |  | TBD | TBD | -Infinity | TBD |
| SS-RSRP Note 3,5 | dBm/SCS kHz | 1 | -94 | -94 | TBD | TBD | -Infinity | TBD |
|  |  | 2 | -94 | -94 | TBD | TBD | -Infinity | TBD |
|  |  | 3 | -91 | -91 | TBD | TBD | -Infinity | TBD |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | TBD | TBD | TBD | TBD |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | TBD | TBD | TBD | TBD |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | TBD | TBD | TBD | TBD |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |  |  |  |  |

##### A.13.3.1.3.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX

##### A.13.3.1.4.1	Test purpose and environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2A.6.1 and 9.2A.6.2.

##### A.13.3.1.4.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), and two cells on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model: SCell (Cell 2) and a neighbour cell (Cell 3). The test parameters for the three cells are given in Table A.13.3.1.4.2-1 and A.13.3.1.4.2-2 below. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information of Cell 3.

FFS: The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

The test is conducted for SS-RSRP, SS-RSRQ, and SS-SINR:

- In Test 1 and Test 2, the UE is configured with SS-RSRP as Event A3 measurement quantity.

- In Test 3 and Test 4, the UE is configured with SS-RSRQ as Event A3 measurement quantity.

- In Test 5 and Test 6, the UE is configured with SS-SINR as Event A3 measurement quantity.

Table A.13.3.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.3.1.4.2-2: General test parameters for intra-frequency event triggered reporting without gap with DRX

| Parameter | Unit | Test configuration | Value |  |  |  |  |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  | Test 2 |  | Test 3 |  | Test 4 |  | Test 5 | Test 6 |  |
| Active PCell |  | 1, 2, 3 | Cell 1 |  |  |  |  |  |  |  |  |  |  |
| Active SCell |  | 1, 2, 3 | Cell 2 |  |  |  |  |  |  |  |  |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 3 |  |  |  |  |  |  |  |  |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and Cell 3 |  |  |  |  |  |  |  |  |  |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |  |  |  |  |  |  |  |  |  |
| Measurement gap repitition periodicity | ms | 1, 2, 3 | 40 |  |  |  |  |  |  |  |  |  |  |
| Measurement gap length | ms | 1, 2, 3 | [6] |  |  |  |  |  |  |  |  |  |  |
| Measurement gap offset | ms | 1, 2, 3 | [39] |  |  |  |  |  |  |  |  |  |  |
| DL CCA model |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |  |  |  |  |  |  |
| UL CCA model |  |  | As specified in clause A.3.20.2.2 |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | 1 | Cell 1: SSB.1 FR1 Cell 2,3: TBD |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | Cell 1: SSB.1 FR1 Cell 2,3: TBD |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | Cell 1: SSB.2 FR1 Cell 2,3: TBD |  |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | 1 | Cell 1: SMTC.2 Cell 2,3: N/A |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | Cell 1: SMTC.1 Cell 2,3: N/A |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | Cell 1: SMTC.1 Cell 2,3: N/A |  |  |  |  |  |  |  |  |  |  |
| DBT window configuration |  | 1, 2, 3 | Cell 1: N/A Cell 2,3: TBD |  |  |  |  |  |  |  |  |  |  |
| CSI-RS parameters in Cell 1 |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |  |  |  |  |  |  |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |  |  |  |  |  |  |  |  |
| Event A3 measurement quantity |  |  | SS-RSRP |  |  | SS-RSRQ |  |  |  | SS-SINR |  |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |  |  |  |  |  |  |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |  |  |  |  |  |  |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |  |  |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  |  |  |  |  |  |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 | DRX.2 |  | DRX.1 |  | DRX.2 |  | DRX.1 |  | DRX.2 |  |
| Time offset between Cell 2 and Cell 3 |  | 1 | 3 ms |  |  |  |  |  |  |  |  |  | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | 2 | 3 ms |  |  |  |  |  |  |  |  |  | Synchronous cells |
|  |  | 3 | 3 ms |  |  |  |  |  |  |  |  |  | Synchronous cells |
| T1 | s | 1, 2, 3 | TBD |  |  |  |  |  |  |  |  |  |  |
| T2 | s | 1, 2, 3 | TBD | TBD |  | TBD |  | TBD |  | TBD |  | TBD |  |

Table A.13.3.1.4.2-3: Cell-specific test parameters for intra-frequency event-triggered reporting without gap

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| DL CCA probability PCCA_DL |  | 1, 2, 3 | N/A |  | TBD | TBD | TBD | TBD |
| UL CCA probability PCCA_UL |  | 1, 2, 3 | N/A |  | TBD | TBD | TBD | TBD |
| TDD configuration |  | 1 | N/A |  | TBD |  | TBD |  |
|  |  | 2 | TDDConf.1.1 |  | TBD |  | TBD |  |
|  |  | 3 | TDDConf.2.1 |  | TBD |  | TBD |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | CR.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | CR.2.1 TDD |  | TBD |  | TBD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | CCR.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | CCR.2.1 TDD |  | TBD |  | TBD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | TBD |  | TBD |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TBD |  | TBD |  |
|  |  | 2 | TRS.1.1 TDD |  | TBD |  | TBD |  |
|  |  | 3 | TRS.1.2 TDD |  | TBD |  | TBD |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | DLBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  | TBD |  | TBD |  |
|  |  | 2 | -98 |  | TBD |  | TBD |  |
|  |  | 3 | -95 |  | TBD |  | TBD |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  | TBD |  | TBD |  |
|  |  | 2 | -98 |  | TBD |  | TBD |  |
|  |  | 3 | -95 |  | TBD |  | TBD |  |
| Note 5 | dB | 1 | 4 | -1.46 | TBD | TBD | -Infinity | TBD |
|  |  | 2 |  |  | TBD | TBD | -Infinity | TBD |
|  |  | 3 |  |  | TBD | TBD | -Infinity | TBD |
| Note 5 | dB | 1 | 4 | 4 | TBD | TBD | -Infinity | TBD |
|  |  | 2 |  |  | TBD | TBD | -Infinity | TBD |
|  |  | 3 |  |  | TBD | TBD | -Infinity | TBD |
| SS-RSRP Note 3,5 | dBm/SCS kHz | 1 | -94 | -94 | TBD | TBD | -Infinity | TBD |
|  |  | 2 | -94 | -94 | TBD | TBD | -Infinity | TBD |
|  |  | 3 | -91 | -91 | TBD | TBD | -Infinity | TBD |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | TBD | TBD | TBD | TBD |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | TBD | TBD | TBD | TBD |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | TBD | TBD | TBD | TBD |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. NOTE 5: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |  |  |  |  |

##### A.13.3.1.4.3	Test Requirements

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1 and Test 2, SS-RSRQ in Test 3 and Test 4, SS-SINR in Test 5 and Test 6), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD (D1 is different for different DRX configurations).

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.13.3.1.5	RSSI measurement reporting on SCC

##### A.13.3.1.5.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the intra-frequency RSSI measurement reporting requirements in Section 9.2A.7.1.

##### A.13.3.1.5.2	Test parameters

In the test, the UE is configured to perform intra-frequency RSSI measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.13.3.1.5.2-1. There are two cells in the test: Cell 1 is PCell on a licensed FR1 band, and Cell 2 is SCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2.

Table A.13.3.1.5.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.3.1.5.2-2: General test parameters.

| Parameter | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  |  | 1 | 2 |
| BWchannel |  | MHz | 40 | 40 |
| DL CCA model |  |  | N/A | PCCA_DL=1.0 |
| UL CCA model |  |  | N/A | PCCA_UL=1.0 |
| Measurement bandwidth |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | TBD |  |
| PDSCH Reference measurement channel defined in TBD |  |  | TBD | TBD |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in TBD |  |  | TBD | TBD |
| OCNG Patterns defined in TBD |  |  | TBD | TBD |
| Other general configuration parameters: TBD |  |  | TBD | TBD |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  | dBm/SCS | TBD | TBD |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  | dBm/SCS | TBD | TBD |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  | dB | TBD | TBD |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  | dB | TBD | TBD |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  | dBm/SCS | TBD | TBD |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | TBD | TBD |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  | dBm/BW | TBD | TBD |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  | dBm/BW | TBD | TBD |
| Propagation condition |  | - | AWGN |  |

#### A.13.3.1.6	Channel occupancy measurement reporting on SCC

##### A.13.3.1.6.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the intra-frequency channel occupancy measurement reporting requirements in Section 9.2A.7.2.

##### A.13.3.1.6.2	Test parameters

In the test, the UE is configured to perform intra-frequency channel occupancy measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.13.3.1.6.2-1. There are two cells in the test: Cell 1 is PCell on a licensed FR1 band, and Cell 2 is SCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2.

Table A.13.3.1.6.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.3.1.6.2-2: General test parameters.

Editor’s note: Table is TBD

### A.13.3.2	Inter-frequency measurements

#### A.13.3.2.1	RSSI measurement reporting

##### A.13.3.2.1.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports RSSI measurements. This test will partly verify the inter-frequency RSSI measurement reporting requirements in Section 9.3A.8.

##### A.13.3.2.1.2	Test parameters

In the test, the UE is configured to perform inter-frequency RSSI measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.13.3.2.1.2-1. There are two cells in the test: Cell 1 is PCell on a licensed FR1 band, and Cell 2 is SCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2. The RSSI measurement is performed on an inter-frequency under CCA.

Table A.13.3.2.1.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.3.2.1.2-2: General test parameters.

Editor’s note: Table TBD

#### A.13.3.2.2	Channel occupancy measurement reporting

##### A.13.3.2.2.1	Test purpose and environment

The purpose of this test is to verify that the UE correctly reports channel occupancy measurements. This test will partly verify the inter-frequency channel occupancy measurement reporting requirements in Section 9.3A.13.

##### A.13.3.2.2.2	Test parameters

In the test, the UE is configured to perform inter-frequency channel occupancy measurements on a carrier frequency under CCA.

Supported test configurations are shown in Table A.13.3.2.2.2-1. There are two cells in the test: Cell 1 is PCell on a licensed FR1 band, and Cell 2 is SCell operating on a carrier frequency under CCA. Prior to the start of the time duration T1, the UE is connected to Cell 1 and Cell 2. The channel occupancy measurement is performed on an inter-frequency under CCA.

Table A.13.3.2.2.2-1: Supported test configurations.

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.3.2.2.2-2: General test parameters.

Editor’s note: Table is TBD

#### A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used

##### A.13.3.2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements for NR cell with CCA in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR1 with CCA on NR RF channel 2 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 3.  The test parameters are given in Tables A.13.3.2.3.1-1, A.13.3.2.3.1-2 and A.13.3.2.3.1-3.

In test 1, measurement gap pattern configuration # 0 as defined in Table A.13.3.2.3.1-2 is provided for UE that does not support per-FR gap. In test 2, measurement gap pattern configuration #4 as defined in Table A.13.3.2.3.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.13.3.2.3.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, NR cell without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.13.3.2.3.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. Channels 2 and 3 are with CCA. |
| Active cells |  | Config 1,2,3 | NR cell 1 (PCell), NR cell 2 with CCA (SCell) |  | NR cell 1 is on NR RF channel number 1. NR cell 2 is on NR RF channel number 2 with CCA. |
| Neighbour cell |  | Config 1,2,3 | NR cell 3 with CCA |  | NR cell 3 is on NR RF channel number 3 with CCA. |
| DL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.7 | 1.7 |  |

Table A.13.3.2.3.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  |  | Config 1 | FDD |  | TDD |  | TDD |  |
|  |  |  |  | Config 2,3 | TDD |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 1 | Not Applicable |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
|  |  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
|  |  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_UL=1 |  | PCCA_UL=1 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  |  |  | Config 1,2,3 | Not Applicable |  | 12 |  | 12 |  |
| WCCA_DL |  |  | ms | Config 1,2,3 | Not Applicable |  | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| BWchannel |  |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP configuration | Initial DL BWP |  |  | Config 1,2,3 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  |  |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  |  |  |
| TRS configuration |  |  |  | Config 1 | TRS.1.1 FDD |  | TRS.1.2 TDD |  |  |  |
|  |  |  |  | Config 2 | TRS.1.1 TDD |  | TRS.1.2 TDD |  |  |  |
|  |  |  |  | Config 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 | SR.1.1 FDD |  | SR.1.1 CCA |  |  |  |
|  |  |  |  | Config 2 | SR.1.1 TDD |  | SR.1.1 CCA |  |  |  |
|  |  |  |  | Config 3 | SR2.1 TDD |  | SR.1.1 CCA |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1 | CR.1.1 FDD |  | CR.1.1 CCA |  |  |  |
|  |  |  |  | Config 2 | CR.1.1 TDD |  | CR.1.1 CCA |  |  |  |
|  |  |  |  | Config 3 | CR2.1 TDD |  | CR.1.1 CCA |  |  |  |
| SSB |  | Semi- |  | Config 1 | SSB.1 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
| parameters |  | static channel Note 5,7 |  | Config 2 | SSB.1 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
|  |  |  |  | Config 3 | SSB.2 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
|  |  | Dynamic |  | Config 1 | SSB.1 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
|  |  | channel |  | Config 2 | SSB.1 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
|  |  | Access Note 6,7 |  | Config 3 | SSB.2 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
| DBT window configuration |  |  |  | Config 1,2,3 | Not Applicable |  | As defined in A.3.28.1 |  | As defined in A.3.28.1 |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1,2,3 | SMTC.1 |  | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 15 |  | 30 |  | 30 |  |
|  |  |  |  | Config 2 | 15 |  | 30 |  | 30 |  |
|  |  |  |  | Config 3 | 30 |  | 30 |  | 30 |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1,2,3 | -98 |  | -104 |  | -104 |  |
| Note2 |  |  | dBm/SCS | Config 1,2 | -98 |  | -101 |  | -101 |  |
|  |  |  |  | Config 3 | -95 |  | -101 |  | -101 |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1,2 | -94 | -94 | -91 | -91 | -Infinity | -88 |
|  |  |  |  | Config 3 | -91 | -91 | -91 | -91 | -Infinity | -88 |
|  |  |  | dB | Config 1,2 | 4 | 4 | 4 | 4 | -Infinity | 7 |
|  |  |  | dB | Config 1,2 | 4 | 4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -58.49 | -58.49 | -63.94 | -56.15 |
|  |  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |

##### A.13.3.2.3.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used

##### A.13.3.2.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR1 with CCA on NR RF channel 2 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 3.  The test parameters are given in Tables A.13.3.2.4.1-1, A.13.3.2.4.1-2 and A.13.3.2.4.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.13.3.2.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.13.3.2.4.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.13.3.2.4.1-1: SA event triggered reporting tests without SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, NR cell without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.13.3.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  |  |  | Three FR1 NR carrier frequencies are used. Channels 2 and 3 are with CCA. |
| Active cells |  | Config 1,2,3 | NR cell 1 (PCell), NR cell 2 with CCA (SCell) |  |  |  | NR cell 1 is on NR RF channel number 1. NR cell 2 is on NR RF channel number 2 with CCA. |
| Neighbour cell |  | Config 1,2,3 | NR cell 3 with CCA |  |  |  | NR cell 3 is on NR RF channel number 3 with CCA. |
| DL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 2.5 | 17 | 2.5 | 17 |  |

Table A.13.3.2.4.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA without SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  |  | Config 1 | FDD |  | TDD |  | TDD |  |
|  |  |  |  | Config 2,3 | TDD |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 1 | Not Applicable |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
|  |  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
|  |  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_UL=1 |  | PCCA_UL=1 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  |  |  | Config 1,2,3 | Not Applicable |  | 5 |  | 5 |  |
| WCCA_DL |  |  | ms | Config 1,2,3 | Not Applicable |  | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| BWchannel |  |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP configuration | Initial DL BWP |  |  | Config 1,2,3 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  |  |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  |  |  |
| TRS configuration |  |  |  | Config 1 | TRS.1.1 FDD |  | TRS.1.2 TDD |  |  |  |
|  |  |  |  | Config 2 | TRS.1.1 TDD |  | TRS.1.2 TDD |  |  |  |
|  |  |  |  | Config 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 | SR.1.1 FDD |  | SR.1.1 CCA |  |  |  |
|  |  |  |  | Config 2 | SR.1.1 TDD |  | SR.1.1 CCA |  |  |  |
|  |  |  |  | Config 3 | SR2.1 TDD |  | SR.1.1 CCA |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1 | CR.1.1 FDD |  | CR.1.1 CCA |  |  |  |
|  |  |  |  | Config 2 | CR.1.1 TDD |  | CR.1.1 CCA |  |  |  |
|  |  |  |  | Config 3 | CR2.1 TDD |  | CR.1.1 CCA |  |  |  |
| SSB |  | Semi- |  | Config 1 | SSB.1 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
| parameters |  | static channel Note 5,7 |  | Config 2 | SSB.1 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
|  |  |  |  | Config 3 | SSB.2 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
|  |  | Dynamic |  | Config 1 | SSB.1 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
|  |  | channel |  | Config 2 | SSB.1 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
|  |  | Access Note 6,7 |  | Config 3 | SSB.2 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
| DBT window configuration |  |  |  | Config 1,2,3 | Not Applicable |  | As defined in A.3.28.1 |  | As defined in A.3.28.1 |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1,2,3 | SMTC.1 |  | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 15 |  | 30 |  | 30 |  |
|  |  |  |  | Config 2 | 15 |  | 30 |  | 30 |  |
|  |  |  |  | Config 3 | 30 |  | 30 |  | 30 |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1,2,3 | -98 |  | -104 |  | -104 |  |
| Note2 |  |  | dBm/SCS | Config 1,2 | -98 |  | -101 |  | -101 |  |
|  |  |  |  | Config 3 | -95 |  | -101 |  | -101 |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1,2 | -94 | -94 | -91 | -91 | -Infinity | -88 |
|  |  |  |  | Config 3 | -91 | -91 | -91 | -91 | -Infinity | -88 |
|  |  |  | dB | Config 1,2 | 4 | 4 | 4 | 4 | -Infinity | 7 |
|  |  |  | dB | Config 1,2 | 4 | 4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -58.49 | -58.49 | -63.94 | -56.15 |
|  |  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |

Table A.13.3.2.4.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.13.3.2.4.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.13.3.2.4.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_without_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

Tidentify_inter_cca_without_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used

##### A.13.3.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR1 with CCA on NR RF channel 2 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 3.   The test parameters are given in Tables A.13.3.2.5.1-1, A.13.3.2.5.1-2 and A.13.3.2.5.1-3.

In test 1 measurement gap pattern configuration # 0 as defined in Table A.13.3.2.5.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table A.13.3.2.5.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 2. Otherwise it is only required to pass test 1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

Table A.13.3.2.5.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, NR cell without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.13.3.2.5.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. Channels 2 and 3 are with CCA. |
| Active cells |  | Config 1,2,3 | NR cell 1 (PCell), NR cell 2 with CCA (SCell) |  | NR cell 1 is on NR RF channel number 1. NR cell 2 is on NR RF channel number 2 with CCA. |
| Neighbour cell |  | Config 1,2,3 | NR cell 3 with CCA |  | NR cell 3 is on NR RF channel number 3 with CCA. |
| DL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.1 |  |  |
| UL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.2 |  |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 2 | 2 |  |

Table A.13.3.2.5.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  |  | Config 1 | FDD |  | TDD |  | TDD |  |
|  |  |  |  | Config 2,3 | TDD |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 1 | Not Applicable |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
|  |  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
|  |  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_UL=1 |  | PCCA_UL=1 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  |  |  | Config 1,2,3 | Not Applicable |  | 5 |  | 5 |  |
| WCCA_DL |  |  | ms | Config 1,2,3 | Not Applicable |  | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| BWchannel |  |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP configuration | Initial DL BWP |  |  | Config 1,2,3 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  |  |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  |  |  |
| TRS configuration |  |  |  | Config 1 | TRS.1.1 FDD |  | TRS.1.2 TDD |  |  |  |
|  |  |  |  | Config 2 | TRS.1.1 TDD |  | TRS.1.2 TDD |  |  |  |
|  |  |  |  | Config 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 | SR.1.1 FDD |  | SR.1.1 CCA |  |  |  |
|  |  |  |  | Config 2 | SR.1.1 TDD |  | SR.1.1 CCA |  |  |  |
|  |  |  |  | Config 3 | SR2.1 TDD |  | SR.1.1 CCA |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1 | CR.1.1 FDD |  | CR.1.1 CCA |  |  |  |
|  |  |  |  | Config 2 | CR.1.1 TDD |  | CR.1.1 CCA |  |  |  |
|  |  |  |  | Config 3 | CR2.1 TDD |  | CR.1.1 CCA |  |  |  |
| SSB |  | Semi- |  | Config 1 | SSB.1 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
| parameters |  | static channel Note 5,7 |  | Config 2 | SSB.1 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
|  |  |  |  | Config 3 | SSB.2 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
|  |  | Dynamic |  | Config 1 | SSB.1 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
|  |  | channel |  | Config 2 | SSB.1 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
|  |  | Access Note 6,7 |  | Config 3 | SSB.2 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
| DBT window configuration |  |  |  | Config 1,2,3 | Not Applicable |  | As defined in A.3.28.1 |  | As defined in A.3.28.1 |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1,2,3 | SMTC.1 |  | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 15 |  | 30 |  | 30 |  |
|  |  |  |  | Config 2 | 15 |  | 30 |  | 30 |  |
|  |  |  |  | Config 3 | 30 |  | 30 |  | 30 |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1,2,3 | -98 |  | -104 |  | -104 |  |
| Note2 |  |  | dBm/SCS | Config 1,2 | -98 |  | -101 |  | -101 |  |
|  |  |  |  | Config 3 | -95 |  | -101 |  | -101 |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1,2 | -94 | -94 | -91 | -91 | -Infinity | -88 |
|  |  |  |  | Config 3 | -91 | -91 | -91 | -91 | -Infinity | -88 |
|  |  |  | dB | Config 1,2 | 4 | 4 | 4 | 4 | -Infinity | 7 |
|  |  |  | dB | Config 1,2 | 4 | 4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -58.49 | -58.49 | -63.94 | -56.15 |
|  |  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration |  |  |  |  |  |  |  |  |  |  |

##### A.13.3.2.5.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and 2 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For test 1, MGRP = 40 ms and for test 2 MGRP = 20 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used

##### A.13.3.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3A.4 and 9.3A.5.

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR1 with CCA on NR RF channel 2 and NR cell 3 as neighbour cell in FR1 with CCA on NR RF channel 3.  The test parameters are given in Tables A.13.3.2.6.1-1, A.13.3.2.6.1-2 and A.13.3.2.6.1-3.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table A.13.3.2.6.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table A.13.3.2.6.1-2 is provided for UE that supports per-FR gap. If a UE supports per-FR gap and gap pattern configuration #4, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

Table A.13.3.2.6.1-1: SA event triggered reporting tests with SSB index reading for FR1-FR1 with CCA

| Config | Description |
| --- | --- |
| 1 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR cell without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR cell with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, NR cell without CCA: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.13.3.2.6.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  |  |  | Three FR1 NR carrier frequencies are used. Channels 2 and 3 are with CCA. |
| Active cells |  | Config 1,2,3 | NR cell 1 (PCell), NR cell 2 with CCA (SCell) |  |  |  | NR cell 1 is on NR RF channel number 1. NR cell 2 is on NR RF channel number 2 with CCA. |
| Neighbour cell |  | Config 1,2,3 | NR cell 3 with CCA |  |  |  | NR cell 3 is on NR RF channel number 3 with CCA. |
| DL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.1 |  |  |  |  |
| UL CCA model |  | Config 1,2,3 | As specified in clause A.3.26.2.2 |  |  |  |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 3 | 20 | 3 | 20 |  |

Table A.13.3.2.6.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with CCA with SSB time index detection

| Parameter |  |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  |  | Config 1 | FDD |  | TDD |  | TDD |  |
|  |  |  |  | Config 2,3 | TDD |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 1 | Not Applicable |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
|  |  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
|  |  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.1.1 CCA |  | TDDConf.1.1 CCA |  |
| DL CCA probability PCCA_DL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_DL=0.9375 |  | PCCA_DL=0.9375 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| UL CCA probability PCCA_UL |  | Semi-static channel access Note 5,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_UL=1 |  | PCCA_UL=1 |  |
|  |  | Dynamic channel access Note 6,7 |  | Config 1,2,3 | Not Applicable |  | PCCA_UL=1 |  | PCCA_UL=1 |  |
| LCCA_DL |  |  |  | Config 1,2,3 | Not Applicable |  | 2 |  | 2 |  |
| WCCA_DL |  |  | ms | Config 1,2,3 | Not Applicable |  | TPSS/SSS_sync_inter_cca |  | TPSS/SSS_sync_inter_cca |  |
| BWchannel |  |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  |  | MHz | Config 1,2 | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
|  |  |  |  | Config 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP configuration | Initial DL BWP |  |  | Config 1,2,3 | DLBWP.0.1 |  | DLBWP.0.1 |  |  |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  |  |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  |  |  |
| TRS configuration |  |  |  | Config 1 | TRS.1.1 FDD |  | TRS.1.2 TDD |  |  |  |
|  |  |  |  | Config 2 | TRS.1.1 TDD |  | TRS.1.2 TDD |  |  |  |
|  |  |  |  | Config 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  |  | Config 1 | SR.1.1 FDD |  | SR.1.1 CCA |  |  |  |
|  |  |  |  | Config 2 | SR.1.1 TDD |  | SR.1.1 CCA |  |  |  |
|  |  |  |  | Config 3 | SR2.1 TDD |  | SR.1.1 CCA |  |  |  |
| CORESET Reference Channel |  |  |  | Config 1 | CR.1.1 FDD |  | CR.1.1 CCA |  |  |  |
|  |  |  |  | Config 2 | CR.1.1 TDD |  | CR.1.1 CCA |  |  |  |
|  |  |  |  | Config 3 | CR2.1 TDD |  | CR.1.1 CCA |  |  |  |
| SSB |  | Semi- |  | Config 1 | SSB.1 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
| parameters |  | static channel Note 5,7 |  | Config 2 | SSB.1 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
|  |  |  |  | Config 3 | SSB.2 FR1 |  | SSB.1 CCA |  | SSB.1 CCA |  |
|  |  | Dynamic |  | Config 1 | SSB.1 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
|  |  | channel |  | Config 2 | SSB.1 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
|  |  | Access Note 6,7 |  | Config 3 | SSB.2 FR1 |  | SSB.2 CCA |  | SSB.2 CCA |  |
| DBT window configuration |  |  |  | Config 1,2,3 | Not Applicable |  | As defined in A.3.28.1 |  | As defined in A.3.28.1 |  |
| SMTC configuration defined in A.3.11 |  |  |  | Config 1,2,3 | SMTC.1 |  | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 15 |  | 30 |  | 30 |  |
|  |  |  |  | Config 2 | 15 |  | 30 |  | 30 |  |
|  |  |  |  | Config 3 | 30 |  | 30 |  | 30 |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | Config 1,2,3 | -98 |  | -104 |  | -104 |  |
| Note2 |  |  | dBm/SCS | Config 1,2 | -98 |  | -101 |  | -101 |  |
|  |  |  |  | Config 3 | -95 |  | -101 |  | -101 |  |
| SS-RSRP Note 3 |  |  | dBm/SCS | Config 1,2 | -94 | -94 | -91 | -91 | -Infinity | -88 |
|  |  |  |  | Config 3 | -91 | -91 | -91 | -91 | -Infinity | -88 |
|  |  |  | dB | Config 1,2 | 4 | 4 | 4 | 4 | -Infinity | 7 |
|  |  |  | dB | Config 1,2 | 4 | 4 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -58.49 | -58.49 | -63.94 | -56.15 |
|  |  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 6: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 7: For UE supporting both semi-static and dynamic channel access, the UE must be tested under dynamic channel access configuration. |  |  |  |  |  |  |  |  |  |  |

Table A.13.3.2.6.1-4: DRX-Configuration for SA inter-frequency event triggered reporting without SSB time index detection

| Field | Test1&3 | Test2&4 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| drx-onDurationTimer | ms1 | ms1 | As specified in clause 6.3.2 in TS 38.331 [2] |
| drx-InactivityTimer | ms1 | ms1 |  |
| drx-RetransmissionTimerDL | sl1 | sl1 |  |
| drx-RetransmissionTimerUL | sl1 | sl1 |  |
| drx-LongCycleStartOffset | ms40 | Ms640 |  |
| shortDRX | disable | disable |  |

Table A.13.3.2.6.1-5: TimeAlignmentTimer -Configuration SA inter-frequency event triggered reporting without SSB time index detection

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | ms500 | As specified in clause 6.3.2 in TS 38.331 [2] |

##### A.13.3.2.6.2	Test Requirements

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_inter_cca_with_index from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

Tidentify_inter_cca_with_index = (TPSS/SSS_sync_inter_cca + T SSB_measurement_period_inter_cca + TSSB_time_index_inter_cca) ms, where

TPSS/SSS_sync_inter_cca: it is the time period used in PSS/SSS detection given in table 9.3A.4-1.

TSSB_time_index_inter_cca: it is the time period used to acquire the index of the SSB being measured given in table 9.3A.4-2.

T SSB_measurement_period_inter_cca: equal to a measurement period of SSB based measurement given in table 9.3A.5-1.

For tests 1 and 2, MGRP = 40 ms and for tests 3 and 4 MGRP = 20 ms.

For tests 1 and 3, DRX cycle = 40 ms and for tests 2 and 4 DRX cycle = 640 ms.

SMTC period = 20 ms.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.13.3.3	L1-RSRP measurements for beam reporting

#### A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used

##### A.13.3.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.13.3.3.1.1-1.

Table A.13.3.3.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.13.3.3.1.2	Test parameters

There are two cells in the tests, FR1 PCell (Cell 1) and FR1 SCell (Cell 2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model. The test parameters and applicability for Cell 1 and Cell 2 are given in Table A.13.3.3.1.2-1 and Table A.13.3.3.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.13.3.3.1.2-1: General test parameters

| Parameter | Config | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| Active PCell/SCell Configuration |  |  | PCell | SCell |
| RF Channel Number |  |  | 1 | 2 |
| DL CCA model | 1~3 |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model | 1~3 |  | N/A | As specified in A.3.20.2.2 |
| Duplex mode | 1 |  | FDD | TDD |
|  | 2 |  | TDD |  |
|  | 3 |  | TDD |  |
| TDD Configuration | 1 |  | N/A | TDDConf.1.1 CCA |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| BWchannel | 1 | MHz | 10: NRB,c = 52 | 40: NRB,c = 106 |
|  | 2 |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | SR.1.1 CCA |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | CR.1.1 CCA |
|  | 2 |  | CR.1.1 TDD |  |
|  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | CCR.1.1 CCA |
|  | 2 |  | CCR.1.1 TDD |  |
|  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration | 1 |  | SSB.3 FR1 | SSB.3 CCA for semi-static channel access |
|  | 2 |  | SSB.3 FR1 | SSB.4 CCA for dynamic channel access |
|  | 3 |  | SSB.4 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 | N/A |
| DBT Window Configuration | 1~3 |  | N/A | DBT.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD | TRS.1.2 TDD |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| DRX configuration | 1~3 |  | Off | Off |
| reportConfigType | 1~3 |  | periodic | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 | 80 |
| T1 | 1~3 | s | 5 | 5 |
| T2 | 1~3 | s | 1 | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1~3 |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |  |

Table A.13.3.3.1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1,2,3 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1,2,3 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1,2,3 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1,2,3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2,3 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1,2,3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2,3 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2,3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1,2,3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS Res when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.13.3.3.1.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

The UE shall send L1-RSRP report of both SSB0 and SSB1 in Cell 2.

NOTE: The actual overall delays measured in the test may be up to 2xTTI DCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used

##### A.13.3.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5A.4.1, with the testing configurations for NR cells in Table A.13.3.3.1.1-1.

Table A.13.3.3.2.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.13.3.3.2.2	Test parameters

There are two cells in the tests, FR1 Pcell (Cell 1) and FR1 Scell (Cell 2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model. The test parameters and applicability for Cell 1  and Cell 2 are given in Table A.13.3.3.2.2-1 and Table A.13.3.3.2.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs and report periodically. The UE transmits the reporting according to UL CCA model. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.13.3.3.2.2-1: General test parameters

| Parameter | Config | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| Active Pcell/Scell Configuration |  |  | Pcell | Scell |
| RF Channel Number |  |  | 1 | 2 |
| DL CCA model | 1~3 |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model | 1~3 |  | N/A | As specified in A.3.20.2.2 |
| Duplex mode | 1 |  | FDD | TDD |
|  | 2 |  | TDD |  |
|  | 3 |  | TDD |  |
| TDD Configuration | 1 |  | N/A | TDDConf.1.1 CCA |
|  | 2 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  |
| BWchannel | 1 | MHz | 10: NRB,c = 52 | 40: NRB,c = 106 |
|  | 2 |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | SR.1.1 CCA |
|  | 2 |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | CR.1.1 CCA |
|  | 2 |  | CR.1.1 TDD |  |
|  | 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | CCR.1.1 CCA |
|  | 2 |  | CCR.1.1 TDD |  |
|  | 3 |  | CCR.2.1 TDD |  |
| SSB configuration | 1 |  | SSB.3 FR1 | SSB.3 CCA for semi-static channel access |
|  | 2 |  | SSB.3 FR1 | SSB.4 CCA for dynamic channel access |
|  | 3 |  | SSB.4 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 | N/A |
| DBT Window Configuration | 1~3 |  | N/A | DBT.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD | TRS.1.2 TDD |
|  | 2 |  | TRS.1.1 TDD |  |
|  | 3 |  | TRS.1.2 TDD |  |
| DRX configuration | 1~3 |  | DRX.3 | DRX.3 |
| reportConfigType | 1~3 |  | periodic | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 | 80 |
| T1 | 1~3 | s | 5 | 5 |
| T2 | 1~3 | s | 1 | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1~3 |  | AWGN | AWGN |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. |  |  |  |  |

Table A.13.3.3.2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| DL CCA Probability PCCA_DL Note 4,6 | 1,2,3 |  | [0.9375] | [0.9375] | [0.9375] | [0.9375] |
| DL CCA Probability PCCA_DL Note 4.7 | 1,2,3 |  | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] | [0.75]/[0.75] |
| UL CCA probability PCCA_UL | 1,2,3 |  | [1.0] | [1.0] | [1.0] | [1.0] |
| Note2 | 1,2,3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2,3 | dBm/SSB SCS | -91.65 |  |  |  |
|  | 1,2,3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2,3 | dBm/SSB SCS | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2,3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1,2,3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: DL and UL CCA probabilities apply for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy. Note 5: The signal levels apply for SSS Res when the discovery burst is transmitted during DBT windows. Note 6: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 7: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. The first value corresponds PCCA_DL1 and the second value corresponds to the PCCA_DL2. |  |  |  |  |  |  |

##### A.13.3.3.2.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 640 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2. The rate of correct events observed during repeated tests shall be at least 90%.

The UE shall send L1-RSRP report of both SSB0 and SSB1 in Cell 2.

NOTE: The actual overall delays measured in the test may be up to 2xTTI DCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.13.4	Measurement performance

### A.13.4.1	SS-RSRP

#### A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA

##### A.13.4.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy on the carrier frequency with CCA is within the specified limits. This test will verify the requirements in clauses 10.1.36.1.1 and 10.1.36.1.2 for intra-frequency measurements under CCA.

##### A.13.4.1.1.2	Test parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), and two cells on the same carrier frequency with CCA and transmit SSBs in DBT windows according to DL CCA model: SCell (Cell 2) and a neighbour cell (Cell 3). Supported test configurations are shown in table A.13.4.1.1.2-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in A.13.4.1.1.2-2.

Table A.13.4.1.1.2-1: SS-RSRP  Intra frequency SS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR carrier with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR carrier without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR carrier with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR carrier without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR carrier with CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR carrier without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

Table A.13.4.1.1.2-2: SS-RSRP Intra frequency test parameters

| Parameter |  |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Cell 2 | Cell 3 |  | Cell 2 |  | Cell 3 |  |  |  | Cell 2 |  |  | Cell 3 |  |  |
| Cell ID |  |  |  |  |  |  | 489 | 0 |  | 489 |  | 0 |  |  |  | 489 |  | 0 |  |  |  |
| SSB ARFCN |  |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |  |  | freq1 |  |  |  |  |  |
| DL CCA model |  |  |  |  |  |  | As specified in clause A.3.20.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UL CCA model |  |  |  |  |  |  | As specified in clause A.3.20.2.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCA_DL |  |  |  |  |  |  | [TBD] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCA_UL |  |  |  |  |  |  | [TBD] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  |  | Config 1,2,3 |  |  | TDDConf.1.1 CCA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  |  | Config 1,2,3 |  | MHz | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  |  | Config 1,2,3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CCA model |  |  |  | Config 1,2,3 |  |  | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  | Config 1,2,3 |  | TRS.1.2 TDD |  | NA |  | TRS.1.2 TDD |  |  | NA | TRS.1.2 TDD |  |  |  |  |  | NA |
| DRX Cycle |  |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | Config 1,2,3 |  |  | SR.1.1 CCA |  | - | SR.1.1 CCA |  | - |  |  |  | SR.1.1 CCA |  |  |  |  | - |
| RMSI CORESET Reference Channel |  |  |  | Config 1,2,3 |  |  | CR.1.1 CCA |  | - | CR.1.1 CCA |  | - |  |  |  | CR.1.1 CCA |  |  |  |  | - |
| Control channel RMC |  |  |  | Config 1,2,3 |  |  | CR.1.1 CCA |  | - | CR.1.1 CCA |  | - |  |  |  | CR.1.1 CCA |  |  |  |  | - |
| SSB configuration for semi-static channel access |  |  |  | Config 1,2,3 |  |  | SSB.1 CCA |  | SSB.1 CCA | SSB.1 CCA |  | SSB.1 CCA |  |  |  | SSB.1 CCA |  |  |  |  | SSB.1 CCA |
| SSB configuration for dynamic channel access |  |  |  | Config 1,2,3 |  |  | SSB.2 CCA |  | SSB.2 CCA | SSB.2 CCA |  | SSB.2 CCA |  |  |  | SSB.2 CCA |  |  |  |  | SSB.2 CCA |
| DBT window configuration |  |  |  | Config 1,2,3 |  |  | DBT.1 |  | DBT.1 | DBT.1 |  | DBT.1 |  |  |  | DBT.1 |  |  |  |  | DBT.1 |
| Time offset with Cell 1 |  | Config 1,2,3 |  |  |  | s | - |  | 3 | - |  | 3 |  |  |  | - |  |  |  | 3 |  |
| SMTC configuration |  | Config 1,2,3 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1,2,3 |  | kHz | 30 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  | dB | 0 |  | 0 | 0 |  |  | 0 |  |  |  | 0 |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,3 |  |  |  | NR_TDD_FR1_I |  | Not applicableNote 5 |  |  | -94 |  |  |  |  |  |  | TBD |  |  |  |  |
| Note2 | Config 1,2,3 | NR_TDD_FR1_I |  |  |  | dBm/SCS | Not applicableNote 5 |  |  | -91 |  |  |  |  |  |  | TBD |  |  |  |  |
| Note6 |  |  |  |  |  | dB | 2.46 |  | -5.97 | 2.46 |  |  | -5.97 |  |  |  | TBD |  |  |  | TBD |
| Note6 |  |  |  |  |  | dB | 6 |  | 1 | 6 |  |  | 1 |  |  |  | TBD |  |  |  | TBD |
| SS-RSRPNote3,6 | Config 1,2,3 |  | NR_TDD_FR1_I |  |  | dBm/SCS | Not applicableNote 5 |  | Not applicableNote 5 | -85 |  |  | -90 |  |  |  | TBD |  |  |  | TBD |
| IoNote3 | Config 1,2,3 |  |  | NR_TDD_FR1_I |  | dBm/ 38.16MHz | Not applicableNote 5- |  |  | -51.99 |  |  |  |  |  |  | TBD |  |  |  |  |
| Propagation condition |  |  |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: Subtest 1 is not used when testing with 30kHz SSB SCS. NOTE 6: The signal levels apply for SSS REs when the discovery burst is transmitted during DBT windows. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.13.4.1.1.3	Test Requirements

The SS-RSRP measurement accuracy for cell 2 and cell 3 shall fulfil absolute requirement in clause 10.1.36.1.1 and relative requirement in clause 10.1.36.1.2.

### A.13.4.2	SS-RSRQ

### A.13.4.3	SS-SINR

### A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell

#### A.13.4.4.1	SSB based L1-RSRP measurement

##### A.13.4.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.1.33.1 for L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.13.4.4.1.1-1.

Table A.13.4.4.1.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.13.4.4.1.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a SCell under CCA (Cell2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model.

Two sub-tests (Test 1 and Test 2) are provided with different Noc  on Cell 2. The test parameters for the Cell 1 and Cell 2 are given in Table A.13.4.4.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.13.4.4.1.2-1.

The same test is applicable for UE supporting any one or both semi-static channel access or dynamic channel access and for network configuring any of semi-static channel occupancy or dynamic channel occupancy.

There is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources. On Cell 2, UE is configured to perform L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.13.4.4.1.2-1: FR1 SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |  |
| Active PCell/SCell Configuration |  | 1~3 |  | PCell | SCell |  | PCell | SCell |  |
| SSB GSCN |  | 1~3 |  | freq1 | freq2 |  | freq1 | freq2 |  |
| DL CCA model |  | 1~3 |  | N/A | As specifieed in A.3.20.2.1 |  | N/A | As specifieed in A.3.20.2.1 |  |
| UL CCA model |  | 1~3 |  | N/A | As specified in A.3.20.2.2 |  | N/A | As specified in A.3.20.2.2 |  |
| Duplex mode |  | 1 |  | FDD | TDD |  | FDD | TDD |  |
|  |  | 2,3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A | TDDConf.1.1 CCA |  | N/A | TDDConf.1.1 CCA |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 40: NRB,c = 106 |  | 10: NRB,c = 52 | 40: NRB,c = 106 |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 CCA |  | SR.1.1 FDD | SR.1.1 CCA |  |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 CCA |  | CR.1.1 FDD | CR.1.1 CCA |  |
|  |  | 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 CCA |  | CCR.1.1 FDD | CCR.1.1 CCA |  |
|  |  | 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| SSB configuration for Semi-static channel access |  | 1 |  | SSB.3 FR1 | SSB.3 CCA |  | SSB.3 FR1 | SSB.3 CCA |  |
|  |  | 2 |  | SSB.3 FR1 |  |  | SSB.3 FR1 |  |  |
|  |  | 3 |  | SSB.4 FR1 |  |  | SSB.4 FR1 |  |  |
| SSB configuration for Dynamic channel access |  | 1 |  | SSB.3 FR1 | SSB.4 CCA |  | SSB.3 FR1 | SSB.4 CCA |  |
|  |  | 2 |  | SSB.3 FR1 |  |  | SSB.3 FR1 |  |  |
|  |  | 3 |  | SSB.4 FR1 |  |  | SSB.4 FR1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.2 TDD |  | TRS.1.1 FDD | TRS.1.2 TDD |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| SMTC configuration |  | 1~3 |  | SMTC.1 |  | N/A | SMTC.1 |  | N/A |
| DBT Window Configuration |  | 1~3 |  | N/A |  | DBT.1 | N/A |  | DBT.1 |
| reportConfigType |  | 1~3 |  | periodic |  |  | periodic |  |  |
| reportQuantity |  | 1~3 |  | ssb-Index-RSRP |  |  | ssb-Index-RSRP |  |  |
| Number of reported RS |  | 1~3 |  | 2 |  |  | 2 |  |  |
| L1-RSRP reporting period |  | 1~3 |  | slot80 |  |  | slot80 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  |  | -94.65 | - |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_I |  |  |  |  |  | - | -113 |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/SSB SCS | -91.65 |  |  | -91.65 | - |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_I |  |  |  |  |  | - | -110 |  |
|  |  | 1~3 | dB | 10 |  |  | 10 | -3 |  |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/SCS | -81.65 |  |  | -81.65 | - |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_I |  |  |  |  |  | - | -113 |  |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/ 38.16MHz | -50.19 |  |  | -50.19 | - |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | - |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | - |  |
|  | NR_TDD_FR1_I |  |  |  |  |  | - | -77.19 |  |
|  |  | 1~3 | dB | 10 |  |  | 10 | -3 |  |
| Propagation condition |  | 1~3 |  | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

##### A.13.4.4.1.3	Test Requirements

In both Test 1 and Test 2, the L1-RSRP measurement accuracy for SSB#0 and SSB#1 of Cell 2 shall fulfil the requirements in clauses 10.1.33.1.

### A.13.4.5	RSSI

### A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA

#### A.13.4.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the RSSI measurement accuracy is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in Section 10.1.34.1.

#### A.13.4.5.1.2	Test parameters

In all test cases, Cell 1 is the PCell on a licensed FR1 band and Cell 2 is the SCell with CCA. RSSI is measured on channel number 2. Supported test configurations are shown in table A.13.4.5.1.2-1. The accuracy of RSSI intra-frequency measurements is tested by using the parameters in A.13.4.5.1.2-2 and A.13.4.5.1.2-3.

Table A.13.4.5.1.2-1: Intra frequency RSSI supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.4.5.1.2-2: RSSI Intra frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2,3 |  | Configuration 1,2: SSB.1 FR1 Configuration 3: SSB.2 FR1 | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2,3 |  | Configuration 1,2: SSB.1 FR1 Configuration 3: SSB.2 FR1 | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 CCA |
|  |  | 2,3 |  | SR.1.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 CCA |
|  |  | 2,3 |  | CR.1.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 CCA |
|  |  | 2,3 |  | CCR.1.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.13.4.5.1.2-3: RSSI RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.13.4.5.1.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 10.1.34.1. The nominal RSSI used to evaluate the requirement shall be based on Io in slots corresponding to RSSI measurement time configuration (RMTC).

### A.13.4.5.2			Inter-frequency RSSI measurement accuracy on a carrier with CCA

#### A.13.4.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the RSSI measurement accuracy is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in Section 10.1.34.2.

#### A.13.4.5.2.2	Test parameters

In all test cases, Cell 1 is the PCell on a licensed FR1 band and Cell 2 is the neighbour with CCA. RSSI is measured on channel number 2. Supported test configurations are shown in table A.13.4.5.2.2-1. The accuracy of RSSI inter-frequency measurements is tested by using the parameters in A.13.4.5.2.2-2 and A.13.4.5.2.3.

Table A.13.4.5.2.2-1: Inter frequency RSSI supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.4.5.2.2-2: RSSI Inter frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2,3 |  | Configuration 1,2: SSB.1 FR1 Configuration 3: SSB.2 FR1 | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2,3 |  | Configuration 1,2: SSB.1 FR1 Configuration 3: SSB.2 FR1 | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | NA |
|  |  | 2,3 |  | SR.1.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | NA |
|  |  | 2,3 |  | CR.1.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | NA |
|  |  | 2,3 |  | CCR.1.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 | NA |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | NA |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.13.4.5.2.2-3: RSSI RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.13.4.5.2.3	Test Requirements

The average RSSI measurement accuracy shall fulfil the requirements in sections 10.1.34.2. The nominal RSSI used to evaluate the requirement shall be based on Io in slots corresponding to RSSI measurement time configuration (RMTC).

### A.13.4.6	Channel occupancy

### A.13.4.6.1 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA

#### A.13.4.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy measurement accuracy is within the specified limits. This test will partially verify the channel occupancy measurement accuracy requirements in Section 10.1.35.1.

#### A.13.4.6.1.2	Test parameters

In all test cases, Cell 1 is the PCell on a licensed FR1 band and Cell 2 is the SCell with CCA. Channel occupancy is measured on channel number 2. Supported test configurations are shown in table A.13.4.6.1.2-1. The accuracy of channel occupancy intra-frequency measurements is tested by using the parameters in A.13.4.6.1.2-2 and A.13.4.6.1.2-3.

Table A.13.4.6.1.2-1: Intra frequency CO supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.4.6.1.2-2: CO Intra frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2,3 |  | Configuration 1,2: SSB.1 FR1 Configuration 3: SSB.2 FR1 | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2,3 |  | Configuration 1,2: SSB.1 FR1 Configuration 3: SSB.2 FR1 | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 CCA |
|  |  | 2,3 |  | SR.1.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 CCA |
|  |  | 2,3 |  | CR.1.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 CCA |
|  |  | 2,3 |  | CCR.1.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| channelOccupancyThreshold |  |  | dBm | -83 |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.13.4.6.1.2-3: CO RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.13.4.6.1.3	Test Requirements

The nominal reported channelOccupancy shall be TBD. At least 90% of channel occupancy reports made by the UE shall indicate this value.

### A.13.4.6.2			Inter-frequency channel occupancy measurement accuracy on a carrier with CCA

#### A.13.4.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the channel occupancy measurement accuracy is within the specified limits. This test will partially verify the channel occupancy measurement accuracy requirements in Section 10.1.35.2.

#### A.13.4.6.2.2	Test parameters

In all test cases, Cell 1 is the PCell on a licensed FR1 band and Cell 2 is the neighbour with CCA. Channel occupancy is measured on channel number 2. Supported test configurations are shown in table A.13.4.6.2.2-1. The accuracy of channel occupancy inter-frequency measurements is tested by using the parameters in A.13.4.6.2.2-2 and A.13.4.6.2.3.

Table A.13.4.6.2.2-1: Inter frequency CO supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 2 | Without CCA: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 3 | Without CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode With CCA: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.13.4.6.2.2-2: CO Inter frequency test parameters

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2,3 |  | Configuration 1,2: SSB.1 FR1 Configuration 3: SSB.2 FR1 | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2,3 |  | Configuration 1,2: SSB.1 FR1 Configuration 3: SSB.2 FR1 | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | TBD |
| PCCA_UL |  |  |  | 1 | TBD |
| DL CCA model |  |  |  | N/A | As specifieed in A.3.20.2.1 |
| UL CCA model |  |  |  | N/A | As specified in A.3.20.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | NA |
|  |  | 2,3 |  | SR.1.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | NA |
|  |  | 2,3 |  | CR.1.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | NA |
|  |  | 2,3 |  | CCR.1.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 | NA |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | NA |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -101.6 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/BW | -101.6 | -87 |
| Propagation condition |  |  | - | AWGN |  |
| channelOccupancyThreshold |  |  | dBm | -83 |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table A.13.4.6.2.2-3: CO RMTC parameters

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

#### A.13.4.6.2.3	Test Requirements

The nominal reported channelOccupancy shall be TBD. At least 90% of channel occupancy reports made by the UE shall indicate this value.