## 6.6	Measurement procedures

### 6.6.1	Intra-frequency measurements

#### 6.6.1.0	Minimum conformance requirements

##### 6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap

Same as 4.6.1.0.1

##### 6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap

[TS 38.133 [6], clause 9.2.6.2, 9.2.6.3]

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra  ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.1.0.2-1.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.1.0.2-2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in TS 38.133 [6] section 9.1.5.2.2 for measurement within outside measurement gaps.

Table 6.6.1.0.2-1: Time period for PSS/SSS detection (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle > 320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

Table 6.6.1.0.2-2: Measurement period for intra-frequency measurements with gaps (Frequency Range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max( 200ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 200ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] Sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.2]

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_without_index defined in TS 38.133 [6] section 9.2.6.2. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.6.2 and 9.2.6.3.

##### 6.6.1.0.3	Void

##### 6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading

[TS 38.133 [6], clause 9.2.6.2, 9.2.6.3]

UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index.

Tidentify_intra_with_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.1.0.4-1.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 6.6.1.0.4-2.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.1.0.4-3.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap, i in TS 38.133 section 9.1.5.2.2 for measurement conducted within measurement gaps.

When intrafrequency SMTC is fully non overlapping with measurement gaps or intrafrequency SMTC is fully overlapping with MGs, Kp=1

When intrafrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP.

Table 6.6.1.0.4-1: Time period for PSS/SSS detection (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | [5] x max(MGRP, DRX cycle) x CSSFintra |

Table 6.6.1.0.4-2: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 6.6.1.0.4-3: Measurement period for intra-frequency measurements with gaps (Frequency Range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max( 200ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 200ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.2]

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_with_index defined in TS 38.133 [6] section 9.2.6.2. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.6.2 and 9.2.6.3.

##### 6.6.1.0.5	 Void

#### 6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX

6.6.1.1.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under non-DRX.

6.6.1.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

6.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.1.

6.6.1.1.4	Test description

6.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.1.4.1-1.

Table 6.6.1.1.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.1-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.1-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.1-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.1.1.4.1-2.

Table 6.6.1.1.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.1.4.1-3 below.

2. Message contents are defined in clause 6.6.1.1.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.1.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under non-DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.1.1.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.1.4.1-3 and Table 6.6.1.1.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.1.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.1.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 802 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current NR cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
Or
- if the device has been switched off, switcheson the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.1.4.3-1: Common Exception messages SA intra frequency event triggered reporting tests without gap under non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for configuration 6.6.1.1-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.1-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM |

6.6.1.1.5	Test requirement

Table 6.6.1.1.4.1-3 and Table 6.6.1.1.5-1 define the primary level settings including test tolerances for NR SA event triggered reporting test without gap under non-DRX.

Table 6.6.1.1.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under non-DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report to NR Cell 2.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intr= max [ 600 ms, ceil (5 × Kp) × SMTC period ] × CSSFintra = 600 ms

T SSB_measurement_period_intra= max [ 200 ms, ceil( 5 × Kp) × SMTC period ] × CSSFintra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 802 ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX

6.6.1.2.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under DRX.

6.6.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.2.

6.6.1.2.4	Test description

6.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.2.4.1-1.

Table 6.6.1.2.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.2-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.2-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.2-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.2.4.1-2.

Table 6.6.1.2.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.2.4.1-3 below.

2. Message contents are defined in clause 6.6.1.2.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.2.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX | ms | 1, 2, 3 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
|  |  | 2 | 3 us |  | Synchronous cells |
|  |  | 3 | 3 us |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

6.6.1.2.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.2.4.1-3 and Table 6.6.1.2.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.2.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.2.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms for Test 1 or less than 6402 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current NR cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.1.2.4.1-1 as appropriate.

6.6.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.2.4.3-1: Common Exception messages for SA intra frequency event triggered reporting tests without gap under DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO SSB.1 FR1, SMTC.2 for configuration 6.6.1.2-1 Table H.3.1-3 with Condition INTRA-FREQ MO SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.2-2 Table H.3.1-3 with Condition INTRA-FREQ MO SSB.2 FR1, SMTC .1 and synchronous cells for configuration 6.6.1.2-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM Table H.3.7-1 with Condition DRX.1 and Offset for test 1 Table H.3.7-1 with Condition DRX.7 and Offset for test 2 |

6.6.1.2.5	Test requirement

Table 6.6.1.2.4.1-3 and Table 6.6.1.2.5-1 define the primary level settings including test tolerances for NR event triggered reporting in synchronous cells when DRX is used test.

Table 6.6.1.2.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

In Test 1 when DRX cycle length = 40 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report on PUSCH.

In Test 2 when DRX cycle length = 640 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report to NR Cell 2 on PUSCH.

For both tests:

The overall delays measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report to NR Cell 2.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delay measured when DRX cycle length is 40 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intra = max[600ms, ceil(1.5 × 5 x Kp) × max(SMTC period, DRX cycle)] × CSSFintra= 600ms

T SSB_measurement_period_intra = max[200ms, ceil(1.5 × 5 × Kp) x max(SMTC period, DRX cycle)] × CSSFintra= 320ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 40 ms shall be less than a total of 922 ms.

The overall delay measured when DRX cycle length is 640 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200ms

T SSB_measurement_period_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of 6402 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX

6.6.1.3.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under non-DRX within intra-frequency cell search with gaps requirements.

6.6.1.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards supporting 5GS NR SA FR1, CSI-RS-based RLM and BWP operation without bandwidth restriction.

6.6.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.3.

6.6.1.3.4	Test description

6.6.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.3.4.1-1.

Table 6.6.1.3.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.3-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.3-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.3-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.3.4.1-2.

Table 6.6.1.3.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.3.4.1-3.

2. Message contents are defined in clause 6.6.1.3.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.3.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.1.3.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.3.4.1-3 and Table 6.6.1.3.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.3.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.3.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 802 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.3.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for Configuration 6.6.1.3-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and Synchronous cells for Configuration 6.6.1.3-2 Table H.3.1-3 with ConditionINTRA-FREQ MO, SSB.2 FR1, SMTC.1 and Synchronous cells for Configuration 6.6.1.3-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.1.3.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

6.6.1.3.5	Test requirement

Table 6.6.1.3.4.1-3 and Table 6.6.1.3.5-1 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in non-DRX test.

Table 6.6.1.3.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.2 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 802 ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX

6.6.1.4.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under DRX within intra-frequency cell search with gaps requirements.

6.6.1.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle.

6.6.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.4.

6.6.1.4.4	Test description

6.6.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.4.4.1-1.

Table 6.6.1.4.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with gap in DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.4-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.4-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.4-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.4.4.1-2.

Table 6.6.1.4.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.4.4.1-3.

2. Message contents are defined in clause 6.6.1.4.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.4.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with gap in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |  |
| Measurement gap repetition periodicity | ms | 1, 2, 3 | 40 |  |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX | ms | 1, 2, 3 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

6.6.1.4.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.4.4.1-3 and Table 6.6.1.4.4.2-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

In Test 1 when DRX cycle = 40 ms is used, UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Timer Alignment Timer to keep the UE uplink time alignment. Furthermore, the UE is allocated with PUSCH resource at every DRX cycle. In Test 2 when DRX = 640 ms is used, the uplink time alignment is not maintained and the UE needs to use RACH to obtain uplink allocation for measurement reporting.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.4.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.4.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms for Test 1 or less than 6402 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.1.4.4.1-3 as appropriate.

6.6.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.4.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for configuration 6.6.1.4-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.4-2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.4-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.7-1 with Condition DRX.1 and Gap for test 1 Table H.3.7-1 with Condition DRX.7 and Gap for test 2 Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.1.4.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

6.6.1.4.5	Test requirement

Table 6.6.1.4.4.1-3 and Table 6.6.1.4.5-1 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in DRX test.

Table 6.6.1.4.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.2 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

In Test 1 when DRX cycle length = 40 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report on PUSCH.

In Test 2 when DRX cycle length = 640 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report on PUSCH.

For both tests:

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 1: The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2: In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

TPSS/SSS_sync_intra = 600 ms for Test 1, and TPSS/SSS_sync_intra =3200 ms for Test 2

TSSB_measurement_period_intra = 320 ms for Test 1, and TSSB_measurement_period_intra = 3200 ms for Test 2

TTI insertion uncertainty = 2 ms

For Test 1, the overall delays measured shall be less than a total of 922 ms (note: this gives a total of 920 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For Test 2, the overall delays measured shall be less than a total of 6402 ms (note: this gives a total of 6400 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

6.6.1.5.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event within intra-frequency cell search without gaps requirements.

6.6.1.5.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

6.6.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.5.

6.6.1.5.4	Test description

6.6.1.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.5.4.1-1.

Table 6.6.1.5.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Configuration | Description |
| --- | --- |
| 6.6.1.5-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.5.4.1-2.

Table 6.6.1.5.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.5.4.1-3.

2. Message contents are defined in clause 6.6.1.5.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.5.4.1-3: General test parameters for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

6.6.1.5.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.5.4.1-3 and Table 6.6.1.5.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.5.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.5.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 clause 4.6 with the following exceptions:

Table 6.6.1.5.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for Configuration 6.6.1.5-1 Table H.3.1-4 with SSB Index and A3-offset = -4.5 dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ and SSB Index Table H.3.1-8 with Condition SSB RLM |

6.6.1.5.5	Test requirement

Table 6.6.1.5.4.1-3 and Table 6.6.1.5.5-1 define the primary level settings including test tolerances for SA event triggered reporting without gap under non-DRX with SSB index reading test.

Table 6.6.1.5.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_with_index

Tidentify_intra_with_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_time_index_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_time_index_intra = 120 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 922 ms in this test case (note: this gives a total of 920 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

6.6.1.6.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event within intra-frequency cell search with gaps requirements.

6.6.1.6.2	Test applicability

This test applies to all types of NR UE release 15 onwards supporting 5GS NR SA FR1, CSI-RS-based RLM and BWP operation without bandwidth restriction.

6.6.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.6.

6.6.1.6.4	Test description

6.6.1.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.6.4.1-1.

Table 6.6.1.6.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Configuration | Description |
| --- | --- |
| 6.6.1.6-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.3.4.1-2.

Table 6.6.1.6.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.6.4.1-3.

2. Message contents are defined in clause 6.6.1.6.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.6.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1 | 40 |  |
| Measurement gap length | ms | 1 | 6 |  |
| Measurement gap offset | ms | 1 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

6.6.1.6.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.6.4.1-3 and Table 6.6.1.6.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.6.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.3.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.6.4.3	Message contents

Message contents are according to TS 38.508-1 clause 4.6 with the following exceptions:

Table 6.6.1.6.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for Configuration 6.6.1.6-1 Table H.3.1-4 with SSB index and A3-offset = -4.5dB. Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ and SSB index Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.1.6.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

6.6.1.6.5	Test requirement

Table 6.6.1.6.4.1-3 and Table 6.6.1.6.5-1 define the primary level settings including test tolerances for SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading test.

Table 6.6.1.6.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_with_index

Tidentify_intra_with_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_time_index_intra = 120 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 922 ms in this test case (note: this gives a total of 920 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

6.6.1.7.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under DRX for UE configured with highSpeedMeasFlag-r16.

6.6.1.7.2	Test applicability

This test applies to all types of NR UE release 15 onwards that supports measurement enhancements in high speed scenario.

6.6.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.7.

6.6.1.7.4	Test description

6.6.1.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.7.4.1-1.

Table 6.6.1.7.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 6.6.1.7-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.1.7-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.1.7-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.7.4.1-2.

Table 6.6.1.7.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.7.4.1-3 below.

2. Message contents are defined in clause 6.6.1.7.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.7.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasFlag-r16 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.7 | 640ms DRX cycle |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 6 |  |

6.6.1.7.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.7.4.1-3 and Table 6.6.1.7.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.7.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.7.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 5122 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.7.4.3-1: Common Exception messages for SA intra frequency event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasFlag-r16

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM Table H.3.7-1 with Condition DRX.7 and Offset |
| Specific message contents exceptions for Test Configuration 6.6.1.7-1 | Table H.3.1-3 with Condition INTRA-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.7-2 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.7-3 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.2 FR1 |

6.6.1.7.5	Test requirement

Table 6.6.1.7.4.1-3 and Table 6.6.1.7.5-1 define the primary level settings including test tolerances for NR event triggered reporting in synchronous cells when DRX is used

Table 6.6.1.7.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4 | -1.46 |  | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4 | 4 |  | -Infinity | 4 |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94+TT | -94 |  | -Infinity | -94 |
|  |  | 2 | -94 | -94 |  | -Infinity | -94 |
|  |  | 3 | -91 | -91 |  | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 |  | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25T |  | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN 1944Hz Note 4 |  |  |
|  |  | 3 | AWGN |  | AWGN 3334Hz Note 5 |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report to NR Cell 2. When DRX cycle length = 640 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report to NR Cell 2 on PUSCH.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delay measured when DRX cycle length is 640 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200ms

T SSB_measurement_period_intra = ceil(Y × Kp) × DRX cycle × CSSFintra = 1920ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of 5122 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.8	NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

6.6.1.8.1	Test purpose

The purpose of this test is to verify TBD

6.6.1.8.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward supporting enhanced NR CA measurement requirements in high-speed scenario.

6.6.1.8.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.1.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.8.

6.6.1.8.4	Test description

6.6.1.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.8.4.1-1.

Table 6.6.1.8.4.1-1: Supported test configurations for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Configuration | Description |
| --- | --- |
| 6.6.1.8-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.1.8-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.1.8-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.8.4.1-2.

Table 6.6.1.8.4.1-2: Initial conditions for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.1.8.4.1-3.

2.	Message contents are defined in clause 6.6.1.8.4.3.

3. There is two carriers and three cells specified in the test. NR Cell 1 is the PCell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test. NR Cell 2 is is a FR1 deactivated SCell and NR Cell 3 is a neighbour cell on the same frequency as the SCell.

Table 6.6.1.8.4.1-3: General test parameters for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasCA-Scell-r17 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and Cell 3 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.6 |  |
| measCycleSCell |  | 1, 2, 3 | 640ms |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

6.6.1.8.4.2	Test procedure

Same test procedure as in 6.6.1.7.4.2 except following step 6:

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 5760 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

6.6.1.8.4.3	Message contents

Table 6.6.1.8.4.3-1: Common Exception messages for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM Table H.3.7-1 with Condition DRX.6 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-1 | Table H.3.1-3 with Condition INTRA-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-2 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-3 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.2 FR1 |

Table 6.6.1.8.4.3-2: ReportConfigNR-DEFAULT(a6-offset): NR report configuration for event A6

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition EVENT_A6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA6 SEQUENCE { |  |  |  |
| a6-Offset CHOICE { |  |  |  |
| rsrp | -9 | The actual value is field value * 0.5 dB. |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.1.8.4.3-3: ServingCellConfigCommon

| Derivation Path: TS 38.508-1[14], Table 4.6.3-168 with condition R17 HST FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| HighSpeedConfig-v1700 SEQUENCE { |  |  |  |
| highSpeedMeasCA-Scell-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.1.8.4.3-4: MeasObjectNR-DEFAULT: measCycleSCell configuration for SCell intra-frequency measurements

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| measCycleSCell-v1530 | sf160 |  |  |
| } |  |  |  |

6.6.1.8.5	Test requirement

Table 6.6.1.8.5-1 defines the primary level settings including test tolerances for EN-DC FR1 interruptions at SRS carrier based switching in asynchronous EN-DC.

Table 6.6.1.8.5-1: NR cell specific test parameters for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD configuration |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98+TT |  |  |  |  |  |
|  |  | 2 | -98+TT |  |  |  |  |  |
|  |  | 3 | -95+TT |  |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98+TT |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4+TT | -1.46+TT | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4+TT | 4+TT | 4+TT | 4+TT | -Infinity | 4+TT |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94+TT | -94+TT | -94+TT | -94+TT | -Infinity | -94+TT |
|  |  | 2 | -94+TT | -94+TT | -94+TT | -94+TT | -Infinity | -94+TT |
|  |  | 3 | -91+TT | -91+TT | -91+TT | -91+TT | -Infinity | -91+TT |
| Io | dBm/9.36 MHz | 1 | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
|  | dBm/9.36 MHz | 2 | -64.6+TT 0 | -62.25+TT | -64.60+TT | -62.25+TT | -64.60+TT | -62.25+TT |
|  | dBm/38.16 MHz | 3 | -58.50+TT | -56.16+TT | -58.50+TT | -56.16+TT | -58.50+TT | -56.16+TT |
| Propagation Condition |  | 1,2 | AWGN |  |  |  | AWGN 1944Hz Note 4 |  |
|  |  | 3 | AWGN |  |  |  | AWGN 3334Hz Note 5 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |  |

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 5760 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.2	Inter-frequency measurements

#### 6.6.2.0	Minimum conformance requirements for Inter-frequency measurements

##### 6.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps

Same as clause 4.6.2.0 when measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps.

##### 6.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index. It is assumed that when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2, the following condition is met:

-	SFN and frame boundary across serving cell and inter-frequency neighbour cells is aligned

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 6.6.2.0-1

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 6.6.2.0-2.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 6.6.2.0-3 and table 6.6.2.0-4.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e. when inter frequency SMTC is fully non overlapping or partially overlapping with measurement gaps or according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps, i.e. when inter frequency SMTC is fully overlapping with measurement gaps.

Mpss/sss_sync_inter: For a UE supporting FR2 power class 1, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2 power class 2, Mpss/sss_sync_inter = 24 samples. For a UE supporting FR2 power class 3, Mpss/sss_sync_inter = 24 samples. For a UE supporting FR2 power class 4, Mpss/sss_sync = 24 samples.

Mmeas_period_inter: For a UE supporting FR2 power class 1, Mmeas_period_inter =40 samples. For a vehicle mounted UE supporting FR2 power class 2, Mmeas_period_inter =24 samples. For a UE supporting FR2 power class 3, Mmeas_period_inter =24 samples. For a UE supporting FR2 power class 4, Mmeas_period_inter = 24 samples.

When inter frequency SMTC is fully non overlapping with measurement gaps or inter frequency SMTC is fully overlapping with MGs, Kp=1.

When inter frequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP.

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by SSB-ToMeasure and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

Table 6.6.2.0-1: Time period for PSS/SSS detection, (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. |  |

Table 6.6.2.0-2: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. |  |

The UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively, as shown in table 6.6.2.0-3 and 6.6.2.0-4, if UE supports inter-frequency measurement without measurement gaps:

Table 6.6.2.0-3: Measurement period for inter-frequency measurements without gaps ((FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. |  |

Table 6.6.2.0-4: Measurement period for inter-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_inter x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mmeas_period_inter xKp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. |  |

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE is required to be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction.  Note that the SSB symbols to be measured in the following clauses are the SSB symbols indicated by SSB-ToMeasure [2], if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [3] are included.

The scheduling availability requirements when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2 in TS 38.133 [6] clause 9.3.9.3.1~9.3.9.3.3 are valid under the following conditions:

-	SFN and frame boundary across serving cell and inter-frequency neighbour cells is aligned, and

When UE performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

When UE performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration.

When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to one serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

For UE which do not support simultaneousRxDataSSB-DiffNumerology-Inter-r16 [14] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If UE performs inter-frequency measurements without measurement gaps in a TDD band, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

-	If UE performs inter-frequency measurements without measurement gaps in a FDD band, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration.

When intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.4, 9.3.5, 9.3.6.3.

#### 6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX

6.6.2.1.1	Test purpose

To verify that the UE makes correct reporting of an event in non-DRX within inter-frequency NR cell search requirements without SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

6.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.1.

6.6.2.1.4	Test description

6.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.1.4.1-2. Test environment parameters are given in Table 6.6.2.1.4.1-3.

Table 6.6.2.1.4.1-1: SA FR1-FR1 event triggered reporting tests in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.1.4.1-2: SA FR1-FR1 general test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in non DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in TS 38.133 clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1 | 1 |  |

Table 6.6.2.1.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.1.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

6.6.2.1.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 6.6.2.1.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 6.6.2.1.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.1.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.1.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 920 ms  for Test 1 and 760 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.  Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR:
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.1.4.1-2 as appropriate.

6.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.1.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 for Test 1 Table H.3.1-6 with Conditions gapFR1, Pattern #4 and gap offset = 9 for Test 2 Table H.3.1-7 with Condition INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.1.5	Test requirement

Table 6.6.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.1.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in non-DRX

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |  |
| OCNG Patterns |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |  |
| SMTC configuration |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%. with a confidence level of 95%

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 760 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX

6.6.2.2.1	Test purpose

To verify that the UE makes correct reporting of an event in DRX within inter-frequency NR cell search requirements without SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle. Test 1 and Test 2 are applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

6.6.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.2.

6.6.2.2.4	Test description

6.6.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.2.4.1-2. Test environment parameters are given in Table 6.6.2.2.4.1-3.

Table 6.6.2.2.4.1-1: SA FR1-FR1 event triggered reporting tests in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.2-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.2-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.2-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in TS 38.133 clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 11 | 1.1 | 11 |  |

Table 6.6.2.2.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.2.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

6.6.2.2.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 6.6.2.2.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 6.6.2.2.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.2.4.1-2 and 6.6.2.2.5-1.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.2.5-1. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1080 ms for Test 1, 10240 ms for Test 2, 1080 ms for Test 3 and 10240 ms for Test 4 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.  Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.2.4.1-2 as appropriate.

6.6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.2.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 for Test 1 and Test 2 Table H.3.1-6 with Conditions gapFR1, Pattern #4 and gap offset = 9 for Test 3 and Test 4 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 and Gap and INTER-FREQ for Test 1 and Test 3 Table H.3.7-1 with Condition DRX.7 and Gap and INTER-FREQ for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 6.6.2.2-1 | Table H.3.1-3 with Conditions INTER-FREQ MO and Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.2-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.2-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.2.5	Test requirement

Table 6.6.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.2.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |  |
| OCNG Patterns |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  | - |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  | - |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |  |
| SMTC configuration |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.2 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.3	Void

#### 6.6.2.4	Void

#### 6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection

6.6.2.5.1	Test purpose

To verify that the UE makes correct reporting of an event in non-DRX within inter-frequency NR cell search requirements with SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.5.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

6.6.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.5.

6.6.2.5.4	Test description

6.6.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.5.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.5.4.1-2. Test environment parameters are given in Table 6.6.2.5.4.1-3.

Table 6.6.2.5.4.1-1: SA FR1-FR1 event triggered reporting tests in non-DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.5-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.5-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.5-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.5.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2 |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in TS 38.133 clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 1 |  |

Table 6.6.2.5.4.1-3: Environment test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.5.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

6.6.2.5.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 6.6.2.5.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 6.6.2.5.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.5.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.5.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1040 ms for Test 1 and 880 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.  Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.5.4.1-2 as appropriate.

6.6.2.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.5.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and Condition SSB Index Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 for Test 1 Table H.3.1-6 with Conditions gapFR1, Pattern #4 and gap offset = 9 for Test 2 Table H.3.1-7 with Conditions INTER-FREQ and SSB Index |
| Specific message contents exceptions for Test Configuration 6.6.2.5-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.5-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.5-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.5.5	Test requirement

Table 6.6.2.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.5.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |  |
| OCNG Patterns |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  | - |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  | - |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |  |
| SMTC configuration |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.2 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.4 | -58.49 |  | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1040 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 880 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1 and 2 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection

6.6.2.6.1	Test purpose

To verify that the UE makes correct reporting of an event in DRX within inter-frequency NR cell search requirements with SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.6.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle. Test 1 and Test 2 are applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

6.6.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.6.

6.6.2.6.4	Test description

6.6.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.6.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.6.4.1-2. Test environment parameters are given in Table 6.6.2.6.4.1-3.

Table 6.6.2.6.4.1-1: SA FR1-FR1 event triggered reporting tests in DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.6-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.64-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.6-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.6.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  |  |  | NR cell 2 is on NR RF channel number 2 |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in TS 38.133 clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 1.3 | 13.5 | 1.3 | 13.5 |  |

Table 6.6.2.6.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.6.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Table Annex C.1.2.

6.6.2.6.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 6.6.2.6.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 6.6.2.6.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.6.4.1-2 and Table 6.6.2.6.5-1.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.6.5-1. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1280 ms for Test 1, 13440 ms for Test 2, 1280 ms for Test 3 and 13440 ms for Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.6.4.1-2 as appropriate.

6.6.2.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.6.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and Condition SSB Index Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 for Test 1 and Test 2 Table H.3.1-6 with Conditions gapFR1 and Pattern #4 and gap offset = 9 for Test 3 and Test 4 Table H.3.1-7 with Conditions INTER-FREQ and SSB Index Table H.3.7-1 with Condition DRX.1 and Gap and INTER-FREQ for Test 1 and Test 3 Table H.3.7-1 with Condition DRX.7 and Gap and INTER-FREQ for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 6.6.2.6-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.6-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.6-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.6.5	Test requirement

Table 6.6.2.6.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.6.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |  |
| OCNG Patterns |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  | - |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  | - |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |  |
| SMTC configuration |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.26 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 13440 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 13440 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.7	Void

#### 6.6.2.8	Void

#### 6.6.2.9	NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing.

-	TT analysis is missing.

-	Test applicability needs to be updated

-	Cell mapping is missing in Annex E

6.6.2.9.1	Test purpose

To verify that the UE makes correct reporting of an event when mandatory gap pattern with 3ms MGL is configured within inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.4.

6.6.2.9.2	Test applicability

This test applies to all types of NR UE release 16 onwards.

6.6.2.9.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.9.

6.6.2.9.4	Test description

6.6.2.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.9.4.1-1.

Table 6.6.2.9.4.1-1: SA FR1 event triggered reporting tests with additional mandatory gap pattern supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.2.9-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 6.6.2.9-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 6.6.2.9-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| NOTE.1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.9.4.1-2.

Table 6.6.2.9.4.1-2: Test Environment parameters for SA FR1 event triggered reporting tests with additional mandatory gap pattern

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2.. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.9.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.9.4.1-3.

2.	Message contents are defined in clause 6.6.2.9.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.9.4.1-3: General test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 3 | 2 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1 | 1 |  |

6.6.2.9.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 3 as defined in Table 6.6.2.9.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #2 as defined in Table 6.6.2.9.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.9.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.9.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1280 ms for Test 1 and 920 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR:

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.9.4.1-2 as appropriate.

6.6.2.9.4.3	Message contents

TBD

6.6.2.9.5	Test requirement

Table 6.6.2.9.5-1 defines the primary level settings including test tolerances for SA inter-frequency event triggered reporting with additional mandatory gap pattern.

Table 6.6.2.9.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.1 |  |  | SMTC.4 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98+TT |  | -98+TT |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  |  |
|  |  |  | Config 3 | -95+TT |  | -95+TT |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT |  | -Infinity | -91+TT |
|  |  |  | Config 3 | -91+TT | -91+TT |  | -Infinity | -88+TT |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT | 4+TT |  | -Infinity | 7+TT |
|  |  | dB | Config 1,2,3 | 4+TT | 4+TT |  | -Infinity | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT |  | -70.05+TT | -62.26+TT |
|  |  | dBm/38.16MHz | Config 3 | -58.49+TT | -58.49+TT |  | -63.94+TT | -56.15+TT |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

In test 1 and 2 UE is not required to report SSB time index.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 6.6.2.10	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing.

-	TT analysis is missing.

-	Test applicability needs to be updated

-	Cell mapping is missing in Annex E

6.6.2.10.1	Test purpose

To verify that the UE makes correct reporting of an event when no gap is configured and DRX is used within inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.9.

6.6.2.10.2	Test applicability

This test applies to all types of NR UE release 16 which supports interFrequencyMeas-Nogap-r16.

6.6.2.10.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.10.

6.6.2.10.4	Test description

6.6.2.10.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.10.4.1-1.

Table 6.6.2.10.4.1-1: SA event triggered reporting tests for FR1 without gap when DRX is used supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.2.10-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.2.10-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.2.10-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.10.4.1-2.

Table 6.6.2.10.4.1-2: Test Environment parameters for SA event triggered reporting tests for FR1 without gap when DRX is used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2.. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.10.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.10.4.1-3.

2.	Message contents are defined in clause 6.6.2.10.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.10.4.1-3: General test parameters for SA event triggered reporting tests for FR1 without gap when DRX is used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

6.6.2.10.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.10.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.10.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1080 ms for Test Configuration 1 and 900 ms for Test Configuration 2 and 3then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR:

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.10.4.1-2 as appropriate.

6.6.2.10.4.3	Message contents

TBD

6.6.2.10.5	Test requirement

Table 6.6.2.10.5-1 defines the primary level settings including test tolerances for SA event triggered reporting tests for FR1 without gap when DRX is used.

Table 6.6.2.10.5-1: Cell specific test parameters for SA event triggered reporting tests for FR1 without gap when DRX is used

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | NA |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | NA |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98+TT |  | -98+TT |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  |  |  |  |
|  |  |  | Config 3 | -95+TT |  | -95+TT |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT |  |  | -Infinity | -91+TT |  |
|  |  |  | Config 3 | -91+TT | -91+TT |  |  | -Infinity | -88+TT |  |
|  |  | dB | Config 1,2,3,4,5,6 | 4+TT | 4+TT |  |  | -Infinity | 7+TT |  |
|  |  | dB | Config 1,2,3 | 4+TT | 4+TT |  |  | -Infinity | 7+TT |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT |  |  | -70.05+TT | -62.2+TT |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49+TT | -58.49+TT |  |  | -63.94+TT | -56.15+TT |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

In test config 1, UE is required to report SSB time index. UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

In test config 2 and 3, UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 900 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.11	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing.

-	TT analysis is missing.

-	Test Procedure is FFS.

-	Test applicability needs to be updated

-	Cell mapping is missing in Annex E

6.6.2.11.1	Test purpose

To verify that the UE makes correct reporting of an event when no gap is configured and DRX is not used within inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.9.

6.6.2.11.2	Test applicability

This test applies to all types of NR UE release 16 which supports interFrequencyMeas-Nogap-r16.

6.6.2.11.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.11.

6.6.2.11.4	Test description

6.6.2.11.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.11.4.1-1.

Table 6.6.2.11.4.1-1: SA event triggered reporting tests for FR1 without gap when DRX is not used supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.2.11-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 6.6.2.11-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 6.6.2.11-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| NOTE.1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.11.4.1-2.

Table 6.6.2.11.4.1-2: Test Environment parameters for SA event triggered reporting tests for FR1 without gap when DRX is not used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2.. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.11.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.10.4.1-3.

2.	Message contents are defined in clause 6.6.2.10.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.11.4.1-3: General test parameters for SA event triggered reporting tests for FR1 without gap when DRX is not used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

6.6.2.11.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.11.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.11.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 800 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR:

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.11.4.1-2 as appropriate.

6.6.2.11.4.3	Message contents

TBD

6.6.2.11.5	Test requirement

Table 6.6.2.11.5-1 defines the primary level settings including test tolerances for SA event triggered tests for FR1 without gap when DRX is not used.

Table 6.6.2.11.5-1: Cell specific test parameters for SA event triggered reporting tests for FR1 without gap when DRX is not used

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98+TT |  | -98+TT |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  |  |
|  |  |  | Config 3 | -95+TT |  | -95+TT |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT |  | -Infinity | -91+TT |
|  |  |  | Config 3 | -91+TT | -91+TT |  | -Infinity | -88+TT |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7+TT |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT |  | -70.05+TT | -62.26+TT |
|  |  | dBm/38.16MHz | Config 3 | -58.49+TT | -58.49+TT |  | -63.94+TT | -56.15+TT |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE is not required to read the neighbour cell SSB index in this test.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 6.6.2.12	NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

6.6.2.12.1	Test purpose

The purpose of this test is to verify TBD

6.6.2.12.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward supporting enhanced inter-frequency NR measurement requirements in high-speed scenario

6.6.2.12.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.12

6.6.2.12.4	Test description

6.6.2.12.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.12.4.1-1.

Table 6.6.2.12.4.1-1: Supported test configurations for NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

| Config | Description |
| --- | --- |
| 6.6.2.12-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.2.12-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.2.12-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.12.4.1-2.

Table 6.6.2.12.4.1-2: Initial conditions for NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.12.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.12.4.1-3.

2.	Message contents are defined in clause 6.6.2.12.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.12.4.1-3: General test parameters for NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.4 |  | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 2.3 | 2.3 |  |

6.6.2.12.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.12.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.12.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 2240 ms Test 1 and Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR:
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.12.4.1-2 as appropriate.

6.6.2.12.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.12.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 for Test 1 Table H.3.1-6 with Conditions gapFR1, Pattern #4 and gap offset = 9 for Test 2 Table H.3.1-7 with Condition INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

Table 6.6.2.12.4.3-2: SIB4: NR inter frequency cell re-selection with highSpeedMeasInterFreq-r17

| Derivation Path: Table H.2.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v1700 ::= SEQUENCE { |  |  |  |
| highSpeedMeasInterFreq-r17 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.2.12.5	Test requirement

Table 6.6.2.12.5-1 defines the primary level settings including test tolerances for EN-DC FR1 interruptions at SRS carrier based switching in asynchronous EN-DC.

Table 6.6.2.12.5-1: NR cell specific test parameters for NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | NA |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  | NA |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  | NA |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  | NA |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  | NA |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  |  | -Infinity | -91 |  |
|  |  |  | Config 3 | -91 | -91 |  |  | -Infinity | -88 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  |  | -Infinity | 7 |  |
|  |  | dB | Config 1,2,3 | 4 | 4 |  |  | -Infinity | 7 |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  |  | -70.05 | -62.2 |  |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 |  |  | -63.94 | -56.15 |  |
| Propagation Condition |  |  | Config 1,2 | AWGN |  |  | AWGN 1944Hz Note 5 |  |  |  |
|  |  |  | Config 3 | AWGN |  |  | AWGN 3334Hz Note 6 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 6: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |  |  |  |

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

In test 1&2, UE is not required to report SSB time index.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.3	Inter-RAT Measurements

#### 6.6.3.0	Minimum conformance requirements

##### 6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD

The requirements are applicable for NR−E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN FDD cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RSRQ related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RS-SINR related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [23].

6.6.3.0.1.1	Requirements when no DRX is used

When the UE requires measurement gaps to identify and measure inter-RAT cells and an appropriate measurement gap pattern is scheduled, the UE shall be able to identify a new detectable FDD cell within TIdentify, E-UTRAN FDD according to the following expression:

TIdentify,E-UTRAN FDD=TBasicIdentify* 480TInter1*CSSFinterRAT     ms,

where:

TBasicIdentify = 480 ms,

TInter1 is defined in TS 38.133 [6] section 9.4.1,

CSSFinterRAT = CSSFwithin_gap_i_ is the scaling factor for the measured inter-RAT E-UTRA carrier i which is calculated as specified in TS 38.133 [6] section 9.1.5.2.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN FDD defined in Table 6.6.3.0.1.1-1.

Table 6.6.3.0.1.1-1: Measurement period and measurement bandwidth

| Configuration | Physical Layer Measurement period: TMeasure, E-UTRAN FDD [ms] | Measurement bandwidth [RB] |
| --- | --- | --- |
| 0 | 480 x CSSFinterRAT | 6 |
| 1 (note 1) | 240 x CSSFinterRAT | 50 |
| NOTE 1: This configuration is optional. |  |  |

The UE shall be capable of identifying and performing NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements of at least 4 E-UTRAN FDD cells per E-UTRA FDD carrier frequency layer for up to 7 E-UTRA FDD carrier frequency layers.

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.5.

6.6.3.0.1.2	Requirements when DRX is used

When DRX is in use and measurement gaps are configured, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in Table 6.6.3.0.1.2-1. When RRM enhancement for high speed is configured the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in Table 6.6.3.0.1.2-2.

Table 6.6.3.0.1.2-1: Requirement to identify a newly detectable E-UTRAN FDD cell

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in Section 6.6.3.0.1.1 apply | Non-DRX requirements in Section 6.6.3.0.1.1 apply |
| 0.256 | 5.12*K (20*CSSFinterRAT) | 7.68*K (30*CSSFinterRAT) |
| 0.32 | 6.4*K (20*CSSFinterRAT) | 7.68*K (24*CSSFinterRAT) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.1.1. |  |  |

Table 6.6.3.0.1.2-2: Requirement to identify a newly detectable E-UTRAN FDD cell for UE configured with RRM enhancement for high speed

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.2.2 apply | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.16<DRx cycle<=0.32 | Note 1(15*CSSFinterRAT) |  |
| 0.32<DRx cycle <= 0.64 | Note 1(10*CSSFinterRAT) |  |
| DRx cycle = 1.024 | Note 1(10*CSSFinterRAT) | Note 1(10*CSSFinterRAT) |
| DRx cycle = 1.28 | Note 1(8*CSSFinterRAT) | Note 1(8*CSSFinterRAT) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. |  |  |

When DRX is in use, the UE shall be capable of performing NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements of at least 4 identified E-UTRAN FDD cells per E-UTRA FDD frequency layer during each layer 1 measurement period, for up to 7 E-UTRA FDD carrier frequency layers, and the UE physical layer shall be capable of reporting NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements to higher layers with the measurement period Tmeasure, E-UTRAN FDD specified in Table 6.6.3.0.1.2-3.

Table 6.6.3.0.1.2-3: Requirement to measure E-UTRAN FDD cells

| DRX cycle length (s) | Tmeasure, E-UTRAN FDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX requirements in Section 6.6.3.0.1.1 apply |
| 0< DRX-cycle ≤10.24 | Note1 (5* CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.1.1. |  |

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.5.

6.6.3.0.1.3	Measurement reporting requirements for Event-Triggered Reporting

The reported NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements contained in event-triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.2.2, 10.2.3, and 10.2.5, respectively.

The UE shall not send any event-triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T Identify, E-UTRAN FDD defined in clauses 6.6.3.0.1.1 and 6.6.3.0.1.2 without DRX and with DRX, respectively. When L3 filtering is used, an additional delay can be expected.

If a cell which has been detectable at least for the time period TIdentify, E-UTRAN FDD becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event as per TS 38.331 [13], the event triggered measurement reporting delay shall be less than TMeasure, E-UTRAN FDD provided the timing to that cell has not changed more than  50 Ts while measurement gap has not been available and the L3 filter has not been used.

The normative reference for this requirement is TS 38.133 [6] clause 9.4.2.

##### 6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD

The requirements are applicable for NR−E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN TDD cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RSRQ related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RS-SINR related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [23].

6.6.3.0.2.1	Requirements when no DRX is used

When the UE requires measurement gaps to identify and measure inter-RAT cells and an appropriate measurement gap pattern is scheduled, the UE shall be able to identify a new detectable TDD cell within TIdentify, E-UTRAN TDD according to the following expression:

-	When configuration 0 or configuration 1 in Table 6.6.3.0.2-1 is applied,

TIdentify,E-UTRAN TDD=TBasicIdentify* 480TInter1*CSSFinterRAT     ms ,

-	When configuration 2 or configuration 3 in Table 6.6.3.0.2-1 is applied,

TIdentify,E-UTRAN TDD=(TBasicIdentify* 480TInter1+240)*CSSFinterRAT    ms,

where:

TBasicIdentify = 480 ms,

TInter1 is defined in TS 38.133 [6] section 9.4.1,

CSSFinterRAT = CSSFwithin_gap_i_ is the scaling factor for the measured inter-RAT E-UTRA carrier i which is calculated as specified in TS 38.133 [6] section 9.1.5.2.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN TDD defined in Table 6.6.3.0.2.1-1.

Table 6.6.3.0.2.1-1: TMeasure, E-UTRAN TDD for different configurations

| Configuration | Measurement bandwidth [RB] | Number of UL/DL sub-frames per half frame (5 ms) |  | DwPTS |  | TMeasure, E-UTRAN TDD [ms] |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | Normal CP | Extended CP |  |
| 0 | 6 | 2 | 2 |  |  | 480 x CSSFinterRAT |
| 1 (note 1) | 50 | 2 | 2 |  |  | 240 x CSSFinterRAT |
| 2 | 6 | 1 | 3 |  |  | 720 x CSSFinterRAT |
| 3 (Note 1) | 50 | 1 | 3 |  |  | 480 x CSSFinterRAT |
| NOTE 1: This configuration is optional. NOTE 2: Void |  |  |  |  |  |  |

The UE shall be capable of identifying and performing NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements of at least 4 E-UTRAN TDD cells per E-UTRA TDD carrier frequency layer for up to 7 E-UTRA TDD carrier frequency layers.

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN TDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.2. The NR – E-UTRAN TDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.3. The NR – E-UTRAN TDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.5.

6.6.3.0.2.1	Requirements when DRX is used

When DRX is in use and measurement gaps are configured, the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in Table 6.6.3.0.2.1-1. When RRM enhancement for high speed is configured the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in Table 6.6.3.0.2.1-2.

Table 6.6.3.0.2.1-1: Requirement to identify a newly detectable E-UTRAN TDD cell

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in Section 6.6.3.0.2.1 apply | Non-DRX requirements in Section 6.6.3.0.2.1 apply |
| 0.256 | 5.12*K (20*CSSFinterRAT) | 7.68*K (30*CSSFinterRAT) |
| 0.32 | 6.4*K (20*CSSFinterRAT) | 7.68*K (24*CSSFinterRAT) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.2.1. |  |  |

Table 6.6.3.0.2.1-2: Requirement to identify a newly detectable E-UTRAN TDD cell for UE configured with RRM enhancement for high speed

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.3.2 apply | Non-DRX requirements in clause 9.4.3.2 apply |
| 0.16<DRx cycle<=0.32 | Note 1(15*CSSFinterRAT) |  |
| 0.32<DRx cycle <= 0.64 | Note 1(10*CSSFinterRAT) |  |
| DRx cycle = 1.024 | Note 1(10*CSSFinterRAT) | Note 1(10*CSSFinterRAT) |
| DRx cycle = 1.28 | Note 1(8*CSSFinterRAT) | Note 1(8*CSSFinterRAT) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.3.2. |  |  |

When DRX is in use, the UE shall be capable of performing NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements of at least 4 identified E-UTRAN TDD cells per E-UTRA TDD frequency layer during each layer 1 measurement period, for up to 7 E-UTRA TDD carrier frequency layers, and the UE physical layer shall be capable of reporting NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements to higher layers with the measurement period Tmeasure, E-UTRAN TDD specified in Table 6.6.3.0.2.1-3.

Table 6.6.3.0.2.1-3: Requirement to measure E-UTRAN TDD cells

| DRX cycle length (s) | Tmeasure, E-UTRAN TDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX Requirements in Section 6.6.3.0.2.1 apply |
| 0.128 | For configuration 2, non-DRX requirements in section 6.6.3.0.2.1 apply, Otherwise: Note1 (5*CSSFinterRAT) |
| 0.128<DRX-cycle≤10.24 | Note1 (5*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.2.1. |  |

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN TDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.2. The NR – E-UTRAN TDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.3. The NR – E-UTRAN TDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.5.

6.6.3.0.2.3	Measurement reporting requirements for Event-Triggered Reporting

The reported NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements contained in event-triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.2.2, 10.2.3, and 10.2.5, respectively.

The UE shall not send any event-triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T Identify, E-UTRAN TDD defined in clauses 6.6.3.0.2.1 and 6.6.3.0.2.2 without DRX and with DRX, respectively. When L3 filtering is used, an additional delay can be expected.

If a cell which has been detectable at least for the time period TIdentify, E-UTRAN TDD becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event as per TS 38.331 [13], the event triggered measurement reporting delay shall be less than TMeasure, E-UTRAN TDD provided the timing to that cell has not changed more than  50 Ts while measurement gap has not been available and the L3 filter has not been used.

The normative reference for this requirement is TS 38.133 [6] clause 9.4.3.

#### 6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX

Editor's Note:

- Test Tolerances need to be updated to the latest 38.133 power levels and propagation conditions

6.6.3.1.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 under the cell search and measurement requirements.

6.6.3.1.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 from Release 15 onwards.

6.6.3.1.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.1.

6.6.3.1.3.1	NR – E-UTRAN FDD requirement

The minimum conformance requirements are specified in clause 6.6.3.0.1.

6.6.3.1.3.2	NR – E-UTRAN TDD requirement

The minimum conformance requirements are specified in clause 6.6.3.0.2.

6.6.3.1.4	Test description

6.6.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.1.4.1-1.

Table 6.6.3.1.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.3.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.1-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.1-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.1-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.1.4.1-2 and Table 6.6.3.1.4.1-3.

Table 6.6.3.1.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.1.5-1 and Table 6.6.3.1.5-2.. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.1.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables 6.6.3.1.5-1 and 6.6.3.1.5-2. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause TS 38.133 [6] Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in Table 6.6.3.1.5-1 |  |  |  |

1.	Message contents are defined in clause 6.6.3.1.4.3.

2.	Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.6.3.1.4.2	Test procedure

The test consists of two successive time periods, with time durations of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. Gap pattern configuration is configured before T2 begins to enable inter-frequency monitoring.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.3.1.5-1 and 6.6.3.1.5-2. Propagation conditions are set according to Annex C clause C.2.2.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.3.1.5-1 and 6.6.3.1.5-2.

6.	UE shall transmit a MeasurementReport message triggered by Event B2. If the measurement reporting delay from the beginning of time period T2 is less than 3842ms the number of successful tests is increased by one. If the UE fails to report the event within the measurement reporting delay requirement then the number of failure tests is increased by one.

7.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED according to TS 38.508-1 [14] clause 4.5.4 (if the paging fails, switches off and on the UE and ensures the UE is in the state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5,
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.3.1.4.3-1: Common Exception messages NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-RAT Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 with Condition INTER-RAT Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT |
| Specific message content exceptions for Test Configurations 6.6.3.1-1 and 6.6.3.1-4 | Table H.3.1-3 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message content exceptions for Test Configurations 6.6.3.1-2, 6.6.3.1-3, 6.6.3.1-5 and 6.6.3.1-6 | Table H.3.1-3 with Condition Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

6.6.3.1.5	Test requirement

Table 6.6.3.1.5-1 and Table 6.6.3.1.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.3.1.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORESET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG pattern note1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| Noc note2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104+TT |  |
| Noc note2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104+TT |  |
|  |  |  |  | 3, 6 | -101+TT |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16+TT | 0+TT |
| Ês/Iot note3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16+TT | 0+TT |
| SS-RSRP note3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88+TT | -104+TT |
|  |  |  |  | 3, 6 | -85+TT | -101+TT |
| SSB_RP note3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88+TT | -104+TT |
|  |  |  |  | 3, 6 | -85+TT | -101+TT |
| Io note3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94+TT | -73.04+TT |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84+TT | -66.93+TT |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.1.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configuration note1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configuration note1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel note2 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel note2 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Patterns note2 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RA note3 |  |  |  |  |
| OCNG_RB note3 |  |  |  |  |
| Noc note4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104+TT |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17+TT |
| Ês/Iot note5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17+TT |
| RSRP note5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87+TT |
| SCH_RP note5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87+TT |
| Io note5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -73.21+TT+10log (NRB,c /50) | -56.12+TT+10log (NRB,c /50) |
| Propagation Condition not 6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3842ms from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The overall delays measured test requirement is expressed as:

Tidentify,E-UTRAN FDD = TBasicIdentify * 480 / TInter1 * CSSFinterRAT ms

Which:

TBasicIdentify = 480,

TInter1 = 60,

CSSFinterRAT = 1

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 3842 ms in this test case (note: this gives a total of 3840 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

The rate of correct events observed during repeated tests shall be at least 90% with confidence level of 95%.

#### 6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX

Editor's Note:

Test Tolerances need to be updated to the latest 38.133 power levels and propagation conditions

6.6.3.2.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used under the cell search and measurement requirements.

6.6.3.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle.

6.6.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 6.6.3.0.1 and 6.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.2.

6.6.3.2.4	Test description

6.6.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.2.4.1-1.

Table 6.6.3.2.4.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Test Case ID | Description |
| --- | --- |
| 6.6.3.2-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.2-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.2-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.2-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.2-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.2-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.2.4.1-2 and Table 6.6.3.2.4.1-3.

Table 6.6.3.2.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.2.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1 |  |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 |  |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables 6.6.3.2.5-1 and 6.6.3.2.5-2. |  |  |  |
| Active cell |  | Cell 1 |  |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  |  | As specified in Clause TS 38.133 [6] Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP |  |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | note 1 |  |  | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 |  |  | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |  |  |
| TimeToTrigger | s | 0 |  |  |  |
| Filter coefficient |  | 0 |  |  | L3 filtering is not used |
| DRX |  | DRX.1 |  | DRX.7 | DRX cycle configurations DRX.1 and DRX.7 are defined in Table A.3.3.1-1 and Table A.3.3.2-1 respectively. |
| T1 | s | 5 |  |  |  |
| T2 | s | 5 | 15 |  |  |
| NOTE 1: Values are defined in Table 6.6.3.2.5-1 |  |  |  |  |  |

1.	Message contents are defined in clause 6.6.3.2.4.3.

2.	There are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

6.6.3.2.4.2	Test procedure

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore the UE shall be allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.3.2.5-1 and 6.6.3.2.5-2.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Tables 6.6.3.2.5-1 and 6.6.3.2.5-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 3.48 s for Test 1 or less than 12.8 s for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off .

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14+2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.3.2.4.3-1: Common Exception messages NR SA FR1 – E-UTRAN event-triggered reporting in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-RAT Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 with Condition INTER-RAT Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT Table H.3.7-1 with Condition DRX.1 and Gap for Test 1 Table H.3.7-1 with Condition DRX.7 and OFFSET for Test 2 |
| Specific message content exceptions for Test Configurations 6.6.3.2-1 and 6.6.3.2-4 | Table H.3.1-3 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message content exceptions for Test Configurations 6.6.3.2-2, 6.6.3.2-3, 6.6.3.2-5 and 6.6.3.2-6 | Table H.3.1-3 with Condition Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

6.6.3.2.5	Test requirement

Table 6.6.3.2.5-1 and Table 6.6.3.2.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.6.3.2.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -104+TT |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104+TT |  |
|  |  |  |  | 3, 6 | -101+TT |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 16+TT | 0+TT |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 16+TT | 0+TT |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88+TT | -104+TT |
|  |  |  |  | 3, 6 | -85+TT | -101+TT |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88+TT | -104+TT |
|  |  |  |  | 3, 6 | -85+TT | -101+TT |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.94+TT | -73.04+TT |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.84+TT | -66.93+TT |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.2.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104+TT |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17+TT |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17+TT |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87+TT |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87+TT |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -73.21+TT+10log (NRB,c /50) | -56.12+TT +10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

In test 1, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

In test 2, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 12.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### 6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16

6.6.3.3.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used under the cell search and measurement requirements for UE configured with RRM enhancement for high speed.

6.6.3.3.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 configured with RRM enhancement for high speed from Release 15 onwards.

6.6.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 6.6.3.0.1 and 6.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.3.

6.6.3.3.4	Test description

6.6.3.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.3.4.1-1.

Table 6.6.3.3.4.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Test Case ID | Description |
| --- | --- |
| 6.6.3.3-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.3-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.3-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.3-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.3-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.3-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.3.4.1-2 and Table 6.6.3.3.4.1-3.

Table 6.6.3.3.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.3.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.3.1-2 and A.6.6.3.3.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX.6 | DRX cycle configurations DRX.6 is defined in Table A.3.3.1-6. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in TS 38.133 Table 6.6.3.3.5-1 |  |  |  |

1.	Message contents are defined in clause 6.6.3.3.4.3.

2.	Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

3. UE is configured with highSpeedMeasFlag-r16

6.6.3.3.4.2	Test procedure

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore the UE shall be allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.3.3.5-1 and 6.6.3.3.5-2.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Tables 6.6.3.3.5-1 and 6.6.3.3.5-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than  4.8s then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off .

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14+2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.3.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks on condition of HighSpeedMeas with exceptions | Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-RAT Table H.3.1-3 with Condition SSB.1 FR1 for configuration 6.6.3.3-1, 6.6.3.3-2, 6.6.3.3-4, 6.6.3.3-5 Table H.3.1-3 with Condition SSB.2 FR1 for configuration 6.6.3.3-3, 6.6.3.3-6 Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 with Condition INTER-RAT Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT Table H.3.7-1 with Condition DRX.6 and Gap |

6.6.3.3.5	Test requirement

Table 6.6.3.3.5-1 and Table 6.6.3.3.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.6.3.3.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.3.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NRB,c /50) | -59.16+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN1944 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 4.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

### 6.6.4	L1-RSRP measurement for beam reporting

#### 6.6.4.0	Minimum conformance requirements

##### 6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting

Same as clause 4.6.4.0.1

The normative reference for this requirement is TS 38.133 [6] clause 9.5.3.1, 9.5.4.1 and 9.5.5.1.

##### 6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting

Same as clause 4.6.4.0.2

The normative reference for this requirement is TS 38.133 [6] clauses 9.5.3.1, 9.5.4.2 and 9.5.5.2.

#### 6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX

6.6.4.1.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

6.6.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.1.

6.6.4.1.4	Test description

6.6.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.1.4.1-2. Test environment parameters are given in Table 6.6.4.1.4.1-3.

Table 6.6.4.1.4.1-1: NR SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.1.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.1.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.1.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.1.4.1-2 and 6.6.4.1.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.6.4.1.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.4.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.6.4.1.5-1. T1 starts.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 80 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in 6.6.4.1.5-1. T2 starts.

5.	The UE shall start sending valid L1-RSRP reports. The SS shall check following requirements:

- R1:	the UE shall start to transmit valid reports no later than 720 ms for configuration 1, 2, 4 and 5 and no later than 680 ms for configuration 3 and 6 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement for SSB#1 (Table 6.6.4.1.5-2 for test configurations 1, 2, 4 and 5 and Table 6.6.4.1.5-3 for test configurations 3 and 6) and the relative L1-RSRP requirement for SSB#0 in Table 6.6.4.1.5-4. If the first valid report is received earlier than the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	the UE shall transmit reports every 80 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

- R3:	The L1-RSRP value of SSB#1 reported by the UE is compared to the expected L1-RSRP value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.4.1.5-2 for test configurations 1, 2, 4 and 5 and in Table 6.6.4.1.5-3 for test configurations 3 and 6 or the UE fails to report the measurement value for SSB#1, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-R4:	The DIFF RSRP value of SSB#0 reported by the UE is compared to the expected DIFF RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.4.1.5-4 or the UE fails to report the measurement value for SSB#0, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.4.1.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with condition SSB |

Table 6.6.4.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.4.1.5	Test requirement

Table 6.6.4.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.1.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.14 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.49 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 4.6.4.1.5-2 for for test configurations 1, 2, 4 and 5 and the corresponding absolute accuracy requirements in Table 4.6.4.1.5-3 for test configurations 3 and 6 and the corresponding relative accuracy requirements in Table 4.6.4.1.5-4 for all test configurations.

Table 6.6.4.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 55 |
| Highest reported value (SSB#1) | - | 75 |

Table 6.6.4.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 58 |
| Highest reported value (SSB#1) | - | 78 |

Table 6.6.4.1.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 0 |
| Highest DIFF RSRP reported (SSB#0) | - | 3 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R4) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in  DCCH.

#### 6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX

6.6.4.2.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.2.

6.6.4.2.4	Test description

6.6.4.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.2.4.1-2. Test environment parameters are given in Table 6.6.4.2.4.1-3.

Table 6.6.4.2.4.1-1: SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.2.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.2.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.2.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.2.4.1-2 and 6.6.4.2.5-1. UE is configured to perform RLM and BFD based on the SSBs. DRX is configured as specified in Table 6.6.4.2.4.1-2.

6.6.4.2.4.2	Test procedure

Same test procedure as in subclause 6.6.4.1.4.2 with tables 6.6.4.1.4.1-2 and 6.6.4.1.5-1 replaced by tables 6.6.4.2.4.1-2 and 6.6.4.2.5-1.

6.6.4.2.4.3	Message contents

Same message content as in subclause 6.6.4.1.4.3 with the following exception:

Table 6.6.4.2.4.3-1: Common Exception messages EN-DC SSB based L1-RSRP measurement in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.7-1 with condition DRX.3 |

6.6.4.2.5	Test requirement

Table 6.6.4.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.2.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.14 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.49 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.2.5-2 for for test configurations 1, 2, 4 and 5 and the corresponding absolute accuracy requirements in Table 6.6.4.2.5-3 for test configurations 3 and 6 and the corresponding relative accuracy requirements in Table 6.6.4.2.5-4 for all test configurations.

Table 6.6.4.2.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 55 |
| Highest reported value (SSB#1) | - | 75 |

Table 6.6.4.2.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 58 |
| Highest reported value (SSB#1) | - | 78 |

Table 6.6.4.2.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 0 |
| Highest DIFF RSRP reported (SSB#0) | - | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX

6.6.4.3.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

6.6.4.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.3.

6.6.4.3.4	Test description

6.6.4.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.3.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.3.4.1-2. Test environment parameters are given in Table 6.6.4.3.4.1-3.

Table 6.6.4.3.4.1-1:NR SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.3-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.3.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.3.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.3.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.3.4.1-2 and 6.6.4.3.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.3.4.2	Test procedure

The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. UE is also configured to measure L1-RSRP based on SSB. Upon receiving the DCI trigger, UE provides the report back based on the reporting configuration as defined in table 6.6.4.3.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.4.3.5-1. T1 starts.

3.	After 80ms from the start of the test the SS transmits the DCI trigger in slot 0 for configuration 1,2 and slot 8 for configuration 3. The corresponding CSI-RS set is transmitted with the offset of 0 slots after the DCI trigger.

4.	The SS shall check following requirements:

- R1:	the UE shall send L1-RSRP report at slot 8 from the reception of DCI trigger. If the report is received at slot 8 from the reception of DCI trigger, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	The L1-RSRP value of CSI-RS#1 reported by the UE is compared to the expected L1-RSRP value for CSI-RS #1. If the resulting value is outside the limits in Table 6.6.4.3.5-2 for test configurations 1, 2, 4 and 5 and in Table 6.6.4.3.5-3 for test configurations 3 and 6 or the UE fails to report the measurement value for CSI-RS #1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

-R3:	The DIFF RSRP value of CSI-RS #0 reported by the UE is compared to the expected DIFF RSRP value. If the resulting value is outside the limits in Table 6.6.4.3.5-4 or the UE fails to report the measurement value for CSI-RS #0, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

5.	Void.

6.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.4.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.4.3.4.3-1: Common Exception messages NR SA CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions APERIODIC and CSI-RSRP Table H.3.6-3 with conditions CSI-RS TS 38.508-1 [14] Table 7.3.1-21 with condition APERIODIC |

Table 6.6.4.3.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

6.6.4.3.5	Test requirement

Table 6.6.4.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.3.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.15 |
|  | 3 |  | -91.65 | -88.14 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.49 |
|  | 1~3 | dB | 0 | 3.5 |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1.

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.3.5-2 for for test configurations 1, 2, 4 and 5 and the corresponding absolute accuracy requirements in Table 6.6.4.3.5-3 for test configurations 3 and 6 and the corresponding relative accuracy requirements in Table 6.6.4.3.5-4 for all test configurations.

Table 6.6.4.3.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 55 |
| Highest reported value (CSI-RS#1) | 75 |

Table 6.6.4.3.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 58 |
| Highest reported value (CSI-RS#1) | 78 |

Table 6.6.4.3.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF RSRP reported (CSI-RS#0) | 0 |
| Highest DIFF RSRP reported (CSI-RS#0) | 3 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R3) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX

6.6.4.4.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.4.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.4.

6.6.4.4.4	Test description

6.6.4.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.4.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.4.4.1-2. Test environment parameters are given in Table 6.6.4.4.4.1-3.

Table 6.6.4.4.4.1-1: NR SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.4-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.4.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.4.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.4.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.4.4.1-2 and 6.6.4.4.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.4.4.2	Test procedure

Same test procedure as in subclause 6.6.4.3.4.2 with tables 6.6.4.3.4.1-2 and 6.6.4.3.5-1 replaced by tables 6.6.4.4.4.1-2 and 6.6.4.4.5-1.

6.6.4.4.4.3	Message contents

Same message content as in subclause 6.6.4.3.4.3 with the following exception:

Table 6.6.4.4.4.3-1: Common Exception messages NR SA CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.7-1 with condition DRX.3 and Offset |

6.6.4.4.5	Test requirement

Table 6.6.4.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.4.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.15 |
|  | 3 |  | -91.65 | -88.14 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.49 |
|  | 1~3 | dB | 0 | 3.5 |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1.

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.4.5-2 for for test configurations 1, 2, 4 and 5 and the corresponding absolute accuracy requirements in Table 6.6.4.4.5-3 for test configurations 3 and 6 and the corresponding relative accuracy requirements in Table 6.6.4.4.5-4 for all test configurations.

Table 6.6.4.4.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 55 |
| Highest reported value (CSI-RS#1) | 75 |

Table 6.6.4.4.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 58 |
| Highest reported value (CSI-RS#1) | 78 |

Table 6.6.4.4.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF RSRP reported (CSI-RS#0) | 0 |
| Highest DIFF RSRP reported (CSI-RS#0) | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16

Editor's note: This test case is incomplete. The following aspects are either missing or not yet determined:

- The feasibility of configuring different channel models to different SSBs needs further study.

- The TT analysis may need to be redone after concluding if the new test configuration is testable.

6.6.4.5.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements for UE configured with highSpeedMeasFlag-r16 in TS 38.133 clause 9.5.4.1

6.6.4.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1, measurement enhancements in high speed scenario and long DRX cycle

6.6.4.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.5.

6.6.4.5.4	Test description

6.6.4.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.5.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.5.4.1-2. Test environment parameters are given in Table 6.6.4.5.4.1-3.

Table 6.6.4.5.4.1-1: SA SSB based L1-RSRP measurement supported test configurations for UE configured with highSpeedMeasFlag-r16

| Test Case ID | Description |
| --- | --- |
| 6.6.4.5-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.5-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.5-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.5.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.5.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.4.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n=1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 with n=1 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.5.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in Table 6.6.4.5.4.1-2 and 6.6.4.5.5-1. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs. DRX is configured as specified in Table 6.6.4.5.4.1-2.

6.6.4.5.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.6.4.5.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.4.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.6.4.1.5-1. T1 starts.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 80 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in 6.6.4.1.5-1. T2 starts.

5.	If the UE sends L1-RSRP reports  meeting the corresponding absolute accuracy requirements in Table 6.6.4.5.5-2 for test configurations 1 and 2 the corresponding absolute accuracy requirements in Table 6.6.4.5.5-3 for test configurations 3 and the corresponding relative accuracy requirements in Table 6.6.4.5.5-4 for all test configurations every 80 slots from no later than 640 ms plus 80 slots for all configurations from the beginning of time period T2 until the end of time period T2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.4.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.4.5.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with conditions SSB Table H.3.7-1 with condition DRX.3 |

Table 6.6.4.5.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.4.5.5	Test requirement

Table 6.6.4.5.5-1: SSB specific test parameters for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.6 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.53 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.14 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.49 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| Propagation conditions | 1,2 |  | AWGN |  | AWGN 1944 HzNote4 |  |
|  | 3 |  | AWGN |  | AWGN 3334 Hz Note5 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. Note 5: The AWGN 3334 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.5.5-2 for for test configurations 1 and 2 and the corresponding absolute accuracy requirements in Table 6.6.4.5.5-3 for test configurations 3 and the corresponding relative accuracy requirements in Table 6.6.4.5.5-4 for all test configurations.

Table 6.6.4.5.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 55 |
| Highest reported value (SSB#1) | - | 75 |

Table 6.6.4.5.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 58 |
| Highest reported value (SSB#1) | - | 78 |

Table 6.6.4.5.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 0 |
| Highest DIFF RSRP reported (SSB#0) | - | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.5	UTRAN inter-RAT measurement

#### 6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX

6.6.5.1.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 under the cell search and measurement requirements.

6.6.5.1.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 from Release 16 onwards and support UTRA FDD.

6.6.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.5.1.

6.6.5.1.4	Test description

6.6.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.5.1.4.1-1.

Table 6.6.5.1.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.5.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, UTRAN FDD |
| 6.6.5.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, UTRAN FDD |
| 6.6.5.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, UTRAN FDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.5.1.4.1-2 and Table 6.6.5.1.4.1-3.

Table 6.6.5.1.4.1-2: Initial conditions for SA inter-RAT UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-3 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.5.1.5-1 and Table 6.6.5.1.5-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | SS LTE in Figure A.3.1.8.2 is replaced by SS UTRA LTE TX/RX in Figure A.3.2.3.2 is replaced by UTRA TX/RX |  |  |

Table 6.6.5.1.4.1-3: General test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  | 2 | 1 UTRA carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Table 6.6.5.1.5-1 and Table 6.6.5.1.5-2 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table FFS Per-UE gap pattern. |
| Inter-RAT UTRA measurement quantity |  | CPICH Ec/Io | Measurement quantity for Cell 2 |
| b1-ThresholdUTRA-FDD | dB | -16.5 | CPICH Ec/Io threshold for SS-RSRP measurement on cell1 for event B1 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.5.1.1-3 |  |  |  |

1.	Message contents are defined in clause 6.6.5.1.4.3.

2.	Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN inter-RAT neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 Annex C.0 and C.1.

6.6.5.1.4.2	Test procedure

The test consists of two successive time periods, with time durations of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. Gap pattern configuration is configured before T2 begins to enable inter-frequency monitoring.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.5.1.5-1 and 6.6.5.1.5-2. Propagation conditions are set according to Annex C clause C.2.2.T1 starts.

3.	SS shall transmit an RRCReconfiguration message to confogure Event B1 measurement reporting.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.5.1.5-1 and 6.6.5.1.5-2.

6.	UE shall transmit a MeasurementReport message triggered by Event B1. If the measurement reporting delay from the beginning of time period T2 is less than 3842ms the number of successful tests is increased by one. If the UE fails to report the event within the measurement reporting delay requirement then the number of failure tests is increased by one.

7.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.	Set Cell 2 primary scrambling code = ((current cell 2 primary scrambling code - 50) mod 200 + 100) for next iteration of the test procedure loop.

9.	Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED according to TS 38.508-1 [14] clause 4.5.4 (if the paging fails, switches off and on the UE and ensures the UE is in the state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5,
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.5.1.4.3	Message contents

Table 6.6.5.1.4.3-1: RRCReconfiguration (Step 3)

| Derivation Path: TS 38.508-1 [14] Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 6.6.5.1.4.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.5.1.4.3-2: MeasConfig (Table 6.6.5.1.4.3-1)

| Derivation Path: Table H.3.1-2 with condition INTER-RAT and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectUTRA-FDD-r16 | MeasObjectUTRA-FDD | Table 6.6.5.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT(16) specified in 38.508-1 [14] Table 4.6.3-141 with condition EVENT_B1_UTRA | Actual value of UTRA threshold = (16-49)/2 = -16.5dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig specified in Table H.3.1-6 with condition gapUE and Pattern #0 |  |  |
| } |  |  |  |

Table 6.6.5.1.4.3-3: MeasObjectUTRA-FDD (Table 6.6.5.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-77A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA-FDD-r16 ::= SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN-ValueUTRA-FDD-r16 for UTRA Cell 2 |  |  |
| } |  |  |  |

Table 6.6.5.1.4.3-4: MeasurementReport (Step 6)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| measurementReport SEQUENCE { |  |  |  |  |
| measResults |  | MeasResults | Table 6.6.5.1.4.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.5.1.4.3-5: MeasResults (Table 6.6.5.1.4.3-4)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-79 with condition B1_UTRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListUTRA-FDD-r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultUTRA-FDD-r16 { | 1 entry |  |  |
| MeasResultUTRA-FDD-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | PhysCellIdUTRA-FDD-r16 of UTRA Cell 2 |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| utra-FDD-EcN0-r16 | INTEGER (0..49) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.5.1.5	Test requirement

Table 6.6.5.1.5-1 and Table 6.6.5.1.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.5.1.5-1: PCell specific test parameters for SA inter-RAT UTRA FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3 | TDDConf.1.2 |  |
| BWchannel |  |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1 | SR.1.1 FDD |  |
|  |  |  |  | 2 | SR.1.1 TDD |  |
|  |  |  |  | 3 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1 | CR.1.1 FDD |  |
|  |  |  |  | 2 | CR.1.1 TDD |  |
|  |  |  |  | 3 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2 | SSB.1 FR1 |  |
|  |  |  |  | 3 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1 | TRS.1.1 FDD |  |
|  |  |  |  | 2 | TRS.1.1 TDD |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2 | -106 |  |
|  |  |  |  | 3 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3 | ETDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.5.1.5-2: UTRAN neighbour cell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |  |

The UE shall send one Event B1 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 2.4s from the start of period T2, i.e. when Cell 2 becomes detectable. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### 6.6.6

#### 6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX

Editor's NOTE: This test case is incomplete in following aspects:

-	Message contents are missing.

-	TT analysis is missing.

-	Test Procedure might need update.

-	Test applicability needs to be updatedW

6.6.6.1.1	Test purpose

To verify that the UE makes correct reporting of SRS-RSRP measurement within SRS-RSRP measurement requirements in TS 38.133 [6] clause 9.7.2.5.

6.6.6.1.2	Test applicability

FFS.

6.6.6.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.6.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.6.1.

6.6.6.1.4	Test description

6.6.6.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.6.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.6.1.4.1-2. Test environment parameters are given in Table 6.6.6.1.4.1-3.

Table 6.6.6.1.4.1-1: Applicable NR configurations for FR1 SRS-RSRP test

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | NR 30 kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.6.1.4.1-2: General test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table A.6.6.6.1.2-3 |
|  |  | 2 | SRSConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -97 |  |
|  |  | 2 | -95 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between DL from serving cell and SRS from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table 6.6.6.1.4.1-3: Test Environment parameters for EN-DC SRS-RSRP
event triggered reporting for PSCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

- Message contents are defined in clause 6.6.6.1.4.3.

- Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is NR FR1 cell (PSCell). Cell 2 is the target for SRS-RSRP measurements. In the measurement control information, a measurement object is configured for the frequency of the PSCell, and it is indicated to the UE that event-triggered reporting with Event I1 is used. The test consists of two successive time periods, with time duration of T1 and T2, respectively.

6.6.6.1.4.2	Test procedure

FFS

6.6.6.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

6.6.6.1.5	Test requirement

Table 6.6.6.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.6.1.5-1: NR Cell specific test parameters for SRS-RSRP event triggered reporting for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |
|  |  | 2 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |
|  |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |
|  |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

Table 6.6.6.1.5-2: NR Cell specific test parameters for SRS-RSRP event triggered reporting for neighbour cell UE

| Parameter | Unit | Test configuration | Neighbour cell UE |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
|  | dB | 1 | -infinity | 4 |
|  |  | 2 |  |  |
|  | dB | 1 | -infinity | 4+TT |
|  |  | 2 |  |  |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | -infinity | -94+TT |
|  |  | 2 | -infinity | -91+TT |
| Io | dBm/9.36 MHz | 1 | -70.05 | -64.59 |
|  | dBm/38.16 MHz | 2 | -63.96 | -58.50 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 6.6.6.1.5-3: SRS configuration for measurement reporting

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 12 | 12 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset | sl640, 4 | sl640, 9 |  |
|  | sequenceId | 0 | 0 | Any 10 bit number |

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 1920 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.7

### 6.6.8	L1-SINR measurement for beam reporting

#### 6.6.8.0	Minimum conformance requirements

##### 6.6.8.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

Same as clause 4.6.7.0.1

The normative reference for this requirement is TS 38.133 [6] clause 9.8.3, 9.8.4.1 and 9.8.5.2.

##### 6.6.8.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured

Same as clause 4.6.7.0.2

The normative reference for this requirement is TS 38.133 [6] clause 9.8.3, 9.8.4.2 and 9.8.5.

##### 6.6.8.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

Same as clause 4.6.7.0.3

The normative reference for this requirement is TS 38.133 [6] clause 9.8.3, 9.8.4.3 and 9.8.5.

#### 6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX

6.6.8.1.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.1.

6.6.8.1.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting L1-SINR measurement and long DRX cycle.

6.6.8.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.8.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.8.1.

6.6.8.1.4	Test description

6.6.8.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.8.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.8.1.4.1-2. Test environment parameters are given in Table 6.6.8.1.4.1-3.

Table 6.6.8.1.4.1-1: NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.8.1.4.1-2: General test parameters for NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.8.1.4.1-3: Test Environment parameters for NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.8.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.5 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.8.1.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.8.1.4.1-2 and 6.6.8.1.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.8.1.4.2	Test procedure

The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. UE is also configured to measure L1-SINR based on SSB. Upon receiving the DCI trigger, UE provides the report back based on the reporting configuration as defined in Table 6.6.8.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.8.1.5-1. T1 starts.

3.	After 80ms from the start of the test the SS transmits the DCI trigger in slot 1 for configuration 1, 2 and slot 8 for configuration 3. The corresponding CSI-RS set is transmitted with the offset of 4 slots after the DCI trigger.

4.	The SS shall check following requirements:

- R1:	the UE shall send L1-SINR report at slot 26 from the reception of DCI trigger. If the report is received at slot 26 from the reception of DCI trigger, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	The L1-SINR value of CSI-RS#1 reported by the UE is compared to the expected L1-SINR value for CSI-RS#1. If the resulting value is outside the limits in Table 6.6.8.1.5-2 for all test configurations or the UE fails to report the measurement value for CSI-RS #1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

- R3:	The DIFF SINR value of CSI-RS #0 reported by the UE is compared to the expected DIFF SINR value. If the resulting value is outside the limits in Table 6.6.8.1.5-4 or the UE fails to report the measurement value for CSI-RS#0, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one..

5.	If after T1 expiry no report is received or received report did not contain L1-SINR of both CSI-RS#0 and CSI-RS#1 or UE sent the L1-SINR report at different slot than 26 from the reception of DCI trigger, the number of ‘failed’ iterations is increased by one, otherwise, the number of ‘passed’ iterations is increased by one.

6.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.8.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.8.1.4.3-1: Common Exception messages NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions APERIODIC and CSI-SINR Table H.3.6A-2 with conditions CSI-RS and APERIODIC Table H.3.7-1 with condition DRX.3 |

Table 6.6.8.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

6.6.8.1.5	Test requirement

Table 6.6.8.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.8.1.5-1: CSI-RS specific test parameters for NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 and CSI-RS#1.

Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.8.1.5-2 for all test configurations and the corresponding relative accuracy requirements in Table 6.6.8.1.5-4 for all test configurations.

Table 6.6.8.1.5-2: L1-SINR absolute accuracy requirements for the reported values for all test configurations

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 41 |
| Highest reported value (CSI-RS#1) | 64 |

Table 6.6.8.1.5-3: Void

Table 6.6.8.1.5-4: L1-SINR relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF SINR reported (CSI-RS#0) | 0 |
| Highest DIFF SINR reported (CSI-RS#0) | 7 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R3) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others..

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX

6.6.8.2.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in non-DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.2.

6.6.8.2.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting L1-SINR measurement.

6.6.8.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.8.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.8.2.

6.6.8.2.4	Test description

6.6.8.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.8.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.8.2.4.1-2. Test environment parameters are given in Table 6.6.8.2.4.1-3.

Table 6.6.8.2.4.1-1: NR SA SSB based CMR and dedicated IMR L1-SINR measurement supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.8.2.4.1-2: General test parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | c |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.1A FDD |
|  | 2 |  | CSI-RS 1.1A TDD |
|  | 3 |  | CSI-RS 2.1A TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity-r16 | 1~3 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| L1-SINR reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.8.2.4.1-3: Test Environment parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.8.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4TBD |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.5 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.8.2.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.8.2.4.1-2, 6.6.8.2.5-1 and 6.6.8.2.5-2. UE is configured to perform RLM and BFD based on the SSBs.

6.6.8.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above Table 6.6.8.2.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-SINR measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.8.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.6.8.2.5-1 and Table 6.6.8.2.5-2. T1 starts. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 80 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in Table 6.6.8.2.5-1 and Table 6.6.8.2.5-2. T2 starts. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

5.	If the UE sends L1-SINR reports meeting the corresponding absolute accuracy requirements in Table 6.6.8.2.5-3 for test configurations 1 and 2 the corresponding absolute accuracy requirements in Table 6.6.8.2.5-4 for test configurations 3 and the corresponding relative accuracy requirements in Table 6.6.8.2.5-5 for all test configurations every 80 slots from no later than 720 ms for configuration 1 and 2 and no later than 680 ms for configuration 3 from the beginning of time period T2 until the end of time period T2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one. The UE shall start sending valid L1-SINR reports. The SS shall check following requirements:

- R1:	the UE shall start to transmit valid reports no later than 720 ms for configuration 1,2  and no later than 680 ms for configuration 3 from the beginning of time period T2. A valid report shall meet the absolute L1-SINR requirement for SSB#1 (Table 6.6.8.2.5-3 for all configurations) and the relative L1-SINR requirement for SSB#0 in Table 6.6.8.2.5-5. If the first valid report is received earlier than the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	the UE shall transmit reports every 80 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

- R3:	The L1-SINR value of SSB#1 reported by the UE is compared to the expected L1-SINR value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.8.2.5-3 for all configurations or the UE fails to report the measurement value for SSB#1, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-R4:	The DIFF SINR value of SSB#0 reported by the UE is compared to the expected DIFF SINR value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table6.6.8.2.5-5 for all configurations or the UE fails to report the measurement value for SSB#0, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.8.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.8.2.4.3-1: Common Exception messages NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and SS-SINR and CSI-RS_IMR Table H.3.6A-2 with conditions SSB and PERIODIC Table H.3.6A-3 with condition PERIODIC |

Table 6.6.8.2.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.8.2.5	Test requirement

Table 6.6.8.2.5-1 and Table 6.6.8.2.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.8.2.5-1: SSB specific test parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0.5 | 0.5 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.15 | -94.15 | -Infinity | -91.65 |
|  | 3 |  | -91.14 | -91.14 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.43 | -63.43 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.33 | -57.33 | -60.61 | -55.84 |
|  | 1~3 | dB | 0.5 | 0.5 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.8.2.5-2: CSI-RS specific test parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 |  | CSI-RS#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/CSI-RS SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0.5 | 0.5 | -Infinity | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/CSI-RS SCS | -94.15 | -94.15 | -Infinity | -91.65 |
|  | 3 |  | -91.14 | -91.14 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.43 | -63.43 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.33 | -57.33 | -60.61 | -55.84 |
|  | 1~3 | dB | 0.5 | 0.5 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-SINR report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-SINR report including results of both SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1. Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.8.2.5-3 for all test configurations and the corresponding relative accuracy requirements in Table 6.6.8.2.5-4 for all test configurations.

Table 6.6.8.2.5-3: L1-SINR absolute accuracy requirements for the reported values for all test configurations

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 44 |
| Highest reported value (SSB#1) | - | 61 |

Table 6.6.8.2.5-4: Void

Table 6.6.8.2.5-5: L1-SINR relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF SINR reported (SSB#0) | - | 0 |
| Highest DIFF SINR reported (SSB#0) | - | 5 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R4) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others..

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX

6.6.8.3.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in non-DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.3.

6.6.8.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting L1-SINR measurement.

6.6.8.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.8.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.8.3.

6.6.8.3.4	Test description

6.6.8.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.8.3.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.8.3.4.1-2. Test environment parameters are given in Table 6.6.8.3.4.1-3.

Table 6.6.8.3.4.1-1: NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.8.3.4.1-2: General test parameters for NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| CSI-IM configuration | 1 |  | CSI-IM.1.2 FDD |
|  | 2 |  | CSI-IM.1.2 TDD |
|  | 3 |  | CSI-IM.2.2 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.8.3.4.1-3: Test Environment parameters for NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.8.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.5 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.8.3.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.8.3.4.1-2 and 6.6.8.3.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.8.3.4.2	Test procedure

The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. UE is also configured to measure L1-SINR based on SSB. Upon receiving the DCI trigger, UE provides the report back based on the reporting configuration as defined in table 6.6.8.3.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.8.3.5-1. T1 starts.

3.	After 80ms from the start of the test the SS transmits the DCI trigger in slot 1 for configuration 1, 2 and slot 8 for configuration 3. The corresponding CSI-RS set is transmitted with the offset of 4 slots after the DCI trigger.

4.	The SS shall check following requirements:

- R1:	the UE shall send L1-SINR report at slot 26 from the reception of DCI trigger. If the report is received at slot 26 from the reception of DCI trigger, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	The L1-SINR value of CSI-RS#1 reported by the UE is compared to the expected L1-SINR value for CSI-RS#1. If the resulting value is outside the limits in Table 6.6.8.3.5-2 for all test configurations or the UE fails to report the measurement value for CSI-RS #1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

- R3:	The DIFF SINR value of CSI-RS #0 reported by the UE is compared to the expected DIFF SINR value. If the resulting value is outside the limits in Table 6.6.8.3.5-4 or the UE fails to report the measurement value for CSI-RS#0, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.5.	If after T1 expiry no report is received or received report did not contain L1-SINR of both CSI-RS#0 as CMR + CSI-IM#0 as IMR and CSI-RS#1 as CMR + CSI-IM#1 as IMR or UE sent the L1-SINR report at different slot than 26 from the reception of DCI trigger, the number of ‘failed’ iterations is increased by one, otherwise, the number of ‘passed’ iterations is increased by one.

6.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.8.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.8.3.4.3-1: Common Exception messages NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions APERIODIC and CSI-SINR and CSI-IM_IMR Table H.3.6A-2 with conditions CSI-RS and APERIODIC Table H.3.6A-4 with condition APERIODIC |

Table 6.6.8.3.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

6.6.8.3.5	Test requirement

Table 6.6.8.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.8.3.5-1: CSI-RS specific test parameters for NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
|  | 1~3 | dB | 0 | 3 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 as CMR + CSI-IM#0 as IMR and CSI-RS#1 as CMR + CSI-IM#1 as IMR.

Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.8.3.5-2 for all test configurations and the corresponding relative accuracy requirements in Table 6.6.8.3.5-4 for all test configurations.

Table 6.6.8.3.5-2: L1-SINR absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 43 |
| Highest reported value (CSI-RS#1) | 62 |

Table 6.6.8.3.5-3: Void

Table 6.6.8.3.5-4: L1-SINR relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF SINR reported (CSI-RS#0) | 0 |
| Highest DIFF SINR reported (CSI-RS#0) | 6 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R3) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.9	Idle Mode CA/DC Measurements

#### 6.6.9.0	Minimum conformance requirements

For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement reporting provided T331 has not expired, the serving cell is supporting idle mode CA/DC measurement reporting and the serving cell is in the validity area.

This subclause defines the requirements for the detected cell status for the idle mode CA/DC measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode. The requirements are applicable to an NE-DC and NR carrier aggregation capable UE which has been configured with one or more of following, one or more SCells, one E-UTRAN PSCell or one or more downlink E-UTRAN SCells during the Connected mode and which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16. The requirements are applicable for SCell(s) and E-UTRAN FDD and TDD PSCell and SCells.

Upon releasing the connection and if the UE has been configured with idle mode CA measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signalling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signalling, and

-	The detected cell and SSBs remains detectable until UE reconnect to the network and transmits the early measurement report, and

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-RAT E-UTRAN cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in TS 36.133 [23] Annex B.1.1 and Annex B.1.2 for a corresponding Band. An inter-frequency cell is considered detectable according to the conditions in TS 38.133 [6] Annex B.1.2 and B.1.3 for a corresponding band. An SSB of an inter-frequency cell is considered detectable according to SSB_RP and SSB Ês/Iot defined in TS 38.133 [6] Annex B.1.2 and B.1.3 for a corresponding Band.

While T331 is running, the UE shall perform measurement on the configured inter-frequency carriers for idle mode CA/DC measurement reporting according to the UE measurement capability.

A UE which supports idleInactiveNR-MeasReport-r16 shall support idle mode CA/DC measurements of:

-	at least 7 inter-frequency carriers which are also configured for inter-frequency mobility measurements, and

-	at least 7 inter-frequency carriers which are not configured for inter-frequency mobility measurements.

The UE shall be capable of monitoring a total of at least 7 inter-frequency carriers for idle mode CA/DC measurements comprising of carriers configured for inter-frequency mobility measurements and carriers not configured for inter-frequency mobility measurements.

For inter-frequency carriers configured for idle mode CA/DC measurements, if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ the inter-frequency measurement requirements in TS 38.133 [6] clause 4.2.2.4 shall apply, where UE shall search for and measure inter-frequency layers configured for idle mode CA/DC measurements in preparation for possible reporting. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements at least every Thigher_priority_search where Thigher_priority_search is described in TS 38.133 [6] clause 4.2.2.7, where UE shall search for and measure inter-frequency layers configured for idle mode CA/DC measurements in preparation for possible reporting.

For UE supporting idleInactiveNR-MeasBeamReport-r16, if the UE is configured with beamMeasConfigIdle-r16 for idle mode CA/DC measurement, the UE shall be capable of performing SS-RSRP, SS-RSRQ for at least

-	7 SSBs with different SSB index and/or PCI on an inter-frequency layer in FR1,

-	10 SSBs with different SSB index and/or PCI on an inter-frequency layer in FR2.

For UE supporting idleInactiveNR-MeasBeamReport-r16, if the UE is configured with beamMeasConfigIdle-r16 for idle mode CA/DC measurement, the UE shall be able to acquire the SSB index for a newly detectable inter-RAT NR cell and perform RSRP/RSRQ measurement within the requirements defined in TS 38.133 [6]  clause 4.2.2.4 plus TSSB_index,NR, where TSSB_index,NR is the additional time period used to acquire the index of the SSB being measured as defined in table 4.4.2.2-1.

Table 6.6.9.0-1: TSSB_index,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | TSSB_index,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |
| 0.32 | 1 | 8 | N2 x 1.28 x N1 x 1.5 (N2 x 4 x N1 x 1.5) |
| 0.64 |  | 5 | N2 x 1.28 x N1 (N2 x 2 x N1) |
| 1.28 |  | 4 | N2 x 1.28 x N1 (N2 x 1 x N1) |
| 2.56 |  | 3 | N2 x 2.56 x N1 (N2 x 1 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. NOTE 2: N2 = 3 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1, and N2 = 5 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR2. |  |  |  |

In the absence or expiration of T331, it is up to UE implementation to perform the idle mode CA/DC measurement.

For inter-frequency carriers configured for idle mode CA/DC measurements, the UE shall be capable of performing SS-RSRP and SS-RSRQ measurements of the carriers, and the UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the carriers configured for idle mode CA/DC measurements to higher layers, with measurement accuracy as specified in clauses [38.133] and [38.133], respectively.

The UE shall be able to report idle mode CA/DC measurements when idle mode CA/DC measurement reporting is requested by the network.

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in clause 4.2.2.2 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.2B, 10.1.3B, 10.1.7B and 10.1.8B.

#### 6.6.9.1	NR SA FR1 Idle mode CA/DC measurement for FR1

6.6.9.1.1	Test purpose

The purpose of this test is to verify that the UE performs the required measurements on the serving cell and the configured inter-frequency carrier for idle mode measurement reporting after the UE has entered Idle mode. This test will partly verify the Idle mode CA/DC measurements requirements in TS 38.133 [6] clause 4.4.

6.6.9.1.2	Test applicability

This test applies to all types of NR UE release 16 onwards, supporting 2DL CA and idleInactiveNR-MeasReport-r16.

6.6.9.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.9.0.

The normative reference for this requirement is TS 38.133 [6] clause 4.4and A.6.6.9.1

6.6.9.1.4	Test description

6.6.9.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.9.1.4.1-1.

Table 6.6.9.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.9.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.9.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.9.1-3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 6.6.9.1.4.1-2.

Table 6.6.9.1.4.1-2: Initial conditions for SA Idle mode CA/DC measurement for FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.9.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

1. The general test parameter settings are set up according to Table 6.6.9.1.4.1-3.

2. Message contents are defined in clause 6.6.9.1.4.3.

3. The test scenario comprises of two NR Cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.9.1.4.1-3: General test parameters for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 300 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells |
| T1 | s | Config 1,2,3 | 10 |  |
| T2 | s | Config 1,2,3 | 11.52 |  |
| T3 | s | Config 1,2,3 | 10 |  |

6.6.9.1.4.2	Test procedure

Two cells are deployed in the test, one FR1 NR PCell (Cell 1) on RF channel 1 and an FR1 NR neighbour cell (Cell 2) on RF channel 2. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. During T1, UE is connected to Cell 1 and has not any timing information about Cell 2. UE is configured with early measurement reporting with channel 2. Beam level reporting for early measurements is not configured.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.9.1.5-1. T1 starts.

3. When T1 expires, the SS shall transmit an RRCRelease message including measIdleCarrierListNR-r16 (Table 6.6.9.1.4.3-3) and switch the parameters according to T2 in Table 6.6.9.1.5-2. T2 starts.

4. Ensure the UE is in state RRC_IDLE until T2 expires. T3 starts.

6. The SS transmits in Cell a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in State RRC_CONNECTED according to TS 38.508-1 [14]clause 4.5.

7. During the connection setup the UE is reports idleMeasAvailable-r16 in the RRCSetupComplete message.

8. TE shall send the UEInformationRequest message, requesting UE to provide ide mode measurements results (Table 6.6.9.1.4.3-5).

9. If UE responds with UEInformationResponse message containing the idle mode measurement results with SS-RSRP values within the limits in Table 6.6.9.1.5-3 for configuration 1 and 2 and Table 6.6.9.1.5-4 for configuration 3, and SS-RSRQ values within the limits in Table 6.6.9.1.5-5 for configuration 1, 2 and 3, the number of passed iterations is increased by one. Otherwise, the number of failed iterations is increased by one.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.9.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.9.1.4.3-1: Common Exception messages for SA Idle mode CA/DC measurement for FR1

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.6.9.1.4.3-2: SIB1 for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleModeMeasurementsNR-r16 |  | TRUE | This field indicates that a UE that is configured for NR idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform NR idle/inactive measurements. |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.9.1.4.3-3: RRCRelease for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |  |
| measIdleCarrierListNR-r16 ::= SEQUENCE { |  |  | Serving cell not included |  |
| MeasIdleCarrierNR-r161 ::= SEQUENCE { |  |  |  |  |
| carrierFreq-r16 |  | ARFCN value of NR channel 2 |  |  |
| ssbSubcarrierSpacing-r16 |  | kHz15 | Config 1,2 |  |
|  |  | kHz30 | Config 3 |  |
| frequencyBandList |  | Not present |  |  |
| measCellListNR-r16 |  | Not present |  |  |
| reportQuantities-r16 |  | both |  |  |
| qualityThreshold-r16 |  | Not present |  |  |
| ssb-MeasConfig-r16 SEQUENCE { |  |  |  |  |
| nrofSS-BlocksToAverage-r16 |  | Not present |  |  |
| absThreshSS-BlocksConsolidation-r16 |  | Not present |  |  |
| smtc-r16 |  | SSB-MTC | TS 38.508-1 [14], Table 7.3.1-3 |  |
| ssb-ToMeasure-r16 |  | SSB-ToMeasure | TS 38.508-1 [14], Table 7.3.1-31 |  |
| deriveSSB-IndexFromCell-r16 |  | false |  |  |
|  |  | true |  | FR1_TDD, FR2_TDD |
| ss-RSSI-Measurement-r16 |  | Not present |  |  |
| } |  |  |  |  |
| beamMeasConfigIdle-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measIdleCarrierListEUTRA-r16 |  | Not present |  |  |
| measIdleDuration-r16 |  | sec300 |  |  |
| validityAreaList-r16 ::= SEQUENCE { |  |  |  |  |
| ValidityArea-r16 ::= SEQUENCE { |  |  |  |  |
| carrierFreq-r16 |  | ARFCN value of NR channel 1 |  |  |
| validityCellList-r16 ::= SEQUENCE { |  |  |  |  |
| PCI-Range |  | PhysCellId of Cell 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.9.1.4.3-4: RRCSetupComplete for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-22: |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| idleMeasAvailable-r16 |  | TRUE | Indication that the UE has idle/inactive measurement report available. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.9.1.4.3-5: UEInformationRequest

| Derivation Path: 38.508-1 14, Table 4.6.1-32A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |  |
| idleModeMeasurementReq-r16 |  | TRUE | UE shall report the idle/inactive measurement information, if available, to the network in the UEInformationResponse message. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.9.1.4.3-6: UEInformationResponse

| Derivation Path: 38.508-1 14, Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |  |
| measResultIdleNR-r16 ::= SEQUENCE { |  |  |  |  |
| rsrp-Result-r16 |  | INTEGER(0..127) | Integer value for RSRP measurements is according to Table 10.1.6.1-1 in TS 38.133 14 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.6.9.1.5	Test requirements

Tables 6.6.9.1.5-1 and 6.6.9.1.5-2 define the primary level settings including test tolerances and tables 6.6.9.1.5-3, 6.6.9.1.5-4, 6.6.9.1.5-5 and 6.6.9.1.5-6 define accuracy requirements define for SA Idle mode CA/DC measurement for FR1.

Table 6.6.9.1.5-1: Cell specific test parameters for connected mode for SA Idle mode CA/DC measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 |  | T3 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 | -91 | -91 | -infinity |  | -98 |  | -98 |
|  |  |  | Config 3 | -88 | -88 | -88 | -infinity |  | -95 |  | -95 |
|  |  | dB | Config 1,2,3,4,5,6 | 7 | 7 | 7 | - infinity |  | 0 |  | 0 |
|  |  | dB | Config 1,2,3 | 7 | 7 | 7 | infinity |  | 0 |  | 0 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -62.26 | -62.26 | -62.26 | -70.5 |  | -67.04 |  | -67.04 |
|  |  | dBm/38.16MHz | Config 3 | -56.15 | -56.15 | -56.15 | -63.94 |  | -60.93 |  | -60.93 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.6.9.1.5-2: Cell specific test parameters for idle mode for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1,2,3 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 | -infinity | 12 | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 7 | 7 | 7 | -infinity | 0 | 0 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  | 2 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  | 3 | -88 | -88 | -88 | -infinity | -95 | -95 |
| Io | dBm/9.36 MHz | 1 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  | dBm/9.36 MHz | 2 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  | dBm/38.16 MHz | 3 | -56.15 | -56.15 | -56.15 | -63.94 | -60.93 | -60.93 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  |  | Not sent |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table 6.6.9.1.5-3: Absolute accuracy requirements for Cell 1 and Cell 2 SS-RSRP reported values for test configurations 1 and 2

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 1) | 54 |
| Highest reported value (Cell 1) | 77 |
| Lowest reported value (Cell 2) | 47 |
| Highest reported value (Cell 2) | 70 |

Table 6.6.9.1.5-4: Absolute accuracy requirements for Cell 1 and Cell 2 SS-RSRP reported values for test configurations 3

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 1) | 57 |
| Highest reported value (Cell 1) | 80 |
| Lowest reported value (Cell 2) | 50 |
| Highest reported value (Cell 2) | 73 |

Table 6.6.9.1.5-5: Absolute accuracy requirements for Cell 1 and Cell 2 SS-RSRQ reported values for test configurations 1, 2 and 3

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 1) | 55 |
| Highest reported value (Cell 1) | 72 |
| Lowest reported value (Cell 2) | 51 |
| Highest reported value (Cell 2) | 67 |

During the time period T2 the UE is in Idle mode and the signal level of cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to Section 4.4.

At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for Cell 2 meets the requirements in Section 10.1.4B for SS-RSRP and 10.1.8B for SS-RSRQ and test ends.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

### 6.6.10 to 6.6.14

### 6.6.15	Idle Mode inter-RAT CA/DC Measurements

#### 6.6.15.0	Minimum conformance requirements

For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement reporting provided T331 has not expired, the serving cell is supporting idle mode CA/DC measurement reporting and the serving cell is in the validity area.

This subclause defines the requirements for the detected cell status for the idle mode CA/DC measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode. The requirements are applicable to an NE-DC and NR carrier aggregation capable UE which has been configured with one or more of following, one or more SCells, one E-UTRAN PSCell or one or more downlink E-UTRAN SCells during the Connected mode and which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16. The requirements are applicable for SCell(s) and E-UTRAN FDD and TDD PSCell and SCells.

Upon releasing the connection and if the UE has been configured with idle mode CA measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signalling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signalling, and

-	The detected cell and SSBs remains detectable until UE reconnect to the network and transmits the early measurement report, and

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-RAT E-UTRAN cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in TS 36.133 [23] Annex B.1.1 and Annex B.1.2 for a corresponding Band. An inter-frequency cell is considered detectable according to the conditions in TS 38.133 [6] Annex B.1.2 and B.1.3 for a corresponding band. An SSB of an inter-frequency cell is considered detectable according to SSB_RP and SSB Ês/Iot defined in TS 38.133 [6] Annex B.1.2 and B.1.3 for a corresponding Band.

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in clause 4.2.2.2 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.2B, 10.1.3B, 10.1.7B and 10.1.8B.

While T331 is running, the UE shall perform measurement on the configured inter-RAT carriers for idle mode CA/DC measurement reporting according to the UE measurement capability.

A UE which supports idleInactiveEUTRA-MeasReport-r16 shall support idle mode DC measurements of:

-	at least 7 E-UTRAN inter-RAT carriers which are also configured for E-UTRAN inter-RATmobility measurements, and

-	at least 1 E-UTRAN inter-RAT carrier which is not configured for E-UTRAN inter-RAT mobility measurements.

The UE shall be capable of monitoring a total of at least 7 inter-RAT carriers for idle mode CA/DC measurements comprising of carriers configured for inter-RAT mobility measurements and carriers not configured for inter-RAT mobility measurements.

For inter-RAT carriers configured for idle mode CA/DC measurements, if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ the inter-RAT measurement requirements in TS 38.133 [6] clause 4.2.2.5 shall apply, where UE shall search for and measure inter-RAT layers configured for idle mode CA/DC measurements in preparation for possible reporting. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ the UE shall search for inter-RAT layers configured for idle mode CA/DC measurements at least every Thigher_priority_search where Thigher_priority_search is described in TS 38.133 [6] clause 4.2.2, where UE shall search for and measure inter-RAT layers configured for idle mode CA/DC measurements in preparation for possible reporting.

For overlapping inter-RAT carriers configured for idle mode CA/DC measurements, the UE shall be capable of performing RSRP and RSRQ measurements of the carriers, and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the carriers configured for idle mode CA/DC measurements to higher layers, with measurement accuracy as specified in TS 36.133 [23] clauses in 9.1.3B.3 and 9.1.6B.2, respectively.

The UE shall be able to report idle mode CA measurements when idle mode CA measurement reporting is requested by the network.

#### 6.6.15.1	NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting

6.6.15.1.1	Test purpose

The purpose of this test is to verify that the UE properly retains the detected cell status for the idle mode CA/DC measurement when UE transitions from RRC Connected mode to Idle mode, when the UE has entered Idle mode. Additionally, test that the UE performs the required measurements on the serving cell and the configured inter-RAT carrier for idle mode measurement reporting. This test will partly verify the Idle mode CA measurements in TS 38.133 [6] clause 4.4. In the test, connected mode DRX configuration is not configured in either PCell or PSCell.

Additionally, the purpose of this test is to verify that the SS-RSRP, SS-RSRQ, RSRP and RSRQ measurement accuracy is within the specified limits. This test will verify the accuracy requirements in TS 38.133 [6] clauses 10.1.2B and 10.1.7B for intra-frequency measurements and clause 10.2.4 and 10.2.5 for the inter-RAT measurements.

6.6.15.1.2	Test applicability

This test applies to all types of NR UE release 16, supporting NE-DC and idleInactiveEUTRA-MeasReport-r16.

6.6.15.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.15.0.

The normative reference for this requirement is TS 38.133 [6] clauses 4.4, 10.1.2B, 10.1.7B, 10.2.4 and 10.2.5 and A.6.6.15

6.6.15.1.4	Test description

6.6.15.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.15.1.4.1-1.

Table 6.6.15.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.15.1-1 | FR1 FDD SSB SCS 15kHz BW 10MHz – LTE FDD 10MHz |
| 6.6.15.1-2 | FR1 FDD SSB SCS 15kHz BW 10MHz – LTE TDD 10MHz |
| 6.6.15.1-3 | FR1 TDD SSB SCS 30kHz BW 40MHz – LTE FDD 10MHz |
| 6.6.15.1-4 | FR1 TDD SSB SCS 30kHz BW 40MHz – LTE TDD 10MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.15.1.4.1-2.

Table 6.6.15.1.4.1-2: Initial conditions for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.15.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

1. The general test parameter settings are set up according to Table 6.6.15.1.4.1-3 and Table 6.6.15.1.4.1-4.

2. Message contents are defined in clause 6.6.15.1.4.3.

3. The test scenario comprises of two NR Cells. (Cell 1 and Cell 2). Cell 1 is configured according to Annex C.1.2 and C.1.3.

Table 6.6.15.1.4.1-3: General test parameters for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| PSCell |  | Cell 2 | PSCell on RF channel number 2 in LTE. Cell 2 is of higher priority. |
| DRX |  | OFF | For both PCell and PSCell once configured |
| PRACH configuration in Cell 2 |  | PRACH_2CE | PRACH configuration as specified in Clause A.3.16 in TS 36.133 |
| T331 | s | 300 |  |
| CSI reporting periodicity and offset configuration for Cell 2 | ms | 2 |  |
| T1 | s | 0.5 | During this time the PCell is known and PSCell is configured. |
| T2 | s | 0.5 | PSCell access. |
| T3 + T4 | s | 66 | During this time the UE is configured to perform inter-RAT measurements in idle mode on the PSCell carrier. |
| T5 | s | 0.5 | UE is paged and connection is setup. Network requests measurement report from the UE. |

Table 6.6.15.1.4.1-4: General idle mode test parameters for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Serving cell |  | 1, 2, 3, 4 | Cell1 | The UE camps on cell 1 which is the former PCell. |
| Neighbour cell |  | 1, 2, 3, 4 | Cell2 | The UE shall perform inter-RAT measurements on cell 2 which is the former PSCell. |
| RF Channel Number |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  | 1, 2, 3, 4 | 3 s | Synchronous cells |
| Access Barring Information | - | 1, 2, 3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  | 1, 2 | SSB.1 FR1 | Serving cell |
|  |  | 3, 4 | SSB.2 FR1 | Serving cell |
| SMTC configuration Serving cell |  | 1, 2, 3, 4 | SMTC.2 |  |
| DRX cycle length | s | 1, 2, 3, 4 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  | 1, 2, 3, 4 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  | 1, 2, 3, 4 | Not configured |  |
| T3 | s | 1, 2, 3, 4 | 0.5 | T3 needs to be defined so that cell measurement time is taken into account. |
| T4 | s | 1, 2, 3, 4 | 65 | T4 needs to be defined so that cell measurement time is taken into account. |

6.6.15.1.4.2	Test procedure

Two cells are deployed in the test, cell 1, which is the PCell in connected, and serving cell in idle mode, on radio channel 1 in FR1, and cell 2, which is the PSCell in connected, and measured LTE inter-RAT cell in idle mode, on radio channel 2 in LTE. The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. During T1 cell 2, the PSCell, shall be configured.

- Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

- The SS shall set the parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate. T1 starts.

- The SS then shall transmit RRCReconfiguration message with condition NE-DC according to TS 38.508-1 [6] Table 4.6.1-13 to add E-UTRA cell (PSCell).

- The UE shall transmit an RRCReconfigurationComplete message.

- The UE shall send a PRACH to PSCell during T1.

- After the UE has transmitted the random-access preamble on the PSCell, the SS shall set T2 parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate. T2 starts.

- After T2, the SS shall set T3 parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate and send the RRCRelease. T3 starts.

- During the time periods T3 and T4 the UE is in Idle mode configured to perform inter-RAT idle mode CA/DC measurements on Cell 2. The UE shall not perform reselection.

- After T3 is expired, the SS shall set T4 parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate. T4 starts.

- After T4 expires the SS shall set T5 parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate, T5 starts.

- The SS transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity). During the connection setup the UE is reports idleMeasAvailable-r16 in the RRCSetupComplete message.

- The SS shall send the UEInformationRequest message, requesting UE to report idle mode CA/DC measurements.

- If UE responds with UEInformationResponse message containing the idle mode measurement results with SS-RSRP values within the limits in Table 6.6.15.1.5-3 for configuration 1 and 2 and Table 6.6.15.1.5-4 for configuration 3, and SS-RSRQ values within the limits in Table 6.6.15.1.5-5 for configuration 1, 2 and 3, the number of passed iterations is increased by one. Otherwise, the number of failed iterations is increased by one.

- If paging in step 11 succeeds, go to step 15, otherwise switch off the UE.

- Switch on the UE and ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR with Connected without release On according to TS 38.508-1 [14] clause 4.5.

- Repeat step 2-15 until a test verdict has been achieved.

6.6.15.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.15.1.4.3-1: Common Exception messages for

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.6.15.1.4.3-2: SIB1 for NR SA FR1 Idle mode CA/DC measurement of inter-RAT DC candidate cells for early reporting

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleModeMeasurementsEUTRA-r16 |  | TRUE | This field indicates that a UE that is configured for EUTRA idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform EUTRA idle/inactive measurements. |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.15.1.4.3-3: RRCRelease for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |  |
| measIdleCarrierListNR-r16 ::= SEQUENCE { |  | Not present |  |  |
| measIdleCarrierListEUTRA-r16 ::= SEQUENCE { |  |  |  |  |
| MeasIdleCarrierEUTRA-r16 ::= SEQUENCE { |  |  |  |  |
| carrierFreqEUTRA-r16 |  | ARFCN value of Cell 2 |  |  |
| EUTRA-AllowedMeasBandwidth |  | mbw6 |  |  |
| measCellListEUTRA-r16 |  | Not present |  |  |
| reportQuantitiesEUTRA-r16 |  | both |  |  |
| qualityThresholdEUTRA-r16 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measIdleDuration-r16 |  | sec300 |  |  |
| validityAreaList-r16 ::= SEQUENCE { |  |  |  |  |
| ValidityArea-r16 ::= SEQUENCE { |  |  |  |  |
| carrierFreq-r16 |  | ARFCN value of NR channel 1 |  |  |
| validityCellList-r16 ::= SEQUENCE { |  |  |  |  |
| PCI-Range |  | PhysCellId of Cell 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.15.1.4.3-4: RRCSetupComplete for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-22: |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| idleMeasAvailable-r16 |  | TRUE | Indication that the UE has idle/inactive measurement report available. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.15.1.4.3-5: UEInformationRequest

| Derivation Path: 38.508-1 14, Table 4.6.1-32A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |  |
| idleModeMeasurementReq-r16 |  | TRUE | UE shall report the idle/inactive measurement information, if available, to the network in the UEInformationResponse message. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.15.1.4.3-6: UEInformationResponse

| Derivation Path: 38.508-1 14, Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |  |
| measResultIdleEUTRA-r16 ::= SEQUENCE { |  |  |  |  |
| measResultsPerCarrierListIdleEUTRA-r16 SEQUENCE |  |  |  |  |
| carrierFreqEUTRA-r16 |  | ARFCN of Cell 2 |  |  |
| measResultsPerCellListIdleEUTRA-r16 SEQUENCE |  |  |  |  |
| rsrp-ResultEUTRA-r16 |  | INTEGER (0..97) |  |  |
| rsrq-ResultEUTRA-r16 |  | INTEGER (-30..46) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.6.15.1.5	Test requirements

Tables 6.6.15.1.5-1 and 6.6.15.1.5-2 define the primary level settings including test tolerances and tables 6.6.15.1.5-3, 6.6. 15.1.5-4, 6.6. 15.1.5-5 and 6.6.15.1.5-6 define accuracy requirements define for Idle mode CA/DC measurement.

Table 6.6.15.1.5-1: Cell specific test parameters for NR cell for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | T1 | T2 | T3 |  |  |  | T4 |  |  |  | T5 |
| AoA setup |  | 1,2,3,4 | N/A |  |  |  |  |  |  |  |  |  |  |
| Assumption for UE beams Note 5 R: Rough |  | 1,2,3,4 | N/A | N/A | R |  |  |  | R |  |  |  | N/A |
| Frequency Range |  | 1,2,3,4 | FR1 |  |  |  |  |  |  |  |  |  |  |
| Duplex mode |  | 1, 2 | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | 3, 4 | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD Configuration 1: TDDConf.1.1 2: TDDConf.2.1 |  | 1,2 | - |  |  |  |  |  |  |  |  |  |  |
|  |  | 3,4 | 1 | 1 | 2 |  |  |  | 2 |  |  |  | 1 |
| BWchannel 1: 10: NRB,c = 52 2: 40: NRB,c = 106 | MHz | 1, 2 | 1 | 1 | - |  |  |  | - |  |  |  | 1 |
|  |  | 3, 4 | 2 | 2 | - |  |  |  | - |  |  |  | 2 |
| Initial Downlink BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Initial Uplink BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Dedicated Downlink BWP configuration 1: DLBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - |  |  | - |  |  |  |  | 1 |
| Dedicated Uplink BWP configuration 1: ULBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - |  |  | - |  |  |  |  | 1 |
| PDSCH Reference Measurement Channel 1: SR.1.1 FDD 2: SR.2.1 TDD | FDD | 1,2 | 1 | 1 | 1 |  |  | 1 |  |  |  |  | 1 |
|  | TDD | 3,4 | 2 | 2 | 2 |  |  | 2 |  |  |  |  | 2 |
| TRS configuration |  | 1,2,3,4 | - |  |  |  |  |  |  |  |  |  |  |
| TCI state |  | 1,2,3,4 | - |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET parameters | FDD | 1,2 | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  | TDD | 3,4 | CR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET parameters | FDD | 1,2 | CCR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  | TDD | 3,4 | CCR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG PatternsNote1 |  | 1,2,3,4 | OP.1 defined in A.3.2.1 |  |  |  |  |  |  |  |  |  |  |
| SSB configuration 1: SSB.1 FR1 2: SSB.2 FR1 |  | 1,2 | 1 |  |  |  |  |  |  |  |  |  |  |
|  |  | 3,4 | 2 |  |  |  |  |  |  |  |  |  |  |
| SMTC configuration |  | 1,2,3,4 | SMTC.2 |  |  |  |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3,4 | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PBCH to PBCH DMRS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDCCH DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of PDSCH to PDSCH |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| EPRE ratio of OCNG DMRS to SSS |  |  | 0 | 0 | - |  |  | - |  |  |  |  | 0 |
| Noc Note2 | dBm/ 15kHz | 1,2 | -98 | -98 | -98 |  |  | -98 |  |  |  |  | -98 |
|  |  | 3,4 |  |  |  |  |  |  |  |  |  |  |  |
| Noc Note2 | dBm/SCS | 1,2 | -98 | -98 | -98 |  | -98 |  |  |  |  | -98 |  |
|  |  | 3,4 | -95 | -95 | -95 |  | -95 |  |  |  |  | -95 |  |
| Ês/Iot | dB | 1,2,3,4 | 5 | 5 | 5 |  | 5 |  |  |  |  | 5 |  |
| Ês/Noc | dB | 1,2,3,4 | 5 | 5 | 5 |  | 5 |  |  |  |  | 5 |  |
| SS-RSRPNote3,4 | dBm/SCS | 1,2 | -93 | -93 | -93 |  | -93 |  |  |  |  | -93 |  |
|  |  | 3,4 | -90 | -90 | -90 |  | -90 |  |  |  |  | -90 |  |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2 | -63.85 | -63.85 | -63.85 | -63.85 |  |  |  |  | -63.85 |  |  |
|  | dBm/ 38.16 MHz | 3,4 | -57.76 | -57.76 | -57.76 | -57.76 |  |  |  |  | -57.76 |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | - | - | -140 |  |  |  |  | - |  |  |  |
|  | dBm/SCS | 3, 4 | - | - | -137 |  |  |  |  | - |  |  |  |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 |  |  |  | - |  |  |  |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 |  |  |  | - |  |  |  |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 |  |  |  | - |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | SS-RSRP |  |  |  |  |  |  |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  |  |  |  | - |  |  |  |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | Not sent |  |  |  |  | - |  |  |  |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | Not sent |  |  |  |  | - |  |  |  |
| Threshx, high | dB | 1,2,3,4 | - |  | 52 |  |  |  |  | - |  |  |  |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  |  |  |  | - |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.6.15.1.5-2: Cell specific test parameters for LTE cell for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Frequency Range |  | 1,2,3,4 | LTE |  |  |  |  |
| Duplex mode |  | 1, 3 | FDD |  |  |  |  |
|  |  | 2, 4 | TDD |  |  |  |  |
| BWchannel | MHz | 1,2,3,4 | 10 | 10 | - | - | 10 |
| Measurement bandwidth |  | 1,2,3,4 | - | - | 22-27 | 22-27 | - |
| PDSCH Reference Measurement Channel 1: R.1 FDD 2: R.1 TDD | FDD | 1,3 | 1 | 1 | - | - | 1 |
|  | TDD | 2,4 | 2 | 2 | - | - | 2 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 and A.3.1.2.2 in 36.133 1: R.6 FDD 2: R.6 TDD |  | 1,3 | 1 | 1 | - | - | 1 |
|  |  | 2,4 | 2 | 2 | - | - | 2 |
| OCNG Patterns defined in A.3.2.1.1 (OP.2 FDD) and A.3.2.1.2 (OP.2 TDD) in 36.133 1: OP.2 FDD 2: OP.2 TDD |  | 1,3 | 1 |  |  |  |  |
|  |  | 2,4 | 2 |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |
| PBCH_RA | dB | 1,2,3,4 | N/A | N/A | 0 | 0 | N/A |
| PBCH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PSS_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| SSS_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PCFICH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PHICH_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PHICH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PDCCH_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PDCCH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PDSCH_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PDSCH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| OCNG_RANote 1 |  |  | N/A | N/A | 0 | 0 | N/A |
| OCNG_RBNote 1 |  |  | N/A | N/A | 0 | 0 | N/A |
| Noc Note2 | dBm/ 15kHz | 1,2 | -98 | -98 | -98 | -98 | -98 |
|  |  | 3,4 |  |  |  |  |  |
| Ês/Iot | dB | 1,2,3,4 | 5 | 5 | -3 | 8 | 5 |
| Ês/Noc | dB | 1,2,3,4 | 5 | 5 | -3 | 8 | 5 |
| RSRPNote3,4 | dBm/SCS | 1,2,3,4 | -93 | -93 | -101 | -90 | -93 |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2, 3, 4 | -61.01 | -61.01 | -65.44 | -58.57 | -61.01 |
| Qrxlevmin | dBm/SCS | 1,2,3,4 | - | - | -140 |  | - |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | RSRP and RSRQ |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  | - |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | Not sent |  | - |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | Not sent |  | - |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table 6.6.15.1.5-3: Absolute accuracy requirements for the Cell 1 and Cell 2 RSRP reported values for test configurations 1 and 2

| Normal Conditions | T5 |
| --- | --- |
| Lowest reported value (Cell 1) | 52 |
| Highest reported value (Cell 1) | 75 |
| Lowest reported value (Cell 2) | 39 |
| Highest reported value (Cell 2) | 62 |

Table 6.6.15.1.5-4: Absolute accuracy requirements for the Cell 1 and Cell 2 RSRP reported values for test configurations 3

| Normal Conditions | T5 |
| --- | --- |
| Lowest reported value (Cell 1) | 55 |
| Highest reported value (Cell 1) | 78 |
| Lowest reported value (Cell 2) | 39 |
| Highest reported value (Cell 2) | 62 |

Table 6.6.15.1.5-5: Absolute accuracy requirements for the Cell 1 and Cell 2 RSRQ reported values for test configurations 1, 2 and 3.

| Normal Conditions | T5 |
| --- | --- |
| Lowest reported value (Cell 1) | 54 |
| Highest reported value (Cell 1) | 71 |
| Lowest reported value (Cell 2) | 6 |
| Highest reported value (Cell 2) | 27 |

During time durations T1 the UE shall start transmitting preamble on PSCell. During T2 the UE perform intra-frequency measurements on the PCell and the PSCell.

During the time-period T3 the connection is released, and UE enters idle mode. During the time period T3 and T4 the UE is camped in Idle mode and at T4 the signal level of cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode DC/CA measurement according to Section 4.4.

At the start of T5 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell including idle mode CA/DC measurement from cell 2.

After receiving the requested early measurement report, the test equipment verifies that the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in Sections 10.1.2B and 10.1.7B and Sections 10.2.4 and 10.2.5, respectively and test ends.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

### 6.6.16

### 6.6.17

### 6.6.18	SA event triggered reporting tests with concurrent gaps

#### 6.6.18.0	Minimum conformance requirements

##### 6.6.18.0.1	Minimum conformance requirements for Intra-frequency measurement

The UE shall be able to identify a new detectable intra-frequency cell within the time period specified in TS 38.133 [6].

Intra-frequency measurements without measurement gaps:

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

Kp = 1 when Navailable = 0.

-	Otherwise, when UE is not configured with or UE does not support concurrent measurement gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp =1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with the ML of NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [13]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

If the higher layer signalling in TS38.331 [13] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, time period requirements are not specified to identify a new detectable intra-frequency cell.

Intra-frequency measurements with measurement gaps:

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps or not supporting [concurrent measurement gaps]. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for measurement gap collisions by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

When concurrent measurement gaps are configured, requirements in this clause do not apply ifNavailable =0.

If the higher layer signalling in TS 38.331 [13] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, time period requirements are not specified for identify a new detectable intra-frequency cell.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.3, 9.2.5 and 9.2.6.

##### 6.6.18.0.2	Minimum conformance requirements for Inter-frequency measurement

The UE shall be able to identify a new detectable inter-frequency cell within the time period specified in TS 38.133 [6].

Inter-frequency measurements without measurement gaps:

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is a scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the SSB frequency layer, and starting at the beginning of any SMTC occasion:

Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG occasions within the window, and

Navailable is the number of SMTC occasions that are not overlapped with any MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule provided that concurrent measurement gaps are configured.

Kp = 1 when Navailable = 0.

Otherwise, when UE is not configured with or UE does not support concurrent measurement gaps:

When inter-frequency SMTC is fully non overlapping with measurement gaps or inter-frequency SMTC is fully overlapping with MGs, Kp =1.

When inter-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When inter-frequency SMTC is partially overlapping with the VIL of NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP.

Inter-frequency measurements with measurement gaps:

Kgap is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gap(s) and per-FR measurement gap(s) within the same FR, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W, after accounting for collisions between the measurement gaps by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

Kgap is only applicable for UE supporting concurrentMeasGap-r17. When concurrent measurement gaps are configured, requirements in this clause do not apply if Navailable =0.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.4, 9.3.5, 9.3.6.3, 9.3.9.

##### 6.6.18.0.3	Minimum conformance requirements for Inter-RAT measurement

The requirements are application for NR−E-UTRAN FDD and NR−E-UTRAN TDD when DRX is used or not.

When the UE requires measurement gaps to identify and measure inter-RAT cells and an appropriate measurement gap pattern is scheduled, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are scheduled, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable FDD or TDD cell within the time period specified in TS 38.133 [6].

For a UE supporting and configured with concurrent measurement gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent measurement gaps.

For a window W of duration MGRP_max, where MGRP_max is the maximum MGRP across all configured per-UE measurement gap(s) and per-FR measurement gap(s) for FR1, and starting from the beginning of any associated gap occasion:

Ntotal is the total number of associated gap occasions within the window, including those overlapped with other MG occasions within the window, and

Navailable is the number of non-dropped associated measurement gap occasions after accounting for collisions between the measurement gaps by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

Requirements do not apply for UE configured with concurrent measurement gaps, if Navailable=0

The normative reference for this requirement is TS 38.133 [6] clause 9.4.2, 9.4.3.

##### 6.6.18.0.4	Minimum conformance requirements for PRS measurement

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP specified in TS 37.355 [38], the UE shall be able to measure multiple (up to the UE capability specified in TS 38.133 [6] Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [10], during the measurement period defined in TS 38.133 [6] Clause 9.9.2.5

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

-	For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including those overlapped with other MG occasions within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable =0.

The normative reference for this requirement is TS 38.133 [6] clause 9.9.2.

#### 6.6.18.1	NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers

Editor's Note: This test case is incomplete in following aspects:

- Concurrent gap configuration is missing in Annex Table H.3.1-6

- TT analysis is missing.

- Initial conditions and message contents contain []

6.6.18.1.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting for concurrent gaps non-overlap with SSB-based measurement in both inter-frequency layers within inter-frequency cell search requirements in TS 38.133 [6] clause 9.3.4.

6.6.18.1.2	Test applicability

This test applies to all types of NR UEs supporting concurrent gap capability from release 17 onwards.

6.6.18.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.18.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.18.1.

6.6.18.1.4	Test description

6.6.18.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.18.1.4.1-1.

Table 6.6.18.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.6.18.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.18.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.18.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Test environment parameters are given in Table 6.6.18.1.4.1-2.

Table 6.6.18.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.18.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.18.1.4.1-3.

2. Message contents are defined in clause 6.6.18.1.4.3.

3. There are three NR carriers and three cells specified in the test. Cell 1 (NR cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.18.1.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overlapping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cells 2 and 3 |  | NR cell 2 is on NR RF channel number 2. NR cell 3 is on NR RF channel number 3. |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #0 1 for MeasGapId #1 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 for MeasGapId #0 19 for MeasGapId #1 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cell 1 |  | Config 1, 2, 3 | 3s |  | Synchronous. |
| Time offset between serving and neighbour cell 2 |  | Config 1, 2, 3 | [5ms] |  | [Asynchronous. The timing of Cell 3 is 5ms later than the timing of Cell 1.] |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1.5 |  |

6.6.18.1.4.2	Test procedure

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 (Cell 2) as neighbour cell 1 in FR1 on NR RF channel 2, and NR cell 3 (Cell 3) as neighbour cell 2 in FR1 on NR RF channel 3. The test parameters are given in Table 6.6.18.1.4.1-3 and Table 6.6.18.1.5-1 respectively.

Two measurement gap patterns (MeasGapId #0 and MeasGapId #1) are configured with the gap pattern ID #0 and #1 as defined in Table 6.6.18.1.4.1-3. MeasGapId #1 is configured with a higher priority than MeasGapId #0. MeasGapId #0 and MeasGapId #1 are associated with the MOs for RF channel numbers #2 and #3 respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.18.1.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.18.1.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If Cell2’s overall delays measured from the beginning of time period T2 is less than 920 ms, and Cell3’s overall delays measured from the beginning of time period T2 is less than 1280 ms, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop. And set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.18.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.18.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with conditions [TBD] Table H.3.1-7 with condition INTER-FREQ |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Cell 3 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |

6.6.18.1.5	Test requirement

Table 6.6.18.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.18.1.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overlapping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for each neighbouring cell, with a measurement reporting delay less than 920 ms for cell 2 and 1280ms for cell 3 from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.18.2	NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers

Editor's Note: This test case is incomplete in following aspects:

- Concurrent gap configuration is missing in Annex Table H.3.1-6

- TT analysis is missing.

- Message contents and test requirement contain []

6.6.18.2.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting for concurrent gaps partially-overlap with SSB-based measurement in both inter-frequency layers within inter-frequency cell search requirements in TS 38.133 [6] clause 9.3.4.

6.6.18.2.2	Test applicability

This test applies to all types of NR UEs supporting concurrent gap capability from release 17 onwards.

6.6.18.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.18.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.18.2.

6.6.18.2.4	Test description

6.6.18.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.18.2.4.1-1.

Table 6.6.18.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.6.18.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.18.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.18.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Test environment parameters are given in Table 6.6.18.2.4.1-2.

Table 6.6.18.2.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.18.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.18.2.4.1-3.

2. Message contents are defined in clause 6.6.18.2.4.3.

3. There are three NR carriers and three cells specified in the test. Cell 1 (NR cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.18.2.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partial overlapping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cells 2 and 3 |  | NR cell 2 is on NR RF channel number 2. NR cell 3 is on NR RF channel number 3. |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset | ms | Config 1,2,3 | 39 for MeasGapId #1 4 for MeasGapId #2 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cell 1 |  | Config 1, 2, 3 | 3s |  | Synchronous. |
| Time offset between serving and neighbour cell 2 |  | Config 1, 2, 3 | 5ms |  | Asynchronous. The timing of Cell 3 is 5ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1.5 |  |

6.6.18.2.4.2	Test procedure

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 (Cell 2) as neighbour cell 1 in FR1 on NR RF channel 2, and NR cell 3 (Cell 3) as neighbour cell 2 in FR1 on NR RF channel 3.  The test parameters are given in Table 6.6.18.2.4.1-3, and Table 6.6.18.2.5-1 respectively. The TE schedules continuous DL data on PCell throughout the test.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #1 as defined in Table 6.6.18.2.4.1-3. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #2 and #3, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.18.2.5-1.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission in Cell 1.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.18.2.5-1. T2 starts.

7. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1280 ms for both Cell 2 and Cell 3, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

8. After the SS receives the MeasurementReport message in step 7 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

9.  Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop. And set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR:
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.18.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.18.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with condition [TBD] Table H.3.1-7 with condition INTER-FREQ |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Cell 3 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition [SMTC.Y] |

6.6.18.2.5	Test requirement

Table 6.6.18.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.18.2.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partial-overlapping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | [SMTC.Y] |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for each neighbouring cell, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.18.3	NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

Editor's Note: This test case is incomplete in following aspects:

- Concurrent gap configuration is missing in Annex Table H.3.1-6

- TT analysis is missing.

- Initial conditions and message contents contain []

6.6.18.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct event-triggered reporting of concurrent inter-RAT E-UTRAN and NR FR1 measurements when operating in standalone (SA) operation with PCell in FR1.

6.6.18.3.2	Test applicability

This test applies to all types of NR UEs supporting concurrent gap capability from release 17 onwards.

6.6.18.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.18.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.18.3.

6.6.18.3.4	Test description

6.6.18.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.18.3.4.1-1.

Table 6.6.18.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.18.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD 10MHz |
| 6.6.18.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD 10MHz |
| 6.6.18.3-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD 10MHz |
| 6.6.18.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD 10MHz |
| 6.6.18.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD 10MHz |
| 6.6.18.3-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD 10MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in Table 6.6.18.3.4.1-2.

Table 6.6.18.3.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.18.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.18.3.4.1-3.

2. Message contents are defined in clause 6.6.18.3.4.3.

3. There are two NR carriers and one E-UTRA cell specified in the test. Cell 1 (NR cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.18.3.4.1-3: General test parameters for NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1 - 6 | 1, 2 | 2 NR carrier frequency is used in the test. 1 FR1 and FR2 carrier frequency. |
| LTE RF Channel Number |  |  |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | 1 - 6 | As specified in Tables A.6.6.3.1.1-2 and A.6.6.3.1.1-3. |  |
| Active cell |  | 1 - 6 | NR Cell 1 (PCell in FR1) | Cell 1 is on RF channel number 1 |
| Neighbour cell 1 |  | 1 - 6 | Cell 2 (NR FR2) | Cell 2 is on RF channel number 2 |
| Neighbour cell 2 |  | 1 - 6 | Cell 3 (LTE) | Cell 3 is on RF channel number 3 |
| Gap Pattern Id #1 |  | 1 - 6 | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| Gap Pattern Id #2 |  | 1 - 6 | 0 | Gap Pattern Id #2 is offset 20ms compared to Gap Pattern Id #1 |
| Measurement gap offset |  | 1 – 6 | 19 for Gap pattern Id #1 |  |
| NR measurement quantity |  |  | SS-RSRP | Measurement quantity for Cell 1 |
| NR measurement quantity |  |  | SS-RSRP | Measurement quantity for Cell 2 |
| Inter-RAT E-UTRAN measurement quantity |  |  | RSRP | Measurement quantity for Cell 3 |
| CSI-RS for tracking parameters on NR RF Channel 1 |  | 1, 4 | TRS.1.1 FDD |  |
|  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  | 3, 6 | TRS.1.2 TDD |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | 1, 4 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | 2, 5 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | 3, 6 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | 1 - 6 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| b2-Threshold1 | dBm |  | Note 1 | SS-RSRP threshold for SS-RSRP measurement on Cell 3 for event B2 |
| b2-Threshold2EUTRA | dBm |  | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on Cell 3 for event B2 |
| Hysteresis | dB | 1 – 6 | 0 |  |
| TimeToTrigger | s | 1 – 6 | 0 |  |
| Filter coefficient |  | 1 – 6 | 0 | L3 filtering is not used |
| offsetMO | dB | 1 - 6 | 6 | NR Cell 2 |
| a4-Threshold | dBm | 1 – 6 | -105 | NR Cell 2 |
| DRX |  |  | OFF | OFF |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms | Asynchronous cells NR cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s | Synchronous NR cells. |
| T1 | s | 1 – 6 | 5 | for LTE Cell 3 and NR FR2 Cell 2 |
| T2 | s | 1 - 6 | 5 | for LTE Cell 3 |
|  |  |  | 5.2 for PC1; 3.5 for other PC | for NR FR2 Cell 2 |

6.6.18.3.4.2	Test procedure

In this test, there are three cells: Cell 1 is the NR PCell, Cell 2 is an Inter-frequency NR FR1 neighbour cell on NR RF channel 2 and Cell 3 is an inter-RAT E-UTRAN neighbour cell. The test parameters are given in Table 6.6.18.3.4.1-3 and Tables 6.6.18.3.5-1/2 respectively.

In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used for the E-UTRAN cell (cell 3). In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used for the NR FR1 cell (cell 2).

Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2 and Cell 3.

Two concurrent per-UE measurement gap pattern configurations # 0 as defined in Table 6.6.18.3.4.1-3 are provided for a UE. Two measurement gap patterns (MeasGapId #0 and MeasGapId #1) are configured with the gap pattern ID #0 as defined in Table 6.6.18.3.4.1-3. MeasGapId #1 is configured with a higher priority than MeasGapId #0. MeasGapId #0 and MeasGapId #1 are associated with the MOs for RF channel numbers #1 and #2, respectively.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Tables 6.6.18.3.5-1/2.

3. SS shall transmit an RRCReconfiguration message to configure Event A4 triggered measurement report for Cell 2, Event B2 triggered measurement report for Cell 3 to the PCell and two concurrent per-UE measurement gap patterns.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Tables 6.6.18.3.5-1/2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A4. If Cell2’s overall delays measured from the beginning of period T2 is less than 5120 ms for UE supporting power class 1, or 3200 ms for UE supporting other power class, and Cell3’s overall delays measured from the beginning of period T2 is less than 3.84 s, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop. And set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.18.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.18.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-RAT and GAP NEEDED Table H.3.1-4 Table H.3.1-5 with condition INTER-RAT Table H.3.1-6 with conditions [TBD] Table H.3.1-7 with condition INTER-RAT |
| Specific message content exceptions for Test Configurations 6.6.18.3-1 and 6.6.18.3-4 | Table H.3.1-3 with conditions INTER-RAT MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message content exceptions for Test Configurations 6.6.18.3-2, 6.6.18.3-3, 6.6.18.3-5 and 6.6.18.3-6 | Table H.3.1-3 with condition INTER-RAT MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |

6.6.18.3.5	Test requirement

Tables 6.6.18.3.5-1/2 define the primary level settings including test tolerances for all tests.

Table 6.6.18.3.5-1: Cell specific test parameters of NR cells for NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | N/A |  | Setup 1 as specified in clause A.3.15 |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 52 |  | 66 |  |
|  |  |  | Config 2 | 52 |  | 66 |  |
|  |  |  | Config 3 | 106 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1,2,3 |  |  | -Infinity | -87 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2 |  |  | -Infinity | -87 |
|  |  | Note5 | Config 3 |  |  | -Infinity | -87 |
| BB Note 8 |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 14.69 |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 | A.3.7A |  | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table 6.6.18.3.5-2: Cell specific test parameters LTE cell for NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

| Parameter | Unit | Configuration | Cell 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.22+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

The UE shall send one Event A4 triggered measurement report for Cell 2, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

The UE shall send one Event B2 triggered measurement report for Cell 3 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

Editor's Note: This test case is incomplete in following aspects:

- Concurrent gap configuration is missing in Annex Table H.3.1-6

- TT analysis is missing.

- Initial conditions and message contents contain []

6.6.18.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event.

6.6.18.4.2	Test applicability

This test applies to all types of NR UEs supporting concurrent gap and independent measurement gap configurations for PRS measurement capability from release 17 onwards.

6.6.18.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.18.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.18.4.

6.6.18.4.4	Test description

6.6.18.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.18.4.4.1-1.

Table 6.6.18.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.18.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.18.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.18.4-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in Table 6.6.18.4.4.1-2.

Table 6.6.18.4.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.18.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.18.4.4.1-3.

2. Message contents are defined in clause 6.6.18.4.4.3.

3. There are three NR carriers specified in the test. Cell 1 (NR cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.18.4.4.1-3: General test parameters for NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1,2,3 | 1: Cell 1 and Cell 3 2: Cell 2 | Two TDD carrier frequencies are used for the NR cells. |
| Active cell |  | 1,2,3 | NR cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1,2,3 | NR cell 2, NR cell 3 | Cell 2 is an inter-frequency cell neighbour cell Cell 3 is a neighbour cell in the positioning assistance data. |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |
|  |  | 2 | 10: NRB,c = 52 |  |
|  |  | 3 | 40: NRB,c = 106 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #0 24 for MeasGapId #1 |  |
| Measurement gap offset | ms | Config 1,2,3 | 7 for MeasGapId #0 11 for MeasGapId #1 |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

6.6.18.4.4.2	Test procedure

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as neighbour cell in FR1 on NR RF channel 2 and NR cell 3 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Table 6.6.18.4.4.1-3 and Table 6.6.18.4.5-1 respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3. Cell 1 and cell 3 transmit PRS during T2.

Two measurement gap patterns (MeasGapId #0 and MeasGapId #1) are configured with the gap pattern ID #0 and #1 as defined in Table 6.6.18.4.1-3. MeasGapId #1 is configured with a higher priority than MeasGapId #0. MeasGapId #0 and MeasGapId #1 are associated with the MOs for RF channel numbers #1 and #2, respectively.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 [38] shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance of MeasGapId #1 containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.18.4.5-1.

3. SS shall transmit an RRCReconfiguration message to configure Event A3 triggered measurement report and two independent measurement gap patterns for PRS measurement.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.18.4.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If Cell2’s overall delays measured from the beginning of period T2 is less than 1840 ms, and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop. And set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.18.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.18.4.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with conditions [TBD] Table H.3.1-7 with condition INTER-FREQ |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Cell 3 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |

6.6.18.4.5	Test requirement

Tables 6.6.18.4.5-1 define the primary level settings including test tolerances for all tests.

Table 6.6.18.4.5-1: Cell specific test parameters of NR cells for NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |  | T1 | T2 |
| TDD configuration |  |  | 1 | N/A |  | N/A |  |  | N/A |  |
|  |  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  |  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| BWchannel |  | MHz | Config 1,2,3 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1,2,3 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |  | OP.1 |  |
| PDSCH RMC configuration |  |  | 1 | SR.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  |  | 1 | CR.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | CR.1.1 TDD |  | N/A |  |  | N/A |  |
|  |  |  | 3 | CR.2.1 TDD |  | N/A |  |  | N/A |  |
| Dedicated CORESET RMC configuration |  |  | 1 | CCR.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | CCR.1.1 TDD |  | N/A |  |  | N/A |  |
|  |  |  | 3 | CCR.2.1 TDD |  | N/A |  |  | N/A |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR2 |  | SSB.1 FR2 |  |  | SSB.1 FR2 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.4 |  | SMTC.4 |  |  | SMTC.4 |  |
| TRS Configuration |  |  | 1 | TRS.1.1 FDD |  | N/A |  |  | N/A |  |
|  |  |  | 2 | TRS.1.1 TDD |  | N/A |  |  | N/A |  |
|  |  |  | 3 | TRS.1.2 TDD |  | N/A |  |  | N/A |  |
| PRS configuration |  |  | 1 | PRS.1.4 FR1 |  | N/A |  |  | PRS.1.4 FR1 |  |
|  |  |  | 2 | PRS.1.4 FR1 |  | N/A |  |  | PRS.1.4 FR1 |  |
|  |  |  | 3 | PRS.2.4 FR1 |  | N/A |  |  | PRS.2.4 FR1 |  |
| PRS muting configuration |  |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  |  | ‘01’ |  |
| Note2 |  | dBm/15kHz Note5 | 1 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 2 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 3 | -98 |  | -98 |  |  | -98 |  |
| Note2 |  | dBm/SCS Note4 | 1 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 2 | -98 |  | -98 |  |  | -98 |  |
|  |  |  | 3 | -95 |  | -95 |  |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | 1 | -94 | -94 | -Infinity |  | -94 | N/A | N/A |
|  |  |  | 2 | -94 | -94 | -Infinity |  | -94 | N/A | N/A |
|  |  |  | 3 | -91 |  |  |  |  |  |  |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | 1 | -Infinity | -101 | N/A |  | N/A | -Infinity | -108 |
|  |  |  | 2 | -Infinity | -101 | N/A |  | N/A | -Infinity | -108 |
|  |  |  | 3 | -Infinity | -98 | N/A |  | N/A | -Infinity | -105 |
| PRS |  | dB | 1,2,3 | -Infinity | -3 | N/A |  | N/A | -Infinity | -10 |
| PRS |  | dB | 1,2,3 | -Infinity | -3 | N/A |  | N/A | -Infinity | -10 |
| IoNote3 |  | dBm/95.04 MHz Note5 | 1 | -62.25 |  | -64.60 |  | -62.25 | -62.25 |  |
|  |  |  | 2 | -62.25 |  | -64.60 |  | -62.25 | -62.25 |  |
|  |  |  | 3 | -56.16 |  | -58.50 |  | -56.16 | -56.16 |  |
| Propagation Condition |  |  | 1,2,3 | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for cell 2, with a measurement reporting delay less than 1840 ms from the beginning of period T2.

The PRS RSRP measurement time fulfils the requirements specified in TS 38.133[6] Clause 9.9.3.5. The UE shall perform and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in TS 38.133[6] section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90% with a confidence level of 95%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in TS 38.133[6] Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

IUE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.19

### 6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation

Editor’s note: This test case is incomplete. The following aspects are missing or TBD:

- Test purpose contains []

- Connection diagram is TBD

- Measurement Uncertainties and Test Tolerances are missing

#### 6.6.20.0	Minimum conformance requirements

[38.133 clause 9.12.4.1, 9.12.5 and 10.1.25.3.1] for measurement time

When UE is configured with prs-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with PRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_PRS, where:

TUERxTx, PRS=NPRSslotN'*Nsample*Klayer3*LCM(TPRS, TDRX)

Where:

NPRSslot is the maximum number of DL PRS resources configured in a slot,

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumberPRS-ResourceProcessedPerSlot-r17 as specified in TS38.331 [2],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

TPRS is the PRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length when DRX is in use, 1ms otherwise.

Klayer3 is

Ntotal / Navailable, when Navailable>0

Where,

-	For a window W of duration LCM(TPRS, MGRP, TSMTC), where TSMTC is the periodicity of SMTC for intra-frequency measurement without gap, starting at the beginning of any PRS resource occasion:

-	Ntotal is the total number of PRS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Navailable is the number of PRS resource occasions that are not overlapped with:

-	For FR1: any measurement gap occasions within the window W

No requirements shall apply if Navailable=0.

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

If PRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the

measurement report mapping requirements specified below.

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024×Tc to 985024×Tc with the resolution step of 2k×Tc.

Table 6.6.20.0-1: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

[TS 38.133 clause 10.1.39.2] for measurement accuracy

The error in the reported value of UE Rx-Tx time difference measurement, including both the measurement error and the reporting quantization error, should be within the accuracy requirements specified in this clause.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one PDC DL PRS resource from the serving cell (PCell).

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in Table 6.6.20.0-2 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex [TBD] for a corresponding Band.

AWGN propagation condition.

Table 6.6.20.0-2: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± [78+d+ℇ] | -3 | ≥[24] | 15 | ≥[4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| ± [59+d+ℇ] |  | ≥[52] |  | ≥[1] | Note 6 | NOTE 6 | NOTE 6 |
| ± [30+d+ℇ] |  | >[104] |  | ≥[1] | Note 6 | NOTE 6 | NOTE 6 |
| ± [57+d+ℇ] |  | ≥[24] | 30 | ≥[4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -117 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -115 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 |  |
| ± [30+d+ℇ] |  | ≥[48] |  | ≥[1] | NOTE 6 | NOTE 6 | NOTE 6 |
| ± [15+d+ℇ] |  | ≥[132] |  | ≥[1] | NOTE 6 | NOTE 6 | NOTE 6 |
| ± [29+d+ℇ] |  | ≥[24] | 60 | ≥[4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -114 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 |  |
| ± [15+d+ℇ] |  | ≥[64] |  | ≥[1] | NOTE 6 | NOTE 6 | NOTE 6 |
| ± [7+d+ℇ] |  | ≥[132] |  | ≥[1] | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [38]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.39.2-3. NOTE 8: ℇ is the margin for reporting quantization error and ℇ=16 Tc. |  |  |  |  |  |  |  |

Table 6.6.20.0-3: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | [160] |
| ≥ 52 | ≥ 24 | N/A | [80] |
| ≥ 104 | ≥ 48 | ≥ 24 | [56] |
| N/A | ≥ 132 | ≥ 64 | [24] |
| N/A | N/A | ≥ 132 | [24] |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

#### 6.6.20.1	UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA

6.6.20.1.1	Test purpose

The purpose of the test is to verify that the UE Rx-Tx measurement for RTT-based PDC meets the requirements specified in TS 38.133[6] clause 9.12.4.1 for measurement delay and clause 10.1.X for measurement accuracy in AWGN propagation condition in FR1 in standalone scenario.

6.6.20.1.2	Test applicability

This test applies to all types of NR UE release 17 onwards supporting rtt-BasedPDC-PRS-r17 or gNB-SideRTT-BasedPDC-r17.

6.6.20.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.20.0.

The normative reference for this requirement is TS 38.133 [6] clauses 9.12.4.1, 9.12.5, 10.1.39.2 and A.6.6.20.

6.6.20.1.4	Test description

6.6.20.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.20.1.4.1-1

Table 6.6.20.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.20-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.20-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.20-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.20.1.4.1-2.

Table 6.6.20.1.4.1-2: Initial conditions for UE Tx-Rx time difference measurement for FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.20.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram |  |  |  |

1. The general test parameter settings are set up according to Table 6.6.20.1.4.1-3.

2. Message contents are defined in clause 6.6.20.1.4.3.

3. The test scenario comprises of 1 NR Cell. The connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.20.1.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |
| --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 1 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |
|  |  | 2 | 10: NRB,c = 52 |
|  |  | 3 | 40: NRB,c = 106 |
| SSB configuration |  | 1 | SSB.1 FR1 |
|  |  | 2 | SSB.1 FR1 |
|  |  | 3 | SSB.2 FR1 |
| SMTC configuration |  | 1 | SMTC.2 |
|  |  | 2 | SMTC.1 |
|  |  | 3 | SMTC.1 |
| CP length |  | 1, 2, 3 | Normal |
| DRX |  | 1, 2, 3 | OFF |
| T1 | s | 1, 2, 3 | 5 |
| T2 | s | 1, 2, 3 | 10 |

6.6.20.1.4.2	Test procedure

The test is considered with one cell (Cell 1) in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. The Cell 1 mutes PRS transmission during T1 and transmits PRS during T2.

measObject  with measObjectRxTxDiff-r17, and NR-DL-PRS-PDC-Info is defined in as per 6.6.20.1.4.3 and shall be provided to the UE during T1.

The last TTI containing the RRC configuration shall be provided to the UE DT ms before the start of T2, where DT = [TBD] ms. The UE is configured to transmit SRS during T2.

The general and cell specific test parameters for PCell is given in Table 6.6.20.1.4.1-3 and Table 6.6.20.1.5-1, respectively. In the measurement control information, a measurement object is configured for the frequency of the PCell, and. The test consists of two successive time periods, with time duration of T1, and T2 respectively.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.20.1.5-1 as appropriate.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.20.1.5-1. T2 starts.

6. The UE shall transmit periodically MeasurementReport messages.

7. After 10s wait from Step 3, the SS shall check the result-k5-r17 reported values in the periodic MeasurementReport. If the result-k5 value is outside the limits in Table 6.6.20.0-1 or the UE fails to report the measurement value for serving cell, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

8. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6.6.20.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table 4.6.3-76B in 38.508-1 [14] with condition PRS_REF |
| Specific message contents exceptions for Test Configuration 6.6.20-1 and 6.6.20-2 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS15 |
| Specific message contents exceptions for Test Configuration 6.6.20-3 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS30 |

6.6.20.1.5	Test requirements

Table 6.6.20.1.4.1-3 and Table 6.6.20.1.5-1 define the primary level settings including test tolerances for UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA.

Table 6.6.20.1.5-1: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  |
| SRS configuration |  | 1 | [PDC-SRS.1] |  |
|  |  | 2 | [PDC-SRS.1] |  |
|  |  | 3 | [PDC-SRS.2] |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -98 |  |
| PRS | dB | 1 | -Infinity | -2.41 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | -Infinity | -2 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100 |
|  |  | 2 | -Infinity | -100 |
|  |  | 3 | -Infinity | -97 |
| Io | dBm/9.36 MHz | 1 | N/A | -67.67 |
|  | dBm/9.36 MHz | 2 |  | -67.67 |
|  | dBm/38.16 MHz | 3 |  | -61.57 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: The noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

The UE Rx-Tx time difference measurement time shall fulfils the requirements specified in clause 6.6.20.0. The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause [10.1.25.3.1].

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 6.6.20.0 for Cell 1.

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS

Editor’s note: This test case is incomplete. The following aspects are missing or TBD:

-	Test purpose contains []

- Connection diagram is TBD

- Measurement Uncertainties and Test Tolerances are missing

#### 6.6.21.0	Minimum conformance requirements

[38.133 clause 9.12.4.2, 9.12.5 and 10.1.25.3.1] for measurement time

When UE is configured with csi-RS-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [2] and provided with TRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_TRS, where:

TUERxTx,TRS=Nsample *Klayer3*LCM(TTRS, TDRX)

Where

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is [4],

TTRS is the TRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length when DRX is in use, 1ms otherwise.

Klayer3 is

Ntotal / Navailable when Navailable>0

Where,

-	For a window W of duration LCM(TTRS, MGRP, TSMTC), where TSMTC is the periodicity of SMTC for intra-frequency measurement without gap, starting at the beginning of any TRS resource occasion:

-	Ntotal is the total number of TRS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Navailable is the number of TRS resource occasions that are not overlapped with

-	For FR1: any measurement gap occasions within the window W

No requirements shall apply if Navailable=0.

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP. When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

If TRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the measurement report mapping requirements specified in below.

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024×Tc to 985024×Tc with the resolution step of 2k×Tc.

Table 6.6.21.0-1: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

[TS 38.133 clause 10.1.39.3] for measurement accuracy

The error in the reported value of UE Rx-Tx time difference measurement, including both the measurement error and the reporting quantization error, should be within the accuracy requirements specified in this clause.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

-	NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

-	if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one PDC TRS resource from the serving cell (PCell).

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in Table 6.6.21.0-2 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex [TBD] for a corresponding Band.

AWGN propagation condition.

Table 6.6.21.0-2: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TRS Ês/Iot | Minimum TRS bandwidth | TRS SCS | NR operating band groupsNote 2 | IoNote 3 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 4 | dB | RB | kHz |  | dBm / SCSTRS | dBm/BW |
| [116++ℇ] | -3 | ≥[24] | 15 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -118 |  |
|  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| [60++ℇ] |  | ≥[52] |  | Note 5 | NOTE 5 | NOTE 5 |
| [29++ℇ] |  | >[104] |  | Note 5 | NOTE 5 | NOTE 5 |
| [56++ℇ] |  | ≥[24] | 30 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -117.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -117 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 |  |
|  |  |  |  | NR_FDD_FR1_F | -115.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -115 |  |
|  |  |  |  | NR_FDD_FR1_H | -114.5 |  |
| [29++ℇ] |  | ≥[48] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| [15++ℇ] |  | ≥[132] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| [29++ℇ] |  | ≥[24] | 60 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -114.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -114 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 |  |
|  |  |  |  | NR_FDD_FR1_F | -113.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -113 |  |
|  |  |  |  | NR_FDD_FR1_H | -111.5 |  |
| [14++ℇ] |  | ≥[64] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| [7++ℇ] |  | ≥[132] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: The Io is defined in TRS slots. The same Io range applies to TRS and non-TRS symbols. Io levels are different in TRS and non-TRS symbols within the same slot. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the TRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: is the margin determined from Table 10.1.39.3-3. NOTE 7: ℇ is the margin for reporting quantitization error and ℇ=16 Tc. |  |  |  |  |  |  |

Table 6.6.21.0-3: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(TRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | [160] |
| ≥ 52 | ≥ 24 | N/A | [80] |
| ≥ 104 | ≥ 48 | ≥ 24 | [56] |
| N/A | ≥ 132 | ≥ 64 | [24] |
| N/A | N/A | ≥ 132 | [24] |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and TRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

#### 6.6.21.1	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA

6.6.21.1.1	Test purpose

The purpose of the test is to verify that the UE Rx-Tx measurement with TRS for RTT-based PDC meets the requirements specified in clause 9.12.4.2 for measurement delay and clause 10.1.X.2 for measurement accuracy in AWGN propagation condition in FR1 in standalone scenario.

6.6.21.1.2	Test applicability

This test applies to all types of NR UE release 17 onwards supporting rtt-BasedPDC-CSI-RS-ForTracking-r17 or gNB-SideRTT-BasedPDC-r17.

6.6.21.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.21.0.

The normative reference for this requirement is TS 38.133 [6] clauses 9.12.4.2, 9.12.5, 10.1.39.3 and A.6.6.21.

6.6.21.1.4	Test description

6.6.21.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.21.1.4.1-1

Table 6.6.21.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.21-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.21-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.21-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.21.1.4.1-2.

Table 6.6.21.1.4.1-2: Initial conditions for UE Tx-Rx time difference measurement for FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.21.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |

1. The general test parameter settings are set up according to Table 6.6.21.1.4.1-3.

2. Message contents are defined in clause 6.6.21.1.4.3.

3. The test scenario comprises of 1 NR Cell. The connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.21.1.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |
| --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 1 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |
|  |  | 2 | 10: NRB,c = 52 |
|  |  | 3 | 40: NRB,c = 106 |
| SSB configuration |  | 1 | SSB.1 FR1 |
|  |  | 2 | SSB.1 FR1 |
|  |  | 3 | SSB.2 FR1 |
| SMTC configuration |  | 1 | SMTC.2 |
|  |  | 2 | SMTC.1 |
|  |  | 3 | SMTC.1 |
| CP length |  | 1, 2, 3 | Normal |
| DRX |  | 1, 2, 3 | OFF |
| T1 | s | 1, 2, 3 | 1 |
| T2 | s | 1, 2, 3 | 1 |

6.6.21.1.4.2	Test procedure

The test is considered with one cell (Cell 1) in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 does not have TRS transmission during T1 and transmits TRS during T2.

The measurement control information with MeasObjectRxTxDiff set to ‘csi-RS-Ref’ as defined in table 6.6.21.1.4.3-1, shall be provided to the UE during T1. The last TTI containing the RRC message shall be provided to the UE T ms before the start of T2, where T = 10 ms is the maximum processing time of the measurement request. The UE is configured to transmit SRS during T2.

The general and cell specific test parameters for PCell is given in Table 6.6.21.1.4.1-3 and Table 6.6.21.1.5-1, respectively. The test consists of two sub-tests with different TRS BW.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.21.1.5-1 as appropriate.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.21.1.5-1. T2 starts.

6. The UE shall transmit periodically MeasurementReport messages.

7. After 10s wait from Step 3, the SS shall check the result-k5-r17 reported values in the periodic MeasurementReport. If the result-k5 value is outside the limits in Table 6.6.21.0-2 or the UE fails to report the measurement value for serving cell, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

8. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6.6.21.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table 4.6.3-76B in 38.508-1 [14] with condition CSI-RS_REF |
| Specific message contents exceptions for Test Configuration 6.6.21-1 and 6.6.21-2 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS15 |
| Specific message contents exceptions for Test Configuration 6.6.21-3 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS30 |

6.6.21.1.5	Test requirements

Table 6.6.21.1.4.1-3 and Table 6.6.21.1.5-1 define the primary level settings including test tolerances for UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA.

Table 6.6.21.1.5-1: Cell specific test parameters

| Parameter | Unit | Test configuration | Values |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |
| SRS configuration |  | 1 | [PDC-SRS.1] |  |
|  |  | 2 | [PDC-SRS.1] |  |
|  |  | 3 | [PDC-SRS.2] |  |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS | dB | 1 | -Infinity | -3 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS | dB | 1 | -Infinity | -3 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101 |
|  |  | 2 | -Infinity | -101 |
|  |  | 3 | -Infinity | -98 |
| Io | dBm/9.36 MHz | 1 | N/A | -68.28 |
|  | dBm/9.36 MHz | 2 |  | -68.28 |
|  | dBm/38.16 MHz | 3 |  | -62.19 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: TRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 6.6.21.0. The UE shall be able to report the Rx-Tx time difference measurement results no later than TBD ms from the beginning of T2.

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 6.6.21.0.

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## 6.7	Measurement performance requirements

### 6.7.1	SS-RSRP

#### 6.7.1.0	Minimum conformance requirements

##### 6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.1.

##### 6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.2.

##### 6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.3.

##### 6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.4.

#### 6.7.1.1	Intra-frequency measurements

##### 6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy

6.7.1.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.1.

6.7.1.1.1.4	Test description

6.7.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.1.1.4.1-1.

Table 6.7.1.1.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.1.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.1.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.1.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.1.1.4.1-2.

Table 6.7.1.1.1.4.1-2: Initial conditions for SS-RSRP intra frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 6.7.1.1.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.1.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.1.1.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Table 6.7.1.1.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.1.1.1.5-1 as appropriate and repeat steps 5-7.

6.7.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.1.1.1.4.3-1: Common Exception messages for NR SA FR1 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-1 | Table H.3.1-3 with Condition SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-2 | Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-3 | Table H.3.1-3 with Condition SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.1.1.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRP Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.1.1.1.5	Test requirement

Table 6.7.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.1.1.1.5-3 for test configuration 3.

Table 6.7.1.1.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  |  |  | Test 2 |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| Physical cell ID |  |  |  | 489 | 0 |  |  | 489 | 0 |  | 489 |  | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  |  | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |  |
| SSB configuration |  | Config 1 |  | SSB 1.FR1 |  | SSB.1 FR1 |  | SSB 1.FR1 | SSB.1 FR1 |  | SSB 1.FR1 |  |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB 1.FR1 |  | SSB.1 FR1 |  | SSB 1.FR1 | SSB.1 FR1 |  | SSB 1.FR1 |  |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB 2.FR1 |  | SSB.2 FR1 |  | SSB 2.FR1 | SSB.2 FR1 |  | SSB 2.FR1 |  |  | SSB.2 FR1 |
| Time offset with Cell 2 |  | Config 1 | ms | - |  | 3 |  | - | 3 |  | - |  |  | 3 |
|  |  | Config 2,3 | s | - |  | 3 |  | - | 3 |  | - |  |  | 3 |
| SMTC Configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | Depending on band group | dBm/15KhZ | -107.5 |  |  |  | -88 |  |  |  | -116 + ΔBG_offset |  |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  |  |  | -94 |  |  |  | -116 + ΔBG_offset |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -107.4 |  |  |  | -88 |  |  |  | Same as Noc/15kHz |  |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  |  |  | -91 |  |  |  | -113 + ΔBG_offset |  |  |
|  |  |  | dB | 1.88 |  |  | -5.57 | 1.88 |  | -5.57 |  | 0.09 |  | -5.56 |
|  |  |  | dB | 6 |  |  | 1.4 | 6 |  | 1.4 |  | 3 |  | -0.8 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -101.5 |  |  | -106.1 | -82 |  | -86.6 |  | -113 + ΔBG_offset |  | -116.8 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  |  | N/ANote 6 | -85 |  | -89.6 |  | -110+ ΔBG_offset |  | -113.8+ ΔBG_offset |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -71.68 |  |  |  | -52.18 |  |  |  | -82.39+ ΔBG_offset |  |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | N/ANote 6 |  |  |  | -51.91 |  |  |  | -76.12 + ΔBG_offset |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. Note 6: Subtest 1 is not used when testing with 30kHz SSB SCS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.1.1.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 44 | 60 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 34 |
|  |  |  | Bands NR_FDD_FR1_B | 34 |
|  |  |  | Bands NR_TDD_FR1_C | 35 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 36 |
|  |  |  | Bands NR_FDD_FR1_G | 37 |
|  |  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | 56 | 79 | NR_FDD_FR1_A, NR_TDD_FR1_A | 46 |
|  |  |  | NR_FDD_FR1_B | 46 |
|  |  |  | NR_TDD_FR1_C | 47 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | 47 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | 48 |
|  |  |  | NR_FDD_FR1_G | 49 |
|  |  |  | NR_FDD_FR1_H | 49 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | 40 | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 29 |
|  |  |  | Bands NR_FDD_FR1_B | 30 |
|  |  |  | Bands NR_TDD_FR1_C | 30 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 31 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 31 |
|  |  |  | Bands NR_FDD_FR1_G | 33 |
|  |  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 61 | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  |  | Bands NR_FDD_FR1_B | 51 |
|  |  |  | Bands NR_TDD_FR1_C | 51 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 52 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 52 |
|  |  |  | Bands NR_FDD_FR1_G | 53 |
|  |  |  | Bands NR_FDD_FR1_H | 54 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

Table 6.7.1.1.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 37 |
|  |  |  | Bands NR_FDD_FR1_B | 37 |
|  |  |  | Bands NR_TDD_FR1_C | 38 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 38 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 39 |
|  |  |  | Bands NR_FDD_FR1_G | 40 |
|  |  |  | Bands NR_FDD_FR1_H | 40 |
| Highest reported value (Cell 2) | N/A | 76 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 49 |
|  |  |  | Bands NR_FDD_FR1_B | 49 |
|  |  |  | Bands NR_TDD_FR1_C | 50 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 50 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 51 |
|  |  |  | Bands NR_FDD_FR1_G | 52 |
|  |  |  | Bands NR_FDD_FR1_H | 52 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | N/A | 54 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 32 |
|  |  |  | Bands NR_FDD_FR1_B | 33 |
|  |  |  | Bands NR_TDD_FR1_C | 33 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 34 |
|  |  |  | Bands NR_FDD_FR1_G | 35 |
|  |  |  | Bands NR_FDD_FR1_H | 36 |
| Highest reported value (Cell 2) | N/A | 79 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 53 |
|  |  |  | Bands NR_FDD_FR1_B | 54 |
|  |  |  | Bands NR_TDD_FR1_C | 54 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 55 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 55 |
|  |  |  | Bands NR_FDD_FR1_G | 56 |
|  |  |  | Bands NR_FDD_FR1_H | 57 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy

6.7.1.1.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.1.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.1.

6.7.1.1.2.4	Test description

6.7.1.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.1.2.4.1-1.

Table 6.7.1.1.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.1.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.1.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.1.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.1.2.4.1-2.

Table 6.7.1.1.2.4.1-2: Initial conditions for SS-RSRP intra frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 6.7.1.1.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.1.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.1.1.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRP reported values of Cell 1 and Cell 2 in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 6.7.1.1.2.5-2 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.1.1.2.5-1 as appropriate and repeat steps 5-7.

6.7.1.1.2.4.3	Message contents

Message contents are same as in clause 6.7.1.1.1.4.3.

6.7.1.1.2.5	Test requirement

Table 6.7.1.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.1.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.1.2.5-2.

Table 6.7.1.1.2.5-1: Same as Table 6.7.1.1.1.5-1 with the following exceptions:

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| Note2 | Config 1,2 | Depending on band group | dBm/15KhZ | -106 |  |  | -88 |  |  |  | -116 + ΔBG_offset |  |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  |  | -94 |  |  |  | -116 + ΔBG_offset |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | Same as Noc/15kHz |  |  | Same as Noc/15kHz |  |  |  | Same as Noc/15kHz |  |  |
|  | Config 3,6 | Depending on band group |  | N/ANote 6 |  |  | -91 |  |  |  | -113 + ΔBG_offset |  |  |
|  |  |  | dB | 1.88 |  | -4.97 | 1.88 |  | -4.97 |  | -0.01 |  | -4.76 |
|  |  |  | dB | 6 |  | 2 | 6 |  | 2 |  | 3 |  | 0 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -100 |  | -104 | -82 |  | -86 |  | -113 + ΔBG_offset |  | -116 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  | N/ANote 6 | -85 |  | -89 |  | -110 + ΔBG_offset |  | -113 + ΔBG_offset |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -70.05 |  |  | -52.05 |  |  |  | -82.20+ ΔBG_offset |  |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | N/ANote 6 |  |  | -51.77 |  |  |  | -75.93 + ΔBG_offset |  |  |

Table 6.7.1.1.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| RSRP_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.1.2	Inter-frequency measurements

##### 6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy

6.7.1.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.2.

6.7.1.2.1.4	Test description

6.7.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.2.1.4.1-1.

Table 6.7.1.2.1.4.1-1: NR SA FR1-FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.2.1.4.1-2.

Table 6.7.1.2.1.4.1-2: Initial conditions for SS-RSRP inter frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.1.2.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in a different FR1 frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.2.1.4.2	Test procedure

Same as in clause 6.7.1.1.1.4.2 but replacing Table 6.7.1.1.1.5-1 and 6.7.1.1.1.5-2 with 6.7.1.2.1.5-1 and 6.7.1.2.1.5-2, respectively.

6.7.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.1.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.1.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRP Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.1.2.1.5	Test requirement

Table 6.7.1.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.2.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.1.2.1.5-3 for test configuration 3.

Table 6.7.1.2.1.5-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |  |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 |  | freq1 | freq2 |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| Gap pattern ID |  |  |  | 0 |  |  | 0 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |  |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |  |
|  |  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |  |
|  |  | 3 |  | CR.2.1 TDD | - |  | CR.2.1 TDD | - |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |  |
|  |  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |  |
|  |  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |  |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  | - |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| Time offset between Cell 2 and Cell 3 |  | 1 | ms | 3 |  |  | 3 |  |  |
|  |  | 2,3 | s | 3 |  |  | 3 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |  | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -94.65 |  | ( for Cell 2 +8dB) + ΔBG_offset | -115+ ΔBG_offset |  |
| Note2 | Depending on band group | 3 | dBm/15kHz | -96 | -96 |  | ( for Cell 2 +8dB) + ΔBG_offset | -115+ ΔBG_offset |  |
| Note2 | Depending on band group | 1,2 | dBm/SSB SCS | -94.65 | -94.65 |  | ( for Cell 2 +8dB) + ΔBG_offset | -115+ ΔBG_offset |  |
|  | Depending on band group | 3 |  | -93 | -93 |  | ( for Cell 2 +8dB) + ΔBG_offset | -112.00+ ΔBG_offset |  |
|  |  | 1~3 | dB | 10 | 10 |  | 13 | -3 |  |
| SS-RSRPNote3 | Depending on band group | 1,2, | dBm/SCS | -84.65 | -84.65 |  | (RSRP for Cell 2 +25dB) + ΔBG_offset | -118.00+ ΔBG_offset |  |
|  | Depending on band group | 3 |  | -83 | -83 |  | (RSRP for Cell 2 +25dB) + ΔBG_offset | -115.00+ ΔBG_offset |  |
| IoNote3 | Depending on band group | 1,2 | dBm/ 9.36MHz | 56.28 | 56.28 |  | (Io for Channel 2 +19.75dB) + ΔBG_offset | -85.28+ ΔBG_offset |  |
|  | Depending on band group | 3 | dBm/ 38.16MHz | -51.53 | -51.53 |  | (Io for Channel 2 +19.75dB) + ΔBG_offset | -79.19+ ΔBG_offset |  |
|  |  | 1~3 | dB | 10 | 10 |  | 13 | -3 |  |
| Propagation condition |  | 1~3 | - | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5 The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. Note 6: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |

Table 6.7.1.2.1.5-2: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 1, 2, 4 and 5

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 32 |
|  |  | Bands NR_FDD_FR1_B | 33 |
|  |  | Bands NR_TDD_FR1_C | 33 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 34 |
|  |  | Bands NR_FDD_FR1_G | 35 |
|  |  | Bands NR_FDD_FR1_H | 36 |
| Highest reported value (Cell 2) | 81 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 45 |
|  |  | Bands NR_FDD_FR1_B | 45 |
|  |  | Bands NR_TDD_FR1_C | 46 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 46 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 47 |
|  |  | Bands NR_FDD_FR1_G | 48 |
|  |  | Bands NR_FDD_FR1_H | 48 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 28 |
|  |  | Bands NR_FDD_FR1_B | 28 |
|  |  | Bands NR_TDD_FR1_C | 29 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 29 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 30 |
|  |  | Bands NR_FDD_FR1_G | 31 |
|  |  | Bands NR_FDD_FR1_H | 31 |
| Highest reported value (Cell 2) | 84 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 49 |
|  |  | Bands NR_FDD_FR1_B | 50 |
|  |  | Bands NR_TDD_FR1_C | 50 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 51 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 51 |
|  |  | Bands NR_FDD_FR1_G | 52 |
|  |  | Bands NR_FDD_FR1_H | 53 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 6.7.1.2.1.5-3: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 3 and 6

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 64 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 35 |
|  |  | Bands NR_FDD_FR1_B | 36 |
|  |  | Bands NR_TDD_FR1_C | 36 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 37 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 37 |
|  |  | Bands NR_FDD_FR1_G | 38 |
|  |  | Bands NR_FDD_FR1_H | 39 |
| Highest reported value (Cell 2) | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 49 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  | Bands NR_FDD_FR1_G | 51 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 61 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 32 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 33 |
|  |  | Bands NR_FDD_FR1_G | 34 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 86 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 52 |
|  |  | Bands NR_FDD_FR1_B | 53 |
|  |  | Bands NR_TDD_FR1_C | 53 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 54 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 54 |
|  |  | Bands NR_FDD_FR1_G | 55 |
|  |  | Bands NR_FDD_FR1_H | 56 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy

6.7.1.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.2.

6.7.1.2.2.4	Test description

6.7.1.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.2.2.4.1-1.

Table 6.7.1.2.2.4.1-1: NR SA FR1-FR1 SS-RSRP relative measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.2.2.4.1-2.

Table 6.7.1.2.2.4.1-2: Initial conditions for SS-RSRP inter frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.1.2.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in a different FR1 frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.2.2.4.2	Test procedure

Same as in clause 6.7.1.1.2.4.2 but replacing Table 6.7.1.1.2.5-1 and 6.7.1.1.2.5-2 with 6.7.1.2.2.5-1 and 6.7.1.2.2.5-2, respectively.

6.7.1.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.1.2.1.4.3.

6.7.1.2.2.5	Test requirement

Table 6.7.1.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.2.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.2.2.5-2.

Table 6.7.1.2.2.5-1: same as Table 6.7.1.2.1.5-1

Table 6.7.1.2.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SS-RSRP_x - 7 | SS-RSRP_x - 31 |
| Highest reported value (Cell 2) | SS-RSRP_x + 7 | SS-RSRP_x - 18 |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SS-RSRP_x - 9 | SS-RSRP_x - 33 |
| Highest reported value (Cell 2) | SS-RSRP_x + 9 | SS-RSRP_x - 17 |
| SS-RSRP_x is the reported value of Cell 1 |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.2	SS-RSRQ

#### 6.7.2.0	Minimum conformance requirements

##### 6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements

Same as in clause 4.7.2.0.1.

##### 6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements

Same as in clause 4.7.2.0.2.

##### 6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements

Same as in clause 4.7.2.0.3.

#### 6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy

6.7.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRQ measurement accuracy is within the specified limits for all bands.

6.7.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.2.1.

6.7.2.1.4	Test description

6.7.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.2.1.4.1-1.

Table 6.7.2.1.4.1-1: NR SA FR1 SS-RSRQ measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.2.1.4.1-2.

Table 6.7.2.1.4.1-2: Initial conditions for SS-RSRQ intra frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.2.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.3.

6.7.2.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.2.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 6.7.2.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.2.1.5-1 as appropriate and repeat steps 5-7.

6.7.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.2.1.4.3-1: Common Exception messages for NR SA FR1 SS-RSRQ measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.2.1-1 | Table H.3.1-3 with Condition SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.2.1-2 | Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.2.1-3 | Table H.3.1-3 with Condition SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRQ Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.2.1.5	Test requirement

Table 6.7.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 6.7.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.2.1.5-2.

Table 6.7.2.1.5-1: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - |  | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  | - |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 | s | - |  | 3 | - |  | 3 |  | - |  | 3 |
|  |  | Config 1 | ms | - |  | 3 | - |  | 3 |  | - |  | 3 |
| SMTC configuration |  | Config 2, 3 |  | SMTC.1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |  | 0 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | Depending on band group | dBm/15kHz | -86.5 |  |  | -101 |  |  | -114+ ΔBG_offset |  |  |  |
|  | Config 3 | Depending on band group |  | -92.6 |  |  | - |  |  | -114+ ΔBG_offset |  |  |  |
| Note2 | Config 1,2 | Depending on band group | dBm/SC S | -86.5 |  |  | -101 |  |  | -114+ ΔBG_offset |  |  |  |
|  | Config 3 | Depending on band group |  | -89.6 |  |  | - |  |  | -111+ ΔBG_offset |  |  |  |
|  |  |  | dB | -1.76 |  |  | -4.7 |  |  | -5.46 |  | -5.46 |  |
|  |  |  | dB | 3 | 3 |  | -2.9 | -2.9 |  | -4 |  | -4 |  |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -83.5 | -83.5 |  | -103.9 | -103.9 |  | -118+ΔBG_offset |  | -118+ ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | -86.6 | -86.6 |  | - | - |  | -115+ ΔBG_offset |  | -115+ ΔBG_offset |  |
| SS-RSRQ Note3 |  |  | dB | -14.77 | -14.77 |  | -16.76 | -16.76 |  | -17.34 |  | -17.34 |  |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -51.57 |  |  | -70 |  |  | -83.28+ ΔBG_offset |  |  |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | -51.56 |  |  | - |  |  | -76.67+ ΔBG_offset |  |  |  |
| Propagation condition |  |  | - | AWGN | AWGN |  | AWGN | AWGN |  | AWGN |  | AWGN |  |
| Antenna configuration |  |  |  | 1x2 | 1x2 |  | 1x2 | 1x2 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. Note 6: Subtest 2 is not used when testing with 30kHz SSB SCS. Note 7: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.2.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_52 | SS-RSRQ_46 | SS-RSRQ_44 |
| Highest reported value (Cell 2) | SS-RSRQ_62 | SS-RSRQ_60 | SS-RSRQ_59 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_49 | SS-RSRQ_45 | SS-RSRQ_43 |
| Highest reported value (Cell 2) | SS-RSRQ_65 | SS-RSRQ_61 | SS-RSRQ_60 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy

##### 6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy

6.7.2.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRQ absolute measurement accuracy is within the specified limits for all bands.

6.7.2.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.2.2.1.

6.7.2.2.1.4	Test description

6.7.2.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.2.2.1.4.1-1.

Table 6.7.2.2.1.4.1-1: NR SA FR1-FR1 SS-RSRQ measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.2.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.2.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.2.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.2.2.1.4.1-2.

Table 6.7.2.2.1.4.1-2: Initial conditions for SS-RSRQ inter frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.2.2.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the different frequency as Cell 1. Cell 2 is the target cell for SS-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.2.2.1.4.2	Test procedure

Same as in clause 6.7.2.1.1.4.2 but replacing Table 6.7.2.1.1.5-1 and 6.7.2.1.1.5-2 with 6.7.2.2.1.5-1 and 6.7.2.2.1.5-2, respectively.

6.7.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.2.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.2.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.2.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.2.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.2.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRQ Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.2.2.1.5	Test requirement

Table 6.7.2.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.2.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.2.2.1.5-2.

Table 6.7.2.2.1.5-1: SS-RSRQ Inter frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| SSB ARFCN |  |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| Gap pattern ID |  |  |  |  | 0 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD | - | R.1.1 FDD | - | CR.1.1 FDD |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| TRS Configuration |  | Config 1 |  |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 |  | s | 3 |  |  |  |  |  |  |
|  |  | Config 1 |  | ms | 3 |  |  |  |  |  |  |
| SMTC configuration |  | Config 2, 3 |  |  | SMTC.1 |  |  |  |  |  |  |
|  |  | Config 1 |  |  | SMTC.2 |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  |  | SSB.1 in FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SSB.2 in FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 30 kHz |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | Depending on band group | dBm/15kHz | -81.68 | -81.68 | -106 | -106 | -116 + ΔBG_offset | -116 + ΔBG_offset |  |
| Note2 | Config 3 |  | Depending on band group | dBm/15kHz | -87.80 | -87.80 | -113 | -113 | -116+ ΔBG_offset | -116+ ΔBG_offset |  |
| Note2 | Config 1,2 |  | Depending on band group | dBm/SCS | -81.68 | -81.68 | -106 | -106 | -116 + ΔBG_offset | -116 + ΔBG_offset |  |
|  | Config 3 |  | Depending on band group |  | -84.8 | -84.8 | -110 | -110 | -113+ ΔBG_offset | -113+ ΔBG_offset |  |
|  |  |  |  | dB | -1.75 | -1.75 | -1.75 | -1.75 | 3 | -1.75 |  |
|  |  |  |  | dB | -1.75 | -1.75 | -1.75 | -1.75 | 3 | -1.75 |  |
| SS-RSRPNote3 | Config 1,2 |  | Depending on band group | dBm/SCS | -83.43 | -83.43 | -107.75 | -107.75 | -113+ ΔBG_offset | -117.75+ ΔBG_offset |  |
|  | Config 3 |  | Depending on band group |  | -86.54 | -86.54 | -111.75 | -111.75 | -110+ ΔBG_offset | -114.75+ ΔBG_offset |  |
| SS-RSRQNote3 |  |  |  | dB | -14.76 | -14.76 | -14.76 | -14.76 | -12.56 | -14.76 |  |
| IoNote3 | Config 1,2 |  | Depending on band group | dBm/Ch BW | -51.51 | -51.51 | -75.83 | -75.83 | -83.28+ ΔBG_offset | -85.83+ ΔBG_offset |  |
|  | Config 3 |  | Depending on band group |  | -51.52 | -51.52 | -76.73 | -76.73 | -77.19+ ΔBG_offset | -79.73+ ΔBG_offset |  |
| Propagation condition |  |  |  | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |  |
| Antenna configuration |  |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.2.2.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_52 | SS-RSRQ_52 | SS-RSRQ_52 |
| Highest reported value (Cell 2) | SS-RSRQ_62 | SS-RSRQ_62 | SS-RSRQ_62 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_49 | SS-RSRQ_49 | SS-RSRQ_49 |
| Highest reported value (Cell 2) | SS-RSRQ_65 | SS-RSRQ_65 | SS-RSRQ_65 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy

6.7.2.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRQ relative measurement accuracy is within the specified limits for all bands.

6.7.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.2.2.2.

6.7.2.2.2.4	Test description

6.7.2.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.2.2.2.4.1-1.

Table 6.7.2.2.2.4.1-1: NR SA FR1-FR1 SS-RSRQ measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.2.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.2.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.2.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.2.2.2.4.1-2.

Table 6.7.2.2.2.4.1-2: Initial conditions for SS-RSRQ inter frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.2.2.2.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.2.2.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.2.2.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the SS-RSRQ value of Cell 1 reported by the UE. If the difference between both values is outside the limits in Table 6.7.2.2.2.5-2 or the UE fails to report the measurement value for Cell 2 or Cell 1, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.2.2.2.5-1 as appropriate and repeat steps 5-7.

6.7.2.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.2.2.1.4.3.

6.7.2.2.2.5	Test requirement

Table 6.7.2.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 6.7.2.2.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.2.2.2.5-2.

Table 6.7.2.2.2.5-1: same as Table 6.7.2.2.1.5-1

Table 6.7.2.2.2.5-2: SS-RSRQ Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x - 7 | SS-RSRQ_x - 7 | SS-RSRQ_x - 11 |
| Highest reported value (Cell 2) | SS-RSRQ_x + 7 | SS-RSRQ_x + 7 | SS-RSRQ_x + 2 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x - 9 | SS-RSRQ_x - 9 | SS-RSRQ_x – 13 |
| Highest reported value (Cell 2) | SS-RSRQ_x + 9 | SS-RSRQ_x + 9 | SS-RSRQ_x + 4 |
| RSRQ_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.3	SS-SINR

#### 6.7.3.0	Minimum conformance requirements

##### 6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements

Same as in clause 4.7.3.0.1.

##### 6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements

Same as in clause 4.7.3.0.2.

##### 6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements

Same as in clause 4.7.3.0.3.

#### 6.7.3.1	NR SA FR1 SS-SINR measurement accuracy

6.7.3.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-SINR measurement accuracy is within the specified limits for all bands.

6.7.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards, which support ss-SINR-Meas.

6.7.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.3.1.

6.7.3.1.4	Test description

6.7.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.3.1.4.1-1.

Table 6.7.3.1.4.1-1: NR SA FR1 SS-SINR measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.3.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.3.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.3.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.3.1.4.1-2.

Table 6.7.3.1.4.1-2: Initial conditions for SS-SINR intra frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.3.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.3.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.3.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS-SINR value of Cell 2 reported by the UE is compared to the expected SS-SINR. If the value is outside the limits in Table 6.7.3.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.3.1.5-1 as appropriate and repeat steps 5-7.

6.7.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.3.1.4.3-1: Common Exception messages for NR SA FR1 SS-SINR measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.3.1-1 | Table H.3.1-3 with Condition SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.3.1-2 | Table H.3.1-3 with Condition Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.3.1-3 | Table H.3.1-3 with Condition Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.3.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-SINR Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.3.1.5	Test requirements

Table 6.7.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 6.7.3.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.3.1.5-2

Table 6.7.3.1.5-1: SS-SINR Intra frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  |  | Config 1 |  |  | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| TRS configuration |  |  | Config 1 |  |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  |  | TRS.1.1 TDD | - |  | TRS.1.1 TDD |  |  | - |
|  |  |  | Config 3 |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |
|  |  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | SR.2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |
|  |  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  |  | Config 2, 3 | s | - |  | 3 |  | - | 3 |  |
|  |  |  |  | Config 1 | ms | - |  | 3 |  | - | 3 |  |
| SMTC configuration |  |  |  | Config 2, 3 |  | SMTC.1 |  |  |  |  |  |  |
|  |  |  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 |  | kHz | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | Depending on band group |  | dBm/15kHz | -93 |  |  | -116+ ΔBG_offset |  |  |  |
| Note2 | Config 1,2 |  |  |  | dBm/SCS | -93.2 |  |  | Same as Noc for 15kHz |  |  |  |
|  | Config 3 |  | Depending on band group |  |  | -90.2 |  |  | -113+ ΔBG_offset |  |  |  |
|  |  |  |  |  | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  |  |  | dB | 4.54 | 2.66 |  | -3.5 |  |  | -3.5 |
| SS-RSRPNote3 | Config 1,2 |  | Depending on band group |  | dBm/SCS | -88.46 | -90.34 |  | -119.5+ ΔBG_offset |  |  | -119.5+ ΔBG_offset |
|  | Config 3 |  | Depending on band group |  |  | -85.65 | -87.53 |  | -116.5+ ΔBG_offset |  |  | -116.5+ ΔBG_offset |
| SS-SINR Note3 |  |  |  |  | dB | 0 | -3.19 |  | -5.1 |  |  | -5.1 |
| IoNote3 |  | Config 1,2 | Depending on band group |  | dBm/ 9.36MHz | -57.5 |  |  | -85.28+ ΔBG_offset |  |  |  |
|  |  | Config 3 | Depending on band group |  | dBm/ 38.16MHz | -51.59 |  |  | -79.17+ ΔBG_offset |  |  |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  | - | 1x2 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.3.1.5-3: SS-SINR Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SS-SINR_31 | SS-SINR_28 |
| Highest reported value (Cell 2) | SS-SINR_49 | SS-SINR_45 |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SS-SINR_30 | SS-SINR_27 |
| Highest reported value (Cell 2) | SS-SINR_50 | SS-SINR_46 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.3.2	Inter-Frequency SS-SINR measurement accuracy

##### 6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy

6.7.3.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR absolute measurement accuracy is within the specified limits for all bands.

6.7.3.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards, which support ss-SINR-Meas.

6.7.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.3.2.1.

6.7.3.2.1.4	Test description

6.7.3.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.3.2.1.4.1-1.

Table 6.7.3.2.1.4.1-1: NR SA FR1-FR1 SS-SINR measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.3.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.3.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.3.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.3.2.1.4.1-2.

Table 6.7.3.2.1.4.1-2: Initial conditions for SS-SINR inter frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.3.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.3.2.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.3.2.1.4.2	Test procedure

Same as in clause 6.7.3.1.4.2 but replacing Table 6.7.3.1.5-1 and 6.7.3.1.5-2 with 6.7.3.2.1.5-1 and 6.7.3.2.1.5-2, respectively.

6.7.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.3.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-SINR absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.3.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.3.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, and Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.3.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, and Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.3.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-SINR Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.3.2.1.5	Test requirements

Table 6.7.3.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 6.7.3.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.3.2.1.5-2.

Table 6.7.3.2.1.5-1: SS-SINR Inter frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  | Test 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |  |  |
| SSB ARFCN |  |  |  |  | freq1 | freq2 |  | freq1 |  | freq2 |  | freq1 |  | freq2 |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Gap pattern ID |  |  |  |  | 0 |  | - |  | 0 |  | - |  | 0 |  |  | - |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD | - |  | TRS.1.1 TDD |  | - |  | TRS.1.1 TDD |  | - |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 |  | s | - |  | 3 |  | - |  | 3 |  | - |  |  | 3 |
|  |  | Config 1 |  | ms | - |  | 3 |  | - |  | 3 |  | - |  |  | 3 |
| SMTC configuration |  | Config 2, 3 |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 1 |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | Depending on band group | dBm/15kHz | -88 | -88 |  | -108.5 |  | -108.5 |  | -119.5+ ΔBG_offset |  |  | -119.5+ ΔBG_offset |  |
| Note2 | Config 1,2 |  |  | dBm/SCS | -88 | -88 |  | -108.5 |  | -108.5 |  | Same as Noc for 15kHz |  |  | Same as Noc for 15kHz |  |
|  | Config 3 |  | Depending on band group |  | -85 | -85 |  | -105.5 |  | -105.5 |  | -116.5+ ΔBG_offset |  |  | 116.5+ ΔBG_offset |  |
|  |  |  |  | dB | -1.75 | -1.75 |  | 20 |  | 20 |  | -3.2 |  |  | -3.2 |  |
|  |  |  |  | dB | -1.75 | -1.75 |  | 20 |  | 20 |  | -3.2 |  |  | -3.2 |  |
| SS-RSRPNote3 | Config 1,2 |  | Depending on band group | dBm/SCS | -89.75 | -89.75 |  | -88.5 |  | -88.5 |  | -122.7+ ΔBG_offset |  |  | -122.7+ ΔBG_offset |  |
|  | Config 3 |  | Depending on band group |  | -86.75 | -86.75 |  | -85.5 |  | -85.5 |  | -119.7+ ΔBG_offset |  |  | -119.7+ ΔBG_offset |  |
| SS-SINR Note3 |  |  |  | dB | -1.75 | -1.75 |  | 20 |  | 20 |  | -3.2 |  |  | -3.2 |  |
| IoNote3 | Config 1,2 |  | Depending on band group | dBm/ 9.36MHz | -57.83 | -57.83 |  | -60.5 |  | -60.5 |  | -89.85+ ΔBG_offset |  |  | -89.85+ ΔBG_offset |  |
|  | Config 3 |  | Depending on band group | dBm/ 38.16MHz | -51.73 | -51.73 |  | -54.41 |  | -54.41 |  | -83.75+ ΔBG_offset |  |  | -83.75+ ΔBG_offset |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.3.2.1.5-2: SS-SINR Inter frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_35 | SS-SINR_79 | SS-SINR_32 |
| Highest reported value (Cell 2) | SS-SINR_51 | SS-SINR_94 | SS-SINR_49 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_33 | SS-SINR_77 | SS-SINR_31 |
| Highest reported value (Cell 2) | SS-SINR_53 | SS-SINR_96 | SS-SINR_50 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy

6.7.3.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR relative measurement accuracy is within the specified limits for all bands.

6.7.3.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards, which support ss-SINR-Meas.

6.7.3.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.3.2.2.

6.7.3.2.2.4	Test description

6.7.3.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.3.2.2.4.1-1.

Table 6.7.3.2.2.4.1-1: NR SA FR1-FR1 SS-SINR measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.3.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.3.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.3.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.3.2.2.4.1-2.

Table 6.7.3.2.2.4.1-2: Initial conditions for SS-SINR inter frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.3.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.3.2.2.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.3.2.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.3.2.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS- SINR value of Cell 2 reported by the UE is compared to the SS- SINR value of Cell 1 reported by the UE. If the difference between both values is outside the limits in Table 6.7.3.2.2.5-2 or the UE fails to report the measurement value for Cell 2 or Cell 1, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.3.2.2.5-1 as appropriate and repeat steps 5-7.

6.7.3.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.3.2.1.4.3.

6.7.3.2.2.5	Test requirements

Table 6.7.3.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 6.7.3.2.2.5-1 shall meet the corresponding relative accuracy requirements in Table 6.7.3.2.2.5-2

Table 6.7.3.2.2.2.5-1: same as Table 6.7.3.2.2.1.5-1

Table 6.7.3.2.2.5-2: SS-SINR Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x - 10 | SS-SINR_x - 10 | SS-SINR_x - 11 |
| Highest reported value (Cell 2) | SS-SINR_x + 10 | SS-SINR_x + 10 | SS-SINR_x + 11 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x - 12 | SS-SINR_x - 12 | SS-SINR_x - 12 |
| Highest reported value (Cell 2) | SS-SINR_x + 12 | SS-SINR_x + 12 | SS-SINR_x + 12 |
| RSRQ_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.4	L1-RSRP

#### 6.7.4.0	Minimum conformance requirements

##### 6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.1.

##### 6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.2.

##### 6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.3.

##### 6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.4.

#### 6.7.4.1	SSB based L1-RSRP measurements

##### 6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy

6.7.4.1.1.1	Test purpose

The purpose of this test is to verify that the SSB based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.4.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.1.

6.7.4.1.1.4	Test description

6.7.4.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.1.1.4.1-1.

Table 6.7.4.1.1.4.1-1: NR SA FR1 SSB based L1-RSRP absolute measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.1.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.1.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.1.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.1.1.4.1-2.

Table 6.7.4.1.1.4.1-2: Initial conditions for SSB based L1-RSRP absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.1.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for SSB-based L1-RSRP measurements. The UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.1.1.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.1.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.1.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.1.1.5-1.

3.	The UE shall start sending L1-RSRP report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of SSB#0 or SSB#1 in the periodic L1-RSRP reports. If the value for the strongest SSB is within the limits in Table 6.7.4.1.1.5-2 or Table 6.7.4.1.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.1.1.5-1 as appropriate and repeat steps 3-5.

6.7.4.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.4.1.1.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with conditions SSB and PERIODIC Table H.3.5-8 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.4.1.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.4.1.1.5	Test requirement

Table 6.7.4.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.4.1.1.5-3 for test configuration 3.

Table 6.7.4.1.1.5-1: L1-RSRP test parameters

| Parameter |  |  | Config |  | Unit |  | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  |  | 1~3 |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  |  | 1 |  |  |  | FDD |  | FDD |  |
|  |  |  | 2 |  |  |  | TDD |  | TDD |  |
|  |  |  | 3 |  |  |  | TDD |  | TDD |  |
| TDD Configuration |  |  | 1 |  |  |  | N/A |  | N/A |  |
|  |  |  | 2 |  |  |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  |  | 3 |  |  |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel |  |  | 1 |  | MHz |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  |  | 2 |  |  |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  |  | 3 |  |  |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel |  |  | 1 |  |  |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
|  |  |  | 2 |  |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  | 3 |  |  |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | 1 |  |  |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  |  | 2 |  |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  | 3 |  |  |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | 1 |  |  |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  |  | 2 |  |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  | 3 |  |  |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| SSB configuration |  |  | 1 |  |  |  | SSB.3 FR1 |  | SSB.3 FR1 |  |
|  |  |  | 2 |  |  |  | SSB.3 FR1 |  | SSB.3 FR1 |  |
|  |  |  | 3 |  |  |  | SSB.4 FR1 |  | SSB.4 FR1 |  |
| OCNG Patterns |  |  | 1~3 |  |  |  | OP.1 |  | OP.1 |  |
| TRS configuration |  |  | 1 |  |  |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  |  |  | 2 |  |  |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  |  | 3 |  |  |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration |  |  | 1~3 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  |  | 1~3 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  |  | 1~3 |  |  |  | SMTC.1 |  | SMTC.1 |  |
| reportConfigType |  |  | 1~3 |  |  |  | periodic |  | periodic |  |
| reportQuantity |  |  | 1~3 |  |  |  | ssb-Index-RSRP |  | ssb-Index-RSRP |  |
| Number of reported RS |  |  | 1~3 |  |  |  | 2 |  | 2 |  |
| L1-RSRP reporting period |  |  | 1~3 |  |  |  | slot80 |  | slot80 |  |
| EPRE ratio of PSS to SSS |  |  | 1~3 |  | dB |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | Depending on band group | 1,2 |  | dBm/15kHz |  | -94.65 |  | -117+ ΔBG_offset |  |
|  |  |  | 3 |  |  |  | -96.00 |  | 117+ ΔBG_offset |  |
| Note2 |  |  | 1,2 |  | dBm/SSB SCS |  | -94.65 |  | -117+ ΔBG_offset |  |
|  |  |  | 3 |  |  |  | -93.00 |  | -114+ ΔBG_offset |  |
|  |  |  | 1~3 |  | dB |  | 10 |  | -2.2 |  |
| SSB RSRP Note3 |  | Depending on band group | 1,2 |  | dBm/SSB SCS |  | -84.65 |  | -119.2 + ΔBG_offset |  |
|  |  |  | 3 |  |  |  | -83.00 |  | -116.2 + ΔBG_offset |  |
| Io Note3 |  | Depending on band group | 1,2 |  | dBm/9.36 MHz |  | -56.28 |  | -87.00 + ΔBG_offset |  |
|  |  |  | 3 |  | dBm/38.16 MHz |  | -51.53 |  | -80.90 + ΔBG_offset |  |
|  |  |  | 1~3 |  | dB |  | 10 |  | -2.2 |  |
| Propagation condition |  |  | 1~3 |  |  |  | AWGN |  | AWGN |  |
| Antenna configuration |  |  | 1~3 |  |  |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |

Table 6.7.4.1.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 32 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 33 |
|  |  | Bands NR_FDD_FR1_G | 34 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 1) | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 44 |
|  |  | Bands NR_FDD_FR1_B | 45 |
|  |  | Bands NR_TDD_FR1_C | 45 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 46 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 46 |
|  |  | Bands NR_FDD_FR1_G | 47 |
|  |  | Bands NR_FDD_FR1_H | 48 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | 61 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 30 |
|  |  | Bands NR_FDD_FR1_B | 30 |
|  |  | Bands NR_TDD_FR1_C | 31 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 31 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 32 |
|  |  | Bands NR_FDD_FR1_G | 33 |
|  |  | Bands NR_FDD_FR1_H | 33 |
| Highest reported value (Cell 1) | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 45 |
|  |  | Bands NR_FDD_FR1_B | 46 |
|  |  | Bands NR_TDD_FR1_C | 46 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 47 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 47 |
|  |  | Bands NR_FDD_FR1_G | 48 |
|  |  | Bands NR_FDD_FR1_H | 49 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 6.7.4.1.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | 63 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 34 |
|  |  | Bands NR_FDD_FR1_B | 34 |
|  |  | Bands NR_TDD_FR1_C | 35 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 36 |
|  |  | Bands NR_FDD_FR1_G | 37 |
|  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 1) | 84 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 47 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 48 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 49 |
|  |  | Bands NR_FDD_FR1_G | 50 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  | Bands NR_FDD_FR1_B | 33 |
|  |  | Bands NR_TDD_FR1_C | 34 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
|  |  | Bands NR_FDD_FR1_G | 36 |
|  |  | Bands NR_FDD_FR1_H | 36 |
| Highest reported value (Cell 1) | 85 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  | Bands NR_FDD_FR1_B | 49 |
|  |  | Bands NR_TDD_FR1_C | 49 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 50 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  | Bands NR_FDD_FR1_G | 51 |
|  |  | Bands NR_FDD_FR1_H | 52 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy

6.7.4.1.2.1	Test purpose

The purpose of this test is to verify that the SSB based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.4.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.1.

6.7.4.1.2.4	Test description

6.7.4.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.1.2.4.1-1.

Table 6.7.4.1.2.4.1-1: NR SA FR1 SSB based L1-RSRP relative measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.1.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.1.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.1.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.1.2.4.1-2.

Table 6.7.4.1.2.4.1-2: Initial conditions for SSB based L1-RSRP relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.1.2.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for SSB-based L1-RSRP measurements. The UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.1.2.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.1.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.1.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.1.2.5-1.

3.	The UE shall start sending L1-RSRP report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of SSB#0 and SSB#1 in the periodic L1-RSRP reports. The DIFF-RSRP value of SSB#0 or SSB#1 reported by the UE is compared to the expected DIFF-. If the DIFF-RSRP value is within the limits in Table 6.7.4.1.2.5-2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.1.2.5-1 as appropriate and repeat steps 3-5.

6.7.4.1.2.4.3	Message contents

Message contents  are same as in Clause 6.7.4.1.1.4.3.

6.7.4.1.2.5	Test requirement

Table 6.7.4.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.1.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.1.2.5-2.

Table 6.7.4.1.2.5-1: Same as Table 6.7.4.1.1.5-1

Table 6.7.4.1.2.5-2: L1-RSRP relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest DIFF RSRP reported value | 0 | 0 |
| Highest DIFF RSRP reported value | +3 | +3 |
| Extreme Conditions |  |  |
| Lowest DIFF RSRP reported value | 0 | 0 |
| Highest DIFF RSRP reported value | +4 | +4 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.4.2	CSI-RS based L1-RSRP measurements

##### 6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy

6.7.4.2.1.1	Test purpose

The purpose of this test is to verify that the CSI-RS based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.4.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.2.

6.7.4.2.1.4	Test description

6.7.4.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.2.1.4.1-1.

Table 6.7.4.2.1.4.1-1: NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.2.1.4.1-2.

Table 6.7.4.2.1.4.1-2: Initial conditions for CSI-RS based L1-RSRP absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.2.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-RSRP measurements. The UE is configured to perform RLM and BFD measurement based on the SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.2.1.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.2.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.2.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.2.1.5-1.

3.	The UE shall start sending L1-RSRP report including results of both CSI-RS#0 and CSI-RS#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of CSI-RS#0 or CSI-RS#1 in the periodic L1-RSRP reports. If the value for the strongestCSI-RS is within the limits in Table 6.7.4.2.1.5-2 or Table 6.7.4.2.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.2.1.5-1 as appropriate and repeat steps 3-5.

6.7.4.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.4.2.1.4.3-1: Common Exception messages EN-DC CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and CSI-RSRP Table H.3.6-3 with conditions CSI-RS and PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.4.2.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSB. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.4.2.1.5	Test requirement

Table 6.7.4.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.2.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.4.2.1.5-3 for test configuration 3.

Table 6.7.4.2.1.5-1: L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1,4 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2,5 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3,6 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity |  | 1~3 |  | cri-RSRP | cri-RSRP |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -96.00 | 117+ ΔBG_offset |
| Note2 |  | 1,2 | dBm/CSI-RS SCS | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -93.00 | -114+ ΔBG_offset |
|  |  | 1~3 | dB | 10 | -2.2 |
| CSI-RSRP Note3 | Depending on band group | 1,2 | dBm/CSI-RS SCS | -84.65 | -119.2 + ΔBG_offset |
|  |  | 3 |  | -83.00 | -116.2 + ΔBG_offset |
| Io Note3 | Depending on band group | 1,2 | dBm/9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  |  | 3 | dBm/38.16 MHz | -51.53 | -80.90 + ΔBG_offset |
|  |  | 1~3 | dB | 10 | -2.2 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |

Table 6.7.4.2.1.5-2: Same as Table 6.7.4.1.1.5-2

Table 6.7.4.2.1.5-3: Same as Table 6.7.4.1.1.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy

6.7.4.2.2.1	Test purpose

The purpose of this test is to verify that the CSI-RS based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.4.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.2.

6.7.4.2.2.4	Test description

6.7.4.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.2.2.4.1-1.

Table 6.7.4.2.2.4.1-1: NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.2.2.4.1-2.

Table 6.7.4.2.2.4.1-2: Initial conditions for CSI-RS based L1-RSRP relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.2.2.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-RSRP measurements. The UE is configured to perform RLM and BFD measurement based on the SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.2.2.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.2.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.2.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.2.2.5-1.

3.	The UE shall start sending L1-RSRP report including results of both CSI-RS#0 and CSI-RS#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of CSI-RS#0 and CSI-RS#1 in the periodic L1-RSRP reports. The DIFF-RSRP value of CSI-RS#0 or CSI-RS#1 reported by the UE is compared to the expected DIFF-. If the DIFF-RSRP value is within the limits in Table 6.7.4.2.2.5-2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.2.2.5-1 as appropriate and repeat steps 3-5.

6.7.4.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.4.2.1.4.3.

6.7.4.2.2.5	Test requirement

Table 6.7.4.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.2.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.2.2.5-2.

Table 6.7.4.2.2.5-1: Same as Table 6.7.4.2.1.5-1

Table 6.7.4.2.2.5-2: Same as Table 6.7.4.1.2.5-2

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.5	E-UTRAN RSRP

#### 6.7.5.0	Minimum conformance requirements

##### 6.7.5.0.1	E-UTRAN RSRP absolute accuracy

The measurement period of E-UTRA RSRP in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3 of TS 38.133 [6].

The accuracy requirements of E-UTRA RSRP measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRP Accuracy Requirements in clause 9.1.3 of TS 36.133 [23]:

The requirements for absolute accuracy of RSRP in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Table 6.7.5.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.3 of TS 36.133 [23] for a corresponding Band.

Table 6.7.5.0.1-1: RSRP Inter frequency absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 4.5 | 9 | -6 dB | FDD_A, TDD_A | -121 | N/A | -70 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | N/A | -70 |
|  |  |  | FDD_C, TDD_C | -120 | N/A | -70 |
|  |  |  | FDD_D | -119.5 | N/A | -70 |
|  |  |  | FDD_E, TDD_E | -119 | N/A | -70 |
|  |  |  | FDD_F | -118.5 | N/A | -70 |
|  |  |  | FDD_G | -118 | N/A | -70 |
|  |  |  | FDD_H | -117.5 | N/A | -70 |
|  |  |  | FDD_N | -114.5 | N/A | -70 |
| 8 | 11 | -6 dB | FDD_A, TDD_A, FDD_B1, FDD_B2, FDD_C, TDD_C, FDD_D, FDD_E, TDD_E, FDD_F, FDD_G, FDD_H, FDD_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5 of TS 36.133 [23]. |  |  |  |  |  |  |

The reporting range and mapping specified for RSRP measurements in clause 9.1.4 of TS 36.133 [23] shall apply:

The reporting range of RSRP is defined from -156 dBm to -44 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 6.7.5.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.5.0.1-2: RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSRP_-17 | RSRP< -156 | dBm |
| RSRP_-16 | -156 RSRP< -155 | dBm |
| … | … | … |
| RSRP_-03 | -143 RSRP< -142 | dBm |
| RSRP_-02 | -142 RSRP< -141 | dBm |
| RSRP_-01 | -141 RSRP< -140 | dBm |
| RSRP_00 | RSRP -140 | dBm |
| RSRP_01 | -140 RSRP < -139 | dBm |
| RSRP_02 | -139 RSRP < -138 | dBm |
| … | … | … |
| RSRP_95 | -46 RSRP < -45 | dBm |
| RSRP_96 | -45 RSRP < -44 | dBm |
| RSRP_97 | -44 RSRP | dBm |

#### 6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy

6.7.5.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT E-UTRAN RSRP absolute measurement accuracy is within the specified limits for all bands, when the serving cell is NR FR1 and the target cell is E-UTRA.

6.7.5.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

6.7.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.5.1.

6.7.5.1.4	Test description

6.7.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.5.1.4.1-1.

Table 6.7.5.1.4.1-1:  test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.5.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: FDD |
| 6.7.5.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.5.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.5.1-4 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: TDD |
| 6.7.5.1-5 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: TDD |
| 6.7.5.1-6 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.5.1.4.1-2.

Table 6.7.5.1.4.1-2: initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.5.1.4.3.

2. There are two carriers and two cells specified in the test, where NR Cell 1 is the NR PCell on the NR carrier and Cell 2 is the E-UTRA neighbour cell on the E-UTRA carrier and the target for the measurements.

6.7.5.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.5.1.5-1 and Table 6.7.5.1.5-2 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the RSRP reported values in the periodic MeasurementReport. The RSRP value of Cell 2 reported by the UE is compared to the expected RSRP. If the value is outside the limits in Table 6.7.5.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.5.1.5-2  as appropriate and repeat steps 5-7.

6.7.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.5.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-RAT and GAP NEEDED Table H.3.1-3 Table H.3.1-3a Table H.3.1-7 with condition INTER-RAT Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.5.1.4.3-1A: MeasConfig (Test procedure step 3)

| Derivation path: Table H.3.1-2 with condition INTER-RAT and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigE-UTRA-DEFAULT(Periodical) | Table 6.7.5.1.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-EVENT | Table 6.7.5.1.4.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.5.1.4.3-1B: ReportConfigInterRAT-EVENT (Table 6.7.5.1.4.3-1A)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-141 with condition EVENT_B1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  |  |
| b1-ThresholdEUTRA CHOICE { |  |  |  |
| rsrp | 97 | Set threshold to -44dBm to ensure measId 2 will never be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.5.1.4.3-2: ReportConfigE-UTRA-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-141 with Condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.5.1.5	Test requirement

Table 6.7.5.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Tables 6.7.5.1.5-1 and 6.7.5.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 6.7.5.1.5-3.

Table 6.7.5.1.5-1: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 6.7.5.1.5-2: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| E-UTRA RF channel number |  |  | 1 |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |
|  | Config 4, 5, 6 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | Depending on band group | dBm/15kHz | -91.65 | -117 + ΔBG_offset |
| Ês/Noc |  | dB | 10 | -3.2 |
| Ês/IotNote5 |  | dB | 10 | -3.2 |
| RSRPNote5 | Depending on band group | dBm/15kHz | -81.65 | -120.2+ ΔBG_offset |
| SCH_RPNote5 | Depending on band group | dBm/15kHz | -81.65 | -120.2+ ΔBG_offset |
| IoNote5 | Depending on band group | dBm/Ch BW | -53.45 + 10log(NRB,c/50) | -87.52+ ΔBG_offset + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: Void Note 8: Void. Note 9: Void Note 10: Void Note 11: ΔBG_offset for LTE band group is defined in TS 36.521-3 [26] clause 3.5.1, Table 3.5.1-1A. |  |  |  |  |

Table 6.7.5.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 |  |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 48 | FDD_A, TDD_A | 14 |  |
|  |  | FDD_B | 15 |  |
|  |  | TDD_C | 15 |  |
|  |  | FDD_D, TDD_D | 16 |  |
|  |  | FDD_E, TDD_E | 16 |  |
|  |  | FDD_G | 17 |  |
|  |  | FDD_H | 18 |  |
| Highest reported value (Cell 2) | 70 | FDD_A, TDD_A | 27 |  |
|  |  | FDD_B | 27 |  |
|  |  | TDD_C | 28 |  |
|  |  | FDD_D, TDD_D | 28 |  |
|  |  | FDD_E, TDD_E | 29 |  |
|  |  | FDD_G | 30 |  |
|  |  | FDD_H | 30 |  |
| Extreme Conditions | Test 1 All bands | Test 2 |  |  |
| Lowest reported value (Cell 2) | 46 | FDD_A, TDD_A | 11 |  |
|  |  | FDD_B | 11 |  |
|  |  | TDD_C | 12 |  |
|  |  | FDD_D, TDD_D | 12 |  |
|  |  | FDD_E, TDD_E | 13 |  |
|  |  | FDD_G | 14 |  |
|  |  | FDD_H | 14 |  |
| Highest reported value (Cell 2) | 72 | FDD_A, TDD_A | 30 |  |
|  |  | FDD_B | 31 |  |
|  |  | TDD_C | 31 |  |
|  |  | FDD_D, TDD_D | 32 |  |
|  |  | FDD_E, TDD_E | 32 |  |
|  |  | FDD_G | 33 |  |
|  |  | FDD_H | 34 |  |
| Note 1: E-UTRA operating band groups are as defined in TS 36.521-3 [26] clause 3.5.1 |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.6	E-UTRAN RSRQ

#### 6.7.6.0	Minimum conformance requirements

##### 6.7.6.0.1	E-UTRAN RSRQ absolute accuracy

The measurement period of E-UTRA RSRQ in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3 on TS 38.133 [6].

The accuracy requirements of E-UTRA RSRQ measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRQ Accuracy Requirements in clause 9.1.6 of TS 36.133 [23]:

The requirements for absolute accuracy of RSRQ in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Table 6.7.6.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.3 of TS 36.133 [23] for a corresponding Band

Table 6.7.6.0.1-1: RSRQ Inter frequency absolute accuracy

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| 2.5 | 4 | -3 dB | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 3.5 | 4 | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5 of TS 36.133 [23]. |  |  |  |  |  |

The reporting range and mapping specified for RSRQ measurements in clause 9.1.7 of TS 36.133 [23] shall apply:

The reporting range of RSRQ is defined from -34 dB to 2.5 dB with 0.5 dB resolution.

The mapping of measured quantity is defined in table 6.7.6.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.6.0.1-2: RSRQ measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSRQ_-30 | RSRQ -34 | dB |
| RSRQ_-29 | -34 RSRQ < -33.5 | dB |
| … | … | … |
| RSRQ_-02 | -20.5 RSRQ < -20 | dB |
| RSRQ_-01 | -20 RSRQ < -19.5 | dB |
| RSRQ_00 | RSRQ -19.5 | dB |
| RSRQ_01 | -19.5 RSRQ < -19 | dB |
| RSRQ_02 | -19 RSRQ < -18.5 | dB |
| … | … | … |
| RSRQ_32 | -4 RSRQ < -3.5 | dB |
| RSRQ_33 | -3.5 RSRQ < -3 | dB |
| RSRQ_34 | -3 RSRQ | dB |
| RSRQ_35 | -3 RSRQ < -2.5 | dB |
| RSRQ_36 | -2.5 RSRQ < -2 | dB |
| … | … | … |
| RSRQ_45 | 2 RSRQ < 2.5 | dB |
| RSRQ_46 | 2.5 RSRQ | dB |

Note: The ranges from RSRQ_-30 to RSRQ_-01 and from RSRQ_35 to RSRQ_46 apply for the UE who can support extended RSRQ range.

#### 6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy

6.7.6.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT E-UTRAN RSRQ absolute measurement accuracy is within the specified limits for all bands, when the serving cell is NR FR1 and the target cell is E-UTRA.

6.7.6.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

6.7.6.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.6.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.6.1.

6.7.6.1.4	Test description

6.7.6.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.6.1.4.1-1.

Table 6.7.6.1.4.1-1: test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.6.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: FDD |
| 6.7.6.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.6.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.6.1-4 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: TDD |
| 6.7.6.1-5 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: TDD |
| 6.7.6.1-6 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.6.1.4.1-2.

Table 6.7.6.1.4.1-2: initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.6.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.6.1.4.3.

2. There are two carriers and two cells specified in the test, where NR Cell 1 is the NR PCell on the NR carrier and Cell 2 is the E-UTRA neighbour cell on the E-UTRA carrier and the target for the measurements.

6.7.6.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.6.1.5-1 and Table 6.7.6.1.5-2 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the RSRQ reported values in the periodic MeasurementReport. The RSRQ value of Cell 2 reported by the UE is compared to the expected RSRQ. If the value is outside the limits in Table 6.7.6.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.6.1.5-2  as appropriate and repeat steps 5-7.

6.7.6.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.6.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-RAT and GAP NEEDED Table H.3.1-3 Table H.3.1-3a Table H.3.1-7 with condition INTER-RAT Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.6.1.4.3-1A: MeasConfig (Test procedure step 3)

| Derivation path: Table H.3.1-2 with condition INTER-RAT and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigE-UTRA-DEFAULT(Periodical) | Table 6.7.6.1.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-EVENT | Table 6.7.6.1.4.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.6.1.4.3-1B: ReportConfigInterRAT-EVENT (Table 6.7.6.1.4.3-1A)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-141 with condition EVENT_B1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  |  |
| b1-ThresholdEUTRA CHOICE { |  |  |  |
| rsrq | 34 | Set threshold to -3dB to ensure measId 2 will never be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.6.1.4.3-2: ReportConfigE-UTRA-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-141 with Condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.6.1.5	Test requirement

Table 6.7.6.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Tables 6.7.6.1.5-1 and 6.7.6.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 6.7.6.1.5-3.

Table 6.7.6.1.5-1: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 6.7.6.1.5-2: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | Depending on band group | dBm/15kHz | -83 | -104.70 | -119.5+ ΔBG_offset |
| Ês/Noc |  | dB | -1.75 | -3.2 | -3.2 |
| Ês/IotNote5 |  | dB | -1.75 | -3.2 | -3.2 |
| RSRPNote5 | Depending on band group | dBm/15kHz | -84.75 | -107.90 | -122.7+ ΔBG_offset |
| RSRQNote5 | Depending on band group | dB | -14.76 | -15.69 | -15.69 |
| IoNote5 | Depending on band group | dBm/Ch BW | -53 + 10log(NRB,c /50) | -75.22 + 10log(NRB,c /50) | -90.02+ ΔBG_offset + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: Void Note 8: Void Note 9: Void Note 10: Void Note 11: ΔBG_offset for LTE band group is defined in TS 36.521-3 [26] clause 3.5.1, Table 3.5.1-1A. |  |  |  |  |  |

Table 6.7.6.1.5-3: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 4 | 0 | 0 |
| Highest reported value (Cell 2) | 16 | 16 | 16 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest reported value (Cell 2) | 1 | 0 | 0 |
| Highest reported value (Cell 2) | 19 | 17 | 17 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.7	E-UTRAN RS-SINR

#### 6.7.7.0	Minimum conformance requirements

##### 6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy

The measurement period of E-UTRA RS-SINR in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3 of TS 38.133 [6].

The accuracy requirements of E-UTRA RS-SINR measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RS-SINR Accuracy Requirements in clause 9.1.17.3 of TS 36.133 [23]:

The reporting range and mapping for E-UTRA RS-SINR measurements shall be the same as specified for RS-SINR measurements in clause 9.1.17.1 of TS 36.133 [23]:

The requirements for absolute accuracy of intra-frequency RS-SINR in this clause apply to a cell on the same frequency as that of the serving cell.

The accuracy requirements in Table 6.7.7.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.18 of TS 36.133 [23] for a corresponding Band.

Table 6.7.7.0.1-1: Intra-frequency RS-SINR absolute accuracy

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| 3.0 | 4 | -3 dB Note 5 | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 3.5 | 4 | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5 of TS 36.133 [23]. NOTE 5: The requirements apply for Ês/Iot ≤ 25 dB. |  |  |  |  |  |

The reporting range of RS-SINR measurement is defined from -23 dB to 40 dB with 0.5 dB resolution.

The mapping of the measured quantity is defined in table 6.7.7.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.7.0.1-2: RS-SINR measurement report mapping

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RS-SINR_000 | RS-SINR -23 | dB |
| RS-SINR_001 | -23 RS-SINR < -22.5 | dB |
| … | … | … |
| RS-SINR_126 | 39.5 RS-SINR < 40 | dB |
| RS-SINR_127 | 40 RS-SINR | dB |

#### 6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy

6.7.7.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT E-UTRAN RS-SINR absolute measurement accuracy is within the specified limits for all bands, when the serving cell is NR FR1 and the target cell is E-UTRA.

6.7.7.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA and rs-SINR-MeasEUTRA.

6.7.7.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.7.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.7.1.

6.7.7.1.1.4	Test description

6.7.7.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.7.1.4.1-1.

Table 6.7.7.1.4.1-1: test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.7.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: FDD |
| 6.7.7.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.7.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.7.1-4 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: TDD |
| 6.7.7.1-5 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: TDD |
| 6.7.7.1-6 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.7.1.4.1-2.

Table 6.7.7.1.4.1-2: initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.7.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.7.1.4.3.

2. There are two carriers and two cells specified in the test, where NR Cell 1 is the NR PCell on the NR carrier and Cell 2 is the E-UTRA neighbour cell on the E-UTRA carrier and the target for the measurements.

6.7.7.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.7.1.5-1 and Table 6.7.7.1.5-2 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the RS-SINR reported values in the periodic MeasurementReport. The RS-SINR value of Cell 2 reported by the UE is compared to the expected RS-SINR. If the value is outside the limits in Table 6.7.7.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.7.1.5-2 as appropriate and repeat steps 5-7.

6.7.7.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.7.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-RAT and GAP NEEDED Table H.3.1-3 Table H.3.1-3a Table H.3.1-7 with condition INTER-RAT Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.7.1.4.3-1A: MeasConfig (Test procedure step 3)

| Derivation path: Table H.3.1-2 with condition INTER-RAT and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigE-UTRA-DEFAULT(Periodical) | Table 6.7.7.1.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-EVENT | Table 6.7.7.1.4.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.7.1.4.3-1B: ReportConfigInterRAT-EVENT (Table 6.7.7.1.4.3-1A)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-141 with condition EVENT_B1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  |  |
| b1-ThresholdEUTRA CHOICE { |  |  |  |
| sinr | 127 | Set threshold to 40dB to ensure measId 2 will never be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.7.1.4.3-2: ReportConfigE-UTRA-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-141 with Condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.7.1.5	Test requirement

Table 6.7.7.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Tables 6.7.7.1.5-1 and 6.7.7.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 6.7.7.1.5-3.

Table 6.7.7.1.5-1: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 6.7.7.1.5-2: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| Noc1Note4 | Depending on band group | dBm/15kHz | -88 | -108.50 | -119.5+ ΔBG_offset |
| CRS Ês/Noc1 |  | dB | -1.75 | 20.0 | -3.2 |
| CRS Ês/IotNote5 |  | dB | -1.75 | 20.0 | -3.2 |
| RSRPNote5 | Depending on band group | dBm/15kHz | -89.75 | -88.50 | -122.7+ ΔBG_offset |
| RS-SINRNote5 |  | dB | -1.75 | 20 | -3.2 |
| IoNote5 | Depending on band group | dBm/Ch BW | -58.00 + 10log(NRB,c /50) | -60.68 + 10log(NRB,c /50) | -90.02+ ΔBG_offset + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over CRS subcarriers and time and shall be modelled as AWGN of appropriate power for Noc1 to be fulfilled. Note 4a: Void Note 5: CRS Ês/Iot, RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: Void Note 8: Void Note 9: Void Note 10: Void Note 11: ΔBG_offset for LTE band group is defined in TS 36.521-3 [26] clause 3.5.1, Table 3.5.1-1A. |  |  |  |  |  |

Table 6.7.7.1.5-3: RS-SINR Intra frequency absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 35 | 79 | 32 |
| Highest reported value (Cell 2) | 51 | 94 | 49 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest reported value (Cell 2) | 33 | 77 | 31 |
| Highest reported value (Cell 2) | 53 | 96 | 50 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.8	CLI Measurements

#### 6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing.

-	TT analysis is missing.

-	Test Procedure is FFS.

-	Test applicability needs to be updated

##### 6.7.8.1.1	Test purpose

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits with SRS-RSRP measurement requirements in TS 38.133 [6] clause 10.1.22.1.1.

##### 6.7.8.1.2	Test applicability

FFS

##### 6.7.8.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.6.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.8.1.

##### 6.7.8.1.4	Test description

6.7.8.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.8.1.4.1-1. Test environment parameters are given in Table 4.7.6.2.4.1-2.

Table 6.7.8.1.4.1-1: Applicable NR configurations for FR1 SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 15kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 2 | 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 6.7.8.1.4.1-2: SRS Configuration parameters for EN-DC SRS-RSRP accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.6.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

- Message contents are defined in clause 6.7.8.1.4.3.

- Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is NR FR1 cell (PSCell). Cell 2 is the target for SRS-RSRP measurements. Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS configuration in Table 6.7.8.1.5-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

6.7.8.1.4.2	Test procedure

FFS

6.7.8.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

FFS

##### 6.7.8.1.5	Test requirement

Table 6.7.8.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.7.8.1.5-1: FR1 test parameters for SRS-RSRP accuracy for PCell

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~2 |  | freq1 | freq1 | freq1 |
| Duplex mode |  | 1~2 |  | TDD | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 2 |  | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 40: NRB,c = 106 | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 2 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 2 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~2 |  | OP.1 | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 2 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~2 |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1~2 | s | 17.67 | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1~2 | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 4 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 3 | 2 |  | Not applicableNote 4 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 4: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table 6.7.8.1.5-2: FR1 test parameters for SRS-RSRP accuracy for neighbour cell UE

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| on SRS |  | 1~2 | dB | 1 | 1 | 1 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -105+TT | -87+TT | -113+TT |
|  | NR_TDD_FR1_C |  |  |  |  | -112+TT |
|  | NR_TDD_FR1_D |  |  |  |  | -111.5+TT |
|  | NR_TDD_FR1_E |  |  |  |  | -111+TT |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -87 | -110 |
|  | NR_TDD_FR1_C |  |  |  |  | -109 |
|  | NR_TDD_FR1_D |  |  |  |  | -108.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -108 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/9.36 MHz | -74.51 | -56.51 | -82.51 |
|  | NR_TDD_FR1_C |  |  |  |  | -81.51 |
|  | NR_TDD_FR1_D |  |  |  |  | -81.01 |
|  | NR_TDD_FR1_E |  |  |  |  | -79.51 |
|  | NR_TDD_FR1_A NOTE 5 | 2 | dBm/38.16 MHz | Not applicableNote 6 | -53.42 | -76.42 |
|  | NR_TDD_FR1_C |  |  |  |  | -75.42 |
|  | NR_TDD_FR1_D |  |  |  |  | -74.92 |
|  | NR_TDD_FR1_E |  |  |  |  | -74.42 |
| on SRS |  | 1~2 | dB | 1+TT | 1+TT | 1+TT |
| Propagation condition |  | 1~2 |  | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1~2 |  | 1x2 | 1x2 | 1x2 |
| SRS configuration |  | 1 |  | SRSConf.1 | SRSConf.1 | SRSConf.1 |
|  |  | 2 |  | SRSConf.2 | SRSConf.2 | SRSConf.2 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table 6.7.8.1.5-3: SRS configuration parameters for FR1 SRS-RSRP accuracy

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |
|  | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
| SRS-Resource | SRS-ResourceId | 0 | 0 |
|  | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n1 | n1 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 12 | 12 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20, 9 | sl40, 19 |
|  | sequenceId | 0 | 0 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.9	L1-SINR measurement for beam reporting

#### 6.7.9.0	Minimum conformance requirements

##### 6.7.9.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off

The UE shall be capable of performing L1-SINR measurements with the CSI-RS configured as CMR and no dedicated resource configured as IMR for L1-SINR computation, and the UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only is defined in Table 6.7.9.0.1-1 for FR1 and in Table 6.7.9.0.1-2 for FR2, where

For the value of M,

-	For periodic and semi-persistent CSI-RS resources as CMR, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise;

-	For aperiodic CSI-RS resources as CMR, M=1.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.28.1 and 10.1.28.3 of TS 38.133 [6] if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured for all resources in the resource set.

For the value of P in FR1,

-	P=11-TCSI-RSMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For the value of P in FR2,

-	P=1, when CSI-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSMRGP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P=3, when CSI-RS is not overlapped with measurement gap and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P= 11-TCSI-RSMRGP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P=31-TCSI-RSMRGP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,MGRP), when CSI-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P=31-TCSI-RSMRGP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

Where:

TSMTCperiod = the configured SMTC1 period or SMTC2 period if configured.

TCSI-RS = the periodicity of CSI-RS configured for L1-SINR measurement

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Note: The overlap between CSI-RS for L1-SINR measurement and SMTC means that CSI-RS for L1-SINR measurement is within the SMTC window duration.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Table 6.7.9.0.1-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

Table 6.7.9.0.1-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

The accuracy requirements in Table 6.7.9.0.1-3 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.1 for a corresponding Band for each relevant CSI-RS based CMR.

-	The bandwidth of CSI-RS as CMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR is equal to or better than the accuracy requirements in Table 6.7.9.0.1-3.

Table 6.7.9.0.1-3: L1-SINR absolute accuracy for CSI-RS based CMR only in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±5.5 | ±6.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.8.4.1 and 10.1.27.1.

##### 6.7.9.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR

The UE shall be capable of performing L1-SINR measurements with the SSB configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to SSB configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_SSB_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as SSB configured as CMR.

The value of TL1-SINR_Measurement_Period_SSB_CMR_IMR is defined in Table 6.7.9.0.2-1 for FR1 and in Table 6.7.9.0.2-2 for FR2, where

For the value of M

-	For periodic or semi-persistent NZP CSI-RS or CSI-IM resource as dedicated IMR, M=1 if the higher layer parameters timeRestrictionForChannelMeasurements and/or timeRestrictionForInterferenceMeasurements are configured, and M=3 otherwise;

For the value of N in FR2

-	N = 8.

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	the value of PCMR shall be derived in the same way as the value of P used for SSB based L1-RSRP measurement in clause 9.5.4.1 of TS 38.133 [6], in which the occasions and period of the SSB for CMR shall be used instead.

-	the value of PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2 of TS 38.133 [6], in which the occasions and period of the NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For L1-SINR measurement with SSB as CMR and CSI-RS or CSI-IM as IMR, the requirement shall apply if the CSI-RS is configured as IMR with repetition field as “repetition = OFF” or CSI-IM is configured as IMR.

For L1-SINR measurement with SSB as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if SSB occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap

Table 6.7.9.0.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. |  |

Table 6.7.9.0.2-2: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. |  |

The accuracy requirements in Tables 6.7.9.0.2-3 and 6.7.9.0.2-4 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.2 for a corresponding Band for each relevant SSB based CMR and IMR.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 6.7.9.0.2-3 and 6.7.9.0.2-4.

Table 6.7.9.0.2-3: L1-SINR absolute accuracy for SSB based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | NZP-IMR Ês/Iot |  | Io Note 1 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±4.0 | ±5.0 | 0 |  | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 6.7.9.0.2-4: L1-SINR absolute accuracy for SSB based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±4.5 | ±5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.8.4.2 and 10.1.27.2.

##### 6.7.9.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR

The UE shall be capable of performing L1-SINR measurements with the CSI-RS resource configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to CSI-RS resource configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as CSI-RS resource configured as CMR.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR is defined in Table 6.7.9.0.3-1 for FR1 and in Table 6.7.9.0.3-2 for FR2, where

For the value of M,

-	M=1 shall be applied if

-	aperiodic NZP-CSI-RS as CMR or dedicated IMR, or

-	aperiodic CSI-IMR as dedicated IMR, or

-	periodic and semi-persistent NZP-CSI-RS as CMR or dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured, or

-	periodic and semi-persistent CSI-IM as dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured;

-	M=3 otherwise.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.28.1 and 10.1.28.3 of TS 38.133 [6] if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured for all resources in the resource set.

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	The value of PCMR and PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2 of TS 38.133 [6], in which the occasions and period of the CSI-RS for CMR and NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead respectively.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS as IMR, the requirement shall apply only if CSI-RS resources as CMR and IMR are configured with the same repetition field and the number of CSI-RS resources in the resource sets for CMR and IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-IM as IMR, the requirement shall apply only if the number of CSI-RS resources in the resource set for CMR and the number of CSI-IM resources in the resource set for IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if CSI-RS occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap.

Table 6.7.9.0.3-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

Table 6.7.9.0.3-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

The accuracy requirements in Tables 6.7.9.0.3-3 and 6.7.9.0.3-4 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.3 for a corresponding Band for each relevant CSI-RS based CMR and IMR.

-	The bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 6.7.9.0.3-3 and 6.7.9.0.3-4.

Table 6.7.9.0.3-3: L1-SINR absolute accuracy for CSI-RS based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | NZP-IMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±4.0 | ±5.0 | 0 | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 6.7.9.0.3-4: L1-SINR absolute accuracy for CSI-RS based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±4.5 | ±5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.8.4.3 and 10.1.27.3.

#### 6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement

##### 6.7.9.1.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy

6.7.9.1.1.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

6.7.9.1.1.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support of L1-SINR measurements.

6.7.9.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.9.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.9.1.

6.7.9.1.1.4	Test description

6.7.9.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.1.1.4.1-1.

Table 6.7.9.1.1.4.1-1: Applicable NR configurations for FR1 L1-SINR test with CSI-RS based CMR and no dedicated IMR configured

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.9.1.1.4.1-2.

Table 6.7.9.1.1.4.1-2: Initial conditions for CSI-RS based L1-SINR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.9.1.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-SINR measurements. The UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.9.1.1.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.1.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.1.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.1.1.5-1.

3.	The UE shall start sending L1-SINR report including results of both CSI-RS#0 and CSI-RS#1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS#0 and CSI-RS#1 in the periodic L1-RSRP reports. If the value for both CSI-RSs is within the limits in Table 6.7.9.1.1.5-2 or Table 6.7.9.1.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.9.1.1.5-1 as appropriate and repeat steps 3-5.

6.7.9.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.9.1.1.4.3-1: Common Exception messages NR SA CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and CSI-SINR Table H.3.6A-2 with conditions CSI-RS and PERIODIC Table H.3.6A-3 with condition PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.9.1.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSB. |  |
| } |  |  |  |
| } |  |  |  |

6.7.9.1.1.5	Test requirement

Table 6.7.9.1.1.5-1 defines the primary level settings excluding test tolerances for all tests.

Each L1-SINR measurement report for each of the tests in Table 6.7.9.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.9.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.9.1.1.5-3 for test configuration 3.

Table 6.7.9.1.1.5-1: FR1 CSI-RS based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 |  | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -117+ ΔBG_offset |
|  |  |  | 3 |  | -96.00 | -117+ ΔBG_offset |
| Note2 |  |  | 1,2 | dBm/SSB SCS | -94.65 | -117+ ΔBG_offset |
|  |  |  | 3 |  | -93.00 | -114+ ΔBG_offset |
|  |  |  | 1~3 | dB | 10 | -2.2 |
| SSB RSRP Note3 |  | Depending on band group | 1,2 | dBm/SSB SCS | -84.65 | -119.2 + ΔBG_offset |
|  |  |  | 3 |  | -83.00 | -116.2 + ΔBG_offset |
| Io Note3 |  | Depending on band group | 1,2 | dBm/9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  |  |  | 3 | dBm/38.16 MHz | -51.53 | -80.90 + ΔBG_offset |
|  |  | 1~3 | dB | 10 | -2.2 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table 6.7.9.1.1.5-2: Same as Table Table 4.7.7.1.1.5-2

Table 6.7.9.1.1.5-3: Same as Table Table 4.7.7.1.1.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.9.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy

6.7.9.1.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR relative measurement accuracy is within the specified limits.

6.7.9.1.2.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support of L1-SINR measurements.

6.7.9.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.7.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.7.7.1.

6.7.9.1.2.4	Test description

6.7.9.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.1.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.7.9.1.2.4.1-2.

Table 6.7.9.1.2.4.1-1: Applicable NR configurations for FR1 L1-SINR test with CSI-RS based CMR and no dedicated IMR configured

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 6.7.9.1.2.4.1-2: Initial conditions for CSI-RS based L1-SINR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 6.7.9.1.2.4.3.

2.	Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-SINR measurements. The UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.9.1.2.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.1.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.1.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.1.1.5-1.

3.	The UE shall start sending L1-SINR report including results of both CSI-RS#0 and CSI-RS #1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS #0 and CSI-RS #1 in the periodic L1-SINR reports. The L1-SINR value for CSI-RS #1 is compared to the L1-SINR value for CSI-RS #0. If the difference is within the limits in Table 6.7.9.1.2.5-2 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.9.1.1.5-1 as appropriate and repeat steps 3-5.

6.7.9.1.2.4.3	Message contents

Same message contents as described in section 6.7.9.1.1.4.3

6.7.9.1.2.5	Test requirement

Table 6.7.9.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-SINR measurement report for each of the tests in Table 6.7.9.1.2.5-1 shall meet the corresponding relative accuracy requirements in Table 6.7.9.1.2.5-2.

Table 6.7.9.1.2.5-1: Same as Table 4.7.7.1.2.5-1

Table 6.7.9.1.2.5-2: Same as Table 4.7.7.1.2.5-2

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy

6.7.9.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

6.7.9.2.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support for L1-SINR measurements on the NR PSCell.

6.7.9.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.9.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.9.2.

6.7.9.2.4	Test description

6.7.9.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.2.4.1-1.

Table 6.7.9.2.4.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.9.2.4.1-2.

Table 6.7.9.2.4.1-2: Initial conditions for SSB based and CSI-RS based L1-SINR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 6.7.9.2.4.3.

2.	Cell 1 is the NR FR1 cell. Cell 1 is the target for SSB-based L1-RSRP measurements. The UE is configured one SSB resource set with two SSB resources and one CSI-RS resource set with two CSI-RS resource. UE is configured to perform RLM and BFD measurement based on the SSB resources 0 and 1. UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-RS resources as IMR. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

6.7.9.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.2.5-1. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

3.	The UE shall start sending L1-SINR report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 in the periodic L1-RSRP reports. If the value for both SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 is within the limits in Table 6.7.9.2.5-2 and Table 6.7.9.2.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6.	Set the parameters according to each sub-test in Table 6.7.9.2.5-1 as appropriate and repeat steps 3-5.

6.7.9.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.4.1.1.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and SS-SINR and CSI-RS_IMR Table H.3.6A-2 with conditions SSB and PERIODIC Table H.3.6A-3 with condition PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.4.1.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.9.2.5	Test requirement

Table 6.7.9.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.9.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.9.2.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.9.2.5-3 for test configuration 3.

Table 6.7.9.2.5-1: FR1 SSB based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| CSI-RS configuration |  | 1 |  | CSI-RS 1.1A FDD | CSI-RS 1.1A FDD |
|  |  | 2 |  | CSI-RS 1.1A TDD | CSI-RS 1.1A TDD |
|  |  | 3 |  | CSI-RS 2.1A TDD | CSI-RS 2.1A TDD |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | ssb-Index-SINR-r16 | ssb-Index-SINR-r16 |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -96.00 | -117+ ΔBG_offset |
| Note2 |  | 1,2 | dBm/SSB SCS | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -93.00 | -114+ ΔBG_offset |
|  |  | 1~3 | dB | 10 | -2.2 |
| SSB RSRP Note3 | Depending on band group | 1,2 | dBm/SSB SCS | -84.65 | -119.2 + ΔBG_offset |
|  |  | 3 |  | -83.00 | -116.2 + ΔBG_offset |
| Io Note3 | Depending on band group | 1,2 | dBm/9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  |  | 3 | dBm/38.16 MHz | -51.53 | -80.90 + ΔBG_offset |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table 6.7.9.2.5-2: Same as Table 4.7.7.2.5-2

Table 6.7.9.2.5-3: Same as Table 4.7.7.2.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy

##### 6.7.9.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy

6.7.9.3.1.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

6.7.9.3.1.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support of L1-SINR measurements.

6.7.9.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.9.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.9.3.

6.7.9.3.1.4	Test description

6.7.9.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.3.1.4.1-1.

Table 6.7.9.3.1.4.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.9.3.1.4.1-2.

Table 6.7.9.3.1.4.1-2: Initial conditions for CSI-RS based CMR and CSI-IM based IMR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.9.3.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based CMR and CSI-IM based IMR measurements. The UE is configured one CSI-RS resource set with two CSI-RS resources and one CSI-IM resource set with two CSI-IM resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. UE is configured to perform L1-SINR measurement based on the configured CSI-RS as CMR and CSI-IM as IMR. The connection setup is done according to the settings in Annex C.1.1.

6.7.9.3.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.3.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.3.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.3.1.5-1. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

3.	The UE shall start sending L1-SINR report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS#0+CSI-IM#0 and CSI-RS#1+CSI-IM#1 in the periodic L1-SINR reports. If the value for both CSI-RSs is within the limits in Table 6.7.9.3.1.5-2 and Table 6.7.9.3.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6.	Set the parameters according to each sub-test in Table 6.7.9.3.1.5-1 as appropriate and repeat steps 3-5.

6.7.9.3.1.4.3	Message contents

Same message content as in subclause 6.7.9.1.4.3 with the following exception:

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.9.3.1.4.3-1: Common Exception messages NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and CSI-SINR and CSI-IM_IMR Table H.3.6A-2 with conditions CSI-RS and PERIODIC Table H.3.6A-3 with condition PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.9.3.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

6.7.9.3.1.5	Test requirement

Table 6.7.9.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-SINR measurement report for each of the tests in Table 6.7.9.3.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.9.3.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.9.3.1.5-3 for test configuration 3.

Table 6.7.9.3.5-1: FR1 CSI-RS based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -96.00 | -117+ ΔBG_offset |
| Note2 |  | 1,2 | dBm/SSB SCS | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -93.00 | -114+ ΔBG_offset |
|  |  | 1~3 | dB | 10 | -2.2 |
| SSB RSRP Note3 | Depending on band group | 1,2 | dBm/SSB SCS | -84.65 | -119.2 + ΔBG_offset |
|  |  | 3 |  | -83.00 | -116.2 + ΔBG_offset |
| Io Note3 | Depending on band group | 1,2 | dBm/9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  |  | 3 | dBm/38.16 MHz | -51.53 | -80.90 + ΔBG_offset |
|  |  | 1~3 | dB | 10 | -2.2 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table 6.7.9.3.5-2: Same as Table 4.7.7.3.5-2

Table 6.7.9.3.5-3: Same as Table 4.7.7.3.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.9.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy

6.7.9.3.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR relative measurement accuracy is within the specified limits.

6.7.9.3.2.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support of L1-SINR measurements.

6.7.9.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.7.9.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.9.3.

6.7.9.3.2.4	Test description

6.7.9.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.3.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.7.9.3.2.4.1-2.

Table 6.7.9.3.2.4.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.9.3.2.4.1-2.

Table 6.7.9.3.2.4.1-2: Initial conditions for CSI-RS based CMR and CSI-IM based IMR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.9.3.2.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based CMR and CSI-IM based IMR measurements. The UE is configured one CSI-RS resource set with two CSI-RS resources and one CSI-IM resource set with two CSI-IM resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. UE is configured to perform L1-SINR measurement based on the configured CSI-RS as CMR and CSI-IM as IMR. The connection setup is done according to the settings in Annex C.1.1.

6.7.9.3.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.3.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.3.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.3.2.5-1.

3.	The UE shall start sending L1-SINR report including results of both CSI-RS#0 and CSI-RS #1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS #0 and CSI-RS #1 in the periodic L1-SINR reports. The L1-SINR value for CSI-RS #1 is compared to the L1-SINR value for CSI-RS #0. If the difference is within the limits in Table 6.7.9.3.2.5-2 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.9.3.2.5-1 as appropriate and repeat steps 3-5.

6.7.9.3.2.4.3	Message contents

Same message contents as described in section 6.7.9.3.1.4.3

6.7.9.3.2.5	Test requirement

Table 6.7.9.3.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-SINR measurement report for each of the tests in Table 6.7.9.3.2.5-1 shall meet the corresponding relative accuracy requirements in Table 6.7.9.3.2.5-2.

Table 6.7.9.3.2.5-1: Same as Table 6.7.9.3.1.5-1

Table 6.7.9.3.2.5-2: Same as Table 4.7.7.3.2.5-2

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.