| 3GPP TR 38.860 V0.2.0 (2021-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on extended 600MHz NR band; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background	7

4.1	Options for study: B1	7

4.2	Options for study: B2	7

4.3	Options for study: BX (if required)	7

5	Regulatory landscape	7

5.1	General	7

5.2	ITU Region 3	9

6	Frequency band arrangements and regulatory background	10

6.1	Operating band and channel bandwidth	10

6.2	Adjacent 3GPP bands	10

6.3	Nearby non-3GPP Services	10

6.3.1	Introduction	10

6.3.2	Coexistence between 600 MHz and TV Services	10

6.3.3	Coexistence between 600 MHz Radio Astronomy Service (RAS)	10

6.4	Duplex filter implementation issues	10

7	List of band specific issues for APT600 MHz frequency band	10

8	Study of NR specific issues	11

8.1	UE aspect issues	11

8.1.1	UE transmitter requirements	11

8.1.2	UE receiver requirements	11

8.2	BS aspect issues	11

X	Examples for styles	11

X.1	Heading styles	11

X.2	Other common styles	11

B.1	Heading levels in an annex	13

Annex <B>: <Informative annex title for a Technical Report>	14

Annex <X> (informative): Change history	15

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This clause is mandatory; do not alter the text in any way other than to choose between "Specification" and "Report".

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document is a technical report for the study item of APT 600 MHz NR FDD band.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[10]	3GPP TS 38.101: " 5G; NR; User Equipment (UE) radio transmission and reception

[11]	3GPP TS 38.104: " 5G; NR; Base Station (BS) radio transmission and reception

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Background

## 4.1	Options for study: B1

## 4.2	Options for study: B2

## 4.3	Options for study: BX (if required)

# 5	Regulatory landscape

## 5.1	General

The 470 - 694 MHz frequency range is allocated to the broadcasting service and mobile service on a co-primary basis in Region 3 [1]. The frequency band 470 - 698 MHz, or parts thereof, was identified by WRC-15 in 7 countries in Region 3 through footnote No. 5.296A for use by those administrations as listed wishing to implement terrestrial IMT systems. In addition, there was interest from other significant markets to do the same. Elsewhere, USA, Mexico and several other countries in Region 2 also identified this band for IMT through footnotes 5.295 and 5.308A. It is noted that resolves 2 of Resolution 224 (Rev.WRC-19) encourages the following:

encourages administrations to take into account results of the existing relevant ITU Radiocommunication Sector studies, when implementing IMT applications/systems in the frequency bands

- 694-862 MHz in Region 1,

- in the frequency band 470-806 MHz in Region 2,

- in the frequency band 790-862 MHz in Region 3,

- in the frequency band 470-698 MHz, or portions thereof, for those administrations mentioned in No. 5.296A, and

- in the frequency band 698-790 MHz, or portions thereof, for those administrations mentioned in No. 5.313A.

The regulatory landscape overview for all regions in frequency range 470 – 694 MHz is provided in table x.1-1, based on ITU radio regulations [3].

Table x.1-1: ITU-R frequency allocation to services [3]

NOTE: 	Since regulatory study of frequency range around 600 MHz is for Region 3, the outcomes of this study item will not impact any requirements defined for US 600 MHz band, i.e. band n71.

Below, ITU RR footnotes referred in this clause are listed:

5.149		In making assignments to stations of other services to which the bands: 13 360-13 410 kHz, 25 550-25 670 kHz, 37.5-38.25 MHz, 73-74.6 MHz in Regions 1 and 3, 150.05-153 MHz in Region 1, 322-328.6 MHz, 406.1-410 MHz, 608-614 MHz in Regions 1 and 3, 1 330-1 400 MHz, 1 610.6-1 613.8 MHz, 1 660-1 670 MHz, 1 718.8-1 722.2 MHz, 2 655-2 690 MHz, 3 260-3 267 MHz, 3 332-3 339 MHz, 3 345.8-3 352.5 MHz, 4 825-4 835 MHz, 4 950-4 990 MHz, 4 990-5 000 MHz, 6 650-6 675.2 MHz, 10.6-10.68 GHz, 14.47-14.5 GHz, 22.01-22.21 GHz, 22.21-22.5 GHz, 22.81-22.86 GHz, 23.07-23.12 GHz, 31.2-31.3 GHz, 31.5-31.8 GHz in Regions 1 and 3, 36.43-36.5 GHz, 42.5-43.5 GHz, 48.94-49.04 GHz, 76-86 GHz, 92-94 GHz, 94.1-100 GHz, 102-109.5 GHz, 111.8-114.25 GHz, 128.33-128.59 GHz, 129.23-129.49 GHz, 130-134 GHz, 136-148.5 GHz, 151.5-158.5 GHz, 168.59-168.93 GHz, 171.11-171.45 GHz, 172.31-172.65 GHz, 173.52-173.85 GHz, 195.75-196.15 GHz, 209-226 GHz, 241-250 GHz, 252-275 GHz are allocated, administrations are urged to take all practicable steps to protect the radio astronomy service from harmful interference. Emissions from spaceborne or airborne stations can be particularly serious sources of interference to the radio astronomy service (see Nos. 4.5 and 4.6 and Article 29). (WRC-07)

5.295		In the Bahamas, Barbados, Canada, the United States and Mexico, the frequency band 470-608 MHz, or

portions thereof, is identified for International Mobile Telecommunications (IMT) – see Resolution 224 (Rev.WRC-19). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. Mobile service stations of the IMT system within the frequency band are subject to agreement obtained under No. 9.21 and shall not cause harmful interference to, or claim protection from, the broadcasting service of neighbouring countries. Nos. 5.43 and 5.43A apply. (WRC-19)

5.296A	In Micronesia, the Solomon Islands, Tuvalu and Vanuatu, the frequency band 470-698 MHz, or portions thereof, and in Bangladesh, Maldives and New Zealand, the frequency band 610-698 MHz, or portions thereof, are identified for use by these administrations wishing to implement International Mobile Telecommunications (IMT) – see Resolution 224 (Rev.WRC-19). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. The mobile allocation in this frequency band shall not be used for IMT systems unless subject to agreement obtained under No. 9.21 and shall not cause harmful interference to, or claim protection from, the broadcasting service of neighbouring countries. Nos. 5.43 and 5.43A apply. (WRC-19)

5.305	Additional allocation: in China, the band 606-614 MHz is also allocated to the radio astronomy service on a primary basis.

5.306	Additional allocation: in Region 1, except in the African Broadcasting Area (see Nos. 5.10 to 5.13), and

in Region 3, the band 608-614 MHz is also allocated to the radio astronomy service on a secondary basis.

5.307		Additional allocation: in India, the band 608-614 MHz is also allocated to the radio astronomy service on

a primary basis.

5.308A 	In the Bahamas, Barbados, Belize, Canada, Colombia, the United States, Guatemala and Mexico, the

frequency band 614-698 MHz, or portions thereof, is identified for International Mobile Telecommunications (IMT) – see Resolution 224 (Rev.WRC-19). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. Mobile service stations of the IMT system within the frequency band are subject to agreement obtained under No. 9.21 and shall not cause harmful interference to, or claim protection from, the broadcasting service of neighbouring countries. Nos. 5.43 and 5.43A apply. (WRC-19)

5.313A	The frequency band, or portions of the frequency band 698-790 MHz, in Australia, Bangladesh, Brunei Darussalam, Cambodia, China, Korea (Rep. of), Fiji, India, Indonesia, Japan, Kiribati, Lao P.D.R., Malaysia, Myanmar (Union of), New Zealand, Pakistan, Papua New Guinea, the Philippines, the Dem. People’s Rep. of Korea, Solomon Islands, Samoa, Singapore, Thailand, Tonga, Tuvalu, Vanuatu and Viet Nam, are identified for use by these administrations wishing to implement International Mobile Telecommunications (IMT). This identification does not preclude the use of these frequency bands by any application of the services to which they are allocated and does not establish priority in the Radio Regulations. (WRC-19)

## 5.2	ITU Region 3

Editor’s note: refer to the APT LS; any APT specific regulatory matters

ITU Region 3 specific regulatory aspects for the frequency range 470 – 694 MHz are provided in this clause.

Referring to the APT LS, AWG-26 agreed to undertake the study to revise the APT/AWG/REP-79 [4] to develop frequency arrangements in the band 470-703 MHz for APT Members that wish to implement both the APT700 and a 600 MHz frequency arrangements that is optimal for APT Members.

According to Radio Regulations, in Region 3, the 610 – 890 MHz band is allocated to Fixed Service, Mobile Service and Broadcasting Service on a primary basis. Additionally, 606 – 614 MHz band is allocated for Radio Astronomy Service in China (i.e. footnote 5.305) and band 608 - 614 MHz is allocated for Radio Astronomy Service in India (i.e. footnote 5.307). Footnote 5.149 urges administrator to protect the radio astronomy service from harmful interference.

Digital television has various channel allocations depending on country allocations in Region 3. IMT and DTV coexistence may require additional consideration on a per-country basis, leading to potential need for a guard bands.

No specific regional regulatory requirements on unwanted emissions were identified for the IMT operation in frequency range 470 – 694 MHz in Region 3, so far.

# 6	Frequency band arrangements and regulatory background

## 6.1	Operating band and channel bandwidth

## 6.2	Adjacent 3GPP bands

## 6.3	Nearby non-3GPP Services

### 6.3.1	Introduction

### 6.3.2	Coexistence between 600 MHz and TV Services

### 6.3.3	Coexistence between 600 MHz Radio Astronomy Service (RAS)

## 6.4	Duplex filter implementation issues

# 7	List of band specific issues for APT600 MHz frequency band

Below is the list of issues that are studied for the APT 600 MHz Band xx

# 8	Study of NR specific issues

## 8.1	UE aspect issues

### 8.1.1	UE transmitter requirements

### 8.1.2	UE receiver requirements

## 8.2	BS aspect issues

# X	Examples for styles

The main text of the document should start here, after the above clauses have been added.

The following styles and editing techniques are aimed to help in the formatting of the document using the 3GPP Template: 3GPP_70.dot, available from the 3GPP FTP site (ftp://ftp.3gpp.org/Information).

## X.1	Heading styles

Heading styles are included in the 3GPP TS Template and are used as follows:

Do not use any built-in automatic numbering for 3GPP documents. Although this is sometimes useful in the early drafting stages of a document, once the document has been placed under change control, the clause numbering needs to be fixed in order to keep cross-reference consistency as the 3GPP specification set develops.

Heading 1:	Used for Main clauses (1, 2, 3, etc.). Also used for Annex clauses (A.1, A.2, etc.).

Heading 2:	Used for Main clauses (4.1, 4.2, 5.1, 5.2, etc.). Also used for Annex clauses (A.1.1, A.1.2, etc.).

Heading 3:	Used for 2nd level clauses (4.1.1, 4.1.2, 5.1.1, 5.1.2, etc.). Also used for Annex clauses (A.2.1.1, A.2.1.2, etc.).

Heading 4 & 5:	Used for 3rd and 4th level clauses and Annex clauses.

Heading 6 & 7:	Not used, instead use style "H6" so that the title appears in the document, but does not appear in the Table of Contents.

Heading 8:	Used for Main Annex titles in Specifications (3GPP TS) (e.g. Annex A (normative): ).

Heading 9:	Used for Main Annex titles in Reports (3GPP TR) (e.g. Annex A: ).

## X.2	Other common styles

Normal:	Used for main document text.

NO:	Used for Notes in the text (Allows Tab and Indent). See example below.

NW:	Same as NO, but Without line space after. Used when there are many notes in sequence.

NOTE 1:	This is an example of a note formatted in style NW. The style is designed to allow space for note numbering and line wrap with a hanging indent. There is no line space after.

NOTE 2:	This is an example of a note formatted in style NO. The style is designed to allow space for note numbering and line wrap with a hanging indent. There is a line space after.

Bullet styles:	The following bullet styles are provided.

B1:	Bullet level 1 for main bullet points.

B2:	Bullet level 2 for sub bullets.

B3-B5:	for further sub bullets.

NOTE:	Bullets are usually formatted manually, using a hyphen ( - ) or alphanumeric identifiers: a), b), or 1), 2) etc. followed by a tab character. Automatic bullet features should not be used as they may be lost if template styles are re-applied later.

Table styles:	TAH, TAL, TAC, TAR, TAN, for TAble Headers, Left justified, Centred, Right justified and Notes in tables: Style TH is used for the Table Heading (title or caption). See example below.

Table 1: Example of table styles

| Col 1 Header (TAH) | Col 2 Header (TAH) | Col 3 Header (TAH) |
| --- | --- | --- |
| Left Justified (TAL) | Centred (TAC) | Right Justified (TAR) |
| NOTE: A special style is provided for notes within a table (TAN). |  |  |

Warning: The default setting for table cells is to disallow rows to break at a page boundary. If you include tables with very long cells, likely to extend beyond the bottom of the page (bearing in mind the table header and the page header and footers, and the margin settings), then you must enable that row's "Allow row to break across pages" setting.

Figure styles:	Figures and graphics are formatted with style "TH" which keeps the figure with the following paragraph, usually the figure title. Figure Titles (captions) are formatted with style "TF". See example below.

Figure 1: Example figure layout. To remove "float over text" select the graphic and "Format Object ..." - De-select "float over text" in the Position Tab
Annex <B> (informative):
<Informative annex for a Technical Specification>

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

# B.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

Annex <B>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-02 | RAN4#98-e | R4-2103265 |  |  |  | Report skeleton R4-2103265, “Blank TR for extended 600MHz NR band” | 0.0.0 |
| 2021-04 | RAN4#98-bis-e | R4-21xxxxx |  |  |  | Conversion to the latest version of the TR template and TR number added. Agreed Text Proposals in RAN4#98-e: R4-2103267, "Regulatory aspects for the 600MHz range in APT region” | 0.2.0 |