| 3GPP TR 38.914 V0.3.0 (2025-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on 6G Scenarios and Requirements; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	9

4	Deployment scenarios	10

4.0	General	10

4.1	Indoor hotspot	10

4.2	Dense urban	13

4.3	Rural	17

4.4	Urban macro	19

4.5	Sub-Urban macro	23

4.6	High speed Train	24

4.7	Extreme long-distance coverage in low density area	28

4.8	Urban coverage for massive connection	28

4.9	Air-to-Ground Scenario	29

4.10	Non-Terrestrial Network	30

4.11	Urban grid	32

4.12	Highway Scenario	35

4.13	 Indoor Factory (InF)	37

5	Requirements and key technical principles	40

5.1	Key performance indicators	40

5.2	Requirements for architecture and migration	47

5.3	Operational requirements	48

5.3.1	Spectrum	48

5.3.2	Diverse device types	49

5.3.3	RAN sharing	49

5.3.4	Ease of operation and Self Organization	49

5.3.5	Network Resilience	49

5.3.6	Service awareness in RAN	49

5.4	Requirements of New and Existing Services	49

5.5	Key technical principles	50

5.6	Testing and Conformance requirements	51

Annex A (informative): Change history	51

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

In June 2023, ITU-R Working Party 5D (WP5D) completed the development of 6G Framework in ITU-R M.2160 [2]. Compared with IMT-2020, the 6G framework presents:

-	Expanded and new usage scenarios, and

-	Enhanced and new capabilities.

In December 2024, ITU WP 5D invited External Organizations to provide their inputs on the minimum technical performance requirements (TPR) for IMT-2030 radio interface technologies, including but not limited to,

-	Proposed candidate items for minimum technical performance requirements based on M.2160 Usage Scenarios and Capabilities, including necessary background information and justification, which are requested to be provided preferably by WP 5D #48 (4-13 February 2025).

-	Proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49 (24 June – 3 July 2025, TBC)

Therefore, one purpose of this study is to investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160 [2], and the associated target values for the identified minimum TPRs. The output of this study is expected to be shared with ITU-R in the form of official liaisons.

5G has revolutionised the way we connect, offering unprecedented throughput, low latency, and the capacity to handle a vast number of connected devices, thus driving innovation in the consumer market and various verticals such as healthcare, automotive, industrial automation, satellite and smart cities. Society is now very much dependent on mobile communications. Business momentum triggered by 5G is contributing to the digital transformation of the society, and frontiers of network capabilities are expanding. Networks are being rolled out that include many of the 5G planned capabilities. The lessons learnt from the development, deployment and operation of 5G networks around the world (e.g. system complexity and large number of migration options) will be considered to ensure that 6G will support future opportunities, e.g. based on new technology trends that enable continuous innovation.

6G aims to support societal advancements and to bring value to society in the 2030s and beyond in secure, resilient, environmentally and economically sustainable ways. In addition to providing diverse and novel 6G services, other considerations are needed, e.g. CAPEX/OPEX reduction, improvement of overall 3GPP system performance, and migration from and interworking with 5G aspects.

Therefore, this study aims to identify typical usage scenarios for 6G radio technology and the required capabilities for each scenario.The study will also translate these required capabilities into technical metrics that can be evaluated from a RAN perspective.

1	Scope

The present document reports on the scenarios and requirements for 6G Radio, which can be used as not only guidance to the technical work in 3GPP RAN WGs but also input for ITU-R when developing IMT-2030 technical performance requirements.

The objectives of the study item are:

-	investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

-	The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

-	Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

-	Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

-	Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

-	Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

-	Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

-	Energy efficiency and energy saving: both for network and device

-	Enhanced spectral efficiency

-	Enhanced overall coverage, focus on cell-edge performance and UL coverage

-	Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

-	Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

-	Target scalable and forward compatible design for diverse device types

-	Improved spectrum utilization and operations taking into account diverse spectrum allocations

-	Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

-	Aim at a harmonized 6G Radio design for TN and NTN, including their integration

-	System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

-	Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas:

-	Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc…

-	Overall high-level aspects of 5G to 6G migration

-	RAN architecture and interfaces, including RAN-Core interface

-	Coordination of 6G AI/ML framework

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

[3]	Report ITU-R M.[IMT-2030. TECH PERF REQ]

[4]	Report ITU-R M.[IMT-2030.EVAL]

[5]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies".

[6]	3GPP TR 38.760-3: "Study on 6G Radio RAN3 aspects".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

Carea	area traffic capacity

ρ	TRxP density (TRxP/m2)

M	number of TRxPs

N	number of users

Q	number of bands

ri	normalized user throughput of user i

R	total peak data rate

Ri (T)		number of correctly received bits by user i (downlink) or from user i (uplink)

Ri (Ti)	number of correctly received bits of user i

Ruser	user experienced data rate

Rp	peak data rate

SEavg	average spectral efficiency

SEp	peak spectral efficiency in a band

SEpi	peak spectral efficiency in a component band i

SEuser	5th percentile user spectral efficiency

T		time over which the data bits are received

Ti	active session time for user i

W	channel bandwidth

Wi	bandwidth for a component band i

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

AI	Artificial Intelligence

BS	Base Station

CAPEX	Capital Expenditure

CDF	Cumulative Distribution Function

DL	Downlink

HRLLC	Hyper Reliable and Low Latency Communication

IC	Immersive Communication

IMT	International Mobile Telecommunications

IP	Internet Protocol

ISD	Inter-Site Distance

ITU	International Telecommunication Union

ITU-R	International Telecommunication Union Radiocommunication Sector

KPI	Key Performance Indicator

OPEX	Operational Expenditure

PDU	Protocol Data Unit

QoS	Quality of Service

RAN	Radio Access Network

RF	Radio Frequency

RIT	Radio Interface Technology

SDU	Service Data Unit

SRIT	Set of Radio Interface Technologies

TR	Technical Report

TRxP	Transmission and Reception Point

Tx	Transmitter

UE	User Equipment

UL	Uplink

WG	Working Group

4	Deployment scenarios

Editor note: Sensing related aspects will be included in section 4

Editor note: More deployment scenarios can be added

4.0	General

High-level descriptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD / Orbit, BS / UE antenna elements, UE distribution / speed and service profile are proposed in this TR. It is assumed that more detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. should be defined in a different TR.

In the deployment scenarios Indoor hotspot, Dense urban, Rural, Urban macro, Sub-Urban macro, High Speed Train, Urban grid, Highway and Indoor Factory, one or more types of sensing targets including at least UAV, human, vehicle, AGV can be dropped, with the detailed sensing related assumptions, e.g. distribution, mobility of the sensing targets referring to TR 38.901. The deployment scenarios in this section are defined for the sole purpose of evaluating RAN performance requirements or features, not intended to mandate actual deployments or network/device implementation.

Note: Regarding the various carrier frequency configurations and options listed in the tables below, only a sub-set of them will be required to be evaluated for ITU IMT-2030 self-evaluation and 3GPP internal evaluation, others may be optionally used for specific RAN features or requirements evaluation.

4.1	Indoor hotspot

The indoor hotspot deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor.

Some of its attributes are listed in Table 4.1.

Table 4.1: Attributes for indoor hotspot

| Attributes | Values or assumptions |  |
| --- | --- | --- |
| Carrier Frequency NOTE1 | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |  |
| Aggregated system bandwidth NOTE2, NOTE3 | Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |  |
| Layout | Single layer: - Indoor floor (Open office), 120 m x 50 m |  |
| ISD | 20m (Equivalent to 12TRxPs per 120 m x 50 m) |  |
| BS antenna elements NOTE5 | Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |  |
| UE antenna elements NOTE5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements Up to 16 Rx antenna elements (for RX only and intended only for FWA) Note: Above does not impact future discussions on UE capability |  |
| User distribution and UE speed NOTE4 | 10 users per TRxP 100% Indoor, 3 km/h |  |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |  |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700 MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990 MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes. Consider larger aggregated system bandwidth if 20 MHz cannot meet requirement.

NOTE4: 	10 users per TRxP is the baseline with full buffer traffic. For evaluation of UL user experienced data rate,  the number of users per TRxP can be reduced to 2

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

NOTE6: 	For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

4.2	Dense urban

The dense urban micro cellular deployment scenario focuses on macro TRxPs with or without micro TRxPs and high user densities and traffic loads in city centres and dense urban areas. The key characteristics of this deployment scenario are high traffic loads, outdoor (for example large city squares) and outdoor-to-indoor coverage. This scenario will be interference-limited, using macro TRxPs with or without micro TRxPs. A continuous cellular layout and the associated interference shall be assumed.

Some of its attributes are listed in Table 4.2.

Table 4.2: Attributes for dense urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 4 GHz + Around 7 GHz Around 7 GHz + Around 4 GHz + Around 2 GHz + Around 700 MHz Micro: Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 4 GHz + Around 4 GHzAround 4 GHz + Around 30 GHz Around 7 GHz + Around 7 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth NOTE2, NOTE3 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 200 m Micro: 3 micro TRxPs per macro TRxP All micro TRxPs are all outdoor |
| BS antenna elements NOTE5 | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements NOTE5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements Up to 16 Rx antenna elements (intended only for FWA) Note: Above does not impact future discussions on UE capability |
| User distribution and UE speed NOTE4 | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50] Opt1: 80% indoor (3 km/h); 20% outdoor (30 km/h) Opt2: 40% indoor (3 km/h); 40% outdoor (3 km/h); 20% outdoor (30 km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700 MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990 MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes. Consider larger aggregated system bandwidth if 20 MHz cannot meet requirement.

NOTE4: 	10 users per TRxP is the baseline with full buffer traffic. For evaluation of UL user experienced data rate,  the number of users per TRxP can be reduced to 2

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

NOTE6: 	For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

4.3	Rural

The rural deployment scenario focuses on larger and continuous coverage. The key characteristics of this scenario are continuous wide area coverage supporting stationary and nomadic devices as well as high speed vehicles (cars, trains). This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Some of its attributes are listed in Table 4.3.

Table 4.3: Attributes for rural scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro layer: Around 700 MHz (for ISD 1 or ISD 2) Around 4 GHz (for ISD 1) Around 700 MHz + Around 2 GHz (for ISD 2) Around 7 GHz (ISD 1) Around 700 MHz + Around 7 GHz (for ISD 1) |
| Aggregated system bandwidth NOTE2, NOTE3 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | ISD 1: 1732 m ISD 2: 5000 m |
| BS antenna elements NOTE5 | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements NOTE5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements Up to 16 Rx antenna elements (intended only for FWA) Note: Above does not impact future discussions on UE capability |
| User distribution and UE speed NOTE4 | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50] Opt1: 50% indoor (3 km/h); 50% outdoor (120 km/h). Opt2: 20% indoor (3 km/h); 40% outdoor (60 km/h ); 40% outdoor (120 km/h) |
| Service profile NOTE6 | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700 MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2 GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990 MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes. Consider larger aggregated system bandwidth if 20 MHz cannot meet requirement.

NOTE4: 	10 users per TRxP is the baseline with full buffer traffic. For evaluation of UL user experienced data rate,  the number of users per TRxP can be reduced to 2

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

NOTE6: For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

4.4	Urban macro

The urban macro deployment scenario focuses on large cells providing continuous and ubiquitous coverage; there is no requirement that the continuous coverage is provided on all bands of a multi-band BS. The key characteristics of this scenario are continuous and ubiquitous coverage for stationary and nomadic devices in urban areas. This scenario will be interference-limited, using macro TRxPs (i.e. radio access points above rooftop level).

Some of its attributes are listed in Table 4.4.

Table 4.4: Attributes for urban macro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 2 GHz + Around 4 GHz Around 2 GHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 4 GHz + Around 7 GHz Around 7 GHz + Around 4 GHz + Around 2 GHz + Around 700MHz Micro: Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz Macro + Micro: Around 2 GHz + Around 30 GHz Around 4 GHz + Around 4 GHz Around 4 GHz + Around 30 GHz Around 7 GHz + Around 7 GHz Around 7 GHz + Around 30 GHz |
| Aggregated system bandwidth NOTE2, NOTE3 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop |
| ISD | Macro: 500 m Micro: [3, 6] micro TRxPs per macro TRxP All micro TRxPs are all outdoor |
| BS antenna elements NOTE5 | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements NOTE5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements Up to 16 Rx antenna elements (intended only for FWA) Note: Above does not impact future discussions on UE capability |
| User distribution and UE speed | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50] Opt1: 80% indoor (3 km/h); 20% outdoor (30 km/h) Opt2: 40% indoor (3 km/h); 40% outdoor (3 km/h); 20% outdoor (30 km/h) |
| Service profile NOTE6 | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2 GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990 MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes. Consider larger aggregated system bandwidth if 20 MHz cannot meet requirement.

NOTE4: 	10 users per TRxP is the baseline with full buffer traffic. For evaluation of UL user experienced data rate,  the number of users per TRxP can be reduced to 2

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

NOTE6: 	For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

4.5	Sub-Urban macro

The suburban deployment scenario focuses on providing large cells and continuous coverage; there is no requirement that the continuous coverage is provided on all bands of a multi-band BS. The key characteristics of this scenario are continuous and ubiquitous wide area coverage supporting pedestrian and moderate speed vehicles, energy saving, positioning performance. This scenario will be noise-limited and/or interference-limited, using macro TRxPs.

Therefore this scenario deployment can be considered for 6G as well and the values or assumptions are described as Table 4.5.

Table 4.5: Attributes for Sub-Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro layer: Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 700 MHz + Around 7 GHz Around 4 GHz + Around 7 GHz Around 2 GHz + Around 7 GHz + Around 30 GHz Around 2 GHz+ Around 700 MHz Around 7 GHz +Around 4 GHz + Around 2 GHz + Around 700 MHz |
| Aggregated bandwidth | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 15 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer Hex. Grid |
| ISD | ISD 1: 1299 m ISD 2: 1732 m |
| BS antenna elements NOTE5 | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 15 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements NOTE5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements Up to 16 Rx antenna elements (intended only for FWA) Note: Above does not impact future discussions on UE capability |
| User distribution and UE speed | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50] Opt1: 10% Outdoor pedestrian: 3 km/h ; 10% Outdoor in cars: 40 km/h ; 80% Indoor in houses: 3 km/h Opt2: 20% outdoor in cars: 40 km/h; 80% indoor in houses: 3 km/h |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700 MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2 GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990 MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes. Consider larger aggregated system bandwidth if 20 MHz cannot meet requirement.

NOTE4: 	10 users per TRxP is the baseline with full buffer traffic. For evaluation of UL user experienced data rate,  the number of users per TRxP can be reduced to 2

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

NOTE6: 	For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

4.6	High speed Train

Editor note: airborne will not be captured in section 4.6.

The high speed deployment scenario focuses on continuous coverage along track in high speed trains. The key characteristics of this scenario are consistent passenger user experience and critical train communication reliability with very high mobility. In this deployment scenario, dedicated linear deployment along railway line and the deployments including SFN scenarios captured in Section 6.2 of 3GPP TR 36.878 [4] are considered, and passenger UEs are located in train carriages. The high speed deployment scenario focuses on train scenario will be noise-limited and/or interference-limited, using macro TRxPs.Figure 4.6-1 illustrates a typical layout of high speed deployment scenario. In this deployment scenario, multiple RRHs connect to one BBU with fiber and share the same cell ID. For each RRH, usually there are two TRPs which orient opposite directions along the track, and 8 RRHs are connected to one BBU with fiber and share the same cell ID to form one large macro cell.

Figure 4.6-1. Layout illustration of high speed deployment scenario

Some of its attributes are listed in Table 4.6-1.

Table 4.6-1: High Speed

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Macro only: Around 4 GHz or Around 7 GHz NOTE2 Macro relay nodes: 1) For BS to relay: Around 4 GHz or Around 7 GHz - For relay to UE: Around 30 GHz or Around 7 GHz or Around 4 GHz 2) For BS to relay: Around 30 GHz - For relay to UE: Around 30 GHz or Around 7 GHz or Around 4 GHz |
| Aggregated bandwidth NOTE2, NOTE3 | Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around 30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Macro only: - Around 4 GHz or 7 GHz: Dedicated linear deployment along the railway line as in Figure 1. - RRH site to railway track distance: 100 m Macro + relay nodes: - Around 4 GHz or Around 7 GHz: Dedicated linear deployment along the railway line as following. RRH site to railway track distance: 100 m - Around 30 GHz: Dedicated linear deployment along the railway line as in following. RRH site to railway track distance: 5 m. |
| ISD | - Around 4 GHz or Around 7 GHz: ISD 500 m or 1732 m between RRH sites, two TRxPs per RRH site. See Figure 1. - Around 30GHz: 1732 m between BBU sites, 3 RRH sites connected to 1 BBU, one TRxP per RRH site, inter RRH site distance (580 m, 580 m, 572 m). - Small cell within carriages: ISD = 25 m |
| BS antenna elements NOTE5 | Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements NOTE5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements [Up to 16 Tx and Rx antenna elements (for RX only and intended only for FWA)] Note: Above does not impact future discussions on UE capability Relay Tx: Up to 256 antenna elements Relay Rx: Up to 256 antenna elements |
| User distribution and UE speed | eMBB with high reliability: 100% of users in train For non-full buffer, 300 UEs per macro cell (assuming 1000 passengers per high-speed train and at least 10% activity ratio) Maximum mobility speed: 500 km/h |
| Service profile | Alt 1: Full buffer Alt 2: FTP model 1/2/3 with packet size 0.5 Mbytes, 0.1 Mbytes (other value is not precluded) Other traffic models are not precluded, e.g., for critical train communications. NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700 MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2 GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes. Consider larger aggregated system bandwidth if 20 MHz cannot meet requirement.

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

NOTE6: 	For non-full-buffer traffic with QoS requirement, use XR traffic models in TR 38.838 for XR composite requirement evaluation, use FTP-3 with packet delay budget requirement for energy efficiency evaluation, and use AI/ML traffic models for AI/ML related capability evaluation.

4.7	Extreme long-distance coverage in low density area

The extreme Long Range deployment scenario is defined to allow for the provision of services for very large areas with low density of users whether they are humans and machines (e.g. wilderness, river or water, remote village or island areas, etc). The key characteristics of this scenario are Macro cells with very large area coverage supporting [at least] basic data speeds and voice services, with low to moderate user throughput and low user density.

Table 4.7.2:  Extreme long-distance coverage in low density area

| Attributes |  | Values or assumptions |
| --- | --- | --- |
| Carrier Frequency |  | Around 700 MHz |
| Aggregated Bandwidth |  | 40 MHz (DL+UL) |
| Layout |  | Single layer: Isolated Macro cells |
| Cell range |  | 100 km range (Isolated cell) to be evaluated through link level simulations for communication. Feasibility of Higher Range shall be evaluated through Link level evaluation (for example in some scenarios ranges up to 150-300km may be required). |
| User distribution and UE speed |  | TBD on distribution Speed up to 160 km/h |
| Traffic model |  | TBD |

4.8	Urban coverage for massive connection

Editor note: focusing on MTC

The urban coverage for massive connection scenario focuses on large cells and continuous coverage to provide mMTC. The key characteristics of this scenario are continuous and ubiquitous coverage in urban areas, with very high connection density of mMTC devices. This deployment scenario is for the evaluation of the KPI of connection density.

Some of its attributes are listed in Table 6.1.7-1.

Table 6.1.7-1: Attributes of urban coverage for massive connection

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz Around 2 GHz |
| Network deployment including ISD | Macro only, ISD = 1732 m, 500 m One layers for Around 2 GHz + Around 700 MHz: - Macro layer: Hex. Grid, 2 GHz + 700 MHz (FFS joint or separate BS scheduler for different frequencies) |
| Device deployment | Indoor, and outdoor in-car devices |
| Maximum mobility speed | 20% of users are outdoor in cars (100 km/h) or 20% of users are outdoors (3 km/h) 80% of users are indoor (3 km/h) Users dropped uniformly in entire cell |
| Service profile | Non-full buffer with small packets |
| BS antenna elements NOTE5 | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements |
| UE antenna elements NOTE5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Note: Above does not impact future discussions on UE capability |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700 MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2 GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990 MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

4.9	Air-to-Ground Scenario

The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds.

Some of the characteristics of this deployment scenario are listed in Table 4.9.

Table 4.9: Attributes for commercial Air to Ground Scenario

| Attributes |  | Values or assumptions |
| --- | --- | --- |
| Carrier Frequency | Macro: Around 4 GHz Around 7 GHz |
| Aggregated Bandwidth | Around 4 GHz: Up to 200 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout NOTE1 | Single layer: - Hex. Grid Details TBD |
| Cell range | Macro cell: 100 km range to be evaluated through system level simulations. Feasibility of Higher Range shall be evaluated through Link level evaluation. |
| BS antenna elements | Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements | Antenna mounted at the bottom of the plane TBD on antenna elements |
| User density and UE speed NOTE2 | End user density per Macro: UE speed: Up to [1500] km/h Altitude: Up to 15 km |
| Traffic model | TBD |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	BS to relay link should be the priority for study compared to relay to UE link.

NOTE2:	Evaluate how many users can be served per cell site when the range edge users are serviced with the target user experience data rate.

4.10	Non-Terrestrial Network

Editor note: orbit type and payload type will be captured in section 4.10

NTN deployment scenarios are defined to allow for the provision of services:

- where terrestrial network services are not or poorly available on a permanent (e.g. low-density populated areas and maritime areas) or temporary (e.g. where disasters disrupt terrestrial networks or damage undersea communication cables) basis including in vertical space areas (e.g., up to several kilometers altitude), and

- that can be efficiently supported by Non-Terrestrial Networks.

Accordingly, NTN will mainly contribute to:

- Achieving ubiquitous connectivity and availability of services in remote and sparsely populated areas,

- Improving overall 6G network resiliency by supporting in priority critical communications and emergency communications (messaging, voice calls).

The attributes of the NTN deployment scenarios are listed in Table 4.	10

Table 4.10: Attributes for Non-Terrestrial Network Deployment Scenarios

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 1.5 GHz Around 2 GHz Around 11/14 GHz Around 20/30 GHz |
| Aggregated system Bandwidth | Up to [60] MHz (DL+UL): Around 1.5 GHz and Around 2 GHz Up to [400] MHz (DL+UL): Around 11/14 GHz Up to [800] MHz (DL+UL): Around 20/30 GHz |
| Orbit types NOTE1 | Any orbit from VLEO (300 to 600 km altitude) to GSO (~35786 km altitude), including LEO (600 to 2000 km altitude) and MEO (2000 to ~35000 km altitude), |
| Architecture NOTE2 | Transparent payload: RAN on ground (at feederlink gateway) Regenerative payload: RAN on board |
| Satellite access node’s antenna characteristics | Antenna array of max [2000] radio elements Other types TBD |
| Radio Cell type NOTE3 | Earth fixed (GSO), Quasi Earth fixed (NGSO) or Earth moving (NGSO) |
| Beam hopping parameter | Number of simultaneous active beams: [32/16]; Number of beams per satellite footprint: 1058 |
| UE antenna elements | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements Note: Above does not impact future discussions on UE capability |
| User speed | Up to 1500 km/h for aircraft mounted terminals TBD for other UEs |
| User distribution | Below 7.125 GHz: - Case 1: 100% Outdoor - Case 2: [TBD%] Outdoor and [TBD%] in adverse propagation conditions (e.g. light indoor, in vehicle) Above 10.7 GHz: Outdoor only: [100%] Outdoor |

NOTE1: 	The orbit types can be characterised with a set of parameters (e.g. Maximum Round Trip Delay (ms), Maximum Doppler shift (Hz), Max Differential delay (ms), Max Differential Doppler (Hz), Max Doppler rate (Hz/s), Max delay variation (s-2). The NTN channel model type in TR 38.811 can be considered for performance evaluation.

NOTE2: 	A transparent payload forwards transparently the received 3GPP radio protocol. For regenerative architecture, the feeder link may be based on 3GPP or non-3GPP defined NTN radio access technology.

NOTE3: 	A radio cell is able to serve UEs on the earth surface as well as in the space above. It may be realised with one or several beams generated by satellites. Only part of the beams generated by a given satellite may be active.

4.11	Urban grid

The Urban Grid deployment scenario focuses on scenario of highly densely deployed vehicles and pedestrians placed in urban area. It could cover a scenario where freeways lead through an urban grid.

Some of its attributes are listed in Table 4.11.

Table 4-11: Attributes for Urban grid

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 2 GHz Around 4 GHz Around 7 GHz |
| Aggregated bandwidth NOTE2, NOTE3 | Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid Baseline: Macro only (with the road configuration in Figure 4.11-1 |
| ISD | ISD 1: 500 m ISD 2: 250 m |
| BS height | Macro BS: antenna height 25 m |
| Building size | 413 m x 230 m x 20 m |
| BS antenna elements NOTE5 | Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements NOTE5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements [Up to 16 Rx antenna elements (for RX only and intended only for FWA) Note: Above does not impact future discussions on UE capability] |
| User distribution and UE speed | Urban grid model (car lanes and pedestrian/bicycle sidewalks are placed around a road block. 2 lanes (3.5m) in each direction, 4 lanes in total, 1 sidewalk (3m), one block size: 433m x 250m) 100% outdoor 20% Vehicular UE in the lanes with up to 120km/h 80% Vehicular UE in the lanes with up to 50km/h [Pedestrian UE in the sidewalks with 3km/h UE changes its direction at the intersection as follows: - Go straight with probability 0.5 - Turn left with probability 0.25 - Turn right with probability 0.25] [The distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. - All the vehicles in the same lane have the same speed. - Vehicle type distribution is not dependent of the lane.] |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2 GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes. Consider larger aggregated system bandwidth if 20 MHz cannot meet requirement.

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

Figure 4.11-1: Road configuration for urban grid

4.12	Highway Scenario

The highway deployment scenario focuses on a scenario of vehicles placed in highways with high speeds. The main KPIs evaluated under this scenario would be reliability, availability under high speeds/mobility (and thus frequent handover operations).

Some of its attributes are listed in Table 4.12.

Table 4.12-1: Attributes of Highway

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 7 GHz + Around 4 GHz Around 2 GHz + Around 700 MHz [Around 7GHz + Around 4 GHz + Around 2 GHz + Around 700MHz] |
| Aggregated bandwidth NOTE2, NOTE3 | Around 700 MHz: Up to 60 MHz (DL+UL) Around 2 GHz: Up to 200 MHz (DL+UL) Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid (only around the highway sides) |
| ISD | Macro BS: ISD 1: 500 m - [25 m BS height for ISD 500 m] ISD 2: 1732 m (for around 4 and around 7 GHz) - [35 m BS height for ISD 1732 m] [ISD 3: 5000 m] |
| BS antenna elements NOTE5 | Around 700 MHz: Up to 64 Tx and Rx antenna elements Around 2 GHz: Up to 288 Tx and Rx antenna elements Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements |
| UE antenna elements NOTE5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements [Up to 16 Rx antenna elements (for RX only and intended only for FWA)] Note: Above does not impact future discussions on UE capability |
| User distribution and UE speed | Opt1: 85% outdoor vehicles (120 km/h), 15% outdoor vehicles (200 km/h) Opt2: 100% vehicle, Speed is 140 km/h 100% in vehicles Randomly distributed in the middle of each lane. - Average inter-vehicle distance (between two vehicles’ center) in the same lane is 2sec * average vehicle speed (average speed: 100-200 km/h) - All the vehicles in the same lane have the same speed. Vehicle type distribution is not dependent of the lane. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic with/without QoS requirement depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

[5]	3GPP TR 38.913: “Study on Scenarios and Requirements for Next Generation Access Technologies”

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700 MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2 GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990 MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes. Consider larger aggregated system bandwidth if 20 MHz cannot meet requirement.

NOTE4: 	10 users per TRxP is the baseline with full buffer traffic. For evaluation of UL user experienced data rate,  the number of users per TRxP can be reduced to 2

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

Figure 4.12-1: Road configuration for highway scenario

4.13		Indoor Factory (InF)

The InF scenario focuses on factory halls of varying sizes and with varying levels of density of "clutter", e.g. machinery, assembly lines, storage shelves, etc. The key characteristics of this deployment scenario are clutter type, Clutter density, height of Tx and Rx and LOS/NLOS. The InF scenario is classified further into InF-SL, InF-DL, InF-SH, InF-DH and InF-HH based on a) density of the clutter (S-sparse and D-dense) and b) the height of Tx and Rx (L- low and H-high).

In addition, the indoor factory deployment scenario focuses on small coverage per site/TRxP (transmission and reception point) and high user throughput or user density in buildings. The key characteristics of this deployment scenario are high capacity, high user density and consistent user experience indoor [supporting High-Reliability-Low-Latency communication, positioning and sensing].

Some of its attributes are listed in Table 4.13-1 and 4.13-2.

Table 4.13-1: Attributes for Indoor Factory (InF)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency NOTE1 | Around 4 GHz Around 7 GHz Around 30 GHz |
| Aggregated bandwidth NOTE2, NOTE3 | Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) Around30 GHz: Up to 1 GHz (DL+UL) |
| Layout | Single layer - Indoor floor (Factory) |
| ISD | 50 m (Equivalent to 18 TRxPs per 300 m x 150 m) |
| BS antenna elements Note5 | Around 4 GHz: Up to 576 Tx and Rx antenna elements Around 7 GHz: Up to 2304 Tx and Rx antenna elements Around 30 GHz: Up to 4096 Tx and Rx antenna elements |
| UE antenna elements Note5 | For the purpose of further study in RAN and RAN WGs for frequency up to 7 GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements Note: Above does not impact future discussions on UE capability |
| User distribution and UE speed | - Profile 1 100% Indoor: 3 km/h - Profile 2 (Optional, stationary machine) 100% Indoor: 0 km/h - [Profile 3 (Optional, AGV) 100% Indoor: 30 km/h] [10 users per TRxP] |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |

NOTE1:	The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options. A range of bands from 450 MHz – 960 MHz identified for WRC-15 are currently being considered and around 700 MHz is chosen as a proxy for this range. A range of bands from 1427 – 2690 MHz identified for WRC-15 are currently being considered and around 2GHz is chosen as a proxy for this range. A range of bands from 3300 – 4990 MHz identified for WRC-15 are currently being considered and around 4 GHz is chosen as a proxy for this range. A range of bands from 6425-7125 MHz identified for WRC-23 are currently being considered and around 7 GHz is chosen as a proxy for this range.

NOTE2:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is not allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth.

NOTE3:	Aggregated system bandwidth assumes availability of three FDD carriers each of 10/20 MHz channel bandwidth in different spectrum band in the Around 700 MHz/Around 2 GHz spectrum regions, respectively. Aggregation of the spectrum into one band in each of the bandwidth regions is allowed for simulation purposes. Consider larger aggregated system bandwidth if 20 MHz cannot meet requirement.

NOTE4: 	10 users per TRxP is the baseline with full buffer traffic. For evaluation of UL user experienced data rate,  the number of users per TRxP can be reduced to 2

NOTE5: 	3GPP needs to strive to meet the target with typical antenna configurations. The specific typical antenna configurations may be different for different device types/characteristics and need further study.

Table 4.13-2: Attributes for Indoor Factory (InF)

| Parameters |  | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (high Tx, high Rx) |
| --- | --- | --- | --- | --- | --- | --- |
| Layout | Room size | Rectangular: 45000 m2 i.e. 300 x 150 m |  |  |  |  |
|  | Ceiling height | 5-25 m | 5-15 m | 5-25 m | 5-15 m | 5-25 m |
|  | Effective clutter height hc | < Ceiling height, 0-10 m |  |  |  |  |
|  | External wall and ceiling type | Concrete or metal walls and ceiling with metal-coated windows |  |  |  |  |
| Clutter type |  | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Any |
| Typical clutter size, dclutter |  | 10 m | 2 m | 10 m | 2 m | Any |
| Clutter density r (percentage of surface area occupied by clutter) |  | Low clutter density (<40%) | High clutter density (≥40%) | Low clutter density (<40%) | High clutter density (≥40%) | Any |
| BS antenna height |  | Clutter-embedded, i.e. the BS antenna height is below the average clutter height |  | Above clutter |  | Above clutter |
| UT location | LOS/NLOS | LOS and NLOS |  |  |  | 100% LOS |
|  | Height | Clutter-embedded |  |  |  | Above clutter |
| User distribution and UE speed |  | 100% Indoor, 3 km/h |  |  |  |  |
| Service profile |  | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. |  |  |  |  |
| Applicable KPI |  | UE positioning, Sensing related capability, Reliability |  |  |  |  |

5	Requirements and key technical principles

5.1	Key performance indicators

Editor note: More KPIs can be added

### 5.1.0	General

Editor note:	Aspects related to IMT-2030 TPRs and 3GPP RAN KPIs will be included in this section

Editor note:	The definitions are subject to revision based on further discussion and progress in ITU

Editor note:	Evaluation methodology to be added for each KPI.

This section describes the definitions and requirements of all KPIs.

The KPI targets defined in this section contain KPI targets from Report ITU-R M.[IMT-2030. TECH PERF REQ] [3] and 3GPP specific requirements; both shall be supported by next generation access technologies. The evaluation methodology defined in Report ITU-R M.[IMT-2030.EVAL] [4] shall be used to evaluate the targets specified in Report ITU-R M.[IMT-2030. TECH PERF REQ] [3].

### 5.1.1	Peak data rate

[Peak data rate is the theoretical maximum [achievable] data rate under ideal conditions, which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands, then the total peak data rate is

R=i=1Q  Wi × SEpi

where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is TBD Gbit/s.

–	Uplink peak data rate is TBD Gbit/s.]

### 5.1.2	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is 60 bit/s/Hz.

–	Uplink peak spectral efficiency is 30 bit/s/Hz.

These values were defined assuming an antenna configuration to enable TBD spatial layers (streams) in the downlink and TBD spatial layers (streams) in the uplink.

### 5.1.3	5th percentile user data rate

The 5th percentile user data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

For the evaluation of uplink user experienced data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account.

In case of one frequency band and one layer of transmission reception points (TRxP), the downlink 5th percentile user data rate could be derived from the 5th percentile user spectral efficiency through the following equation. Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the 5th percentile user data rate, Ruser is given by:

Ruser = W × SEuser

For the downlink, in case bandwidth is aggregated across multiple bands (one or more TRxP layers), the 5th percentile user data rate will be summed over the bands.

The targets in Table 5.1.3-1 shall be fulfilled using parameters in accordance with [4].

Table 5.1.3-1: 5th percentile user data rate

| Deployment Scenario | DL 5th percentile user data rate (Mbit/s) | UL 5th percentile user data rate (Mbit/s) |
| --- | --- | --- |
| Dense Urban | 300 | 50 |

### 5.1.4	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (T) denoting the number of correctly received bits of user i, T the time over which the data bits are received and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to the following equation:

The minimum requirements for 5th percentile user spectral efficiency for various deployment scenarios are summarized in Table 5.1.4-1.

Table 5.1.4-1: 5th percentile user spectral efficiency

| Deployment Scenario | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot | 0.9 | 0.63 |
| Dense Urban | 0.675 | 0.45 |
| Rural | 0.36 | 0.135 |

### 5.1.5	Average spectral efficiency

Average spectral efficiency (corresponds to "spectrum efficiency" in Recommendation ITU R M.2160 [2]) is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to the following equation:

The minimum requirements for average spectral efficiency for various deployment scenarios are summarized in Table 5.1.5-1.

Table 5.1.5-1: Average spectral efficiency

| Deployment Scenario | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot | 27 | 20.25 |
| Dense Urban | 23.4 | 16.2 |
| Rural | 9.9 | 4.8 |

### 5.1.6	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation:

Carea = ρ × W × SEavg

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

The target value for area traffic capacity in the downlink is 40 Mbit/s/m² for Indoor Hotspot deployment scenario.

### 5.1.7	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active [or battery efficient (e.g. inactive)] state.

The minimum requirements for user plane latency are:

-	4ms for IC

-	1ms for HRLLC

Assuming unloaded conditions for small IP packets, (e.g. 0 byte payload + IP header), for both downlink and uplink.

### 5.1.8	Control plane latency

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state).

The minimum requirement for control plane latency is 20 ms.

### 5.1.9	Connection density

Connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a given bandwidth and a given number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

The minimum requirement for connection density is 106 devices per km2.

This requirement applies to Urban coverage for massive connection deployment scenario.

### 5.1.10	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

The minimum requirement for the reliability is 1-10-5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Indoor Factory deployment scenario, assuming small application data (e.g. 20 bytes application data + protocol overhead).

### 5.1.11	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

-	Stationary: 0 km/h

-	Pedestrian: 0 km/h to 10 km/h

-	Vehicular: 10 km/h to 120 km/h

-	High speed vehicular: 120 km/h to 500 km/h

-	Airplane: 500 km/h to 1000/1200 km/h (3GPP internal KPI)

Table 5.1.11-1 defines the mobility classes that shall be supported.

Table 5.1.11-1: Mobility classes

|  | Indoor Hotspot | Dense Urban | Rural |
| --- | --- | --- | --- |
| Mobility classes supported | Stationary, Pedestrian | Stationary, Pedestrian, Vehicular | Pedestrian, Vehicular, [High speed vehicular] |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 5.1.11-2. This assumes the user is moving at the maximum speed in that mobility class in each of the deployment scenarios.

Table 5.1.11-2: Traffic channel link data rates normalized by bandwidth

| Deployment Scenario | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot | 2.25 | 10 |
| Dense Urban | 1.68 | 30 |
| Rural | 0.88~1.2 | 120 |
|  | 0.495~0.675 | 500 |

These values were defined assuming an antenna configuration as described in Report ITU-R M.[IMT-2030.EVAL] [4].

### 5.1.12	Mobility interruption time

Mobility interruption time is the shortest time duration supported by the system during which a UE cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the UE and the radio access network.

Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. Mobility interruption time in other scenarios can be reported, where applicable.

### 5.1.13	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the 6GR is defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is at least 400 MHz.

### 5.1.14	Positioning

Positioning is the ability to estimate the position of connected UEs.

The positioning accuracy is the 90% point of the CDF of the device positioning error for [both] horizontal [and vertical] direction[s].

Horizontal positioning accuracy is defined as the positioning accuracy in the horizontal plane (i.e., x/y axis). [Vertical positioning accuracy is defined as the positioning accuracy in the vertical direction (i.e., z-axis, or altitude)].

The minimum requirements for the device positioning accuracy for various deployment scenarios are summarized in Table 5.1.14-1.

Table 5.1.14-1: Positioning accuracy

| Deployment Scenario | Horizontal Accuracy |  | [Vertical Accuracy] |  |
| --- | --- | --- | --- | --- |
| Indoor Factory |  |  |  |  |
| Urban Macro |  |  |  |  |

### 5.1.15	Energy efficiency

Editor note: The text captured is based on ITU-R WP5D and is for ITU-R evaluation; 3GPP internal evaluation metrics will be captured in future meetings.

Energy efficiency is an important metric of sustainability. Energy efficiency of the 6GR and 6G RAN can be characterized by evaluating network and device energy efficiency.

Energy efficiency can be estimated as the relative energy consumption (in terms of percentage) for the selected load case(s) relative to a fully loaded reference case.

6GR shall have the capability to support low relative energy consumption in the selected load cases, considering at least the unloaded (L= 0%) and loaded scenario of [0% < L ≤ 15%] / [15% < L ≤ 30%] / [30% < L ≤ 50%] / [0% < L ≤ 30%], where L is load level, i.e., network resource utilization ratio.

The evaluated energy efficiency values and the power model used for the evaluation can be reported, as well as impacts on communication performance associated with the evaluated energy saving technologies.

Conditions for evaluation are included in Report ITU-R M.[IMT-2030.EVAL].

### 5.1.16	Composite requirement

Editor note: The text captured is based on ITU-R WP5D and is for ITU-R evaluation; 3GPP internal evaluation metrics will be captured in future meetings.

The composite requirement refers to simultaneously satisfying data rate, latency, packet success probability, and the number of users per TRxP.

This requirement is defined as the number of users per TRxP of which at least 90% are satisfied, where each satisfied user achieves the required packet success probability of transmitting a layer 2/3 packet within a required latency. The latency is the time it takes to deliver a data packet for a given data rate from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface.

The minimum performance requirement of composite requirement is summarized in Table 5.1.16-1.

Table 5.1.16-1: Composite requirement

| Deployment Scenario | Data rate (Mbit/s) | Latency (ms) | Packet success probability | Number of users (/TRxP) |
| --- | --- | --- | --- | --- |
| Dense Urban | DL = 30 Mbps/UL=10 Mbps | DL=10 ms / UL=30 ms (or DL+UL total 40 ms) | 99% | 6 |

### 5.1.17	Sensing-related requirements

Sensing-related capabilities are measured in terms of the following technical performance requirements:

•	Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing target when it is present, and false alarm probability is the associated probability that a sensing target is detected when no sensing target is present.

•	Horizontal/Vertical Localization Accuracy: It is defined as the absolute difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing target. The value of localization accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the location estimation error. Both the horizontal and vertical localization accuracy are in Cartesian coordinates.

•	Velocity Accuracy: Velocity accuracy is defined as the absolute difference between the estimated velocity and the actual velocity of the sensing target. The value of velocity accuracy shall be obtained assuming 90% confidence level, which is the 90th percentile point of the cumulative distribution function (CDF) of the velocity estimation error.

In the above technical performance requirements, the sensing target is a passive object, which is not connected to the radio network for communication.

The minimum requirements for Sensing-related requirements are summarized in Table 5.1.17-1.

Table 5.1.17-1: Sensing-related requirements

|  | Indoor Factory | Urban Macro |
| --- | --- | --- |
| Detection Probability |  | [95%] |
| False Alarm Probability |  | [5%] |
| Horizontal Localization Accuracy |  | [5m or 10m] |
| Vertical Localization Accuracy |  |  |
| Velocity Accuracy |  |  |

### 5.1.18	AI-related requirement

### 5.1.19	Resilience and Extended Connectivity

### 5.1.20	Coverage (3GPP internal KPI)

6GR aims to re-use existing 5G mid-band (~3.5 GHz) site grid for 6G deployments in at least around 7 GHz and targeting:

-	Same coverage (as 5G mid-band) for initial access

-	Comparable to same (as 5G mid-band) coverage for data channels with same data rate

6GR aims to provide enhanced overall coverage compared to 5G in the same band, focusing on cell-edge performance and UL coverage.

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The design of the 6G RAN architecture shall enable lower energy consumption with respect to current networks to achieve sustainability.

-	The 6G RAN architecture shall allow non-public networks.

-	The 6G RAN shall support network performance monitoring.

-	The design of the 6G RAN architecture shall allow the collection and transport of standardized measurement data (e.g. AIML data and sensing data) in an efficient way and under operator control to fulfil existing and new service requirements.

-	User data privacy and anonymity is supported and preserved as part of data collection, transmission and operation.

-	User consent as part of outcome of SA.

-	Operator control: operator can control and configure the data to be collected, to have visibility of the collected data, and decide whether to start or stop data collection based on user consent where necessary, while ensuring user privacy and anonymity.

-	Note: The term ‘user consent’ may be further modified depending on SA discussion/outcome.

-	The 6G RAN shall at least support idle mode inter-RAT mobility between the 6GR and E-UTRAN.

-	The 6GR shall support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration.

5.3	Operational requirements

Editor note: Spectrum, co-existence, topology and other operational requirements

5.3.1	Spectrum

Editor note: e.g. spectrum and co-existence related

6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR.

6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

6GR aims to support spectrum allocation scalability, including the fragmented, irregular spectrum allocations.

6GR aims to support UEs with different BW capability in the same spectrum and in the same base station.

6GR shall support the designated frequency ranges between [410MHz] and 52.6GHz for TN

- The supported frequency ranges include existing NR TN operating bands

- The lower bound of the considered frequency ranges can be revisited based on operators’ input

5.3.2	Diverse device types

5.3.3	RAN sharing

6G RAT shall support Network Sharing (i.e., MOCN, MORAN, and Indirect Network Sharing).

Additional description and requirements for RAN sharing are provided in TR 38.760-3 [6].

5.3.4	Ease of operation and Self Organization

Self-organization facilitates the automatic deployment of network nodes, including the establishment of basic operational parameters without manual intervention.

6GR and 6G RAN architecture shall support automatic configuration and optimization of the RAN, including ANR and collection of MDT data.

5.3.5	Network Resilience

The 6G RAN shall be designed to ensure accessibility and service continuity with minimum interruption time in any failure condition.

5.3.6	Service awareness in RAN

6G RAN shall be able to understand and be capable of adapting to traffic demands of applications while maintaining overall system efficiency in terms of radio resource utilization and energy consumption.

5.4	Requirements of New and Existing Services

### 5.4.0	General

This section describes the requirements of new and existing services.

### 5.4.1	Mobile Broadband

The 6GR and 6G RAN architecture shall support mobile broadband services with enhanced performance.

### 5.4.2	Immersive Communication

The 6GR and 6G RAN architecture shall support immersive communications.

The composite requirement in Clause 5.1.16 is used for immersive communication.

Editor note:	FFS on other requirements

### 5.4.3	Massive Communication (IoT)

The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT):

- 6G Massive Communication (IoT) shall be supported for FR1.

  - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB

    - Prioritize 6GR design for eMBB

  - The above usage scenario should not overlap with Ambient IoT and NB-IoT

- PHY minimum peak data rate is 10 Mbps in DL and 10 Mbps in UL for lowest-tier device.

Editor note:  It was agreed in RAN#109 that for 6G Massive Communication (IoT), the minimum number of UE receive/transmit antenna is 1 for lowest-tier device. Reflect this agreement in the device type section of the TR

### 5.4.4	Sensing

Editor note:	More sensing use cases can be included depending on further discussion.

The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

### 5.4.5	AI

6GR and 6G RAN architecture shall support efficient transmission/mechanism for AI applications and services.

### 5.4.6	Voice

The 6GR and 6G RAN architecture shall support Voice in 6G.

Editor note: whether and how to support voice services if they are not provided by 6G is to be decided by SA2. 6GR and 6G RAN architecture requirements will be aligned with that.

### 5.4.7	Regulatory Services

The 6GR and 6G RAN architecture shall provide mechanisms to enable regulatory services such as emergency call, location services for regulatory compliance, Public Warning System (PWS), Lawful Interception (LI), eCall (for automotive), Disaster roaming.

### 5.4.8	Fixed wireless access (FWA)

The 6GR and 6G RAN architecture shall support fixed wireless access (FWA).

### 5.4.9	UAV communications

The 6GR shall provide coverage, high-capacity connectivity and mobility for UAV. The 6GR shall allow for serving UAV with both UAV dedicated and UAV non-dedicated deployments.

### 5.4.10	Vehicular services

The 6GR and 6G RAN architecture shall support services to vehicles over the Uu interface.

5G NR sidelink is reused as is for 6G V2X services.

### 5.4.11	Mission Critical Communication

The 6GR and 6G RAN architecture shall support Mission Critical Communications services (e.g. MCPTT, MCVideo, MCData).

5G broadcast, 5G NR multicast and 5G NR sidelink are reused as is for 6G mission critical communications services.

### 5.4.12	Positioning

5.5	Key technical principles

5.6	Testing and Conformance requirements

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-06 | RAN#108 | RP-251393 |  |  |  | Skeleton for Study on 6G Scenarios and Requirements | 0.0.1 |
| 2025-06 | RAN#108 | RP-251819 |  |  |  | Skeleton for Study on 6G Scenarios and Requirements as agreed by RAN #108 | 0.0.2 |
| 2025-06 | RAN#108 | RP-251873 |  |  |  | Updated TR including pCRs approved at RAN #108 (RP-251872, RP-251880); v0.1.0 was agreed by RAN #108 | 0.1.0 |
| 2025-09 | RAN #109 | RP-252120 |  |  |  | Updated TR as provided by MCC to align with 3GPP drafting rules | 0.1.1 |
| 2025-09 | RAN #109 | RP-252882 |  |  |  | Updated TR including pCRs approved at RAN#109 (RP-252947, RP-252960, RP-252870, RP-252951, RP‑252888); v0.2.0 was agreed by RAN #109 | 0.2.0 |
| 2021-12 | RAN#110 | RP-253750 |  |  |  | Updated TR including pCRs approved at RAN#110 (RP-253873, RP-253869, RP-253870, RP-253754, RP-253756, RP-253757); v0.3.0 was agreed by RAN #110 | 0.3.0 |