| 3GPP TR 38.921 V17.0.0 (2021-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Co-existence study	9

4.1	Co-existence simulation scenarios	9

4.2	Co-existence simulation assumption	9

4.2.1	Network layout model	9

4.2.1.1	Urban macro	9

4.2.1.2	Dense urban	11

4.2.1.3	Indoor	11

4.2.2	Propagation model	12

4.2.2.1	Path loss	12

4.2.2.2	LOS probability	14

4.2.2.3	O-to-I penetration loss	15

4.2.2.3.1	O-to-I building penetration loss	15

4.2.2.3.2	O-to-I car penetration loss	16

4.2.3	Antenna and beam forming pattern modelling	16

4.2.4	Transmission power control model	16

4.2.5	Received power model	16

4.2.6	ACLR and ACS modelling	17

4.2.7	Link level performance for 5G NR coexistence	17

4.2.8	Other simulation parameters	18

4.2.9	Co-existence simulation methodology	18

4.3	Co-existence simulation results	19

4.3.1	Urban Macro scenario	19

4.3.1.1	Downlink	19

4.3.1.2	Uplink	21

4.3.2	Indoor scenario	23

4.3.2.1	Downlink	23

4.3.2.2	Uplink	27

5	General parameters	31

5.1	Duplex mode	31

5.2	Channel Bandwidth	31

5.3	Signal Bandwidth	31

6	BS parameters	31

6.1	Transmitter characteristics	31

6.1.1	Power dynamic range	31

6.1.2	Spectral mask	31

6.1.3	ACLR	32

6.1.4	Spurious emissions	32

6.1.5	Maximum output power	33

6.1.6	Average output power	33

6.2	Receiver characteristics	33

6.2.1	Noise figure	33

6.2.2	Sensitivity	34

6.2.3	Blocking response	34

6.2.4	ACS	35

7	UE parameters	36

7.1	Transmitter characteristics	36

7.1.1	Power dynamic range	36

7.1.2	Spectral mask	36

7.1.3	ACLR	36

7.1.4	Spurious emissions	36

7.1.5	Maximum output power	37

7.1.6	Average output power	37

7.2	Receiver characteristics	37

7.2.1	Noise figure	37

7.2.2.	Sensitivity	37

7.2.3	Blocking response	37

7.2.4	ACS	37

8	Antenna characteristics	38

8.1	BS antenna characteristics	38

8.1.1	 Array antenna model	38

8.1.2	Array antenna parameters	38

8.2	UE antenna characteristics	39

9	Other information relevant for the sharing and compatibility studies	40

9.1	Spatial emission	40

9.2	Interference management	40

9.2.0	General	40

9.2.1	Beam nulling	40

9.2.2	Amplitude weighting/tapering	41

9.2.3	Asymmetric side lobe shaping	41

Annex A (informative):	Change history	43

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for the study item on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz [2], covering the study on transmitter and receiver characteristics for both NR BS and NR UE, and related parameters for answering requests from ITU-R WP5D.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-200513: "Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz"

[3]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[4]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[6]	ERC Recommendation 74-01, "Unwanted emissions in the spurious domain"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

array element: subdivision of a passive antenna array, consisting of a single radiating element or a group of radiating elements, with a fixed radiation pattern

beamwidth: beam which has a half-power contour that is essentially elliptical, the half-power beamwidths in the two pattern cuts that respectively contain the major and minor axis of the ellipse

front-to-back ratio: ratio of maximum directivity of an antenna to its directivity in a specified rearward direction

gain: ratio of the radiation intensity, in a given direction, to the radiation intensity that would be obtained if the power accepted by the antenna were radiated isotropically

NOTE:	If the direction is not specified, the direction of maximum radiation intensity is implied.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

A	Peak normalized array element pattern in dB

AA	Composite antenna array pattern in dBi

AE	Array element pattern in dBi

Am	Front to back ratio in dB

dh	Horizontal element separation in meters

dv	Vertical element separation in meters

GE,max	Array element peak gain in dBi

SLAv	Side lobe suppression in dB

3dB 	Horizonal half power beamwidth

3dB	Vertical half power beamwidth

etilt	Electrical down-tilt angle in degrees (defined from antenna array normal and downwards)

escan	Electrical scan angle in degrees

	Horizontal angle (defined between -180° and 180°).

	Vertical angle of the signal direction (defined between -0° and 180°,90° represents the direction perpendicular to the antenna array

NRB	Transmission bandwidth configuration, expressed in resource blocks

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS	Active Antenna System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AWGN	Additive White Gaussian Noise

BS	Base Station

BW	Bandwidth

FDD	Frequency Division Duplex

FR	Frequency Range

FRC	Fixed Reference Channel

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

LOS	Line-Of-Sight

MR	Medium Range

NR	New Radio

OTA	Over The Air

O-to-I	Outdoor-to-Indoor

RB	Resource Block

RF	Radio Frequency

SCS	Sub-Carrier Spacing

TDD	Time division Duplex

WA	Wide Area

# 4	Co-existence study

## 4.1	Co-existence simulation scenarios

Table 4.1-1 summarizes the proposed scenarios to be considered for 6.425-7.125GHz and 10.0-10.5 GHz.

Table 4.1-1: Summary of considered scenario

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 7 GHz | Urban macro |  |
| 2 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 7 GHz | Indoor hotspot |  |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 7 GHz | Dense urban | Down-prioritized |
| 4 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Urban macro |  |
| 5 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Indoor hotspot |  |
| 6 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 7 GHz | Dense urban | Down-prioritized |
| 7 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 10 GHz | Urban macro |  |
| 8 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 10 GHz | Indoor hotspot |  |
| 9 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 10 GHz | Dense urban | Down-prioritized |
| 10 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 10 GHz | Urban macro |  |
| 11 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 10 GHz | Indoor hotspot |  |
| 12 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 10 GHz | Dense urban | Down-prioritized |

## 4.2	Co-existence simulation assumption

### 4.2.1	Network layout model

#### 4.2.1.1	Urban macro

Details on urban macro network layout model are listed in Tables 4.2.1.1-1 and 4.2.1.1-2.

Table 4.2.1.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.45 km (urban) (Note 1) 0.9 km (suburban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 20 m (urban) 25 m (suburban) |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Note: Results with 0.4 km ISD at 10 GHz can also be provided if simulation results show 5%-tile throughput cannot be achieved with 0.45 km ISD at 10 GHz. |  |  |  |

Table 4.2.1.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) and un-coordinated operation (100% Grid Shift) | RAN4 has long been using un-coordinated operation in below 6 GHz coexistence simulation |

Figure 4.2.1.1-1: Coordinated operation: each network with co-location of sites

Figure 4.2.1.1-2: Uncoordinated operation: second network’s sites are located at the first network’s cell edge

#### 4.2.1.2	Dense urban

It is agreed to down-prioritized the dense urban scenario in this coexistence study, because it has the least demanding ACIR requirements among the three simulated scenarios in TR 38.803.

#### 4.2.1.3	Indoor

Details on indoor network layout model are listed in Tables 4.2.1.3-1 and 4.2.1.3-2.

Table 4.2.1.3-1: Single operator layout for indoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | 50 m x 120 m, 12 BSs |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 3 m | ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office |  |
| Shadowing correlation |  | NA |  |

Figure 4.2.1.3-1: Network layout for indoor

Table 4.2.1.3-2: Multi operators layout for indoor

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Coordinated operation (0% Grid Shift) |  |

### 4.2.2	Propagation model

#### 4.2.2.1	Path loss

The pathloss models are summarized in Table 4.2.2.1-1 and the distance definitions are indicated in Figures 4.2.2.1-1 and 4.2.2.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 4.2.2.1-1.

Figure 4.2.2.1-1: Definition of d2D and d3D for outdoor UTs

Figure 4.2.2.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs

Note that

d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2	(4.2.2-1)

Table 4.2.2.1-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 4 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| UMa | LOS | PLUMa-LOS=PL110m≤d2D≤dBP'PL2dBP'≤d2D≤5km, see note 1 PL1=28.0+22log10(d3D)+20log10(fc) PL2=28.0+40log10(d3D)+20log10(fc) -9log10((dBP')2+(hBS-hUT)2) | σSF=4 | 1.5m≤hUT≤22.5m hBS=25m |
|  | NLOS | PLUMa-NLOS=max(PLUMa-LOS,PLUMa-NLOS') for 10m≤d2D≤5km | σSF=6 | 1.5m≤hUT≤22.5m hBS=25m Explanations: see note 3 |
|  |  | Optional PL=32.4+20log10fc+30log10d3D | σSF=7.8 |  |
| InH - Office | LOS | PLInH-LOS=32.4+17.3log10(d3D)+20log10(fc) | σSF=3 | 1m≤d3D≤150m |
|  | NLOS | PLInH-NLOS=max(PLInH-LOS,PLInH-NLOS') PLInH-NLOS'=38.3log10d3D+17.30+24.9log10fc | σSF=8.03 | 1m≤d3D≤150m |
|  |  | Optional PLInH-NLOS'=32.4+20log10fc+31.9log10d3D | σSF=8.29 | 1m≤d3D≤150m |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by Cd2D,hUT=0,hUT<13mhUT-13101.5gd2D,13m≤hUT≤23m, where gd2D=0,d2D≤18m54d2D1003exp-d2D150,18m<d2D. Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

#### 4.2.2.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 4.2.2.2-1.

Table 4.2.2.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMa | PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out631-18d2D-out1+C'hUT54d2D-out1003exp-d2D-out150,18m<d2D-outwhere C'(hUT)=0,hUT≤13mhUT-13101.5,13m<hUT≤23m |
| Indoor - Mixed office | PrLOS=1,d2D-in≤1.2mexp-d2D-in-1.24.7,1.2m<d2D-in<6.5mexp-d2D-in-6.532.6⋅0.32,6.5m≤d2D-in |
| Indoor - Open office | PrLOS=1,d2D-in≤5mexp-d2D-in-570.8,5m<d2D-in≤49mexp-d2D-in-49211.7⋅0.54,49m<d2D-in |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

#### 4.2.2.3	O-to-I penetration loss

##### 4.2.2.3.1	O-to-I building penetration loss

The pathloss incorporating O2I building penetration loss is modelled as in the following:

PL=PLb+PLtw+PLin+N0,σP2	(4.2.2-2)

where PLb is the basic outdoor path loss given in clause 4.2.2.1, where d3D is replaced by d3D-out+d3D-in . PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP is the standard deviation for the penetration loss.

PLtw is characterized as:

PLtw=PLnpi-10log10i=1Npi×10Lmaterial_i-10	(4.2.2-3)

PLnpi is an additional loss is added to the external wall loss to account for non-perpendicular incidence;  Lmaterial_i=amaterial_i+bmaterial_i⋅f, is the penetration loss of material i, example values of which can be found in Table 4.2.2.3-1; pi is proportion of i-th materials, where  i=1Npi=1; and N is the number of materials.

Table 4.2.2.3-1: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2f |
| IRR glass | LIIRglass=23+0.3f |
| Concrete | Lconcrete=5+4f |
| Wood | Lwood=4.85+0.12f |
| Note: f is in GHz. |  |

Table 4.2.2.3-2 gives PLtw, PLin and σP for two O2I penetration loss models. The O2I penetration is UT-specifically generated and is added to the SF realization in the log domain.

Table 4.2.2.3-2: O2I building penetration loss model

|  | Path loss through external wall: PLtw in [dB] | Indoor loss: PLin in [dB] | Standard deviation: σP in [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5 d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIIRglass10+0.3⋅10-Lconcrete10 | 0.5 d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for UMa and UMi-Street Canyon, and between 0 and 10 m for RMa. d2D-in should be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

Only the high-loss model is applicable to InF.

##### 4.2.2.3.2	O-to-I car penetration loss

The pathloss incorporating O2I car penetration loss is modelled as in the following:

PL=PLb+Νμ,σP2	(4.2.2-4)

where PLb is the basic outdoor path loss given in clause 4.2.2.1. μ = 9, and σP = 5. The car penetration loss should be UT-specifically generated. Optionally, for metallized car windows, μ = 20 can be used. The O2I car penetration loss models are applicable for at least 0.6 - 60 GHz.

### 4.2.3	Antenna and beam forming pattern modelling

The BS antenna is modelled as described in clause 8.1.1 using parameters for different BS deployments listed in clause 8.1.2.

The UE antenna is modelled as described in clause 8.2 using isotropic antenna pattern.

### 4.2.4	Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in clause 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = 88 + 10*log10(200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1

### 4.2.5	Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

### 4.2.6	ACLR and ACS modelling

For DL it seems reasonable from the perspective of simulating worst case scenarios that we assume BS ACLR is modelled as flat in space, and the UE ACS can be modelled flat in space.

If this assumption is for DL, then the similar assumption could be made for the UL because:

-	UE has a much small number of antennas, thus the effect of directivity should be smaller for ACLR (or the adjacent channel interference). It can also be reasonably assumed that the UE ACLR will play a dominant role than the BS ACS in the adjacent channel interference.

-	Again, BS ACS flat in space might mean worse coexistence performance than actual performance because BS has better capability of steering its receive antennas to suppress interference.

1 user scheduling is baseline assumption for coexistence evaluation and the two step ACLR model shown in Table 4.2.6-1 could be used for 3 uplink user scheduling simulation, where a UE occupies a smaller bandwidth than the channel bandwidth for transmission, to avoid overly estimating interference, similar as done in E-UTRA coexistence study (as recorded in TR 36.942).

Table 4.2.6-1: Uplink ACIR value

| Frequency offset between aggressor (91RBs) and victim (91RBs) | ACIR value |
| --- | --- |
| 0 - 90 RBs | 30 + X |
| 91 - 181RBs | 43 + X |
| > 181RBs | 43+ X |

Therefore, it is assumed that both ACLR (or the adjacent channel interference) and ACS are flat in both space and frequency. The ACIR model can be express as:

(assuming ACLR, ACS and ACIR to be linear).

### 4.2.7	Link level performance for 5G NR coexistence

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR< SNIRMIN                                        ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

-	S(SNIR)		Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz

-					Attenuation factor, representing implementation losses

-	SNIRMIN		Minimum SNIR of the code set, dB

-	SNIRMAX		Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in Table 4.2.7-1 represent a baseline case, which assumes:

-	1:1 antenna configuration

-	AWGN channel model

-	Link Adaptation (see Table 4.2.7-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 4.2.7-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Note that the parameters proposed in Table 4.2.7-1 are targeted for eMBB coexistence scenario.

### 4.2.8	Other simulation parameters

Table 4.2.8-1: Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Carrier frequency | 7 GHz, 10 GHz | 7 GHz, 10 GHz | Down-prioritized |
| Channel bandwidth | 100 MHz | 100 MHz | Down-prioritized |
| Scheduled channel bandwidth per UE (DL) | 98.28 MHz | 98.28 MHz | Down-prioritized |
| Scheduled channel bandwidth per UE (UL) | 98.28 MHz (1 user, Note 4) 32.76 MHz (3 user) | 98.28 MHz (1 user, Note 4) 32.76 MHz (3 user) | Down-prioritized |
| The number of active UE (DL) (Note 1) | 1 | 1 | Down-prioritized |
| The number of active UE (UL) (Note 1) | 1 or 3 | 1 or 3 | Down-prioritized |
| Traffic model | Full buffer | Full buffer | Down-prioritized |
| DL power control | NO | NO | Down-prioritized |
| UL power control | YES | YES | Down-prioritized |
| BS max TX power in dBm | 24 | 43 (Note 3) | Down-prioritized |
| UE max TX power in dBm | 23 or 20 (Note 2) | 23 or 20 (Note 2) | Down-prioritized |
| UE min TX power in dBm | -33 | -33 | Down-prioritized |
| BS Noise figure in dB | 14 (@7 GHz) 15 (@10 GHz) | 6 (@7 GHz) 7 (@10 GHz) | Down-prioritized |
| UE Noise figure in dB | 9 or 13 (Note 5) | 9 or 13 (Note 5) | Down-prioritized |
| Handover margin | 3 dB | 3 dB | Down-prioritized |
| Note 1 Same as the number of BS beam(s); Note 2: 20 dBm as optional case where CLx-ile should be reduced by 3 dB; Note 3: BS max TX power is defined per polarization; Note 4: 1 user scheduling is a baseline assumption for coexistence evaluation; Note 5: NF of 9 dB is a baseline assumption and NF of 13 dB is as optional case. |  |  |  |

### 4.2.9	Co-existence simulation methodology

Adopt following simulation steps.

1. Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2. UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at BS and UE with isotropic antenna.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

## 4.3	Co-existence simulation results

### 4.3.1	Urban Macro scenario

#### 4.3.1.1	Downlink

For Urban macro scenarios, simulation results for DL throughput loss with baseline assumption ACLR = 45 dBc at BS and ACS = 33 dBc at UE are summarized in Table 4.3.1.1-1/2/3/4.

Table 4.3.1.1-1: DL throughput loss of victim UE for 6425 – 7125 MHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | AAS based BS | Average throughput loss in % (7 GHz) | 1.65 | 1.89 | 2.10 | 2.33 | 2.63 | 2.99 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 4.39 | 5.18 | 5.98 | 7.07 | 7.87 | 9.06 |
| Nokia | AAS based BS | Average throughput loss in % (7 GHz) | 1.11 | 1.28 | 1.48 | 1.70 | 1.94 | 2.21 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 6.22 | 6.23 | 6.24 | 7.51 | 8.99 | 9.23 |
| Huawei | AAS based BS | Average throughput loss in % (7 GHz) | 1.04 | 1.39 | 1.47 | 1.77 | 2.30 | 2.39 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 3.00 | 4.63 | 4.96 | 6.37 | 6.78 | 7.16 |

Table 4.3.1.1-1a: DL throughput loss of victim UE for 6425 – 7125 MHz [Absolute ACIR value]

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average | 2.7 | 2.4 | 2.1 | 1.8 | 1.6 | 1.4 | 1.2 | 1.0 | 0.9 | 0.8 | 0.7 |
|  | Throughput lost at 5%-tile | 13.1 | 10.6 | 10.0 | 8.3 | 6.6 | 6.1 | 5.5 | 4.7 | 3.1 | 2.2 | 2.1 |
| CATT | Throughput lost at Average | 2.945446 | 2.615785 | 2.318919 | 2.050108 | 1.808206 | 1.592518 | 1.399160 | 1.226961 | 1.073971 | 0.936971 | 0.814444 |
|  | Throughput lost at 5%-tile | 11.345302 | 10.038643 | 8.745891 | 7.593149 | 6.594121 | 5.813514 | 5.023313 | 4.235969 | 3.589495 | 3.063706 | 2.576903 |
| Qualcomm | Throughput lost at Average |  |  | 3.0285 | 2.6700 | 2.3630 | 2.0876 | 1.8414 | 1.6318 | 1.4200 |  |  |
|  | Throughput lost at 5%-tile |  |  | 12.3347 | 10.6516 | 9.2041 | 7.9086 | 6.7639 | 5.8122 | 4.9061 |  |  |

Table 4.3.1.1-2: DL throughput loss of victim UE for 10 - 10.5 GHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | AAS based BS | Average throughput loss in % (7 GHz) | 1.52 | 1.68 | 1.93 | 2.12 | 2.31 | 2.62 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 3.36 | 3.79 | 4.79 | 5.78 | 6.41 | 7.63 |
| Nokia | AAS based BS | Average throughput loss in % (7 GHz) | 1.03 | 1.20 | 1.38 | 1.60 | 1.83 | 2.09 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 4.38 | 4.38 | 4.86% | 5.23 | 6.01 | 7.40 |
| Huawei | AAS based BS | Average throughput loss in % (7 GHz) | 1.21 | 1.22 | 1.30% | 1.72 | 1.99 | 2.44 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 3.04 | 3.33 | 4.14% | 5.06 | 6.41 | 8.67 |

Table 4.3.1.1-2a: DL throughput loss of victim UE for 10-10.5GHz [Absolute ACIR value]

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average | 2.1 | 1.9 | 1.7 | 1.4 | 1.3 | 1.1 | 0.9 | 0.8 | 0.7 | 0.6 | 0.5 |
|  | Throughput lost at 5%-tile | 11.8 | 10.0 | 9.6 | 9.4 | 7.8 | 6.3 | 4.8 | 4.7 | 4.2 | 3.9 | 3.5 |
| CATT | Throughput lost at Average | 2.874567 | 2.547846 | 2.252212 | 1.986030 | 1.748135 | 1.537007 | 1.348903 | 1.181463 | 1.032390 | 0.899829 | 0.782843 |
|  | Throughput lost at 5%-tile | 9.975828 | 8.600968 | 7.468747 | 6.454207 | 5.566782 | 4.750499 | 4.068893 | 3.494199 | 2.988947 | 2.552896 | 2.076688 |
| Qualcomm | Throughput lost at Average |  |  | 2.788 | 2.4497 | 2.1611 | 1.9031 | 1.6734 | 1.4787 | 1.2825 |  |  |
|  | Throughput lost at 5%-tile |  |  | 11.0936 | 9.6118 | 8.3690 | 7.2691 | 6.2895 | 5.4504 | 4.5726 |  |  |

In addition, coexistence simulation results for UE with NF 13 dB is summarized in Table 4.3.1.1.3 and 4.3.1.1.4.

Table 4.3.1.1-3: 7 GHz ACIR with 13 dB NF [Qualcomm, R4-2016236]

| ACIR [dB] | 31 | 30 | 29 | 28 | 27 | 26 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7 GHz) | 1.3706% | 1.5769% | 1.7811% | 2.0213% | 2.2902% | 2.5902% | 2.9408% |
| 5%-tile throughput loss (7 GHz) | 4.5693% | 5.4639% | 6.3517% | 7.4031% | 8.5919% | 9.9359% | 11.5323% |

Table 4.3.1.1-4: 10 GHz ACIR with 13 dB NF [Qualcomm, R4-2016236]

| ACIR [dB] | 31 | 30 | 29 | 28 | 27 | 26 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (10 GHz) | 1.2320% | 1.4223% | 1.6112% | 1.8344% | 2.0853% | 2.3663% | 2.6961% |
| 5%-tile throughput loss (10 GHz) | 3.2501% | 3.9646% | 4.7371% | 5.6903% | 6.7960% | 8.0620% | 9.5717% |

Based on the above simulation results, the required DL ACIR for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for urban macro scenarios are in summarized in Table 4.3.1.1-5.

Table 4.3.1.1-5: DL simulation results for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz

|  | ACIR (dB) |  |
| --- | --- | --- |
| Company | 6.425 - 7.125 GHz | 10.0 - 10.5 GHz |
|  | Urban macro uncoord. | Urban Macro uncoord. |
| CATT | 29.5 | 28 |
| Nokia | 32.7 | 30.7 |
| Huawei | 30.7 | 29.7 |
| Ericsson | 30 | 29 |
| ZTE | 31.7 | 29.7 |
| Qualcomm | 30.9 | 30.5 |
| Average | 30.9 | 29.6 |
| Average after removing highest and lowest values | 30.9 | 29.5 |

#### 4.3.1.2	Uplink

For Urban macro scenarios, simulation results for UL throughput loss with baseline assumption ACLR = 30 dBc at UE and ACS = 46 dBc at BS are summarized in Table 4.3.1.2-1/2/3/4.

Table 4.3.1.2-1: UL throughput loss of victim BS for 6425-7125MHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | AAS based BS | Average throughput loss in % (7 GHz) | 0.99 | 1.09 | 1.23 | 1.40 | 1.55 | 1.76 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 2.72 | 2.84 | 5.28 | 6.01 | 6.71 | 7.20 |
| Nokia | AAS based BS | Average throughput loss in % (7 GHz) | 1.08 | 1.24 | 1.43 | 1.65 | 1.89 | 2.15 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 1.01 | 1.68 | 3.14 | 3.98 | 5.76 | 8.08 |
| Huawei | AAS based BS | Average throughput loss in % (7 GHz) | 0.64 | 0.69 | 0.69 | 0.92 | 1.01 | 1.07 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 3.8 | 4.3 | 5.0 | 6.2 | 7.6 | 8.7 |

Table 4.3.1.2-1a: UL throughput loss of victim BS for 6425 – 7125 MHz [Absolute ACIR value]

|  | ACIR (dB) | 23 | 24 | 25 |  | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average | 1.5 | 1.3 | 1.1 |  | 1.0 | 0.8 | 0.7 | 0.6 | 0.5 | 0.5 | 0.4 | 0.3 |
|  | Throughput lost at 5%-tile | 6.3 | 6.2 | 5.5 |  | 4.3 | 2.3 | 1.6 | 1.5 | 1.4 | 0.8 | 0.6 | 0.5 |
| CATT | Throughput lost at Average | 1.489821 | 1.301164 | 1.134213 |  | 0.986689 | 0.856389 | 0.741773 | 0.641199 | 0.553419 | 0.476770 | 0.409968 | 0.351849 |
|  | Throughput lost at 5%-tile | 7.283069 | 6.142583 | 5.227730 |  | 4.495608 | 3.745409 | 3.051767 | 2.429319 | 1.621495 | 0.968716 | 0.671695 | 0.526234 |
| Qualcomm | Throughput lost at Average | 1.5 | 1.2 | 1.0 |  | 0.9 | 1.0 | 0.9 | 0.8 | 0.6 | 0.5 |  |  |
|  | Throughput lost at 5%-tile | 4.3 | 3.4 | 2.4 |  | 2.2 | 2.4 | 2.2 | 1.8 | 1.5 | 1.3 |  |  |

Table 4.3.1.2-2: UL throughput loss of victim BS for 10 - 10.5 GHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | AAS based BS | Average throughput loss in % (7 GHz) | 0.79 | 0.87 | 1.11 | 1.23 | 1.41 | 1.59 |
|  |  | 5%-tile throughput loss in % (7 GHz) | N/A | N/A | N/A | N/A | N/A | N/A |
| Nokia | AAS based BS | Average throughput loss in % (7 GHz) | 0.90 | 1.04 | 1.21 | 1.39 | 1.60 | 1.84 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 1.98 | 2.27 | 2.28 | 3.90 | 4.00 | 6.41 |
| Huawei | AAS based BS | Average throughput loss in % (7 GHz) | 0.53 | 0.59 | 0.61 | 0.76 | 0.91 | 1.05 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 1.7 | 2.3 | 3.3 | 4.0 | 5.3 | 6.4 |

Table 4.3.1.2-2a: UL throughput loss of victim BS for 10 - 10.5 GHz [Absolute ACIR value]

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average | 1.3 | 1.1 | 1.0 | 0.8 | 0.7 | 0.6 | 0.5 | 0.5 | 0.4 | 0.3 | 0.3 |
|  | Throughput lost at 5%-tile | 3.0 | 2.3 | 1.3 | 1.0 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.2 |
| CATT | Throughput lost at Average | 1.535229 | 1.345764 | 1.177367 | 1.027196 | 0.894277 | 0.777219 | 0.673935 | 0.582684 | 0.502769 | 0.432740 | 0.371237 |
|  | Throughput lost at 5%-tile | 9.496269 | 7.816610 | 6.444894 | 5.273296 | 4.180206 | 3.433665 | 3.082907 | 2.691289 | 2.325946 | 2.024768 | 1.625941 |
| Qualcomm | Throughput lost at Average | 1.5 | 1.2 | 1.0 | 0.9 | 0.8 | 0.6 | 0.5 | 0.4 |  |  |  |
|  | Throughput lost at 5%-tile | 4.0 | 3.5 | 2.9 | 2.6 | 2.2 | 2.0 | 1.6 | 1.4 |  |  |  |

In addition, coexistence simulation results for UE with Tx power 20 dBm is summarized in Table 4.3.1.2.3 and 4.3.1.2.4.

Table 4.3.1.2-3: ACIR results at 7 GHz with 20 dBm UE max Tx power [Qualcomm, R4-2016601]

| ACIR [dB] | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7 GHz) | 0.4% | 0.5% | 0.6% | 0.8% | 0.9% | 1.0% | 1.2% | 1.5% | 1.8% |
| 5%-tile throughput loss (7 GHz) | 0.8% | 1.2% | 1.6% | 1.8% | 2.2% | 2.5% | 3.4% | 4.2% | 5.0% |

Table 4.3.1.2-4: ACIR results at 10 GHz with 20 dBm UE max Tx power [Qualcomm, R4-2016601]

| ACIR [dB] | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (10 GHz) | 0.3% | 0.4% | 0.5% | 0.6% | 0.7% | 0.8% | 1.0% | 1.2% | 1.4% | 1.6% |
| 5%-tile throughput loss (10 GHz) | 0.8% | 1.2% | 1.4% | 1.8% | 2.1% | 2.3% | 3.2% | 4.0% | 4.7% | 5.6% |

Based on the above simulation results, the required UL ACIR for 6.425-7.125GHz and 10.0-10.5 GHz for urban macro scenarios are summarized in Table 4.3.1.2-5.

Table 4.3.1.2-5: UL simulation results for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz

|  | ACIR (dB) |  |
| --- | --- | --- |
| Company | 6.425 - 7.125 GHz | 10.0 - 10.5 GHz |
|  | Urban macro uncoord. | Urban Macro uncoord. |
| CATT | 26 | 26.2 |
| Nokia | 26.9 | 25.9 |
| Huawei | 27.9 | 25.9 |
| Ericsson | 27 | 23 |
| ZTE | 27.9 | 24.9 |
| Qualcomm | 22 (Note 1) | 22 / 21.5(Note 1, 2) |
| Average value | 26.3 | 24.6 |
| Average after removing highest and lowest values | 27 | 24.9 |
| Note 1: 3 UL UEs are assumed, where the same antenna gain derived by the BS beam forming pattern modelling described in clause 8.1.1 is used for all UEs. Note 2: 22 with 23 dBm UE maximum Tx power and 21.5 with 20 dBm UE maximum Tx power. |  |  |

### 4.3.2	Indoor scenario

#### 4.3.2.1	Downlink

For indoor scenarios, simulation results for DL throughput loss with baseline assumption ACLR = 45 dBc at BS and ACS = 33 dBc at UE are summarized in Table 4.3.2.1-1/1a/2/2a/3/3a/4/4a.

Table 4.3.2.1-1: DL throughput loss of victim UE for 6425 – 7125 MHz with omni antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | Omni based BS | Average throughput loss in % (7 GHz) | 0.1371 | 0.1724 | 0.2168 | 0.2726 | 0.3425 | 0.4302 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.1429 | 0.1798 | 0.2263 | 0.2847 | 0.3123 | 0.3427 |
| Nokia | Omni based BS | Average throughput loss in % (7 GHz) | 0.13 | 0.17 | 0.21 | 0.26 | 0.33 | 0.41 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.07 | 0.09 | 0.11 | 0.14 | 0.20 | 0.33 |

Table 4.3.2.1-1a: DL throughput loss of victim UE for 6425 – 7125 MHz with omni antenna [Absolute ACIR value]

|  | ACIR (dB) | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average |  |  |  |  |  | 1.8 | 2.2 | 2.8 | 3.5 | 4.3 | 5.4 |
|  | Throughput lost at 5%-tile |  |  |  |  |  | 0.9 | 1.1 | 1.4 | 1.8 | 2.3 | 2.8 |
| CATT | Throughput lost at Average |  |  |  |  |  |  | 3.198061 | 3.945391 | 4.852136 | 5.946106 | 7.258307 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 1.970774 | 2.470131 | 3.106864 | 3.901198 | 4.918388 |
| Qualcomm | Throughput lost at Average | 0.59 | 0.76 | 0.9 | 1.1 | 1.4 | 1.8 | 2.3 | 2.9 | 3.6 | 4.4 | 5.4 |
|  | Throughput lost at 5%-tile | 0.31 | 0.39 | 0.44 | 0.53 | 0.66 | 0.8 | 1.1 | 1.4 | 1.9 | 2.5 | 3.1 |

Table 4.3.2.1-2: DL throughput loss of victim UE for 6425 – 7125 MHz with AAS antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | AAS based BS | Average throughput loss in % (7 GHz) | 0.3043 | 0.3817 | 0.4775 | 0.5947 | 0.7378 | 0.9123 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.0054 | 0.0067 | 0.0085 | 0.0107 | 0.0135 | 0.1421 |
| Huawei | AAS based BS | Average throughput loss in % (7 GHz) | 0.15 |  | 0.30 |  | 0.36 |  |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.02 |  | 0.06 |  | 0.07 |  |
| Nokia | AAS based BS | Average throughput loss in % (7 GHz) | 0.27 | 0.33 | 0.42 | 0.52 | 0.65 | 0.81 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.05 | 0.06 | 0.08 | 0.10 | 0.12 | 0.16 |

Table 4.3.2.1-2a: DL throughput loss of victim UE for 6425 – 7125 MHz with AAS antenna [Absolute ACIR value]

|  | ACIR (dB) | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average |  |  |  |  |  |  |  |  |  |  |  |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  |  |  |  |  |  |
| CATT | Throughput lost at Average |  |  |  |  |  |  | 4.311783 | 5.036284 | 5.868329 | 6.803309 | 7.846610 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 0.988785 | 1.284747 | 1.673493 | 2.221040 | 2.921901 |
| Qualcomm | Throughput lost at Average | 1.27 | 1.57 | 1.88 | 2.27 | 2.72 | 3.24 | 3.87 | 4.52 | 5.29 | 6.16 |  |
|  | Throughput lost at 5%-tile | 0.17 | 0.26 | 0.34 | 0.46 | 0.60 | 0.77 | 1 | 1.2 | 1.6 | 1.9 |  |

Table 4.3.2.1-3: DL throughput loss of victim UE for 10 - 10.5 GHz with omni antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | Omni based BS | Average throughput loss in % (7 GHz) | 0.1561 | 0.1956 | 0.2447 | 0.3058 | 0.3815 | 0.4750 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.1096 | 0.1380 | 0.1736 | 0.2369 | 0.3323 | 0.4823 |
| Nokia | Omni based BS | Average throughput loss in % (7 GHz) | 0.13 | 0.17 | 0.21 | 0.26 | 0.33 | 0.41 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.09 | 0.11 | 0.13 | 0.16 | 0.19 | 0.23 |

Table 4.3.2.1-3a: DL throughput loss of victim UE for 10 - 10.5 GHz with omni antenna [Absolute ACIR value]

|  | ACIR (dB) | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average |  |  |  |  |  | 1.8 | 2.2 | 2.8 | 3.5 | 4.3 | 5.4 |
|  | Throughput lost at 5%-tile |  |  |  |  |  | 0.9 | 1.1 | 1.4 | 1.8 | 2.3 | 2.8 |
| CATT | Throughput lost at Average |  |  |  |  |  |  | 3.122018 | 3.855086 | 4.745063 | 5.820303 | 7.109105 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 1.938403 | 2.462824 | 3.109281 | 3.943454 | 4.927987 |
| Qualcomm | Throughput lost at Average | 0.59 | 0.76 | 0.94 | 1.18 | 1.4 | 1.85 | 2.3 | 2.9 | 3.6 | 4.46 | 5.4 |
|  | Throughput lost at 5%-tile | 0.5 | 0.66 | 0.7 | 0.83 | 0.9 | 1.18 | 1.4 | 1.9 | 2.4 | 3.0 | 3.72 |

Table 4.3.2.1-4a: DL throughput loss of victim UE for 10 - 10.5 GHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | AAS based BS | Average throughput loss in % (7 GHz) | 0.3188 | 0.3979 | 0.4950 | 0.6133 | 0.7587 | 0.9337 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.0481 | 0.0553 | 0.0645 | 0.0760 | 0.0969 | 0.1978 |
| Huawei | AAS based BS | Average throughput loss in % (7 GHz) | 0.13 |  | 0.21 |  | 0.32 |  |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.01 |  | 0.07 |  | 0.09 |  |
| Nokia | AAS based BS | Average throughput loss in % (7 GHz) | 0.27 | 0.34 | 0.43 | 0.54 | 0.67 | 0.82 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.00 | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 |

Table 4.3.2.1-4b: DL throughput loss of victim UE for 6425 – 7125 MHz with AAS antenna [Absolute ACIR value]

|  | ACIR (dB) | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average |  |  |  |  |  |  |  |  |  |  |  |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  |  |  |  |  |  |
| CATT | Throughput lost at Average |  |  |  |  |  |  | 4.235205 | 4.964440 | 5.793027 | 6.735404 | 7.788678 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 1.414406 | 1.703417 | 2.085369 | 2.595632 | 3.227734 |
| Qualcomm | Throughput lost at Average | 1.2 | 1.5 | 1.8 | 2.2 | 2.6 | 3.1 | 3.8 | 4.4 | 5.2 | 6.0 |  |
|  | Throughput lost at 5%-tile | 0.26 | 0.3 | 0.307 | 0.31 | 0.312 | 0.314 | 0.4 | 0.64 | 0.93 | 1.28 |  |

Based on the above simulation results, the required DL ACIR for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for indoor hotpot scenarios are in summarized in Table 4.3.2.1-5.

Table 4.3.2.1-5: DL simulation results for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz

| DL simulations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ACIR (dB) for Indoor |  |  |  | Can we keep agreed BS ACLR (38-37) and UE ACS (32-31) for indoor? |
|  | 6.425 - 7.125 GHz |  | 10.0 - 10.5 GHz |  |  |
| ACIR for macro BS | 31.0 |  | 30.0 |  |  |
|  | Omni | AAS | Omni | AAS |  |
| CATT | 18 | 20 | 18 | 19 | Yes |
| Huawei |  | <25.9 |  | <25.9 | Yes |
| Nokia | <ACIR - 5 | <ACIR – 5 | <ACIR - 5 | <ACIR - 5 | Yes |
| ZTE | <ACIR – 5 | <ACIR - 5 | <ACIR – 5 | <ACIR - 5 | Yes |
| Ericsson | 17 |  | 17 |  | Yes |
| Qualcomm | 16 | 18 | 16 | 18 | Yes |

#### 4.3.2.2	Uplink

For indoor scenarios, simulation results for UL throughput loss with baseline assumption ACLR = 30 dBc at UE and ACS = 46 dBc at BS are summarized in Table 4.3.2.2-1/1a/2/2a/3/3a/4/4a.

Table 4.3.2.2-1: UL throughput loss of victim BS for 6425 – 7125 MHz with omni antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | Omni based BS | Average throughput loss in % (7 GHz) | 0.1433 | 0.1794 | 0.2245 | 0.2805 | 0.3499 | 0.4356 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.0078 | 0.0098 | 0.0123 | 0.0155 | 0.0212 | 0.0246 |
| Nokia | Omni based BS | Average throughput loss in % (7 GHz) | 0.30 | 0.37 | 0.46 | 0.58 | 0.73 | 0.91 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.05 | 0.08 | 0.10 | 0.12 | 0.19 | 0.27 |

Table 4.3.2.2-1a: UL throughput loss of victim BS for 6425 – 7125 MHz with omni antenna [Absolute ACIR value]

|  | ACIR (dB) | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average | 1.8 | 2.2 | 2.8 | 3.5 | 4.3 | 5.3 | 6.6 | 8.1 | 9.9 |  |  |
|  | Throughput lost at 5%-tile | 1.2 | 1.5 | 1.9 | 2.4 | 3.1 | 3.9 | 4.8 | 5.8 | 7.4 |  |  |
| CATT | Throughput lost at Average |  | 2.392580 | 2.954531 | 3.639646 | 4.471517 | 5.476805 | 6.684997 | 8.127856 | 9.843106 | 11.858804 | 14.204583 |
|  | Throughput lost at 5%-tile |  | 1.391922 | 1.752115 | 2.163126 | 2.636267 | 3.216128 | 3.893275 | 4.758722 | 6.027059 | 7.603577 | 9.503131 |
| Qualcomm | Throughput lost at Average |  | 2.5 | 3.0 | 4.0 | 5.0 | 6.0 | 7.0 | 8.5 | 11.0 | 13.0 | 15.0 |
|  | Throughput lost at 5%-tile |  | 0.8 | 1.0 | 1.5 | 1.8 | 2.0 | 2.5 | 3.0 | 4.5 | 5.5 | 6.5 |

Table 4.3.2.2-2: UL throughput loss of victim BS for 6425 – 7125 MHz with AAS antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | AAS based BS | Average throughput loss in % (7 GHz) | 0.1414 | 0.1770 | 0.2211 | 0.2759 | 0.3437 | 0.4275 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.0033 | 0.0042 | 0.0053 | 0.0378 | 0.0880 | 0.1511 |
| Nokia | AAS based BS | Average throughput loss in % (7 GHz) | 0.16 | 0.20 | 0.24 | 0.30 | 0.38 | 0.47 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.04 | 0.05 | 0.07 | 0.08 | 0.10 | 0.13 |

Table 4.3.2.2-2a: UL throughput loss of victim BS for 6425 – 7125 MHz with AAS antenna [Absolute ACIR value]

|  | ACIR (dB) | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average |  |  |  |  |  |  |  |  |  |  |  |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  |  |  |  |  |  |
| CATT | Throughput lost at Average |  |  |  |  |  |  | 3.165202 | 3.814549 | 4.575053 | 5.458766 | 6.477323 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 2.151940 | 2.644977 | 3.314275 | 4.158239 | 5.164969 |
| Qualcomm | Throughput lost at Average |  | 2.0 | 2.5 | 3 | 3.5 | 4.5 | 5.0 | 5.5 | 7.5 | 9.0 |  |
|  | Throughput lost at 5%-tile |  | 0.7 | 1.0 | 1.5 | 1.8 | 2.0 | 2.4 | 3.0 | 4.0 | 5.0 |  |

Table 4.3.2.2-3: UL throughput loss of victim BS for 10 - 10.5 GHz with omni antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | Omni based BS | Average throughput loss in % (7 GHz) | 0.1510 | 0.1892 | 0.2367 | 0.2957 | 0.3689 | 0.4593 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.0214 | 0.0347 | 0.0362 | 0.0382 | 0.0407 | 0.1147 |
| Nokia | Omni based BS | Average throughput loss in % (7 GHz) | 0.30 | 0.38 | 0.47 | 0.59 | 0.74 | 0.92 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.07 | 0.09 | 0.12 | 0.19 | 0.37 | 0.59 |

Table 4.3.2.2-3a: UL throughput loss of victim BS for 10 - 10.5 GHz with omni antenna [Absolute ACIR value]

|  | ACIR (dB) | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average | 1.8 | 2.2 | 2.8 | 3.5 | 4.3 | 5.3 | 6.6 | 8.1 | 9.9 |  |  |
|  | Throughput lost at 5%-tile | 1.2 | 1.5 | 1.9 | 2.4 | 3.1 | 3.9 | 4.8 | 5.8 | 7.4 |  |  |
| CATT | Throughput lost at Average |  | 2.260160 | 2.793478 | 3.445105 | 4.238153 | 5.198868 | 6.356444 | 7.742539 | 9.400835 | 11.352258 | 13.623966 |
|  | Throughput lost at 5%-tile |  | 1.537452 | 1.949692 | 2.435648 | 2.994580 | 3.642921 | 4.428238 | 5.338709 | 6.554238 | 8.002240 | 9.693387 |
| Qualcomm | Throughput lost at Average |  | 2.5 | 3.0 | 4.0 | 5.0 | 6.0 | 7.0 | 9 | 11.0 | 13.0 | 15.0 |
|  | Throughput lost at 5%-tile |  | 0.8 | 1.0 | 1.5 | 1.8 | 2.2 | 2.5 | 3.5 | 5 | 5.5 | 7.0 |

Table 4.3.2.2-4: UL throughput loss of victim BS for 10 - 10.5 GHz with AAS antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE | AAS based BS | Average throughput loss in % (7 GHz) | 0.1536 | 0.1920 | 0.2395 | 0.2983 | 0.3709 | 0.4602 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.0900 | 0.0904 | 0.0910 | 0.0918 | 0.0927 | 0.0938 |
| Nokia | AAS based BS | Average throughput loss in % (7 GHz) | 0.16 | 0.20 | 0.25 | 0.31 | 0.38 | 0.48 |
|  |  | 5%-tile throughput loss in % (7 GHz) | 0.00 | 0.01 | 0.01 | 0.01 | 0.02 | 0.02 |

Table 4.3.2.2-4a: UL throughput loss of victim BS for 10 - 10.5 GHz with AAS antenna [Absolute ACIR value]

|  | ACIR (dB) | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson | Throughput lost at Average |  |  |  |  |  |  |  |  |  |  |  |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  |  |  |  |  |  |
| CATT | Throughput lost at Average |  |  |  |  |  |  | 3.302668 | 3.975666 | 4.761144 | 5.670780 | 6.715829 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 2.197258 | 2.860650 | 3.679917 | 4.658938 | 5.877432 |
| Qualcomm | Throughput lost at Average |  | 2.0 | 2.5 | 3.0 | 4.0 | 4.5 | 5.0 | 6.0 | 7.5 | 9.0 | 10.5 |
|  | Throughput lost at 5%-tile |  | 0.5 | 1.0 | 1.5 | 1.8 | 2.0 | 2.4 | 3.0 | 4.0 | 4.8 | 5.5 |

Based on the above simulation results, the required UL ACIR for 6.425-7.125GHz and 10.0-10.5 GHz for indoor hotpot scenarios are in summarized in Table 4.3.2.1-5.

Table 4.3.2.2-5: UL simulation results for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz

| UL simulations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ACIR (dB) for Indoor |  |  |  | Can we keep agreed UE ACLR (26-24) and BS ACS (42-40) for indoor? |
|  | 6.425-7.125GHz |  | 10.0-10.5GHz |  |  |
| Macro urban | 25.9 |  | 23.9 |  |  |
|  | Omni | AAS | Omni | AAS |  |
| CATT | 17 | 13 | 17 | 13 | Yes |
| Huawei |  | <23.9 |  | <23.9 | Yes |
| Nokia | <ACIR - 5 | <ACIR - 5 | <ACIR – 5 | <ACIR - 5 | Yes |
| ZTE | <ACIR – 5 | <ACIR – 5 | <ACIR - 5 | <ACIR - 5 | Yes |
| Ericsson | 17 |  | 17 |  | Yes |
| Qualcomm | 17 | 15 | 17 | 15 | Yes |

# 5	General parameters

## 5.1	Duplex mode

For both frequency ranges, even if FDD is not precluded, it’s most likely that TDD should be used in these frequency ranges.

## 5.2	Channel Bandwidth

A pragmatic, simple and non-ambiguous answers should be provided to ITU-R. While a number of channel bandwidth would be specified for these frequency ranges, 100 MHz has been considered as a representative channel bandwidth that will be used.

## 5.3	Signal Bandwidth

The signal bandwidth for a 100 MHz channel bandwidth signal is calculated based on the NR spectrum utilization for 30 kHz SCS:

Signal bandwidth = NRB x SCS x 12

with NRB: Number of Resource block for 100 MHz channel bandwidth and 30kHz SCS, as specified in TS 38.104 [3].

# 6	BS parameters

## 6.1	Transmitter characteristics

### 6.1.1	Power dynamic range

There is no power control in downlink and fixed power per resource block is assumed in the co-existence simulation. Hence 0 dB power dynamic range was agreed for the LS reply.

### 6.1.2	Spectral mask

Existing FR1 operating band unwanted emission mask is the same as for LTE (5 MHz ~ 20 MHz CBW). For both 6.425 - 7.125 GHz and 10.0 - 10.5 GHz band, it is foreseen the smaller channel bandwidth such as less than 50 MHz CBW is less attractive. Hence the basic limits for OOB emission mask should be updated as below. It is agreed that it does not mean that 50 MHz will be the minimum channel bandwidth when RAN4 specified the channel bandwidth and emission requirements.

Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -13 dBm | 1MHz |

Table 6.1.2-2: Wide Area BS operating band unwanted emission limits for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -15 dBm | 1MHz |

### 6.1.3	ACLR

According to the simulation results in clause 4.3, it is agreed to specify 38 dB ACLR for 6.425 - 7.125 GHz and 37 dB ACLR for 10.0 - 10.5 GHz.

For 6425 – 7125 MHz and 10 - 10.5 GHz, the ACLR should be higher than the value specified in table 6.1.3-1.

Table 6.1.3-1: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 40, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 38 dB for 6425-7125 MHz 37 dB for 10-10.5 GHz |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 38 dB for 6425-7125 MHz 37 dB for 10-10.5 GHz |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). |  |  |  |  |

### 6.1.4	Spurious emissions

From the Rel-15 discussion, the offset ΔfOBUE for the boundary between OBUE and spurious emissions is the most demanding parameters for OTA or Hybrid limits AAS BS. Based on the implementation evaluation and regulatory consideration, it is agreed to adopt ΔfOBUE = 100 MHz for both 6.425 - 7.125 GHz and 10.0 - 10.5 GHz.

Regarding spurious emission requirements for 6.425 - 7.125 GHz, as this frequency range is still within FR1 band definition, it is agreed to reuse the existing spurious emission requirements defined in TS 38.104 [3] and further update needed are to define upper frequency of Tx spurious emission to be 26 GHz instead of 5th harmonic of DL frequency according to ITU-R SM 329-10 [5] recommendation. The general spurious emissions for 6.425-7.125 GHz is defined in Table 6.1.4-1.

Regarding spurious emission requirements for 10-10.5 GHz, according to ERC 74-01 report [6], the lower frequency limit could be defined as 30 MHz. Without any specified spurious limit for AAS BS in between 6 and 24 GHz in ERC 74-01 [6], in relation to the existing frequency ranges specified, with observations of the foreseen implementation of AAS BS and how they are to be deployed and used, the spurious limits specified for above 24 GHz with upper frequency limited to 26 GHz will be used for 10.0-10.5 GHz as defined in Table 6.1.4-2.

Table 6.1.4-1: BS spurious emission limits for 6.425 - 7.125 GHz

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz – 150 kHz | -36 dBm | 1 kHz |
| 150 kHz – 30 MHz |  | 10 kHz |
| 30 MHz – 1 GHz |  | 100 kHz |
| 1 GHz – 26 GHz | -30 dBm | 1 MHz |

Table 6.1.4-2: BS spurious emission limits for 10 - 10.5 GHz

| Spurious frequency range | Basic limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18 GHz – 26 GHz | -20 dBm | 10 MHz |

### 6.1.5	Maximum output power

The maximum output power will be provided in the antenna parameter table. It was agreed to be aligned with antenna characteristics.

The Total Radiated Power for two polarizations was agreed as shown in table 6.1.5-1 below.

Table 6.1.5-1: The Total Radiated Power

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- |
| Total Radiated Power for two polarizations (dBm) | 46 | 46 | 37 |

### 6.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS.

## 6.2	Receiver characteristics

### 6.2.1	Noise figure

From the TR 38.820 for 7 - 24 GHz, the typical Noise Figure for a Wide Area BS operating at 10 GHz is 7 dB (12 dB for Medium Range BS and 15 dB for Local Area BS).

Table 6.2.1-1: Typical noise figure for 7 – 24 GHz example frequencies

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE (dB) |
| --- | --- | --- |
| 10 | 7 | 9 |
| 15 | 8 | 10 |
| 20 | 9 | 10 |

For 6.425 - 7.125 GHz, the typical Noise Figure for a Wide Area BS operating at 7 GHz was agreed to be 6 dB (11 dB for Medium Range BS and 14 dB for Local Area BS).

### 6.2.2	Sensitivity

As it is not clear if it will have a conducted sensitivity requirement for both frequency ranges, however the OTA sensitivity requirement will be needed either way and will be based on the NF and the antenna gain:

EISREFSENS=PkT+10*log10BW+NF+IM+SNR-G    (dBm)

Where:

-	BW is the configured bandwidth of the FRC,

-	NF is the noise figure,

-	IM is implementation margin not related to antenna array,

-	SNR is the required SNR to reach 95% throughput, and

-	G is the antenna gain including RF losses and 3dB off peak margin.

However, the sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter.

### 6.2.3	Blocking response

The in-band blocking requirement should apply from FUL,low - ΔfOOB to FUL,high + ΔfOOB, excluding the downlink frequency range of the FDD operating band. It is agreed to adopt ΔfOOB = 100 MHz for both 6.425 - 7.125 GHz and 10.0 - 10.5 GHz. The in-band blocking levels are reused from existing FR1 requirements.

Table 6.2.3-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area BS: -43 Medium Range BS: -38 Local Area BS: -35 | ±30 | 20 MHz DFT-s-OFDM NR signal 15 kHz SCS, 100 RBs |
| NOTE: PREFSENS depends on the RAT. |  |  |  |  |

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz for FR1 bands. -15 dBm CW interfering signal is reused for out-of-band blocking for both 6.425-7.125GHz and 10.0-10.5GHz.

### 6.2.4	ACS

According to the simulation results in clause 4.3, it is agreed to specify 42 dB ACS for 6.425 - 7.125 GHz and 40 dB ACLR for 10.0 - 10.5 GHz.

# 7	UE parameters

## 7.1	Transmitter characteristics

### 7.1.1	Power dynamic range

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. For existing FR1 bands, the minimum output power is -33 dBm for 100 MHz channel bandwidth. The minimum output power can be reused for both frequency ranges, 6.425 - 7.125 GHz and 10.0 - 10.5 GHz, i.e. power dynamic range is 56 dB for 100 MHz channel bandwidth.

### 7.1.2	Spectral mask

The spectral mask for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz is defined in below table which is relaxed general NR FR1 spectrum at the FOOB edge ± 0 – 5 MHz by 3 dB.

Table 7.1.2-1: Spectrum emission mask for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz

|  | Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Measurement bandwidth |
| ± 0-1 | -10 | -10 | -10 | -10 |  |  |  |  |  |  | 1 % channel bandwidth |
| ± 0-1 |  |  |  |  | -21 | -21 | -21 | -21 | -21 | -21 | 30 kHz |
| ± 1-5 | -7 | -7 | -7 | -7 | -7 | -7 | -7 | -7 | -7 | -7 | 1 MHz |
| ± 5-6 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 | -13 |  |
| ± 6-10 |  |  |  |  |  |  |  |  |  |  |  |
| ± 10-15 |  |  |  |  |  |  |  |  |  |  |  |
| ± 15-20 |  |  |  |  |  |  |  |  |  |  |  |
| ± 20-25 | -25 |  |  |  |  |  |  |  |  |  |  |
| ± 25-30 |  | -25 |  |  |  |  |  |  |  |  |  |
| ± 30-35 |  |  | -25 |  |  |  |  |  |  |  |  |
| ± 35-40 |  |  |  |  |  |  |  |  |  |  |  |
| ± 40-45 |  |  |  | -25 |  |  |  |  |  |  |  |
| ± 45-50 |  |  |  |  |  |  |  |  |  |  |  |
| ± 50-55 |  |  |  |  | -25 |  |  |  |  |  |  |
| ± 55-60 |  |  |  |  |  |  |  |  |  |  |  |
| ± 60-65 |  |  |  |  |  | -25 |  |  |  |  |  |
| ± 65-70 |  |  |  |  |  |  |  |  |  |  |  |
| ± 70-75 |  |  |  |  |  |  | -25 |  |  |  |  |
| ± 75-80 |  |  |  |  |  |  |  |  |  |  |  |
| ± 80-85 |  |  |  |  |  |  |  | -25 |  |  |  |
| ± 85-90 |  |  |  |  |  |  |  |  |  |  |  |
| ± 90-95 |  |  |  |  |  |  |  |  | -25 |  |  |
| ± 95-100 |  |  |  |  |  |  |  |  |  |  |  |
| ± 100-105 |  |  |  |  |  |  |  |  |  | -25 |  |

For the last 5 MHz of UE SEM, it is agreed that we might revisit the value pending MRP definition in WI phase.

### 7.1.3	ACLR

According to the simulation results in clause 4.3, it is agreed to specify 26 dB ACLR for 6.425 - 7.125 GHz and 24 dB ACLR for 10.0 - 10.5 GHz.

### 7.1.4	Spurious emissions

The general spurious emissions defined in TS 38.101-1 [4] clause 6.5.3.1 can apply to both frequency ranges, 6.425 - 7.125 GHz and 10.0 - 10.5 GHz.

-	30 MHz ≤ f ≤ 1 GHz: - 36 dBm / 100 kHz

-	1 GHz ≤ f ≤ 26 GHz: - 30 dBm / 1 MHz

### 7.1.5	Maximum output power

TR 38.820 indicates that 23 dBm is feasible at 10 - 10.5 GHz and this is an assumption for co-existence analysis. Hence the UE maximum output power for the considered frequency ranges could be 23 dBm. Other UE power classes are not precluded for both frequency ranges, 6.425 - 7.125 GHz and 10.0 - 10.5 GHz.

### 7.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS to WP5D.

## 7.2	Receiver characteristics

### 7.2.1	Noise figure

The noise figure of 9 dB is the baseline assumption and 13 dB is the optional assumption for co-existence study for 6425 – 7125 MHz and 10 - 10.5 GHz.

A noise figure in the [9, 13] dB interval was finally agreed for reporting to ITU WP5D sharing studies. Note that the noise figure value in [9, 13] dB interval should be used only for WP5D response. The actual noise figure to be used to define RF requirements should be further studied in the WI phase.

### 7.2.2.	Sensitivity

The sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter.

### 7.2.3	Blocking response

The blocking characteristic specified in clause 7.6 of TS 38.101-1 [4] for frequency larger than 3300 MHz could be applied for 6425 – 7125 MHz and 10 - 10.5 GHz.

### 7.2.4	ACS

According to the simulation results in clause 4.3, adjacent channel selectivity (ACS) is agreed as 32 dBc for 6425 – 7125 MHz and 31 dBc for 10 - 10.5 GHz.

# 8	Antenna characteristics

## 8.1	BS antenna characteristics

### 8.1.1		Array antenna model

In Table 8.1.1-1, the parameters used by the parameterized array antenna model are described.

Table 8.1.1-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.1-2.

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan | dBi |

### 8.1.2	Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 8.1.2-1 in dBi as:

GE,max=DE,max-LE	(8.1.2-1)

, where the peak directivity DE,max is calculated from given values on 3dB, 3dB, dh and dv as:

DE,max=10log104πAlinθ,φmax-ππ0πAlinθ,φsinθdθdφ	(8.1.2-2)

, where Alin(,) is defined in linear scale as:

Alinθ,φ=10Aθ,φ10	(8.1.2-3)

Table 8.1.2-1: BS array antenna parameters

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Small cell indoor |
| --- | --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 | 30 |
| 3dB (deg.) | 90 | 90 | 90 | 90 |
| 3dB (deg.) | 65 | 90 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 | 2.0 |
| (M, N) | (16, 8) | (16, 8) | (8,8) | (4, 4) |
| dh (m) | 0.5 | 0.5 | 0.5 | 0.5 |
| dv (m) | 0.7 | 0.5 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 | N/A |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 | N/A |
| Conducted power (before ohmic loss) per antenna element (dBm) | 22 (Note 4) | 22 (Note 4) | 16 (Note 5) | 9 (Note 6) |
| Mechanical downtilt (deg.) | 6 | 10 | N/A | N/A (Note 7) |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: The conducted power per element assumes 4x4x2 elements (i.e. power per H/V polarized element). Note 7: Boresight direction is perpendicular to the ceiling. |  |  |  |  |

## 8.2	UE antenna characteristics

The outcome of the RAN4 study for collecting technical background information relevant for the frequency range 7 to 24 GHz indicated that the frequency range 7.25-[10-13] GHz would have “FR1 like” requirements, and as such we can assume that in the 10 - 10.5 GHz range this applies. The UE will most likely therefore have a conducted interface with an assumed isotropic radiation pattern antenna and no beamforming.

# 9	Other information relevant for the sharing and compatibility studies

## 9.1	Spatial emission

Traditionally, antenna data sheets provide information on not only the antenna peak gain in the intended direction but also the gain in unwanted directions. To describe characteristics in unintended directions the following metrics are used:

1.	The antenna front-to-back ratio, which is the power ratio of radiative power in the main beam directions to the power radiated in the backward direction.

2.	The antenna side lobe ratio, which is the power ratio between the main beam direction and the power of the strongest side lobe.

However, for an AAS base station it is clear that these traditional metrics of antenna characteristics are not directly relevant, since an AAS base station has the ability to adaptively shape the spatial characteristics to maintain optimum network throughput. Therefore, additional declarations were defined in TS 38.141-2 [x], Annex G. The additional declarations include information on how much power is radiated outside the intended coverage region in relation to the power radiated with in the intended coverage region.

## 9.2	Interference management

### 9.2.0	General

Given an antenna array with M multiplied by N identical elements, the radiation pattern of the array antenna can be described according to the pattern multiplication theorem as:

Rθ,φ=REθ,φRAθ,φ 	(Eq. 9.2-1)

, where is RE is the radiation pattern for the array elements and RA is the radiation pattern associated to the array factor. The element patten, RE(, is based on a parameterized Gaussian shaped element, with floors to model for side-lobe levels and front-to-back ratio. The element peak gain is directivity normalized, hence the peak element gain GE,max, element loss LE and half power beam widths 3dB and 3dB should be selected carefully to maintain correct antenna gain.

It can be noticed that both the element factor and the array factor can be used to shape the composite radiation pattern. The element pattern can be used to suppress side-lobe characteristics. For a limited steering range, a sub-array element can be used to suppress side-lobes better than a single element configuration. Typically for an AAS base station implementation the element radiation pattern and the array factor are customized to optimize the coverage within a specific coverage range for a given deployment scenario.

For a general array antenna, the array factor radiation pattern for transmitting array antenna with MN element per polarization can be expressed as:

RAθ,φ=n=1MNwnejkr2 	(Eq. 9.2-2)

, where

-	wn is the complex array excitation,

-	k is the wave vector of the transmitted wave, and

-	r is the element location matrix.

### 9.2.1	Beam nulling

Beam nulling technology is used to suppress the unwanted spatial emission by inserting nulls in the radiation pattern for the direction of the interference. One typical applicable scenarios is multiple beams transmission. As shown in Figure 2 as one example, Beam 1 is the serving beam of UE 1 but the side lobe of beam 1 would interfere the UE2. The SINR for UE2 will be affected. In order to support high order modulation scheme such as DL 256 QAM, using beam nulling technology, a null can be placed at the direction to UE2 for beam 1. The weight of each antenna array of beam 1 can be obtained by specific algorithms so that SINR of beam1 is maximized in the direction to UE1 and the transmission power in the main lobe is maintained, and the side lobe are suppressed in the direction to UE2. Beam nulling can also be used in the inter-cell or inter system scenarios as long as the location or direction of the protected station is known.

Figure 9.2.1-1: Side lobe interference

### 9.2.2	Amplitude weighting/tapering

AAS offer a wide range of opportunities on optimizing the directivity patterns through amplitude and phase control. High directivity antenna array also have side lobes which are often undesirable since they may cause intra-cell or inter-cell interference. Side lobe levels can be reduced via tailoring the amplitude across the antenna array which is often referred as amplitude weighting technology. Whilst amplitude weighting/tapering reduces the side lobes it also makes the main lobe wider and hence reduces gain. It needs tradeoff between antenna gain and side lobe suppression. For example:

Figure 9.2.2-1: Examples for amplitude weighting/tapering: left: Kaiser (a=3), right: Chebwin (R=35dB)

### 9.2.3	Asymmetric side lobe shaping

In addition to beam tapering it is possible to manipulate the amplitude and the phase of the window (for example using modified Taylor series) to modify the side lobe levels asymmetrically. This technique is widely used with passive BS arrays to make the ground side lobes larger to fill in the angles between the main beam and the antenna. The same technique can be used to minimise radiation in specified unwanted directions. For example:

Figure 9.2.3-1: Example for modified Taylor window

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-03 | RAN4#94bis-e | R4-2004477 |  |  |  | TR skeleton | 0.0.1 |
| 2020-08 | RAN4#96-e | R4-2010370 |  |  |  | 1. Agreed Text Proposal in RAN4#95-e: R4-2008928, “TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz” 2. A new clause is added on general parameters | 0.1.0 |
| 2020-10 | RAN4#97-e | R4-2015675 |  |  |  | Agreed Text Proposal in RAN4#96-e: R4-2011829, “TP to TR 38.921: BS IMT technology related parameters” R4-2011830, “TP to TR 38.921: Addition of BS antenna model and parameters in subclause 4.2.3 and subclause 8.1” | 0.2.0 |
| 2021-02 | RAN4#98-e | R4-2101494 |  |  |  | Agreed Text Proposal in RAN4#97-e: R4-2016903 Maintenance TP to TR38.921 R4-2016906 TP to TR38.921 : BS spurious emission R4-2016901 TP to TR 38.921: Clarification of system level simulation assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz R4-2017817 TP to TR38.921: uplink ACIR model R4-2014478 TP to TR 38.921: Clarification of BS array antenna element peak gain for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz R4-2016902 TP to TR 38.921: Correction to antenna parameter table in clause 3 and sub-clause 8.1 R4-2016907 TP on spatial emission and interference mitigation | 0.3.0 |
| 2021-03 | RAN#91-e | RP-210463 |  |  |  | 1. Agreed Text Proposal in RAN4#98-e: R4-2103408 TP to TR 38.921 UE transmitter requirements R4-2103398 TP to TR 38.921: BS remaining parameters R4-2103107 TP to TR 38.921 summary of simulation results R4-2101793 TP to TR 38.921: Clarification of BS maximum transmit power on system level simulation assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz R4-2103109 TP to TR 38.921: Addition of in-door antenna parameters and correction to model in subclause 8.1 1. Editorial changes to align the TR with drafting rules. 2. Submitted to RAN #91-e for 1-step Approval. | 1.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-03 | RAN#91 |  |  |  |  | Approved by plenary – Rel-17 spec under change control | 17.0.0 |