| 3GPP TR 23.700-56 V1.1.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on system architecture enhancement for next generation real time communication phase 3; Stage 2 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	11

1	Scope	13

2	References	13

3	Definitions of terms, symbols and abbreviations	13

3.1	Terms	13

3.2	VOID	14

3.3	Abbreviations	14

4	Architectural Assumptions and Requirements	14

4.1	Architectural Assumptions	14

5	Key Issues	14

5.1	Key Issue #1: ADC establishment in alerting phase for early media	14

5.2	Key Issue #2: Enhancements to capability exposure framework for eMMTel services	15

5.3	Key Issue #3: Supporting dynamic configuration of MDC2 endpoints	15

6	Solutions	16

6.0	Mapping of Solutions to Key Issues	16

6.1	Solution #1: ADC establishment in alerting phase initiated by the UE	16

6.1.0	High-level solution Principles	16

6.1.1	Description	16

6.1.2	Procedure	16

6.1.3	Impacts on Services, Entities and Interfaces	20

6.2	Solution #2: Support for NW initiated data channel application in early media session with subscription	20

6.2.1	High level principles	20

6.2.2	Description	20

6.2.3	Procedures	22

6.2.4	Impacts to Services, Entities and Interfaces	25

6.3	Solution #3: IMS ADC establishment during alerting phase when initial INVITE does have BDC offer	25

6.3.0	High-level solution Principles	25

6.3.1	Description	25

6.3.2	Procedures	26

6.3.3	Impacts on Services, Entities and Interfaces	28

6.4	Solution #4: IMS ADC establishment during alerting phase when initial INVITE does not have BDC offer	29

6.4.0	High-level solution Principles	29

6.4.1	Description	29

6.4.2	Procedures	30

6.4.3	Impacts on Services, Entities and Interfaces	32

6.5	Solution #5: Early Media in Alerting Phase (CAT) Based on DC AS Initiated A2P DC	32

6.5.0	High-level solution Principles	32

6.5.1	Description	33

6.5.2	Procedure	33

6.5.3	Impacts on Services, Entities and Interfaces	34

6.6	Solution #6: Support Alerting phase P2A ADC establishment through Autolaunch/Autoload Parameter	35

6.6.0	High-level solution Principles	35

6.6.1	Description	35

6.6.2	Procedure	36

6.6.2.1	Application Download Procedure	36

6.6.2.2	Originating-side Application Data Channel Establishment Procedure in P2A Mode	37

6.6.2.3	Terminating-side Application Data Channel Establishment Procedure in P2A Mode	38

6.6.2.4	Terminating Application Data Channel After the Call Established	38

6.6.3	Impacts on Existing Nodes and Functionality	39

6.7	Solution #7: IMS DC Events Supported by DCSF	39

6.7.0	High-level solution Principles	39

6.7.1	Description	39

6.7.2	Procedure	40

6.7.3	Impacts on Services, Entities and Interfaces	42

6.8	Solution #8: Subscription and notification of DC application downloading events	42

6.8.0	High-level solution principles	42

6.8.1	Types of DC application downloading events	42

6.8.2	Procedure of DC application downloading event subscription and notification	43

6.8.3	Procedure for reporting DC application usage event analytics	44

6.8.4	Impacts on services, entities and interfaces	46

6.9	Solution #9: Capability exposure of IMS DC events supported by DCSF	47

6.9.0	High-level solution Principles	47

6.9.1	Description	47

6.9.2	Service definition	48

6.9.2.1	General	48

6.9.2.1.1	HSS Service	48

6.9.2.1.2	DCSF Service	50

6.9.3	Procedure	51

6.9.3.1	DCSF instance registration in HSS	51

6.9.3.2	DCSF address discovery by AF	51

6.9.3.3	Subscription for IMS DC events supported by DCSF	52

6.9.4	Impacts on Services, Entities and Interfaces	53

6.10	Solution #10: DC3/DC4 Enhancement to Support Subscription, Notification and Instruction for P2A and P2A2P DC establishment	54

6.10.0	High-level solution Principles	54

6.10.1	Description	54

6.10.2	Service definition	54

6.10.2.1	General	54

6.10.2.2	Ndcsf_EventExposure	55

6.10.2.2.1	General	55

6.10.2.2.2	Ndcsf_EventExposure_Subscribe	55

6.10.2.2.3	Ndcsf_EventExposure_Unsubscribe	55

6.10.2.2.4	Ndcsf_EventExposure_Notify	55

6.10.2.3	Ndcsf_MediaControl	55

6.10.2.3.1	General	55

6.10.2.3.2	Ndcsf_MediaControl_MediaInstruction	56

6.10.3	Procedure	56

6.10.3.1	Service Subscription Procedure	56

6.10.3.2	P2A Application Data Channel Establishment Procedure	56

6.10.3.3	P2A2P Application Data Channel Establishment Procedure	58

6.10.4	Impacts on Services, Entities and Interfaces	59

6.11	Solution #11: DC3/DC4 based on DCSF services	59

6.11.0	High-level solution principles	59

6.11.1	Description	60

6.11.2	Service definition	60

6.11.2.1	General	60

6.11.2.2	Ndcsf_SessionEvent	61

6.11.2.2.1	General	61

6.11.2.2.2	Ndcsf_SessionEvent_Subscribe	61

6.11.2.2.3	Ndcsf_SessionEvent_Unsubscribe	61

6.11.2.2.4	Ndcsf_SessionEvent_Notify	61

6.11.2.3	Ndcsf_ADCControl	62

6.11.2.3.1	General	62

6.11.2.3.2	Ndcsf_ADCControl_Create	62

6.11.2.3.4	Ndcsf_ADCControl_Delete	62

6.11.2.3.4	Nimsas_MediaControl_MediaInstruction	62

6.11.3	Procedure	62

6.11.3.1	Person-to-Application (P2A) Application Data Channel Setup	62

6.11.3.2	Person-to-Application-to-Person (P2A2P) Procedure	64

6.11.4	Impacts on services, entities and interfaces	66

6.12	Solution #12: Support dynamic configuration of MDC2 endpoints with new DCSF services	67

6.12.0	High-level solution Principles	67

6.12.1	Description	67

6.12.2	Procedures	67

6.12.2.1	P2A DC procedure	67

6.12.2.2	P2A2P DC procedure	69

6.12.3	DCSF Services	71

6.12.3.1	Ndcsf_MDC2Connection	71

6.12.3.1.1	General	71

6.12.3.1.2	Ndcsf_MDC2Connection_CreateNotify service operation	71

6.12.3.1.3	Ndcsf_MDC2Connection_UpdateNotify service operation	72

6.12.3.1.4	Ndcsf_MDC2Connection_DeleteNotify service operation	72

6.12.4	Impacts on Services, Entities and Interfaces	72

6.13	Solution #13: DC3/DC4 enhancements with new media management	73

6.13.1	Description	73

6.13.2	Service definition	73

6.13.2.1	General	73

6.13.2.2	Ndcas_Media_Management	74

6.13.2.2.1	General	74

6.13.2.2.2	Ndcas_Media_Management_Create	74

6.13.2.2.3	Ndcas_Media_Management_Update	74

6.13.2.2.4	Ndcas_Media_Management_Delete	74

6.13.2.3	Nimsas_ImsParameterProvision Service API	75

6.13.2.3.1 	Type: ImsPpDataEntry	75

6.13.2.4	Nimsas_SessionEventControl Service API	75

6.13.2.4.1	Type: SessionEventNotification	75

6.13.3	Procedure	77

6.13.3.1	Person-to-Application (P2A) Application Data Channel Setup	77

6.13.3.2	Person-to-Application and Application-to-Person (P2A2P) Procedure	79

6.13.4	Impacts on Services, Entities and Interfaces	81

6.14	Solution #14: Dynamic MDC2 endpoint configuration IMS Data Channel	82

6.14.0	High-level solution Principles	82

6.14.1	Description	82

6.14.2	Service definition	83

6.14.2.1	General	83

6.14.2.2	NEF Service	83

6.14.2.2.1	Service Summary table Update	83

6.14.2.2.2	Update to Nnef_ImsEventExposure (ImsEE) service (clause 5.2.6.39.2 of TS 23.502 [8])	83

6.14.2.3	HSS Service	84

6.14.2.3.1	Service Summary table Update	84

6.14.2.3.2	Update to Nhss_ImsUECM_Registration service operation	85

6.14.2.3.3	Update to Nhss_ImsUECM_Deregistration service operation	85

6.14.2.3.4	Nhss_ImsUECM_DcsfInfoGet service operation	85

6.14.2.3.5	Nhss_ImsUECM_DcsfUnsubscribe	85

6.14.2.3.6	Nhss_ImsUECM_DcsfNotify	86

6.14.2.4	DCSF Service	86

6.14.2.4.0	General	86

6.14.2.4.1	Ndcsf_ADCControl_MDC2 service operation	86

6.14.2.4.2	Service Description	86

6.14.2.4.3	Ndcsf_ADCControl_MDC2Create	86

6.14.2.4.4	Ndcsf_ADCControl_MDC2Update	87

6.14.2.4.5	Ndcsf_ADCControl_MDC2Delete	87

6.14.2.5	Ndcsf_SessionEvent	87

6.14.2.5.1	General	87

6.14.2.5.2	Ndcsf_SessionEvent_Subscribe	87

6.14.2.5.3	Ndcsf_SessionEvent_Unsubscribe	88

6.14.2.5.4	Ndcsf_SessionEvent_Notify	88

6.14.3	Procedure	88

6.14.3.1	DCSF instance registration in HSS	88

6.14.3.2	DCSF address discovery by DC AS	89

6.14.4	Impacts on Services, Entities and Interfaces	90

6.15	Solution #15: Early Media in Alerting Phase (CRS) Based on DC AS Initiated A2P DC	91

6.15.0	High-level solution Principles	91

6.15.1	Description	91

6.15.2	Procedure	91

6.15.3	Impacts on Services, Entities and Interfaces	93

6.16	Solution #16: Enhancements to capability exposure framework and IMS AS service to support audio, video capability in addition with DC capability	93

6.16.1	High level Solution principles	93

6.16.2	Description	94

6.16.3	Procedures	95

6.16.3.1	DC AS initiated IMS audio session addition with a standalone A2P application data channel	95

6.16.3.2	Person-to-Application (P2A) MMTel media (audio) and Application Data Channel Setup	98

6.16.4	Service definition	100

6.16.4.1	Nnef_ImsSessionManagement service	100

6.16.4.1.1	General	100

6.16.4.1.2	Nnef_ImsSessionManagement_Create service operation	100

6.16.2.1.3	Nnef_ImsSessionManagement_Update service operation	101

6.16.4.2	Nimsas_ImsSessionManagement Service	102

6.16.4.2.1	General	102

6.16.4.2.2	Nimsas_ImsSessionManagement_Create service operation	102

6.16.4.2.3	Nimsas_ImsSessionManagement_Update service operation	103

6.16.4.3	Nmf_MediaResourceManagement (MRM) service	103

6.16.4.3.1	General	103

6.16.4.3.2	Nmf_MRM_Create service operation	104

6.16.5	Impacts on Services, Entities and Interfaces	106

6.17	Solution #17: DC3/DC4 Enhancement to Support Implicit Subscription and Notification for P2A and P2A2P DC establishment	106

6.17.0	High-level solution Principles	106

6.17.1	Description	107

6.17.2	Service definition	107

6.17.2.1	General	107

6.17.2.2	Ndcsf_EventExposure	107

6.17.2.2.1	General	107

6.19.2.2.2	Ndcsf_EventExposure_Notify	107

6.17.2.3	Ndcsf_MediaControl	107

6.17.2.3.1	General	107

6.17.2.3.2	Ndcsf_MediaControl_MediaInstruction	108

6.17.3	Procedure	108

6.17.3.1	P2A Application Data Channel Establishment Procedure	108

6.17.3.2	P2A2P Application Data Channel Establishment Procedure	110

6.17.4	Impacts on Services, Entities and Interfaces	112

7	 VOID	113

8	Conclusions	113

8.1	Conclusions for Key Issue #1	114

8.2	Conclusions for Key Issue #2	114

8.3	Conclusions for Key Issue #3	115

Annex A: Change History	116

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document investigates further enhancements to the capabilities of IMS data channel (DC), specifically in the areas of ADC establishment in alerting phase, IMS capability exposure and dynamic configuration of MDC2 endpoints.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 26.114: "Media handling and interaction".

[4]	3GPP TS 23.203: "Policy and charging control architecture".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[7]	3GPP TS 23.392: "Application enablement aspects for MMTel".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[9]	3GPP TS 29.175: "IP Multimedia Subsystem (IMS) Application Server (AS) Services Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	VOID

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2P	Application to Person

ADC	Application Data Channel

API	Application Programming Interface

DC	Data Channel

DCAR	Data Channel Application Repository

DCMF	Data Channel Media Function

DCMTSI	Data Channel Multimedia Telephony Service for IMS

DCSF	Data Channel Signalling Function

IMS	IP Multimedia Subsystem

MF	Media Function

MRF	Media Resource Function

NEF	Network Exposure Function

NF	Network Function

OMA	Open Mobile Alliance

P2A	Person to Application

P2A2P	Person to Application to Person

P2P	Person to Person

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

The following Architectural Assumptions are applicable to this Study:

-	The IMS and IMS DC architecture defined in TS 23.228 [2] is used as the basis for any enhancements.

-	An AS may be trusted or untrusted.

-	The functionality of existing standardized APIs (e.g. OMA APIs) exposing MMTEL services may be used where applicable when the MMTEL service does not depend on IMS DC media.

-	OMA APIs are not extended to support Data Channel and the principles outlined in TS 23.228 [2] Annex AD (IMS Subscribe/Notify framework for event monitoring).

# 5	Key Issues

## 5.1	Key Issue #1: ADC establishment in alerting phase for early media

This key issue will investigate how to support IMS application data channel establishment in alerting phase to provide early media for a normal IMS DC session.

This Key Issue will study:

-	Whether and how the capability of supporting ADC establishment in alerting phase is exchanged between UE and IMS network.

-	How the use of ADC during alerting phase is controlled.

-	Whether and how to download a specific list of DC applications that can be used during alerting phase.

-	How to establish and terminate an ADC during alerting phase.

## 5.2	Key Issue #2: Enhancements to capability exposure framework for eMMTel services

This Key Issue will investigate how to enhance IMS capability exposure framework to expose IMS capability for eMMTel services (voice/video) and related events besides Data Channel, supporting IMS session establishment, modification and release to enable eMMTel services that may require enhancing audio/video services with DC capabilities.

Solutions should describe how to enhance Rel-19 IMS event exposure framework and event definition to support IMS DC events supported by DCSF (e.g. application downloading event) and non-DC events, including subscriber specific and non-subscriber specific supported by IMS AS.

NOTE 1: 	This Key Issue will analyse relevant use cases, the gap between OMA and 3GPP APIs and specify exposure functionality missing from OMA APIs.

NOTE 2:	This Key Issue does not intend to transfer the responsibility of OMA API(s) to 3GPP.

This Key Issue will study:

-	Identification of new events and/or capabilities to be exposed, in addition to those already listed in Table AD.2.5.3-1 of TS 23.228 [2].

-	Assessment of whether existing OMA APIs support the exposure of the newly identified events and/or capabilities.

-	Possible enhancement of exposure functionality where the assessment above identifies gaps in supporting exposure of the newly identified events and/or capabilities.

## 5.3	Key Issue #3: Supporting dynamic configuration of MDC2 endpoints

This Key Issue will investigate whether and how to define interfaces between DCSF and NEF/AF to complete existing P2A/P2A2P Data Channel procedures by specifying DC3/DC4 interfaces to enable dynamic configuration of MDC2 endpoints.

This Key Issue will study:

-	the services and information elements that are exposed via DC3/DC4 for UE initiated P2A and P2A2P ADC session establishment.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 | X |  |  |
| #5 | X |  |  |
| #6 | X |  |  |
| #7 |  | X |  |
| #8 |  | X |  |
| #9 |  | X |  |
| 10 |  |  | X |
| 11 |  |  | X |
| 12 |  |  | X |
| 13 |  |  | X |
| 14 |  |  | X |
| 15 | X |  |  |
| 16 |  | X |  |
| 17 |  |  | X |

## 6.1	Solution #1: ADC establishment in alerting phase initiated by the UE

### 6.1.0	High-level solution Principles

This solution addresses Key Issue #1: DC establishment in alerting phase for early media.

The solution reuses Rel-19 TS 23.228 [2] with the addition that

-	Based on the UE request (via a specific root URL) , the DSCF sends to the UE(s) an application list related with the alerting phase-	The calling party receive BDC from the terminating network during alerting phase.

-	The UE(s) decide whether and which application to download (as for Rel-19 specifications).

-	The UE(s) decide when to release an application that was running in alerting phase; they may decide to continue this application after the alerting phase has finished. (as for Rel-19 specifications).

The solution may e.g. be used for the case where the originating UE1 calls a help centre and is immediately provided with an App that the user will be using when interacting with the call centre.

### 6.1.1	Description

This solution provides a procedure on how to establish a P2A ADC in alerting phase.

Editor's note:	Current version of the solution supports P2A ADC establishment in alerting phase. it is FFS whether it can support other forms of ADC, i.e. P2A2P and P2P.

### 6.1.2	Procedure

The pre-condition of ADC establishment in alerting phase is the capability negotiation during the IMS registration phase on whether the UE supports the DC in alerting phase.

Based on the IMS DC capability negotiation defined in clause AC.7.0 of TS 23.228 [2], when the UE supporting DC in alerting phase and the UE is allowed to use DC in alerting phase, it includes the media feature tag of supporting DC in alerting phase in the Contact header field of the initial REGISTER request and any subsequent REGISTER request to allow the home IMS network to discover its capability of supporting DC in alerting phase.

If the IMS network supports DC in alerting phase, the S-CSCF includes a Feature-Caps header field indicating its capability of supporting DC in alerting phase in the 200 OK response to the initial and any subsequent REGISTER request, which is used by the UE to discover the IMS data channel capability of its home IMS network.

The UE may receive a Feature-Caps header field indicating IMS's capability of supporting DC in alerting phase in the 200 OK response to a subsequent REGISTER request when the network starts supporting DC in alerting phase after successful initial registration of the UE.

When the UE supporting DC in alerting phase initiates an IMS session and if the UE is allowed to use DC in alerting phase, it includes the media feature tag in the Contact header field of the initial INVITE message, regardless of data channel media being part of the SDP or not.

The UE shall not include the media feature tag in the Contact header field and data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE request message, if the S-CSCF has not included the capability indication of supporting DC in alerting phase in the Feature-Caps header field in the 200 OK response either to the initial REGISTER or a subsequent REGISTER request message.

If the UE is not configured whether to use DC in alerting phase, it is up to UE implementation whether or not to include the media feature tag in the Contact header field.

With a successful capability negotiation about the support of DC in alerting phase, the procedure is illustrated in Figure 6.1.2-1, which is based on clause AC.7.2.2 in TS 23.228 [2].

Figure 6.1.2-1: Procedure of ADC establishment in alerting phase

The steps in the procedure are as follows:

1.	Same as step 1-14 in clause AC.7.1 of TS 23.228 [2], UE#1 initiates a SIP INVITE towards the IMS AS. After validating the user's subscription, the IMS AS notifies the DCSF about DC support. The DCSF provides the IMS AS with instructions and media information for establishing BDC. The IMS AS allocates resources with MF accordingly and updates the INVITE with the negotiated media. The updated SIP INVITE is forwarded to the terminating network/UE#2 for terminating network negotiation.

2-3.	UE#2 and terminating network returns an 18x response with the SDP answer to BDC to originating network. According to the received SDP answer, MF may update data channel media resource information for UE#2.

4.	The IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, Session ID, Media Info List) to DCSF.

5.	The DCSF responds to the Nimsas notification request.

6.	The SIP message18x is forwarded to UE#1 which indicates the BDC has been established and that the alerting phase has started.

7.	The originating network P-CSCF executes QoS procedure for BDC as specified in TS 23.203 [4] and TS 23.503 [5].

8.	PRACK and 200 OK for PRACK procedure are performed.

9-10.	When the BDCs have been established between terminating MF and UE#1/UE#2, the DC application list for alerting phase and DC application for alerting phase is requested and downloaded to UE#1 and UE#2 from terminating DCSF.

The DCSF provides via MF the application list for alerting phase from the received specific URL , further to UE#1 and UE#2, based on their data channel capabilities and their choices.

The UE(s) use a specific URL related with the fact that they request a list of applications available in the alerting phase

Editor's note:	It is FFs how the UE is alerted to initiate the download.

11.	UE#1 initiates a P2A ADC for the selected DC application for alerting phase. UE#1 sends SIP UPDATE with an updated SDP to IMS AS via originating network P-CSCF and S-CSCF. The updated SDP contains the BDC information, as well as the requested ADC and the associated DC application binding information.

12.	Same as step 3-14 of clause AC.7.2.2-1 in TS 23.228 [2], the IMS AS notifies the DCSF about DC support. The DCSF instructs the IMS AS to establish MF anchoring via MDC2. The IMS AS provisions the necessary media resources at the MF and returns the information to the DCSF. The DCSF then initiates ADC setup toward the DC Application Server via DC3/DC4. After receiving the DC AS's response with its own MDC2 media resource, the DCSF instructs the IMS AS to update the MF resource accordingly. The IMS AS completes the MF configuration and notifies the DCSF.

13-14.	IMS AS sends the SIP UPDATE to remote network side and UE#2, via the originating S-CSCF, which does not include a request in the SDP for the ADC.

15-17.	UE#2 and terminating network returns a SIP UPDATE 200 OK response with SDP answer for audio/video/bootstrap.

18.	IMS AS notifies the DCSF about the successful result of media confirmation.

19.	DCSF replies to the notification.

20.	The IMS AS includes SDP answer for ADC to UE#1 in SIP UPDATE 200 OK response that it sends towards the UE 1 via the S-CSCF and P-CSCF.

21.	The originating network P-CSCF executes QoS procedure for ADC media based on the SDP answer information from the 200 OK response.

22.	SIP UPDATE 200 OK forwarded to UE#1 with ADC SDP answer.

23.	The ADC between UE#1 and DC AS is established via MF. MF forwards data channel traffic between UE#1 and DC AS based on MDC2 media point information.

24-25.	UE#2 answers the call and SIP INVITE 200 OK response is sent to the IMS AS via I/S-CSCF.

26.	The IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (Session ID) of the call state change.

27.	DCSF replies to the notification.

28-29.	SIP INVITE 200 OK forwarded to UE#1 which indicates the call is answered and ACK procedure is performed.

30.	Both UE#1 and UE#2 retrieve a new application list via BDC since call is answered. The new list supersedes the previously downloaded list. The UEs send request to MF to request application list by providing a root URL, via the established BDC with its data channel capabilities. The DCSF provides the application list from the root URL provided by UE#1 via MF, further to UE#1 and UE#2, based on their data channel capabilities and their choices through MF.

31.	Subsequent procedures for DC application, as defined in clause AC.7.2 of TS 23.228 [2]. Depending on the application and user interaction, the UE(s) determine when to release the application initiated in alerting phase but this aspect is out of scope of specifications and does not require a new UE behaviour.

### 6.1.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

-	supporting retrieving DC application list and DC application for alerting phase; this includes providing a specific URL to get from DCSF the application list for alerting phase,

-	supporting ADC establishment in alerting phase.

IMS AS:

-	supporting negotiation of ADC in alerting phase.

Editor's note:	Impacts on IMS AS and DCSF are FFS.

DCSF:

-	providing DC application list based on the specific URL received from the UE and associated with the application list for alerting phase.

## 6.2	Solution #2: Support for NW initiated data channel application in early media session with subscription

### 6.2.1	High level principles

This solution has the following high level principles, in order to support IMS data channel application in early media session:

-	Data Channel Application in Alerting Phase (DCAP) service can be configured by the user and stored in the HSS as part of the subscription data.

-	IMS AS triggers the application data channel establishment based on UE subscription data when receives INVITE from remote UE.

-	IMS AS performs as a B2BUA to update the SDP for data channel in early session and media flow for the main session.

-	IMS AS activates the application data channel upon receiving 180 Ringing from the serving UE.

-	The application data channel may coexistent with other early session services, e.g. CAT, CRS.

### 6.2.2	Description

This is a solution for Key issue #1 " DC establishment in alerting phase for early media".

The user of UE#2 is able to subscribe to the Data Channel Application in Alerting Phase (DCAP) service, including activate (or de-activate) the service and update the settings, e.g. to change by configuration the active data channel application. The DCAP subscriber is able to refine the data channel application selection behaviour with configured rules, e.g. time, calling party's location, called party's location, the identity of the calling and called party. The DCAP service is able to select the appropriate data channel application according to the rules.

DCAP is a terminating network service, but can also have an originating network functional component. That is, DCAP can be selected on behalf of the called subscriber for presentation to the calling party, but the calling (IMS) subscriber can also subscribe to and activate the DCAP service.

### 6.2.3	Procedures

The overall procedures of data channel establishment in early media phase are illustrated in figure 6.2.3-1.

Figure 6.2.3-1: ADC establishment in early media phase

0.	The user of UE#2 subscribes Data Channel Application in Alerting Phase (DCAP) service to his/her serving IMS network. The data channel application to be initiated in alerting phase is selected by the user and configured in UE#2's subscription data.

1.	UE#1 sends a SIP INVITE targeting UE#2. The initial SDP contains offers for audio/video flow, bootstrap data channel. UE#1 indicates support for preconditions, reliable provisional responses and indication of early-session supported.

2.	Base on UE#2’s subscription data for DCAP service, the IMS AS decides to trigger the pre-configured data channel application to UE#1 in early media session. The IMS AS sends Nimsas_SessionEventControl_Notify to the selected DCSF to notify the session establishment, including the application data channel (inactive) and bootstrap data channel. The IMS AS may add the remote bootstrap data channel to UE#1 (BDC from UE#1 to the DCSF in the terminating network) if not presented.

3.	The DCSF decides whether the requested data channels are allowed and determines the control policy and media information for the application data channel and bootstrap data channels.

4.	The DCSF invokes the Nimsas_MediaControl_MediaInstruction instructing the IMS AS how to set up the application data channel and bootstrap data channels with MF both for originating and terminating side.

5.	The IMS AS instruct the MF to allocate required bootstrap data channels. The IMS AS indicates to the DC AS that the media stream of ADC is inactive.

6.	The IMS AS responds to the MediaInstruction request received in step 4.

7.	The DCSF indicate the DC AS reserving the media resource for the application data channel and MDC2 end points to the MF. The DCSF indicates to the DC AS that the media stream of ADC is inactive.8.	The DCSF responds to the Notify Request received in step 2.

9.	The IMS AS sends the INVITE to UE#2.

10.	UE#2 determines the subset of the media flows and responds with an Offer Response message back to the IMS AS.

11.	The IMS AS forwards the Offer Response message to UE#1.

12.	The IMS AS initiates an UPDATE message to UE#1, which includes the SDP for the and remote bootstrap data channel and inactive application data channel for early session. The IMS AS may add a remote bootstrap data channel with UE#1 if not request in the INVITE message.

13.	Based on UE implementation and local policy, UE#1 may alert the user that an application data channel in early media is triggered. The user can decide to accept or reject the application.

14.	If accepted by the user, UE#1 reserves the media resource for the application data channel and bootstrap data channel. UE#1 sends the200 OK, which acknowledges the UPDATE message to the IMS AS.

15.	The BDC between UE#1 and the DCSF is established as part of the SDP Offer in the early session. If the requested data channel application is not locally available, UE#1 downloads the data channel application via the established BDC and update the early session with the IMS AS.

16.	The IMS AS forwards the PRACK message to UE#2.

17-18.	UE#2 reserves resource and sends a SIP 200 (OK) response for the SIP PRACK request to UE#1. The application data channel between the DC AS and UE#1 is established and UE#1.

19.	UE alerts the user and sends Ringing indication to the IMS AS.

20.	The IMS AS decides to activate the pre-configured data channel application to UE#1.

21.	The IMS AS instructs the MF to activate the media resource for the application data channel. The IMS AS may notify the DC AS to activate the application towards UE#1.

22.	The IMS AS updates the IMS session with UE#1 to activate the application data channel in early session.

23.	UE#1 confirms the activation of the application data channel.

24.	The interaction between UE#1 and the DC AS via application data data channel in early session is started.

25.	UE#2 accepts the session and sends 200 OK to the INVITE.

26-27.	The IMS AS updates UE#1 with SDP offer to initiate the media flow of the main session and terminate the application data channel in the early session.

28.	The IMS AS instructs the MF to update the media resource and terminate the application data channel.

29-30.	The IMS AS forwards 200 OK to UE#1. UE#1 sends ACK to UE#2.

31.	The media flow of the main session between UE#1 and UE#2 starts. The requested BDCs for both UE#1 and UE#2 in the main session are established.

### 6.2.4	Impacts to Services, Entities and Interfaces

UE:

-	Indicates supporting of early media session in SIP INVITE message, as specified in existing specification, e.g. TS 24.229 [6].

-	Supports application data channel in the early session.

-	Notifies user the requested data channel application in early session.

HSS:

-	Supports management of user subscription for Data Channel Application in Alerting Phase (DCAP) service.

-	Supports the user configuration of data channel application(s) in alerting phase.

IMS AS:

-	Triggers the application data channel establishment when receiving INVITE from the originating UE.

-	Activates the established application data channel when receiving 180 Ringing from the terminating UE.

-	Terminates the application data channel when the call accepted by the terminating UE.

-	Performs as B2BUA to update the media flow in the SDP exchanged with UE.

## 6.3	Solution #3: IMS ADC establishment during alerting phase when initial INVITE does have BDC offer

### 6.3.0	High-level solution Principles

The main principle of this solution for support of application data channel establishment during alerting phase of normal IMS DC session as follows:

-	The calling party will receive bootstrap data channel offer from the terminating network during alerting phase.

-	The calling party will establish the bootstrap data channel, download the application and based on the interest, trigger application data channel establishment by selecting one particular application from the terminating network.

-	The called party after getting notification about the application running status in the calling party device, will establish bootstrap data channel, download the application and once accepts the session, the call will be established.

### 6.3.1	Description

This paper proposes solution for supporting application data channel establishment during alerting phase when calling party has offered bootstrap data channel in the initial INVITE.

In this proposal, the calling party and the called party will establish bootstrap data channel(s), download application or application list and establish application data channel while call has not yet been established.

### 6.3.2	Procedures

The procedure of establishing application data channel during alerting phase.

Figure 6.3.3-1: ADC establishment in early media phase

1.	UE-A sends an initial SIP INVITE request to the originating IMS AS through P-CSCF and S-CSCF with an initial SDP offer for audio/video and for the bootstrap data channel establishment with bootstrap DC stream ID. The SDP contains bootstrap data channel offers for both the originating and the terminating side. P-CSCF may include the P-Early-Media header field to indicate the support for early media authorization in the initial SIP INVITE request.	NOTE: The UE is assumed to support P-Early-Media.

2-13.	Steps 2-13 of clause AC.7.1 in TS 23.228 [2] apply.

14.	UE-B returns a SIP 18X response with an SDP answer for bootstrap data channel to the originating network. The terminating IMS AS includes P-Early-Media header field in the SIP 18X response indicating early media is allowed based on the SDP answer not including any application data channel.

15.	The bootstrap data channel has been established between originating MF and UE-A. The UE shall request for a data channel application or data channel applications list from the MF via the established bootstrap data channel. The DCSF provides the data channel application or the data channel application list to UE-A through MF. UE-A may download the data channel application or the data channel application list through the established bootstrap data channel..

16.	The bootstrap data channel has also been established between terminating MF and UE-A.

17.	UE-A sends a SIP PRACK request towards the terminating side before or during step 15.

18.	UE-B returns a SIP 200 OK response for the PRACK. Simultaneously, UE-B may alert the terminating user that the incoming session is a normal IMS DC session.

19.	After the DC application is downloaded, UE-A initiates a SIP UPDATE request to update the IMS session for adding the application data channel and to inform UE-B about the associated DC application binding information. The IMS AS shall notify DCSF about the media change request event and request MF to allocate media resources for the application data channels based on instructions received from the DCSF if MF is anchoring application data channel. Once acknowledgement is received from the DCSF, the IMS AS shall send the update request to the terminating side and UE-B through S-CSCF. The SIP UPDATE request with the modified SDP offer includes the negotiated bootstrap data channel media description, requested application data channel media description and the associated data channel application binding information.

The IMS AS includes P-Early-Media header field in the SIP UPDATE request to indicate that early media is allowed based on the previously received SDP answer not indicating any application data channel.

20-22.	The bootstrap data channel has been established between the originating MF and UE-B. The bootstrap data channel has also been established between the terminating MF and UE-B. UE-B may alert the terminating user that the data channel application is running in UE-A and to download the data channel application if not already available in UE-B. UE-B shall request for the data channel application and the DC application list from the MF for downloading via the established bootstrap data channels if needed.

23-29.	When the DC application is available in UE-B, then the terminating network sends a 200 OK response to the originating side based on which the IMS AS notifies DCSF of the successful bootstrap DC and application DC establishment. Once the DCSF responds to the session establishment request, 200 OK response is forwarded to the UE-A. The originating network P-CSCF executes QoS procedures for bootstrap data channel. The IMS AS may include P-Early-Media header field in the 200 OK response to the SIP UPDATE request to indicate that early media is allowed. UE-B alerts the terminating user for running the DC application.

30.	UE-B returns a SIP 180 response indicating that UE-B is ringing.

31.	Once UE-B accepts to run the DC application, UE-B answers the IMS session. UE-B sends 200 OK to the initial SIP INVITE request and subsequent procedures follow.

Editor's note:	The consideration on originating user consent of NW initiated ADC during alerting phase is FFS.

### 6.3.3	Impacts on Services, Entities and Interfaces

Editor's note:	Impacts are FFS.

## 6.4	Solution #4: IMS ADC establishment during alerting phase when initial INVITE does not have BDC offer

### 6.4.0	High-level solution Principles

The main principle of this solution for support of application data channel establishment during alerting phase of normal IMS DC session as follows:

-	The calling party will receive bootstrap data channel offer from the terminating network during alerting phase.

-	The calling party will establish the bootstrap data channel based on the interest, download the application and , trigger application data channel establishment by selecting one particular application from the terminating network.

-	The called party after getting notification about the application running status in the calling party device, will establish bootstrap data channel, download the application and once accepts the session, the call will be established.

-	It is assumed that the interactive ring back tone will not co-exist with legacy ring back tone.

### 6.4.1	Description

This clause proposes a solution for supporting application data channel establishment during alerting phase when calling party has not offered bootstrap data channel in the initial INVITE.

In this proposal, one scenario is explained how the called party (e.g. restaurant customer service) network is providing bootstrap data channel offer to the calling party during alerting phase so that the calling party can establish bootstrap data channel, download the application or application list and then session is established once called party user accepts it.

Scenario.

-	The calling party triggers a normal call to restaurant customer service with intention of reserving seats for lunch.

-	The called party has taken an interactive ringback tone service which means whenever any incoming session is received for that restaurant, the network will offer bootstrap data channel and list of applications provided by the restaurant.

-	The calling party will establish the bootstrap data channel, download the list of applications and select "seat reserve" application.

-	The called party based on the notification of running application on the calling party side will download the application if not available and then accepts the session.

### 6.4.2	Procedures

The procedure of establishing application data channel during alerting phase.

Figure 6.4.3-1: ADC establishment in early media phase

0.	The terminating user has opted for an interactive ring back tone service from its network. That is, for every incoming SIP INVITE, the network will send an SDP offer for bootstrap data channel and a list of applications towards the originating UE-A.

1-2.UE-A sends an initial SIP INVITE request to the terminating side through P-CSCF and S-CSCF with an initial SDP offer for audio/video. P-CSCF may include the P-Early-Media header field to indicate the support for early media authorization in the initial SIP INVITE request.

NOTE:	The UE is assumed to support P-Early-Media.

3.	UE-B returns a SIP 18X response with an SDP answer for audio/video session to the originating network. The terminating IMS AS includes P-Early-Media header field in the SIP 18X response indicating early media is allowed.

4-5.	UE-A sends a SIP PRACK request towards the terminating side and UE-B returns a SIP 200 OK response for the PRACK.

6.	UE B sends 180 Ringing.

6A.	Since the terminating user has subscribed to an interactive ringback tone service from its network, the terminating IMS AS triggers a SIP UPDATE towards the originating side. The SIP update contains the SDP offer for audio/video and bootstrap offer for the originating side. Steps 2-10 of clause AC.7.1 in TS 23.228 [2] apply in which the terminating IMS AS determines to notify the terminating DCSF based on DC subscription and UE-B DC capability and the DCSF instructs the IMS AS and MF to establish the resource required for the terminating side. In this scenario, the DCSF allows for application selection from the remote UE, even before the application has been selected by the peer UE.

7.	IMS AS sends the modified SIP UPDATE request including the SDP offer for audio/video and bootstrap data channel adding media information of MF to the originating IMS AS.

8.	Originating IMS AS determines to notify DCSF based on DC subscription and UE DC capability. If the network does not support DC service or UE-A does not have DC capability, then the originating network sends an SDP answer to setup only audio/video session and remove any bootstrap data channel media information.

9.	Originating IMS AS forwards the SIP UPDATE with the SDP offer for audio/video and bootstrap data channel to the originating UE-A.

10.	Based on UE implementation and local policy, UE-A may alert the user that bootstrap DC for early media has been offered and the user can accept or reject the bootstrap DC setup.

11.	If the user accepts the offer, the bootstrap DC shall be established between the terminating MF and UE-A. The UE shall request for a data channel application or data channel applications list from the MF via the established bootstrap data channel. The DCSF provides the data channel application or the data channel application list to UE-A through MF. UE-A may download the data channel application or the data channel application list through the established bootstrap data channel.

12.	Once the bootstrap data channel has been established, UE-A sends 200 OK response for the SIP UPDATE request to the terminating side.

13.	UE-B alerts the terminating user to indicate an incoming IMS session.

14.	After the DC application is downloaded, UE-A initiates a SIP UPDATE request to update the IMS session for adding the application data channel and to inform UE-B about the associated DC application binding information. The IMS AS shall notify DCSF about the media change request event. Once acknowledgement is received from the DCSF, the IMS AS shall send the update request to the terminating side and UE-B through S-CSCF. The SIP UPDATE request with the modified SDP offer includes the negotiated bootstrap data channel media description, requested application data channel media description and the associated data channel application binding information.

The IMS AS includes P-Early-Media header field in the SIP UPDATE request to indicate that early media is allowed based on the previously received SDP answer not indicating any application data channel.

15.	The bootstrap data channel has been established between the terminating MF and UE-B. UE-B may alert the terminating user that the data channel application is running in UE-A and to download the data channel application if not already available in UE-B. UE-B shall request for the data channel application and the DC application list from the MF for downloading via the established bootstrap data channels if needed.

16.		After the DC application is downloaded, UE-B alerts the terminating user for running the DC application.

17-22.	The terminating network sends a 200 OK response to the originating side based on which the IMS AS notifies DCSF of the successful bootstrap DC and application DC establishment. Once the DCSF responds to the session establishment request, 200 OK response is forwarded to the UE-A. The originating network P-CSCF executes QoS procedures for bootstrap data channel and application data channel. The IMS AS may include P-Early-Media header field in the 200 OK response to the SIP UPDATE request to indicate that early media is allowed.

23.	Application data channel has been established between UE-A and UE-B. The application data channel media in the alerting phase is exchanged between UE-A and UE-B.

24.	UE-B answers the call and sends 200 OK to the SIP INVITE. The ADC established during alerting phase is terminated after 200 OK to INVITE is sent by UE B.

25.	Subsequent procedures continue for the main session.

NOTE:	The DCSF and MF shown in the figure belongs to originating IMS network.

### 6.4.3	Impacts on Services, Entities and Interfaces

This solution may have the following impacts to existing entities and interfaces:

IMS AS:

-	Terminating IMS AS will provide a new offer for IMS bootstrap data channel based on the interactive ringback tone service subscription of called party user.-	Terminating IMS AS will notify the session establishment to the selected DCSF including the bootstrap data channel although the incoming INVITE does not have the bootstrap DC offer.

UE:

-	Supports bootstrap DC and application DC establishment during alerting phase.

-	Notifies user the incoming bootstrap DC offer during the alerting phase.

## 6.5	Solution #5: Early Media in Alerting Phase (CAT) Based on DC AS Initiated A2P DC

### 6.5.0	High-level solution Principles

This solution addresses Key Issue #1: ADC establishment in alerting phase for early media.

The solution focuses on the DC based Customized Alerting Tones (CAT) service which is subscribed by the called party and it will provide customized content to the calling party as an early media during the calling process. The same principle could be applied to both calling and called party-side subscription and providing the A2P ADC to either side of the UE. The different procedures only vary in the request that the DC AS sends to the IMS AS.

The solution has the following principles:

-	The called UE B has subscribed to the DC based CAT service.

-	The calling UE A and the network serving the called UE B have the early media capability and UE A has negotiated with the serving network regarding this capability.

-	The solution needs an enhancement of the subscribe/notify framework for event monitoring procedure to notify the DC AS the calling event which needs an A2P DC to deliver the early media

-	The solution reuses the IMS DC capability exposure procedure to establish an A2P DC which is used to transmit the early media to the UE.

-	The IMS-AS serving the called UE B notifies the calling event to the DC AS; the DC AS triggers the A2P DC establishment process to the calling UE A.

### 6.5.1	Description

This solution provides a procedure on how to establish an A2P ADC in alerting phase to provide customized DC content to the calling party as an early media.

### 6.5.2	Procedure

The pre-condition of the following procedure is that the called UE B has subscribed to the DC based CAT service and the calling UE A and the serving network of called UE B have the  early media capability and  UE A has negotiated with the serving network regarding this capability. It assumes that the DC based CAT service will not coexists with the legacy CAT service; it means the UE will subscribe only one of the CAT services.

Figure 6.5.2-1: Procedure of A2P ADC establishment in alerting phase

The steps in the procedure are as follows:

1.	UE A initiates SIP INVITE including Audio/Video/BDC SDP; step 1-13 of Figure AC.7.1-1 in TS 23.228 [2] for BDC establishment apply.

2.	BDC has been established for UE A.

3.	Based on the subscription of UE B, IMS AS-2 decides to notify the DC AS at the network serving UE B this BDC establishment event.

NOTE 1:	It assumes that the DC AS has subscribed the BDC establishment event of UE B.5.

4.	The IMS AS-2 sends the Nimsas_ImsEE_Notify Request to the NEF the calling event including the calling UE A and the called UE B information, as well as IMS Session ID.

5.	The NEF sends the Nnef_ImsEE_Notify Request to the DC AS the calling event including the corresponding information.

6.	The DC AS decides to establish A2P ADC to UE A to deliver the DC early media.

7.	UE B sends 180 Ringing to IMS AS-2.

8.	The DC AS establishes A2P ADC with UE A; and step 1-21 of Figure AG.2.1.1-1 in TS 23.228 [2] for adding ADC to an existing IMS session apply with the following differences.

-	The A2P ADC is added to an IMS session in establishment phase not an existing IMS session.

-	The SIP signal UPDATE is used instead of re-INVITE in the procedure of AG.2.1.1 in TS 23.228 [2].

-	The destination of A2P ADC is UE A rather than UE B in the terminating network.

UE A may download the application used in the alert phase.

The DC AS may send the A2P ADC addition request to the IMS AS-2 prior to the 180 Ringing. The IMS AS-2 will hold the request until it receives the 180 Ringing from the UE B.

9.	The UE B/terminating network and UE A/originating network exchange 180 Ringing, SIP signals.

10.	The A2P ADC between UE A and the DC AS which goes through the MF is activated and the early media (customized alerting tones) of this IMS session is delivered from the DC AS to UE A. The user of UE A experiences with the early media.

11.	The user of UE B picks up the phone and the UE B/terminating network and UE A/originating network exchange 200 OK (INVITE) SIP signal.

12.	The IMS AS-2 notifies the DC AS at the called network the IMS session establishment event.

13.	The DC AS releases the A2P ADC with UE A.

14.	Subsequent procedures continue.

NOTE 2:	The detailed steps for interaction between the IMS-AS and the DCSF, the IMS-AS and the MF are not illustrated in the procedure, which reuse the corresponding steps in Figure AC.7.1-1 and Figure AG.2.1.1-1.

### 6.5.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

-	supporting ADC establishment in alerting phase.

IMS AS:

-	supporting report of IMS session establishment events to DC AS;

-	supporting A2P DC establishment for delivering early media.

MF:

-	receiving early media from DC AS and providing it to UE through A2P DC.

DC AS:

-	supporting receiving IMS session establishment events from IMS AS through NEF;

-	supporting initiating and releasing A2P DC for delivering early media.

## 6.6	Solution #6: Support Alerting phase P2A ADC establishment through Autolaunch/Autoload Parameter

### 6.6.0	High-level solution Principles

This solution supports application data channel establishment during the alerting phase. This solution is based on the following high-level solution principles:

-	The bootstrap data channel media negotiation will follow the existing procedure defined in clause AC.7 of TS 23.228 [2] and that the bootstrap data channel will be established before SIP 180 message from the terminating party.

-	In order to optimize the user experience during data channel establishment in the alerting phase, the IE Autolaunch and/or Autoload defined in Table 8.2.2.3.1-2 of TS 23.392 [7] will be used by the UE to automatically launch the application that need to be used during the alerting phase. The configuration of data channel application profile is defined in TS 23.392 [7].

-	Only P2A/A2P application data channel is supported. Any P2P or P2A2P application data channel should be established after the 200 OK of initial SIP INVITE. This solution only focuses on the P2A application data channel procedures.

### 6.6.1	Description

This solution proposes to support application data channel establishment during the alerting phase to enhance specific subscription-based services. Some examples of such services include:

-	Customized Alerting Tone (CAT)

-	Customized Ringing Signal (CRS)

-	Originating Identification Presentation (OIP)

-	Terminating Identification Presentation (TIP)

NOTE 1:	The established ADC could be used either as the complement of existing services or be used to replace the existing services.

This solution supports establishment of 4 types of P2A application data channel during the alerting phase:

-	From originating network to originating subscriber. (e.g. CAT, TIP)

-	From originating network to terminating subscriber. (e.g. CRS, OIP)

-	From terminating network to originating subscriber. (e.g. CAT, TIP)

-	From terminating network to terminating subscriber. (e.g. CRS, OIP)

In the P2A (i.e. the UE initiates the application data channel),the overall procedure is as following:

1.	UE-A initiates the session. The bootstrap data channel is established before the 180 ringing.

2.	The DCSF sends the data channel application profile contained in the root application/application list. The profile contains the Autolaunch/Autoload parameter based on configuration or subscription. UE-A/UE-B automatically download and launch the application based on the Autolaunch/Autoload parameter.

3.	The application established the ADC after the 180 ringing.

NOTE 2:	If the 200 OK of the SIP INVITE is received by UE-A before the application download completion, the UE needs to continue with the call establishment and skip step 3 above. Whether the UE will stop the download immediately or complete the download after the call establishment is up to UE implementation.

### 6.6.2	Procedure

#### 6.6.2.1	Application Download Procedure

The TS 23.392 [7] defines the Autolaunch/Autoload parameter in the data channel application profile, which could be used to automatically download and launch data channel application used during the ringing phase.

NOTE 1:	The privacy and user consent issue should comply with the definition in NOTE 3 and NOTE 4 in Table 8.2.2.3.1-2 of TS 23.392 [7] in SA WG6. The security aspect should be considered in SA WG3.

Figure 6.6.2.1-1: Application Download Procedure

1.	Steps 1-14 of clause AC.7.1 of TS 23.228 [2] applies to establish IMS session and bootstrap data channel.

2.	UE-A/UE-B sends HTTP Get request to download the root application/application list from its serving network.

3.	DCSF-A/DCSF-B sends the root application/application list through the HTTP get response. The root application/application list contains the data channel application profiles, which one of the profiles may contain the Autolaunch/Autoload parameter. The configuration of data channel application profile is defined in TS 23.392 [7].

Editor’s note:	How the SA WG6 architecture of application and application profile download maps to the existing SA WG2 architecture is FFS. Whether the autolaunch parameter could be used during early session is FFS. Further collaboration with SA WG6 might be needed.

NOTE 2:	Steps 2-3 are out of scope of SA WG2 and defined in SA WG6.

NOTE 3:	It is also possible that UE-A sends HTTP Get request to the DCSF-B and UE-B sends HTTP Get request to the DCSF-A. The procedure will follow the existing specification.

4.	Optionally, UE-A/UE-B can automatically download and launch a given data channel application if it contains Autolaunch/Autoload parameter in its data channel application profile. The definition and usage of the parameter are defined in TS 23.392 [7].

NOTE 4:	How the UE authorize the download are based on the existing procedure defined in SA WG6 and UE implementation. It is also up to the UE implementation and local regulations that whether the UE need to inform the user and/or acquire consent from the user before initiating the download based on the Autolaunch/Autoload parameter.

#### 6.6.2.2	Originating-side Application Data Channel Establishment Procedure in P2A Mode

The UE-A/UE-B may initiate application data channel establishment procedure after the corresponding data channel application has been downloaded and launched. The depicted procedure starts after the UE-B sends the 180 ringing to the UE-A.

Figure 6.6.2.2-1: Originating-side Application Data Channel Establishment Procedure in P2A Mode

1.	UE-B sends 180 ringing message indicating it starts to ring.

2.	UE-A sends SIP Update to establish P2A application data channel for application downloaded with the procedure specified in Clause 6.6.2.1.

3.	Steps 2-21 of clause AC.7.2.2 of TS 23.228 [2] applies to establish P2A ADC with the difference that the SIP Update instead of SIP re-INVITE has been used.

The DCSF can optionally instruct the IMS AS to anchor the audio and/or video channel to the MF/DC AS if any additional media needs to be sent to the UE based on the subscription and type of services that requesting the ADC, e.g. play additional alerting tone alongside with the established ADC.

4.	The IMS AS forwards the 200 OK response of SIP Update from terminating UE to the UE-A.

5.	The terminating network sends 200 OK of SIP INVITE, indicating the session has been established.

6.	UE-A sends ACK.

#### 6.6.2.3	Terminating-side Application Data Channel Establishment Procedure in P2A Mode

Figure 6.6.2.3-1: Terminating-side Application Data Channel Establishment Procedure in P2A Mode

1.	UE-B sends 180 ringing indicating it starts to ring.

2.	UE-B sends SIP Update to establish P2A application data channel for application downloaded with the procedure specified in clause 6.6.2.1.

3.	Steps 2-21 in clause AC.7.2.2 of TS 23.228 [2] applies to establish P2A ADC.

The DCSF can optionally instruct the IMS AS to anchor the audio and/or video channel to the MF/DC AS if any additional media needs to be sent to the UE based on the subscription and type of services that requesting the ADC, e.g. play additional alerting tone alongside with the established ADC.

4.	The IMS AS forwards the 200 OK response of SIP Update from originating UE to the UE-B.

5.	The terminating network sends 200 OK of SIP INVITE, indicating the session has been established.

6.	UE-A sends ACK.

#### 6.6.2.4	Terminating Application Data Channel After the Call Established

Figure 6.6.2.4-1: Terminating Application Data Channel After the Call Established

1.	The terminating network sends 200 OK of SIP INVITE, indicating the session has been established.

2.	UE-A sends ACK.

3.	UE-A initiates a SIP reINVITE request to terminate the ADC based on clause AC.7.6 of TS 23.228 [2].

### 6.6.3	Impacts on Existing Nodes and Functionality

UE:

-	supporting ADC establishment in alerting phase.

-	supporting Autolaunch/Autoload parameter in the DC App profile.

IMS AS/DCSF/MF/DC AS:

-	supporting ADC establishment in alerting phase.

MF:

-	providing early media to UE through DC/audio/video based on the instruction from DC AS.

## 6.7	Solution #7: IMS DC Events Supported by DCSF

### 6.7.0	High-level solution Principles

This solution addresses Key Issue #2: enhancements to capability exposure framework for eMMTel services, especially how to support IMS DC events supported by DCSF.

The solution has the following principles:

-	The DCSF can expose some IMS DC events to NEF or AF.

-	The exposure interfaces will reuse DC3 between DCSF and NEF and DC4 between DCSF and AF.

-	The AF supports subscribing the events exposed by the DCSF and receiving the IMS DC events from the DCSF.

### 6.7.1	Description

This solution provides a procedure on how to support IMS DC events supported by DCSF.

The Table 6.7.1-1 which is the modification of Table AD.2.5.2-1 List of events for monitoring capability in clause AD.2.5.2 of TS 23.228 [2] includes the IMS DC events supported by DCSF.

Table 6.7.1-1: (modified Table AD.2.5.2-1of TS 23.228 [2]) List of events for monitoring capability (changes in RED)

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Bootstrap Data Channel established | Detected when a Bootstrap data channel is established. Network detects that a Bootstrap data channel is established when the SDP for an established IMS DC includes the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [3]. | IMS AS |
| Bootstrap Data Channel Release | Detected when an established Bootstrap data channel is released. Network detects that a Bootstrap data channel is released when an IMS DC that includes an SDP with the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [3] is released, or an IMS DC is updated to remove the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [3]. | IMS AS |
| Application Data Channel established | Detected when an Application Data Channel is established. Network detects that an Application data channel is established when the SDP for an established IMS DC includes the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [3]. The Application DC stream ID can be explicit or any stream ID. | IMS AS |
| Application Data Channel Release | Detected when an Application data channel is released. Network detects that an Application data channel is released when an IMS DC that includes an SDP with the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [3] is released, or an IMS DC is updated to remove the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [3]. | IMS AS |
| Application Data Channel requires interworking is established | Detected when an Application Data Channel that requires interworking is in the process of establishment. Network detects that an P2A Application data channel is in process of establishment and the MDC2 resources are assigned for the Application data channel. | IMS AS |
| Application operation | Detected when an application is operated. Network detects that an application is operated, e.g. downloading. | DCSF |

### 6.7.2	Procedure

The procedure for DCSF event exposure is very similar to the subscribe/notify procedure for non-subscriber specific IMS events in clause AD.3.3 of TS 23.228 [2]. The following differences apply:

-	the subscribing destination is DCSF rather than IMS AS;

Figure 6.7.2-1: Procedure for subscribe/notify of events supported by DCSF

The steps in the procedure are as follows:

1.	The AF subscribes to NEF initiating the Nnef_imsEE_Subscribe Request for non-subscriber specific IMS DC events supported by the DCSF.

2.	If the subscription to the non-subscriber specific IMS event is authorized by the NEF, the NEF locates all DCSF instances that support the requested IMS DC events via NRF or by local configuration.

3.	For each DCSF instance that supports the requested IMS DC event, NEF subscribes to the DCSF for the requested IMS event via DC3.

4.	The DCSF returns to the NEF the Subscription Response via DC3.

5.	The NEF returns to AF the Nnef_ImsEE_Subscribe Response.

6.	At some point, the requested IMS DC event is detected by the DCSF.

7.	The DCSF sends a notification to the NEF.

8.	The NEF sends Nnef_ImsEE_Notify Request to the AF.

NOTE 1:	For the trusted AF, the AF may send the subscription request to the DCSF and receive the event notification from the DCSF via DC4 directly.

NOTE 2:	For the subscribing request from AF when it is subscriber specific, the NEF may discover the DCSF from the HSS; and it is assumed that the DCSF has registered to the HSS as described in other Solution #9.

### 6.7.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DCSF:

-	supporting detecting IMS DC events which are subscribed by an AF;

-	supporting IMS DC event subscription and notification;

-	supporting registering to the HSS.

NEF:

-	supporting transferring the subscription request from AF to DCSF;

-	supporting transferring the event notification from DCSF to AF.

AF:

-	supporting subscription and receiving IMS DC events detected by DCSF.

HSS:

-	supporting registration and discovery of the DCSF.

## 6.8	Solution #8: Subscription and notification of DC application downloading events

### 6.8.0	High-level solution principles

This solution targets KI2.

This solution enables an AF (a DC AS) to subscribe to and being notified about events related to the downloading of DC applications. Four types of events are supported: these events can be related to specific or any UE(s), as well as independent or dependent on application(s) (see clause 6.8.1). Subscription requests optionally include filter information to control the granularity and frequency of notifications. inducing usage of NWDAF to enforce statistics or predictions on the notifications related with application download. The AF (DC AS) can use the corresponding notifications to assess the popularity of applications (especially P2P applications).

Once the IMS AS has received a subscription request from the AF as defined in Annex AD.3 of TS 23.228 [2] and when a BDC is established, the IMS AS, based on the type of the DC application downloading events (see clause 6.8.1), provides the subscription to the selected DCSF handling the BDC (this assumes the definition of an IMS AS event requiring the IMS AS is to forward to the selected DCSF). The solution does not assume a 1-1 mapping between IMS AS and DCSF. The selected DCSF is responsible for detecting these events and reports the relevant information directly to the AF/DC AS (or to the NEF).

### 6.8.1	Types of DC application downloading events

To support enhanced exposure capabilities, the IMS DC exposure framework provides mechanism for a DC AS to subscribe to events related to the downloading of DC applications. Four types of DC application downloading events are supported:

-	Type 1: Subscriber-specific and DC application-specific event: The subscription request relates to one or more UEs (identified by their IMPUs) and one or more DC application identifiers. A notification shall be triggered only when one of the UE(s) downloads one of the DC applications.

-	Type 2: Subscriber-specific and DC application-agnostic event: The subscription request relates to one or more UEs, without indicating any specific DC application identifier. The DC AS identifier shall be included in the subscription request. A notification shall be triggered when one of the specified UE(s) download any DC application associated with the subscribing DC AS. Notifications for DC applications associated with other DC ASs shall not be provided.

-	Type 3: Subscriber-agnostic and DC application-specific event: The subscription request relates to one or more DC application identifiers, without specifying any UE. A notification shall be triggered when a UE downloads the specified DC application(s).

-	Type 4: Subscriber-agnostic and DC application-agnostic event: The subscription request shall not specify any UE or DC application identifier. The DC AS identifier shall be included in the subscription request. A notification shall be triggered when a UE downloads any DC application associated with the subscribing DC AS. Notifications for DC applications associated with other DC ASs shall not be provided.

NOTE 1:	It is assumed that the DCSF can determine that a DC application is associated with a DC AS either via configuration or via proper application naming convention.

Subscription requests optionally include filter information to enable the DC AS to specify criteria for event reporting, such as periodic notifications or reporting of event statics to prevent excessive signalling for individual events. The IMS network may negotiate the event filter parameters with the DC AS to ensure alignment with network capabilities, resource availability and operational policies.

The event subscription provided to the DCSF contains the AF Id of the DC AS allowing the DSCF to control whether to send a notification related to a given application to the DC AS.

NOTE 2:	This is to ensure that a DC AS can receive notifications only about "its own applications".

### 6.8.2	Procedure of DC application downloading event subscription and notification

Figure 6.8.2-1 depicts a signalling flow diagram for DC application downloading event subscription and notification.

Figure 6.8.2-1: Procedure of DC application downloading event subscription and notification

1.	The AF (DC AS) sends a subscription request (see clause 6.8.1) to the DC application download event(s) to IMS AS via the NEF. For Type 1 or Type 2 event targeting specific UE(s), clause AD.3.2 of TS 23.228 [2] is followed. For Type 3 or Type 4 event targeting any UE, clause AD.3.3 of TS 23.228 [2] is followed.

The IMS AS instance(s) locally store the subscription request(either globally for subscription targeting Any UE or in association with the IMS registration for subscription targeting an individual UE). The IMS AS needs to understand the event type and the targeted UE(s), but does not need to understand other information about this event in the subscription request.

NOTE 1:	This corresponds to a new IMS AS event requiring the IMS AS to forward the corresponding event subscription to the selected DCSF.

2.	The UE requests BDC establishment as defined in clause AC.7.1 of TS 23.228 [2]. During this process, the IMS AS selects a DCSF.

3.	The IMS AS on behalf of the NEF/DC AS forwards the event subscription request received in step 1 to the selected DCSF for: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc.) it provides the DC AS / NEF subscription request to the DCSF.

4.	The DCSF responds to the IMS AS, indicating successful subscription.

5.	The UE downloads a DC application via the established BDC.

6.	The DCSF checks whether the downloaded DC application is associated with the DC AS having issued the subscription request and whether the Event is authorized for this UE. If it does not, the event shall not be notified.

NOTE 2:	This is to ensure that a DC AS can receive notifications only about "its own applications". To be able to detect whether the downloaded DC application is associated with the DC AS having issued the subscription request either the DSCF can be configured with the mapping between DC applications and the AF ID associated with the DC AS or the application Id (negotiated with the DC AS) contains a reference to this AF ID,

7-8.	The DCSF notifies the NEF of the DC application downloading event. The NEF responds to the notification.

9-10.	The NEF notifies the DC AS of the DC application downloading event. The DC AS responds to the notification.

### 6.8.3	Procedure for reporting DC application usage event analytics

Figure 6.8.3-1 depicts a signalling flow diagram for reporting of DC application usage event analytics.

The AF / DC AS requests for analytics of DC application usage (e.g. download) events providing criteria for analytics reporting, e.g. reporting the number of DC app A downloading within the next 10 days or per period of 10.

If no analytics filter is provided, the procedure in clause 6.8.2 applies.

NOTE 1: 	Analytics of DC application downloading events allows providing AF / DC AS with information about DC application usage without burdening AF / DC AS each time there is an application download.

Figure 6.8.3-1: Procedure for analytics related with DC application usage event subscription

1.	The AF (DC AS) sends a subscription request (Nnef_AnalyticsExposure_Subscribe) for the DC application download event(s) (see clause 6.8.1) to the NEF, including a Notification Target Address (+ Notification Correlation ID) for this subscription request. The difference with similar step in clause 6.8.2 is that filter information specifying criteria for analytics event reporting, is provided as part of the subscription request. Target of Analytic Reporting may indicate an individual UE or any UE.

2.	NEF creates a Notification Target Address (+ Notification Correlation ID) for the received subscription request. The NEF selects a NWDAF (that supports the newly defined analytics related with DC application usage event subscription) and sends the subscription request to the NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe including the Notification Target Address (+ Notification Correlation ID).

3.	NWDAF creates a NWDAF Notification Target Address (+ Notification Correlation ID) for receiving the DC application downloading related event report. NWDAF responds to the NEF including the NWDAF Notification Target Address (+ Notification Correlation ID).

4.	NEF subscribes to IMS network for the event described in the subscription request and includes the NWDAF Notification Target Address (+ Notification Correlation ID) received in step 3 in the subscription request. The way of subscription follows clauses AD.3.2 and AD.3.3 of TS 23.228 [2], as described in step 1 of clause 6.8.2. This subscription is on behalf of the NWDAF.

5.	Event detection by DCSF as defined steps 2-6 in clause 6.8.2.

6-7.	The DCSF reports the detected event to the NWDAF using the NWDAF Notification Target Address (+ Notification Correlation ID) received in step 4. The NWDAF responds to the report.

8-9.	NWDAF aggregates the events based on the Notification Correlation ID contained in the messages from step 6 and generates the analytics result. NWDAF notifies NEF about the analytics result using Notification Target Address (+ Notification Correlation ID) received in step 2 via Nnwdaf_AnalyticsSubscription_Notify. The NEF responds to the notification.

10-11.	The NEF determines the DC AS based on the Notification Correlation ID received in step 1 and notifies the DC AS about the analytics result via Nnef_AnalyticsExposure_Notify. The DC AS responds to the notification.

As an alternative described in Figure 6.8.3-2, the NWDAF may directly subscribes to the IMS network.

NOTE 2:	Only One of alternative will be specified: either the alternative described in Figure 6.8.3-1 or the alternative described in Figure 6.8.3-2 where the NWDAF directly subscribes to the IMS network.

Figure 6.8.3-2: Procedure for analytics related with DC application usage event subscription (NWDAF directly subscribes to the IMS network)

-	Steps 1 and 2 are the same as in figure 6.8.3-1.

-	Step 3: The NWDAF subscribes to the IMS network for the event described in the subscription request and includes the NWDAF Notification Target Address (+ Notification Correlation ID). The way of subscription follows clauses AD.3.2 and AD.3.3 of TS 23.228 [2], but where the NWDAF initiates the subscription instead of the NEF.

-	Step 4: The NWDAF answers to the subscription request and the answer is provided by the NEF to the DC AS.

-	The rest of the steps are as in figure 6.8.3-1.

### 6.8.4	Impacts on services, entities and interfaces

The variant with DC application usage event analytics assumes a deployment where the IMS network can access to a NWDAF; this means the definition of a new Reference point in TS 23.228 [2].

DC AS:

-	Requesting new events on about DC application downloading.

-	Providing analytics filter information about the DC application downloading event for procedure of DC application downloading event analytics in clause 6.8.3.

NEF:

-	Definition of new event about DC application downloading.

IMS AS:

-	Forwarding of new event subscription request to DCSF based on event type and target UE(s).

-	Definition of an IMS AS event requiring the IMS AS to forward the corresponding subscription to the selected DCSF.

DSCF:

-	Receiving new event subscription request.

-	checking the association between DC application and DC AS.

-	Notifying of the event to the consumer (DC AS, or to NWDAF for DC application downloading event analytics).

NOTE:	For DC application downloading event analytics, the DCSF interfaces NWDAF when sending notifications to NWDAF but based on SBA principles, the DCSF does not need to be aware which entity it is sending a notification to.

NWDAF (DC application usage event analytics):

-	Generating analytics about DC application downloading events based on the filter information and the received event reports.

-	(if Figure 6.8.3-2 applies) subscription following clauses AD.3.2 and AD.3.3 of TS 23.228 [2], where the NWDAF initiates the subscription instead of the NEF.

## 6.9	Solution #9: Capability exposure of IMS DC events supported by DCSF

### 6.9.0	High-level solution Principles

This solution addresses Key Issue #2: Capability exposure of IMS DC events supported by DCSF.

The main principle of this solution for support capability exposure of IMS DC events supported by DCSF as follows:

-	The trusted AF or untrusted AF via NEF shall discover and select the DCSF.

-	To discover DCSF address, DC AS subscribe with HSS and HSS notifies once DCSF register its address in HSS.

-	Similar to third party registration done by S-CSCF to IMS AS, IMS AS does third party registration with DCSF and then DCSF register its address in HSS.

-	Upon successful discovery and selection, DCSF subscribes for the establishment of supported events with DCSF and get notified when DCSF detects the events.

NOTE:	This solution involves DCSF during registration of IMPU while it was invoked during session earlier. The implications of this approach will not be discussed further in this solution.

### 6.9.1	Description

This solution proposes how the trusted AF and untrusted AF via NEF discovers the DCSF and subscribes to get notification for the supported events by DCSF.

Discovery of DCSF:

The trusted AF can directly discover the DCSF with the help of HSS and in case of untrusted AF, the discovery happens through NEF. For this purpose, HSS can expose one service operation using which DC AS/NEF will subscribe and get notification about the DCSF address for the UE.

When UE registers with IMS network, S-CSCF does third party registration with IMS AS and then IMS AS registers its address with HSS. Similarly, this solution proposed IMS AS does third party registration with DCSF and then DCSF registers its address with HSS. When HSS receives the subscription request for DCSF address from NEF or trusted AF and the DCSF address is not present in HSS (i.e. in the case when the IMPU is not yet registered or de-registered) then HSS stores the subscription request. Once HSS comes to know the DCSF address (i.e. when IMPU does IMS registration and DCSF register its address to HSS), HSS notifies the DCSF address with NEF/AF.

Subscription and Notification with DCSF:

Upon successful discovery, AF/NEF shall subscribe with DCSF for the supported events and get notified when DCSF detects the event.

Table 6.9.1-1 List of events supported by DCSF

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Successful Application download to UE | Detected when an Application Data Channel is established. DCSF detects that UE has been successfully downloaded one specific application | DCSF |

NOTE:	This solution identifies changes to existing specifications, these changes are shown using RED text.

### 6.9.2	Service definition

#### 6.9.2.1	General

The following table shows the required service operation.

##### 6.9.2.1.1	HSS Service

The following table shows the services exposed by an SBI capable HSS.

Table 6.9.2.1.1: (Updated Table AA.2.1.1-1 from TS 23.228 [2]): IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
|  | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| ImsSubscriber Data | Subscribe | Subscribe/Notify | S-CSCF, AS |
| Management (_ImsSDM) | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
|  | Registration | Request/Response | S-CSCF, IMS AS, DCSF |
|  | DeregistrationNotification | Subscribe/Notify | S-CSCF |
|  | Deregistration | Request/Response | S-CSCF, IMS AS |
| Ims UE Context | Authorize | Request/Response | I-CSCF |
| Management (_ImsUECM) | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
|  | AsInfoGet | Request/Response | NEF |
|  | DcsfInfoGet | Subscribe/Notify | NEF, AF |
|  | DcsfInfoUnsubscribe | Subscribe/Notify | NEF, AF |
|  | DcsfInfoNotify | Subscribe/Notify | NEF, AF |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |

6.9.2.1.1.0	Nhss_ImsUECM_Registration service operation (modified)

Clause AA.2.1.2.1 of TS 23.228 [2] is modified to:

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

If the DCSF is the consumer, this service operation registers the DCSF instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name and Registration Type (e.g. Initial Registration, Unregistered) if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer, DCSF instance Id if the DCSF is the consumer.

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

6.9.2.1.1.1	Nhss_ImsUECM_Deregistration service operation (modified)

Service operation name: Nhss_ImsUECM_Deregistration

Description: This service operation deregisters the S-CSCF allocated to a public identity if the consumer is S-CSCF and deregisters the IMS AS instance assigned to the public identity if the consumer is IMS AS.

Inputs, Required: S-CSCF name if the S-CSCSF is the consumer, IMS AS instance ID if the IMS AS is the consumer, DCSF instance Id if the DCSF is the consumer, Deregistration Type.

Inputs, Optional: User Identity (Private Identity and/or Public Identity), P-CSCF Restoration indication, Session Priority.

Outputs, Required: Result indication.

Outputs, Optional: None.

6.9.2.1.1.2	Nhss_ImsUECM_DcsfInfoGet service operation

Service operation name: Nhss_ImsUECM_DcsfInfoGet

Description: Service consumer like NEF or DC AS subscribes with HSS to receive the DCSF address assigned for the UE. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the DCSF address is removed from HSS (IMPU is de-registered) 2) DCSF address is added to HSS (IMPU is registered) 3) DCSF address is updated.

Inputs, Required: Public Identity.

Inputs, Optional: Private Identity.

Outputs, Required: Result.

Outputs, Optional: None.

6.9.2.1.1.3	Nhss_ImsUECM_DcsfUnsubscribe

Service operation name: Nhss_ImsUECM_DcsfUnsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.9.2.1.1.4	Nhss_ImsUECM_DcsfNotify

Service operation name: Nhss_ImsUECM_DcsfNotify

Description: This service operation enables the HSS to notify the consumer with the updated DCSF address where there is a change of DCSF happens for the UE.

Inputs, Required: DCSF address.

Inputs, Optional: IMPU, IMPI.

Outputs, Required: Result.

Outputs, Optional: None.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.9.2.1.2	DCSF Service

6.9.2.1.2.1	Ndcsf_SessionEvent

6.9.2.1.2.1.1	General

Service description: This service enables the consumer to subscribe with the supported events by DCSF and get notified when the event is detected.

The following operations are available for this service:

-	Subscribing and unsubscribing with events supported by DCSF.

-	Notifying after event is detected to consumers, e.g. NEF, AF.

6.9.2.1.2.1.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Notification Target address, Event ID

Only one Event ID is currently defined: successful download of specific application by UE.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application ID

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (otherwise an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

6.9.2.1.2.1.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.9.2.1.2.1.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, DC Application ID.

Inputs, Optional: Calling party and called party ID.

Outputs, Required: Result.

### 6.9.3	Procedure

#### 6.9.3.1	DCSF instance registration in HSS

Figure 6.9.3.1-1depicts a signalling flow diagram for an DCSF registering in HSS the DCSF instance assigned to a registering UE.

Figure 6.9.3.1-1: DCSF Instance Registration in HSS

1.	UE performs initial IMS Registration.

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS AS performs third party Registration with the DCSF instance assigned to the registering UE. This is over http and not SIP.

4.	DCSF assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

NOTE:	Figure 6.9.3.1-1 shows only SBA services. Corresponding Sh procedure is not included in the Figure.

#### 6.9.3.2	DCSF address discovery by AF

Figure 6.9.3.2-1 depicts a signalling flow diagram for discovering DCSF address by AF.

.

Figure 6.9.3.2-1: DCSF address discovery by DC AS

1.	DCSF address is already registered as per Fig 6.9.3.1-1 solution.

2.	AF discovers DCSF by sending Nnef_ImsUECM_DcscfInfoGet service operation to NEF.

3.	NEF discovers DCSF by sending Nhss_ImsUECM_DcscfInfoGet service operation to HSS

4.	HSS provides DCSF address in Nhss_ImsUECM_DcscfInfoGet response service operation to NEF.

5.	NEF provides DCSF address in Nnef_ImsUECM_DcscfInfoGet response service operation to AF.

#### 6.9.3.3	Subscription for IMS DC events supported by DCSF

Figure 6.9.3.3-1depicts a signalling flow diagram for subscriptions for IMS DC events supported by DCSF.

Figure 6.9.3.2-1: DCSF address discovery by DC AS

1.	AF subscribes with NEF using Nnef_ImsEE_Subscribe request.

2.	NEF provides Nnef_ImsEE_Subscribe Response.

3.	Based on the requested events, NEF decides that the request need to be sent to DCSF. It is assumed that NEF has discovered DCSF based on clause 6.9.2. NEF then sends the Ndcsf_ImsEE_Subscribe request with IMPU and Application ID, event ID as input.

4.	DCSF provides Ndcsf_ImsEE_Subscribe Response.

5.	DCSF detected that the application which was subscribed by NEF has been downloaded by the UE.

4.	DCSF sends Ndcsf_ImsEE_Notify to NEF.

7.	NEF sends Nnef_ImsEE_Notify to AF.

### 6.9.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

AF/NEF:

-	subscribing with DCSF for specific application download by UE event and get notified when the event is detected by DCSF.

-	subscribing with HSS to discover the DCSF address.

DCSF:

-	receiving third party registration from IMS AS.

-	registering DCSF address in HSS.

-	receiving subscription for specific successful application download event from NEF or AF and notifying when UE successfully downloads the application.

HSS:

-	receiving subscription from AF/NEF for discovering DCSF address.

-	notify with DCSF address once DCSF register its address to HSS.

IMS AS:

-	IMS AS does third party registration with DCSF over http (not SIP).

## 6.10	Solution #10: DC3/DC4 Enhancement to Support Subscription, Notification and Instruction for P2A and P2A2P DC establishment

### 6.10.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints. This solution enhances the current P2A and P2A2P ADC establishment procedure defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2].

### 6.10.1	Description

This solution requires the DC AS to subscribe to the call events from DCSF so that DC AS can give media instruction after receiving the notification from DCSF. This solution defines two services to enable DC AS to subscribe to the DCSF and to instruct DCSF.

This solution also enhances and unifies the timing of notifying DC AS in the current P2A and P2A2P ADC establishment procedure defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2].

### 6.10.2	Service definition

#### 6.10.2.1	General

The following table shows the DCSF Services defined.

Table 6.10.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
|  | Subscribe | Subscribe/Notify | NEF, DC AS |
| Ndcsf_EventExposure | Unsubscribe | Subscribe/Notify | NEF, DC AS |
|  | Notify | Subscribe/Notify | NEF, DC AS |
| Ndcsf_MediaControl | MediaInstuction | Request/Response | NEF, DC AS |

#### 6.10.2.2	Ndcsf_EventExposure

##### 6.10.2.2.1	General

Service description: This service enables the consumer to subscribe and to be notified when the given event is detected.

##### 6.10.2.2.2	Ndcsf_EventExposure_Subscribe

Service operation name: Ndcsf_EventExposure_Subscribe

Description: This service operation enables consumers to subscribe to specific events.

Inputs, Required: Notification Target address, Event ID

Three Event ID is currently defined: application data channel establishment request, application data channel establishment success, application data channel establishment failure.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the notification will be sent to the consumer for all users.

When no application binding information is given, the notification will be sent to the consumer for all IMS DC applications.

Outputs, Required: Result indication, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.10.2.2.3	Ndcsf_EventExposure_Unsubscribe

Service operation name: Ndcsf_EventExposure_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the subscription.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 6.10.2.2.4	Ndcsf_EventExposure_Notify

Service operation name: Ndcsf_EventExposure_Notify

Description: This service operation enables the DCSF to notify the consumer about the subscribed event.

Inputs, Required: Session ID, Event ID

Inputs, Optional: User IDs of calling party and called party, application binding information.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.10.2.3	Ndcsf_MediaControl

##### 6.10.2.3.1	General

Service description: This service enables the consumer to give media instruction to DCSF. In this release, the instruction is only used to establish P2A and P2A2P data channel.

##### 6.10.2.3.2	Ndcsf_MediaControl_MediaInstruction

Service operation name: Ndcsf_MediaControl_MediaInstruction

Description: This service operation enables the consumer to provide instruction to the DCSF.

Inputs, Required: Session ID, Media information of MDC2 interface.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

### 6.10.3	Procedure

#### 6.10.3.1	Service Subscription Procedure

Figure 6.10.3.1-1: service subscription procedure

1.	DC AS sends Ndcsf_EventExposure_Subscribe to DCSF to subscribe to specific ADC establishment event. The DC AS can find the DCSF using NRF or configuration.

If the subscription request is non-subscriber specific, the DC AS will send this request to all DCSF. If the subscription request is subscriber specific, the DC AS will send this request to any DCSF. The DCSF will write the subscription request in the DCSF repository data in the HSS using Nhss_imsSDM_Update service, which will be retrieved by DCSF during the data channel establishment procedure.

2.	DCSF sends 200 OK.

#### 6.10.3.2	P2A Application Data Channel Establishment Procedure

Figure 6.10.3.2-1 depicts the P2A application data channel establishment procedure.

Figure 6.10.3.2-1: P2A ADC establishment procedure

1.	Steps 0-3 of clause AC.7.2.2 of TS 23.228 [2] applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the subscription from DC AS.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS sends Ndcsf_MediaControl_MediaInstruction to instruct the DCSF about the MDC2 related media information.

6.	The DCSF sends 200 OK indicating it receives the instruction.

NOTE:	It is up to operator's configuration and implementation that how the DCSF knows that the DC AS will send instruction to the DCSF. A possible implementation is that the DCSF will continue the following steps after a fix timer is expired.

7.	Steps 11-20 of clause AC.7.2.2 of TS 23.228 [2] applies.

8.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS.

9.	The DC AS sends 200 OK indicating it receives the notification.

10.	Steps 21-27 of clause AC.7.2.2 of TS 23.228 [2] applies.

#### 6.10.3.3	P2A2P Application Data Channel Establishment Procedure

Figure 6.10.3.3-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.10.3.3-1: P2A2P Application Data Channel Establishment Procedure.

1.	Steps 0-3 of clause AC.7.2.3 of TS 23.228 [2] applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the subscription from DC AS.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS detects that this is a P2A2P ADC so it allocates two sets of MDC2 endpoints. The DC AS sends Ndcsf_MediaControl_MediaInstruction to instruct the DCSF about the MDC2 related media information.

6.	The DCSF sends 200 OK indicating it receives the instruction.

NOTE:	It is up to operator's configuration and implementation that how the DCSF knows that the DC AS will send instruction to the DCSF. A possible implementation is that the DCSF will continue the following steps after a fix timer is expired.

7.	Steps 6-8 of clause AC.7.2.3 of TS 23.228 [2] applies. The DCSF shall send all the MDC2 media related information received from the DC AS to the MF. In this case, the MF receives two sets of MDC2 endpoints.

8.	Steps 10-18 of clause AC.7.2.3 of TS 23.228 [2] applies. The DCSF will instruct the MF via IMS AS to update the media related information after receiving SDP answer from the terminating side.

9.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS. In the P2A2P scenario, two sets of endpoint information shall be sent to the DC AS.

10.	The DC AS sends 200 OK indicating it receives the notification.

11.	Steps 19-26 of clause AC.7.2.3 of TS 23.228 [2] applies.

### 6.10.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	sending subscription request to the DCSF.

-	receiving notification from the DCSF which may include MDC2 related media information.

-	sending DC AS side MDC2 related media information to DCSF.

DCSF:

-	receiving subscription from DC AS.

-	writing subscription information of subscriber-specific subscription request in the DCSF repository data in the HSS.

-	notifying the session events to the DC AS.

-	sending MF side MDC2 related media information to DC AS.

-	receiving DC AS side MDC2 related media information from DC AS.

## 6.11	Solution #11: DC3/DC4 based on DCSF services

### 6.11.0	High-level solution principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints.

The DC AS can subscribe to events related with MDC2 channels, targeting specific UE(s) or any targeting any UE. The subscription can be either implicitly or explicitly established. For an implicitly establishment, the subscription is configured at the DCSF. For an explicitly established, the subscription request is delivered to IMS AS by following the procedure defined in clause AD.3.2 of TS 23.228 [2] or in clause AD.3.3 of TS 23.228 [2]. Once, the UE(s) initiates a P2A/P2A2P ADC establishment, the IMS AS selects a DCSF instance for handling the ADC as defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2] and forwards the DC AS subscription information to the selected DCSF instance. The subscription at DCSF lasts only as long as the corresponding end user session.

The solution does not assume a 1-1 mapping between IMS AS and DCSF.

Upon receipt of a P2A/P2A2P ADC establishment request meeting the subscription filter conditions, the DCSF invokes a new Ndcsf_SessionEvent_Notify to notify the DC AS of the event that MDC2 needs to be established. This notification includes MF-side information required for MDC2 configuration, a Resource URL where the DCSF expects further instructions from the DC AS, and security information enabling security between UE/MF and DC AS.

Based on the received notification from the DCSF, the DC AS derives the DC AS-side MDC2 configuration information, DC AS's certificate fingerprint for security between UE/MF and DC AS, and optionally QoS requirement for the DC application. and the DC AS transfers this information to the DCSF via a new Ndcsf_ADCControl_Create operation towards the Resource URL received in Ndcsf_SessionEvent_Notify.

### 6.11.1	Description

This solution specifies the DCSF services exposed over the DC3/DC4 interface to support UE-initiated P2A and P2A2P ADC establishment. Specifically, the following services are defined: Ndcsf_SessionEvent and Ndcsf_ADCControl. The associated service parameters and corresponding procedures are defined in clause 6.11.2.

The DC AS subscribes to (or is subscribed to) events notifications related with MDC2 channel establishment need . Such an event subscription can be explicitly initiated by the DC AS or implicitly configured in the DCSF. Events can be UE related (i.e. targeting specific UE(s)) or UE unrelated (i.e. targeting any UE) and associated with filter conditions such as P2A/P2A2P ADC associated with specific applications.

Upon receipt of a P2A or P2A2P ADC establishment request meeting the subscription filter conditions, the DCSF invokes Ndcsf_SessionEvent_Notify to notify the DC AS of the MDC2 channel establishment event. This notification includes MF-side information and security information (e.g. UE's certificate fingerprint) for security between UE/MF and DC AS required for MDC2 configuration. Based on the notification received, the DC AS derives the DC AS-side MDC2 configuration information and security information (e.g. DC AS's certificate fingerprint) for security between UE/MF and DC AS, optionally QoS requirement for the DC application. DCAS transfers this information to the DCSF via the Ndcsf_ADCControl_Create service operation. The DCSF subsequently proceeds with the MDC2 configuration based on the information received from the DC AS.

This enhancement to the DC3/DC4 interface is crucial for ensuring consistent and up-to-date MDC2 media resources between the DCSF and the DC AS. The DC AS and the MF may dynamically deploy/insert MDC2 terminations and thus use the dynamically determined IP address. The DC port and/or DC SCTP port may be dynamically associated to the session by the MF and/or the DC AS. Furthermore, the network (DCSF) may need to provide the DC AS with the UE certificate fingerprint received from the UE over IMS signalling.

The current P2A/P2A2P data channel procedures lack a mechanism for MDC2 media resource negotiation, leading to potential inconsistencies of MDC2 media resource between the two MDC2 endpoints. This proposed enhancement, by enabling the MDC2 media negotiation and dynamic configuration, significantly improves the robustness and flexibility of these procedures, preventing failures due to outdated or missing MDC2 media resource information.

### 6.11.2	Service definition

#### 6.11.2.1	General

The following table shows the DCSF Services and DCSF Service Operations.

Table 6.11.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
|  | Subscribe | Subscribe/Notify | IMS AS |
| Ndcsf_SessionEvent | Unsubscribe | Subscribe/Notify | IMS AS |
|  | Notify | Subscribe/Notify | NEF, Trusted AF/DC AS |
|  | Create | Request/Response | NEF, Trusted AF/DC AS |
| Ndcsf_ADCControl | Update | Request/Response | NEF, Trusted AF/DC AS |
|  | Delete | Request/Response | NEF, Trusted AF/DC AS |

NOTE:	The security information transferred between DCSF and DC AS for security between UE/MF and DC AS will be aligned with SA WG3.

#### 6.11.2.2	Ndcsf_SessionEvent

##### 6.11.2.2.1	General

Service description: This service enables the consumer to subscribe to and to be notified about events related with MDC2 channels.

The following operations are available for this service:

-	Subscribing and unsubscribing to receive event related with MDC2 channels.

-	Notifying the event to consumers, e.g. NEF, trusted DC AS.

##### 6.11.2.2.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event related with MDC2 channels (in this release only event is defined “the establishment of a MDC2 channel is needed”).

Inputs, Required: Notification Target address, Event ID

NOTE: 	The Notification Target address contains the Notification target address of the NEF or of the subscribing trusted DC AS.

Only one Event Id is currently defined: establishment of a MDC2 channel is needed.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (as an alternative, to be decided by stage 3, an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.11.2.2.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from an event .

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.11.2.2.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), Resource URL(for any Ndcsf_ADCControl operation that would correspond to the same session), DC application ID

One or two MF-side MDC2 addressing information are provided depending on whether the session is established in P2A or P2A2P mode.

Inputs, Optional: Calling party and called party IDs, application ID, security Setup if UE initiates the DTLS negotiation, certificate fingerprint of UE for security between UE/MF and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.11.2.3	Ndcsf_ADCControl

##### 6.11.2.3.1	General

Service description: This service enables the consumer to create, update or terminate an ADC.

##### 6.11.2.3.2	Ndcsf_ADCControl_Create

Service operation name: Ndcsf_ADCControl_Create

Description: This service operation enables the consumer to create information used by the DCSF for ADC establishment in an IMS session.

Inputs, Required: Session ID, DC AS-side MDC2 endpoint information (e.g. IP address, port number),.

One or two MF-side MDC2 addressing information are provided depending on whether the session is established in P2A or P2A2P mode.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation, certificate fingerprint of DC AS for  security between UE/MF and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode, QoS requirement for the DC application.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.11.2.3.4	Ndcsf_ADCControl_Delete

Service operation name: Ndcsf_ADCControl_Delete

Description: This service operation enables the consumer to provide information used by the DCSF for ADC termination in an IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.11.2.3.4	Nimsas_MediaControl_MediaInstruction

Nimsas_MediaControl_MediaInstruction as defined in clause AA.2.4.3.2 of TS 23.228 [2] is updated to add optional QoS information in the Media instruction.

### 6.11.3	Procedure

#### 6.11.3.1	Person-to-Application (P2A) Application Data Channel Setup

Figure 6.11.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.11.3.1-1: P2A ADC establishment with a trusted DC AS

0.	A subscription of the DC AS about the need for establishment of a MDC2 channel is created; this subscription is either implicitly configured at the DCSF (e.g. based on the DC Application), or explicitly provided to the IMS AS following the exposure framework defined in Annex AD of TS 23.228 [2]

1.	IMS session and BDC are established. The IMS AS selects a DCSF as specified in Figure AC.7.1-1 step 2 of TS 23.228 [2]

2-9.	UE#1 requests the establishment of P2A ADC establishment. The IMS AS forwards the DC AS subscription request to the selected DCSF instance as part of a modified step 3 of Figure AC.7.2.1-1 of TS 23.228 [2]: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc.), the IMS AS provides the DC AS subscription request to the DCSF. The DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating media resources.

NOTE 1:	The IMS AS does not need to know whether the DC is requested for P2A ADC establishment. It sends the subscription via Nimsas_SessionEventControl_Notify and it is up to DCSF to determine whether the subscription applies (as the DC is requested for P2A ADC establishment)

10.	The DCSF notifies DC AS about the need to establish a MDC2 channel (indicating the session is established in P2A mode) using Ndcsf_SessionEvent_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL to be used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE. The DC AS responds to (acknowledges) the notification message.

11.	The DCSF waits for ADC control instruction (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

12.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message with the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info and may include the certificate fingerprint of DC AS and possibly QoS requirement for the DC application.

NOTE 2:	Work on the handling of security information like certificate fingerprints is to be co-ordinated with SA WG3.

13.	The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.

14-17.	The DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information and the received QoS requirement. This follows already specified interactions (steps 11 to 14 of Figure AC.7.2.2-1 of TS 23.228 [2]) adding QoS requirements to Nimsas_MediaControl_MediaInstruction request if QoS requirements have been received in step 12. Then, the IMS AS sends response to DCSF.

18-29.	IMS AS performs SDP negotiation with terminating network. With a successful SDP negotiation, the application data channel is established. This follows already specified interactions with the addition that the IMS AS when new QoS requirements are provided in Nimsas_MediaControl_MediaInstruction provides them in sip/sdp signalling towards the UEs.

#### 6.11.3.2	Person-to-Application-to-Person (P2A2P) Procedure

Figure 6.11.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application to person use case.

Figure 6.11.3.2-1: P2A2P ADC establishment with a DC AS

0.	As in step 0 of Figure 6.11.3.1-1.

1.	As in step 1 of Figure 6.11.3.1-1.

2-9.	UE#1 requests the establishment of P2A2P ADC establishment. The IMS AS forwards the DC AS subscription request to the selected DCSF instance: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc…), the IMS AS provides the DC AS subscription request to the DCSF.

NOTE 1:	The IMS AS does not need to know whether the DC is requested for P2A2P ADC establishment. It sends the subscription via Nimsas_SessionEventControl_Notify and it is up to DCSF to determine whether the subscription applies (as the DC is requested for P2A2P ADC establishment).

The DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating and terminating media resources.

10.	DCSF notifies DC AS about the session establishment (indicating the session is established in P2A2P mode) using Ndcsf_SessionEvent_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE. DC AS responds to (acknowledges) the notification message.

11.	DCSF waits for ADC control instructions (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

12.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message to the DCSF with the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info and may include the certificate fingerprint of DC AS and possibly QoS requirement for the DC application.

NOTE 2:	Handling of security information like certificate fingerprints is to be co-ordinated with SA WG3.

13.	The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.

14-21.	IMS AS performs SDP negotiation with terminating network. And the IMS AS notifies the DCSF about the successful result of the SDP negotiation. This follows already specified interactions

22.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information, the received media resource information of the terminating network, and the received QoS requirement. Then, the IMS AS sends response to DCSF. This follows existing procedures e.g. Steps 8 of Figure AC.7.2.3-1 of TS 23.228 [2] adding QoS requirements to Nimsas_MediaControl_MediaInstruction request if QoS requirements have been received in step 12.

23-29.	IMS AS continues performing SDP negotiation with the originating network. With a successful SDP negotiation, the application data channel is established. This follows already specified interactions with the addition that the IMS AS when new QoS requirements are provided in Nimsas_MediaControl_MediaInstruction provides them in sip/sdp signalling towards the UEs.

### 6.11.4	Impacts on services, entities and interfaces

The solution has the following impacts:

DC AS:

-	Subscribing to the event of "establishment of a MDC2 channel is needed" and receiving corresponding notifications containing MF-side information for MDC2 configuration.

-	Sending DC AS-side information for MDC2 configuration in Ndcsf_ADCControl_Create that may include QoS requirement for the DC application.

-	Sending/receiving certificate fingerprints for security between UE/MF and DC AS.

DCSF:

-	Receiving subscription to the "event of establishment of a MDC2 channel is needed" and sending corresponding notification, containing MF-side information for MDC2 configuration.

-	Receiving DC AS-side information for MDC2 configuration.

-	Sending/receiving certificate fingerprints for security between UE/MF and DC AS.

-	Receiving QoS requirement for the DC application.

IMS AS:

-	This solution assumes the definition of an IMS AS event requiring the IMS AS is to forward to the selected DCSF.

-	Sending subscription request to the DCSF instance.

-	Using optional QoS requirements added to Nimsas_MediaControl_MediaInstruction.

## 6.12	Solution #12: Support dynamic configuration of MDC2 endpoints with new DCSF services

### 6.12.0	High-level solution Principles

This solution addresses KI#3: Supporting dynamic configuration of MDC2 endpoints.

This solution takes the existing Rel-18 procedures of P2A and P2A2P DC establishment as the basis and updates the existing steps between DCSF and DC AS by defining new service of DCSF.

### 6.12.1	Description

This solution defines a new service of DCSF which is used to request to establish, update or delete a MDC2 connection. This service exchanges the MDC2 media points information on MF and DC AS to each other. The connection ID is allocated by DCSF and is used to correlate the MDC2 connection.

The interaction between DCSF and DC AS is designed as event exposure of MDC2 connection request to exchange the MDC2 media endpoint information. By this principle, the DC AS from third party customer is not required to provide any service when using IMS DC services from the operator. Accordingly the DCSF services use Subscription/Notification mode. In this solution implicit subscription is used based on the following consideration:

-	If explicit subscription is used, it is very difficult to ensure the DCSF selected by the DC AS to subscribe is the same DCSF selected by the IMS AS during DC establishment, especially when multiple DCSFs are deployed in a pool.

-	Explicit subscription saves cost interaction.

The solution supports both P2A and P2A2P DC management. When P2A DC is established, the media information of one pair of MDC2 media points between MF and DC AS is exchanged. When P2A2P DC is established, the media information of two pairs of MDC2 media points for originating side and terminating side between MF and DC AS is exchanged.

This solution uses event notification and notification response messages to exchange MDC2 media endpoints information. The reliable transmission of the messages is ensured by the transportation layer, e.g. TCP or SCTP.

To support implicit subscription and notification, the solution also clarifies that the DCSF selects the correct DC AS and get the address of the selected DC AS URL based on binding information which indicates which DC AS is serving the application. The mapping of the app id and the DC AS URL is locally configured on DCSF or is retrieved from the application data in the HSS. This mechanism has been specified in Rel-19 to enable the DCSF to fetch Avatar ID list from a DC AS (see clause AC.11.1 of TS 23.228 [2]).

### 6.12.2	Procedures

#### 6.12.2.1	P2A DC procedure

This procedure depicts the procedure of UE initiated P2A DC establishment, P2A DC modification and P2A DC deletion, which reuses the P2A DC establishment procedure in clause AC.7.2.2 of TS 23.228 [2] as the basis and updates original steps 9 and 10 to clarify the interaction between DCSF and DC AS.

Figure 6.12.2.1-1: Procedure of P2A DC establishment

The steps in the procedure are as follows:

Steps 0-8 are the same with steps 0-8 in clause AC.7.2.2 of TS 23.228 [2];

9.	The DCSF selects the DC AS serving this session based on the binding information of this P2A DC. The DCSF gets the DC AS URL of the selected DC AS by local configuration or retrieves it from DCSF service specific data stored in HSS.

10.	The DCSF invokes Ndcsf_MDC2Connection_Create service operation as defined in clause 6.12.3.2 to request the selected DC AS to establish MDC2 connection with the MF for the P2A DC. The service operation includes the MDC2 media endpoint information on the MF.

11.	The DC AS responds to Ndcsf_MDC2Connection_Create service operation with connection ID and its MDC2 media endpoint information for the MDC2 connection.

12-28.	The same with steps 11-27 in clause AC.7.2.2 of TS 23.228 [2].

29-30.	The UE initiates IMS session modification to update the established P2A DC. The MDC2 media endpoint on the MF is updated and the information is returned to the DCSF.

31.	The DCSF invokes Ndcsf_MDC2Connection_Update service operation as defined in clause 6.12.1.3.3 to notify the DC AS the media endpoint on the MF for the established P2A DC is updated. The service operation includes the connection ID and the updated MDC2 media endpoint information on the MF.

32.	The DC AS updates its media endpoint accordingly and responds to Ndcsf_MDC2Connection_Update service operation with updated MDC2 media endpoint information.

33-34.	The IMS session is updated.

35.	The UE releases the P2A DC or the whole IMS session.

36-37.	The DCSF notifies the DC AS the P2A DC is deleted. The DC AS removes its media resource of the P2A DC.

#### 6.12.2.2	P2A2P DC procedure

This procedure depicts the procedure of UE initiated P2A2P DC establishment, P2A2P DC modification and P2A2P DC deletion, which reuses the P2A2P DC establishment procedure in clause AC.7.2.3 of TS 23.228 [2] as the basis and updates original step 9 and 10 to clarify the interaction between DCSF and DC AS.

Figure 6.12.2.1-1 Procedure of P2A2P DC establishment

The steps in the procedure are as follows:

Steps 0-8 are the same with steps 0-8 in clause AC.7.2.3 of TS 23.228 [2];

9.	The DCSF determines to establish P2A2P DC and invokes Ndcsf_MDC2Connection_Create service operation to request the DC AS to establish MDC2 connection with the MF for the P2A2P DC. The service operation includes the MDC2 media endpoint information for originating side and terminating side on the MF.

10.	The DC AS responds to Ndcsf_MDC2Connection_Create service operation with connection ID and its MDC2 media endpoint information for originating side and terminating side.

11-26.	The same with steps 11-27 in clause AC.7.2.3 of TS 23.228 [2].

27-28.	The UE initiates IMS session modification to update the established P2A2P DC. The MDC2 media endpoint on the MF is updated and the MDC2 information of originating side is returned to the DCSF.

29.	The DCSF invokes Ndcsf_MDC2Connection_Update service operation as defined in clause 6.12.1.3.3 to notify the DC AS the media endpoint for originating side on the MF for the established P2A2P DC is updated. The service operation includes the connection ID and the updated MDC2 media endpoint information on the MF.

30.	The DC AS updates its media endpoint accordingly and responds to Ndcsf_MDC2Connection_Update service operation with updated MDC2 media endpoint information.

31.	The IMS session is updated.

32.	The UE releases the P2A DC or the whole IMS session.

33-34.	The DCSF notifies the DC AS the P2A DC is deleted. The DC AS removes its media resource of the P2A DC.

### 6.12.3	DCSF Services

This solution defines DCSF services to support interaction between DCSF and DC AS for establishing the P2A or P2A2P DC.

Table 6.12.3.1-1: DCSF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_MDC2ConnectionRequest | CreateNotify | Subscription/Notification | DC AS |
|  | UpdateNotify | Subscription/Notification | DC AS |
|  | DeleteNotify | Subscription/Notification | DC AS |

#### 6.12.3.1	Ndcsf_MDC2Connection

##### 6.12.3.1.1	General

Service description: This service enables the consumer to be requested to establish, update or delete MDC2 connection towards the MDC2 endpoint on the MF.

##### 6.12.3.1.2	Ndcsf_MDC2Connection_CreateNotify service operation

Service operation name: Ndcsf_MDC2Connection_CreateNotify

Description: This service operation enables DCSF to notify the consumer of the event of establishing a MDC2 connection with the MF towards the MDC2 endpoint on the MF and responds with its own MDC2 endpoint information on the MF.

Inputs, Required: Session ID, Connection ID, DC type, remote MDC2 media endpoint information.

Session ID is the identity of the IMS session to which the MDC2 connection belongs.

Connection ID is the identity of the MDC2 connection. It is allocated by the DCSF.

DC type indicates the type of the established DC, i.e. P2A, P2A2P.Remote MDC2 media endpoint information contains the following:

-	Endpoint type: Originating side, Terminating side.

-	The IP address and port of the MDC2 media endpoint on MF.

NOTE:	If the DC is P2A DC, only media endpoint information of originating side is included.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. The Calling ID and Called ID can be used by DC AS to decide whether a P2A2P DC should be further established.

Outputs, Required: Result indication.

Outputs, Optional:  Local MDC2 media endpoint information if the result is successful.

Local MDC2 media endpoint information contains the following:

-	Endpoint type: Originating side, Terminating side.

-	The IP address and port of the MDC2 media endpoint on MF.

NOTE:	If the DC is P2A DC, only media endpoint information of originating side is included.

##### 6.12.3.1.3	Ndcsf_MDC2Connection_UpdateNotify service operation

Service operation name: Ndcsf_MDC2Connection_UpdateNotify

Description: This service operation enables DCSF to notify the consumer to the event of updating an existing MDC2 connection on the MF.

Inputs, Required: Session ID, Connection ID, updated remote MDC2 media endpoint information.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional:  Local MDC2 media endpoint information if the result is successful.

##### 6.12.3.1.4	Ndcsf_MDC2Connection_DeleteNotify service operation

Service operation name: Ndcsf_MDC2Connection_DeleteNotify

Description: This service operation enables DCSF to notify the consumer to the event of deleting an existing MDC2 connection on the MF.

Inputs, Required: Session ID, Connection ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

### 6.12.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DCSF:

-	Supports to select the DC AS serving the session based on the binding information of this P2A DC.

-	supports Ndcsf_MDC2ConnectionRequest service to interact with DC AS to establish, update or delete MDC2 connection for P2A or P2A2P DC.

DC AS:

-	supports Ndcsf_MDC2ConnectionRequest service to interact with DCSF to establish, update or delete MDC2 connection for P2A or P2A2P DC.

## 6.13	Solution #13: DC3/DC4 enhancements with new media management

### 6.13.1	Description

This solution addresses KI#3.

This solution specifies the DCAS services exposed over the DC3/DC4 interface to support UE-initiated P2A and P2A2P ADC establishment. Specifically, the following new service function is defined: Ndcas_Media_Management. the associated service parameters and corresponding procedures are also defined.

The DC AS provisions the service triggering (triggering information) criteria for establishing P2A/P2A2P ADC at the DCSF using the Nimsas_ImsParameterProvision_Create service operation (see clause AA2.4.6 of TS 23.228 [2], v19.0.0). Such provisioning can be explicitly initiated by the DC AS or implicitly configured per DC AS in the IMS AS and/or in the service document in HSS (Repository data). Triggering can be user specific or user unrelated (i.e. PSI) and associated with filter conditions such as P2A/P2A2P ADC associated with specific applications. Upon receipt of a P2A or P2A2P ADC establishment request meeting the triggering filter conditions, the IMS AS invokes Nimsas_SessionEventControl_Notify to notify the DCSF of a media change request event and the availability of an P2A/P2A2P ADC. This notification includes the SDP information for the P2A/P2A2P ADC representation on the Mb interface. Based on the notification received, the DCSF will request anchoring of media in the MF for the originating MF-side of the MDC2 interface. The DCSF will then provide the originating MF-side of the MDC2 to the DCAS and request the DCAS-side for the MDC2 via the Ndcas_Media_Management_Create service operation. The DCSF will provide the received media information from the DCAS to IMS AS over the Nimsas_mediaControl service with the requested MDC2 configuration based on the information received from the DC AS.

Note that the proposal utilizes the principle that configuration/trigger can be sent to any (not all) IMS AS, according to operator choice and the data will be then stored in HSS over N71 as repository data (using the same mechanisms as defined in clause AF.4 of TS 23.228 [2]) and can subsequently be used by any IMS AS serving specific subscriber or non-subscriber specific triggers.

The use of IMS Nimsas_SessionEventControl_Notify was purposefully done in order to reuse (thus no need for new interfaces in DCSF, etc.). The provisioning paradigm used is a request-resp interface and it is not connected to IMS events (as covered in KI#2) ensuring that all IMS ASs can avail of the data even if not always applicable only to a specific subscriber. These triggers are static in nature and are similar to iFC triggers.

This enhancement to the DC3/DC4 interface is crucial for ensuring a time sensitive and 3GPP SBA principle compliant consistent handling of MDC2 media resources between the DCSF and the DC AS. The DC AS and the MF may dynamically deploy/insert MDC2 terminations and thus use the dynamically determined IP address. The DC port and/or DC SCTP port may be dynamically associated to the session by the MF and/or the DC AS. Furthermore, the network (DCSF) may need to provide the DC AS with the UE certificate fingerprint received from the UE over IMS signalling using the Dcas_Media_Management_Update service operation.

The current P2A/P2A2P data channel procedures lack a 3GPP SBA service self-containment complaint mechanism for MDC2 media resource negotiation, leading to potential inconsistencies of MDC2 media resource between the two MDC2 endpoints. This proposed enhancement, by enabling the MDC2 media negotiation and dynamic configuration, significantly improves the robustness and flexibility of these procedures, preventing failures due to outdated or missing MDC2 media resource information. The DCSF availing of the DC AS offered / provided service, Dcas_Media_Management ensures that it is the resource control entity when interworking with DC AS.

Furthermore, this solution ensures that DC AS can be selected by DCSF (for establishing P2A/P2A2P ADC) on a per session basis without creating long lived dependencies between the entities.

NOTE:	This solution identifies changes to existing specifications, these changes are shown using RED text.

### 6.13.2	Service definition

#### 6.13.2.1	General

The following table shows the DC AS Services and DC AS Service Operations.

Table 6.13.2-1: NF services provided by the DC AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcas_Media_Management | Create | Request/Response | DCSF |
|  | Update | Request/Response | DCSF |
|  | Delete | Request/Response | DCSF |

#### 6.13.2.2	Ndcas_Media_Management

##### 6.13.2.2.1	General

Service description: This service enables the consumer (DCSF) to create, update or terminate a media resource controlled by the producer (DCAS).

##### 6.13.2.2.2	Ndcas_Media_Management_Create

Service operation name: Ndcas_Media_Management_Create

Description: This service operation enables the consumer to create a media resource (e.g. ADC (P2A, P2A2P) or other media resources like audio/Video. The information is used by the DCAS to create a resource and establish a media resource in an IMS session.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode, trigger Information.

Inputs, Optional:

Outputs, Required: Result. The result contains the following data elements: session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent operations.

Outputs, Optional: None.

##### 6.13.2.2.3	Ndcas_Media_Management_Update

Service operation name: Ndcas_Media_Management_Update

Description: This service operation enables the consumer to update a media resource information used by the DCAS.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), destination URL

Inputs, Optional: certificate, fingerprint of the UE, trigger Information

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.13.2.2.4	Ndcas_Media_Management_Delete

Service operation name: Ndcas_Media_Management_Delete

Description: This service operation enables the consumer to provide information used by the DCAS for media termination in an IMS session

Inputs, Required: Session ID, mediaInformation, destination URL

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.13.2.3	Nimsas_ImsParameterProvision Service API

##### 6.13.2.3.1 	Type: ImsPpDataEntry

Table 6.13.2.3.1-1 New Parameters for:Table 6.5.6.2.2-1: Definition of type ImsPpDataEntry, TS 29.175 [9]

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rcdServerInformation | RcdServerInfo | O | 0..1 | The RCD server information provisioned to the IMS AS. |
| triggerinformation | TriggerInfo | O | 0..1 | Triggering information to invoke for example dcas media management |

Editor’s note:	The TriggerInfo data type needs to be defined as common data and is FFS.

NOTE:	An additional data type is required (existing defined operation only contains RCD related info) to trigger the service to invoke DCSF (and locate the correct DCAS) for this session.

#### 6.13.2.4	Nimsas_SessionEventControl Service API

##### 6.13.2.4.1	Type: SessionEventNotification

The parameters that are new are: afInstance, triggerinformation

Table 6.13.2.4.1-1: New Parameters for Table 6.1.6.2.2-1 of TS 29.175 [9]: Definition of type SessionEventNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notificationEvent | NotificationEvent | M | 1 | The NotificationEvent corresponds to a report on one subscribed event. |
| sessionId | SessionId | M | 1 | The session ID is the identity of the IMS session for which the event relates to. This IE contains the information in the Call-ID header which is the typical header of SIP message. |
| sessionInfo | SessionInfo | C | 0..1 | The sessionInfo is the IMS session related information, e.g. the calling and called identity. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "MEDIA_CHANGE_REQUEST", "EXTERNAL_SESSION_CREATE", "SESSION_ESTABLISHMENT_SUCCESS" (NOTE). |
| mediaInfoList | map(MediaInfo) | C | 1..N | Media info list includes a list of media related information, e.g. media identity, media type and media specifications. The mediaId attribute within the MediaInfo data type is the key of the map. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "SESSION_ESTABLISHMENT_PROGRESS", "SESSION_ESTABLISHMENT_ALERTING", "SESSION_ESTABLISHMENT_SUCCESS", "MEDIA_CHANGE_REQUEST", "MEDIA_CHANGE_SUCCESS", "MEDIA_CHANGE_FAILURE", or "MEDIA_CHANGE_CANCEL". |
| psDataOff | PsDataOffStatus | C | 0..1 | It indicates the active/inactive status of PS Data Off for the served UE. It shall be present if the attribute eventType of the notificationEvent is set to one of the following: - "SESSION_ESTABLISHMENT_REQUEST", when IMS session establishment is invoked; - "PS_DATA_OFF_STATUS_CHANGE", when the status change of PS Data Off happens without any other changes to the IMS session; It may be present in other cases if the change of PS Data Off status happens together with other events (e.g. media changes). |
| mediaInfoSet | array(MediaInfoExternal) | C | 0..1 | Media information set includes a set of media information. It shall be contained when the attribute eventType of the notificationEvent is one of the "EXTERNAL_SESSION_UPDATE" "EXTERNAL_SESSION_CREATE" |
| afInstance | string | O | 0..1 | The string identifying the served DCAS service instance |
| triggerinformation | TriggerInfo | O | 0..1 | Triggering information to invoke dcas media management |
| NOTE: The sessionInfo shall be present when the attribute eventType of the notificationEvent is set to "SESSION_ESTABLISHMENT_SUCCESS" and the originating IMS AS received the redirected-to party identity via the History-Info header field in the 200 OK responses to the initial INVITE request. |  |  |  |  |

### 6.13.3	Procedure

#### 6.13.3.1	Person-to-Application (P2A) Application Data Channel Setup

Figure 6.13.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.13.3.1-1: P2A ADC establishment with a trusted DC AS

1.	The DCAS provisions service triggers using the Nimsas_ImsParameterProvision.

NOTE:	The IMS Parameter Provisioning flow is simplified for illustration purposes. The IMS Parameter Provisioning flow is defined in clause AF.4 of TS 23.228 [2] and uses an IMS AS instance that does not need to be the one assigned to the specific IMS user. Since the provisioned parameters are stored as repository data in HSS, the IMS AS assigned to the IMS user will retrieve the DCAS provided service triggers from HSS.

Editor's note:	How the IMS Parameter Provisioning procedure can support provisioning of service triggers for any UE is FFS.

2.	The IMSAS responds the request to the DC AS confirming the provisioned data stored for the DCAS.

3.	IMS session and BDC has been established.

4-11.	UE#1 requests the establishment of P2A ADC establishment. IMSAS evaluate the service triggers provided in step 1. IMSAS provide the service triggers and the DCAS instance to the DCSF. DCSF determines the DC control policy and request media endpoint for the MDC2 from the IMSAS. IMS AS to request MF to reserving the originating media resources.

12.		DCSF request the DC AS to establish a media endpoint for the MDC2 via Ndcas_Media_Management_Create message. The message includes calling and called parting id, application id/DCAS instance, session id, MF-side MDC2 media resource info. source URL used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE.

13.		DC AS allocates DC AS-side MDC2 media resource for this session. DC AS respond to the DCSF with the DC AS endpoint for the MDC2 via Ndcas_Media_Management_Create response. The message contains: result, session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent Ndcas_Media_Management messages to update or terminate the connection. Update might contain e.g. the certificate fingerprint of UE.

14-16.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

17-29.	IMS AS performs SDP negotiation with terminating network. With a successful SDP negotiation, the application data channel established.

#### 6.13.3.2	Person-to-Application and Application-to-Person (P2A2P) Procedure

Figure 6.13.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.13.3.2-1: P2A2P ADC establishment with a trusted DC AS

1.	The DCAS provisions service triggers using the Nimsas_ImsParameterProvision.

NOTE:	The IMS Parameter Provisioning flow is simplified for illustration purposes. The IMS Parameter Provisioning flow is defined in clause AF.4 of TS 23.228 [2] and uses an IMS AS instance that does not need to be the one assigned to the specific IMS user. Since the provisioned parameters are stored as repository data in HSS, the IMS AS assigned to the IMS user will retrieve the DCAS provided service triggers from HSS.

Editor's note:	How the IMS Parameter Provisioning procedure can support provisioning of service triggers for anyUE is FFS.

2.	The IMSAS responds the request to the DC AS confirming the provisioned data stored for the DCAS.

3.	IMS session and BDC has been established.

4-11.	UE#1 requests the establishment of P2A ADC establishment. IMSAS evaluate the service triggers provided in step 1. IMSAS provide the service triggers and the DCAS instance to the DCSF. DCSF determines the DC control policy and request media endpoint for the MDC2 from the IMSAS. IMS AS to request MF to reserving the originating media resources.

12.	DCSF request the DC AS to establish a media endpoint for the MDC2 via Ndcas_Media_Management_Create message. The message includes calling and called parting id, application id / DC AS ID, session id, MF-side MDC2 media resource info. source URL used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE.

13.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS respond to the DCSF with the DC AS endpoint for the MDC2 via Ndcas_Media_Management_Create response. The message contains: result, session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent Ndcas_Media_Management messages to update or terminate the connection. Update might contain e.g. the certificate fingerprint of UE.

14-16.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

17-24.	IMS AS performs SDP negotiation with terminating network. And the IMS AS notifies the DCSF about the successful result of the SDP negotiation.

25.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information and the media resource information of the terminating network. Then, the IMS AS sends response to DCSF.

26-33.	IMS AS continues performing SDP negotiation with the originating network. With a successful SDP negotiation, the application data channel established.

### 6.13.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	provisioning service triggers for the UE-initiated P2A/P2A2P ADC establishment;

-	receiving MF-side information for MDC2 configuration;

-	receiving the certificate fingerprint of UE for end-to-end security between UE and DC AS;

-	sending DC AS-side information for MDC2 configuration;

-	sending the certificate fingerprint of DC AS for end-to-end security between UE and DC AS.

DCSF:

-	receiving subscription of the event UE-initiated P2A/P2A2P ADC establishment;

-	notifying about the event of UE-initiated P2A/P2A2P ADC establishment;

-	sending MF-side information for MDC2 configuration;

-	sending the certificate fingerprint of UE for end-to-end security between UE and DC AS;

-	sending request to create DC AS-side for the MDC2;

-	receiving DC AS-side information for MDC2 configuration;

-	receiving the certificate fingerprint of DC AS for end-to-end security between UE and DC AS;

-	requesting IMS AS for MDC2 configuration.

IMS AS:

-	receiving provisioning of trigger information from the DCAS;

-	receiving MDC2 configuration request from DCSF;

-	instructing MF for MDC2 configuration.

MF:

-	receiving MDC2 configuration instructions from IMS AS;

-	performing MDC2 configuration.

## 6.14	Solution #14: Dynamic MDC2 endpoint configuration IMS Data Channel

### 6.14.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints.

The main principle of this solution for support of dynamic MDC2 endpoint configuration for IMS Data Channel during UE initiated P2A and P2A2P session as follows

-	The DC AS shall discover and select the DCSF handling the UE for P2A or P2A2P session

-	To discover DCSF address, DC AS subscribe with HSS and HSS notifies once DCSF register its address in HSS.

-	Similar to third party registration done by S-CSCF to IMS AS, IMS AS does third party registration with DCSF and then DCSF register its address in HSS.

-	Upon successful discovery and selection, DCSF subscribes for the establishment of P2A or P2A2P events with DCSF and get notified when DCSF detects the events.

-	After receiving notification, If DC AS accepts P2A application data channel establishment request, DC AS sends an MDC2 reserved media resource in the newly proposed Ndcsf_MediaControl service operation message.

-	DCSF after receiving MDC2 endpoint information from DC AS, invoke IMS AS to provide the MDC2 endpoint address of DC AS and request for MDC2 endpoint from MF side.

-	DCSF provides the MF side MDC2 endpoint address to DC AS in response to Ndcsf_MediaControl service operation.

NOTE:	This solution involves DCSF during registration of IMPU while it was invoked during session earlier. The implications of this approach will not be discussed further in this solution.

### 6.14.1	Description

This solution proposes how the DC AS discovers the DCSF and subscribes to get notification for the UE initiated P2A and P2A2P session and then shares the MDC2 endpoint addresses. This will overcome the existing gaps of the P2A and P2A2P session which does not provide any flexibility in dynamically exchanging MDC2 endpoint addresses between DC AS and MF.

NOTE:	This solution identifies changes to existing specifications, these changes are shown using RED text.

Discovery of DCSF by DC AS:

The trusted DC AS can directly discover the DCSF with the help of HSS and in case untrusted DC AS, the discovery happens through NEF. For this purpose, HSS can expose one service operation using which DC AS/NEF will subscribe and get notification about the DCSF address for the UE.

When UE registers with IMS network, S-CSCF does third party registration with IMS AS and then IMS AS registers its address with HSS. Similarly, this solution proposed IMS AS does third party registration with DCSF and then DCSF registers its address with HSS. When HSS receives the subscription request for DCSF address from NEF or trusted AF and the DCSF address is not present in HSS (i.e. in the case when the IMPU is not yet registered or de-registered) then HSS stores the subscription request.  Once HSS comes to know the DCSF address (i.e. when IMPU does IMS registration and DCSF register its address to HSS), HSS notifies DCSF address with the NEF/ DC AS.

Subscription with DCSF by DC AS:

Upon successful discovery, DC AS shall subscribe with DCSF for the UE initiated P2A or P2A2P session establishment events.

Providing MDC2 endpoint address by DC AS:

Upon notified by DCSF about establishment of P2A or P2A2P call, DC AS provides its MDC2 endpoint address with DCSF.

Table 6.14.1-1 List of events supported by DCSF

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| P2A pr P2A2P Application Data Channel established | Detected when an Application Data Channel is established. Network detects that an Application data channel offer takes DC Application Server as target endpoint and requires to anchor in the MF | DCSF |

### 6.14.2	Service definition

#### 6.14.2.1	General

The following tables shows the required service updates in TS 23.502 [8] and TS 23.228 [2].

#### 6.14.2.2	NEF Service

The following shows the required NEF service updates in TS 23.502 [8].

##### 6.14.2.2.1	Service Summary table Update

The NEF services for IMS Event exposure is already defined in table 5.2.6.1-1 of TS 23.502 [8].

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_ImsEventExposure | Subscribe | Subscribe/Notify | AF, DC AS |
|  | Unsubscribe | Subscribe/Notify | AF, DC AS |
|  | Notify | Subscribe/Notify | AF, DC AS |

##### 6.14.2.2.2	Update to Nnef_ImsEventExposure (ImsEE) service (clause 5.2.6.39.2 of TS 23.502 [8])

The following describes the updated service operations for the IMS Event Exposure service:

6.14.2.2.2.1	Update to Nnef_ImsEE_Subscribe service operation

Service operation name: Nnef_ImsEE_Subscribe

Description: The NF consumer (for e.g. AF or DC AS) subscribes for a specific IMS event.

Inputs, Required: Event-ID(s) representing the event of interest, Event Reporting Information as defined in Table 4.15.1-1.

Inputs, Optional: List one or more IMS Public Subscriber identity, Event Filters, Notification Target address, Notification Correlation Information, Expiry time, Subscription Correlation ID (in the case of modification of the event subscription), AF Identifier.

IMS Public subscriber identity entry is only present for subscriber specific IMS events.

NOTE 1:	The semantics of the Event ID and Event Filters are transparent to the NEF.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can expire based on the operator's policy).

Outputs, Optional: If applicable to the event type, the current status related to the subscribed event type is included, if Immediate Reporting Flag is included in the request within Event Reporting Information and if available (see clause 4.15.1).

NOTE:	If the current status related to the subscribed event type is available and included in the response, it is not considered as an event report (i.e. it is not considered a one-time notification on event detection). That is, it is not deducted from the maximum number of reports if included in the request within the Event Reporting Information.

##### 6.14.2.3	HSS Service

The following shows the required HSS service updates in TS 23.228 [2].

##### 6.14.2.3.1	Service Summary table Update

The HSS services for IMS Services provided by an SBI capable HSS are already defined in table AA.2.1.1-1 of TS 23.228 [2].

Table 6.14.2.3.2.1-1: Update to Table AA.2.1.1-1 of TS 23.228 [2]: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| ImsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| Ims UE Context | Registration | Request/Response | S-CSCF, IMS AS, DCSF |
| Management (_ImsUECM) | DeregistrationNotification | Subscribe/Notify | S-CSCF |
|  | Deregistration | Request/Response | S-CSCF, IMS AS, DCSF |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
|  | AsInfoGet | Request/Response | NEF |
|  | DcsfInfoGet | Subscribe/Notify | NEF, DC AS |
|  | DcsfInfoUnsubscribe | Subscribe/Notify | NEF, DC AS |
|  | DcsfInfoNotify | Subscribe/Notify | NEF, DC AS |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |

##### 6.14.2.3.2	Update to Nhss_ImsUECM_Registration service operation

The following describes the Nhss_ImsUECM_Registration service operation from clause AA.2.1.2.1 of TS 23.228 [2], and the updates for this solution.

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

If the DCSF is the consumer, this service operation registers the DCSF instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name and Registration Type (e.g. Initial Registration, Unregistered) if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer, DCSF instance Id if the DCSF is the consumer.

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

##### 6.14.2.3.3	Update to Nhss_ImsUECM_Deregistration service operation

Service operation name: Nhss_ImsUECM_Deregistration

Description: This service operation deregisters the S-CSCF allocated to a public identity if the consumer is S-CSCF and deregisters the IMS AS instance assigned to the public identity if the consumer is IMS AS.

Inputs, Required: S-CSCF name if the S-CSCSF is the consumer, IMS AS instance ID if the IMS AS is the consumer, DCSF instance Id if the DCSF is the consumer, Deregistration Type.

Inputs, Optional: User Identity (Private Identity and/or Public Identity), P-CSCF Restoration indication, Session Priority.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 6.14.2.3.4	Nhss_ImsUECM_DcsfInfoGet service operation

The following describes a new service for IMS UE Context Management used for this solution:

Service operation name: Nhss_ImsUECM_DcsfInfoGet

Description: Service consumer like NEF or DC AS subscribes with HSS to receive the DCSF address assigned for the UE. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF.

Inputs, Required: Public Identity.

Inputs, Optional: Private Identity.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.14.2.3.5	Nhss_ImsUECM_DcsfUnsubscribe

The following describes a new service for IMS UE Context Management used for this solution:

Service operation name: Nhss_ImsUECM_DcsfUnsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.14.2.3.6	Nhss_ImsUECM_DcsfNotify

The following describes a new service for IMS UE Context Management used for this solution:

Service operation name: Nhss_ImsUECM_DcsfNotify

Description: This service operation enables the HSS to notify the consumer with the updated DCSF address where there is a change of DCSF happens for the UE.

Inputs, Required: DCSF address.

Inputs, Optional: IMPU, IMPI.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.14.2.4	DCSF Service

##### 6.14.2.4.0	General

The following table shows the new DCSF Services and new DCSF Service Operations added to table AA.2.4.1-1 of TS 23.228 [2].

Table 6.14.2.4.0-1: Updated Table AA.2.4.1-1 from TS 23.228 [2]: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_ADCControl_MDC2 | Create | Request/Response | NEF, DC AS |
|  | Update | Request/Response | NEF, DC AS |
|  | Delete | Request/Response | NEF, DC AS |

##### 6.14.2.4.1	Ndcsf_ADCControl_MDC2 service operation

Service operation name: Ndcsf_ADCControl_MDC2

##### 6.14.2.4.2	Service Description

This service enables the consumer to create, update or terminate an ADC.

##### 6.14.2.4.3	Ndcsf_ADCControl_MDC2Create

Service operation name: Ndcsf_SessionEvent_Create

Description: This service operation enables the consumer to create information used by the DCSF for ADC establishment in an IMS session.

Inputs, Required: Session ID, DC AS-side MDC2 endpoint information (e.g. IP address, port number), certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.14.2.4.4	Ndcsf_ADCControl_MDC2Update

Service operation name: Ndcsf_ADCControl_Update

Description: This service operation enables the consumer to update information used by the DCSF for ADC updating in an IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.14.2.4.5	Ndcsf_ADCControl_MDC2Delete

Service operation name: Ndcsf_ADCControl_Delete

Description: This service operation enables the consumer to provide information used by the DCSF for ADC termination in an IMS session

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.14.2.5	Ndcsf_SessionEvent

##### 6.14.2.5.1	General

Service description: This service enables the consumer to subscribe with the supported events by DCSF and get notified when the event is detected.

The following operations are available for this service:

-	Subscribing and unsubscribing with events supported by DCSF.

-	Notifying after event is detected to consumers, e.g. NEF, AF.

##### 6.14.2.5.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Notification Target address, Event ID

Only one Event Id is currently defined: establishment of a MDC2 channel is needed

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (otherwise an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.14.2.5.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.14.2.5.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), Resource URL(for any Ndcsf_ADCControl operation that would correspond to the Notification), DC application ID, certificate fingerprint of UE for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: Calling party and called party IDs, application ID, securitySetup if UE initiates the DTLS negotiation.

Outputs, Required: Result.

### 6.14.3	Procedure

#### 6.14.3.1	DCSF instance registration in HSS

Figure 6.14.3.1-1depicts a signalling flow diagram for an DCSF registering in HSS the DCSF instance assigned to a registering UE.

Figure 6.14.3.1-1: DCSF Instance Registration in HSS

1.	UE performs initial IMS Registration.

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS AS performs third party Registration with the DCSF instance assigned to the registering UE. This is over http and not SIP.

4.	DCSF assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

NOTE:	Figure 6.14.3.1-1 shows only SBA services. Corresponding Sh procedure is not included in the Figure.

#### 6.14.3.2	DCSF address discovery by DC AS

Figure 6.14.3.2-1depicts a signalling flow diagram for discovering DCSF address by DC AS.

.

Figure 6.14.3.2-1: DCSF address discovery by DC AS

1.	DCSF address is already registered as for the Figure 6.14.3.1-1.

2.	DCAS discovers DCSF by sending Nnef_ImsUECM_DcscfInfoGet service operation to NEF.

3.	NEF discovers DCSF by sending Nhss_ImsUECM_DcscfInfoGet service operation to HSS.

4.	HSS provides DCSF address in Nhss_ImsUECM_DcscfInfoGet response service operation to NEF.

5.	NEF provides DCSF address in Nnef_ImsUECM_DcscfInfoGet response service operation to DC AS.

The P2A and P2A2P call flow is addressed in clause 6.11.3.

### 6.14.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS/NEF:

-	subscribing the event of UE-initiated P2A/P2A2P ADC establishment.

-	receiving notification about the event of UE-initiated P2A/P2A2P ADC establishment.

-	receiving MF-side MDC2 endpoint information in the notify message.

-	sending DC AS-side MDC2 endpoint information in a new service operation message.

-	subscribing with HSS to discover the DCSF address.

DCSF:

-	receiving third party registration from IMS AS.

-	registering DCSF address in HSS.

-	receiving subscription for UE-initiated P2A/P2A2P ADC establishment event from NEF or DC AS and sending MF-side MDC2 endpoint information in notification message.

-	receiving DC AS-side MDC2 endpoint information in a new service operation message.

HSS:

-	receiving subscription from AF/NEF for discovering DCSF address.

-	notify with DCSF address once DCSF register its address to HSS.

IMS AS:

-	IMS AS does third party registration with DCSF over http (not SIP).

## 6.15	Solution #15: Early Media in Alerting Phase (CRS) Based on DC AS Initiated A2P DC

### 6.15.0	High-level solution Principles

This solution addresses Key Issue #1: ADC establishment in alerting phase for early media.

The solution focuses on the DC based Customized Ringing Signal (CRS) service which is subscribed by the calling party and it will provide customized content to the called party as an early media during the calling process.

The solution has the following principles:

-	The calling UE A has subscribed to the DC based CRS service.

-	The network serving the calling UE A and the called UE B have the early media capability, and the UE B has negotiated with the serving network regarding this capability.

-	The solution needs an enhancement of the subscribe/notify framework for event monitoring procedure to notify the DC AS the calling event which needs an A2P DC to deliver the early media

-	The solution reuses the IMS DC capability exposure procedure to establish an A2P DC which is used to transmit the early media to the UE.

-	The IMS-AS serving the calling UE A notifies the calling event to the DC AS; the DC AS triggers the A2P DC establishment process to the called UE B.

### 6.15.1	Description

This solution provides a procedure on how to establish an A2P ADC in alerting phase to provide customized DC content to the called party as an early media for CRS.

### 6.15.2	Procedure

The pre-condition of the following procedure is that the calling UE A has subscribed to the DC based CRS service and the network serving the calling UE A and the called UE B have the early media capability and the UE B has negotiated with the serving network regarding this capability. It assumes that the DC based CRS service will not coexists with the legacy CRS service; it means the UE will subscribe only one of the CRS services.

Figure 6.15.2-1: Procedure of A2P ADC establishment in alerting phase for CRS

The steps in the procedure are as follows:

1.	UE A initiates SIP INVITE including Audio/Video/BDC SDP; step 1-14 of Figure AC.7.1-1 in TS 23.228 [2] for BDC establishment apply.

2.	BDC has been established for UE B and UE B may download the application list and the application for CRS service.

3.	Based on the subscription of UE A, IMS AS-1decides to notify the DC AS at the network serving the UE A this BDC establishment event.

NOTE:	It assumes that the DC AS has subscribed the BDC establishment event of UE A.

4.	The IMS AS-1 sends the Nimsas_ImsEE_Notify Request to the NEF the calling event including the calling UE A and the called UE B information, as well as IMS Session ID.

5.	The NEF sends the Nnef_ImsEE_Notify Request to the DC AS the calling event including the corresponding information.

6.	The IMS AS-1 receives 180 Ringing from the terminating network.

7.	The DC AS decides to establish A2P ADC to UE B to deliver the DC early media for CRS.

8.	The DC AS establishes A2P ADC with UE B; and step 1-21 of Figure AG.2.1.1-1 in TS 23.228 [2] for adding ADC to an existing IMS session apply with the following differences.

-	The A2P ADC is added to an IMS session in establishment phase not an existing IMS session.

-	The SIP signal UPDATE is used instead of re-INVITE in the procedure of AG.2.1.1 in TS 23.228 [2].

UE B may download the application used in the alert phase.

The DC AS may send the A2P ADC addition request to the IMS AS-1 prior to the 180 Ringing. The IMS AS-1 will hold the request until it receives the 180 Ringing from the UE B.

9.	The UE B/terminating network and UE A/originating network exchange 180 Ringing SIP signals.

10.	The A2P ADC between UE B and the DC AS which goes through the MF is activated and the early media (customized alerting tones) of this IMS session is delivered from the DC AS to UE B. The user of UE B experiences with the early media for CRS.

11.	The user of UE B picks up the phone and the UE B/terminating network and UE A/originating network exchange 200 OK (INVITE) SIP signal.

12.	The IMS AS-2 notifies the DC AS at the calling network the IMS session establishment event.

13.	The DC AS releases the A2P ADC with UE B.

14.	Subsequent procedures continue.

### 6.15.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

-	Supporting ADC establishment in alerting phase for CRS.

IMS AS:

-	Supporting report of IMS session establishment events to DC AS.

-	Supporting A2P DC establishment for delivering early media.

MF:

-	Receiving early media from DC AS and providing it to UE through A2P DC.

DC AS:

-	Supporting receiving IMS session establishment events from IMS AS through NEF.

-	Supporting initiating and releasing A2P DC for delivering early media.

## 6.16	Solution #16: Enhancements to capability exposure framework and IMS AS service to support audio, video capability in addition with DC capability

### 6.16.1	High level Solution principles

This solution addresses KI#2.

Release 19 NG_RTC_Ph2 has specified IMS data channel capability exposure procedures and the related services to enables the AF/AS to request the IMS AS via the NEF to create or release an IMS session with DC media or modify the DC media in a specific IMS session. But in some cases, the NF/AF needs to create an IMS session with audio/video media and DC media at the same time.

OMA has specified a RESTful Network API for Audio Call, but it cannot support DC media. If a 3rd party AF/AS wants to create an IMS session with audio and DC media, it has to invoke the OMA API to create a IMS session with audio firstly and then it invokes Nnef_ImsSessionManagement API specified by 3GPP to add DC media in the established IMS session. It means that the 3rd party AF/AS need to utilize two sets of APIs with different styles (OMA style and 3GPP style) and it cannot use both audio and DC media at alerting phase.

To ease the development of the 3rd party AF/AS and enable that audio and DC media can be used at alerting phase, both Nnef_ImsSessionManagement, Nimsas_ImsSessionManagement and Nmf_MediaResourceManagement service specified in Release 19 need to be enhanced to support audio and video media.

The main principles of the solution are to align with and expand on existing specified functionality:

-	to add exposure capabilities to introduce support for establishment, modification and release of MMTel media (e.g. audio & video).

-	addition of MMTel media (e.g. audio & video) exposure capabilities is in the context of data channel (e.g. enhancing audio/video services with DC capabilities or enhancing DC services with audio/video services capabilities).

NOTE:	The term MMTEL media refers to media that is not sent through/ via a data channel.

### 6.16.2	Description

The solution specifies enhancements to the IMS exposure framework ensuring that exposure capabilities for session establishment, modification and release are media agnostic, i.e. other MMTel media (e.g. audio, video) are also supported in addition to the existing specified DC media.

The solution caters for the addition of MMTel media (e.g. audio, video) to the existing specified relevant service operations. The proposal builds on aligning with already specified functionality where the establishment, modification and release of MMTel media (e.g. audio, video) is supported in the following services:

-	Nimsas_MediaControl_MediaInstruction service operation (see clause AA.2.4.3.2 of TS 23.228 [2])

-	Nmf_MRM_Create service operation (see clause AA.2.5.2.2 of TS 23.228 [2]).

3GPP stage 3 has also defined support for audio / video (for example, see clause 6.1.6.3.4 of TS 29.175 [9]).

The extended functionality will for example enable a DC AS to add, modify or release MMTel media (e.g. audio/video) to / from a standalone ADC or enable a person / user to add, modify or release MMTel media (e.g. audio/video) towards DC AS (for DC enabled services).

The IMS AS services that are amended to indicate support of MMTel media (e.g. audio / video) in addition to the existing DC media are described in clause 6.16.4 Service definition enhancements.

Support for MMTel media (e.g. audio/video) shall also be included in the yet undefined services on DC3 & DC4 reference point.

The following two example procedures show the addition of MMTel media (e.g. audio and video) in addition to an A2P application data channel, between the UE-1 and DC AS over MDC2. As result, the MMTel media is anchored in the MF.

### 6.16.3	Procedures

#### 6.16.3.1	DC AS initiated IMS audio session addition with a standalone A2P application data channel

Figure 6.16.3.1-1 depicts a signalling flow diagram initiated by DC AS for adding MMTel media (e.g. audio) to a standalone Application Data Channel A2P application data channel with UE A. This signalling flow extends the flow described in clause AG.2.2.2 of TS 23.228 [2].

Figure 6.16.3.1-1: Establishment procedure of an adding an IMS audio session with a standalone A2P application data channel

0.	The DC AS decides to add an IMS audio session with an already established standalone A2P application data channel with UE A.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add an IMS audio session as addition to an already created standalone A2P application DC with UE A.

The DC AS sends the session ID (of previously created ADC) and UE A's IMPU in addition to the media operation set including the parameters for the media type set as "Audio" to NEF. The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified for the progress of the session creation.

2.	If the NEF authorizes the request, the NEF queries the HSS with Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE A.

3.	The NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-1 requiring to add an IMS audio session. The request includes the session ID (of previously created ADC) and UE A's IMPU in addition to the media operation set. The media operation set includes the parameters for the media type set as "Audio", binding info & the MDC2 media endpoint. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified of the session create progress.

4.	The IMS AS-1 validates the user subscription data and checks the IMS audio & DC capability of UE A to determine whether the request should be notified to DCSF-1. If UE A does not have subscription of IMS data channel and MMTel media (e.g. audio), then the request shall be rejected with appropriate cause and the subsequent steps are skipped. If the IMS AS-1 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Update response generated by the IMS AS-1 to the DC AS.

6.	The IMS AS-1 sends a Nimsas_SessionEventControl_Notify request to the DCSF-1 with event set to ExternalSessionCreateEvent. The notification includes the necessary parameters based on the information extracted from the media operation set (including MediaInfoList: MediaType - "Audio") received in step 3 and other stored parameters in the IMS AS-1 if applicable.

7.	After receiving the session event control notification, the DCSF-1 determines the policy how to process the A2P media audio establishment request based on the related parameters in the notification and/or DCSF-1 service specific policy.

8.	The DCSF-1 determines that the A2P media audio requires the DCAS audio to be anchored in MF-1.

9.	The DCSF-1 instructs the IMS AS-1 to set up the A2P audio by sending a Nimsas_MediaControl_MediaInstruction request.

10.	Based on the instruction from DCSF-1, the IMS AS-1 interacts with the MF-1 to allocate audio media resource towards UE A and MDC2 media resource towards the DC AS.

11.	The IMS AS-1 returns the Nimsas_MediaControl_MediaInstruction response to the DCSF-1.

12.	DCSF send a connection (audio) establishment request offer to DCAS including the MDC2 media resource.

13.	DCAS accepts the request and provides MDC2 media resource (audio).

14. The DCSF-1 instructs the IMS AS-1 to set up the audio by sending a Nimsas_MediaControl_MediaInstruction request.

15.	Based on the instruction from DCSF-1, the IMS AS-1 interacts with the MF-1 to request update of DCAS side media resource information.

16.	The IMS AS-1 returns the Nimsas_MediaControl_MediaInstruction response to the DCSF-1.

17.	The DCSF-1 returns the Nimsas_SessionEventControl_Notify response to the IMS AS-1.

18.	The IMS AS-1 generates a re-INVITE request in which the SDP offer contains the media information of the audio A2P application required by the DC AS, binding information and the session ID.

The IMS AS-1 sends the re-INVITE request to UE A.

19.	DC media negotiation may be completed through 18X/PRACK/200 OK(PRACK)/200 OK(INVITE)/ACK procedure.

20.	The IMS AS-1 requests MF-1 to update UE-A side media (Audio) resource information.

21.	UE A established audio with DC AS.

22.	If the NEF included a Notification Target Address in the Nimsas_ImsSessionManagement_Update request in step 3, the IMS AS-1 notifies the NEF for the progress of the audio session creation with the Nimsas_ImsSessionManagement_Notify request.

23.	If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in step 1, the NEF notifies the DC AS for the progress of the session creation with the Nnef_ImsSessionManagement_Notify request.

24.	The DC AS returns a Nnef_ImsSessionManagement_Notify response to the NEF.

25.	The NEF returns a Nimsas_ImsSessionMangement_Notify response to the IMS AS-1.

#### 6.16.3.2	Person-to-Application (P2A) MMTel media (audio) and Application Data Channel Setup

Figure 6.16.3.1-2 depicts a signalling flow diagram for adding an IMS audio session in a in a person to application use case. This signalling flow extends the flow described in clause AC.7.2.2-1 of TS 23.228 [2] where a P2A ADC is setup.

Figure 6.16.3.2-1: Person-to-Application (P2A) audio & application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0.a.	It is assumed that DC AS has provisioned triggers in IMS AS to be informed of media setup (e.g. audio (setup) media request).

0.b.	As in the base signalling procedure (AC.7.2.2-1), steps 0-3 of clause AC.7.2.1 applies. It is assumed that audio A-B has also been established.

Editor's note:	Steps 1-3 are to be updated.

NOTE 1:	The SIP re-INVITE (step 1) is initiated by the served users UE (in this case UE#1).

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel / audio establishment request based on the related parameters (i.e. associated DC application binding information) in the notification and/or DCSF service specific policy.

5.	The DCSF determines that the added Application Data Channel / audio media of the offer takes DC Application Server as target endpoint and requires to anchor in the MF.

6.	The DCSF invokes Nimsas_MediaControl service operation to instruct IMS AS to terminate the media flow of the originating UE to MF. The instruction also includes information to be consumed by the MF that the audio media shall be relayed via the MDC2 interface.

7.	The IMS AS invokes Nmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct MF on application data channel / audio establishment and data channel / audio media resource reservation based on the DC / audio media information received from DCSF.

8.	The IMS AS notifies the MediaControl instruction control response to DCSF.

9.	The DCSF stores the media resource information and sends a P2A (audio / DC) application data channel establishment request (including the MDC2 media resource received from MF) to the DC Application Server via DC3/DC4.

10.	DC Application Server accepts the P2A application data channel establishment request, returning an MDC2 reserved media resource as answer and is prepared for UE#1 traffic through MDC2.

NOTE 2:	Details on how DCSF communicates with the DC Application Server is for study as part of KI#3.

11.	DCSF requests IMS AS to update the MF resource with MDC2 media endpoint information of DC Application Server.

12.	IMS AS updates the MF resource.

13.	IMS AS notifies the MediaControl instruction control response to DCSF.

14.	DCSF replies to the notification received in step 3.

15-16.	IMS AS sends the re-INVITE to remote network side and UE#2, via the originating S-CSCF, which does not include application data channel request in the SDP for the application data channel.

17-19.	UE#2 and terminating network returns a 200 OK response with SDP answer for audio/video.

20.	IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

21.	DCSF replies to the notification.

22-23.	The IMS AS includes SDP answer for application data channels / audio to UE#1 in 200 OK response and sends 200 OK response to S-CSCF and P-CSCF.

24.	The originating network P-CSCF executes QoS procedure for application data channel / audio media based on the SDP answer information from the 200 OK response.

25.	CSCF returns the 200 OK response to UE#1.

26.	UE#1 send ACK to the terminating network.

27.	The application data channel / audio between UE#1 and DC Application Server is established via MF. MF forwards data channel / audio traffic between UE#1 and DC Application Server based on MDC2 media point information received in steps 9 and 12.

### 6.16.4	Service definition

#### 6.16.4.1	Nnef_ImsSessionManagement service

##### 6.16.4.1.1	General

The Media Type and Media Parameter set parameter in the following clauses need to be extended to support audio and video media. To be clear, revision mode is used to update the service definition of Nnef_ImsSessionManagement API in the following clauses.

##### 6.16.4.1.2	Nnef_ImsSessionManagement_Create service operation

Service operation name: Nnef_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with the requested media (e.g. standalone data channel media).

Inputs, Required: Calling ID, Called ID, Media information set.

Calling ID: It is the public identity of calling IMS Subscriber or the identity of the consumer NF, e.g. DC AS.

Called ID: It is the public identity of called IMS Subscriber.

Media information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created. It is allocated by the consumer NF.

-	Media Type: It indicates the media type used in the created session, e.g. Data Channel, Audio or Video.

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	App Binding Information;

-	Target ID(s): the public identity(ies) of the IMS Subscriber the DC is targeted;

NOTE:	For P2P ADC, the Calling ID and Called ID of the session are used as the Target IDs.

-	If PA2 or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC;

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	the MDC1 media endpoint address.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [2].

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 6.16.2.1.3	Nnef_ImsSessionManagement_Update service operation

Service operation name: Nnef_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the data channel media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the type of operated media (e.g. Data Channel, Audio or Video).

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	App Binding Information;

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	If P2A or P2A2P is selected, if the Operation Type indicates adding or updating the media, the MDC2 media endpoint address of the application layer is indicated.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted

-	if the Operation Type indicates adding or updating the media, the MDC1 media endpoint address.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [2].

Outputs, Required: Result indication.

Outputs, Optional: None.

#### 6.16.4.2	Nimsas_ImsSessionManagement Service

##### 6.16.4.2.1	General

The Media Type and Media Parameter set parameter in the following clauses need to be extended to support audio and video media. To be clear, revision mode is used to update the service definition of Nimsas_ImsSessionManagement API in the following clauses.

##### 6.16.4.2.2	Nimsas_ImsSessionManagement_Create service operation

Service operation name: Nimsas_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with the requested media (e.g. standalone data channel media).

NOTE 1:	Only data channel media is supported in this Release.

Inputs, Required: Calling ID, Called ID, Media Information set.

Calling ID: It is the public identity of calling IMS Subscriber or the identity of the consumer NF, e.g. DC AS.

Called ID: It is the public identity of called IMS Subscriber.

Media Information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created. It is allocated by the consumer NF.

-	Media Type: It indicates the media type used in the created session, e.g. Data Channel, Audio or Video.

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	Application Binding Information; and

-	Target ID(s): the public identity(ies) of the IMS Subscriber the DC is targeted;

NOTE 2:	For P2P ADC, the Calling ID and Called ID of the session are used as the Target IDs.

-	If P2A or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [2].

Outputs, Required: Result indication.

Outputs, Optional:  Session ID allocated by IMS AS.

##### 6.16.4.2.3	Nimsas_ImsSessionManagement_Update service operation

Service operation name: Nimsas_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the type of operated media (e.g. Data Channel, Audio or Video).

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	Application Binding Information; and

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	If P2A or P2A2P is selected, if the Operation Type indicates adding or updating the media, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [2].

Outputs, Required: Result indication.

Outputs, Optional: None.

#### 6.16.4.3	Nmf_MediaResourceManagement (MRM) service

##### 6.16.4.3.1	General

The Media Type and Media Parameter set parameter in the following clauses need to be extended to support audio and video media. To be clear, revision mode is used to update the service definition of Nmf_MediaResourceManagement API in the following clauses.

##### 6.16.4.3.2	Nmf_MRM_Create service operation

Service operation name: Nmf_MRM_Create

Description: The consumer NF requests the MF to create a media context including one or multiple media terminations and reserve media resources for anchoring one or multiple media streams of Mb interface in each termination. The consumer NF may also include media application function specification information requested by the application layer, e.g. DCSF, to be applied on the media stream by the MF.

Inputs, Required: List of Media Termination Descriptors.

Each Media Termination Descriptor of the list includes:

-	Termination ID: A unique identity of the termination within the media context resource. The identity is allocated by the producer and must be set to value 'null' by the consumer when adding a termination to a context.

-	List of Media Stream Descriptors belonging to the Termination.

Each Media Stream Descriptor of the list includes:

-	Media ID: A unique identity of the media stream within the media context instance. The Media ID value is set by the consumer.

-	associated Media ID: This parameter only exists when a multiplexed media is de-multiplexed to multiple media and is used to associate different media streams which are de-multiplexed from a multiplexed media component. This parameter is set to the value of the media ID of the original multiplexed media.

NOTE 1:	Different Termination IDs associated with the same Media ID or associated media ID corresponding to the same Media ID represent the media stream flow for data forwarding.

-	Remote Mb Specification: Media Specification specifying SDP parameters of a remote media endpoint e.g. media stream IP address and ports allocated at the remote UE.

NOTE 2:	In the case of reserving termination media resource before receiving media negotiation information from the UE, Remote Mb Specification should be set to Null.

-	Media Function Specifications: Specification of the media function to be applied to the media stream as requested by the application layer, which includes:

-	Media Function Type: The value is set based on the type of the media function required, e.g. "DC media function type, Audio, Video or AR media function type.

-	Media Function Description: Application specific media function description specifying how the media function shall handle and process the media stream. Data content depends on type of media function.

If the Media Function Type is "Audio" or "Video", then the Media Function Description incudes:

-	Codecs and Formats;

If the Media Function Type is "DC Media Function", then the Media Function Description incudes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	Remote MDC1/MDC2 media endpoint address.

-	Replacement HTTP URL per Stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Data Channel SCTP Port: This attribute identifies the SCTP port for the Data Channel.

-	Security Setup: This attribute identifies the security setup of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity

NOTE 3:	Some of the above attributes are optional depending on the scenario and proxy configuration.

If the Media Function Type is "AR Media Function", then the Media Function Description includes:

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies the how the media stream should be processed.

NOTE 4:	The detail of Media Processing Specification is defined by SA4 and needs to be further aligned.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Resource URL: URL that allocated by DC AS for MF to retrieve the avatar representation.

-	Rendering mode: identifies the mode of rendering avatar media, i.e. UE centric, network centric with MF rendering, network centric with DC AS rendering.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE 5:	Avatar Media Specification are optional depending on the scenario.

Inputs, Optional: None.

Outputs, Required: Result indication for the entire list, Media Resource Context ID, List of allocated Media Termination Descriptor Resource information. For each Media Termination Descriptor; the Termination ID and the list of media stream descriptors corresponding to the input list of media stream descriptor:

Media Resource Context ID uniquely identifies the media resources created by the MF. This identity shall be used as reference when updating or deleting the created media resource.

List of allocated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor Resources allocated. The list will include same number of terminations, in the same order as provided in the request. Each Media Termination Descriptor Resource includes:

-	Termination ID: Unique identity allocated by the consumer. The identity shall be used when updating/deleting the Media Termination.

-	List of Media Stream Descriptor Resource information elements allocated within the media termination. The list will include same number of media streams, in the same order as provided in the request.

Media Stream Descriptor Resource information includes:

-	Media ID: Unique identity of the media stream within the media context instance allocated by the consumer. This is identical to the Media ID used in the input.

-	associated Media ID: identical to the associated Media ID received from the consumer.

-	Local Mb Specification: Media specification specifying SDP parameters representing the media endpoint of the provider e.g. media stream IP address and port.

-	Media Application Function Output Description: Optional media application function description including information required by the application layer. The Media Application Function Output Description shall be relayed to the DCSF by the consumer NF. Data content depends on type of media function. If the Media type is "DC media function", the Media Application Function Output Description includes:

-	MDC1/MDC2 media point information (IP address/port number, etc.) which shall be relayed to the DCSF by the consumer NF.

Outputs, Optional: None.

### 6.16.5	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

NEF:

-	Enhance Nnef_ImsSessionManagement API to support audio and video media when creating, updating and removing an IMS session.

DC AS:

-	sending establishment, modification and release requests for MMTel Audio / Video.

-	sending DC AS-side information for MDC2 configuration for Audio / Video.

-	receiving MF-side (audio/video) information for MDC2 configuration.

DCSF:

-	receiving subscription of the event ExternalSessionCreateEvent (with MediaType=Audio/Video).

-	requesting IMS AS for MDC2 (audio/Video) configuration.

-	sending MF-side (audio/video) information for MDC2 configuration to DC AS.

-	sending request to create DC AS-side for the MDC2 (Audio/Video).

-	receiving DC AS-side information for MDC2 (Audio/Video) configuration.

-	requesting IMS-AS for MF (Audio/Video) configuration in e.g. controlling which media transformation the MF has to carry out.

IMS AS:

-	Enhance Nimsas_ImsSessionManagement API to support audio and video media when creating, updating and removing an IMS session.

-	receiving MDC2 (Audio/Video) configuration request from DCSF.

-	instructing MF for MDC2 (Audio/Video) configuration.

-	have to re-anchor voice media towards an MF/from MF based on DCSF instructions.

MF:

-	Enhance Nmf_MediaResourceManagement API to support creating, updating and removing audio and video media.

-	support Codec, formats and relevant audio/video media capabilities.

-	receiving MDC2 (Audio/Video) configuration instructions from IMS AS.

-	performing MDC2 (Audio/Video) configuration for DC AS service like e.g. translation & transcription services.

## 6.17	Solution #17: DC3/DC4 Enhancement to Support Implicit Subscription and Notification for P2A and P2A2P DC establishment

### 6.17.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints. This solution enhances the current P2A and P2A2P ADC establishment procedure defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2].

### 6.17.1	Description

This solution does not require the DC AS to subscribe to the call events from DCSF. Instead, the DCSF is implicated subscribed to and will notify the DC AS about the ADC establishment request event when receiving the notification from IMS AS. The DC AS will send the its MDC2 endpoint in the response if needed.

This solution also enhances and unifies the timing of notifying DC AS in the current P2A and P2A2P ADC establishment procedure defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2].

### 6.17.2	Service definition

#### 6.17.2.1	General

The following table shows the DCSF Services defined.

Table 6.17.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_EventExposure | Notify | Subscribe/Notify | NEF, DC AS |
| Ndcsf_MediaControl | MediaInstuction | Request/Response | NEF, DC AS |

#### 6.17.2.2	Ndcsf_EventExposure

##### 6.17.2.2.1	General

Service description: This service enables the consumer to be notified when the given event is detected.

##### 6.19.2.2.2	Ndcsf_EventExposure_Notify

Service operation name: Ndcsf_EventExposure_Notify

Description: This service operation enables the DCSF to notify the consumer about the subscribed event.

Inputs, Required: Session ID, Event ID.

Three Event ID is currently defined: application data channel establishment request, application data channel establishment success, application data channel establishment failure.

Inputs, Optional: application binding information, MF-side MDC2 address, indication of requesting DC AS-side MDC2 address.

Outputs, Required: Result.

Outputs, Optional:None.

#### 6.17.2.3	Ndcsf_MediaControl

##### 6.17.2.3.1	General

Service description: This service enables the consumer to give media instruction to DCSF. In this release, the instruction is only used to establish P2A and P2A2P data channel.

NOTE:	If the Event ID in the Ndcsf_EventExposure_Notify is application data channel establishment request, then the DCSF will expect the Ndcsf_MediaControl_MediaInstruction from the DC AS.

##### 6.17.2.3.2	Ndcsf_MediaControl_MediaInstruction

Service operation name: Ndcsf_MediaControl_MediaInstruction

Description: This service operation enables the consumer to provide instruction to the DCSF.

Inputs, Required: Session ID, Media information of MDC2 interface.

Inputs, Optional: AR media specification, avatar media specification.

Outputs, Required: Result indication.

Outputs, Optional: None.

### 6.17.3	Procedure

#### 6.17.3.1	P2A Application Data Channel Establishment Procedure

Figure 6.17.3.1-1 depicts the P2A application data channel establishment procedure.

Figure 6.17.3.1-1: P2A ADC establishment procedure

1.	Steps 0-8 of clause AC.7.2.2 of TS 23.228 [2] applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the implicit subscription. The DCSF will obtain the MF-side MDC2 point in the previous steps.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received and send the MF-side MDC2 point. The DCSF needs to indicates that it is requesting an MDC2 endpoint address from the DC AS side.

If the Event ID is application data channel establishment request, then the DCSF will expect the Ndcsf_MediaControl_MediaInstruction from the DC AS.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS sends Ndcsf_MediaControl_MediaInstruction to the DCSF to inform the MDC2 endpoint of the DC AS. Optionally, DC AS may send AR media instruction and avatar media instruction in the message if required.

6.	The DCSF sends 200 OK.

7.	Steps 11-27 in clause AC.7.2.2 of TS 23.228 [2] applies.

#### 6.17.3.2	P2A2P Application Data Channel Establishment Procedure

Figure 6.17.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.17.3.2-1: P2A2P Application Data Channel Establishment Procedure

1.	Steps 0-8 of TS 23.228 [2] Annex AC.7.2.3 applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the implicit subscription. The DCSF will obtain the MF-side MDC2 point in the previous steps.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received and send the MF-side MDC2 point. The DCSF needs to indicates that it is requesting an MDC2 endpoint address from the DC AS side.

If the Event ID is application data channel establishment request, then the DCSF will expect the Ndcsf_MediaControl_MediaInstruction from the DC AS.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS sends Ndcsf_MediaControl_MediaInstruction to the DCSF to inform the MDC2 endpoint of the DC AS. Optionally, DC AS may send AR media instruction and avatar media instruction in the message if required.

6.	The DCSF sends 200 OK.

7.	Steps 10-26 in clause AC.7.2.3 of TS 23.228 [2] applies. The DCSF will instruct the MF via IMS AS to update the media related information after receiving SDP answer from the terminating side.

### 6.17.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	receiving notification from the DCSF which may include MDC2 related media information.

-	sending DC AS side MDC2 related media information to DCSF in the notification response.

-	instructing AR media specification and avatar media specification to DCSF when needed.

DCSF:

-	notifying the session events to the DC AS.

-	sending MF side MDC2 related media information to DC AS.

-	receiving DC AS side MDC2 related media information from DC AS.

# 7	 VOID

# 8	Conclusions

## 8.1	Conclusions for Key Issue #1

The following conclusions for Key Issue #1: “ADC establishment in alerting phase for early media”:

1.	Only the A2P ADC is supported in this release.

2.	The IMS AS will be enhanced to support notification of events to the DC AS for establishment of DC in the alerting phase. This notification could be based on explicit subscription from DC AS to IMS AS, and/or be based on configuration. The DC AS will initiate the A2P ADC establishment procedure and proceed by IMS AS using a SIP Update.

3.	The initial INVITE sent by calling party may or may not have IMS DC in the SDP offer. If the IMS DC is not in the initial SIP INVITE, the network shall include both the BDC and ADC offer in the SIP Update.

4.	The bootstrap data channel media negotiation will follow the existing procedure defined in Annex AC of TS 23.228 [2].

5.	Users subscribe to Data Channel Application in Alerting Phase (DCAP) service. The subscription is stored in the HSS as a part of transparent data. The IMS AS will use this transparent data to authorize the subscription request from DC AS and the notification towards the DC AS.

6.	The application downloaded during alerting phase may continue and used for the established session after receiving 200 OK to the initial INVITE.

The establishment of data channels during alerting phase may require the DCMTSI UEs and the local and remote networks to handle multiple parallel SIP early-dialogs. The situation may occur when the session including bootstrap and application data channels is forked due to service execution (e.g. forking, call diversion, customized alerting tone, etc) in the terminating domain. Although the above issue is well covered in specifications for other media types (e.g. Voice, Video), it is not addressed in case of data channel media.

NOTE: 	Addressing the issue requires CT1 / CT4 and potentially SA4 feedback. SA2 final normative work on this KI will be on hold until feedback on whether there can be solution found on protocol level.

## 8.2	Conclusions for Key Issue #2

The following conclusions for Key Issue #2 "Enhancements to capability exposure framework for eMMTel services":

1.	Exposure of DC application downloading events are supported by the IMS capability exposure framework. Following four types of DC application downloading events are defined: subscriber specific and application specific event, subscriber agnostic and application specific event, subscriber specific and application agnostic event, subscriber agnostic and application agnostic event.

2.	An AF, particularly a DC AS, can subscribe to these events, utilizing the NEF for untrusted AFs.

3.	The subscriptions to these events are propagated (by the NEF to the IMS AS as currently defined in TS 23.228 [2] Annex AD. The IMS AS forwards the event subscription request to the DCSF instance, which has been selected by the IMS AS for controlling the BDC of the UE.

4.	The DCSFs are responsible for detecting the download of a DC application and for notifying the subscribing AF/DC AS (possibly via NEF).

5.	The DCSF ensures that event notifications are only provided to a DC AS for applications associated with the subscribing DC AS.

6.	Add following text in 23.228

“

IMS sessions can be established or controlled via APIs exposed by the IMS subsystem. Two kinds of APIs are available

- NEF API specified in the current document.

- Other APIs e.g. OMA APIs.

NOTE:	The IMS session ID can be used to ensure correlation between NEF APIs and other APIs (e.g. OMA APIs) targeting the same IMS session.

NOTE: 	The usage of OMA API and performing the correlation between NEF API and other APIs are outside the scope of 3GPP. No changes to OMA APIs are meant by these conclusions.

“

7.	Enhancements to , Nimsas_ImsSessionManagement, Nimsas_SessionEventControl_Notify (ExternalSessionCreateEvent), Nimsas_MediaControl_MediaInstruction and Nmf_MediaResourceManagement to support MMTel media (e.g. audio and video) when creating, updating and removing an IMS session that requires enhancing audio/video services with DC capabilities.

8.	Enhancement of Nnef_ImsSessionManagement to support MMTel media (e.g., audio and video) when creating, updating and removing an IMS session that requires enhancing audio/video services with DC capabilities.

9.	DCSF availing of the enhanced services like Nimsas_SessionEventControl_Notify (ExternalSessionCreateEvent), Nimsas_MediaControl_MediaInstruction and the newly introduced service / API for establishing MDC2 with support for MMTel media (e.g., audio and video) when creating, updating and removing an IMS session.

10.	DC AS availing of the enhanced services, Nimsas_ImsSessionManagement and the newly introduced service Ndcas_Media_Management supports MMTel media (e.g., audio and video) when creating, updating and removing an IMS session.

11.	All the enhancements above are neither meant to control supplementary services (CF, CDIV, etc…) nor specific audio media manipulation (transcoding, conferencing, etc…).

## 8.3	Conclusions for Key Issue #3

The following conclusions for Key Issue #3: “Supporting dynamic configuration of MDC2 endpoints”:

1.	The subscriptions to P2A and P2A2P ADC establishment request events are propagated (by the NEF) to the IMS AS as currently defined in TS 23.228 [2] Annex AD.

2.	The IMS AS forwards the event subscription request to the DCSF instance, which has been selected by the IMS AS for controlling the BDC of the UE.

3.	The DCSF notifies the DC AS based on notification from IMS AS triggered by ADC establishment request or based on configuration.

4.	The DC AS invokes Ndcsf service operation to inform the MDC2 endpoint of DC AS to the DCSF.

5.	The DC AS may include QoS and/or security related parameters that would be used in the SDP request of the ADC establishment in the message sending to the DCSF.

Annex A:
Change History

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA2#170 | S2-2507417 | - | - | - | Skeleton approved at SA2#170 | 0.0.0 |
| 2025-08 | SA2#170 | S2-2507417, S2-2507546, S2-2507547, S2-2507551, S2-2507610, S2-2507611, S2-2507612, S2-2507614, S2-2507615, S2-2507648, S2-2507655, S2-2507656 | - | - | - | Baseline Doc, Scope, References, Architectural Assumptions Key issue 1, 2 &3, Solution 1, 2, 3, &4 | 0.1.0 |
| 2025-10 | SA2#171 | S2-2509268, S2-2509270, S2-2509274, S2-2509275, S2-2509276, S2-2509277, S2-2509598, S2-2509599, S2-2509600, S2-2509621, S2-2509622 | - | - | - | Solution #2 update & add new solutions (#6, #7, #8, #9, #10, #11, #12,#13 and #14) | 0.2.0 |
| 2025-11 | SA2#172 | S2-2510816, S2-2510820, S2-2510822, S2-2510825, S2-2510828, S2-2510830, S2-2510916, S2-2510917, S2-2510918, S2-2510919, S2-2510920, S2-2510921, S2-2510952, S2-2510953 | - | - | - | Solution updates & add new solutions (#15, #16, and #17) | 0.3.0 |
| 2025-12 | SA#110 | SP-251349 | - | - | - | MCC Editorial update for presentation to TSG SA#110 for information | 1.0.0 |
| 2026-02 | SA#111 | S2-2600779, S2-2600909, S2-2601262, S2-2601442, S2-2601446, S2-2601460, S2-2601479, | - | - | - | Update Solutions #13 and #17, Add conclusions for KI#1, KI#2, and KI#3 | 1.1.0 |