| 3GPP TR 23.801-01 V0.4.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Architecture for 6G System; Stage 2 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

Introduction	11

1	Scope	12

2	References	12

3	Definitions of terms, symbols and abbreviations	13

3.1	Terms	13

3.2	Symbols	13

3.3	Abbreviations	13

4	Architectural Assumptions and Requirements	13

4.1	Architectural Assumptions	13

4.2	Architectural Requirements	14

5	Key Issues	14

5.1	Key Issue #1: Study the support for control signalling for 6G System	14

5.2	Key Issue #2: SBA framework	15

5.3	Key Issue #3: Support of Network Slicing in the 6G system	15

5.4	Key Issue #4: User Plane Architecture	15

5.5	Key Issue #5: QoS Framework for 6G	16

5.6	Key Issue #6: Policy and charging control framework	16

5.7	Key Issue #7: Network Exposure	17

5.8	Key Issue #8: Network Sharing in the 6G system	19

5.9	Key Issue #9: Localized Service Access	19

5.10	Key Issue #10: FWA / Fixed Wireless Access	19

5.11	Key Issue #11: Support of non-3GPP access	19

5.12	Key Issue #12: Voice Services for 6G	20

5.13	Key Issue #13: Emergency Voice Services for 6G	20

5.14	Key Issue #14: Location Services for 6G	21

5.15	Key Issue #15: Messaging Services for 6G	21

5.16	Key Issue #16: Other Essential/Regulatory Services for 6G	21

5.17	Key Issue #17: Migration and Interworking	21

5.18	Key Issue #18: AI for 6G architecture	22

5.19	Key Issue #19: 6G Network for AI	23

5.20	Key Issue #20: Integrated Sensing and Communication	24

5.21	Key Issue #21: 6G data framework in SA2	24

5.22	Key Issue #22: 6G Computing Support	25

5.23	Key Issue #23: Support of 6G NTN	26

5.24	Key Issue #24: Analyse 5GS IoT features and solutions	26

5.X	Key Issue #X: Key Issue Title	27

6	Solutions	27

6.0	Mapping of Solutions to Key Issues	27

6.1	Solutions to KI#1	27

6.2	Solutions to KI#2	28

6.3	Solutions to KI#3	28

6.4	Solutions to KI#4	28

6.5	Solutions to KI#5	29

6.5.0	Summary of Solution Variants	29

6.5.1	Solution variant #5.1: QoS flow considerations for 6G - Variant 1	31

6.5.1.0	Topics addressed and High-level Solution Principles	31

6.5.1.1	Description	31

6.5.1.2	Procedures	31

6.5.1.3	Services, Entities and Interfaces	31

6.5.1.4	Issues	31

6.5.1a	Solution variant #5.1a: QoS Flow considerations for 6G - Variant 2	31

6.5.1a.0	Topics addressed and High-level Solution Principles	31

6.5.1a.1	Description	31

6.5.1a.2	Procedures	32

6.5.1a.3	Services, Entities and Interfaces	32

6.5.1a.4	Issues	32

6.5.2	Solution variant #5.2: Enhanced QoS handling for /delay/error tolerant applications	32

6.5.2.0	Topics addressed and High-level Solution Principles	32

6.5.2.0	Description	33

6.5.2.2	Procedures	33

6.5.2.3	Services, Entities and Interfaces	33

6.5.2.4	Issues	33

6.5.3	Solution variant #5.3: QoS collaboration for a group of flows/UEs	34

6.5.3.0	Topics addressed and High-level Solution Principles	34

6.5.3.1	Description	34

6.5.3.2	Procedures	34

6.5.3.3	Services, Entities and Interfaces	34

6.5.3.4	Issues	34

6.5.4	Solution variant #5.4: Support a QoS flow with dynamic QoS requirements	34

6.5.4.0	Topics addressed and High-level Solution Principles	34

6.5.4.1	Description	34

6.5.4.2	Procedures	35

6.5.4.3	Services, Entities and Interfaces	35

6.5.4.4	Issues	35

6.5.5	Solution variant #5.5: Supporting dynamic QoS requirement by using multiple QoS flows	35

6.5.5.0	Topics addressed and High-level Solution Principles	35

6.5.5.1	Description	35

6.5.5.2	Procedures	36

6.5.5.3	Services, Entities and Interfaces	36

6.5.5.4	Issues	36

6.5.6	Solution variant #5.6: Enhanced UE/NW QoS collaboration via a collaboration layer between UE/application and UPF via user plane signalling	36

6.5.6.0	Topics addressed and High-level Solution Principles	36

6.5.6.1	Description	36

6.5.6.2	Procedures	37

6.5.6.3	Services, Entities and Interfaces	37

6.5.6.4	Issues	37

6.5.7	Solution variant #5.7: Enhanced QoS collaboration based on enhanced control plane signalling functionality	38

6.5.7.0	Topics addressed and High-level Solution Principles	38

6.5.7.1	Description	38

6.5.7.2	Procedures	38

6.5.7.3	Services, Entities and Interfaces	38

6.5.7.4	Issues	38

6.5.8	Solution variant #5.8: NW/DN QoS collaboration via user plane signalling	38

6.5.8.0	Topics addressed and High-level Solution Principles	38

6.5.8.1	Description	38

6.5.8.2	Procedures	39

6.5.8.3	Services, Entities and Interfaces	39

6.5.8.4	Issues	39

6.6	Solutions to KI#6	39

6.6.0	Mapping of bullets to solution variance	39

6.6.1	Bullet 1a: Solution variant #6.1: Convergence of Policy Association	39

6.6.1.0	Topics addressed and High-level Solution Principles	39

6.6.1.1	Description	40

6.6.1.2	Procedures	40

6.6.1.3	Services, Entities and Interfaces	40

6.6.1.4	Issues	40

6.6.2	Bullet 1a: Solution variant #6.2: Converged PCF and convergence of Policy Association and context across policy types	40

6.6.2.0	Topics addressed and High-level Solution Principles	40

6.6.2.1	Description	41

6.6.2.2	Procedures	41

6.6.2.3	Services, Entities and Interfaces	41

6.6.2.4	Issues	41

6.6.3	Bullet 1b: Solution variant #6.3: UE request for UE policies with UE determined parameters	41

6.6.3.0	Topics addressed and High-level Solution Principles	41

6.6.3.1	Description	42

6.6.3.2	Procedures	42

6.6.3.3	Services, Entities and Interfaces	42

6.6.3.4	Issues	42

6.6.4	Bullet 1b: Solution variant #6.4: UE policy control without UE determined parameters	42

6.6.4.0	Topics addressed and High-level Solution Principles	42

6.6.4.1	Description	43

6.6.4.2	Procedures	43

6.6.4.3	Services, Entities and Interfaces	43

6.6.4.4	Issues	43

6.6.5	Bullet 5: Solution variant #6.5: Support UE input for PCC decision	43

6.6.5.1	Description	43

6.6.5.2	Procedures	44

6.6.5.3	Services, Entities and Interfaces	44

6.6.5.4	Issues	44

6.7	Solutions to KI#7	44

6.8	Solutions to KI#8	44

6.8.0	High Level Summary of Solution variants for KI#8	44

6.8.1	Solution variant #8.1: 6G Multi-Operator Core Network	44

6.8.1.0	Topics addressed and High-level Solution Principles	44

6.8.1.1	Description	44

6.8.1.2	Procedures	45

6.8.1.3	Services, Entities and Interfaces	45

6.8.1.4	Issues	45

6.8.2	Solution variant #8.2: 6G Indirect Network Sharing	45

6.8.2.0	Topics addressed and High-level Solution Principles	45

6.8.2.1	Description	46

6.8.2.2	Procedures	46

6.8.2.3	Services, Entities and Interfaces	46

6.8.2.4	Issues	46

6.9	Solutions to KI#9	46

6.10	Solutions to KI#10	47

6.11	Solutions to KI#11	47

6.12	Solutions to KI#12	47

6.12.1	Solution variant #12.1: IMS based voice service using IP-CAN in the 6GS	47

6.12.1.0	Topics addressed and High-level Solution Principles	47

6.12.1.1	Description	47

6.12.1.1.1	UE's usage setting	47

6.12.1.1.2	(Access) Domain selection	47

6.12.1.1.3	UE capability indicates to 6G RAN	48

6.12.1.1.4	UE Capability Match Request procedure	48

6.12.1.1.5	DNN	48

6.12.1.1.6	Transport of IMS signalling and IMS media traffic	48

6.12.1.1.7	Voice continuity	49

6.12.1.1.8	P-CSCF discovery and selection	49

6.12.1.1.9	P-CSCF restoration	49

6.12.1.1.10	HSS discovery and selection	49

6.12.1.1.11	PCF aspects	50

6.12.1.1.12	Identity aspects	50

6.12.1.1.13	Roaming aspects	50

6.12.1.1.14	Paging aspects	50

6.12.1.1.15	Avoid late availability of Vo6G	50

6.12.1.1.16	Misc aspects	51

6.12.1.2	Procedures	51

6.12.1.3	Services, Entities and Interfaces	51

6.12.2	Solution variant #12.2: IMS Voice Call Fallback Mechanism	51

6.12.2.0	Topics addressed and High-level Solution Principles	51

6.12.2.0.1	UE-based fallback mechanism	52

6.12.2.0.2	Network-based fallback mechanism	52

6.12.2.1	Description	52

6.12.2.1.1	UE-based fallback mechanism	52

6.12.2.1.2	Network-based fallback mechanism	53

6.12.2.2	Procedures	53

6.12.2.3	Services, Entities and Interfaces	53

6.12.3	Solution variant #12.3: Compatibility with existing deployed IMS	53

6.12.3.0	Topics addressed and High-level Solution Principles	53

6.12.3.1	Description	54

6.12.3.2	Procedures	54

6.12.3.3	Services, Entities and Interfaces	54

6.13	Solutions to KI#13	54

6.13.1	Solution variant #13.1: Support for emergency services via 6GS	54

6.13.1.0	Topics addressed and High-level Solution Principles	54

6.13.1.1	Description	55

6.13.1.2	Procedures	55

6.13.1.3	Services, Entities and Interfaces	55

6.13.1.4	Issues	56

6.13.2	Solution variant #13.2: Support for Emergency Services Fallback to NR and E-UTRAN	56

6.13.2.0	Topics addressed and High-level Solution Principles	56

6.13.2.1	Description	56

6.13.2.2	Procedures	56

6.13.2.3	Services, Entities and Interfaces	58

6.13.2.4	Issues	58

6.14	Solutions to KI#14	58

6.14.1	Solution variant #14.1: Enhance 5GS LCS architecture for 6GS	58

6.14.1.0	Topics addressed and High-level Solution Principles	58

6.14.1.1	Description	58

6.14.1.2	Procedures	59

6.14.1.2.1	6GC-MT-LR procedure for the regulatory location service	60

6.14.1.2.2	6GC-MT-LR multiple location procedure for the regulatory location service	60

6.14.1.2.3	6GC-NI-LR Procedure	61

6.14.1.2.4	6GC-MT-LR Procedure without UDM Query	61

6.14.1.2.5	Network Assisted Positioning Procedure with direct periodic measurement report based on solution #14.3	62

6.14.1.2.6	Terminate direct connection between 6G-RAN and 6G-LMF based on solution #14.3	62

6.14.1.3	Services, Entities and Interfaces	63

6.14.1.3.1	Entities reused and enhanced from 5G	63

6.14.1.3.2	Interfaces	63

6.14.1.4	Open issues	64

6.14.2	Solution variant #14.2: Redesign 5G LCS architecture related to AMF (mobility management) involvement	64

6.14.2.0	Topics addressed and High-level Solution Principles	64

6.14.2.1	Description	65

6.14.2.2	Procedures	66

6.14.2.2.1	Procedures based on solution #14.4	66

6.14.2.2.2	Procedures based on solution #14.5	69

6.14.2.3	Services, Entities and Interfaces	69

6.14.2.3.1	Following Entities and functionalities are defined based on Solution #14.4	69

6.14.2.3.2	Following Entities and functionalities are defined based on second Solution of #14.5	69

6.14.2.4	Open issues	70

6.15	Solutions to KI#15	70

6.15.1	Solution variant #15.1: Support for SMS services via 6GS	70

6.15.1.0	Topics addressed and High-level Solution Principles	70

6.15.1.1	Description	70

6.15.1.1.1	SMS over NAS	70

6.15.1.1.2	SMS over IP	71

6.15.1.1.3	Domain selection for UE originating SMS	71

6.15.1.1.4	Secure SMS	71

6.15.1.2	Procedures	71

6.15.1.3	Services, Entities and Interfaces	71

6.16	Solutions to KI#16	71

6.17	Solutions to KI#17	72

6.18	Solutions to KI#18	72

6.19	Solutions to KI#19	72

6.19.0	Mapping of Solution variants to KI#19 bullets	72

6.19.1	Solution variant #19.1: 6G CN support the UE AI agent registration, discovery and communication	72

6.19.1.0	Topics addressed and High-level Solution Principles	72

6.19.1.1	Description	73

6.19.1.2	Procedures	74

6.19.1.3	Services, Entities and Interfaces	74

6.19.1.4	Issues	74

6.19.2	Solution variant #19.2: UE AI agent registration, discovery and communication within application layer of the operator control domain	74

6.19.2.0	Topics addressed and High-level Solution Principles	74

6.19.2.1	Description	74

6.19.2.2	Procedures	74

6.19.2.3	Services, Entities and Interfaces	74

6.19.2.4	Issues	74

6.19.3	Solution variant #19.3: Network Capability Exposure Functionalities to AI agent on AF(s)	75

6.19.3.0	Topics addressed and High-level Solution Principles	75

6.19.3.1	Description	75

6.19.3.2	Procedures	75

6.19.3.3	Services, Entities and Interfaces	75

6.19.3.4	Issues	75

6.19.4	Solution variant #19.4: 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE)	75

6.19.4.0	Topics addressed and High-level Solution Principles	75

6.19.4.1	Description	76

6.19.4.2	Procedures	76

6.19.4.3	Services, Entities and Interfaces	76

6.19.4.4	Issues	76

6.19.5	Solution variant #19.5: 6G System impacts based on the characteristics of AI traffic	77

6.20	Solutions to KI#20	77

6.21	Solutions to KI#21	77

6.22	Solutions to KI#22	77

6.22.0	Solution Category Overview	77

6.23	Solutions to KI#23	79

6.24	Solutions to KI#24	79

6.24.1	Solution variant #24.1: Features applicable for 6G IoT solution.	79

6.24.1.0	Topics addressed and High-level Solution Principles	79

6.24.1.1	Description	79

6.24.1.2	Procedures	80

6.24.1.3	Services, Entities and Interfaces	80

6.24.1.4	Issues	80

6.X	Solutions to KI#X	80

6.X.Y	Solution #X.Y: <Solution Title>	80

6.X.Y.0	High-level solution Principles	80

6.X.Y.1	Description	81

6.X.Y.2	Procedures	81

6.X.Y.3	Impacts on Services, Entities and Interfaces	81

7	Interim agreements	81

7.1	Agreed Principles	81

7.1.Y	Agreed Principles for KI#Y	81

8	Consolidated 6G architecture interim agreements	81

9	Conclusions	81

Annex A:	Work Tasks	83

A.0	Introduction	83

A.0a	Mapping between Work Tasks and Key Issues	83

A.X	Work Task X	83

A.1	Work Task 1	84

A.1.1	Work Task 1.1: Study the support for control signalling for 6G System	84

A.1.2	Work Task 1.2	84

A.1.2.1	SBA framework	84

A.1.2.2	Network Slicing	84

A.1.2.3	User Plane Architecture	85

A.1.2.4	QoS Framework for 6G	85

A.1.2.5	Policy Framework	85

A.1.2.6	Network Exposure	87

A.1.2.7	Network Sharing in the 6G system	88

A.1.2.8	Localized service access	88

A.1.2.9	FWA	88

A.1.3	Work Task 1.3: Non-3GPP access	88

A.1.4	Work Task 1.4: 6G Essential Services	89

A.2	Work Task 2: Migration and Interworking	89

A.3	Work Task 3: AI	90

A.3.0	General	90

A.3.1	WT#3.1: AI for 6G architecture	90

A.3.2	WT#3.2 6G Network for AI:	91

A.4	Work Task 4: Integrated Sensing and Communication	92

A.5	Work Task 5: 6G data framework in SA WG2	92

A.6	Work Task 6: 6G Computing Support	93

A.7	Work Task 7: Support of 6G NTN	94

A.8	Work Task 8: Cellular IoT enablers in 6G	94

Annex B:	Topics for further resolution	96

B.X	Topics for further resolution for KI#X	96

Annex C:	List of submitted solutions	97

C.1	List of submitted solutions for KI#1	97

C.2	List of submitted solutions for KI#2	97

C.3	List of submitted solutions for KI#3	97

C.4	List of submitted solutions for KI#4	97

C.5	List of submitted solutions for KI#5	98

C.6	List of submitted solutions for KI#6	100

C.7	List of submitted solutions for KI#7	100

C.8	List of submitted solutions for KI#8	101

C.9	List of submitted solutions for KI#9	101

C.10	List of submitted solutions for KI#10	101

C.11	List of submitted solutions for KI#11	101

C.12	List of submitted solutions for KI#12	101

C.13	List of submitted solutions for KI#13	102

C.14	List of submitted solutions for KI#14	102

C.15	List of submitted solutions for KI#15	102

C.16	List of submitted solutions for KI#16	102

C.17	List of submitted solutions for KI#17	102

C.18	List of submitted solutions for KI#18	102

C.19	List of submitted solutions for KI#19	103

C.19.1	KI#19 bullets 1&2	103

C.19.2	Void	103

C.19.3	KI#19 bullet 3	103

C.19.4	KI#19 bullet 4	104

C.19.5	KI#19 bullet 5	104

C.20	List of submitted solutions for KI#20	104

C.21	List of submitted solutions for KI#21	104

C.22	List of submitted solutions for KI#22	105

C.23	List of submitted solutions for KI#23	105

C.24	List of submitted solutions for KI#24	106

Annex D:	Change History	107

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The study will work towards goals endorsed at TSG#107 (Mar 2025) to "create lean and streamlined standards for 6G, e.g. by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified".

# 1	Scope

The present document studies system architecture for 6G mobile networks for improvement of existing services and support of new services, to meet the 6G system requirements as captured by TS 22.abc [x].

Editor's note:	The above reference should be replaced with a reference to normative specification, when available. Until the normative work is available, the study can take into account progress made in SA WG1 and TSG RAN.

The study investigates the architectural requirements, assumptions, high-level principles, key issues, as well as solution and system architecture recommendations for 6G architecture.

The conclusions of this study will form the basis for the normative work.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[7]	RP-252912: "Revised SID: Study on 6G Radio", 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025.

[8]	3GPP TS 22.101: "Service aspects; Service principles".

[9]	3GPP TS 22.261: "Service requirements for the 5G system".

[10]	3GPP TS 22.105: "Services and service capabilities".

[11]	3GPP TR 22.870: "Study on 6G Use Cases and Service Requirements".

[12]	3GPP TS 23.380: "IMS Restoration Procedures".

[13]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[14]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[15]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[17]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

6G Core Network (6G CN): In this study, it refers to the Core Network for the 6G System.

NOTE 1:	To what degree 6G CN can be considered as evolution of 5GC or as a new Core Network will be determined during the study.

6G RAN: In this study, it refers to the radio access network for 6G radio access technology (6GR) being studied by RAN WGs RP-252912 [7].

6G System (6GS): 3GPP System consisting of 6G RAN, 6G CN and UE and supports also connectivity of UEs via specific non-3GPP access(es).

NOTE 2:	Which specific non-3GPP access(es) are supported is studied under WT#1.3.

Non-Access Stratum: The Non-Access Stratum (NAS) is the stratum for the control plane between the UE and the 6G CN.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

Editor's note:	This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1.	The framework of SBA specified for 5GC is assumed as a starting point for discussion.

2.	This study assumes that control and user plane of the 6G RAN connects to a single core network type, i.e. 6G CN. It is assumed that the 6G RAN is a single technology framework based on a stand-alone architecture.

NOTE:	This does not exclude network sharing scenarios such as Multi-Operator Core Network.

3.	The 6G system is assumed to maintain the RAN and CN functionality split as in 5GS.

4.	The study aims to avoid duplication of functionality between 6G RAN and 6G CN.

5.	Real-time voice and video services under operator's control and in 3GPP scope (MMTel) will be provided by IMS.

6.	The 6G system is assumed to natively support both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

## 4.2	Architectural Requirements

The followings are architectural requirements of 6G System:

The 6G System shall:

1.	Support roaming.

2.	Support multi-vendor interoperable interfaces.

The 6G System should:

3.	Follow design principles such as: utilization of cloud nativeness, improve sustainability and energy efficiency, robustness and resiliency.

4.	Minimize inter dependencies between NFs when introducing a new feature or procedure for an NF.

The architectural requirements apply to all WTs and corresponding key issues in this study.

# 5	Key Issues

Editor's note:	This clause identifies key issues to be addressed.

## 5.1	Key Issue #1: Study the support for control signalling for 6G System

Key Issue #1.1

1.	Study the support for control signalling for 6G System, including at least the following:

a)	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE 1:	It is assumed that this key issue KI#1.1 bullet 1a covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2:	For the above KI#1.1 bullet 1a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

NOTE 3:	6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signalling interaction between RAN-CN need coordination and alignment with RAN WG3.

NOTE 4:	Overall Interworking and migration aspects are covered by key issues for WT#2 (KI#17). Impact to IWK due to features introduced by this key issue will be covered by this key issue in alignment with WT#2 (KI#17).

Key Issue #1.2

1.	Study the support for control signalling for 6G System, including at least the following:

a)	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 5:	This KI#1.2 bullet 1a can include any transport mechanism such as using NAS or UP or new plane. KI#1.2 bullet 1a can have dependency on other key issues defined for e.g. KI#1.1 bullet 1a, KI#18, KI#19, KI#20, KI#21, KI#22.

NOTE 6:	6G System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signalling interaction between RAN-CN need coordination and alignment with RAN WG3.

NOTE 7:	Overall Interworking and migration aspects are covered by key issues for WT#2 (KI#17). Impact to IWK due to features introduced by this key issue will be covered by this key issue in alignment with WT#2 (KI#17).

NOTE 8:	This KI#1 covers operator services.

NOTE 9:	Solutions may apply for only KI#1.1 or only KI#1.2 or both KI#1.1 and KI#1.2.

## 5.2	Key Issue #2: SBA framework

The following is key issue for SBA related work task.

1	Study whether and how to optimize NF/NF service registration, discovery and selection for efficient message forwarding compared with 5G.

2	Study whether and how to improve NF/NF service resiliency, scalability, efficiency and load balancing, compared with 5G.

NOTE:	This key issue covers generic aspects for SBA framework. The study of other key issues can also result in potential enhancements on the SBA framework and will be studied as part of respective key issues.

## 5.3	Key Issue #3: Support of Network Slicing in the 6G system

This Key Issue aims to study the support of Network Slicing in the 6G system, including the following aspects:

1.	Study the overall design and functionalities of the network slicing in 6G assuming the network slicing in 5GS as starting point for discussions including the following:

a)	Identify and address any areas of possible improvement and simplification of network slicing.

b)	Identify and address any improvements of the application traffic mapping to network slice(s)/user plane connection(s).

NOTE 1:	Any feedback from network slicing deployments can be considered in order to facilitate improvements.

NOTE 2:	Coordination between bullet 1b and the KI#6 on Policy is expected.

2.	Study impacts of the network slicing in 6G on the interworking and migration.

NOTE 3:	Impacts to RAN are possible and coordination with RAN WGs is expected.

## 5.4	Key Issue #4: User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

1.	Whether and how to enhance CP-UP functional split and interaction for better multi-vendor interoperability.

2.	Whether and how to enhance user plane flexibility (by user plane function (re)selection) for different service requirements, session continuity and mobility with consideration of user plane function capability and path performance between access network and data network.

NOTE 1:	This work task may require coordination with RAN WGs for user plane interface.

3.	Whether and how to enhance resilience, scalability, and high availability of user plane function.

NOTE 2:	The outcome of this study may include architectural requirements based on which stage 3 working group can work on the user plane protocols. The stage 3 protocol aspects, including how to design and select user plane protocols, are not in the scope of stage 2.

## 5.5	Key Issue #5: QoS Framework for 6G

This key issue is based on the WT#1.2 on the QoS Framework for 6G. This KI includes following aspects:

1.	Investigate whether and what new functionality in 6G QoS framework is required considering, the emerging new traffic characteristics and application needs, e.g. AI/ML application traffic.

NOTE 1:	Which working groups will determine the characteristics of AI traffic is subject to TSG SA plenary (SA#110) decision.

NOTE 2:	Coordination with KI#19 on 6G network for AI may be needed.

2.	Whether and how to enhance QoS mechanisms to:

a)	support QoS targets fulfilling application's QoS requirements (e.g. dynamic QoS requirements) in a less resource intensive manner than the existing GBR, Delay Critical-GBR.

3.	Study whether and what enhancements are needed to adjust the QoS targets, in case the current QoS targets cannot be met, to minimize the impacts to application operation or user experience.

4.	Study whether and how to support the UE, application and network QoS collaboration to improve awareness in the 6GS of application traffic needs, e.g. traffic pattern, dynamic QoS requirements and to improve awareness in the application/UE of what can be provided by the network, e.g. maximum bitrate.

NOTE 3:	Applications consist of UE aspect and DN aspects.

5.	Whether and how to enhance QoS monitoring, based on the identified needs, e.g. to determine the end-to-end per-packet delay, packet loss rate between the PSA-UPF and the UE for the verification of the QoS target fulfilment.

6.	Study whether and what enhancements are needed to enforce QoS differentiation, e.g. in case of frequent address change of the server, application traffic with multiplexed media flows.

NOTE 4:	QoS aspects of interworking will be considered based on the KI#17 for migration and interworking.

NOTE 5:	This KI will coordinate with SA WG4 and RAN WGs.

NOTE 6:	The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion.

NOTE 7:	This KI may study impacts to the QoS for 6G based on the input identified and provided by other KIs.

## 5.6	Key Issue #6: Policy and charging control framework

To support the Policy and charging control in 6GS, the policy and charging control framework defined in 5G for policies (i.e. SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, the following aspects are to be studied:

1.	Whether and how to simplify the policy and charging control framework for 6G, including:

a)	whether and how to reduce the number of policy control services e.g. the same policy control association for both AM and UE policies or the number of interactions to efficiently provide policies to the different policy enforcement points.

NOTE 1:	The example above does not preclude that other examples are covered in the study phase if the reason for simplification is identified.

b)	whether and how to support the UE to request UE policies from the 6G CN and/or the 6G CN to provide UE policies to the UE.

2.	Whether and how to consider user preferences during UE policy evaluation at UE, without conflicting with network provided UE policies.

NOTE 2:	The statement that refers to considering user preferences refers to the ability to influence the evaluation of UE policies.

3.	Whether and how to improve the External Parameter Provisioning aspects of policy control for 6G.

NOTE 3:	General Authorization of the external parameter provisioning is handled in the KI#7 - Network Exposure.

4.	Whether and how to improve events notification about policies to the AF (e.g. to notify the AF that policy and charging control, e.g. charging key changed).

5.	Whether and what parameters the UE can provide as input for PCC decision, e.g. related to QoS control to the 6G CN. The UE input for PCC decision related to QoS is based on the progress in KI#5 - QoS.

NOTE 4:	The example above does not preclude that other examples are covered in the study phase if those are identified.

NOTE 5:	The AF input for session management defined in 5G can be treated as starting point for discussion.

NOTE 6:	The UE can decide if and when to provide the above input the request to the network.

NOTE 7:	The UE is not assumed to know whether AF input for QoS is available.

NOTE 8:	Whether the UE can provide any input for PCC decisions as the AF provides for UE or AM policies are out of the scope of this bullet.

NOTE 9:	The assumption is that either the AF or the UE provides input for PCC decision for e.g. QoS control.

The following notes are applicable to all bullets in this key issue:

NOTE 10:	If the energy related information, either based on 5G mechanisms, or 6G mechanisms, is available then this key issue will study how to take energy efficiency and consumption aspects into account for policy decisions.

NOTE 11:	This key issue has IWK aspects that are scoped under KI#17 - Migration and Interworking.

NOTE 12:	This key issue can study potential enhancements to the policy control framework for 6G services based on the input identified and provided by other key issues.

NOTE 13:	This key issue depends on the overall 6G roaming architecture based on the output of KI#1 - Support of 6G Control signalling. Solutions for this key issue will address roaming if applicable.

NOTE 14:	Any improvements of the application traffic mapping to network slice(s)/user plane connection(s) are out of the scope of this key issue.

NOTE 15:	Identify and address any feedback from markets where policy control framework technologies are already being commercially deployed.

## 5.7	Key Issue #7: Network Exposure

This key issue is to study whether and how to support a common network exposure framework in 6G. The capabilities that are exposed via the framework fall into one of two categories.

1.	The first category is new capabilities. New capabilities refer to capabilities that are not exposed in the 5G network. This category of exposed capabilities is dependent on other key issues. This key issue will include a review of exposure requirements that derive from other key issues.

2.	The second category refers to capabilities that are exposed in the 5G network and are used as a starting point for discussion of this key issue. This category of exposed capabilities may be dependent on other key issues.

Since this key issue depends on other key issues, time will not be allocated for solutions for this key issue by SA WG2 before SA2 #175.

NOTE 1:	This key issue requires coordination with SA WG3, SA WG6, and SA WG5.

NOTE 2:	This key issue has dependency on service requirements to be specified by SA WG1.

NOTE 3:	Information exposure is based on operator policies/SLAs.

NOTE 4:	Authorization, Privacy protection and user consent aspects require coordination with SA WG3, and application layer user consent aspects may require coordination SA WG6.

Aspects that can be studied under this key issue are:

1.	How to support a common network exposure framework in 6G for the exposure of network capabilities to an AF.

a)	Whether and how to support a common network exposure framework in 6G for the exposure of network capabilities to AFs, in an access independent manner (to the extent that exposure in an access independent manner is possible). The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this key issue.

b)	Whether and how to support co-existence with the 5G exposure framework.

NOTE 5:	The common network exposure framework in 6G in SA WG2 does not duplicate the SA WG6 Framework.

NOTE 6:	Examples of network capabilities include information exposure such as information about UE related events, network events, analytics, and information provisioning such as UE related network configuration parameters and service specific parameters.

c)	Whether and how to support privacy protection, user consent, data anonymization, and service authorization mechanisms.

d)	Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary.

2.	How to support Capability Exposure to the UE and Application Endpoints.

Depending on the outcome of the QoS key issue, this key issue is about whether and how to integrate solutions from the QoS key issue for collaboration between the UE, applications and network in a common exposure framework. As a prerequisite for this key issue, the QoS key issue is expected to determine if specific information needs to be exchanged. This key issue will cover:

a)	Based on the work in the QoS key issue, whether and how to integrate solutions from the QoS key issue into a common exposure framework in 6G in order to improve awareness in applications and the UE of what information can be provided by the network.

NOTE 7:	Exposure should be done in an operator-controlled manner.

NOTE 8:	Solutions to this key issue can be based on the control plane and/or user plane.

b)	Whether there is a need to and how to have the common framework support how to discover what information can be provided by the network and authorize the network to provide specific information to the applications and the UE.

3.	Whether and how to support Intent-Based Exposure.

a)	Whether, how, and what intents should be supported on the northbound interface (i.e. the interface to an AF in a DN) of a common network exposure framework in 6G. The intent-based work in this key issue is limited to the northbound interface of a common network exposure framework in 6G and needs to be coordinated with the AI key issue. AI aspects will be covered under the AI key issues and will not be covered under this key issue.

## 5.8	Key Issue #8: Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

1)	How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

## 5.9	Key Issue #9: Localized Service Access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, which consists of the following:

1.	Whether and how to support localized service access provided via PLMN, and study how to perform authorization and authentication of UE for localized service access, and minimize service interruption during UE mobility, including:

a)	Whether, how and which aspects (e.g. provisioning, updating, retrieval) for the part of subscription (that is needed for localized service access) can be performed/managed via the NFs present locally in the network providing localized service access.

NOTE 1:	Network selection and access control are not to be studied for bullet 1 and bullet 2.

NOTE 2:	For the case when localized service access is provided via PLMN, solutions are expected not to introduce additional UE functionality that is specific for localized service access (i.e. anything that requires UE dependency beyond what is needed for general PLMN access).

NOTE 3:	Authentication aspects need to be coordinated with SA WG3.

2.	Study whether and how to make use of 5G NPN (SNPN and PNI-NPN) for localized service access in 6G.

NOTE 4:	Which of the existing features will be supported will be determined during the study.

NOTE 5:	Access control principles defined for NPN (SNPN and PNI-NPN) in 5G are assumed to be reused. Network selection principles defined for SNPN are assumed to be re-used, but remit stays within CT WG1.

NOTE 6:	Session and service continuity between PLMN and SNPN using direct interface between PLMN and SNPN is out of scope of this work task.

## 5.10	Key Issue #10: FWA / Fixed Wireless Access

The key issue aims to:

1)	Analyse issues encountered in 5G deployments to efficiently support FWA and determine requirements to be taken by other KIs for 6G. The result of this analysis will serve as the basis for architectural requirements to relevant Key Issues related with WT 1.1 and WT 1.2.

NOTE:	As the result of this analysis will serve as the basis for architectural requirements to Key Issues related with other Work Tasks, this analysis needs to conclude for June 2026.

## 5.11	Key Issue #11: Support of non-3GPP access

NOTE 1:	The UE can access the 6G CN via non-3GPP access even if there is no 3GPP access available.

NOTE 2:	Coordination with SA3 for authentication aspects will be needed for this KI.

NOTE 3:	The dependency/interaction of non-3GPP access and 3GPP access should be minimized.

For non-3GPP access:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

2.	Study how to support service continuity between 3GPP access and non-3GPP access in the above bullet(s).

3.	Study how to support Interworking aspects between 5GS and 6GS related to non-3GPP access.

NOTE 4:	Coordination with other KIs is needed.

NOTE 5:	Solutions to above bullets may support connectivity via non-3GPP access only.

NOTE 6:	Solution discussion on Simultaneous connectivity via 3GPP and non-3GPP access with only per flow granularity can only start from SA2#175, but with low priority at SA2#175 if time allows.

For Non/Seamless WLAN Offload:

4.	How to support UE policy necessary for Non-Seamless WLAN Offload (NSWO).

## 5.12	Key Issue #12: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (referring to the mechanisms used in 5GS to support voice services as the starting point for discussion) - the following aspects will be studied:

1.	How the 6GS natively supports IMS based voice over the 6G access network, including how a UE supporting 6G RAN selects and connects to 6G network for voice service.

2.	Whether and how to update the IMS specification to support an IP-CAN in the 6GS.

3.	How to support the voice continuity between 6G RAT and selected 3GPP RATs or between 6G RAT and selected non-3GPP Accesses when 6G RAT supports voice natively.

4.	How to support migration to Vo6G.

5.	Whether and how to support voice services if they are not provided in the serving 6G Network (Assuming this covers both domain selection and RAT/System selection at the time of the call).

6.	How to support the compatibility between 6G CN and the existing deployed IMS with minimum impacts on the existing deployed DIAMETER interfaces (e.g. Rx, Cx, Sh).

NOTE 1:	For bullet 6, coordination with CT is required, including to consider whether SA WG2 or a CT WG leads the study.

NOTE 2:	Support of voice services via 6G NTN need to be coordinated with WT#7 (KI#23).

NOTE 3:	Interworking, migration and service continuity aspects need to be coordinated with WT#2 (KI#17). Interworking with 2G/3G and CS (Circuit switch) for voice are not considered in this Key issue.

NOTE 4:	The existing IMS architecture is used as a starting point for the study.

NOTE 5:	The study will be aligned with RAN and coordinated where needed.

NOTE 6:	This Key issue should consider roaming aspects.

## 5.13	Key Issue #13: Emergency Voice Services for 6G

This key issue will investigate how to support emergency voice services over the 6G system using the capabilities defined for voice service in Key Issue 12.

Additionally, this key issue will study:

1.	The functionality required in the UE and the 6G system to enable emergency voice service for 6GS.

2.	The functionality required in the UE and the 6G system to establish emergency session via 6GS for the emergency voice service.

3.	How to identify emergency services in 6GS in case of non-UE detectable emergency call, such that the required treatment for an emergency call is applied.

4.	How to support a UE in Limited-Service state so that it can establish emergency session via 6GS.

5.	Whether and how to support emergency voice services if they are not provided in the serving 6G Network (Assuming this covers both domain selection and RAT/System selection at the time of the call).

## 5.14	Key Issue #14: Location Services for 6G

This Key Issue will investigate how to support location service in 6G - the following aspects will be studied:

1.	The architecture to support location services in 6GS.

2.	How to support regulatory location services for emergency voice service in 6GS.

3.	How to support location service exposure.

NOTE 1:	The focus in this release of the specifications will be on the required location service capability to support emergency and other regulatory services.

NOTE 2:	A unified location service architecture should be pursued (using 5GS as the starting point for discussion), by considering the location service requirement in this KI and from other KI(s) if any.

NOTE 3:	Coordination with RAN WG for RAN aspects, SA3 for privacy and user consent aspects and SA5 for OAM and charging aspects will be needed.

## 5.15	Key Issue #15: Messaging Services for 6G

For messaging service in 6G this Key Issue will study:

1.	How to support short message services (SMS) in 6GS as per the service requirements specified in TS 22.101 [8], TS 22.261 [9], TS 22.105 [10].

NOTE:	For SMS to Emergency centre support it is assumed that the outcome of 5G-Advanced Rel-20 study will be adopted also for 6G.

## 5.16	Key Issue #16: Other Essential/Regulatory Services for 6G

This Key Issue will study the following services:

1.	How to support Multimedia Priority Services (MPS) in 6GS.

2.	How to provide priority treatment to access control, signalling and media packets delivery for Mission Critical Services (MCX Services) in 6GS.

NOTE 1:	It is assumed that 6GS will support Public Warning System (PWS). Stage 2 capabilities for PWS will be specified by TSG CT.

NOTE 2:	The scope will keep alignment with TSG RAN.

## 5.17	Key Issue #17: Migration and Interworking

This key issue studies how to support migration and interworking, including:

1.	How to support migration to 6GS.

2.	How to support interworking with 5GS.

3.	Whether and how to support interworking with EPS.

NOTE 1:	Whether to support interworking with EPS will depend on SA WG1 requirement.

4.	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is required to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:	The detailed migration study scope and migration options will be coordinated and aligned with RAN.

NOTE 4:	Additional migration options beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied no earlier than June 2026 in alignment with TSG RAN (if needed).

NOTE 5:	It is assumed that interworking for roaming is within the scope of this work task.

NOTE 6:	This work task focuses on general interworking procedures. The specific interworking aspects studied in other KIs (if any) need to follow the general interworking procedures.

## 5.18	Key Issue #18: AI for 6G architecture

Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 1:	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA WG1 work is still in progress, adapting the definition of AI agent from SA WG1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

NOTE 2:	It is assumed that operators can decide whether and which AI capabilities and technologies to deploy in their network.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	Enable the 6G CN to leverage AI capabilities and technologies in the 6G CN (e.g. AI agent), subject to operator policies and configuration and using 6G CN functionalities available in the network:

a)	Determine how to fulfil requests from the UEs or AFs when intent is included and, in order to ensure interoperability of intents and supporting system test cases, determine the constraints on the use and expression of intents sent from the UEs and the AFs so that the intents can be processed and interpreted unambiguously by the AI capable entities in the 6G CN, and define the mechanisms required to support these constraints.

b)	Determine how to fulfil requests when intent is not included.

c)	In order to enable AI capable entities in 6G CN to dynamically compose parts of procedures to fulfil requests from UEs and AFs, determine the design principles and constraints for the modularisation of the procedures for the 6G CN that will be applicable to the system procedures defined in the normative work. How to enable the AI capable entities to compose these procedures in SA2 specifications will be determined by the study.

NOTE 3:	An intent refers to expectations including requirements, goals, conditions, guidelines, and constraints, without specifying how to achieve them. What intent means and how to specify intents in SA2 specifications will be determined by the study. Intent is to be specified as a result of this key issue.

NOTE 4:	Whether to support intents in a 6G CN is up to operator choice. It is assumed it is not required for the MT stack of UE to produce nor understand the intent. The MT stack of UE is assumed to be agnostic to whether or not the network uses 6G CN AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

NOTE 5:	This should not result in duplicated procedures in 3GPP specifications. It is assumed that the 6G study will still describe 6G system procedures for the 3GPP features, without necessarily requiring AI capabilities.

2.	Enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN.

3.	Enable entities in 6G CN to access network AI capabilities provided by 6G NFs.

4.	Enable AI capable entities in 6G CN to access trusted external capabilities provided by AF.

NOTE 6:	Whether and what external capabilities provided by AF is accessed by AI capable entities will be discussed as part of the study.

5.	Enable the monitoring of the performance of all AI capable entities in 6G CN.

6.	Enable the operator to control the network's use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of autonomy based on operational needs, including the option to use or not use any AI capabilities.

7.	Support roaming scenarios, e.g. how visited networks using or not using AI technologies can interwork with home network using or not using AI technologies.

8.	Enable NFs of the 6G CN to have AI/ML capabilities, ML model provisioning, inferencing, training and monitoring.

9.	Enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G.

NOTE 7:	How to process the intent when AI capabilities are not being used is to be studied as part of this key issue.

NOTE 8:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 9:	Further alignments with SA WG1 consolidated requirements may be needed.

NOTE 10:	This key issue does not cover AI agents in UE, which is covered by KI#19.

NOTE 11:	Coordination across the KIs in the 6G study is needed.

NOTE 12:	This key issue will not duplicate the work in the scope of KI#1.

## 5.19	Key Issue #19: 6G Network for AI

This key issue is based on the WT#3.2 about the 6G network for AI.

NOTE 1:	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA WG1 work is still in progress, adapting the definition of AI agent from SA WG1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

3.	Study whether and how to enhance network capability exposure functionalities to AI agent on AF(s).

NOTE 2:	Coordination with KI#7 for Network Exposure is required.

NOTE 3:	During the study, which aspects will belong to SA WG2 or SA WG6, or required coordination with SA WG6 on the application layer aspects will be discussed, if needed. And coordination with SA WG3 on security aspects may be needed.

NOTE 4:	When AI agent is on the UE, this AI agent is assumed not part of the MT. It is also assumed that this KI is not replacing UE to Core Network interaction, NAS.

NOTE 5:	The AI agent in this KI is outside of RAN and Core Network for 6G.

4.	Study whether and how the 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE).

5.	Study the potential system impacts based on the characteristics of AI traffic.

NOTE 6:	Coordination with KI#5 QoS may be needed.

NOTE 7:	System enhancement related to QoS for AI traffic will be handled by KI#5 QoS.

NOTE 8:	Which working groups will determine the characteristics of AI traffic is subject to TSG SA plenary (SA#110) decision.

NOTE 9:	Coordination with SA WG3, SA WG5 and SA WG6 may be needed on AI inferencing and AI training.

NOTE 10:	Coordination with KI#21 and KI#22 may be needed.

NOTE 11:	New service charging part may be coordinated with SA WG5 Charging group.

## 5.20	Key Issue #20: Integrated Sensing and Communication

Study the integration of Sensing and Communication over 3GPP access, covering RAN node and UE operating as 3GPP Sensing Entities in the different sensing modes in alignment with TSG RAN, and considering multiple sources of sensing data.

The following aspects will be studied:

1)	Architecture and functional support for Sensing Service, including:

a)	Sensing Service authorization and revocation.

b)	Discovery and (re-)selection of Sensing function and Sensing Entities (i.e. RAN nodes, UEs), including sensing modes selection.

c)	Configuration parameters and policy provisioning to support sensing service, e.g. to the selected Sensing Entities, Sensing function, etc.

d)	Collection and transport of 3GPP sensing related data for sensing result generation.

e)	Exposure framework of sensing result to the Sensing service consumer (i.e. AFs, UEs, Core NFs).

NOTE 1:	How the Sensing service consumer makes use of the sensing results is not part of this key issue.

2)	Whether and how to support Sensing Service in consideration of the mobility of the UE as sensing entity.

3)	Whether and how to support providing sensing data, which is not obtained from 3GPP radio signals, to the Sensing Function for sensing result generation.

NOTE 2:	This includes sensing data obtained using technologies that are different from 3GPP defined ISAC mechanisms, such as sensing based on IEEE 802.11, camera, sonar, lidar, etc.

NOTE 3:	The scope of this KI assumes RAN node and UE operating as Sensing Entities in different sensing modes in alignment with TSG RAN.

NOTE 4:	Coordination with other KIs will be needed if identified, e.g. Sensing data aspects with KI#21, architectural aspect with KIs corresponding to WT#1, etc.

NOTE 5:	Coordination with RAN WG for RAN aspects, SA WG3 for privacy and user consent aspects, and SA WG5 for OAM and charging aspects will be needed. Coordination with SA WG6 may be needed on aspects to be identified during the study.

NOTE 6:	The scope of this KI will be aligned with RAN for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit WT/KI descriptions and determine if solutions can be discussed in Q2 2026.

## 5.21	Key Issue #21: 6G data framework in SA2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA WG5 is required. Common functionalities for data handling may be identified between SA WG2 and SA WG5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The Key issue is based on WT#5, which includes the following aspects:

1.	Identify high level use cases (e.g. AIML in the core, UE data collection for UE-sided model training, Sensing) potentially subject to the data framework.

NOTE 2:	This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3:	The use of the term "data framework" does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

2.	Study required data framework functionalities to support the identified use cases, e.g.:

a)	data discovery and data registration.

b)	data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution)).

c)	data labelling/metadata handling.

d)	data processing (e.g. data anonymization, data analysis, data generation, etc.).

e)	data storage and retrieval.

f)	data exposure.

The functionalities above may need to consider the quality of data, latency and data volume.

NOTE 4:	The bullet 1 and bullet 2 can be studied in parallel, e.g. each solution is expected to include or refer to the corresponding use case(s).

NOTE 5:	Coordination with SA WG5 may be required for charging aspects.

NOTE 6:	Consideration on MNO's requirements of visibility and controllability during data handling processes is required.

NOTE 7:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

3.	Study potential system impacts of user consent, privacy, security and data governance from SA WG2 architectural and system level perspective for data generated in 6G system corresponding to the use cases and functionalities in aspects 1 and 2.

NOTE 8:	Coordination with SA WG3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA WG2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA WG3. SA WG2 should consider the output of SA WG3 before specifying any system procedure on this topic.

## 5.22	Key Issue #22: 6G Computing Support

Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination among UE, core network and application server, exposure of computing service in the core network, etc.).

In order to support computing, the following aspects need to be studied:

1.	Identification of the architectural requirements and computing resource(s):

a)	Derive architectural requirements for scenario(s) and service requirements defined by SA WG1 to be enabled by the Computing Service.

b)	Whether and how to define the computing resource (e.g. computing resource type and/or status, location of the computing resource, etc.).

NOTE 1:	The use cases and the terms of Computing Service and the Service Hosting Environment defined in TR 22.870 [11] can be used as starting point for further refinement during solution development phase. The detailed computing resource type (e.g. software component, hardware component, etc.) can be identified during study phase.

2.	Enablement/authorization of computing service to UE or AF.

a)	Whether and how to enable Computing Service.

b)	Whether and how the operator network may expose the computing resource related information (e.g. its hosting capability (e.g. using compute resources at Service Hosting Environment)) and/or network metrics to AF.

3.	Coordination of communication and computing, service continuity and QoS aspects:

a)	Whether and how to coordinate (e.g. within or outside the core network) communication (for the traffic transferred over user plane) and computing resource(s) in the Computing Site.

b)	Whether and how to identify, collect/monitor, provision the computing resource related information.

c)	Whether and how to improve the service experience.

d)	Whether there is a need to improve QoS to satisfy Computing Service requirements.

e)	Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g. due to UE mobility, computing load balancing, AF influence etc.).

4.	Discovery and (re-)selection of compute site(s) for the Computing Service:

a)	Whether and how to discover and (re-)select Computing Site(s) for the Computing Service.

NOTE 2:	Whether and how to utilise and/or enhance Edge Computing mechanisms specified in 5G to address the aspects in the scope of this KI will be discussed in the study phase.

NOTE 3:	Which of above aspects should fall into SA WG5 and/or SA WG6 scope will be further identified in study phase, coordination with SA WG5 and SA WG6 may be needed.

NOTE 4:	Potential coordination with other KIs (e.g. KI#1 on service enablement and UE-CN interaction, key issues on QoS, exposure framework and UP aspects, etc.) is needed.

NOTE 5:	Computing Site represents the computing resources in a specific location of network topology. The computing site can offer computing services by allocating its computing resources to run application workloads upon demand of a Computing Service consumer. From networking perspective, the Computing Site is located within 6G CN (via e.g. user plane function in 6G) or Data Network (either owned and controlled by the operator or 3rd party).

## 5.23	Key Issue #23: Support of 6G NTN

Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for this KI will be coordinated and aligned with RAN in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions.

NOTE 2:	This KI has IWK aspects that are scoped under WT#2 (KI#17).

NOTE 3:	This KI has dependency on mobility aspects that are scoped under WT#1.1 (KI#1).

NOTE 4:	This KI has voice aspects that are scoped under KIs corresponding to WT#1.4.

## 5.24	Key Issue #24: Analyse 5GS IoT features and solutions

This key issue will study how to support cellular IoT enablers in 6G including:

1	Analyse whether and which 5GS cellular IoT features are applicable to 6G and identify which of these features that should be further studied.

NOTE 1:	Enhancements in 6G to the selected 5GS IoT features are not precluded.

NOTE 2:	Features driven by this key issue are expected to be generic and may apply to any UEs in 6G.

NOTE 3:	The detailed scope for this key issue will be coordinated and aligned with Massive Communication (IoT) requirements from RAN and SA WG1. Ambient IoT, NB-IoT and non-3GPP access are not in the scope of the key issue.

## 5.X	Key Issue #X: Key Issue Title

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1.1 | 1.2 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| 5.1 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1a |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.2 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.3 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.4 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.5 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.6 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.7 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.8 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.4 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.2 |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.1 |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.2 |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.3 |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 13.2 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| 14.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| 14.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| 15.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 19.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 19.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 19.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 19.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 19.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| 22.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| 24.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.1	Solutions to KI#1

Editor's note:	The content of this clause is FFS.

## 6.2	Solutions to KI#2

Editor's note:	The content of this clause is FFS.

## 6.3	Solutions to KI#3

Editor's note:	The content of this clause is FFS.

## 6.4	Solutions to KI#4

Editor's note:	The content of this clause is FFS.

## 6.5	Solutions to KI#5

### 6.5.0	Summary of Solution Variants

Table 6.5.0-1: Summary of solution variants per Key Issue 5 bullet

| Bullet 1: |  |  |  |
| --- | --- | --- | --- |
| Solution Variant | Title | Solutions Proposals used as a basis (see Annex C, clause C.5, Table C.5-1) | Comments |
| 5.1 | QoS flow considerations for 6G - Variant 1 | 11, 28 | Introducing considerations for QoS concepts for 6G |
| 5.1a | QoS flow considerations for 6G - Variant 2 | 42 | Introducing considerations for QoS concepts for 6G |
| 5.2 | Enhanced QoS handling for delay/error tolerant applications | 22, 26, 32, 38, 42, 48 | Introduce new QoS parameters to support new QoS handling for new 6G services such as delay, error tolerant applications. |
| 5.3 | QoS collaboration for a group of flows/UEs | 7, 19, 34 | The key principle proposed by this solution variant is to support new QoS concept for handling QoS for a group of flows/UEs |
| Bullet 2 |  |  |  |
| 5.4 | Support a QoS flow with dynamic QoS requirements | 1, 8, 12, 17, 20, 23, 24, 29, 33, 35, 36, 42, 46 | - A new type of configuration for a QoS flow is proposed to be supported in 6G which is defined by dynamic QoS requirements. - Multiple QoS Profiles, or range of values for QoS parameter(s) are provided to the 6G RAN and optionally to the UE for this QoS Flow. - The 6G RAN adjusts the QoS based on network condition and/or the input from the UE and/or the UPF. |
| 5.5 | Supporting dynamic QoS requirement by using multiple QoS flows | 24, 28, 46, 49 | - Multiple QoS Flows are associated for a single service data flow . - The 6G UE determine which QoS flow of the multiple QoS flow to use for a service data flow in uplink - The 6G UPF determine which QoS flow of the multiple QoS flow to use for a service data flow in downlink. |
| Bullet 4 |  |  |  |
| 5.6 | Enhanced UE/NW QoS collaboration via a collaboration layer between UE/application and UPF via user plane signalling | 2, 6, 9, 10, 14, 21, 31, 37, 41, 45, 46, 47, 51, 53 | UE/application and 6G UPF use an additional user plane layer (collaboration layer) to exchange additional information (e.g. traffic characteristics, QoS requirements, etc.) for a service data flow, QoS flow or a PDU session. - The additional information provided by the UE/application is used by the network to fine tune/adapt the QoS requirements for the application session. - The additional information provided by the network (UPF) to the UE/application is used by the application in the UE to adapt their behaviour. |
| 5.7 | Enhanced QoS collaboration based on enhanced control plane signalling functionality | 12, 50, 52 | UE exchanges QoS related information with the 6G core network using enhanced control plane signalling. |
| 5.8 | NW/DN QoS collaboration via user plane signalling | 21, 41 | QoS collaboration between the network and the application server in the DN via user plane signalling. |

### 6.5.1	Solution variant #5.1: QoS flow considerations for 6G - Variant 1

#### 6.5.1.0	Topics addressed and High-level Solution Principles

This solution variant addresses Bullet 1 of Key Issue 5.

Solution proposals 11, 28 of Annex C, clause C.5, Table C.5-1, are used as a basis for defining this solution variant.

The key principles proposed by this solution variant are:

-	Introducing considerations of general principles and new QoS concepts for 6G.

#### 6.5.1.1	Description

The main proposal by this solution variant is to propose considerations for a general QoS concept in 6G.

A QoS Flow designates the finest granularity for packet (or PDU) treatment in the 6G System.

It is proposed that for a 6G QoS flow, UL and/or DL packet filters are associated with the 6G QoS flow and the QoS is characterised by:

-	A 6G QoS identifier.

-	Allocation and Retention Priority (ARP) as defined in 5GS.

-	Priority Level as defined in 5GS.

Editor's note:	Additional QoS characteristics are FFS.

Editor's note:	Additional considerations for PDU set based handling is FFS, e.g. QoS flow information may include a packet discard importance which is associated with the description of PSI (PDU set importance).

Editor's note:	The structure of the QoS parameters/characteristics is FFS.

#### 6.5.1.2	Procedures

#### 6.5.1.3	Services, Entities and Interfaces

#### 6.5.1.4	Issues

### 6.5.1a	Solution variant #5.1a: QoS Flow considerations for 6G - Variant 2

#### 6.5.1a.0	Topics addressed and High-level Solution Principles

This solution variant addresses bullet 1 of Key Issue 5.

Solution proposal 42 of Annex C, clause C.5, Table C.5-1, is used as a basis for defining this solution variant.

The key principles proposed by this solution variant are:

-	Introducing considerations of general principles and new QoS concepts for 6G.

#### 6.5.1a.1	Description

The main proposal by this solution variant is to propose considerations for a general QoS concept in 6G.

It is proposed to introduce a concept of a QoS Flow that designates the finest granularity for packet (or PDU) forwarding treatment in the 6G System. All packets mapped to the same QoS Flow are subject to the same packet forwarding treatment (e.g. scheduling policy, queue management policy, RLC configuration, etc.) based on the current QoS configuration to be used.

Each QoS Flow is associated with:

-	6G QoS Identifier (6QI) (mandatory).

-	Optionally, Allocation and Retention Priority (ARP) (when absent, it is assumed that it is provisioned in 6G RAN and referenced by the 6QI). ARP inherits its functionality as specified in 5GS.

-	Optionally, Priority Level (when absent, it is assumed that it is provisioned in 6G RAN and referenced by the 6QI).

-	Optionally, Packet (PDU) Treatment Target (PTT), for example a throughput target (when absent, it is assumed that it is provisioned in 6G RAN and referenced by the 6QI).

6G QoS Identifier (6QI), a scalar that is used as a reference to a specific Packet (PDU) Treatment Target (PTT) (resulting in behaviour, like throughput) to be provided to a QoS Flow. This may be implemented in the 6G Radio Access Network (6G RAN) by the 6QI, by referencing node specific parameters that control the PTT (e.g. scheduling weights, admission thresholds, queue management thresholds, link layer protocol configuration, etc.).

The Priority Level consists of two components:

-	Absolute Priority Level (APL): is used by 6G RAN when all Packet Forwarding Treatment Targets cannot be fulfilled to decide which QoS Flows' targets should be fulfilled first; and

-	Relative Priority Level (RPL): is used by 6G RAN to decide how to distribute resources for the QoS Flows within the same APL.

A Packet (PDU) Treatment Target (PTT) specifies the targets of the transport characteristics of a network connection and details the performance that application data traffic can expect to experience with respect to e.g. throughput and delay.

Editor's note:	The details of the PTT are FFS.

#### 6.5.1a.2	Procedures

#### 6.5.1a.3	Services, Entities and Interfaces

#### 6.5.1a.4	Issues

### 6.5.2	Solution variant #5.2: Enhanced QoS handling for /delay/error tolerant applications

#### 6.5.2.0	Topics addressed and High-level Solution Principles

This solution variant addresses bullet 1 of Key Issue 5.

Solution proposals 22, 26, 32, 38, 42, 48 of Annex C, clause C.5, Table C.5-1, are used as a basis for defining this solution variant.

The key principles proposed by this solution variant are:

-	Introduce new QoS parameters to support new QoS handling for new 6G services such as delay, error tolerant applications.

Editor's note:	QoS handling for AI traffic related application is to be handled based on Key Issue 19 outcome.

#### 6.5.2.0	Description

The main proposal by this solution variant is to support new QoS concepts and new QoS parameters for, delay, error tolerant applications (i.e. when AL-FEC is used).

The main principles are:

-	AF or UE request QoS requirements for the error/delay tolerant application.

-	Option 1: The request may also include an indicator to enable PDU set QoS handling for the service data flow and a PDU set redundancy information for error tolerance.

Editor's note:	It is FFS about the definition of PDU Set redundancy information for error tolerance.

Editor's note:	For error tolerant applications other potential QoS handling considering error tolerance requirements is FFS.

-	Option 2: The request may also include an indicator to enable PDU set QoS handling for the service data flow and include alternative QoS requirements in association with PDU Set Importance.

NOTE:	This option will not impact the principle that the QoS flow is finest granularity of packet treatment.

-	Option 3: The request may include a relaxed delay indication, traffic characteristics (e.g. periodicity, inter-packet arrival interval) for a service data flow.

Editor's note:	The QoS requirements for delay tolerant/error tolerant applications is FFS.

-	The 6G CN provides QoS configurations to the UE, 6G RAN and the 6G UP NF separately.

-	The 6G RAN and 6G UP function use the QoS configuration to prioritize resource efficiency.

-	Option 1: If PDU set handling is enabled, the 6G RAN may consider the PDU set redundancy information for each PDU Set marked by the 6G UPF when delivering this PDU Set.

-	Option 2: If PDU set handling is enabled, the UPF identifies and marks PSI for DL packets sent to RAN. When the current QoS cannot be met, assuming one media service data flow is transmitted via this QoS flow, RAN may determine to change to an alternative QoS with lower priority which are applied to the data packets marked with a certain PSI value.

-	Option 3: Based on QoS configuration (relaxed delay indication, traffic characteristics) from CN, RAN may take them into account for resource management during data transmission.

Editor's note:	Coordination with RAN is needed to define the RAN behaviour.

#### 6.5.2.2	Procedures

#### 6.5.2.3	Services, Entities and Interfaces

#### 6.5.2.4	Issues

### 6.5.3	Solution variant #5.3: QoS collaboration for a group of flows/UEs

#### 6.5.3.0	Topics addressed and High-level Solution Principles

This solution variant addresses bullet 1 of Key Issue 5.

Solution proposals 7, 19, 34 of Annex C, clause C.5, Table C.5-1, are used as a basis for defining this solution variant.

The key principle proposed by this solution variant is to support new QoS concept for handling QoS for a group of flows/UEs.

Editor's note:	The enhancements/delta compared to the existing procedure using MMSID need to be clarified, e.g. how a new Group ID is different from MMSID at concept level.

#### 6.5.3.1	Description

#### 6.5.3.2	Procedures

#### 6.5.3.3	Services, Entities and Interfaces

#### 6.5.3.4	Issues

### 6.5.4	Solution variant #5.4: Support a QoS flow with dynamic QoS requirements

#### 6.5.4.0	Topics addressed and High-level Solution Principles

This solution variant addresses bullets 2 of Key Issue 5.

Solution proposals 1, 8, 12, 17, 20, 23, 24, 29, 33, 35, 36, 42, 46 of Annex C, clause C.5, Table C.5-1, are used as a basis for defining this solution.

The key principles proposed by this solution variant are:

-	A new type of configuration for a QoS flow is proposed to be supported in 6G which is defined by dynamic QoS requirements.

-	Multiple QoS Profiles, or range of values for QoS parameter(s) are provided to the 6G RAN and optionally to the UE for this QoS Flow.

-	The 6G RAN adjusts the QoS based on network condition and/or the input from the UE and/or the UPF.

#### 6.5.4.1	Description

The main principles of this solution variant are:

-	The new configuration for the QoS flow is characterised by multiple QoS requirements which are provided to the 6G RAN, UE and 6G UPF. The multiple QoS requirements may be defined by:

-	Option 1: A range of values for QoS parameters (e.g. min-max PDB as a range) for to a QoS flow.

-	Option 2: Multiple QoS profiles, each profile having different QoS requirements associated to the same QoS flow.

Editor's note:	The information contained in the QoS profile is FFS (e.g. whether it contains GBR and non-GBR QoS profile).

Editor's note:	Additional options are FFS (e.g. combination of Option 1 and Option 2).

-	The new configuration for QoS flow may include additional mapping information.

Editor's note:	The mapping information provided is FFS (e.g. contain a mapping of traffic characteristics/content type to QoS requirements).

-	The 6G PCF decides for a service data flow, a QoS flow with dynamic QoS requirements. This may be based on an input from AF/UE or based on subscription and policies.

-	Option 1: The 6G RAN determines and adjusts the QoS requirement to be used from the multiple QoS requirements of the QoS flow based on certain conditions. The 6G RAN may then inform the UE and UPF of the adjusted QoS. The 6G RAN may adjust the QoS based on:

-	the network conditions.

-	the traffic characteristics provided by the UE and/or 6G UP NF.

-	Option 2: the UE and/or 6G UP NF determines based on the traffic characteristics provided by the UE and/or 6G UP NF, the QoS requirement to be used from the multiple QoS requirements of the QoS flow, and informs RAN which QoS to apply.

Editor's note:	As multiple applications (or service data flows) can share a single QoS flow, how and on what basis QoS requirements are determined for a QoS flow needs to be clarified.

#### 6.5.4.2	Procedures

#### 6.5.4.3	Services, Entities and Interfaces

#### 6.5.4.4	Issues

### 6.5.5	Solution variant #5.5: Supporting dynamic QoS requirement by using multiple QoS flows

#### 6.5.5.0	Topics addressed and High-level Solution Principles

This solution variant addresses bullets 2 of Key Issue 5.

Solution proposals 24, 28, 46, 49 of Annex C, clause C.5, Table C.5-1, are used as a basis for defining this solution variant.

The key principles proposed by this solution variant are:

-	Multiple QoS Flows are associated for a single service data flow.

-	The 6G UE determine which QoS flow of the multiple QoS flow to use for a service data flow in uplink.

-	The 6G UPF determine which QoS flow of the multiple QoS flow to use for a service data flow in downlink.

#### 6.5.5.1	Description

The main procedure is:

-	Based on the 6G PCF PCC Rules, a service data flow is associated with multiple/alternative QoS flows and UE and 6G UPF are provisioned with rules accordingly. The multiple QoS flows are associated with a service data flow identified by one packet filter.

-	In the uplink the UE determines the QoS requirements to be used and uses the QoS rules to determine via which QoS flow to route application traffic.

-	Option 1: The UE decision is opportunistic, i.e. if there is an available established QoS flow that satisfies the QoS requirements of the service data flow then the UE may change the QoS flow and route the data flow via another QoS flow.

-	Option 2: Additionally, QoS rules can contain traffic limit conditions that the UE uses to map traffic to QoS Flows while staying within the constraints of the traffic limit conditions (e.g. traffic limits may be time duration).

-	In the downlink direction the UPF decides via which QoS flow to route the data flow based on the traffic characteristics.

-	If the UPF has received N4 rules with traffic limit conditions that the UPF use to map traffic to QoS flows while staying within the constraints of the traffic limit conditions (e.g. traffic limits may be time duration).

#### 6.5.5.2	Procedures

#### 6.5.5.3	Services, Entities and Interfaces

#### 6.5.5.4	Issues

### 6.5.6	Solution variant #5.6: Enhanced UE/NW QoS collaboration via a collaboration layer between UE/application and UPF via user plane signalling

#### 6.5.6.0	Topics addressed and High-level Solution Principles

This solution variant addresses bullet 4 of Key Issue 5.

Solution proposals 2, 6, 9, 10, 14, 21, 31, 37, 41, 45, 46, 47, 51, 53 of Annex C, clause C.5, Table C.5-1, are used as a basis for defining this solution variant.

The key principles proposed by this solution variant are:

-	UE/application and 6G UPF use an additional user plane layer (collaboration layer) to exchange additional information (e.g. traffic characteristics, QoS requirements, etc.) for a service data flow, QoS flow or a PDU session.

-	The additional information provided by the UE/application is used by the network to fine tune/adapt the QoS requirements for the service data flow.

-	The additional information provided by the network (UPF) to the UE/application is used by the application in the UE to adapt their behaviour.

#### 6.5.6.1	Description

The main principles of this solution variant are:

-	The collaboration layer between the UE/application and 6G UPF may be based on:

-	Option 1: On-path signalling methods (e.g. Connect-UDP, MoQ).

-	Option 2: Using the PMF protocol as described in clause 5.32.5 of TS 23.501 [2] with the necessary enhancements to support collaboration layer.

-	Option 3: An encapsulation header on top of PDU layer of PDU session which can be defined by 3GPP without involvement of IETF.

-	Option 4: Adding a Context-Aware Header between IP and SDAP layer within 6GS between UE and UPF, visible for both RAN and UPF.

-	Option 5: Enable support for IETF SCONE.

-	Option 6: A secure channel based on TS 33.535 [14].

Editor's note:	Other options are FFS.

Editor's note:	How the collaboration layer is established between UE/application and a UPF is FFS.

-	UE includes additional information for the service data flow via the collaboration layer. The additional information may be:

-	A requested QoS.

-	Traffic characteristics.

Editor's note:	The information provided by the UE/application via the collaboration layer is FFS.

Editor's note:	How the UE determines QoS requirements and traffic characteristics is FFS.

-	The 6G UPF:

-	Option 1: Provides additional information to the PCF (via SMF) for Policy determination (e.g. updated QoS rules).

-	Option 2: Provides additional information to the RAN for better resource management of the associated QoS flow.

Editor's note:	Details on additional information provided by the UPF is FFS (e.g. whether the additional information is the information provided by the UE/Application via the collaboration layer).

Editor's note:	Other options are FFS.

-	In addition, based on configuration or AF request the network exposes network related information via the collaboration layer to the UE/Application. The network related information may be:

-	Network status information as per QoS monitoring parameters.

-	An operator configured maximum available data rate.

Editor's note:	The information provided by the network to the UE via the collaboration layer is FFS.

#### 6.5.6.2	Procedures

#### 6.5.6.3	Services, Entities and Interfaces

#### 6.5.6.4	Issues

### 6.5.7	Solution variant #5.7: Enhanced QoS collaboration based on enhanced control plane signalling functionality

#### 6.5.7.0	Topics addressed and High-level Solution Principles

This solution variant addresses bullet 4 of Key Issue 5.

Solution proposals 12, 50, 52 of Annex C, clause C.5, Table C.5-1, are used as a basis for defining this solution.

The key principles proposed by this solution variant are:

-	UE exchanges QoS related information with the 6G core network using enhanced control plane signalling.

#### 6.5.7.1	Description

The main principles of this solution variant are:

-	UE provides additional QoS requirements or traffic characteristics to the core network via control plane signalling that is exchanged between the UE and the 6G SMF.

-	The UE may include additional QoS requirements to allow signalling of individual QoS parameters/characteristic and alternative QoS requirements from the UE to the network.

-	The information provided is of the same level of information that the AF can provide e.g. as that defined in clauses 6.1.3.22 and 6.1.3.27 of TS 23.503 [4].

Editor's note:	Which AF provided information can be treated as the information requested by the UE is FFS.

-	The UE may determine the QoS handling for the detected service data flow based on UE policy.

Editor's note:	How the UE requests additional QoS requirements is FFS.

#### 6.5.7.2	Procedures

#### 6.5.7.3	Services, Entities and Interfaces

#### 6.5.7.4	Issues

### 6.5.8	Solution variant #5.8: NW/DN QoS collaboration via user plane signalling

#### 6.5.8.0	Topics addressed and High-level Solution Principles

This solution variant addresses bullet 4 of Key Issue 5.

Solution proposals 21, 41 of Annex C, clause C.5, Table C.5-1, are used as a basis for defining this solution variant

The key principles proposed by this solution variant are to allow QoS collaboration between the network (6G UPF) and the application server in the DN via user plane signalling.

#### 6.5.8.1	Description

The main principles of this solution variant is that the 6G UPF and the Application Server at DN use a collaboration layer over N6 to exchange additional information. The collaboration layer may be based on:

-	On path signalling methods (e.g. Connect UDP).

-	Using IETF SCONE.

The information provided via the collaboration layer may be:

-	Traffic flow performance information (e.g. QoS monitoring information).

-	Traffic flow characteristics adjustment advice (e.g. Burst Arrival Time and Periodicity adjustment information).

#### 6.5.8.2	Procedures

#### 6.5.8.3	Services, Entities and Interfaces

#### 6.5.8.4	Issues

## 6.6	Solutions to KI#6

### 6.6.0	Mapping of bullets to solution variance

Table 6.6.0-1: Summary of solution variants

| Bullet 1a (Simplification of Policy framework) |  |  | Comments |
| --- | --- | --- | --- |
| 1 | Converge of Policy framework for AM and UE policies | 6.1, 6.2 |  |
| 2 | Converged PCF and convergence of Policy Association and context across policy types | 6.2 |  |
| Bullet 1b (Request and Delivery of UE policies) |  |  |  |
| 1 | UE request for UE policies with UE determined parameters | 6.3 |  |
| 2 | UE policy control without UE determined parameters | 6.4 |  |
| Bullet 2 (User preferences for UE Policies) |  |  |  |
| Bullet 3 (External Parameter Provisioning) |  |  |  |
| Bullet 4 (Event Notification of policies - AF) |  |  |  |
| Bullet 5 (UE inputs for policy decisions) |  |  |  |
| 1 | UE inputs and transport mechanisms | 6.5 |  |

NOTE:	Depending on the progress of other KIs (e.g. KI#3, KI#11) it is required to decide whether there is still UE Policy in 6G.

### 6.6.1	Bullet 1a: Solution variant #6.1: Convergence of Policy Association

#### 6.6.1.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#6, bullet 1a and is extracted from Solutions #1, #2, #3, #5 of Annex C, clause C.6, Table C.6-1.

Following are the high-level solution principles that are proposed:

1.	Enable operators to have single PCF for AM and UE policies and have only one association for a UE from the Access and Mobility Management (MM) module to the PCF; by also considering the association indicators known from the UDM or local policies.

2.	Expose single service API for the services provided to Mobility Management module for both AM and/or UE policies.

3.	In roaming case, the interaction between V-PCF and H-PCF is enhanced to support both AM policy and UE policy.

#### 6.6.1.1	Description

Editor's note:	FFS.

#### 6.6.1.2	Procedures

Editor's note:	FFS.

#### 6.6.1.3	Services, Entities and Interfaces

Editor's note:	FFS.

#### 6.6.1.4	Issues

1.	How (for example number of messages and PSI and the containers) to deliver the UE policies to the UE during the registration procedures.

2.	IWK aspects needs to align.

Editor's note:	How the UE policy containers are delivered to the UE as part of the NAS container, will be aligned with KI#1.1 and KI#1.2.

### 6.6.2	Bullet 1a: Solution variant #6.2: Converged PCF and convergence of Policy Association and context across policy types

#### 6.6.2.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#6, bullet 1a and is extracted from Solutions #3, #4, #5 of Annex C, clause C.6, Table C.6-1.

Following are the high-level solution principles that are proposed:

1.	This solution considers all the bullets from clause 6.6.1.0 and additionally:

a.	Enable operators to deploy the same PCF for AM, UE and SM policies for a UE.

b.	Have only one context for a UE maintained at the PCF which can be used by various network functions e.g. Mobility Management module and Session Management (SM) module for various PDU sessions with a single association; by also considering the association indicators known from the UDM or local policies.

c.	Expose single service API for the services provided to Mobility Management module and Session Management module for handling AM, UE, SM policies (when a single PCF is serving all the DNN, Slices).

d.	A single association is used for the same PCF that is handling multiple PDU sessions for the same UE and if a different PCF is required, e.g. for a specific DNN/Slice , then a new association from the SM would be created to the other PCF.

NOTE 1:	This solution variant also considers improvements to related policies that may be required for supporting additional services that may be outcome of other KI#s (e.g. for KI#21).

NOTE 2:	Based on discussions in KI#1.1, KI#1.2, this solution variant would require further updates and discussion.

Editor's note:	If there are alternatives to select a PCF other than the NRF based mechanisms could be a discussed as a sub variant and is FFS.

#### 6.6.2.1	Description

Editor's note:	FFS.

#### 6.6.2.2	Procedures

Editor's note:	FFS.

#### 6.6.2.3	Services, Entities and Interfaces

Editor's note:	FFS.

#### 6.6.2.4	Issues

1.	How (for example number of messages and PSI and the containers) to deliver the UE policies to the UE during the registration procedures.

Editor's note:	How the UE policy containers are delivered to the UE as part of the NAS container, will be aligned with KI#1.1 and KI#1.2.

2.	IWK aspects needs to align.

3.	Life cycle of the policy context at the PCF (during the PDU session termination for example) would be elaborated and is FFS.

### 6.6.3	Bullet 1b: Solution variant #6.3: UE request for UE policies with UE determined parameters

#### 6.6.3.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#6, bullet 1b and is extracted from Solutions #7, #8, #9, #10, #12, #23 of Annex C, clause C.6, Table C.6-1.

Following are the high-level solution principles that are proposed:

Editor's note:	This solution proposes aspects of URSP/ANDSP, and this is dependent on KI#3/KI#11. Whether URSP/ANDSP shall be used or something new would be considered is dependent on KI3 and is FFS.

1.	UE policy including URSP rules do not need to be pre-provisioned with all the rules but are dynamically requested or provisioned by UE or network.

2.	The UE can request the UE Policies when needed (during registration or after registering to the network) and the network may derive and provide the UE policies based on the received information from the UE.

-	When UE requests provisioning of UE policies (e.g. when detecting a new application traffic, during registration, or any), UE may indicate the requested UE policy e.g. requested policy type, application traffic, etc.

Editor's note:	When UE requests a provisioning of URSP rule(s) for application traffic(s), what information the UE may provide (e.g. traffic descriptors, application category, QoS information required for application traffic, etc.) is FFS.

-	For URSP and ANDSP, validity time may be provided for each and UE may request a policy when validity time expires.

Editor's note:	The exhaustive list of conditions of when the UE would request for policies is to be documented and is FFS.

NOTE:	Existing signalling may be reused and if a new method is required, is FFS.

3.	URSP rules and WLANSP rule may be linked so that for an application, if WLAN access is preferred, WLANSP Rule(s) may be indicated.

Editor's note:	This is dependent on KI#11 and is FFS.

4.	NW may provide VPLMN Specific URSP rules with a list of conditions that indicate when the policies in a Policy Section can be used.

#### 6.6.3.1	Description

Editor's note:	FFS.

#### 6.6.3.2	Procedures

Editor's note:	FFS.

#### 6.6.3.3	Services, Entities and Interfaces

Editor's note:	FFS.

#### 6.6.3.4	Issues

1.	Section 6.6.3.0 and sub sections of clause 6.6.3.1 does not indicate the relevance to bullet 2.

Editor's note:	Relevance to bullet 2 (user preference) needs to be clarified and updated. FFS.

2.	PCF monitoring the conditions and instructing the UE when and for which traffic descriptors re-evaluation is required, thereby minimizing UE monitoring requirements is to be discussed.

### 6.6.4	Bullet 1b: Solution variant #6.4: UE policy control without UE determined parameters

#### 6.6.4.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#6, bullet 1b, 2 and is extracted from Solutions #11 of Annex C, clause C.6, Table C.6-1.

Following are the high-level solution principles that are proposed:

1.	Network learns the policy types the UE supports by UE indicating it.

2.	There is no need for UE to request for UE Policies.

NOTE:	This also aims to have uniform approach for all the UE policy types (for e.g. V2X, Prose) is expected.

3.	As per existing procedures, it shall be network decision on whether to create/update/delete UE Policy and network learns if required if and what policies are available at the UE using the PSI mechanisms.

#### 6.6.4.1	Description

Editor's note:	FFS.

#### 6.6.4.2	Procedures

Editor's note:	FFS.

#### 6.6.4.3	Services, Entities and Interfaces

Editor's note:	FFS.

#### 6.6.4.4	Issues

1.	Depending on the progress of other KIs (e.g. KI#3, KI#11) it is required to decide whether there is still UE Policy in 6G.

2.	The 6G PCF can also store the Policy Set Entries in the 6G Data Framework (dependent on KI#21).

3.	User preferences during UE policy evaluation shall be discussed based on use cases.

### 6.6.5	Bullet 5: Solution variant #6.5: Support UE input for PCC decision

This solution variant addresses KI#6, bullet 5 and is extracted from Solutions #18, #20, #6, #19 of Annex C, clause C.6, Table C.6-1.

Following are the high-level solution principles that are proposed:

1.	UE input may be:

a.	Application information (e.g. applications used by the UE, etc.).

b.	QoS related PCC input which may be discussed in KI#5.

c.	Non-QoS related PCC input: AF input for influencing the traffic routing is reused as the UE input.

NOTE:	The use and the aspect of being made use of these parameters needs to be validated.

Editor's note:	Whether other AF input defined in "Npcf_PolicyAuthorization_Create service operation" in clause 4.2.2 of TS 29.514 [17] or clause 5.2.5.3 of TS 23.502 [3] can be considered is FFS.

Editor's note:	Application information sharing by the UE to the Core network may involve privacy aspects and would be aligned with SA WG3 and is FFS.

2.	UE provides the UE input to the network for PCC decisions (for both ongoing or future PDU sessions) through UE CN interaction.

Editor's note:	Further details on the method of the interaction would be aligned with KI#1, KI#5 and KI#7.

3.	The PCF may consider the inputs from UE for policy determination.

#### 6.6.5.1	Description

Editor's note:	FFS.

#### 6.6.5.2	Procedures

Editor's note:	FFS.

#### 6.6.5.3	Services, Entities and Interfaces

Editor's note:	FFS.

#### 6.6.5.4	Issues

1.	Communication path (for the proposed options) to the core network for transporting UE inputs for PCC decision is FFS.

2.	Each of the proposed transport options of UE inputs aspects (for e.g. authorization at the NEF, mention of CMF relaying to the PCF) needs to be discussed and is FFS and may require co-ordination with KI#1 and SA WG3.

## 6.7	Solutions to KI#7

Editor's note:	The content of this clause is FFS

## 6.8	Solutions to KI#8

### 6.8.0	High Level Summary of Solution variants for KI#8

This clause shows the summary of solution variants for KI#8: 6G Network sharing.

Table 6.8.0-1: Summary of solution variants for KI#8

| Classification of solutions for KI#8: |  |  | Other considerations |
| --- | --- | --- | --- |
| 1 | 6G Multi-Operator Core Network | Solution#1 |  |
| 2 | 6G Indirect Network Sharing | Solution#1 |  |

### 6.8.1	Solution variant #8.1: 6G Multi-Operator Core Network

#### 6.8.1.0	Topics addressed and High-level Solution Principles

This solution is extracted from Solution#1 of Annex C, clause C.8 to propose 6G Multi-Operator Core Network (6G MOCN) mechanism for KI#8 (How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G). It gives the architecture design, the broadcasting information, the general network selection and the basic procedures for 6G MOCN.

5G MOCN is assumed as a starting point for further study on 6G MOCN.

#### 6.8.1.1	Description

Network sharing is a way for operators to share the heavy deployment costs for mobile networks, especially in the roll-out phase. For the network sharing in 6G, 6G Multi-Operator Core Network (6G MOCN) network sharing architecture, in which the 6G radio access network is shared in 6G System, should be designed. The architecture of 6G MOCN where the shared 6G RAN has the direct connection with the 6G CN(s) of the participating operator(s) is illustrated as Figure 6.8.1.1-1.

Figure 6.8.1.1-1: 6G Multi-Operator Core Network (6G MOCN) in which multiple 6G CNs are connected to the same 6G RAN

NOTE:	The inter-PLMN interfaces between the 6G RAN and the 6G CN should follow the conclusion of other KIs (e.g. KI#1, KI#4).

6G MOCN for 6G System, including UE, shared 6G RAN and 6G CN, shall support operators' ability to use more than one PLMN IDs. The shared 6G RAN should broadcast the PLMN IDs of the participating operators for selection by the UE.

A UE that has a subscription to one of the participating operators shall be able to select the PLMN ID of this operator while within the coverage area of the shared network and to receive subscribed services from that operator.

#### 6.8.1.2	Procedures

Editor's note:	It is FFS how to perform the basic procedures to access the network through 6G MOCN after the PLMN selection.

Editor's note:	It is FFS how to perform the mobility procedures between the non-shared network and shared network or between the different shared networks.

#### 6.8.1.3	Services, Entities and Interfaces

Editor's note:	If any enhancement in the clause 6.X.1.2 can be identified, the impacts on services, entities and interfaces can be added.

#### 6.8.1.4	Issues

### 6.8.2	Solution variant #8.2: 6G Indirect Network Sharing

#### 6.8.2.0	Topics addressed and High-level Solution Principles

This solution is extracted from Solution#1 of Annex C, clause C.8 to propose 6G Indirect Network Sharing (6G INS) mechanism for KI#8 (How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G). It gives the architecture design, the broadcasting information, the general network selection and the basic procedures for 6G INS.

5G Indirect Network Sharing is assumed as a starting point for further study on 6G INS.

#### 6.8.2.1	Description

The 6G System should support 6G Indirect Network Sharing (6G INS) between hosting operator and participating operator (see Figure 6.8.2.1-1), in which the 6G RAN is shared. The communication between the shared 6G RAN and the 6G CN of the participating operator is routed through the 6G CN of the hosting operator that connects to the shared 6G RAN.

Figure 6.8.2.1-1: 6G Indirect Network Sharing in which multiple participating operators' CNs connect to hosting operator's CN to share 6G RAN

NOTE:	The interfaces between the 6G RAN and the 6G CN and the inter-PLMN 6G CNs' interfaces should follow the conclusion of other KIs (e.g. KI#1, KI#4).

6G INS, including UE, shared 6G RAN and the hosting operator's 6G CN, shall support operators' ability to use more than one PLMN IDs.

For 6G INS, the shared 6G RAN should broadcast the PLMN ID(s) representing the participating operator(s) for selection by the UE.

A UE that has a subscription to one of the participating operators shall be able to select the PLMN ID representing this operator while within the coverage area of the shared network and to receive subscribed services from that operator.

#### 6.8.2.2	Procedures

Editor's note:	It is FFS how to perform the basic procedures to access the network through 6G INS after the PLMN selection.

Editor's note:	It is FFS how to perform the mobility procedures between the non-shared network and shared network or between the different shared networks.

#### 6.8.2.3	Services, Entities and Interfaces

Editor's note:	If any enhancement in clause 6.8.2.2 can be identified, the impacts on services, entities and interfaces can be added.

#### 6.8.2.4	Issues

## 6.9	Solutions to KI#9

Editor's note:	The content of this clause is FFS.

## 6.10	Solutions to KI#10

Editor's note:	The content of this clause is FFS.

## 6.11	Solutions to KI#11

Editor's note:	The content of this clause is FFS.

## 6.12	Solutions to KI#12

NOTE:	In the description in (sub)clauses of clause 6.12, a "{x}" denotes a reference to submitted solution "x" in Annex C, clause C.12 Table C.12-1.

### 6.12.1	Solution variant #12.1: IMS based voice service using IP-CAN in the 6GS

#### 6.12.1.0	Topics addressed and High-level Solution Principles

The solution addresses KI#12, bullet#1, #2 and #3:

1.	How the 6GS natively supports IMS based voice over the 6G access network, including how a UE supporting 6G RAN selects and connects to 6G network for voice service.

2.	Whether and how to update the IMS specification to support an IP-CAN in the 6GS.

3.	How to support the voice continuity between 6G RAT and selected 3GPP RATs or between 6G RAT and selected non-3GPP Accesses when 6G RAT supports voice natively.

Clause 4.1 bullet #5 is the assumption of this solution:

5.	Real-time voice and video services under operator's control and in 3GPP scope (MMTel) will be provided by IMS. {2}

The solution is mainly based on re-using or enhancing existing TS 23.228 [5] mechanisms. {2} {5} {6}

#### 6.12.1.1	Description

##### 6.12.1.1.1	UE's usage setting

If the UE is configured to support IMS voice, the UE shall include the information element "UE's usage setting" in Registration Request message (or its equivalent message in 6GS). The UE's usage setting indicates whether the UE behaves in a "voice centric" or "data centric" way (Similar to clause 5.16.3.7 of TS 23.501 [2] UE's usage setting) {6} {5} {1}.

##### 6.12.1.1.2	(Access) Domain selection

NOTE:	Aspects related to KI#12 bullet #5 (Whether and how to support voice services if they are not provided in the serving 6G Network (Assuming this covers both domain selection and RAT/System selection at the time of the call)) is addressed in Solution variant #12.2.

During the procedure (e.g. registration procedure (or its equivalent procedure in 6GS)) to get authorization to access the 6G CN, the UE receives an indication of whether IMS voice over PS session is supported. {1} {2}

The serving PLMN 6GS can consider the following aspects to decide whether to send the "IMS voice over PS session supported indication" to the UE during registration procedure (or its equivalent procedure in 6GS):

-	6G RAN capability to support IMS voice service. {1}

-	Roaming agreement on voice service when roaming. {1}

-	Operator local policy. {1} {10}

-	Network configuration. {5}

-	How extended 6GR coverage is. {10}

-	UE capability. {10}

-	HPLMN. {10}

Editor's note:	What aspects to be checked when decide whether to send the "IMS voice over PS session supported indication" to the UE are FFS.

For UE originating calls, the UE performs access domain selection. The UE can take following factors into account for access domain selection decision:

-	Whether "IMS voice over PS session supported indication" is received from 6G core network. {1} {5}

-	The state of the UE in the IMS. The state information shall include: Registered, Unregistered. {10} {5}{1}

-	Whether the UE is expected to behave in a "voice centric" or "data centric" way. {1} {10} {5}

-	UE capability of supporting IMS PS voice. {1}{10}

-	6G CN capability. {2}

-	6G RAN capability. {2}

Editor's note:	What aspects to be taken into account when the UE performs access domain selection are FFS.

If the UE fails to use IMS for voice, e.g. due to not receive "IMS voice over PS session supported indication" from the 6GS, the UE behaves as described below:

-	A UE set to "voice centric" for 6G system shall always try to ensure that Voice service is possible. A voice centric UE unable to obtain voice service in 6G system shall not select a 6GR cell. By disabling capabilities to access 6G system, the UE re-selects to another RAT. {10}{1}{5}

-	A UE set to "data centric" for 6GS may not need to perform reselection to another RAT if voice services cannot be obtained from 6G system. {10}

##### 6.12.1.1.3	UE capability indicates to 6G RAN

The UE indicates it supports IMS voice service to 6G RAN (e.g. 6G voice capability, 5G voice capability) in 6G RRC establishment procedure. {1} {5}

Editor's note:	Whether needed and the details of IMS voice capability in 6G RRC are FFS and may require coordination with RAN WG.

##### 6.12.1.1.4	UE Capability Match Request procedure

The 6GC can re-use the UE Capability Match Request procedure (e.g. as specified in the clause 4.2.8a of TS 23.502 [3]) to check the Voice Support Match Indicator for the UE. {5}

##### 6.12.1.1.5	DNN

The 6GC uses the well-known DNN for IMS. {1}

##### 6.12.1.1.6	Transport of IMS signalling and IMS media traffic

The UE supports IMS voice registers (or its equivalent procedure in 6GS) to the 6GS and requests PDU Session (or its equivalent in 6GS) for IMS service. {6}

As the same principle of existing IMS voice service, the IMS signalling and media are both transported over UP (user plane) of 6GS. The UP for IMS signalling and media supports IP type packets delivery.{1}

The 6GS allows, via interaction with the UE via PCF (or its equivalent NF in 6GS), the establishment, modification and removal of at least: {2} {6}

-	one QoS flow (or its equivalent in 6GS) with the QoS characteristics required for IMS signalling.{2}{6}

-	one QoS flow (or its equivalent in 6GS) with the QoS characteristics required for IMS voice. {2} {6}

-	one QoS flow (or its equivalent in 6GS) with the QoS characteristics required for IMS video. {2}

##### 6.12.1.1.7	Voice continuity

When the UE moves between 6G RAT and 5G RAT, SSC Mode 1 is used and the IMS voice service continuity is achieved by maintaining the IP address for the PDU Session (or its equivalent in 6GS) used for IMS connectivity. Maintaining the IP address ensures that the UE does not need to perform a fresh initial registration to the IMS. The detailed procedure for PDU Session handover between 6G RAT and 5G RAT with IP address continuity will be specified in Key Issue #17: Migration and Interworking. {6} {1} {5}

##### 6.12.1.1.8	P-CSCF discovery and selection

The 6GC network supports P-CSCF discovery and selection for the UE. {1} {6}

The P-CSCF information is pre/locally-configured in 6G core network or is dynamically generated by NRF based service discovery according to IMS DNN, S-NSSAI, and UE location; utilizing similar mechanisms as in 5G following the principles as described in clause 5.16.3.11 of TS 23.501 [2] {1} {2} {5} {6}

The 6GC supports capability to send the P-CSCF address(s) to the UE during the procedure when the UE establishes the PDU Session (or its equivalent in 6GS). It is proposed to use similar mechanisms as 5G for P-CSCF address delivery to the UE, i.e.:

-	the 6G SMF (or its equivalent NF in 6GS) to provide the P-CSCF information in a Protocol Configuration Option (PCO). {6} {5}

The UE then selects a P-CSCF:

-	either using a DHCP procedure; or {6}

-	based on the P-CSCF address(es)/FQDN(s) received from the 6G core network during the PDU Session (or its equivalent in 6GS) establishment for IMS service. {6} {1}

If a list of P-CSCF addresses/FQDNs are provided, the UE selects the P-CSCF based on its local policy. {1}

In roaming scenario:

-	For the LBO roaming scenario:

-	the P-CSCF in visited network is provided and selected. {1}

-	the P-CSCF address(s) information is sent by 6G SMF (or its equivalent NF in 6GS) in the VPLMN. {6}

-	For the HR roaming scenario:

-	the P-CSCF in home network is provided and selected. {1}

##### 6.12.1.1.9	P-CSCF restoration

6GC Core Network can support P-CSCF restoration procedure by using procedures defined by CT4 e.g. similar to what is currently defined in the clause 5.8 of TS 23.380 [12]. {2} {6}

##### 6.12.1.1.10	HSS discovery and selection

IMS subscriptions are stored and managed through the HSS. {6}

The I-CSCF/S-CSCF/IMS-AS uses the HSS discovery and selection functionality to select the appropriate HSS. The HSS discovery and selection principles as specified in Annex AA.3.3 of TS 23.228 [5] are re-used in 6GC. {6}

##### 6.12.1.1.11	PCF aspects

IMS can interact with 6G CN through a 6G PCF (or its equivalent NF in 6GS) for: {2}

-	receiving 6G CN / 6G RAN information (e.g. User location information and/or UE Time Zone);

-	subscribing to 6G CN events (e.g. PLMN Change, IP CAN/RAT change); and

-	provisioning of service information (e.g. request the establishment/release of dedicated QoS flows (or its equivalent in 6GS)) and AF Signalling flow;

-	enabling of IP flows (Gate control);

-	handling of SIP forking.

##### 6.12.1.1.12	Identity aspects

IMS network can support usage of IMS identities, i.e. IMPI and IMPU, which are derived from 6G USIM using 6G identity. {1}

Editor's note:	Whether 6G USIM is required needs coordination with CT WG6.

Editor's note:	Details of 6G identity depends on Key Issue #1: Study the support for control signalling for 6G System.

##### 6.12.1.1.13	Roaming aspects

6GS can support IMS Services for roaming users with: {2}

-	Architecture without IMS-level roaming interfaces (Home routed the UE's IP point of presence is located in the home PLMN as well as whole IMS).

-	Architecture with IMS-level roaming interfaces where the UE's IP point of presence is in VPLMN as well as P-CSCF while rest of IMS are in HPLMN).

##### 6.12.1.1.14	Paging aspects

6GS can support Paging policy differentiation. {2} {6}

The RAN node can support Paging Policy Differentiation (PPD) and Paging Priority to handle specific services like IMS voice during the paging, by reusing Annex E.9 of TS 23.228 [5]. {5}

##### 6.12.1.1.15	Avoid late availability of Vo6G

Reasons for the late availability of VoLTE/VoNR seem to include: {9, 12}

a)	incomplete radio coverage on the new generation RAN. While this could be solved by handover from the new RAN to a legacy RAN this was hampered by:

i)	lack of UE and RAN functionality and/or interoperability testing opportunities for mobile features such as radio measurement on the legacy RAT and 'measurement gap control', etc.

ii)	the need for inter-CN handover functionality on the new CN, e.g. on both SMF and AMF. (The "source adapts to target" concept means that no changes were normally needed on the legacy CN).

b)	issues with connection to equipment in the LI recipients, e.g. the LI authorities' difficulties:

i)	in handling (in their IMS kit) the longer cell IDs in 5GS compared to EPS; and

ii)	installing and running a SEPP.

c)	inflexibility to deploy many new 6G UPFs supporting basic functionalities for voice: {9}

i)	deploy essential functionality for IMS service (e.g. Charging, LI) for 6G UPFs in a large scale;

ii)	combined 6G UPF and 5G UPF for service continuity.

NOTE:	Not all issues are related to standards but related to implementation and IoDT.

To avoid late availability of Vo6G it seems useful to consider how to avoid similar issues arising from the design of the 6G system, for example by:

1)	using the same NFs for 5GS and 6GS for the AMF and SMF.

Editor's note:	Whether using the same NFs for 5GS and 6GS for the AMF and SMF depends on the Key Issue #1: Study the support for control signalling for 6G System.

2)	keeping the same length of cell ID in 6GS as in 5GS and use an additional, optional, parameter to differentiate between NR and 6GR.

Editor's note:	The length of cell ID in 6GS depends on RAN WG(s) decision.

3)	reusing HR roaming/INS like PDU session establishment for establishing a IMS PDU session via 6G UPF to anchor 5G UPF. The service continuity between 6G and 5G can be guaranteed as the anchor 5G UPF will not be changed. {9}

Editor's note:	Whether to adopt the "Cross-system PDU session" is FFS and may depend on Key Issue #1: Study the support for control signalling for 6G System and Key Issue #4: User Plane Architecture.

##### 6.12.1.1.16	Misc aspects

6GS can support: {2}

-	Retrieval of User Status and IMEI.

-	Network initiated session release - P-CSCF initiated.

-	Provide NRF services to IMS NFs.

-	Support of RAN Assisted Codec Adaptation.

The charging aspects will be addressed at a later stage. {2}

The UE sets the RRC establishment cause (e.g. mo-VoiceCall for originating calls) to ensure appropriate priority handling by the RAN. {5}

#### 6.12.1.2	Procedures

#### 6.12.1.3	Services, Entities and Interfaces

### 6.12.2	Solution variant #12.2: IMS Voice Call Fallback Mechanism

#### 6.12.2.0	Topics addressed and High-level Solution Principles

Editor's note:	Whether to adopt the voice fallback solution and which voice fallback option as way forward are FFS.

Editor's note:	Whether and how to do IMS Voice Call Fallback to EPS is to be FFS depending on Key Issue #17: Migration and Interworking.

Editor's note:	The IMS Voice Call Fallback to NR, if needed, may depend on other Key Issue(s), e.g. Key Issue #1: Study the support for control signalling for 6G System and Key Issue #17: Migration and Interworking.

This solution addresses Key Issue #12 bullet 5:

5.	Whether and how to support voice services if they are not provided in the serving 6G Network (Assuming this covers both domain selection and RAT/System selection at the time of the call).

##### 6.12.2.0.1	UE-based fallback mechanism

If

-	the network or the UE are not able to support IMS voice over PS session over 6GR connected to 6GC; and

-	the network and the UE are able to support IMS voice over PS session over NR connected to 5GC;

then the AMF (or its equivalent NF in 6GS) sends the "Support of IMS voice over PS session" indication to the UE during the Registration procedure (or its equivalent procedure in 6GS), and also provides the UE with:

-	"support voice natively in 6GR" indication or "support voice via fallback" indication: {4} {7} {10}

-	regarding the "support voice via fallback", for UE-based fallback mechanism, there are 2 variant options:

[Option 1] UE-based fallback mechanism with fallback by "RAN triggered" interworking (handover or redirection) to NR connected to 5GC at QoS Flow establishment for IMS voice (or its equivalent in 6GS); or {10} {8}

[Option 2] UE-based fallback mechanism with fallback by "UE triggered" interworking procedure to 5GS/EPS, based on: {4} {7}

2a)	"fallback assistance info" provisioned in the registration procedure (or its equivalent in 6GS). {4}

2b)	"fallback assistance info" in 6G RAN broadcasted information. {7}

The "fallback assistance info" can contain NR, frequency or cell information.

##### 6.12.2.0.2	Network-based fallback mechanism

The Network-based fallback mechanism is a mapping of "EPS fallback procedure of 5GS" into "5GS fallback procedure of 6GS". {8}

#### 6.12.2.1	Description

##### 6.12.2.1.1	UE-based fallback mechanism

For UE-based fallback mechanism, for different voice call scenarios, for different [Option 1] and [Option 2] specified in clause 6.12.2.0.1, the following principle applies:

Case 1)	MO call, IDLE UE:

[Option 1] the UE initiates a Service Request with a "voice fallback" indication. then the UE sends a SIP INVITE over the connection, after that the 6GS trigger voice fallback procedure (handover or redirection) to the 5GS. {10}

[Option 2] the UE triggers interworking (e.g. cell reselection) to 5GS according to the "fallback assistance info" and optionally UE measurement result, then initiates the SIP INVITE procedure. {4} {7}

Editor's note:	FFS whether there is technical issue for UE locally initiate cell reselection without any network interaction.

Case 2)	MO call, CONNECTED UE:

[Option 1] same as Case 1) [Option 1]. {10}

[Option 2] the UE initiates a Service Request procedure with a "voice fallback" indication, after the 6GS triggers interworking (handover or redirect) to 5GS with a "voice fallback" indication, the SIP INVITE procedure starts. {4}

Case 3)	MT call, IDLE UE:

[Option 1] Paging message includes "Paging Cause Indication" is sent to UE. If the paging cause is for Voice the UE and network follows the procedures described as for Case 1) [Option 1]. {10}

[Option 2] Paging message includes "Paging Cause Indication" is sent to UE. If the paging cause is for Voice the UE and network follows the procedures described as for Case 1) [Option 2]. {4} {7}

NOTE:	This indication is currently defined in clause 5.38.3 of TS 23.501 [2] but is restricted to be used by UE and network supporting Multi-USIM.

Case 4)	MT call, CONNECTED UE:

[Option 1] FFS.

[Option 2 the 6GS triggers interworking (handover or redirect) to 5GS with a "voice fallback" indication, after switching/interworking to 5GS the UE initiates the SIP INVITE procedure. {4}

##### 6.12.2.1.2	Network-based fallback mechanism

For 6G RRC establishment, the UE reports its IMS voice capability (e.g. 6G voice capability, 5G voice capability) and voiceFallbackIndication to 6G RAN. (It can refer TS 38.331 [15] as an example.) {5}

Editor's note:	Whether needed, and the details of IMS voice capability in 6G RRC are FFS and may require coordination with RAN WGs.

If the 6GR does not support IMS voice natively, it may use the 5GS fallback to support IMS voice, similar to "EPS fallback defined in 5GS", with following characteristics: {5}{8}

-	When a QoS Flow (or its equivalent in 6GS) establishment request for voice reaches the 6G RAN, the 6G RAN  (e.g. based on configuration and UE capability) rejects the request with a specific fallback cause. The 6G RAN may initiate measurement report solicitation from the UE including NG-RAN as measurement target.

-	The 6G RAN triggers 5GS Fallback via handover or redirection.

-	The 6G RAN may send the fallback indication to UE in the Handover command or RRC release.

-	After inter-system to 5GS, the UE may initiate 5G registration procedure (5GS fallback).

-	The 5GS re-initiates the setup of the voice QoS flow (5GS fallback) for IMS voice.

-	The IMS voice call is established over the target system.

#### 6.12.2.2	Procedures

#### 6.12.2.3	Services, Entities and Interfaces

### 6.12.3	Solution variant #12.3: Compatibility with existing deployed IMS

#### 6.12.3.0	Topics addressed and High-level Solution Principles

The solution addresses KI#12, bullet#6:

6.	How to support the compatibility between 6G CN and the existing deployed IMS with minimum impacts on the existing deployed DIAMETER interfaces (e.g. Rx, Cx, Sh).

Editor's note:	Two sub-variants proposed but which one to adopt requires further study.

#### 6.12.3.1	Description

The current definition of the IMS architecture includes reference points based on SBI and Diameter for certain NFs (e.g. P-CSCF, S-CSCF, IMS AS, HSS, DCSF):

[Option 1]

-	Only define the IMS architecture only based on SBI where there is an equivalent reference point based on Diameter:{3}

-	Existing reference points based on SBI between 6G CN and IMS (e.g. N5 or equivalent) and within the IMS architecture shall be kept as the only option, avoiding impacts on existing interfaces based on Diameter (e.g. Rx, Cx, Sh, Sc) {3}

-	Enhancements on procedures over reference points indicated in the previous bullet to support IMS-based services over 6G access shall be based on using only service-based interfaces. {3}

-	Functional parity between SBI and Diameter reference points in IMS shall not be maintained, avoiding duplication of new functionality {3}

[Option 2]

-	6G core networks policy control function supports both Diameter interface (Rx) and Service Based Interfaces (SBI) towards the P-CSCF to enable IMS service: {6} {5}

-	Existing interfaces based on Diameter (e.g. Rx, Cx, Sh, Sc) are maintained to support voice services in 6G.

Editor's note:	It is FFS whether the DIAMETER based interfaces are updated to continue to support IMS basic services once the 6G Core architecture has sufficiently progressed to enable an assessment of the impacts.

#### 6.12.3.2	Procedures

#### 6.12.3.3	Services, Entities and Interfaces

## 6.13	Solutions to KI#13

### 6.13.1	Solution variant #13.1: Support for emergency services via 6GS

#### 6.13.1.0	Topics addressed and High-level Solution Principles

This solution addresses the following aspects of KI#13:

-	The functionality required in the UE and the 6G system to enable emergency voice service for 6GS.

-	The functionality required in the UE and the 6G system to establish emergency session via 6GS for the emergency voice service.

-	How to identify emergency services in 6GS in case of non-UE detectable emergency call, such that the required treatment for an emergency call is applied.

-	How to support a UE in Limited-Service state so that it can establish emergency session via 6GS.

The principles of clause 7.1.Y apply to this solution. In addition:

-	5GC procedures for emergency services described in clause 5.16.4 of TS 23.501 [2] apply.

-	If the network or the UE are not able to support emergency services over PS session over 6GR connected to CN for 6G, but the network and the UE are able to support emergency services over PS session over NR connected to 5GC or E-UTRAN connected to EPC and the 6G system support procedures for emergency services fallback to NR or E-UTRAN or both the 6G Core send an Emergency Services Support toward the UE during the Registration procedure.

#### 6.13.1.1	Description

The 6G RAN node broadcasts support for emergency services using the ims-EmergencySupport flag to indicate that the cell may be considered acceptable by the UE.

When the UE is camped on a cell, and not in limited service state, during Registration procedure, the 6G Core includes an indication for Emergency Services Support within the Registration Accept to the UE. For 3GPP access, the Emergency Services Support indication is valid within the current Registration Area per RAT (i.e. this is to cover cases when the same registration area supports multiple RATs and they have different capability).

NOTE:	The details of 6G registration will be determined by key issue#1.

The Emergency Services Support is configured by the operator according to local regulations and network capabilities. The 6G Core includes Emergency Services Support indicator if the following condition is true within the current Registration Area:

-	The Network is able to support Emergency Services natively over 6GS.

UEs that are in limited-service state initiate the Registration procedure by indicating that the registration is to receive Emergency Services, referred to as Emergency Registration and a Follow-on request is included in the Registration Request to initiate PS session establishment procedure with a Request Type indicating "Emergency Request". UE without a USIM shall use the PEI as identifier. Based on local regulation, the network supporting Emergency Services for UEs in limited-service state provides Emergency Services to these UE, regardless of whether the UE can be authenticated, has roaming or Mobility Restrictions or a valid subscription.

When the UE can detect an emergency session (e.g. by evaluating the SIP-URI or the dialled numbers) it indicates the emergency session to the network.

If the UE fails to detect that the requested SIP-URI or dialled numbers correspond to an emergency session, but the P-CSCF detects that this is a request to establish an emergency session, based upon operator policy, the P-CSCF may reject the session initiation request with an indication that the UE shall select a domain for the emergency session and initiate an emergency session. The detail treatment of non-UE detectable Emergency Session is specified in clause 7.1.2 of TS 23.167 [6].

Emergency Services fallback may be used when the CN for 6G does not indicate support for Emergency Services and indicates support for Emergency Services fallback. The 6G CN includes an Emergency Services Fallback support indicator to indicate whether Emergency Services can be supported via 5GS or EPS. The Emergency Services Fallback support indicator is valid within the current Registration Area per RAT.

#### 6.13.1.2	Procedures

Detailed procedures for Emergency Services establishment via 6GS are FFS.

New procedures for Emergency Services Fallback described in clause 6.13.2 are triggered when the UE receives indication from 6G CN that Emergency Services over PS session is not supported natively over 6GR but through handover or redirection to NR connected to 5GC or E-UTRAN connected to EPC.

#### 6.13.1.3	Services, Entities and Interfaces

Existing UE and 6G CN procedures described in clauses 5.16.4 of TS 23.501 [2] and TS 23.167 [6] apply. In addition:

6G CN:

-	If Emergency Services over PS session are not supported "natively" over 6GR, the 6G CN provides indication to UE in Registration Accept whether Emergency Services Fallback over NR connected to 5GC or E-UTRAN connected to EPC or both are supported.

UE:

-	If Emergency Services over PS session are not supported "natively" over 6GR, and the UE receives indication in Registration Accept that Emergency Services over PS session is supported over NR connected to 5GC or E-UTRAN connected to EPC or both, it executes procedures described in clause 6.13.2.

#### 6.13.1.4	Issues

-	The LS from ETSI TC EMTEL in SP-251285 presents additional requirements on emergency communications which can be invoked for persons with disabilities, including relay services, multiparty session control and translating services. The study of emergency service in 6G should take the afore mentioned requirements into account and identify the potential impacts to 6GS and IMS system.

-	Not fully clear relationship between Emergency Service Indicator and Emergency Services Fallback indicator.

### 6.13.2	Solution variant #13.2: Support for Emergency Services Fallback to NR and E-UTRAN

#### 6.13.2.0	Topics addressed and High-level Solution Principles

This solution addresses the following aspects of KI#13:

-	Whether and how to support emergency voice services if they are not provided in the serving 6G Network (Assuming this covers both domain selection and RAT/System selection at the time of the call).

The principles of clause 7.1.Y apply. In addition:

-	5GC procedures for emergency services described in clause 4.13.4 of TS 23.502 [3] apply.

-	the UE sends a Service Request message indicating that it requires emergency services fallback. If both emergency services fallback to NR connected to 5GC and E-UTRAN connected to EPC are supported (as per what was indicated in the last Registration Accept message), the UE also indicates whether it prefers to fallback to NR connected to 5GC or E-UTRAN connected to EPC.

NOTE:	How the UE is handed over or redirect to 5GS/EPS, depends on the key issue#17.

#### 6.13.2.1	Description

This solution builds upon the principles described in solution variant #13.1 (Support for emergency services via 6GS) and assumes the UE has received indication in Registration Accept from the 6G CN that emergency services over PS session is not "natively" support over 6GR in the system but Emergency Services Fallback through handover or redirection to either NR connected to 5GC or E-UTRAN connected to EPC is supported.

#### 6.13.2.2	Procedures

Figure 6.13.2.2-1 describes the procedure for Emergency Services fallback.

Figure 6.13.2.2-1: Emergency Service Fallback

1.	UE camps on 6GR in the 6G system.

2.	UE has a pending emergency service session request from the upper layers.

3.	If the 6G CN has indicated support for emergency services fallback via the Registration Accept message for the current RAT, the UE sends a Service Request message indicating that it requires emergency services fallback. If both emergency services fallback to NR connected to 5GC and E-UTRAN connected to EPC are supported (as per what was indicated in the last Registration Accept message), the UE also indicates whether it prefers to fallback to NR connected to 5GC or E-UTRAN connected to EPC.

4.	CN for 6G triggers a request for Emergency Services Fallback by executing a procedure in which it indicates to 6G-gNB that this is a fallback for emergency services. The 6G CN based on the support of Emergency Services in EPC or 5GC may indicate the target CN for the RAN node to know whether inter-RAT fallback or inter-system fallback is to be performed.

5.	Based on the target CN if indicated in message 4 or otherwise based on the RAN configuration, one of the following procedures is executed by 6G-gNB:

5a.	6G-gNB initiates handover or redirection to a NR cell.

5b. 6G-gNB initiates handover or redirection to E-UTRAN connected to EPC.

If the redirection procedure is used either in 5a. or 5b. the target CN type (EPC or 5GC) is also conveyed to the UE in order to be able to perform the appropriate NAS procedures (S1 or N1 Mode). The Registration or the Tracking Area Update requests should contain Follow-on request and active flag respectively to indicate that the UE has "user data pending".

6.	After handover or redirection to the target cell the UE establishes a PDU Session / PDN connection for IMS emergency services over PS session and performs the IMS procedures for establishment of an IMS session (e.g. voice).

At least for the duration of the emergency voice call, NR or E-UTRAN are configured to not trigger any handover to 6GR.

#### 6.13.2.3	Services, Entities and Interfaces

UE and 6G CN follow the procedures described.

#### 6.13.2.4	Issues

-	Whether handover to EPS is supported or not depends on KI#17 and KI#12.

## 6.14	Solutions to KI#14

### 6.14.1	Solution variant #14.1: Enhance 5GS LCS architecture for 6GS

#### 6.14.1.0	Topics addressed and High-level Solution Principles

Solution addresses all bullet points from KI:

1.	The architecture to support location services in 6GS.

2.	How to support regulatory location services for emergency voice service in 6GS.

3.	How to support location service exposure.

The high-level principles based on the first solution in #14.5 and solution #14.3 for enhancing 5GC LCS framework for 6GS:

-	enhance LCS framework as defined in TS 23.273 [13] to support location services 6GS;

-	enhance the procedure as defined in clauses 6.1.1, 6.1.3 and 6.10 TS 23.273 [13] for the regulatory location services, including emergency voice service;

-	enhance the mechanism as defined in clause 5.5 of TS 23.273 [13] for location service exposure;

-	enhance 5G LCS architecture for 6G-RAN;

-	new 6G-RAT specific positioning methods support can be added based on RAN WG conclusions;

-	based on first solution in #14.5: optionally, depending on solution to KI#17, reuse the mechanism as defined in clause 6.19 of TS 23.273 [13] for LCS continuity between 6GS and 5GS if needed;

-	based on solution #14.3: introducing a new interface between 6G-RAN and 6G-LMF to handle periodical measurement reports in order to reduce the frequent data transfer load in 6G-AMF in case of periodic data delivery.

NOTE:	Reducing data transfer load in 6G-AMF functionality needs to coordinate with KI#1 discussion, because the topic on offloading 6G AMF is discussed.

#### 6.14.1.1	Description

Figure 6.14.1.1-1: Non-roaming reference architecture for Location Service for 6G (first solution in #14.5)

NOTE:	AF and User Plane is only used for commercial services therefore those are out of scope in this release for this variant.

Figure 6.14.1.1-2: Non-roaming reference architecture for Regulatory Location Service for 6G with new direct interface between 6G-RAN and 6G-LMF (solution #14.3)

#### 6.14.1.2	Procedures

All signalling procedure as defined in TS 23.273 [13] can be enhanced for 6G version of the procedures:

##### 6.14.1.2.1	6GC-MT-LR procedure for the regulatory location service

Figure 6.14.1.2.1-1: 6G-MT-LR procedure for the regulatory location service

Procedure itself is identical with 5G procedure as written in clause 6.1.1 of TS 23.273 [13], but Entities are enhanced with 6G capabilities and support 6G-RAN. Enhancements are listed in clause 6.14.1.3.

##### 6.14.1.2.2	6GC-MT-LR multiple location procedure for the regulatory location service

Figure 6.14.1.2.2-1: 6GC-MT-LR multiple location procedure for the regulatory location service

Procedure itself is identical with 5G procedure as written in clause 6.1.3 of TS 23.273 [13], but Entities are enhanced with 6G capabilities and support 6G-RAN. Enhancements are listed in clause 6.14.1.3.

##### 6.14.1.2.3	6GC-NI-LR Procedure

Figure 6.14.1.2.3-1: 6GC-NI-LR

Procedure itself is identical with 5G procedure as written in clause 6.10.1 of TS 23.273 [13], but Entities are enhanced with 6G capabilities and support 6G-RAN. Enhancements are listed in clause 6.14.1.3.

##### 6.14.1.2.4	6GC-MT-LR Procedure without UDM Query

Figure 6.14.1.2.4-1: 6GC-MT-LR Procedure without UDM Query

Procedure itself is identical with 5G procedure as written in clause 6.10.2 of TS 23.273 [13], but Entities are enhanced with 6G capabilities and support 6G-RAN. Enhancements are listed in clause 6.14.1.3.

##### 6.14.1.2.5	Network Assisted Positioning Procedure with direct periodic measurement report based on solution #14.3

Figure 6.14.1.2.5-1: Network Assisted Positioning Procedure with direct periodic measurement report based on solution #14.3

Extended Network Assisted Positioning Procedure proposed by solution #14.3 to be introduced compared to 5G as enhancement. This procedure allows 6G-RAN and 6G-LMF to establish a direct connection in order to transfer periodical measurements report.

1.	If 6G-LMF decides to ask periodical measurement report from 6G-RAN then 6G-LMF includes LMF NF URI as 6G-LMF address in N1N2messageTransfer message.

2.	6G-AMF forwards the message to 6G-RAN.

3.	6G-RAN identifies that the request from 6G-LMF is for periodic measurement report therefore 6G-RAN initiates a direct connection establishment towards 6G-LMF using the address received in LMF NF URI parameter.

4.	If 6G-LMF accepts the Connection Establishment, then it sends a Connection Acknowledgment message to 6G-RAN.

5.	6G-RAN Response back via 6G-AMF using control plane signals to accept the measurement request.

6.	6G-AMF forwards the Response to 6G-LMF.

7.	6G-RAN provides the location related measurement report to 6G-LMF directly (not via 6G-AMF).

8.	6G-RAN repeats step 7 until 6G-LMF terminates the session via Control Plane.

Procedure is also applicable when 6G-LMF uses non-UE associated positioning methods for periodical location report similar that 5G supports. In this case NonUeN2Transfer like messages are used.

6G-RAN can identify using NF-URI that 6G-RAN has already direct connection with the 6G-LMF who sends the request. In this case 6G-RAN reuses the already existing connection and does not create a new one.

Editor's note:	How does 6G-RAN send periodic measurements reports via the direct 6G-RAN interface and how connection is established is FFS and needs coordination with RAN WG3.

For non-periodic measurements Network Assisted Positioning Procedure defined in clause 6.11.2 of TS 23.273 [13] is used and enhanced with 6G-RAN capabilities.

##### 6.14.1.2.6	Terminate direct connection between 6G-RAN and 6G-LMF based on solution #14.3

If the direct connection between 6G-RAN and 6G-LMF is not used as proposed in solution #14.3, then 6G-LMF can decide to terminate it based on Figure 6.

Figure 6.14.1.2.6-1. Terminate direct connection between 6G-RAN and 6G-LMF based on solution #14.3

1.	6G-LMF decides to terminate direct connection between 6G-RAN and 6G-LMF and sends a termination message via NonUeN2messageTransfare message including 6G-LMF NF_URI.

2.	6G-AMF forwards the message to 6G-RAN.

3.	After 6G-RAN terminates the direct connection, it sends a termination response message to 6G-AMF.

4.	6G-AMF forwards the termination response message to 6G-LMF.

#### 6.14.1.3	Services, Entities and Interfaces

##### 6.14.1.3.1	Entities reused and enhanced from 5G

The following Entities are reused and enhanced from 5G:

-	6G-LMF: 5G LMF enhanced with 6G-RAN capabilities. Based on solution 14.3 this includes a new interface (eNLp) to send periodical measurements data from 6G-RAN to 6G-LMF. 6G-LMF supports 6G Positioning methods.

-	6G-UDM: Enhanced to store UE information resides on 6G network (UE IDs, serving eAMF).

-	6G-GMLC: Enhanced to communicate with 6G-UDM, 6G-AMF, 6G-NEF and to handle UEs reside on 6G access. It is able to select 6G-AMF in case of 6G UE and forwards positioning request to 6G-AMF.

-	6G-AMF: Enhanced to handle UE attached to 6G network and select a 6G capable 6G-LMF for positioning.

-	6G-NEF: Enhanced to expose positioning data received from 6G-GMLC.

-	6G-RAN: It has capability to establish a direct connection towards 6G-LMF for sensing periodic measurements reports.

##### 6.14.1.3.2	Interfaces

-	eNLp: based on solution 14.3 this is a new interface in 6G introduced between 6G-RAN and 6G-LMF in order to receive location related measurement reports periodically.

-	Le: interface towards LCS client is proposed to be kept. Interface is widely used in earlier generations for regulatory purposes mostly using Mobile Location Protocol (MLP).

-	eNL1: interface between 6G-AMF and 6G-LMF to send determine location request for a UE resides on 6G network.

-	eNL2: interface between 6G-GMLC and 6G-AMF to send provide positioning request for a UE resides on 6G network.

-	eNL6: interface between 6G-GMLC and 6G-UDM enhanced to query UE related information (UE IDs, serving 6G-AMF, etc) from 6G-UDM when UE resides on 6G network.

#### 6.14.1.4	Open issues

First solution in #14.5 expects backward compatibility for signalling but solution #14.3 does not state this since different interfaces may be enhanced due to another KIs.

Solution variant depends on the conclusion of KI#1.

### 6.14.2	Solution variant #14.2: Redesign 5G LCS architecture related to AMF (mobility management) involvement

#### 6.14.2.0	Topics addressed and High-level Solution Principles

Solution addresses all bullet points from KI:

1.	The architecture to support location services in 6GS.

2.	How to support regulatory location services for emergency voice service in 6GS.

3.	How to support location service exposure.

The high-level common principles based on the second solution in #14.5 and solution in #14.4:

-	Both solutions propose to reduce LCS interaction with mobility management functionality (AMF / (CRF/MMF)).

-	The 6G GMLC interacts directly with the 6G LMF for Location Service request and results.

-	The location service consumers can request the service directly to the 6G GMLC or via the 6G exposure function.

-	Both solutions propose direct 6G-RAN and LMF connection for interaction between 6G-RAN and LMF.

Both solutions (#14.5, 14.4) provide different ways to solve direct connection between 6G LMF and 6G-RAN/UE.

Additional high-level principles based only on the second solution in #14.5:

-	support a user plane connection over PDU Session from UE to the 6G LMF for the location service support, including the control signalling and the location service data transport;

-	reuse the LPP protocol for the Location Service support between UE and the 6G LMF, with potential extensions to support new 6G specific positioning methods;

-	the interactions between the 6G RAN and 6G LMF depends on the conclusions on RAN and CN interactions;

-	depending on the RAN WG conclusions, to use NRPPa or a similar protocol to support the Location Service control between the RAN node and 6G LMF, supporting the 6G RAN specific positioning methods;

-	the 6G GMLC provides interworking and mobility support of Location Service between the 6G system and 5G or 4G systems.

Additional high-level principles based only on the second solution in #14.4:

-	Support Nld interface between 6G RAN and 6G LMF to transfer LPP message and NRPPa+ message.

#### 6.14.2.1	Description

Figure 6.14.2.1-1: Architecture 6G location service in reference point representation (Sol#14.4)

Figure 6.14.2.1-2: Architecture for 6G location service in SBI representation (Sol#14.4)

Figure 6.14.2.1-3 Non-roaming reference architecture for user-plane based Location Service for 6G (second solution in #14.5)

Figure 6.14.2.1-4 Roaming reference architecture for user-plane based Location Service for 6G (second solution in #14.5)

The interfaces related to Location Service operation are as follows defined for solution #14.5:

-	LCS-UP: The interface between the UE and 6G LMF over a user plane connection. It supports the Location Service protocol, e.g. LPP.

-	NLx: The interface between 6G RAN and 6G LMF for the location service support. The transport mechanism depends on the RAN-CN design for the 6G system. It supports the Location Service protocol, e.g. NRPPa or extensions.

-	NL10: The interface between the 6G GMLC and 6G LMF for Location Service control and results exchange. It is an extension to the NL10 defined in TS 23.273 [13].

-	Le, NL3, NL5, NL8, NL6 are the interfaces as defined in TS 23.273 [13].

-	N33 is similar to the interface defined in TS 23.501 [2] with potential extensions based on solutions to KI#7.

NOTE:	AF is only used for commercial services which is out of scope in this Release.

#### 6.14.2.2	Procedures

##### 6.14.2.2.1	Procedures based on solution #14.4

6.14.2.2.1.0	Positioning Message Transfer

The positioning messages include LPP+ message and NRPPa+ message. The 6G location service architecture supports NLd interface between 6G RAN and 6G LMF to transfer the messages.

Editor's note:	The details of the NLd interface (e.g. NLd interface is P2P interface or service based interface) is FFS and coordination with RAN3 is needed.

The LPP+ message is transferred between Target UE and 6G LMF via the 6G RRC and the NLd interface as follows.

-	DL LPP+ message transfer: Based on cell ID of UE and locally stored TRP information, 6G LMF determines the serving 6G RAN of the Target UE. The 6G LMF sends the LPP+ message to the 6G RAN via NLd interface.

Editor's note:	How does 6G RAN route the LPP+ message to 6G UE is FFS and needs coordination with RAN WGs.

-	UL LPP+ message transfer: 6G UE sends the LPP+ message and routing identifier which is the 6G LMF identifier to 6G RAN. Based on the routing identifier, the 6G RAN sends the LPP+ message to the 6G LMF. The routing identifier is provided by 6G LMF to UE during DL LPP+ message routing.

The NRPPa+ message is transferred between 6G RAN and 6G LMF via the NLd interface as follows:

-	DL NRPPa+ message transfer: 6G LMF selects 6G RAN node(s) based on cell ID of target UE and the locally stored TRP information. And 6G LMF sends the NRPPa+ message to the selected 6G RAN node(s) via NLd interface.

-	UL NRPPa+ message transfer: 6G RAN sends the NRPPa+ message to 6G LMF based on the information of the DL NRPPa+ message (e.g. LMF IP address).

6.14.2.2.1.1	6GC-MT-LR procedure for the regulatory location service

Figure 6.14.2.2.1.1-1 illustrates the general network positioning for the LCS clients external to the PLMN for the regulatory location service for non-roaming scenario.

This procedure is applicable to a request from an LCS client for a current location of the target UE, and it is assumed that the LCS client is authorised to use the location service and no privacy verification is required.

Figure 6.14.2.2.1.1-1: 6GC-MT-LR procedure for the regulatory location service

1.	The external location services client sends a request to the 6G GMLC for a location for the target UE. The request may include the required QoS and Supported GAD shapes.

2.	The 6G GMLC invokes a Nudm_UECM_Get service operation towards the UDM+.

3.	The UDM+ returns the network addresses of the current serving mobility management function (e.g. CRF/MMF).

NOTE:	The CRF/MMF supports mobility management function which is to be studied in KI#1.

4.	The 6G GMLC invokes the Ncrf/mmf_UEInfor_Get Request service operation towards the CRF/MMF to request the information of the Target UE.

5.	If the UE is in CM IDLE state, the CRF/MMF initiates a network triggered Service Request procedure to obtain the cell ID and TAI of the UE.

6.	The CRF/MMF invokes the Ncrf/mmf_UEInfor_Get Response service operation towards the 6G GMLC to provide the cell ID and TAI of the target UE.

7.	The 6G GMLC selects a 6G LMF via NRF+ or based on local configuration.

8.	The 6G GMLC invokes the Nlmf_Location_DetermineLocation service operation towards the 6G LMF to request the current location of the UE. The service operation includes a LCS Correlation identifier, the serving cell identity, the client type, the required QoS, and Supported GAD shapes.

9.	The 6G LMF performs one or more of the positioning procedures described in clause 6.14.2.2.1.2.

10.	The 6G LMF returns the Nlmf_Location_DetermineLocation Response towards the 6G GMLC to return the current location of the UE. The service operation includes the LCS Correlation identifier, the location estimate, its age and accuracy and may include information about the positioning method and the timestamp of the location estimate.

11.	The GMLC sends the location service response to the external location services client. The service operation includes the location estimate, its age and accuracy and may include information about the positioning method and the timestamp of the location estimate.

6.14.2.2.1.2	UE Positioning Procedure

6.14.2.2.1.2.1	UE Assisted and UE Based Positioning Procedure

Figure 6.14.2.2.1.2.1-1 shows a positioning procedure used by a 6G LMF to support UE based positioning, UE assisted positioning and delivery of assistance data. The procedure is based on use of the LPP+ protocol between the LMF and UE.

Figure 6.14.2.2.1.2.1-1: UE Assisted and UE Based Positioning Procedure

1.	The 6G LMF sends Transfer (LPP+ message, UE ID) to 6G RAN.

Editor's note:	The UE ID is FFS.

2.	The 6G RAN node sends Transport (LPP+ message) to 6G UE.

3.	The 6G UE stores any assistance data provided in the LPP+ message and performs any positioning measurements and/or location computation requested by the LPP+ message.

4.	The 6G UE sends Transport (LPP+ message) to 6G RAN.

5.	The 6G RAN sends Transfer (LPP+ message) to 6G LMF.

6.14.2.2.1.2.2	Network Assisted Positioning Procedure

Figure 6.14.2.2.1.2.2-1 shows a procedure that may be used by a 6G LMF to support network assisted and network based positioning. The procedure may be based on NRPPa+ signalling between the LMF and NG-RAN.

Figure 6.14.2.2.1.2.2-1: Network Assisted Positioning Procedure

1.	The 6G LMF sends Transfer (NRPPa+ message) to 6G RAN.

2.	The 6G RAN node obtains any location information for the UE requested in step 1.

3.	The 6G RAN sends Transfer (NRPPa+ message) to 6G LMF.

##### 6.14.2.2.2	Procedures based on solution #14.5

A 6G UE establishes the PDU session for the operator services support, based on the URSP or UE configurations, when it registers to the network.  For a UE that supports LCS, it can discover a 6G LMF to connect to, based on configurations, or a self-formulated FQDN. The connection between the UE and the LMF can be based on TLS, as that used for 5G LCS-UP.

The (R)AN node also establishes the connection to the 6G LMF when needed. The mechanism for such connection establishment depends on other key issue.

Over the connections, the Location Service protocols, e.g. LPP and NRPPa, would be used to manage the location service operations.

The exact procedures to support the location service for different 6G services and emergency voice service, can be defined using the procedures in TS 23.273 [13] as baseline, once those 6G service procedures are available.

#### 6.14.2.3	Services, Entities and Interfaces

##### 6.14.2.3.1	Following Entities and functionalities are defined based on Solution #14.4

The 6G GMLC supports the following functions:

-	Support Le interface to LCS Client.

-	Support privacy check.

-	Obtain serving CRF information of target UE from UDM+.

-	Get cell ID and TAI of Target UE from the 6G mobility management function when location request is received.

NOTE 1:	The 6G mobility management function which is to be studied in KI#1.

-	Select 6G LMF.

-	Send the location request to 6G LMF and receive the location response from 6G LMF.

The 6G LMF supports the following functions:

-	Select positioning method.

-	Select proper 6G RAN node(s) to perform corresponding measurement

-	Obtain positioning measurements via the NLd interface from 6G RAN

-	Calculate location of Target UE.

-	Receive the location request from 6G GMLC and send location response to 6G GMLC.

The 6G CRF supports the following functions:

-	Provide the information of Target UE (e.g. cell ID, TAI) to 6G GMLC.

NOTE 2:	The 6G CRF supports connection management function and message routing function. The 6G CRF is to be studied in KI#1.1.

The 6G RAN supports the following functions:

-	Support NRPPa+/LPP+ message transfer via the NLd interface between 6G RAN and 6G LMF.

##### 6.14.2.3.2	Following Entities and functionalities are defined based on second Solution of #14.5

-	The solution reuses the services provided by the solutions from other key issues, e.g. KI#1, KI#7.

-	6G LMF and 6G GMLC would be used for location service support.

-	6G network exposure function would be used for location service exposure based on conclusion of KI#7.

-	The interfaces defined in TS 23.273 [13] for location services are reused and adapted.

-	NL10 is enhanced to support direct control between 6G GMLC and 6G LMF.

#### 6.14.2.4	Open issues

Two solutions in this alternative propose different way to avoid AMF between 6G LMF and 6G-RAN/UE.

-	Solution of #14.5 proposes to use user plane as only option to send data between 6G LMF and UE, additionally proposes new NLs interface between 6G RAN and 6G LMF to transfer data between those entities.

-	Solution of #14.4 proposes to define a new NLd interface between 6G RAN and 6G LMF to send data between 6G LMF and 6G-RAN/UE.

Also, it is different between these two solutions that #14.5 requires AMF(CRF) interaction from 6G-GMLC to get cell ID and TAI of Target UE. However, it is not clear if those parameters are enough for 6G-LMF selection.

Editor's note:	It is FFS what parameters are required from AMF to select 6G-RAN by 6G LMF.

For solution mentioned in #14.4 it is not clear how LMF selects 6G-RAN without any AMF interaction. The assumption is this part of the solution depends on outcome of other KIs.

Editor's note:	It is FFS how 6G LMF selects 6G-RAN.

Solution variant depends on the conclusion of K#1.

User plane was standardized in rel18 only for commercial services. Now based on Solution in 14.5 used for Variant #2 user plane would be also used for regulatory services.

Editor's note:	It is FFS whether and what enhancements are required for 3GPP User plane solution to be used for regulatory services.

## 6.15	Solutions to KI#15

### 6.15.1	Solution variant #15.1: Support for SMS services via 6GS

#### 6.15.1.0	Topics addressed and High-level Solution Principles

This solution addresses the following aspects of KI#15:

-	How to support short message services (SMS) in 6GS as per the service requirements specified in TS 22.101 [8], TS 22.261 [9], TS 22.105 [10].

The principles of the solution are as follows:

-	5G Architecture to support SMS over NAS described in clause 4.4.2 of TS 23.501 [2] apply.

-	5GC procedures for SMS over NAS described in clause 4.13.3 of TS 23.502 [3] apply.

-	The 5G SMSF can be reused if the evolution is limited to the 5G core. However, if a new core network architecture is required for 6G, an equivalent network function of 5G SMSF should be introduced.

-	Supporting SMS over generic IP-CANs ("SMS over IP") described in clause 4.13.3 of TS 23.204 [16] apply.

#### 6.15.1.1	Description

##### 6.15.1.1.1	SMS over NAS

Following functionality is required for supporting SMS over NAS in 6G System:

-	Support for SMS over NAS transport between UE and AMF (or equivalent for CN for 6G).

-	Support for AMF (or equivalent for CN for 6G) determining the SMSF (or equivalent for CN for 6G) for a given UE.

-	Support for subscription checking and actual transmission of MO/MT-SMS transfer by the SMSF (or equivalent for CN for 6G).

-	Support for MO/MT-SMS transmission for both roaming and non-roaming scenarios.

-	Support for selecting proper domains for MT SMS message delivery including initial delivery and re-attempting in other domains.

During Registration procedure, a UE that wants to use SMS provides an "SMS supported" indication over NAS signalling indicating the UE's capability for SMS over NAS transport. "SMS supported" indication indicates whether UE can support SMS delivery over NAS. If the core network supports SMS functionality, the AMF (or equivalent for CN for 6G) includes "SMS allowed" indication to the UE and whether SMS delivery over NAS is accepted by the network.

SMS is transported via NAS transport message, which can carry SMS messages as payload.

##### 6.15.1.1.2	SMS over IP

The IP-SM-GW acts as an SIP-AS in the IMS domain to provide the protocol interworking for the delivery of the short message between the UE and the Service Centre. All functionalities and interfaces of IP-SM-GW are defined in TS 23.204 [16].

NOTE:	Adaptations required in IMS to support 6GR and CN for 6G will be studied under KI#12.

##### 6.15.1.1.3	Domain selection for UE originating SMS

To allow for appropriate domain selection for SMS delivery, it should be possible to provision UEs with the following HPLMN operator preferences on how an IMS enabled UE is supposed to handle SMS services:

-	SMS is not to be invoked over IP networks: the UE does not attempt to deliver SMS over IP networks. The UE attempts to deliver SMS over NAS signalling.

-	SMS is preferred to be invoked over IP networks: the UE attempts to deliver SMS over IP networks. If delivery of SMS over IP networks is not available, the UE attempts to deliver SMS over NAS signalling.

##### 6.15.1.1.4	Secure SMS

SA WG1 identified security requirements for 6G SMS Services in clause 5.7.3 of TR 22.870 [11], including operator-supported sender verification and SMS integrity protection.

NOTE:	Coordination with SA WG3 and C WGT1 is required to identify the potential impacts to 6G architecture in supporting SMS service with enhanced security mechanisms.

#### 6.15.1.2	Procedures

The procedures for SMS over NAS described in TS 23.502 [3] are reused for 6G SMS over NAS.

#### 6.15.1.3	Services, Entities and Interfaces

Existing UE and SMSF (or equivalent for CN for 6G) described in TS 23.501 [2] apply.

## 6.16	Solutions to KI#16

Editor's note:	The content of this clause is FFS.

## 6.17	Solutions to KI#17

Editor's note:	The content of this clause is FFS.

## 6.18	Solutions to KI#18

Editor's note:	The content of this clause is FFS.

## 6.19	Solutions to KI#19

### 6.19.0	Mapping of Solution variants to KI#19 bullets

Table 6.19-1: Mapping of Solution variants to KI#19 bullets

|  | Key Issues # 19 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | Bullet#1 | Bullet#2 | Bullet#3 | Bullet#4 | Bullet#5 |
| #19.1 | X | X |  |  |  |
| #19.2 | X | X |  |  |  |
| #19.3 |  |  | X |  |  |
| #19.4 |  |  |  | X |  |
| #19.5 |  |  |  |  | X |

### 6.19.1	Solution variant #19.1: 6G CN support the UE AI agent registration, discovery and communication

#### 6.19.1.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#19, bullet 1 and bullet 2.

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

This solution variant is extracted from Solutions#1,2,3,4,5,6,8,9,10,11,12,13,14,15,16,17,18,19, 20, 21 (20 solutions) of Annex C, clause C.19.

The followings are high-level solution principles:

The AI Agent Identification, Authorization, AI Agent Discovery and communication are under the control of 6G CN.

(1)	UE AI agent ID assignment/generation (Optional)

NOTE:	This AI agent ID assignment/generation is not applicable to all solutions.

AI agent Identification Allocation Functionality in 6G CN is responsible for the AI agent ID assignment/generation for specific UE AI agent.

-	Option 1: UE AI agent requests the AI agent Identification Allocation Functionality to assignment AI agent ID via NAS message. (Sol#2, #3, #8, #13, #18)

-	Option 2: On behalf of UE AI agent, the AF may send the UE AI agent registration request to the AI agent Identification Management Functionality, which may trigger the AI agent ID assignment/generation. (Sol#9)

(2)	UE AI agent registration, authentication and authorization.

-	Option1: UE AI agent registers in the 6G AI agent Management Functionality via NAS signalling. The 6G AI agent Management Functionality can authenticate and authorize the registration. (Sol#2, #3, #4, #5, #6, #8, #11, #13, #14, #15, #18, #19)

-	Option2: UE AI agent registration, authentication and authorization with AI agent Management Functionality via user plane (Sol#20, Sol#18)

-	Option3: AF, on behalf of UE, trigger the UE AI agent registration, authentication and authorization. (Sol#8, #9)

Editor's note:	Coordination with SA3 is needed on the authentication.

(3)	UE AI agent discovery

UE AI agent sends discovery request to AI agent Discovery Control Functionality.

-	Option1: via NAS signalling. (Sol#1, #4, #5, #10, #11, #13, #8, #17, #18, #14, #19, #20, #21)

Editor's note:	How the NAS solutions work in the case of UE fallback to 5G network is FFS.

-	Option2: via user plane. (Sol#20)

-	Option3: AF triggered discovery (Sol#8, 9)

(4)	UE AI agent communication

-	One-to-one communication:

-	Option1: The source UE AI agent send traffics to the target UE AI agents via 6G user plane. (Sol#15, #12);

-	UE AI agent to UE AI agent communication via 6G UP/AI agent proxy in network. (Sol#11, #20, #15)

-	During the communication, authorization between UE AI agents is performed and the distributed authorization can be used (Sol#15).

-	Option2: The source UE AI agent send message to the target UE AI agents via 6G CP NF. (Sol#2)

-	Group communication.

The group communication can be supported via user plane.

Editor's note:	What group communication refers to is FFS.

-	Option1: The 6G UE AI agent may request the group communication via NAS signalling to AI agent group management functionality. (Sol#2, #4, #6, #11, #13, #18, #21).

-	Option2: AF may request the group communication to AI agent communication management functionality. (Sol#3, #18).

-	Option3: Agent skill discovery can be used to support the group communication. (Sol#20, #16)

Editor's note:	The definition of the UE AI Agent is FFS.

Editor's note:	Inter-PLMN roaming for UE AI agents is FFS. UE AI agent handling when UE is in roaming is FFS.

Editor's note:	AI agent Management Functionality needs to coordinate with KI#18.

Editor's note:	Whether the above functionalities are separate or combined is FFS.

#### 6.19.1.1	Description

#### 6.19.1.2	Procedures

#### 6.19.1.3	Services, Entities and Interfaces

#### 6.19.1.4	Issues

### 6.19.2	Solution variant #19.2: UE AI agent registration, discovery and communication within application layer of the operator control domain

#### 6.19.2.0	Topics addressed and High-level Solution Principles

This solution variant addresses KI#19, bullet 1 and bullet 2.

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

This solution variant is extracted from Solution#7 ,18 (2 solutions) of Annex C, clause C.19.1.

The followings are high-level solution principles:

-	The AI Agent Identification and Authorization and AI Agent Discovery is performed using application layer signalling that is transported over user plane, as such transparent to the 6G CN or any other CN such as 5G and not impacting NAS. And the application layer is in the operator control domain

-	AI agents can exchange application-layer agent registration/discovery messages with the server over the UP connection

NOTE:	Application layer is under operator control domain.

Editor's note:	The impact for 6GC is FFS and which part will be discussed in SA WG2 is FFS.

#### 6.19.2.1	Description

#### 6.19.2.2	Procedures

#### 6.19.2.3	Services, Entities and Interfaces

#### 6.19.2.4	Issues

### 6.19.3	Solution variant #19.3: Network Capability Exposure Functionalities to AI agent on AF(s)

#### 6.19.3.0	Topics addressed and High-level Solution Principles

This solution variant focuses on KI#19: 6G Network for AI, and the topic addressed is the following:

3.	Study whether and how to enhance network capability exposure functionalities to AI agent on AF(s).

The following are high-level solution principles captured by this solution variant based on the proposals in Solutions #1, #2, #3 and #4 in Annex , clause C.19.3, with further description available in clause 6.19.3.1:

1.	The network capability exposure interactions between 6G CN and external AI agents on AF(s) can be realized via agentic protocols [Solutions #1, #2 and #3 in Annex C, clause C.19.3].

2.	A translation function in the CN is required to enable such interactions, enabling combination of CN-level services into abstractions understandable by AI agents on AF(s) [Solutions #1, #2 in Annex C, clause C.19.3]. The translation function may reside in NEF, AF or in a new NF.

3.	The CN is largely agnostic to any agentic protocol used for exposure towards external AI agents [Solutions #1, #2, #4 in Annex C, clause C.19.3].

4.	The network capability exposure architecture should support intent-based requests e.g. from external AI agents on AFs [Solutions #1, #3, #4 in Annex C, clause C.19.3].

5.	External AI agents on AF(s) may interact with an AI Agent Proxy in the 6G CN [Solution #3 in Annex C, clause C.19.3].

NOTE 1:	The processing of intents within the CN and potential interactions with CN AI Agents are in scope of KI#18.

NOTE 2:	Not all solutions impact the network capability exposure when processing intent.

#### 6.19.3.1	Description

#### 6.19.3.2	Procedures

#### 6.19.3.3	Services, Entities and Interfaces

#### 6.19.3.4	Issues

### 6.19.4	Solution variant #19.4: 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE)

#### 6.19.4.0	Topics addressed and High-level Solution Principles

This solution variant addresses Bullet#4 of KI#19:

4.	Study whether and how the 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE).

Based on the submitted solutions, the following architectural and high-level principles are extracted to enable the 6G CN to provide network AI services to the application in UE or AF.

Common high-level principles for both AI inference and AI training services:

-	Principle 1. Enable in 6G CN to fully support network AI services for applications (including AI model training and/or inference operation).

-	Principle 2. Enable network exposure of AI service for AI model training service and/or inference service to AF.

NOTE 1:	Network capability exposure will be coordinated and addressed under KI#7 Network Exposure.

-	Principle 3. Enable the 6G CN to authorise or reject the AI service request from AF and/or UE.

High-level principles for network AI model training service:

-	Principle 4. Enable the application in UE and/or AF to request the network to provide AI training service.

-	Principle 5. Enable a UE to send request/subscription to the core network to request network AI service for AI model training via e.g. CP, UP

-	Principle 6. Enable an AF to send request/subscription to the core network to request network AI service for AI model training.

-	Principle 7. Enable data collection from different sources for AI model training.

-	Principle 8. Enable 6G CN to send the AI service results to AI service consumer.

High-level principles for network inference service:

-	Principle 9. Enable the application in UE and/or AF to request the network to provide inference service.

-	Principle 10. Enable a UE to send request/subscription to the core network to request network AI service for AI inference, e.g. via CP, UP.

-	Principle 11. Enable an AF to send request/subscription to the core network to request network AI service for AI model inference.

-	Principle 12. Enable data collection from different sources for AI model inference, e.g. via UP or data framework.

-	Principle 13. Enable 6G CN to send the AI service results to AI service consumer.

NOTE 2:	If the request to the core network is transferred via NAS, coordination with KI#1 might be needed.

NOTE 3:	If data framework aspects will be deployed for data collection, coordination with KI# 21 might be needed.

NOTE 4:	Coordination with computing service in KI#22 may be needed.

#### 6.19.4.1	Description

#### 6.19.4.2	Procedures

#### 6.19.4.3	Services, Entities and Interfaces

#### 6.19.4.4	Issues

### 6.19.5	Solution variant #19.5: 6G System impacts based on the characteristics of AI traffic

Editor's note:	Which working groups will determine the characteristics of AI traffic is subject to TSG SA plenary decision.

Editor's note:	This clause will be filled out once characteristics of AI traffic are clarified.

## 6.20	Solutions to KI#20

Editor's note:	The content of this clause is FFS.

## 6.21	Solutions to KI#21

Editor's note:	The content of this clause is FFS.

## 6.22	Solutions to KI#22

### 6.22.0	Solution Category Overview

The solutions for KI#22 are structured as following in Table 6.22.0-1:

Table 6.22.0-1: Solution Category overview

| Solution Groups | Addressed bullet # | Solution category for the group | Variants for each category | Referred input paper in Annex C, clause .22 | Solution # in TR |
| --- | --- | --- | --- | --- | --- |
| 1. Architecture Requirements, | 1a, 1b | (1) Architectural requirements | FFS | Solution #11, 22, 27, 29 | 22.1 |
| Definitions, Location of |  | (2) Computing related definitions | FFS | FFS |  |
| Computing Node/Site |  | (3) Computing Site location | FFS | FFS |  |
| 2. Architecture and Coordination of Communication and Computing | 1b, 3a, 3c, 3d, 3e, 4a | (1) Coordination between Communication and Computing is controlled by 6G CN (e.g. 6G SMF, CMF, CCF, CCCE, etc.): | UE Requested Computing Support in 6G by using SM NAS | Solution #1, 6 (collocating CMF with NRF), 8, 13, 15 (Over CP), 20, 21, 25, 33 (UE Requested), 34 | 22.2 |
|  |  |  | UE Triggered Computing Service Support in 6G by using SM NAS and UP connectivity to a Computing Site in the SHE, an Operator controlled domain other than 6G CN | Solution #7 |  |
|  |  |  | UE Requested Computing Support in 6G by using Computing NAS for Computing Site connecting to 6G UPF | Solution #3, 5, 10, 11, 15, 26, 36, 38, 39, 40 |  |
|  |  |  | UE Requested Computing Support in 6G by using Computing NAS for Computing Site connecting to 6G Data Plane Function | Solution #11, 15 |  |
|  |  |  | UE Requested Computing Support with Computing messages over User Plane | Solution #14, 15, 23 |  |
|  |  |  | UE Requested Computing Support in 6G over new Plane (e.g. service plane) | Solution #16, 26 |  |
|  |  |  | UE Initiated Computing Service Support and SMF Controlled Coordinated selection of UPFs and compute sites | Solution #35 |  |
|  |  |  | AF Requested Computing Support via CP Function | Solution #5, 10, 16, 17, 33 |  |
|  |  | (2) Coordination between Communication and Computing is performed by AF |  | Solution #27, 28, 30 | 22.3 |
|  |  | (3) Use SA6 defined Edge Enabler Layer to support computing in 6G |  | Solution #9 | 22.4 |
| 3. Computing Service Authorization to UE/AF | 2a | (1) UE Policy (i.e. URSP, new Computing Policy) based authorization to UE by PCF/CCF |  | Solution #12, 18, 25 | 22.5 |
|  |  | (2) Non-UE policy based authorization | A. Computing Service Authorization to UE/AF by NEF | Solution #38 | 22.6 |
|  |  |  | B. Computing Service Authorization to UE by AMF | Solution #34 |  |
|  |  | (3) Subscription based authorization without explicit authorization to UE/AF |  | Solution #7, 33 | 22.7 |
|  |  | (4) Computing resource authorization by computing framework (no need for computing resource authorization by 6G CN) |  | Solution #27, 28, 30, 35 | 22.8 |
| 4. Computing Exposure to AF | 2b | (1) Computing Related Information exposure |  | Solution #2, 24, 31, 37 | 22.9 |
|  |  | (2) Communication Delay exposure |  | Solution #24, 28 | 22.10 |
| 5. Computing Resource Monitoring and Management by 6G CN | 3b | (1) CP Function (e.g. 6G SMF, CMF, CCF, etc.) monitors the Computing Resources status in Computing Node/Site (e.g. Computing Node/Site, etc.) |  | Solution #5, 7, 19 | 22.11 |
|  |  | (2) CP Function (e.g. CSMF, etc.) monitors the QoS for Computing Service from other NFs (e.g. from SMF, PCF, NWDAF, etc.) in 6G CN |  | Solution #34 | 22.12 |
|  |  | (3) CP Function (i.e. EASDF) monitors the Computing Resources in DN via AF |  | Solution #32 | 22.13 |

Editor's note:	Solution details and open issues will be added in the next meeting and are FFS. For Solution group #4 and 5, additional categories will be added for solutions that don’t require computing resource management and exposure by 6GS.

## 6.23	Solutions to KI#23

Editor's note:	The content of this clause is FFS.

## 6.24	Solutions to KI#24

### 6.24.1	Solution variant #24.1: Features applicable for 6G IoT solution.

#### 6.24.1.0	Topics addressed and High-level Solution Principles

This solution addresses KI#24 and specifically provides a list of 5GS IoT features that should be further studied and possible enhanced to support 6G IoT.

#### 6.24.1.1	Description

The 5GS CIoT features can be treated as the start point for the 6G IoT study.

Features driven by this key issue are expected to be generic and may apply to any UEs in 6G.

Based on clause 5.31 "Support for cellular IoT" in TS 23.501 [2], this solution has the following analysis and observations.

Following features are necessary for the 6G IoT, so SA WG2 should study these features in this key issue. Enhancements on these features may be needed. The study of a feature below may conclude that not all options supported in 5G will be supported in 6G.

-	5.31.5	Non-IP Data Delivery (NIDD).

-	5.31.7	Power Saving Enhancements.

NOTE 1:	Some of 5G IoT power saving features (e.g. eDRX) have high dependence on RAN WGs and coordination with RAN WGs is needed.

NOTE 2:	Other 5G IoT power saving features have little or no RAN dependence (e.g. MICO).

NOTE 3:	Other RAN led power saving features e.g. LP-WUS etc. may also be supported in 6G and may then require alignment work in SA WG2.

-	5.31.8	High latency communication.

Editor's note:	High latency communication solutions have high dependence on whether data is routed via CP or UP.

-	5.31.9	Support for Monitoring Events.

-	Efficient data communication. Both MO and MT data communication is included and should consider small data and larger data communication aspects. 5G specifies options for the efficient data communication e.g. over the control plane (as in clause 5.31.4 Control Plane CIoT 5GS Optimisation) and over the user plane (including using RRC Inactive with CN buffering and as in clause 5.31.18 User Plane CIoT 5GS Optimisation).

Editor's note:	Whether 6G will support data routing over CP, UP or both is FFS. This feature may also have dependence on which power saving features to be supported in 6G.

NOTE 4:	Study how to enable an AF to send MT data to an IoT device when IP connectivity has been lost due to e.g. inactivity has caused Firewall and/or NAT to interrupt connectivity, or long inactivity has caused entities outside 3GPP to perform cleanup of unused IP connectivity.

-	Roaming.

NOTE 5:	It is expected that support of roaming is studied in other KI as it is a generic 6G feature.

-	Subscription control to prevent SIMs intended for use in low capability devices being used in high capability devices.

The following features have clear RAN dependency and should be led by RAN WGs. Alignment work may be needed in SA W|G2 based on RAN progress on coverage enhancement for 6G RAT.

NOTE 6:	Enhanced Coverage/coverage enhancements were not specified for NR in 5G.

-	5.31.12	Restriction of use of Enhanced Coverage.

-	5.31.13	Paging for Enhanced Coverage.

Features in clause 5.31 "Support for cellular IoT" in TS 23.501 [2] that are listed below are considered not applicable for 6G IoT.

-	5.31.3		Selection, steering and redirection between EPS and 5GS.

-	5.31.10	NB-IoT UE Radio Capability Handling.

NOTE 7:	This KI#24 will not discuss the feature in 5.31.11	Inter-RAT idle mode mobility to and from NB-IoT. Interworking aspects will be handled in KI#17.

-	5.31.17	Inter-UE QoS for NB-IoT.

-	5.31.19	QoS model for NB-IoT.

#### 6.24.1.2	Procedures

#### 6.24.1.3	Services, Entities and Interfaces

#### 6.24.1.4	Issues

The following features need more discussion before agreeing on the applicability for 6G IoT.

-	5.31.6		Reliable Data Service.

-	5.31.14	Support of rate control of user data.

-	5.31.15	Control Plane Data Transfer Congestion Control.

-	5.31.16	Service Gap Control.

-	Reduced Capability signalling.

## 6.X	Solutions to KI#X

### 6.X.Y	Solution #X.Y: <Solution Title>

#### 6.X.Y.0	High-level solution Principles

Editor's note:	A description of which KI(s), and which parts of each KI, the solution addresses should be included, in addition to the high-level solution principles.

Editor's note:	Where possible similar/overlapping solution proposals should be documented as a single solution in the TR.

Documentation of more than one solution from the same company for the same KI is allowed but it shouldn't be expected that all of them will be documented in the TR.

#### 6.X.Y.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). Further (sub-)clause(s) may be added to capture details.

#### 6.X.Y.2	Procedures

Editor's note:	This clause will describe the high-level procedures and information flows for the solution.

#### 6.X.Y.3	Impacts on Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#Y

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in Annex B gets resolved and a principle is agreed.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

# 8	Consolidated 6G architecture interim agreements

Editor's note:	This clause can be used to consolidate system architecture aspects based on the consensus building around solutions proposed in clause 6. Whether to use this clause and how it relates to clause 9 will be determined as the study progresses.

# 9	Conclusions

Editor's note:	This clause will capture conclusions for the study.

By consensus interim agreements can become part of the final conclusions of the study.

Annex A:
Work Tasks

# A.0	Introduction

Editor's note:	This Annex will capture updates to WT scope based on the 6G SID in SP-250806. The plan is to update the Study Item Description with updated Work Task descriptions from this Annex.

This Annex contains clarifications of Work Task scope, as preparation for Key Issue drafting.

# A.0a	Mapping between Work Tasks and Key Issues

Table A.0a-1: Mapping of Work Tasks to Key Issues

| Work | Key Issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tasks | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| WT#1.1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 SBA |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 network slicing |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 user plane |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 QoS |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 policy framework |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 network exposure |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 network sharing |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 localized service access |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.2 FWA |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.3 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WT#1.4 |  |  |  |  |  |  |  |  |  |  |  | X | X | X | X | X |  |  |  |  |  |  |  |  |
| WT#2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| WT#3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| WT#3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| WT#4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| WT#5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| WT#6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| WT#7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| WT#8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |

# A.X	Work Task X

Editor's note:	Description of Work Task X. Clauses can be used if needed.

# A.1	Work Task 1

## A.1.1	Work Task 1.1: Study the support for control signalling for 6G System

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE 1:	It is assumed that WT#1.1 bullet 1a covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2:	For the above bullet 1a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

2.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3:	This WT#1.1 bullet 2a can include any transport mechanism such as using NAS or UP or new plane. WT1.1 bullet 2a can have dependency on other work tasks e.g. WT#1.1 bullet 1a, WT#3, WT#4, WT#5, WT#6.

NOTE 4:	This WT#1.1 covers operator services.

## A.1.2	Work Task 1.2

### A.1.2.1	SBA framework

The following is work scope for SBA related work task.

1	Study whether and how to optimize NF/NF service registration, discovery and selection for efficient message forwarding compared with 5G.

2	Study whether and how to improve NF/NF service resiliency, scalability, efficiency and load balancing, compared with 5G.

NOTE:	This WT covers generic aspects for SBA framework. The study of other WTs can also result in potential enhancements on the SBA framework and will be studied as part of respective WTs.

### A.1.2.2	Network Slicing

The scope of the network slicing work task includes:

1.	Study the overall design and functionalities of the network slicing in 6G assuming the network slicing in 5GS as starting point for discussions including the following:

a.	Identify and address any areas of possible improvement and simplification of network slicing.

b.	Identify and address any improvements of the application traffic mapping to network slice(s)/user plane connection(s).

NOTE 1:	Coordination between bullet 1b and the WT 1.2 on Policy is expected.

2.	Study impacts of the network slicing in 6G on the interworking and migration.

NOTE 2:	Impacts to RAN are possible and coordination with RAN WGs is expected.

### A.1.2.3	User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

1.	Whether and how to enhance CP-UP functional split and interaction for better multi-vendor interoperability.

2.	Whether and how to enhance user plane flexibility (by user plane function (re)selection) for different service requirements, session continuity and mobility with consideration of user plane function capability and path performance between access network and data network.

NOTE 1:	This work task may require coordination with RAN WG for user plane interface.

3.	Whether and how to enhance resilience, scalability, and high availability of user plane function.

NOTE 2:	The outcome of this study may include architectural requirements based on which stage 3 working group can work on the user plane protocols. The stage 3 protocol aspects, including how to design and select user plane protocols, are not in the scope of stage 2.

### A.1.2.4	QoS Framework for 6G

1.	Investigate whether and what new functionality in 6G QoS framework is required considering, the emerging new traffic characteristics and application needs, e.g. AI/ML application traffic.

NOTE 1:	Which working groups will determine the characteristics of AI traffic is subject to SA plenary (SA#110) decision.

NOTE 2:	Coordination with WT#3.2 may be needed.

2.	Whether and how to enhance QoS mechanisms to:

a.	Support QoS targets fulfilling application’s QoS requirements (e.g. dynamic QoS requirements) in a less resource intensive manner than the existing GBR, Delay Critical-GBR.

3.	Study whether and what enhancements are needed to adjust the QoS targets, in case the current QoS targets cannot be met, to minimize the impacts to application operation or user experience.

4.	Study whether and how to support the UE, application and network QoS collaboration to improve awareness in the 6GS of application traffic needs, e.g. traffic pattern, dynamic QoS requirements and to improve awareness in the application/UE of what can be provided by the network, e.g. maximum bitrate.

NOTE 3:	Applications consist of UE aspect and DN aspects.

5.	Whether and how to enhance QoS monitoring, based on the identified needs, e.g. to determine the end-to-end per-packet delay, packet loss rate between the PSA-UPF and the UE for the verification of the QoS target fulfilment.

6.	Study whether and what enhancements are needed to enforce QoS differentiation, e.g. in case of frequent address change of the server, application traffic with multiplexed media flows.

NOTE 4:	QoS aspects of interworking will be considered based on the WT#2.

NOTE 5:	This WT will coordinate with SA WG4 and RAN WGs.

NOTE 6:	The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion.

NOTE 7:	This WT may study impacts to the QoS for 6G based on the input identified and provided by other WTs.

### A.1.2.5	Policy Framework

To support the Policy Framework in 6GS, the policy and charging control Framework defined in 5G for policies (i.e. SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, the following aspects are to be studied:

1.	Whether and how to simplify the policy control framework for 6G, including:

a.	whether and how to reduce the number of policy control services e.g. the same policy control association for both AM and UE policies or the number of interactions to efficiently provide policies to the different policy enforcement points.

NOTE 1:	The example above does not preclude that other examples are covered in the study phase if the reason for simplification is identified.

b.	whether and how to support the UE to request UE policies from the 6G CN and/or the 6G CN to provide UE policies to the UE.

2.	Whether and how to consider user preferences during UE policy evaluation at UE, without conflicting with network provided UE policies.

NOTE 2:	the statement that refers to considering user preferences refers to the ability to influence the evaluation of UE policies.

3.	Whether and how to improve the External Parameter Provisioning aspects of policy control for 6G.

NOTE 3:	General Authorization of the external parameter provisioning is handled in the WT#1.2 Network Exposure.

4.	Whether and how to improve events notification about policies to the AF (e.g. to notify the AF that e.g. charging key changed).

5.	Whether and what parameters the UE can provide as input for PCC decision, e.g. related to QoS control to the 6G CN. The UE input for PCC decision related to QoS is based on the progress in WT#1.2 QoS.

NOTE 4:	The example above does not preclude that other examples are covered in the study phase if those are identified.

NOTE 5:	The AF input for session management defined in 5G can be treated as starting point for discussion.

NOTE 6:	The UE can decide if and when to provide the above input the request to the network.

NOTE 7:	The UE is not assumed to know whether AF input for QoS is available.

NOTE 8:	Whether the UE can provide any input for PCC decisions as the AF provides for UE or AM policies are out of the scope of this bullet.

NOTE 9:	The assumption is that either the AF or the UE provides input for PCC decision for e.g. QoS control.

The following notes are applicable to all bullets in this WT:

NOTE 10:	If the energy related information, either based on 5G mechanisms, or 6G mechanisms, is available then this WT will study how to take energy efficiency and consumption aspects into account for policy decisions.

NOTE 11:	This WT has IWK aspects that are scoped under WT#2.

NOTE 12:	This WT can study potential enhancements to the policy control framework for 6G services based on the input identified and provided by other WTs.

NOTE 13:	This WT depends on the overall 6G roaming architecture based on the output of WT#1.1. Solutions for this WT will address roaming if applicable.

NOTE 14:	Any improvements of the application traffic mapping to network slice(s)/user plane connection(s) are out of the scope of this WT.

NOTE 15:	Identify and address any feedback from markets where policy control framework technologies are already being commercially deployed.

### A.1.2.6	Network Exposure

This work task is to study whether and how to support a common network exposure framework in 6G. The capabilities that are exposed via the framework fall into one of two categories.

1.	The first category is new capabilities. New capabilities refer to capabilities that are not exposed in the 5G network. This category of exposed capabilities is dependent on other work tasks. This work task will include a review of exposure requirements that derive from other work tasks.

2.	The second category refers to capabilities that are exposed in the 5G network and are used as a starting point for discussion of this work task. This category of exposed capabilities may be dependent on other work tasks.

Since this work task depends on other work tasks, time will not be allocated for solutions for this work task by SA WG2 before SA2#175.

NOTE 1:	This work task requires coordination with SA WG3, SA WG6, and SA WG5.

NOTE 2:	This work task has dependency on service requirements to be specified by SA WG1.

NOTE 3:	Information exposure is based on operator policies/SLAs.

NOTE 4:	Authorization, Privacy protection and user consent aspects require coordination with SA WG3, and application layer user consent aspects may require coordination SA WG6.

Aspects that can be studied under this work task are:

1.	How to support a common network exposure framework in 6G for the exposure of network capabilities to an AF.

a.	Whether and how to support a common network exposure framework in 6G for the exposure of network capabilities to AFs, in an access independent manner (to the extent that exposure in an access independent manner is possible). The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task.

b.	Whether and how to support co-existence with the 5G exposure framework.

NOTE 5:	The common network exposure framework in 6G in SA2 does not duplicate the SA WG6 Framework.

NOTE 6:	Examples of network capabilities include information exposure such as information about UE related events, network events, analytics, and information provisioning such as UE related network configuration parameters and service specific parameters.

c.	Whether and how to support privacy protection, user consent, data anonymization, and service authorization mechanisms.

d.	Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary.

2.	How to support Capability Exposure to the UE and Application Endpoints.

Depending on the outcome of WT#1.2 (QoS), this work task is about whether and how to integrate WT#1.2 (QoS) solutions for collaboration between the UE, applications and network in a common exposure framework. As a prerequisite for this work task, WT#1.2 (QoS) is expected to determine if specific information needs to be exchanged. This work task will cover:

a.	Based on the work in WT#1.2 (QoS), whether and how to integrate solutions from WT#1.2 into a common exposure framework in 6G in order to improve awareness in applications and the UE of what information can be provided by the network.

NOTE 7:	Exposure should be done in an operator-controlled manner.

NOTE 8:	Solutions to this work task can be based on the control plane and/or user plane.

b.	Whether there is a need to and how to have the common framework support how to discover what information can be provided by the network and authorize the network to provide specific information to the applications and the UE.

3.	Whether and how to support Intent-Based Exposure.

a.	Whether, how, and what intents should be supported on the northbound interface (i.e. the interface to an AF in a DN) of a common network exposure framework in 6G. The intent-based work in this work task is limited to the northbound interface of a common network exposure framework in 6G and needs to be coordinated with WT#3. AI aspects will be covered under WT#3 and will not be covered under this work task.

### A.1.2.7	Network Sharing in the 6G system

Study on how to support network sharing in 6G, including the following aspects:

-	How to support the following network sharing architectures in 6G: Multi-Operator Core Network in 6G and Indirect Network Sharing in 6G.

### A.1.2.8	Localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, which consists of the following:

1.	Whether and how to support localized service access provided via PLMN, and study how to perform authorization and authentication of UE for localized service access, and minimize service interruption during UE mobility, including

a.	Whether, how and which aspects (e.g. provisioning, updating, retrieval) for the part of subscription (that is needed for localized service access) can be performed/managed via the NFs present locally in the network providing localized service access.

NOTE 1:	Network selection and access control are not to be studied for bullet 1 and bullet 2.

NOTE 2:	For the case when localized service access is provided via PLMN, solutions are expected not to introduce additional UE functionality that is specific for localized service access (i.e. anything that requires UE dependency beyond what is needed for general PLMN access).

NOTE 3:	Authentication aspects need to be coordinated with SA WG3.

2.	Study whether and how to make use of 5G NPN (SNPN and PNI-NPN) for localized service access in 6G.

NOTE 4:	Which of the existing features will be supported will be determined during the study.

NOTE 5:	Access control principles defined for NPN (SNPN and PNI-NPN) in 5G are assumed to be reused. Network selection principles defined for SNPN are assumed to be re-used, but remit stays within CT WG1.

NOTE 6:	Session and service continuity between PLMN and SNPN using direct interface between PLMN and SNPN is out of scope of this work task.

### A.1.2.9	FWA

Study support for Fixed Wireless Access in 6G:

1)	Analyse issues encountered in 5G deployments to efficiently support FWA and determine requirements to be taken by other WT for 6G. The result of this analysis will serve as the basis for architectural requirements to relevant Key Issues related with WT 1.1 and WT 1.2.

NOTE:	As the result of this analysis will serve as the basis for architectural requirements to Key Issues related with other Work Tasks, this analysis needs to conclude for June 2026.

## A.1.3	Work Task 1.3: Non-3GPP access

Study whether and how to support and/or enhance non-3GPP access in 6G as well as session continuity between 3GPP access and non-3GPP access.

NOTE 1:	The UE can access the 6G CN via non-3GPP access even if there is no 3GPP access available.

NOTE 2:	Coordination with SA3 for authentication aspects will be needed for this WT.

NOTE 3:	The dependency/interaction of non-3GPP access and 3GPP access should be minimized.

WT#1.3.1. For non-3GPP access:

1.	Study how to support untrusted non-3GPP access in 6G System architecture.

2.	Study how to support service continuity between 3GPP access and non-3GPP access in the above bullet(s).

3.	Study how to support Interworking aspects between 5GS and 6GS related to non-3GPP access.

NOTE 4:	Coordination with other WTs is needed.

NOTE 5:	Solutions to above bullets may support connectivity via non-3GPP access only.

NOTE 6:	Solution discussion on Simultaneous connectivity via 3GPP and non-3GPP access with only per flow granularity can only start from SA2#175, but with low priority at SA2#175 if time allows.

WT#1.3.2. For Non-Seamless WLAN Offload:

1.	How to support UE policy necessary for Non-Seamless WLAN Offload (NSWO).

## A.1.4	Work Task 1.4: 6G Essential Services

Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

# A.2	Work Task 2: Migration and Interworking

Study migration and interworking, including

1.	How to support migration to 6GS.

2.	How to support interworking with 5GS.

3.	Whether and how to support interworking with EPS.

NOTE 1:	Whether to support interworking with EPS will depend on SA1 requirement.

4.	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is required to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:	The detailed migration study scope and migration options will be coordinated and aligned with RAN.

NOTE 4:	Additional migration options beyond stand-alone, MRSS and inter-RAT mobility between NR and 6GR, will be studied no earlier than June 2026 in alignment with TSG RAN (if needed).

NOTE 5:	It is assumed that interworking for roaming is within the scope of this work task.

NOTE 6:	This work task focuses on general interworking procedures. The specific interworking aspects studied in other WTs (if any) need to follow the general interworking procedures.

# A.3	Work Task 3: AI

## A.3.0	General

Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE:	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA WG1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

## A.3.1	WT#3.1:	AI for 6G architecture

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

NOTE 1:	It is assumed that operators can decide whether and which AI capabilities and technologies to deploy in their network.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	Enable the 6G CN to leverage AI capabilities and technologies in the 6G CN (e.g. AI agent), subject to operator policies and configuration and using 6G CN functionalities available in the network:

1.1	Determine how to fulfil requests from the UEs or AFs when intent is included and, in order to ensure interoperability of intents and supporting system test cases, determine the constraints on the use and expression of intents sent from the UEs and the AFs so that the intents can be processed and interpreted unambiguously by the AI capable entities in the 6G CN, and define the mechanisms required to support these constraints.

1.2	Determine how to fulfil requests when intent is not included.

1.3	In order to enable AI capable entities in 6G CN to dynamically compose parts of procedures to fulfil requests from UEs and AFs, determine the design principles and constraints for the modularisation of the procedures for the 6G CN that will be applicable to the system procedures defined in the normative work. How to enable the AI capable entities to compose these procedures in SA2 specifications will be determined by the study.

NOTE 2:	An intent refers to expectations including requirements, goals, conditions, guidelines, and constraints, without specifying how to achieve them. What intent means and how to specify intents in SA WG2 specifications will be determined by the study. Intent is to be specified as a result of this work task.

NOTE 3:	Whether to support intents in a 6G CN is up to operator choice. It is assumed it is not required for the MT stack of UE to produce nor understand the intent. The MT stack of UE is assumed to be agnostic to whether or not the network uses 6G CN AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

NOTE 4:	This should not result in duplicated procedures in 3GPP specifications. It is assumed that the 6G study will still describe 6G system procedures for the 3GPP features, without necessarily requiring AI capabilities.

2.	Enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN.

3.	Enable entities in 6G CN to access network AI capabilities provided by 6G NFs.

4.	Enable AI capable entities in 6G CN to access trusted external capabilities provided by AF.

NOTE 5:	Whether and what external capabilities provided by AF is accessed by AI capable entities will be discussed as part of the study.

5.	Enable the monitoring of the performance of all AI capable entities in 6G CN.

6.	Enable the operator to control the network's use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of autonomy based on operational needs, including the option to use or not use any AI capabilities.

7.	Support roaming scenarios, e.g. how visited networks using or not using AI technologies can interwork with home network using or not using AI technologies.

8.	Enable NFs of the 6G CN to have AI/ML capabilities, ML model provisioning, inferencing, training and monitoring.

9.	Enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G.

NOTE 6:	How to process the intent when AI capabilities are not being used is to be studied as part of this work task.

NOTE 7:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 8:	Further alignments with SA1 consolidated requirements may be needed.

NOTE 9:	This sub work task does not cover AI agents in UE, which is covered by WT3.2.

NOTE 10:	Coordination across the WTs in the 6G study is needed.

NOTE 11:	This work task will not duplicate the work in the scope of WT#1.1.

## A.3.2	WT#3.2 6G Network for AI:

1.	Study whether and how to support an AI agent on a UE to discover another AI agent on a different UE via the 6G network(s).

2.	Study whether and how to enable communication for AI agents on different UEs via the 6G network(s) e.g. identification and authorization of an AI agent on a UE.

3.	Study whether and how to enhance network capability exposure functionalities to AI agent on AF(s).

NOTE 1:	Coordination with WT#1.2 for Network Exposure is required.

NOTE 2:	During the study, which aspects will belong to SA WG2 or SA WG6, or required coordination with SA WG6 on the application layer aspects will be discussed, if needed. And coordination with SA WG3 on security aspects may be needed.

NOTE 3:	When AI agent is on the UE, this AI agent is assumed not part of the MT. It is also assumed that this WT is not replacing UE to Core Network interaction, NAS.

NOTE 4:	The AI agent in WT#3.2 is outside of RAN and Core Network for 6G.

4.	Study whether and how the 6G CN can provide AI services (i.e. AI inferencing and AI training) to applications (AF or in UE).

5.	Study the potential system impacts based on the characteristics of AI traffic.

NOTE 5:	Coordination with WT1.2 QoS may be needed.

NOTE 6:	System enhancement related to QoS for AI traffic will be handled by WT1.2 QoS,

NOTE 7:	Which working groups will determine the characteristics of AI traffic is subject to TSG SA plenary (SA#110) decision.

NOTE 8:	Coordination with SA WG3, SA WG5 and SA WG6 may be needed on AI inferencing and AI training.

NOTE 9:	Coordination with WT#5 and WT#6 may be needed.

NOTE 10:	New service charging part may be coordinated with SA5 Charging group.

# A.4	Work Task 4: Integrated Sensing and Communication

Study the integration of Sensing and Communication over 3GPP access, covering RAN node and UE operating as 3GPP Sensing Entities in the different sensing modes in alignment with TSG RAN, and considering multiple sources of sensing data.

The following aspects will be studied:

1.	Architecture and functional support for Sensing Service, including:

a.	Sensing Service authorization and revocation.

b.	Discovery and (re-)selection of Sensing function and Sensing Entities (i.e. RAN nodes, UEs), including sensing modes selection.

c.	Configuration parameters and policy provisioning to support sensing service, e.g. to the selected Sensing Entities, Sensing function, etc.

d.	Collection and transport of 3GPP sensing related data for sensing result generation.

e.	Exposure framework of sensing result to the Sensing service consumer (i.e. AFs, UEs, Core NFs).

NOTE 1:	How the Sensing service consumer makes use of the sensing results is not part of this work task.

2.	Whether and how to support Sensing Service in consideration of the mobility of the UE as sensing entity.

3.	Whether and how to support providing sensing data, which is not obtained from 3GPP radio signals, to the Sensing Function for sensing result generation.

NOTE 2:	This includes sensing data obtained using technologies that are different from 3GPP defined ISAC mechanisms, such as sensing based on IEEE 802.11, camera, sonar, lidar, etc.

NOTE 3:	The scope of WT#4 assumes RAN node and UE operating as Sensing Entities in different sensing modes in alignment with TSG RAN.

NOTE 4:	Coordination with other WTs will be needed if identified, e.g. Sensing data aspects with WT#5, architectural aspect with WT#1, etc.

NOTE 5:	Coordination with RAN WGs for RAN aspects, SA WG3 for privacy and user consent aspects, and SA WG5 for OAM and charging aspects will be needed. Coordination with SA6 may be needed on aspects to be identified during the study.

NOTE 6:	The scope of WT#4 will be aligned with RAN for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit WT/KI descriptions and determine if solutions can be discussed in Q2 2026.

# A.5	Work Task 5: 6G data framework in SA WG2

NOTE 1:	Coordination (as requested by SA plenary LS SP-251261) with SA WG5 is required. Common functionalities for data handling may be identified between SA WG2 and SA WG5 to be coordinated with each other to avoid incompatible or duplicated solutions for data framework.

The WT includes the following aspects:

1.	Identify high level use cases (e.g. AIML in the core, UE data collection for UE-sided model training, Sensing) potentially subject to the data framework.

NOTE 2:	This identification does not predetermine whether the use cases will ultimately adopt the data framework solutions. The coordination with other WTs for specific use cases is needed.

NOTE 3:	The use of the term “data framework” does not imply that all use cases that will be identified need to be supported by the same set of functionalities, procedures, NFs, services and/or use the same data structure/format.

2.	Study required data framework functionalities to support the identified use cases, e.g.:

a.	Data discovery and data registration.

b.	Data collection and transfer (including configuration of the data source (e.g. about collected data, reporting period and mechanisms to use), transfer of the collected data (including data distribution)).

c.	Data labelling/metadata handling.

d.	Data processing (e.g. data anonymization, data analysis, data generation, etc.).

e.	Data storage and retrieval.

f.	Data exposure.

The functionalities above may need to consider the quality of data, latency and data volume.

NOTE 4:	The bullet 1 and bullet 2 can be studied in parallel, e.g. each solution is expected to include or refer to the corresponding use case(s).

NOTE 5:	Coordination with SA WG5 may be required for charging aspects.

NOTE 6:	Consideration on MNO’s requirements of visibility and controllability during data handling processes is required.

NOTE 7:	The outcome of 5GS studies in R20 and normative work in previous releases on data handling will be considered.

3.	Study potential system impacts of user consent, privacy, security and data governance from SA WG2 architectural and system level perspective for data generated in 6G system corresponding to the use cases and functionalities in aspects 1 and 2.

NOTE 8:	Coordination with SA WG3 is required for user consent, privacy, security and data governance related topics to avoid duplicated or incompatible design. SA WG2 should study first which use cases and functionalities in aspects 1 and 2 above apply to aspect 3 and then coordinate with SA WG3. SA WG2 should consider the output of SA WG3 before specifying any system procedure on this topic.

# A.6	Work Task 6: 6G Computing Support

Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination among UE, core network and application server, exposure of computing service in the core network, etc.).

In order to support computing, the following aspects need to be studied:

WT#6.1: Identification of the architectural requirements and computing resource(s):

a.	Derive architectural requirements for scenario(s) and service requirements defined by SA WG1 to be enabled by the Computing Service.

b.	Whether and how to define the computing resource (e.g. computing resource type and/or status, location of the computing resource, etc.).

NOTE 1:	The use cases and the terms of Computing Service and the Service Hosting Environment defined in TR 22.870 [11] can be used as starting point for further refinement during solution development phase. The detailed computing resource type (e.g. software component, hardware component, etc.) can be identified during study phase.

WT#6.2: Enablement/authorization of computing service to UE or AF.

a.	Whether and how to enable Computing Service.

b.	Whether and how the operator network may expose the computing resource related information (e.g. its hosting capability (e.g. using compute resources at Service Hosting Environment)) and/or network metrics to AF.

WT#6.3: Coordination of communication and computing, service continuity and QoS aspects:

a.	Whether and how to coordinate (e.g. within or outside the core network) communication (for the traffic transferred over user plane) and computing resource(s) in the Computing Site.

b.	Whether and how to identify, collect/monitor, provision the computing resource related information.

c.	Whether and how to improve the service experience.

d.	Whether there is a need to improve QoS to satisfy Computing Service requirements.

e.	Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g. due to UE mobility, computing load balancing, AF influence etc.).

WT#6.4: Discovery and (re-)selection of compute site(s) for the Computing Service:

a.	Whether and how to discover and (re-)select Computing Site(s) for the Computing Service.

NOTE 2:	Whether and how to utilise and/or enhance Edge Computing mechanisms specified in 5G to address the aspects in the scope of WT#6 will be discussed in the study phase.

NOTE 3:	Which of above aspects should fall into SA WG5 and/or SA WG6 scope will be further identified in study phase, coordination with SA WG5 and SA WG6 may be needed.

NOTE 4:	Potential coordination with other WTs (e.g. WT#1.1 on service enablement and UE-CN interaction, WT#1.2 on QoS, exposure framework and UP aspects, etc.) is needed.

NOTE 5:	Computing Site represents the computing resources in a specific location of network topology. The computing site can offer computing services by allocating its computing resources to run application workloads upon demand of a Computing Service consumer. From networking perspective, the Computing Site is located within 6G CN (via e.g. user plane function in 6G) or Data Network (either owned and controlled by the operator or 3rd party).

# A.7	Work Task 7: Support of 6G NTN

Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed before discussing solutions.

NOTE 2:	This WT has IWK aspects that are scoped under WT#2.

NOTE 3:	This WT has dependency on mobility aspects that are scoped under WT#1.1.

NOTE 4:	This WT has voice aspects that are scoped under WT#1.4.

# A.8	Work Task 8: Cellular IoT enablers in 6G

Study how to support cellular IoT enablers in 6G including:

1	Analyse whether and which 5GS cellular IoT features are applicable to 6G and identify which of these features that should be further studied.

NOTE 1:	Enhancements in 6G to the selected 5GS IoT features are not precluded.

NOTE 2:	Features driven by this WT are expected to be generic and may apply to any UEs in 6G.

NOTE 3:	The detailed scope for WT#8 will be coordinated and aligned with Massive Communication (IoT) requirements from RAN and SA WG1. Ambient IoT, NB-IoT and non-3GPP access are not in the scope of the WT.

Annex B:
Topics for further resolution

# B.X	Topics for further resolution for KI#X

Editor's note:	This clause will include the topics for further resolution as work progresses for the specific KI#X. Eventually this clause should only contain topics for further resolution that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.X) and can either be then marked as not pursued or postponed to a future Release.

Annex C:
List of submitted solutions

# C.1	List of submitted solutions for KI#1

Editor's note:	The content of this clause is FFS.

# C.2	List of submitted solutions for KI#2

Editor's note:	The content of this clause is FFS.

# C.3	List of submitted solutions for KI#3

Editor's note:	The content of this clause is FFS.

# C.4	List of submitted solutions for KI#4

Editor's note:	The content of this clause is FFS.

# C.5	List of submitted solutions for KI#5

Table C.5-1: List of submitted solutions

| Meeting | Solution# | Solution variant | TDoc | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 | 5.4 | S2-2600055 | [KI#5, bullet #2] Graded QoS/ Solution #X.Y: Graded QoS Profile |
| SA2#173 | 2 | 5.6, | S2-2600091 | [KI#5, Bullet 4] Traffic Agent for Adaptive QoS/ Solution #5.Y: Traffic Agent for Adaptive QoS |
| SA2#173 | 6 | 5.6, | S2-2600189 | [KI#5, KI#18] Context-Guided Session Coordination for Adaptive QoS/ Solution #5.X: Context-Guided Session Coordination for Adaptive QoS in 6G |
| SA2#173 | 7 | 5.3 | S2-2600202 | New QoS parameter for AIML operations/ Solution #X.Y: QoS Framework for AI/ML application traffic |
| SA2#173 | 8 | 5.4 | S2-2600217 | [KI#5 bullets 2,3] Supporting QoS for applications with dynamic change of traffic characteristics/ Solution #X.Y: Supporting QoS for applications with dynamic change of traffic characteristics |
| SA2#173 | 9 | 5.6 | S2-2600218 | [KI#5, bullet 4, KI#6 bullet 5] Solution for a UE providing QoS requirements for an application service directly to the 3GPP network/ Solution #X.Y: Solution for a UE providing QoS requirements for an application service directly to the 3GPP network |
| SA2#173 | 10 | 5.6, | S2-2600264 | [KI#5,bullet#4,#5] supporting QoS Collaboration between Network and Application on the UE/ Solution #X.Y: QoS Collaboration between Network and Application on the UE. |
| SA2#173 | 11 | 5.1, | S2-2600271 | [KI#5, bullet #1, 4, 5] : QoS framework for 6G Solution #5.X: <QoS framework with QoS Flow and XaaS Task Flow> |
| SA2#173 | 12 | 5.4, 5.7 | S2-2600271 | KI#5, bullet #1, 4, 5] : QoS framework for 6G Solution #5.Y: <Two-way QoS negotiation mechanism> |
| SA2#173 | 14 | 5.6, | S2-2600336 | [KI#5, bullet #4] <UE and Network QoS Collaboration>/ Solution #X.Y: <UE and network QoS collaboration> |
| SA2#173 | 17 | 5.4 | S2-2600347 | [KI#5, bullet #2] Solution on dynamic QoS requirements/ Solution #5.Y: Applying GBR and non-GBR QoS dynamically to a SDF |
| SA2#173 | 19 | 5.3 | S2-2600353 | [KI#5, bullet #6] multimodality communication support for the differentiated QoS flows derived from multiplexed media flows/ Solution #5.Y: multimodal communication support for the differentiated QoS flows derived from multiplexed media flows |
| SA2#173 | 20 | 5.4 | S2-2600366 | [KI#5, bullet #1,2,3] Adaptive QoS in 6G/ Solution #5.X: Adaptive QoS in 6G |
| SA2#173 | 21 | 5.6, 5.8 | S2-2600367 | [KI#5, bullet #4] New solution to support UE, application and network QoS collaboration via a Proxy in the UPF/ Solution #5.Y: UE, Application and Network QoS Collaboration via a Proxy in the UPF |
| SA2#173 | 22 | 5.2 | S2-2600377 | [KI#5, bullet#5] Traffic characteristics aware QoS Monitoring Framework for 6G/ Solution #5.5: Traffic characteristics aware QoS Framework for 6G |
| SA2#173 | 23 | 5.4 | S2-2600387 | [KI#5.2] new solution of Conditional-GBR for dynamic QoS requirement// Solution #X.Y: Conditional-GBR for dynamic QoS requirement |
| SA2#173 | 24 | 5.4, 5.5 | S2-2600389 | [KI#5, bullet#2#4] Support Dynamical QoS/ Solution #5.Y: Dynamic QoS for UE, application and network QoS collaboration |
| SA2#173 | 26 | 5.2 | S2-2600399 | [KI#5.3] Alternative QoS profile in association with PDU Set Importance/ Solution #X.Y: Alternative QoS in association with PDU Set Importance |
| SA2#173 | 27 |  | S2-2600416 | [KI#5, Bullet#6] solution for TEID and SDAP entity of L4S traffic/ Solution #X.Y: Multiple TEID |
| SA2#173 | 28 | 5.1, 5.5 | S2-2600418 | [KI#5] Common 6G QoS Framework for different 6G Sessions/ Solution #X.Y: Common 6G QoS framework for 6G PDU Session and 6G Data Plane Session. |
| SA2#173 | 29 | 5.4 | S2-2600428 | Enable dynamic QoS adjustment/ Solution #X.Y: Enable UE to access / offload computing service |
| SA2#173 | 31 | 5.6 | S2-2600435 | [KI#5, bullet#4] Support UE and network collaboration/ Solution #X.Y: Support UE and network collaboration |
| SA2#173 | 32 | 5.2 | S2-2600440 | [KI#5, bullet#1] New Functionality in 6G QoS Framework considering AI Traffic Characteristics/ Solution #5.Y: New Functionality in 6G QoS Framework Considering AI Traffic Characteristics |
| SA2#173 | 33 | 5.4 | S2-2600441 | [KI#5, bullet#3] Content based Adaptive QoS to support the User Experience/ Solution #5.Y: Content based Adaptive QoS to support the User Experience |
| SA2#173 | 34 | 5.3 | S2-2600442 | [KI#5, bullet#4] Joint Task Awareness to support the QoS Collaboration/ Solution #5.Y: Joint Task Awareness to support the QoS Collaboration |
| SA2#173 | 35 | 5.4 | S2-2600461 | [KI#5, bullet #2] Solution to support dynamic QoS requirement/ Solution #5.Y: Solution to support dynamic QoS requirement |
| SA2#173 | 36 | 5.4 | S2-2600463 | [KI#5, bullet#2] New Sol for QoS Flow with Flexible QoS/ Solution #X.Y: QoS Flow with flexible QoS |
| SA2#173 | 37 | 5.6,, | S2-2600464 | [KI#5, bullet#4&#5] New Sol for support of the UE, application and network QoS collaboration/ Solution #X.Y: Real-time collaboration mechanism between UE, application and network |
| SA2#173 | 38 | 5.2 | S2-2600465 | [KI#5, bullet#1] New Sol of PDU Set based QoS handling considering the error tolerant traffic characteristics/ Solution #X.Y: PDU Set based QoS handling considering the error tolerant traffic characteristics |
| SA2#173 | 41 | 5.6, 5.8 | S2-2600483 | [KI#5, bullet #4] UE, application and network QoS collaboration based on SCONE in 6G./ Solution #X.Y: UE, application and network QoS collaboration based on SCONE in 6G |
| SA2#173 | 42 | 5.1a, 5.4 | S2-2600484 | KI#5, bullet #1, 2, 3, 4] Principles and aspects of the QoS Framework for 6GS/ Solution #X.Y: General concepts for the 6G QoS Framework |
| SA2#173 | 45 | 5.6 | S2-2600527 | [KI#5, bullet #4#5] Exposure of QoS monitoring parameters to UE/ Solution #X.Y: <Solution Title> |
| SA2#173 | 46 | 5.4, 5.5, 5.6, | S2-2600538 | [KI#5, Bullets #2, #3, and #4] QoS Framework for 6G/ Solution #5.Y: QoS Framework |
| SA2#173 | 47 | 5.6 | S2-2600543 | [KI#5, bullet #2a, #3, #4, #5] Unified QoS-related signalling framework for dynamic signalling/ Solution #5.Y: Unified QoS-related signalling framework for dynamic signalling |
| SA2#173 | 48 | 5.2 | S2-2600549 | [KI#5, bullet #1] Solution for data transmission for delay relaxed traffic/ Solution #5.X: Solution for data transmission for delay relaxed traffic |
| SA2#173 | 49 | 5.5 | S2-2600552 | [KI#5, bullet #4] Supporting Dynamic change of QoS for UE traffic/ Solution #5.Y: <Solution Title> Authorizing additional QoS flows for a service data flow to support dynamic QoS change |
| SA2#173 | 50 | 5.7 | S2-2600567 | [KI#5] Solution proposals on QoS Framework for 6G Solution #5.X: Enhancement to UE initiated QoS signalling |
| SA2#173 | 51 | 5.6 | S2-2600567 | [KI#5] Solution proposals on QoS Framework for 6G Solution #5.Y: UE provided traffic type information over user plane |
| SA2#173 | 52 | 5.7 | S2-2600567 | [KI#5] Solution proposals on QoS Framework for 6G Solution #5.Z: UE Policy based UE QoS flow detection and control |
| SA2#173 | 53 | 5.6 | S2-2600605 | KI#5 Solution: E2E Context-Aware System for Predictive QoS/ Solution #X.Y: Context-Aware Header (CAH) for Predictive QoS |

# C.6	List of submitted solutions for KI#6

Table C.6-1: List of submitted solutions

| Meeting | Solution# | Variant # | TDoc | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 | 6.1 | S2-2600277 | [KI#6, bullet #1] Combine of AM and UE Policy Associations |
| SA2#173 | 2 | 6.1 | S2-2600476 | [KI#6] AM policy and UE Policy combination |
| SA2#173 | 3 | 6.1, 6.2 | S2-2600283 | [KI#6, bullet#1] Solution to support policy framework simplification |
| SA2#173 | 4 | 6.2 | S2-2600419 | [KI#6] Common 6G Policy and Charging Control Framework |
| SA2#173 | 5 | 6.1, 6.2 | S2-2600220 | [KI#6, bullet 1] Solution for policy association coordination for a UE |
| SA2#173 | 6 | 6.5 | S2-2600612 | [KI#6, bullet#1] Policy association for SMF |
| SA2#173 | 7 | 6.3 | S2-2600232 | [KI#6, bullet #1 and bullet #2] Efficient UE Policy control for 6G System |
| SA2#173 | 8 | 6.3 | S2-2600408 | [KI#6, bullet#1b] New solution on supporting UE to request UE Policy from 6G CN consistently |
| SA2#173 | 9 | 6.3 | S2-2600437 | [KI#6, bullet #1.b] Support UE to request the URSP |
| SA2#173 | 10 | 6.3 | S2-2600482 | [KI#6, bullet #1b] UE request for UE policies |
| SA2#173 | 11 | 6.4 | S2-2600180 | [KI#6, Bullet 1b, Bullet 2] New Sol: Solution for Policy Control on UE Policy for 6G system |
| SA2#173 | 12 | 6.3 | S2-2600544 | [KI#6, bullet #1b, bullet#5] QoS-based UE policy provisioning and evaluation |
| SA2#173 | 18 | 6.5 | S2-2600436 | [KI#6, bullet#5] Support UE input for PCC decision |
| SA2#173 | 19 | 6.5 | S2-2600443 | [KI#6, bullet#5] UE input for PCC decision to support QoS Control |
| SA2#173 | 20 | 6.5 | S2-2600478 | [KI#6, bullet #5] UE inputs for PCC decision through northbound APIs |
| SA2#173 | 23 | 6.3 | S2-2600429 | [KI#6] Enabling UE requested policy provisioning procedure |

Table C.6-2: List of submitted solutions related to unhandled bullets of KI#6

| SA2#173 | 13 | Not handled | S2-2600196 | [KI#6] Support of multiple connectivity profiles and related URSPs and network policies |
| --- | --- | --- | --- | --- |
| SA2#173 | 14 | Not handled | S2-2600553 | [KI#6, bullet #2] Evaluation of UE Policies considering user/local preferences |
| SA2#173 | 15 | Not handled | S2-2600611 | [KI #6, bullet #2] User Preference defintion |
| SA2#173 | 16 | Not handled | S2-2600568 | [KI#6] Solution proposals on Policy and charging control framework |
| SA2#173 | 17 | Not handled | S2-2600259 | [KI#6, Bullet#5]Network Policy for UE's Energy Saving |
| SA2#173 | 21 | Not handled | S2-2600355 | [KI#6, Bullet#3] solution for verifying the authenticity of applications |
| SA2#173 | 22 | Not handled | S2-2600481 | [KI#6, bullet #3] External Parameter Provisioning aspects of policy control for 6G |

# C.7	List of submitted solutions for KI#7

Editor's note:	The content of this clause is FFS.

# C.8	List of submitted solutions for KI#8

Table C.8-1: List of submitted solutions

| Meeting | Solution# | TDoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600069 | [KI#8] Solution Description on 6G Network Sharing |

# C.9	List of submitted solutions for KI#9

Editor's note:	The content of this clause is FFS.

# C.10	List of submitted solutions for KI#10

Editor's note:	The content of this clause is FFS.

# C.11	List of submitted solutions for KI#11

Editor's note:	The content of this clause is FFS.

# C.12	List of submitted solutions for KI#12

Table C.12-1: List of submitted solutions

| Meeting | Solution# | TDoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | {1} | S2-2600142 | KI#12: New solution on Voice Services for 6G |
| SA2#173 | {2} | S2-2600251 | [KI12, bullet #1 bullet #2] Native support of IMS based voice services over the 6G access network |
| SA2#173 | {3} | S2-2600252 | [KI#12, bullet #6 ] Compatibility between 6G CN and the existing deployed IMS |
| SA2#173 | {4} | S2-2600273 | [KI#12.5 Solution] UE-oriented normal IMS voice call fallback |
| SA2#173 | {5} | S2-2600342 | [KI#12] Support IMS voice and voice Fallback in the 6G |
| SA2#173 | {6} | S2-2600350 | Solution for Key Issue #12: Support for Voice service in 6G |
| SA2#173 | {7} | S2-2600363 | [KI#12, bullet 5] Solution proposal on UE-based fallback mechanism |
| SA2#173 | {8} | S2-2600364 | [KI#12, bullet#5] Solution proposal on NW-based fallback mechanism |
| SA2#173 | {9} | S2-2600415 | [KI#12.1] New solution on native voice service support in 6GS |
| SA2#173 | {10} | S2-2600451 | [KI#12] Voice Services for 6G |
| SA2#173 | {11} | S2-2600508 | Interim agreements for Key Issue #12: Voice service for 6G |
| SA2#173 | {12} | S2-2600599 | [KI#12, bullets #1, 3, 4, 5] Avoiding obstacles to day 1 Vo6G support |

NOTE:	Interim agreements proposed in {11} is not considered in clause 6.

# C.13	List of submitted solutions for KI#13

Table C.13-1: List of submitted solutions

| Meeting | Solution# | TDoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 13.1 | S2-2600147 | Used the solution Support for emergency services via 6GS as baseline |
| SA2#173 | 13.2 | S2-2600147 | Used the solution Support for Emergency Services Fallback to NR and E-UTRAN as baseline |
| SA2#173 | - | S2-2600343 | Merged Emergency service support in 6G into X.Y and X.Z |

# C.14	List of submitted solutions for KI#14

Table C.14-1: List of submitted solutions

| Meeting | Solution# | TDoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 14.1 | S2-2600118 | Privacy-Preserving Regulatory Location Sharing |
| SA2#173 | 14.2 | S2-2600155 | LPAF for 6G Location Architecture |
| SA2#173 | 14.3 | S2-2600248 | [[KI#14, bullet 1,2,3] 6G LCS architecture and related regulatory services |
| SA2#173 | 14.4 | S2-2600490 | [KI#14] Support of location service for 6G |
| SA2#173 | 14.5 | S2-2600579 | [KI#14] Solution proposals on Location Service for 6G |

# C.15	List of submitted solutions for KI#15

Table C.15-1: List of submitted solutions

| Meeting | Solution# | TDoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2 #173 | 1 | S2-2600148 | Solution for SMS over NAS and SMS over IP re-using 5GC features & procedures |
| SA2 #173 | 2 | S2-2600272 | Solution for SMS over NAS and SMS over IP with SMS originator ID verification |

# C.16	List of submitted solutions for KI#16

Editor's note:	The content of this clause is FFS.

# C.17	List of submitted solutions for KI#17

Editor's note:	The content of this clause is FFS.

# C.18	List of submitted solutions for KI#18

Editor's note:	The content of this clause is FFS.

# C.19	List of submitted solutions for KI#19

## C.19.1	KI#19 bullets 1&2

Table C.19.1-1: Input papers at SA2#173 for KI#19 bullet 1&2

| Meeting | Solution# | Tdoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600061 | 23.801-01: [KI#19, bullet1] new solution for AI Agent discovery via the 6G networks(s) |
| SA2#173 | 2 | S2-2600062 | 23.801-01: [KI#19, bullet2] new solution for AI Agent communication via the 6G networks(s) |
| SA2#173 | 3 | S2-2600078 | 23.801-01: [KI#19 bullets 1, 2] New solution on digital identity distribution, registration, authentication and session establishment for UE AI Agent |
| SA2#173 | 4 | S2-2600094 | 23.801-01: [KI#19 bullet 1&2] Solution for Key Issue #19: 6G Network for AI-Enable AI agent communication |
| SA2#173 | 5 | S2-2600186 | 23.801-01: [KI#19, bullet#1, bullet#2] New Solution on 6G network support UE agent identification, authorization and discovery |
| SA2#173 | 6 | S2-2600223 | 23.801-01: KI#19-Skill-Based Distributed Al Agents Coordination Framework |
| SA2#173 | 7 | S2-2600245 | 23.801-01: [KI#19, bullet #1, bullet#2] Support for communication between AI Agents on the application layer via 6G CN |
| SA2#173 | 8 | S2-2600356 | 23.801-01: [KI#19, bullet#1&bullet#2] New Solution for AI agent registration and discovery |
| SA2#173 | 9 | S2-2600360 | 23.801-01: [KI#19, bullet#1] AI Agent Discovery via Agent Repository Function (ARF) |
| SA2#173 | 10 | S2-2600381 | 23.801-01: [KI#19, bullet #1] New Solution for AI Agent Discovery via the AI Agent Exposure Function in 6G |
| SA2#173 | 11 | S2-2600424 | 23.801-01: [KI#19, bullet #1 & bullet#2] New solution: Group-based AI agent communication |
| SA2#173 | 12 | S2-2600425 | 23.801-01: [KI#19, bullet #1 & bullet#2] New solution: AI agent addressing and direct one- to-one communication |
| SA2#173 | 13 | S2-2600432 | 23.801-01: [KI#19, bullet#1,2] Solution for AI agent Communication Network |
| SA2#173 | 14 | S2-2600520 | 23.801-01: [KI#19, bullet #1] UE AI Agent Discovery using UE Agent Capability Profiles |
| SA2#173 | 15 | S2-2600530 | 23.801-01: [KI#19, bullet 2] Solution for KI#19: distributed authorization for AI agents' communications |
| SA2#173 | 16 | S2-2600536 | 23.801-01: [KI#19] 6G Network for AI Solution |
| SA2#173 | 17 | S2-2600546 | 23.801-01: [KI#19, bullet #1, bullet#2] UE AI agent Discovery and UE AI agent Session Establishment via 6G CN |
| SA2#173 | 18 | S2-2600561 | 23.801-01: [KI#19] AI agent Collaboration based on AI Agent Registration/Discovery and AI Agent Group Communication |
| SA2#173 | 19 | S2-2600369 | 23.801-01: [KI#18, KI#19] New Solution - Discovery of Al agents via Al agent Repository Function |
| SA2#173 | 20 | S2-2600573 | 23.801-01: [KI#18/KI#19] Native AI and agentic AI support for 6G |
| SA2#173 | 21 | S2-2600629 | 23.801-01: [KI#19] Solution for NAS-based LLM Service Commanding in 6G Core Network |

## C.19.2	Void

## C.19.3	KI#19 bullet 3

Table C.19.3-1: Input papers at SA2#173 for KI#19 bullet 3

| Meeting | Solution# | Tdoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600224 | [KI#19, bullet 3] Supporting intent-based exposure from AI agent in an AF |
| SA2#173 | 2 | S2-2600452 | [KI#19, bullet#3] New solution to enhance network capability exposure functionalities to AI agent on AFs |
| SA2#173 | 3 | S2-2600573 | 23.801-01: [KI#18/KI#19] Native AI and agentic AI support for 6G. |
| SA2#173 | 4 | S2-2600244 | [KI#18, bullet#1a and bullet#1b] How to fulfil requests from the UE or the AF that includes intents. |

## C.19.4	KI#19 bullet 4

Table C.19.4-1: Input papers at SA2#173 for KI#19 bullet 4

| Meeting |  | Solution# | Tdoc | Subject/Comment |
| --- | --- | --- | --- | --- |
| SA2#173 | 1 |  | S2-2600063 | [KI#19, bullet4] new solution for 6G CN provides AI services to applications(AF or in UE) |
| SA2#173 | 2 |  | S2-2600079 | [KI#19 bullet 4] New solution on UE access AI services |
| SA2#173 | 3 |  | S2-2600096 | [KI#9 bullet 4] Solution for KI#19: AI model training and inference performed by 6GC |
| SA2#173 | 4 |  | S2-2600158 | [KI#19, bullet#4] 6G AI service to UE |
| SA2#173 | 5 |  | S2-2600171 | [KI#19, bullet #4] New Solution for 6G CN Providing AI Inferencing Services |
| SA2#173 | 6 |  | S2-2600191 | [KI#19, bullet #9] new solution: Support AI Training Service in 6G core network |
| SA2#173 | 7 |  | S2-2600414 | [KI#19, bullet#4] 6G Network for AI services |
| SA2#173 | 8 |  | S2-2600454 | [KI#19, bullet#4] New solution to enable 6G CN to support AI services to applications (UE or in AF) |
| SA2#173 | 9 |  | S2-2600540 | [KI#19] Enabling UE application-based AI agent interactions with the network. |

## C.19.5	KI#19 bullet 5

Table C.19.5-1: Input papers at SA2#173 for KI#19 bullet 5

| Meeting | Solution# | Tdoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600095 | [KI#19, bullet #5] Solution for KI#19 on Network Enhancement based on the characteristics of AI traffic |
| SA2#173 | 2 | S2-2600173 | [KI#19, bullet#5] New Sol#X: “Traffic Detection based on AI-related traffic type” |
| SA2#173 | 3 | S2-2600393 | [KI#19, bullet#7] AI traffic characteristics for Generative AI |
| SA2#173 | 4 | S2-2600394 | [KI#19, bullet#7] Token related AI traffic |
| SA2#173 | 5 | S2-2600403 | [KI#19] Discussion and proposal for traffic of AI service and AI codec |
| SA2#173 | 6 | S2-2600467 | [KI#19, bullet#5] [KI#19, bullet#5] AI/ML traffic characteristics |
| SA2#173 | 7 | S2-2600537 | [KI#19, bullet#5] Solution to Support GenAI Traffic |
| SA2#173 | 8 | S2-2600540 | 23.801-01: [KI#19] Enabling UE application-based AI agent interactions with the network. |

# C.20	List of submitted solutions for KI#20

Editor's note:	The content of this clause is FFS.

# C.21	List of submitted solutions for KI#21

Editor's note:	The content of this clause is FFS.

# C.22	List of submitted solutions for KI#22

Table C.22-1: Input papers at SA2#173

| Meeting | Solution# | Tdoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600098 | [KI#22, bullet #1&2&4] Solution for KI#22 of 6G architecture enhancement to support 6G computing service |
| SA2#173 | 2 | S2-2600125 | NF computing capability collection and exposure for 6G Computing Supporting |
| SA2#173 | 3 | S2-2600156 | [KI#22] 6G computing service to UE |
| SA2#173 | 4 | S2-2600174 | [KI#22, bullet#1(b)] New Sol#X: “Computing Resource Profile” |
| SA2#173 | 5 | S2-2600181 | [KI#22] New Sol: Solution to support 6G Computing |
| SA2#173 | 6 | S2-2600183 | Capability Aware Discovery and selection of compute sites |
| SA2#173 | 7 | S2-2600190 | [KI#22] New Sol: Solution to support computing service |
| SA2#173 | 8 | S2-2600192 | [KI#22, bullet#1,#2,#4] 6G computing service |
| SA2#173 | 9 | S2-2600247 | [KI#22, bullet 1] Use of Edge Enabler Layer to support 6G computing support |
| SA2#173 | 10 | S2-2600262 | [KI#22] Solution to support computing task offloading to the 6G network |
| SA2#173 | 11 | S2-2600267 | [KI#22, bullet #1, #3] Identification of architecture requirements and computing resource |
| SA2#173 | 12 | S2-2600268 | [KI#22, bullet #2, #4] Support computing service for UE |
| SA2#173 | 13 | S2-2600358 | [KI#22] PDU session establishment for Computing Service |
| SA2#173 | 14 | S2-2600359 | [KI#22] Exposed services for Computing Service |
| SA2#173 | 15 | S2-2600374 | [KI#22, bullet #1,3,4] Solution on 6G Computing Architecture |
| SA2#173 | 16 | S2-2600383 | [KI#22, bullet #2.a, KI#1.2 bullet #a)] A service-plane-based architectural solution to support computing services in 6G systems |
| SA2#173 | 17 | S2-2600402 | [KI#22, bullet #1,2,3,4] Solution for 6G Computing Support |
| SA2#173 | 18 | S2-2600409 | [KI#22, bullet#2a] New Solution on Enabling Computing Service to UE |
| SA2#173 | 19 | S2-2600410 | [KI#22, bullet#3b] New Solution on Computing Resource Collection and Management in 6G CN |
| SA2#173 | 20 | S2-2600411 | [KI#22, bullet#4a] New Solution on Computing Node and Site Discovery and Selection |
| SA2#173 | 21 | S2-2600417 | [KI#22] New solution for 6G computing resource allocation and coordination |
| SA2#173 | 22 | S2-2600421 | [KI#22] Architectural requirement of 6G computing |
| SA2#173 | 23 | S2-2600426 | [KI#22, bullet #1, bullet#2, bullet#3] New solution: Computing service support for UE request |
| SA2#173 | 24 | S2-2600427 | [KI#22, bullet #2] New solution: Computing service resource information exposure to AF |
| SA2#173 | 25 | S2-2600430 | Enabling computing service access and offload |
| SA2#173 | 26 | S2-2600453 | [KI#22] Computing Framework and Computing Service Enablement Mechanism |
| SA2#173 | 27 | S2-2600477 | [KI#22, bullet#1] Scenarios and architectural requirements for 6G compute |
| SA2#173 | 28 | S2-2600479 | [KI#22 bullets 2-4] Coordination of communication and computing |
| SA2#173 | 29 | S2-2600480 | [KI#22 bullet 1] Architectural requirements on 6G Computing support |
| SA2#173 | 30 | S2-2600486 | [KI#22, bullet#4] Discovery and (re)selection |
| SA2#173 | 31 | S2-2600487 | [KI#22, bullet 2-b and 3-b] Management and exposure |
| SA2#173 | 32 | S2-2600488 | [KI#22, bullet#3] CS re-discovery with monitoring |
| SA2#173 | 33 | S2-2600502 | [KI#22] Solution proposals on 6G Computing Support |
| SA2#173 | 34 | S2-2600548 | [KI#22, Bullets #1, #2a, #3a, b, c, & d, and #4] 6G Computing Service provisioning, configuration and monitoring |
| SA2#173 | 35 | S2-2600575 | [KI#22] Coordinated selection of UPFs and compute sites without HLOS and UE application dependencies |
| SA2#173 | 36 | S2-2600584 | [KI#22, bullet 1] Solution for Architecture of Computing resource and definition of computing resource |
| SA2#173 | 37 | S2-2600585 | [KI#22, bullet 2] Solution for Exposure of computing service |
| SA2#173 | 38 | S2-2600586 | [KI#22, bullet 2] Solution for Enablement of computing service |
| SA2#173 | 39 | S2-2600587 | [KI#22, bullet 3] Solution for application aware computing orchestration |
| SA2#173 | 40 | S2-2600588 | [KI#22, bullet 3] Solution for dynamic computing configuration based on QoS and UE mobility |

# C.23	List of submitted solutions for KI#23

Editor's note:	The content of this clause is FFS.

# C.24	List of submitted solutions for KI#24

Table C.24-1: List of submitted solutions

| Meeting | Solution# | TDoc | Subject/Comment |
| --- | --- | --- | --- |
| SA2#173 | 1 | S2-2600121 | Only control plane based features |
| SA2#173 | 2 | S2-2600246 | Only user plane based features |
| SA2#173 | 3 | S2-2600316 | Only user plane based features |
| SA2#173 | 4 | S2-2600332 | Support user plane solution, but not the UP CIoT optimization feature in 5GS. |
| SA2#173 | 5 | S2-2600380 | Include support for both Control plane and User plane based features. |
| SA2#173 | 6 | S2-2600396 | Only control plane based features |
| SA2#173 | 7 | S2-2600401 | Only user plane based features |
| SA2#173 | 8 | S2-2600444 | Include support for both Control plane and User plane based features. |
| SA2#173 | 9 | S2-2600529 | Include support for both Control plane and User plane based features. |
| SA2#173 | 10 | S2-2600590 | Includes important features for 6G IoT, but not connected to CP or UP solution |

Annex D:
Change History

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA2#170 | S2-2507937 | - | - | - | TR skeleton for FS_6G_ARC | 0.0.0 |
| 2025-08 | SA2#170 | - | - | - | - | Documented approved p-CRs at S2#170: S2-2508022, S2-2508034 | 0.1.0 |
| 2025-10 | SA2#171 | - | - | - | - | Documented approved p-CRs at S2#171: S2-2509788, S2-2509821, S2-2509835 | 0.2.0 |
| 2025-11 | SA2#172 | - | - | - | - | Documented approved p-CRs at S2#172: S2-2511123, S2-2511142, S2-2511144, S2-2511180, S2-2511196, S2-2511197, S2-2511201, S2-2511212, S2-2511213, S2-2511214, S2-2511215, S2-2511217, S2-2511218, S2-2511240, S2-2511241, S2-2511244, S2-2511245, S2-2511252, S2-2511253, S2-2511254, S2-2511261, S2-2511297 | 0.3.0 |
| 2026-02 | SA2#173 | - | - | - | - | Documented approved p-CRs at S2#173: S2-2601433, S2-2601434, S2-2601467, S2-2601470, S2-2601484, S2-2601501, S2-2601625, S2-2601642, S2-2601701, S2-2601702 | 0.4.0 |