| 3GPP TR 23.949 V0.3.0 (2026-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; SEAL services usage guidance; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Overview of SEAL	9

4.1	The role and responsibility of SEAL within 3GPP system	9

4.1.1	General description	10

4.1.2	The interaction with rest part of 3GPP system	11

4.2	SEAL Entities	12

4.2.1	General description	12

4.2.2	NRM (network resource management) server/client	12

4.2.3	SEALDD (service enabler architecture layer) server/client	13

4.2.4	NSCE (Network Slice Capability Exposure) server/client	14

4.2.5	LM (location management) server/client	16

4.2.6	AIMLE (AIML enabler) server/client	16

4.2.7	Spatial Anchor server/client	16

4.2.8	Spatial Map server/client	17

4.2.9	Digital Assets server/client	17

4.2. A	Group Management server/client	17

5	The use cases and advantages to applications	17

5.1	Summary of SEAL services	17

5.2	Use cases and advantages for SEAL Service provided by SEALDD	19

5.2.1	Use case	19

5.2.2	Advantages and values of SEALDD Services	19

5.3	Use cases and advantages for SEAL Services provided by NRM	21

5.3.1	Use case	21

5.3.2	 Advantages and values of NRM Services	21

5.4	Use cases and advantages for SEAL Services provided by NSCE	22

5.4.1	Use case	22

5.4.2	Advantages and values of NSCE	22

5.5	Use cases and advantages for SEAL Services provided by metaverse service	23

5.5.1	Use case	23

5.5.2	Advantages and values of spatial anchor Services	24

5.5.3	Advantages and values of Digital asset Service	25

5.5.4	Advantages and values of Spatial Map Service	25

6	Deployment of SEAL entities and business relationship among stakeholders	25

6.1	Ecosystem stakeholders	25

6.2	Deployment alternatives	26

6.2.1	Common exposure entities	26

6.2.2	Separated exposure entities for different domain API	27

Annex A: Relationship between SEAL Enablers and external organizations	29

A.1	General description	29

A.2	Vertical applications consume 3GPP APIs directly	29

A.3	SEAL layer services as the SBI of external API platform	30

Annex B: Change history	31

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This TR provides guidelines for SEAL services usage for the benefit of the Application providers (application developers, application service provider) and application ecosystem partners (e.g., third party application platform providers).

# 1	Scope

This TR provides guidelines for  SEAL services usage for the benefit of the Application providers (developers/ application service providers) and application ecosystem partners (e.g., third party application platform providers).

It includes the overall introduction of SEAL layer covering the below aspects:

-	Roles and responsibilities of SEAL within 3GPP exposure system.

-	Relationship between SEAL and external SDOs.

-	Use cases related to SEAL applicability, advantages and value-add it offers

-	Deployment models for SEAL entities and business relationship among different stakeholders

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications; Stage2".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Stage 2".

[4]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service; Stage 2".

[5]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service; Stage 2".

[6]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial mapping and Spatial anchors management Stage 2".

[7]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital asset".

[8]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[9]	3GPP TS26.522 5G Real-time Media Transport Protocol Configurations

[10]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[11]	ITU-T Rec H.265 (08/2021): "High efficiency video coding" | ISO/IEC 23008-2:2023: "High Efficiency Coding and Media Delivery in Heterogeneous Environments – Part 2: High Efficiency Video Coding".

[12]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[13]	ITU-T Rec H.264 (08/2021): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2022: "Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding".

[14]	3GPP TS 28.533: " Management and orchestration; Architecture framework”

[15]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[16]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios".

[17]	3GPP TS 28.538: " Edge Computing Management".

[18]	3GPP TS 23.558: " Architecture for enabling Edge Applications".

[19]	3GPP TS 23.286: "Application layer support for Vehicle-to-Everything (V2X) services Functional architecture and information flows".

[20]	3GPP TS 23.255: "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows".

[21]	3GPP TS 23.239: "Application enablement aspects for MMTel".

[22]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[23]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[24]                      3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the terms given in clause 3 of 3GPP TS 23.222 [2] and clause 3 of 3GPP TS 29.222 [3] shall also apply.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the abbreviations given in clause 3 of 3GPP TS 23.222 [2] and clause 3 of 3GPP TS 29.222 [3] shall also apply.

# 4	Overview of SEAL

## 4.1	The role and responsibility of SEAL within 3GPP system

Editor's Note:	This clause will describe the general role and responsibilities of SEAL layer

### 4.1.1	General description

The services at this layer defined in 3GPP TS 23.434 [3], 3GPP TS 23.435 [4], 3GPP TS 23.436 [5], 3GPP TS 23.437 [6], 3GPP TS 23.438 [7], 3GPP TS 23.482 [12].

As depicted in figure 4.1.1-1, SEAL is on the 3GPP services layerwhich provides the value-added services to enable application layer to utilize 3GPP network and UE capabilities in an efficient and easy-to-use way. SEAL services are generic and it caters to different applications.

Services at this layer are created by utilizing network connection layer capabilities from one or multiple Network Functions deployed in telecom system. It can also utilize the resources/information of SEAL client on the UE.

Services exposed at this layer is normally in larger granularity than the services exposed by single network function at network connection layer and provide an application layer meaningful function. Implementation complexities reduces significantly if the application layer choose to utilize the services exposed at SEAL layer  -instead of directly consuming network connection layer services by invoking APIs one by one.

SEAL layer includes SEAL server(s) and SEAL client(s).

-	SEAL server(s): it is implemented in network by MNO or 3rd part and are authorized to (1) provide services to application and SEAL client, and (2) consume 3GPP network services by invoking network APIs.

-	SEAL client(s): it is implemented in UE and can be provided by same organization as SEAL service server or other origination e.g., UE vendor/OS and are authorized to (1) provide services to consumers by APIs, (2) consuming network services by invoking APIs from SEAL service server (3) trigger UE modem to initiate NAS procedures to request network capability so as to provide service result to application.

Figure 4.1.1-1: Layered representation of SEAL service layer in 3GPP network exposure system

### 4.1.2	The interaction with rest part of 3GPP system

Figure 4.1.2-1: Service based Architecture of 3GPP system supporting SEAL services

As the figure 4.1.2-1, SEAL services which are part of the 3GPP system are deployed either by the MNO or third party.

SEAL server(s) invokes service APIs exposed by 3GPP core network functions directly over the 3GPP core network SBA when deployed as trusted AF in MNO's trusted domain as figure 4.1.2-1.

NOTE 1:	In the real case, SEAL server(s) may invoke service APIs exposed by 3GPP core network functions via NEF as a non-trusted AF e.g., when deployed by the third party.

NOTE2:	SEAL server(s) when deployed as trusted AF in MNO's trusted domain may also invoke APIs from 3GPP core network functions via NEF.

SEAL server(s) invokes service APIs exposed by the OAM system over the OAM SBMA when deployed by MNO as specified in 3GPP TS28.533 [14].

NOTE 3:	In the real case, SEAL server(s) may invoke service APIs exposed by 3GPP core network functions via AEF (e.g., when deployed by the third party, or even by MNO).

SEAL clients take two different responsibilities:

1)	consumer of SEAL server API(s)

2)	providing required terminal side information/resource to support the SEAL server function(s)

SEAL services APIs are provided and exposed by MNOs or third party, and they are consumed by the Application specific layer (VAL layer) or vertical application enabler layer (e.g., VAE server, UAE server), Edge enabler layer (e.g., EES, ECS), SEAL client(s) or third party's platform.

The interactions between the vertical application server(s) and SEAL server(s) for discovery, authentication and authorization are supported by CAPIF as specified in 3GPP TS 23.222 [9]. The VAL server acts as CAPIF's API invoker and SEAL server acts as CAPIF's API exposing function.

NOTE 4:	 It is up to application developer's implementation how to invoke the SEAL server's APIs in their application layer software.

NOTE 5:	The API invokers as defined in 3GPP TS 23.222 [9] can either be on the network side (Application server) or on the UE side (e.g., SEAL clients, applications on UE), to use SEAL server API(s).

## 4.2	SEAL Entities

Editor's Note:	This clause will describe the service entities (SEAL servers and SEAL Clients) in SEAL layer.

### 4.2.1	General description

The following service entities as shown in figure 4.2.1-1in SEAL layer are defined in 3GPP TS 23.433 [2], 3GPP TS 23.434[3], 3GPP TS 23.435 [4], 3GPP TS 23.436 [5], 3GPP TS 23.437 [6], 3GPP TS 23.438 [7], 3GPP TS 23.482 [12].

The service entities at this layer can work independent with other entities. It is up to operator’s business decision to deploy one or multiple services servers.

For each SEAL server, there is a related SEAL client resides on UE.  The SEAL client connects to only the associated SEAL server.

Figure 4.2.1-1: SEAL functional entities

### 4.2.2	NRM (network resource management) server/client

NRM server provides the value-added services by utilizing capabilities of 3GPP core network (e.g., NEF/PCF/MBSMF). It enables third party‘s application (application server or application client) to request 3GPP system to provide QoS service to improve the performance of applications traffic(s) in an easy-to-use way.

NOTE:	The detailed information about NRM advantages to application developers is described in clause 5.3.

Table 4.2.4-1 Example of NRM APIs for third party

| APIs | Usage of the APIs for third party |
| --- | --- |
| SS_NetworkResourceAdaptation | This API enables the third party to request/reserve different network resource to support different service requirements by providing indication (using API operation): Request/reserve resource for the unicast connection used for application data with required transmission performance. or Request/reserve resource for the multicast connection used for transmitting application data from application server to multiple UEs with required transmission performance. Or Request/reserve resource for background data transmission. Or Request/reserve resource for TSC stream data with required transmission performance. |
| SS_EventsMonitoring | This API enables the third party to get the information of UE network connection statues, e.g., LOSS_OF_CONNECTIVITY, COMMUNICATION_FAILURE. This information can be further used by application server of third party to determine application layer behaviour related to data transmission to the VAL UE, e.g., for DL only case. |
| SS_NetworkResourceMonitoring | This API enables the third party to request to be notified about QoS statues of a target stream, VAL, a UE or group of UEs. This information can be further used by application layer of third party to determine application layer behavior related to data transmission. |
| SS_MMetaServiceRequirements | This API enable the service exposed to Metaverse service provider who deploys the virtual UEs. This API can be used by third party to request 3GPP system to discover VAL UEs within the network coverage area and identifies and reserve network resources for all the application sessions among the VAL UEs (VAL UE -to- network -to- VAL UE) and between the VAL UEs and the VAL servers (where the virtual UEs are deployed) required for the end-to-end Metaverse service. |

NRM client supports two types of functional role:

1)	Provide assistant information or resource required by NRM server, to enable certain NRM service (e.g., providing MBMS listening status report required by NRM server to determine the data transmission mode), or

2)	Trigger (acting as NRM service consumer) NRM server to execute NRM service logic, e.g., request NRM to manage End-to-End QoS service for traffic between application on one UE to application on another UE.

### 4.2.3	SEALDD (service enabler architecture layer) server/client

SEALDD server provides application layer data(content) storage and forwarding services utilizing 3GPP network capabilities.  By caching content closer to end users and co-operating with 3GPP network for improving transmission performance, it will reduce latency and improve user experience of applications.

SEALDD client provides the capabilities which enable the application on the UE to request/receiving certain content from SEALDD server via SEALDD Uu.

Table 4.2.4-1: Example of SEAL DD APIs for third party

| Usage Type | APIs | Usage of the APIs for third party |
| --- | --- | --- |
| Application layer signalling transmission / Application layer content transmission over single SEAL-DD flow | Sdd_RegularTransmission | This API enable third party application server (VAL server) to request SEAL-DD server to support the following purpose: -Request SEAL-DD server to monitor and forward any application layer signalling toward VAL server from any UE. -Request SEAL-DD server to monitor and forward uplink data from UE to the address that third party application server indicate, or forward the DL applications data to UEs. |
|  | Sdd_ConnectionStatusEvent | This API enables third party application server to know the statue of SEAL-DD connection., e.g., whether the connection is available to transmit data, or not available due to the reason of UE unreachable. |
|  | Sdd_PolicyConfiguration | This API enables third party (e.g., administrator) to configure the policy which requires SEALDD server to apply for the transmission of application layer content. |
| Redundant transmission of application layer content | Sdd_URLLCTransmission | This API enables third party to request SEAL-DD server establish redundant transmission resource for the transmission of application layer content. |
| Data storage and deliver | Sdd_ DataStorage_Creation | This API enables third party to request 3GPP system to store application data (caching data in SEAL DD server), which can be further fetched by clients, or other VAL server, other SEAL DD server |
|  | Sdd_ DataStorage_Fetch | This API enables VAL UE or VAL server, other SEAL DD server to request the data cached in SEAL -DD or pulled from VAL content server if not available. |
|  | Sdd_ DataStorage_Management | This API enables third party to request 3GPP system to delete/update application data (caching data in SEAL DD server), which can be further fetched by clients, or other VAL server, other SEAL DD server |
| Application layer content delivering for XR type application | Sdd_multi-modalTransmission | This API enables third party application server to request to establish resource transmission of multi-application layer data flows to meet with the synchronization requirement among flows. |

### 4.2.4	NSCE (Network Slice Capability Exposure) server/client

NSCE server provides the value-added services by utilizing slice related capabilities of 3GPP core network and OAM. It enables third party’s application (application server or application client) to request 3GPP system to provide slice services to meet with the third party’s requirement in an easy-to-use way.

NOTE:	The detailed information about slice advantages to application developers is described in clause 5.4.

Table 4.2.4-1: Example of NSCE APIs for third party

| APIs | Usage of the APIs for third party |
| --- | --- |
| SS_NSCE_SliceApiConfiguration | This API enables the third party to configure/ possible subsequent update their VAL Application requirements that a network slice should support. |
| SS_NSCE_Multiplayer | This API enables the third party to delegate NSCE server to performs the triggering the management operation on a network slice during the slice lifecycle, including making the network slice lifecycle management operation decision (e., g the slice SLA requirement that support VAL Application requirements) and triggering OAM system to perform the network slice life-cycle management operations (create update, delete a network slice from 5G system), notifying the consumer about the network slice information.) |
| SS_NSCE_PolicyManagement | This API enables the third party to configure (possible subsequent update/delete) the policy which can be used as criteria to trigger some network slice operations (e.g., network slice parameters modification) when the pre-configured thresholds are met (max number of PDU sessions or max number of UE is reached). This API also support the service operation for third party to request NSCE server to provide VAL server policy usage reporting data. |
| SS_NSCE_NSOptimization | This API enables the third party to request NSCE server to activating the policies for specific slice been configured by SS_NSCE_PolicyManagement. |
| SS_NSCE_Management_Service_Discovery | This API enables the third party to subscribe been notified about the information with expected exposure capability type and the related permissions for a target slice or for a given VAL application. |
| SS_NSCE_PerfMonitoring API | This API enables the third party to request to monitor network slice related performance and analytic. |
| SS_NSCE_PerfReportSubscription | This API enables the third party to request to report network slice related performance and analytic. |
| SS_NSCE_PerfReport | This API enables the third party to request for getting notify of network slice related performance and analytic report. |
| SS_NSCE_MultiSlicePerfReport | The consumer requests to get multiple slices related performance and analytics consolidated report. |
| SS_NSCE_MultiSlices_Optimization | This API enables the third party to request NSCE server to realize optimized and efficient resource usage between different slices, based on preferred QoS request from the vertical applications, the performance monitoring of multiple slices of one network (PNI-NPN slice and PLMN slice) and resource adjustment between different slices. |
| SS_NSCE_NetworkSliceAdaptation API or SS_ NSCE_NetworkSliceAdaptationTrigge | This API enables the third party to request NSCE server to remap VAL application to different network slice (and optionally DNN) and provide the new mapping information to VAL UE within the VAL applications. The request optionally includes the adaptation threshold of network slice adaptation. |
| SS_NSCE_Network Slice Diagnostics | This API enables the third party to request NSCE server to provide network slice diagnostics information, if the VAL server of third party has identified there is specific event where the application has experienced service degradation (reported errors from VAL client for degraded service (bad quality) |

NSCE Client supports two types of functional role:

1)	Provide assistant information required by NSCE server, to enable certain NSCE service (e.g., providing QoE information associated to certain applications), or

2)	 Acting as NSCE service consumer, to retrieval the slice information (associated to applications on UE) from NSCE server.

### 4.2.5	LM (location management) server/client

LM server provides the value-added services by utilizing UE position related capabilities of 3GPP core network by combining the information from CN and location client (e., g if the CN cannot retrieve non-3GPP location information from UE modem, or UE using non-3GPP connection). It enables third party’s application (application server or application client) to invoke the value-added location services in an easy-to-use way.

NOTE:	The detailed information about advantages of Location management service to application developers is described in clause 5.x.

Table 4.2.4-1: Example of LMS APIs for third party

| API Name | Usage of the APIs for third party |
| --- | --- |
| SS_LocationReporting | This API enables the third party to configure the location reporting trigger to the LM server, e.g., what location information is requested (e.g., geographic area), when the requesting client/VAL server is expecting to receive the location report. |
| SS_LocationInfoEvent | This API enables the third party to subscribe, receive and unsubscribe the UEs location information from the LM server, as well as modify an active subscription. |
| SS_LocationInfoRetrieval | This API enables the third-party request UE location information at any time by sending a location information request to the location management server, which may trigger LM server to immediately send the location report. |
| SS_LocationAreaInfoRetrieval | This API enables the third-party to obtain UE(s) information in an application defined proximity range of a location from the LM server. |
| SS_LocationHistoryInfoEvent | This API enables the third-party to configure a list of location tracing parameters to the LM server and obtain the UE history location information. |
| SS_SLPositioningManagement | This API enables the third-party to subscribe to LM Server for configuring VAL UEs for a Side Link positioning / ranging service. |
| SS_SRPositioningInformation | This API enables third-party to obtain Short-range based positioning information from the LM server |
| SS_ConfirmLocation | This API enables third-party application in the UE to verify the location measured from the UE (non-3GPP location information, e.g., GNSS) is valid location or not through the LM server for providing the appropriate services to the user based on the location. |
| SS_LocationMonitoring | This API enables the third-party to monitor the VAL UE's location in a given area of interest through the LM server. |

LM Client supports two types of functional role:

1)	Provide assistant information required by LM server, to enable certain location service (e.g., reporting the location information of UE that LM client resides on), or

2)	Acting as location service consumer, to request location service from LM server.

### 4.2.6	AIMLE (AIML enabler) server/client

Editor's Note:	This clause will provide introduction of AIMLE server and client.

### 4.2.7	Spatial Anchor server/client

The entities offering the spatial anchor services are Spatial Anchor server and Spatial Anchor client. These entities provides the capabilities to efficiently manage the spatial anchors belonging to the vertical application layer (VAL) entities that are offering metaverse services (e.g. VAL Server , VAL Client). Functionalities offered by the Spatial anchor services includes CRUD operations for managing the spatial anchors, spatial anchor discovery, spatial anchor grouping etc.

### 4.2.8	Spatial Map server/client

The Spatial map service offers the capabilities to the vertical application layer entities (VAL server, VAL client) for managing the spatial maps. The main functional entities are Spatial Map server and client that provides client and server-side functionalities respectively for managing spatial map for the vertical applications. Spatial map service provides a mechanism for the data sources to register with the Spatial Map enabler layer with the information like the type of data they offer and their availability period etc.

### 4.2.9	Digital Assets server/client

The Digital assets service provides functionalities to manage the digital assets related to metaverse services particularly Digital Avatars. The core entities of the Digital Service enabler are Digital Asset server and Digital Asset client. The digital asset service supports management of digital assets profile, digital asset media and discovery of digital assets.

The DA client/server supports the client/server-side functionalities for digital assets discovery, digital asset profile creation, update, retrieve and delete operations.

### 4.2. A	Group Management server/client

The group management is a SEAL service that offers the group management related capabilities to one or more vertical applications. Main functional entities of this service are Group management server and client.

Group management server offers functionalities to the VAL application layer entities for managing the groups. Apart from the regular support of CRUD operations, the Group management server provides capabilities such as location-based group creation, temporary group formation by combining multiple groups etc. that can be utilized by the VAL layer entities to support enriched features.

The group management client functional entity acts as the interface to the VAL application client for management of groups. A VAL system maintains groups corresponding to one or more vertical applications. The group management client interacts with the group management server for group management related operations. The group management client also supports interactions with the corresponding group management client between the two UEs.

# 5	The use cases and advantages to applications

## 5.1	Summary of SEAL services

Editor's Note:	Brief summary of the value of each the SEAL services.

This clause is to address the overall summary of the values and beneficial use case(s) of each SEAL service.

The supporting of the following are the key services that make SEAL layer distinct with other non-3GPP basis developer API Platform:

-	NSCE/NRM/SEAL-DD which are specific to the usage of 5G/4G network capabilities.

-	E-MMTel enabler which is specific the usage of IMS system capabilities.

-	LMS utilize both 3GPP based position and non-3GPP based position.

-	AI/ML enabler could support AI/ML service related to mobile UE, leveraging on the 3GPP network-for-AI capabilities

| Service type | Service description | Benefits to application provides |
| --- | --- | --- |
| NRM | Provide value added APIs by abstrating the capabilties served by core network layer entities MB-SMF/PCF/NEF APIs/SCEF/BMSC, to enable third party to use 3GPP QoS service in a simple way | Simply the usage of App providers (APP deveolper, APP owner) of 3GPP network capabilities to: - Improve qualitiy of application layer communication by using NRM API to tigger the 3GPP network connection with QoS supporting, compared to application layer communication over best effort 3GPP network connection. |
| SEAL DD | Provide Caching and Distribution between application content server and mobile UE by using various 3GPP network capabilities designed for different type of application data to optimization purpose. For example reliable transmission mechanism specific to URLLC data. | Futher improve QoE of the applications when using Caching and Distribution service, by supporting the usage of 3GPP network capabilities(including control plane and user plane) for QoS optimization |
| NSCE | Provide the API services to enable third party to retrive the slice SLA requirement automaticly, order slice product from MNO system, mornitor/change slice SLA, by aggregating the API services from OAM and Core network, SEAL layer services (service KQI information) | Provide the required functions(e.g deriving and recomending slice SLA to let a slice customer e.g., enterprise knows how much resource is required for there services) to enable third party to perform self-management operation on their dedicated slice, when NaaS is provided by operator OAM. |
| AI/ML enabler | AI/ML AIMLE provides APIs for enabling and managing AI/ML services It leverages the network capabilities of 5G systems to support secure and efficient connection for mobile application to utilize AI services from MNO | It could enable an MNO operator to use their AI capabilities to launch the new business to AI applications. A mobile AI application can gain from offload the AI task to another node to promote the model performance or latency performance. |
| LMS | LMS provides location management related functionality and location-based services to the 3rd party applications. It utilizes the core network related location capabilities and value add on top of it in-order to provide an enriched location service. | Availability of enriched Location information from core network along with the GPS co-ordinates. provides readily available supplementary location based services such as geofencing, location deviation monitoring etc., apart from reporting and fetching of location information. |
| Group management | Group management service provides the group management related operations considering the needs of multiple vertical applications. Apart from regular CRUD operations it provides the value-add services like location-based group creation, group regrouping, group member registration etc. | Re-usability : Single group management framework catering to multiple vertical needs easing the development time. |
| Configuration management | Configuration management is to enable ASP to provide configuring data applicable to different vertical applications. | Re-usability : Single configuration management framework catering to multiple vertical needs easing the development time. |
| Digital asset | Provides application enablement to Metaverse application by supporting following functionalities: Digital asset profile management: which includes creating, updating, retrieving and deleting the digital asset profile Digital asset discovery Digital asset media management – which includes uploading and downloading the digital asset media. | Capabilities to re-use the digital asset across multiple Metaverse applications (e.g. digital assets server provided by the MNO, which is trusted by different Metaverse applications to manage digital assets). |
| Spatial Anhor enabler and spitial mapping enabler | Offers the platform for managing the spatial anchors and spatial maps for metaverse services. Enables the re-use of spatial anchors and spatial maps across multiple verticals and thus improving the metaverse experience for the end users. | Capability to re-use the spatial anchors and maps across multiple verticals. Provides the insight on the spatial anchor usage to improve the effeciency |

## 5.2	Use cases and advantages for SEAL Service provided by SEALDD

Editor's Note:	Details about the value of each SEAL services and how it brings advantages to applications

### 5.2.1	Use case

CDN is known as a content distribution network. CDN is a global network server system that works together to provide super-fast delivery of Internet content such as website webpages, HTML pages, JavaScript files, stylesheets, images, and videos. When people try to access a website if using a CDN, then visitors will see a copy of the website that is cached from the main server.

Existing widely deployed CDN server use Over-The-Top mechanism. It assumes network layer provide transparent forwarding of IP packets carrying application data. Normally it treats 3GPP mobile network as the same as other type of connection type.

Figure 5.2.1-1: Existing CDN data transmission by transparent forwarding

3GPP network specifies several mechanisms for improving the transmission quality of application data (e.g., PDU Set based QoS handling as defined by 3GPP TS 23.501 [8] and 3GPP TS 23.502 [23]), which requires application layer entity (e.g., application server) support some enhanced features e.g., using RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in 3GPP TS 26.522 [9] and together with RTP Payload Format;

SEALDD provides some enhanced content delivery functions, which fully use of 5G network to perform content delivery e.g., by providing application layer information required for 3GPP network layer transmission quality enhancement compared to transparent forwarding, the SEALDD data gets the best performance supporting from network. Thus, SEALDD provides content distribution service to third party with better service performance than CDN using Over-The-Top mechanism.

### 5.2.2	Advantages and values of SEALDD Services

As it shows in figure 5.2.2-1, the application data information will be provided by SEAL DD in the way 3GPP system can read. So that 3GPP network layer can apply some enhancement transmission on SEALDD data.

Figure 5.2.2-1: SEALDD data transmission using differentiating-processing capability by 3GPP network

In addition to the user plane co-operation between SEALDD and Network layer, SEALDD server also use the network APIs from 5G core control plane NF by behaving as trusted or untrusted AF, to trigger PDU Set based QoS handling for different data type, retrieving the network layer information to improving application layer data transmission mechanism (as figure 5.1.2-2).

Note:	In some case, SEALDD server can also uses other SEAL service API e.g., NRM if deployed separated as 3GPP TS 23.433 [2].

Figure 5.2.2-2: SEALDD utilize 3GPP network APIs for different data traffic type

Compared to existing CDN, which provide the same transmission service to different applications. SEALDD could provide different mechanism to meet with different needs of applications as the following:

-	Application layer signalling between AC and AS for different applications with different data transmission requirements.

-	Best effort demanding applications;

-	Low latency demanding applications like signalling for MCX call.

-	Transmitting the application layer data for different applications with different data transmission requirement:

-	Low latency and high reliable demanding application like remote robot/machine control.

-	High data rate with low latency demanding application like Realtime XR video applications, Machine vision application….

-	Low latency like video call/voice call.

-	Low cost demanding like background data

-	Low power demanding like IoT device applications.

-	Multiple flows synchronisation transmission service like multi-modal XR applications

Benefits to application providers (application owners/developers):

-	Different applications will get different QoS assurance services for different data type when compared to using over-the-top CDN.

## 5.3	Use cases and advantages for SEAL Services provided by NRM

### 5.3.1	Use case

Below list specifies the needs of the various applications utilizing the 3GPP network capabilities:

-	URLLC as mentioned in clause 7.2 of 3GPP TS 22.261 [15]

-	High data rate and low latency as mentioned in clause 7.6 of 3GPP TS 22.261 [15]

-	Low latency e.g IMS/MCX Voice call/video call

-	Background data transfer

-	supporting low power IOT device

The QoS mechanism supported by 5G network layer (NEF/SCEF or PCF/PCRF, MB-SMF) can be used for the case where the application providers negotiate the QoS parameters required for their applications and sign on business by offline way, or single QoS service for allow all application types can dynamically activating the network QoS service.

Since QoS assurance for different type of target applications requires different network resources and can cause different charging policy with different price, operator can need to further splitting QoS services into different types of QoS service. NRM service is defined to transform the 4G/5G network layer APIs to SEAL layer service API, to ease the usage of 3GPP QoS service (e.g., selecting the QoS services based on application needs).

### 5.3.2		Advantages and values of NRM Services

Figure 5.3.2-1: Capabilities of 5G system transformed by NRM service

The NRM service transforms 3GPP network capabilities and UE capabilities into the following types of service, enabling third party application providers to select and use network layer connection with different QoS services based on application needs, as shown in Figure 5.y.2-1:

NOTE:	The NRM sever can directly connect to the core network entities which provides required capabilities when deployed in trusted domain of MNO. NRM may also connect to SCEF/NEF when deployed in untrusted domain or even in trusted domain.

-	4G/5G unicast network connection with different QoS

-	4G/5G multicast/broadcast network connection with QoS

-	5G network connection for TSC/TSN data flows

-	5G connection for background data transfer

-	PDU session connection for low power IOT device

Benefits to application providers (application owners/developers):

-	Simplifies the usage of 3GPP network capabilities by App providers (APP deveolper, APP owner)

-	By using the NRM service APIs, QoE of the application can be improved when compared to application layer communication over best effort 3GPP network connection.

## 5.4	Use cases and advantages for SEAL Services provided by NSCE

### 5.4.1	Use case

Some of the vertical industry players could rent dedicated network slice(s) for their vertical applications. They can self-manage the network slice related operation themselves, including determining the services required, network slice SLA, ordering a network slice product with the determined SLA from MNO, triggering the activation and de-activation of network slices, changing the configuration etc.

When utilizing dedicated network slice services from MNO, vertical industry players do not know exactly how much network resources needed to be configured per slice. Vertical application usage of the network resources can be dynamic in nature and hence there is a need for vertical industry players to detect the changing demand for network resources and accordingly request MNO to increase/ decrease the network resources allocated for the network slice from what was initially provisioned.

The NSCE support the required capabilities and services to assist the network slice customer to manage the network slice(s) efficiently.

### 5.4.2	Advantages and values of NSCE

To enable the self-management of network slice(s) by vertical industry players, NSCE needs to utilize the network slice related APIs exposed by OAM, core network and UE client to offer the NSCE services as illustrated in Figure 5.4.2-1. When the NSCE is deployed within operator's trusted domain, it can access the 5GC directly, otherwise it should access the 5GC via NEF. The following are the example how NSCE can use different services from 3GPP system:

-	Deriving the appropriate recommendation on the Slice SLA parameters, by combining the UE reported application QoE, Slice information from Core network and OAM.

-	Allow third party to monitor and control the slice lifecycle.

-	Allow third party make the remapping of a slice to the applications

Figure 5.4.2-1: The required underlying APIs services for NSCE

Benefits to third party:

-	Availability of easier and effective way of managing the network slices (e.g., signing on appropriate network slice related SLA with MNO, making dynamic adaption on slice configuration by monitoring the Slice resource utilization and QoE, Re-mapping of slice to the application).

## 5.5	Use cases and advantages for SEAL Services provided by metaverse service

### 5.5.1	Use case

Spatial anchor, Spatial mapping, and digital asset are typical Metaverse Services for support Metaverse application.

A spatial anchor is an association between space and service information. The Spatial anchors SEAL service offers spatial anchors related capabilities (i.e., CRUDS) to one or more vertical applications. Spatial anchors enable the authorized users to place digital information at specific locations.

Spatial anchors can be used in many ways as described in clause 5.1 of 3GPP TR 22.856 [24]:

-	To show the path to the destination without obstructing the user's perception of their proximity.

-	To show the content that may be relevant to the user (i.e., potential shopper)

-	To provide personalized message (like the food of a restaurant liked by the user)

-	To display extensive information about the products of a shop (as described in clause 5.4 of 3GPP TR 22.856 [24])

Figure 5.5.1 -1: Localized Mobile Metaverse Services offering relevant information (from clause 5.1 of 3GPP TR 22.856 [24])

Detailed use case it captured in clause 5.1 and clause 5.5 of 3GPP TR 22.856 [24].

Digital asset is another service which can be offered to support metaverse application development as captured in clause 5.13 of 3GPP TR 22.856 [24]. It refers to the digitally stored information that is uniquely identifiable and can be used to realize value according to their licensing conditions and applicable regulations. Examples of digital assets include digital image (avatar), software licenses, gift certificates and files (e.g., music files) that have been purchased under a license that allows resale.

The network operators offer the digital asset management services for the users, with which some information (e.g., IDs) can be certified by the operator. The digital asset management services can also include

- 	The management of the digital asset container is performed according to the applicable regulations.

- 	The digital asset container has security properties (cannot be spoofed, access control with a policy determined by the user, etc.).

In the case of immersive XR media services, the user can choose, in the digital asset container, his/her digital representation and the related information, for example, the digital representation of the user (e.g., avatar), electronic money and associated financial services, identity, purchased items (the format of this information is at application layer and is not studied in 3GPP). This information can be used when accessing immersive XR media services or for real life services as the presentation of the identity.

### 5.5.2	Advantages and values of spatial anchor Services

A spatial anchor is an association between space and service information. The Spatial anchors SEAL service offers spatial anchors related capabilities (i.e., CRUDS) to one or more vertical applications. Spatial anchors enable the authorized users to place digital information at specific locations.

Spatial anchors can be used in many ways as described in clause 5.1 of 3GPP TR 22.856 [24]:

-	To show the path to the destination without obstructing the user's perception of their proximity

-	To show the content that may be relevant to the user (i.e., potential shopper)

-	To provide personalized message (like the food of a restaurant liked by the user)

### 5.5.3	Advantages and values of Digital asset Service

The SEAL digital assets service enables management and usage of digital assets in a secure and controllable way. Examples of digital assets include digital representation (avatar), software licenses, gift certificates, tokens and files (e.g., music files) that have been purchased.

Digital asset service can be used in many ways:

-	Avatar of a human can be used in collaborative and concurrent engineering in an XR meeting (clause 5.3 of 3GPP TR 22.856 [24])

-	To play immersive games and to attend digital live concert (clause 5.6 of 3GPP TR 22.856 [24])

-	To predict remote user's action (clause 5.9 of 3GPP TR 22.856 [24])

-	To operate patient in critical heath care (clause 5.10 of 3GPP TR 22.856 [24])

-	To have immersive calling experience (clause 5.11 of 3GPP TR 22.856 [24])

### 5.5.4	Advantages and values of Spatial Map Service

For spatial map service, sensing data gathered transparently is processed in order to identify the static and transient forms.

Spatial map service can be used in many ways (as described in clause 5.5 of 3GPP TR 22.856 [24]):

-	A government conducting a digital city project can build a 3D spatial map of outdoor environment of an entire city or public spaces such as outdoor parks or indoor offices of their government building.

-	A navigation service provider (or Spatial Localization Service) can build a 3D spatial map of outdoor environment of entire roads or public spaces. An operator’s partner or operator also can build a 3D spatial map of outdoor environment.

-	A customer wanting mapping of their indoor environment, e.g., the interior of a commercial space such as the cheese shop.

Detailed use case is captured in clause 5.5 of 3GPP TR 22.856 [24].

# 6	Deployment of SEAL entities and business relationship among stakeholders

## 6.1	Ecosystem stakeholders

Figure 6.1-1: Ecosystem stakeholders for the exposure of MNO’s service

Figure 6.1-1 shows the Ecosystem stakeholders for the exposure of MNO’s service. The following ecosystem stakeholders are covered:

-	The stakeholders who develop different parts of MNO system, including Network vendors for network equipment, platform vendors for exposure platform (supporting API framework entities), and Application enabler layer Developers for application enabler layer entities.

-	The stakeholders who manage marketplace used for display MNO APIs product to consumers. They can be MNO who own MNO marketing place, or third party who own third party’s Marketplaces

-	The stakeholders who use MNO APIs to develop applications, or produce new APIs.

## 6.2	Deployment alternatives

### 6.2.1	Common exposure entities

In the deployment alternative as figure 6.2.1-1, it assumes that one MNO would like to deploy the common exposure framework entities to expose all the APIs from difference domain to third party.

Within each domain (e.g.  OAM and CN), domain API gateway(NEF or EGMF) for some reasons (e.g., inter-domain security isolation ) may be optionally deployed.

Figure 6.2.1-1: common exposure among different domain

### 6.2.2	Separated exposure entities for different domain API

Figure 6.2.2-1: Separated exposure framework among different domains

In the deployment alternative as figure 6.2.1-1, it assumes that one MNO would like to deploy the separated exposure entitles to expose the APIs from difference domain to third party.

The O&M and CN domain API are managed and discovered by Common CCF, but CAPIF-2/CAPIF-2e are supported by different API gateway:

1)	CN APIs are invoked from the API gateway with the co-location of NEF and AEF;

2)	OAM APIs are invoked from the API gateway with the co-location of NEF and EGMF.

Annex A:
Relationship between SEAL Enablers and external organizations

Editor's Note:	This clause will describe how SEAL layer could be consumed in different vertical scenarios based on the analysis in solution#6 in the 3GPP TR 23.700-35.

# A.1	General description

SEAL layer services provides 3GPP enabler layer services, which are produced based on 3GPP system's capabilities. 3GPP SEAL layer services can be directly exposed to any applications, for example vertical applications of 5GAA,5GACIA as clause A.2.  Meanwhile there is also other organzaitions working on the APP developer fcous APIs in their domain.

From 3GPP perspective, 3GPP API consumers can also be a third-party platform. There are no general restrictions on the API(s) consumer. It is possible that 3GPP application enabler layer( includingSEAL layer services) are selected by another API platform as SBI (e.g OP platform defined in GSMA Open gateway, or any hyperscaler’s API platforms), and making API formate transforming when needed to serve their target consumers as clause A.3.

NOTE:	The content in Annex A is to provide the 3GPP understanding on the potential mapping relationship between 3GPP SEAL and the architecture defined by external organizations. It does not mandate any external SDO's decision on whether to adopt 3GPP SEAL service or re-specify their own value-add services by extra standard efforts.

# A.2	Vertical applications consume 3GPP APIs directly

Figure A.2-1: Independent value-added service system with 3GPP SEAL layer

3GPP NBI including the APIs supporting network layer services, and also the APIs supporting application enabler layer services. These APIs can be directly invoked by vertical applications including applications defined in 5GAA, or 5GACIA etc.

External SDO platform can also support APIs for applications. It can only invoke 3GPP APIs supporting network layer services and defined their self-defining value-added layer services, then expose the 3GPP network layer services and value-added layer services by their defining non-3GPP NBI.

The non-3GPP NBI can expose the same services or different services with what are exposed by 3GPP NBI (possible with different stage-3 API design).

It up to each external platform provider/SDO's decision whether and how to make alignment with 3GPP NBI for the same services.

MNO operator can deploy 3GPP platform (together with 3GPP enabler layer services) and /or external SDO platform.

# A.3	SEAL layer services as the SBI of external API platform

Figure A.3-1: Standard alternatives of exposure platform for market

3GPP application enabler layer including SEAL layer APIs are selected by external SDO as SBI In this solution, it is assumed external platform selects the 3GPP API(s) including network layer and application enabler layer APIs as southbound. Based on their customer's requirement, external platform can use the network layer services by adopting network API. Or it can use application enabler layer services by adopting 3GPP application enabler API(s).

External platform can perform API transformation upon 3GPP API(s), if 3GPP API(s) cannot be directly matched to their application layer's requirement.

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-10 | SA6#69 | S6-254792 |  |  |  | TS skeleton | 0.0.0 |
| 2025-10 | SA6#69 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-254660, S6-254661, S6-254662, S6-254692, S6-255590, S6-255591, S6-255592, S6-255672, S6-255674, S6-255675, S6-255676 | 0.1.0 |
| 2025-11 | SA6#70 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-255590, S6-255591, S6-255592, S6-255672, S6-255674, S6-255675, S6-255676 | 0.2.0 |
| 2026-02 | SA6#71 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-260287, S6-260288, S6-260360, S6-260524, S6-260525, S6-260527, S6-260528, S6-260529, S6-260530, S6-260532, S6-260534, S6-260756, S6-260775 | 0.3.0 |