3GPP RAN TSG Meeting #100		RP-231176

Taipei, Taiwan, 12 – 16th June 2023

Agenda item:	9.2.1

Source:	Apple Inc.

Title:	Regulatory update for the 6GHz frequency range

WI/SI:	FS_6GHz_LTE_NR

Release:	Rel-18

Document for:	Approval

# 1	Introduction

Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations is the RAN level study item, which aims at capturing the latest information and status of the regulatory decisions for the 6GHz frequency range. In this discussion paper we present a text proposal for the corresponding TR 37.890 that captures the following information:

-	ITU region 2: Regulatory decisions in Argentina.

-	ITU region 3: Regulatory decisions in Singapore.

# 2	Text proposal

-------------------------------------------------- TP BEGIN --------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	RP-172804: “Feasibility Study on 6 GHz for LTE and NR”, Ericsson, Verizon Wireless, Qualcomm Incorporated.

[3]					ITU-R Radio Regulations, Articles, Edition 2016;

[4]		FCC ONLINE TABLE OF FREQUENCY ALLOCATIONS, 47 C.F.R. § 2.106, December 13, 2017;

[5]	FCC 17-104, Notice of Inquiry, “Expanding Flexible Use in Mid-Band Spectrum Between 3.7 and 24 GHz”;

[6]	Comments of IEEE 802, in GN Docket No. 17-183;

[7]	APPLE INC., BROADCOM LIMITED,,CISCO SYSTEMS, INC., FACEBOOK, INC., GOOGLE LLC, HEWLETT PACKARD ENTERPRISE, INTEL CORPORATION,MEDIATEK INC., MICROSOFT CORPORATION, and QUALCOMM INCORPORATED, in GN Docket No. 17-183;

[8] 	Reply Comments of the Wireless Internet Service Providers Association, in GN Docket No. 17-183;

[9]	Comments of Ericsson, in GN Docket No. 17-183;

[10]	Comments of T-Mobile USA, in GN Docket No. 17-183;

[11]	Comments of Verizon, in GN Docket No. 17-183;

[12]	Reply Comments of the Satellite Industry Association, in GN Docket No. 17-183;

[13]	Reply Comments of the Fixed Wireless Communications Coalition, in GN Docket No. 17-183;

[14]	Comments of Dynamic Spectrum Alliance, in GN Docket No. 17-183;

[15]	Comments of the National Spectrum Management Association, in GN Docket No. 17-183;

[16]	Comments of CTIA, in GN Docket No. 17-183;

[17]	Reply Comments of Cisco Systems, Inc., in GN Docket No. 17-183;

[18] 	Reply Comments of WI-FI Alliance, in GN Docket No. 17-183;

[19]	PART 15 - Radio Frequency Devices, Title 47 of electronic Code of Federal Regulations;

[20]	The European Table of Frequency Allocations and applications in the frequency range 8.3 kHz and 3000 GHz (ECA Table), October 2017;

[21]	RSCOM17-53rev1- Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for wireless access systems including radio local area networks in the band 5925-6425 MHz for the provision of wireless broadband services.

[22]	ETSI TR 103 524 System Reference document (SRdoc), “Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 5 925 MHz to 6 725 MHz”, v1.1.1, October 2018

[23]	CEPT/ERC/REC 74-01: “Unwanted Emissions in the Spurious Domain”;

[24]	ECC Report 302, “Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz, May 2019.

[25]	FCC Notice of Proposed Rulemaking. FCC 18-147. October 24, 2018

[26]	ETSI TR 103 612, "IMT cellular networks; Mobile/Fixed Communication Network (MFCN) in the frequency range 6 425 - 7 125 MHz", v1.1.1, December 2019

[27]	ETSI TR 103 631, "Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 6 725 MHz to 7 125 MHz", v1.1.1, March 2019

[28]	ECC Report 302, “Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz”

[29]	CCSA-TC5-WG8-2019-003 Project Proposal on the feasibility study of IMT system using 5925-7125MHz frequency band, http://www.ccsa.org.cn/tc/meeting.php?meeting_id=6243#

[30]	World Radiocommunication Conference 2019 (WRC-19) Provisional Final Acts, ITU-R https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.13-2019-PDF-E.pdf

[31]				Report and order and further notice of proposed rulemaking, FCC 20-51

[32]	ECC Report 316, “Sharing studies assessing short-term interference from Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) into Fixed Service in the frequency band 5925-6425 MHz”, 21 May 2020

[33]	CEPT Report 075, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report B: Harmonised technical parameters for WAS/RLANs operating on a coexistence basis with appropriate mitigation techniques and/or operational compatibility/coexistence conditions, operating on the basis of a general authorisation. , November 2020

[34]	ECC Decision (20)01; “On the harmonised use of the frequency bands 5945 to 6425 MHz for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLAN)”, November 2020

[35]	ETSI TR 103 524, "System Reference document (SRDoc); Wireless access systems including radio local area networks (WAS/RLANs) in the band 5925 MHz to 6725 MHz"

[36]	EN 303 687, “"6 GHz RLAN Harmonised Standard for access to radio spectrum", Draft

[37]	doc 2.1_LS to 3GPP TSG RAN – ENG (18th meeting of the RCC Commission on Spectrum and Satellite Orbits), [to be updated]

[38]	CEPT Report 073, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report A: Assessment and study of compatibility and coexistence scenarios for WAS/RLANs in the band 5925-6425 MHz, Approved on 6 March 2020 by ECC.

[39]	Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

[40]	Ofcom,  "Improving spectrum access for Wi-Fi; Spectrum use in the 5 GHz and 6 GHz bands", July 2020, URL: https://www.ofcom.org.uk/__data/assets/pdf_file/0036/198927/6ghz-statement.pdf

[41]	Communications & Information Technology Commission, "Spectrum Outlook for Commercial and Innovative Use 2021- 2023", January 2021, URL: https://www.citc.gov.sa/ar/new/publicConsultation/Documents/Spectrum%20Outlook%20for%20Commercial%20and%20Innovative%20(2021-2023).pdf

[42]	Innovation, Science and Economic Development Canada, "Decision on the Technical and Policy Framework for Licence-Exempt Use in the 6 GHz Band", May 2021, URL: https://www.ic.gc.ca/eic/site/smt-gst.nsf/eng/sf11698.html

[43]	National Telecommunications Agency (ANATEL), "ACT NO. 1306", February 2021, URL: https://sei.anatel.gov.br/sei/modulos/pesquisa/md_pesq_documento_consulta_externa.php?eEP-wqk1skrd8hSlk5Z3rN4EVg9uLJqrLYJw_9INcO7uvjUt3vSOwT_4Z5fukj9yIzPErY4KWH5cpE9W_9hcTZkCG-vLPIdpXyuhgMG-L9M-uBLoSdAAXO0clb3SIt1i

[44]	Ministerio de Transportes y Comunicaciones, "Resolución Ministerial N° 373-2021-MTC/01", April 2021, URL: https://cdn.www.gob.pe/uploads/document/file/1861732/Resolución%20Ministerial%20nro%20373-2021-MTC/01.pdf

[45]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 1985 EXENTA", October 2020, URL: https://www.bcn.cl/leychile/navegar?idNorma=1109333&idParte=9841504&idVersion=&r_c=6

[46]	RP-210957, Liaison statement to 3GPP TSG RAN on inclusion of the frequency band 6425-7125 MHz in 3GPP specification for 5G-NR/IMT-2020 systems, Regional Commonwealth in the filed of Communications.

[47]		COMMISSION IMPLEMENTING DECISION (EU) 2021/1067, on the harmonised use of radio spectrum in the 5 945-6 425 MHz frequency band for the implementation of wireless access systems including radio local area networks (WAS/RLANs), 17 June 2021.

[48]		The National Telecommunications Commission (CONATEL) of Honduras, "Resolution NR 003/21", March 2021, URL: http://www.conatel.gob.hn/doc/Regulacion/resoluciones/2021/NR003-21.pdf

[49]		The Superintendencia de Telecomunicaciones (SUTEL), "DECRETO EJECUTIVO N° 42924-MICITT", April 2021, URL: https://www.imprentanacional.go.cr/pub/2021/04/30/ALCA87_30_04_2021.pdf

[50]	Communications & Information Technology Commission, "Radio Spectrum Allocation and Use Regulation for WLAN Application", June 2021, URL: https://www.citc.gov.sa/en/new/publicConsultation/Documents/144207-en.pdf

[51]	Communications & Information Technology Commission, “Public Consultation on Spectrum Light Licensing”, August 2021, URL: https://www.citc.gov.sa/ar/new/publicConsultation/Documents/EN_PublicConsultationonLightLicensing-144301.pdf

[52]	Communications & Information Technology Commission, “WLAN regulations”, January 2022, URL: https://www.citc.gov.sa/en/mediacenter/pressreleases/Pages/2022022101.aspx

[53]	UAE Telecommunication Regulatory Authority, "Ultra-Wide Band and Short Range Devices", Version 4.0, December 2020, URL: https://tdra.gov.ae/-/media/About/regulations-and-ruling/EN/UWB-and-SRD-Regs-V4-0-pdf.ashx

[54]	National Telecommunications Regulatory Agency (ANRT), "Decision ANRT/DG/N°07/21", May 2021, URL: https://www.anrt.ma/sites/default/files/decision_a2fp_-vf-_mod_07.05.2021.pdf

[55]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 1325 EXENTA", July 2021, URL: https://www.bcn.cl/leychile/navegar?idNorma=1162978&idParte=10255091

[56]	Agencia Nacional del Espectro (ANE), "Por medio de la cual se modifica la Resolución 105 de 2020 y se actualiza el Cuadro Nacional de Atribución de Bandas de Frecuencia", October 2021, URL: https://www.ane.gov.co/Documentos%20compartidos/ArchivosDescargables/Normatividad/Planeacion_del_espectro/RESOLUCIÓN%20No%20000105%20DE%2027-03-2020(1)%20(1).pdf

[57]	African Telecommunication Union, "ATU-R Recommendation 005-0", July 2021, URL: https://www.atuuat.africa/wp-content/uploads/2021/08/En_ATU-R-Recommendation-005-0.pdf

[58]	African Telecommunication Union, "2nd ATU Preparatory Meeting for WRC-23 (APM23-2); Agenda Item 1.2", September 2021, URL: https://docs.google.com/document/d/1u0Oz6o3gnH6oTGY-sE6_BTarNf2RUuSx/edit

[59]		RCC Recommendation 1/21 “Harmonization of the technical conditions for 5G-NR / IMT-2020 systems in the RCC countries in the frequency band 6 425-7 125 MHz or in its portions” (RP-213605).

[60]	Recommendation ITU-R SM.329-12, Unwanted emissions in the spurious domain, 09/2012.

[61]	The Australian Communications and Media Authority, "Radiocommunications (Low Interference Potential Devices) Class Licence Variation 2022 (No. 1)", March 2022, URL: https://www.legislation.gov.au/Details/F2022L00249/Html/Text

[62]	HKCA 1081, " PERFORMANCE SPECIFICATION FOR RADIOCOMMUNICATIONS APPARATUS OPERATING IN THE 6 GHz BAND FOR WIRELESS LOCAL AREA NETWORK", April 2022, URL: https://www.ofca.gov.hk/filemanager/ofca/en/content_401/hkca1081.pdf

[63]	Radio Spectrum Management, "WLAN use in the 6 GHz Band: Outcomes and Other Updates", August 2022, URL: https://www.rsm.govt.nz/projects-and-auctions/completed-projects/wlan-use-in-the-6-ghz-band/

[64]	Ministry of Internal Affairs and Communications, "Technical conditions for introduction of 6GHz band wireless LAN", April 2022, URL: https://www.soumu.go.jp/menu_news/s-news/01kiban12_02000142.html

[65]	Malaysian Communications and Multimedia Commission (MCMC), "Class Assignment No. 1 of 2022", January 2022, URL: https://www.mcmc.gov.my/skmmgovmy/media/General/CA-No-1-of-2022_-signed_19012022.pdf

[66]	Communications Authority of Kenya (CAK), "Guidelines On the Use of Radiofrequency Spectrum by Short Range Devices", June 2022, URL: https://www.ca.go.ke/wp-content/uploads/2022/07/Guidelines-on-the-Use-of-Radiofrequency-Spectrum-by-Short-Range-Devices-2022.pdf

[67]	Communications Regulatory Authority (CRA), "Class License for the use of RLAN devices over 5925-6425 MHz Band", April 2022, URL: https://www.cra.gov.qa/-/media/System/D/2/5/8/D258CF18B83A5613B0D590193CB799CB/Class-License-WIFI-6E-Final-English-032022--V3.ashx

[68]	The Hashemite Kingdom of Jordan Telecommunications Regulatory Commision (TRC), "Short Range Station License", May 2022, URL: https://trc.gov.jo/EchoBusV3.0/SystemAssets/PDF/AR/الطيف%20الترددي%20الراديوي/ترخيص%20الترددات/التعليمات%20والاجراءات/20be3e3e-3013-47cf-8cbc-b0280118acc0_SRD%20license.pdf

[69]	<to be updated>

[70]	MINISTERIO DE TRANSPORTES Y TELECOMUNICACIONES; SUBSECRETARÍA DE TELECOMUNICACIONES, "RESOLUCIÓN 2844 EXENTA  MODIFICA RESOLUCIÓN N° 1.985 EXENTA, DE 2017, DE LA SUBSECRETARÍA DE TELECOMUNICACIONES", September 2022, URL: https://www.bcn.cl/leychile/navegar?idNorma=1181305&idVersion=2022-09-14

[71]	Agencia Nacional del Espectro (ANE), " RESOLUCIÓN No. 000737", November 2022, URL: https://www.mintic.gov.co/portal/715/articles-273182_recurso_1.pdf

[72]	Instituto Dominicano de las Telecomunicaciones (INDOTEL), “RESOLUCIÓN NÚM. 082-2022”, September 2022, URL: https://transparencia.indotel.gob.do/media/217225/res_082_2022.pdf

[73]	Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000217697&vSct=202275

[74]	Ministry of the digital development and communication, "Заседание ГКРЧ от 23 декабря 2022 г. (протокол № 22-65)", December 2022, URL: https://digital.gov.ru/ru/documents/8628/

[75]	Instituto Federal de Telecomunicaciones, “El pleno del IFT clasifica la banda de 5925-6425 MHz como espectro libre”, February 2023, press release 013/2023

[76]	Ente Nacional de Comunicaciones (ENACOM), "Resolución 756/2023", May 2023, URL: https://www.boletinoficial.gob.ar/detalleAviso/primera/287126/20230524

-------------------------------------------------- next section --------------------------------------------------

### 4.2.9	Colombia

On October 2021, Colombian National Agency (Agencia Nacional del Espectro) released a draft resolution [56] according to which the 5925-7125MHz frequency range is opened for the license-exempt operation. On November 2022 another resolution No. 000737 [71] was released setting the following technical parameters:

-	only indoor usage is allowed;

-	the use of devices located on oil rigs, automobiles, trains, boats and aircraft is not allowed with the exception of aircraft flying above 10,000 feet;

-	the operation of equipment intended to control or communicate with unmanned aerial vehicles is not allowed;

-	access point devices must be powered directly from the commercial electrical power network, the use of batteries is not allowed, and their structure must not support outdoor use;

-	access point devices must only work with an antenna integrated into their structure;

-	maximum EIRP of 30dBm for AP devices with maximum PSD of 5dBm/MHz;

-	maximum EIRP of 24dBm for CL devices with maximum PSD of -1dBm/MHz;

-	the spectral density should be attenuated 20 dB at a distance of 1 MHz from the end of the channel, 28 dB at a distance of one channel apart from the center of the channel, and 40 dB at a distance of 1.5 channels apart from the center of the channel;

-	emissions outside the 5925-7125MHz frequency range should be limited to -27 dBm/MHz;

### 4.2.10	Dominican Republic

On September 2022 the Instituto Dominicano de las Telecomunicaciones (INDOTEL) has published Resolución No. 082-2022 [72], which establishes operating conditions for Low and Very Low Power Equipment in the 5925-7125 MHz frequency band with the following conditions:

-	For indoor access points operating and for devices subordinate to it, the maximum power spectral density must not exceed 8 dBm/MHz. The maximum EIRP over the operating frequency band should not exceed 30 dBm.

-	For client-type devices the maximum power spectral density shall not exceed 2 dBm/MHz in any 1MHz chunk and the maximum EIRP over the operating frequency band should not exceed 24 dBm.

-	Very Low Power (VLP) devices with maximum power spectral density not exceeding -8 dBm/MHz and maximum EIRP over the operating frequency band not exceeding 14 dBm may be used outdoors.

-	For transmitters any emissions outside the 5925-7125 MHz frequency band must not exceed -27 dBm/MHz.

-	The power spectral density must be suppressed by 20dB at 1MHz outside the edge of the channel, by 28dB at channel width from the center from the channel and by 40dB at one and a half times the channel width from the center of the channel. At frequencies between 1MHz outside the channel edge and one channel width from the center of the channel, the limits should be linearly interpolated between 20 dB and 28 dB of suppression. At frequencies between one channel width plus 1MHz, and one channel with one-half times the channel width the limits must be linearly interpolated between 28 dB and 40 dB of suppression.

-	The operation of indoor access points and fixed client devices is prohibited in automobiles, trains, ships and airplanes, except for large aircraft while flying above 3000 meters and only access points indoor operating in the frequency band 5925-6425MHz.

-	Usage of indoor access points and fixed client-type devices is prohibited for control or communications with unmanned aircraft systems.

-	Indoor access points and fixed client-type devices must use an antenna integrated or permanently fixed to its body or structure, as well as use a contention-based communications protocol.

### 4.2.11	Argentina

On 24th May 2023, Argentinian Ente Nacional de Comunicaciones (ENACOM) decided to allocate full 6GHz band, 5925 - 7125MHz, for the unlicensed usage [76].

-	Operation is for exclusive indoor use, both private and for the provision of ICT services.

-	Emissions may not cause interference to authorized stations of a service assigned with primary category, nor claim protection against harmful interference caused by stations of a service assigned with category primary.

-	An access point or subordinate device may operate in the 5925-7125MHz band with a maximum Equivalent Isotropically Radiated Power (EIRP) of 30 dBm and a maximum EIRP spectral density of 5 dBm/MHz.

-	A client device can operate in the 5925-7125MHz band with a maximum Equivalent Isotropically Radiated Power (EIRP) of 24 dBm and a maximum EIRP spectral density of -1 dBm/MHz.

-	Emissions outside the band 5925-7125MHz should not exceed an EIRP of -27 dBm/MHz.

-	The operation of access points, client devices and subordinate devices is restricted to inside buildings only.

-	Both access points, client devices and subordinate devices must use a "contntion protocol" that allows several users to share the same spectrum.

-	The operation of these devices is prohibited in vehicles (cars, trains, boats and planes). However, the operation of this equipment will be allowed in aircraft when they fly above 10,000 feet.

-------------------------------------------------- next section --------------------------------------------------

### 4.3.7	Malaysia

In January 2022, Malaysian Communications and Multimedia Commission (MCMC) published a new class assignment document, which in particular specifies unlicensed operation in the 5925-6425MHz frequency range with the following technical conditions:

-	The maximum EIRP for indoor use only is limited to 200mW;

-	The radiated PSD for indoor devices is limited to 10mW/MHz;

-	The maximum EIRP for indoor and outdoor devices is limited to 25mW;

-	The radiated PSD for outdoor and indoor devices is limited to 1.25mW/Hz;

-	The radiated PSD for narrowband usage (channel bandwidth below 20MHz) for outdoor and indoor devices is limited to 10mW/MHz;

### 4.3.8	Singapore

On 12th May 2023, the Singapore authority Infocomm Media Development Authority (IMDA) decided to allocate additional spectrum for Wi-Fi use in the lower segment of the 6GHz band, in 5925 - 6425MHz.

NOTE:	The exact regulatory parameters are not decided yet.

-------------------------------------------------- next section --------------------------------------------------

## 4.5	Regulatory parameters comparative for license-exempt

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
|  | UAE | LPI (see 4.1.5) | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI (see 4.1.7) | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP (see 4.1.7) |  | 14 dBm |  |  |
|  | Saudi Arabia | LPI (see 4.1.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 10dBm/MHz |  |
|  | Kenya | LPI (see 4.1.8) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.1.8) |  | 14dBm | 1dBm/MHz |  |
|  | Qatar | LPI (see 4.1.9) | 5925 – 6425MHz | 23dBm |  |  |
|  |  | VLP (see 4.1.9) |  | 14dBm |  |  |
|  | Jordan | LPI (see 4.1.10) | 5925 – 6425MHz | 23dBm |  |  |
|  |  | VLP (see 4.1.10) |  | 14dBm |  |  |
|  | Russian Federation | LPI (see 4.1.11) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.1.11) |  | 14dBm | 1dBm/MHz |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 6425MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  |  | VLP (4.2.5) |  | 17 dBm |  |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  | Dominican Republic | LPI (see 4.2.10) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 8dBm/MHz (AP) 2dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.10) |  | 14dBm | -8dBm/MHz |  |
|  | Argentina | LPI (see 4.2.11) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |
|  | Hong Kong | LPI (see 4.3.3) | 5925 – 6425MHz | 23dBm | 10dBm/MHz | In accordance with ETSI EN 303 687 |
|  |  | VLP (see 4.3.3) |  | 14dBm | 1dBm/MHz |  |
|  | Australia | LPI (see 4.3.4) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.4) |  | 14dBm | 1dBm/MHz |  |
|  | New Zealand | LPI (see 4.3.5) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.5) |  | 14dBm | 1dBm/MHz |  |
|  | Japan | LPI (see 4.3.6) | 5925 – 6425MHz | 23dBm |  | -27dBm/MHz (below 5925MHz) -13dBm/MHz and -19dBm/MHz (above 6425MHz) |
|  |  | VLP (see 4.3.6) |  | 14dBm |  | -37dBm/MHz (below 5925MHz) -13dBm/MHz and -19dBm/MHz (above 6425MHz) |
|  | Malaysia | LPI (see 4.3.7) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.3.7) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) |  |
| Note 1: For the exact conditions of operation, refer to the sub-clause which is mentioned in parenthesis (e.g. for the exact conditions for operating SP in US, refer to sub-clause 4.2.1.) |  |  |  |  |  |  |

-------------------------------------------------- TP END --------------------------------------------------