3GPP TSG RAN WG1 #115	        		   			R1-2312784

# Chicago, USA, November 13th – 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0585 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Maintenance of support for NR NTN enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections on support for NR NTN enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture that the UE transmits any PUCCH with repetitions over the indicated number of slots when the UE uses a common PUCCH resource to transmit the PUCCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for NR NTN enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged parts are omitted ***

### 9.2.6	PUCCH repetition procedure

A UE that does not have dedicated PUCCH resource configuration and indicates a capability to transmit with repetitions a PUCCH with HARQ-ACK information [11, TS 38.321], determines a number of NPUCCHrepeat slots for repetitions of a PUCCH transmission with HARQ-ACK information based on an indication by numberOfPUCCHforMsg4HARQACK-RepetitionsList. If numberOfPUCCHforMsg4HARQACK-RepetitionsList provides more than one values, the DAI field in a DCI format 1_0 with CRC scrambled by a TC-RNTI scheduling a PDSCH reception that includes a UE contention resolution identity indicates NPUCCHrepeat from the more than one values. The UE transmits any PUCCH with repetitions over NPUCCHrepeat slots before dedicated PUCCH resource configuration is provided. The UE transmits each repetition of the PUCCH using frequency hopping as described in Clause 9.2.1.

In the remaining of this clause, a UE without dedicated PUCCH resource configuration determines a value of a parameter, if applicable, according to Table 9.2.1-1 and/or as specified above in this clause for a PUCCH transmission with repetitions from the UE.

A UE can be indicated to transmit a PUCCH over NPUCCHrepeat slots using a PUCCH resource, where

-	if the PUCCH resource is indicated by a DCI format and includes pucch-RepetitionNrofSlots, NPUCCHrepeat is provided by pucch-RepetitionNrofSlots

-	otherwise, NPUCCHrepeat is provided by nrofSlots

## *** Unchanged parts are omitted ***