3GPP TSG RAN Meeting #103								RP-240086

Maastricht, Netherlands, March 18-21, 2024

## Status Report to TSG

Agenda item:			9.3.1.1

| WI / SI Name | NR MIMO Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_Ph5 |  |  |  |  |
| Unique ID | 1020089 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-234007 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 15% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Darcy Tsai |
|  | Company | MediaTek |
|  | Email | darcy.tsai@mediatek.com |
| Rapporteur | Name | Chai Li |
|  | Company | China Mobile |
|  | Email | chaili@chinamobile.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#116, the following agreements were made.

UE-initiated/event-driven beam management

Agreement

On UE-initiated/event-driven beam report, at least of following aspects should be included:

- Trigger-event detection for beam reporting by UE

  - UE monitors RS to assess if a beam-reporting trigger condition has been met

  - FFS: Trigger condition for declaring beam-reporting event

- Beam-report transmission by UE

  - Signaling contents in the beam report

  - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission

    - MAC-CE

    - UCI

    - Others are not precluded.

On UE-initiated/event-driven beam report, the following aspects may be included:

- UE requesting UL resource(s) for the beam report

- UE notifying transmission of beam report

- gNB preconfigured resources

Other procedure(s) as required

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold.

- Further study trigger events, including the following example as a starting point

  - Event-1: Quality of the current beam is worse than a certain threshold.

  - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - Event-3: Quality of a new beam is better than a certain threshold.

  - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

  - Others are not precluded.

- Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event

Agreement

On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

- Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting.

- FFS: Semi-persistent CSI-RS and aperiodic CSI-RS.

- FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused.

- FFS: Whether/how to specify filtering operation for L1-RSRP.

Agreement

On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP

- FFS: Study and decide whether additional contents can be supported.

- FFS: L1-RSRP format, e.g., absolute and/or differential value.

- Note: Above does not imply to preclude discussion on L1-RSRP filtering.

- The actual reported content depends on the triggering event

  - Support of one or multiple events will be discussed separately

CSI

Agreement

For the Rel-19 NR Type-I/II codebook refinement and CRI-based CSI for up to 128 CSI-RS ports, reuse the EVM agreed for Rel-18 NR Type-II Doppler codebooks (cf. R1-2205289) for SLS with the following refinement:

- Center frequencies can be replaced by the following options (companies should state their assumption): 3.5GHz, 4.9GHz, 6GHz, 7GHz.

- Companies should state their assumptions on TXRU structure and the associated TXRU-to-port virtualization for >32 ports

- The UEs can be placed both indoor (with 80% probability, 3kmph) and outdoor (with 20% probability, companies should state the assumed UE speed)

- Lower RU can also be assumed (companies should state the assumed RU)

- Companies should state the assumed traffic models and ISD

Agreement

For the Rel-19 NR CJT calibration reporting, reuse the EVM agreed for Rel-18 NR Type-II CJT codebooks (cf. R1-2205289) for SLS with the following refinement:

- Adding ISD=500m as an option for inter-site CJT

- Delay offsets caused by propagation delays from the UE to the TRPs and, whenever applicable, the ISI incurred by the composite (summed across selected TRP(s)) channel impulse response should be modelled/accounted

  - In addition, initial timing offsets among TRPs can be included, e.g. from TR 38.855 for positioning or uniformly distributed [-0.5CP:0.5CP]

- Per-TRP frequency offset (relative to the UE) is modelled as a uniform random variable with [-x:x]

  - Companies should state the assumed value of x, e.g. 0.05ppm, 0.1ppm (per Table 6.5.1.2-1 of TS.38.104)

- For TDD, per-TRP DL/UL phase misalignment is modelled as a uniform random variable with [-y:y] where y is {}

- For TDD, per-TRP DL/UL timing misalignment is modelled as a uniform random variable with [-z:z]

  - Companies should state the assumed value of z, e.g. 65ns(from RAN4)

- For FDD scenarios, Rel-18 Type-II CJT codebook is assumed

- For TDD scenario, SRS-based (e.g. SVD) precoding is assumed

- Companies should state the assumed sub-carrier spacing, e.g. 15kHz, 30kHz

Agreement

For the Rel-19 NR CJT calibration reporting, use the following EVM for LLS with the following refinement:

- Purpose: alternative to SLS to observe the impact of misalignment and gain by proposed solutions, as the frequency offset/time misalignment have impacts in granularities of subcarrier/symbol levels

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | CDL-C channel model in TR 38.901 |
| Delay Spread | 300ns |
| BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (2, 8, 2, 1, 1; 2, 8), (dH, dV) = (0.5, 0.8) |
| TRP number | 2 |
| UE antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2), (dH, dV) = (0.5, 0.5) |
| UE number | 1, 4 |
| MCS | Link Adaption |
| Bandwidth | 20RB, 145RB |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| MIMO Rank | rank = 2 per UE |
| UE speed | 3km/h |
| Precoding granularity | 2RB, 4RB, 8RB, 16RB, 32RB |
| SRS periodicity | 10ms |
| DMRS | Type 2 DMRS, double-symbol, or Type 1 DMRS |
| DL DMRS channel estimation | LMMSE channel estimation |
| Frequency offset | Uniformly distributed delay difference between [0, x], companies should state the assumed value of x, e.g., 0.05ppm, 0.1ppm. |
| Delay difference | a uniformly distributed delay difference between [0, y], companies should state the assumed value of y, e.g., CP length, 1.67us, 65ns. |

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for up to 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, support aggregating at least K=2, 3, or 4 legacy NZP CSI-RS resources with equal number of ports

- FFS (by RAN1#116bis): Mapping from CSI-RS resource index/port index per resource and port index to CSI/PMI calculation, also considering co-existence with pre-Rel-19 UEs

- FFS (by RAN1#116bis): whether the Rel-18 CJT CMR restrictions (where all resources shall be located within 2 consecutive slots) are reused, or additional restriction(s) are introduced (e.g. PCoffset, CDM type, RS density, TD (co-located in a slot)/FD locations, QCL, …)

- FFS (by RAN1#116bis): Whether legacy resource configuration for interference measurement is reused, or additional restriction(s) are introduced

- FFS: Whether all the K CSI-RS resources are associated with a same CSI-RS resource set or not

- Note: If the supported number of ports does not require aggregation of 3 resources, K=3 can be removed

Agreement

For the Rel-19 Type-II codebook refinement for up to 128 CSI-RS ports, in accordance to the WID, the following enhancement areas are supported:

- Adding new (N1, N2) values for the Rel-16 eType-II regular and Rel-18 Type-II Doppler regular codebooks where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and O1=O2=4

  - FFS: How to configure the aggregated NZP CSI-RS resources when AP-CSI-RS resources are configured as CMR for Rel-18 Type-II Doppler codebooks

- Adding new PCSI-RS values for Rel-17 FeType-II Port Selection (PS) codebook where PCSI-RS (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources

There will be separate UE feature groups for each of the enhanced codebooks.

Note: Per WID objective 2b,

- No other legacy codebook design aspects (such as SD/FD/DD basis design including O1/O2, W2/combining coefficient design, codebook parameter definitions and respective values) can be modified.

- Only RI=1-4 is supported

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, the following use cases are assumed:

- For per-TRP delay offset reporting:

  - Use case 1.1: TRP selection

  - Use case 1.2: delay offset compensation for at least one TRP to ensure the CJT-composite delay spread doesn’t exceed a pre-defined dynamic range/threshold

- For per-TRP frequency offset (FO) reporting:

  - Use case 2.1: TRP selection

  - Use case 2.2: per-TRP FO compensation at NW side

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, support the following:

- The UE is configured with NTRP NZP CSI-RS resources/resource sets via higher-layer (RRC) signalling where NTRP∈{1, 2, 3, 4}

  - FFS (by RAN1#116bis): Whether further restriction(s) on applicable NZP CSI-RS resources/resource sets need to be introduced (e.g. number of ports, only TRS with multiple resource sets, TD/FD locations, QCL assumptions)

- For the purpose of CJT calibration reporting, decide, by RAN1#116bis, from the following

  - Opt1:  The UE reports for all the configured NTRP NZP CSI-RS resources/resource sets

  - Opt2: The UE reports for N out of NTRP NZP CSI-RS resources/resource sets where the selection of N resources/resource sets is dynamically signalled by the NW to the UE

  - Opt3: The UE reports for N out of NTRP NZP CSI-RS resources/resource sets where the selection of N resources/resource sets is performed by the UE and included in the CSI report

- Interference measurement is not supported, hence neither CSI-IM nor NZP CSI-RS resource for interference measurement can be configured (analogous to Rel-18 TDCP)

- FFS: One-part or two-part UCI on PUSCH (analogous to Rel-18 TDCP)

- The priority of the CSI report(s) is the same as CSI report(s) not carrying L1-RSRP or L1-SINR (analogous to Rel-18 TDCP)

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {(Dn,offset, dn), n=0, 1, …, N – 1} where

- Dn,offset is a B-bit indicator representing the delay offset associated with the n-th CSI-RS resource/resource set

  - For the reference CSI-RS resource/resource set nref, the value of Dnref,offset is assumed 0 and not reported

    - FFS (by RAN1#116bis): Whether nref is fixed, NW-configured, or is included in the report (selected by the UE)

  - The value of Dn,offset indicates the interval [δi,δi+1) which the delay offset falls into

    - Down-select, by RAN1#116bis, from the following

      - Alt1: δ0,δ1,…,δM-2 is uniformly spaced between 0 and AD, i.e. δi=i×ADM-2,   i=0,1,…, M-2, with M=2B

      - Alt2: δ0,δ1,…,δM-2 is uniformly spaced between -AD and AD, i.e. δi=-AD+i×2ADM-2,   i=0,1,…, M-2, with M=2B

    - Each interval [δi,δi+1)  corresponds to a codepoint, and (-∞,δ0) and/or [δM-2,∞) represent ‘out-of-range’

    - FFS (by RAN1#116bis): supported quantization alphabet(s) (including AD, M)

- dn is a 1-bit indicator associated with the n-th CSI-RS resource/resource set, indicating whether the measured delay offset, plus delay spread, is inside or outside a pre-defined range/interval

  - FFS (RAN1#116bis): The pre-defined range(s), e.g. CP length or its multiple

- FFS: Detailed UCI design on codepoint encoding details

- FFS: The need for a new QCL assumption

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, in addition to the already agreed use cases, the following use cases are assumed:

- For per-TRP DL/UL Rx-Tx phase misalignment reporting:

  - Use case 3.1: TRP selection

  - Use case 3.2: per-TRP DL/UL Rx-Tx phase compensation at NW side for reciprocity (e.g. using both CSI-RS and SRS for measurement)

Agreement

For the Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, at least for RI=1-4, study and decide, by RAN1#116bis, from the following:

- Scheme1 (baseline): Adding new (N1, N2) values for the Rel-15 Type-I single-panel codebook where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources

  - FFS: Whether to further down-select between mode-1 (L=1) and mode-2 (L=4)

  - FFS: For rank-3/4, follow legacy mechanisms for <16 ports, or for >=16 ports

- Scheme2: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - For each layer, reuse legacy Rel-16 eType-II SD basis with L=1 to determine the DFT-based SD basis candidates

      - FFS: Whether the indication of selected SD basis indices follows Rel-16 eType II or Rel-15 Type I

    - For 4≥RI>1, L=1 SD basis vector is independently selected for different layers

      - FFS: SD basis selection restriction to reduce SD overhead for RI>4

  - W2 structure: Layer-specific inter-polarization M-PSK co-phasing where M is further down-selected from {2, 4, 8, 16}

  - FFS: Common SD vector selection for a pair of layers (reduced total number of bits for SD basis vector selection); layer multiplexing via orthogonal polarization co-phasing for the layer pairs with common SD vector (reduced number of bits for co-phasing indication for the layer pairs with common SD vector).

  - FFS: Additional support for L>1

- Scheme2B: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - For each layer, determine L=1 DFT-based SD basis candidate

      - FFS: Whether the indication of selected SD basis indices follows Rel-16 eType-II or Rel-15 Type-I

    - For 4≥RI>1, L=1 SD basis vector is independently selected for different layers

    - FFS: Common SD vector selection for a pair of layers (reduced total number of bits for SD basis vector selection), SD basis selection restriction to reduce SD overhead for RI>4

  - W2 structure:

    - Option 1: Layer-specific inter-polarization amplitude and phase scaling (single scaling coefficient per polarization)

      - FFS: WB/SB amplitude and phase reporting.

    - Option 2: Layer-specific intra-polarization (two scaling coefficients per polarization) amplitude and phase scaling.

      - FFS: WB/SB amplitude and phase reporting.

    - FFS: Rel-15 3-bit WB amplitude and M-PSK co-phasing and M is further down-selected from {2, 4, 8, 16}.

- Scheme3: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - W1 structure:

    - Reuse legacy Rel-16 eType-II SD basis with L>1 to determine the DFT-based SD basis candidates, and indication of SD basis indices follows Rel-16 eType-II

    - For 4≥RI>1, L>1 SD basis vectors are commonly selected across layers

      - FFS: SD basis selection restriction to reduce SD overhead for RI>4

  - W2 structure:

    - Option 1: Layer-specific sub-band SD basis selection (1 out of L) and inter-polarization M-PSK co-phasing where M is further down-selected from {2, 4, 8, 16}

    - Option 2: Layer-specific wideband SD basis linear combination and inter-polarization scaling coefficient (e.g., amplitude scaling + M-PSK co-phasing) where M is further down-selected from {2, 4, 8, 16}

- Scheme4: Using legacy Rel-15 Type-I codebook including legacy (N1, N2) values per NZP CSI-RS resource (or port group) where the PMI (associated with W1 and W2) is calculated according to

  - W1 structure: Reuse legacy Rel-15 Type-I SD basis with L=1 or L=4 for either each or some of the NZP CSI-RS resources (or port groups)

  - W2 structure: inter-NZP CSI-RS resource (or port group) co-phasing along with reusing legacy Rel-15 Type-I inter-polarization co-phasing per NZP CSI-RS resource (or port group)

    - inter-CSI-RS resource (or port group) co-phasing is used to combine the different PMIs to come up with a single precoder with >32 ports

- Scheme5: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and extending the set of orthogonal beams for the selection of the second beam based on the Rel-15 Type-I single-panel codebook

  - (i1,1, i1,2) is used to refer to the 1st beam as in legacy Rel-15 Type-I

  - The 2nd beam is selected from the extended set of orthogonal beams of size: N1-1N2O2+N2-1N1O1-N1-1N2-1

  - FFS: whether to apply any restrictions to the extended orthogonal set of beams

- Scheme6: Adding new (N1, N2) values where 2N1N2 (>32) is the total number of CSI-RS ports across aggregated NZP CSI-RS resources, and

  - Beam(s) is(are) selected for each antenna group or NZP CSI-RS resource.

  - Inter-group (or CSI-RS resource) co-phasing along with inter-polarization co-phasing per group (or CSI-RS resource) are used to combine different beam(s), FFS using scalar quantization or vector quantization for the co-phasings

FFS (by RAN1#116bis): Down-select (O1, O2) value between (2,2) and (4,4), whether (O1, O2) and/or (q1, q2) is layer-common or layer-specific

FFS (by RAN1#116bis): Whether extension of Rel-15 Type-I MP codebook for Rel-19 Type-I is also supported

FFS (by RAN1#116bis): Whether to introduce larger L values (e.g. 6, 8, 10)

FFS: Whether to refine CBSR design to reduce RRC overhead

Agreement

For the Rel-19 Type-II codebook refinement for up to 128 CSI-RS ports,

- Fully reuse the legacy Rel-16 eType-II design (and for PS codebook, the Rel-17 FeType-II PS design) for UCI omission rules

- On the supported parameter combinations, decide, by RAN1#116bis, whether further restriction on the the legacy Rel-16 eType-II design (and for PS codebook, the Rel-17 FeType-II PS design) to reduce/limit PMI overhead and/or UE complexity is necessary

- On the definition and detailed design of UCI parameters, fully reuse the legacy Rel-16 eType-II design (and for PS codebook, the Rel-17 FeType-II PS design), except for SD basis selection indication

  - On SD basis selection indication, decide, by RAN1#116bis, whether refinement on the legacy Rel-16 eType-II design (and for PS codebook, the Rel-17 FeType-II PS design) is necessary to reduce UE memory requirements

- On CBSR, decide, by RAN1#116bis, whether refinement on the legacy Rel-16 eType-II design (and for PS codebook, the Rel-17 FeType-II PS design) is necessary to reduce RRC overhead (including moving (N1,N2) configuration out from CBSR IE)

- Further study the rules on CPU occupation, resource counting, and Z2/Z2’ in conjunction with Rel-19 Type-I

Agreement

For the Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support also RI=5-8, with lower priority than RI=1-4:

- FFS: Reduced complexity design, whether to reuse legacy RI=5-8 structure, based on the outcome of RI=1-4

Agreement

For the Rel-19 Type-I and Type-II codebook refinement for up to 128 CSI-RS ports, at least support the legacy time-domain behaviours for CSI reporting (and the applicable CSI-RS time-domain behaviors). That is,

- For Rel-19 Type-I:

  - Aperiodic CSI (channel(s) and multiplexing rules follows legacy)

  - Periodic CSI (channel(s) and multiplexing follows legacy), hence wideband PMI only

  - Semi-persistent CSI (channel(s) and multiplexing rules follows legacy)

- For Rel-19 Type-II:

  - Semi-persistent CSI on PUSCH (channel(s) and multiplexing rules follows legacy)

  - Aperiodic CSI (channel(s) and multiplexing rules follows legacy)

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, in accordance to the WID, extend the Rel-15 CRI-based CSI reporting as follows:

- A UE is configured to measure KS>1 NZP CSI-RS resources with equal number of ports, with up to 32 ports per NZP CSI-RS resource

  - Note: The maximum number of ports per NZP CSI-RS resource for a given value of KS will be discussed separately

- Containing the information of M “quadruplets” {(CRIn, RIn, PMIn, CQIn), n=0, …, M–1} in one CSI reporting instance where the value range of M (≤KS) is {1, …, min(X, KS)}

  - FFS (by RAN1# 116bis): The supported value(s) of X (candidates are 2, 4, 6, KS)

  - FFS (by RAN1# 116bis): Whether the value of M is NW-configured via higher-layer (RRC) signalling, or UE-selected (as a part of CSI report), or a combination of the two

- A same legacy codebook (with up to 32 ports) is configured for (associated with) all M “quadruplets”

FFS: detailed UCI design/optimization (e.g. overhead reduction)

FFS: Whether solution to allow CSI reporting for larger number of CSI-RS resources across multiple CSI reports is supported

FFS: whether further restriction(s) on CMR configuration is needed, including relation with IMR

FFS: the packing order of the information of M “quadruplets”, CSI omission rule

FFS: Whether all the K CSI-RS resources are associated with a same CSI-RS resource set or not

FFS: Whether KS, maximum # ports per resource, and X depend on codebook type

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, the supported combinations of KS value and the maximum number of ports per NZP CSI-RS resource are as follows:

- FFS: UE capability on KS and the number of ports per resource

| KS | Maximum # ports per resource |
| --- | --- |
| 2, 3, 4 | 32 |
| 5, 6, 7, 8 | 16 |

Conclusion

The Rel-17 NCJT CSI is not extended to accommodate the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports

Agreement

For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II and Rel-18 Type-II Doppler for up to 128 CSI-RS ports, as well as Rel-19 Type-I codebook refinement for up to 128 CSI-RS ports, support the following (N1, N2) values:

| Total # CSI-RS ports across aggregated resources (=P) | (N1, N2) |
| --- | --- |
| 48 | (8,3) |
|  | (6,4) |
| 64 | (16,2) |
|  | (8,4) |
| 128 | (16,4) |
|  | (8,8) |

The support of total # CSI-RS ports across aggregated resources (=P) and (N1, N2) are subject to UE capability.

- For the Rel-19 Type-II codebook refinement based on Rel-16 eType-II regular codebook, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities

- For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler regular codebook, the (N1,N2) values for P=64 are supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, in addition to the already agreed use cases, the following use cases are assumed for study:

- Use case 3.3: For TDD reciprocity, timing offset report for at least one pair of TRPs to assist TRP synchronization (i.e. to align TRP inherent timing without propagation delay)

Whether there is any spec support associated with this use case is FFS

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, support reporting, in one CSI reporting instance, {FOn , n=0, 1, …, N – 1, n≠nref}, where FOn denotes the measured frequency offset associated with the n-th CSI-RS resource/resource set relative to the reference CSI-RS resource/resource set nref

- For the reference CSI-RS resource/resource set nref, the value of FOnref is assumed 0 and not reported

  - FFS (by RAN1#116bis): Whether nref is fixed, NW-configured, or is included in the report (selected by the UE)

  - FFS (by RAN1#116bis): whether the UE assumes that the measured and reported per-TRP frequency offsets can include Doppler shift (if existent) associated with the reference CSI-RS resource/resource set nref

  - FFS: Measurement resource/resource set for FO reporting

- Down-select, by RAN1#116bis, from the following

  - Alt1. The value of FOn indicates a uniformly quantized FO between –AFO and AFO, or 0 and AFO

    - FFS (by RAN1#116bis): supported quantization alphabet(s) (including AFO and resolution) for FOn

  - Alt2. The value of FOn indicates the interval [φi,φi+1) which the FO falls into

    - Alt2A: φ0,φ1,…,φM-1 is uniformly spaced between -AFO and AFO, i.e. φi=-AFO+i×2AFOM-1,   i=0,1,…, M-1

    - Alt2B: φ0,φ1,…,φM-1 is uniformly spaced between 0 and AFO, i.e. φi=i×AFOM-1,   i=0,1,…, M-1

- FFS: whether “out-of-range” value/interval is needed, or whether TRP selection value is needed

- FFS: If N<NTRP, the rest (NTRP–N) resources/resource sets are indicated with a state “out of range”

- FFS: Detailed UCI design

- FFS: The need for a new QCL assumption

- FFS the unit of AFO: e.g. absolute (e.g. in Hz) or relative (e.g. in ppm/ppb relative to carrier frequency, or fraction of SCS), dependence on RS configuration

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, given the NTRP configured NZP CSI-RS resources/resource sets and the selected N resources/resource sets, study and decide, by RAN1#116bis, whether to support reporting, in one CSI reporting instance, {n,m n=0, 1, …, N – 1, n≠nref, m=0,1,…,M-1}, where n,m denotes the measured phase offset between the n-th CSI-RS resource/resource set and the reference CSI-RS resource/resource set/ nref for the m-th frequency unit

- FFS: whether M>1 (sub-band reporting) is needed or not (M=1, i.e. wideband reporting)

- For the reference CSI-RS resource/resource set nref, the value of nref is assumed 0 and not reported

  - FFS (by RAN1#116bis): Whether nref is fixed, NW-configured, or is included in the report (selected by the UE)

- The value n,m indicates a uniformly quantized phase between –A and A, or 0 and A

  - FFS (by RAN1#116bis): supported quantization alphabet(s) (including A and resolution) for n,m

- FFS: Detailed UCI design

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding the supported codebook(s) for calculating CQI/PMI/RI on each of the M CRI(s), decide, in RAN1#116bis, between the two alternatives:

- Alt1: only Rel-15 Type-I Single Panel codebook

- Alt2: Rel-15 Type-I Single Panel codebook and the Rel-16 eType-II codebook

Agreement

For the Rel-19 Type-II refinement based on Rel-17 FeType-II PS, support the following PCSI-RS value(s) {48, 64}

- For the Rel-19 Type-II codebook refinement based on Rel-17 FeType-II PS, PCSI-RS =64 is supported as a part of the respective basic feature, while PCSI-RS =48 is supported as a separate UE capability

3Tx uplink

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission.

13100,13010,13001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission.

13100100, 13100001, 13 001001

Agreement

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, down-select one of

- Alt1 – Support configuration of X 4-port SRS resources in a resource set where one the ports is muted

- Alt2 – Support configuration of X SRS resources with equal/unequal number of ports (e.g. 2 + 1 or 1 + 1 + 1) in a resource set,

The value for X is FFS, and it will be determined according to the selected alternative.

Agreement

For a 3TX UE, down-select one of the following options for the number of PTRS ports,

- Option-1: A single PTRS port is supported.

- Option- 2: Up to 2 PTRS port may be configured.

Agreement

For a 3-antenna-port codebook-based UL transmission, study PTRS-DMRS association.

Agreement

For a 3-antenna-port codebook-based UL transmission, study power split for each port of SRS and PUSCH.

Agreement

For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability.

Conclusion

There is no consensus in RAN1 to support antenna switching for 3TX UE in Rel-19

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for LLS evaluation

- Companies may provide additional evaluation results per their case of interest

- LLS is optionally used for 3Tx UL evaluation, if needed

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Waveform | CP-OFDM |
| SCS | 30 KHz |
| System bandwidth | 20 MHz, 100 MHz |
| Scheduled PRBs | 5, 25, 50, 260 PRBs |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ |
| UE speed | 3 Km/h |
| Number of Layers | Adaptive, Fixed (reported by company) |
| AMC | Adaptive, Fixed (reported by company) |
| DMRS configuration | Type 1; 1 front loaded + 1 additional symbol |
| Channel estimation | Real |
| Channel Model | CDL-A (30ns), CDL-B (100ns), CDL-C (300ns) |

Agreement

For performance evaluation of 3TX UE, adopt the following Table as the reference EVM for SLS evaluation.

- Companies may provide additional evaluation results per their case of interest.

Note: The considered EVM is to be used as a baseline set of assumption for future potential studies related to 3TX.

| Parameter | Value |
| --- | --- |
| Frequency range | 3.5 GHz |
| Multiple access | OFDMA |
| Numerology | 14 CP-OFDM symbol slot SCS , 30 KHz |
| Scenario | eMBB: Dense Urban (200m), 3.5GHz Outdoor FWA: UMa (500m), 3.5GHz |
| UE Outdoor/Indoor (%) | eMBB: 80%, 20% Outdoor FWA: 100%, 0% |
| System bandwidth | 20 MHz, 100 MHz |
| gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝) | (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Optional: Classical: two 8x1 xpols, 4λ apart; 4 TXRUs tilt=[104°] |
| gNB antenna radiation pattern parameters | - Outdoor/Indoor Per 38.901, Table 7.3-1 |
| gNB receiver noise figure | 5dB |
| gNB receiver | MMSE-IRC |
| gNB scheduler | Single user with proportional fair |
| Modulation | - Up to 64 QAM - Up to 256QAM |
| MIMO scheme | SU-MIMO with rank adaptation |
| UE speed | 3 Km/h |
| UE antenna config | eMBB: - Xpol+1pol; isotropic ULA - Xpol+1pol; 110°, 4 dBi Outdoor FWA: - Xpol+1pol; isotropic ULA - 3 directional 1pol: 110°, 4 dBi |
| Traffic model | - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional) |
| Suggested benchmarking | Rel-15 2Tx non-coherent |
| Precoder granularity | Wideband |
| Power control | Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario |
| UE power rating | eMBB: 23 dBm, UL FPTx mode 0 or Rel-15 power scaling Outdooe FWA: 31 dBm, UL FPTx mode 0 |
| Metric | UL mean-user throughput, 5%-ile and 95%-ile UPT |

Agreement

For performance evaluation of 3TX UE, consider following reference configurations,

- A linear array (1D) of single-polarized antenna configuration with a spacing of 0.5λ,

  - For example:		 |←0.5λ→|←0.5λ→|

- A configuration of a cross-polarized and a single-polarized antennas,

  - For example:		 ×←0.5λ → ⁄

Agreement

For SRS configuration supporting codebook-based UL transmission by a 3TX UE, one SRS resource set is configured for single TRP operation.

Agreement

For codebook-based transmission by a 3TX UE,

- Only PUSCH antenna ports 1000, 1001, 1002 are used

- Option- 2: Subject to UE capability, up to 2 PTRS ports may be configured in PTRS-UplinkConfig,

  - FFS whether a single bit or 2 bits are used for PTRS-DMRS association indication.

Above is only for single panel transmission.

Asymmetric DL-sTRP/UL-mTRP

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset:

- When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

- Note: it does not intend to increase the number of maintained PLs per cell.

- FFS: whether to support associating joint TCI state (if supported) with a PL offset.

Further study whether/how to apply a PL offset on PDCCH-order PRACH transmission too.

- FFS: how to determine the Tx beam of PRACH towards UL TRP

- Note: this does not imply to support 2 TA for single-DCI based system.

Agreement

To facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, support configuring two closed-loop PC adjustment states for SRS in one CC, both of which are separate from that of the PUSCH.

Agreement

Study how to indicate TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured two SRS CLPC adjustment states, down-select from the following options:

- Option 1: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA;

- Option 2: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeB;

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

- Option 4: enhance DCI format 1_1 and/or 0_1 to indicate TPC for SRS CLPC adjustment states

- Option 5: enhance the legacy DCI format 2_3 by introducing a new Type for higher layer parameter srs-TPC-PDCCH-Group

- Option 6: new DCI format to indicate TPC for SRS CLPC adjustment states

- Other options are not precluded.

For the Options1, 2, 3 and 5, consider at least the following Alts as possible examples:

- Alt1: In DCI format 2_3, add one additional TPC command for each CC configured with two SRS CLPC adjustment states,

  - the first TPC command is associated with the first SRS CLPC adjustment state and the second TPC command is associated with the second SRS CLPC adjustment state.

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

- Alt3: use two different TPC-SRS-RNTIs for DCI format 2_3:

  - DCI format 2_3 with CRC scrambled with the first TPC-SRS-RNTI and the second TPC-SRS-RNTI indicates the TPC command for the first and second SRS CLPC adjustment state, respectively.

- Alt4: Implicit method:

Agreement

To support two SRS CLPC adjustment states, study and possibly down-select at least one from the following Alts:

- Alt1: SRS CLPC adjustment state is associated with SRS resource set

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 in the TCI state indicates one of the SRS CLPC adjustment states

- Alt3: Add one extra parameter in P0AlphaSet-r17 of TCI state to indicate one of those two SRS CLPC adjustment states

- Alt4: SRS CLPC adjustment state is associated with SRS resource usage type

Note: Other alternatives are not precluded

Agreement

Down-select one from the following alternatives:

- Alt1: Use only RRC to update the PL offset associated with the UL TCI state

- Alt2: In addition to RRC, MAC-CE can be used to update the PL offset associated with the UL TCI state

  - FFS: Details on MAC CE

Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2.

- Joint TCI state mode can be configured at least for FR1

#### 2.1.2	Remaining Open issues

UE-initiated/event-driven beam management:

- Remaining details on enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - Remaining details on UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - Remaining details on UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

CSI:

- Remaining details on CSI support for up to 128 CSI-RS ports, targeting FR1

  - Remaining details on Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Remaining details on Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Remaining details on extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

- Remaining details on UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Remaining details on inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

3Tx uplink:

- Remaining details on non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Asymmetric DL-sTRP/UL-mTRP:

- Remaining details on enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Remaining details on two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

## 2.2	RAN2

#### 2.2.1	Agreements

- Not yet started

#### 2.2.2	Remaining Open issues

- Not yet started

.

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

- Not yet started

#### 2.4.2	Remaining Open issues

- Not yet started

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template