3GPP TSG-RAN5 Meeting #102	R5-241186

# Athens, Greece, 26th February – 1st March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4303 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UPIP TC 7.1.3.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPIP_SEC_LTE-RAN-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reference to core requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added reference to core requirements, TS 36.331 clause.5.3.1.2, that was listed in test cases now covered by this test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The list of core requirements will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.3.2.6	Integrity protection / Correct functionality of UP integrity protection / multiple DRBs

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when {  Functionality of integrity algorithms is taken into use on the MCG DRB }

then { UE performs correct integrity protection function in PDCP entities associated with MCG DRB }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when {  Functionality of integrity algorithms is taken into use on the SCG DRB }

then { UE performs correct integrity protection function in PDCP entities associated with SCG DRB }

}

7.1.3.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 33.401, clauses 7.3.3 and 7.3.4, TS 36.300, clause 14.1 and TS 36.331, clause.5.3.1.2.

[TS 33.401, clause 7.3.3]

NOTE a:	Only EN-DC capable UEs and EN-DC capable eNBs support UP integrity protection. Therefore, the eNB can only activate UP integrity protection with a UE that is EN-DC capable and supports user plane integrity protection with EPS.

If the UE indicates that it supports user plane integrity protection with EPS in EIA7 in the EPS security capability, the MME shall provide UP integrity protection policy for each E-RAB to the eNB during the Attach/Dedicated bearer activation/Dedicated bearer modification procedure as specified in TS 23.401 [2]. The MME receives UP integrity protection policy from SMF+PGW-C via SGW.

NOTE 1:	The SMF+PGW-C can be locally configured with UP integrity protection and confidentiality policy. However, the SMF+PGW-C only sends UP integrity protection policy to the upgraded SGW. The SMF+PGW-C, SGW and MME can use GTP-C signalling compatibility concepts to jugde whether to send UP integrity protection policy to the peer.

The UP integrity protection policy shall indicate whether UP integrity protection shall be activated or not for all DRBs belonging to that E-RAB.

The eNB shall be locally configured with UP integrity protection policy. If the eNB receives UP integrity protection policy from the MME, the eNB shall use the received UP integrity protection policy, otherwise, the eNB shall use the locally configured UP integrity protection policy if EIA7 in the EPS security capability indicates that the UE supports user plane integrity protection with EPC.

The locally configured UP integrity protection policy on eNB should be set as "preferred".

The eNB shall activate UP integrity protection per each DRB, according to the UP integrity protection policy, using RRC signalling as defined in clause 7.3.4. If the UP integrity protection policy indicates "Required", the eNB shall activate UP integrity protection. If the eNB cannot activate UP integrity protection, and when the UP integrity protection policy is "Required", the eNB shall reject establishment of UP resources for the E-RAB and indicate reject-cause to the MME. If the UP integrity protection policy is " Not needed ", the eNB shall not activate UP integrity protection.

…

[TS 33.401, clause 7.3.4]

AS UP integrity protection activation shall be done as part of the DRB addition procedure using RRC Connection Reconfiguration procedure as described in this clause, see Figure 7.3.4 -1.

As defined in Clause 7.3.3, the MME may send the UP integrity protection policy to the eNB. If the MME does not send the UP integrity protection policy, the eNB may use locally configured UP integrity protection policy.

Figure 7.3.4-1: User plane (UP) integrity protection activation mechanism

1a.	This RRC Connection Reconfiguration procedure which is used to add DRBs shall be performed only after RRC security and UP ciphering have been activated as part of the AS security mode command procedure defined in Clause 7.2.4.5 and the UE indicates that it supports use of user plane integrity protection with EPC.

1b.	The eNB shall send the RRC Connection Reconfiguration message to the UE for UP security activation containing indication for the activation of UP integrity protection for each DRB according to the security policy.

The eNB shall select the NR integrity algorithm and indicate it in the RRC Connection Reconfiguration procedure to the UE. The selected NR integrity algorithm  corresponds to the EPS integrity algorithm which the eNB selected and indicated to the UE in the AS Security Mode Command procedure.

1c.	If UP integrity protection is activated for DRBs as indicated in the RRC Connection Reconfiguration message, and if the eNB does not have KUPint, the eNB shall generate KUPint and UP integrity protection for such DRBs shall start at the eNB.

2a.	UE shall verify the RRC Connection Reconfiguration message. If successful, if UP integrity protection is activated for DRBs as indicated in the RRC Connection Reconfiguration message, and if the UE does not have KUPint, the UE shall generate KUPint and UP integrity protection for such DRBs shall start at the UE.

2b.	If the UE successfully verifies integrity of the RRC Connection Reconfiguration message, the UE shall send the RRC Connection Reconfiguration Complete message to the eNB.

When the UE receives the RRC Connection Reconfiguration message then the UE shall use the EPS algorithm which corresponds to the NR algorithm indicated in the RRC Connection Reconfiguration message for UP integrity protection.

If UP integrity protection is not activated for DRBs, the eNB and the UE shall not integrity protect the traffic of such DRB and shall not put MAC-I into PDCP packet.

[TS 36.300, clause 14.1]

The following principles apply to E-UTRAN security:

…

-	Separate AS and NAS level security mode command procedures are used. AS level security mode command procedure configures AS security (RRC and user plane) and NAS level security mode command procedure configures NAS security. Both integrity protection and ciphering for RRC are activated within the same AS SMC procedure. User plane ciphering is activated at the same time as RRC ciphering. An EN-DC capable UE supporting user plane integrity protection (see TS 24.301 [20]) when connected to E-UTRA/EPC (as specified in TS 33.401 [22]) shall support integrity protection for all DRBs (MN and SN terminated) at any data rate, up to and including the highest data rate supported by the UE for both UL and DL. When supported, user plane integrity protection with NR PDCP can be activated (on a per radio bearer basis) upon DRB addition.

…

[TS 36.331, clause.5.3.1.2]

…

The AS applies different security keys: one for the integrity protection of RRC signalling (KRRCint), one for the ciphering of RRC signalling (KRRCenc) and one for the ciphering of user data (KUPenc). For the UE capable of user plane integrity protection when it is connected to E-UTRA/EPC (TS 33.401 [32]), the AS applies a security key for integrity protection of user data (KUPint) for the DRBs that are configured to apply integrity protection of user data. All AS keys are derived from the KeNB key. The KeNB is based on the KASME key for E-UTRA/EPC, or KAMF for E-UTRA/5GC, which is handled by upper layers.

…

7.1.3.2.6.3	Test description

7.1.3.2.6.3.1	Pre-test conditions

-	Same Pre-test conditions as in clause 7.1.3.0 with Connectivity set to EN-DC without message condition UE TEST LOOP MODE A, then RRCConnectionReconfiguration message including MobilityControlInfo IE is transmitted on E-UTRA Cell 1 to reconfigure SRB1, SRB2 and MCG DRB from E-UTRA PDCP to NR PDCP and Test Loop Function (On) with UE test loop mode A (message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU) according to TS 38.508-1 [4]

7.1.3.2.6.3.2	Test procedure sequence

Table 7.1.3.2.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits PDCP PDU on MCG DRB integrity protected. | <-- | PDCP PDU | - | - |
| 2 | Check: Does the UE transmit looped back PDCP PDU integrity protected on MCG DRB? | --> | PDCP PDU | 1 | P |
| 3 | SS transmits PDCP PDU on SCG DRB integrity protected. | <-- | PDCP PDU | - | - |
| 4 | Check: Does the UE transmit looped back PDCP PDU integrity protected on SCG DRB? | --> | PDCP PDU | 2 | P |

7.1.3.2.6.3.3	Specific message contents

Table 7.1.3.2.6.3.3-1: ATTACH REQUEST (Preamble)

| Derivation Path: 36.508 [7] Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE network capability |  |  |  |  |
| All octets with the exception of octet 4, bit 1 |  | Any allowed value |  |  |
| EPS-UPIP supported (octet 4, bit 1) |  | 1 | EPS-UPIP supported |  |

Table 7.1.3.2.6.3.3-2: SecurityModeCommand message (Preamble)

| Derivation Path: TS 36.508 [7] Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| securityModeCommand-r8 SEQUENCE { |  |  |  |  |
| securityConfigSMC SEQUENCE { |  |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |  |
| cipheringAlgorithm |  | Set according to PIXIT parameter for default ciphering algorithm |  |  |
| integrityProtAlgorithm |  | Set according to PIXIT parameter for default integrity protection algorithm | null algorithm is not allowed |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.2.6.3.3-3: RRCConnectionReconfiguration (Preamble)

| Derivation Path: 36.508[47] Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| mobilityControlInfo |  | MobilityControlInfo-HO-SameCell | As per Table 7.1.3.2.6.3.3-4 |  |
| radioResourceConfigDedicated |  | RadioResourceConfigDedicated-DRB-Rel-Add | As per Table 7.1.3.3.1.3.3-5 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message according to TS 38.508-1 [4], table 4.6.1-13 with condition EN-DC_HO. |  |  |
| } |  |  |  |
| } |  |  |  |
| sk-Counter-r15 | Increment the value by 1 from the previous value |  |  |
| nr-RadioBearerConfig1-r15 | RadioBearerConfig-MCG-SCG-DRB-NR-PDCP | Table 7.1.3.2.6.3.3-6 |  |
| nr-RadioBearerConfig2-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.2.6.3.3-4: MobilityControlInfo-HO-SameCell (Table7.1.3.2.6.3.3-3)

| Derivation Path: 36.508 [7], Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of E-UTRA Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 7.1.3.2.6.3.3-5: RadioResourceConfigDedicated-DRB-Rel-Add (Table 7.1.3.2.6.3.3-3)

| Derivation Path: 36.508 [7], Table 4.6.3-19AAAAAD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |
| RadioResourceConfigDedicated-SRB2-DRB ::= SEQUENCE { |  |  |  |  |  |  |
| srb-ToAddModList | Not present |  |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |  |  |  |
| drb-ToAddMod[1] | DRB-ToAddMod-DEFAULT (8) using condition AM except pdcp-Config not included |  | entry 1 See TS 36.508 subclause 4.8.2 |  |  |  |
| } |  |  |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { | 1 entry |  |  |  |  |  |
| DRB-Identity[1] | 8 |  | entry 1 Same as the DRB Identity associated with the default EPS bearer |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 7.1.3.2.6.3.3-6: RadioBearerConfig-MCG-SCG-DRB-NR-PDCP (Table 7.1.3.2.6.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132, condition MCG_NR_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 2 entries |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | Same as the default EPS bearer Identity | MCG DRB |  |
| } |  |  |  |
| drb-Identity | Same as the DRB identity associated with the default EPS bearer |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config | Table 7.1.3.2.6.3.3-7 |  |
| } |  |  |  |
| DRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | true |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config | Table 7.1.3.2.6.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Same as the ciphering algorithm configured Table 7.1.3.2.6.3.3-2 |  |  |
| integrityProtAlgorithm | Same as the integrity algorithm configured Table 7.1.3.2.6.3.3-2 |  |  |
| } |  |  |  |
| keyToUse | secondary |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.2.6.3.3-7: PDCP-Config (Table 7.1.3.2.6.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| integrityProtection | true |  |  |
| } |  |  |  |
| } |  |  |  |