3GPP TSG-RAN5 Meeting #102	R5-240219

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3008 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update cell specific parameters for NB-IoT NTN RA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For NB-IoT NTN random access test cases, there are some typo errors for serving satellite configuration. _RA and _RB values for 1Tx cells are incorrect, too. Core spec has already been corrected by R4-2321472. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct typo errors for serving satellite configuraiton. - _RA and _RB values for 1Tx cells should be 0dB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases are unclear, and work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 13.3.2	Random Access for Satellite Access

This clause provides the list of Random Access test cases for category NB1 UEs when connecting to a NTN cell using satellite access. The list of supported test configurations is provided in Table 13.3.2-1.

Table 13.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

#### 13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

##### 13.3.2.1.1	Test purpose

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Normal Coverage is according to the requirements when connected to a NTN NB-IoT cell, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach[5]. This test will verify the requirements in 3GPP TS 36.133[4] Clause 6.6A.2, Clause 6.6A.3 and Clause 7.20A.2 in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables 13.3.2.1.5-1, 13.3.2.1.5-2 and 13.3.2.1.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using AT command-based test approach.

##### 13.3.2.1.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both from release 17 and forwards.

##### 13.3.2.1.3	Minimum conformance requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window. The RA response window shall be started at the point in time indicated by clause 5.1.4 in TS 36.321[11].

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of re-transmissions defined by maxNumPreambleAttemptCE in the table 13.3.2.1.5-3 is reached.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

The UE shall send ACK if the Contention Resolution is successful.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

The UE shall select NPRACH resources and transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0. The rate of correct coverage enhancement level selection during repeated tests shall be at least 90%.

Note:	Correct coverage enhancement level selection is a prerequisite for testing the other NPRACH requirements.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.3.2.1.

##### 13.3.2.1.4	Test description

13.3.2.1.4.1	Initial conditions

Test Environment: Normal, TL/VL, TL/VH, TH/VL, TH/VH; as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.3.2.1.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

13.3.2.1.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 1 Ncell.

1.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	Set the parameters according to Tables 13.3.2.1.5-1 to 13.3.2.1.5-3 selecting values appropriate to the test condition of normal or extreme. Propagation conditions are set according to Annex B clause B.1.1.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when receiving random access response reception

4.1.	In Test 1, the UE shall send the preamble to the SS. In response to the first 2 preamble transmission attempts (the preamble is transmitted 2 times in each attempt), the SS shall transmit a random access response not corresponding to the transmitted random access preamble.

4.2.	The UE shall consider the random access response reception not successful then re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received random access response contain random access preamble identifiers that do not match the transmitted random access preamble.

4.3.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after 3 preamble transmission attempts have been received by the SS.

4.4.	The UE shall consider this random access response reception successful and transmit the msg3.

4.5.	Measure the power and timing of the first preamble transmission attempt. The power of the first preamble transmission attempt shall be -25 dBm with the accuracy specified in clause 6.3B.4 of TS 36.102. Measure the relative power and timing applied to additional preamble transmission attempts. The relative power of the last 2 preamble transmission attempts shall not exceed the values specified in clause 6.3B.4 of TS 36.102. The transmit timing of all NPRACH transmissions shall be within the accuracy specified in TS36.133 clause 7.20A.2. If the UE transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power, relative power and timing are only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

5.	Test 2: Correct behaviour when not receiving random access response reception

5.1.	Repeat step 1-3.

5.2.	In Test 2, the UE shall send the preamble to the SS. The SS shall not respond to the first 2 preamble transmission attempts (the preamble is transmitted 2 times in each attempt).

5.3.	The UE shall consider the random access response reception not successful then re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no random access response is received within the RA Response window. The RA response window shall be started at the point in time indicated by clause 5.1.4 in TS 36.321.

5.4.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after 3 preamble transmission attemps have been received by the SS.

5.5.	The UE shall consider this random access response reception successful and transmit the msg3.

5.6.	Measure the power and timing of the first preamble transmission attempt. The power of the first preamble transmission attempt shall be -25 dBm with the accuracy specified in clause 6.3B.4 of TS 36.102. Measure the relative power and timing applied to additional preamble transmission attempts. The relative power of the last 2 preamble transmission attempts shall not exceed the values specified in clause 6.3B.4 of TS 36.102. The transmit timing of all NPRACH transmissions shall be within the accuracy specified in TS36.133 clause 7.20A.2. If the UE transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power, relative power and timing are only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

6.	Test 3:  Not applicable.

7.	Test 4: Correct behaviour when receiving an incorrect message over Temporary C-RNTI

7.1.	Repeat step 1-3.

7.2.	In Test 4, the UE shall send the preamble to the SS. If the UE transmits an NPRACH preamble using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0, then count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

7.3.	The UE shall consider this random access response reception successful and transmit the msg3.

7.4.	The SS shall send a message addressed to the Temporary C-RNTI with a UE contention resolution identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

7.5.	The UE shall consider the contention resolution not successful then re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires. If the UE transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power is only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

8.	Test 5: Correct behaviour when receiving a correct message over Temporary C-RNTI

8.1.	In Test 5, the SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS. If the UE transmits an NPRACH preamble using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0, then count a success for the event "CE selection". Otherwise count a fail for the event "CE selection".

8.2.	The UE shall consider this random access response reception successful and transmit the msg3.

8.3.	The SS shall send a message addressed to the Temporary C-RNTI with a UE contention resolution identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

8.4.	The UE shall send ACK and consider the contention resolution successful and the random access procedure successfully completed.

9.	Test 6: Correct behaviour when contention resolution timer expires

9.1.	Repeat step 1-3.

9.2.	In Test 6, the UE shall send the preamble to the SS. The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS. If the UE transmits an NPRACH preamble using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0, then count a success for the event "CE selection". Otherwise count a fail for the event "CE selection".

9.3.	The UE shall consider this random access response reception successful and transmit the msg3.

9.4.	The SS shall send an ACK for msg3 but not send msg4 allowing the contention resolution timer to expire.

9.5.	The UE shall consider the contention resolution not successful then re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires. If the UE transmits an NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 0, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power is only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

9.6.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

9.7.	The UE shall consider this random access response reception successful and transmit the msg3.

10.	Void.

13.3.2.1.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.5B and 8.1.6 with following exceptions.

Table 13.3.2.1.4.3-1: RadioResourceConfigCommonSIB-NB-DEFAULT

| Derivation Path: 36.508 clause 8.1.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-Config-v1330 ::= SEQUENCE { |  |  |  |
| nprach-ParametersList-v1330 ::= SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-v1330 | 3 entries |  |  |
| NPRACH-Parameters-NB-v1330[1] ::= SEQUENCE { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-v1330[2] ::= SEQUENCE { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-v1330[3] ::= SEQUENCE { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.3.2.1.4.3-2: RACH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n3 |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB2 |  |  |
| preambleInitialReceivedTargetPower | dBm-112 |  |  |
| } |  |  |  |
| rach-InfoList-r13 (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF RACH-Info-NB-r13 { | 3 entries |  |  |
| RACH-Info-NB-r13[1] SEQUENCE { |  |  |  |
| ra-ResponseWindowSize-r13 | pp2 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| RACH-Info-NB-r13[2] SEQUENCE { |  |  |  |
| ra-ResponseWindowSize-r13 | pp2 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| RACH-Info-NB-r13[3] SEQUENCE { |  |  |  |
| ra-ResponseWindowSize-r13 | pp2 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| } |  |  |  |
| powerRampingParameters-v1450 | Not present |  |  |
| } |  |  |  |

Table 13.3.2.1.4.3-3: NPRACH-ConfigSIB-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-CP-Length-r13 | us66dot7 |  |  |
| rsrp-ThresholdsPrachInfoList-r13 ::= SEQUENCE (SIZE(1)) OF RSRP-Range { | 2 entries |  |  |
| RSRP-Range[1] | 25 | -116dBm | Normal |
| RSRP-Range[2] | 40 | -101dBm |  |
| } |  |  |  |
| RSRP-Range[1] | 25 | -116dBm | Extreme |
| RSRP-Range[2] | 36 | -105dBm |  |
| nprach-ParametersList-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r13 { | 3 entries |  |  |
| NPRACH-Parameters-NB-r13[1] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms40 |  |  |
| nprach-StartTime-r13 | ms8 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n2 |  |  |
| npdcch-NumRepetitions-RA-r13 | r4 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v1dot5 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[2] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms240 |  |  |
| nprach-StartTime-r13 | ms64 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n6 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n8 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v1dot5 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[3] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms1280 |  |  |
| nprach-StartTime-r13 | ms512 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n10 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n64 |  |  |
| npdcch-NumRepetitions-RA-r13 | r128 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v1dot5 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.3.2.1.4.3-4: NPDSCH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | -5 (dBm) |  |  |
| } |  |  |  |

13.3.2.1.5	Test requirement

The test parameters are given in tables 13.3.2.1.5-1, 13.3.2.1.5-2 and 13.3.2.1.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using [a configuration as described in Table 13.3.2.1.5-2]. Tables 13.3.2.1.5-4 and 13.3.2.1.5-5 define the absolute and relative power control requirements including test tolerances. Table 13.3.2.1.5-6 defines the uplink timing error limit including test tolerances.

Table 13.3.2.1.5-1: nCell specific test parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| NB-IOT operational mode |  | Standalone |  |
| BWchannel | kHz | 200 |  |
| NPDSCH parameters Note 2 |  | R.18 HD-FDD | As defined in TS36.133 A.3.1.5.3 |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD | As defined in TS36.133 A.3.1.6.3 |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB | 0 |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RA Note 1 | dB |  |  |
| NOCNG_RB Note 1 | dB |  |  |
| DRX |  | OFF |  |
|  | dBm/15 kHz | -98 |  |
|  | dB | 3.1 |  |
| Note 3 | dB | 3.1 |  |
| NRSRP Note 3 | dBm/15 kHz | -94.9 |  |
| Io Note 3 | dBm/180 KHz | -82.38 |  |
| Propagation Condition | - | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

Table 13.3.2.1.5-2: NTN specific test parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Parameter | Value | Comment |
| --- | --- | --- |
| Configuration 1 | SSC.1 | GSO Test Configuration |
| Configuration 2 | SSC.2 | NGSO Test Configuration |

Table 13.3.2.1.5-3: NPRACH-Configuration parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {40, 55} |  |  | Corresponding to {-116, -101} dBm as defined in TS36.133 Section 9.1.22.9 |
|  | {40, 51} |  |  | Extreme condition: Corresponding to {-116, -105} dBm as defined in 3GPP TS 36.331 Section 9.1.22.9 |
| nprach-CP-Length | us66dot7 |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  | As defined in clause 6.7.3 in TS 36.331. |
| Backoff Parameter Index | 1 |  |  | As defined in table 7.2-2 in TS 36.321 |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  |  | As defined in clause 6.2B.4 in TS 36.102 |
| powerRampingStep | dB2 |  |  |  |
| preambleInitialReceivedTargetPower | dBm-112 |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |
| nprach-Periodicity | ms40 | ms240 | ms1280 |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |
| numRepetitionsPerPreambleAttempt | n2 | n8 | n64 |  |
| npdcch-NumRepetitions-RA | r4 | r16 | r128 |  |
| npdcch-StartSF-CSS-RA | v1dot5 | v1dot5 | v1dot5 |  |
| npdcch-Offset-RA | zero | zero | Zero |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp8 | pp8 | pp8 |  |
| Note 1: See TS36.133 Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |

Test 1: Correct behaviour when receiving random access response reception

-	The power of the first preambles shall be -25 dBm to within the accuracy specified in Table 13.3.2.1.5-4.

-	The relative power for preamble ramping step shall be 2 dB to within the accuracy specified in Table 13.3.2.1.5-5.

-	The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Table 13.3.2.1.5-6.

Test 2: Correct behaviour when not receiving random access response reception-

-	The power of the first preambles shall be -25 dBm to within the accuracy specified in Table 13.3.2.1.5-4.

-	The relative power for preamble ramping step shall be 2 dB to within the accuracy specified in Table 13.3.2.1.5-5.

-	The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Table 13.3.2.1.5-6.

Test 3: Correct behaviour when receiving a NACK on msg3

-	The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of HARQ retransmission is reached.

Test 4: Correct behaviour when receiving an incorrect message over Temporary C-RNTI

-	The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires unless the received message includes a UE contention resolution identity MAC control element and the UE contention resolution identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

Test 5: Correct behaviour when receiving a correct message over Temporary C-RNTI

-	The UE shall send ACK if the contention resolution is successful.

Test 6: Correct behaviour when contention resolution timer expires

-	The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the back off time expires if the contention resolution timer expires.

Correct behaviour when UE selects NPRACH resources

-	The rate of correct coverage enhancement level 0 selection events "CE selection" observed during repeated tests shall be at least 90% with a confidence level of 95%.

Decide the test pass, if all the the power, relative power and timing measurements pass and the CE selection events pass, otherwise fail the UE.

Table 13.3.2.1.5-4: Absolute power tolerance for HD-FDD contention based random access test for UE category NB1

| Conditions | Tolerance |
| --- | --- |
| Normal | ±10.4 dB |
| Extreme | ±13.4 dB |

Table 13.3.2.1.5-5: Relative power tolerance for HD-FDD contention based random access test for UE category NB1

| Power step P [dB] | NPRACH [dB] |
| --- | --- |
| ΔP = 0 | 2.2 |
| ΔP = 2 | ±2.7 |
| ΔP = 4 | ±4.2 |
| ΔP = 6 | ±4.7 |
| NOTE: For extreme conditions an additional ± 2.0 dB relaxation is allowed. |  |

Table 13.3.2.1.5-6: Test requirements for Te Timing Error Limit for HD-FDD contention based random access test for UE category NB1

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 100*TS |
| NOTE: TS is the basic timing unit defined in 3GPP TS 36.211 |  |

#### 13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

##### 13.3.2.2.1	Test purpose

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Enhanced Coverage is according to the requirements when connected to a NTN NB-IoT cell, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach[5]. This test will verify the requirements in TS36.133 Clause 6.6A.2, Clause 6.6A.3 and Clause 7.20A.2 in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables 13.3.2.1.5-1, 13.3.2.1.5-2 and 13.3.2.1.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using AT command-based test approach.

##### 13.3.2.2.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both from release 17 and forwards.

##### 13.3.2.2.3	Minimum conformance requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window. The RA response window shall be started at the point in time indicated by clause 5.1.4 in TS 36.321[11].

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of re-transmissions defined by maxNumPreambleAttemptCE in the table 13.3.2.2.5-3 is reached.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

The UE shall send ACK if the Contention Resolution is successful.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

The UE shall select NPRACH resources and transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1. The rate of correct coverage enhancement level selection during repeated tests shall be at least 90%.

Note:	Correct coverage enhancement level selection is a prerequisite for testing the other NPRACH requirements.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.3.2.2.

##### 13.3.2.2.4	Test description

13.3.2.2.4.1	Initial conditions

Test Environment: Normal,TL/VL, TL/VH, TH/VL, TH/VH;  as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.3.2.2.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

13.3.2.2.4.2	Test procedure

The test scenario comprises of one NB-IoT carrier with 1 Ncell.

1.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 in Ncell 1.

2.	Set the parameters according to Tables 13.3.2.2.5-1 to 13.3.2.2.5-3 selecting values appropriate to the test condition of normal or extreme. Propagation conditions are set according to Annex B clause B.1.1.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when receiving random access response reception.

4.1.	In Test 1, the UE shall send the preamble to the SS. In response to the first 4 preamble transmission attempts (the preamble is transmitted 8 times in each attempt), the SS shall transmit a random access response not corresponding to the transmitted random access preamble.

4.2.	The UE shall consider the random access response reception not successful then re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received random access response contain random access preamble identifiers that do not match the transmitted random access preamble.

4.3.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after 5 preamble transmission attempts have been received by the SS.

4.4.	The UE shall consider this random access response reception successful and transmit the msg3.

4.5.	Measure the power and timing of the first preamble transmission attempt. The power of the first preamble transmission attempt shall be 23 dBm for power class 3, 20 dBm for power class 5 with the accuracy specified in clause 6.3B.4 of TS 36.102. Measure the relative power and timing applied to additional preamble transmission attempts. The relative power of the last 4 preamble transmission attempts shall not exceed the values specified in clause 6.3B.4 of TS 36.102. The transmit timing of all NPRACH transmissions shall be within the accuracy specified in TS36.133 clause 7.20A.2. If the UE transmits or re-transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power, relative power and timing are only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

5.	Test 2: Correct behaviour when not receiving random access response reception

5.1.	Repeat step 1-3.

5.2.	In Test 2, the UE shall send the preamble to the SS. The SS shall not respond to the first 4 preamble transmission attempts (the preamble is transmitted 8 times in each attempt).

5.3.	The UE shall consider the random access response reception not successful then re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no random access response is received within the RA Response window. The RA response window shall be started at the point in time indicated by clause 5.1.4 in TS 36.321.

5.4.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after 5 preamble transmission attemps have been received by the SS.

5.5.	The UE shall consider this random access response reception successful and transmit the msg3.

5.6.	Measure the power and timing of the first preamble transmission attempt. The power of the first preamble transmission attempt shall be 23 dBm for power class 3, 20 dBm for power class 5 with the accuracy specified in clause 6.3B.4 of TS 36.102. Measure the relative power and timing applied to additional preamble transmission attempts. The relative power of the last 4 preamble transmission attempts shall not exceed the values specified in clause 6.3B.4 of TS 36.102. The transmit timing of all NPRACH transmissions shall be within the accuracy specified in TS36.133 clause 7.20A.2. If the UE transmits or re-transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power, relative power and timing are only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

6.	Test 3:  Not applicable.

7.	Test 4: Correct behaviour when receiving an incorrect message over Temporary C-RNTI

7.1.	Repeat step 1-3.

7.2.	In Test 4, the UE shall send the preamble to the SS. If the UE transmits an NPRACH preamble using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

7.3.	The UE shall consider this random access response reception successful and transmit the msg3.

7.4.	The SS shall send a message addressed to the Temporary C-RNTI with a UE contention resolution identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

7.5.	The UE shall consider the contention resolution not successful then re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires. If the UE transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power is only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

8.	Test 5: Correct behaviour when receiving a correct message over Temporary C-RNTI

8.1.	In Test 5, the SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS. If the UE transmits an NPRACH preamble using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then count a success for the event "CE selection". Otherwise count a fail for the event "CE selection".

8.2.	The UE shall consider this random access response reception successful and transmit the msg3.

8.3.	The SS shall send a message addressed to the Temporary C-RNTI with a UE contention resolution identity included in the MAC control element matching the CCCH SDU transmitted in the msg3 uplink message.

8.4.	The UE shall send ACK and consider the contention resolution successful and the random access procedure successfully completed.

9.	Test 6: Correct behaviour when contention resolution timer expires

9.1.	Repeat step 1-3.

9.2.	In Test 6, the UE shall send the preamble to the SS. The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS. If the UE transmits an NPRACH preamble using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then count a success for the event "CE selection". Otherwise count a fail for the event "CE selection".

9.3.	The UE shall consider this random access response reception successful and transmit the msg3.

9.4.	The SS shall send an ACK for msg3 but not send msg4 allowing the contention resolution timer to expire.

9.5.	The UE shall consider the contention resolution not successful then re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires. If the UE transmits an NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1, then for each preamble count a success for the event "CE selection". Otherwise count a fail for the event "CE selection". The power is only measured if the coverage enhancement level selection was correct, and any results relating to a wrong selection are discarded.

9.6.	The SS shall transmit a random access response containing a random access preamble identifier corresponding to the transmitted random access after the first preamble has been received by the SS.

9.7.	The UE shall consider this random access response reception successful and transmit the msg3.

10.	Void.

13.3.2.2.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.5B and 8.1.6 with following exceptions.

Table 13.3.2.2.4.3-1: RadioResourceConfigCommonSIB-NB-DEFAULT

| Derivation Path: 36.508 clause 8.1.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-Config-v1330 ::= SEQUENCE { |  |  |  |
| nprach-ParametersList-v1330 ::= SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-v1330 | 3 entries |  |  |
| NPRACH-Parameters-NB-v1330[1] ::= SEQUENCE { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-v1330[2] ::= SEQUENCE { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-v1330[3] ::= SEQUENCE { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.3.2.2.4.3-2: RACH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n6 |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB2 |  |  |
| preambleInitialReceivedTargetPower | dBm-120 |  |  |
| } |  |  |  |
| rach-InfoList-r13 (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF RACH-Info-NB-r13 { | 3 entries |  |  |
| RACH-Info-NB-r13[1] SEQUENCE { |  |  |  |
| ra-ResponseWindowSize-r13 | pp2 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| RACH-Info-NB-r13[2] SEQUENCE { |  |  |  |
| ra-ResponseWindowSize-r13 | pp2 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| RACH-Info-NB-r13[3] SEQUENCE { |  |  |  |
| ra-ResponseWindowSize-r13 | pp2 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| } |  |  |  |
| powerRampingParameters-v1450 | Not present |  |  |
| } |  |  |  |

Table 13.3.2.2.4.3-3: NPRACH-ConfigSIB-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-CP-Length-r13 | Us266dot7 |  |  |
| rsrp-ThresholdsPrachInfoList-r13 ::= SEQUENCE (SIZE(1)) OF RSRP-Range { | 2 entries |  |  |
| RSRP-Range[1] | 35 | -121dBm | Normal |
| RSRP-Range[2] | 56 | -100dBm |  |
| RSRP-Range[1] | 31 | -125dBm | Extreme |
| RSRP-Range[2] | 60 | -96dBm |  |
| } |  |  |  |
| nprach-ParametersList-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r13 { | 3 entries |  |  |
| NPRACH-Parameters-NB-r13[1] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms40 |  |  |
| nprach-StartTime-r13 | ms8 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n2 |  |  |
| npdcch-NumRepetitions-RA-r13 | r4 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v1dot5 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[2] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms240 |  |  |
| nprach-StartTime-r13 | ms64 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n6 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n8 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v1dot5 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[3] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms1280 |  |  |
| nprach-StartTime-r13 | ms512 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n10 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n64 |  |  |
| npdcch-NumRepetitions-RA-r13 | r128 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v1dot5 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.3.2.2.4.3-4: NPDSCH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | -5 (dBm) |  |  |
| } |  |  |  |

##### 13.3.2.2.5	Test requirement

The test parameters are given in tables 13.3.2.2.5-1, 13.3.2.2.5-2 and 13.3.2.2.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using [a configuration as described in Table 13.3.2.2.5-2]. Tables 13.3.2.2.5-4 and 13.3.2.2.5-5 define the absolute and relative power control requirements including test tolerances. Table 13.3.2.2.5-6 defines the uplink timing error limit including test tolerances.

Table 13.3.2.2.5-1: nCell specific test parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Enhanced Coverage

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| NB-IOT operational mode |  | Standalone |  |
| BWchannel | kHz | 200 |  |
| NPDSCH parameters Note 2 |  | R.18 HD-FDD | As defined in TS36.133 A.3.1.5.3 |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD | As defined in TS36.133 A.3.1.6.3 |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB |  |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RA Note 1 | dB |  |  |
| NOCNG_RB Note 1 | dB |  |  |
| DRX |  | OFF |  |
|  | dBm/15 kHz | -98 |  |
|  | dB | -12.5 |  |
| Note 3 | dB | -12.5 |  |
| NRSRP Note 3 | dBm/15 kHz | -110.5 |  |
| Io Note 3 | dBm/180KHz | -86.97 |  |
| Propagation Condition | - | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

Table 13.3.2.2.5-2: NTN specific test parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Enhanced Coverage

| Parameter | Value | Comment |
| --- | --- | --- |
| Configuration 1 | SSC.1 | GSO Test Configuration |
| Configuration 2 | SSC.2 | NGSO Test Configuration |

Table 13.3.2.2.5-3: NPRACH-Configuration parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Enhanced Coverage

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {34, 57} |  |  |  | Corresponding to {-122, -99} dBm as defined in TS36.133 Section 9.1.22.9 |
|  | {31, 60} |  |  |  | Extreme condition: Corresponding to {-125, -96} dBm as defined in 3GPP TS 36.133 Section 9.1.22.9 |
| nprach-CP-Length | us266dot7 |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  |  |  | As defined in clause 6.2B.4 in TS 36.102 |
| powerRampingStep | dB2 |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-120 |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | ms40 | ms240 | ms1280 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n2 | n8 | n64 |  |  |
| npdcch-NumRepetitions-RA | r4 | r16 | r128 |  |  |
| npdcch-StartSF-CSS-RA | v1dot5 | v1dot5 | v1dot5 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp8 | pp8 | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |

Test 1: Correct behaviour when receiving random access response reception

-	The power of the first preamble shall be 23 dBm for power class 3, 20 dBm for power class 5 within the accuracy specified in Table 13.3.2.2.5-4.

-	The relative power for preamble ramping step shall be 2 dB to within the accuracy specified in Table 13.3.2.2.5-5.

-	The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Table 13.3.2.2.5-6.

Test 2: Correct behaviour when not receiving random access response reception

-	The power of the first preamble shall be 23 dBm for power class 3, 20 dBm for power class 5 within the accuracy specified in Table 13.3.2.2.5-4.

-	The relative power for preamble ramping step shall be 2 dB to within the accuracy specified in Table 13.3.2.2.5-5.

-	The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Table 13.3.2.2.5-6.

Test 3: Correct behaviour when receiving a NACK on msg3

-	The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of HARQ retransmission is reached.

Test 4: Correct behaviour when receiving an incorrect message over Temporary C-RNTI

-	The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires unless the received message includes a UE contention resolution identity MAC control element and the UE contention resolution identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

Test 5: Correct behaviour when receiving a correct message over Temporary C-RNTI

-	The UE shall send ACK if the contention resolution is successful.

Test 6: Correct behaviour when contention resolution timer expires

-	The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the back off time expires if the contention resolution timer expires.

Correct behaviour when UE selects NPRACH resources

-	The rate of correct coverage enhancement level 1 selection events "CE selection" observed during repeated tests shall be at least 90% with a confidence level of 95%.

Decide the test pass, if all the the power, relative power and timing measurements pass and the CE selection events pass, otherwise fail the UE.

Table 13.3.2.2.5-4: Absolute power tolerance for HD-FDD contention based random access test for UE category NB1

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 14.3 dB |
| Extreme | ±17.3 dB |

Table 13.3.2.2.5-5: Relative power tolerance for HD-FDD contention based random access test for UE category NB1

| Power step P [dB] | NPRACH [dB] |
| --- | --- |
| ΔP = 0 | ±2.2 |
| ΔP = 2 | ±2.7 |
| ΔP = 4 | ±4.2 |
| ΔP = 6 | ±4.7 |
| NOTE: For extreme conditions an additional ± 2.0 dB relaxation is allowed. |  |

Table 13.3.2.2.5-6: Test requirements for Te Timing Error Limit for HD-FDD contention based random access test for UE category NB1

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 100*TS |
| NOTE: TS is the basic timing unit defined in 3GPP TS 36.211 |  |

#### 13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

##### 13.3.2.3.1	Test purpose

The purpose of this test is to verify whether the behavior of the random access procedure of a category NB1 UE in Enhanced Coverage is according to the requirements, whether the NPRACH power settings and timing are within specified limits, and whether the UE determines properly the enhanced coverage level based on the NRSRP measurement and the configured criterion in NRSRP-ThresholdsPrach [5]. This test will verify the requirements in TS36.133 Clause 6.6A.2, Clause 6.6A.3 and Clause 7.20A2 in an AWGN model.

For this test a single NB-IoT cell is used. The test parameters are given in tables 13.3.2.3.5-1, 13.3.2.3.5-2 and 13.3.2.3.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using AT command-based test approach.

##### 13.3.2.3.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD category NB1 UEs supporting GSO or NGSO or both  from release 17 and forwards.

Applicability requires support for sending NPRACH on non-anchor carrier.

##### 13.3.2.3.3	Minimum conformance requirements

Contention based random access is triggered by not explicitly assigning a random access preamble via dedicated signalling in the downlink.

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window. The RA response window shall be started at the point in time indicated by clause 5.1.4 in TS 36.321[11].

The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of re-transmissions defined by maxNumPreambleAttemptCE in the table 13.3.2.3.5-3 is reached.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

The UE shall send ACK if the Contention Resolution is successful.

The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

The UE shall select NPRACH resources and transmits or re- transmits NPRACH preambles using the NPRACH resources and NPRACH configuration corresponding to the coverage enhancement level 1. The rate of correct coverage enhancement level selection during repeated tests shall be at least 90%.

Note:	Correct coverage enhancement level selection is a prerequisite for testing the other NPRACH requirements.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause A.13.3.2.3.

##### 13.3.2.3.4	Test description

13.3.2.3.4.1	Initial conditions

Test Environment: Normal, TL/VL, TL/VH, TH/VL, TH/VH as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: Ncell bandwidth is as specified in Table 13.3.2.3.5-1.

1.	Connect the SS and AWGN noise source to the UE antenna connectors as shown in 3GPP TS 36.508 [7] Annex A Figure A.18 using only UE main Tx/Rx antenna.

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one NB-IoT cell specified in the test. Ncell 1 is the cell used for registration with the power level set according to Annex C.0 and C.1 for this test. In Ncell 1 one anchor carrier and one non-anchor carrier are configured.

4.	UE location according to TS 36.508 [12] clause 8.4.6.1 is provided to the UE through any preconfigured means.

5.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] clause 8.4.6.2.1 depending on the type of satellite under test. Test system shall send same SIB31-NB information during the duration of the test as defined in TS 36.508 [12] clause 8.4.6.3.1.

6.	Deactivate UE prediction of satellite trajectory through any preconfigured means.

13.3.2.3.4.2	Test procedure

The test scenario comprises of 2 NB-IoT carriers with 1 Ncell.

The test procedures are the same as test procedures defined in 13.3.2.2.4.2, with step 4 and forward performed on the non-anchor carrier.

13.3.2.3.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.5B and 8.1.6 with following exceptions.

Table 13.3.2.3.4.3-1: SystemInformationBlockType22-NB

| Derivation Path: 36.508 Table 8.1.4.3.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| ul-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 SEQUENCE { |  |  |  |
| UL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-ParametersList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r14 SEQUENCE { |  |  |  |
| NPRACH-Parameters-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms40 |  |  |
| nprach-StartTime-r14 | ms8 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | zero |  |  |
| npdcch-NumRepetitions-RA-r14 | r4 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v1dot5 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r14[2] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms240 |  |  |
| nprach-StartTime-r14 | ms64 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | zero |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v1dot5 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n20 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r14[3] SEQUENCE { |  |  |  |
| nprach-Parameters-r14 SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms1280 |  |  |
| nprach-StartTime-r14 | ms512 |  |  |
| nprach-SubcarrierOffset-r14 | n0 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | zero |  |  |
| npdcch-NumRepetitions-RA-r14 | r128 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v1dot5 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n40 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nprach-ProbabilityAnchorList-r14 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ProbabilityAnchor-NB-r14 SEQUENCE { |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[1] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[2] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| NPRACH-ProbabilityAnchor-NB-r14[3] SEQUENCE { |  |  |  |
| nprach-ProbabilityAnchor-r14 | zero |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 13.3.2.3.4.3-2: RadioResourceConfigCommonSIB-NB-DEFAULT

| Derivation Path: 36.508 clause 8.1.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-Config-v1330 ::= SEQUENCE { |  |  |  |
| nprach-ParametersList-v1330 ::= SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-v1330 | 3 entries |  |  |
| NPRACH-Parameters-NB-v1330[1] ::= SEQUENCE { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-v1330[2] ::= SEQUENCE { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-v1330[3] ::= SEQUENCE { |  |  |  |
| nprach-NumCBRA-StartSubcarriers-r13 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.3.2.2.4.3-3: RACH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n6 |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB2 |  |  |
| preambleInitialReceivedTargetPower | dBm-120 |  |  |
| } |  |  |  |
| rach-InfoList-r13 (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF RACH-Info-NB-r13 { | 3 entries |  |  |
| RACH-Info-NB-r13[1] SEQUENCE { |  |  |  |
| ra-ResponseWindowSize-r13 | pp2 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| RACH-Info-NB-r13[2] SEQUENCE { |  |  |  |
| ra-ResponseWindowSize-r13 | pp2 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| RACH-Info-NB-r13[3] SEQUENCE { |  |  |  |
| ra-ResponseWindowSize-r13 | pp2 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| } |  |  |  |
| powerRampingParameters-v1450 | Not present |  |  |
| } |  |  |  |

Table 13.3.2.2.4.3-4: NPRACH-ConfigSIB-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-CP-Length-r13 | Us266dot7 |  |  |
| rsrp-ThresholdsPrachInfoList-r13 ::= SEQUENCE (SIZE(1)) OF RSRP-Range { | 2 entries |  |  |
| RSRP-Range[1] | 35 | -121dBm | Normal |
| RSRP-Range[2] | 56 | -100dBm |  |
| RSRP-Range[1] | 31 | -125dBm | Extreme |
| RSRP-Range[2] | 60 | -96dBm |  |
| } |  |  |  |
| nprach-ParametersList-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r13 { | 3 entries |  |  |
| NPRACH-Parameters-NB-r13[1] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms40 |  |  |
| nprach-StartTime-r13 | ms8 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n2 |  |  |
| npdcch-NumRepetitions-RA-r13 | r4 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v1dot5 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[2] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms240 |  |  |
| nprach-StartTime-r13 | ms64 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n6 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n8 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v1dot5 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| NPRACH-Parameters-NB-r13[3] ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r13 | ms1280 |  |  |
| nprach-StartTime-r13 | ms512 |  |  |
| nprach-SubcarrierOffset-r13 | n0 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | zero |  |  |
| maxNumPreambleAttemptCE-r13 | n10 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n64 |  |  |
| npdcch-NumRepetitions-RA-r13 | r128 |  |  |
| npdcch-StartSF-CSS-RA-r13 | v1dot5 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.3.2.2.4.3-5: NPDSCH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.508 Table 8.1.6.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | -5 (dBm) |  |  |
| } |  |  |  |

##### 13.3.2.3.5	Test requirement

The test parameters are given in tables 13.3.2.3.5-1, 13.3.2.3.5-2 and 13.3.2.3.5-3. The UE shall perform timing pre-compensation before the initial NPRACH transmission using [aconfiguration as described in Table 13.3.2.3.5-2]. Tables 13.3.2.3.5-4 and 13.3.2.3.5-5 define the absolute and relative power control requirements including test tolerances. Table 13.3.2.3.5-6 defines the uplink timing error limit including test tolerances.

Table 13.3.2.3.5-1: nCell specific test parameters for HD-FDD contention based random access on non-achor carrier test for UE category NB1 Standalone mode in Enhanced Coverage

| Parameter | Unit | Value | Comments |
| --- | --- | --- | --- |
| NB-IOT operational mode |  | Standalone |  |
| BWchannel | kHz | 200 |  |
| NPDSCH parameters Note 2 |  | R.18 HD-FDD | As defined in TS36.133 A.3.1.5.3 |
| NPDCCH parameters Note 2 |  | R.30 HD-FDD | As defined in TS36.133 A.3.1.6.3 |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB |  |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RA Note 1 | dB |  |  |
| NOCNG_RB Note 1 | dB |  |  |
| DRX |  | OFF |  |
|  | dBm/15 kHz | -98 |  |
|  | dB | -12.5 |  |
| Note 3 | dB | -12.5 |  |
| NRSRP Note 3 | dBm/15 kHz | -110.5 |  |
| Io Note 3 | dBm/180 KHz | -86.97 |  |
| Propagation Condition | - | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The NPDSCH and NPDCCH reference measurement channels are used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: Es/Iot, NRSRP and Io level has been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |

Table 13.3.2.3.5-2: NTN specific test parameters for HD-FDD contention based random access test for UE category NB1 Standalone mode in Normal Coverage

| Parameter | Value | Comment |
| --- | --- | --- |
| Configuration 1 | SSC.1 | GSO Test Configuration |
| Configuration 2 | SSC.2 | NGSO Test Configuration |

Table 13.3.2.3.5-3: NPRACH-Configuration parameters for HD-FDD contention based random access on non-anchor carrier test for UE category NB1 Standalone mode in Enhanced Coverage

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {34, 57} |  |  |  | Corresponding to {-122, -99} dBm as defined in TS36.133 Section 9.1.22.9 |
|  | {31, 60} |  |  |  | Extreme condition: Corresponding to {-125, -96} dBm as defined in 3GPP TS 36.133 Section 9.1.22.9 |
| nprach-CP-Length | us266dot7 |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5 |  |  |  | As defined in clause 6.2B.4 in TS 36.102 |
| powerRampingStep | dB2 |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-120 |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-ProbabilityAnchor | zero | zero | zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| nprach-Periodicity | ms40 | ms240 | ms1280 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n2 | n8 | n64 |  |  |
| npdcch-NumRepetitions-RA | r4 | r16 | r128 |  |  |
| npdcch-StartSF-CSS-RA | v1dot5 | v1dot5 | v1dot5 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp8 | pp8 | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |

Test 1: Correct behaviour when receiving random access response reception

-	The power of the first preamble shall be 23 dBm for power class 3, 20 dBm for power class 5 within the accuracy specified in Table 13.3.2.3.5-4.

-	The relative power for preamble ramping step shall be 2 dB to within the accuracy specified in Table 13.3.2.3.5-5.

-	The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Table 13.3.2.3.5-6.

Test 2: Correct behaviour when not receiving random access response reception

-	The power of the first preamble shall be 23 dBm for power class 3, 20 dBm for power class 5 within the accuracy specified in Table 13.3.2.3.5-4.

-	The relative power for preamble ramping step shall be 2 dB to within the accuracy specified in Table 13.3.2.3.5-5.

-	The transmit timing of all NPRACH transmissions shall be within the accuracy specified in Table 13.3.2.3.5-6.

Test 3: Correct behaviour when receiving a NACK on msg3

-	The UE shall re-transmit the msg3 upon the reception of a NACK on msg3 until the maximum number of HARQ retransmission is reached.

Test 4: Correct behaviour when receiving an incorrect message over Temporary C-RNTI

-	The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires unless the received message includes a UE contention resolution identity MAC control element and the UE contention resolution identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

Test 5: Correct behaviour when receiving a correct message over Temporary C-RNTI

-	The UE shall send ACK if the contention resolution is successful.

Test 6: Correct behaviour when contention resolution timer expires

-	The UE shall re-select a preamble and transmit with the calculated NPRACH transmission power when the back off time expires if the contention resolution timer expires.

Correct behaviour when UE selects NPRACH resources

-	The rate of correct coverage enhancement level 1 selection events "CE selection" observed during repeated tests shall be at least 90% with a confidence level of 95%.

Decide the test pass, if all the the power, relative power and timing measurements pass and the CE selection events pass, otherwise fail the UE.

Table 13.3.2.3.5-4: Absolute power tolerance for HD-FDD contention based random access test for UE category NB1

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 14.3 dB |
| Extreme | ±17.3 dB |

Table 13.3.2.3.5-5: Relative power tolerance for HD-FDD contention based random access test for UE category NB1

| Power step P [dB] | NPRACH [dB] |
| --- | --- |
| ΔP = 0 | ±2.7 |
| ΔP = 2 | ±2.7 |
| ΔP = 4 | ±4.2 |
| ΔP = 6 | ±4.7 |
| NOTE: For extreme conditions an additional ± 2.0 dB relaxation is allowed. |  |

Table 13.3.2.3.5-6: Test requirements for Te Timing Error Limit for HD-FDD contention based random access test for UE category NB1

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 100*TS + TT |
| NOTE: TS is the basic timing unit defined in 3GPP TS 36.211 |  |

<< End of changes >>