3GPP TSG-RAN5 Meeting #102	R5-240144

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0009 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update content of Statistical testing of Performance Requirements with probability of misdetection in Annex G.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek (Hefei) Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ‘Statistical testing of Performance Requirements with probability of misdetection’ part is FFS in 36.521-4. This part is needed when people trying to figure out the minimum test time of NPDCCH TC 8.3.1.2.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add sub-clause G.4 align with 36.521-1 G.4 since the NPDCCH test case in 36.521-4 is reused from 36.521-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ‘Statistical testing of Performance Requirements with probability of misdetection’ part remains not complete in 36.521-4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

G.4	Statistical testing of Performance Requirements with probability of misdetection

G.4.1	General

The test of receiver performance characteristics is two fold.

1.	A signal or a combination of signals is offered to the RX port(s) of the receiver.

2.	The ability of the receiver to demodulate /decode this signal is verified by analyzing the reaction of the UE to this signal.

In (2) is the statistical aspect of the test and is treated here.

The minimum requirement for those receiver performance tests are 1% or 0.1% misdetection probability

All receiver performance tests are performed in fading conditions. In addition to the statistical considerations, this requires the definition of a minimum test time.

NOTE:	All demodulation performance tests (state from version 9.5.0) require a minimum test time, which exceeds the maximum test time in tables G.4.4. Under this circumstances only the test limit at the end of tables G.4.4.-1 resp. G.4.4.-2 is applicable.

G.4.2	Mapping the UE reaction to error ratio

The UE can not indicate the detection or misdetection of the physical channel under test directly. Indirect methods are described in the procedure of the applicable test.

G.4.3	Design of the test

G.2.3 applies, exception:

Limit ER = 0.01

G.4.4	Numerical definition of the pass fail limits

Table G.4.4-1: pass fail limits for ER = 0.01

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 344 | NA | 40 | 3929 | 2553 | 80 | 7033 | 5874 | 120 | 10036 | 9354 |
| 1 | 485 | NA | 41 | 4009 | 2632 | 81 | 7109 | 5960 | 121 | 10110 | 9442 |
| 2 | 607 | 10 | 42 | 4089 | 2712 | 82 | 7185 | 6046 | 122 | 10184 | 9530 |
| 3 | 719 | 33 | 43 | 4168 | 2792 | 83 | 7261 | 6131 | 123 | 10259 | 9619 |
| 4 | 826 | 66 | 44 | 4247 | 2873 | 84 | 7336 | 6217 | 124 | 10333 | 9707 |
| 5 | 929 | 107 | 45 | 4327 | 2953 | 85 | 7412 | 6303 | 125 | 10407 | 9796 |
| 6 | 1029 | 152 | 46 | 4406 | 3034 | 86 | 7488 | 6389 | 126 | 10481 | 9884 |
| 7 | 1127 | 202 | 47 | 4484 | 3115 | 87 | 7564 | 6475 | 127 | 10555 | 9972 |
| 8 | 1223 | 255 | 48 | 4563 | 3196 | 88 | 7639 | 6561 | 128 | 10629 | 10061 |
| 9 | 1317 | 311 | 49 | 4642 | 3278 | 89 | 7715 | 6648 | 129 | 10703 | 10150 |
| 10 | 1409 | 370 | 50 | 4720 | 3359 | 90 | 7790 | 6734 | 130 | 10777 | 10238 |
| 11 | 1501 | 430 | 51 | 4799 | 3441 | 91 | 7866 | 6820 | 131 | 10851 | 10327 |
| 12 | 1592 | 492 | 52 | 4877 | 3523 | 92 | 7941 | 6907 | 132 | 10925 | 10416 |
| 13 | 1681 | 555 | 53 | 4955 | 3605 | 93 | 8017 | 6993 | 133 | 10999 | 10504 |
| 14 | 1770 | 620 | 54 | 5033 | 3688 | 94 | 8092 | 7080 | 134 | 11073 | 10593 |
| 15 | 1858 | 686 | 55 | 5111 | 3770 | 95 | 8167 | 7167 | 135 | 11147 | 10682 |
| 16 | 1946 | 754 | 56 | 5189 | 3853 | 96 | 8242 | 7253 | 136 | 11221 | 10771 |
| 17 | 2032 | 822 | 57 | 5267 | 3935 | 97 | 8317 | 7340 | 137 | 11295 | 10860 |
| 18 | 2119 | 891 | 58 | 5344 | 4018 | 98 | 8393 | 7427 | 138 | 11369 | 10949 |
| 19 | 2204 | 961 | 59 | 5422 | 4101 | 99 | 8468 | 7514 | 139 | 11442 | 11038 |
| 20 | 2290 | 1032 | 60 | 5499 | 4185 | 100 | 8543 | 7601 | 140 | 11516 | 11127 |
| 21 | 2374 | 1103 | 61 | 5577 | 4268 | 101 | 8618 | 7688 | 141 | 11590 | 11216 |
| 22 | 2459 | 1175 | 62 | 5654 | 4352 | 102 | 8693 | 7775 | 142 | 11664 | 11305 |
| 23 | 2543 | 1248 | 63 | 5731 | 4435 | 103 | 8768 | 7863 | 143 | 11737 | 11394 |
| 24 | 2627 | 1321 | 64 | 5809 | 4519 | 104 | 8843 | 7950 | 144 | 11811 | 11483 |
| 25 | 2710 | 1395 | 65 | 5886 | 4603 | 105 | 8917 | 8037 | 145 | 11885 | 11573 |
| 26 | 2793 | 1470 | 66 | 5963 | 4687 | 106 | 8992 | 8125 | 146 | 11958 | 11662 |
| 27 | 2876 | 1544 | 67 | 6039 | 4771 | 107 | 9067 | 8212 | 147 | 12032 | 11751 |
| 28 | 2958 | 1620 | 68 | 6116 | 4855 | 108 | 9142 | 8300 | 148 | 12105 | 11840 |
| 29 | 3040 | 1696 | 69 | 6193 | 4940 | 109 | 9216 | 8387 | 149 | 12179 | 11930 |
| 30 | 3122 | 1772 | 70 | 6270 | 5024 | 110 | 9291 | 8475 | 150 | 12252 | 12019 |
| 31 | 3204 | 1848 | 71 | 6346 | 5109 | 111 | 9366 | 8562 | 151 | 12326 | 12109 |
| 32 | 3285 | 1925 | 72 | 6423 | 5193 | 112 | 9440 | 8650 | 152 | 12399 | 12198 |
| 33 | 3366 | 2003 | 73 | 6499 | 5278 | 113 | 9515 | 8738 | 153 | 12473 | 12288 |
| 34 | 3447 | 2080 | 74 | 6576 | 5363 | 114 | 9589 | 8826 | 154 | 12546 | 12377 |
| 35 | 3528 | 2158 | 75 | 6652 | 5448 | 115 | 9664 | 8914 | 155 | 12620 | 12467 |
| 36 | 3609 | 2237 | 76 | 6728 | 5533 | 116 | 9738 | 9002 | 156 | 12693 | 12556 |
| 37 | 3689 | 2315 | 77 | 6805 | 5618 | 117 | 9813 | 9090 | 157 | 12767 | 12646 |
| 38 | 3769 | 2394 | 78 | 6881 | 5704 | 118 | 9887 | 9178 | 158 | 12840 | 12736 |
| 39 | 3850 | 2473 | 79 | 6957 | 5789 | 119 | 9962 | 9266 | 159 | 12913 | 12826 |
|  |  |  |  |  |  |  |  |  | 160 | NA | 12915 |
|  |  |  |  |  |  |  |  |  | Test limit = 1.2352E-2 |  |  |

NOTE 1:	The first column is the number of errors (ne = number of misdetections)

NOTE 2:	The second column is the number of samples for the pass limit (nsp, ns=Number of Samples= number misdetections + number of detections)

NOTE 3:	The third column is the number of samples for the fail limit (nsf)

NOTE 4:	The test limit at the end of the table is applicable, when the minimum test time in clause 3.5 governs the test. Pass the test for ER ≤ Test limit, otherwise fail.

G.4.5	Pass fail decision rules

G.2.5 applies

NOTE:	For ER=0.01 an ideal DUT passes after 344 samples. The maximum test time is 12913 samples.

G.4.6	Minimum Test time

Table G.4.6-1: Minimum Test time for NPDCCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNS (Calculation) | MNSF (Min No Sub Frames, mandatory) |
| --- | --- | --- | --- | --- |
|  |  |  | HD-FDD | HD-FDD |
| 8.3.1.2.1: Test 1 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 36166 | 15430827 | 15431000 |
| 8.3.1.2.1: Test 2 | R.NB.3 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 36166 | 55550976 | 55551000 |

<<< END OF CHANGES 1>>>