3GPP TSG-RAN Meeting #103	RP-240403

Maastrich, Niederlands 18th March – 22nd March 2024

Agenda item:	    9.7.2.7

Source:	Vodafone, Apple

Title:	Access Control for 2 RX XR UEs in Legacy Networks

Document for:	Discussion and Decision

# 1	Introduction

During the RAN Plenary RAN#102 the following agreements were done:

Task RAN2 and RAN3 to develop signalling support for ‘2Rx non-REDCAP XR devices’, send corresponding Release-18 draft CR(s) to RAN#103:

- A new dedicated UE capability indication per band and setting of corresponding existing UE capability(ies). [RAN2]
Note: no need is foreseen to add an indication to Msg1 or Msg3.

Indication in SIB, to be used to re-direct to another frequency layer, or to bar the device altogether. [RAN2]

Taking RAN Plenary agreements above into account, RAN WG2 discussed based on R2-2400582 and agreed to technically endorse 2 sets of CRs.

The difference of these two sets of CRs is that in one of them the 2Rx XR UEs are barred from initial access “by default” ( i.e. barred from access in the cells operating on bands where 4Rx is mandatory in networks where the new 2RX XR UE related barring IEs are not broadcasted (e.g. legacy networks)) and in other set of CRs initial access would be allowed in legacy cells.

# 2	Justification and Proposals

Fast rollout of new services is important to our customers.

Whether or not to tolerate the capacity and coverage impacts of 2Rx XR UEs should be a choice for the operator.

Most operators have multiple RAN vendors. The “default barred” approach requires all of the operator’s RAN vendors to upgrade their equipment before the operator can offer customers service. This means that our customers are impacted by the RAN vendor that delivers slowest!

Competition in the RAN vendor space is vital and hence “default barred” is not an acceptable approach.

Additional observations:

- Default barred does not protect the legacy NWs, because if the 2RX XR UE would be barred in 4Rx cell, it could access through 2Rx cell and legacy RAN would handover such UE to the 4Rx cell.

- A non-upgraded RAN and non-upgraded Core Network can reject 2Rx XR UEs using NAS signalling that is driven by “prohibited list” response to the IMEI (Permanent Equipment Identifier/PEI) check with the Equipment ID Register.

b)There are no technical reasons why a legacy NW could not support initial access of 2Rx non-REDCAP XR UEs once such UEs are in coverage of the corresponding cell. This is different e.g. to RedCAPs devices where a legacy 100 MHz bandwidth cell would not be able to serve such devices due to the limited bandwidth of RedCAP devices.

c) ‘barred by default’ will lead to additional complexity with regard to emergency call (and emergency SMS) handling as barring also disables establishment of emergency calls

d)  For V-PLMNs that are not interested in XR and hence not buying Rel 18 features: ‘barred by default ‘would lead to the situations, roaming devices would not be able to be supported at all

It is proposed to agree on the following proposals:

- Proposal 1: If 2RX XR UE related barring information is not broadcasted in SIB1, 2RX XR UE shall treat the corresponding cell as unbarred.

- Proposal 2: It is proposed to agree on R2-2401986  CR – 0382, R2-2401987  CR – 4572, R2-2401988  CR – 1052,R2-2401989  CR - 0813