3GPP TSG-RAN WG4 Meeting # 110															R4-2402599

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.113 | CR | 0094 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE-RF) CR to TS 36.113: clarification on the limit value for the step frequency case, Rel-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-RF |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In this CR we provide clarification for the selection of the limit values for the step frequency. The proposed approach is reused from CISPR 32, annex I. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification on the limit value selection for the step frequency. - Other editorials corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on the limit value would remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3, 8.3.3, 8.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 8.2.2	Radiated emission, Ancillary equipment

This test is applicable to ancillary equipment. This test shall be performed on a representative configuration of the ancillary equipment.

#### 8.2.2.1	Definition

This test assesses the ability of ancillary equipment to limit unwanted emission from the enclosure port.

#### 8.2.2.2	Test method

The test method shall be in accordance with CISPR 32 25.

#### 8.2.2.3	Limits

The ancillary equipment shall meet the limits according to CISPR 32 25 table A.4 and table A.5.

Where there is a step in the relevant limit, the lower value should be applied at the transition frequency, as per CISPR 32 [25].

Table 8.2.2.3-1: Void

| - |  |
| --- | --- |
| - |  |

Table 8.2.2.3-2: 
Void

Alternatively, for ancillary equipment intended to be used in telecommunication centres only, the class A limits given in CISPR 32 [25] table A.2 and table A.3 shall be used.

------------------------------ Next modified section -------------------------

### 8.3.3	Limits

The equipment shall meet the limits according to CISPR 32 [11] table A.9, which are defined for average detector receiver and for quasi-peak detector receiver--. If the average limit is met when using a quasi-peak detector, the equipment shall be deemed to meet both limits and measurement with the average detector receiver is not necessary.

Where there is a step in the referred limit values, the lower value shall be applied at the transition frequency.

Table 8.3.3-1: Void

| - |  |  |
| --- | --- | --- |
| - |  |  |

------------------------------ Next modified section -------------------------

### 8.4.3	Limits

The equipment shall meet the limits according to CISPR 32 [11] table A.10, which are defined for the average detector receiver and for quasi-peak detector receiver--. If the average limit is met when using a quasi-peak detector, the equipment shall be deemed to meet both limits and measurement with the average detector receiver is not necessary.

For the referred limit values following shall apply:

Where the limits value varies over a given frequency range, it changes linearly with respect to the logarithm of the frequency.

Where there is a step in the relevant limit, the lower value shall be applied at the transition frequency.

Table 8.4.3-1: Void

| - |  |  |
| --- | --- | --- |
| - |  |  |

Alternatively, for equipment intended to be used in telecommunication centres the limits given in CISPR 32 [11] table A.9 shall be used.

Table 8.4.3-2: Void

| - |  |  |
| --- | --- | --- |
| - |  |  |

------------------------------ End of modified section -------------------------