3GPP TSG RAN #109			RP-251925

Beijing, China, September 15 – 18, 2025

Agenda item: 9.1.4.5

Source:	Apple

Title:   	Moderator's summary for RAN4 REL-20 5G-Adv: Demodulation topics

Document for: Information

# Introduction

In RP-251879 (WF for RAN4 led REL-20 topics) led by RAN4 chair, the following agreements were reached:

The high-level scope for WI on Demod enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - Normative work on SCM based requirements for SU-MIMO in FR1 - The exact scope also depends on the conclusion of R19 study - Study on SCM for MU-MIMO - Performance requirements for 6Rx UE with interference - Up to 4 MIMO layers for the target cell is considered - No RAN1/2 impact is expected - Target 0.5 TU allocation from Q1’26 and 1 TU from Q2’26.

In RP-251928 (Release 20 5G-Adv RAN4 Package) prepared by RAN/RAN1/RAN2/RAN4 Chairs, the following proposed scope is provided for discussion in RAN#109.

This document summarizes the relevant contributions on this topic and provides moderator recommendations for further discussions with an aim to reach an agreement on R20 WI scope.

# List of relevant contributions and their views

| RP-252037 | Views on Rel-20 demodulation topics for 5G-A | ZTE Corporation, Sanechips | - Normative work on SCM based requirements for SU-MIMO in FR1 Core part for SCM based SU-MIMO: - To specify SCM channel model - Applicable for both FR1 FDD and TDD bands - Applicable scenarios: - SU-MIMO PDSCH - 4 layers for 4Rx and 8layers for 8Rx - SU-MIMO PMI - 4 layers for 4Rx - Take Rel-19 SCM study outcome in TR 38.753 into account Perf part for SCM based SU-MIMO: - To define the UE demod requirement based on SCM channel model - Study on SCM for MU-MIMO - 4Rx and 8Rx as baseline; - MMSE-IRC receiver as baseline; Performance requirements for 6Rx UE with interference - Up to 4 MIMO layers for the serving cell is considered - Use MMSE-IRC receiver as baseline; - With both inter-cell interference and intra-cell inter-user interference considered; - Take the legacy approach to derive the interference profile. - No RAN1/2 impact is expected Target 0.5 TU allocation from Q1’26 and 1 TU from Q2’26. |
| --- | --- | --- | --- |
| RP-252070 | Views on 5G-A Rel-20 demodulation topics | Samsung | Normative work for SU-MIMO in FR1 ○Limited test cases selected ○Only one SCM candidate selected ○Band agnostic requirements to be evaluated ○Rel-19 TS38.901 update on scaling of angles to be evaluated ○Fixed angular properties and non-uniform PMI statistics to be overcomed Study on SCM for MU-MIMO ○6G-oriented ○Applicable for both 5GA and 6G ○Different desired angles for target and co-scheduled UE ○Angular distribution model for co-scheduled UE(s) ○Extended channel model including user information |
| RP-252487 | Qualcomm views on Demod topics fo 5GA Rel-20 | Qualcomm Incorporated | - SCM based requirements for SU-MIMO in FR1 - Requirements should be limited to a few simple test cases as “trial” - 4L/4Rx and potentially 8Rx/8L demod tests - PMI results not very well aligned in the Rel-19 SI. Further study needed whether PMI tests could be defined - No mandatory requirements. Requirements valid with UE declaration - Study on SCM - Continue the Rel-19 study with channels suitable for MU-MIMO - Include also study on usage of SCM/CDL for temporal prediction considering recent findings about channel correlation - Requirements for 6Rx with interference - Should reuse the interference scenarios from previous work (4Rx/8Rx) - Same requirements for handheld device and FWA - Requirements for SU-MIMO and MU-MIMO (same as for 8Rx) - Release independence back to same Release as 6Rx without interference |
| RP-252497 | Apple’s view on Rel-20 RAN4 package for Demodulation | Apple | Define the following UE performance requirements for 6Rx handheld and FWA devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver Reuse the Rel-17 interference model for 2/4Rx UE as a baseline Up to 4 MIMO layers for target cell - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver Reuse the existing interference model for 2/4Rx UE as a baseline Up to 4 MIMO layers for target cell |
| RP-252514 | Rel-20 RAN4 Demodulation | MediaTek Inc. | Priority: 2 > 3 > 1 - SCM SU-MIMO tests do not require any enhanced UE implementations. In other words, it does not improve the current NR cell performance. - Study on SCM for MU-MIMO was the main intention to introduced this work in Rel-19. Dropping this is not preferred. - In 6G SI, the Demoduationpart may also start the corresponding study for channel model. A clear boundary between 5G and 6G channel model studies is needed, e.g., AI vs non-AI, legacy FR vs new FR. - Neutral to 6Rx requirement with interference. - Up 4-layer MIMO limitation is reasonable, since it is unlikely that >4-layer MIMO can be useful under |
| RP-252521 | Proposal on Demod project scope | Xiaomi Technology | Proposal 1: Specify MMSE-IRC receiver requirements for 6Rx UE with following objective •PDSCH demodulation requirements with inter-cell/intra-cell interference scenarios •CQI reporting requirements for inter-cell interference scenario Note: Reusing existing interference profile and test set-up for 2Rx/4Rx/8Rx UE Proposal 2: Spatial channel model handling (avoid mixing study item and work item in the same release) •SpecifyPDSCH/PUSCH SU-MIMO requirements in FR1 for Rel-20 5G-A •Continue the study for unified spatial channel model incl. MU-MIMO, FR2 for uplink and downlink under 6G •The study in 6G can be leveraged into 5G-A at later stage if needed. |
| RP-252536 | Views on RAN4 Rel-20 Demodulation performance evolution WI | China Telecom | - Proposal 1: It is proposed to firstly start the normative work (6Rx UE with interference, SCM requirements for SU-MIMO). The study work (SCM for MU-MIMO) can be started after the new SCM requirements are stable. - Define the following performance requirements for 6Rx TDD UEs with MMSE-IRC receiver: - PDSCH demodulation requirements with inter-cell and with intra-cell inter-user interference - CQI reporting requirements with inter-cell interference - The existing interference model for 2/4/8 Rx UE is a start point - Define release independent rules if feasible. - Spatial channel modeling (SCM) - Phase I: Define new SCM based UE performance requirements for FR1 SU-MIMO (Starts from RAN4#116bis) - Channel model are selected from the Rel-19 RAN4 study in TR38.753. - Simulation parameters agreed in the Rel-19 RAN4 study in TR38.753 is a starting point. - The following requirements will be defined: - Define PDSCH demodulation requirements for 8T8R 8 layer - [PMI reporting test case based on SCM with Type I/eType II] (To be discussed during RAN#109) - Phase II: Study the feasibility of SCM based performance requirements under DL MU-MIMO scenario for FR1 (Starts from [RAN4#120]) - Both baseline MMSE-IRC [and advanced R-ML] receivers can be considered in the study - Both Type I [and eType II] codebook can be considered in the study. |
| RP-252573 | Views of RAN4 5G-A Demodulation topics | Ericsson | Proposal 1 Create optional demodulation requirements based on SCM: o PDSCH requirement for SU-MIMO (8 layers) o Type II PMI reporting requirement for SU-MIMO Proposal 2 Study further MU-MIMO SCM. Within the context of 6G, further study SCM for AI/ML. |
| RP-252608 | Views on Rel-20 Demodulation Topics | CMCC | Objective for Rel-20 • Normative work on SCM based requirements for SU-MIMO in FR1 • Define PDSCH requirements and PMI requirements by using CDL channel model defined in TR 38.753 • Study on SCM for MU-MIMO • FR1 is prioritized Performance requirements for 6Rx UE with interference • Only handheld UE, up to 4 MIMO layers for the target cell is considered • Both inter-cell interference and intra-cell inter-user interference • Reuse the existing interference model |
| RP-252663 | Considerations on RAN4-led Rel-20 5G-Adv demodulation topics | Nokia | Observation 1: The introduction of SCM based requirements does not necessitate the development of new receivers; rather it provides assurance and lower bounds for the performance of existing (and future) receivers in typical MIMO environments that were blind spots in legacy channel models. Hence, the requirements may be optional in 5GA. Observation 2: Rel-19 6Rx performance requirements (introduced in NR_ENDC_RF_Ph4) do not currently cover interference scenarios. With 6Rx a substantial gain of advanced receivers in high throughput scenarios is expected. Observation 3: The SCM topic is foundational for 6G and TUs are expected to be heavily limited for 5GA. - Rel-20 demod work shall be focused on specifying performance requirements using the rCDL based spatial channel model detailed in TR 38.753 for SU-MIMO in FR1; covering at least 8T8R 2CW, 4T4R 1CW and 8T4R eTypeII use cases. Additionally, rCDL based practical spatial channel modelling methodology extensions for MU-MIMO demodulation requirements and CSI reporting requirements shall be studied; covering at least eTypeII and advanced receiver use cases. - Additionally, Rel-20 demod work may be considered to define performance requirements for 6Rx UE with interference; at least the case with up to 4 MIMO layers for the target cell may be considered. Whether both inter cell and inter-user interference, and both PDSCH and PMI requirements are needed shall be discussed. |
| RP-252714 | Views on demodulation works for Rel-20 5G-A | CATT | Proposal 1: RAN to consider the following objective for normative works on SCM based requirements for SU-MIMO in FR1. Define the following UE performance requirements based on spatial channel models: PDSCH demodulation performance and CQI reporting requirements under SU-MIMO scenarioorCDL-C1 and/or xTDL-C1 based - oAntenna configuration: at least 4T4R, 8T8R - oMMSE-IRC receiver as baseline - oTest metrics: consider 30% and 70% maximum throughput Proposal 2: RAN to consider the following objective for spatial channel modelling under MU-MIMO scenario. Study on SCM for MU-MIMO •Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements and CSI reporting requirements: •Further identify the limitations of the current (i.e. up to and including Release 18) channel models under MU-MIMO scenario •leverage Rel-19 works on spatial channel model of TR38.753 for SU-MIMO scenario. •Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. •The methodology shall be for FR1 (conducted). Proposal 3: RAN to consider the following objective for Performance requirements for 6Rx UE. - Specify UE demodulation requirements for 6Rx UE with interference - MMSE-IRC receiver as baseline - Intra-cell inter-user interference and inter-cell interference respectively - Up to 4 MIMO layers for the target cell |
| RP-252742 | Proposals for RAN4-led Rel-20 Demod topics | Huawei, HiSilicon | Motivation for SCM −A suitable spatial channel model should have the following characteristics −Be applicable for all scenarios including SU-MIMO and MU-MIMO, all frequency ranges including FR1, FR2 and FR3 −Be applicable for both RRM and demodulation performance requirements testing for both UE and BS −Be applicable for testing of new scenarios and features, such as AI/ML −Such spatial channel model should be ready and used in performance requirements testing for both UE and BS in 6G Day1 −RAN already started the 6GR study, the demodulation and performance aspects are included, more efforts should focus on 6G study −It is in 5G late stage, less interest from industry to spend much efforts to test only 4 aligned cases studied in Rel-19 SI Proposal 1: Have a comprehensive study on CDL channel model to ensure it can be used for RRM and demodulation performance requirements testing for both UE and BS in 6G Day 1 •Consolidate and improve the study of CDL channel for SU-MIMO and study the CDL channel model for MU-MIMO 1)Study feasibility to define a generic CDL channel or a set of CDL channel models that can be applicable for all frequency ranges. 2)Introduce varying spatial properties to CDL channel to make PMI test more meaningful. 3)Consider new UE antenna modeling and updated scaling factor procedure defined in Rel-19 version of TS 38.901 4)Study Uplink CDL channel model as per Rel-19 version of TS 38.901 Motivation for 6Rx demod: Considering the limited 5G TU, the parallel 5G and 6G session and workload in RAN4, it is suitable to only consider the definition of demodulation requirements for 6Rx with interference in R20 Proposal 2: Only define the demodulation performance requirements for 6Rx with interference for 5G |
| RP-252748 | Discussion of WID objectives for NR demodulation performance for Rel-20 | BT, AT&T, Bouygues Telecom, Deutsche Telekom, Orange, SK Telecom, T-Mobile USA, Telefónica, TIM, Vodafone, Nokia | - Adopt the following as objectives of the WI: - Specify performance requirements under a spatial channel model for SU-MIMO in FR1 - Using the rCDL based spatial channel model detailed in TR 38.753 - Specify PDSCH demodulation performance requirements for the 4T4R case (1 codeword) and the 8T8R (2 codeword) use case. - Specify CSI reporting performance requirements for the 4T4R, 8T4R and 32T4R eTypeII precoding use case as studied in the TR38.753. - Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements and CSI reporting requirements - Base work on the rCDL based spatial channel model detailed in TR 38.753. - Consider spatially consistent approaches based both on creating a single channel for a single UE, as well as, creating channels for all UEs. - At least, consider the TPUT performance of advanced receivers, as well as the CSI selection performance of advanced UEs, both under MU interference. - Decouple the work into normative (1st bullet) and study phase (2nd bullet). |
| RP-252751 | Views on RAN4 Rel-20 5G-A - Demodulation topics | Intel Corporation | Proposal #1: Demodulation objective “SCM based requirements” - Define FR1 SU-MIMO UE performance requirements for CDL-based models requirements defined in TR 38.753 for a limited set of test cases - PDSCH rank 4 demodulation for 4Rx UEs - PDSCH rank 8 demodulation for 8Rx UEs - eTypeII PMI reporting requirements Proposal #2: Demodulation objective “Further study on SCM” - The study objectives shall include - Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements: - Consider CDL-based channel modelling approach. Use CDL models in TR 38.753 as the starting point and identify any necessary changes. - The methodology shall include both FR1 and FR2, with first priority for FR1. - The channel modelling methodology should be applicable for 5G-A and 6G testing - Further discuss how to handle possible overlap with potential 6G SI RAN4 work on channel models for performance requirements. Proposal #3: Demodulation objective “Performance requirements for 6Rx UE with interference” - Confirm RAN#108 agreements with work planned for 6-9 months |

# Moderator recommendations for discussions

## SCM

It is recommended to consider the following WI scope in the WID:

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

  - RAN4 is tasked to down-select only one channel model from the two candidates, rCDL-C1 and xTDL-c1, in the first quarter. If there is no down-selection made in RAN4, RAN is expected to make a down-selection.

    - Criteria to use for down-selection:

      - [The model that’s closest to the one used in RAN1 shall be selected.]

      - To be closest to real-world propagation

      - To meet the test purpose to verify higher MIMO layer performance

      - To minimize the test complexity

  - [Using the down-selected spatial channel model detailed in TR 38.753 as starting point]

    - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case.

    - Specify CSI performance requirements for the 4T4R and 8T4R eTypeII precoding use case.

    - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding.

  - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

  - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

- Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements and CSI reporting requirements

  - Base work on the rCDL based spatial channel model detailed in TR 38.753 as staring point.

    - Consider applicability to and support for AI/ML scenarios

  - Note 2: The methodology shall include both FR1 and FR2, with first priority for FR1.

  - Note 3: The channel modelling methodology should be applicable for 5G-A and can be considered 6G testing

  - Note 4: The study is aimed to consider both UL and DL.

Offline Discussion

| Company | Comments |
| --- | --- |
| Qualcomm | The sub-bullet on “Specify CSI performance requirements for the 4T4R and 8T4R eTypeII precoding use case” should be changed to “study the feasibility of introducing requirements for CSI performance with 4T4R and 8T4R eTypeII precoding . Requirement definition to be further discussed based on the feasibility study”. There were some issues in the PMI simulation alignment so far and it is not clear if introducing these requirements is feasible. For the MU-MIMO, we should only have a study for now, the wording implies we could already define requirements. We should either have a Note saying no requirements definition in Rel-20 or at least say “future introduction of MU-MIMO demodulation requirements. |
| China Telecom | We support the recommended detailed scope. Same time, as we pointed out in our contribution, it is very challenge to start 3 objectives simultaneously from October considering there are totally 0.5 TUs. We propose to first start the SU-MIMO normative work in Oct and the MU-MIMO study can be started later after the SU-MIMO requirements are stable. Its workload can be much reduced by inheriting the progress from the SU-MIMO normative work, which not only has little impact to the study progress, but also avoid overlapping discussion and save efforts of the group. |
| Huawei | - Specify the PDSCH normative work on SU-MIMO based on the alignment results during the SI with further consideration of the left open issues identified and captured in the TR 38.753: - Band agnostic requirements. - Consider both UE antenna modelling and updated angular scaling procedure studied by RAN1 in Rel-19. - Consider both rCDL as well as eTDL outcomes of the Rel-19 SI, leave RAN4 to decide on the solution to specify. - Further study the alignment of PMI testing is needed. - For the study of MU-MIMO part for both FR1 and FR2, considering the limited TU for 5G-A; better to discuss it under 6G Demod study item. Further consideration on agreement in RAN1 need to be considered. |
| Ericsson | We support the objectives. In our view, it would be helpful for RAN to guide RAN4 to use the rCDL model for the requirements work. This would save time and focus the RAN4 work. |
| Nokia | From the offline discussion, there is nothing that RAN4 will be able to do in the next quarter that cannot be done already at RAN#109 to select the channel model for normative requirements. The rCDL-C1 model had approximately double the number of contributors to the alignment work and is aligned with RAN1’s work on MIMO feature development, so it seems inconceivable that RAN#110 would not select it; therefore the decision can already be made at RAN#109 without wasting further time to discuss in RAN4. |
| Orange | We agree with Nokia’s comments and fully support a decision at this plenary to use of rCDL in RAN4 work. |
| ‍BT | We support getting the guidance of RAN. A study has been done already. Further study will consume too much RAN4 time. |
| Vodafone | We agree with Nokia’s comments. |
| Telenor | Telenor: We support the comments of Ericsson and Nokia. |

## Performance requirements for 6Rx UE with interference

It is recommended to consider the following WI scope in the WID:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference

  - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver

    - Use the Rel-17 interference model for 2/4Rx UE as a baseline

    - Up to 4 MIMO layers for target cell

  - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Note: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference.

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Use the existing interference model for 2/4Rx UE as a baseline

    - Up to 2 MIMO layers for each user

Offline Discussion

| Company | Comments |
| --- | --- |
| China Telecom | We support the recommended detailed scope. Regarding release independent, we believe it should follow the R19 Core part for 6Rx UE feature, which is still under discussion. We propose to add ‘consider release independent if feasible’ at this stage. |
| Qualcomm | For the intra-cell inter-user interference, the last bullet should be as below: o Up to 2 MIMO layers for each user target cell target cell is a bit confusing. 2 MIMO layers for each user should be enough. |
| Ericsson | OK with the objectives and the Qualcomm ammendment |

# Conclusion

Based on the email discussion and breakout room discussion, the following WI scope is agreeable:

## SCM

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

  - RAN4 is tasked to down-select only one channel model from the two candidates, rCDL-C1 and xTDL-c1, in the first quarter. If there is no down-selection made in RAN4, RAN is expected to make a down-selection.

  - Using the down-selected spatial channel model detailed in TR 38.753 as starting point

    - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case.

    - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding.

  - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias

  - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

- Study practical spatial channel modelling methodology for MU-MIMO demodulation requirements and CSI reporting requirements

  - Base work on the rCDL based spatial channel model detailed in TR 38.753 as staring point.

    - Consider applicability to and support for AI/ML scenarios

  - Note 3: The methodology shall include both FR1 and FR2, with first priority for FR1.

  - Note 4: The channel modelling methodology should be applicable for 5G-A and can be considered 6G testing

  - Note 5: The study is aimed to consider both UL and DL.

## Performance requirements for 6Rx UE with interference

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference

  - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver

    - Use the Rel-17 interference model for 2/4Rx UE as a baseline

    - Up to 4 MIMO layers for target cell

  - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Note: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference.

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Use the existing interference model for 2/4Rx UE as a baseline

    - Up to 2 MIMO layers for each user