3GPP TSG RAN Meeting #109												RP-251936

Beijing, China, September 15-18th, 2025

Title:	Text proposal for TR 38.914 for NTN performance requirements

Source:	Thales, ESA, Airbus DS, Fraunhofer IIS, Fraunhofer HHI, Eutelsat, Iridium, Gatehouse Satcom, TNO, Novamint, ST Engineering iDirect

Agenda Item:	8.2.1.2 Key performance indicators

Document for:	Discussion/Decision

# Introduction

This contribution presents a Text proposal for TR 38.914 on NTN performance requirements.

======================Start of the changes========================

# 5.	Requirements and key technical principles

## 5.1	Key performance indicators

This section describes the definitions and requirements of all KPIs.

The KPI targets defined in this section as well as in Report ITU-R M.[IMT-2030. TECH PERF REQ] shall be supported by next generation access technologies. The evaluation methodology defined in Report ITU-R M.[IMT-2030.EVAL] shall be used to evaluate the targets specified in Report ITU-R M.[IMT-2030. TECH PERF REQ].

Editor note: aspects related to IMT-2030 TPRs and 3GPP RAN KPIs will be included in this section

Editor note: The KPIs captured in RAN#108 focus on ITU IMT-2030 TPRs. The definitions are subject to revision based on further discussion and progress in ITU

Editor note: Evaluation methodology to be added for each KPI.

### 5.1.1	Peak data rate

[Peak data rate is the maximum achievable data rate under ideal conditions (in bit/s), which is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let W denote the channel bandwidth and SEp denote the peak spectral efficiency in that band. Then the peak data rate Rp is given by:

Rp = W × SEp

Peak spectral efficiency and available bandwidth may have different values in different frequency ranges. In case bandwidth is aggregated across multiple bands, the peak data rate will be summed over the bands. Therefore, if bandwidth is aggregated across Q bands then the total peak data rate is

R=i=1Q  Wi × SEpi

where Wi and SEpi (i = 1, …, Q) are the component bandwidths and spectral efficiencies respectively.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak data rate are as follows:

–	Downlink peak data rate is TBD Gbit/s.

–	Uplink peak data rate is TBD Gbit/s.]

For NTN, the following requirements apply:

The following Table indicates the required peak data rate and, in notes, the maximum bandwidth that may be used to achieve the required rate.

| [Test Environment] | Downlink (Mbit/s) | Uplink (Mbit/s) |
| --- | --- | --- |
| Pedestrian outdoor (handheld UE) | More than [140] (1) | More than [2] (1) |
| Cars, small airplane, drones and railway (VSAT type 1) |  |  |
| Large airplane and vessels (VSAT type 2) | [1800] | [1800] |

(1) Requirements are derived using an assignable bandwidth of up to 30 +30 MHz in paired spectrum over one satellite beam.

(2) Requirements are derived using an assignable bandwidth of up to 800 + 800 MHz in paired spectrum over one satellite beam.

### 5.1.2	Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalised by channel bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assuming error-free conditions assignable to a single mobile station, when all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots, guard bands and guard times).

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for peak spectral efficiencies are as follows:

–	Downlink peak spectral efficiency is TBD bit/s/Hz.

–	Uplink peak spectral efficiency is TBD bit/s/Hz.

These values were defined assuming an antenna configuration to enable TBD spatial layers (streams) in the downlink and TBD spatial layers (streams) in the uplink.

For NTN, the following requirements apply:

The following Table indicates the minimum achievable peak spectral efficiency requirements.

| [Test Environment] | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Pedestrian outdoor (handheld UE) | More than [3] (1) | More than [1.5] (1) |
| Cars, small airplane, drones and railway (VSAT type 1) |  |  |
| Large airplane and vessels (VSAT type 2) |  |  |

(1) Requirements are derived using an assignable bandwidth of up to 30 +30 MHz in paired spectrum over one satellite beam.

(2) Requirements are derived using an assignable bandwidth of up to 800 + 800 MHz in paired spectrum over one satellite beam.

### 5.1.3	User experienced data rate

Editor note: name of this KPI may need to be changed

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

For the evaluation of uplink user experienced data rate, use system level simulation with no bandwidth scaling taking UE maximum output power into account.

For the evaluation of downlink user experienced data rate, in case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency,  let W denote the channel bandwidth and SEuser denote the 5th percentile user  spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser

In case bandwidth is aggregated across multiple bands (one or more TRxP layers), the user experienced data rate will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for the user experienced data rate are as follows in the Dense Urban – IC [test environment]:

–	Downlink user experienced data rate is TBD Mbit/s.

–	Uplink user experienced data rate is TBD Mbit/s.

For NTN, the following requirements apply:

The following Table indicates the minimum achievable user experienced data rate.

| [Test Environment] | Downlink (Mbps) | Uplink (Mbps) |
| --- | --- | --- |
| Pedestrian outdoor (handheld UE) | More than [1] | More than [0.1] |
| Cars, small airplane, drones and railway (VSAT type 1) |  |  |
| Large airplane and vessels (VSAT type 2) |  |  |

### 5.1.4	5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

With Ri (Ti) denoting the number of correctly received bits of user i, Ti the active session time for user i and W the channel bandwidth, the (normalized) user throughput of user i, ri, is defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for 5th percentile user spectral efficiency for various [test environments] are summarized in Table 5.1.4-1.

Table 5.1.4-1: 5th percentile user spectral efficiency

| [Test Environment] | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – IC | 0.9 | TBD |
| Dense Urban – IC | 0.675 | TBD |
| Rural – IC | TBD | TBD |

For NTN, the following requirements apply:

The following Table indicates the minimum achievable 5th percentile user spectral efficiency.

| [Test Environment] | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Pedestrian outdoor (handheld UE) | More than [0.03] | More than [0.003] |
| Cars, small airplane, drones and railway (VSAT type 1) |  |  |
| Large airplane and vessels (VSAT type 2) |  |  |

### 5.1.5	Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalized appropriately considering the uplink/downlink ratio.

Let Ri (T) denote the number of correctly received bits by user i (downlink) or from user i (uplink) in a system comprising a user population of N users and M TRxPs. Furthermore, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency, SEavg is then defined according to the following equation:

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirements for average spectral efficiency for various [test environments] are summarized in Table 5.1.5-1.

Table 5.1.5-1: Average spectral efficiency

| [Test Environment] | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – IC | 27 | TBD |
| Dense Urban – IC | 23.4 | TBD |
| Rural – IC | TBD | TBD |

For NTN, the following requirements apply:

The following Table indicates the minimum achievable average spectral efficiency.

| [Test Environment] | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Pedestrian outdoor (handheld UE) | More than [0.5] | More than [0.1] |
| Cars, small airplane, drones and railway (VSAT type 1) |  |  |
| Large airplane and vessels (VSAT type 2) |  |  |

### 5.1.6	Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/m2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived for a particular use case (or deployment scenario) of one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg through the following equation:

Carea = ρ × W × SEavg

In case bandwidth is aggregated across multiple bands, the area traffic capacity will be summed over the bands.

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

The minimum requirement for area traffic capacity in downlink is TBD Mbit/s/m² in the Indoor Hotspot – IC [test environment].

For NTN, the following requirements apply:

The following Table indicates the minimum achievable Area traffic capacity.

| [Test Environment] | Downlink (kbit/s/km2) | Uplink (kbit/s/km2) |
| --- | --- | --- |
| Pedestrian outdoor (handheld UE) | More than [8] | More than [1.5] |
| Cars, small airplane, drones and railway (VSAT type 1) |  |  |
| Large airplane and vessels (VSAT type 2) |  |  |

The area capacity is averaged over the coverage area of a single satellite beam.

### 5.1.7	User plane latency

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirements for user plane latency are:

–	4ms for IC

–	1ms for HRLLC

The above assumes unloaded conditions (i.e. a single user) for small IP packets (e.g. 0 byte payload + IP header), for both downlink and uplink.

When operating with NTN, the minimum requirement for user plane latency is from [4] ms up to [650] ms.

### 5.1.8	Control plane latency

Control plane latency refers to the time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE).

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for control plane latency is 20 ms.

Note: Proponents are encouraged to consider lower control plane latency, e.g. 10 ms.

When operating with NTN, the minimum requirement for control plane latency is from [4] ms up to [1150] ms.

### 5.1.9	Connection density

Connection density is the number of connections fulfilling a specific quality of service (QoS) per unit area (per km2).

Connection density should be achieved for a given bandwidth and a given number of TRxPs. The target QoS is to support delivery of a message of a certain size within a certain time and with a certain success probability.

This requirement is defined for the purpose of evaluation in the Massive Communication usage scenario.

The minimum requirement for connection density is 106 connections per km2.

For NTN, the following requirements apply:

The minimum requirement for connection density is [103 TBC] connections per km2. This assumes a 30 + 30 MHz spectrum allocation (DL/UL bandwidth).

### 5.1.10	Reliability

Reliability relates to the capability of transmitting a given amount of traffic within a predetermined time duration with high success probability.

Reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

This requirement is defined for the purpose of evaluation in the Hyper Reliable and Low Latency Communication usage scenario.

The minimum requirement for the reliability is 1-10-5 success probability of transmitting a layer 2 PDU (protocol data unit) of 32 bytes within 1 ms in channel quality of coverage edge for the Urban Macro-HRLLC [test environment], assuming small application data (e.g. 20 bytes application data + protocol overhead).

Proponents are encouraged to consider larger packet sizes, e.g. layer 2 PDU size of up to 100 bytes.

### 5.1.11	Mobility

Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h).

The following classes of mobility are defined:

–	Stationary: 0 km/h

–	Pedestrian: 0 km/h to 10 km/h

–	Vehicular: 10 km/h to 120 km/h

–	High speed vehicular: 120 km/h to 500 km/h

–	Airplane: 500 km/h to 1500 km/h

–	Drones: TBD km/h and max acceleration TBD

This requirement is defined for the purpose of evaluation in the Immersive Communication usage scenario.

Table 5.1.11-1 defines the mobility classes that shall be supported in the respective [test environments].

Table 5.1.11-1: Mobility classes

|  | [Test environments] for IC |  |  |
| --- | --- | --- | --- |
|  | Indoor Hotspot – IC | Dense Urban – IC | Rural – IC |
| Mobility classes supported |  |  |  |

A mobility class is supported if the traffic channel link data rate on the uplink, normalized by bandwidth, is as shown in Table 5.1.11-2. This assumes the user is moving at the maximum speed in that mobility class in each of the [test environments].

Table 5.1.11-2: Traffic channel link data rates normalized by bandwidth

| [Test environment] | Normalized traffic channel link data rate (bit/s/Hz) | Mobility (km/h) |
| --- | --- | --- |
| Indoor Hotspot – IC | 2.25 | 10 |
| Dense Urban – IC | 1.68 | 30 |
| Rural – IC | [0.8~1.2] | 120 |
|  | [0.45~0.675] | 500 |

These values were defined assuming an antenna configuration as described in Report ITU-R M.[IMT-2030.EVAL].

For NTN, the following requirements apply:

The following Table indicates the minimum achievable Area traffic capacity.

| [Test Environment] | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Pedestrian outdoor (handheld UE) | More than [0.005] @ 500 km/h (on-board high-speed train) | TBD |
| Cars, small airplane, drones and railway (VSAT type 1) |  |  |
| Large airplane and vessels (VSAT type 2) |  |  |

### 5.1.12	Mobility interruption time

[Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

This requirement is defined for the purpose of evaluation in the Immersive Communication and Hyper Reliable and Low Latency Communication usage scenarios.

The minimum requirement for mobility interruption time is TBD.

For NTN, the following requirements apply:

- The requirement for mobility interruption time during transition between 2 satellites at same orbit is [0] ms

- The requirement for mobility interruption time during transition between NTN and TN is [TBD] ms, excluding the differential delay between NTN and TN.

### 5.1.13	Bandwidth

Bandwidth is the maximum aggregated system bandwidth. The bandwidth may be supported by single or multiple radio frequency (RF) carriers. The bandwidth capability of the RIT/SRIT is defined for the purpose of IMT-2030 evaluation.

The requirement for bandwidth is at least 400 MHz.

For NTN, the requirement for bandwidth is at least 800 MHz

### 5.1.14	Positioning

Positioning is the ability to calculate the approximate position of connected devices. Positioning accuracy is defined as the difference between the calculated horizontal/vertical position and the actual horizontal/vertical position of a device.
This requirement is defined for the purpose of evaluation in the Immersive Communication, Hyper-Reliable and Low Latency Communication, Ubiquitous Connectivity, and Integrated Sensing and Communication usage scenarios.

Regulatory requirements are considered as a minimum performance targets for NTN positioning. For regulatory use cases, the following requirements are considered as a minimum performance targets for NR positioning:

-	Horizontal positioning error <= 50m for 80% of UEs

-	Vertical positioning error <5 m for 80% of UEs

-	End to end latency and TTFF < 30 seconds

For NTN, additional requirements apply:

| Usage scenari | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Airplane en-route | [50] m | [50] m | [99 %] | [1] s | Outdoor | Up to [1500] km/h | Airplane mounted |
| Airplane landing | [10] m | [10] m | [99 %] | [1] s | Outdoor | Up to [350] km/h | Airplane mounted |
| UAV positioning | [1] m | [1] m | [99 %] | [1] s | Outdoor | Up to [160] km/h | UAV mounted |
| NOTE: Multiple satellites can be used to support 3GPP positioning technologies. |  |  |  |  |  |  |  |

### 5.1.15	Energy efficiency

[Note: Energy efficiency definition may be different in idle versus connected mode]

### 5.1.16	Composite requirement

Editor note: KPI related to data rate, latency, reliability, capacity. Name of this KPI may need to be changed

### 5.1.17	Sensing

### 5.1.18	AI

### 5.1.xx	XXXX

Editor note: More KPIs can be added based on companies input and discussion

======================End of the changes========================

# 4	Conclusion

Proposal: Add the text proposal in clauses 5.1 of the TR 38.914.

Average spectral efficiency corresponds to “spectrum efficiency” in Recommendation ITU-R M.2160.