3GPP TSG RAN Meeting #108						RP-251952

Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.2

Source:	Ericsson

Title:	Technical Performance Requirements (TPR) for IMT-2030 related to WP5D

Document for:	Approval

# Background

At TSG RAN#106, a study was initiated for 6G Scenarios and requirements (RP-243327), where in the first phase the objective is to investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160 and to share the outcome with ITU-R WP5D as suitable. A first LS response was sent from TSG RAN#106 to ITU-R WP5D in RP-243325 with a list candidate TPR and a second LS was sent from TSG RAN#108 with target TPR values (RP-251825).

At TSG RAN#108, the first version of TR 38.914 (v0.1.0) for the Study on 6G Scenarios and Requirements was created. It already contains many deployment scenarios, key performance indicators (KPI) and other requirements for 6G, in particular for legacy requirements from 5G.

This contribution gives input to the continuing work on the 6G scenarios and requirements study for TSG RAN, based on the ongoing discussions in WP5D on the TPR Working document and WP5D input documents:

- Candidate IMT-2030 TPR in WP5D (Section 2)

- TPR Values (Section 3)

# Candidate IMT-2030 TPR in WP5D

The ITU-R WP5D Working document for the draft new Report on TPR (latest version in ITU-R 5D/792 Annex 5.14) lists technical performance requirements, based on the capabilities in ITU-R Recommendation M.2160. Definitions are derived from that recommendation, the legacy definitions for IMT-2020 in Report ITU-R report M.2410, plus additional input made to WP5D. Being a “working document” means that the text is under discussion and has varying levels of stability, ranging from text with wide consensus to some proposed TPR with weak support within WP5D. No text is formally agreed at this point.

The following table shows the Key performance indicators in TR 38.914 and what the status is of the related TPR in ITU-R WP5D. Note that any “agreement” is not yet formal and there has not yet been a discussion of values for the TPR in WP5D. Discussion of target values is planned for the coming WP5D#50 meeting in October. Definitions of most legacy requirements inherited from IMT-2020 are stable.

Table 1: Relation to and status of TPR in WP5D

| KPI in TR 38.914 |  | Corresponding TPR in WP5D Working document |  | Status in WP5D |
| --- | --- | --- | --- | --- |
| 5.1.1 | Peak data rate | 4.1 | Peak data rate | Agreed with definition |
| 5.1.2 | Peak spectral efficiency | 4.3.1 | Peak spectral efficiency | Agreed with definition |
| 5.1.3 | User experienced data rate | 4.2 | 5th percentile user data rate | Agreed with definition |
| 5.1.4 | 5th percentile user spectral efficiency | 4.3.3 | 5th percentile user spectral efficiency | Agreed with definition |
| 5.1.5 | Average spectral efficiency | 4.3.2 | Average spectral efficiency | Agreed with definition |
| 5.1.6 | Area traffic capacity | 4.4 | Area traffic capacity | Agreed with definition |
| 5.1.7 | User plane latency | 4.7.1 | User plane latency | Agreed with definition |
| 5.1.8 | Control plane latency | 4.7.2 | Control plane latency | Agreed with definition |
| 5.1.9 | Connection density | 4.5 | Connection density | Agreed with definition |
| 5.1.10 | Reliability | 4.8 | Reliability | Agreed with definition |
| 5.1.11 | Mobility | 4.6 | Mobility | Agreed with definition |
| 5.1.12 | Mobility interruption time | 4.6.1 | Mobility interruption time | Agreed with definition |
| 5.1.13 | Bandwidth | 4.12 | Bandwidth | Agreed with definition |
| 5.1.14 | Positioning | 4.11 | Positioning | Draft definition in place |
| 5.1.15 | Energy efficiency | 4.15 | Energy efficiency for sustainability | Draft definition in place, agreed for Network and UE. |
| 5.1.16 | Composite requirement | [4.9 | Composite requirement] | Draft definition in place with large support, no final agreement |
| 5.1.17 | Sensing | 4.13 | Sensing-related capabilities | Agreed with draft definition in place, full scope is not yet agreed |
| 5.1.18 | AI | 4.14 | AI-related capabilities | Agreed with draft definition |
| - | - | [4.10 | Coverage] | Draft definition in place, no final agreement |
| - | - | [4.16 | Security] | Draft definition in place, but not yet agreed as TPR |
| - | - | 4.17 | Resilience | Agreed with draft definition in place |
| - | - | [4.X | Extended connectivity] | Draft definition, but not yet agreed as TPR |

# TPR values

The LS from ITU-R WP5D in RP-243202 asked for candidate items for TPR and also “proposed associated target values for the above candidate items for the minimum technical performance requirements which are requested to be provided preferably by WP 5D #49”. A TSG RAN LS response was given in RP-251825.  Table 1 below with proposed TPR values is further input to that discussion.

As a background to setting values for the TPRs, it is observed that:

- Traffic in mobile networks continues to increase.

- IMT-2030 should enable new use cases.

- Different emerging services for IMT-2030 rely on a different combination of TPR performance, such as data rate, delay, and reliability.

- The uptakes of the usage categories mMTC and URLLC introduced in IMT-2020 do not seem to be limited by the capabilities of the RIT.

Based on these observations, the TPR values in the fourth (blue) column of  Table 1 below are proposed.

Table 2: Proposed TPR values for quantitative TPRs

| TPR | IMT-2020 req | TSG RAN LS response (RP-251825) | IMT-2030 proposal | Comments |
| --- | --- | --- | --- | --- |
| Peak data rate | 20 Gbps 10 Gbps | TBD | DL: 50 Gbps UL: 25 Gbps |  |
| 5th percentile user data rate (previously User experienced data rate) | 100 Mbps 50 Mbps | TBD on the values 3GPP RAN agreed to use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. Due to the different evaluation methodology, a new title can be considered to avoid the confusion with previous IMT-2020 values. | DL: 100Mbps UL: 30Mbps | The proposals are values to be reached without bandwidth scaling. In addition to a having a requirement for the Dense Urban test environment. A lower requirement for e.g. the Rural test environment can be considered. |
| Peak spectral efficiency | 30 bps/Hz 15 bps/Hz | TBD on the values | DL: 50 bps/Hz UL: 25 bps/Hz |  |
| Average and 5%tile spectral efficiency | DL: 3.3-9.0, 0.12-0.3 UL: 1.6-6.75,0.045-0.21 (bits/s/Hz) | Indoor hotspot-IC/Dense urban-IC: - 3x of IMT-2020 for DL - TBD for UL Assuming 7GHz with: - BS: up to 1024 elements - UE: 4Rx or 6Rx or 8Rx Form factor and other operational considerations may limit the number of receiver antennas of UE | 2-3x IMT-2020 | This is enabled with more antenna elements than used for IMT-2020. Different factors of improvements could be considered in different test environments. |
| Area traffic capacity | 10 Mbps/m2 | TBD on values | 30 Mbps/m2 |  |
| Connection density | 106/km2 | 10⁶ (devices/km2) for massive communication | 106/km2 | Not seen as limiting for mMTC/MC usage. |
| Mobility | 10-500 km/h and 1.5-0.45 bps/Hz | Indoor Hotspot-IC: 2.25 (bit/s/Hz) @10 km/h Dense Urban- IC: 1.68 (bit/s/Hz) @30 km/h Rural-IC: [0.8~1.2] (bit/s/Hz)@120 km/h [0.45~0.675] (bit/s/Hz) @500 km/h NOTE1: Maximum velocity is up to 1000/1200km/h. Do not set spectrum efficiency target for this velocity for ITU TPRs. NOTE2: For Rural-IC, TBD on whether to include other velocities, e.g. 30km/h, 60km/h | TBD | Air-to-ground links may require higher speeds than 500 km/h. Possibly that does not need to be represented by an IMT-2030 TPR. |
| Mobility Interruption Time | 0 ms | TBD |  |  |
| Latency (UP and CP) | 1 ms URLLC, 4ms MBB 20ms (10ms optional) | IC: 4ms HRLLC: 1ms | Same as IMT-2020 | It is not seen that the IMT-2020 values are limiting usage. Proposed to focus on the composite requirement on bitrate, delay and reliability TPR instead. |
| Reliability | 99.999% for 32B in 1 ms | 1-10-5 with same IMT-2020 assumptions | Same as IMT-2020 | Not seen as limiting for URLLC usage. |
| Bandwidth | 200 MHz | 400MHz (same definition as IMT-2020) | 400MHz for system bandwidth (same definition as IMT-2020) |  |
| Positioning | - | Quantitative TPR, TBD on the values | TBD |  |
| Sensing | - | TBD | TBD |  |
| Energy efficiency | (Inspection only) | TBD | TBD |  |
| Composite requirement | - | TPR related to data rate, latency, reliability, capacity etc. Applicable usage scenario: immersive communication | For XR, 30Mbps DL, 10Mbps UL, 40ms two-way delay (DL + UL <= 40ms, e.g. DL = 10ms & UL = 30ms) with 99% reliability, capacity average of 5 simultaneous users per TRxP, or 433 per sqkm for 200m ISD at 95% satisfaction ratio. For video conferencing, 1Mbps DL, 1Mbps UL, 100ms two-way delay with 98% reliability. | Note: the proponent may evaluate DL and UL separately, with delay requirements that add to <=40ms. |

Coverage is presently a candidate TPR in WP5D (item 4.10 in the Working document), but with no agreement. While coverage may not become a dedicated TPR, link budget and related link performance would be sufficient and should be submitted by the proponents.

# Proposals

The values proposed in Table 2 of Section 3 are proposed as minimum requirements for the IMT-2030 TPR.