3GPP TSG RAN Meeting #109 																					RP-251970

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.3

Source:	OPPO

Title:	Discussion on requirement on architecture and migration

Document for:	Discussion and decision

Introduction

As outlined in SID, one key aspect to address is

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: […]

- Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas:

  - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc…

  - Overall high-level aspects of 5G to 6G migration

  - RAN architecture and interfaces, including RAN-Core interface

  - Coordination of 6G AI/ML framework

And corresponding clause in the TR intends to cover

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

This discussion paper is to address the corresponding part.

Discussion

Referring back to the legacy work for 5G, similar clause can be found in TR 38.913.

3	Requirements for architecture and migration of Next Generation Radio Access Technologies

The RAN design for the Next Generation Radio Access Technologies shall be designed to fulfil the following requirements:

-	The RAN architecture shall support tight interworking between the new RAT and LTE.

- Considering high performing inter-RAT mobility and aggregation of data flows via at least dual connectivity between LTE and new RAT. This shall be supported for both collocated and non-collocated site deployments.

-	The RAN architecture shall support connectivity through multiple transmission points, either collocated or non-collocated.

- The RAN architecture shall enable a separation of control plane signalling and user plane data from different sites.

- The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

-	Different options and flexibility for splitting the RAN architecture shall be allowed.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable context aware service delivery, low latency services, etc...

-	The RAN architecture shall allow for C-plane/U-plane separation.

-	The RAN architecture shall allow deployments using Network Function Virtualization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	The RAN architecture shall allow for the operation of Network Slicing3GPP TR 23.799 [7].

-	The RAN architecture shall support sharing of the RAN between multiple operators.

-	The design of the RAN architecture shall allow the deployment of new services rapidly and efficiently.

-	The design of the RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

-	The design of the RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

-	RAN-CN interfaces and RAN internal interfaces (both between new RAT logical nodes/functions and between new RAT and LTE logical nodes/functions) shall be open for multi-vendor interoperability.

-	The RAN architecture shall support operator-controlled sidelink (device-to-device) operation, both in coverage and out of coverage.

Requirement on architecture

Firstly, there are legacy requirements of architecture,

- Which has been proved to be useful/beneficial, that 6G should inherit from 5G

- Which is of much less interest, that 6G should de-prioritize instead

So, a summary is provided as follows.

Table 1 5G RAN architecture requirement to be inherited

| Text in TR 38.913 | Comment | Applicability to 6G |
| --- | --- | --- |
| The RAN architecture shall support connectivity through multiple transmission points, either collocated or non-collocated. | Multi-TRP is the basis of RAN architecture. | Yes |
| The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination. | Inter-node scheduling is a key component to enable MIMO operation. | Yes |
| The design of the RAN architecture shall allow the deployment of new services rapidly and efficiently. The design of the RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational) | This requirement is very straightforward, i.e., both legacy and new services have to be supported smoothly. | Yes (Yet the concerned service types have to be refined) |
| The design of the RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services. | This requirement is very straightforward, and align with the 6G target of “TCO reduction” | Yes |
| RAN-CN interfaces and RAN internal interfaces (both between new RAT logical nodes/functions and between new RAT and LTE logical nodes/functions) shall be open for multi-vendor interoperability. | The migration related part is left to the next sub-clause. For the other part, it is very straightforward. | Yes |

So firstly, RP can confirm to inherit the requirements that are still applicable.

RP confirms that 6G shall inherit the requirement on RAN architecture including: support for MIMO operation via multi-TRP, support for existing and new services, TCO reduction, Inter-operability for RAN-CN interface and RAN internal interfaces.

Other than those requirements which are quite straightforward to be inherited, some requirement worth a further check, before applying it to 6G

Table 2 5G RAN architecture requirement to be revisited, for the aggregation method of multiple spectrum/sites

| Text in TR 38.913 | Comment | Applicability to 6G |
| --- | --- | --- |
| The RAN architecture shall enable a separation of control plane signalling and user plane data from different sites. The RAN architecture shall allow for C-plane/U-plane separation. | CP / UP separation essentially is from the perspective that CP concerns more on coverage, while UP concerns more on capacity, so that the two may impose different radio requirement. | To be revisited |

Firstly, 5G highlighted CP / UP separation, yet it is unclear what is the real commercial benefit it offers to the consumer, even though there exist function(s) to enable that in 5G system design. While in our view, what brings benefit to consumers should be something that brings visible communication QoS/QoE improvement. From that perspective, CP/UP separation is more a solution without clear target / objective.

Instead, what brings real benefit to the consumer is the

- Robust connection to the network (e.g., carrying control plane data and also maybe essential user plane data like for emergency service), for which a lower frequency band is needed to secure the always-on low-bit-rate link;

- On-demand connection to the network (e.g., carrying high-data-rate traffic data for entertainment service, and/or for load re-distribution), for which temporary available low/medium/high frequency band is required.

Then eventually, what consumer needs is a RAN which is robust and always-on to carry a moderate-bit-rate connection, and also able to take additional load / higher bit-rate traffic when necessary. That is the ultimate goal of efficient usage / aggregation of spectrum.

Instead of CP/UP separation, what a consumer actually expects is a RAN which is robust / always-on to carry a moderate-bit-rate connection, and also able to take additional load / higher bit-rate traffic when necessary.

Then down to solutions, since 4G, RAN has deployed multiple operation options for such requirement (other than MIMO where aggregation happens at layer-1), i.e., 1) Carrier aggregation, where the aggregation happens at lower layer-2, and 2) (intra-RAT) Dual connectivity, where the aggregation happens at higher layer-2. Between the two,

- CA has been widely deployed since 4G, supporting both collocated and non-collocated scenario. And it is expected to achieve higher efficiency due to the potential of joint lower layer design (including scheduling, HARQ and etc.), which is especially important when used to aggregation spectrum fragment.

- While commercialization of (intra-RAT) DC is relatively limited, considering the worse performance and the additional complexity (e.g., the complex inter-node coordination design) compared to CA.

Although both CA and DC like solution act as tools for spectrum aggregation, CA shows superior performance and has been commercialized widely, with less specification effort.

Then in the sprit of minimize the adoption of multiple options for the same functionality, it is to preferred to focus on CA architecture to reach the requirement.

RP confirms 6G shall target at a RAN architecture which is able to aggregate spectrum to offer robust / always-on connection for coverage, together with on-demand connection for capacity.

Furthermore, for CA like architecture in 6G, further enhancement can be considered, i.e., one can model multiple fragment spectrum as a single cell, in order that

- System overhead can be reduced since less SS/PBCH/system information needs to be transmitted on fragmented carriers. And from a scheduling perspective, efficiency can be improved through reduced DCI overhead, less physical layer resource fragmentation caused by resource allocation, and lower Layer 2 header or padding and Layer 1 CRC overhead when larger transport blocks (TBs) are scheduled.

- It also enables initial access offloading and allows for a flexible association between downlink (DL) and uplink (UL) carriers, meaning that access can be performed on one or more carriers as needed.

- Then the number of RF chain and baseband processing unit can decrease from M to 1, M is number of fragment carrier within one band. From perspective of baseband processing, decoding number for related channels can decreases significantly, especially for PDCCH blind decoding cost.

- Transmission performance is further enhanced because a single HARQ entity shared by carriers within one cell can achieve lower latency.

A further evolved CA architecture, i.e., aggregating different spectrum as a single cell, can further improve performance by 1) reduced system-level and UE-level overhead, 2) dynamic load balance, 3) lower implementation complexity via RF sharing, 4) higher spectral efficiency via higher HARQ diversity gain.

RP confirms 6G shall target at a RAN architecture which can aggregating spectrum in a more efficient manner than 5G, in terms of further reduced overhead, more dynamic load balance, lower implementation complexity and higher spectrum efficiency.

Table 3 5G RAN architecture requirement to be revisited, for RAN internal interface

| Text in TR 38.913 | Comment | Applicability to 6G |
| --- | --- | --- |
| Different options and flexibility for splitting the RAN architecture shall be allowed. | Considering the 6G design principle: minimize the adoption of multiple options for the same functionality This requirement needs to be re-visited | To be revisited |

Currently, there are multiple options on the table

- CU/DU split

- BBU/RRU split

Where the former one has been standardized in 5G, yet the commercialization for the F1 interface is rather limited in 5G, and the function split between CU and DU has led to not only concern on latency of RRC signaling, and thus higher specification effort on MAC-CE based control, which unnecessarily complicates the system design considering the typical deployment is collocated CU/DU, but also complexity of procedure design, since CU and DU, although function-wise split, but procedure-wise are tightly coupled with each other. Note that RAN WGs has been communicated / clarified with each other to avoid misunderstanding on the usage of MAC-CE and RRC signaling.

On the other hand, (e)CPRI has been an interface that has been deployed widely in real deployment, and has less impact to the 3GPP standardized procedure. And O-RAN now is working on the related aspect(s).

CU/DU split leads to big specification effort in 5G, while the commercialization is limited.

RP confirms 6G RAN architecture shall strive for reduced option(s) for function splitting.

Requirement on migration

Table 4 Comparison of requirement on RAN architecture for 5G and 6G

| Text in TR 38.913 | Comment | Applicability to 6G |
| --- | --- | --- |
| The RAN architecture shall support tight interworking between the new RAT and LTE. | The high-level requirement is straightforward | Yes |
| Considering high performing inter-RAT mobility and aggregation of data flows via at least dual connectivity between LTE and new RAT. This shall be supported for both collocated and non-collocated site deployments. RAN-CN interfaces and RAN internal interfaces (both between new RAT logical nodes/functions and between new RAT and LTE logical nodes/functions) shall be open for multi-vendor interoperability. | Inter-RAT mobility is straightforward. While there are wide-spread negative view on the inter-RAT DC architecture. | Yes for Inter-RAT mobility To-be-revisited for inter-RAT DC (i.e., RAN-level integration) |
| The RAN architecture shall allow for the RAN and the CN to evolve independently. | Considering eLTE which has been standardized but failed to be commercialized, this requirement needs to be revisited | To be revisited |

For inter-RAT mobility, clearly it has to be supported.

RP confirms 6G shall target at a RAN architecture supporting inter-RAT mobility.

While inter-RAT mobility assumes N26-like CN level interface between 5G-6G, a further problem is whether we assume there is a need for RAN-level integration similar to ENDC type architecture. The following table gives a brief comparison between the two architectures.

Table 5 Comparison of integration with and without RAN level interface

|  | Integration without RAN-level interface | Integration with RAN-level interface |
| --- | --- | --- |
| Definition | Standalone CP + UP in both RATs. Convergence point at Core network / APP and UE. | Single CP (SRB1,2) in the anchor RAT, and UP in both RAT. Convergence point at RAN and UE |
| Network Side | New 6GC with at least the basic functionalities (Mobility Management, Session Management, Security) re-designed. No MSG/SCG role difference between 5/6G-RAN. | No new 6G core is needed. One RAN as MCG, the other RAN as SCG. |
| UE side | Simultaneous Tx or TX switching between 5G and 6G. | Simultaneous Tx or TX switching between 5G and 6G. |
| Constraint | Loose RAN integration may affect the performance, i.e., slow adaptation between the two RATs (Yet question is whether that is needed, whether that is implemented efficiently in ENDC) | Huge specification effort, implementation complexity and market split. |

Clearly, the comparison is between potentially superior performance of RAN-level interface and the huge specification, implementation complexity and market split it caused. Note that this issue is independent of CN architecture selection, which is fully up to SA work.

For the adaption of RAN-level integration, the comparison is between potentially superior performance of RAN-level interface and the huge specification, implementation complexity and market split it caused.

RP confirm 6G does not target at a RAN architecture requiring RAN-level integration between 5G and 6G.

While for the RAN, and CN evolvement, RAN should at least avoid the eLTE type architecture, while leave the CN evolvement decision to SA.

RP confirm 6G shall target at RAN evolvement compatible with CN evolvement decision by SA.

Conclusion

The proposals are summarized as follows.

Proposal 1	RP confirms that 6G shall inherit the requirement on RAN architecture including: support for MIMO operation via multi-TRP, support for existing and new services, TCO reduction, Inter-operability for RAN-CN interface and RAN internal interfaces.

Proposal 2	RP confirms 6G shall target at a RAN architecture which is able to aggregate spectrum to offer robust / always-on connection for coverage, together with on-demand connection for capacity.

Proposal 3	RP confirms 6G shall target at a RAN architecture which can aggregating spectrum in a more efficient manner than 5G, in terms of further reduced overhead, more dynamic load balance, lower implementation complexity and higher spectrum efficiency.

Proposal 4	RP confirms 6G RAN architecture shall strive for reduced option(s) for function splitting.

Proposal 5	RP confirms 6G shall target at a RAN architecture supporting inter-RAT mobility.

Proposal 6	RP confirm 6G does not target at a RAN architecture requiring RAN-level integration between 5G and 6G.

Proposal 7	RP confirm 6G shall target at RAN evolvement compatible with CN evolvement decision by SA.

References

- RP-25xxxx

Text Proposal

TP Start

5.2	Requirements for architecture and migration

Editor note: 6G RAN architecture, 5G-6G migration

The RAN design for the Next Generation Radio Access Technologies shall be designed to fulfil the following requirements:

-	The RAN architecture shall support interworking between the new 6G RAT and NR.

-	Supporting high performing inter-RAT mobility between the new 6G RAT and NR. This shall be supported for both collocated and non-collocated site deployments.

-	The RAN architecture shall support connectivity through multiple transmission points, either collocated or non-collocated.

-	The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination.

-	The RAN architecture shall be able to aggregate spectrum to offer robust connection for coverage, together with on-demand connection for capacity via same or different sites.

-	The RAN architecture shall support aggregating spectrum in a more efficient manner than 5G, in terms of further reduced overhead, more dynamic load balance, lower implementation complexity and higher spectrum efficiency.

-	Different options for splitting the RAN architecture shall be avoided.

-	The RAN architecture evolvement shall be compatible with CN archiecture decision.

-	The design of the RAN architecture shall allow the deployment of existing services (e.g. IoT and eMBB traffic) and new services (e.g., AI and sensing) rapidly and efficiently.

-	The design of the RAN architecture shall enable lower TCO with respect to current networks to achieve the same level of services.

End of TP