3GPP TSG RAN Meeting #109                                                        RP-252214

Beijing, September 15 – 18, 2025

Source:	Indian Institute of Technology (Madras), Tejas Networks, CEWiT

Title:	Views on 6G Key Performance Indicators

Agenda Item:	8.2.1.2 – Key Performance Indicators

Document for:	Discussion and Decision

Introduction

In RAN #108, a revised RAN level study was agreed [1], on 6G scenarios and requirements. Two of the important aspects of the study include:

- The investigation of a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs.

  - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN

- The identification of typical and practical deployment scenarios [2] defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

In this contribution, we present our views on:

- Possible target values for IMT 2030 TPRs

- Definition and possible evaluation methodology for the Coverage TPR

Target Values for TPRs

Our views on the target values for the IMT-2030 TPRs are shown in the table below.

Table 1: Our views on target values for IMT-2030 TPRs

| ITU IMT-2030 TPR |  | IMT-2020 Value | Our view |
| --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | 22 Gbps for an aggregated bandwidth of 400 MHz and 45 Gbps for an aggregated bandwidth of 800 MHz. The proponent shall report the bandwidth used in the calculation. |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | DL: 55 UL: 20 |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | Dense Urban-IC: DL: 150 UL: 75 – 100 Rural-UC: DL: 1-2 UL: 1-2 |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | Dense Urban-IC: DL: 0.3 UL: 0.2 - 0.3 Indoor Hotspot-IC: DL: 0.45 UL: 0.3 - 0.4 Rural-UC: DL: 0.1 UL: 0.1 |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | Dense Urban-IC: DL: 12 UL: 7-10 Indoor Hotspot-IC: DL: 13 UL: 9-12 Rural-UC: DL: 2 UL: 1 |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | 20-100 |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | IC: 2-4 HRLLC: 0.5-1 |
|  | Control plane latency (ms) | 20 | 10-20 |
| Connection density (devices/km²) |  | 10⁶ | 0.5 x 107 - 1 x 107 |
| Reliability |  | - 10⁻⁵ | 1-10−5 to 1-10−6 |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | Stationary: 0 km/h Pedestrian: 0-10 km/h Vehicular: TBD 10-120 km/h High speed vehicular: TBD 120-500 km/h. |
| Mobility interruption time (ms) |  | 0 | 0 ms |
| Bandwidth (MHz) |  | 100 | At least 200 MHz (As per our present understanding, Indian regulators are unlikely to allocate 400 MHz per operator in the 7GHz band) |
| Coverage (Maximum Link Distance) |  | NA | 3-8 km for 2-3 Mbps data rate |
| Positioning |  | NA | Indoor Factory ‒ HRLLC: Horizontal accuracy (m): 0.1-3 Vertical accuracy (m): 0.1-3 Dense Urban-IC: Horizontal accuracy (m): 1-5 Vertical accuracy (m): 1-3 |

Coverage/Maximum Link Distance – TPR Definition and Evaluation Steps

Our goal is increased and improved terrestrial coverage with larger cells. We envision terrestrial coverage to be an important component of Ubiquitous Connectivity, and it should evolve with every new generation. In IMT-2030, TN will be complemented by NTN wherever terrestrial coverage is missing. TN is expected to provide coverage to 95% of the population and with a higher user experienced data rate and cell edge data rate than NTN.

While NTN can be a big driver for serving under connected areas where there is not much habitation of people, we strongly believe that enhanced terrestrial coverage is still necessary to serve under connected areas which do have significant habitation of people (example: rural areas including villages and small towns such as those in the Indian subcontinent). NTN is not expected to handle the very large population density seen in rural environments of developing countries including India, and is also not expected to provide the above-mentioned user experienced data rate and cell edge data rate.

One type of deployment that we envision includes sparse deployment of large cells without contiguous multi-cell coverage. We believe that this type of deployment is to offer broadband services and is therefore best suited for a new test environment, which we call Rural-UC. The evaluation parameters should be revisited (compared to Rural-eMBB in IMT-2020) to include different UE heights and different UE power classes.

This test environment can be used to evaluate the coverage TPR, which we define as follows:

Figure 1: Rural-UC test environment covering multiple small villages with a large standalone cell.

Coverage/Maximum Link Distance is defined as the cell edge distance at which a minimum data rate of [2-3] Mbps is achieved. It is evaluated by single cell link budget analysis.

This requirement is defined for the purpose of evaluation in the Ubiquitous Connectivity Usage Scenario.

This requirement is defined for the purpose of evaluation in the Rural-UC test environment.

| Test Environment | Cell Edge Distance |
| --- | --- |
| Rural-UC | 3-8 km for a single cell |

Evaluation methodology for the Coverage TPR:

Step 1:	Run link level simulations for downlink and uplink data channels with the given configuration as per section 8.4.X. Obtain the SNR that corresponds to a minimum data rate of [2-3] Mbps.

Step 2:	Perform a link budget analysis for uplink and downlink using the SNR obtained from Step 1, along with other parameters specified in section 8.4.X. Obtain the MCL values from the link budget.

Step 3:	Calculate the corresponding distance for NLOS, for the associated channel model for this test environment.

Detailed evaluation results are provided in our companion contribution RP-252118 [3].

Conclusion

Proposal 1: Consider the target values presented in column 3 of Table 1, towards the discussion on IMT-2030 TPR values.

Proposal 2: For peak data rate calculations, the RIT proponent shall report the bandwidth used in the calculation.

Proposal 3: For test environments related to Ubiquitous Connectivity, lower values compared to IMT-2020 (Rural-eMBB) should be considered for the following TPRs: User Experienced Data Rate, Avg Spectral Efficiency and 5th Percentile Spectral Efficiency.

Proposal 4: For the Ubiquitous Connectivity usage scenario, the study should consider coverage (maximum link distance), user experienced data rate, 5th percentile and average spectral efficiency as TPRs. A new test environment should be defined for Ubiquitous Connectivity which includes at least the Rural geographical area, with an emphasis on large cells.

Proposal 5: Single cell rural deployments should be included in a new test environment called Rural-UC for the evaluation of TPRs related to Ubiquitous Connectivity. The Rural-UC test environment should include the sparse deployment of large cells without contiguous multi-cell coverage, as shown in Figure 1.

Proposal 6: The evaluation parameters for the Rural-UC test environment should include different UE types. For example, a CPE type device with 5m height and 31dBm power class should also be considered in addition to the 1.5m handheld UE.

Proposal 7: The evaluation of coverage (maximum link distance) in Rural-UC can done with the following steps:

Step 1:	Run link level simulations for downlink and uplink data channels with the given configuration as per section 8.4.X. Obtain the SNR that corresponds to a minimum data rate of [2-3] Mbps.

Step 2:	Perform a link budget analysis for uplink and downlink using the SNR obtained from Step 1, along with other parameters specified in section 8.4.X. Obtain the MCL values from the link budget.

Step 3:	Calculate the corresponding distance for NLOS, for the associated channel model for this test environment.

References

[1] RP-251395 Revised SID: Study on 6G scenarios and requirements

[2] TR 38.914 Study on 6G Scenarios and Requirements, Release 20, 2025.

[3] RP-252118 Discussion on 6G Deployment Scenarios, RAN #109, Beijing, September 2025