3GPP TSG-RAN5 Meeting #108	R5-253731

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0617 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Test applicability for NR NTN RA TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc.,Qualcomm Technologies International, Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-07-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test applicability of NR NTN RA TCs are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test applicability for NR NTN RA TCs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WP can’t be finished. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.533 CR 4008 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

## 4.2	RRM conformance test cases

<<< skip unchanged part>>>

Table 4.2-14: Applicability of NR-NTN SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Subtest Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 14.1 | RRC_IDLE state mobility |  |  |  |  |  |  |
| 14.1.1 | NR SA FR1 Cell Reselection for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.1.2 | NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access | Rel-17 | C332l | UE supporting FDD FR1 satellite access and enhanced requirements for measurements in IDLE mode | NOTE 1 |  |  |
| 14.1.3 | NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access | Rel-17 | C332j | UE supporting FDD FR1 satellite access and time-based measurement initiation | NOTE 1 |  |  |
| 14.1.4 | NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access | Rel-17 | C332k | UE supporting FDD FR1 satellite access and location-based measurement initiation | NOTE 1 |  |  |
| 14.1.5 | NR SA FR1-FR1 Cell Reselection for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.1.6 | NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access | Rel-17 | C332l | UE supporting FDD FR1 satellite access and enhanced requirements for measurements in IDLE mode | NOTE 1 |  |  |
| 14.1.7 | NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access | Rel-17 | C332j | UE supporting FDD FR1 satellite access and time-based measurement initiation | NOTE 1 |  |  |
| 14.1.8 | NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access | Rel-17 | C332k | UE supporting FDD FR1 satellite access and location-based measurement initiation | NOTE 1 |  |  |
| 14.1.9 | NR SA FR1-FR1 Cell reselection for UE fulfilling low mobility relaxed measurement criterion for Satellite Access | Rel-17 | C332m | UE supporting FDD FR1 satellite access and relaxed RRM measurements in IDLE mode |  |  |  |
| 14.1.10 | NR SA FR1-FR1 Cell reselection for UE fulfilling not-at-cell edge relaxed measurement criterion for Satellite Access | Rel-17 | C332m | UE supporting FDD FR1 satellite access and relaxed RRM measurements in IDLE mode |  |  |  |
| 14.2 | RRC_CONNECTED state mobility |  |  |  |  |  |  |
| 14.2.1 | Handover |  |  |  |  |  |  |
| 14.2.1.1 | NR SA FR1 Handover for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.2.1.2 | NR SA FR1-FR1 Handover for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.2.1.3 | NR SA FR1 Time-based Conditional Handover for NR Satellite Access | Rel-17 | C332h | UE supporting FDD FR1 satellite access and time-based conditional handover | NOTE 1 |  |  |
| 14.2.1.4 | SA FR1-FR1 SAN time-based conditional Handover for NR satellite access | Rel-17 | C332h | UE supporting FDD FR1 satellite access and time-based conditional handover | NOTE 1 |  |  |
| 14.2.1.5 | SA FR1 SAN distance-based conditional Handover for NR satellite access | Rel-17 | C332i | UE supporting FDD FR1 satellite access, distance-based conditional handover, event D1 measurement trigger and inter-satellite measurements | NOTE 1 |  |  |
| 14.2.1.6 | SA FR1-FR1 SAN distance-based conditional Handover for NR satellite access | Rel-17 | C332i | UE supporting FDD FR1 satellite access, distance based conditional handover, event D1 measurement trigger and inter-satellite measurements | NOTE 1 |  |  |
| 14.2.2 | RRC Connection Mobility Control |  |  |  |  |  |  |
| 14.2.2.1 | RRC Re-establishment for SAN |  |  |  |  |  |  |
| 14.2.2.1.1 | NR SA FR1 RRC Re-establishment for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.2.2.1.2 | NR SA FR1-FR1 RRC Re-establishment for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.2.2.2 | Random Access for SAN |  |  |  |  |  |  |
| 14.2.2.2.1 | NR SA FR1 4-step contention based random access for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  |  |
| 14.2.2.2.2 | NR SA FR1 4-step non-contention based random access for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  | Subtest 2: F001 |
| 14.3 | Timing for Satellite Access |  |  |  |  |  |  |
| 14.3.1 | UE transmit timing for Satellite Access |  |  |  |  |  |  |
| 14.3.1.1 | NR SA FR1 UE transmit timing accuracy for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  | Subtest 2: F002 |
| 14.3.2 | UE transmit timing for Satellite Access |  |  |  |  |  |  |
| 14.3.2.1 | NR SA FR1 timing advance adjustment accuracy for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.4 | Signalling characteristics |  |  |  |  |  |  |
| 14.4.1 | Radio Link Monitoring for SAN |  |  |  |  |  |  |
| 14.4.1.1 | NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  |  |
| 14.4.1.2 | NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in non-DRX mode for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  |  |
| 14.4.1.3 | NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access | Rel-17 | C332a | UE supporting FDD FR1 satellite access and long DRX cycle |  |  |  |
| 14.4.1.4 | NR SA FR1 Radio Link Monitoring In-sync for PCell configured with SSB-based RLM RS in DRX mode for Satellite Access | Rel-17 | C332a | UE supporting FDD FR1 satellite access and long DRX cycle |  |  |  |
| 14.4.1.5 | NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access | Rel-17 | C332b | UE supporting FDD FR1 satellite access and CSI-RS based RLM |  |  |  |
| 14.4.1.6 | NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in non-DRX mode for Satellite Access | Rel-17 | C332b | UE supporting FDD FR1 satellite access and CSI-RS based RLM |  |  |  |
| 14.4.1.7 | NR SA FR1 Radio Link Monitoring Out-of-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access | Rel-17 | C332c | UE supporting FDD FR1 satellite access and CSI-RS based RLM and long DRX cycle |  |  |  |
| 14.4.1.8 | NR SA FR1 Radio Link Monitoring In-sync for PCell configured with CSI-RS-based RLM RS in DRX mode for Satellite Access | Rel-17 | C332c | UE supporting FDD FR1 satellite access and CSI-RS based RLM and long DRX cycle |  |  |  |
| 14.4.2 | Beam Failure Detection and Link recovery procedures for SAN |  |  |  |  |  |  |
| 14.4.2.1 | NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in non-DRX mode for Satellite Access | Rel-17 | C332d | UE supporting FDD FR1 satellite access and beam failure and link recovery | NOTE 1 |  |  |
| 14.4.2.2 | NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with SSB-based BFD and LR in DRX mode for Satellite Access | Rel-17 | C332e | UE supporting FDD FR1 satellite access and beam failure and link recovery and long DRX cycle | NOTE 1 |  |  |
| 14.4.2.3 | NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in non-DRX mode for Satellite Access | Rel-17 | C332f | UE supporting FDD FR1 satellite access and CSI-RS based beam failure and link recovery | NOTE 1 |  |  |
| 14.4.2.4 | NR SA FR1 Beam Failure Detection and Link Recovery Test for PCell configured with CSI-RS-based BFD and LR in DRX mode for Satellite Access | Rel-17 | C332g | UE supporting FDD FR1 satellite access and CSI-RS based beam failure and link recovery and long CDRX cycle | NOTE 1 |  |  |
| 14.4.3 | Active BWP Switch for SAN |  |  |  |  |  |  |
| 14.4.3.1.1 | NR SA FR1 DCI-based DL active BWP switch with non-DRX for Satellite Access | Rel-17 | C332v | UE supporting FDD FR1 satellite access and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) | NOTE 1 |  |  |
| 14.4.3.2 | RRC-based Active BWP Switch |  |  |  |  |  |  |
| 14.4.3.2.1 | NR SA FR1 RRC-based DL active BWP switch with non-DRX for Satellite Access | Rel-17 | C332w | UE supporting FDD FR1 satellite access and (Support of BWP adaptation upto2 or upto4) | NOTE 1 |  |  |
| 14.4.4 | UE specific CBW change for SAN |  |  |  |  |  |  |
| 14.4.4.1 | NR SA FR1 UE specific CBW change on PCell in non-DRX for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.4.5 | Pathloss reference signal switching delay for SAN |  |  |  |  |  |  |
| 14.4.5.1 | NR SA FR1 MAC-CE based pathloss reference signal switch delay for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.5 | Measurement procedure |  |  |  |  |  |  |
| 14.5.1 | Intra-frequency Measurements |  |  |  |  |  |  |
| 14.5.1.1 | SA FR1 event triggered reporting tests without gap under non-DRX for NR satellite access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  |  |
| 14.5.1.2 | SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access | Rel-17 | C332a | UE supporting FDD FR1 satellite access and long DRX cycle |  |  |  |
| 14.5.1.3 | SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  |  |
| 14.5.1.4 | SA FR1 event triggered reporting tests with single measurement gap under non-DRX for satellite access for NR satellite access | Rel-17 | C332b | UE supporting FDD FR1 satellite access and CSI-RS based RLM |  |  |  |
| 14.5.1.5 | SA FR1 event triggered reporting tests with FNO concurrent gaps under DRX for satellite access for NR satellite access | Rel-17 | C332t | UE supporting FDD FR1 satellite access, long DRX cycle, 2 parallel measurement gaps and CSI-RS based RLM |  |  |  |
| 14.5.1.6 | SA FR1 event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access for NR satellite access | Rel-17 | C332s | UE supporting FDD FR1 satellite access, 2 parallel measurement gaps and CSI-RS based RLM |  |  |  |
| 14.5.2 | Inter-frequency Measurements |  |  |  |  |  |  |
| 14.5.2.1 | NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  |  |
| 14.5.2.2 | NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access | Rel-17 | C332a | UE supporting FDD FR1 satellite access and long DRX cycle |  |  |  |
| 14.5.2.3 | NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  |  |
| 14.5.2.4 | NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access | Rel-17 | C332u | UE supporting FDD FR1 satellite access and 2 parallel measurement gaps |  |  |  |
| 14.5.2.5 | Void |  |  |  |  |  |  |
| 14.5.2.6 | NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access | Rel-17 | C332u | UE supporting FDD FR1 satellite access and 2 parallel measurement gaps |  |  |  |
| 14.5.3 | L1-RSRP measurement for beam reporting for SAN |  |  |  |  |  |  |
| 14.5.3.1 | NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.5.3.2 | NR SA FR1 SSB based L1-RSRP measurement when DRX is used for Satellite Access | Rel-17 | C332a | UE supporting FDD FR1 satellite access and long DRX cycle | NOTE 1 |  |  |
| 14.6 | Measurement Performance requirements |  |  |  |  |  |  |
| 14.6.1 | SS-RSRP for SAN |  |  |  |  |  |  |
| 14.6.1.1 | NR SA FR1 SS-RSRP measurement accuracy for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.6.1.2 | NR SA FR1-FR1 SS-RSRP measurement accuracy for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.6.2 | SS-RSRQ for SAN |  |  |  |  |  |  |
| 14.6.2.1 | NR SA FR1 SS-RSRQ measurement accuracy for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.6.2.2 | NR SA FR1-FR1 SS-RSRQ measurement accuracy for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access | NOTE 1 |  |  |
| 14.6.3 | SS-SINR |  |  |  |  |  |  |
| 14.6.3.1 | NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access | Rel-17 | C332p | UE supporting FDD FR1 satellite access and SS-SINR measurements |  |  |  |
| 14.6.3.2 | NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access | Rel-17 | C332p | UE supporting FDD FR1 satellite access and SS-SINR measurements |  |  |  |
| 14.6.4 | L1-RSRP measurement for beam reporting |  |  |  |  |  |  |
| 14.6.4.1 | NR SA FR1 SSB based L1-RSRP Measurement Accuracy for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  |  |
| 14.6.4.2 | NR SA FR1 CSI-RS based L1-RSRP Measurement Accuracy for Satellite Access | Rel-17 | C332 | UE supporting FDD FR1 satellite access |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. |  |  |  |  |  |  |  |

<<< END OF CHANGES 1>>>