3GPP TSG-RAN5 Meeting #108	R5-253816

# Bengaluru, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4010 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR-NTN Test Case SS-RSRP 14.6.1.1 Including TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RRM NR-NTN SS-RSRP test case 14.6.1.1 is incomplete in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been corrected, Test Tolerance has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TT will not be recorded. WP cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 1043 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### 14.6.1.1	NR SA FR1 SS-RSRP measurement accuracy for Satellite Access

Editor’s note: This test case is incomplete in following aspects:

- Message contents may need to be updated.

- Call setup and test procedure may need to be updated.

- Test requirements contain parameters in square brackets.

14.6.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

14.6.1.1.2	Test applicability

This test applies to all types of NR UE from Release 17 and onwards supporting satellite access.

14.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 14.6.1.0.1 and 14.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.14.6.1.1.

14.6.1.1.4	Test description

Supported test configurations are shown in table 14.6.1.1.4.1-1. Both absolute and relative accuracy of SS-RSRP intra-frequency measurements are tested by using the parameters in 14.6.1.1.5-1. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell

14.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.6.1.1.4.1-1.

Table 14.6.1.1.4.1-1: Supported test configurations for NR SA FR1 SS-RSRP measurement accuracy for Satellite Access

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.6.1.1.4.1-2.

Table 14.6.1.1.4.1-2: Initial conditions for NR SA FR1 SS-RSRP measurement accuracy for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table [E.12-1] and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. [4Rx support is FFS, diagram may need to be removed] |
|  | [TE Part 4Rx] | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | [DUT Part 4Rx] | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. The general test parameter settings are set up according to Table 14.6.1.1.5-1

2. Message contents are defined in clause 14.6.1.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

4.	The initial test environment conditions are setup according to section 14.0.5.

14.6.1.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 14.6.1.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1 configuring the UE to perform intra-frequency measurements as specified in section 14.6.1.1.4.3.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Tables 14.6.1.1.5-2 and 14.6.1.1.5-3 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 14.6.1.1.5-1 as appropriate and repeat steps 5-7.

14.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 14.6.1.1.4.3-1: Common Exception messages for NR SA FR1 SS-RSRP measurement accuracy for Satellite Access

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition SSB.1 FR1 Table H.3.1-5 Table H.3.1-7 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |

Table 14.6.1.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRP measurement accuracy for Satellite Access

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

14.6.1.1.5	Test requirement

Table 14.6.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 14.6.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 14.6.1.1.5-2 and the relative accuracy requirements in Table 14.6.1.1.5-3, correspondingly.

Table 14.6.1.1.5-1: Cell specific test parameters for NR SA FR1 SS-RSRP measurement accuracy for Satellite Access

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |  |  | Test 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |  |  | Cell 1 |  |  | Cell 2 |  |  |
| Cell ID |  |  |  | 489 | 0 |  | 489 |  | 0 |  |  |  | 489 |  | 0 |  |  |  |
| SSB ARFCN |  |  |  | freq1 |  |  | freq1 |  |  |  |  |  | freq1 |  |  |  |  |  |
| BWchannel |  | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1,2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1,2 |  | TRS.1.1 FDD |  | NA |  | TRS.1.1 FDD |  |  | NA | TRS.1.1 FDD |  |  |  |  |  | NA |
| DRX Cycle |  | Config 1,2 | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,2 |  | SR.1.1 FDD |  | - | SR.1.1 FDD |  | - |  |  |  | SR.1.1 FDD |  |  |  |  | - |
| RMSI CORESET Reference Channel |  | Config 1,2 |  | CR.1.1 FDD |  | - | CR.1.1 FDD |  | - |  |  |  | CR.1.1 FDD |  |  |  |  | - |
| Control channel RMC |  | Config 1,2 |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD |  | - |  |  |  | CCR.1.1 FDD |  |  |  |  | - |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  | SSB.1 FR1 | SSB.1 FR1 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |
| Time offset with Cell 1 |  | Config 1,2 | ms | - |  | 3 | - |  | 3 |  |  |  | - |  |  |  | 3 |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 | 0 |  |  | 0 |  |  |  | 0 |  |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A | dBm/15Khz | -107.55 |  |  | -88 |  |  |  |  |  |  | -114 |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -107.55 |  |  | -88 |  |  |  |  |  |  | Same as Noc/15kHz |  |  |  |  |
|  |  |  | dB | 2.23 |  | -5.57 | 2.23 |  |  | -5.57 |  |  |  | -0.01 |  |  |  | -4.76 |
|  |  |  | dB | 6 |  | 1.40 | 6 |  |  | 1.40 |  |  |  | 3 |  |  |  | 0 |
| SS-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/SCS | -101.55 |  | -106.15 | -82 |  |  | -86.60 |  |  |  | -111.00 |  |  |  | -114.00 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A | dBm/ 9.36MHz | -71.56 |  |  | -52.01 |  |  |  |  |  |  | -80.03 |  |  |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 14.6.1.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |  |
| --- | --- | --- | --- | --- |
|  | All bands | All bands | All bands |  |
| Normal Conditions |  |  |  |  |
| Lowest reported value (Cell 2) | 44 | 60 | 36 |  |
| Highest reported value (Cell 2) | 56 | 79 | 49 |  |
| Extreme Conditions |  |  |  |  |
| Lowest reported value (Cell 2) | 40 | 57 | 32 |  |
| Highest reported value (Cell 2) | 61 | 82 | 53 |  |

Table 14.6.1.1.5-3: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRP_x - 9 | SS-RSRP_x - 9 | SS-RSRP_x - 7 |
| Highest reported value (Cell 2) | SS-RSRP_x - 2 | SS-RSRP_x - 2 | SS-RSRP_x + 1 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRP_x - 9 | SS-RSRP_x - 9 | SS-RSRP_x - 7 |
| Highest reported value (Cell 2) | SS-RSRP_x - 2 | SS-RSRP_x - 2 | SS-RSRP_x + 1 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

< End of changes >