3GPP TSG-RAN5 Meeting #108	R5-253977

# Bangalore, India, 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0015 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction of clause 5.4.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial issues remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.4.4.3	NR/5GC signalling

5.4.4.3.1	Initial conditions

As specified in clause 5.4.4.1.1 with the following clarifications:

- An MCX PDU session with default QoS rule #10 (5QI 69) according to table 4.8.2.1-10 in TS 38.508-1 [132] is established for MCX and SIP signalling.

NOTE 1:	The assumptions for the PDU session support, including the default QoS flow 5QI requirements in regard to the different PDU session are described in 5.4.1B.

5.4.4.3.2	Definition of system information messages

The NR/5GC default system information messages as defined in TS 38.508-1 [132] are used.

5.4.4.3.3	Procedure

Table 5.4.4.3.3-1: NR/5GC signalling for MCX CT communication

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging |
| 2-8 | Steps 2-8 of the RRC establishment procedure in TS 38.508-1 [132] Table 4.5.4.2-3 take place. | - | - |
| - | EXCEPTION: Steps 9a1-9a3 describe behaviour that depends on the context in which the procedure is used: The steps take place when the procedure is used for MCPTT or MCVideo call establishment and MCData communication establishment for using the media plane. | - | - |
| 9a1 | The SS configures a new RLC-UM data radio bearer, associated with the QoS flow to be added to the MCX PDU session and transmits an RRCReconfiguration message containing a PDU SESSION MODIFICATION COMMAND accordingly. | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND |
| - | EXCEPTION: Depending upon UE implementation steps 8a2 and 8a3 can occur in any order. | - | - |
| 9a2 | The UE transmits an RRCReconfigurationComplete message to confirm the establishment of the data radio bearer associated with the added QoS flow. | --> | NR RRC: RRCReconfigurationComplete |
| 9a3 | The UE transmits a PDU SESSION MODIFICATION COMPLETE message. | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE |

5.4.4.3.4	Specific message contents

All specific NR/5GC signalling message contents shall be referred to TS 38.508-1 [132] clause 4.6 and 4.7 7 with the following clarifications:

-	The SERVICE REQUEST message at step 4 of Table 5.4.4.3.3-1 is expected to have the service type IE set to "mobile terminated services".

-	The RRCReconfiguration message at step 9a1 adds a single RLC-UM non-default DRB (configuration DRB(0,1)) in RRC messages and information elements of TS 38.508-1 [132]).

-	PDU SESSION MODIFICATION COMMAND message at step 9a1 adds a non-default QoS flow to the MCX PDU session as shown in Table 5.4.4.3.4-1.

Table 5.4.4.3.4-1: PDU SESSION MODIFICATION COMMAND (step 8a1, Table 5.4.4.3.3-1)

| Derivation Path: TS 38.508-1 [132] Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as in PDU SESSION ESTABLISHMENT REQUEST message at establishment of the MCX PDU session |  |  |
| Authorized QoS rules |  | Reference QoS rule #11 as defined in TS 38.508-1 [132] Table 4.8.2.1-7 using condition MCPTT, MCVIDEO or MCDATA (NOTE 1) |  |  |
| Authorized QoS flow descriptions |  | Reference QoS flow #10 as defined in TS 38.508-1 [132] Table 4.8.2.3-5 using condition MCPTT, MCVIDEO or MCDATA (NOTE 1) |  |  |
| NOTE 1: Depending on the MC service (condition MCPTT, MCVIDEO or MCDATA) the QoS flow uses MCPTT: QoS rule id=10, QFI=12 and 5QI = 65 MCVideo: QoS rule id=11, QFI=13 and 5QI = 67 MCData: QoS rule id=12, QFI=14 and 5QI = 70 |  |  |  |  |