3GPP TSG-RAN5 Meeting #108	R5-255076

Bengaluru, India, August 25-29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-7 | CR | 0006 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.9.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet, Ericsson, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case ‘6.9.4 On-network / Regroup using a preconfigured group / Creating a user regroup using preconfigured group / Removing a user regroup using preconfigured group / Client Terminated (CT)’ included in the Work Plan, is not implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case ‘6.9.4 On-network / Regroup using a preconfigured group / Creating a user regroup using preconfigured group / Removing a user regroup using preconfigured group / Client Terminated (CT)’ according to the Work Plan. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case ‘6.9.4 On-network / Regroup using a preconfigured group / Creating a user regroup using preconfigured group / Removing a user regroup using preconfigured group / Client Terminated (CT)’ included in the Work Plan will not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9 and 6.9.4 (new clauses) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 37.579-1 CR 0018 TS 37.579-4 CR 0005 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final version of R5-254191 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

Table 6.8.4.3.3-6: HTTP DELETE from the UE (Step 7, Table 6.8.4.3.2-1; step 2, TS 37.579-1 [2] Table 5.3C.15.2-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.4.5-1, condition MSG_STORE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_MsgSF_BaseURL & "subscriptions/subscriptId0001" | The subscription to be deleted |  |  |

## 6.9	Group regroup using a preconfigured group

## 6.9.4	On-network / Regroup using a preconfigured group / Creating a user regroup using preconfigured group / Removing a user regroup using preconfigured group / Client Terminated (CT)

6.9.4.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorised for MCData Service }

ensure that {

when { UE (MCData Client) receives a request via a SIP MESSAGE message to create an MCData user regroup using a preconfigured group }

then { UE (MCData Client) responds with a SIP 200 (OK) message }

}

(2)

with { UE (MCData Client) having established an MCData user regroup using a preconfigured group }

ensure that {

when { UE (MCData Client) receives a request to establish an MCData On-demand Pre-arranged Group Call using the user regroup }

then { UE (MCData Client) sends a SIP INVITE message using the user regroup to establish an MCData On-demand Pre-arranged Group Call }

}

(3)

with { UE (MCData Client) having an ongoing On-demand Pre-arranged Group Call }

ensure that {

when { MCData User requests to terminate the ongoing MCData Group Call }

then { UE (MCData Client) sends a SIP BYE message and leaves the MCData session }

}

(4)

with { UE (MCData Client) having an affiliation with an established MCData user regroup }

ensure that {

when { UE (MCData Client) receives a request to remove the user regroup via a SIP MESSAGE message }

then { UE (MCData Client) responds with a SIP 200 (OK) message }            }

6.9.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.282 clauses 23.3.1.3, 23.2.1.3, 23.3.1.4, 23.2.1.4. Unless otherwise stated these are Rel-17 requirements.

[TS 24.282 clause 23.3.1.3]

The procedure in clause 23.2.1.3 is used by the MCData client when the MCData server notifies the MCData client of the creation of a user regroup using preconfigured group.

[TS 24.282 clause 23.2.1.3]

Upon receiving a "SIP MESSAGE request to the MCData client to request creation of a regroup using preconfigured group", the MCData client:

1)	should notify the MCData user of the creation of the regroup using preconfigured group;

2)	shall send a 200 (OK) response to the MCData server according to 3GPP TS 24.229 [5];

3)	in the application/vnd.3gpp.mcdata-regroup+xml MIME body contained in the incoming SIP MESSAGE request:

a)	if a <users-for-regroup> element is included in that MIME body, shall store the value of the <mcdata-regroup-uri> element as the temporary group identity and associate that with the group identity received in the <mcdata-regroup-uri> element, along with the information that the created regroup is a user regroup and should store the contents of the <users-for-regroup> element as the list of the users that are part of that user regroup: or

b)	if a <groups-for-regroup> element is included in that MIME body, shall store the value of the <mcdata-regroup-uri> element as the temporary group identity and associate that with the group identity received in the <mcdata-regroup-uri> element, along with the information that the created regroup is a group regroup and should store the contents of the <groups-for-regroup> element as the list of groups that are part of that group regroup:

4)	shall consider that the MCData Client is affiliated with the regroup;

5)	should not initiate calls targeting any of the constituent groups but instead target the regroup for the duration of a group regroup; and

6)	if the regroup is a chat group, the MCData client should join the regroup when this notification of creation is received.

[TS 24.282 clause 23.3.1.4]

The procedure in clause 23.2.1.4 is used by the MCData client when the MCData server notifies the MCData client of the removal of a user regroup using preconfigured group.

[TS 24.282 clause 23.2.1.4]

Upon receiving a "SIP MESSAGE request to the MCData client to request removal of a regroup using preconfigured group", the MCData client:

1)	should notify the MCData user of the removal of the regroup using preconfigured group;

2)	shall send a 200 (OK) response to the MCData server according to 3GPP TS 24.229 [5]; and

3)	shall consider that the MCData client is de-affiliated from the regroup.

6.9.4.3	Test description

6.9.4.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.9.4.3.2	Test procedure sequence

Table 6.9.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Procedure 'MCX CO communication' as described in TS 37.579-1 [2] clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 1 | Make the UE (MCData client) request creation of a new group (group B). (NOTE 1) | - | - | - | - |
| 2 | The UE (MCData client) correctly performs procedure 'MCX CO Group Creation' as described in TS 37.579-1 [2] Table 5.3.26.3-1 to create group B. | - | - | - | - |
| 3 | The procedure 'MCX communication release' as described in TS 37.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 4 | Check: Does the UE (MCData client) correctly perform procedure ‘MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to establish an MCData user regroup using a preconfigured group? | - | - | 1 | P |
| 5 | Make the UE (MCData client) request the establishment of a SDS group session using the user regroup. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCData client) correctly perform procedure ' CO MCData Call Establishment ' as described in TS 37.579-1 [2] Table 5.3C.2.3-1 to establish a SDS group session using the user regroup? | - | - | 2 | P |
| 7 | Make the UE (MCData client) release the call. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCData client) correctly perform procedure ' CO MCData call release ' as described in TS 37.579-1 [2] Table 5.3C.6.3-1? | - | - | 3 | P |
| 9 | Check: Does the UE (MCData client) correctly perform procedure ‘MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to remove the user regroup? | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.9.4.3.3	Specific message contents

Table 6.9.4.3.3-1: HTTP PUT (step 2, Table 6.9.4.3.2-1;
step 1, TS 37.579-1 [2], Table 5.3.26.3-1)

Derivation Path: TS 37.579-1 [2], Table 5.5.4.4-1, condition GROUPCREATE

Table 6.9.4.3.3-2: SIP MESSAGE from the SS (Step 4, Table 6.9.4.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_B |  | TS 24.282 [31] clause 23.2.1.3 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.9.4.3.3-3 |  | TS 24.282 [31] clause 23.2.1.3 |  |
| MIME body part |  | MCData-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcdata-regroup+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCData-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCData-Regroup as described in Table 6.9.4.3.3-4 |  | TS 24.282 [31] clause 23.2.1.3 |  |

Table 6.9.4.3.3-3: MCData-Info in SIP MESSAGE (Table 6.9.4.3.3-2)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | not present |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| mcdata-client-id | Encrypted <mcdata-client-id> with mcdataString set to px_MCX_Client_B_ID | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-3 | TS 24.282 [31] clause 23.2.1.3 |  |

Table 6.9.4.3.3-4: MCData-Regroup in SIP MESSAGE (Table 6.9.4.3.3-2)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.3-3, condition USER_REGROUP

Table 6.9.4.3.3-5: SIP INVITE from the UE (step 6, Table 6.9.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP Message as described in Table 6.9.4.3.3-6 |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6. 9. 4.3.3-7 |  |  |  |

Table 6.9.4.3.3-6: SDP in SIP INVITE (Table 6.9.4.3.3-5)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.1-3, condition INITIAL_SDP_OFFER, MCDATA_SDS, SDP_OFFER, SDS_SESSION

Table 6.9.4.3.3-7: MCData-Info in SIP INVITE (Table 6.9.4.3.3-5)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | Encrypted <mcdata-request-uri> with mcdataURI set to px_MCData_Group_T_ID | Encryption according to NOTE 1 and NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-3 |  |  |

Table 6.9.4.3.3-8: SIP 200 (OK) from the SS (step 6, Table 6.9.4.3.2-1; 
step 4, TS 37.579-1 [2] Table 5.3C.2.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.9.4.3.3-9 |  |  |  |

Table 6.9.4.3.3-9: SDP in SIP 200 (OK) (Table 6.9.4.3.3-8)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.2-3, condition SDP_ANSWER, SDS_SESSION, MCDATA_SDS

Table 6.9.4.3.3-10: SIP MESSAGE from the SS (Step 9, Table 6.9.4.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_B |  | TS 24.282 [31] clause 23.2.1.4 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.9.4.3.3-3 |  | TS 24.282 [31] clause 23.2.1.4 |  |
| MIME body part |  | MCData-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcdata-regroup+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCData-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCData-Regroup as described in Table 6.9.4.3.3-11 |  | TS 24.282 [31] clause 23.2.1.4 |  |

Table 6.9.4.3.3-11: MCData-Regroup in SIP MESSAGE (Table 6.9.4.3.3-10)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.3-2, condition REMOVE