3GPP TSG-RAN5 Meeting #108	R5-254553

# Bengaluru, India, Aug.25th – 29rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4088 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ATG L1-SINR accuracy RRM test cases 19.6.5.x with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To update following ATG RRM test cases based on TT analysis results. - 19.6.5.1.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG - 19.6.5.1.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG - 19.6.5.2 NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG - 19.6.5.3.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG - 19.6.5.3.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor’s notes are removed. - Annex F is updated based on TT analysis results. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.6.5.1.1, 19.6.5.1.2, 19.6.5.2, 19.6.5.3.1, 19.6.5.3.2, F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 1075 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT Associated 38.903 CR: R5-254555 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

19.6.5.1.1	NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG

19.6.5.1.1.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

19.6.5.1.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.1.

19.6.5.1.1.4	Test description

19.6.5.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.1.1.4.1 with following exceptions:

- During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

- During the test, the specific gNB reference location is emulated by test system.

- Table 6.7.9.1.1.4.1-1 is replaced by Table 19.6.5.1.1.4.1-1;

- Table 6.7.9.1.1.4.1-2 is replaced by Table 19.6.5.1.1.4.1-2;

Table 19.6.5.1.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 19.6.5.1.1-1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.5.1.1-2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.5.1.1-3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 19.6.5.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.5.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

19.6.5.1.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.9.1.1.4.2 with following exceptions:

- Table 6.7.9.1.1.5-1 is replaced by Table 19.6.5.1.1.5-1.

19.6.5.1.1.4.3	Message contents

Same as the message contents given in clause 6.7.9.1.1.4.3.

19.6.5.1.1.5	Test requirement

Same as the test requirements given in clause 6.7.9.1.1.5 with following exceptions:

- Table 6.7.9.1.1.5-1 is replaced by Table 19.6.5.1.1.5-1.

Table 19.6.5.1.1.5-1: NR Cell specific test parameters (Same as Table 6.7.9.1.1.5-1 with following exceptions)

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Propagation condition | 1, 2 |  | AWGN+220 Hz | AWGN+220 Hz |
|  | 3 |  | AWGN+500 Hz | AWGN+500 Hz |

19.6.5.1.2	NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG

19.6.5.1.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR relative measurement accuracy is within the specified limits.

19.6.5.1.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.1.

19.6.5.1.2.4	Test description

19.6.5.1.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.1.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.9.1.2.4.1-1 is replaced by Table 19.6.5.1.2.4.1-1;

-	Table 6.7.9.1.2.4.1-2 is replaced by Table 19.6.5.1.2.4.1-2;

Table 19.6.5.1.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 19.6.5.1.2-1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.5.1.2-2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.5.1.2-3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 19.6.5.1.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.5.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

19.6.5.1.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.9.1.2.4.2 with following exceptions:

-	Table 6.7.9.1.1.5-1 is replaced by Table 19.6.5.1.1.5-1;

19.6.5.1.2.4.3	Message contents

Same as the message contents given in clause 6.7.9.1.1.4.3.

19.6.5.1.2.5	Test requirement

Same as the test requirements given in clause 6.7.9.1.2.5 with following exceptions:

-	Table 6.7.9.1.1.5-1 is replaced by Table 19.6.5.1.1.5-1;

<End of Change 1>

<Start of Change 2>

19.6.5.2	NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG

19.6.5.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

19.6.5.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.2.

19.6.5.2.4	Test description

19.6.5.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.9.2.4.1-1 is replaced by Table 19.6.5.2.4.1-1;

-	Table 6.7.9.2.4.1-2 is replaced by Table 19.6.5.2.4.1-2;

Table 19.6.5.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 19.6.5.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.5.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.5.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 19.6.5.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

19.6.5.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.9.2.4.2 with following exceptions:

-	Table 6.7.9.2.5-1 is replaced by Table 19.6.5.2.5-1.

19.6.5.2.4.3	Message contents

Same as the message contents given in clause 6.7.9.2.4.3.

19.6.5.2.5	Test requirement

Same as the test requirements given in clause 6.7.9.2.5 with following exceptions:

-	Table 6.7.9.2.5-1 is replaced by Table 19.6.5.2.5-1.

Table 19.6.5.2.5-1: NR Cell specific test parameters (Same as Table 6.7.9.2.5-1 with following exceptions)

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Propagation condition | 1, 2 |  | AWGN+220 Hz | AWGN+220 Hz |
|  | 3 |  | AWGN+500 Hz | AWGN+500 Hz |

<End of Change 2>

<Start of Change 3>

19.6.5.3.1	NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG

19.6.5.3.1.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

19.6.5.3.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.3.

19.6.5.3.1.4	Test description

19.6.5.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.3.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.9.3.1.4.1-1 is replaced by Table 19.6.5.3.1.4.1-1;

-	Table 6.7.9.3.1.4.1-2 is replaced by Table 19.6.5.3.1.4.1-2;

Table 19.6.5.3.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 19.6.5.3.1-1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.5.3.1-2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.5.3.1-3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 19.6.5.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.5.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

19.6.5.3.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.9.3.1.4.2 with following exceptions:

-	Table 6.7.9.3.1.5-1 is replaced by Table 19.6.5.3.1.5-1.

19.6.5.3.1.4.3	Message contents

Same as the message contents given in clause 6.7.9.3.1.4.3.

19.6.5.3.1.5	Test requirement

Same as the test requirements given in clause 6.7.9.3.1.5 with following exceptions:

-	Table 6.7.9.3.1.5-1 is replaced by Table 19.6.5.3.1.5-1.

Table 19.6.5.3.1.5-1: NR Cell specific test parameters (Same as Table 6.7.9.3.1.5-1 with following exceptions)

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Propagation condition | 1, 2 |  | AWGN+220 Hz | AWGN+220 Hz |
|  | 3 |  | AWGN+500 Hz | AWGN+500 Hz |

19.6.5.3.2	NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG

19.6.5.3.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR relative measurement accuracy is within the specified limits.

19.6.5.3.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards and supporting L1-SINR measurements.

19.6.5.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.5.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.5.3.

19.6.5.3.2.4	Test description

19.6.5.3.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.9.3.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.9.3.2.4.1-1 is replaced by Table 19.6.5.3.2.4.1-1;

-	Table 6.7.9.3.2.4.1-2 is replaced by Table 19.6.5.3.2.4.1-2;

Table 19.6.5.3.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 19.6.5.3.2-1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.6.5.3.2-2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.6.5.3.2-3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 19.6.5.3.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.5.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

19.6.5.3.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.9.3.2.4.2 with following exceptions:

-	Table 6.7.9.3.1.5-1 is replaced by Table 19.6.5.3.1.5-1.

19.6.5.3.2.4.3	Message contents

Same as the message contents given in clause 6.7.9.3.1.4.3.

19.6.5.3.2.5	Test requirement

Same as the test requirements given in clause 6.7.9.3.2.5 with following exceptions:

-	Table 6.7.9.3.1.5-1 is replaced by Table 19.6.5.3.1.5-1.

<End of Change 3>

<Start of Change 4>

Table F.1.1.2-17: Maximum test system uncertainty for RRM requirements for SA FR1 Air-to-ground test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 19.2.1.1 Intra-frequency handover from FR1 to FR1; known target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 19.2.1.2 Inter-frequency handover from FR1 to FR1; unknown target cell | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 19.6.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.5.1.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 19.6.5.1.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 19.6.5.2 NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG | Same as 6.7.9.2 | Same as 6.7.9.2 |
| 19.6.5.3.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 19.6.5.3.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |

<End of Change 4>

<Start of Change 5>

Table F.1.3.2-15: Derivation of test requirements for RRM requirements for SA FR1 Air-to-ground test cases

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 19.2.1.1 Intra-frequency handover from FR1 to FR1; known target cell | Same as 6.3.1.1 | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 19.2.1.2 Inter-frequency handover from FR1 to FR1; unknown target cell | Same as 6.3.1.3 | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 19.6.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.5.1.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 |
| 19.6.5.1.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |
| 19.6.5.2 NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG | Same as 4.7.7.2 | Same as 4.7.7.2 | Same as 4.7.7.2 |
| 19.6.5.3.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 4.7.7.3.1 | Same as 4.7.7.3.1 | Same as 4.7.7.3.1 |
| 19.6.5.3.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |

<End of Change 5>