3GPP TSG RAN #109			RP-252335

Beijing, China, Sept 12-20, 2025

Source:	Sharp

Title:	NTN HD-FDD Mobile VSAT for Ka and Ku bands

Agenda Item:	9.1.4.6

Document for:	Discussion and Decision

# Background

At RAN#108, the list of potential RAN4 5GAdv items is endorsed in [1]. Below is a section of that listing as related to NTN enhancements that are expected to be further discussed at RAN#109 as part of the drafting process of a Rel-20 WID.

# NTN enhancements ## Agreements: High level scope for WI on NTN enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - NTN HPUE - …text removed for brevity…. - NTN HD-FDD VSAT for Ka/Ku bands - RAN4 will start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm). - Once the mobile VSAT with small antennas work is done, - NTN HD-FDD for fixed VSAT and the mobile VSAT with large/normal antenna will be discussed, including how the agreements for the mobile VSAT with small antenna can be leveraged as applicable - RAN1/2 impact is expected and will be further discussed in RAN#109 - Leverage RedCap over NTN HD-FDD solution to NTN as applicable. - NTN DL CA - …text removed for brevity…. - Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements - …text removed for brevity…. - Target 1TU in Q4’25 and 1.5 TU allocation including RF, RRM and demod from Q1’26 ……

The list identifies Ultra-Small MVSAT and HD-FDD as two interdependent items, as both are part of a NR TN-NTN solutions space that would employ an active phase array antenna integrated into a terminal. For the purposes of NTN communications, an active phased array antenna can provide the terminal with efficient means to steer and direct radiated RF energy to achieve the desired effective isotropic radiated power (EIRP) on transmit and the gain over noise temperature (G/T) on receive. Such efficiencies are considered necessary for the successful deployment of 3GPP NTN broadband services for the passenger automobile market in a NR 5GAdv system.

Such a terminal with an integrated phase array antenna may employ a Full Duplex Frequency-Division Duplexing (FD-FDD) communication link. However, a FD-FDD configuration may require a design trade-off to optimize either terminal size or antenna performance. For example, a phase array antenna design optimised for performance may use a first set of antenna array elements for Tx operations and second set of antenna array elements physically separate from the first set for Rx operations, while an alternate design optimized for reduced size may use the same antenna array elements for both Tx and Rx operations. In the case of the first design optimized for performance, using a separate set of antenna array elements for Tx and Rx operations reduces self-interference but at the expense of additional space requirements. In the case of the second design optimized for reduced size, using the same set of antenna array elements for both Tx and Rx operation provides for a minimal physical footprint of the antenna, however that second design must contend with high self-interference by the Tx signal onto the Rx.

Sharp’s view on the inclusion of Ultra-Small MVSAT and HD-FDD in the RAN4 WID is discussed in subsequent sections.

# Discussion

# Ultra Small Mobile VSAT

The auto industry has provided multiple contributions to RAN regarding the requirements of a Mobile VSAT terminal that would be suitable for deployment on passenger type vehicles [3-13]. One of the key requirements identified is the availability of broadband service via NTN connectivity by the end of the decade [6]. Another key requirement identified is the availability of specifications for an Ultra Small form factor phase array antenna (e.g. < 20x20cm) suitable for mounting on a passenger automobile [5]. As noted in the auto industry publication [13], the average time from exterior design freeze to first production of a passenger vehicle is about 27 months. Therefore, given the current schedule for Rel-20 (RAN4 Perf part, June of 2027 [2]) we can see that to align with the auto industry pre-production schedule of a model year 2030 vehicle that would include a 3GPP compliant 20x20cm phase array, the work for the specification of Ultra Small VSAT needs to start in the Rel-20 5GAdv timeframe.

Observation 1: The automobile industry has identified phase arrays antennas of < 20x20cm as suitable for installation in passenger vehicles.

Observation 2: The automobile industry is looking for NTN broadband service in passenger vehicles by the end of the decade.

Observation 3: Lead time for terminal integration into manufacture of passenger automobile is 27 months.

Observation 4: The current schedule for the completion of RAN4 Perf part for Rel-20 5GAdv is June of 2027.

Proposal 1: RAN4 to perform relevant analyse and normative work to enable ultra small Mobile VSAT in Ka and Ku bands in Rel-20 5GAdv.

# Half Duplex Frequency-Division Duplexing (HD-FDD)

A FD-FDD transceiver may transmit and receive signals simultaneously on different frequencies separated by a guard band. A phase array antenna used by such a FD-FDD transceiver may be designed with a sperate set of array elements for Tx and for Rx operations as a means to avoid the strong self-interference generated when transmitting and receiving simultaneously via a shared-antenna configuration (i.e. the same physical array elements).

While a phase array antenna design with sperate array elements for Tx and Rx operation provides a reduction in self-interference due to increased isolation of the antenna elements, the design would at least double the antenna’s overall size when accounting for the guard space required between the two sets of array elements.

Alternatively, a phase array antenna may be designed to use a shared-antenna configuration for both Tx and Rx operations as a means to minimize the impact of the antenna on the overall physical size of the terminal. However, while the shared-antenna configuration design requires less space, it suffers from self-interference due to poor isolation between the antenna’s receiving and transmitting operations.

As noted above and in the background section, a FD-FDD transceiver integrated to a phase array antenna must contend with a trade-off of size vs self-interference. To mitigate the impacts of the design trade-offs identified when using an FD-FDD transceiver and a phase array antenna (i.e. size vs self-interference), a hy-bread Tx/Rx transceiver mode knows as Half Duplex Frequency-Division Duplexing (HD-FDD) can be used. In this mode, the transceiver interleaves the operations for Tx and Rx in the time domain as in a TDD mode, while separate frequences are used for the Tx and Rx operations as in a FDD mode.

By utilizing such a hy-bread transceiver operation and antenna design, a terminal’s size is minimized to a single antenna array for both Tx and Rx operations, while avoiding self-interference by interleaving Tx and Rx operation across the time domain.

To meet the auto industry goals of a flat panel phase array antenna with form factor of < 20x20cm, while maintaining suitable system performance, a HD-FDD design is necessary.

Proposal 2: Study and specify the relevant set of requirements for defining HD-FDD operations for Mobile VSAT with small antennas (e.g 20x20cm) in R20 5GAdv.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: RAN4 to perform relevant analyse and normative work to enable ultra small Mobile VSAT in Ka and Ku bands in Rel-20 5GAdv.

Proposal 2: Study and specify the relevant set of requirements for defining HD-FDD operations for Mobile VSAT with small antennas (e.g 20x20cm) in R20 5GAdv.

# References

- RP-251879, Way forward on RAN4 R20 package

- RP-240790, Additional Considerations for 6G Timeline (RAN Perspective)

- RWS-230164, “5GAA input to 3GPP Rel-19 Workshop”, Taipei, Taiwan, 2023-06

- RP-232733, “Automotive Requirements for Non-Terrestrial Network (NTN) Enhancements for Release 19”, Edinburgh, Scotland, 2023-12

- RP-250856, LS to RANp on automotive priorities in 5G-Advanced NR-NTN Rel-20

- RP-242482, NTN in 5G Advanced Remaining Topics BMW

- RP-250542, NTN in 5G-Advanced Rel-20 / Automotive Terminal / VSAT support

- RP-250543, Remaining NR NTN topics for BMW

- RP-251553, View on Automotive Requirements for 6G RAN-Level Study Item

- RP-251561, NTN in 5GA: Connected Mode Mobility, Automotive Terminal / VSAT Support

- RP-251590, RAN WG 6G SI – an Automotive view

- RP-251767, View on Automotive Requirements for 6G RAN-Level Study Item

- https://www.kearney.com/industry/automotive/article/speeding-up-the-automotive-product-development-process