3GPP TSG RAN Meeting #109					RP-252340

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.1

Source:	Google

Title:	Deployment Scenarios for 6G

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussions on deployment scenarios for 6G.

# Discussion

Supporting single-panel UE for FR2 in 6G is a crucial design consideration to achieve a balance between performance and practicality. While multi-panel designs can provide superior beamforming gains and link robustness, they often come with increased complexity, higher cost, and greater power consumption for devices. By enabling single-panel FR2 operation, 6G can extend high-frequency millimeter-wave capabilities to a broader range of devices, including more compact and energy-efficient form factors, like IoT devices and affordable smartphones. This approach would simplify the device architecture, reduce manufacturing costs, and potentially improve battery life, making the benefits of high-capacity FR2 bands accessible to a wider market. This is especially important for the mass adoption of 6G, as it provides a pathway for a simpler, more streamlined ecosystem that can still deliver the multi-gigabit speeds required for immersive experiences and advanced applications.

Proposal 1: Support single-panel UE for FR2 deployment scenarios in 6G.

Further, usually the number of antenna element is limited in the UE side. It is not practical to form a narrow beam in UE side. Thus, the beamforming gain for multi-UE-beam operation is limited compared to single-UE-beam operation. Compared to the multi-UE-beam operation, the single-UE-beam operation would benefit for not only UE power reduction but also latency and complexity reduction for beam management. Further, single-UE-beam operation is also more friendly to carrier aggregation framework. UE is able to receive multiple downlink signals with different QCL properties simultaneously. Therefore, for FR2, the single-panel UE should be supported. The UE based on single-UE-beam operation should also be included.

Proposal 2: Support single-UE-beam operation for FR2 deployment scenarios in 6G.

With regard to diverse types of devices, the UEs with different antenna architectures should be considered. The 6G deployment scenario should consider all types of UEs. Thus, for FR1/FR3, the UEs with 1, 2 or 4 antennas should be considered. The UEs with omni-directional antennas should be considered for the deployment scenario.

Proposal 3: Support UEs with 1/2/4 omni-directional antennas for FR1/FR3 deployment scenarios in 6G.

Further, for UEs with multiple antennas, different capabilities in terms of the maximum transmission power for different antennas should be considered. For example, for UEs with two antennas, one antenna may be able to support up to 26 dBm transmission power, and the other antenna may be able to support up to 23 dBm transmission power. Therefore, when defining the maximum transmission power for the UE, UEs with different maximum transmission power for different antennas should be supported for the deployment scenarios in 6G.

Proposal 4: Support UEs with different maximum transmission power for different antennas for the deployment scenarios in 6G.

# Conclusion

In this contribution, we provided discussion on the deployment scenarios for 6G. Based on the discussion, the following proposals are provided.

Proposal 1: Support single-panel UE for FR2 deployment scenarios in 6G.

Proposal 2: Support single-UE-beam operation for FR2 deployment scenarios in 6G.

Proposal 3: Support UEs with 1/2/4 omni-directional antennas for FR1/FR3 deployment scenarios in 6G.

Proposal 4: Support UEs with different maximum transmission power for different antennas for the deployment scenarios in 6G.