3GPP TSG RAN #109		RP-252368
Beijing, China, Sep 15-18, 2025

Title:	RAN4 R20 RRM

Source:	OPPO

Agenda item:	9.1.4.4

Document for:	Approval

- Introduction

A WF on RAN4 led REL-20 topics was approved in RAN#108. For RRM enhancements, the following agreements are achieved.

The high-level scope for WI on RRM enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - (e)RedCap UE enhancement - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 LT5 WI - This also include example bands, UE RF scope including potential complexity reduction, UE demod - Fast SCell activation - Further discussion on the possible down scoping in RAN#109 based on the following two objectives - SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026. - A-TRS/TRS based unknown SCell activation including both inter-band CA and intra-band non-contiguous CA - Consider simplification of solution to maintain reasonable work scope in consideration of the TU allocation - No RAN1/2 scope is expected - Target 0.5 TU (0.25RD and 0.25RF) in Q4’25 and 1 TU allocation (0.75RD,0.25RF) from Q2’26

In this document, we will provide our view on R20 RRM topics.

- Discussion

  - RLM/BFD relaxation for Redcap UE

In R20, RAN4 RRM WI should focus on the real demands from markets, and control the workload led by RAN4.

Power saving is important for both UE and network, in previous releases several features and corresponding RRM requirements have been introduced, e.g., Rel-16 RRM measurement relaxation for idle mode, Rel-17 RLM/BFD relaxation for connected mode, Rel-17 UE supporting Redcap, and R18~R19 network energy saving, and Rel-19 LP-WUS/WUR.

For Redcap UE, the Rel-16 RRM measurement relaxation for idle mode, and R19 LP-WUS/WUR were agreed to be supported. However, RLM/BFD relaxation is still missing.

Considering Redcap UE has already been commercialized, we support to introduce RLM/BFD relaxation for Redcap UE, including both 1Rx and 2Rx. About Less than 5MHz for (e)RedCap, we are open to this considering the RRM work is not much.

Proposal 1: 	Specify RLM/BFD relaxation requirements for Redcap UE, including both 1Rx and 2Rx. And R17 UE power saving WI conclusions can be reused as much as possible.

  - SCell activation delay reduction

As background, below enhancements have been done before:

- For Scell activation

  - Intra-band contiguous Scell (for UE supporting scellWithoutSSB) is already supported from R15.

- For delay reduction

  - MAC CE based A-TRS/TRS based fast Scell activation in R17 CA/DC enhancement,

  - Inter-band collocated SSB-less Scell (for UE supporting SSBwithoutSSB-interbandCA) in R18 NES WI,

  - Further reducing L1 and L3 measurement delay (i.e., UE supporting ReduceForCellDetection, ReduceForSSBL1-RSRP-Meas and ShortMeasInterval) and MAC CE base L3 report in R18 RRM enhancement.

In R20, we can further consider SCell activation delay reduction for intra-band co-located CA including contiguous and non-contiguous, as it is also common commercial deployment.

Based on the observation of current requirements, the total activation time is THARQ+ Tactivation_time +TCSI_reporting.

In the case of intra-band non-contiguous CA, SSB-less Scell can be considered to further reduce Tactivation_time by skipping cell detection and L1/L3 measurement.

In the case of intra-band continuous CA, TCSI_reporting is quite large, for example, Tactivation_time is 3 m, THARQ is 2ms but TCSI_reporting is as least 15ms in test case configuration and even 40/80ms configuration in practical network. Since the channel conditions are comparable on the to-be-activated SCC and another continuous activated CC, CSI reporting procedure is not necessary before completing the SCell activation. Thus, whether and how to reduce CSI reporting delay can be also considered.

Proposal 2: 	Consider SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting in Rel-20.

Besides, A-TRS for fast Scell activation has been supported for FR1 known and unknown SCell and FR2 known SCell, and A-TRS/TRS based SCell activation have been supported for UE supporting scellWithoutSSB-InterBandCA-r18 in FR1. However, intra-band non-contiguous CA and FR2 unknown cases are deprioritized in both R17 and R18 normal work, since these scenarios are quite complicated and not common.

Thus, we are slight negative to further involve A-TRS/TRS based unknown SCell activation in R20. No further enhancement of FR2 is preferred since RAN4 already paid a lot of efforts to support dedicated FR2 Scell activation delay reduction in R18 and the benefit of A-TRS/TRS seems less. If possible, only TRS based FR1 unknown SCell activation can be discussed and considered.

Proposal 3: 	Prefer to have no further enhancement for A-TRS/TRS based FR2 unknown SCell activation.

- Conclusions

This paper discussed the Rel-20 RRM candidates. And got below proposals.

Proposal 1: 	Specify RLM/BFD relaxation requirements for Redcap UE, including both 1Rx and 2Rx. And R17 UE power saving WI conclusions can be reused as much as possible.

Proposal 2: 	Consider SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting in Rel-20.

Proposal 3: 	Prefer to have no further enhancement for A-TRS/TRS based FR2 unknown SCell activation.

- References

[1] RP-251879, WF for RAN4 led REL-20 topics