3GPP TSG RAN Meeting #109	RP-252373
Beijing, China, September 15-18, 2025

Agenda Item:	9.1.4.4

Source:	Ericsson

Title:	Discussion on RRM proposals for 5G-Advanced Release 20

Document for:	Discussion

# 1	Introduction

During RAN#107, first discussions were held on the RAN4 5G-A package. The candidate topics were down selected at RAN#108 and final topics are expected to be approved at RAN#109. This contribution is provided for information with our views in each of the areas identified at RAN#108.

# 2	RRM

In the area of RRM generally there are very large number of proposals. Therefore, our aim is to focus on those areas which are urgent and have substantial practical significance and cannot be covered in work items led by other working groups. Based on the arguments we have the following two proposals for RRM.

- Further enhanced RedCap

- Scell activaiton delay enhancement

## 2.1 Further enhanced RedCap

Firstly, RAN4 introduced requirements for relaxed Radio Link Monitoring (RLM) and Beam Failure Detection (BFD) in release 17 for regular NR UEs as part of the UE power saving WI [2]. This feature allows the UE to achieve power saving gain by measuring less often when fulfilling some criteria, see Figure 1.

The criteria introduced are related to radio conditions of the UE with respect to the measured cell (i.e. good serving cell) and mobility state (low mobility criteria) of the UE. However, these requirements are currently not applicable for RedCap devices. Power saving is a relevant feature for RedCap UEs as they have reduced capability and cost. Therefore, we propose to extend this CONNECTED mode power saving feature for both 1 Rx and 2 Rx RedCap. For 2Rx RedCap, the legacy requirements including the criteria can be reused while some more discussions are needed in the working group regarding 1Rx and HD-FDD impact. It should be emphasized that both the criteria for applying relaxed measurements and the method for defining these requirements shall be reused. This approach helps managing the workload, avoids creating multiple solutions for the same feature, and leverages the UE power-saving work from Release 17.

It was noted from the last plenary discussions that many companies have expressed support for this particular enhancement for RedCap. Thus it was highlighted as candidate topic with common interest among the companies in RAN4#107, see the summary document in [1].

Add RLM/BFD relaxation for Redcap devices in Rel-20 RRM based on Rel-17 relaxed RLM/BFD requirements and criteria defined for non-RedCap UEs.

Agreements in RAN#108:

The high-level scope for WI on RRM enhancement (the exact scope and WID draft will be discussed and decided in RAN#109):

- (e)RedCap UE enhancement

  - RLM/BFD relaxation based on the conclusions of R17 UE power saving WI

  - Less than 5MHz for (e)RedCap, including both 1Rx and 2Rx, based on the conclusions of R18 LT5 WI

    - This also include example bands, UE RF scope including potential complexity reduction, UE demod

Based on the discussions and agreements from RAN#108, it was noted that there is commercial interest in supporting less than 5MHz operation for (e)RedCap in both 1Rx and 2Rx UEs. We therefore support introducing the necessary RRM requirements to enable this new scenario. It is worth recalling that less than 5MHz operation for 2Rx UEs was already introduced in Release 18. Our proposal is to build on the conclusions from that work and focus only on the remaining cases, namely 1Rx and HD-FDD.

If less than 5MHz for eRedCap is introduced, RAN4 to leverage on conclusions from release 18 less than 5MHz work and only work on the missing requirements for 1Rx and HD-FDD cases.

## 2.2 Other proposals

Faster SCell activation of CA is highly important for CA performance in bursty traffic. It is important that the network and UE can make use of CA quickly when the UE has traffic bursts without the signaling and energy overheads of keeping carriers active when the UE does not have data. It is important to take into account that fast SCell activation has already been the subject of much attention.

RAN4 has defined RRM requirements for various types of SCell activation procedures. However, due to the limited timeframe for the Rel-20 RRM package, efforts should be focused on areas relevant to existing deployments and/or those with commercial interest. While multiple SCell activation procedures are supported in the specifications, only a subset is currently implemented and used in real-world networks. Therefore, enhancements or optimizations for SCell activation procedures that are not commercially deployed should be deprioritized.

Agreements in RAN#108:

The high-level scope for WI on RRM enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - Fast SCell activation - Further discussion on the possible down scoping in RAN#109 based on the following two objectives - SCell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting - Interested companies can propose to extend this work to inter-band case in the check-point in June 2026. - A-TRS/TRS based unknown SCell activation including both inter-band CA and intra-band non-contiguous CA - Consider simplification of solution to maintain reasonable work scope in consideration of the TU allocation - No RAN1/2 scope is expected - Target 0.5 TU (0.25RD and 0.25RF) in Q4’25 and 1 TU allocation (0.75RD,0.25RF) from Q2’26

Based on the agreements from RAN#108, we consider intra-band co-located CA to be a type of SCell activation procedure with greater commercial relevance compared to A-TRS/TRS-based approaches.

It is also recalled that RAN4 did FR2 measurement delay reduction work for CA scenario as part of Rel-19 RRM enhancement WI. However, the enhancements introduced in this WI were limited to FR2 only. Extending the Rel-19 solutions to FR1 CA scenario could be a candidate for Rel-20 Fast SCell activation work since the Rel-19 solutions can be reused and the work required would be to identify the conditions under which the Rel-19 FR2 solutions would work.

Observation 1: Extending the Rel-19 FR2 CA measurement delay reduction work to FR1 may not require significant amount of work.

As stated earlier, given that RAN4 already did enhancements to SCell activation procedures in previous releases and RRM workload is an issue in general, we are also fine to not do any further RAN4 work on Scell activation work in Rel-20.

Observation 2: RAN4 RRM workload is an issue in general, and not approving any further work on fast scell activation requirements is yet another option.

Nevertheless, if companies are interested in further enhancing SCell activation requirements, we are open to consider solutions to reduce SCell activation delay for intra-band co-located CA, including SSB-less scenarios and improved CSI reporting.

Further enhancing Scell activation delay requirements is in Rel-20 is not necessary. If there is any introduction of fast SCell activation objective, RAN4 to consider Scell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting or apply Rel-19 FR2 delay reduction requirements to FR1.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

- Add RLM/BFD relaxation for Redcap devices in Rel-20 RRM.

If less than 5MHz for eRedCap is introduced, RAN4 to leverage on conclusions from release 18 less than 5MHz work and only work on the missing requirements for 1Rx and HD-FDD cases.

- Observation 1: Extending the Rel-19 FR2 CA measurement delay reduction work to FR1 may not require significant amount of work.

Observation 2: RAN4 RRM workload is an issue in general, and not approving any further work on fast scell activation requirements is yet another option.

Further enhancing Scell activation delay requirements is in Rel-20 is not necessary. If there is any introduction of fast SCell activation objective, RAN4 to consider Scell activation delay reduction for intra-band co-located CA, including SSB-less cases and enhanced CSI reporting. As an another alternative apply Rel-19 FR2 delay reduction requirements to FR1.

# 4	References

[1]	RP-251879, “WF for RAN4 led REL-20 topics”, RAN4 Chair (Apple)