3GPP TSG-RAN Meeting #109	RP-252443

Beijing, China , Sept 15th – 18th, 2025

Source:	Nokia

Title:	Considerations on RAN4-led REL-20 5G-Adv Cross area topics

Agenda item:	9.1.4.7

Document for:	Discussion

Introduction

In the last RAN plenary meeting #108 in Prague, the WF for RAN4 led REL-20 topics in [1] was endorsed. During the meeting a number of agreements were made while some aspects are still open for further discussions. In this paper we discuss these remaining open aspects regarding the cross area topics in RAN4 led Rel-20 package.

Discussion

## 2.1	Cross RF and baseband topics

From [1], the following two areas are endorsed to be introduced in R20, and two further areas are under discussion:

- Extension of LBCA (the exact scope and WID draft will be discussed and decided in RAN#109):

  - Further discussion on the possible scoping in RAN#109 based on the following two objectives

    - Study and if feasible, specify non-RRC based switching solution, e.g. dynamic switching, by taking the RAN1 impacts into consideration

    - Dual DL solution

    - RAN1/2 impact is expected and will be further discussed in RAN#109

  - Target 0.5 TU (0.25 RF and 0.25 RRM) in Q4’25 and 0.75 TU allocation (0.25 RF and 0.5 RRM) from Q1’26

- UAV (the exact scope and WID draft will be discussed and decided in RAN#109):

  - UAV UE with antenna array with up to 33dBm maximum output power on TDD spectrum

  - RAN1/2 scope is not expected

  - Target 0.5 TU allocation (0.25 RF and 0.25 RRM)

It was also endorsed that the following two proposals will be discussed at RAN#109, where it will be decided whether or not to include either of the proposals:

  - Fragmented DL:

    - Introducing RF/RRM requirements related to new UE behavior for reducing the number of UE Rx RF chains

    - RAN1/2 scope is TBC

  - Intra-band non-collocated EN-DC/NR-CA deployment

    - Type 3 UE requirements assuming MRTD<CP and power imbalance < [xx] dB for handheld

    - RAN1/2 scope is TBC

While the importance of the above two topics under discussion to some operators is recognised, the challenge of TU availability remains because a non-negligible amount of discussion would be required; it also remains uncertain to which extent device vendors would intend to implement the these features in the context of 5G-Advanced. These aspects might be better resolved across the ecosystem in 6G.

RAN can consider not to agree more cross RF and baseband WIs than the two endorsed. Remaining proposals can be considered in relation to 6G.

## 2.7	TU discussion

The overall TU allocation for 5G-Adv topics in Rel-20 needs to take into account the time required for the 6G SI. Great care therefore needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items.

Great care needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items.

Conclusion

The following Observations and Proposals were made on the Cross area domain:

Proposal 1: RAN can consider not to agree more cross RF and baseband WIs than the two endorsed. Remaining proposals can be considered in the relation to 6G.

Proposal 2: Great care needs to be taken not to overload the 5G-Adv track, nor to underestimate the TUs required for individual items.

- 4	References

- RP-251879; WF for RAN4 led REL-20 topics; RAN4 Chair

- RP-250901, Other cross domain topics for Rel-20 5G-A, Nokia