3GPP TSG RAN WG1 #122	        		   			R1-2506663

# Bangalore, India, August 25th – 29th, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0724 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on evolution of NR duplex operation: Sub-band full duplex (SBFD) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-09-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Missing description that for PUSCH transmission in SBFD symbols, RBs are in the UL sub-band (in addition to active UL BWP) in clause 7.1.1. - Missing description that Msg3 PUSCH, or PUCCH with HARQ-ACK, transmission in SBFD symbols needs to be associated with PRACH transmission in second ROs in order to consider the UL sub-band when determining NBWPsize in clauses 8.3 and 9.2.1. - Incorrect association of PDSCH receptions in SBFD/non-SBFD symbols with RRC parameter sbfd-Config2-Transmission in clause 11.1. - Misaligned RRC parameter names in clauses 8.3 and 11.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture that that for PUSCH transmission in SBFD symbols, RBs are in the UL sub-band (in addition to active UL BWP) in clause 7.1.1. - Add description that Msg3 PUSCH, or PUCCH with HARQ-ACK, transmission in SBFD symbols needs to be associated with PRACH transmission in second ROs in order to consider the UL sub-band when determining NBWPsize in clauses 8.3 and 9.2.1. - Associate PDSCH receptions in SBFD/non-SBFD symbols with RRC parameter sbfd-Config2-Reception in clause 11.1. - Align RRC parameter names in clauses 8.3 and 11.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete/incorrect specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1, 8.3, 9.2.1, 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged parts are omitted ***

### 7.1.1	UE behaviour

If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l

-	if the UE is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and is configured with multipanelSchemeSDM or multipanelSchemeSFN or sTx-2Panel, and the UE determines to apply both the first TCI-State or TCI-UL-State and the second TCI-State or TCI-UL-State in PUSCH transmission occasion i, the UE determines the PUSCH transmission power PPUSCH,b,f,c, k(i,j,qd,l) for the k-th indicated TCI-State or TCI-UL-State as

PPUSCH,b,f,c,ki,j,qd,l=minPCMAX,f,c,kiPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l   [dBm]

*** Unchanged parts are omitted ***

-	MRB,b,f,cPUSCH(i) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c and μ is a SCS configuration defined in [4, TS 38.211]. If the PUSCH transmission occasion i is in SBFD symbols, MRB,b,f,cPUSCH(i) is the number of RBs in both the active UL BWP and in the UL sub-band as described in [6, TS 38.214].

-	PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c

*** Unchanged parts are omitted ***

## 8.3	PUSCH scheduled by RAR UL grant

An active UL BWP with SCS configuration μ, as described in clause 12 and in [4, TS 38.211], for a PUSCH transmission scheduled by a RAR UL grant is indicated by higher layers.

If useInterlacePUCCH-PUSCH is not provided by BWP-UplinkCommon and BWP-UplinkDedicated, for determining the frequency domain resource allocation for the PUSCH transmission within the active UL BWP

-	if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP, the initial UL BWP is used

-	else, the RB numbering starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP

The frequency domain resource allocation is by uplink resource allocation type 1 [6, TS 38.214]. For an initial UL BWP size of NBWPsize RBs, a UE processes the frequency domain resource assignment field as follows

-	if NBWPsize≤180, or for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided if NBWPsize≤90

-	truncate the frequency domain resource assignment field to its log2NBWPsize∙NBWPsize+12 least significant bits and interpret the truncated frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212]

-	else

-	insert

-	log2NBWPsize∙NBWPsize+12-12 most significant bits, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided;

-	log2NBWPsize∙NBWPsize+12-14 most significant bits, otherwise;

with value set to '0' after the NUL,hop bits to the frequency domain resource assignment field, where NUL,hop=0  if the frequency hopping flag is set to '0' and NUL,hop is provided in Table 8.3-1 if the hopping flag bit is set to '1', and interpret the expanded frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212]

-	end if

If the PUSCH transmission is in SBFD symbols and associated with a PRACH transmission in second PRACH occasions, as described in clause 11.1, NBWPsize in Table 8.3-1 is the number of RBs that are both in the active UL BWP and in the UL sub-band for the determination of the frequency offset for the second hop.

If useInterlacePUCCH-PUSCH is provided by BWP-UplinkCommon or BWP-UplinkDedicated, the frequency domain resource allocation is by uplink resource allocation type 2 [6, TS 38.214]. A UE processes the frequency domain resource assignment field as follows

-	truncate the frequency domain resource assignment field to the X=6 LSBs if μ=0, or to the X=5 LSBs if μ=1

-	for interlace allocation of a PUSCH transmission, interpret the X MSBs of the truncated frequency domain resource assignment field for the active UL BWP as for the X MSBs of the frequency domain resource assignment field in DCI format 0_0 [6, TS 38.214]

-	for RB set allocation of a PUSCH transmission, the RB set of the active UL BWP is the RB set of the PRACH transmission associated with the RAR UL grant. The UE assumes that the RB set is defined as when the UE is not provided intraCellGuardBandsUL-List [6, TS 38.214].

A UE determines whether or not to apply transform precoding as described in [6, TS 38.214].

For a PUSCH transmission with frequency hopping scheduled by RAR UL grant or for a Msg3 PUSCH retransmission, the frequency offset for the second hop [6, TS 38.214] is given in Table 8.3-1.

Table 8.3-1: Frequency offset for second hop of PUSCH transmission with frequency hopping scheduled by RAR UL grant or of Msg3 PUSCH retransmission

| Number of PRBs in initial UL BWP | Value of NUL,hop Hopping Bits | Frequency offset for 2nd hop |
| --- | --- | --- |
| NBWPsize<50 | 0 | NBWPsize2 |
|  | 1 | NBWPsize4 |
| NBWPsize≥50 | 00 | NBWPsize2 |
|  | 01 | NBWPsize4 |
|  | 10 | -NBWPsize4 |
|  | 11 | Reserved |

A SCS for the PUSCH transmission is provided by subcarrierSpacing in BWP-UplinkCommon. A UE transmits PRACH and the PUSCH on a same uplink carrier of a same serving cell.

A UE transmits a transport block in a PUSCH scheduled by a RAR UL grant in a corresponding RAR message using redundancy version number 0, if the PUSCH transmission is without repetitions. If a TC-RNTI is provided by higher layers, the scrambling initialization of the PUSCH corresponding to the RAR UL grant in clause 8.2 is by TC-RNTI. Otherwise, the scrambling initialization of the PUSCH corresponding to the RAR UL grant in clause 8.2 is by C-RNTI.

If a UE is provided tag2-Id, the UE transmits a transport block in a PUSCH scheduled by a RAR UL grant in a corresponding RAR message with timing advance corresponding to a TAG indicated by the RAR message.

Msg3 PUSCH retransmissions, if any, of the transport block, are scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI provided in the corresponding RAR message [11, TS 38.321].

With reference to slots for a PUSCH transmission scheduled by a RAR UL grant, if a UE receives a PDSCH with a RAR message ending in slot n for a corresponding PRACH transmission from the UE, the UE transmits the PUSCH in slot n+k2+∆+2μ∙Kcell,offset, where k2 and ∆ are provided in [6, TS 38.214] and Kcell,offset is provided by cellSpecificKoffset; otherwise, if not provided, Kcell,offset=0.

A UE can be provided in BWP-UplinkCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. The UE does not apply sbfd-Config2-Transmission, if provided for a PUSCH transmission with repetitions. If the UE requests repetitions for the PUSCH transmission [11, TS 38.321], the UE transmits the PUSCH over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 from a set of four values provided by numberOfMsg3-RepetitionsList or from {1, 2, 3, 4} if numberOfMsg3-RepetitionsList is not provided. The UE determines an MCS for the PUSCH transmission by the 2 LSBs of the MCS field in the RAR UL grant or by the 3 LSBs of the MCS field in the DCI format 0_0, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆+2μ∙Kcell,offset where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, and where Kcell,offset is provided by cellSpecificKoffset; otherwise, if not provided, Kcell,offset=0. For paired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆+2μ∙Kcell,offset where Kcell,offset is provided by cellSpecificKoffset; otherwise, if not provided, Kcell,offset=0.

The UE may assume a minimum time between the last symbol of a PDSCH reception conveying a RAR message with a RAR UL grant and the first symbol of a corresponding PUSCH transmission scheduled by the RAR UL grant is equal to NT,1+NT,2+0.5 msec, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] and, for determining the minimum time, the UE considers that N1 and N2 correspond to the smaller of the SCS configurations for the PDSCH and the PUSCH. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

*** Unchanged parts are omitted ***

### 9.2.1	PUCCH Resource Sets

*** Unchanged parts are omitted ***

If the PUCCH transmission is in SBFD symbols and is associated with a PRACH transmission in second PRACH occasions, as described in clause 8, or if the PUCCH transmission is in SBFD symbols and is for a PDSCH reception other than a PDSCH reception that includes UE contention resolution identityNBWPsizeRBUL_SBstart=0, NBWPsize is the number of RBs in both the active UL BWP and in the UL sub-band and RBUL_SBstart is the index of the first RB from the RBs that are in the active UL BWP and in the UL sub-band as described in clause 11.1; otherwise, NBWPsize is the number of RBs in the active UL BWP and RBUL_SBstart=0.

If rPUCCH8=0 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

-	the UE determines the lowest PRB index of the PUCCH transmission in the first hop as RBUL_SBstart+RBBWPoffset⋅NRB+rPUCCHNCS∙NRB and the lowest PRB index of the PUCCH transmission in the second hop as RBUL_SBstart+NBWPsize-RBBWPoffset∙NRB-1+rPUCCHNCS∙NRB, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes

-	the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS

If rPUCCH8=1 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

-	the UE determines the lowest PRB index of the PUCCH transmission in the first hop as RBUL_SBstart+NBWPsize-RBBWPoffset∙NRB-1+rPUCCH-8NCS∙NRB and the lowest PRB index of the PUCCH transmission in the second hop as RBUL_SBstart+RBBWPoffset∙NRB+rPUCCH-8NCS∙NRB

-	the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as

If a UE is provided a PUCCH resource by pucch-ResourceCommon and is provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon

-	the UE determines for the PUCCH resource an interlace index m as m=m0+rPUCCHNCSmodM where M is a number of interlaces [4, TS 38.211] and m0=RBBWPoffset is an interlace index offset and RBBWPoffset is as given in Table 9.2.1-1

-	the UE determines an initial cyclic shift index in a set of initial cyclic shift indexes as rPUCCHmodNCS, where NCS is the total number of initial cyclic shifts indexes in the set of initial cyclic shift indexes in Table 9.2.1-1

-	if pucch-ResourceCommon indicates

-	index 0: the first symbol is 9 for a PUCCH resource with PUCCH format 0 if rPUCCH≥10

-	index 1 or 2: the first symbol is 9 for a PUCCH resource with PUCCH format 0 if rPUCCH=15

-	index 3, 7, or 11: an orthogonal cover code with index 1 is used for a PUCCH resource with PUCCH format 1 if rPUCCH≥10; otherwise, an orthogonal cover code with index 0 is used for a PUCCH resource with PUCCH format 1

-	the UE does not expect pucch-ResourceCommon to indicate index 15

*** Unchanged parts are omitted ***

## 11.1	Slot configuration

*** Unchanged parts are omitted ***

A UE expects that the reference SCS configuration μref is smaller than or equal to a SCS configuration μ for any configured DL BWP or UL BWP. Each slot provided by pattern1 or pattern2 is applicable to 2μ-μref consecutive slots in the active DL BWP or the active UL BWP where the first slot starts at a same time as a first slot for the reference SCS configuration μref and each downlink or flexible or uplink symbol for the reference SCS configuration μref corresponds to 2μ-μref consecutive downlink or flexible or uplink symbols for the SCS configuration μ.

A downlink or flexible symbol provided by tdd-UL-DL-ConfigurationCommon can include an UL sub-band provided by ul-SubbandlocationAndBandwidth, a first DL sub-band provided by firstDL-SubbandlocationAndBandwidth and may additionally include a second DL sub-band provided by secondDL-SubbandlocationAndBandwidth, for a SCS configuration μ of any configured UL BWP or DL BWP, respectively, as provided by scs-SpecificCarrierList [4, TS 38.211]. The downlink or flexible symbol is then referred to as an SBFD symbol; otherwise, it is referred to as a non-SBFD symbol. Unless otherwise stated, the UE considers symbols in a slot indicated as downlink and as SBFD by tdd-UL-DL-ConfigurationCommon to be available for transmissions. Uplink symbols by tdd-UL-DL-ConfigurationCommon are non-SBFD symbols. An SBFD symbol or a non-SBFD symbol provided by tdd-UL-DL-ConfigurationCommon cannot change to a non-SBFD symbol or to an SBFD symbol, respectively, by other information. The UE is not provided coresetPoolIndex and is not configured to receive PDSCH according to more than one TCI states mapped to one TCI codepoint [6, TS 38.214] for a serving cell where the UE is provided SBFD symbols.

SBFD symbols are consecutive, start from a first slot provided by SBFD-StartingSlotIndex and from a first symbol in the first slot provided by SBFD-StartingSymbolIndex, and end in a second slot provided by SBFD-EndingSlotIndex and in a second symbol in the second slot provided by SBFD-EndingSymbolIndex. SBFD symbols can be provided in any of pattern1 and, if provided, pattern2. A configuration period for SBFD symbols is P msec when only pattern1 is provided, or P+P2 when pattern2 is additionally provided.

Except for a PRACH transmission in a PRACH occasion determined based on a second RACH configuration provided by sbfd-RACHDualConfig as described in clause 8, the UE transmits or receives a physical channel or signal either only in SBFD symbols or only in non-SBFD symbols.

When the UE is provided sbfd-Config2-Transmission, the UE can

-	transmit a first PUCCH or PUSCH or a first repetition of a PUCCH or PUSCH, and

-	transmit a second PUCCH or PUSCH or a second repetition of the PUCCH or PUSCH, in SBFD symbols

When the UE is provided sbfd-Config2-Reception, the UE can,

-	receive a first PDSCH or a first repetition of a PDSCH in non-SBFD symbols, and

-	receive a second PDSCH or a second repetition of the PDSCH in SBFD symbols

When a UE is not provided sbfd-Config2-Transmission

-	for a PUCCH or PUSCH transmission with repetitions, if the first repetition is either in SBFD symbols or in non-SBFD symbols, the remaining repetitions are also either in SBFD symbols or in non-SBFD symbols, respectively

-	for a Type 2 CG PUSCH transmission, or for a PUSCH transmission with SP-CSI reports, if the first transmission after a latest activation is either in SBFD symbols or in non-SBFD symbols, the remaining transmissions associated with the latest activation are also either in SBFD symbols or in non-SBFD symbols, respectively

-	for a Type 1 CG PUSCH, or for a PUCCH transmission with SP-CSI reports, or for a PUCCH with SR, [LRR, UEIRI], or P-CSI, or SRS, all respective transmissions are either only in SBFD symbols or only in non-SBFD symbols as indicated by the respective configurations [12, TS 38.331]

When a UE is not provided sbfd-Config2-Reception,

-	for a PDSCH reception with repetitions, if the first repetition is either in SBFD symbols or in non-SBFD symbols, the remaining repetitions are also either in SBFD symbols or in non-SBFD symbols, respectively

-	for a SPS PDSCH reception, if the first reception after a latest activation is either in SBFD symbols or in non-SBFD symbols, the remaining receptions associated with the latest activation are also either in SBFD symbols or in non-SBFD symbols, respectively

In an SBFD symbol, and except for cross-link interference measurements [6, TS 38.214], the UE transmits or receives only in RBs that are both in the active UL BWP and in the UL sub-band, or both in the active DL BWP and the DL sub-band(s), respectively. The UE does not transmit in SBFD symbols indicated for presence of SS/PBCH blocks within the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon.

*** Unchanged parts are omitted ***