3GPP TSG RAN Meeting #109			                                     RP-252829

Beijing, China, Sep. 15-18, 2025

Agenda Item:		10.5.3

Source:			EBU

Title:				WI summary for WI Core part: LTE-based 5G Broadcast Phase 2

WI code(s):			LTE_terr_bcast_Ph2

leading WG:		RAN1

Release:			Rel-19

### 1	Introduction

Time and frequency interleavers are crucial components for enhancing transmission robustness in time-varying (e.g., mobile reception) and frequency-selective (e.g., SFN deployments) channels. Physical-layer time interleaving, in particular, offers significant performance benefits, especially in high-mobility scenarios.

For MBMS-dedicated cells, this Rel-19 work item specifies time-frequency interleavers [RAN1] and corresponding signalling [RAN2, RAN3].

### 2	Description

Following on the design principles of NR TBoMS, the basic approach of the proposed Time- Frequency Interleaver is to reuse the building blocks behind HARQ retransmissions and combining, which allows to spread the coded bits of a transport block (TB) across multiple non-consecutive slots/subframes.The HARQ chip memory remains unused for dedicated MBMS carriers (i.e. no unicast transmissions) and can be repurposed to perform PMCH time-interleaving, i.e. no additional in-chip memory may be required.

In Figure 1, this mechanism is depicted: n 'redundancy versions' (RV) of each of m TBs are generated and then interleaved across a time unit of m× n subframes. To maintain the same overall throughput with non-interleaved transmission, the TBS is scaled according to n. Hence, the same HARQ memory can be reused to store the LLRs corresponding to each redundancy version and decode after all n redundancy versions for a TB have been received.

Figure 1: Subframe mapping of TBs for time interleaved PMCH transmission.

Frequency interleaving is key to counteract the erasure of OFDM subcarriers due to periodic frequency selectivity. This is a well-known effect of Single Frequency Network (SFN) scenarios, where frequency selectivity can erase entire codeblocks. Spreading the information of each codeblock across the entire system bandwidth would optimally mitigate the issue.

The main principles behind the proposed frequency interleaver are:

1. The tones corresponding to the codeblocks are written column-wise.

2. Pseudo random shifts are applied to the columns. This prevents periodic fades in the frequency domain (e.g., from two strong taps spaced apart in the delay domain) from affecting specific codeblocks negatively.

3. Data symbols in an OFDM symbol are read row-wise.

### 3	References

[1] RP-250794, “Revised WID on LTE-based 5G Broadcast Phase 2”, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 2025;

[2] RP-251975, Status Report for “New WID on LTE-based 5G Broadcast Phase 2”