3GPP TSG RAN#71									                   RP-160573

Gothenburg, Sweden, March 7 - 10, 2016

Agenda Item:	9.2.2

Source:			Orange, IAESI, Telecom Italia, Telefonica, ZTE, Telenor, China Unicom

Title:			Text proposal to TR 38.913 from email discussion [5G-AH-10] eHealth

Document for:		Approval

### 1	Introduction

Following the email discussion on the 3GPP RAN reflector [5G-AH-10] eHealth which summary report can be found in [RP-160370] we propose to approve the text proposal below to be included in TR38.913.

### 3		Proposed text for TR38.913

-------------------------------------------------- BEGIN TEXT PROPOSAL ----------------------------------------------------------

## 7.5	User plane latency

The time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point via the radio interface in both uplink and downlink directions, where neither device nor Base Station reception is restricted by DRX.

The target for user plane latency should be 1ms for UL, and 1ms for DL. This value is target value is to be met by URLLC while meeting the reliability requirement of 1-10-5 within 1ms.

[Editor’s notes: Detailed definition to be discussed.]

------------------------------------------------- END TEXT PROPOSAL ----------------------------------------------------------

-------------------------------------------------- BEGIN TEXT PROPOSAL ----------------------------------------------------------

### 7.9	Reliability

Reliability can be evaluated by the success probability of transmitting [X] bytes within [1 ms], which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface, at a certain channel quality (e.g., coverage-edge).

The target for reliability should be 1-10-5 within 1ms.

[Editor’s notes: The relevant use cases (V2V, V2I, or any others), deployment scenarios and the traffic model should be clarified.]

3GPP system shall support reliability up to be 1-10-5 within 1ms.for use cases such as eHealth surgical robots operating mainly in very deep indoor environment. This reliability performance shall be supported together with user experienced data rate in the order of [300Mbps].

[Editor’s notes: The requirement expressed above as specific to eHealth can be moved later to a separate section if we agree to have a dedicated section to use cases special combinations of KPIs to be met together]

### 7.10	Coverage

"Maximum coupling loss" (MCL) in uplink and downlink between device and Base Station site (antenna connector(s)) for a data rate of [X bps], where the data rate is observed at the egress/ingress point of the radio protocol stack in uplink and downlink.

The target for coverage should be [164dB].

### 7.11	UE battery life

UE battery life can be evaluated by the battery life of the UE without recharge. For mMTC, UE battery life in extreme coverage shall be based on the activity of mobile originated data transfer consisting of [TBD bytes] UL per day followed by [TBD bytes] DL from MCL of [tbd] dB, assuming a stored energy capacity of [TBD].

The target for UE battery life should be [15 years].

------------------------------------------------- END TEXT PROPOSAL ----------------------------------------------------------

-------------------------------------------------- BEGIN TEXT PROPOSAL ----------------------------------------------------------

### 7.17	Connection density

Connection density refers to total number of devices fulfilling specific QoS per unit area (per km2). QoS definition should take into account the amount of data or access request generated within a time t_gen that can be sent or received within a given time, t_sendrx, with x% probability.

The target for connection density should be 1 000 000 device/km2 in urban environment.

Editor’s notes: The details of QoS definition is FFS.

Connection density for other environments is FFS

------------------------------------------------- END TEXT PROPOSAL ----------------------------------------------------------

-------------------------------------------------- BEGIN TEXT PROPOSAL ----------------------------------------------------------

## 9.2	Location/Positioning Service

The target for positioning accuracy is [<1m] Indoor and outdoor.

-------------------------------------------------- END TEXT PROPOSAL ----------------------------------------------------------