3GPP TSG-RAN WG4 Meeting #80 	R4-165561

# Gothenburg, Sweden, 22 – 26 Aug, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3723 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce PDCCH test for LAA demodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2016-08-5 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In RAN4#79 Ad hoc meeting, PDCCH test is agreed to be included in LAA demodulation. This agreement is not captured in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce PDCCH test for LAA demodulation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no PDCCH test in LAA demodulation |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | Y |  | Test specifications |  |  | TS36.521 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

### 8.4.3	LAA

The parameters specified in Table 8.4.3-1 are valid for all LAA PDCCH tests unless otherwise stated.

Table 8.4.3-1: Common test Parameters for PDCCH

| Parameter |  | Unit | Transmit diversity |
| --- | --- | --- | --- |
| Downlink power allocation (Note 1) | PDCCH_RA PHICH_RA OCNG_RA | dB | -3 |
|  | PCFICH_RB PDCCH_RB PHICH_RB OCNG_RB | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| PHICH Ng (Note 1) |  |  | 1 |
| PHICH duration |  |  | Normal |
| Unused RE-s and PRB-s (Note 2) |  |  | OCNG |
| Cell ID |  |  | 0 |
| Cyclic prefix |  |  | Normal |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: In LAA Scell(s), PCFICH_RB, PHICH_RA, and PHICH_RB are not available. Note 2: OCNG is applied only within LAA burst. |  |  |  |

#### 8.4.3.1	Transmit diversity performance

##### 8.4.3.1.1	FDD Pcell (FDD single carrier)

##### 8.4.3.1.1.1	Minimum Requirement 2 Tx Antenna Port

The average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.3.1.1.1-2 for Pcell and in Table 8.4.3.1.1.1-3 for LAA Scell(s), with the addition of the parameters in Table 8.4.3-1, and Table 8.4.3.1.1.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.3.1.1.1-1: Test Parameters for LAA Scell(s)

| Parameter | Unit | Value |
| --- | --- | --- |
| DMTC Periodicity | ms | 80 |
| dmtc-PeriodOffset-r12 ms80-r12 |  | [0] |
| Downlink Burst transmission pattern for LAA SCell |  | As specified in B.7 |
| The number of subframes set (S1) in a burst |  | {1,3, 5, 8} |
| subframeStartPosition |  | ‘s07’ |
| Occupied OFDM symbols set in the last subframe |  | {6, 9, 12,14} |
| timing error relative of LAA SCell to PCell | Μs | TBD |
| Frequency offset of th i-th LAA SCell relative to PCell | Hz | [200] |
| Note 1: The same PDSCH transmission mode is applied to each component carrier. Note 2: The OCNG shall be applied for the non-scheduled OFDM symbols within the burst, and which OFDM symbols are scheduled within the burst is according to UE capability. |  |  |

Table 8.4.3.1.1.1-2: Single carrier performance for CCs which are not LAA Scells for multiple CA configurations

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 CCE | R.16 FDD | OP.1 TDD | EVA5 | 2 x 2 Low | 1 | -0.6 |

Table 8.4.3.1.1.1-3: Single carrier performance for LAA Scell(s) for multiple CA configurations

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 MHz | 4 CCE | R.16-1 FDD | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | TBD |
| 2 | 20 MHz | 4 CCE | R.16-1 FDD | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | TBD |
| 3 | 20 MHz | 4 CCE | R.16-1 FDD | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | TBD |
| 4 | 20 MHz | 4 CCE | R.16-1 FDD | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | TBD |
| Note1： UE is required to fulfill only one test among test 1-4 depending on it’s capability for endingDwPTS and secondSlotStartingPosition. For UE don’t support endingDwPTS and secondSlotStartingPosition, it is required to fulfill test 1; For UE don’t support endingDwPTS and support secondSlotStartingPosition, it is required to fulfill test 2; For UE support endingDwPTS and don’t support secondSlotStartingPosition, it is required to fulfil test 3; and For UE support both endingDwPTS and secondSlotStartingPosition, it is required to fulfill test 4. |  |  |  |  |  |  |  |  |

##### 8.4.3.1.2	TDD Pcell (TDD single carrier)

##### 8.4.3.1.2.1	Minimum Requirement 2 Tx Antenna Port

The average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.4.3.1.2.1-2 for Pcell and in Table 8.4.3.1.2.1-3 for LAA Scell(s), with the additional of the parameters in Table 8.4.3-1, and Table 8.4.3.1.2.1-1. The downlink physical setup is in accordance with Annex C.3.2.

Table 8.4.3.1.2.1-1: Test Parameters for LAA Scell(s)

| Parameter | Unit | Value |
| --- | --- | --- |
| DMTC Periodicity | ms | 80 |
| dmtc-PeriodOffset-r12 ms80-r12 |  | [0] |
| Downlink Burst transmission pattern for LAA SCell |  | As specified in B.7 |
| The number of subframes set (S1) in a burst |  | {1,3, 5, 8} |
| subframeStartPosition |  | ‘s07’ |
| Occupied OFDM symbols set in the last subframe |  | {6, 9, 12,14} |
| timing error relative of LAA SCell to PCell | μs | TBD |
| Frequency offset of th i-th LAA SCell relative to PCell | Hz | [200] |
| Note 1: The same PDSCH transmission mode is applied to each component carrier. Note 2: The OCNG shall be applied for the non-scheduled OFDM symbols within the burst, and which OFDM symobls are scheduled within the burst is according to UE capability. |  |  |

Table 8.4.3.1.2.1-2: Single carrier performance for CCs which are not LAA Scells for multiple CA configurations

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 MHz | 4 CCE | R.16 FDD | OP.1 TDD | EVA5 | 2 x 2 Low | 1 | -0.6 |

Table 8.4.3.1.2.1-3: Single carrier performance for LAA Scell(s) for multiple CA configurations

| Test number | Bandwidth | Aggregation level | Reference Channel | OCNG Pattern | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 20 MHz | 4 CCE | R.16-1 TDD | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | TBD |
| 2 | 20 MHz | 4 CCE | R.16-1 TDD | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | TBD |
| 3 | 20 MHz | 4 CCE | R.16-1 TDD | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | TBD |
| 4 | 20 MHz | 4 CCE | R.16-1 TDD | OP.1 FS3 | EVA5 | 2 x 2 Low | 1 | TBD |
| Note1： UE is required to fulfill only one test among test 1-4 depending on it’s capability for endingDwPTS and secondSlotStartingPosition. For UE don’t support endingDwPTS and secondSlotStartingPosition, it is required to fulfill test 1; For UE don’t support endingDwPTS and support secondSlotStartingPosition, it is required to fulfill test 2; For UE support endingDwPTS and don’t support secondSlotStartingPosition, it is required to fulfil test 3; and For UE support both endingDwPTS and secondSlotStartingPosition, it is required to fulfill test 4. |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

### A.3.5.1	FDD

Table A.3.5.1-1: Reference Channel FDD

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.15 FDD | R.15-1 FDD | R.15-2 FDD | R.16 FDD | R.17 FDD |
| Number of transmitter antennas |  | 1 | 2 | 2 | 2 | 4 |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 5 |
| Number of OFDM symbols for PDCCH | symbols | 2 | 3 | 2 | 2 | 2 |
| Aggregation level | CCE | 8 | 8 | 8 | 4 | 2 |
| DCI Format |  | Format 1 | Format 1 | Format 1 | Format 2 | Format 2 |
| Cell ID |  | 0 | 0 | 0 | 0 | 0 |
| Payload (without CRC) | Bits | 31 | 31 | 31 | 43 | 42 |

Table A.3.5.1-2: Reference Channel FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.16-1 FDD |
| Number of transmitter antennas |  | 2 |
| Channel bandwidth | MHz | 10 |
| Number of OFDM symbols for PDCCH | symbols | 2 |
| Aggregation level | CCE | 4 |
| DCI Format |  | Format 2A |
| Cell ID |  | 0 |
| Payload (without CRC) | Bits | 48 |

### A.3.5.2	TDD

Table A.3.5.2-1: Reference Channel TDD

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.15 TDD | R.15-1 TDD | R.15-2 TDD | R.16 TDD | R.17 TDD |
| Number of transmitter antennas |  | 1 | 2 | 2 | 2 | 4 |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 | 5 |
| Number of OFDM symbols for PDCCH | symbols | 2 | 3 | 2 | 2 | 2 |
| Aggregation level | CCE | 8 | 8 | 8 | 4 | 2 |
| DCI Format |  | Format 1 | Format 1 | Format 1 | Format 2 | Format 2 |
| Cell ID |  | 0 | 0 | 0 | 0 | 0 |
| Payload (without CRC) | Bits | 34 | 34 | 34 | 46 | 45 |

Table A.3.5.2-2: Reference Channel TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.16-1 TDD |
| Number of transmitter antennas |  | 2 |
| Channel bandwidth | MHz | 10 |
| Number of OFDM symbols for PDCCH | symbols | 2 |
| Aggregation level | CCE | 4 |
| DCI Format |  | Format 2A |
| Cell ID |  | 0 |
| Payload (without CRC) | Bits | 51 |

## A.5.3	OCNG Patterns for frame structure type 3

The following OCNG patterns are used for modelling allocations to virtual UEs (which are not under test). The OCNG pattern for each sub frame specifies the allocations that shall be filled with OCNG, and furthermore, the relative power level of each such allocation.

In each test case the OCNG is expressed by parameters OCNG_RA and OCNG_RB which together with a relative power level () specifies the PDSCH EPRE-to-RS EPRE ratios in OFDM symbols with and without reference symbols, respectively. The relative power, which is used for modelling boosting per virtual UE allocation, is expressed by:

where  denotes the relative power level of the i:th virtual UE. The parameter settings of OCNG_RA, OCNG_RB, and the set of relative power levels are chosen such that when also taking allocations to the UE under test into account, as given by a PDSCH reference channel, a constant transmitted power spectral density that is constant on an OFDM symbol basis is targeted.

Moreover the OCNG pattern is accompanied by a PDCCH reference channel which specifies the control region. For any aggregationthe PDCCH are padded with resource element groups with a power level given respectively by PDCCH_RA/RB as specified in the test case such that a total power spectral density in the control region that is constant on an OFDM symbol basis is targeted.

For the performance requirements of UE with the CA capability, the OCNG patterns apply for eachLAA Scell.

### A.5.3.1	OCNG FS3 pattern 1: One sided dynamic OCNG frame structure type 3 pattern

This OCNG Pattern fills with OCNG all empty PRB-s (PRB-s with no allocation of data or system information) of the DL sub-frames, when the unallocated area is continuous in frequency domain (one sided).

Table A.5.1.1-1: OP.1 FS3: One sided dynamic OCNG frame structure type 3 Pattern

| Relative power level [dB] |  |  | PDSCH Data |
| --- | --- | --- | --- |
| Subframe |  |  |  |
| 0 | 5 | 1 – 4, 6 – 9 |  |
| Allocation |  |  |  |
| First unallocated PRB – Last unallocated PRB | First unallocated PRB – Last unallocated PRB | First unallocated PRB – Last unallocated PRB |  |
| 0 | 0 | 0 | Note 1 |
| Note 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of PDSCH. Note 2: If two or more transmit antennas with CRS are used in the test, the OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS according to transmission mode 2. The parameter applies to each antenna port separately, so the transmit power is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in section 7.1 in 3GPP TS 36.213. Note 3： Subframes available for DL transmission and Occupied OFDM symbols in each subframe depend on the downlink burst transmission pattern and its corresponding configuration |  |  |  |

-----------------------------------------------------------------End of Change ------------------------------------------------------------