3GPP TSG-RAN4 Meeting #83	R4-1706028

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.171 | CR | 0014 | rev | 1 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | InDoPos: New WLAN delay test case in dynamic range conditions with LTE FDD and TDD (Rel-14) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_LTE_iPos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2017-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the last R4 meetings, WLAN AP identifications core requirements have been agreed in R4-1702881, which include also delay reporting time in reference sensitivity and dyamic range conditions. The agreed WP in R4-1704295 prescribes TC for nominal conditions, but leaves open (TBD) the decision for TC in other conditions. In our opinion dynamic range conditions represent a very common use case in reality and shall be tested. The added value of this test increases when considering that it also covers indirectly reference sensitivity conditions, making the necessity for such test redundant . |  |  |  |  |  |  |  |  |
| Summary of change: |  | WLAN test requirements for dynmic range conditions have been added. The test cases evaluate the ability of a UE to identify and report WLAN AP within the maximum response time under dynamic range conditions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WLAN test requirements for dynamic range conditions (including sensitivity conditions) will remain untested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.X.4/5 (new subclauses) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 37.571-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

# A.3	UE Measurement Procedures

< Unchanged sections omitted >

#### A.3.X.4	LTE-FDD: WLAN AP Identification and reporting delay under dynamic range conditions test

##### A.3.X.4.1 Test purpose and Environment

The purpose of this test is to verify the requirements in Clause 7.4 for WLAN AP measurements.

In this test, there are cell1 (E-UTRAN FDD) and [3] WLAN APs transmitting beacon signals at least every 102.3 ms. The APs are transmitting in 3 non-overlapping frequency channels in the same WLAN Frequency Band. There is [1] AP in every channel. The test consists of two successive time periods, with duration of T1 and T2, respectively. WLAN-RequestLocationInformation message shall be provided to the UE during T1. WLAN Access Points only transmit signal during T2.

Table A.3.X.4.1-1: General test parameters for WLAN AP Identification and reporting delay under dynamic range conditions test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Number of Access Points | N/A | 3 | AP1-AP3 |
| Time Slot 1 | ms | 1 | AP1, AP2, AP3 |
| Beacon Interval | ms | [20] | [In order to ensure that the UE is in passive scan mode] |
| T1 | s | 5 | During this time the WLAN signal is not transmitted |
| T2 | s | 35 | UE shall report WLAN measurement information within 30s |

Table A.3.X.4.1-2: E-UTRAN FDD Cell specific test parameters for WLAN AP Identification and reporting delay under dynamic range conditions test

| Parameter | Unit | Cell 1 |  | AP 1 |  | AP 2 |  | AP 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | N/A |  | N/A |  | N/A |  |
| WLAN Channel Number |  | N/A |  | 1 |  | 2 |  | 3 |  |
| BWchannel |  | 10MHz |  | 20 MHz |  | 20 MHz |  | 20 MHz |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 FDD |  | N/A |  | N/A |  | N/A |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 FDD |  | N/A |  | N/A |  | N/A |  |
| OCNG Patterns |  | OP.1 FDD |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA | dB | 0 |  | N/A |  | N/A |  | N/A |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Noc1Note 2 | dBm/15 KHz | -98 |  | N/A |  | N/A |  | N/A |  |
| Noc2Note 3 | dBm/20 MHz | N/A |  | [-85] |  | [-85] |  | [-85] |  |
| Ês/Noc1 | dB | 3 | 3 | N/A |  | N/A |  | N/A |  |
| Ês/Iot Note 4 | dB | 3 | 3 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -95 | -95 |  |  |  |  |  |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 | -95 |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -65.5 | -65.5 |  |  |  |  |  |  |
| WLAN Received Power Level | dBm/20 MHz | N/A | N/A | - inf | -70 | - inf | -47 | - inf | -70 |
| WLAN SNRNote 4 | db | N/A |  | [15] |  | [38] |  | [15] |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  | - |  | - |  | - |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc1 to be fulfilled. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over the bandwidth and time and shall be modelled as AWGN of appropriate power for Noc2 to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP, Io and WLAN SNR have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |  |

##### A.3.X.4.2 Test Requirements

The WLAN Response Time shall fulfil the requirements in section 4.3 and the WLAN AP report shall fulfil the requirements in section 7.4.

#### A.3.X.5	LTE-TDD: WLAN AP Identification and reporting delay under dynamic range conditions test

##### A.3.X.5.1 Test purpose and Environment

The purpose of this test is to verify the requirements in Clause 7.4 for WLAN AP measurements.

In this test, there are cell1 (E-UTRAN TDD) and [3] WLAN APs transmitting beacon signals at least every 102.3 ms. The APs are transmitting in 3 non-overlapping frequency channels in the same WLAN Frequency Band. There is [1] AP in every channel. The test consists of two successive time periods, with duration of T1 and T2, respectively. WLAN-RequestLocationInformation message shall be provided to the UE during T1. WLAN Access Points only transmit signal during T2.

Table A.3.X.5.1-1: General test parameters for WLAN AP Identification and reporting delay under dynamic range conditions test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Number of Access Points | N/A | 3 | AP1-AP3 |
| Time Slot 1 | ms | 1 | AP1, AP2, AP3 |
| Beacon Interval | ms | [20] | [In order to ensure that the UE is in passive scan mode] |
| T1 | s | 5 | During this time the WLAN signal is not transmitted |
| T2 | s | 35 | UE shall report WLAN measurement information within 30s |

Table A.3.X.5.1-2: E-UTRAN TDD Cell specific test parameters for WLAN AP Identification and reporting delay under dynamic range conditions test

| Parameter | Unit | Cell 1 |  | AP 1 |  | AP 2 |  | AP 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | N/A |  | N/A |  | N/A |  |
| WLAN Channel Number |  | N/A |  | 1 |  | 2 |  | 3 |  |
| BWchannel |  | 10MHz |  | 20 MHz |  | 20 MHz |  | 20 MHz |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 TDD |  | N/A |  | N/A |  | N/A |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 TDD |  | N/A |  | N/A |  | N/A |  |
| OCNG Patterns |  | OP.1 TDD |  | N/A |  | N/A |  | N/A |  |
| PBCH_RA | dB | 0 |  | N/A |  | N/A |  | N/A |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Noc1Note 2 | dBm/15 KHz | -98 |  | N/A |  | N/A |  | N/A |  |
| Noc2Note 3 | dBm/20 MHz | N/A |  | [-85] |  | [-85] |  | [-85] |  |
| Ês/Noc1 | dB | 3 | 3 | N/A |  | N/A |  | N/A |  |
| Ês/Iot Note 4 | dB | 3 | 3 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -95 | -95 |  |  |  |  |  |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 | -95 |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -65.5 | -65.5 |  |  |  |  |  |  |
| WLAN Received Power Level | dBm/20 MHz | N/A | N/A | - inf | -70 | - inf | -47 | - inf | -70 |
| WLAN SNRNote 4 | db | N/A |  | [15] |  | [38] |  | [15] |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  | - |  | - |  | - |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc1 to be fulfilled. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over the bandwidth and time and shall be modelled as AWGN of appropriate power for Noc2 to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP, Io and WLAN SNR have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |  |

##### A.3.X.5.2 Test Requirements

The WLAN Response Time shall fulfil the requirements in section 4.3 and the WLAN AP report shall fulfil the requirements in section 7.4.

< End of changes >