3GPP TSG-RAN WG4 Meeting #83                                                     		R4-1706155

Hangzhou, China, 15 - 19 May, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 37.105 | CR | 0060 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.105: Correction of the spurious emissions requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Core |  |  |  |  |  |  | Date: |  | 2017-04-20 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This is Cat. A CR for the Rel-14. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Additonal reference added for the spurious emissions regional requirement (based on R4-1701206) - Correction of wrong reference to SM.329 - Texrt correction to address various OBUE/SEM exclusion zones, dependinf on the RAT considered - Minor editorial corrections and text improvements - Text modification in sections 7.6.2.1 and 7.6.3 were aligned |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UTRA SEM frequency limits would not be properly addressed for the spurious emissions requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5, 6.6.1, 6.6.6.1, 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---------------------------------- Modified section ----------------------------------

## 4.5	Regional Requirements

Some requirements in the present document may only apply in certain regions either as optional requirements, or as mandatory requirements set by local and regional regulation. It is normally not stated in the 3GPP specifications under what exact circumstances the regional requirements apply, since this is defined by local or regional regulation.

Table 4.5-1 lists all requirements in the present specification that may be applied differently in different regions. Non-AAS BS requirements are applicable as defined in the present document. In many cases, such requirements include regional requirements that are implicitly referenced from the present specification, and listed in the specification for the specifications concerned 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] and 3GPP TS 37.104 [5].

Table 4.5-1: List of regional requirements

| Clause number | Requirement | Comments |
| --- | --- | --- |
| 4.6 | Operating bands and Band Categories | Some bands may be applied regionally. |
| 6.2.2.4.2 | Base station output power | These requirements apply in Japan for an E-UTRA BS operating in band 34 and Band 41. |
| 6.6.2 | Occupied bandwidth | The requirement may be applied regionally. There may also be regional requirements to declare the Occupied bandwidth according to the definition. |
| 6.6.4 | Spectrum emission mask | The mask specified may be mandatory in certain regions. In other regions this mask may not be applied. Additional spectrum protection requirements may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | The BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied and under the conditions declared by the manufacturer. |
| 6.6.5 | Operating band unwanted emissions | The requirements for unsynchronized TDD co-existence may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | The requirements for protection of DTT may apply regionally. |
| 6.6.5 | Operating band unwanted emissions | Regional requirement as defined in 3GPP TS 37.104, subclause 6.6.2.4.4 [9] may be applied for the protection of systems operating in frequency bands adjacent to band 1 as defined in 3GPP TS 37.104, subclause 4.5, [9] in geographic areas in which both an adjacent band service and UTRA and/or E-UTRA are deployed. |
| 6.6.5 | Operating band unwanted emissions | Additional requirements for band 41 may apply in certain regions as additional operating band unwanted emission limits. |
| 6.6.5 | Operating band unwanted emissions | Additional band 32 unwanted emissions requirements may apply in certain regions |
| 6.6.6 | Spurious emissions | Category A limits are mandatory for regions where Category A limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [14] apply. Category B limits are mandatory for regions where Category B limits for spurious emissions, as defined in Recommendation ITU-R SM.329 [14] apply. |
| 6.6.6 | Spurious emissions | Additional spurious emissions requirements may be applied for the protection of system operating in frequency ranges other than the AAS BS operating band as described in 3GPP TS 37.104 [9] subclause 6.6.1.3 (NOTE). |
| 6.6.6 | Spurious emissions | In addition to 3GPP requirements, the BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer. |
| 6.6.6 | Spurious emissions | Co-location spurious emissions requirements may be applied for the protection of other BS receivers when an MSR BS operating in another frequency band is co-located with an AAS BS. |
| 6.7 | Transmitter intermodulation | Additional requirements may apply in certain regions. |
| 7.5 | Blocking | Co-location blocking requirements may be applied for the protection of the BS receiver when a BS operating in another frequency band is co-located with an AAS BS. |
| NOTE: AAS BS does not support Band 46 operation, but additional spurious emissions requirements for Band 46 as described in 3GPP TS 37.104 [9] subclause 6.6.1.3, are still applicable for AAS BS for protection of Band 46 operation. |  |  |

---------------------------------- Next modified section ----------------------------------

## 6.6	Unwanted Emissions

### 6.6.1	General

Unwanted emissions consist of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 [14]. In ITU terminology, out of band emissions are unwanted emissions immediately outside the channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

For AAS BS in single RAT E-UTRA operation and MSR operation, the out-of-band emissions requirement for the AAS BS transmitter is specified in terms of an operating band unwanted emissions requirement that defines limits for emissions in each supported downlink operating band plus the frequency ranges 10 MHz above and 10 MHz below each band. Emissions outside of this frequency range are limited by a spurious emissions requirement. For UTRA FDD single RAT AAS BS, the out of band emission requirement for AAS BS transmitter is specified in terms of spectrum emission mask requirement.

The unwanted emission level limit of a TAB connector TX min cell group is in general defined by the unwanted emission basic limit which is the same as the corresponding applicable non-AAS BS per transmitter requirement specified in 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] or 3GPP TS 37.104 [5], and its scaling by NTXU,countedpercell. The unwanted emission requirements are applied per the TAB connector TX min cell groups for all the configurations supported by the AAS BS. The basic limits and corresponding scaling are defined in each relevant subclause.

There is in addition a requirement for occupied bandwidth and an ACLR requirement.

---------------------------------- Next modified section ----------------------------------

### 6.6.6	Spurious emission

#### 6.6.6.1	General

The conducted transmitter spurious emission limits apply from 9 kHz to 12.75 GHz, excluding the following RAT-specific frequency ranges:

- UTRA TDD BS, 1.28 Mcps option as specified in TS 25.105 [3]: from 4 MHz below the lowest frequency of each operating band to 4 MHz above the highest frequency of each operating band.

- UTRA FDD BS as specified in TS 25.104 [2]: from 12.5MHz below the lowest carrier frequency used up to 12.5MHz above the highest carrier frequency used.

- E-UTRA BS as specified in TS 36.104 [4]: from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band.

- MSR BS as specified in TS 37.104 [5]: from 10 MHz below the lowest frequency of the downlink operating band up to 10 MHz above the highest frequency of the downlink operating band.

For some operating bands the upper frequency limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329 [14]. In some exceptional cases, requirements apply also closer than 10 MHz from the downlink operating band; these cases are highlighted in the requirement tables in respective referenced UTRA, E-UTRA or MSR specifications. For operating bands supported by multi-band TAB connectors exclusion bands apply to each supported band.

The requirements applies for both single band TAB connectors and multi-band TAB connectors (except for frequencies at which exclusion bands or other multi-band provisions apply) and for all transmission modes foreseen by the manufacturer's specification. Unless otherwise stated, all requirements are measured as mean power.

For operation in Region 2, where the FCC guidance for MIMO systems in [18] is applicable, NTXU,countedpercell shall be equal to 1 for the purposes of calculating the spurious emissions limits in subclauses 6.6.6.2, 6.6.6.3 or 6.6.6.4. For all other unwanted emissions requirements, NTXU,countedpercell shall be the value calculated according to subclause 6.1.

The AAS BS requirements for spurious emissions limits which are specified for Band 46 in 3GPP TS 37.104 [5], are applicable for AAS BS.

---------------------------------- Next modified section ----------------------------------

## 7.6	Receiver spurious emissions

### 7.6.1	General

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at the TAB connector. The requirements apply to all AAS BS with separate RX and TX TAB connectors.

NOTE:	In this case for FDD AAS BS the test is performed when both TX and RX are ON, with the TX TAB connector terminated.

For a TAB connector supporting both RX and TX in TDD, the requirements apply during the transmitter OFF period. For a TAB connector supporting both RX and TX in FDD, the receiver spurious requirements are superseded by the TX spurious requirements in subclause 6.6.6.

For RX only multi-band TAB connector(s), the RX spurious emissions requirements are subject to exclusion zones in each supported operating band. For multi-band TAB connector(s) that both transmit and receive in operating band supporting TDD, RX spurious emissions requirements are applicable during the TX OFF period, and are subject to exclusion zones in each supported operating band. The unwanted emission level limit of a TAB connector RX min cell group is in general defined by the unwanted emission basic limit which is the same as the corresponding applicable non-AAS BS per transmitter requirement specified in 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] or 3GPP TS 37.104 [5], and its scaling by NRXU,countedpercell. The basic limits and corresponding scaling are defined in each relevant subclause. The receiver spurious emission requirements are applied per the TAB connector RX min cell groups for all the configurations supported by the AAS BS.

### 7.6.2	Minimum requirement for MSR operation

#### 7.6.2.1	General minimum requirement

The general MSR RX spurious emission basic limits are provided in table 7.6.2.1-1.

Table 7.6.2.1-1: General spurious emissions basic limits

| Frequency range | Maximum level | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30MHz - 1 GHz | -57 dBm | 100 kHz |  |
| 1 GHz - 12.75 GHz | -47 dBm | 1 MHz |  |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | Applies only for Bands 22, 42 and 43. |
| NOTE: The frequency range from FBW RF,DL,low -10 MHz to FBW RF,DL,high + 10 MHz may be excluded from the requirement. For multi-band TAB connectors, the exclusion applies for all supported operating bands for those connectors. |  |  |  |

The RX spurious emissions requirements for an MSR AAS BS are that for each applicable basic limit specified in table 7.6.2.1-1 for each TAB connector RX min cell group, the power sum of emissions at the TAB connectors of the TAB connector RX min cell group shall not exceed an AAS BS limit specified as the basic limits + 10log10(NRXU,countedpercell).

In addition to the basic limits in Table 7.6.2.1-1, additional spurious emissions requirements in 3GPP TS 37.104 [9], subclause 6.6.1.3 form basic limits for additional receiver spurious emission requirements.

In case of FDD BS (for BC1 and BC2), the levels specified for Protection of the BS receivers of own or different BS in 3GPP TS 37.104 [9], subclause 6.6.1.2 form basic levels for additional receiver spurious emission requirements.

In addition, the requirements for co-location with other base stations specified in 3GPP TS 37.104 [9], subclause 6.6.1.4 may also form basic levels for co-location spurious emission requirements.

NOTE:	Conformance to the AAS BS receiver spurious emissions requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the spurious emissions power measured on each TAB connector in the TAB connector RX min cell group shall be less than or equal to the AAS BS limit as defined above for the respective frequency span.

Or

2)	The spurious emissions power at each TAB connector shall be less than or equal to the AAS BS limit as defined above for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector RX min cell group.

### 7.6.3	Minimum requirement for single RAT UTRA operation

The single RAT UTRA FDD wide area, medium range area and local area RX spurious emissions basic limits are the same as those specified in 3GPP TS 25.104 [6], subclause 7.7.1.

The single RAT UTRA TDD wide area and local area RX spurious emissions basic limits are the same as those specified in 3GPP TS 25.105 [7], subclause 7.7.1.2.

The RX spurious emissions requirements for a single RAT UTRA AAS BS are that for each applicable basic limit as specified in 3GPP TS 25.104 [6] or 3GPP TS 25.105 [7], for each TAB connector RX min cell group, the power sum of emissions at the TAB connectors of the TAB connector RX min cell group shall not exceed an AAS BS limit specified as the basic limits + 10log10(NRXU,countedpercell).

NOTE:	Conformance to the AAS BS receiver spurious emissions requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the spurious emissions power measured on each TAB connector in the TAB connector RX min cell group shall be less than or equal to the AAS BS limit as defined above for the respective frequency span.

Or

2)	The spurious emission power at each TAB connector shall be less than or equal to the AAS BS limit as defined above for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector RX min cell group.

### 7.6.4	Minimum requirement for single RAT E-UTRA operation

The single RAT E-UTRA wide area, medium range area and local area RX spurious emissions basic limits are the same as those specified in 3GPP TS 36.104 [8], subclause 7.7.1.

The RX spurious emissions requirements for a single RAT E-UTRA AAS BS are that for each applicable basic limit specified in 3GPP TS 36.104 [4] for each TAB connector RX min cell group, the power sum of emissions at the TAB connectors of the TAB connector RX min cell group shall not exceed an AAS limit specified as the basic limits + 10log10(NRXU,countedpercell).

NOTE:	Conformance to the AAS BS receiver spurious emissions requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector RX min cell group shall be less than or equal to the AAS BS limit as defined above for the respective frequency span.

Or

2)	The spurious emission power at each TAB connector shall be less than or equal to the AAS BS limit as defined above for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector RX min cell group.

---------------------------------- End of modified section ----------------------------------