3GPP TSG RAN Meeting 76                                               	RP-171447

West Palm Beach, USA, 5th – 9th June 2017

Source: 	Thales, CTTC, CNES, Telekom R&D Sdn. Bhd., STMicroelectronics, HNS, Avanti, ESA, Dish Network, Fraunhofer IIS

Title:	Non-Terrestrial Network overview

TDOC type:	Discussion

Agenda Item:	9.3.1  (sFS_NR_nonterr_nw)

Document for:	Approval

Release:	Rel-15

# Abstract

The objective of this document is to propose a text for the 3rd and 4th clauses of the TR 38.811 “Study on NR to support Non-Terrestrial Networks” to be drafted as part of the study item NR-NTN.

# Discussion

The chapter “Non-Terrestrial Networks overview” aims at identifying the potential role and use cases of NTN in 5G system, and providing architecture types to down select the reference deployment scenarios.

# Proposed text for approval

It is proposed to add the following texts to TR 38.811 “Study on NR to support Non-Terrestrial Networks”.

* * * Start of changes * * * * (new text)

# 2. References

[1]	3GPP TR 38.913: "Study on Scenarios and Requirements for  Next Generation Access Technologies (Release 14)".

[2]	3GPP TS 36.101: "Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 14)".

[3] 3GPP TR 38.801: “Technical Specification Group Radio Access Network; Study on new radio access technology: Radio access architecture and interfaces (Release 14)”

[4]	3GPP TR 38.804: "Technical Specification Group Radio Access Network; Study on New Radio Access Technology; Radio Interface Protocol Aspects (Release 14) ".

# 3. Definitions, symbols and abbreviations

## 3.1. Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aerial: an airborne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, typically at an altitude between 8 to 50 km.

Airborne vehicles: High Altitude Platforms (HAPs) encompassing Unmanned Aircraft Systems (UAS) including tethered UAS and Lighter than Air UAS (LTA), Heavier than Air UAS (HTA), all operating in altitude typically between 8 and 50 km

Availability : % of time during which the RAN is available for the targeted communication. Unavailable communication for shorter period than [Y] ms shall not be counted. The RAN may contain several access network components among which an NTN to achieve multi-connectivity or link aggregation.

Beam throughput: data rate provided in a beam

Bentpipe payload: payload that changes the frequency carrier of the uplink RF signal, filters and amplifies it before transmitting it on the downlink

Connectivity: capability to establish and maintain data / voice / video transfer between networks and parts thereof

Geostationary Earth orbit: Circular orbit at 35,786 kilometres above the Earth's equator and following the direction of the Earth's rotation. An object in such an orbit has an orbital period equal to the Earth's rotational period and thus appears motionless, at a fixed position in the sky, to ground observers.

Low Earth Orbit: Orbit around the around Earth with an altitude between 500 kilometers (orbital period of about 88 minutes), and 2,000 kilometres (orbital period of about 127 minutes).

Medium Earth Orbit: region of space around the Earth above low Earth orbit and below geostationary Earth Orbit.

Non Geostationary Satellites: Satellites ( LEO and MEO) orbiting around the Earth with a period that varies approximately between 1.5 hour and 10 hours. It is necessary to have a constellation of several Non Geostationary satellites associated with handover mechanisms to ensure a service continuity.

Non-terrestrial Networks : Networks, or segments of networks, using an airborne or spaceborne vehicle to embark a transmission equipment relay node or base station.

On Board processing: digital processing carried out on uplink RF signals aboard a satellite or an aerial.

One way latency: time required to propagate through the RAN from a terminal to the gateway or from the gateway to the terminal. This is especially used for voice and video conference applications.

Regenerative payload: payload that transforms and amplifies an uplink RF signal before transmitting it on the downlink. The transformation of the signal refers to digital processing that may include demodulation, decoding, re-encoding, re-modulation and/or filtering .

Relay node: Relay of Uu radio interface. The relay function can take place at Layer 1, 2 or 3.

Reliability : probability that the RAN performs in a satisfactory manner for a given period of time when used under specific operating conditions. The RAN may contain several access network components among which an NTN to achieve multi-connectivity or link aggregation.

Round Trip Delay: time required for a network communication to travel from a terminal to the gateway or from the gateway to the terminal and back. This is especially used for web based applications.

Satellite: a spaceborne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO) typically at an altitude between 500 km to 2000 km, Medium-Earth Orbit (MEO) typically at an altitude between 8000 to 20000 km, or Geostationary-satellite Earth Orbit (GEO) at 35 786 km altitude.

Spaceborne vehicles: Satellites including Low Earth Orbiting (LEO) satellites, Medium Earth Orbiting (MEO) satellites, Geostationary Earth Orbiting (GEO) satellites as well as Highly Elliptical Orbiting (HEO) satellites

User Connectivity: capability to establish and maintain data / voice / video transfer between networks and Terminals

User Throughput: data rate provided to a terminal

## 3.2. Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3. Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AWGN:	Additive White Gaussian Noise

BLOS:	Beyond Line of Sight

BW:	Bandwidth

C2:	Control and Command

eMBB:	enhanced Mobile Broadband

FOTA:	Firmware Over The Air services

FSS:	Fixed Satellite Services

GEO:	Geostationary Earth Orbiting

gNB:	next Generation Node B

HD:	High Definition

HEO:	Highly Elliptical Orbiting

IoT:	Internet of Things

KPI:	Key Performance Indicator

IMUX:	Input MUltipleXer

ISL:	Inter-Satellite Links

LEO:	Low Earth Orbiting

LMS:	Land Mobile Satellite

LoS:	Line of Sight

Mbps:	Mega bit per second

MEO:	Medium Earth Orbiting

mMTC:	massive Machine Type Communications

NGSO:	Non Geostationary Satellite Orbit

NT:	Non Terrestrial

NTN: 	Non Terrestrial Network

OMUX:	Output MUltipleXer

POI:	Point of Interest

RAN:	Radio Access Network

RAT:	Radio Access Technology

SOTA:	Software Over The Air services

TV:	Television

UAS:	Unmanned Aerial System

UE:	User Equipment

URLCC:	ultra-Reliable Low latency Communications

# 4. Non-Terrestrial Networks Overview – background information

## 4.1. Roles for Non-Terrestrial networks in 5G system

Thanks to the wide service coverage capabilities and reduced vulnerability of space/airborne vehicles to physical attacks and natural disasters, Non-Terrestrial networks are expected to

- foster the roll out of 5G service in un-served areas that cannot be covered by terrestrial 5G network (isolated/remote areas, on board aircrafts or vessels) and underserved areas (e.g. sub-urban/rural areas) to upgrade the performance of limited terrestrial networks in cost effective manner,

- reinforce the 5G service reliability by providing service continuity for M2M/IoT devices or for passengers on board moving platforms (e.g. passenger vehicles-aircraft, ships, high speed trains, bus) or ensuring service availability anywhere especially for critical communications, future railway/maritime/aeronautical communications, and to

- enable 5G network scalability by providing efficient multicast/broadcast resources for data delivery towards the network edges or even user terminal.

The benefits relate to either Non-Terrestrial networks operating alone or to integrated terrestrial and Non-Terrestrial networks. They will impact Coverage, User Bandwidth, System capacity, Service reliability or Service availability, Energy consumption, Connection density (See [1]).

A role for Non-Terrestrial network components in the 5G system is expected for the following verticals: , Transport, Public Safety, Media and Entertainment, eHealth, Energy, Agriculture, Finance, Automotive.

## 4.2. 5G Use Cases wherein Non-Terrestrial Network components have a role

### 4.2.1. 5G use cases introduction

A use case, typically refers to the interactions between a role and a system, to achieve a specific goal. Hence, it is necessary to identify the goal of the service enabled by a Non-Terrestrial network component integrated in the 5G system.

The tables in the clauses after respectively identify for each of the 5G service enablers, the use cases wherein Non-Terrestrial Network components have a role to play.

- 5G service enablers refer to eMBB (enhanced Mobile Broadband), uRLCC (ultra-Reliable Low latency Communications) and mMTC (massive Machine Type Communications).

- 5G use cases correspond to the interactions between a stakeholder (user, operator, service provider) and the 5G system, to achieve a specific goal.

- The role of the Non-Terrestrial Network refers to services enabled by the Non-Terrestrial Network component in the 5G system to support the use case.

- 3GPP reference documents are provided in which the use cases are mentioned.

Table 1: 5G use cases for Satellite access networks

| 5G Service enabler | 5G Use case | 5G Use case description | Satellite service | 3GPP References |
| --- | --- | --- | --- | --- |
| eMBB | Multi connectivity | Users in underserved areas (home or in Small Offices, big events in ad-hoc built-up facilities) are connected to the 5G network via multiple network technologies and benefit from 50 Mbps+. Delay sensitive traffic may be routed over short latency links while less delay sensitive traffic can be routed over the long latency links. | Broadband connectivity to cells or relay node in underserved areas in combination with terrestrial wireless/cellular or wire line access featuring limited user throughput. | TR 22.864, §5.5: Backhauling TR 22.863, §5.6: Fixed Mobile Convergence TR 22.863, §5.7: Femto cell TR 22.863, §5.4: Higher user mobility TS 22.261 §6, 6.6, 6.7, 6.8, 6.9, 6.10, 6.11, 6.13, 6.15 |
| eMBB | Fixed cell connectivity | Users in isolated villages or industry premises (Mining, off shore platform) access 5G services and benefit from 50 Mbps+. | Broadband connectivity between the core network and the cells in un-served areas (isolated areas). | TR 22.863, §5.3: Deployment and coverage |
| eMBB | Mobile cell connectivity | Passengers on board vessels or aircrafts access 5G services and benefit from 50 Mbps+. | Broadband connectivity between the core network and the cells on board a moving platform (e.g. aircraft or vessels). | TR 22.863, §5.3: Deployment and coverage TS 22.261 |
| eMBB | Network resilience | Some critical network links requires high availability which can be achieved through the aggregation of two or several network connections in parallel. The intent is to prevent the primary network connection outage. | Secondary/backup connection (although potentially limited in capability compared to the primary network connection). | TR 22.862, §5.5: Higher availability TS 22.261 |
| eMBB | Trunking | A network operator may want to deploy or restore (disaster relief) 5G service in an isolated area (not connected to public data network). A network operator may want to interconnect various 5G local access network wide apart | Broadband connectivity between the public data network and a mobile network anchor point or between the anchor points of two mobile networks. | TR 22.863, §5.3: Deployment and coverage TS 22.261 |
| eMBB | Edge network Delivery | Media and entertainment content such as live broadcasts, ad-hoc broadcast/multicast streams, group communications, Mobile Edge Computing’s Virtual Network Function updates are transmitted in multicast mode to a RAN equipment at the network edge where it may be stored in a local cache or further distributed to the User Equipment. The intent is to off load popular content from the mobile network infrastructure (especially at backhaul level). | Broadcast channel to support Multicast delivery to 5G network edges. | TR 22.864, §5.4: Efficient content delivery TS 22.261 (related 6.x) |
| eMBB | Mobile cell hybrid connectivity | Passengers on board public transport vehicles (e.g. high speed/regular trains, buses, river boats) access reliable 5G services. They are served by a base station which is connected by an hybrid cellular/satellite connection. The cellular connectivity may be intermittent and limited user throughput. | Broadband connectivity combined with terrestrial cellular access to connect a cell/group of cells or relay node(s) on board moving platforms. | TR 22.863, §5.3: Deployment and coverage TR 22.862, §5.5: Higher availability TS 22.261 (related to) |
| eMBB | Direct To Node broadcast | TV or multimedia service delivery to home premises or on board a moving platform | Broadcast/Multicast service to access points in homes or on board moving platforms. | TR 22.864, §5.4: Efficient content delivery TS 22.261 (related to) |
| mMTC | Wide area IoT service | Global continuity of service for telematic applications based on a group of sensors/actuators (IoT devices, battery activated or not) scattered over or moving around a wide area and reporting information to or controlled by a central server. These sensors and/or actuators may be used for example the following telematics applications: - Automotive and road transport: high density platooning, HD map updates, Traffic flow optimisation, Vehicle software updates, automotive diagnostic reporting, user base insurance information (e.g. speed limit, driving behaviour), safety status reporting (e.g. air-bag deployment reporting), advertising based revenue, Context awareness information (e.g. neighbouring bargain opportunities based on revenue), remote access functions (e.g. remote door unlocking). - Energy: Critical surveillance of oil/gas infrastructures (e.g. pipeline status) - Transport: Fleet management, asset tracking, digital signage, remote road alerts - Agriculture: Livestock management, farming | Connectivity between IoT devices (battery activated sensors/actuators or not) and spaceborne platform. Continuity of service across spaceborne platforms and terrestrial base stations is needed. | TR 22.861, §5.2: connectivity aspects TR 22.864, §5.6: Access TR 22.862, §5.1: Higher reliability and lower latency TS 22.261 (related to) |
| mMTC | Local area IoT service | Group of sensors that collect local information, connect to each other and report to a central point. The central point may also command a set of actuators to take local actions such as on-off activities or far more complex actions. The sensors/actuators served by a local area network may be located in a smart grid sub-system (Advanced Metering), on board a moving platform (e.g. container on board a vessel, a truck or a train). | Connectivity between mobile core network and base station serving IoT devices in a cell or a group of cells. | TR 22.863, §5.3: Deployment and coverage TS 22.261 (related to) |

Table 2: 5G use cases for Aerial access networks

| 5G Service enabler | 5G Use case | 5G Use case description | Aerial access service | 3GPP References |
| --- | --- | --- | --- | --- |
| eMBB | Hot spot on demand | Users in un/underserved areas (big events) are connected to the 5G network and benefit from 50 Mbps+. | Broadband connectivity to cells or relay node in un/underserved areas. | TR 22.863, §5.3: Deployment and coverage TS 22.261 (related to) |

## 4.3. Satellite and aerial access network architecture principles

Non-Terrestrial access Network typically features the following elements:

- NTN Terminal: It may refer to directly the 3GPP UE or a terminal specific to the satellite system in case the satellite doesn’t serve directly 3GPP UEs.

- A service link which refer to the radio link between the user equipment and the space/airborne platform

- A space or an airborne platform embarking a payload which may implement

- A bent pipe payload: Radio Frequency filtering, Frequency conversion and amplification:

- A regenerative payload: Radio Frequency filtering, Frequency conversion and amplification as well as demodulation/decoding, switch and/or routing, coding/modulation

- Inter satellite/aerial links in case of regenerative payload and a constellation of satellites. They may operate in RF frequency or optical bands

- Gateways that connects the satellite or aerial access network to the core network

- Feeder links which refer to the radio links between the Gateways and the space/airborne platform

We shall distinguish between two types of Satellite access network

- Broadband access network serving Very Small Aperture Terminals that can be fixed or mounted on a moving platform (e.g. bus, train, vessel, aircraft, etc.). Broadband, in such system, refer 50 Mbps+ data rate and even up to several hundred Mbps on the downlink. The service links operate in frequency bands allocated to satellite services (Fixed, Mobile) above 6 GHz.

- Narrowband access network serving terminals equipped with Omni or semi directional antenna (e.g. handheld terminal). Narrowband, in such system, refer to less than 1 or 2 Mbps data rate on the downlink. The service links operate typically in frequency bands allocated to mobile satellite services below 6 GHz.

As per Aerial access network, embarking a gNB, they can serve handheld devices with broadband service.

Based on these principles, the figures below illustrate possible satellite and aerial access network architectures

Figure 1: Satellite access network (No ISL) which service link operates above 6 GHz frequency bands allocated to Fixed and Mobile Satellite Services (FSS and MSS)

Figure 2: Satellite access network which service link operates above 6 GHz frequency bands allocated to Fixed and Mobile Satellite Services (FSS and MSS)

Figure 3: Satellite access network which service link operates below 6 GHz frequency bands allocated to Mobile Satellite Services (MSS)

Figure 4: Aerial access network which service link operates below 6 GHz frequency bands allocated to Mobile Services (MS)

It is recommended to select a range of Deployment scenarios with either bent pipe or regenerative payloads. Note that the impact of ISL or not is beyond the scope of the study because it is assumed that NR will not be considered for such interface as the cost constraints of on board terminal are different from the VSAT or handheld terminals.

## 4.4. Characteristics of NTN Terminals for satellite / aerial access network

The following table gives the minimum RF characteristics of the terminals operating respectively in Ka band (e.g. very small aperture terminals) and in S band (e.g. handheld terminals).

Table 3: Typical minimum RF characteristics of UE in satellite and aerial access networks

|  | Very Small Aperture Terminal (fixed or mounted on Moving Platforms) | Handheld or IoT devices (3GPP class 3, see [2]) |
| --- | --- | --- |
| Frequency band | Downlink: 19.7-21.2 GHz Uplink: 27.5 – 30.0 GHz | Downlink: 2170-2200 MHz Uplink: 1980-2010 MHz |
| Transmit Power | 2 W (33 dBm) | 200 mW (23 dBm) |
| Antenna type | 60 cm equivalent aperture diameter (circular polarisation) | Omnidirectional antenna (linear polarisation) |
| Antenna gain | Tx: 43.2 dBi Rx: 39.7 dB | Tx and Rx: 0 dBi |
| Noise figure | 1.2 dB | 9 dB |
| EIRP | 45.75 dBW | -7 dBW |
| G/T (Note 1) | 18.5 dB/K | -33.6 dB/K |
| Polarisation (Note 2) | Circular | Linear |

Note 1 : For the computation of G/T or figure of merit, following formula applies in dB:

G/T = Ga – NF – 10*LOG(To+(Ta-To)/(100.1*NF))

where

- Antenna Gain : Ga in dBi

- Ambient Temperature : T0 (usually 290 K)

- Antenna temperature : Ta (Typically 290 K with 0 dBi gain and 150 K with >30 dBi gain)

- Noise Figure : NF in dB

Note 2 : For S band, we assume that the User Equipment features an omni-directional antenna of linear polarization, while the antenna on board spaceborne or airborne platforms features typically circular polarization. Hence a polarization mismatch of 3 dB has to be taken into account for the radio link budget computation. This will impact the UE RF characteristics as below:

- Equivalent EIRP of 20 dBm (-10 dBW) under satellite coverage.

- Equivalent G/T of -36,6 dB/K under satellite coverage.

It is recommended to select at least 2 Deployment scenarios each with both frequency bands (S and Ka bands).

## 4.5. Air/Space borne vehicle characteristics

The table below provides characteristics of aerial and satellite vehicles that are relevant for the purpose of the study item:

Table 4: Typical characteristics of Airborne or Spaceborne vehicles

| Characteristics | Geostationary satellites | Non-Geostationary satellites | Airborne platforms |
| --- | --- | --- | --- |
| Altitude | 35 786 km | Low Earth Orbiting satellites: From 600 km up to 1500 km Medium Earth Orbiting satellites: From 7000 up to 20000 km | Typically from 8 to 50 km |
| Motion | Typically within a cube of 50-100 km side around the theoretical orbital position fixed in terms of elevation/azimuth with respect to a given earth point | We shall assume here only circular orbits around the earth | Typically in motion within TBD km from the theoretical position fixed in terms of elevation/azimuth with respect to a given earth point |
| Elevation angle (angle under which the flying platform is seen by the terminal) | Typically more than 10° for user terminal and more than 5° for gateways |  |  |

The characteristics of the air/spaceborne vehicles create specific Doppler and propagation delay conditions that NR has to cope with.

## 4.6	Coverage pattern of NTN

Satellite or aerial vehicles typically generate several beams over a given area. The foot print of the beams are typically elliptic shape.

The beam footprint may be moving over the earth with the satellite or the aerial vehicle motion on its orbit.

Alternatively, the beam foot print may be earth fixed, in such case some beam pointing mechanisms (mechanical or electronic steering feature) will compensate for the  satellite or the aerial vehicle motion.

Table 5: Typical beam foot print size

| Attributes | GEO | Non-GEO | HAPS |
| --- | --- | --- | --- |
| Beam foot print size in diameter | 200 – 1000 km | 100 – 500 km | 50 - 200 km |

Typical beam patterns of the various NTN are depicted below:

Figure 5: NTN Beam patterns

## 4.7 Non-Terrestrial Network architecture options

Here under are depicted the possible options of NTN architecture in 5G context based on the RAN architecture principles described in [3]:

Figure 6: NTN featuring an access network serving UEs and based on a satellite/aerial with bent pipe payload

In figure 6, the satellite or the aerial will relay a “Satellite friendly” NR signal between the gNB and the UEs in a transparent manner.

Figure 7: NTN featuring an access network serving UEs and based on a satellite/aerial with gNB on board

In figure 7, the satellite or the aerial embarks full or part of a gNB to generate/receive a “Satellite friendly” NR signal to/from the UEs. This requires sufficient on board processing capabilities to be able to embark gNB or a Relay Node functions

Figure 8: NTN featuring an access network serving Relay Nodes and based on a satellite/aerial with bent pipe payload

In figure 8, the satellite or the aerial will relay a “Satellite friendly” NR signal between the gNB and the Relay Nodes in a transparent manner.

Figure 9: NTN featuring an access network serving Relay Nodes and based on a satellite/aerial with gNB

In figure 9, the satellite or the aerial embarks full or part of a gNB to generate/receive a “Satellite friendly” NR signal to/from the Relay Nodes. This requires sufficient on board processing capabilities to be able to embark gNB or a Relay Node functions.

Table 6: 5G system elements mapping in NTN architecture

|  | 5G elements - NTN elements mapping |  |  |
| --- | --- | --- | --- |
| NTN architecture options | NTN Terminal | NTN vehicle | NTN Gateway |
| A1: access network serving UEs via bentpipe satellite/aerial | UE | Remote Radio Head (Bent pipe relay of Uu radio interface signals) | gNB |
| A2: access network serving UEs with gNB on board satellite/aerial | UE | gNB or Relay Node functions | Router interfacing to Core network |
| A3: access network serving Relay Nodes via bent pipe satellite/aerial | Relay Node | Remote Radio Head (Bent pipe relay of Uu radio interface signals) | gNB |
| A4: access network serving Relay Nodes with gNB on board satellite/aerial | Relay Node | gNB or Relay Node functions | Router interfacing to Core network |

* * * End of Changes * * * *