3GPP TSG-RAN WG4 Meeting #82bis		was R4-1703953

Spokane, WA, US, 3 - 7 April, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 38.803 | CR | 0006 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.803: RRM requirements cleanup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-03-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During previous RAN4#82 meeting, the first version of the RAN4 NR TR 38.803 was agreed. Number of misalignments with NR SI technical reports from other RAN WGs were identified for the RRM and testability sections in TR 38.803. This Cat. F CR implements cleanup and text corrections of the RRM requirements section and testability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Terminology alignment with NR SI technical reports from other RAN WGs - Corrections in the table for power consumption predictions model - Cleanup and text corrections, missing abbreviations added rev.1: WI code corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Text improvements and alignments across the first release of the TR would be needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 9, 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---------------------------------- Modified section ----------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP RP-160671: "New SID Proposal: Study on New Radio Access Technology".

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	Recommendation ITU-R SM.328-10, “Spectra and Bandwidth of Emissions”.

[4]	Recommendation ITU-R SM.329-12, “Unwanted emissions in the spurious domain”

[5]	“International Telecommunications Union Radio Regulations”, Edition 2016, Volume 1 – Articles, ITU.

[6]	“Title 47 of the Code of Federal Regulations (CFR)”, Federal Communications Commission.

[7]	Recommendation ITU-R SM.1539-1, “Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541 and ITU-R SM.329”.

[8]	Recommendation ITU-R SM.1540, “Unwanted emissions in the out-of-band domain falling into adjacent allocated bands”.

[9]	Recommendation ITU-R SM.1541-6, “Unwanted emissions in the out-of-band domain”.

[10]	Recommendation ITU-R M.2012, “Detailed specifications of the terrestrial radio interfaces of International Mobile Telecommunications-Advanced (IMT-Advanced)”.

[11]	Recommendation ITU-R M.2070, “Generic unwanted emission characteristics of base stations using the terrestrial radio interfaces of IMT-Advanced”.

[12]	Recommendation ITU-R M.2071, “Generic unwanted emission characteristics of mobile stations using the terrestrial radio interfaces of IMT-Advanced”.

[13]	Report ITU-R M.2292, “Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses”.

[14]	ECC Recommendation (02)05, “Unwanted Emissions”, October 2002, amended March 2012.

[15]	CEPT/ERC/RECOMMENDATION 74-01, “Unwanted Emissions in the Spurious Domain”, Cardiff  2011.

[16]	ETSI EN 301 908, “IMT cellular networks; Harmonized EN covering the essential requirements of article 3.2 of the R&TTE Directive;” (22 parts).

[17]	FCC Report and Order and Further Notice of Proposed Rulemaking, “Use of Spectrum Bands Above 24 GHz For Mobile Radio Services…”, FCC 16-89, July 14, 2016.

[18]	3GPP TR 25.942, Technical Report,”3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Frequency (RF) system scenarios”

[19]	3GPP TR 36.942, Technical Report, ”3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios”

[20]	3GPP TR 37.900, Technical Report, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Frequency (RF) requirements for Multicarrier and Multiple Radio Access Technology (Multi-RAT) Base Station (BS)”

[21]           	TR 37.842, Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)

[22]          	R4-166432, AAS and NR BS requirements, Huawei

[23]	R4-164168, "On PA models", Ericsson

[24]		R4-165901, "Further elaboration on PA models for NR", Ericsson.

[25]		R4-163314, “Realistic power amplifier model for the New Radio evaluation”, Nokia

[26]		R4-167263, “PA model using a Memory Polynomial”, Intel

[27]	ECC PT1 (16)083_A31, Liaison Statement on IMT 2020/ 5G spectrum in Europe

[28]	ECC decision (11)06, Harmonised frequency arrangements for mobile/fixed communications networks (MFCN) operating in the bands 3400-3600 MHz and 3600-3800 MHz

[29]	“Flex5Gware Project, “Deliverable 2.1: Requirements and concepts for the analogue HW in 5G mobile systems”, Dec. 2015.

[30]	TR 37.977, Technical Report, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Verification of radiated multi-antenna reception performance of User Equipment (UE)”

[31]	RP-170021, “Reply LS to ITU-R WP5D/374 (Attachment 4.13) on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz” (RAN4).

[32]	R4-1610616,”Way forward on IMT parameters WP5D” (Ericsson).

[33]	R4-1700287, "Way Forward on  BS SEM" (Ericsson, Nokia, Alcatel-Lucent Shanghai Bell).

[34]	R4-1700076, "NR unwanted emissions for BS and UE in ITU-R response" (Ericsson).

[36]	R4-1700302, "Way forward on ACLR and ACS for WP5D LS" (Huawei,Nokia, Ericsson, Qualcomm).

[36]	R4-1700303, "Way Forward on UE ACLR and BS ACS" (Qualcomm Incorporated).

[37]	R4-1700288, "Way Forward on  BS Spurious Emissions" (Ericsson).

[38]	R4-1700279, "Way forward on UE and BS NF for mm-waves" (Ericsson).

[39]	R4-1700268, "WF on BS sensitivity and blocking for WP5 response" (Nokia, Alcatel-Lucent Shanghai Bell, Ericsson).

[40]	R4-1700276, "WF on UE sensitivity blocking response for the ITU-R LS" (Ericsson).

[41]	R4-1700254, "Text improvement of throughput vs SINR mapping" (SAMSUNG Electronics Co., Ltd.).

[42]	R4-1609014, "LS on Characteristics of terrestrial IMT systems for frequency sharing / interference analysis in the frequency range between 24.25 GHz and 86 GHz" (ITU-R WP 5D).

[43]	3GPP TR 38.804, Study on New Radio Access Technology; Radio Interface Protocol Aspects

---------------------------------- Next modified section ----------------------------------

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [2] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [2].

BS	Base Station

CA	Carrier Aggregation

DUT	Device Under Test

gNB	NR Node B

IF	Intermediate Frequency

LNA	Low Noise Amplifier

NR	New RAT

NR-SS	NR Synchronization Signals (composed on NR-PSS and NR-SSS)

NSA	Non-StandAlone (NR)

OTA	Over The Air

PA	Power Amplifier

RAT	Radio Access Technology

Rx	Receiver

TRP	Total Radiated Power

TRxP	Transmission Reception Point

Tx	Transmitter

UE	User Equipment

---------------------------------- Next modified section ----------------------------------

# 9	Radio resource management

## 9.1	Mobility aspects

At least two UE states will be specified in NR: RRC_CONNECTED and RRC_IDLE. A new state, RRC_INACTIVE, also is expected to be introduced for NR UE as outlined in TR 38.804 [43], for which the UE and at least one gNB should keep the AS context information. RAN4 RRM requirements have to ensure NR UE performance while in any of the NR RRC states, as well as smooth transition between the RRC states.

NR cell corresponds to one or multiple TRxPs, which may be co-located or non-collocated. A cell is associated with a cell ID. Detecting a cell by a UE means detecting also at least its cell ID. The cell ID is the same ID as that carried by NR-SS.

In the RRC_IDLE state, the UE camps on a best cell and cell-level mobility is supported based on DL cell-level measurements.

The RRC_IDLE state (in at least standalone NR) involves at least the following UE procedures:

- Cell selection (including cell evaluation)

  - Cell selection refers to initial detection of an NR cell during cell selection procedure (i.e., the NR UE is not yet camped on a cell), e.g. at power ON, resulting in UE camping on the selected cell. To select a cell, the UE has to perform measurements and evaluate whether the cell is suitable for being selected.

- Cell reselection (including at least cell evaluation)

  - NR UE which has been camping on a cell may reselect to another cell.

- Cell identification (including at least cell detection)

  - NR UE camping on a cell can perform cell identification.

For mobility, cell reselection and cell identification RRM requirements will be specified.

In the RRC_CONNECTED state, UE mobility is supported based on DL cell-level and/or beam-level measurements. In multi-beam operation, beam-level measurements can be used to derive cell-level quality of the cell associated with the beams. In the RRC_CONNECTED state, the NR UE can perform:

- Cell identification, including at least cell detection,

- Beam identification, performed in an identified cell and included at least beam detection.

Depending e.g. on RAN1 design and RAN2 signaling, the identification time periods (e.g., for cell or beam identification) may also involve performing and reporting a measurement or receiving a part of the system information.

The mobility procedures in the RRC_INACTIVE state may be different from those in the RRC_IDLE and RRC_CONNECTED states and may also depend on how the RRC state transition is performed.

In addition to intra-RAT mobility, inter-RAT mobility at least between standalone NR and LTE will also be supported.

For the relevant RRC states, RAN4 will specify RRM requirements to ensure UE performance in relation to intra-RAT, including intra-frequency and inter-frequency, as well as inter-RAT mobility to support the agreed NR-based network architectures. If beam-level mobility is supported, RAN4 would need to develop the corresponding mobility requirements too. The mobility requirements need to cover, e.g., RRC connection, random access, handover, cell reselection, cell identification, beam identification, etc. The mobility procedures may need to be performed with and without neighbour list provided by the network.

UL-based mobility for NR has also been discussed in RAN2. If UL-based is supported in NR, RAN4 needs to discuss the necessary requirements during the NR WI.

## 9.2	Beam management

The following RRM procedures related to beam management can be further studied:

- Beam determination: RAN4 needs to study requirements for this procedure related to select at least one of its own transmit/receive beam(s).

- Beam measurement: RAN4 needs to study requirements for beam measurement. The detailed requirements need further study since the physical layer design has not been finalized. The potential requirement may include beam measurement period, beam measurement accuracy, etc.

- Beam reporting: RAN4 needs to study the potential requirement for beam reporting. The requirement may include at least reporting delay.

- Beam sweeping: Whether RRM requirements for beam sweeping at both NR UE and TRxP sides are needed needs further study in WI phase.

## 9.3	Timing aspects

Timing aspects of NR RRM shall be investigated in the NR work item phase.

## 9.4	Power consumption related aspects

### 9.4.1 	Power consumption model for RRM

Power consumption models may be useful for investigating the power consumption impact of different RRM requirements. Models are needed for both gNB and UE. The power consumption models are simplified models and are not intended to capture details or in any way limit gNB or UE implementation. Different models are defined for the UE and the gNB.

#### 9.4.1.1	UE power consumption model

Four different power consumption conditions are defined as shown in figure 9.4.1.1-1:

- Deep sleep: The UE is operating in its lowest power consumption mode, with baseband circuits maintaining timing to the lowest level of accuracy and minimal other baseband activity. RF circuits are not active.

- Light sleep: The UE in this state have maintained timing using a clock and activity level which allows reception to be started with a reasonably small delay. This state represents the UE being ready to start to receive with minimal delay.

- Active RX only: The UE is actively receiving, or attempting to receive a signal. This state is characterised by the RF receiver circuit being active.

- Active TX only: The UE is actively transmitting a signal.

Active TX + RX: The UE is actively receiving and transmitting (RX and TX are active). The power consumption in this state is assumed to be the sum of the active RX only power consumption + active TX only power consumption.

Figure 9.4.1.1-1: Power consumption states for the UE model

The UE or gNB power consumption state can change dynamically based on NR UE requirements for reception and transmission. For example, if DRX is active in a UE it can be expected to spend as much of the DRX off duration as possible in deep sleep. Prior to an RX on period, it may move to light sleep state before it enters active RX and TX depending on requirements for reception and transmission. UE will remain in active state as required according to monitoring and scheduling requirements. When UE is no longer required to receive or transmit, it may move back to light or deep sleep again. Power consumption may be estimated by considering the proportion of time that the UE spends in each state.

RRM activities may extend the duration that the UE needs to remain in either the active RX state or the active TX state.

The parameters used for the NR model are shown in table 9.4.1.1-1.

Table 9.4.1.1-1: Predicted power consumption in different states for the UE model

| Parameter | Label/value | Default value |
| --- | --- | --- |
| Absolute power consumption in “Active with data TX” state. Relative upper limit set to get some absolute numbers on the graphs | Pat, CA-N Note 1 | TBD mW |
| Relative power consumption in “Active data/no data RX” state | Par, CA-M Note 2 | TBD*Pat |
| Relative power consumption in “Deep sleep” state | Pds | TBD*Pat |
| Relative power consumption in “Light sleep” state | Pls | TBD*Pat |
| Duration of transition from “Deep sleep” to “Light sleep” state | Td2l | TBD ms |
| Relative power consumption while changing from “Deep sleep” to “Light sleep” state | Pd2l | TBD * Td2l |
| Duration of transition from “Light sleep” to “Active with (no) data RX” state | Tar2l | TBD ms |
| Duration of transition from any active state to any sleep state | Tat2d or Tat2l or Tar2d or Tar2l | TBD ms |
| Note 1: This parameter is determined for each possible UL CA configuration. Note 2: This parameter is determined for each possible DL CA configuration. |  |  |

#### 9.4.1.2	gNB power consumption model

The gNB power consumption model is characterized by 4 states as illustrated in figure 9.4.1.2-1.

Figure 9.4.1.2-1: Power consumption states for the gNB model

The power consumption assumption and the RRC state transition assumptions are listed in table 9.4.1.2-1.

Table 9.4.1.2-1: Predicted gNB Power consumption under different conditions

The principles of the gNB power consumption model are the same as for the NR UE model. gNB can be in different power saving states each of which depends on the activity (transmission and/or reception) of the gNB. A gNB power consumption depends on the configuration and load. If the gNB is 2x2 macro with full load or no load the power consumption is listed in the Load column in table 9.4.1.2-1. If the gNB does not need to be active in next symbol it may enter sleep mode 1 (SM1). If gNB does not need to be active for 1ms it can then enter sleep mode 2 (SM2), etc. The gNB wake up procedure goes back to full activity in a step wise manner.

### 9.4.2	Principles for reducing power consumption

At least the following list of possible techniques for power saving can be studies for NR RRM:

- Power saving opportunities for transmission of reference signals

- Follow lean carrier principles

  - Transmission of NR-SS synchronization signal as infrequently as possible.

  - Transmission of reference signals as infrequently as possible.

  - Transmission of measurement reference signals necessary to perform time and frequency synchronisation, allow identifying a beam and perform beam measurements.

  - Minimise time duration of individual transmissions e.g. using wideband signals in frequency domain.

  - Provide assistance information on reference signal timing in neighbour nodes to reduce NR UE measurement time (e.g. similar to DMTC window in LTE).

- RAN4 should study the effect of the above principles on the NR RRM requirements.

- Power saving opportunities for receiving and measuring reference signals

- RAN4 should explore measurements activity in order to enable NR UE power savings.

  - RAN4 should study wideband measurements versus longer measurement time to reach a given level of accuracy.

- RAN4 should take into account the UE power saving opportunities when developing measurements requirements in order to enable UE power savings, e.g. align performance requirements with DRX.

- RAN4 should enable that available assistance information is used to minimize search and measurement time.

### 9.4.3 	Configurability of requirements

Power consumption in RRM requirements is an engineering trade-off. Performance of measurements needs to be good enough to meet the needs of the system but it would be wasteful to have minimum requirements which exceed the needs of the system.

In determining the RRM requirements for NR the goal should be to ensure that power consumption and measurement delay is configurable over a suitable range.

## 9.5	Measurements and measurement related requirements

### 9.5.1	Measurement procedures

NR UE can perform measurements at least in the RRC_IDLE state (when it is camped on a cell) and RRC_CONNECTED state. The UE will perform measurements according to the corresponding requirements.

The UE will perform measurements at least for the following purposes:

- Mobility

- Beam management

### 9.5.2	Measurement reporting

The NR UE will report measurement results at least in RRC_CONNECTED state.

### 9.5.3	Measurements by carrier frequency relation

#### 9.5.3.1	Intra-frequency measurements

In NR, the UE will support intra-frequency measurements at least for intra-frequency mobility, RRM (e.g., beam management), and RLM. RAN4 needs to study whether UE needs measurements gaps for intra-frequency measurements.

#### 9.5.3.2	Inter-frequency measurements

In NR, UE will support inter-frequency measurements at least for inter-frequency mobility.

The inter-frequency measurements on different frequencies may be for different numerologies, which may further impact the inter-frequency measurement gap configurations.

UE may need measurement gaps to perform inter-frequency measurements.

#### 9.5.3.3	Inter-RAT measurements

In NR, UE will support inter-RAT measurements for inter-RAT mobility at least between NR and LTE. The measurements will be performed according to the corresponding RAN4 requirements.

#### 9.5.3.4	CA measurements

In NR, the UE will support CA measurements at least for serving cells change and RRM (e.g., beam management). RAN4 needs to study whether UE needs measurements gaps for CA measurements.

### 9.5.4	Measurements in relation to beams

The impact of the beamforming on measurements performed by the NR UE is to be further based on at least following aspects:

A UE may perform measurement with at least one of: DL transmit beam, DL receive beam, UL receive beam, and UL transmit beam.

RAN4 should further discuss whether there is a difference between measurements with and without beamforming and if so, what is the exact difference.

### 9.5.5	Measurement bandwidth aspects

Requirements for measurements shall be developed for NR UEs supporting different bandwidths. The requirements may include specification of a minimum measurement bandwidth which is the minimum bandwidth upon which the accuracy requirements are based assuming UE is using minimum measurement bandwidth. The possibility to configure wideband measurements (e.g. the measurement bandwidth size and allocation), for example with RRC signalling, is not precluded.

RAN4 should investigate how the minimum measurement bandwidth may scale according to subcarrier spacing. In addition, RAN4 needs to look at whether different NR use cases (such as mobile broadband, machine type communication and/or URLLC) could lead to different minimum measurement BWs.

RAN4 needs to study whether to define minimum measurement bandwidth during the NR work item for different use cases and subcarrier spacings.

#### 9.5.5.1	Measurements for UEs with different supported BW and numerology

In NR, a UE may support a bandwidth which is less than the NR system bandwidth. In addition, for more capable devices which support at least the NR system bandwidth, bandwidth adaptation is being considered, whereby UEs may change operating bandwidth for balancing between power saving and scheduling purposes on a semi-static or dynamic basis.

In either case, the UE requirements should be developed assuming that UE performing RRM measurements over a bandwidth which is not more than the currently configured UE downlink operating bandwidth, i.e. the UEs current operating radio link.

RAN4 should investigate measurement gaps and if they are provided to allow bandwidth reconfiguration of the receiver.

In NR, a cell may operate with multiple OFDM numerologies simultaneously using either FDM or TDM techniques. NR UEs may support a subset of the numerologies which NR cells are currently operating with. In these cases, measurements may be performed only on resource elements which correspond to numerologies supported by the UE.

## 9.6	Measurement capacity

The impact of the beamforming on the amount of parallel measurements supported by the NR UE should be further studied.

# 10	Testability

## 10.1	RRM requirements testability

### 10.1.1	General

Testability aspects of both gNB and UE have been considered. Unless otherwise indicated below, device under test (DUT) could refer to either gNB or UE nodes. The exact list of RRM tests for UE and gNB can only be determined once the core requirements are settled.

### 10.1.2	Testability of NR RRM requirements on frequency bands below 6GHz

For low frequency bands below 6GHz, the conducted testing is considered as the baseline approach for NR RRM testability.

If BS RRM requirements and test scenarios are developed, the re-use of the AAS BS measurement setup can be investigated.

NR RRM testing can generally be performed using the antenna connectors and following similar approaches as are applicable for E-UTRA UE or eNB below 6 GHz. The exact details of the tests shall be determined when the tests are implemented.

It is possible that for some specific features of NR such as beamforming, over the air tests developed for frequency bands above 6GHz (as in section 10.1.3) could be reused on frequency bands below 6GHz in order to avoid the need to develop both OTA and conducted tests for beam based measurements.

### 10.1.3	Testability of NR RRM requirements on frequency bands above 6GHz

For frequency bands above 6GHz (e.g. mm-wave), conducted antenna connectors are assumed not to be available at DUT and the OTA testing is considered as the baseline approach for NR RRM testability.

The possibility of performing conducted tests using an intermediate frequency (IF) were evaluated. It was decided that this approach would be challenging to standardise for various reasons since IF is an internal interface in the DUT and using a standardised IF (signal level, number of IF ports, IF frequency, etc.) would preclude many different DUT implementations including direct conversion receivers. In addition, IF testing excludes all components which operate at the radio frequency such as RF filters, duplexers, transmit receive switch, low noise amplifier (LNA), power amplifier (PA), analogue beamforming phase shifting elements etc., and the algorithms which control such components from the test.

#### 10.1.3.1	Over the air testing

Further details of a suitable OTA test environment are to be discussed in the work item, and may have impact to the core requirements which are defined. For example, side conditions for the applicability of core requirements should be defined in a way in which they can be ensured in an OTA environment.

The baseline measurement setup of NR RRM characteristics for f > 6 GHz is capable of establishing an OTA link between the DUT and a number of emulated gNB sources and is shown in Figure 10.1.3.1-1 below.

Diagram TBD

Figure 10.1.3.1-1: Baseline measurement setup of RRM characteristics

The RRM baseline measurement setup shares all aspects in common with the UE RF setup defined in 10.2.2.1 and includes the following aspects in addition:

-	A positioning system such that the angle between the N antennas (N ≥ 2) transmitting the emulated gNB sources and the DUT has at least two axes of freedom

-	Where N corresponds to the maximum number of emulated gNB sources defined in the RRM test scenarios

-	It is desirable that at least 1 antenna provides for an angular relationship with the DUT that is independently controllable (or the setup should provide equivalent functionality)

-	It is FFS whether all N antennas need to provide independently controllable angular relationships

-	Requirements on the polarization properties and control of each antenna are FFS

-	It is likely that the measurement uncertainty budget for the RRM setup may contain additional measurement uncertainty elements relative to the setup defined in 10.2.2.1

-	It is FFS how to model propagation conditions between the DUT and the emulated gNB sources

If BS RRM requirements and test scenarios are developed, the re-use of the AAS BS measurement setup can be investigated.

---------------------------------- End of modified section ----------------------------------