3GPP TSG-RAN WG1 Meeting #90	R1-1714717

# Prague, Czech Republic 21th – 25th Aug 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0264 | rev | 3 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NB-IoT DCI format N1 in 36.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  |  | Date: |  | 2017-08-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing reference to the RAN2 spec and missing dependency on UE support for multi-carrier NPRACH |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the reference to the RAN2 spec and condition presence of carrier indication for NPDCCH order in DCI format N1 on UE support for and eNB configuration of the feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

#### 6.4.3.2	DCI Format N1

DCI format N1 is used for the scheduling of one NPDSCH codeword in one cell, random access procedure initiated by a NPDCCH order, and notifying SC-MCCH change. The DCI corresponding to a NPDCCH order is carried by NPDCCH.

The following information is transmitted by means of the DCI format N1:

-	If the format N1 CRC is scrambled by C-RNTI or RA-RNTI:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	NPDCCH order indicator – 1 bit

-	Else if the format N1 CRC is scrambled by a G-RNTI:

-	Information for SC-MCCH change notification – 2 bits as defined in section 5.8a of [6]

Format N1 is used for random access procedure initiated by a NPDCCH order only if NPDCCH order indicator is set to ‘1’, format N1 CRC is scrambled with C-RNTI, and all the remaining fields are set as follows:

-	Starting number of NPRACH repetitions – 2 bits as defined in section 16.3.2 of [3]

-	Subcarrier indication of NPRACH – 6 bits as defined in section 16.3.2 of [3]

-	Carrier indication of NPRACH – 4 bits as defined in section 16.3.2 of [3]. This field is only present if nprach-MultiCarrierConfig is configured and the UE indicates the multiCarrier-NPRACH capability.

-	All the remaining bits in format N1 are set to one

Otherwise,

-	Scheduling delay – 3 bits as defined in section 16.4.1 of [3]

-	Resource assignment – 3 bits as defined in section 16.4.1.3 of [3]

-	Modulation and coding scheme – 4 bits as defined in section 16.4.1.5 of [3]

-	Repetition number – 4 bits as defined in section 16.4.1.3 of [3]

-	New data indicator – 1 bit

-	HARQ-ACK resource – 4 bits as defined in section 16.4.2 of [3].

-	DCI subframe repetition number – 2 bits as defined in section 16.6 in [3]

-	HARQ process number – 1 bit. This field can only be present if 2 HARQ processes are configured.

When the format N1 CRC is scrambled with a RA-RNTI or a G-RNTI, then the following fields among the fields above are reserved for RA-RNTI and not present for G-RNTI:

-	New data indicator

-	HARQ-ACK resource

If the number of information bits in format N1 is less than that of format N0 and the format N1 CRC is not scrambled by G-RNTI, zeros shall be appended to format N1 until the payload size equals that of format N0.

#### < Unchanged parts are omitted >