3GPP TSG RAN Meeting #77	RP-171858

Sapporo, Japan, September 11 - 14, 2017

Agenda Item: 	9.2.1

Source:	Apple

Title:	Single UL transmission support for NR NSA mode UEs

Document for:		Decision

# 1	Introduction

A key enabler for early commercialization of 5G NR is the non-stand-alone (NSA) deployment, which is expected to be adopted in many of the initial deployment world wide. The NSA mode, which is based on LTE-NR dual connectivity, requires UEs to maintain two active uplink transmissions from NR and LTE simultaneously, according to current 3GPP standards.

As explained in [1] [2] [3], depending on the band combinations, sending two concurrent uplinks in two different frequency bands can produce undesirable intermodulation products that could fall right into the NR or LTE receving bands, causing degradation of UE’s receiver sensitivity. This is especially problematic for band combinations for both LTE and NR bands below 6 GHz, resulting in maximum sensitivity degradation (MSD) on the order of 20-30 dB in certain cases. In such scenarios, the impacted receiver bands due to MSD are virtually not usable in practice, and it creates extreme challenges in RF implementations.

Additionally, a full blown quantitative MSD simulation and evaluation of all possible LTE-NR band combinations will inevitably add a tremendous amount of work load to RAN4 work schedule and is expected to last years, as new band combinations keep being introduced in each RAN4 meeting. In order to avoid serious receiver sensitivity degradations on any agreed LTE/NR band combinations, it is imperative that some alternative mechanisms to dual-connectivity to be specified in Rel-15 NR work item phase to avoid simultaneous LTE and NR uplink transmissions from a single UE perspective when such MSD issues occur.

As a result, 3GPP RAN1 has tackled this problem and has reached the following agreements in the past three RAN1 meetings:

In RAN1 #89 in May, it’s agreed to support an operating mode so that the network can configure the NSA mode UEs to avoid simultaneously transmitting on two carriers when such self-interference exists:

Agreements:

- For NR NSA for a UE, NR supports the case that when the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers

  - FFS whether or not there is specification impact

    - If there is RAN1 specification impact, aim to minimize the specification impact for NR

    - Note: this feature by itself is not intended to have any LTE RAN1 specification impact

  - Note: the other case of allowing simultaneous operation on two or more UL carriers is already agreed to be supported

Further in RAN1 Adhoc NR #2 in June, a TDM based solution was selected and agreed:

Agreements:

- Support the following solution to single UL transmission where NW synchronization between eNodeB and gNodeB is assumed (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency)

  - When UE is activated with multiple UL carriers on different frequencies, time-switching of LTE UL carrier and NR UL carrier is used

    - UL transmission timing pattern of LTE carrier and NR carrier is semi-statically shared between eNodeB and gNodeB

    - FFS: Signaling to UE of UL transmission timing pattern

  - UE simultaneously receives signals/channels from both NR DL carrier and LTE DL carrier

    - For scheduling/HARQ timing of LTE FDD carrier, the following timing can be considered, e.g., for LTE:

      - DL-reference UL/DL configuration for TDD

      - DL-reference UL/DL configuration defined for FDD-SCell in TDD-FDD CA with TDD-PCell

      - Up to NW implementation (i.e., no RAN1 spec. impact)

    - For scheduling/HARQ timing of NR carrier, no special handling would be necessary

  - Other solutions are not precluded

- Send an LS to RAN2, RAN3, and RAN4 – Kazuaki (DCM), which is drafted and endorsed in R1-1711877. Final LS approved in R1-1711878.

In RAN1#90 in August, specific TDM patterns were selected and further agreed:

Agreements:

- When the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), but the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers

  - For LTE carrier, UE can be configured with

    - Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell

      - For scheduling/HARQ timing of LTE FDD carrier, DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell is applied

      - UE is allowed to transmit NR UL signals at least in the subframe(s) where LTE UL transmission is not allowed according to the DL-reference UL/DL configuration

      - FFS whether or not a UE-specific subframe offset for the DL-reference UL/DL configuration can be configured considering system resource utilization and potential spec impact

    - Case 2: Release 15 LTE-FDD HARQ timing

      - No impact on LTE RAN1 specifications

    - Note: it doesn’t necessarily imply that UE has to support both cases

Initial discussion had also been performed in both RAN2 and RAN4 respectively.

In this contribution, we provide some clarification on single UL transmission vs. 2 simultaneous UL transmissions, its applicability of single UL transmission on the agreed LTE/NR NSA band combinations, UE capability design, and remaining standardization work across RAN WGs, along with our propsals for completion of the work on single UL transmission for LTE/NR NSA operation by Dec. 2017.

# 2    Single UL vs. two UL transmissions

It is worthwhile to first clarify the motivation and the exact meaning of “single uplink” as refered to in this contribution. Specifically, there are two types of flexibility which potentially could motivate the “single uplink” design, and we denote them as follows:

- Hardware flexibility: whether UE can implement 2 sets of transmit (RF) chains for the LTE and NR uplink carriers separately, or only implement 1 set of transmit chain shared between the two RAT’s uplinks (e.g. in TDM manner).

- Configuration flexibility: given UE will have 2 sets of transmit (RF) chains for the LTE and NR uplink carriers separately, can the network configure the UE to operate in non-simultaneous uplink (e.g. in TDM manner) when needed.

To be clear, by the “single uplink” in this contribution, or at least for the LTE-NR DC scenarios with currently planed band combinations (i.e. NR band at above 3GHz, e.g. n77, n78, n79), we are thinking of the “configuration flexibility”, i.e. UE has dedicated hardware for the two uplink frequencies. In these scenarios, since the LTE and NR spectrum are far apart, different transmit chains are practically needed anyway.

As for the hardware flexibility, it might be potentially useful for some future product categories when new spectrum very close to (or in the same frequemcy bands as) the LTE spectrum is allocated to NR (such as migration of LTE frequency bands to NR while both LTE and NR will be coexisting together for the time being). In such case a single set of transmit chain might be wide enough to cover the two frequencies. However, at least for all the current allocated LTE-NR combinations for initial NSA deployment, we don’t see such a use case for now in LTE-NR DC deployment. With this clarification, we can use single uplink and non-simultaneous uplink transmission interchangeably during this contribution without confusion.

It is also benefial to briefly look at the pros/cons of single uplink in the context of LTE-NR DC. Other than avoiding the impact on downlink sensitivity degradation due to IMD, single uplink can be a useful feature in other context as well, such as cell coverage. It is desirable that a LTE-NR DC device can achieve similar coverage, at least on the LTE RAT, as the legacy LTE deployment, since the initial NR deployment will likely use co-site deployment and reuse the existing LTE basestation sites. Single uplink transmission with TDM across two RATs allow the UE to have full allocation of transmit power on one RAT, and therefore allowing UE to achieve similar coverage in uplink during LTE transmissions.

A potential concern on single uplink transmission is the impact to uplink throughput, specifically on the peak throughput metric, due to the fact that only one uplink can be used at a time. To have better understanding of the impact, we’d like to separate into two scenarios: power limited vs dimension limited.

In power limited scenario, due to the uplink coupling loss, UE can’t achieve the maximal spectral efficiency even when UE’s transmitting at close to peak power. In this case, UE is limited by total transmission power, and creating more dimensions (e.g. adding more frequency resources) does not improve uplink throughput.

In a practical network, it is expected that a large portion of the devices will be in power limited in uplink, other than those located at the cell center. For those devices, uplink throughpout impact due to TDM mode is not an issue, but on the other hand, the downlink sensitivity degradation could be substantial since the devices are transmitting at close to peak power.

In the dimension limited scenario, the uplink coupling loss is small, and UE can achieve maximal spectral efficiency (i.e. largest MCS) even without transmitting at peak power, e.g. when UE is at cell center or in certain lab test setups. In this case, creating more dimensions (e.g. by adding another frequency) could improve throughput. It should be noted that NR uplink supports much higher maximal spectral efficiency than LTE uplink, due to the introduction of higher order modulation, spatial multiplexing, as well as wider bandwidth. Therefore, the difference in UL throughput between single UL and dual UL depends on how many subframes allocated to NR and LTE, respectively.

Observation 1: for LTE-NR DC UE’s that are power limited in the uplink, there is negligible impact on uplink throughput using non-simultaneous UL compared to simultaneous UL.

Proposal 1: For LTE-NR DC band combinations with potential Rx MSD issues due to IMD, both single UL transmission and dual UL transmission are supported by the UE and network to avoid the REFSENS degradation

# 3	Coexistence of available LTE/NR band combinations

According to the co-existence analysis of LTE and NR band combinations in [2], following observations can be summarized:

Observation 2: for the 40 band combinations (with NR in sub-6GHz band) that have been evaluated out of the 242 existing LTE-NR band combinations, 23 have potential IMD interference issue

Observation 3: for the 18 band combinations (with NR in mmW band) that have been evaluated out of the 242 existing LTE-NR band combinations, 0 have potential IMD interference issue

# 4    Self-interference and UE capability

As discussed in section 1, the main motivation for single UL transmission is to avoid self-interference due to IMD. Such self-interference can depend on several factors:

- Specific band combination, i.e. how close the two UL frequencies are

- UE implementation: such as RF components used, how many bands are integrated into the devices, the layout of the RF components, etc.

- Any proprietary interference suppression mechanism used to suppress the interference

Therefore, the existence and severity of the self-interference can be device-dependent, in addition to band combination dependent. The simplest way is to allow UEs to include a 1-bit indicator in the UE RF capability for each band combination, potentially as part of the RRC message.

Proposal 2: Introducing 1-bit indicator per band combination as part of the UE RF capability message, to request for single uplink transmission.

# 5    Remaining standardisation work

As discussed in section 1, RAN1 has completed the functional requirements and solution design to support single UL transmission for UE NSA operations, i.e., UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers. RAN1 discussed several alternative solutions to support single UL transmission, including physical layer TDM approach, physical layer FDM approach, and MAC layer multiplexing with single uplink waveform, and eventually narrowed down to the TDM approach, which we believe is the simplest approach and has the least specification impact. Based on the agreed RAN1 design, following remaining standardisation work is expected to finalize the single UL transmission work.

RAN1:

- Stage 2 descriptions for single UL transmission under LTE-NR coexistence framework. (TS38.300)

- Stage 3 changes (TS38.213)

RAN2:

- UE capability signaling (which may need some RAN4 imput, e.g., whether it’s per band combinations, etc.) (TS38.331, TS38.306)

- RRC signaling for network to configure UE with the UL transmission timing patterns for LTE and NR, based on RAN1 agreements. (TS38.331)

- Additional stage 2 descriptions if needed from RAN2 side (TS38.300)

RAN3:

- X2/Xn signaling to coordinate between LTE eNB and NR gNB about the UL transmission timing patterns for LTE and NR. (TS36.423/TS38.423)

RAN4:

- Identify LTE-NR NSA band combinations, for which single UL transmissions should be supported

Observation 4: The scope of the remaining standardisationwork is limited, and can be finished by Dec. 2017.

Proposal 3: Provide above analysis of RAN1/2/3/4 specification impact as a guidance to complete the single UL transmission work in RAN1/2/3/4.

# 6    Conclusion

In this contribution, we summarize the progress made on single UL transmission for LTE/NR NSA operation, and give our analysis on single UL transmission vs. 2 UL transmissions, applicability of single UL transmission on the agreed LTE/NR NSA band combinations, and remaining standardisation work across RAN WGs. We observed the following:

Observation 1: for LTE-NR DC UE’s that are power limited in the uplink, there is negligible impact on uplink throughput using non-simultaneous UL compared to simultaneous UL.

Observation 2: for the 40 band combinations (with NR in sub-6GHz band) that have been evaluated out of the 242 existing LTE-NR band combinations, 23 have potential IMD interference issue

Observation 3: for the 18 band combinations (with NR in mmW band) that have been evaluated out of the 242 existing LTE-NR band combinations, 0 have potential IMD interference issue

Observation 4: The scope of the remaining standardisationwork is limited, and can be finished by Dec. 2017.

To facilitate the successful commercialization of NR NSA deployment, we propose the following:

Proposal 1: For LTE-NR DC band combinations with potential Rx MSD issues due to IMD, both single UL transmission and dual UL transmission are supported by the UE and network to avoid the REFSENS degradation

Proposal 2: Introducing 1-bit indicator per band combination as part of the UE RF capability message, to request for single uplink transmission.

Proposal 3: Provide above analysis of RAN1/2/3/4 specification impact as a guidance to complete the single UL transmission work in RAN1/2/3/4.

# 7    References

- R1-1708276, “Uplink sharing in NSA mode”, 3GPP RAN1#89, Hangzhou, China, May 15th-19th, 2017.

- R4-1704759, “Co-existence analysis on LTE-NR band combinations”, 3GPP RAN4 #83, Hangzhou, China, May 15th-19th, 2017.

- R4-1703999, “European LTE-NR band combinations, and coexistence analysis”, RAN4#82bis, Spokane, US, April 3-7, 2017