3GPP TSG-RAN4 Meeting #84	R4-1707328

# Berlin, Germany, 21st Aug 2017 - 25th Aug 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4520 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eHST RF: Further clarifications to HST-SFN scenario model (Rel-14) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_high_speed-Perf |  |  |  |  |  | Date: |  |  | 2017-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to offline discussions in R4#83 meeting, in HST-SFN scenarios the total received power by the UE (from all seen RRHs) is constant. This is also required in order to may clearly define the SNR required in the tests. This is achieved by defining power received by each RRH (Pk) as normalized to the total power. This is not yet specified clearly in the HST-SFN scenario definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In HST-SCN scenario definition in Annex B.3A: - Added normalization to to the Pk definition in equation B.3A.4 - Adopted graph in Figure B.3A-2 accordingly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification will lead to different implementation of the HST-SFN scenarion and SNR required in the tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex B.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

# B.3A	HST-SFN scenario

There is an infinite number of RRHs distributed equidistantly along the track (distance = Ds) with the same Cell ID as depicted in figure B.3A-1.

Figure B.3A-1: Deployment of HST-SFN

The location of RRH k is given as:

meters	(B.3A.1)

where:

and is the distance between the RRHs and railway track, while  is the distance of two RRHs, both in meters.

The train location is denoted as:

(B.3A.2)

where:

and a means distance in meters, which means the train is right on the track.

The HST-SFN scenario for the test of the baseband performance is a non fading propagation channel with four taps, namely the four nearest RRHs. Thus RRH k is visible for the train only in the range:

(B.3A.3)

Power level  (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs, is given by:

Pk=-20lgy-xk+20lgDs-10lgi∈i|i*Ds-2*Ds≤a<i*Ds+2*DsDs2y-xi2

for 	(B.3A.4)

Doppler shift (Hz) from kth RRH is given by:

for 	(B.3A.5)

The relative delay  (s) for the signal from kth RRH can be derived as:

for 	(B.3A.6)

In the above v (m/s) is the moving speed of the train, fC (Hz) is the center frequency, and C (m/s) is the velocity of light.

Power level, Doppler shift and relative delay are given by equations B.3A.4 ~ B.3A.6 respectively, where the required input parameters listed in table B.3A-1 and the resulting Doppler shift shown in Figure B.3A-3 are applied for all frequency bands.

Table B.3A-1: HST-SFN scenario

| Parameter | Value |
| --- | --- |
|  | 1000 m |
|  | 50m |
|  | 350 km/h |
|  | 872 Hz |

NOTE 1:	Parameters for HST-SFN scenario in Table B.3A-1 includingand Doppler shift trajectories presented in Figure B.3A-2 were derived from Band 7 and are applied for performance verification in all frequency bands. And the trajectories of ralative power, Doppler shifts and relative delay presented in Figures B.3A-2~ B.3A-4 are derived from the equations B.3A.4 ~ B.3A.6 respectively.

Figure B.3A-2 Ralative power level trajectories

Figure B.3A-3 Doppler shifts trajectories

Figure B.3A-4 Relative delay trajectories

For 2x2 antenna configuration, the same h(t,τ) is used to describe the channel between every pair of Tx and Rx with phase shift according to .

< End of changes >