3GPP TSG RAN WG1 Meeting #90 	R1-1712260

# Prague, Czech Republic 21st – 25th August 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.873 | CR | 0031 | rev |  | Current version: | 12.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 3D-RMa channel model in TR 36.873 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_LTE_3D_channel |  |  |  |  |  |  | Date: |  | 2017-08-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TR 36.873 has some errors in 3D-RMa channel model specification. 1. Table 7.2-1: Pathloss models, it is not clear whether d is in 2d or 3d. 2. In 7.2.3 O2I penetration loss, there is no shadowing fading parameters for O2I links for RMa, but in Table 7.3-6b channel model parameters for 3D-RMa section 7.2.3 was mentioned as the source. 3. “3D-RMa” was not mentioned in Table 7.3-2 Scaling factors for AoA,AoD generation and 7.3-4 Scaling factors for ZoA,ZoD generation. (TR 36.873,V12.5.1, cat.F) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix the typos and issues in 3D-RMa channel model in TR 36.873, including: 1. Add 2D and 3D clarifications in to Table 7.2-1. 2. In Table 7.3-6b, Remove the reference to section 7.2.3, and add fixed shadow fading value there. 3. Add 3D-RMa in Table 7.3-2 and Table 7.3-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Users of channel model do not know how to model the 3D-RMa. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sections 7.2 and 7.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Text Proposal

# < Unchanged parts are omitted >

Table 7.2-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and distance is in meters | Shadow fading std [dB]7) | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| 3D-UMi | LOS | PL = 22.0log10(d3D) + 28.0 + 20log10(fc) PL = 40log10(d3D)+28.0+20log10(fc) –9log10((d'BP)2+(hBS - hUT)2) | σSF=3 σSF =3 | 10m < d2D < d'BP1) d'BP < d2D < 5000m1) hBS = 10m1), 1.5m ≦ hUT≦ 22.5m1) |
|  | NLOS | For hexagonal cell layout: PL = max(PL3D-UMi-NLOS, PL3D-UMi-LOS), PL3D-UMi-NLOS = 36.7log10(d3D) + 22.7 + 26log10(fc) – 0.3(hUT - 1.5) | σSF =4 | 10 m < d2D < 2000m2) hBS = 10m 1.5m ≦ hUT≦ 22.5m |
| 3D-UMa | LOS | PL = 22.0log10(d3D) + 28.0 + 20log10(fc) PL = 40log10(d3D)+28.0+20log10(fc) –9log10((d'BP)2+(hBS - hUT)2) | σSF =4 σSF =4 | 10m < d2D < d'BP4) d'BP < d2D < 5000m4) hBS = 25m4), 1.5m ≦ hUT ≦ 22.5m4) |
|  | NLOS | PL = max(PL3D-UMa-NLOS, PL3D-UMa-LOS), PL3D-UMa-NLOS = 161.04 – 7.1 log10 (W) + 7.5 log10 (h) – (24.37 – 3.7(h/hBS)2) log10 (hBS) + (43.42 – 3.1 log10 (hBS)) (log10 (d3D)-3) + 20 log10(fc) – (3.2 (log10 (17.625)) 2 - 4.97) – 0.6(hUT - 1.5) | σSF =6 | 10 m < d2D < 5 000 m h = avg. building height, W = street width hBS = 25 m, 1.5m ≦ hUT≦ 22.5m, W = 20m, h = 20 m The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1.5 m ≦ hUT ≦ 22.5 m Explanations: see 6) |
| 3D-RMa | LOS | PL1 = 20log10(40d3Dfc /3) + min(0.03h1.72,10)log10(d3D) – min(0.044h1.72,14.77) + 0.002log10(h)d3D PL2 = PL1 (dBP) + 40 log10(d3D /dBP) | SF = 4 SF = 6 | 10 m < d2D < dBP, 8) dBP < d2D < 10 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height, W = street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
|  | NLOS | PL = 161.04 – 7.1 log10 (W) + 7.5 log10 (h) – (24.37 – 3.7(h/hBS)2) log10 (hBS) + (43.42 – 3.1 log10 (hBS)) (log10 (d3D)-3) + 20 log10(fc) – (3.2 (log10 (11.75 hUT)) 2 - 4.97) | SF = 8 | 10 m < d2D < 5 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m h = avg. building height, W = street width The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
| 3D-InH | LOS | PL = 16.9 log10(d3D) + 32.8 + 20 log10(fc) | SF = 3 | 3 m < d2D < 100 m hBS = 3-6 m hUT = 1-2.5 m |
|  | NLOS | PL = 43.3 log10(d3D) + 11.5 + 20 log10(fc) | SF = 4 | 10 m < d2D < 150 m hBS = 3-6 m hUT = 1-2.5 m |
| NOTE 1: Break point distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In 3D-UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. NOTE 2: PL3D-UMi-LOS = Pathloss of 3D-UMi LOS outdoor scenario. NOTE 4: Break point distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In 3D-UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. The function C(d2D, hUT) is defined in Table 7.2-2. Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. NOTE 6: PL3D-UMa-LOS = Pathloss of 3D-UMa LOS outdoor scenario. NOTE 7: The shadow fading values are reused from [4] for simplicity. NOTE 8: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. |  |  |  |  |

< Unchanged parts are omitted >

Table 7.3-6b: Channel model parameters for 3D-RMa

| Scenarios |  | Rural |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -7.49 | -7.43 | -7.47 |
|  | lgDS | 0.55 | 0.48 | 0.24 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | 0.90 | 0.95 | 0.67 |
|  | lgASD | 0.38 | 0.45 | 0.18 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 1.52 | 1.52 | 1.66 |
|  | lgASA | 0.24 | 0.13 | 0.21 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | 0.47 | 0.58 | 0.93 |
|  | lgZSA | 0.40 | 0.37 | 0.22 |
| Shadow fading (SF) [dB] | SF | See Table 7.2-1 |  | 8 |
| K-factor (K) [dB] | K | 7 | N/A | N/A |
|  | K | 4 | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0 | -0.4 | 0 |
|  | ASA vs DS | 0 | 0 | 0 |
|  | ASA vs SF | 0 | 0 | 0 |
|  | ASD vs SF | 0 | 0.6 | 0 |
|  | DS vs SF | -0.5 | -0.5 | 0 |
|  | ASD vs ASA | 0 | 0 | -0.7 |
|  | ASD vs K | 0 | N/A | N/A |
|  | ASA vs K | 0 | N/A | N/A |
|  | DS vs K | 0 | N/A | N/A |
|  | SF vs K | 0 | N/A | N/A |
| Cross-Correlations 1) | ZSD vs SF | 0.01 | -0.04 | 0 |
|  | ZSA vs SF | -0.17 | -0.25 | 0 |
|  | ZSD vs K | 0 | N/A | N/A |
|  | ZSA vs K | -0.02 | N/A | N/A |
|  | ZSD vs DS | -0.05 | -0.10 | 0 |
|  | ZSA vs DS | 0.27 | -0.40 | 0 |
|  | ZSD vs ASD | 0.73 | 0.42 | 0.66 |
|  | ZSA vs ASD | -0.14 | -0.27 | 0.47 |
|  | ZSD vs ASA | -0.20 | -0.18 | -0.55 |
|  | ZSA vs ASA | 0.24 | 0.26 | -0.22 |
|  | ZSD vs ZSA | -0.07 | -0.27 | 0 |
| Delay distribution |  | Exp |  |  |
| AoD and AoA distribution |  | Wrapped Gaussian |  |  |
| ZoD and ZoA distribution |  | Laplacian |  |  |
| Delay scaling parameter r |  | 3.8 | 1.7 | 1.7 |
| XPR [dB] | XPR | 12 | 7 | 7 |
|  | XPR | 4 | 3 | 3 |
| Number of clusters |  | 11 | 10 | 10 |
| Number of rays per cluster |  | 20 | 20 | 20 |
| Cluster ASD () in [deg] |  | 2 | 2 | 2 |
| Cluster ASA () in [deg] |  | 3 | 3 | 3 |
| Cluster ZSA () in [deg] |  | 2 | 2.5 | 3 |
| Per cluster shadowing std  [dB] |  | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] XPR [dB] | DS | 50 | 36 | 36 |
|  | ASD | 25 | 30 | 30 |
|  | ASA | 35 | 40 | 40 |
|  | SF | 37 | 120 | 120 |
|  | K | 40 | N/A | N/A |
|  | ZSA | 15 | 50 | 50 |
|  | ZSD | 15 | 50 | 50 |
| NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. |  |  |  |  |

< Unchanged parts are omitted >

Table 7.3-2: Scaling factors for AOA, AOD generation

| scenario | 3D-UMa, 3D-UMi, 3D-RMa |  |  |  |  |  |  |  |  |  |  | 3D-InH |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N | 4 | 5 | 8 | 10 | 11 | 12 | 14 | 15 | 16 | 19 | 20 | 15 | 19 |
|  | 0.779 | 0.860 | 1.018 | 1.090 | 1.123 | 1.146 | 1.190 | 1.211 | 1.226 | 1.273 | 1.289 | 1.434 | 1.501 |

< Unchanged parts are omitted >

Table 7.3-4: Scaling factors for ZOA, ZOD generation

| scenario | 3D-UMa, 3D-UMi, 3D-InH, 3D-RMa |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| N | 10 | 11 | 12 | 15 | 19 | 20 |
|  | 0.9854 | 1.013 | 1.04 | 1.1088 | 1.1764 | 1.1918 |

< Unchanged parts are omitted >