3GPP TSG RAN WG1 Meeting #90 	R1-1712261

# Prague, Czech Republic 21st – 25th August 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.873 | CR | 0032 | rev |  | Current version: | 12.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on large scale parameters for 3D-InH in TR 36.873 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Fraunhofer HHI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_LTE_3D_channel |  |  |  |  |  |  | Date: |  | 2017-08-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The number of clusters and rays in Table 7.3-6a: Channel model parameters for 3D-InH were removed by a mistake. This issue should be fixed, otherwise this model is not available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 3D-InH parameters into Table 7.3-6a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Users of channel model do not know the parameter values, and cannot implement the model. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Text Proposal

# < Unchanged parts are omitted >

Table 7.3-6a: Channel model parameters for 3D-InH

| Scenarios |  | Indoor Hotspot |  |
| --- | --- | --- | --- |
|  |  | LOS | NLOS |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | –7.70 | –7.41 |
|  | lgDS | 0.18 | 0.14 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | 1.60 | 1.62 |
|  | lgASD | 0.18 | 0.25 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 1.62 | 1.77 |
|  | lgASA | 0.22 | 0.16 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | 1.22 | 1.26 |
|  | lgZSA | 0.23 | 0.67 |
| Shadow fading (SF) [dB] | SF | See Table 7.2-1 |  |
| K-factor (K) [dB] | K | 7 | N/A |
|  | K | 4 | N/A |
| Cross-Correlations | ASD vs DS | 0.6 | 0.4 |
|  | ASA vs DS | 0.8 | 0 |
|  | ASA vs SF | –0.5 | –0.4 |
|  | ASD vs SF | –0.4 | 0 |
|  | DS vs SF | –0.8 | –0.5 |
|  | ASD vs ASA | 0.4 | 0 |
|  | ASD vs K | 0 | N/A |
|  | ASA vs K | 0 | N/A |
|  | DS vs K | –0.5 | N/A |
|  | SF vs K | 0.5 | N/A |
|  | ZSD vs SF | 0.2 | 0 |
|  | ZSA vs SF | 0.3 | 0 |
|  | ZSD vs K | 0 | N/A |
|  | ZSA vs K | 0.1 | N/A |
|  | ZSD vs DS | 0.1 | -0.27 |
|  | ZSA vs DS | 0.2 | -0.06 |
|  | ZSD vs ASD | 0.5 | 0.35 |
|  | ZSA vs ASD | 0 | 0.23 |
|  | ZSD vs ASA | 0 | -0.08 |
|  | ZSA vs ASA | 0.5 | 0.43 |
|  | ZSD vs ZSA | 0 | 0.42 |
| Delay distribution |  | Exp |  |
| AoD and AoA distribution |  | Laplacian |  |
| ZoD and ZoA distribution |  | Laplacian |  |
| Delay scaling parameter r |  | 3.6 | 3 |
| XPR [dB] | XPR | 11 | 10 |
|  | XPR | 3 | 3 |
| Number of clusters |  | 15 | 19 |
| Number of rays per cluster |  | 20 | 20 |
| Cluster ASD () in [deg] |  | 5 | 5 |
| Cluster ASA () in [deg] |  | 8 | 11 |
| Cluster ZSA () in [deg] |  | 9 | 9 |
| Per cluster shadowing std  [dB] |  | 6 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 8 | 5 |
|  | ASD | 7 | 3 |
|  | ASA | 5 | 3 |
|  | SF | 10 | 6 |
|  | K | 4 | N/A |
|  | ZSA | 4 | 4 |
|  | ZSD | 4 | 4 |
| NOTE 1: DS = rms delay spread, ASD = rms azimuth spread of departure angles, ASA = rms azimuth spread of arrival angles, ZSD = rms zenith spread of departure angles, ZSA = rms zenith spread of arrival angles, SF = shadow fading, and K = Ricean K-factor. NOTE 2: The sign of the shadow fading is defined so that positive SF means more received power at UT than predicted by the path loss model. NOTE 3: The following notation for mean (μlgX=mean{log10(X) }) and standard deviation (σlgX=std{log10(X) }) is used for logarithmized parameters X. |  |  |  |

< Unchanged parts are omitted >