3GPP TSG RAN Meeting 78                            	              RP-172438

Lisbon, Portugal, December 18th-21st, 2017

Agenda item:	9.2.1

Source: 	Qualcomm Incorporated

Title: 	NR EN-DC Power sharing capability

Document for:	Discussion/Decision

# Introduction

This contribution discusses the LTE-NR power sharing capability of NSA UEs when configured with LTE-NE DC.

# Discussion

At RAN1#90, the following was agreed.

Agreements:

- At least for LTE-NR NSA operation

  - Maximum allowed power values for LTE (P_LTE) and NR (P_NR) are set separately

    - i.e., when UE is configured for NR, P_LTE can be configured up to P_cmax and  P_NR can be configured up to P_cmax.

    - e.g. P_LTE + P_NR > P_cmax or P_LTE + P_NR = P_cmax

  - Signaling details for P_LTE, P_NR are left to RAN2, RAN4.

  - Note: ‘P_cmax’ is a limit that is similar to ‘The configured maximum UE output power’ that was specified for LTE.

  - Note: The network will still have flexibility to prioritize or reserve certain NR transmission power depending on network implementation

  - All UEs are mandated to handle P_LTE + P_NR = P_cmax while handling of P_LTE + P_NR > P_cmax depends on UE capability

  - At least, when DL/UL LTE sTTI/reduced UE processing time based operation is not configured for the UE, if total transmit power exceeds P_cmax when there is simultaneous NR and LTE UL tx,

    - For NR, UE scales down/drops NR transmission and NR power scaling details are left to UE implementation (note: it is not intended to have RAN4 test from RAN1 perspective)

      - If there are two or more UL carriers, the power scaling or tx dropping can be performed for each of the UL carriers separately or jointly up to UE implementation

    - For LTE, no change in power control procedure

  - FFS the case when DL/UL LTE sTTI/reduced UE processing time based operation is configured for the UE

  - The following is FFS

    - The case when P_NR is configured such that P_NR < P_cmax, and UE can use power up to P_cmax in NR when it knows that there will be no UL transmission in LTE by semi-static configuration (e.g., measurement gap, DL/UL configuration)

The LS to RAN4 capturing the agreement was unclear and misunderstood.

Instead of understanding P_LTE and P_NR as they were intended to be as semi-static configured parameters, RAN4 assumed that P_LTE and P_NR are actual transmit powers.

Since the issue was clarified, there should be no assumption that any UE will exceed P_powerclass under any condition, irrespective of their capability.

The RAN1 agreement assumes that there are two types of NSA UEs supported by the specification:

- UE Type 1 capable of dynamic LTE-NR power sharing

  - Converged NR and LTE modem with high speed interface capability

  - These UEs can operate with P_LTE + P_NR > P_powerclass configuration or with no P_LTE or P_NR configuration

- UE Type 2 not capable of dynamic LTE-NR power sharing

  - Separate NR and LTE modem with limited interface capability

  - These UEs can only operate in EN-DC when a configuration is received with P_LTE + P_NR <= P_powerclass

It is important that both UE types are continued to be supported by the specification.

At RAN1 #91 a further agreement was made [1][2] that applies only to UE Type 2

Agreement:

For LTE/NR NSA operation,

If this UE supports dual UL operation and also supports single UL operation with Case 1 HARQ timing, RRC signaling can configure a UE to operate in one of the following modes:

Dual UL operation

Single UL operation with Case 1 HARQ timing

Single UL operation with Case 2 HARQ timing

For UE supporting single UL operation and with Case 1 HARQ timing if UE does not support power scaling for LTE-NR DC with P_LTE+P_NR>Pcmax, UE shall support the following two operations:

Operation A with Case1: P_LTE + P_NR > Pcmax, in which case the UE assumes that no NR UL transmission takes place in an UL subframe/slot that is designated as LTE UL in the Case 1 reference TDD configuration

Operation B with Case1: P_LTE + P_NR <= Pcmax, in which case NR UL can be scheduled in any UL subframe/slot (while the UE behaviour in case of being simultaneously scheduled on LTE and NR uplinks is not specified)

The operation A vs operation B configuration is implicitly determined based on P_LTE and P_NR

Note that the above agreement does not affect the current status on the optional/mandatory support of power scaling for LTE-NR DC with P_LTE + P_NR > Pcmax

Note that the above agreement can become obsolete if power scaling for LTE-NR DC is mandated to all UEs

When the NR UE capabilities are further discussed, the above RAN1 decision should be endorsed. In particular, UE Type 2 not capable of dynamic LTE-NR power sharing needs to be supported by the specification.

It can be further discussed whether for these UEs the support of Case 1 HARQ timing should be optional or mandatory.

# Conclusion

Proposal:  The RAN1 decision on the specification support of UEs not capable of dynamic LTE-NR power sharing should be endorsed.  It can be further discussed whether for these UEs the support of Case 1 HARQ timing should be optional or mandatory.

# References

[1] R1-1721555, “On UL power sharing for coverage enhancement”, Orange, NTT Docomo, AT&T, China Telecom, China Unicom, DT, Verizon, Bouygues Telecom, Huawei, ZTE, Ericsson, CATT, OPPO, Vivo, Xiaomi, Apple, Hisilicon

[2] R1-1721469 “Power split in TDM cases”, Qualcomm