3GPP TSG RAN #78	RP-172788

Lisbon, Portugal, 18th – 21st Dec 2017

Title:		Mandatory 4Rx antenna performance for NR UE

Source:	Vodafone

Agenda: 	9.2.1

For:		Approval

# Introduction

This document makes a proposal regarding mandating NR UE receiver performance assuming 4 receiver antenna ports for certain bands within the “FR1” range (sub-6GHz).

# Proposal

- For NR Bands n7, n38, n41, n77, n78, and n79 the UE shall be equipped with 4Rx ports as a baseline

  - Applicability to other NR bands is FFS

  - For certain UE types/categories, some exceptions to this requirement may be applicable in future

  - RRC signalling and ASN.1 coding shall be defined in Release 15 such that supported number of UE Rx antenna ports is indicated from UE to the eNB/gNB.

    - “4” shall be the lowest value that is allowed to be indicated.

    - The coding shall enable flexibility such that a lower number than 4 could be indicated in the future for the UE types/categories mentioned above as exceptions.

- UE equipped with 4Rx ports as a baseline shall fulfil all 4Rx demodulation performance requirements defined for downlink data and control

- Fall back to 2Rx shall be allowed to save power

- RRM Requirements will be based on 2Rx

  - RLM requirements shall be defined based on both 2Rx and 4Rx

  - The proper antenna connection similar to LTE (defined in A.3.8.1 of TS36.133) will be defined to make 2Rx based RRM tests be applicable to NR UE equipped with 4Rx port.

- The baseline Rx antenna port assumption for LTE operation for the UE is as defined in 36 series specifications today