3GPP TSG-RAN4 Meeting #86	R4-1802184

# Athens, Greece, 26th Feb 2018 - 2nd Mar 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.307 | CR | 4374 | rev | - | Current version: | 13.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of 4UL CA into TS36.307 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_4DL4UL_B41_B41_B42_B42-Perf, TEI11 |  |  |  |  |  | Date: |  |  | 2018-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the requirement for up to 4UL/4DL inter band CA into the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of new tables for common RRM and UE RF requirements for up to 4DL/4UL inter-band CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for up to 4DL/4UL inter-band CA are still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

3A.2	Additional E-UTRA CA configurations

Requirements for a Rel-13 UE for additional E-UTRA CA configurations compared to TS 36.101 Rel-13 [2] are introduced via this clause.

Table 3A.2-1: Intra-band contiguous CA configurations

| Feature | DL/UL | CA BW Class | Duplex-mode | Release independent from | requirements to be fulfilled (see 36.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- |
| Intra-band contiguous CA configurations | DL | B | FDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | C | FDD, TDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | D | TDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | E | TDD | Rel-11 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | F | TDD | Rel-12 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  | UL | B | FDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
|  |  | C | FDD, TDD | Rel-10 | Table B.2.2-1, Table B.3.2-1, Table B.4.2-1 |
| NOTE1: The duplex mode "FDD, TDD" refers to a CA configuration composed by only FDD bands or only TDD bands, respectively. |  |  |  |  |  |

Table 3A.2-2: Inter-band CA configurations

| Feature | DL/UL | number of bands | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 36.307 of the REL in which the CA configuration was introduced) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations | DL | 2 |  | A, B, C | FDD, TDD | Rel-10 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  |  |  | D | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  |  |  | A, B, C, D | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  | 3 |  | A | FDD, TDD | Rel-10 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  |  |  | B, C | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  |  |  | A | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  | 4 |  | A, C | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  |  |  |  | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  | 5 |  | A | FDD, TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  |  |  |  | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  | UL | 2 | 2-4 | A, C | FDD, TDD | Rel-11 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
|  |  |  | 2 | A | FDD and TDD | Rel-12 | Table B.2.4-1, Table B.3.2-1, Table B.4.3-1 or Table B.4.4-1 |  |
| NOTE1: The duplex mode "FDD, TDD" refers to a CA configuration composed by only FDD bands or only TDD bands, respectively. The duplex mode "FDD and TDD" refers to a CA configuration including both FDD and TDD bands. NOTE2: CA configurations involving downlink only operation in Band 46 are release independent from Rel-13 onwards (LAA was introduced in Rel-13). The 10 MHz channel bandwidth for Band 46 was introduced in TS 36.101 Rel-14 [2] and can be implemented in a release independent way from Rel-13. |  |  |  |  |  |  |  |

For example, CA configuration CA_1A-19A was introduced in the Release 11 specifications. In order to implement a UE conforming to Release 10 but supporting the CA configuration CA_1A-19A, it is necessary for the UE to additionally conform to some parts of the Release 11 specifications (see corresponding Annexes of TS 36.307 Rel-11 which will point to the requirements in the Rel-11 of TS 36.101 [2] or TS 36.133 [3] to be fulfilled), such as the radio frequency and radio resource management requirements for the CA configuration CA_1A-19A.

Table 3A.2-3: Intra-band non-contiguous CA configurations

| Feature | DL/UL | number of sub-blocks | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 36.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| Intra-band non-contiguous CA configurations | DL | 2 | A, C, D | FDD, TDD | Rel-11 | Table B.2.3-1, Table B.3.2-1, Table B.4.5-1 |
|  | UL | 2 | A | FDD | Rel-11 | Table B.2.3-1, Table B.3.2-1, Table B.4.5-1 |
| NOTE1: The duplex mode "FDD, TDD" refers to a CA configuration composed by only FDD bands or only TDD bands, respectively. |  |  |  |  |  |  |

<End of change>