3GPP TSG RAN Meeting 80                            	              RP-180672

La Jolla, CA, USA June 11th-15st, 2018

# Source:	Qualcomm Incorporated

# Title:	New WID on LTE-based 5G Terrestrial Broadcast

# Document for:	Information

Agenda Item:	10.1.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	WID on LTE-based 5G Terrestrial Broadcast

## Acronym: LTE_TERR_BCAST

## Unique identifier:

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  |  |  |
| Don't know |  |  |  | X |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| x | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 710081 | eMBMS enhancements for LTE | Parent WID |

## 3	Justification

During Rel-14, the use cases and scenarios for eMBMS services based on LTE have been expanded to include terrestrial broadcasting (the feature also referred to as “EnTV”). Among others, LTE supports dedicated eMBMS (i.e., networks/eNB that only support eMBMS), deployments with larger intersite distance (by the introduction of new cyclic prefix of 200us), and other aspects such as network sharing and receive-only mode.

At the same time, the requirements for the 5G radio access were defined in TR 38.913. Given that LTE-based eMBMS has undergone a deep transformation in the Rel-14 such that many of the 5G requirements for dedicated broadcast networks may be already fulfilled, we propose to perform a gap-analysis accordingly. The gap analysis will compare the current LTE broadcasting capabilities with the requirements for 5G dedicated broadcast networks in TR 38.913. For those requirements that are not met by the latest version of theLTE standard, corresponding enhancements shall be specified.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

For the broadcast requirements in TR 38.913, and taking as baseline Rel-14 LTE:

- Identify which of the broadcast requirements in TR 38.913 are relevant for dedicated terrestrial broadcast networks [RAN1, RAN2].

  - Capture the analysis and potential solutions in a TR.

- Study and specify necessary enhancements (if any) needed to meet the identified requirements [RAN1, RAN2].

  - The work shall include but is not limited to studying/specifying the following, which will also be specified for non-dedicated networks:

    - For support of large cell radius, evaluate new and existing numerologies for deployments with cell radius of up to 100km and introduce necessary improvements into the specification. [RAN1]

    - Study and, if necessary, specify enhancements to support speeds of up to 250km/h [RAN4, RAN1].

### 4.2	Objective of Performance part WI

Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above mentioned enhancements and core requirements.

## 5	Expected Output and Time scale

| New specifications [If Study Item, one TR is anticipated] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title | 1st rsp. WG | 2nd rsp. WG(s) | Presented for information at plenary# | Approved at plenary # | Comments |
| TR 36.xxx | Evolved Universal Terrestrial Radio Access (E-UTRA);Analysis of 5G requirements for eMBMS | RAN1 | RAN2, RAN4 | RAN#83 | RAN#84 |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 36.201 |  | RAN#84 | Core part |
| 36.211 |  | RAN#84 | Core part |
| 36.212 |  | RAN#84 | Core part |
| 36.213 |  | RAN#84 | Core part |
| 36.214 |  | RAN#84 | Core part |
| 36.306 |  | RAN#84 | Core part |
| 36.331 |  | RAN#84 | Core part |
| 36.101 |  | RAN#84 | Core part |
| 36.104 |  | RAN#84 | Core part |
| 36.101 |  | RAN#86 | Performance part |
| 36.104 |  | RAN#86 | Performance part |

## 6	Work item Rapporteur(s)

-

## 7	Work item leadership

Primary: RAN1

Secondary: RAN2, RAN4

## 8	Aspects that involve other WGs

## 9	Supporting Individual Members

Supporting IM name