3GPP TSG RAN Meeting 80		 RP-180676

La Jolla, CA, U.S.A., June 11st – 14th, 2018

Agenda item:	9.8

Source: 	Qualcomm Incorporated

Title: 	NR UE Features

Document for:	Discussion/Decision

# Discussion

In this document, we discuss remaining issues and proposed changes to the Rel-15 NR UE feature list.

Note that this contribution only addresses eMBB related capabilities (not URLLC).

In this document we highlight some proposals, while the complete set of changes are listed in the attached feature list.

## Add UE Capability #2 for processing time

For UE Capability #2, the corresponding capability needs to be added in the UE feature list.

Proposal 1:

Add UE Capability # 2 for processing time capability indication in the UE feature list.

## Interpretation of Type 3

We propose the following:

Proposal 2:

Unless noted otherwise, the meaning of Type 3 is ‘per band per band combination’

## DL and UL CA operation

For intra-band contiguous CA, the number of different BWs of the different CCs should be limited. For example, instead of 50MHz + 80MHz +100MHz, use 30MHz + 100MHz + 100MHz.

6-5 Basic DL NR-NR CA operation

6-6 Basic UL NR-NR CA operation

| 6-5 | Basic DL NR-NR CA operation | 1) Up to16 DL carriers 2) Same numerology across carrier for data/control channel [at a given time] 3) Up to two different CC BWs for intra-band contiguous CA in a band |  | Yes |  | N.A. | N. A. | N.A. |  | This is conditioned on the support of DL CA band combination(s). The band combination definition is up to RAN4. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6-6 | Basic UL NR-NR CA operation | 1) Up to16 UL carriers 2) Same numerology across carrier for data/control channel [at a given time] 3) One PUCCH group 4) Single TAG 5) Up to two different CC BWs for intra-band contiguous CA in a band | [6-5] | Yes |  | N.A. | N.A. | N.A. |  | This is conditioned on the support of UL CA band combination(s). The band combination definition is up to RAN4. |

Proposal 3:

For intra-band contiguous DL or UL CA, required support is up to two different CC BWs in the band.

Note that this proposal may be addressed in the RAN4 specification.

## BWP configuration

When the UE has many CCs in a band, storing all configurations for all BWPs is excessive. To solve this, we propose limiting the number of different configurations.

In addition, the same condition – i.e. BWP includes BW of the initial DL BWP -- which was introduced to Type A BWP adaptation, should also be added to Basic BWP operation as well.

6-1 Basic BWP operation

| 6-1 | Basic BWP operation | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) Identical BWP (including all RRC parameters associated with BWP) configuration for equal BW SCells in a band |  | Yes |  | N.A. | N.A. | N.A. |  | This feature should be mandatory for at least BWPs which is the same as the set of specified channel BW RAN4 may discuss other BW requirements. UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP |  | [Mandatory without capability signaling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4:

UE is only required to support identical BWP configuration for SCells of equal BW within a band.

## Beam management

2-24 SSB/CSI-RS for beam measurement

| 2-24 | SSB/CSI-RS for beam measurement | 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_1 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 3. Supported density of CSI-RS 4. When intra-band NR CA is configured in a band, the same beam management is applied across all CCs in the band | 2-21, 2-22 or 2-23, 2-23a | Yes | RSRP measurement is not supported | Type 1 | No need | Yes |  |  |  | Component-1, candidate value set for MB_1 is {8, 16, 32, 64} Support MB_1 =8 is mandatory for at least for >6Ghz bands Component-2, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-3: candidate value set: {“1 only”, “3 only”, “both 1 and 3”} At least density of CSI-RS =3 is mandatory at least for FR2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5:

Due to the analog beamforming restriction, there is no need to perform individual beam management on each CC in intra-band contiguous CA. Therefore, in the UE capabilities, CCs participating in intra-band contiguous CA should be considered a single beam management entity.

## CSI-RS switching time

When triggering P3 procedure, the UE needs additional time for turning on Rx antennas that are not currently active. This feature is relevant in FR2, where the UE may employ antenna selection among multiple candidate antennas.

2-28 A-CSI-RS beam switching timing

Proposal 6:

For the A-CSI-RS beam switching timing, adopt Alt. 2.

Note that this proposal was also covered in [7].

## Capturing earlier agreement

In email discussion [92-NR-05] “Issues on UE features” after RAN1 #92, the following change was agreed that has been captured for 2-33 but not for 2-36 in the feature list.

| 2-36 | Type I single panel codebook | 1. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2. Supported Codebook Mode(s) […] | 2-35 | Yes | … | Type 3 | No need |  |  | Note: simultaneously doesn’t mean in the same slot |  | Component-1: the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256} Component-2 candidate values: {Mode-1, Mode-2} […] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7:

Capture the changes already agreed in email discussion [92-NR-05] after RAN1 #92 for feature 2-36.

## Limiting number of CSI reports per CC

There is already a capability associated with the number of reports supported across all CCs. There should be a limit on reports per CC as well, to accommodate UEs that process different CCs in different hardware blocks and have a processing limitation per hardware block.

2-35 CSI report framework

| 2-35 | CSI report framework | 1. Maximum number of periodic CSI report setting per BWP 2. Maximum number of aperiodic CSI report setting per BWP 3. Maximum number of semi-persistent CSI report setting per BWP 4. UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 5. UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. X and Y also count the SRS precoder derivation in case of reciprocity based SRS Tx | 2-32 | Yes | CSI report is not supported | Type 3 | N.A. | N.A. |  | NOTE: Other MIMO capability other than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update | Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5 candidate values: {from 5 to 32} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8:

Introduce a capability for number of simultaneous reports in a CC.

Proposal 9:

Resolve the FFS in 2-35 by including SRS related precoder derivation in the limits.

## Dynamic power sharing capability

Due to the issues discussed in [8], it is not yet clear if UEs that are generally dynamic power sharing capable would be able to perform dynamic power sharing or would benefit from dynamic power sharing in certain intra-band EN-DC cases. Therefore, it would be beneficial to make this capability Type 3. Not that this is irrespective of whether the feature is mandatory or not.

8-1 Dynamic power sharing for EN-DC

| 8-1 | Dynamic power sharing for LTE-NR DC | When total transmission power exceeds Pcmax, UE scales NR transmission power. | EN-DC | Yes |  | Type 3 | Yes | Yes |  |  | RAN |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 10:

Change the capability indication for 8-1 from Type 2 to Type 3.

## Beam reporting timing

For the case of 30 kHz SCS, the current beam reporting timing is too tight, change the upper limit to a larger value.

2-25 Beam reporting timing

| 2-25 | Beam reporting timing | 1. The number of symbols, Xi, between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RBi, where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-24 | Yes | Beam reporting time capability is not known by gNB | Type 1 | N.A. | N.A. |  |  | RAN1/4 | Candidate value sets: X1 is {2, 4, 8} X2 is {4, 8, 14, 28} X3 is {8, 14, 28} X4 is {14, 28, 56} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 11:

Add 28 symbol as a candidate value for 30kHz SCS beam reporting timing.

# Conclusions

In this contribution, the following proposals have been made:

Proposal 1:

Add UE Capability # 2 for processing time capability indication in the UE feature list.

Proposal 2:

Unless noted otherwise, the meaning of Type 3 is ‘per band per band combination’

Proposal 3:

For intra-band contiguous DL or UL CA, required support is up to two different CC BWs in the band.

Proposal 4:

UE is only required to support identical BWP configuration for SCells of equal BW within a band.

Proposal 5:

Due to the analog beamforming restriction, there is no need to perform individual beam management on each CC in intra-band contiguous CA. Therefore, in the UE capabilities, CCs participating in intra-band contiguous CA should be considered a single beam management entity.

Proposal 6:

For the A-CSI-RS beam switching timing, adopt Alt. 2.

Proposal 7:

Capture the changes already agreed in email discussion [92-NR-05] after RAN1 #92 for feature 2-36.

Proposal 8:

Introduce a capability for number of simultaneous reports in a CC.

Proposal 9:

Resolve the FFS in 2-35 by including SRS related precoder derivation in the limits.

Proposal 10:

Change the capability indication for 8-1 from Type 2 to Type 3.

Proposal 11:

Add 28 symbol as a candidate value for 30kHz SCS beam reporting timing.

# References

- “RAN1 Chair’s Notes”, RAN1 #93, May 25, 2018

- R1-1807891, “Updated NR UE features”, NTT DOCOMO, AT&T

- R1-1807896, “LS on NR UE feature list”, RAN1

- R1-1807898, “LS on Lead Time for Beam Switching”, RAN4

- R1-1807616, “NR Features and Capabilities”, Qualcomm

- R1-1807761, “Remaining issues for MIMO feature list”, Qualcomm

- RP-180678, “Support of power efficient panel switch UE feature”, Qualcomm

- R1-1805817, “Reply LS on SRS switching”, RAN4