3GPP TSG-RAN Meeting #80 	R4-180934

# La Jolla, USA, 11-14 June, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0011 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-2: Correction of the spherical coverage requirement for PC3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The spherical coverage requirement for PC3, as captured in the Editor’s CR, includes a note which defines the requirement’s applicability in the following way: “The requirements in this table are only applicable for UE which supports single band in FR2.” Given the use of both 28 GHz and 39 GHz bands in the US (namely, bands n261 and n260), the requirement may not fully address all potential Rel-15 NR deployments. An enhancement of this requirement is needed to define the applicability to UEs which support both bands n261 and n260. A multi-band framework to define spherical coverage requirements was discussed in in RAN4 in R4-1808215. Based on the technical analysis provided in the accompanying discussion paper (RP-180933), this CR proposes to let ΔTMBS = 1.6 dB, applicable to band n260, for UEs which support bands n261 and n260. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defines multi-band applicability of the spherical coverage requirement for PC3 with ΔTMBS = 1.6 dB, applicable to band n260, for UEs which support bands n261 and n260. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spherical coverage requirements for UEs which support bands n261 and n260 are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-2 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The changes in this CR are merged with the Editor’s CR 0010 (R4-1808531). The new content is Table 6.2.1.3-4 and the paragraph above it. |  |  |  |  |  |  |  |  |

<<<start of change>>>

### 6.2.1.3		UE maximum output power for power class 3

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The values listed on the table below are for handheld UE, defined as minimum peak EIRP. The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.3-1: UE minimum peak EIRP for power class 3

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 |  | 22.4 |  |
| n258 | 22.4 |
| n260 | 20.6 |
| n261 | 22.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance |  |

The maximum output power values for TRP and EIRP are found on the Table6.2.1.3-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1-2: UE maximum output power limits for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n260 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.3-3 below. The requirement is verified with the test metric of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.3-3: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50t%-tile CDF (dBm) |
| --- | --- |
| n257 | [11.5] |
| n258 | [11.5] |
| n260 | [8] |
| n261 | [11.5] |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are only applicable for UE which supports single band in FR2 |  |

Spherical coverage performance for UEs which support multiple bands is expected to be optimized over the entire supported range of frequencies and is relaxed from the single-band requirement in Table 6.2.1.3-3 by the factor ΔTMBS, which is defined in Table 6.2.1.3-4 below.

Table 6.2.1.3-4: UE spherical coverage requirement multi-band relaxations for power class 3

| Supported bands |  | ΔTMBS (dB) |
| --- | --- | --- |
| n261 and n260 |  | [1.6] (2) |
| NOTE 1: The requirements in this table are applicable for UEs which support only the indicated bands NOTE 2: Applicable to band n260 only |  |

<<<end of change>>>